MFA TR MF.101 V1.0.2 (2017-02)

Technical Report

MulteFire Alliance

MulteFire Deployment Scenarios and Requirements

(Release 1)

The present document has been developed within MulteFire Alliance and may be further elaborated for the purposes of the MulteFire Alliance.
The MulteFire Alliance accepts no liability for any use of this Specification.

Keywords

multefire, neutral host network, stage 2, NHN, release 1

MulteFire Alliance

Postal address

5177 Brandin Court

Fremont, CA 94538 USA

Internet

http://www.multefire.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016-2017, MulteFire Alliance

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

1	Scope	3

2	References	3

3	Definitions, symbols and abbreviations	3

3.1	Definitions	3

3.2	Symbols	3

3.3	Abbreviations	4

4	Overview	4

5	High-Level Assumptions	4

6	Deployment Scenarios	5

6.1	Standalone Deployment Scenario without External Interworking	5

6.1.1	Description of the deployment scenario	5

6.1.2	Related Use Cases	5

6.1.3	High-Level Requirements	6

6.2	Standalone Deployment Scenario with External Interworking	6

6.2.1	Description of the deployment scenario	6

6.2.2	Related Use Cases	7

6.2.3	High-Level Requirements	7

6.3	Tight 3GPP IW Deployment Scenario	8

6.3.1	Description of the deployment scenario	8

6.3.2	Related Use Cases	9

6.3.3	High-Level Requirements	9

6.4	Combination of deployment scenarios	10

6.4.1	Description of the deployment scenario	10

6.4.2	Related Use Cases	10

7	Summary	10

Annex A (informative): Change History	11

Annex B (informative): Regulatory Aspects	12

# 1	Scope

The present document provides a study on scenarios of deployment of MulteFire technology, and the corresponding network architectures needed to enable each deployment scenario.

Network architecture in the context of this document excludes protocols at the radio layer, e.g. RRC, MAC or PHY. Any changes to these protocols, and resulting changes to other protocols such as X2 or TR-069/196 are not considered in this document. This document includes requirements imposed on MulteFire RRC, MAC and PHY layers by the architecture, where applicable.

# 2	References

For interpretations of references, the defined rules in Section 4 of MFA TR MF.100 [0] apply.

[0]	MulteFire Alliance (MFA). Guidelines for MulteFire Specifications. MFA TR MF.100.

[1]	Third Generation Partnership Project (3GPP). Vocabulary for 3GPP Specifications. 3GPP TR 21.905.

[2]	MulteFire Alliance (MFA). MulteFire Alliance Architecture for PLMN Access Mode (Stage 2) (Release 1). MFA TS MF.201.

[3]	MulteFire Alliance (MFA). Architecture for Neutral Host Network Access Mode: Stage 2 (Release 1). MFA TS MF.202.

[4]	The Internet Engineering Task Force (IETF). Extensible Authentication Protocol (EAP), edited by B. Aboba et al., June 2004. IETF RFC 3748, also available at https://tools.ietf.org/html/rfc3748.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions described in 3GPP TR 21.905 [1], MFA TS MF.201 [2], and the following apply. A term defined in the present document takes precedence over the definition of the same term as described in 3GPP TR 21.905 [1] and MFA TS MF.201 [2].

| Term | Definition |
| --- | --- |
| Mobile Network Operator | A mobile network operator who uses licensed spectrum. |
| MulteFire | An LTE-based wireless system operating in exclusively unlicensed spectrum that is defined in specifications provided by the MulteFire Forum. |
| MulteFire Access Point | An entity providing MulteFire radio services. It has similar role in MuLTE architecture as an eNB in 3GPP architecture. An MulteFire Access Point consists of one or more MulteFire cells. |
| Neutral Host MulteFire Network | A network based on MulteFire radio technology intended to service to subscribers from one or more participating service providers. |
| Participating Service Provider | A provider of credentials and billing support that enables access to a MulteFire network. |

## 3.2	Symbols

## 3.3	Abbreviations

| Abbreviation | Definition |
| --- | --- |
| AAA | Authentication, Authorization, and Accounting |
| CN | Core Network |
| ISP | Internet Service Provider |
| LBT | Listen-before-Talk |
| MF | MulteFire |
| MF-AP | MulteFire Access Point |
| MNO | Mobile Network Operator |
| PWS | Public Warning System |
| RAN | Radio Access Network |

# 4	Overview

MulteFire technology is designed to bring the benefits of LTE technology to unlicensed spectrum. The design objective for MulteFire network architecture is to largely reuse the existing architecture defined by 3GPP. The changes from existing 3GPP architecture specifications depend on the deployment scenario.

This document provides a high-level description of deployment scenarios and associated use-cases for MulteFire deployments. The deployment scenarios are described from a network architecture perspective with a focus on architecture requirements. Physical topology related use cases including aspects such as link budget and fading are out of scope for this document.

# 5	High-Level Assumptions

The following general assumptions are used during the specification of MulteFire:

- There are no restrictions to the implementation of MulteFire network functional elements: they can be implemented as separate entities, or they can be combined during the implementation in any manner (e.g., it is possible that a full network is implemented as a single node).

- The interfaces between the MF RAN and MF CN, and between MF-APs will be specified by the MulteFire Alliance and 3GPP Release 13-based interfaces, in order to enable multivendor deployments.

- The target is that interworking with the MF network does not require any MulteFire-specific features in the MNO network. However, it is possible that some enhanced features or improved performance can only be supported if the MNO network supports some MulteFire-specific features. In those cases, any impacts to the MNO network shall be minimized.

- MulteFire targets one design for unlicensed band including 1.9 GHz (Japan), 3.5 GHz, and 5 GHz. Design focuses on 5 GHz, are more challenging, coexistence-wise.

- The baseline specification for radio is 3GPP Release 13. As a guiding principle, specifications should consider design aspects defined in 3GPP Release 14 whenever appropriate.

- The specification assumes a minimum carrier bandwidth of 10 MHz.

- Design is optimized for low- to medium-mobility scenarios, and should be able to support higher speed
(e.g. 50 kph).

- Maximum transmit power is governed by regulatory bodies in the different regions.

- Design optimizes performance for small cell. Support for macro deployment is to be determined.

# 6	Deployment Scenarios

## 6.1	Standalone Deployment Scenario without External Interworking

## 6.1.1	Description of the deployment scenario

This standalone deployment scenario allows a MulteFire deployment to provide IP services without interworking provided by any external network. A standalone MulteFire deployment includes all the radio and core network functionalities needed to provide IP services. Figure6.1.1-1 shows this deployment scenario. The MulteFire network consists of a MulteFire RAN connected to a MulteFire CN. The MulteFire CN enables access to IP services, session and mobility management services, user authentication, network management and charging. A MulteFire RAN can consist of one or more potentially interconnected MF-APs, and the MF network supports seamless mobility among them in a similar manner as mobility is supported within a 3GPP RAN. It is possible to build an entire network into a single 
MF-AP box, similar to standalone WLAN or utilize more hierarchical approach as in LTE.

Figure 6.1.1-1: MulteFire Standalone Deployment Scenario without External Interworking

## 6.1.2	Related Use Cases

Potential use cases for such a deployment include:

- A MulteFire network deployed for special purpose networks in industrial, mining, off-shore or other isolated environments.

- A MulteFire network deployed to serve fixed UEs that remain within the coverage of MulteFire (e.g., indoor automation).

- A MulteFire network deployed in an enterprise for enterprise-specific services to employees when interworking (e.g., external authentication or mobility) with other networks is not required. Internet connectivity and access to MNO service via the Internet is not excluded in this scenario, but this is not part of the services of the MulteFire network.

- A MulteFire network deployed in a residential scenario when interworking (e.g., external authentication or mobility) with other networks is not required. Internet connectivity and access to MNO service via the Internet is not excluded in this scenario, but this is not part of the services of the MulteFire network.

- A MulteFire network deployed to be used as wireless backhaul for connecting eNBs to a 3GPP CN.

## 6.1.3	High-Level Requirements

| Feature | Description |
| --- | --- |
| A.11 | The UE and the MulteFire network shall perform mutual authentication. Note: EAP-based methods for authentication are recommended (see IETF RFC 3748 [4]). |
| A.12 | Selection and authentication to the MulteFire network are made transparent (visible) to the user. |
| A.13 | Manual network selection shall be supported. Note: Online signup support is needed to support this requirement. |
| A.21 | A UE accessing the MulteFire network shall be able to access the local IP services provided by the MulteFire network. |
| A.22 | The MulteFire specifications shall enable to provide different QoS levels for different IP flows (similar to LTE). |
| A.23 | The MulteFire network shall be able to provide different service levels (similar to LTE) for different users, based on subscriptions. Note: The use of this feature depends on the actual deployment. |
| A.24 | The MulteFire network shall provide mobility experience similar to LTE within the MulteFire network. This includes that the IP address is preserved during mobility events within the MulteFire network. Note: The seamless mobility may be restricted geographically (e.g., An operator may deploy separate MulteFire networks, and mobility between these networks may not be supported). |
| A.31 | A UE may access MulteFire network and a MNO network via 3GPP RAN simultaneously. |
| A.32 | The MulteFire network may collect charging data for subscribers. |
| A.33 | The MulteFire network may provide similar level of security as a network based on 3GPP specifications. |

## 6.2	Standalone Deployment Scenario with External Interworking

## 6.2.1	Description of the deployment scenario

This deployment scenario is based on the previous deployment, and the main role of MulteFire network is to provide IP services. In this scenario, the standalone MulteFire Network is connected to external entities. Figure 6.2.1-1 shows this deployment scenario. The MulteFire network can interact with external AAAs of Participating Service Provider(s) for Authentication, Authorization and Accounting. In the case when the Participating Service Provider is an MNO, SIM card-based credentials are used. In other cases, other credentialing protocols can be used.

Figure 6.2.1-1: MulteFire Standalone Deployment Scenario with External Interworking

The MF CN may be connected to an IP network that is also connected to a 3GPP EPC. In this scenario, the MF CN provides access to its subscribers to the MNO IP Services. However in this deployment scenario this is a service of 3GPP EPC and not provided by the MulteFire network.

Mobility between the MNO network and MulteFire network is achieved by the UE establishing separate sessions with the two networks. To optimize mobility, the UE may be able to maintain two sessions simultaneously, either by dual-radio operation capability, or by switching a single radio between the two networks.

## 6.2.2	Related Use Cases

This deployment scenario enables to provide:

- IP access to users of Participating Service Providers (e.g., MNOs, ISPs)

- Access to MNO IP services for MNO users using the IP access.

Potential use cases for such a deployment include

- A MulteFire network deployed in a venue or public space providing service to subscribers of multiple MNOs (Neutral Host/DAS replacement).

- A MulteFire network deployed over a large area (e.g., Smart City) providing service to subscribers of multiple MNOs (such as a Neutral Host/RAN-sharing replacement).

- A MulteFire network deployed in a venue or public space to offer service for non-MNO types of Participating Service Providers (e.g., a retail model).

- A MulteFire network deployed in an enterprise for enterprise-specific services to employees when interworking (e.g., external authentication or mobility) with other systems is required.

## 6.2.3	High-Level Requirements

| Feature | Description |
| --- | --- |
| B.11 | A UE from any Participating Service Provider shall authenticate to the MulteFire network. When the Participating Service Provider is a MNO, then the UE shall authenticate to the MulteFire network using SIM card-based credentials. Note: EAP-based methods for authentication are recommended (see IETF RFC 3748 [4]). |
| B.12 | Same as A.12 |
| B.13 | Same as A.13 |
| B.14 | A single MulteFire network deployment may be connected to several Participating Service Providers including MNOs. |
| B.15 | A Participating MNO shall be able to control the policy/thresholds used by the UE to initiate a connection with a MulteFire network. |
| B.21 | Same as A.21 |
| B.22 | A UE with MNO credentials shall be able to access MNO services (e.g., IMS) hosted by the corresponding MNO while connected to the MulteFire network if the MNO supports it. The MNO network can be a 3GPP Release-13 or earlier deployment. |
| B.23 | Same as A.24 Note: The MulteFire network does not support session continuity (IP address preservation) between 3GPP and the MF RAN. However, the 3GPP network may provide session continuity with IP address preservation for MNO IP Services, using the PGW as an anchor point. |
| B.24 | Same as A.22 |
| B.25 | Same as A.23 |
| B.31 | Same as A.31 |
| B.32 | The MulteFire shall collect accounting information and make it available to Participating Service Providers when requested. |
| B.33 | Same as A.33 |

## 6.3	Tight 3GPP IW Deployment Scenario

## 6.3.1	Description of the deployment scenario

In this deployment scenario the MulteFire network is connected to the 3GPP CN, and cannot provide IP services independently from the MNO network (see Figure 6.3.1-1). The MulteFire RAN also uses the same identifiers (e.g., PLMN ID) as the 3GPP RAN. Session continuity with IP address preservation is provided between the MF RAN and the 3GPP RAN. In order to support seamless mobility there may be a direct interface between the MF RAN and the 3GPP RAN.

This type of deployment is also possible for operators that do not have 3GPP RAN.

Figure 6.3.1-1: Tight 3GPP Interworking Deployment Scenario

## 6.3.2	Related Use Cases

Potential use cases enabled by this deployment option are:

- A MulteFire RAN operating as a residential Femto.

- A MulteFire RAN deployed in an enterprise and provided by the MNO.

- A MulteFire RAN used to extend the MNO network coverage. This is useful where licensed spectrum is not available for the MNO.

- A MulteFire RAN deployed in a venue providing LTE service to multiple MNOs (e.g., DAS replacement).

- A MulteFire RAN deployed over a large area (e.g., Smart City) providing service to subscribers of multiple MNOs (RAN-sharing replacement)

- A MulteFire RAN provided by a MNO provides access to subscribers of one or more MNOs.

## 6.3.3	High-Level Requirements

| Feature | Description |
| --- | --- |
| C.11 | A UE from any participating MNO shall authenticate to the MulteFire network using EPS authentication as in LTE. |
| C.12 | Selection and authentication to the MulteFire network are made transparent to the user. |
| C.13 | A single MulteFire network deployment may be connected to several Participating MNOs. |
| C.21 | A UE from a participating MNO can access MNO services (e.g., IMS) hosted by the corresponding MNO while connected to the MulteFire network. |
| C.22 | The MulteFire network shall provide mobility experience similar to LTE within the MulteFire network. This includes that the IP address is preserved during mobility events within the MulteFire network. |
| C.23 | The UE shall have session continuity with IP address preservation between the MulteFire RAN and the MNO 3GPP RAN. |
| C.24 | The MulteFire network can provide different service levels (similar to LTE) for different users, based on subscriptions. |
| C.25 | The MulteFire network shall be able to provide different QoS levels for different IP flows (similar to LTE). |
| C.26 | The MulteFire network shall be able to provide connectivity to different external PDNs (such as LTE). |
| C.32 | The provided security level shall be the same as in an MNO system. |
| C.33 | The 3GPP CN can determine when the UE is connected via the MF RAN. |
| C.34 | Access can be restricted (including handover) to the MF RAN. |

## 6.4	Combination of deployment scenarios

## 6.4.1	Description of the deployment scenario

A single MulteFire network can support Standalone Deployment Scenario and Tight 3GPP IW Deployment Scenario, as shown in Figure 6.5.1-1. In the combined deployments at the point of attachment, the UE request decides how the network is accessed by a specific UE; this will not change during the lifetime of the session.

Figure 6.5.1-1: Combination of Standalone Deployment Scenario and Tight 3GPP IW Deployment Scenario

## 6.4.2	Related Use Cases

Potential use cases enabled by this deployment option are:

- A MulteFire operator has different business agreements with different operators.

# 7	Summary

The architecture specifications in MFA TS MF.201 [2] and MFA TS MF.202 [3] address the deployment scenarios presented in the previous sections. MFA TS MF.201 [2] describes solutions for the Standalone Deployment Scenario without External Interworking and Tight 3GPP IW Deployment scenarios (Sections 6.1 and 6.3, respectively). MFA TS MF.202 [3] describes solutions for the Standalone Deployment Scenarios (Sections 6.1 and 6.2).

# Annex A (informative): Change History

| Date | Document Number | Comment |
| --- | --- | --- |
| 2016-11 | MFA TR MF.101 v1.0.0 | Initial Release-1 Version |
| 2017-01 | MFA TR MF.101 v1.0.1 | Approved version from MFA #8 incorporating changes: mf2016.1016 |
| 2017-02 | MFA TR MF.101 v1.0.2 | Approved version from MFA #9 incorporating changes: mf2017.059 |

# Annex B (informative): Regulatory Aspects

This section provides an informative overview on the regulatory requirements that should be considered for MulteFire. The information contained here does not in any way replace nor complement the corresponding regulatory requirements and related legislation of each country.

The basic model is that the one who provides the service carries the responsibility to implement the regulatory requirements. These requirements can be broadly divided into following categories (non-exhaustive):

  - Spectrum usage

  - Service quality, availability and billing

  - Emergency calls

  - Emergency alerting

  - Privacy and other product security requirements

  - Lawful Interception

Spectrum usage-related requirements concerning MulteFire are in most countries less restrictive than with regular LTE, due to operation on unlicensed spectrum only. However, while devices operating on the unlicensed band do not have full regulatory protection against interference from other users on the same band, fair co-existence policy with WiFi can be required in many countries. This can mean Listen-before-Talk (LBT) operation as well as testing, measuring and certifying the co-existence capabilities. In some countries, the regulations can be more restrictive and can concern performance, modulation and other aspects of the radio interface.

Regulatory requirements related to the service quality, availability and billing and emergency calls are typically associated to the service provided, not to the network technology or spectrum used.

Key factors determining the requirements typically are:

  - Is the provided service for private or public use?

  - Is the provided service for Internet access or for telephony services?

  - Does the provided telephony service concern fixed or mobile telephony?

Typically, private services are very lightly regulated, whereas public telephony services are tightly regulated, including support for emergency calls with location information, service availability requirements, and billing accuracy requirements. Exact definitions and interpretations of these definitions can vary country-by-country. The MulteFire network in itself provides only internet access with other services (such as VoLTE) provided by the operator core network. Thus, MulteFire needs to support those regulatory requirements relevant to the internet service provider and those relevant to an access network operator connected to mobile core network that provides VoLTE service. The latter means typically location information. This approach is especially relevant for the Standalone Deployment Scenario with External Interworking (Section 6.2), when the MulteFire network only provides Internet access and may have no business relation to the actual voice service provider.

Emergency alerting services concerning natural disasters (e.g., earthquake, tsunami or flood), or other events requiring alerting the general public, have traditionally relied on radio and television broadcasts. Given the current role of mobile phones and other similar devices, several countries have taken into use wireless emergency alerting systems enabled to send warning messages to mobile devices using cell broadcasting technology of cellular networks. While typically mobile network operator participation to these services is optional, they may be required in some countries. 3GPP has specified requirements for a Public Warning System (PWS). These requirements are relevant to MulteFire mainly when MulteFire is connected to a regular EPC to extend a licensed band LTE network.

Privacy of consumer data (including logs and traces) and other product security-related requirements—such as logging of remote access, security testing and vulnerability reporting—are required in some countries. These can be regarded as implementation requirements, and as such are out-of-scope for MulteFire specifications.

Lawful interception is a process in which a communications network operator or service provider gives authorized officials access to the communications of individuals or organizations. This access includes both the content of the communication, and any related metadata such as location and time. Most countries have the view that lawful interception must be standards-based in order to enable interoperability and simplify cooperation between the authorities and operators and between the authorities of different countries. ETSI has been a major driver behind the specification of related interfaces and of the flow that intercepted data should follow. ETSI has specified a general architecture for lawful interception that allows communication between network operators and authorities over the defined interfaces and in compliance with national legislation. This general architecture applies to any kind of circuit- or packet-switched voice and data networks. These ETSI specifications are now in use globally in a large number of countries that require the Lawful Interception of telecommunications. MulteFire must comply with these ETSI specifications especially, in the case when MulteFire is connected to a regular EPC and to extend a licensed band LTE network.