MFA TS 36.211 V1.0.3 (2017-09)

Technical Specification

MulteFire Alliance

Evolved Universal Terrestrial Radio Access (E-UTRA);

Physical Channels and Modulation

(Release 1.0)

The present document has been developed within MulteFire Alliance and may be further elaborated for the purposes of the MulteFire Alliance.
The MulteFire Alliance accepts no liability for any use of this Specification.

Keywords

MF, multefire, radio, layer 1

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016-2017, MulteFire Alliance

This specification is a modification of 3GPP TS 36.211, Version 13.5.0 © 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC) from which permission has been received.

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

MulteFire Alliance

Postal address

5177 Brandin Court

Fremont, CA 94538 USA

Internet

http://www.multefire.org

Contents

Foreword	9

1	Scope	10

2	References	11

3	Symbols and abbreviations	12

3.1	Symbols	12

3.2	Abbreviations	16

4	Frame structure	17

4.1	Frame structure type 1	17

4.2	Frame structure type 2	17

4.3	Frame structure type 3	19

5	Uplink	20

5.1	Overview	20

5.1.1	Physical channels	20

5.1.2	Physical signals	20

5.2	Slot structure and physical resources	20

5.2.1	Resource grid	20

5.2.2	Resource elements	22

5.2.3	Resource blocks	22

5.2.4	Narrowbands	23

5.2.5	Guard period for narrowband retuning	23

5.3	Physical uplink shared channel	24

5.3.1	Scrambling	24

5.3.2	Modulation	25

5.3.2A	Layer mapping	25

5.3.2A.1	Layer mapping for transmission on a single antenna port	26

5.3.2A.2	Layer mapping for spatial multiplexing	26

5.3.3	Transform precoding	26

5.3.3A	Precoding	27

5.3.3A.1	Precoding for transmission on a single antenna port	27

5.3.3A.2	Precoding for spatial multiplexing	27

5.3.4	Mapping to physical resources	29

5.4	Physical uplink control channel	32

5.4.1	PUCCH formats 1, 1a and 1b	33

5.4.2	PUCCH formats 2, 2a and 2b	35

5.4.2A	PUCCH format 3	37

5.4.2B	PUCCH format 4	38

5.4.2C	PUCCH format 5	39

5.4.2MF1	MF-sPUCCH format 0	40

5.4.2MF2	MF-sPUCCH formats 1, 2, 3	41

5.4.2MF3	MF-ePUCCH format 1	45

5.4.3	Mapping to physical resources	48

5.5	Reference signals	51

5.5.1	Generation of the reference signal sequence	51

5.5.1.1	Base sequences of length  or larger	51

5.5.1.2	Base sequences of length less than 	52

5.5.1.3	Group hopping	55

5.5.1.4	Sequence hopping	55

5.5.1.5	Determining virtual cell identity for sequence generation	56

5.5.2	Demodulation reference signal	56

5.5.2.1	Demodulation reference signal for PUSCH	56

5.5.2.1.1	Reference signal sequence	56

5.5.2.1.2	Mapping to physical resources	60

5.5.2.2	Demodulation reference signal for PUCCH	60

5.5.2.2.1	Reference signal sequence	60

5.5.2.2.1MF1	Reference signal sequence for MF	62

5.5.2.2.2	Mapping to physical resources	65

5.5.2.2.2MF1	Mapping to physical resources for MF	65

5.5.3	Sounding reference signal	66

5.5.3.1	Sequence generation	66

5.5.3.2	Mapping to physical resources	66

5.5.3.3	Sounding reference signal subframe configuration	69

5.5.3MF1	Sounding reference signal for MF UE	70

5.5.3MF1.1	Sequence generation	70

5.5.3MF1.2	Mapping to physical resources	72

5.6	SC-FDMA baseband signal generation	73

5.7	Physical random access channel	74

5.7.1	Time and frequency structure	74

5.7.2	Preamble sequence generation	82

5.7.3	Baseband signal generation	87

5.8	Modulation and upconversion	89

6	Downlink	90

6.1	Overview	90

6.1.1	Physical channels	90

6.1.2	Physical signals	90

6.2	Slot structure and physical resource elements	91

6.2.1	Resource grid	91

6.2.2	Resource elements	92

6.2.3	Resource blocks	93

6.2.3.1	Virtual resource blocks of localized type	94

6.2.3.2	Virtual resource blocks of distributed type	94

6.2.4	Resource-element groups	95

6.2.4A	Enhanced Resource-Element Groups (EREGs)	96

6.2.5	Guard period for half-duplex FDD operation	96

6.2.6	Guard Period for TDD Operation	97

6.2.7	Narrowbands	97

6.2.8	Guard period for narrowband retuning	97

6.3	General structure for downlink physical channels	97

6.3.1	Scrambling	98

6.3.2	Modulation	99

6.3.3	Layer mapping	100

6.3.3.1	Layer mapping for transmission on a single antenna port	100

6.3.3.2	Layer mapping for spatial multiplexing	100

6.3.3.3	Layer mapping for transmit diversity	101

6.3.4	Precoding	102

6.3.4.1	Precoding for transmission on a single antenna port	102

6.3.4.2	Precoding for spatial multiplexing using antenna ports with cell-specific reference signals	102

6.3.4.2.1	Precoding without CDD	102

6.3.4.2.2	Precoding for large delay CDD	103

6.3.4.2.3	Codebook for precoding and CSI reporting	104

6.3.4.3	Precoding for transmit diversity	105

6.3.4.4	Precoding for spatial multiplexing using antenna ports with UE-specific reference signals	106

6.3.5	Mapping to resource elements	107

6.4	Physical downlink shared channel	107

6.4.1	Physical downlink shared channel for BL/CE UEs	109

6.5	Physical multicast channel	111

6.6	Physical broadcast channel	111

6.6.1	Scrambling	111

6.6.2	Modulation	111

6.6.3	Layer mapping and precoding	112

6.6.4	Mapping to resource elements	112

6.7	Physical control format indicator channel	113

6.7.1	Scrambling	114

6.7.2	Modulation	114

6.7.3	Layer mapping and precoding	115

6.7.4	Mapping to resource elements	115

6.8	Physical downlink control channel	115

6.8.1	PDCCH formats	115

6.8.2	PDCCH multiplexing and scrambling	116

6.8.3	Modulation	116

6.8.4	Layer mapping and precoding	116

6.8.5	Mapping to resource elements	117

6.8A	Enhanced physical downlink control channel	118

6.8A.1	EPDCCH formats	118

6.8A.2	Scrambling	119

6.8A.3	Modulation	119

6.8A.4	Layer mapping and precoding	120

6.8A.5	Mapping to resource elements	120

6.8B	MTC physical downlink control channel	121

6.8B.1	MPDCCH formats	121

6.8B.2	Scrambling	122

6.8B.3	Modulation	123

6.8B.4	Layer mapping and precoding	123

6.8B.5	Mapping to resource elements	123

6.9	Physical hybrid ARQ indicator channel	125

6.9.1	Modulation	126

6.9.2	Resource group alignment, layer mapping and precoding	127

6.9.3	Mapping to resource elements	128

6.10	Reference signals	130

6.10.1	Cell-specific Reference Signal (CRS)	131

6.10.1.1	Sequence generation	131

6.10.1.2	Mapping to resource elements	131

6.10.2	MBSFN reference signals	134

6.10.2.1	Sequence generation	134

6.10.2.2	Mapping to resource elements	135

6.10.3	UE-specific reference signals associated with PDSCH	136

6.10.3.1	Sequence generation	137

6.10.3.2	Mapping to resource elements	138

6.10.3A	Demodulation reference signals associated with EPDCCH or MPDCCH	143

6.10.3A.1	Sequence generation	143

6.10.3A.2	Mapping to resource elements	144

6.10.4	Positioning reference signals	146

6.10.4.1	Sequence generation	146

6.10.4.2	Mapping to resource elements	147

6.10.4.3	Positioning reference signal subframe configuration	148

6.10.5	CSI reference signals	148

6.10.5.1	Sequence generation	149

6.10.5.2	Mapping to resource elements	150

6.10.5.3	CSI reference signal subframe configuration	159

6.11	Synchronization signals	160

6.11.1	Primary synchronization signal (PSS)	160

6.11.1.1	Sequence generation	160

6.11.1.2	Mapping to resource elements	160

6.11.1MF1	MF Primary Synchronization Signal (MF-PSS)	161

6.11.1MF1.1	Sequence generation	161

6.11.1MF1.2	Mapping to resource elements	161

6.11.2	Secondary synchronization signal (SSS)	162

6.11.2.1	Sequence generation	162

6.11.2.2	Mapping to resource elements	164

6.11.2MF1	MF secondary synchronization signal (MF-SSS)	164

6.11.2MF1.1	Sequence generation	164

6.11.2MF1.2	Mapping to resource elements	164

6.11A	Discovery signal	164

6.12	OFDM baseband signal generation	166

6.13	Modulation and upconversion	166

7	Generic functions	168

7.1	Modulation mapper	168

7.1.1	BPSK	168

7.1.2	QPSK	168

7.1.3	16QAM	168

7.1.4	64QAM	169

7.1.5	256QAM	171

7.2	Pseudo-random sequence generation	172

8	Timing	173

8.1	Uplink-downlink frame timing	173

9	Sidelink	175

9.1	Overview	175

9.1.1	Physical channels	175

9.1.2	Physical signals	175

9.1.3	Handling of simultaneous sidelink and uplink/downlink transmissions	175

9.2	Slot structure and physical resources	176

9.2.1	Resource grid	176

9.2.2	Resource elements	176

9.2.3	Resource blocks	177

9.2.4	Resource pool	177

9.2.5	Guard period	177

9.3	Physical Sidelink Shared Channel	177

9.3.1	Scrambling	177

9.3.2	Modulation	178

9.3.3	Layer mapping	178

9.3.4	Transform precoding	178

9.3.5	Precoding	178

9.3.6	Mapping to physical resources	178

9.4	Physical Sidelink Control Channel	179

9.4.1	Scrambling	179

9.4.2	Modulation	179

9.4.3	Layer mapping	179

9.4.4	Transform precoding	179

9.4.5	Precoding	180

9.4.6	Mapping to physical resources	180

9.5	Physical Sidelink Discovery Channel	180

9.5.1	Scrambling	180

9.5.2	Modulation	180

9.5.3	Layer mapping	180

9.5.4	Transform precoding	180

9.5.5	Precoding	180

9.5.6	Mapping to physical resources	180

9.6	Physical Sidelink Broadcast Channel	181

9.6.1	Scrambling	181

9.6.2	Modulation	181

9.6.3	Layer mapping	181

9.6.4	Transform precoding	181

9.6.5	Precoding	181

9.6.6	Mapping to physical resources	181

9.7	Sidelink Synchronization Signals	182

9.7.1	Primary sidelink synchronization signal	182

9.7.1.1	Sequence generation	182

9.7.1.2	Mapping to resource elements	182

9.7.2	Secondary sidelink synchronization signal	182

9.7.2.1	Sequence generation	182

9.7.2.2	Mapping to resource elements	182

9.8	Demodulation reference signals	183

9.9	SC-FDMA baseband signal generation	185

9.10	Timing	185

10	Narrowband IoT	187

10.1	Uplink	187

10.1.1	Overview	187

10.1.1.1	Physical channels	187

10.1.1.2	Physical signals	187

10.1.2	Slot structure and physical resources	187

10.1.2.1	Resource grid	187

10.1.2.2	Resource elements	188

10.1.2.3	Resource unit	188

10.1.3	Narrowband physical uplink shared channel	188

10.1.3.1	Scrambling	188

10.1.3.2	Modulation	189

10.1.3.3	Layer mapping	189

10.1.3.4	Transform precoding	189

10.1.3.5	Precoding	189

10.1.3.6	Mapping to physical resources	189

10.1.4	Demodulation reference signal	190

10.1.4.1	Reference signal sequence	190

10.1.4.1.1	Reference signal sequence for 	190

10.1.4.1.2	Reference signal sequence for 	191

10.1.4.1.3	Group hopping	192

10.1.4.2	Mapping to physical resources	193

10.1.5	SC-FDMA baseband signal generation	193

10.1.6	Narrowband physical random access channel	194

10.1.6.1	Time and frequency structure	194

10.1.6.2	Baseband signal generation	196

10.1.7	Modulation and upconversion	196

10.2	Downlink	196

10.2.1	Overview	196

10.2.1.1	Physical channels	196

10.2.1.2	Physical signals	197

10.2.2	Slot structure and physical resource elements	197

10.2.2.1	Resource grid	197

10.2.2.2	Resource elements	197

10.2.2.3	Guard period for half-duplex FDD operation	197

10.2.3	Narrowband physical downlink shared channel	197

10.2.3.1	Scrambling	197

10.2.3.2	Modulation	197

10.2.3.3	Layer mapping and precoding	197

10.2.3.4	Mapping to resource elements	198

10.2.4	Narrowband physical broadcast channel	198

10.2.4.1	Scrambling	198

10.2.4.2	Modulation	198

10.2.4.3	Layer mapping and precoding	199

10.2.4.4	Mapping to resource elements	199

10.2.5	Narrowband physical downlink control channel	199

10.2.5.1	NPDCCH formats	199

10.2.5.2	Scrambling	199

10.2.5.3	Modulation	199

10.2.5.4	Layer mapping and precoding	200

10.2.5.5	Mapping to resource elements	200

10.2.6	Narrowband reference signal (NRS)	200

10.2.6.1	Sequence generation	201

10.2.6.2	Mapping to resource elements	201

10.2.7	Synchronization signals	202

10.2.7.1	Narrowband primary synchronization signal (NPSS)	202

10.2.7.1.1	Sequence generation	202

10.2.7.1.2	Mapping to resource elements	203

10.2.7.2	Narrowband secondary synchronization signal (NSSS)	203

10.2.7.2.1	Sequence generation	203

10.2.7.2.2	Mapping to resource elements	204

10.2.8	OFDM baseband signal generation	204

10.2.9	Modulation and upconversion	205

Annex A (informative): Change history	206

Annex A-MF1 (informative): Change history for MF Specification	213

# Foreword

This Technical Specification has been produced by the MulteFire Alliance (MFA).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as documented in MFA TR MF.100 [MF-1].

# 1	Scope

The present document describes the physical channels for evolved UTRA.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	MFA TS 36.201: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE physical layer; General description".

[3]	MFA TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[4]	MFA TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[5]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements".

[6]	MFA TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[7]	MFA TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[8]	MFA TS 36.321, "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[9]	MFA TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) Protocol specification”

[10]	MFA TS 36.304, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”

[MF-1]	MFA TR MF.100, “Guidelines for MulteFire Specifications (Release 1)”

# 3	Symbols and abbreviations

## 3.1	Symbols

For the purposes of the present document, the following symbols apply:

Resource element with frequency-domain index  and time-domain index

Value of resource element  [for antenna port]

Matrix for supporting cyclic delay diversity

Density of random access opportunities per radio frame

Carrier frequency

PRACH resource frequency index within the considered time-domain location

PRACH frequency hopping offset, expressed as a number of resource blocks

Start symbol in slot 0 for NPDCCH

Start symbol in slot 0 for NPDSCH

Bandwidth for PSBCH transmission, expressed as a number of subcarriers

Bandwidth for PSBCH transmission, expressed as a number of resource blocks

Bandwidth for PSCCH transmission, expressed as a number of subcarriers

Bandwidth for PSCCH transmission, expressed as a number of resource blocks

Bandwidth for PSDCH transmission, expressed as a number of subcarriers

Bandwidth for PSDCH transmission, expressed as a number of resource blocks

Scheduled bandwidth for PSSCH transmission, expressed as a number of subcarriers

Scheduled bandwidth for PSSCH transmission, expressed as a number of resource blocks

Scheduled bandwidth for uplink transmission, expressed as a number of subcarriers

Scheduled bandwidth for uplink transmission, expressed as a number of resource blocks

Scheduled number of repetitions of a NPUSCH transmission

Scheduled number of repetitions of a NPDSCH transmission

Scheduled bandwidth for uplink NPUSCH transmission, expressed as a number of subcarriers

Number of repetitions of identical slots for NPUSCH

Number of coded bits to transmit on a physical channel [for codeword ]

Number of modulation symbols to transmit on a physical channel [for codeword ]

Number of modulation symbols to transmit per layer for a physical channel

Number of modulation symbols to transmit per antenna port for a physical channel

A constant equal to 2048 for , 4096 for  and 8192 for

Downlink cyclic prefix length for OFDM symbol  in a slot

Cyclic shift value used for random access preamble generation

Number of cyclic shifts used for PUCCH formats 1/1a/1b in a resource block with a mix of formats 1/1a/1b and 2/2a/2b

Bandwidth available for use by PUCCH formats 2/2a/2b, expressed in multiples of

The offset used for PUSCH frequency hopping, expressed in number of resource blocks (set by higher layers)

Physical layer cell identity

Narrowband physical layer cell identity

MBSFN area identity

Physical layer sidelink synchronization identity

Downlink bandwidth configuration, expressed in multiples of

Smallest downlink bandwidth configuration, expressed in multiples of

Largest downlink bandwidth configuration, expressed in multiples of

Uplink bandwidth configuration, expressed in multiples of

Smallest uplink bandwidth configuration, expressed in multiples of

Largest uplink bandwidth configuration, expressed in multiples of

Sidelink bandwidth configuration, expressed in multiples of

Number of scheduled subframes for NPDSCH transmission

Number of symbols for NPSS in a subframe

Number of symbols for NSSS in a subframe

Number of consecutive subcarriers in an UL resource unit for NB-IoT

Number of reference signal sequences available for the UL resource unit size

Number of scheduled UL resource units for NB-IoT

Total number of uplink narrowbands

Number of subcarriers in the frequency domain for NB-IoT

Number of consecutive absolute subframes over which the scrambling sequence stays the same

Total number of absolute subframes a PUSCH with repetition spans , expressed as a number of absolute subframes

Number of repetititions of a PUSCH transmission

Number of consecutive absolute subframes over which PUCCH or PUSCH stays at the same narrowband before hopping to another narrowband, expressed as a number of absolute subframes

Narrowband offset between one narrowband and the next narrowband a PUSCH hops to, expressed as a number of uplink narrowbands

Total number of absolute subframes a PUCCH with repetition spans, expressed as a number of absolute subframes

Number of repetititions of a PUCCH transmission

Number of PRACH repetitions per preamble transmission attempt

Number of subframes allowed for preamble transmission within a 1024-frame interval

PRACH starting subframe periodicity

Number of NPRACH repetitions per preamble transmission attempt

NPRACH resource periodicity

Frequency location of the first sub-carrier allocated to NPRACH

Number of sub-carriers allocated to NPRACH

Number of starting sub-carriers allocated for contention based NPRACH random access

NPRACH starting subframe

Fraction for starting subcarrier index for UE support for multi-tone msg3 transmission

Periodicity for NPDSCH/NPDCCH gaps

Duration for NPDSCH/NPDCCH gaps

Threshold for applying NPDDCH/NPDCCH gaps

Total number of downlink narrowbands

Total number of absolute subframes a PDSCH with repetition spans, expressed as a number of absolute subframes

Number of repetititions of a PDSCH transmission

Number of consecutive absolute subframes over which MPDCCH or PDSCH stays at the same narrowband before hopping to another narrowband, expressed as a number of absolute subframes

Number of narrowbands over which MPDCCH or PDSCH frequency hops

Narrowband offset between one narrowband and the next narrowband an MPDCCH or PDSCH hops to, expressed as a number of downlink narrowbands

Number of times a PDSCH carrying SIB1-BR is transmitted over 8 radio frames

Total number of absolute subframes a MPDCCH with repetition spans , expressed as a number of absolute subframes

Number of repetitions of a MPDCCH transmission

Number of ECCEs in a subframe for one MPDCCH

Number of OFDM symbols in a downlink slot

Number of SC-FDMA symbols in an uplink slot

Number of consecutive slots in an UL resource unit for NB-IoT

Number of SC-FDMA symbols in a sidelink slot

Resource block size in the frequency domain, expressed as a number of subcarriers

Number of sub-bands for PUSCH frequency-hopping with predefined hopping pattern

Size of each sub-band for PUSCH frequency-hopping with predefined hopping pattern, expressed as a number of resource blocks

Size of narrow-band random-access resource in number of subcarriers

Number of downlink to uplink switch points within the radio frame

Number of reference symbols per slot for PUCCH

Timing offset between uplink and downlink radio frames at the UE, expressed in units of

Fixed timing advance offset, expressed in units of

Timing offset between sidelink and timing reference frames at the UE, expressed in units of

Resource index for PUCCH formats 1/1a/1b

Resource index for PUCCH formats 2/2a/2b

Resource index for PUCCH formats 3

Number of PDCCHs present in a subframe

Physical resource block number

First physical resource block occupied by PRACH resource considered

First physical resource block available for PRACH

Subcarrier occupied by NPRACH resource considered

Virtual resource block number

Radio network temporary identifier

Sidelink group destination identity

System frame number

Slot number within a radio frame

Absolute subframe number

Index for subframes allowed for preamble transmission

Number of antenna ports used for transmission of a channel

Antenna port number

Codeword number

Index for PRACH versions with same preamble format and PRACH density

Qm	Modulation order: 2 for QPSK, 4 for 16QAM, 6 for 64QAM and 8 for 256QAM transmissions

Time-continuous baseband signal for antenna port  and OFDM symbol  in a slot

Radio frame indicator index of PRACH opportunity

Half frame index of PRACH opportunity within the radio frame

Uplink subframe number for start of PRACH opportunity within the half frame

Radio frame duration

Basic time unit

Slot duration

Precoding matrix for downlink spatial multiplexing

Amplitude scaling for PRACH

Amplitude scaling for NPRACH

Amplitude scaling for PUCCH

Amplitude scaling for PUSCH

Amplitude scaling for NPUSCH

Amplitude scaling for sounding reference symbols

Subcarrier spacing

Subcarrier spacing for the random access preamble

Number of transmission layers

Number of OFDM symbols used for MF-SRS transmission.

Orthogonal cover code sequence for MF-SRS

Number of RBs used for MF-SRS transmission within 1 symbol

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CCE	Control Channel Element

CDD	Cyclic Delay Diversity

CRS	Cell-specific Reference Signal

CSI	Channel-State Information

DCI	Downlink Control Information

DM-RS	Demodulation Reference Signal

ECCE	Enhanced Control Channel Element

EPDCCH	Enhanced Physical Downlink Control CHannel

EREG	Enhanced Resource-Element Group

MPDCCH	MTC Physical Downlink Control Channel

NCCE	Narrowband Control Channel Element

NPBCH	Narrowband Physical Broadcast CHannel

NPDCCH	Narrowband Physical Downlink Control CHannel

NPDSCH	Narrowband Physical Downlink Shared CHannel

NPRACH	Narrowband Physical Random Access CHannel

NPUSCH	Narrowband Physical Uplink Shared CHannel

NPSS	Narrowband Primary Synchronization Signal

NSSS 	Narrowband Secondary Synchronization Signal

NRS	Narrowband Reference Signal PBCH	Physical Broadcast CHannel

PCFICH	Physical Control Format Indicator CHannel

PDCCH	Physical Downlink Control CHannel

PDSCH	Physical Downlink Shared CHannel

PHICH	Physical Hybrid-ARQ Indicator CHannel

PMCH	Physical Multicast CHannel

PRACH	Physical Random Access CHannel

PRB	Physical Resource Block

PRS	Positioning Reference Signal

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSDCH	Physical Sidelink Discovery CHannel

PSSCH	Physical Sidelink Shared CHannel

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared CHannel

REG	Resource-Element Group

SCG	Secondary Cell Group

SRS	Sounding Reference Signal

VRB	Virtual Resource Block

# 4	Frame structure

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed as a number of time units  seconds.

Downlink, uplink and sidelink transmissions are organized into radio frames with  duration. 
Three radio frame structures are supported:

-	Type 1, applicable to FDD only,

-	Type 2, applicable to TDD only,

-	Type 3, applicable to LAA secondary cell operation and MF cells only.

NOTE: LAA secondary cell operation only applies to frame structure type 3.

Transmissions in multiple cells can be aggregated where up to 31  secondary cells can be used in addition to the primary cell. Unless otherwise noted, the description in this specification applies to each of the up to 32  serving cells. In case of multi-cell aggregation, different frame structures can be used in the different serving cells.

## 4.1	Frame structure type 1

Frame structure type 1 is applicable to both full duplex and half duplex FDD only. Each radio frame is  long and consists of 20 slots of length, numbered from 0 to 19. A subframe is defined as two consecutive slots where subframe  consists of slots and. Subframe  in frame  has an absolute subframe number   where  is the system frame number.

For FDD, 10 subframes are available for downlink transmission and 10 subframes are available for uplink transmissions in each 10 ms interval. Uplink and downlink transmissions are separated in the frequency domain. In half-duplex FDD operation, the UE cannot transmit and receive at the same time while there are no such restrictions in full-duplex FDD.

Figure 4.1-1: Frame structure type 1

## 4.2	Frame structure type 2

Frame structure type 2 is applicable to TDD only. Each radio frame of length  consists of two half-frames of length each. Each half-frame consists of five subframes of length. Each subframe  is defined as two slots, and, of length  each. Subframe  in frame  has an absolute subframe number   where  is the system frame number.

The uplink-downlink configuration in a cell may vary between frames and controls in which subframes uplink or downlink transmissions may take place in the current frame. The uplink-downlink configuration in the current frame is obtained according to Section 13 in MFA TS 36.213 [4].

The supported uplink-downlink configurations are listed in Table 4.2-2 where, for each subframe in a radio frame, "D" denotes a downlink subframe reserved for downlink transmissions, "U" denotes an uplink subframe reserved for uplink transmissions and "S" denotes a special subframe with the three fields DwPTS, GP and UpPTS. The length of DwPTS and UpPTS is given by Table 4.2-1 subject to the total length of DwPTS, GP and UpPTS being equal to where X is the number of additional SC-FDMA symbols in UpPTS provided by the higher layer parameter srs-UpPtsAdd if configured otherwise X is equal to 0. The UE is not expected to be configured with 2 additional UpPTS SC-FDMA symbols for special subframe configurations {3, 4, 7, 8} for normal cyclic prefix in downlink and special subframe configurations {2, 3, 5, 6} for extended cyclic prefix in downlink and 4 additional UpPTS SC-FDMA symbols for special subframe configurations {1 2, 3, 4, 6, 7, 8} for normal cyclic prefix in downlink and special subframe configurations {1, 2, 3, 5, 6} for extended cyclic prefix in downlink.

Uplink-downlink configurations with both 5 ms and 10 ms downlink-to-uplink switch-point periodicity are supported.

- In case of 5 ms downlink-to-uplink switch-point periodicity, the special subframe exists in both half-frames.

- In case of 10 ms downlink-to-uplink switch-point periodicity, the special subframe exists in the first half-frame only.

Subframes 0 and 5 and DwPTS are always reserved for downlink transmission. UpPTS and the subframe immediately following the special subframe are always reserved for uplink transmission.

In case multiple cells are aggregated, the UE may assume that the guard period of the special subframe in the cells using frame structure type 2 have an overlap of at least .

In case multiple cells with different uplink-downlink configurations in the current radio frame are aggregated and the UE is not capable of simultaneous reception and transmission in the aggregated cells, the following constraints apply:

-	if the subframe in the primary cell is a downlink subframe, the UE shall not transmit any signal or channel on a secondary cell in the same subframe

-	if the subframe in the primary cell is an uplink subframe, the UE is not expected to receive any downlink transmissions on a secondary cell in the same subframe

-	if the subframe in the primary cell is a special subframe and the same subframe in a secondary cell is a downlink subframe, the UE is not expected to receive PDSCH/EPDCCH/PMCH/PRS transmissions in the secondary cell in the same subframe, and the UE is not expected to receive any other signals on the secondary cell in OFDM symbols that overlaps with the guard period or UpPTS in the primary cell.

Figure 4.2-1: Frame structure type 2 (for 5 ms switch-point periodicity)

Table 4.2-1: Configuration of special subframe (lengths of DwPTS/GP/UpPTS)

| Special subframe configuration | Normal cyclic prefix in downlink |  |  | Extended cyclic prefix in downlink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DwPTS | UpPTS |  | DwPTS | UpPTS |  |
|  |  | Normal cyclic prefix in uplink | Extended cyclic prefix in uplink |  | Normal cyclic prefix in uplink | Extended cyclic prefix in uplink |
| 0 |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  | - | - | - |
| 9 |  |  |  | - | - | - |

Table 4.2-2: Uplink-downlink configurations

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

## 4.3	Frame structure type 3

Frame structure type 3 is applicable to LAA secondary cell and MF operation with normal cyclic prefix only. Each radio frame is  long and consists of 20 slots of length, numbered from 0 to 19. A subframe is defined as two consecutive slots where subframe  consists of slots and.

The 10 subframes within a radio frame are available for downlink or uplink transmissions. Downlink transmissions occupy one or more consecutive subframes, starting anywhere within a subframe and ending with the last subframe either fully occupied or following one of the DwPTS durations in Table 4.2-1. Uplink transmissions occupy one or more consecutive subframes.

For MF, if DwPTS duration is less than or equal to - - mf-sPUCCHgap, the length of UpPTS is , where

- is given by clause 8.1,

- mf-sPUCCHgap = 16us if mf-sPUCCH-LBT is set by higher layer signalling.

- mf-sPUCCHgap = 25us if type 2 channel access procedure is configured by higher layer signalling

otherwise UpPTS length = 0.

# 5	Uplink

## 5.1	Overview

The smallest resource unit for uplink transmissions is denoted a resource element and is defined in clause 5.2.2.

### 5.1.1	Physical channels

An uplink physical channel corresponds to a set of resource elements carrying information originating from higher layers and is the interface defined between 3GPP TS 36.212 [3] and this document. 
The following uplink physical channels are defined:

-	Physical Uplink Shared Channel, PUSCH

-	Physical Uplink Control Channel, PUCCH

-	Physical Random Access Channel, PRACH

- 	MF Short Physical Uplink Control Channel, MF-sPUCCH

- 	MF Extended Physical Uplink Control Channel, MF-ePUCCH

- 	MF Short Physical Random Access Channel, MF-sPRACH

### 5.1.2	Physical signals

An uplink physical signal is used by the physical layer but does not carry information originating from higher layers. The following uplink physical signals are defined:

-	Reference signal

## 5.2	Slot structure and physical resources

### 5.2.1	Resource grid

The transmitted signal in each slot is described by one or several resource grids of  subcarriers and  SC-FDMA symbols. The resource grid is illustrated in Figure 5.2.1-1.  The quantity  depends on the uplink transmission bandwidth configured in the cell and shall fulfil

where  and  are the smallest and largest uplink bandwidths, respectively, supported by the current version of this specification. The set of allowed values for  is given by MFA TS 36.101 [7].

The number of SC-FDMA symbols in a slot depends on the cyclic prefix length configured by the higher layer parameter UL-CyclicPrefixLength and is given in Table 5.2.3-1.

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. There is one resource grid per antenna port. The antenna ports used for transmission of a physical channel or signal depends on the number of antenna ports configured for the physical channel or signal as shown in Table 5.2.1-1. The index  is used throughout clause 5 when a sequential numbering of the antenna ports is necessary.

Figure 5.2.1-1: Uplink resource grid

Table 5.2.1-1: Antenna ports used for different physical channels and signals

| Physical channel or signal | Index | Antenna port number as a function of the number of antenna ports configured for the respective physical channel/signal |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 |
| PUSCH | 0 | 10 | 20 | 40 |
|  | 1 | - | 21 | 41 |
|  | 2 | - | - | 42 |
|  | 3 | - | - | 43 |
| SRS | 0 | 10 | 20 | 40 |
|  | 1 | - | 21 | 41 |
|  | 2 | - | - | 42 |
|  | 3 | - | - | 43 |
| PUCCH | 0 | 100 | 200 | - |
|  | 1 | - | 201 | - |
| MF-sPUCCH | 0 | 100 | - | - |
| MF-ePUCCH | 0 | 100 | - | - |

### 5.2.2	Resource elements

Each element in the resource grid is called a resource element and is uniquely defined by the index pair  in a slot where  and  are the indices in the frequency and time domains, respectively. Resource element  on antenna port  corresponds to the complex value . 
When there is no risk for confusion, or no particular antenna port is specified, the index  may be dropped.  
Quantities  corresponding to resource elements not used for transmission of a physical channel or a physical signal in a slot shall be set to zero.

### 5.2.3	Resource blocks

A physical resource block is defined as consecutive SC-FDMA symbols in the time domain and consecutive subcarriers in the frequency domain, where  and  are given by Table 5.2.3-1. 
A physical resource block in the uplink thus consists of  resource elements, corresponding to one slot in the time domain and 180 kHz in the frequency domain.

Table 5.2.3-1: Resource block parameters

| Configuration |  |  |
| --- | --- | --- |
| Normal cyclic prefix | 12 | 7 |
| Extended cyclic prefix | 12 | 6 |

The relation between the physical resource block number  in the frequency domain and resource elements  in a slot is given by

### 5.2.4	Narrowbands

A narrowband is defined as six non-overlapping consecutive physical resource blocks in the frequency domain. The total number of uplink narrowbands in the uplink transmission bandwidth configured in the cell is given by

The narrowbands are numbered   in order of increasing physical resource-block number where narrowband is composed of physical resource-block indices

where

### 5.2.5	Guard period for narrowband retuning

For BL/CE UEs, a guard period is created for Tx-to-Tx frequency retuning between two consecutive subframes.

-	If the UE retunes from a first narrowband carrying PUSCH to a second narrowband carrying PUSCH, or if the UE retunes from a first narrowband carrying PUCCH to a second narrowband carrying PUCCH,

-	a guard period is created by the UE not transmitting the last SC-FDMA symbol in the first subframe and the first SC-FDMA symbol in the second subframe.

-	If the UE retunes from a first narrowband carrying PUCCH to a second narrowband carrying PUSCH,

-	if the PUCCH uses a shortened PUCCH format, a guard period is created by the UE not transmitting the first SC-FDMA symbol in the second subframe,

-	otherwise a guard period is created by the UE not transmitting the first two SC-FDMA symbols in the second subframe.

-	If the UE retunes from a first narrowband carrying PUSCH to a second narrowband carrying PUCCH,

-	a guard period is created by the UE not transmitting the last two SC-FDMA symbols in the first subframe.

## 5.3	Physical uplink shared channel

The baseband signal representing the physical uplink shared channel is defined in terms of the following steps:

-	scrambling

-	modulation of scrambled bits to generate complex-valued symbols

-	mapping of the complex-valued modulation symbols onto one or several transmission layers

-	transform precoding to generate complex-valued symbols

-	precoding of the complex-valued symbols

-	mapping of precoded complex-valued symbols to resource elements

-	generation of complex-valued time-domain SC-FDMA signal for each antenna port

Figure 5.3-1: Overview of uplink physical channel processing

### 5.3.1	Scrambling

For each codeword , the block of bits , where  is the number of bits transmitted in codeword  on the physical uplink shared channel in one subframe, shall be scrambled with a UE-specific scrambling sequence prior to modulation, resulting in a block of scrambled bits  according to the following pseudo code

Set i = 0

while

if 		// ACK/NACK or Rank Indication placeholder bits

else

if  	// ACK/NACK or Rank Indication repetition placeholder bits

else			// Data or channel quality coded bits, Rank Indication coded bits or ACK/NACK coded bits

end if

end if

i = i + 1

end while

where x and y are tags defined in MFA TS 36.212 [3] clause 5.2.2.6 and where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  corresponds to the RNTI associated with the PUSCH transmission as described in clause 8 in MFA TS 36.213 [4].

For BL/CE UEs, the same scrambling sequence is applied per subframe to PUSCH for a given block of  subframes. For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first uplink subframe intended for PUSCH. The PUSCH transmission spans  consecutive subframes including non-BL/CE UL subframes where the UE postpones the PUSCH transmission. For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2.

Up to two codewords can be transmitted in one subframe, i.e., . In the case of single-codeword transmission, .

### 5.3.2	Modulation

For each codeword, the block of scrambled bits  shall be modulated as described in clause 7.1, resulting in a block of complex-valued symbols . Table 5.3.2-1 specifies the modulation mappings applicable for the physical uplink shared channel.

Table 5.3.2-1: Uplink modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PUSCH | QPSK, 16QAM, 64QAM |

### 5.3.2A	Layer mapping

The complex-valued modulation symbols for each of the codewords to be transmitted are mapped onto one or two layers. Complex-valued modulation symbols   for codeword  shall be mapped onto the layers ,  where  is the number of layers and  is the number of modulation symbols per layer.

#### 5.3.2A.1	Layer mapping for transmission on a single antenna port

For transmission on a single antenna port, a single layer is used, , and the mapping is defined by

with .

#### 5.3.2A.2	Layer mapping for spatial multiplexing

For spatial multiplexing, the layer mapping shall be done according to Table 5.3.2A.2-1. The number of layers  is less than or equal to the number of antenna ports  used for transmission of the physical uplink shared channel. 
The case of a single codeword mapped to multiple layers is only applicable when the number of antenna ports used for PUSCH is four.

Table 5.3.2A.2-1: Codeword-to-layer mapping for spatial multiplexing

| Number of layers | Number of codewords | Codeword-to-layer mapping |  |
| --- | --- | --- | --- |
| 1 | 1 |  |  |
| 2 | 1 |  |  |
| 2 | 2 |  |  |
| 3 | 2 |  |  |
| 4 | 2 |  |  |

### 5.3.3	Transform precoding

For each layer  the block of complex-valued symbols  is divided into  sets, each corresponding to one SC-FDMA symbol. Transform precoding shall be applied according to

resulting in a block of complex-valued symbols . The variable, where  represents the bandwidth of the PUSCH in terms of resource blocks, and shall fulfil

where  is a set of non-negative integers.

### 5.3.3A	Precoding

The precoder takes as input a block of vectors ,  from the transform precoder and generates a block of vectors ,  to be mapped onto resource elements.

#### 5.3.3A.1	Precoding for transmission on a single antenna port

For transmission on a single antenna port, precoding is defined by

where , .

#### 5.3.3A.2	Precoding for spatial multiplexing

Precoding for spatial multiplexing is only used in combination with layer mapping for spatial multiplexing as described in clause 5.3.2A.2. Spatial multiplexing supports  or  antenna ports where the set of antenna ports used for spatial multiplexing is  and , respectively.

Precoding for spatial multiplexing is defined by

where , .

The precoding matrix  of size  is given by one of the entries in Table 5.3.3A.2-1 for  and by Tables 5.3.3A.2-2 through 5.3.3A.2-5 for  where the entries in each row are ordered from left to right in increasing order of codebook indices.

Table 5.3.3A.2-1: Codebook for transmission on antenna ports

| Codebook index | Number of layers |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  | - |
| 2 |  | - |
| 3 |  | - |
| 4 |  | - |
| 5 |  | - |

Table 5.3.3A.2-2: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 7 |  |  |  |  |  |  |  |  |
| 8 – 15 |  |  |  |  |  |  |  |  |
| 16 – 23 |  |  |  |  |  |  |  |  |

Table 5.3.3A.2-3: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 7 |  |  |  |  |
| 8 – 11 |  |  |  |  |
| 12 – 15 |  |  |  |  |

Table 5.3.3A.2-4: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |  |  |  |
| --- | --- | --- | --- | --- |
| 0 – 3 |  |  |  |  |
| 4 – 7 |  |  |  |  |
| 8 – 11 |  |  |  |  |

Table 5.3.3A.2-5: Codebook for transmission on antenna ports  with

| Codebook index | Number of layers |
| --- | --- |
| 0 |  |

### 5.3.4	Mapping to physical resources

For each antenna port  used for transmission of the PUSCH in a subframe the block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in clause 5.1.1.1 in MFA TS 36.213 [4], and mapped in sequence starting with  to physical resource blocks on antenna port  and assigned for transmission of PUSCH. The relation between the index  and the antenna port number  is given by Table 5.2.1-1. The mapping to resource elements  corresponding to the physical resource blocks assigned for transmission and

-	not used for transmission of reference signals, and

-	not part of the last SC-FDMA symbol in a subframe, if the UE transmits SRS in the same subframe in the same serving cell, and

-	not part of the last SC-FDMA symbol in a subframe configured with cell-specific SRS for non-BL/CE UEs and BL/CE UEs in CEModeA, if the PUSCH transmission partly or fully overlaps with the cell-specific SRS bandwidth, and

-	not part of an SC-FDMA symbol reserved for possible SRS transmission in a UE-specific aperiodic SRS subframe in the same serving cell, and

-	not part of an SC-FDMA symbol reserved for possible SRS transmission in a UE-specific periodic SRS subframe in the same serving cell when the UE is configured with multiple TAGs

-	not part of the first SC-FDMA symbol in a subframe if the associated DCI indicates PUSCH starting position ‘01’, ‘10’, or ‘11

-	not part of the last SC-FDMA symbol in a subframe if the associated DCI indicates PUSCH ending symbol ‘1’

shall be in increasing order of first the index , then the index, starting with the first slot in the subframe.

For BL/CE UEs in CEModeB, resource elements in the last SC-FDMA symbol in a subframe configured with cell-specific SRS shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.

For BL/CE UEs, if one or more SC-FDMA symbol(s) are left empty due to guard period for narrowband retuning, the affected SC-FDMA symbol(s) shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.

If uplink frequency-hopping is disabled or the resource blocks allocated for PUSCH transmission are not contiguous in frequency, the set of physical resource blocks to be used for transmission is given by  where  is obtained from the uplink scheduling grant as described in clause 8.1 in MFA TS 36.213 [4].

If uplink frequency-hopping with type 1 PUSCH hopping is enabled, the set of physical resource blocks to be used for transmission is given by clause 8.4.1 in MFA TS 36.213 [4].

If uplink frequency-hopping with predefined hopping pattern is enabled, the set of physical resource blocks to be used for transmission in slot  is given by the scheduling grant together with a predefined pattern according to

where  is obtained from the scheduling grant as described in clause 8.1 in MFA TS 36.213 [4]. The parameter pusch-HoppingOffset,, is provided by higher layers. The size  of each sub-band is given by,

where the number of sub-bands  is given by higher layers. The function  determines whether mirroring is used or not. The parameter Hopping-mode provided by higher layers determines if hopping is "inter-subframe" or "intra and inter-subframe".

The hopping function and the function are given by

where  and the pseudo-random sequence  is given by clause 7.2 and CURRENT_TX_NB indicates the transmission number for the transport block transmitted in slot as defined in MFA TS 36.321 [8].  The pseudo-random sequence generator shall be initialised with  for frame structure type 1 and  for frame structure type 2  at the start of each frame.

For BL/CE UEs, the PRB resources within the narrowband  for PUSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.10 and 5.3.3.1.11 in MFA TS 36.212 [3]. The PUSCH is transmitted with repetitions. The PUSCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the UE postpones the PUSCH transmission if . For BL/CE UE in CEModeA, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set and the frequency hopping flag in DCI format 6-0A indicates frequency hopping, otherwise frequency hopping is disabled. If frequency hopping is not enabled for PUSCH, the PUSCH repetitions are located at the same PRB resources at the same narrowband . If frequency hopping is enabled for PUSCH, PUSCH is transmitted in subframe  within the   consecutive uplink subframes using the same PRB resources within narrowband

where  is the absolute subframe number of the first UL subframe intended for carrying the PUSCH and  and  are cell-specific higher-layer parameters. For the   consecutive subframes, the UE shall not transmit PUSCH in subframe  if it is not a BL/CE UL subframe.

For BL/CE UEs, PUSCH transmission associated with Temporary C-RNTI, frequency hopping of the PUSCH is enabled when higher layer parameter rar-HoppingConfig is set. Further

-	if PRACH CE level 0 or 1  is used for the last PRACH attempt,  is set to the higher layer parameter interval-UlHoppingConfigCommonModeA;

-	if PRACH CE level 2 or 3 is used for the last PRACH attempt,   is set to the higher layer parameter interval-UlHoppingConfigCommonModeB.

For BL/CE UEs, for PUSCH transmission not associated with Temporary C-RNTI, for frame structure type 1, after a transmission duration of   time units (which may include non-BL/CE UL subframes), a gap of   time units shall be inserted, as specified in MFA TS 36.331 [9]. BL/CE UL subframes within the gap of   time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH.

For BL/CE UEs, for PUSCH transmission associated with Temporary C-RNTI for frame structure type 1, after a transmission duration of   time units (which may include non-BL/CE UL subframes), a gap of   time units shall be inserted. BL/CE UL subframes within the gap of   time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH.

## 5.4	Physical uplink control channel

The physical uplink control channel, PUCCH, carries uplink control information. Simultaneous transmission of PUCCH and PUSCH from the same UE is supported if enabled by higher layers. For frame structure type 2, the PUCCH is not transmitted in the UpPTS field.

In an MF cell, MF-sPUCCH and MF-ePUCCH carry uplink control information. MF-sPUCCH is transmitted during the last four symbols of a subframe where the preceding ten symbols of the subframe are not used for uplink transmission.

The physical uplink control channel supports multiple formats as shown in Table 5.4-1 with different number of bits per subframe, where  represents the bandwidth of the PUCCH format 4 as defined by clause 5.4.2B, and  and  are defined in Table 5.4.2C-1. MF-sPUCCH and MF-ePUCCH formats are defined in Table 5.4-1-1, and 5.4-1-2, respectively.

Formats 2a and 2b are supported for normal cyclic prefix only.

Table 5.4-1: Supported PUCCH formats

| PUCCH format | Modulation scheme | Number of bits per subframe, |
| --- | --- | --- |
| 1 | N/A | N/A |
| 1a | BPSK | 1 |
| 1b | QPSK | 2 |
| 2 | QPSK | 20 |
| 2a | QPSK+BPSK | 21 |
| 2b | QPSK+QPSK | 22 |
| 3 | QPSK | 48 |
| 4 | QPSK |  |
| 5 | QPSK |  |

Table 5.4-2: Supported MF-sPUCCH formats

| MF-sPUCCH format | Modulation scheme | Number of bits per subframe, |
| --- | --- | --- |
| 0 | QPSK | N/A |
| 1 | QPSK |  |
| 2 | QPSK |  |
| 3 | QPSK |  |

Table 5.4-3: Supported MF-ePUCCH format

| MF-ePUCCH format | Modulation scheme | Number of bits per subframe, |
| --- | --- | --- |
| 1 | QPSK |  |

All PUCCH formats as well as MF-sPUCCH and MF-ePUCCH use a cyclic shift, , which varies with the symbol number  and the slot number  according to

where the pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with , where  is given by clause 5.5.1.5 with  corresponding to the primary cell, at the beginning of each radio frame.

The physical resources used for PUCCH depends on two parameters,  and , given by higher layers. 
The variable  denotes the bandwidth in terms of resource blocks that are available for use by PUCCH formats 2/2a/2b transmission in each slot. The variable  denotes the number of cyclic shift used for PUCCH formats 1/1a/1b in a resource block used for a mix of formats 1/1a/1b and 2/2a/2b. The value of  is an integer multiple of  within the range of {0, 1, …, 7}, where  is provided by higher layers. No mixed resource block is present if . At most one resource block in each slot supports a mix of formats 1/1a/1b and 2/2a/2b. 
Resources used for transmission of PUCCH formats 1/1a/1b, 2/2a/2b, 3, 4, and 5 are represented by the non-negative indices , , ,  and , respectively.

The physical resources used for MF-sPUCCH, MF-ePUCCH depends on bandwidth parameters given by higher layer configuration.  and  denote the bandwidth in terms of resource blocks available for MF-sPUCCH and MF-ePUCCH, respectively. Resources used for transmission of MF-sPUCCH formats 0/1/2/3 and MF-ePUCCH are represented by the non-negative indices , , , 
nMF-sPUCCH3,p and , respectively.

### 5.4.1	PUCCH formats 1, 1a and 1b

For PUCCH format 1, information is carried by the presence/absence of transmission of PUCCH from the UE. 
In the remainder of this clause,  shall be assumed for PUCCH format 1.

For PUCCH formats 1a and 1b, one or two explicit bits are transmitted, respectively. The block of bits  shall be modulated as described in Table 5.4.1-1, resulting in a complex-valued symbol. 
The modulation schemes for the different PUCCH formats are given by Table 5.4-1.

The complex-valued symbol  shall be multiplied with a cyclically shifted length  sequence  for each of the  antenna ports used for PUCCH transmission according to

where  is defined by clause 5.5.1 with and . The antenna-port specific cyclic shift   varies between symbols and slots as defined below.

The block of complex-valued symbols  shall be scrambled by  and block-wise spread with the antenna-port specific orthogonal sequence  according to

where

and

with  for the two slots in a subframe given by Table 5.4.1-1a. The sequence  is given by Table 5.4.1-2 and Table 5.4.1-3 and  is defined below.

Resources used for transmission of PUCCH format 1, 1a and 1b are identified by a resource index  from which the orthogonal sequence index  and the cyclic shift  are determined according to

where

The resource indices within the two resource blocks in the two slots of a subframe to which the PUCCH is mapped are given by

for  and by

for , where , with for normal CP and for extended CP.

The parameter deltaPUCCH-Shift  is provided by higher layers.

Table 5.4.1-1: Modulation symbol  for PUCCH formats 1a and 1b

| PUCCH format |  |  |
| --- | --- | --- |
| 1a | 0 |  |
|  | 1 |  |
| 1b | 00 |  |
|  | 01 |  |
|  | 10 |  |
|  | 11 |  |

Table 5.4.1-1a: The quantity  for PUCCH formats 1a and 1b

| PUCCH format |  |  |
| --- | --- | --- |
|  | first slot | second slot |
| normal 1/1a/1b | 4 | 4 |
| shortened 1/1a/1b | 4 | 3 |

Table 5.4.1-2: Orthogonal sequences  for

| Sequence index | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |

Table 5.4.1-3: Orthogonal sequences  for

| Sequence index | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |

### 5.4.2	PUCCH formats 2, 2a and 2b

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in clause 7.1, resulting in a block of complex-valued modulation symbols .

Each complex-valued symbol  shall be multiplied with a cyclically shifted length  sequence for each of the  antenna ports used for PUCCH transmission according to

where  is defined by clause 5.5.1 with and .

Resources used for transmission of PUCCH formats 2/2a/2b are identified by a resource index  from which the cyclic shift  is determined according to

where

and

for  and by

for .

For PUCCH formats 2a and 2b, supported for normal cyclic prefix only, the bit(s)  shall be modulated as described in Table 5.4.2-1 resulting in a single modulation symbol  used in the generation of the reference-signal for PUCCH format 2a and 2b as described in clause 5.5.2.2.1.

Table 5.4.2-1: Modulation symbol  for PUCCH formats 2a and 2b

| PUCCH format |  |  |
| --- | --- | --- |
| 2a | 0 |  |
|  | 1 |  |
| 2b | 00 |  |
|  | 01 |  |
|  | 10 |  |
|  | 11 |  |

### 5.4.2A	PUCCH format 3

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The complex-valued symbols  shall be block-wise spread with the orthogonal sequences  and  resulting in  sets of  values each according to

where  for both slots in a subframe using normal PUCCH format 3 and , holds for the first and second slot, respectively, in a subframe using shortened PUCCH format 3. The orthogonal sequences  and  are given by Table 5.4.2A-1. Resources used for transmission of PUCCH formats 3 are identified by a resource index  from which the quantities  and  are derived according to

Each set of complex-valued symbols shall be cyclically shifted according to

where  is given by Subclause 5.4,  is the slot number within a radio frame and  is the SC-FDMA symbol number within a slot.

The shifted sets of complex-valued symbols shall be transform precoded according to

where  is the number of antenna ports used for PUCCH transmission, resulting in a block of complex-valued symbols .

Table 5.4.2A-1: The orthogonal sequence

| Sequence index | Orthogonal sequence |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  | - |

### 5.4.2B	PUCCH format 4

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The block of complex-valued symbols  is divided into  sets, each corresponding to one SC-FDMA symbol. Transform precoding shall be applied according to

where ,  and  are given by Table 5.4.2C-1 for normal PUCCH format 4 and shortened PUCCH format 4, resulting in a block of complex-valued symbols . The variable, where  represents the bandwidth of the PUCCH format 4 in terms of resource blocks, shall fulfil

where  is a set of non-negative integers.

### 5.4.2C	PUCCH format 5

The block of bits  shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits  according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe where  is the C-RNTI.

The block of scrambled bits  shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols  where .

The complex-valued symbols  shall be divided into  sets, each corresponding to one SC-FDMA symbol. Block-wise spreading shall be applied according to

where ,  and  are given by Table 5.4.2C-1 for normal PUCCH format 5 and shortened PUCCH format 5, and  is given by Table 5.4.2C-2 with  provided by higher layers.

The block-wise spread complex-valued symbols shall be transform precoded according to

where , resulting in a block of complex-valued symbols .

Table 5.4.2C-1: The quantities  and

| PUCCH format type | Normal cyclic prefix |  | Extended cyclic prefix |  |
| --- | --- | --- | --- | --- |
| Normal PUCCH format | 6 | 6 | 5 | 5 |
| Shortened PUCCH format | 6 | 5 | 5 | 4 |

Table 5.4.2C-2: Orthogonal sequences

|  | Orthogonal sequences |
| --- | --- |
| 0 |  |
| 1 |  |

### 5.4.2MF1	MF-sPUCCH format 0

The MF-sPUCCH format 0 is based on DMRS of MF in clause 5.5.2.2. This format is used as the physical waveform for MF-sPRACH and scheduling request-only transmissions in MulteFire. The MF-sPUCCH format 0 signal for MF is based on the reference signal sequence  defined by clause 5.5.1.1 with . The MF-sPUCCH format 0 signal is defined as

where

,

denotes the number of subcarriers used for MF-sPUCCH format 0 transmission over

resource blocks,

denotes the number of symbols used for MF-sPUCCH format 0 transmission,  is defined in clause 5.5.1.1, and  is the length-4 inter-OCC as defined in Table 5.4.2MF.3 for . The cyclic shift  is given by

In the case of SR-only transmissions, the indicesnocp, ncspand q to use for a particular transmission within a particular cell are signaled by higher layer and where, p=0, 1, . . . , P-1, and P, the number of antenna ports used for MF-sPUCCH format 0 is given by Table 5.2.1-1. Further, nocpand ncsp are derived as follows

nocp=nMF-sPUCCH(0,p) mod NMF-sPUCCH0

ncsp=nMF-sPUCCH0,p/NMF-sPUCCH0modNscRB

For MF-sPRACH, p=0, and

ncs0∈{0, Ncs, 2Ncs,…, 12Ncs-1⋅Ncs}

where Ncs is defined in clause 5.7.2.

The sequence shall be mapped in sequence starting with

to resource elements  according to

where

,

and

NRSMF-sPUCCH0=NMF-sPUCCH0.

For MF-sPUCCH format 0 transmission, the quantity  is the frequency-domain starting position of the MF-sPUCCH format 0 signal, which is defined by

,

where

nMF-sPUCCH0I=nMF-sPUCCH0,p/NscRBNMF-sPUCCH0.

For MF-sPRACH transmission, the frequency-domain starting position is specified in Section 5.7.2.

### 5.4.2MF2	MF-sPUCCH formats 1, 2, 3

The block of bits

shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits

according to

where the scrambling sequence  is described in Section 7.2. The scrambling sequence generator shall be initialised with

at the start of each subframe where  is the C-RNTI.

The block of scrambled bits

shall be QPSK modulated as described in Section 7.1, resulting in a block of complex-valued modulation symbols

where

and

MRBMF-sPUCCH

is the bandwidth of MF-sPUCCH in resource blocks.

The complex-valued symbols

shall be multiplexed with the orthogonal sequence wnoc,intrap(i) of length NSF,intraMF-sPUCCH  within one SC-FDMA symbol for each of the antenna ports used for MF-sPUCCH transmission according to

ypn⋅NSF,intraMF-sPUCCH+i=dn⋅wnoc,intrapi

n=0,…,Msymb-1

i=0,…,NSF,intraMF-sPUCCH-1

where

NSF,intraMF-sPUCCH=1

for format 1,

NSF,intraMF-sPUCCH=2

for format 2, and

NSF,intraMF-sPUCCH=6

for format 3 and where,

p=0, 1, . . . , P-1, and P,

the number of antenna ports used for MF-sPUCCH formats 1/2/3 is given by Table 5.2.1-1. For

NSF,intraMF-sPUCCH=1, wnoc,intra0=1,

The orthogonal sequence

wnoc,intrai for NSF,intrasPUCCH>1

are listed in Table 5.4.2MF2-1.

Table 5.4.2MF2-1: The orthogonal sequence woc,intra(i)

| Sequence index | Orthogonal sequence [wnoc0⋯wnoc(NSF,intraMF-sPUCCH-1)] |  |
| --- | --- | --- |
|  | NSF, intraMF-sPUCCH=2 | NSF, intraMF-sPUCCH=6 |
| 0 | [+1 +1] | [+1,+1,+j,-1,-j,-j] |
| 1 | [+1 -1] | [-j,-1,+j,+1,+1,-j] |
| 2 | - | [+1,+1,+1,+1,+1,+j] |
| 3 | - | [-1,-j,+j,-1,+1,+j] |
| 4 | - | [+1,-1,-j,-1,+1,-1] |
| 5 |  | [+1,-1,+j,-j,-1,+j] |

The intra-symbol spread modulation symbols

ypi

are divided into

MRBMF-sPUCCH

sets, and each set corresponds to one resource block. Within each set, symbols are interleaved according to

ypi+n⋅NscRB=yp(i'⋅NSF,intraMF-sPUCCH+i''+n⋅NscRB)

i'=i mod Msymb,RBMF-sPUCCH

i''=i/Msymb,RBMF-sPUCCH mod  NSF,intraMF-sPUCCH

Msymb,RBMF-sPUCCH=Msymb/MRBMF-sPUCCH

i=0,…,NscRB-1

n=0,…, MRBMF-sPUCCH-1

After interleaving, the complex-valued symbols

ypi

shall be block-wise spread with the orthogonal sequences

wnoc,interp(n)

of length-2 according to

ynpi=wnoc,interpn⋅yp(i)

i=0,…,MRBMF-sPUCCH⋅NscRB-1

n=0,1

where

wnoc,interp(n)

is the same as the length-2

wnoc,intrap(n)

and is listed in Table 5.4.2MF2-1. Resources used for transmission of MF-sPUCCH are identified by a resource index

nMF-sPUCCH(F,p)

for MF-sPUCCH format F transmissions (F = 1, 2 or 3). The OCC resource indices

noc,intrap and noc,interp

are given by

noc,interp=nMF-sPUCCH(F,p) mod 2

noc,intrap=nMF-sPUCCH(F,p)/2 mod NSF,intraMF-sPUCCH

The complex-valued symbols

ynpi

are transformed precoded according to

zpn⋅MRBMF-sPUCCH⋅NscRB+k=1P1MRBMF-sPUCCH⋅NscRB i=0MRBMF-sPUCCH⋅NscRB-1ynpie-j2πkiMRBMF-sPUCCH⋅NscRB

k=0, …, MRBMF-sPUCCH⋅NscRB-1

n=0, 1

where  is the number of antenna ports used for MF-sPUCCH transmission, resulting in a block of complex-valued symbols

zp0,…, zp(2⋅MRBMF-sPUCCH⋅NscRB-1).

MRBMF-sPUCCH

shall fulfill

where

is a set of non-negative integers.

### 5.4.2MF3	MF-ePUCCH format 1

The block of bits

shall be scrambled with a UE-specific scrambling sequence, resulting in a block of scrambled bits

according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialised with

at the start of each subframe where  is the C-RNTI.

The block of scrambled bits

shall be QPSK modulated as described in Subclause 7.1, resulting in a block of complex-valued modulation symbols

where

and MRBMF-ePUCCH

is the bandwidth of MF-ePUCCH in resource blocks.

The complex-valued symbols

shall be block-wise spread with the orthogonal sequences

and

resulting in

sets of

MRBMF-ePUCCH⋅

values each according to

where

for both slots in a subframe using normal MF-ePUCCH format 1,

NSF,0MF-ePUCCH=4

if the first symbol is excluded from MF-ePUCCH, for example, for LBT purposes and

NSF,1MF-ePUCCH=4

if using shortened MF-ePUCCH format 1. The orthogonal sequences

and

are listed in Table 5.4.2MF.3-1. Resources used for transmission of MF-ePUCCH format 1 triggered by DCI with CRC scrambled by CC-RNTI are identified by a resource index

from which the quantities

and

are derived according to

where

NSFMF-ePUCCH=min⁡{NSF,0MF-ePUCCH,NSF,1MF_ePUCCH}.

For MF-ePUCCH format 1 transmissions scheduled by other DCI formats, the resource index is indicated by the corresponding DCI and the quantities

and

are derived according to the above equations.

Each set of complex-valued symbols shall be cyclically shifted, according to

i=0,…,NscRB-1
n'=0,…,MRBMF-ePUCCH-1

where

is defined in Section 5.4,  is the slot number within a radio frame and  is the SC-FDMA symbol number within a slot.

The shifted sets of complex-valued symbols shall be transform precoded according to

where  is the number of antenna ports used for MF-ePUCCH transmission, resulting in a block of complex-valued symbols

.

Table 5.4.2MF.3-1: The orthogonal sequence

| Sequence index | Orthogonal sequence[wnoc0⋯wnoc(NSF, xMF-ePUCCH-1)] |  |
| --- | --- | --- |
|  | NSF, xMF-ePUCCH=5 | NSF,xMF-ePUCCH=4 |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  | - |

### 5.4.3	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in Subclause 5.1.2.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with  to resource elements. PUCCH uses one or more resource block in each of the two slots in a subframe.

Within the physical resource block(s) used for transmission, the mapping of  to resource elements  on antenna port  and not used for transmission of reference signals shall be in increasing order of first , then  and finally the slot number, starting with the first slot in the subframe. The relation between the index  and the antenna port number  is as defined for MF-sPUCCH and MF-sPUCCH given by Table 5.2.1-1. For MF-sPUCCH formats 1, 2 and 3,

only maps to last two symbols in a subframe in increasing order of first k and then l.

For non-BL/CE UEs, except for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

For BL/CE UEs, PUCCH is transmitted with repetitions. The PUCCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the UE postpones the PUCCH transmission if . The quantity  is given by the higher-layer parameter pucch-NumRepetitionCE-Msg4 for transmission of Msg4 in the random-access procedure MFA TS 36.321 [8], otherwise by pucch-NumRepetitionCE-Format1 for PUCCH format 1/1a and pucch-NumRepetitionCE-Format2 for PUCCH format 2/2a/2b,

The physical resource blocks to be used for transmission of PUCCH in subframe  within the   consecutive subframes are given by

where  is the absolute subframe number of the first uplink subframe intended for PUCCH.

The variable  depends on the PUCCH format.

-	Formats 1, 1a and 1b:

-	Formats 2, 2a and 2b:

-	Format 3:

-	Format 5 (non-BL/CE UEs only):

For non-BL/CE UEs, for PUCCH format 4, the physical resource blocks to be used for transmission of PUCCH in slot  are given by

where  is obtained from MFA TS 36.213 [4].

Mapping of modulation symbols for the physical uplink control channel for PUCCH formats 1 – 3 is illustrated in Figure 5.4.3-1.

In case of simultaneous transmission of sounding reference signal and PUCCH format 1, 1a, 1b, 3, 4 or 5 when there is one serving cell configured, the shortened PUCCH format shall be used where the last SC-FDMA symbol in the second slot of a subframe shall be left empty.

In case of guard period for narrowband retuning for BL/CE UEs, if an SC-FDMA symbol is left empty due to guard period, the SC-FDMA symbol shall be counted in the PUCCH mapping but not used for transmission of the PUCCH. The SC-FDMA symbol affected by the guard period can be the first SC-FDMA symbol in the first slot of a subframe and/or the last SC-FDMA symbol in the second slot of a subframe.

Figure 5.4.3-1: Mapping to physical resource blocks for PUCCH formats 1 – 3 for non-BL/CE UEs.

For MF-sPUCCH format 1/2/3, the physical resource blocks to be used for transmission of MF-sPUCCH in the last two symbols of a subframe are given by

nPRB={m, m+NRBUL/10, …,m+9⋅NRBUL/10}

m=nMF-sPUCCHp/(2NSF,intraMF-sPUCCH)+i

i=0,…,NMF-sPUCCHI-1

where

NMF-sPUCCHI

is the number of the allocated interlaces given by higher layer configuration mf-spucch-NumInterlaces.

For MF-ePUCCH format 1, the physical resource blocks to be used for transmission of MF-ePUCCH are given by

nPRB={m, m+NRBUL/10, …,m+9⋅NRBUL/10}

m=nePUCCH1,p/NSFMF-ePUCCH+i

i=0,…, NMF-ePUCCHI-1

where

NMF-ePUCCHI

is the number of the allocated interlaces given by higher layer configuration mf-epucch-NumInterlaces for MF-ePUCCH transmissions triggered by DCI with CRC scrambled by CC-RNTI. For MF-ePUCCH format 1 transmissions scheduled by other DCI formats, the allocated interlace(s) are indicated by the corresponding DCI.

The mapping to resource elements is performed corresponding to the physical resource blocks assigned for transmission and

- not part of the first SC-FDMA symbol in a subframe if an associated DCI indicates MF-ePUCCH starting position other than the beginning of the first SC-OFDMA symbol, i.e.

.

- not part of the last SC-FDMA symbol in a subframe if an associated DCI indicates

.

## 5.5	Reference signals

Two types of uplink reference signals are supported:

-	Demodulation reference signal, associated with transmission of PUSCH or PUCCH

-	Sounding reference signal, not associated with transmission of PUSCH or PUCCH

The same set of base sequences is used for demodulation and sounding reference signals.

### 5.5.1	Generation of the reference signal sequence

Reference signal sequence  is defined by a cyclic shift  of a base sequence  according to

where  is the length of the reference signal sequence, ,  is defined in subclause 5.5.2.1.2, and the quantity  when the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3, and  otherwise. Multiple reference signal sequences are defined from a single base sequence through different values of .

Base sequences  are divided into groups, where  is the group number and  is the base sequence number within the group, such that each group contains one base sequence () of each length  ,  and two base sequences () of each length , . The sequence group number  and the number  within the group may vary in time as described in clauses 5.5.1.3 and 5.5.1.4, respectively. The definition of the base sequence  depends on the sequence length.

#### 5.5.1.1	Base sequences of length  or larger

For, the base sequence  is given by

where the  root  Zadoff-Chu sequence is defined by

with  given by

The length  of the Zadoff-Chu sequence is given by the largest prime number such that.

#### 5.5.1.2	Base sequences of length less than

For , , , and , the base sequence is given by

where the value of  is given by Table 5.5.1.2-1, Table 5.5.1.2-2, Table 5.5.1.2-3, and Table 5.5.1.2-4 for , , , and , respectively. For , the base sequence  is given by

Table 5.5.1.2-1: Definition of  for .

| 0 | -1 | 1 | 3 | -3 | 3 | 3 | 1 | 1 | 3 | 1 | -3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 3 | 3 | 3 | -1 | 1 | -3 | -3 | 1 | -3 | 3 |
| 2 | 1 | 1 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -3 | 1 | -1 |
| 3 | -1 | 1 | 1 | 1 | 1 | -1 | -3 | -3 | 1 | -3 | 3 | -1 |
| 4 | -1 | 3 | 1 | -1 | 1 | -1 | -3 | -1 | 1 | -1 | 1 | 3 |
| 5 | 1 | -3 | 3 | -1 | -1 | 1 | 1 | -1 | -1 | 3 | -3 | 1 |
| 6 | -1 | 3 | -3 | -3 | -3 | 3 | 1 | -1 | 3 | 3 | -3 | 1 |
| 7 | -3 | -1 | -1 | -1 | 1 | -3 | 3 | -1 | 1 | -3 | 3 | 1 |
| 8 | 1 | -3 | 3 | 1 | -1 | -1 | -1 | 1 | 1 | 3 | -1 | 1 |
| 9 | 1 | -3 | -1 | 3 | 3 | -1 | -3 | 1 | 1 | 1 | 1 | 1 |
| 10 | -1 | 3 | -1 | 1 | 1 | -3 | -3 | -1 | -3 | -3 | 3 | -1 |
| 11 | 3 | 1 | -1 | -1 | 3 | 3 | -3 | 1 | 3 | 1 | 3 | 3 |
| 12 | 1 | -3 | 1 | 1 | -3 | 1 | 1 | 1 | -3 | -3 | -3 | 1 |
| 13 | 3 | 3 | -3 | 3 | -3 | 1 | 1 | 3 | -1 | -3 | 3 | 3 |
| 14 | -3 | 1 | -1 | -3 | -1 | 3 | 1 | 3 | 3 | 3 | -1 | 1 |
| 15 | 3 | -1 | 1 | -3 | -1 | -1 | 1 | 1 | 3 | 1 | -1 | -3 |
| 16 | 1 | 3 | 1 | -1 | 1 | 3 | 3 | 3 | -1 | -1 | 3 | -1 |
| 17 | -3 | 1 | 1 | 3 | -3 | 3 | -3 | -3 | 3 | 1 | 3 | -1 |
| 18 | -3 | 3 | 1 | 1 | -3 | 1 | -3 | -3 | -1 | -1 | 1 | -3 |
| 19 | -1 | 3 | 1 | 3 | 1 | -1 | -1 | 3 | -3 | -1 | -3 | -1 |
| 20 | -1 | -3 | 1 | 1 | 1 | 1 | 3 | 1 | -1 | 1 | -3 | -1 |
| 21 | -1 | 3 | -1 | 1 | -3 | -3 | -3 | -3 | -3 | 1 | -1 | -3 |
| 22 | 1 | 1 | -3 | -3 | -3 | -3 | -1 | 3 | -3 | 1 | -3 | 3 |
| 23 | 1 | 1 | -1 | -3 | -1 | -3 | 1 | -1 | 1 | 3 | -1 | 1 |
| 24 | 1 | 1 | 3 | 1 | 3 | 3 | -1 | 1 | -1 | -3 | -3 | 1 |
| 25 | 1 | -3 | 3 | 3 | 1 | 3 | 3 | 1 | -3 | -1 | -1 | 3 |
| 26 | 1 | 3 | -3 | -3 | 3 | -3 | 1 | -1 | -1 | 3 | -1 | -3 |
| 27 | -3 | -1 | -3 | -1 | -3 | 3 | 1 | -1 | 1 | 3 | -3 | -3 |
| 28 | -1 | 3 | -3 | 3 | -1 | 3 | 3 | -3 | 3 | 3 | -1 | -1 |
| 29 | 3 | -3 | -3 | -1 | -1 | -3 | -1 | 3 | -3 | 3 | 1 | -1 |

Table 5.5.1.2-2: Definition of  for

| 0 | -1 | 3 | 1 | -3 | 3 | -1 | 1 | 3 | -3 | 3 | 1 | 3 | -3 | 3 | 1 | 1 | -1 | 1 | 3 | -3 | 3 | -3 | -1 | -3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -3 | 3 | -3 | -3 | -3 | 1 | -3 | -3 | 3 | -1 | 1 | 1 | 1 | 3 | 1 | -1 | 3 | -3 | -3 | 1 | 3 | 1 | 1 | -3 |
| 2 | 3 | -1 | 3 | 3 | 1 | 1 | -3 | 3 | 3 | 3 | 3 | 1 | -1 | 3 | -1 | 1 | 1 | -1 | -3 | -1 | -1 | 1 | 3 | 3 |
| 3 | -1 | -3 | 1 | 1 | 3 | -3 | 1 | 1 | -3 | -1 | -1 | 1 | 3 | 1 | 3 | 1 | -1 | 3 | 1 | 1 | -3 | -1 | -3 | -1 |
| 4 | -1 | -1 | -1 | -3 | -3 | -1 | 1 | 1 | 3 | 3 | -1 | 3 | -1 | 1 | -1 | -3 | 1 | -1 | -3 | -3 | 1 | -3 | -1 | -1 |
| 5 | -3 | 1 | 1 | 3 | -1 | 1 | 3 | 1 | -3 | 1 | -3 | 1 | 1 | -1 | -1 | 3 | -1 | -3 | 3 | -3 | -3 | -3 | 1 | 1 |
| 6 | 1 | 1 | -1 | -1 | 3 | -3 | -3 | 3 | -3 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -3 | -1 | 1 | -1 | 3 | -1 | -3 |
| 7 | -3 | 3 | 3 | -1 | -1 | -3 | -1 | 3 | 1 | 3 | 1 | 3 | 1 | 1 | -1 | 3 | 1 | -1 | 1 | 3 | -3 | -1 | -1 | 1 |
| 8 | -3 | 1 | 3 | -3 | 1 | -1 | -3 | 3 | -3 | 3 | -1 | -1 | -1 | -1 | 1 | -3 | -3 | -3 | 1 | -3 | -3 | -3 | 1 | -3 |
| 9 | 1 | 1 | -3 | 3 | 3 | -1 | -3 | -1 | 3 | -3 | 3 | 3 | 3 | -1 | 1 | 1 | -3 | 1 | -1 | 1 | 1 | -3 | 1 | 1 |
| 10 | -1 | 1 | -3 | -3 | 3 | -1 | 3 | -1 | -1 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -1 | 1 | 3 | 3 | -1 | 1 | -1 | 3 |
| 11 | 1 | 3 | 3 | -3 | -3 | 1 | 3 | 1 | -1 | -3 | -3 | -3 | 3 | 3 | -3 | 3 | 3 | -1 | -3 | 3 | -1 | 1 | -3 | 1 |
| 12 | 1 | 3 | 3 | 1 | 1 | 1 | -1 | -1 | 1 | -3 | 3 | -1 | 1 | 1 | -3 | 3 | 3 | -1 | -3 | 3 | -3 | -1 | -3 | -1 |
| 13 | 3 | -1 | -1 | -1 | -1 | -3 | -1 | 3 | 3 | 1 | -1 | 1 | 3 | 3 | 3 | -1 | 1 | 1 | -3 | 1 | 3 | -1 | -3 | 3 |
| 14 | -3 | -3 | 3 | 1 | 3 | 1 | -3 | 3 | 1 | 3 | 1 | 1 | 3 | 3 | -1 | -1 | -3 | 1 | -3 | -1 | 3 | 1 | 1 | 3 |
| 15 | -1 | -1 | 1 | -3 | 1 | 3 | -3 | 1 | -1 | -3 | -1 | 3 | 1 | 3 | 1 | -1 | -3 | -3 | -1 | -1 | -3 | -3 | -3 | -1 |
| 16 | -1 | -3 | 3 | -1 | -1 | -1 | -1 | 1 | 1 | -3 | 3 | 1 | 3 | 3 | 1 | -1 | 1 | -3 | 1 | -3 | 1 | 1 | -3 | -1 |
| 17 | 1 | 3 | -1 | 3 | 3 | -1 | -3 | 1 | -1 | -3 | 3 | 3 | 3 | -1 | 1 | 1 | 3 | -1 | -3 | -1 | 3 | -1 | -1 | -1 |
| 18 | 1 | 1 | 1 | 1 | 1 | -1 | 3 | -1 | -3 | 1 | 1 | 3 | -3 | 1 | -3 | -1 | 1 | 1 | -3 | -3 | 3 | 1 | 1 | -3 |
| 19 | 1 | 3 | 3 | 1 | -1 | -3 | 3 | -1 | 3 | 3 | 3 | -3 | 1 | -1 | 1 | -1 | -3 | -1 | 1 | 3 | -1 | 3 | -3 | -3 |
| 20 | -1 | -3 | 3 | -3 | -3 | -3 | -1 | -1 | -3 | -1 | -3 | 3 | 1 | 3 | -3 | -1 | 3 | -1 | 1 | -1 | 3 | -3 | 1 | -1 |
| 21 | -3 | -3 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 3 | 1 | -3 | -1 | 1 | -1 | 1 | -1 | -1 | 3 | 3 | -3 | -1 | 1 | -3 |
| 22 | -3 | -1 | -3 | 3 | 1 | -1 | -3 | -1 | -3 | -3 | 3 | -3 | 3 | -3 | -1 | 1 | 3 | 1 | -3 | 1 | 3 | 3 | -1 | -3 |
| 23 | -1 | -1 | -1 | -1 | 3 | 3 | 3 | 1 | 3 | 3 | -3 | 1 | 3 | -1 | 3 | -1 | 3 | 3 | -3 | 3 | 1 | -1 | 3 | 3 |
| 24 | 1 | -1 | 3 | 3 | -1 | -3 | 3 | -3 | -1 | -1 | 3 | -1 | 3 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -3 | -1 | 3 |
| 25 | 1 | -1 | 1 | -1 | 3 | -1 | 3 | 1 | 1 | -1 | -1 | -3 | 1 | 1 | -3 | 1 | 3 | -3 | 1 | 1 | -3 | -3 | -1 | -1 |
| 26 | -3 | -1 | 1 | 3 | 1 | 1 | -3 | -1 | -1 | -3 | 3 | -3 | 3 | 1 | -3 | 3 | -3 | 1 | -1 | 1 | -3 | 1 | 1 | 1 |
| 27 | -1 | -3 | 3 | 3 | 1 | 1 | 3 | -1 | -3 | -1 | -1 | -1 | 3 | 1 | -3 | -3 | -1 | 3 | -3 | -1 | -3 | -1 | -3 | -1 |
| 28 | -1 | -3 | -1 | -1 | 1 | -3 | -1 | -1 | 1 | -1 | -3 | 1 | 1 | -3 | 1 | -3 | -3 | 3 | 1 | 1 | -1 | 3 | -1 | -1 |
| 29 | 1 | 1 | -1 | -1 | -3 | -1 | 3 | -1 | 3 | -1 | 1 | 3 | 1 | -1 | 3 | 1 | 3 | -3 | -3 | 1 | -1 | -1 | 1 | 3 |

Table 5.5.1.2-3: Definition of  for

| 0 | -1 | -3 | 3 | -3 | 3 | -3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | -1 | 3 | -1 | 1 | 1 | 1 |
| 2 | 3 | -1 | -3 | -3 | 1 | 3 |
| 3 | 3 | -1 | -1 | 1 | -1 | -1 |
| 4 | -1 | -1 | -3 | 1 | -3 | -1 |
| 5 | 1 | 3 | -3 | -1 | -3 | 3 |
| 6 | -3 | 3 | -1 | -1 | 1 | -3 |
| 7 | -1 | -3 | -3 | 1 | 3 | 3 |
| 8 | 3 | -1 | -1 | 3 | 1 | 3 |
| 9 | 3 | -3 | 3 | 1 | -1 | 1 |
| 10 | -3 | 1 | -3 | -3 | -3 | -3 |
| 11 | -3 | -3 | -3 | 1 | -3 | -3 |
| 12 | 3 | -3 | 1 | -1 | -3 | -3 |
| 13 | 3 | -3 | 3 | -1 | -1 | -3 |
| 14 | 3 | -1 | 1 | 3 | 3 | 1 |
| 15 | -1 | 1 | -1 | -3 | 1 | 1 |
| 16 | -3 | -1 | -3 | -1 | 3 | 3 |
| 17 | 1 | -1 | 3 | -3 | 3 | 3 |
| 18 | 1 | 3 | 1 | 1 | -3 | 3 |
| 19 | -1 | -3 | -1 | -1 | 3 | -3 |
| 20 | 3 | -1 | -3 | -1 | -1 | -3 |
| 21 | 3 | 1 | 3 | -3 | -3 | 1 |
| 22 | 1 | 3 | -1 | -1 | 1 | -1 |
| 23 | -3 | 1 | -3 | 3 | 3 | 3 |
| 24 | 1 | 3 | -3 | 3 | -3 | 3 |
| 25 | -1 | -1 | 1 | -3 | 1 | -1 |
| 26 | 1 | -3 | -1 | -1 | 3 | 1 |
| 27 | -3 | -1 | -1 | 3 | 1 | 1 |
| 28 | -1 | 3 | -3 | -3 | -3 | 3 |
| 29 | 3 | 1 | -1 | 1 | 3 | 1 |

Table 5.5.1.2-4: Definition of  for

| 0 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | -1 | -3 | 3 | -1 | 3 | -1 | 3 | -3 | -1 | -1 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | -1 | 1 | -3 | -3 | -3 | 1 | -1 | 3 | -3 | -3 | 1 |
| 2 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | 1 | 3 | -3 | 1 | 1 | -3 | 1 | -3 | 3 | 1 | -1 |
| 3 | -3 | -3 | -3 | -3 | -3 | -1 | 1 | 3 | -3 | -1 | 3 | -1 | 3 | 1 | -1 | -3 | 3 | -3 |
| 4 | -3 | -3 | -3 | -3 | -3 | -1 | 3 | -3 | -1 | 1 | -1 | -3 | 3 | 3 | 1 | -3 | 1 | -1 |
| 5 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | -3 | -3 | 1 | 1 | 1 | -3 | 1 | 1 | -3 | -3 |
| 6 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | 1 | 1 | -3 | -3 | -3 | 1 | -1 | 3 | -1 | 3 |
| 7 | -3 | -3 | -3 | -3 | -3 | 1 | -3 | -1 | 3 | -1 | 3 | 3 | -1 | -1 | 1 | 3 | 3 | -1 |
| 8 | -3 | -3 | -3 | -3 | -3 | 1 | -1 | -1 | -1 | -3 | 3 | -1 | 3 | -3 | 3 | -1 | 1 | 3 |
| 9 | -3 | -3 | -3 | -3 | -3 | 3 | -3 | 1 | -1 | 3 | -3 | 3 | 3 | -1 | -3 | 1 | 1 | -3 |
| 10 | -3 | -3 | -3 | -3 | -3 | 3 | -1 | -3 | -3 | 1 | 1 | 3 | -3 | -1 | 3 | -1 | 3 | 1 |
| 11 | -3 | -3 | -3 | -3 | -3 | 3 | 3 | -1 | -1 | -1 | 3 | 1 | -3 | 3 | -1 | 1 | -3 | 1 |
| 12 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | -3 | 1 | 3 | 1 | -1 | 3 | -3 | -1 | -3 | 1 | 1 |
| 13 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | -1 | -1 | 3 | -3 | -3 | 1 | 3 | 1 | -3 | 1 |
| 14 | -3 | -3 | -3 | -3 | -1 | -3 | -3 | 1 | 3 | -3 | -1 | 3 | 1 | 3 | -1 | 3 | -1 | -3 |
| 15 | -3 | -3 | -3 | -3 | -1 | -3 | -1 | 3 | -3 | 1 | -3 | 1 | -1 | -3 | -3 | 1 | 1 | 3 |
| 16 | -3 | -3 | -3 | -3 | -1 | -1 | 3 | -3 | 3 | -1 | -3 | 1 | 1 | -1 | -3 | -1 | 3 | -3 |
| 17 | -3 | -3 | -3 | -3 | -1 | -1 | 3 | -1 | -3 | 1 | 3 | -1 | -3 | -3 | 1 | 3 | -1 | 1 |
| 18 | -3 | -3 | -3 | -3 | -1 | 3 | -1 | -1 | 3 | 3 | -1 | -3 | 1 | 1 | 1 | -1 | -3 | -1 |
| 19 | -3 | -3 | -3 | -3 | -1 | 3 | 1 | -3 | -1 | -3 | 3 | 1 | -1 | 3 | -1 | 1 | 3 | -1 |
| 20 | -3 | -3 | -3 | -3 | 1 | -3 | -3 | 3 | 1 | 1 | -3 | -1 | 1 | 3 | 3 | -1 | 3 | -1 |
| 21 | -3 | -3 | -3 | -3 | 1 | -3 | 1 | 3 | 1 | -1 | -1 | 3 | 3 | -1 | 1 | 1 | -3 | 3 |
| 22 | -3 | -3 | -3 | -3 | 1 | -3 | 3 | -3 | -1 | 3 | 1 | 1 | -1 | -1 | 3 | 3 | -1 | 3 |
| 23 | -3 | -3 | -3 | -3 | 1 | -3 | 3 | -1 | 3 | -3 | -1 | -1 | -1 | 1 | -3 | -3 | 3 | 1 |
| 24 | -3 | -3 | -3 | -3 | 1 | 1 | 3 | 1 | 1 | -1 | 3 | 1 | 1 | 3 | -1 | -3 | 1 | 3 |
| 25 | -3 | -3 | -3 | -3 | 1 | 3 | 3 | 3 | 1 | -3 | 1 | -3 | -3 | 3 | -3 | 1 | -1 | -3 |
| 26 | -3 | -3 | -3 | -3 | 3 | 1 | 3 | 3 | -1 | 3 | -3 | -3 | -1 | 3 | -1 | -1 | -3 | 1 |
| 27 | -3 | -3 | -3 | -1 | -3 | -3 | -1 | -1 | -3 | 3 | 3 | 1 | -3 | -1 | -1 | 3 | 1 | -3 |
| 28 | -3 | -3 | -3 | -1 | -3 | 1 | -1 | 1 | -3 | 3 | 1 | -3 | -1 | 1 | 3 | 1 | -1 | -1 |
| 29 | -3 | -3 | -3 | -1 | -3 | 3 | 1 | 1 | -1 | -1 | 1 | 3 | 1 | -3 | 1 | -3 | -1 | 1 |

#### 5.5.1.3	Group hopping

The sequence-group number  in slot  is defined by a group hopping pattern  and a sequence-shift pattern  according to

There are 17 different hopping patterns and 30 different sequence-shift patterns. Sequence-group hopping can be enabled or disabled by means of the cell-specific parameter Group-hopping-enabled provided by higher layers. Sequence-group hopping for PUSCH can be disabled for a certain UE through the higher-layer parameter Disable-sequence-group-hopping despite being enabled on a cell basis unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

The group-hopping pattern  may be different for PUSCH, PUCCH and SRS and is given by

where the pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5.

The sequence-shift pattern  definition differs between PUCCH, PUSCH and SRS.

For PUCCH, the sequence-shift pattern  is given by  where  is given by clause 5.5.1.5.

For PUSCH, the sequence-shift pattern  is given by , where  is configured by higher layers,  if no value for  is provided by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, otherwise it is given by  with  given by clause 5.5.1.5.

For SRS, the sequence-shift pattern  is given by where  is given by clause 5.5.1.5.

#### 5.5.1.4	Sequence hopping

Sequence hopping only applies for reference-signals of length .

For reference-signals of length , the base sequence number  within the base sequence group is given by .

For reference-signals of length , the base sequence number  within the base sequence group in slot  is defined by

where the pseudo-random sequence  is given by clause 7.2. The parameter Sequence-hopping-enabled provided by higher layers determines if sequence hopping is enabled or not. Sequence hopping for PUSCH can be disabled for a certain UE through the higher-layer parameter Disable-sequence-group-hopping despite being enabled on a cell basis unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

For PUSCH, MF-ePUCCH and MF-sPUCCH format 1/2/3, the pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5.

For SRS, the pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame where  is given by clause 5.5.1.5 and  is given by clause 5.5.1.3.

#### 5.5.1.5	Determining virtual cell identity for sequence generation

The definition of  depends on the type of transmission.

Transmissions associated with PUSCH:

-	 if no value for  is configured by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure,

-	 otherwise.

Transmissions associated with PUCCH:

-	 if no value for  is configured by higher layers,

-	 otherwise.

Sounding reference signals:

-	.

### 5.5.2	Demodulation reference signal

#### 5.5.2.1	Demodulation reference signal for PUSCH

##### 5.5.2.1.1	Reference signal sequence

The PUSCH demodulation reference signal sequence  associated with layer  is defined by

where

and  if the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3 and  otherwise.

Subclause 5.5.1 defines the sequence  where, for PUSCH demodulation reference signal sequence,  when the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3, and  otherwise. The orthogonal sequence  is given by  for DCI format 0 if the higher-layer parameter Activate-DMRS-with OCC is not set or if the temporary C-RNTI was used to transmit the most recent uplink-related DCI for the transport block associated with the corresponding PUSCH transmission. Otherwise,

-	if higher-layer parameter ul-DMRS-IFDMA is not set,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI [3],

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is not present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field indicates the use of Table 5.5.2.1.1-1, and

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field indicates the use of Table 5.5.2.1.1-3.

The cyclic shift  in a slot  is given as  with

where the value of  is given by Table 5.5.2.1.1-2 according to the parameter cyclicShift provided by higher layers. For non-BL/CE UEs  is given using the most recent uplink-related DCI MFA TS 36.212 [3] for the transport block associated with the corresponding PUSCH transmission as follows:

-	if the higher-layer parameter ul-DMRS-IFDMA is not set,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is not present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI,

-	if higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-1 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field indicates the use of Table 5.5.2.1.1-1, and

-	If higher-layer parameter ul-DMRS-IFDMA is set and the Cyclic Shift Field mapping table for DMRS bit field is present in the most recent uplink-related DCI,  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI when the Cyclic Shift Field mapping table for DMRS bit field indicates the use of Table 5.5.2.1.1-3.

For BL/CE UEs, a cyclic shift field of ‘000’ shall be assumed when determining   from Table 5.5.2.1.1-1.

The first row of Table 5.5.2.1.1-1 shall be used to obtain  and  if there is no uplink-related DCI for the same transport block associated with the corresponding PUSCH transmission, and

-	if the initial PUSCH for the same transport block is semi-persistently scheduled, or

-	if the initial PUSCH for the same transport block is scheduled by the random access response grant.

The quantity  is given by

where the pseudo-random sequence  is defined by clause 7.2. The application of  is cell-specific. The pseudo-random sequence generator shall be initialized with  at the beginning of each radio frame. The quantity   is given by  if no value for  is configured by higher layers or the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, otherwise it is given by .

The vector of reference signals shall be precoded according to

where  is the number of antenna ports used for PUSCH transmission.

For PUSCH transmission using a single antenna port, ,  and .

For spatial multiplexing,  or  and the precoding matrix  shall be identical to the precoding matrix used in clause 5.3.3A.2 for precoding of the PUSCH in the same subframe.

Table 5.5.2.1.1-1: Mapping of Cyclic Shift Field in uplink-related DCI format to  and

| Cyclic Shift Field in uplink-related DCI format [3] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 0 | 6 | 3 | 9 |  |  |  |  |
| 001 | 6 | 0 | 9 | 3 |  |  |  |  |
| 010 | 3 | 9 | 6 | 0 |  |  |  |  |
| 011 | 4 | 10 | 7 | 1 |  |  |  |  |
| 100 | 2 | 8 | 5 | 11 |  |  |  |  |
| 101 | 8 | 2 | 11 | 5 |  |  |  |  |
| 110 | 10 | 4 | 1 | 7 |  |  |  |  |
| 111 | 9 | 3 | 0 | 6 |  |  |  |  |

Table 5.5.2.1.1-2: Mapping of cyclicShift to values

| cyclicShift |  |
| --- | --- |
| 0 | 0 |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 6 |
| 5 | 8 |
| 6 | 9 |
| 7 | 10 |

Table 5.5.2.1.1-3: Mapping of Cyclic Shift Field in uplink-related DCI format to , , and

| Cyclic Shift Field in uplink-related DCI format [3] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 1 | 0 | 6 | 3 | 9 |  |  |  |  |
| 001 | 1 | 6 | 0 | 9 | 3 |  |  |  |  |
| 010 | 1 | 3 | 9 | 6 | 0 |  |  |  |  |
| 011 | 0 | 4 | 10 | 7 | 1 |  |  |  |  |
| 100 | 0 | 2 | 8 | 5 | 11 |  |  |  |  |
| 101 | 0 | 8 | 2 | 11 | 5 |  |  |  |  |
| 110 | 0 | 10 | 4 | 1 | 7 |  |  |  |  |
| 111 | 1 | 9 | 3 | 0 | 6 |  |  |  |  |

##### 5.5.2.1.2	Mapping to physical resources

For each antenna port used for transmission of the PUSCH, the sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to the resource blocks.  when the higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3, and  otherwise.

If higher-layer parameter ul-DMRS-IFDMA is set and the most recent uplink-related DCI contains the Cyclic Shift Field mapping table for DMRS bit field which indicates the use of Table 5.5.2.1.1-3, the mapping to resource elements , with  for normal cyclic prefix and  for extended cyclic prefix, in the subframe shall be in increasing order of first  for all values of  satisfying , then the slot number.  The quantity  is given by Table 5.5.2.1.1-3 using the cyclic shift field in the most recent uplink-related DCI.

For all other cases, the set of physical resource blocks used in the mapping process and the relation between the index  and the antenna port number   shall be identical to the corresponding PUSCH transmission as defined in clause 5.3.4. 
The mapping to resource elements , with  for normal cyclic prefix and  for extended cyclic prefix, in the subframe shall be in increasing order of first, then the slot number.

#### 5.5.2.2	Demodulation reference signal for PUCCH

##### 5.5.2.2.1	Reference signal sequence

The PUCCH demodulation reference signal sequence  for PUCCH formats 1, 1a, 1b, 2, 2a, 2b, and 3 is defined by

where

and  is the number of antenna ports used for PUCCH transmission. For PUCCH formats 2a and 2b,  equals  for , where  is defined in clause 5.4.2. For all other cases,

The sequence is given by clause 5.5.1 with  and  where the expression for the cyclic shift  is determined by the PUCCH format.

For PUCCH formats 1, 1a and 1b,   is given by

where , ,  and  are defined by clause 5.4.1. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-2, respectively.

For PUCCH formats 2, 2a and 2b,   is defined by clause 5.4.2. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-3, respectively.

For PUCCH format 3,  is given by

where  is given by Table 5.5.2.2.1-4 and  and   for the first and second slot in a subframe, respectively, are obtained from clause 5.4.2A. The number of reference symbols per slot  and the sequence  are given by Table 5.5.2.2.1-1 and 5.5.2.2.1-3, respectively.

Table 5.5.2.2.1-1: Number of PUCCH demodulation reference symbols per slot

| PUCCH format | Normal cyclic prefix | Extended cyclic prefix |
| --- | --- | --- |
| 1, 1a, 1b | 3 | 2 |
| 2, 3 | 2 | 1 |
| 2a, 2b | 2 | N/A |

Table 5.5.2.2.1-2: Orthogonal sequences  for PUCCH formats 1, 1a and 1b

| Sequence index | Normal cyclic prefix | Extended cyclic prefix |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  | N/A |

Table 5.5.2.2.1-3: Orthogonal sequences  for PUCCH formats 2, 2a, 2b and 3.

| Normal cyclic prefix | Extended cyclic prefix |
| --- | --- |

Table 5.5.2.2.1-4: Relation between  and   for PUCCH format 3.

| 0 | 0 | 0 |
| --- | --- | --- |
| 1 | 3 | 3 |
| 2 | 6 | 6 |
| 3 | 8 | 9 |
| 4 | 10 | N/A |

The PUCCH demodulation reference signal sequence  for PUCCH formats 4 and 5 is defined by

where

and

Subclause 5.5.1 defines the sequence  where .

The cyclic shift  in a slot  is given as  with

where the values of and  are given by Subclause 5.5.2.1.1 and

with  obtained as described in clause 5.4.2C.

##### 5.5.2.2.1MF1	Reference signal sequence for MF

The demodulation reference signal sequence

for MF-sPUCCH and MF-ePUCCH is defined by

where

and  is the number of antenna ports used for PUCCH transmission.

denotes the number of subcarriers used for RS transmission over

resource blocks.

represents the number of MF-PUCCH RS symbols per slot for MF-ePUCCH and MF-sPUCCH. The sequence

is defined in Section 5.5.1 with

.

The cyclic shift

for MF-ePUCCH and MF-sPUCCH formats 1 and 2 is given by

where

is given by Subclause 5.4,  is the slot number within a radio frame and  is the SC-FDMA symbol number within a slot. For MF-ePUCCH,

is given by the second column of Table 5.5.2.2.1-4 and

and

for the first and second slot in a subframe, respectively, are obtained from clause 5.4.2A. For MF-sPUCCH format 0, the cyclic shift is defined in clause 5.4.2MF.1. For MF-sPUCCH formats 1 and 2

is obtained as

np'=0,   if nMF-sPUCCH(F,p) mod 2NSF,intraMF-sPUCCH=03,   if nMF-sPUCCH(F,p) mod 2NSF,intraMF-sPUCCH=16,   if nMF-sPUCCH(F,p) mod 2NSF,intraMF-sPUCCH=28,   if nMF-sPUCCH(F,p) mod 2NSF,intraMF-sPUCCH=3,

applicable to MF-sPUCCH format F transmissions, where the MF-sPUCCH resource index

nMF-sPUCCH(F,p)

and orthogonal sequence length

are defined in Sections 5.4.2MF2 and 5.4.2MF3.

The cyclic shift for MF-sPUCCH format 3 is given by

where

And

is the MF-sPUCCH resource index defined in Section 5.4.2MF2.

The sequence

is given by Table 5.5.2.2.1MF1-1, where the first row is used for MF-ePUCCH and MF-sPUCCH formats 1 and 2. For MF-sPUCCH format 3, the choice of orthogonal sequence is indicated by higher layers and the sequence index is given by

,

while for MF-sPUCCH format 0, the orthogonal sequence is defined in clause 5.4.2.D.

Table 5.5.2.2.1MF1-1: Orthogonal sequences

| Sequence index | Normal cyclic prefix |
| --- | --- |
| 0 |  |
| 1 |  |

##### 5.5.2.2.2	Mapping to physical resources

The sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to resource elements on antenna port . The mapping shall be in increasing order of first, then  and finally the slot number. The set of values for  and the relation between the index  and the antenna port number  shall be identical to the values used for the corresponding PUCCH transmission. The values of the symbol index  in a slot are given by Table 5.5.2.2.2-1.

Table 5.5.2.2.2-1: Demodulation reference signal location for different PUCCH formats.

| PUCCH format | Set of values for |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 1, 1a, 1b | 2, 3, 4 | 2, 3 |
| 2, 3 | 1, 5 | 3 |
| 2a, 2b | 1, 5 | N/A |
| 4,5 | 3 | 2 |

##### 5.5.2.2.2MF1	Mapping to physical resources for MF

For MF-PUCCH, the sequence

shall be multiplied with the amplitude scaling factor

in order to conform to the transmit power specified in Section 5.1.2.1 in MFA TS 36.213 [4] and mapped in sequence starting with

to resource elements

on antenna port . The mapping shall be in increasing order of first, then , and finally the slot number. The set of values for  and the relation between the index  and the antenna port number  shall be identical to the values used for the corresponding MF PUCCH transmission. The values of the symbol index  in a slot for MF-ePUCCH are given by Table 5.5.2.2.2MF-1. For MF-sPUCCH, the first two symbols of the MF-sPUCCH transmission region defined in clause 5.4.2MF.1 are used for demodulation reference signal transmission.

Table 5.5.2.2.2MF1-1: Demodulation reference signal location for different PUCCH formats.

| PUCCH format | Set of values for |
| --- | --- |
|  | Normal cyclic prefix |
| MF-ePUCCH | 1, 5 |

### 5.5.3	Sounding reference signal

#### 5.5.3.1	Sequence generation

The sounding reference signal sequence  is defined by clause 5.5.1, where  is the sequence-group number defined in clause 5.5.1.3,  is the base sequence number defined in clause 5.5.1.4, and . The cyclic shift  of the sounding reference signal is given as

,

where  is configured separately for periodic and each configuration of aperiodic sounding by the higher-layer parameters cyclicShift and cyclicShift-ap, respectively,  for each UE and  is the number of antenna ports used for sounding reference signal transmission. The parameter   if  , otherwise . The parameter  is given by the higher layer parameter transmissionCombNum if configured, otherwise .

#### 5.5.3.2	Mapping to physical resources

The sequence shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in clause 5.1.3.1 in MFA TS 36.213 [4], and mapped in sequence starting with  to resource elements  on antenna port  according to

where  is the number of antenna ports used for sounding reference signal transmission and the relation between the index  and the antenna port  is given by Table 5.2.1-1. The set of antenna ports used for sounding reference signal transmission is configured independently for periodic and each configuration of aperiodic sounding. The quantity  is the frequency-domain starting position of the sounding reference signal and for  and   is the length of the sounding reference signal sequence defined as

where is given by Table 5.5.3.2-1 through Table 5.5.3.2-4 for each uplink bandwidth . The cell-specific parameter srs-BandwidthConfig,  and the UE-specific parameter srs-Bandwidth, are given by higher layers. For UpPTS,  shall be reconfigured to  if this reconfiguration is enabled by the cell-specific parameter srsMaxUpPts given by higher layers, otherwise if the reconfiguration is disabled ,where  is a SRS BW configuration and  is the set of SRS BW configurations from the Tables 5.5.3.2-1 to 5.5.3.2-4 for each uplink bandwidth ,  is the number of format 4 PRACH in the addressed UpPTS and derived from Table 5.7.1-4.

The frequency-domain starting position  is defined by

where for normal uplink subframes  is defined by

and for UpPTS by

The quantity  is given by

where the relation between the index  and the antenna port  is given by Table 5.2.1-1,  is given by the UE-specific parameter transmissionComb or transmissionComb-ap for periodic and each configuration of aperiodic transmission, respectively, provided by higher layers for the UE, and  is frequency position index. The variable  is equal to 0 for UpPTS in the first half frame and equal to 1 for UpPTS in the second half frame of a radio frame.

The frequency hopping of the sounding reference signal is configured by the parameter , provided by higher-layer parameter srs-HoppingBandwidth. Frequency hopping is not supported for aperiodic transmission. If frequency hopping of the sounding reference signal is not enabled (i.e., ),  the frequency position index  remains constant (unless re-configured) and is defined by  where the parameter  is given by higher-layer parameters freqDomainPosition and freqDomainPosition-ap for periodic and each configuration of aperiodic transmission, respectively. If frequency hopping of the sounding reference signal is enabled (i.e., ),  the frequency position indexes  are defined by

where  is given by Table 5.5.3.2-1 through Table 5.5.3.2-4 for each uplink bandwidth ,

where  regardless of the  value on Table 5.5.3.2-1 through Table 5.5.3.2-4, and

counts the number of UE-specific SRS transmissions, where  is UE-specific periodicity of SRS transmission defined in clause 8.2 of MFA TS 36.213 [4],  is SRS subframe offset defined in Table 8.2-2 of MFA TS 36.213 [4] and  is the maximum value of  for a certain configuration of SRS subframe offset.

The sounding reference signal shall be transmitted in the last symbol of the uplink subframe.

Table 5.5.3.2-1:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 1 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |
| 2 | 24 | 1 | 4 | 6 | 4 | 1 | 4 | 1 |
| 3 | 20 | 1 | 4 | 5 | 4 | 1 | 4 | 1 |
| 4 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |
| 5 | 12 | 1 | 4 | 3 | 4 | 1 | 4 | 1 |
| 6 | 8 | 1 | 4 | 2 | 4 | 1 | 4 | 1 |
| 7 | 4 | 1 | 4 | 1 | 4 | 1 | 4 | 1 |

Table 5.5.3.2-2:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 1 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |
| 2 | 40 | 1 | 20 | 2 | 4 | 5 | 4 | 1 |
| 3 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 4 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |
| 5 | 24 | 1 | 4 | 6 | 4 | 1 | 4 | 1 |
| 6 | 20 | 1 | 4 | 5 | 4 | 1 | 4 | 1 |
| 7 | 16 | 1 | 4 | 4 | 4 | 1 | 4 | 1 |

Table 5.5.3.2-3:  and , , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 72 | 1 | 24 | 3 | 12 | 2 | 4 | 3 |
| 1 | 64 | 1 | 32 | 2 | 16 | 2 | 4 | 4 |
| 2 | 60 | 1 | 20 | 3 | 4 | 5 | 4 | 1 |
| 3 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 4 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |
| 5 | 40 | 1 | 20 | 2 | 4 | 5 | 4 | 1 |
| 6 | 36 | 1 | 12 | 3 | 4 | 3 | 4 | 1 |
| 7 | 32 | 1 | 16 | 2 | 8 | 2 | 4 | 2 |

Table 5.5.3.2-4: and, , values for the uplink bandwidth of

| SRS bandwidth configuration | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  | SRS-Bandwidth |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 96 | 1 | 48 | 2 | 24 | 2 | 4 | 6 |
| 1 | 96 | 1 | 32 | 3 | 16 | 2 | 4 | 4 |
| 2 | 80 | 1 | 40 | 2 | 20 | 2 | 4 | 5 |
| 3 | 72 | 1 | 24 | 3 | 12 | 2 | 4 | 3 |
| 4 | 64 | 1 | 32 | 2 | 16 | 2 | 4 | 4 |
| 5 | 60 | 1 | 20 | 3 | 4 | 5 | 4 | 1 |
| 6 | 48 | 1 | 24 | 2 | 12 | 2 | 4 | 3 |
| 7 | 48 | 1 | 16 | 3 | 8 | 2 | 4 | 2 |

#### 5.5.3.3	Sounding reference signal subframe configuration

The cell-specific subframe configuration period  and the cell-specific subframe offset  for the transmission of sounding reference signals are listed in Tables 5.5.3.3-1 and 5.5.3.3-2, for frame structures type 1 and 2 respectively, where the parameter srs-SubframeConfig is provided by higher layers. Sounding reference signal subframes are the subframes satisfying. For frame structure type 2, a sounding reference signal is transmitted only in uplink subframes or UpPTS.

Table 5.5.3.3-1: Frame structure type 1 sounding reference signal subframe configuration

| srs-SubframeConfig | Binary | Configuration Period (subframes) | Transmission offset (subframes) |
| --- | --- | --- | --- |
| 0 | 0000 | 1 | {0} |
| 1 | 0001 | 2 | {0} |
| 2 | 0010 | 2 | {1} |
| 3 | 0011 | 5 | {0} |
| 4 | 0100 | 5 | {1} |
| 5 | 0101 | 5 | {2} |
| 6 | 0110 | 5 | {3} |
| 7 | 0111 | 5 | {0,1} |
| 8 | 1000 | 5 | {2,3} |
| 9 | 1001 | 10 | {0} |
| 10 | 1010 | 10 | {1} |
| 11 | 1011 | 10 | {2} |
| 12 | 1100 | 10 | {3} |
| 13 | 1101 | 10 | {0,1,2,3,4,6,8} |
| 14 | 1110 | 10 | {0,1,2,3,4,5,6,8} |
| 15 | 1111 | reserved | reserved |

Table 5.5.3.3-2: Frame structure type 2 sounding reference signal subframe configuration

| srs-SubframeConfig | Binary | Configuration Period (subframes) | Transmission offset (subframes) |
| --- | --- | --- | --- |
| 0 | 0000 | 5 | {1} |
| 1 | 0001 | 5 | {1, 2} |
| 2 | 0010 | 5 | {1, 3} |
| 3 | 0011 | 5 | {1, 4} |
| 4 | 0100 | 5 | {1, 2, 3} |
| 5 | 0101 | 5 | {1, 2, 4} |
| 6 | 0110 | 5 | {1, 3, 4} |
| 7 | 0111 | 5 | {1, 2, 3, 4} |
| 8 | 1000 | 10 | {1, 2, 6} |
| 9 | 1001 | 10 | {1, 3, 6} |
| 10 | 1010 | 10 | {1, 6, 7} |
| 11 | 1011 | 10 | {1, 2, 6, 8} |
| 12 | 1100 | 10 | {1, 3, 6, 9} |
| 13 | 1101 | 10 | {1, 4, 6, 7} |
| 14 | 1110 | reserved | reserved |
| 15 | 1111 | reserved | reserved |

### 5.5.3MF1	Sounding reference signal for MF UE

#### 5.5.3MF1.1	Sequence generation

The sounding reference signal for MF is based on the reference signal sequence

defined by Section 5.5.1, where  is the sequence-group number defined in Section 5.5.1.3 and  is the base sequence number defined in Section 5.5.1.4. The MF-SRS sequence is defined as

where

, .

denotes the number of subcarriers used for MF-SRS transmission,

is the number of resource blocks derived from a higher layer parameter mf_frequencyIndex or from resource block assignment in DCI format 0A/0B/4A/4B,

denotes the number of symbols used for SRS transmission where

for trigger type 0 and trigger type 2, and

for trigger type 1. The cyclic shift  of the sounding reference signal is given as

,

where

.

is configured for each configuration of aperiodic sounding by the higher-layer parameter mf- cyclicShift, for each UE and

Nap

is the number of antenna ports used for sounding reference signal transmission.

The orthogonal sequence

w(γp)l

is defined as 1 for

NSRS=1.

Otherwise, it is listed in Table 5.5.3MF1.1-1. The sequence index γp is given by

,

where

is configured by higher-layer parameter mf-transmissionSRSOCCIndex.

Table 5.5.3MF1.1-1: Orthogonal sequences , when

| Sequence index γp | Sequence |
| --- | --- |
| 0 | [1,1,1,1] |
| 1 | [1,-1,1,-1] |
| 2 | [1,-1,-1,1] |
| 3 | [1,1,-1,-1] |

#### 5.5.3MF1.2	Mapping to physical resources

The sequence shall be multiplied with the amplitude scaling factor

in order to conform to the transmit power

specified in Section 5.1.3.1 in MFA TS 36.213 [4], and mapped in sequence starting with

to resource elements

on antenna port  according to

where

,

is the number of interlaces allocated for SRS transmission, and

is the number of antenna ports used for sounding reference signal transmission and the relation between the index  and the antenna port  is given by Table 5.2.1-1. The set of antenna ports used for sounding reference signal transmission is configured independently for each configuration of aperiodic sounding. The quantity

is the frequency-domain starting position of jth interlace allocated for the sounding reference signal, which is defined by

, , .nSRSIj∈0,1,2,…,NRBUL10-1, and

nSRSI(j)

is a monotonically increasing function of j,

For trigger type 0, trigger type 1 with DCI format 1A/2B/2C/2D  and trigger type 2,

is the jth interlace indicated by higher-layer configutation. For trigger type 0, trigger type 1 with DCI format 1A/2B/2C/2D, the interlace nSRSI(j) is given by the higher-layer parameter mf-frequencyIndex which is defined as a 10-bit bitmap corresponding to each interlace in increasing order, where the LSB of the bitmap indicates interlace 0 and “1” indicates that the interlace is allocated. For trigger type 2, nSRSI(j) is given by mf-frequencyIndex which is an integer value.

For trigger type 1 with DCI format 0A/0B/4A/4B,

is the jth interlace configured by DCI format 0A/0B/4A/4B with SRS request bit set to true.

## 5.6	SC-FDMA baseband signal generation

This clause applies to all uplink physical signals and uplink physical channels except the physical random access channel.

The time-continuous signal  for antenna port  in SC-FDMA symbol  in an uplink slot is defined by

for where , ,  and  is the content of resource element  on antenna port .

For frame structure type 3, if the associated DCI indicates PUSCH starting position other than ‘00’,   is given by

where

and  is given by clause 8.1. The UE behaviour if  is undefined.

The SC-FDMA symbols in a slot shall be transmitted in increasing order of , starting with , where SC-FDMA symbol starts at time   within the slot.

Table 5.6-1 lists the values of that shall be used.

Table 5.6-1: SC-FDMA parameters

| Configuration | Cyclic prefix length |
| --- | --- |
| Normal cyclic prefix |  |
| Extended cyclic prefix |  |

## 5.7	Physical random access channel

### 5.7.1	Time and frequency structure

The physical layer random access preamble for an LTE cell, illustrated in Figure 5.7.1-1, consists of a cyclic prefix of length and a sequence part of length. The parameter values are listed in Table 5.7.1-1 and depend on the frame structure and the random access configuration. Higher layers control the preamble format.

Figure 5.7.1-1: Random access preamble format

Table 5.7.1-1: Random access preamble parameters

| Preamble format |  |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 (see Note) |  |  |
| NOTE: Frame structure type 2 and special subframe configurations with UpPTS lengths and only assuming that the number of additional SC-FDMA symbols in UpPTS X in Table 4.2-1 is 0. |  |  |

The physical layer random access preamble for the MF cell uses the MF-sPUCCH format 0 defined by Section  5.4.2MF1 in case of MF-preamble format 0. Higher layers control the MF-preamble format.

For an LTE cell, the transmission of a random access preamble, if triggered by the MAC layer, is restricted to certain time and frequency resources. These resources are enumerated in increasing order of the subframe number within the radio frame and the physical resource blocks in the frequency domain such that index 0 correspond to the lowest numbered physical resource block and subframe within the radio frame. PRACH resources within the radio frame are indicated by a PRACH configuration index, where the indexing is in the order of appearance in Table 5.7.1-2 and Table 5.7.1-4.

For an MF cell, the transmission of a random access preamble, if triggered by the MAC layer, is restricted to certain time and frequency resources. These resources are enumerated in increasing order of the subframe number within the radio frame and the interlace number in the frequency domain, such that index 0 corresponds to the lowest-numbered interlace and subframe within the radio frame. MF-sPRACH resources within the radio frame are indicated by a PRACH configuration index, where the indexing is in the order of appearance as listed in Table 5.7.1-5.

For non-BL/CE UEs there is a single PRACH configuration with  given by the higher-layer parameter prach-FrequencyOffset in an LTE cell.

For BL/CE UEs, for each PRACH coverage enhancement level, there is a PRACH configuration configured by higher layers with a PRACH configuration index (prach-ConfigurationIndex), a PRACH frequency offset  (prach-FrequencyOffset), a number of PRACH repetitions per attempt  (numRepetitionPerPreambleAttempt) and optionally a PRACH starting subframe periodicity  (prach-StartingSubframe). PRACH of preamble format 0-3 is transmitted  times, whereas PRACH of preamble format 4 is transmitted one time only.

There is a single MF-PRACH configuration with the higher-layer parameter mf-PRACH-FrequencyIndex in an MF cell.

For BL/CE UEs and for each PRACH coverage enhancement level, if frequency hopping is enabled for a PRACH configuration by the higher-layer parameter prach-HoppingConfig, the value of the parameter  depends on the SFN and the PRACH configuration index and is given by

-	In case the PRACH configuration index is such that a PRACH resource occurs in every radio frame when calculated as below from Table 5.7.1-2 or Table 5.7.1-4,

-	otherwise

where  is the system frame number corresponding to the first subframe for each PRACH repetition,  corresponds to a cell-specific higher-layer parameter prach-HoppingOffset. If frequency hopping is not enabled for the PRACH configuration then .

For frame structure type 1 with preamble format 0-3, for each of the  PRACH configurations there is at most one random access resource per subframe. 
Table 5.7.1-2 lists the preamble formats according to Table 5.7.1-1 and the subframes in which random access preamble transmission is allowed for a given configuration in frame structure type 1. The start of the random access preamble shall be aligned with the start of the corresponding uplink subframe at the UE assuming , where  is defined in clause 8.1. For PRACH configurations 0, 1, 2, 15, 16, 17, 18, 31, 32, 33, 34, 47, 48, 49, 50 and 63 the UE may for handover purposes assume an absolute value of the relative time difference between radio frame  in the current cell and the target cell of less than . 
The first physical resource block  allocated to the PRACH opportunity considered for preamble formats 0, 1, 2 and 3 is defined as .

Table 5.7.1-2: Frame structure type 1 random access configuration for preamble formats 0-3

| PRACH Configuration Index | Preamble Format | System frame number | Subframe number | PRACH Configuration Index | Preamble Format | System frame number | Subframe number |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Even | 1 | 32 | 2 | Even | 1 |
| 1 | 0 | Even | 4 | 33 | 2 | Even | 4 |
| 2 | 0 | Even | 7 | 34 | 2 | Even | 7 |
| 3 | 0 | Any | 1 | 35 | 2 | Any | 1 |
| 4 | 0 | Any | 4 | 36 | 2 | Any | 4 |
| 5 | 0 | Any | 7 | 37 | 2 | Any | 7 |
| 6 | 0 | Any | 1, 6 | 38 | 2 | Any | 1, 6 |
| 7 | 0 | Any | 2 ,7 | 39 | 2 | Any | 2 ,7 |
| 8 | 0 | Any | 3, 8 | 40 | 2 | Any | 3, 8 |
| 9 | 0 | Any | 1, 4, 7 | 41 | 2 | Any | 1, 4, 7 |
| 10 | 0 | Any | 2, 5, 8 | 42 | 2 | Any | 2, 5, 8 |
| 11 | 0 | Any | 3, 6, 9 | 43 | 2 | Any | 3, 6, 9 |
| 12 | 0 | Any | 0, 2, 4, 6, 8 | 44 | 2 | Any | 0, 2, 4, 6, 8 |
| 13 | 0 | Any | 1, 3, 5, 7, 9 | 45 | 2 | Any | 1, 3, 5, 7, 9 |
| 14 | 0 | Any | 0, 1, 2, 3, 4, 5, 6, 7, 8, 9 | 46 | N/A | N/A | N/A |
| 15 | 0 | Even | 9 | 47 | 2 | Even | 9 |
| 16 | 1 | Even | 1 | 48 | 3 | Even | 1 |
| 17 | 1 | Even | 4 | 49 | 3 | Even | 4 |
| 18 | 1 | Even | 7 | 50 | 3 | Even | 7 |
| 19 | 1 | Any | 1 | 51 | 3 | Any | 1 |
| 20 | 1 | Any | 4 | 52 | 3 | Any | 4 |
| 21 | 1 | Any | 7 | 53 | 3 | Any | 7 |
| 22 | 1 | Any | 1, 6 | 54 | 3 | Any | 1, 6 |
| 23 | 1 | Any | 2 ,7 | 55 | 3 | Any | 2 ,7 |
| 24 | 1 | Any | 3, 8 | 56 | 3 | Any | 3, 8 |
| 25 | 1 | Any | 1, 4, 7 | 57 | 3 | Any | 1, 4, 7 |
| 26 | 1 | Any | 2, 5, 8 | 58 | 3 | Any | 2, 5, 8 |
| 27 | 1 | Any | 3, 6, 9 | 59 | 3 | Any | 3, 6, 9 |
| 28 | 1 | Any | 0, 2, 4, 6, 8 | 60 | N/A | N/A | N/A |
| 29 | 1 | Any | 1, 3, 5, 7, 9 | 61 | N/A | N/A | N/A |
| 30 | N/A | N/A | N/A | 62 | N/A | N/A | N/A |
| 31 | 1 | Even | 9 | 63 | 3 | Even | 9 |

For frame structure type 2 with preamble formats 0-4, for each of the PRACH configurations there might be multiple random access resources in an UL subframe (or UpPTS for preamble format 4) depending on the UL/DL configuration [see table 4.2-2]. Table 5.7.1-3 lists PRACH configurations allowed for frame structure type 2 where the configuration index corresponds to a certain combination of preamble format, PRACH density value,  and version index, . 
For frame structure type 2 with PRACH configuration indices 0, 1, 2, 20, 21, 22, 30, 31, 32, 40, 41, 42, 48, 49, 50, or with PRACH configuration indices 51, 53, 54, 55, 56, 57 in UL/DL configuration 3, 4, 5, the UE may for handover purposes assume an absolute value of the relative time difference between radio frame in the current cell and the target cell is less than .

Table 5.7.1-3: Frame structure type 2 random access configurations for preamble formats 0-4

| PRACH configuration Index | Preamble Format | Density Per 10 ms | Version | PRACH configuration Index | Preamble Format | Density Per 10 ms | Version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.5 | 0 | 32 | 2 | 0.5 | 2 |
| 1 | 0 | 0.5 | 1 | 33 | 2 | 1 | 0 |
| 2 | 0 | 0.5 | 2 | 34 | 2 | 1 | 1 |
| 3 | 0 | 1 | 0 | 35 | 2 | 2 | 0 |
| 4 | 0 | 1 | 1 | 36 | 2 | 3 | 0 |
| 5 | 0 | 1 | 2 | 37 | 2 | 4 | 0 |
| 6 | 0 | 2 | 0 | 38 | 2 | 5 | 0 |
| 7 | 0 | 2 | 1 | 39 | 2 | 6 | 0 |
| 8 | 0 | 2 | 2 | 40 | 3 | 0.5 | 0 |
| 9 | 0 | 3 | 0 | 41 | 3 | 0.5 | 1 |
| 10 | 0 | 3 | 1 | 42 | 3 | 0.5 | 2 |
| 11 | 0 | 3 | 2 | 43 | 3 | 1 | 0 |
| 12 | 0 | 4 | 0 | 44 | 3 | 1 | 1 |
| 13 | 0 | 4 | 1 | 45 | 3 | 2 | 0 |
| 14 | 0 | 4 | 2 | 46 | 3 | 3 | 0 |
| 15 | 0 | 5 | 0 | 47 | 3 | 4 | 0 |
| 16 | 0 | 5 | 1 | 48 | 4 | 0.5 | 0 |
| 17 | 0 | 5 | 2 | 49 | 4 | 0.5 | 1 |
| 18 | 0 | 6 | 0 | 50 | 4 | 0.5 | 2 |
| 19 | 0 | 6 | 1 | 51 | 4 | 1 | 0 |
| 20 | 1 | 0.5 | 0 | 52 | 4 | 1 | 1 |
| 21 | 1 | 0.5 | 1 | 53 | 4 | 2 | 0 |
| 22 | 1 | 0.5 | 2 | 54 | 4 | 3 | 0 |
| 23 | 1 | 1 | 0 | 55 | 4 | 4 | 0 |
| 24 | 1 | 1 | 1 | 56 | 4 | 5 | 0 |
| 25 | 1 | 2 | 0 | 57 | 4 | 6 | 0 |
| 26 | 1 | 3 | 0 | 58 | N/A | N/A | N/A |
| 27 | 1 | 4 | 0 | 59 | N/A | N/A | N/A |
| 28 | 1 | 5 | 0 | 60 | N/A | N/A | N/A |
| 29 | 1 | 6 | 0 | 61 | N/A | N/A | N/A |
| 30 | 2 | 0.5 | 0 | 62 | N/A | N/A | N/A |
| 31 | 2 | 0.5 | 1 | 63 | N/A | N/A | N/A |

Table 5.7.1-4 lists the mapping to physical resources for the different random access opportunities needed for a certain PRACH density value, . Each quadruple of the format  indicates the location of a specific random access resource, where  is a frequency resource index within the considered time instance,  indicates whether the resource is reoccurring in all radio frames, in even radio frames, or in odd radio frames, respectively,  indicates whether the random access resource is located in first half frame or in second half frame, respectively, and where  is the uplink subframe number where the preamble starts, counting from 0 at the first uplink subframe between 2 consecutive downlink-to-uplink switch points, with the exception of preamble format 4 where  is denoted as (*). The start of the random access preamble formats 0-3 shall be aligned with the start of the corresponding uplink subframe at the UE assuming  and the random access preamble format 4 shall start  before the end of the UpPTS at the UE, where the UpPTS is referenced to the UE’s uplink frame timing assuming.

The random access opportunities for each PRACH configuration shall be allocated in time first and then in frequency if and only if time multiplexing is not sufficient to hold all opportunities of a PRACH configuration needed for a certain density value  without overlap in time. For preamble format 0-3, the frequency multiplexing shall be done according to

where  is the number of uplink resource blocks, is the first physical resource block allocated to the PRACH opportunity considered and where  is the first physical resource block available for PRACH.

For preamble format 4, the frequency multiplexing shall be done according to

whereis the system frame number and whereis the number of DL to UL switch points within the radio frame.

For BL/CE UEs, only a subset of the subframes allowed for preamble transmission are allowed as starting subframes for the  repetitions. The allowed starting subframes for a PRACH configuration are determined as follows:

-	Enumerate the subframes that are allowed for preamble transmission for the PRACH configuration as  where   and   correspond to the two subframes allowed for preamble transmission with the smallest and the largest absolute subframe number   , respectively.

-	If a PRACH starting subframe periodicity   is not provided by higher layers, the periodicity of the allowed starting subframes in terms of subframes allowed for preamble transmission is . The allowed starting subframes defined over  are given by  where

-	If a PRACH starting subframe periodicity   is provided by higher layers, it indicates the periodicity of the allowed starting subframes in terms of subframes allowed for preamble transmission. The allowed starting subframes defined over  are given by  where

-	No starting subframe with absolute subframe number   is allowed.

Each random access preamble occupies a bandwidth corresponding to 6 consecutive resource blocks for both frame structures.

Table 5.7.1-4: Frame structure type 2 random access preamble mapping in time and frequency

| PRACH configuration Index (See Table 5.7.1-3) | UL/DL configuration (See Table 4.2-2) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 0 | (0,1,0,2) | (0,1,0,1) | (0,1,0,0) | (0,1,0,2) | (0,1,0,1) | (0,1,0,0) | (0,1,0,2) |
| 1 | (0,2,0,2) | (0,2,0,1) | (0,2,0,0) | (0,2,0,2) | (0,2,0,1) | (0,2,0,0) | (0,2,0,2) |
| 2 | (0,1,1,2) | (0,1,1,1) | (0,1,1,0) | (0,1,0,1) | (0,1,0,0) | N/A | (0,1,1,1) |
| 3 | (0,0,0,2) | (0,0,0,1) | (0,0,0,0) | (0,0,0,2) | (0,0,0,1) | (0,0,0,0) | (0,0,0,2) |
| 4 | (0,0,1,2) | (0,0,1,1) | (0,0,1,0) | (0,0,0,1) | (0,0,0,0) | N/A | (0,0,1,1) |

| 5 | (0,0,0,1) | (0,0,0,0) | N/A | (0,0,0,0) | N/A | N/A | (0,0,0,1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | (0,0,0,2) (0,0,1,2) | (0,0,0,1) (0,0,1,1) | (0,0,0,0) (0,0,1,0) | (0,0,0,1) (0,0,0,2) | (0,0,0,0) (0,0,0,1) | (0,0,0,0) (1,0,0,0) | (0,0,0,2) (0,0,1,1) |
| 7 | (0,0,0,1) (0,0,1,1) | (0,0,0,0) (0,0,1,0) | N/A | (0,0,0,0) (0,0,0,2) | N/A | N/A | (0,0,0,1) (0,0,1,0) |
| 8 | (0,0,0,0) (0,0,1,0) | N/A | N/A | (0,0,0,0) (0,0,0,1) | N/A | N/A | (0,0,0,0) (0,0,1,1) |
| 9 | (0,0,0,1) (0,0,0,2) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) | (0,0,0,1) (0,0,0,2) (0,0,1,1) |
| 10 | (0,0,0,0) (0,0,1,0) (0,0,1,1) | (0,0,0,1) (0,0,1,0) (0,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,1,0) | N/A | (0,0,0,0) (0,0,0,1) (1,0,0,0) | N/A | (0,0,0,0) (0,0,0,2) (0,0,1,0) |
| 11 | N/A | (0,0,0,0) (0,0,0,1) (0,0,1,0) | N/A | N/A | N/A | N/A | (0,0,0,1) (0,0,1,0) (0,0,1,1) |
| 12 | (0,0,0,1) (0,0,0,2) (0,0,1,1) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,0) (1,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) | (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) |
| 13 | (0,0,0,0) (0,0,0,2) (0,0,1,0) (0,0,1,2) | N/A | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,1) | N/A | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,1) |
| 14 | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) | N/A | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,0) | N/A | N/A | (0,0,0,0) (0,0,0,2) (0,0,1,0) (0,0,1,1) |
| 15 | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,1) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,0,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,0,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,1) (1,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,0) (1,0,0,1) (2,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) (4,0,0,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) |
| 16 | (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,1,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,0) (1,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,0) (1,0,0,1) (2,0,0,0) | N/A | N/A |
| 17 | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,0,0) | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,0) (1,0,0,1) | N/A | N/A | N/A |
| 18 | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) (0,0,1,2) | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,0,1) (1,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,0,0) (2,0,1,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (1,0,0,0) (1,0,0,1) (1,0,0,2) | (0,0,0,0) (0,0,0,1) (1,0,0,0) (1,0,0,1) (2,0,0,0) (2,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) (4,0,0,0) (5,0,0,0) | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) (1,0,0,2) |
| 19 | N/A | (0,0,0,0) (0,0,0,1) (0,0,1,0) (0,0,1,1) (1,0,0,0) (1,0,1,0) | N/A | N/A | N/A | N/A | (0,0,0,0) (0,0,0,1) (0,0,0,2) (0,0,1,0) (0,0,1,1) (1,0,1,1) |
| 20 / 30 | (0,1,0,1) | (0,1,0,0) | N/A | (0,1,0,1) | (0,1,0,0) | N/A | (0,1,0,1) |
| 21 / 31 | (0,2,0,1) | (0,2,0,0) | N/A | (0,2,0,1) | (0,2,0,0) | N/A | (0,2,0,1) |
| 22 / 32 | (0,1,1,1) | (0,1,1,0) | N/A | N/A | N/A | N/A | (0,1,1,0) |

| 23 / 33 | (0,0,0,1) | (0,0,0,0) | N/A | (0,0,0,1) | (0,0,0,0) | N/A | (0,0,0,1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 / 34 | (0,0,1,1) | (0,0,1,0) | N/A | N/A | N/A | N/A | (0,0,1,0) |
| 25 / 35 | (0,0,0,1) (0,0,1,1) | (0,0,0,0) (0,0,1,0) | N/A | (0,0,0,1) (1,0,0,1) | (0,0,0,0) (1,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) |
| 26 / 36 | (0,0,0,1) (0,0,1,1) (1,0,0,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) | N/A | (0,0,0,1) (1,0,0,1) (2,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) (1,0,0,1) |
| 27 / 37 | (0,0,0,1) (0,0,1,1) (1,0,0,1) (1,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) | N/A | (0,0,0,1) (1,0,0,1) (2,0,0,1) (3,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) (1,0,0,1) (1,0,1,0) |
| 28 / 38 | (0,0,0,1) (0,0,1,1) (1,0,0,1) (1,0,1,1) (2,0,0,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,0,0) | N/A | (0,0,0,1) (1,0,0,1) (2,0,0,1) (3,0,0,1) (4,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) (4,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) (1,0,0,1) (1,0,1,0) (2,0,0,1) |
| 29 /39 | (0,0,0,1) (0,0,1,1) (1,0,0,1) (1,0,1,1) (2,0,0,1) (2,0,1,1) | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) (2,0,0,0) (2,0,1,0) | N/A | (0,0,0,1) (1,0,0,1) (2,0,0,1) (3,0,0,1) (4,0,0,1) (5,0,0,1) | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) (4,0,0,0) (5,0,0,0) | N/A | (0,0,0,1) (0,0,1,0) (1,0,0,1) (1,0,1,0) (2,0,0,1) (2,0,1,0) |
| 40 | (0,1,0,0) | N/A | N/A | (0,1,0,0) | N/A | N/A | (0,1,0,0) |
| 41 | (0,2,0,0) | N/A | N/A | (0,2,0,0) | N/A | N/A | (0,2,0,0) |
| 42 | (0,1,1,0) | N/A | N/A | N/A | N/A | N/A | N/A |
| 43 | (0,0,0,0) | N/A | N/A | (0,0,0,0) | N/A | N/A | (0,0,0,0) |
| 44 | (0,0,1,0) | N/A | N/A | N/A | N/A | N/A | N/A |
| 45 | (0,0,0,0) (0,0,1,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) |
| 46 | (0,0,0,0) (0,0,1,0) (1,0,0,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) (2,0,0,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) (2,0,0,0) |
| 47 | (0,0,0,0) (0,0,1,0) (1,0,0,0) (1,0,1,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) | N/A | N/A | (0,0,0,0) (1,0,0,0) (2,0,0,0) (3,0,0,0) |
| 48 | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) | (0,1,0,*) |
| 49 | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) | (0,2,0,*) |
| 50 | (0,1,1,*) | (0,1,1,*) | (0,1,1,*) | N/A | N/A | N/A | (0,1,1,*) |
| 51 | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) | (0,0,0,*) |
| 52 | (0,0,1,*) | (0,0,1,*) | (0,0,1,*) | N/A | N/A | N/A | (0,0,1,*) |
| 53 | (0,0,0,*) (0,0,1,*) | (0,0,0,*) (0,0,1,*) | (0,0,0,*) (0,0,1,*) | (0,0,0,*) (1,0,0,*) | (0,0,0,*) (1,0,0,*) | (0,0,0,*) (1,0,0,*) | (0,0,0,*) (0,0,1,*) |
| 54 | (0,0,0,*) (0,0,1,*) (1,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) |
| 55 | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) |
| 56 | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) |
| 57 | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) (2,0,1,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) (2,0,1,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) (2,0,1,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) (5,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) (5,0,0,*) | (0,0,0,*) (1,0,0,*) (2,0,0,*) (3,0,0,*) (4,0,0,*) (5,0,0,*) | (0,0,0,*) (0,0,1,*) (1,0,0,*) (1,0,1,*) (2,0,0,*) (2,0,1,*) |
| 58 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 59 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 60 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 61 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 62 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 63 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

NOTE:	*  UpPTS

For frame structure type 3 with MF-preamble format 0, for each of the MF-PRACH configurations there might be multiple random access resources per subframe, depending on the number of allocated interlaces for MF-sPRACH indicated by the higher-layer parameter mf-PRACH-FrequencyIndex and mf-PRACH-NumInterlaces.
Table 5.7.1-5 lists the MF-preamble format and the subframes in which random access preamble transmission for MF cell is allowed for a given configuration in frame structure type 3. The start of the random access preamble for MF-preamble format 0 shall be aligned with the start of fourth symbol in the second slot of the corresponding subframe at the UE assuming

,

where

is defined in Section 8.1.The number of interlace allocated to each of MF-sPRACH opportunities

given by the higher-layer parameter mf-PRACH-NumInterlaces and considered for MF-preamble format 0 is defined as an enumerated indicator where the value of “oneInterlacePerSPRACH” indicates one interlace allocated for each MF-sPRACH opportunity and the value of “twoInterlacesPerSPRACH” indicates two interlaces allocated for each MF-sPRACH opportunity. The interlace allocation to the first interlace of each MF-sPRACH opportunity given by the higher-layer parameter mf-PRACH-FrequencyIndex and considered for MF-preamble format 0 is defined as a 10-bit bitmap corresponding to each interlace in increasing order, where the value of “1” indicates that the interlace is allocated for the first interlace of MF-sPRACH, and where the LSB of the bitmap indicates interlace 0. The interlace index allocated to the first interlace of MF-sPRACH considered is.

In case of two interlaces per MF-sPRACH opportunity as indicated by

,

the interlace index allocated to the second interlace of MF-sPRACH opportunity considered is

derived from

.

is the first physical resource block allocated to the MF-sPRACH opportunity considered.

Table 5.7.1-5: Frame structure type 3 random access configuration for MF-preamble formats 0

| MF-PRACH Configuration Index | MF-Preamble Format | System frame number | Subframe number | MF-PRACH Configuration Index | MF-Preamble Format | System frame number | Subframe number |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Even | 1 | 32 | N/A | N/A | N/A |
| 1 | 0 | Even | 4 | 33 | N/A | N/A | N/A |
| 2 | 0 | Even | 7 | 34 | N/A | N/A | N/A |
| 3 | 0 | Any | 1 | 35 | N/A | N/A | N/A |
| 4 | 0 | Any | 4 | 36 | N/A | N/A | N/A |
| 5 | 0 | Any | 7 | 37 | N/A | N/A | N/A |
| 6 | 0 | Any | 1, 6 | 38 | N/A | N/A | N/A |
| 7 | 0 | Any | 2 ,7 | 39 | N/A | N/A | N/A |
| 8 | 0 | Any | 3, 8 | 40 | N/A | N/A | N/A |
| 9 | 0 | Any | 1, 4, 7 | 41 | N/A | N/A | N/A |
| 10 | 0 | Any | 2, 5, 8 | 42 | N/A | N/A | N/A |
| 11 | 0 | Any | 3, 6, 9 | 43 | N/A | N/A | N/A |
| 12 | 0 | Any | 0, 2, 4, 6, 8 | 44 | N/A | N/A | N/A |
| 13 | 0 | Any | 1, 3, 5, 7, 9 | 45 | N/A | N/A | N/A |
| 14 | 0 | Any | 0, 1, 2, 3, 4, 5, 6, 7, 8, 9 | 46 | N/A | N/A | N/A |
| 15 | 0 | Even | 9 | 47 | N/A | N/A | N/A |
| 16 | N/A | N/A | N/A | 48 | N/A | N/A | N/A |
| 17 | N/A | N/A | N/A | 49 | N/A | N/A | N/A |
| 18 | N/A | N/A | N/A | 50 | N/A | N/A | N/A |
| 19 | N/A | N/A | N/A | 51 | N/A | N/A | N/A |
| 20 | N/A | N/A | N/A | 52 | N/A | N/A | N/A |
| 21 | N/A | N/A | N/A | 53 | N/A | N/A | N/A |
| 22 | N/A | N/A | N/A | 54 | N/A | N/A | N/A |
| 23 | N/A | N/A | N/A | 55 | N/A | N/A | N/A |
| 24 | N/A | N/A | N/A | 56 | N/A | N/A | N/A |
| 25 | N/A | N/A | N/A | 57 | N/A | N/A | N/A |
| 26 | N/A | N/A | N/A | 58 | N/A | N/A | N/A |
| 27 | N/A | N/A | N/A | 59 | N/A | N/A | N/A |
| 28 | N/A | N/A | N/A | 60 | N/A | N/A | N/A |
| 29 | N/A | N/A | N/A | 61 | N/A | N/A | N/A |
| 30 | N/A | N/A | N/A | 62 | N/A | N/A | N/A |
| 31 | N/A | N/A | N/A | 63 | N/A | N/A | N/A |

### 5.7.2	Preamble sequence generation

The random access preambles for an LTE cell are generated from Zadoff-Chu sequences with zero correlation zone, generated from one or several root Zadoff-Chu sequences. The random access preambles for the MF cell use the MF-sPUCCH format 0 sequences defined in Section 5.4.2MF1 for MF-preamble format 0. The network configures the set of preamble sequences the UE is allowed to use.

There are 64 preambles available in each cell. The set of 64 preamble sequences in an LTE cell is found by including first, in the order of increasing cyclic shift, all the available cyclic shifts of a root Zadoff-Chu sequence with the logical index RACH_ROOT_SEQUENCE, where RACH_ROOT_SEQUENCE is broadcasted as part of the System Information. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found.

The set of 64 preamble sequences in a MF cell is found by including first, in the order of increasing OCC index and then in the order of increasing cyclic shift, all the available cyclic shifts with OCC of the base sequence with the logical index RACH_ROOT_SEQUENCE on the first interlace of the MF-sPRACH opportunity considered, where the base sequence is defined by clause 5.5.1.1 with

and RACH_ROOT_SEQUENCE is broadcasted as part of the System Information for MF. Additional preamble sequences in a MF cell, in case 64 preambles cannot be generated from a single base sequence on the first interlace of the MF-sPRACH opportunity considered, are obtained first from the second interlace if configured and then the base sequences with the consecutive logical root sequence indexes until all the 64 sequences are found.

The logical root sequence order for an LTE cell is cyclic: the logical index 0 is consecutive to 837. The relation between a logical root sequence index and physical root sequence index  is given by Tables 5.7.2-4 and 5.7.2-5 for preamble formats 0 – 3 and 4, respectively.

The  root Zadoff-Chu sequence is defined by

where the length  of the Zadoff-Chu sequence is given by Table 5.7.2-1. From the  root Zadoff-Chu sequence, random access preambles with zero correlation zones of length are defined by cyclic shifts according to

where the cyclic shift is given by

and  is given by Tables 5.7.2-2 and 5.7.2-3 for preamble formats 0-3 and 4, respectively, where the parameter zeroCorrelationZoneConfig is provided by higher layers. The parameter High-speed-flag provided by higher layers determines if unrestricted set or restricted set shall be used.

The variable  is the cyclic shift corresponding to a Doppler shift of magnitude  and is given by

where  is the smallest non-negative integer that fulfils . The parameters for restricted sets of cyclic shifts depend on . For, the parameters are given by

For, the parameters are given by

For all other values of , there are no cyclic shifts in the restricted set.

The logical root sequence order for MF cell is cyclic: the logical index 0 is consecutive to 837. The relation between a logical root sequence index and physical root sequence index  is given by Tables 5.7.2-5A for MF-preamble format 0.

For MF-preamble format 0, from the  root Zadoff-Chu sequence, sequence group number  and the number  within the group for the base sequence

defined by clause 5.5.1.1 with

are given in Table 5.7.2-1B. Random access preambles for MF-preamble format 0 with zero correlation zones of length

and OCC are defined by clause 5.4.2MF.1 where

is given by Table 5.7.2-3A for MF-preamble formats 0 where the parameter zeroCorrelationZoneConfig is provided by higher layers.

Table 5.7.2-1: Random access preamble sequence length

| Preamble format |  |
| --- | --- |
| 0 – 3 | 839 |
| 4 | 139 |

Table 5.7.2-1B: Base sequence parameters for MF-preamble format 0

|  | 3 | 4 | 7 | 8 | 10 | 11 | 14 | 15 | 18 | 19 | 21 | 22 | 25 | 26 | 29 | 30 | 32 | 33 | 36 | 37 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 1 | 1 | 2 | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 |
|  | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |
|  | 40 | 41 | 43 | 44 | 47 | 48 | 51 | 52 | 54 | 55 | 58 | 59 | 61 | 62 | 65 | 66 | 69 | 70 | 72 | 73 |
|  | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 | 19 | 19 |
|  | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |
|  | 76 | 77 | 80 | 81 | 83 | 84 | 87 | 88 | 91 | 92 | 94 | 95 | 98 | 99 | 102 | 103 | 105 | 106 | 109 | 110 |
|  | 20 | 20 | 21 | 21 | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 26 | 27 | 27 | 28 | 28 | 29 | 29 |
|  | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |

Table 5.7.2-2:  for preamble generation (preamble formats 0-3)

| zeroCorrelationZoneConfig | value |  |
| --- | --- | --- |
|  | Unrestricted set | Restricted set |
| 0 | 0 | 15 |
| 1 | 13 | 18 |
| 2 | 15 | 22 |
| 3 | 18 | 26 |
| 4 | 22 | 32 |
| 5 | 26 | 38 |
| 6 | 32 | 46 |
| 7 | 38 | 55 |
| 8 | 46 | 68 |
| 9 | 59 | 82 |
| 10 | 76 | 100 |
| 11 | 93 | 128 |
| 12 | 119 | 158 |
| 13 | 167 | 202 |
| 14 | 279 | 237 |
| 15 | 419 | - |

Table 5.7.2-3:  for preamble generation (preamble format 4)

| zeroCorrelationZoneConfig | value |
| --- | --- |
| 0 | 2 |
| 1 | 4 |
| 2 | 6 |
| 3 | 8 |
| 4 | 10 |
| 5 | 12 |
| 6 | 15 |
| 7 | N/A |
| 8 | N/A |
| 9 | N/A |
| 10 | N/A |
| 11 | N/A |
| 12 | N/A |
| 13 | N/A |
| 14 | N/A |
| 15 | N/A |

Table 5.7.2-3A:  for preamble generation (MF-preamble format 0)

| zeroCorrelationZoneConfig | value |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 6 |
| 5 | 12 |
| 6 | N/A |
| 7 | N/A |
| 8 | N/A |
| 9 | N/A |
| 10 | N/A |
| 11 | N/A |
| 12 | N/A |
| 13 | N/A |
| 14 | N/A |
| 15 | N/A |

Table 5.7.2-4: Root Zadoff-Chu sequence order for preamble formats 0 – 3

| Logical root sequence number | Physical root sequence number (in increasing order of the corresponding logical sequence number) |
| --- | --- |
| 0–23 | 129, 710, 140, 699, 120, 719, 210, 629, 168, 671, 84, 755, 105, 734, 93, 746, 70, 769, 60, 779 2, 837, 1, 838 |
| 24–29 | 56, 783, 112, 727, 148, 691 |
| 30–35 | 80, 759, 42, 797, 40, 799 |
| 36–41 | 35, 804, 73, 766, 146, 693 |
| 42–51 | 31, 808, 28, 811, 30, 809, 27, 812, 29, 810 |
| 52–63 | 24, 815, 48, 791, 68, 771, 74, 765, 178, 661, 136, 703 |
| 64–75 | 86, 753, 78, 761, 43, 796, 39, 800, 20, 819, 21, 818 |
| 76–89 | 95, 744, 202, 637, 190, 649, 181, 658, 137, 702, 125, 714, 151, 688 |
| 90–115 | 217, 622, 128, 711, 142, 697, 122, 717, 203, 636, 118, 721, 110, 729, 89, 750, 103, 736, 61, 778, 55, 784, 15, 824, 14, 825 |
| 116–135 | 12, 827, 23, 816, 34, 805, 37, 802, 46, 793, 207, 632, 179, 660, 145, 694, 130, 709, 223, 616 |
| 136–167 | 228, 611, 227, 612, 132, 707, 133, 706, 143, 696, 135, 704, 161, 678, 201, 638, 173, 666, 106, 733, 83, 756, 91, 748, 66, 773, 53, 786, 10, 829, 9, 830 |
| 168–203 | 7, 832, 8, 831, 16, 823, 47, 792, 64, 775, 57, 782, 104, 735, 101, 738, 108, 731, 208, 631, 184, 655, 197, 642, 191, 648, 121, 718, 141, 698, 149, 690, 216, 623, 218, 621 |
| 204–263 | 152, 687, 144, 695, 134, 705, 138, 701, 199, 640, 162, 677, 176, 663, 119, 720, 158, 681, 164, 675, 174, 665, 171, 668, 170, 669, 87, 752, 169, 670, 88, 751, 107, 732, 81, 758, 82, 757, 100, 739, 98, 741, 71, 768, 59, 780, 65, 774, 50, 789, 49, 790, 26, 813, 17, 822, 13, 826, 6, 833 |
| 264–327 | 5, 834, 33, 806, 51, 788, 75, 764, 99, 740, 96, 743, 97, 742, 166, 673, 172, 667, 175, 664, 187, 652, 163, 676, 185, 654, 200, 639, 114, 725, 189, 650, 115, 724, 194, 645, 195, 644, 192, 647, 182, 657, 157, 682, 156, 683, 211, 628, 154, 685, 123, 716, 139, 700, 212, 627, 153, 686, 213, 626, 215, 624, 150, 689 |
| 328–383 | 225, 614, 224, 615, 221, 618, 220, 619, 127, 712, 147, 692, 124, 715, 193, 646, 205, 634, 206, 633, 116, 723, 160, 679, 186, 653, 167, 672, 79, 760, 85, 754, 77, 762, 92, 747, 58, 781, 62, 777, 69, 770, 54, 785, 36, 803, 32, 807, 25, 814, 18, 821, 11, 828, 4, 835 |
| 384–455 | 3, 836, 19, 820, 22, 817, 41, 798, 38, 801, 44, 795, 52, 787, 45, 794, 63, 776, 67, 772, 72 767, 76, 763, 94, 745, 102, 737, 90, 749, 109, 730, 165, 674, 111, 728, 209, 630, 204, 635, 117, 722, 188, 651, 159, 680, 198, 641, 113, 726, 183, 656, 180, 659, 177, 662, 196, 643, 155, 684, 214, 625, 126, 713, 131, 708, 219, 620, 222, 617, 226, 613 |
| 456–513 | 230, 609, 232, 607, 262, 577, 252, 587, 418, 421, 416, 423, 413, 426, 411, 428, 376, 463, 395, 444, 283, 556, 285, 554, 379, 460, 390, 449, 363, 476, 384, 455, 388, 451, 386, 453, 361, 478, 387, 452, 360, 479, 310, 529, 354, 485, 328, 511, 315, 524, 337, 502, 349, 490, 335, 504, 324, 515 |
| 514–561 | 323, 516, 320, 519, 334, 505, 359, 480, 295, 544, 385, 454, 292, 547, 291, 548, 381, 458, 399, 440, 380, 459, 397, 442, 369, 470, 377, 462, 410, 429, 407, 432, 281, 558, 414, 425, 247, 592, 277, 562, 271, 568, 272, 567, 264, 575, 259, 580 |
| 562–629 | 237, 602, 239, 600, 244, 595, 243, 596, 275, 564, 278, 561, 250, 589, 246, 593, 417, 422, 248, 591, 394, 445, 393, 446, 370, 469, 365, 474, 300, 539, 299, 540, 364, 475, 362, 477, 298, 541, 312, 527, 313, 526, 314, 525, 353, 486, 352, 487, 343, 496, 327, 512, 350, 489, 326, 513, 319, 520, 332, 507, 333, 506, 348, 491, 347, 492, 322, 517 |
| 630–659 | 330, 509, 338, 501, 341, 498, 340, 499, 342, 497, 301, 538, 366, 473, 401, 438, 371, 468, 408, 431, 375, 464, 249, 590, 269, 570, 238, 601, 234, 605 |
| 660–707 | 257, 582, 273, 566, 255, 584, 254, 585, 245, 594, 251, 588, 412, 427, 372, 467, 282, 557, 403, 436, 396, 443, 392, 447, 391, 448, 382, 457, 389, 450, 294, 545, 297, 542, 311, 528, 344, 495, 345, 494, 318, 521, 331, 508, 325, 514, 321, 518 |
| 708–729 | 346, 493, 339, 500, 351, 488, 306, 533, 289, 550, 400, 439, 378, 461, 374, 465, 415, 424, 270, 569, 241, 598 |
| 730–751 | 231, 608, 260, 579, 268, 571, 276, 563, 409, 430, 398, 441, 290, 549, 304, 535, 308, 531, 358, 481, 316, 523 |
| 752–765 | 293, 546, 288, 551, 284, 555, 368, 471, 253, 586, 256, 583, 263, 576 |
| 766–777 | 242, 597, 274, 565, 402, 437, 383, 456, 357, 482, 329, 510 |
| 778–789 | 317, 522, 307, 532, 286, 553, 287, 552, 266, 573, 261, 578 |
| 790–795 | 236, 603, 303, 536, 356, 483 |
| 796–803 | 355, 484, 405, 434, 404, 435, 406, 433 |
| 804–809 | 235, 604, 267, 572, 302, 537 |
| 810–815 | 309, 530, 265, 574, 233, 606 |
| 816–819 | 367, 472, 296, 543 |
| 820–837 | 336, 503, 305, 534, 373, 466, 280, 559, 279, 560, 419, 420, 240, 599, 258, 581, 229, 610 |

Table 5.7.2-5: Root Zadoff-Chu sequence order for preamble format 4

| Logical root sequence number | Physical root sequence number (in increasing order of the corresponding logical sequence number) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 19 | 1 | 138 | 2 | 137 | 3 | 136 | 4 | 135 | 5 | 134 | 6 | 133 | 7 | 132 | 8 | 131 | 9 | 130 | 10 | 129 |
| 20 – 39 | 11 | 128 | 12 | 127 | 13 | 126 | 14 | 125 | 15 | 124 | 16 | 123 | 17 | 122 | 18 | 121 | 19 | 120 | 20 | 119 |
| 40 – 59 | 21 | 118 | 22 | 117 | 23 | 116 | 24 | 115 | 25 | 114 | 26 | 113 | 27 | 112 | 28 | 111 | 29 | 110 | 30 | 109 |
| 60 – 79 | 31 | 108 | 32 | 107 | 33 | 106 | 34 | 105 | 35 | 104 | 36 | 103 | 37 | 102 | 38 | 101 | 39 | 100 | 40 | 99 |
| 80 – 99 | 41 | 98 | 42 | 97 | 43 | 96 | 44 | 95 | 45 | 94 | 46 | 93 | 47 | 92 | 48 | 91 | 49 | 90 | 50 | 89 |
| 100 – 119 | 51 | 88 | 52 | 87 | 53 | 86 | 54 | 85 | 55 | 84 | 56 | 83 | 57 | 82 | 58 | 81 | 59 | 80 | 60 | 79 |
| 120 – 137 | 61 | 78 | 62 | 77 | 63 | 76 | 64 | 75 | 65 | 74 | 66 | 73 | 67 | 72 | 68 | 71 | 69 | 70 | - | - |
| 138 – 837 | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.7.2-5A: Root Zadoff-Chu sequence order for MF-preamble format 0

| Logical root sequence number | Physical root sequence number (in increasing order of the corresponding logical sequence number) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 19 | 8 | 88 | 62 | 103 | 84 | 51 | 70 | 105 | 41 | 43 | 33 | 29 | 4 | 80 | 83 | 72 | 15 | 30 | 91 | 109 |
| 20 – 39 | 44 | 22 | 94 | 98 | 25 | 19 | 10 | 69 | 36 | 32 | 48 | 58 | 55 | 65 | 81 | 77 | 106 | 26 | 95 | 76 |
| 40 – 59 | 37 | 18 | 87 | 7 | 61 | 21 | 47 | 11 | 102 | 66 | 92 | 52 | 14 | 59 | 110 | 73 | 40 | 3 | 54 | 99 |
| 60 – 79 | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 80 – 99 | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100 – 119 | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 120 – 137 | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 138 – 837 | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.7.3	Baseband signal generation

The time-continuous random access signal  is defined by

where,  is an amplitude scaling factor in order to conform to the transmit power  specified in clause 6.1 in MFA TS 36.213 [4], and . The location in the frequency domain is controlled by the parameter  is derived from clause 5.7.1. The factor  accounts for the difference in subcarrier spacing between the random access preamble and uplink data transmission. The variable, the subcarrier spacing for the random access preamble, and the variable, a fixed offset determining the frequency-domain location of the random access preamble within the physical resource blocks, are both given by Table 5.7.3-1.

Table 5.7.3-1: Random access baseband parameters

| Preamble format |  |  |
| --- | --- | --- |
| 0 – 3 | 1250 Hz | 7 |
| 4 | 7500 Hz | 2 |

The time-continuous random access signal  for MF-preamble format 0 is defined by

where

is an amplitude scaling factor in order to conform to the transmit power

specified in Section 6.1 in MFA TS 36.213 [4],

,

and

.

## 5.8	Modulation and upconversion

Modulation and upconversion to the carrier frequency of the complex-valued SC-FDMA baseband signal for each antenna port or the complex-valued PRACH baseband signal is shown in Figure 5.8-1. The filtering required prior to transmission is defined by the requirements in MFA TS 36.101 [7].

Figure 5.8-1: Uplink modulation

# 6	Downlink

## 6.1	Overview

The smallest time-frequency unit for downlink transmission is denoted a resource element and is defined in clause 6.2.2.

A subset of the downlink subframes in a radio frame on a carrier supporting PDSCH transmission can be configured as MBSFN subframes by higher layers. Each MBSFN subframe is divided into a non-MBSFN region and an MBSFN region.

-	The non-MBSFN region spans the first one or two OFDM symbols in an MBSFN subframe where the length of the non-MBSFN region is given according to Subclause 6.7.

-	The MBSFN region in an MBSFN subframe is defined as the OFDM symbols not used for the non-MBSFN region.

For frame structure type 3, MBSFN configuration shall not be applied to downlink subframes in which at least one OFDM symbol is not occupied or discovery signal is transmitted.

Unless otherwise specified, transmission in each downlink subframe shall use the same cyclic prefix length as used for downlink subframe #0.

### 6.1.1	Physical channels

A downlink physical channel corresponds to a set of resource elements carrying information originating from higher layers and is the interface defined between MFA TS 36.212 [3] and this present document MFA TS 36.211. 
The following downlink physical channels are defined:

-	Physical Downlink Shared Channel, PDSCH

-	Physical Broadcast Channel, PBCH

-	Physical Multicast Channel, PMCH

-	Physical Control Format Indicator Channel, PCFICH

-	Physical Downlink Control Channel, PDCCH

-	Physical Hybrid ARQ Indicator Channel, PHICH

-	Enhanced Physical Downlink Control Channel, EPDCCH

-	MTC Physical Downlink Control Channel, MPDCCH

-    MF Physical Broadcast Channel, MF-PBCH

### 6.1.2	Physical signals

A downlink physical signal corresponds to a set of resource elements used by the physical layer but does not carry information originating from higher layers. The following downlink physical signals are defined:

-	Reference signal

-	Synchronization signal

-	Discovery signal

## 6.2	Slot structure and physical resource elements

### 6.2.1	Resource grid

The transmitted signal in each slot is described by one or several resource grids of  subcarriers and  OFDM symbols. The resource grid structure is illustrated in Figure 6.2.2-1. The quantity  depends on the downlink transmission bandwidth configured in the cell and shall fulfil

where  and  are the smallest and largest downlink bandwidths, respectively, supported by the current version of this specification.

The set of allowed values for  is given by MFA TS 36.104 [6].  The number of OFDM symbols in a slot depends on the cyclic prefix length and subcarrier spacing configured and is given in Table 6.2.3-1.

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. For MBSFN reference signals, positioning reference signals, UE-specific reference signals associated with PDSCH and demodulation reference signals associated with EPDCCH, there are limits given below within which the channel can be inferred from one symbol to another symbol on the same antenna port. There is one resource grid per antenna port. The set of antenna ports supported depends on the reference signal configuration in the cell:

-	Cell-specific reference signals support a configuration of one, two, or four antenna ports and are transmitted on antenna ports ,, and , respectively.

-	MBSFN reference signals are transmitted on antenna port. The channel over which a symbol on antenna portis conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols correspond to subframes of the same MBSFN area.

-	UE-specific reference signals associated with PDSCH intended for non-BL/CE UE are transmitted on antenna port(s) , , ,  or one or several of . The channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are within the same subframe and in the same PRG when PRB bundling is used or in the same PRB pair when PRB bundling is not used.

-	-	UE-specific reference signals associated with PDSCH intended for BL/CE UE are transmitted on one or several of antenna port(s) . The channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are in the same set of consecutive subframes and have the same PRB index.

-	Demodulation reference signals associated with EPDCCH are transmitted on one or several of . The channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are in the same PRB pair.

-	-	Demodulation reference signals associated with MPDCCH are transmitted on one or several of . The channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are in the same set of consecutive subframes and have the same PRB index.

-	Positioning reference signals are transmitted on antenna port. The channel over which a symbol on antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only within one positioning reference signal occasion consisting of  consecutive downlink subframes, where  is configured by higher layers.

-	CSI reference signals support a configuration of 1, 2, 4, 8, 12, 16, 20, 24, 28, or 32 antenna ports and are transmitted on antenna ports , , , , , , , ,  and, respectively.

Two antenna ports are said to be quasi co-located if the large-scale properties of the channel over which a symbol on one antenna port is conveyed can be inferred from the channel over which a symbol on the other antenna port is conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, and average delay.

### 6.2.2	Resource elements

Each element in the resource grid for antenna port  is called a resource element and is uniquely identified by the index pair  in a slot where  and  are the indices in the frequency and time domains, respectively. Resource element  on antenna port  corresponds to the complex value. 
When there is no risk for confusion, or no particular antenna port is specified, the index  may be dropped.

Figure 6.2.2-1: Downlink resource grid

### 6.2.3	Resource blocks

Resource blocks are used to describe the mapping of certain physical channels to resource elements. Physical and virtual resource blocks are defined.

A physical resource block is defined as  consecutive OFDM symbols in the time domain and consecutive subcarriers in the frequency domain, where  and  are given by Table 6.2.3-1. A physical resource block thus consists of  resource elements, corresponding to one slot in the time domain and 180 kHz in the frequency domain.

Physical resource blocks are numbered from 0 to  in the frequency domain. The relation between the physical resource block number  in the frequency domain and resource elements  in a slot is given by

Table 6.2.3-1: Physical resource blocks parameters

| Configuration |  |  |  |
| --- | --- | --- | --- |
| Normal cyclic prefix |  | 12 | 7 |
| Extended cyclic prefix |  |  | 6 |
|  |  | 24 | 3 |

A physical resource-block pair is defined as the two physical resource blocks in one subframe having the same physical resource-block number .

A virtual resource block is of the same size as a physical resource block. Two types of virtual resource blocks are defined:

-	Virtual resource blocks of localized type

-	Virtual resource blocks of distributed type

For each type of virtual resource blocks, a pair of virtual resource blocks over two slots in a subframe is assigned together by a single virtual resource block number, .

#### 6.2.3.1	Virtual resource blocks of localized type

Virtual resource blocks of localized type are mapped directly to physical resource blocks such that virtual resource block  corresponds to physical resource block. Virtual resource blocks are numbered from 0 to, where .

#### 6.2.3.2	Virtual resource blocks of distributed type

Virtual resource blocks of distributed type are mapped to physical resource blocks as described below.

Table 6.2.3.2-1: RB gap values

| System BW () | Gap () |  |
| --- | --- | --- |
|  | 1st Gap () | 2nd Gap () |
| 6-10 |  | N/A |
| 11 | 4 | N/A |
| 12-19 | 8 | N/A |
| 20-26 | 12 | N/A |
| 27-44 | 18 | N/A |
| 45-49 | 27 | N/A |
| 50-63 | 27 | 9 |
| 64-79 | 32 | 16 |
| 80-110 | 48 | 16 |

The parameter  is given by Table 6.2.3.2-1. For , only one gap value  is defined and . For , two gap values  and  are defined. Whether  or is signaled as part of the downlink scheduling assignment as described in MFA TS 36.212 [3].

Virtual resource blocks of distributed type are numbered from 0 to, where  for  and  for .

Consecutive  VRB numbers compose a unit of VRB number interleaving, where  for  and  for . Interleaving of VRB numbers of each interleaving unit is performed with 4 columns and  rows, where , and  is RBG size as described in MFA TS 36.213 [4]. VRB numbers are written row by row in the rectangular matrix, and read out column by column.  nulls are inserted in the last  rows of the 2nd and 4th column, where . Nulls are ignored when reading out. The VRB numbers mapping to PRB numbers including interleaving is derived as follows:

For even slot number ;

,

where ,

and ,

where   and  is obtained from the downlink scheduling assignment as described in MFA TS 36.213 [4].

For odd slot number ;

Then, for all ;

.

Virtual resource blocks of distributed type are not applicable to BL/CE UEs.

### 6.2.4	Resource-element groups

Resource-element groups are used for defining the mapping of control channels to resource elements.

A resource-element group is represented by the index pair  of the resource element with the lowest index  in the group with all resource elements in the group having the same value of . The set of resource elements  in a resource-element group depends on the number of cell-specific reference signals configured as described below with , .

-	In the first OFDM symbol of the first slot in a subframe the two resource-element groups in physical resource block  consist of resource elements  with   and , respectively.

-	In the second OFDM symbol of the first slot in a subframe in case of one or two cell-specific reference signals configured, the three resource-element groups in physical resource block  consist of resource elements  with  ,  and , respectively.

-	In the second OFDM symbol of the first slot in a subframe in case of four cell-specific reference signals configured, the two resource-element groups in physical resource block  consist of resource elements  with  and , respectively.

-	In the third OFDM symbol of the first slot in a subframe, the three resource-element groups in physical resource block  consist of resource elements  with  ,  and , respectively.

-	In the fourth OFDM symbol of the first slot in a subframe in case of normal cyclic prefix, the three resource-element groups in physical resource block  consist of resource elements  with  ,  and , respectively.

-	In the fourth OFDM symbol of the first slot in a subframe in case of extended cyclic prefix, the two resource-element groups in physical resource block  consist of resource elements  with    and , respectively.

Mapping of a symbol-quadruplet  onto a resource-element group represented by resource-element  is defined such that elements  are mapped to resource elements  of the resource-element group not used for cell-specific reference signals in increasing order of   and . In case a single cell-specific reference signal is configured, cell-specific reference signals shall be assumed to be present on antenna ports 0 and 1 for the purpose of mapping a symbol-quadruplet to a resource-element group, otherwise the number of cell-specific reference signals shall be assumed equal to the actual number of antenna ports used for cell-specific reference signals. The UE shall not make any assumptions about resource elements assumed to be reserved for reference signals but not used for transmission of a reference signal.

For frame structure type 3, if the higher layer parameter subframeStartPosition indicates ‘s07’ and the downlink transmission starts in the second slot of a subframe, the above definition applies to the second slot of that subframe instead of the first slot.

### 6.2.4A	Enhanced Resource-Element Groups (EREGs)

EREGs are used for defining the mapping of enhanced control channels to resource elements.

There are 16 EREGs, numbered from 0 to 15, per physical resource block pair. Number all resource elements, except resource elements carrying DM-RS for antenna ports  for normal cyclic prefix or  for extended cyclic prefix, in a physical resource-block pair cyclically from 0 to 15 in an increasing order of first frequency, then time. All resource elements with number  in that physical resource-block pair constitutes EREG number .

For frame structure type 3, if the higher layer parameter subframeStartPosition indicates ‘s07’ and the downlink transmission starts in the second slot of a subframe, the above definition applies to the second slot of that subframe instead of the first slot.

### 6.2.5	Guard period for half-duplex FDD operation

For type A half-duplex FDD operation, a guard period is created by the UE by

-	not receiving the last part of a downlink subframe immediately preceding an uplink subframe from the same UE.

For type B half-duplex FDD operation, guard periods, each referred to as a half-duplex guard subframe, are created by the UE by

-	not receiving a downlink subframe immediately preceding an uplink subframe from the same UE, and

-	not receiving a downlink subframe immediately following an uplink subframe from the same UE.

### 6.2.6	Guard Period for TDD Operation

For frame structure type 2, the GP field in Figure 4.2-1 serves as a guard period.

### 6.2.7	Narrowbands

A narrowband is defined as six non-overlapping consecutive physical resource blocks in the frequency domain. The total number of downlink narrowbands in the downlink transmission bandwidth configured in the cell is given by

The narrowbands are numbered   in order of increasing physical resource-block number where narrowband is composed of physical resource-block indices

where

### 6.2.8	Guard period for narrowband retuning

For BL/CE UEs, a guard period is created by the UE not receiving at most the first two OFDM symbols in the second narrowband when

-	the UE retunes from a first downlink narrowband to a second downlink narrowband with a different center frequency, or

-	the UE retunes from a first uplink narrowband to a second downlink narrowband with a different center frequency for frame structure type 2.

## 6.3	General structure for downlink physical channels

This clause describes a general structure, applicable to more than one physical channel.

The baseband signal representing a downlink physical channel is defined in terms of the following steps:

-	scrambling of coded bits in each of the codewords to be transmitted on a physical channel

-	modulation of scrambled bits to generate complex-valued modulation symbols

-	mapping of the complex-valued modulation symbols onto one or several transmission layers

-	precoding of the complex-valued modulation symbols on each layer for transmission on the antenna ports

-	mapping of complex-valued modulation symbols for each antenna port to resource elements

-	generation of complex-valued time-domain OFDM signal for each antenna port

Figure 6.3-1: Overview of physical channel processing

### 6.3.1	Scrambling

For each codeword , the block of bits , where  is the number of bits in codeword  transmitted on the physical channel in one subframe, shall be scrambled prior to modulation, resulting in a block of scrambled bits according to

where the scrambling sequence  is given by clause 7.2. For an LTE cell, the scrambling sequence generator shall be initialised at the start of each subframe, where the initialisation value of  depends on the transport channel type according to

For a MF cell:

Where

and

corresponds to the RNTI associated with the PDSCH transmission as described in clause 7.1 MFA TS 36.213 [4].

For BL/CE UEs, the same scrambling sequence is applied per subframe to PDSCH for a given block of  subframes. For the  block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for PDSCH. The PDSCH transmission spans  consecutive subframes including non-BL/CE UL subframes where the PDSCH transmission is postponed.

For BL/CE UEs,

-	if the PDSCH is carrying SIB1-BR

-

-	else if the PDSCH is carrying SI message (except for SIB1-BR) or if the PDSCH transmission is associated with P-RNTI:

-	 for frame structure type 1 and  for frame structure type 2

-	otherwise

-	 for UEs assuming CEModeA (according to the definition in section 12 of MFA TS 36.213 [4]) or configured with CEModeA

-	 for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in section 12 of MFA TS 36.213 [4]) or configured with CEModeB

Up to two codewords can be transmitted in one subframe, i.e., . In the case of single codeword transmission,  is equal to zero.

### 6.3.2	Modulation

For each codeword , the block of scrambled bits shall be modulated as described in clause 7.1 using one of the modulation schemes in Table 6.3.2-1, resulting in a block of complex-valued modulation symbols .

Table 6.3.2-1: Modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PDSCH | QPSK, 16QAM, 64QAM, 256QAM |
| PMCH | QPSK, 16QAM, 64QAM, 256QAM |

### 6.3.3	Layer mapping

The complex-valued modulation symbols for each of the codewords to be transmitted are mapped onto one or several layers. Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is the number of layers and  is the number of modulation symbols per layer.

#### 6.3.3.1	Layer mapping for transmission on a single antenna port

For transmission on a single antenna port, a single layer is used, , and the mapping is defined by

with .

#### 6.3.3.2	Layer mapping for spatial multiplexing

For spatial multiplexing, the layer mapping shall be done according to Table 6.3.3.2-1. The number of layers  is less than or equal to the number of antenna ports  used for transmission of the physical channel. The case of a single codeword mapped to multiple layers is only applicable when the number of cell-specific reference signals is four or when the number of UE-specific reference signals is two or larger.

Table 6.3.3.2-1: Codeword-to-layer mapping for spatial multiplexing

| Number of layers | Number of codewords | Codeword-to-layer mapping |  |
| --- | --- | --- | --- |
| 1 | 1 |  |  |
| 2 | 1 |  |  |
| 2 | 2 |  |  |
| 3 | 1 |  |  |
| 3 | 2 |  |  |
| 4 | 1 |  |  |
| 4 | 2 |  |  |
| 5 | 2 |  |  |
| 6 | 2 |  |  |
| 7 | 2 |  |  |
| 8 | 2 |  |  |

#### 6.3.3.3	Layer mapping for transmit diversity

For transmit diversity, the layer mapping shall be done according to Table 6.3.3.3-1. There is only one codeword and the number of layers  is equal to the number of antenna ports  used for transmission of the physical channel.

Table 6.3.3.3-1: Codeword-to-layer mapping for transmit diversity

| Number of layers | Number of codewords | Codeword-to-layer mapping |  |
| --- | --- | --- | --- |
| 2 | 1 |  |  |
| 4 | 1 |  | If two null symbols shall be appended to |

### 6.3.4	Precoding

The precoder takes as input a block of vectors,  from the layer mapping and generates a block of vectors,  to be mapped onto resources on each of the antenna ports, where  represents the signal for antenna port.

#### 6.3.4.1	Precoding for transmission on a single antenna port

For transmission on a single antenna port, precoding is defined by

where  is the number of the single antenna port used for transmission of the physical channel and , .

#### 6.3.4.2	Precoding for spatial multiplexing using antenna ports with cell-specific reference signals

Precoding for spatial multiplexing using antenna ports with cell-specific reference signals is only used in combination with layer mapping for spatial multiplexing as described in clause 6.3.3.2. Spatial multiplexing supports two or four antenna ports and the set of antenna ports used is or, respectively.

##### 6.3.4.2.1	Precoding without CDD

Without Cyclic Delay Diversity (CDD), precoding for spatial multiplexing is defined by

where the precoding matrix  is of size  and , .

For spatial multiplexing, the values of   shall be selected among the precoder elements in the codebook configured in the eNodeB and the UE. The eNodeB can further confine the precoder selection in the UE to a subset of the elements in the codebook using codebook subset restrictions. The configured codebook shall be selected from Table 6.3.4.2.3-1 or 6.3.4.2.3-2.

##### 6.3.4.2.2	Precoding for large delay CDD

For large-delay CDD, precoding for spatial multiplexing is defined by

where the precoding matrixis of size  and , . The diagonal size-matrix  supporting cyclic delay diversity and the size- matrix  are both given by Table 6.3.4.2.2-1 for different numbers of layers .

The values of the precoding matrix  shall be selected among the precoder elements in the codebook configured in the eNodeB and the UE. The eNodeB can further confine the precoder selection in the UE to a subset of the elements in the codebook using codebook subset restriction. The configured codebook shall be selected from Table 6.3.4.2.3-1 or 6.3.4.2.3-2.

For 2 antenna ports, the precoder is selected according to  where  denotes the precoding matrix corresponding to precoder index 0 in Table 6.3.4.2.3-1.

For 4 antenna ports, the UE may assume that the eNodeB cyclically assigns different precoders to different vectors on the physical downlink shared channel as follows.  A different precoder is used every vectors, where  denotes the number of transmission layers in the case of spatial multiplexing. In particular, the precoder  is selected according to , where  is the precoder index given by  and  denote precoder matrices corresponding to precoder indices 12,13,14 and 15, respectively, in Table 6.3.4.2.3-2.

Table 6.3.4.2.2-1: Large-delay cyclic delay diversity

| Number of layers |  |  |
| --- | --- | --- |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

##### 6.3.4.2.3	Codebook for precoding and CSI reporting

For transmission on two antenna ports, , and for the purpose of CSI reporting based on two antenna ports  or , the precoding matrix  shall be selected from Table 6.3.4.2.3-1 or a subset thereof. For the closed-loop spatial multiplexing transmission mode defined in MFA TS 36.213 [4], the codebook index 0 is not used when the number of layers is .

Table 6.3.4.2.3-1: Codebook for transmission on antenna ports  and for CSI reporting based on antenna ports  or

| Codebook index | Number of layers |  |
| --- | --- | --- |
|  | 1 | 2 |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  | - |

For transmission on four antenna ports, , the precoding matrix  shall be selected from Table 6.3.4.2.3-2 or a subset thereof. For the purpose of CSI reporting based on four antenna ports  or , the precoding matrix  shall be selected from Table 6.3.4.2.3-2 or a subset thereof except for alternativeCodeBookEnabledFor4TX -r12 =TRUE  in which case the precoding matrix  shall be selected from Tables 7.2.4-0A, 7.2.4-0B, 7.2.4-0C, 7.2.4-0D in MFA TS 36.213 [4] or a subset thereof, and except for advancedCodebookEnabled = TRUE in which case the precoding matrix  shall be selected from Table XX in [4] or a subset thereof. The quantity  denotes the matrix defined by the columns given by the set  from the expression  where  is the  identity matrix and the vector  is given by Table 6.3.4.2.3-2.

Table 6.3.4.2.3-2: Codebook for transmission on antenna ports and for CSI reporting based on antenna ports  or

| Codebook index |  | Number of layers |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 |
| 0 |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |

For the purpose of CSI reporting for 8, 12, 16, 20, 24, 28, and 32 CSI reference signals the codebooks are given in clause 7.2.4 of MFA TS 36.213 [4].

#### 6.3.4.3	Precoding for transmit diversity

Precoding for transmit diversity is only used in combination with layer mapping for transmit diversity as described in clause 6.3.3.3. The precoding operation for transmit diversity is defined for two and four antenna ports.

For transmission on two antenna ports, , the output ,  of the precoding operation is defined by

for with .

If the higher-layer parameter semiOpenLoop is set, for rank=1 transmission on two antenna ports, , the output ,  of the precoding operation is defined by

where .

For transmission on four antenna ports,, the output ,  of the precoding operation is defined by

for  with .

#### 6.3.4.4	Precoding for spatial multiplexing using antenna ports with UE-specific reference signals

Precoding for spatial multiplexing using antenna ports with UE-specific reference signals is only used in combination with layer mapping for spatial multiplexing as described in clause 6.3.3.2. Spatial multiplexing using antenna ports with UE-specific reference signals supports up to eight antenna ports.

If the higher-layer parameter dmrs-tableAlt is set to 1 and the set of antenna ports  is used for two layers transmission, the precoding operation for transmission on the two antenna ports is defined by

where , . Otherwise, the set of antenna ports used is and  the precoding operation for transmission on  antenna ports is defined by

where , .

If the higher-layer parameter semiOpenLoop is set to 1 and the set of antenna ports  is used for rank=2 transmission, the precoding operation for transmission on the two antenna ports is defined by

where  and .

### 6.3.5	Mapping to resource elements

For each of the antenna ports used for transmission of the physical channel, the block of complex-valued symbols  shall conform to the downlink power allocation specified in clause 5.2 in MFA TS 36.213 [4] and be mapped in sequence starting with  to resource elements  which meet all of the following criteria in the current subframe:

-	they are in the physical resource blocks corresponding to the virtual resource blocks assigned for transmission, and

-	they are not used for transmission of the core part of PBCH or MF-PBCH, synchronization signals, and

-	they are assumed by the UE not to be used for cell-specific reference signals, where the positions of the cell-specific reference signals are given by clause 6.10.1.2 with the number of antenna ports for and the frequency shift of cell-specific reference signals derived as described in clause 6.10.1.2 4, and

The mapping to resource elements  on antenna port  not reserved for other purposes shall be in increasing order of first the index  over the assigned physical resource blocks and then the index, starting with the first slot in a subframe.

## 6.4	Physical downlink shared channel

The physical downlink shared channel shall be processed and mapped to resource elements as described in clause 6.3 with the following additions and exceptions:

-	In resource blocks in which UE-specific reference signals are not transmitted, the PDSCH shall be transmitted on the same set of antenna ports as the PBCH or MF-PBCH, which is one of , , or .

-	In resource blocks in which UE-specific reference signals are transmitted, the PDSCH shall be transmitted on antenna port(s) ,,, , , or , where  is the number of layers used for transmission of the PDSCH.

-	If PDSCH is transmitted in MBSFN subframes as defined in MFA TS 36.213 [4], the PDSCH shall be transmitted on one or several of antenna port(s) , where  is the number of layers used for transmission of the PDSCH.

-	PDSCH is not mapped to resource elements used for UE-specific reference signals associated with PDSCH

-	In mapping to resource elements, the positions of the cell-specific reference signals are given by clause 6.10.1.2 with the number of antenna ports and the frequency shift of the cell-specific reference signals derived as described in clause 6.10.1.2, unless other values for these parameters are provided by clause 7.1.9 in MFA TS 36.213 [4], in which case these values are used in the resource blocks indicated by the relevant DCI.

-	If the DCI associated with the PDSCH uses the C-RNTI or semi-persistent C-RNTI, the PDSCH is not mapped to resource elements assumed by the UE to be used for transmission of:

-	zero-power CSI reference signals, where the positions of the CSI reference signals are given by clause 6.10.5.2.The configuration for zero power CSI reference signals is

-	obtained as described in clause 6.10.5.2, unless other values for these parameters are provided by clause 7.1.9 in MFA TS 36.213 [4], in which case these values are used in the resource blocks indicated by the relevant DCI, and

-	obtained by higher-layer configuration of up to five reserved CSI-RS resources as part of the discovery signal configuration following the procedure for zero-power CSI-RS in clause 6.10.5.2.

-	non-zero-power CSI reference signals for CSI reporting, where the positions of the non-zero-power CSI reference signals for CSI reporting are given by clause 6.10.5.2. The configuration for non-zero power CSI reference signals is obtained as described in clause 6.10.5.2.

-	PDSCH is not mapped to any physical resource-block pair(s) carrying an EPDCCH associated with the PDSCH.

-	PDSCH on antenna port 5, 7, 8, 9, 10, 11, 12, 13 or 14 is not mapped to any physical resource-block pair(s) carrying PBCH or MF-PBCH or synchronization signals.

-	For frame structure type 1 and 2, the index  in the first slot in a subframe fulfils  where  is given by clause 7.1.6.4 of MFA TS 36.213 [4].

-	For frame structure type 3,

-	if the higher layer parameter subframeStartPosition indicates ‘s07’ and the downlink transmission starts in the second slot of a subframe

-	the index  in the second slot in a subframe fulfils  where  is given by clause 7.1.6.4 of MFA TS 36.213  [4],

-	otherwise

-	the index  in the first slot in a subframe fulfils  where  is given by clause 7.6.1.4 of MFA TS 36.213  [4],

-	In mapping to resource elements, if the DCI associated with the PDSCH uses the C-RNTI or semi-persistent C-RNTI and transmit diversity according to clause 6.3.4.3 is used and if the higher-layer parameter semiOpenLoop is not set, resource elements in an OFDM symbol assumed by the UE to contain CSI-RS shall be used in the mapping if and only if all of the following criteria are fulfilled:

-	there is an even number of resource elements for the OFDM symbol in each resource block assigned for transmission, and

-	the complex-valued symbols  and , where  is an even number, can be mapped to resource elements  and  in the same OFDM symbol with .

-	In mapping to resource elements, if the DCI associated with the PDSCH uses C-RNTI or semi-persistent C-RNTI and if the higher-layer parameter semiOpenLoop is set, a pair of resource elements ,  shall be used in the mapping if and only if

-	the complex-valued symbols  and  can be mapped to resource elements  and  in the same OFDM symbol and the same PRB with , where  is an even number and  starts from 0 at the lowest subcarrier of the PRB.

-	For MF,

- PDSCH transmitted in a subframe containing DRS can have

or

in the second slot depending on the subframe duration and the subframe duration is determined by channel access rules defined in MFA TS 36.213 [4].

### 6.4.1	Physical downlink shared channel for BL/CE UEs

For BL/CE UEs, the following additions and exceptions hold in addition to those in clause 6.4:

-	Resource elements occupied by CSI reference signals shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

-	Resource elements belonging to synchronization signals, the core part of PBCH or PBCH repetitions shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

- For BL/CE UEs in CEModeB configured in transmission mode 9, in MBSFN subframe(s), resource elements that correspond to the positions of cell-specific reference signals as in subframe #0 shall not be counted in the PDSCH mapping and not used for transmission of the PDSCH.

For BL/CE UEs, if the PDSCH is not carrying SIB1-BR the PRB resources within the narrowband  for PDSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.12, 5.3.3.1.13, and 5.5.1.3.14 in MFA TS 36.212 [3], or provided by higher layers. The PDSCH is transmitted with  repetitions, spanning  consecutive subframes, including non-BL/CE UL subframes where the the PDSCH transmission is postponed.

-	If frequency hopping is not enabled for PDSCH, the PDSCH repetitions are located at the same PRB resources in the same narrowband , and

-	if frequency hopping is enabled for PDSCH, the PDSCH shall be transmitted in subframe  within the   consecutive downlink subframes using the same PRB resources within each narrowband

where  is the absolute subframe number of the first downlink subframe intended for PDSCH and ,  and  are cell-specific higher-layer parameters. For PDSCH carrying SI other than SIB1-BR and for PDSCH associated with P-RNTI, if interval-DlHoppingConfigCommonModeB is signalled in SIB1-BR, then the frequency hopping granularity  is set to interval-DlHoppingConfigCommonModeB; otherwise,  is set to interval-DlHoppingConfigCommonModeA signalled in SIB1-BR.

For BL/CE UE in CEModeA, frequency hopping of PDSCH associated with C-RNTI or SPS-RNTI is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig is set and the frequency hopping flag in DCI format 6-1A indicates frequency hopping, otherwise, frequency hopping of is not enabled.

The UE shall not expect PDSCH in subframe  if it is not a BL/CE UL subframe.

For BL/CE UEs, if the PDSCH carries SIB1-BR, the PDSCH transmission is repeated periodically in every period of 8 radio frames, where a period starts with a radio frame with  where  is the system frame number. The PDSCH is transmitted  times in each period of 8 frames, Let  be the set of narrowbands, excluding narrowbands overlapping with the 72 center subcarriers for , and ordered in increasing order of narrowband index. The PDSCH transmission cycles through the set  of narrowbands in increasing order of , starting with  for the first subframe, according to

where  is the number of narrowbands in the set .

The set of frames and subframes used for SIB1-BR transmission in each period are given by Tables 6.4.1-1 and 6.4.1-2.

Table 6.4.1-1: The set of frames and subframes for SIB1-BR for .

|  |  | Frame structure type 1 |  | Frame structure type 2 |  |
| --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 4 | 1 | 5 |
|  | 1 | 1 | 4 | 1 | 5 |

Table 6.4.1-2: The set of frames and subframes for SIB1-BR for .

|  |  | Frame structure type 1 |  | Frame structure type 2 |  |
| --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 4 | 1 | 5 |
|  | 1 | 1 | 4 | 1 | 0 |
| 8 | 0 | 0, 1 | 4 | 0, 1 | 5 |
|  | 1 | 0, 1 | 9 | 0, 1 | 0 |
| 16 | 0 | 0, 1 | 4, 9 | 0, 1 | 0, 5 |
|  | 1 | 0, 1 | 0, 9 | 0, 1 | 0, 5 |

BL/CE UEs may assume the same precoding matrix being used for a PRB across a block of   consecutive subframes  when UE-specific reference signals are transmitted together with the PDSCH.

For PDSCH transmission associated with SI-RNTI or P-RNTI to BL/CE UEs, frequency hopping of the PDSCH is enabled when higher layer parameter si-HoppingConfigCommon is set.

For PDSCH transmission associated with RA-RNTI or temporary C-RNTI to BL/CE UEs, frequency hopping of the PDSCH is enabled when higher layer parameter rar-HoppingConfig is set. Further

-	if PRACH CE level 0 or 1  is used for the last PRACH attempt,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeA;

-	if PRACH CE level 2 or 3 is used for the last PRACH attempt,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeB.

## 6.5	Physical multicast channel

The physical multicast channel shall be processed and mapped to resource elements as described in clause 6.3 with the following exceptions:

-	No transmit diversity scheme is specified.

-	Layer mapping and precoding shall be done assuming a single antenna port and the transmission shall use antenna port 4.

-	The PMCH can only be transmitted in the MBSFN region of an MBSFN subframe. The index  in the first slot in the MBSFN subframe fulfils  where  is equal to the value given by the higher layer parameter non-MBSFNregionLength (see MFA TS 36.331 [9]).

-	The PMCH shall use extended cyclic prefix.

-	The PMCH is not mapped to resource elements used for transmission of MBSFN reference signals.

## 6.6	Physical broadcast channel

The PBCH is not transmitted for frame structure type 3.

The MF-PBCH is transmitted on a subframe containing DRS. The MF-PBCH is transmitted on subframe 0 whenever PDSCH or PDCCH is transmitted on this subframe.

### 6.6.1	Scrambling

The block of bits, where , the number of bits transmitted on the physical broadcast channel for frame structure type 1 and 2 or the maximum number of bits for potential transmission on the physical broadcast channel for MF, equals 1920 for normal cyclic prefix and 1728 for extended cyclic prefix for frame structure type 1 and 2 and 2688 for MF, shall be scrambled with a cell-specific sequence prior to modulation, resulting in a block of scrambled bits   according to

where the scrambling sequence  is given by clause 7.2. The scrambling sequence shall be initialised with  in each radio frame fulfilling .

### 6.6.2	Modulation

The block of scrambled bits  shall be modulated as described in clause 7.1, resulting in a block of complex-valued modulation symbols. Table 6.6.2-1 specifies the modulation mappings applicable for the physical broadcast channel.

Table 6.6.2-1: PBCH/MF-PBCH modulation schemes.

| Physical channel | Modulation schemes |
| --- | --- |
| PBCH/MF-PBCH | QPSK |

### 6.6.3	Layer mapping and precoding

The block of modulation symbols  shall be mapped to layers according to one of clauses 6.3.3.1 or 6.3.3.3 with  and precoded according to one of  clauses 6.3.4.1 or 6.3.4.3, resulting in a block of vectors , , where  represents the signal for antenna port  and where  and the number of antenna ports for cell-specific reference signals .

### 6.6.4	Mapping to resource elements

The block of complex-valued symbols for each antenna port is transmitted during 4 consecutive radio frames starting in each radio frame fulfilling  and shall be mapped in sequence starting with  to resource elements  constituting the core set of PBCH/MF-PBCH resource elements.

For frame structure type 1 and 2:

The mapping to resource elements  not reserved for transmission of reference signals shall be in increasing order of first the index, then the index  in slot 1 in subframe 0 and finally the radio frame number. The resource-element indices are given by

where resource elements reserved for reference signals shall be excluded. The mapping operation shall assume cell-specific reference signals for antenna ports 0-3 being present irrespective of the actual configuration. The UE shall assume that the resource elements assumed to be reserved for reference signals in the mapping operation above but not used for transmission of reference signal are not available for PDSCH or MPDCCH transmission. The UE shall not make any other assumptions about these resource elements.

For MF:

- The mapping to resource elements  not reserved for transmission of reference signals shall be in increasing order of first the index, then the index  in a subframe containing DRS or in subframe 0 whenever PDSCH or PDCCH is transmitted and finally the radio frame number. The resource-element indices are given by

where resource elements reserved for cell-specific reference signals shall be excluded, and resource elements configured for channel state information resource signals may be excluded. The mapping operation shall assume cell-specific reference signals for antenna ports 0-3 being present, irrespective of the actual configuration. The UE shall assume that the resource elements assumed to be reserved for cell-specific reference signals in the mapping operation above but not used for transmission of reference signal are not available for PDSCH transmission. The UE shall not make any other assumptions about these resource elements. The UE may assume channel state information resource signals are absent for MF-PBCH mapping.

If a cell is configured with repetition of the physical broadcast channel (applicable only to frame structure type 1 and 2)

-	symbols mapped to core resource element   in slot 1 in subframe 0 within a radio frame  according to the mapping operation above, and

-	cell-specific reference signals in OFDM symbols  in slot 1 in subframe 0 within a radio frame  with  according to the mapping operation above

shall additionally be mapped to resource elements  in slot number  within radio frame  unless resource element  is used by CSI reference signals.

For frame structure type 1, , , and  are given by Table 6.6.4-1.

For frame structure type 2,

-	if ,  and  are given by Table 6.6.4-2 and ;

-	if ,  and  are given by Table 6.6.4-2 and , except that repetitions with  and   are not applied.

For both frame structure type 1 and frame structure type 2, repetition of the physical broadcast channel is not applicable if .

Resource elements already used for transmission of cell-specific reference signals in absence of repetition shall not be used for additional mapping of cell-specific reference signals.

Table 6.6.4-1: Frame offset, slot and symbol number triplets for repetition of PBCH for frame structure type 1.

|  | Frame offset, slot and symbol number triplets |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 0 | (1,18,3), (1,19,0), (1,19,4), (0,0,4) | (1,18,3), (1,19,0), (1,19,5) |
| 1 | (1,18,4), (1,19,1). (1,19,5), (0,1,4) | (1,18,4), (1,19,1). (0,0,3) |
| 2 | (1,18,5), (1,19,2), (1,19,6), (0,1,5) | (1,18,5), (1,19,2), (0,1,4) |
| 3 | (1,18,6), (1,19,3), (0,0,3), (0,1,6) | (1,19,3), (1,19,4), (0,1,5) |

Table 6.6.4-2: Slot and symbol number pairs for repetition of PBCH for frame structure type 2.

|  | Slot and symbol number pairs |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 0 | (0,3), (1,4), (10,3), (11,0), (11,4) | (0,3), (10,3), (11,0) |
| 1 | (0,4), (1,5), (10,4), (11,1), (11,5) | (0,4), (10,4), (11,1) |
| 2 | (0,5), (10,5), (11.2) | (0,5), (10,5), (11.2) |
| 3 | (0,6), (10,6), (11.3) | (1,4), (11,3), (11.4) |

## 6.7	Physical control format indicator channel

The physical control format indicator channel carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe. The set of OFDM symbols possible to use for PDCCH in a subframe is given by Table 6.7-1.

Table 6.7-1: Number of OFDM symbols used for PDCCH

| Subframe | Number of OFDM symbols for PDCCH when | Number of OFDM symbols for PDCCH when |
| --- | --- | --- |
| Subframe 1 and 6 for frame structure type 2 or a subframe for frame structure type 3 with the same duration as the DwPTS duration of a special subframe configuration | 1, 2 | 2 |
| MBSFN subframes on a carrier supporting PDSCH, configured with 1 or 2 cell-specific antenna ports | 1, 2 | 2 |
| MBSFN subframes on a carrier supporting PDSCH, configured with 4 cell-specific antenna ports | 2 | 2 |
| Subframes on a carrier not supporting PDSCH | 0 | 0 |
| Non-MBSFN subframes (except subframe 6 for frame structure type 2) configured with positioning reference signals | 1, 2, 3 | 2, 3 |
| All other cases | 1, 2, 3 | 2, 3, 4 |

The UE may assume the PCFICH is transmitted when the number of OFDM symbols for PDCCH is greater than zero unless stated otherwise in Section 12 of MFA TS 36.331 [4].

### 6.7.1	Scrambling

The block of bits  transmitted in one subframe shall be scrambled with a cell-specific sequence prior to modulation, resulting in a block of scrambled bitsaccording to

where the scrambling sequence  is given by clause 7.2. For an LTE cell, the scrambling sequence generator shall be initialised with  at the start of each subframe.

For an MF cell, the scrambling sequence generator shall be initialised with

where

### 6.7.2	Modulation

The block of scrambled bits shall be modulated as described in clause 7.1, resulting in a block of complex-valued modulation symbols. Table 6.7.2-1 specifies the modulation mappings applicable for the physical control format indicator channel.

Table 6.7.2-1: PCFICH modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PCFICH | QPSK |

### 6.7.3	Layer mapping and precoding

The block of modulation symbols  shall be mapped to layers according to one of clauses 6.3.3.1 or 6.3.3.3 with and precoded according to one of  clauses 6.3.4.1 or 6.3.4.3, resulting in a block of vectors , , where  represents the signal for antenna port  and where and the number of antenna ports for cell-specific reference signals . The PCFICH shall be transmitted on the same set of antenna ports as the PBCH.

### 6.7.4	Mapping to resource elements

The mapping to resource elements is defined in terms of quadruplets of complex-valued symbols. Let  denote symbol quadruplet  for antenna port. For each of the antenna ports, symbol quadruplets shall be mapped in increasing order of  to the four resource-element groups in the first OFDM symbol in a downlink subframe or DwPTS with the representative resource-element as defined in clause 6.2.4 given by

where the additions are modulo  ,

and  is the physical-layer cell identity as given by clause 6.11.

## 6.8	Physical downlink control channel

### 6.8.1	PDCCH formats

The physical downlink control channel carries scheduling assignments and other control information. A physical control channel is transmitted on an aggregation of one or several consecutive control channel elements (CCEs), where a control channel element corresponds to 9 resource element groups. The number of resource-element groups not assigned to PCFICH or PHICH is . The CCEs available in the system are numbered from 0 to, where . The PDCCH supports multiple formats as listed in Table 6.8.1-1. A PDCCH consisting of  consecutive CCEs may only start on a CCE fulfilling, where  is the CCE number.

Multiple PDCCHs can be transmitted in a subframe.

Table 6.8.1-1: Supported PDCCH formats

| PDCCH format | Number of CCEs | Number of resource-element groups | Number of PDCCH bits |
| --- | --- | --- | --- |
| 0 | 1 | 9 | 72 |
| 1 | 2 | 18 | 144 |
| 2 | 4 | 36 | 288 |
| 3 | 8 | 72 | 576 |

### 6.8.2	PDCCH multiplexing and scrambling

The block of bits  on each of the control channels to be transmitted in a subframe, where  is the number of bits in one subframe to be transmitted on physical downlink control channel number , shall be multiplexed, resulting in a block of bits , where  is the number of PDCCHs transmitted in the subframe.

The block of bits  shall be scrambled with a cell-specific sequence prior to modulation, resulting in a block of scrambled bits according to

where the scrambling sequence  is given by clause 7.2. For an LTE cell, the scrambling sequence generator shall be initialised with  at the start of each subframe.

For an MF cell, the scrambling sequence generator shall be initialised with

,

where

CCE number  corresponds to bits . If necessary, <NIL> elements shall be inserted in the block of bits prior to scrambling to ensure that the PDCCHs starts at the CCE positions as described in MFA TS 36.213 [4] and to ensure that the length  of the scrambled block of bits matches the amount of resource-element groups not assigned to PCFICH or PHICH.

### 6.8.3	Modulation

The block of scrambled bits shall be modulated as described in clause 7.1, resulting in a block of complex-valued modulation symbols. Table 6.8.3-1 specifies the modulation mappings applicable for the physical downlink control channel.

Table 6.8.3-1: PDCCH modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PDCCH | QPSK |

### 6.8.4	Layer mapping and precoding

The block of modulation symbols  shall be mapped to layers according to one of clauses 6.3.3.1 or 6.3.3.3  with  and precoded according to one of clauses 6.3.4.1 or 6.3.4.3, resulting in a block of vectors ,  to be mapped onto resources on the antenna ports used for transmission, where  represents the signal for antenna port . The PDCCH shall be transmitted on the same set of antenna ports as the PBCH.

### 6.8.5	Mapping to resource elements

The mapping to resource elements is defined by operations on quadruplets of complex-valued symbols. Let  denote symbol quadruplet  for antenna port.

The block of quadruplets , where , shall be permuted resulting in . The permutation shall be according to the sub-block interleaver in clause 5.1.4.2.1 of MFA TS 36.212 [3] with the following exceptions:

- the input and output to the interleaver is defined by symbol quadruplets instead of bits

- interleaving is performed on symbol quadruplets instead of bits by substituting the terms "bit", "bits" and "bit sequence" in clause 5.1.4.2.1 of MFA TS 36.212 [3] by "symbol quadruplet", "symbol quadruplets" and "symbol-quadruplet sequence", respectively

<NULL> elements at the output of the interleaver in MFA TS 36.212 [3] shall be removed when forming . Note that the removal of <NULL> elements does not affect any <NIL> elements inserted in clause 6.8.2.

The block of quadruplets  shall be cyclically shifted, resulting in   where.

Mapping of the block of quadruplets  is defined in terms of resource-element groups, specified in clause 6.2.4, according to steps 1–10 below:

1)	Initialize  (resource-element group number)

2)	Initialize

3)	Initialize

4)	If the resource element  represents a resource-element group and the resource-element group is not assigned to PCFICH or PHICH then perform step 5 and 6, else go to step 7

5)	Map symbol-quadruplet  to the resource-element group represented by  for each antenna port

6)	Increase  by 1

7) Increase  by 1

8) Repeat from step 4 if  , where  corresponds to the number of OFDM symbols used for PDCCH transmission as indicated by the sequence transmitted on the PCFICH

9) Increase  by 1

10) Repeat from step 3 if

## 6.8A	Enhanced physical downlink control channel

For frame structure type 3, for a subframe with the same duration as the DwPTS duration of a special subframe configuration, the enhanced physical downlink control channel is defined the same as that for the corresponding special subframe configuration.

### 6.8A.1	EPDCCH formats

The enhanced physical downlink control channel (EPDCCH) carries scheduling assignments. An enhanced physical downlink control channel is transmitted using an aggregation of one or several consecutive enhanced control channel elements (ECCEs) where each ECCE consists of multiple enhanced resource element groups (EREGs), defined in clause 6.2.4A. The number of ECCEs used for one EPDCCH depends on the EPDCCH format as given by Table 6.8A.1-2 and the number of EREGs per ECCE is given by Table 6.8A.1-1.Both localized and distributed transmission is supported.

An EPDCCH can use either localized or distributed transmission, differing in the mapping of ECCEs to EREGs and PRB pairs.

A UE shall monitor multiple EPDCCHs as defined in MFA TS 36.213 [4]. One or two sets of physical resource-block pairs which a UE shall monitor for EPDCCH transmissions can be configured. All EPDCCH candidates in EPDCCH set  use either only localized or only distributed transmission as configured by higher layers. Within EPDCCH set  in subframe , the ECCEs available for transmission of EPDCCHs are numbered from 0 to  and ECCE number  corresponds to

-	EREGs numbered  in PRB index  for localized mapping, and

-	EREGs numbered  in PRB indices   for distributed mapping,

where ,  is the number of EREGs per ECCE, and  is the number of ECCEs per resource-block pair. The physical resource-block pairs constituting EPDCCH set  are in this paragraph assumed to be numbered in ascending order from 0 to .

Table 6.8A.1-1: Number of EREGs per ECCE,

| Normal cyclic prefix |  |  | Extended cyclic prefix |  |
| --- | --- | --- | --- | --- |
| Normal subframe | Special subframe, configuration 3, 4, 8 | Special subframe, configuration 1, 2, 6, 7, 9 | Normal subframe | Special subframe, configuration 1, 2, 3, 5, 6 |
| 4 |  | 8 |  |  |

Table 6.8A.1-2: Supported EPDCCH formats

| EPDCCH format | Number of ECCEs for one EPDCCH, |  |  |  |
| --- | --- | --- | --- | --- |
|  | Case A |  | Case B |  |
|  | Localized transmission | Distributed transmission | Localized transmission | Distributed transmission |
| 0 | 2 | 2 | 1 | 1 |
| 1 | 4 | 4 | 2 | 2 |
| 2 | 8 | 8 | 4 | 4 |
| 3 | 16 | 16 | 8 | 8 |
| 4 | - | 32 | - | 16 |

Case A in Table 6.8A.1-2 is used when the conditions corresponding to case 1 in clause 9.1.4 of MFA TS 36.213 [4] are satisfied, otherwise case B is used. The quantity  for a particular UE and referenced in MFA TS 36.213 [4] is defined as the number of downlink resource elements  available for EPDCCH transmission in a physical resource-block pair configured for possible EPDCCH transmission of EPDCCH set  and fulfilling all of the following criteria:

-	they are part of any one of the 16 EREGs in the physical resource-block pair, and

-	they are assumed by the UE not to be used for cell-specific reference signals, where the positions of the cell-specific reference signals are given by clause 6.10.1.2 with the number of antenna ports for and the frequency shift of cell-specific reference signals derived as described in clause 6.10.1.2 unless other values for these parameters are provided by clause 9.1.4.3 in MFA TS 36.213 [4], and-

-	they are assumed by the UE not to be used for transmission of CSI reference signals, where the positions of the CSI reference signals are given by clause 6.10.5.2 with the configuration for zero power CSI reference signals obtained as described in clause 6.10.5.2 unless other values are provided by clause 9.1.4.3 in MFA TS 36.213 [4], and with the configuration for non-zero power CSI reference signals obtained as described in clause 6.10.5.2, and

-	for frame structure type 1 and 2, the index  in the first slot in a subframe fulfils  where  is given by clause 9.1.4.1 of MFA TS 36.213 [4], and

-	for frame structure type 3,

-	if the higher layer parameter subframeStartPosition indicates ‘s07’ and if the downlink transmission starts in the second slot of a subframe

-	the index  in the second slot in the subframe fulfils  where  is given by clause 9.1.4.1 of MFA TS 36.213  [4],

-	otherwise

-	the index  in the first slot in the subframe fulfils  where  is given by clause  9.1.4.1 of MFA TS 36.213  [4].

### 6.8A.2	Scrambling

The block of bits  to be transmitted on an EPDCCH in a subframe shall be scrambled, resulting in a block of scrambled bits  according to

where the UE-specific scrambling sequence  is given by clause 7.2. The scrambling sequence generator shall be initialized with  where  is the EPDCCH set number.

### 6.8A.3	Modulation

The block of scrambled bits  shall be modulated as described in clause 7.1, resulting in a block of complex-valued modulation symbols . Table 6.8A.3-1 specifies the modulation mappings applicable for the enhanced physical downlink control channel.

Table 6.8A.3-1: EPDCCH modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| EPDCCH | QPSK |

### 6.8A.4	Layer mapping and precoding

The block of complex-valued modulation symbols shall be mapped to a single layer and precoded according to Section 6.3.4.1 as for , .

### 6.8A.5	Mapping to resource elements

The block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements  on the associated antenna port which meet all of the following criteria:

-	they are part of the EREGs assigned for the EPDCCH transmission, and

-	they are assumed by the UE not to be used for cell-specific reference signals, where the positions of the cell-specific reference signals are given by clause 6.10.1.2 with the number of antenna ports for and the frequency shift of cell-specific reference signals derived as described in clause 6.10.1.2 unless other values for these parameters are provided by clause 9.1.4.3 in MFA TS 36.213 [4], and

-	they are assumed by the UE not to be used for transmission of:

-	zero-power CSI reference signals, where the positions of the CSI reference signals are given by clause 6.10.5.2. The configuration for zero power CSI reference signals is

-	obtained as described in clause 6.10.5.2 unless other values are provided by clause 9.1.4.3 in MFA TS 36.213 [4], and

-	obtained by higher-layer configuration of up to five reserved CSI-RS resources as part of the discovery signal configuration following the procedure for zero-power CSI-RS in clause 6.10.5.2.

-	non-zero-power CSI reference signals for CSI reporting with the configuration for non-zero power CSI reference signals for CSI reporting obtained as described in clause 6.10.5.2, and

-	for frame structure type 1 and 2, the index  in the first slot in a subframe fulfils  where  is given by clause 9.1.4.1 of MFA TS 36.213 [4], and

-	for frame structure type 3,

-	if the higher layer parameter subframeStartPosition indicates ‘s07’ and if the downlink transmission starts in the second slot of a subframe

-	the index  in the second slot in the subframe fulfils  where  is given by clause 9.1.4.1 of MFA TS 36.213  [4],

-	otherwise

-	the index  in the first slot in the subframe fulfils  where  is given by clause 9.1.4.1 of MFA TS 36.213  [4].

The mapping to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe.

For localized transmission, the single antenna port  to use is given by Table 6.8A.5-1 with

where  is the lowest ECCE index used by this EPDCCH transmission in the EPDCCH set,  equals the C-RNTI, and  is the number of ECCEs used for this EPDCCH.

Table 6.8A.5-1: Antenna port to use for localized EPDCCH transmission

|  | Normal cyclic prefix |  | Extended cyclic prefix |
| --- | --- | --- | --- |
|  | Normal subframes, Special subframes, configurations 3, 4, 8 | Special subframes, configurations 1, 2, 6, 7, 9 | Any subframe |
| 0 | 107 | 107 | 107 |
| 1 | 108 | 109 | 108 |
| 2 | 109 | - | - |
| 3 | 110 | - | - |

For distributed transmission, each resource element in an EREG is associated with one out of two antenna ports in an alternating manner, starting with antenna port 107, where  for normal cyclic prefix and  for extended cyclic prefix.

## 6.8B	MTC physical downlink control channel

### 6.8B.1	MPDCCH formats

The MPDCCH formats are defined as in Clause 6.8A.1 with the following exceptions:

-	The term EPDCCH is replaced by MPDCCH.

-	The MTC physical downlink control channel carries downlink control information and is transmitted across  consecutive BL/CE UL subframes. Within each of the  BL/CE UL subframes an MPDCCH is transmitted using an aggregation of one or several consecutive enhanced control channel elements (ECCEs) where each ECCE consists of multiple enhanced resource element groups (EREGs), defined in clause 6.2.4A.

-	For frame structure type 2,

-	For a UE configured in CEModeA, if repetition is not configured for the MPDCCH, the number of EREGs per ECCE is given by Table 6.8A.1-1. If repetition is configured for the MPDCCH, the number of EREGs per ECCE is given by Table 6.8B.1-1.

-	For both CEModeA and CEModeB, for those special subframes where the MPDCCH is not supported, these special subframes are considered BL/CE UL subframes for both MPDCCH and PDSCH transmission, only if they are indicated as BL/CE UL subframe by higher layer signalling.

-	For an MPDCCH associated with 2 or 4 PRBs, if repetition is not configured for the MPDCCH, the supported MPDCCH formats are given by Table 6.8A.1-2. Otherwise, the supported MPDCCH formats are given by Table 6.8B.1-2. However, for MPDCCH format 5, the equation defining the relation between ECCE index and EREG index does not apply and the number of ECCEs refers to the MPDCCH mapping to the REs of the 2+4 PRB set as defined in section 6.8B.5.

Table 6.8B.1-1: Number of EREGs per ECCE, , for frame structure type 2.

| Normal cyclic prefix |  | Extended cyclic prefix |  |
| --- | --- | --- | --- |
| Normal subframe | Special subframe, configuration 3, 4, 8 | Normal subframe | Special subframe, configuration 1, 2, 3, 5, 6 |
| 4 |  | 8 |  |

Table 6.8B.1-2: Supported MPDCCH formats

| MPDCCH format | Number of ECCEs in a subframe for one MPDCCH, |  |  |  |
| --- | --- | --- | --- | --- |
|  | =4 |  | =8 |  |
|  | Localized transmission | Distributed transmission | Localized transmission | Distributed transmission |
| 0 | 2 | 2 | 1 | 1 |
| 1 | 4 | 4 | 2 | 2 |
| 2 | 8 | 8 | 4 | 4 |
| 3 | 16 | 16 | 8 | 8 |
| 4 | - | - | - | - |
| 5 | 24 | 24 | 12 | 12 |

### 6.8B.2	Scrambling

Scrambling shall be perfomed according to Clause 6.8A.2 with EPDCCH replaced by MPDCCH except that the same scrambling sequence is applied per subframe to MPDCCH for a given block of  subframes and  is the MPDCCH set number. For MPDCCH format 5,  is used to generate the scrambling sequence for mapping to REs in 6 PRBs.

For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for the MPDCCH. The MPDCCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the MPDCCH transmission is postponed.

For BL/CE UEs,

-	if the MPDCCH transmission is associated with P-RNTI:

-	 for frame structure type 1 and  for frame structure type 2

-	otherwise

-	 for UEs assuming CEModeA (according to the definition in section 12 of MFA TS 36.213 [4]) or configured with CEModeA:

-	 for frame structure type 1 and  for frame structure type 2 for UEs assuming CEModeB (according to the definition in section 12 of MFA TS 36.213 [4]) or configured with CEModeB.

### 6.8B.3	Modulation

Modulation shall be performed according to 6.8A.3 with EPDCCH replaced by MPDCCH.

### 6.8B.4	Layer mapping and precoding

Layer mapping and precoding shall be done according to Clause 6.8A.4 with EPDCCH replaced by MPDCCH.

### 6.8B.5	Mapping to resource elements

Mapping to resource elements shall be done according to Clause 6.8A.5with the following exceptions:

-	The term EPDCCH shall be replaced by MPDCCH.

-	The mapping shall be repeated across each of the  BL/CE UL subframes.

-	 is the number of ECCEs used for this MPDCCH in the first of the  subframes.

-	For an MPDCCH associated with a 2+4 PRB set as defined in MFA TS 36.213 [4], the mapping to resource elements  on antenna port  shall be in increasing order of first the index  and then the index over the 6 PRBs for MPDCCH format 5 and over the 2 or 4 PRBs for the other MPDCCH formats.

-	For localized transmission and MPDCCH format 5, the single antenna port  to use is given by Table 6.8A.5-1 with

-	Resource elements occupied by CSI reference signals shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH.

-	The UE may assume there is no MPDCCH transmission which uses overlapping sets of subframes as PDSCH transmissions to that UE, where the MPDCCH is located at a different narrowband than the PDSCH.

- For BL/CE UEs in CEModeB, in MBSFN subframe(s), resource elements that correspond to the positions of cell-specific reference signals as in subframe #0 shall not be counted in the MPDCCH mapping and not used for transmission of the MPDCCH.

-	Resource elements belonging to synchronization signals, the core part of PBCH or PBCH repetitions shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH.

-	In the subframes where an MPDCCH or its associated PDSCH is transmitted in response to a physical random access transmission initiated by a PDCCH order, the UE shall receive the MPDCCH or its associated PDSCH, and assume no other UE-specific reception is needed.

-	For MPDCCH transmission associated with C-RNTI or SPS-RNTI, frequency hopping of the MPDCCH is enabled when higher layer parameter mpdcch-pdsch-HoppingConfig is set.

-	For MPDCCH transmission associated with RA-RNTI or temporary C-RNTI, frequency hopping of the MPDCCH is enabled when higher layer parameter rar-HoppingConfig is set. Further

-	if PRACH CE level 0 or 1 is used for the last PRACH attempt,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeA;

-	if PRACH CE level 2 or 3 is used for the last PRACH attempt,  is set to the higher layer parameter interval-DlHoppingConfigCommonModeB.

-	The narrowband  for MPDCCH transmission in the first subframe of MPDCCH monitoring instance is provided by higher layers. Starting subframe configuration of UE-specific search space where UE monitors an MPDCCH is also provided by higher layers. The MPDCCH is transmitted with  repetitions, spanning  consecutive subframes, including non-BL/CE UL subframes where the MPDCCH transmission is postponed.

-	If frequency hopping is not enabled for MPDCCH, the repetitions of an MPDCCH candidate are located at the same PRB resources in the same narrowband , and

-	if frequency hopping is enabled for MPDCCH, an MPDCCH candidate shall be transmitted in subframe  within the  consecutive downlink subframes using the same PRB resources within each narrowband

where  is the absolute subframe number of the first downlink subframe of MPDCCH monitoring instance, and ,  and  are cell-specific higher-layer parameters. The UE shall not expect MPDCCH transmission in subframe  if it is not a BL/CE UL subframe.

-	The UE may assume the same precoding matrix being used for a PRB across a block of   consecutive subframes for MPDCCH.

The UE may assume that an MPDCCH associated with the P-RNTI is transmitted on the set  of narrowbands where  is defined in section 6.4.1. For a UE monitoring an MPDCCH associated with the P-RNTI, the first MPDCCH narrowband is given by  where ,  is the Paging Narrowband (PN) obtained according to MFA TS36.304 [10], and  is the higher-layer parameter paging-narrowBands.

-	If the higher-layer parameter si-HoppingConfigCommon disables frequency hopping for an MPDCCH associated with P-RNTI, each MPDCCH candidate shall be located in the same PRB in narrowband  where .

-	If the higher-layer parameter si-HoppingConfigCommon enables frequency hopping for an MPDCCH with P-RNTI, an MPDCCH candidate shall be located in narrowband  in subframe  within the  consecutive downlink subframes using the same PRB resources within each narrowband  where

where  is the absolute subframe number of the first downlink subframe of MPDCCH monitoring instance according to locations of paging opportunity subframes, and ,  and  are cell-specific higher-layer parameters. For MPDCCH associated with P-RNTI, if interval-DlHoppingConfigCommonModeB is signalled in SIB1-BR, then the frequency hopping granularity  is set to interval-DlHoppingConfigCommonModeB; otherwise,  is set to interval-DlHoppingConfigCommonModeA signalled in SIB1-BR.

The UE shall not expect MPDCCH transmission in subframe  if it is not a BL/CE UL subframe.

## 6.9	Physical hybrid ARQ indicator channel

The PHICH carries the hybrid-ARQ ACK/NACK. Multiple PHICHs mapped to the same set of resource elements constitute a PHICH group, where PHICHs within the same PHICH group are separated through different orthogonal sequences. A PHICH resource is identified by the index pair , where  is the PHICH group number and  is the orthogonal sequence index within the group.

For frame structure type 1 and type 3, the number of PHICH groups  is constant in all subframes and given by

where  is provided by higher layers. The index  ranges from  to .

For MF, Ng is defined to have the value of 1/6.

For frame structure type 2, the number of PHICH groups may vary between subframes and is given by   where  is given by the expression above and  is given by Table 6.9-1 with the uplink-downlink configuration provided by the higher-layer parameter subframeAssignment. The index  in a subframe with non-zero PHICH resources ranges from  to .

Table 6.9-1: The factor  for frame structure type 2

| Uplink-downlink configuration | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 2 | 1 | 0 | 0 | 0 | 2 | 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| 2 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 3 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 6 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |

### 6.9.1	Modulation

The block of bits  transmitted on one PHICH in one subframe shall be modulated as described in clause 7.1, resulting in a block of complex-valued modulation symbols, where . Table 6.9.1-1 specifies the modulation mappings applicable for the physical hybrid ARQ indicator channel.

Table 6.9.1-1: PHICH modulation schemes.

| Physical channel | Modulation schemes |
| --- | --- |
| PHICH | BPSK |

The block of modulation symbols  shall be symbol-wise multiplied with an orthogonal sequence and scrambled, resulting in a sequence of modulation symbols   according to

where

and  is a cell-specific scrambling sequence generated according to clause 7.2. The scrambling sequence generator shall be initialised with  at the start of each subframe.

The sequence  is given by Table 6.9.1-2 where the sequence index  corresponds to the PHICH number within the PHICH group.

Table 6.9.1-2: Orthogonal sequences  for PHICH

| Sequence index | Orthogonal sequence |  |
| --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  | - |
| 5 |  | - |
| 6 |  | - |
| 7 |  | - |

### 6.9.2	Resource group alignment, layer mapping and precoding

The block of symbols  should be first aligned with resource element group size, resulting in a block of symbols , where  for normal cyclic prefix; and  for extended cyclic prefix.

For normal cyclic prefix, , for .

For extended cyclic prefix,

for .

The block of symbols   shall be mapped to layers and precoded, resulting in a block of vectors , , where  represents the signal for antenna port , and the number of cell-specific reference signals . The layer mapping and precoding operation depends on the cyclic prefix length and the number of antenna ports used for transmission of the PHICH. The PHICH shall be transmitted on the same set of antenna ports as the PBCH.

For transmission on a single antenna port, , layer mapping and precoding are defined by clauses 6.3.3.1 and 6.3.4.1, respectively, with .

For transmission on two antenna ports, , layer mapping and precoding are defined by clauses 6.3.3.3 and 6.3.4.3, respectively, with .

For transmission on four antenna ports, , layer mapping is defined by clause 6.3.3.3 with  and precoding by

if  for normal cyclic prefix, or  for extended cyclic prefix, where  is the PHICH group number and , and by

otherwise for .

### 6.9.3	Mapping to resource elements

The sequence  for each of the PHICH groups is defined by

where the sum is over all PHICHs in the PHICH group and  represents the symbol sequence from the :th PHICH in the PHICH group.

PHICH groups are mapped to PHICH mapping units.

For normal cyclic prefix, the mapping of PHICH group  to PHICH mapping unit is defined by

where

,

and where  is given by Table 6.9-1.

For extended cyclic prefix, the mapping of PHICH group and  to PHICH mapping unit  is defined by

where

and where  is given by Table 6.9-1.

Let  ,  denote symbol quadruplet  for antenna port.  Mapping to resource elements is defined in terms of symbol quadruplets according to steps 1–10 below:

1)	For each value of

2)	Let  denote the number of resource element groups not assigned to PCFICH in OFDM symbol

3)	Number the resource-element groups not assigned to PCFICH in OFDM symbol   from 0 to, starting from the resource-element group with the lowest frequency-domain index.

4)	Initialize  (PHICH mapping unit number)

5)	For each value of

6)	Symbol-quadruplet  from PHICH mapping unit  is mapped to the resource-element group represented by  as defined in clause 6.2.4 where the indices  and  are given by steps 7 and 8 below:

7)	The time-domain index  is given by

8)	Set the frequency-domain index  to the resource-element group assigned the number  in step 3 above, where  is given by

in case of extended PHICH duration in MBSFN subframes, or extended PHICH duration in subframes 1 and 6 for frame structure type 2, or extended PHICH duration in subframe with the same duration as the DwPTS duration of a special subframe configuration in frame structure type 3 and by

otherwise.

9)	Increase  by 1.

10)	Repeat from step 5 until all PHICH mapping units have been assigned.

The PHICH duration is configurable by higher layers according to Table 6.9.3-1.

Table 6.9.3-1: PHICH duration in MBSFN and non-MBSFN subframes

| PHICH duration | Non-MBSFN subframes |  |  | MBSFN subframes on a carrier supporting PDSCH |
| --- | --- | --- | --- | --- |
|  | Subframes 1 and 6 in case of frame structure type 2 | Subframe with the same duration as the DwPTS duration of a specifial subframe configuration in case of frame structure type 3 | All other cases |  |
| Normal | 1 | 1 | 1 | 1 |
| Extended | 2 | 2 | 3 | 2 |

## 6.10	Reference signals

Six types of downlink reference signals are defined:

-	Cell-specific Reference Signal (CRS)

-	MBSFN reference signal

-	UE-specific Reference Signal (DM-RS) associated with PDSCH

-	DeModulation Reference Signal (DM-RS) associated with EPDCCH or MPDCCH

-	Positioning Reference Signal (PRS)

-	CSI Reference Signal (CSI-RS)

There is one reference signal transmitted per downlink antenna port.

### 6.10.1	Cell-specific Reference Signal (CRS)

The UE may assume cell-specific reference signals are, unless otherwise stated in Section 12 of MFA TS 36.213 [4], transmitted in

-	all downlink subframes for frame structure type 1,

-	all downlink subframes and DwPTS for frame structure type 2,

-	non-empty subframes for frame structure type 3

in a cell supporting PDSCH transmission.

Cell-specific reference signals are transmitted on one or several of antenna ports 0 to 3.

Cell-specific reference signals are defined for  only.

#### 6.10.1.1	Sequence generation

The reference-signal sequence  is defined by

where  is the slot number within a radio frame and  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol where

#### 6.10.1.2	Mapping to resource elements

The reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference symbols for antenna port  in slot  according to

where

The variables  and  define the position in the frequency domain for the different reference signals where  is given by

The cell-specific frequency shift is given by .

Resource elements  used for transmission of cell-specific reference signals on any of the antenna ports in a slot shall not be used for any transmission on any other antenna port in the same slot and set to zero.

In an MBSFN subframe, cell-specific reference signals shall only be transmitted in the non-MBSFN region of the MBSFN subframe.

Figures 6.10.1.2-1 and 6.10.1.2-2 illustrate the resource elements used for reference signal transmission according to the above definition. The notation  is used to denote a resource element used for reference signal transmission on antenna port.

Figure 6.10.1.2-1. Mapping of downlink reference signals (normal cyclic prefix)

Figure 6.10.1.2-2. Mapping of downlink reference signals (extended cyclic prefix)

### 6.10.2	MBSFN reference signals

MBSFN reference signals shall be transmitted in the MBSFN region of MBSFN subframes only when the PMCH is transmitted. MBSFN reference signals are transmitted on antenna port 4.

MBSFN reference signals are defined for extended cyclic prefix only.

#### 6.10.2.1	Sequence generation

The MBSFN reference-signal sequence  is defined by

where  is the slot number within a radio frame and  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol.

#### 6.10.2.2	Mapping to resource elements

The reference-signal sequence  in OFDM symbol  shall be mapped to complex-valued modulation symbols  with  according to

where

Figure 6.10.2.2-1 illustrates the resource elements used for MBSFN reference signal transmission in case of . In case of  for a MBSFN-dedicated cell, the MBSFN reference signal shall be mapped to resource elements according to Figure 6.10.2.2-3. The notation  is used to denote a resource element used for reference signal transmission on antenna port.

Figure 6.10.2.2-1: Mapping of MBSFN reference signals (extended cyclic prefix, )

Figure 6.10.2.2-3: Mapping of MBSFN reference signals (extended cyclic prefix, )

### 6.10.3	UE-specific reference signals associated with PDSCH

UE-specific reference signals associated with PDSCH

-	are transmitted on antenna port(s) , , ,  ,, or , where  is the number of layers used for transmission of the PDSCH;

-	are present and are a valid reference for PDSCH demodulation only if the PDSCH transmission is associated with the corresponding antenna port according to clause 7.1 of MFA TS 36.213 [4];

-	are transmitted only on the physical resource blocks upon which the corresponding PDSCH is mapped.

A UE-specific reference signal associated with PDSCH is not transmitted in resource elements  in which one of the physical channels or physical signals other than the UE-specific reference signals defined in 6.1 are transmitted using resource elements with the same index pair  regardless of their antenna port .

For frame structure type 3, for PDSCH in a subframe with the same duration as the DwPTS duration of a special subframe configuration, the UE-specific reference signals are defined the same as that for the corresponding special subframe configuration.

#### 6.10.3.1	Sequence generation

For antenna port 5, the UE-specific reference-signal sequence  is defined by

where  denotes the assigned bandwidth in resource blocks of the corresponding PDSCH transmission. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each subframe where  is as described in clause 7.1 MFA TS 36.213 [4].

For any of the antenna ports , the reference-signal sequence  is defined by

.

The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with

at the start of each subframe.

For BL/CE UEs, the same scrambling sequence is applied per subframe to the UE-specific reference-signal sequence for a given block of  subframes. For the block of   subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for PDSCH. The PDSCH transmissions span  consecutive subframes, including non-BL/CE UL subframes where the PDSCH transmission is postponed. For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2.

The quantities , , are given by

-	 if no value for  is provided by higher layers or if DCI format 1A, 2B or 2C is used for the DCI associated with the PDSCH transmission

-	 otherwise

The value of  is zero unless specified otherwise. For a PDSCH transmission on ports 7 or 8,  is given by the DCI format 2B, 2C, 2D or 6-1A in MFA TS 36.212 [3] associated with the PDSCH transmission. 
In the case of DCI format 2B,  is indicated by the scrambling identity field according to Table 6.10.3.1-1. In the case of DCI format 2C or 2D,  is given by Table 5.3.3.1.5C-1 or Table 5.3.3.1.5C-2 in MFA TS 36.212 [3]. For a PDSCH transmission on ports 11 or 13,  is given by the DCI format 2C or 2D in MFA TS 36.212 [3] associated with the PDSCH transmission where  is given by Table 5.3.3.1.5C-2 in MFA TS 36.212 [3].

Table 6.10.3.1-1: Mapping of scrambling identity field in DCI format 2B to values for antenna ports 7 and 8

| Scrambling identity field in DCI format 2B (MFA TS 36.212 [3]) |  |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

#### 6.10.3.2	Mapping to resource elements

For antenna port 5, in a physical resource block with frequency-domain index  assigned for the corresponding PDSCH transmission, the reference signal sequence  shall be mapped to complex-valued modulation symbols  with  in a subframe according to:

Normal cyclic prefix:

Extended cyclic prefix:

where   is the counter of UE-specific reference signal resource elements within a respective OFDM symbol of the PDSCH transmission.

The cell-specific frequency shift is given by .

The mapping shall be in increasing order of the frequency-domain index  of the physical resource blocks assigned for the corresponding PDSCH transmission. The quantity  denotes the assigned bandwidth in resource blocks of the corresponding PDSCH transmission.

Figure 6.10.3.2-1 illustrates the resource elements used for UE-specific reference signals for normal cyclic prefix for antenna port 5.

Figure 6.10.3.2-2 illustrates the resource elements used for UE-specific reference signals for extended cyclic prefix for antenna port 5.

The notation  is used to denote a resource element used for reference signal transmission on antenna port.

Figure 6.10.3.2-1: Mapping of UE-specific reference signals, antenna port 5 (normal cyclic prefix)

Figure 6.10.3.2-2: Mapping of UE-specific reference signals, antenna port 5 (extended cyclic prefix)

For antenna ports , ,,, or , in a physical resource block with frequency-domain index  assigned for the corresponding PDSCH transmission, a part of the reference signal sequence  shall be mapped to complex-valued modulation symbols  in a subframe according to

Normal cyclic prefix:

where

The sequence  is given by Table 6.10.3.2-1.

Table 6.10.3.2-1: The sequence  for normal cyclic prefix

| Antenna port |  |
| --- | --- |
| 7 |  |
| 8 |  |
| 9 |  |
| 10 |  |
| 11 |  |
| 12 |  |
| 13 |  |
| 14 |  |

Extended cyclic prefix:

where

The sequence  is given by Table 6.10.3.2-2.

Table 6.10.3.2-2: The sequence  for extended cyclic prefix

| Antenna port |  |
| --- | --- |
| 7 |  |
| 8 |  |

For extended cyclic prefix, UE-specific reference signals are not supported on antenna ports 9 to 14.

Resource elements  used for transmission of UE-specific reference signals to one UE on any of the antenna ports in the set , where  or  shall

-	not be used for transmission of PDSCH on any antenna port in the same slot, and

-	not be used for UE-specific reference signals to the same UE on any antenna port other than those in  in the same slot.

Figure 6.10.3.2-3 illustrates the resource elements used for UE-specific reference signals for normal cyclic prefix for antenna ports 7, 8, 9 and 10. Figure 6.10.3.2-4 illustrates the resource elements used for UE-specific reference signals for extended cyclic prefix for antenna ports 7, 8.

Figure 6.10.3.2-3: Mapping of UE-specific reference signals, antenna ports 7, 8, 9 and 10 (normal cyclic prefix)

Figure 6.10.3.2-4: Mapping of UE-specific reference signals, antenna ports 7 and 8 (extended cyclic prefix)

### 6.10.3A	Demodulation reference signals associated with EPDCCH or MPDCCH

The demodulation reference signal associated with EPDCCH/MPDCCH

-	is transmitted on the same antenna port  as the associated EPDCCH/MPDCCH physical resource;

-	is present and is a valid reference for EPDCCH/MPDCCH demodulation only if the EPDCCH/MPDCCH transmission is associated with the corresponding antenna port;

-	is transmitted only on the physical resource blocks upon which the corresponding EPDCCH/MPDCCH is mapped.

A demodulation reference signal associated with EPDCCH/MPDCCH is not transmitted in resource elements  in which one of the physical channels or physical signals other than the demodulation reference signals defined in 6.1 are transmitted using resource elements with the same index pair  regardless of their antenna port .

#### 6.10.3A.1	Sequence generation

For any of the antenna ports , the reference-signal sequence  is defined by

.

For non-BL/CE UEs, the pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with

at the start of each subframe where  and  is configured by higher layers. The EPDCCH set to which the EPDCCH associated with the demodulation reference signal belong is denoted .

For BL/CE UEs, the same scrambling sequence is applied per subframe to the demodulation reference signal associated with MPDCCH for a given block of  subframes. For the block of   subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for MPDCCH. The MPDCCH transmissions span  consecutive subframes, including non-BL/CE UL subframes where the MPDCCH transmission is postponed. For a BL/CE UE configured with CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2. The quantities  and  are configured by higher layers. The MPDCCH set to which the MPDCCH associated with the demodulation reference signal belong is denoted . For an MPDCCH associated with a 2+4 PRB set as defined in MFA TS 36.213 [4],  is used to generate the scrambling sequence for the 6 PRBs as well as for the 2 PRBs and 4 PRBs.

#### 6.10.3A.2	Mapping to resource elements

For the antenna port  in a physical resource block  assigned for the associated EPDCCH/MPDCCH, a part of the reference signal sequence  shall be mapped to complex-valued modulation symbols  in a subframe according to

Normal cyclic prefix:

where

The sequence  is given by Table 6.10.3A.2-1.

Table 6.10.3A.2-1: The sequence  for normal cyclic prefix

| Antenna port |  |
| --- | --- |
| 107 |  |
| 108 |  |
| 109 |  |
| 110 |  |

Extended cyclic prefix:

where

The sequence  is given by Table 6.10.3A.2-2.

Table 6.10.3A.2-2: The sequence  for extended cyclic prefix

| Antenna port |  |
| --- | --- |
| 107 |  |
| 108 |  |

For extended cyclic prefix, demodulation reference signals are not supported on antenna ports 109 to 110.

Resource elements  used for transmission of demodulation reference signals to one UE on any of the antenna ports in the set , where  or  shall

-	not be used for transmission of EPDCCH/MPDCCH on any antenna port in the same slot, and

-	not be used for demodulation reference signals to the same UE on any antenna port other than those in  in the same slot.

Replacing antenna port numbers 7 – 10 by 107 – 110 in Figure 6.10.3.2-3 provides an illustration of the resource elements used for demodulation reference signals associated with EPDCCH/MPDCCH for normal cyclic prefix. Replacing antenna port numbers 7 – 8 by 107 – 108 in Figure 6.10.3.2-4 provides an illustration of the resource elements used for demodulation reference signals associated with EPDCCH/MPDCCH for extended cyclic prefix.

For frame structure type 3, for EPDCCH in a subframe with the same duration as the DwPTS duration of a special subframe configuration, the mapping of the demodulation reference signals to the resource elements is the same as that for the corresponding special subframe configuration.

### 6.10.4	Positioning reference signals

Positioning reference signals shall only be transmitted in resource blocks in downlink subframes configured for positioning reference signal transmission. If both normal and MBSFN subframes are configured as positioning subframes within a cell, the OFDM symbols in a MBSFN subframe configured for positioning reference signal transmission shall use the same cyclic prefix as used for subframe #0. If only MBSFN subframes are configured as positioning subframes within a cell, the OFDM symbols configured for positioning reference signals in the MBSFN region of these subframes shall use extended cyclic prefix length. In a subframe configured for positioning reference signal transmission, the starting positions of the OFDM symbols configured for positioning reference signal transmission shall be identical to those in a subframe in which all OFDM symbols have the same cyclic prefix length as the OFDM symbols configured for positioning reference signal transmission.

Positioning reference signals are transmitted on antenna port 6.

The positioning reference signals shall not be mapped to resource elements  allocated to the core part of the PBCH, PSS or SSS regardless of their antenna port .

Positioning reference signals are defined for  only.

#### 6.10.4.1	Sequence generation

The reference-signal sequence  is defined by

where  is the slot number within a radio frame,  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol where

#### 6.10.4.2	Mapping to resource elements

The reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference signal for antenna port  in slot  according to

where

Normal cyclic prefix:

Extended cyclic prefix:

The bandwidth for positioning reference signals  is configured by higher layers and the cell-specific frequency shift is given by .

Figure 6.10.4.2-1: Mapping of positioning reference signals (normal cyclic prefix)

Figure 6.10.4.2-2: Mapping of positioning reference signals (extended cyclic prefix)

#### 6.10.4.3	Positioning reference signal subframe configuration

The cell specific subframe configuration period  and the cell specific subframe offset  for the transmission of positioning reference signals are listed in Table 6.10.4.3-1. The PRS configuration index  is configured by higher layers. Positioning reference signals are transmitted only in configured DL subframes. Positioning reference signals shall not be transmitted in DwPTS. Positioning reference signals shall be transmitted in  consecutive downlink subframes, where  is configured by higher layers.

The positioning reference signal instances, for the first subframe of the  downlink subframes, shall satisfy .

Table 6.10.4.3-1: Positioning reference signal subframe configuration

| PRS configuration Index | PRS periodicity (subframes) | PRS subframe offset (subframes) |
| --- | --- | --- |
| 0 – 159 | 160 |  |
| 160 – 479 | 320 |  |
| 480 – 1119 | 640 |  |
| 1120 – 2399 | 1280 |  |
| 2400-4095 | Reserved |  |

### 6.10.5	CSI reference signals

CSI reference signals are transmitted on 1, 2, 4, 8, 12, 16, 20, 24, 28, or 32 antenna ports using , , , , , ,, ,  and, respectively.

For CSI reference signals using more than eight antenna ports,  CSI-RS configurations in the same subframe, numbered from 0 to , where value 0 corresponds to the configured resourceConfig-r11 or resourceConfig-r10, and value k (k>0) corresponds to the configured k-th entry of nzp-resourceConfigList-r13, are aggregated to obtain  antenna ports in total. Each CSI-RS configuration in such an aggregation corresponds to  antenna ports and one of the configurations in Table 6.10.5.3-1 or 6.10.5.2-1 for normal cyclic prefix, and one of the configurations in Table 6.10.5.3-1 or 6.10.5.2-2 for extended cyclic prefix. The supported configurations of aggregated CSI-RS configurations are shown in Table 6.10.5-1. If the higher layer parameter NZP-TransmissionComb is not configured,  unique CSI-RS configurations from Table 6.10.5.2-1 for normal cyclic prefix and from Table 6.10.5.2-2 for extended cyclic prefix are aggregated to form 12, 16, 20, 24, 28, or 32 antenna ports.

For CSI reference signals using more than sixteen antenna ports, when higher layer parameter NZP-TransmissionComb is configured, the number of unique CSI-RS configurations from Table 6.10.5.2-1 for normal cyclic prefix and from Table 6.10.5.2-2 for extended cyclic prefix that are aggregated to form 20, 24, 28, or 32 antenna ports can be less than or equal to . The number of antenna ports within each such unique CSI-RS resource configuration is an integer multiple of  .

CSI reference signals are defined for  only.

Table 6.10.5-1: Aggregation of CSI-RS configurations.

| Total number of antenna ports | Number of antenna ports per CSI-RS configuration | Number of CSI-RS configurations |
| --- | --- | --- |
| 12 | 4 | 3 |
| 16 | 8 | 2 |
| 20 | 4 | 5 |
| 24 | 8 | 3 |
| 28 | 4 | 7 |
| 32 | 8 | 4 |

#### 6.10.5.1	Sequence generation

The reference-signal sequence  is defined by

where  is the slot number within a radio frame and  is the OFDM symbol number within the slot. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each OFDM symbol where

The quantity  equals  unless configured by higher layers.

#### 6.10.5.2	Mapping to resource elements

In subframes configured for CSI reference signal transmission, the reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference symbols on antenna port  . The mapping depends on the higher-layer parameter CDMType.

For the case of CDMType is not configured or is configured to CDM2:

where

For the case of CDMType equal to CDM4:

where

and where  is given by Table 6.10.5.2-0.

Table 6.10.5.2-0: The sequence  for CDM4.

| 15 | 15,17 |  |
| --- | --- | --- |
| 16 | 16,18 |  |
| 17 | 19,21 |  |
| 18 | 20,22 |  |

If neither of the higher-layer parameters NZP-FrequencyDensity and NZP-TransmissionComb are configured, .

If the UE is configured with one or more of the parameters NZP-FrequencyDensity and NZP-TransmissionComb,

-	if either NZP-FrequencyDensity equals 1,

-	if NZP-FrequencyDensity equals 1/2 and NZP-TransmissionComb equals 0,

-	if NZP-FrequencyDensity equals 1/2 and NZP-TransmissionComb equals 1,

-	if NZP-FrequencyDensity equals 1/3 and NZP-TransmissionComb equals 0,

-	if NZP-FrequencyDensity equals 1/3 and NZP-TransmissionComb equals 1,

-	if NZP-FrequencyDensity equals 1/3 and NZP-TransmissionComb equals 2,

The quantity  and the necessary conditions on  are given by Tables 6.10.5.2-1 and 6.10.5.2-2 for normal and extended cyclic prefix, respectively.

The relation between the antenna port number  and the quantity  depends on the number of CSI-RS antenna ports:

-	for CSI reference signals using up to eight antenna ports,

-	for CSI reference signals using more than eight antenna ports when the higher-layer parameter CDMType equals CDM2

where  is the CSI-RS resource number.

-	for CSI reference signals using more than eight antenna ports when the higher-layer parameter CDMType equals CDM4, antenna port number  where  for CSI-RS resource number ..

For the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 32:

where

The resource elements for the  CDM8 pattern, where  , are determined by aggregating pairs of resource elements  satisfying  from the  aggregated CSI-RS configurations, where at most one pair of resource elements is drawn from each of the  aggregated CSI-RS configurations.  For the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 32, the aggregated CSI-RS configurations from Table 6.10.5.2-1 for normal cyclic prefix and from Table 6.10.5.2-2 for extended cyclic prefix are restricted to one of , , or .  Antenna port number  where  for CSI-RS resource number .  The sequence is given by Table 6.10.5.2-0A.

Table 6.10.5.2-0A: The sequence  for CDM8 with 32 CSI-RS antenna ports.

| 15, 17, 19, 21 |  |
| --- | --- |
| 16, 18, 20, 22 |  |
| 23, 25, 27, 29 |  |
| 24, 26, 28, 30 |  |
| 31, 33, 35, 37 |  |
| 32, 34, 36, 38 |  |
| 39, 41, 43, 45 |  |
| 40, 42, 44, 46 |  |

For the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 24:

where

For the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 24, the aggregated CSI-RS configurations from Table 6.10.5.2-1 for normal cyclic prefix are restricted to  in that order.  Resource elements for CDM8 patterns are determined as follows:

-	Aggregating resource element quadruplet  satisfying  from CSI-RS configuration 1 with resource element quadruplet  satisfying  from CSI-RS configuration 2

-	Aggregating resource element quadruplet  satisfying  from CSI-RS configuration 3 with resource element quadruplet  satisfying  from CSI-RS configuration 1

-	Aggregating resource element quadruplet  satisfying  from CSI-RS configuration 2 with resource element quadruplet  satisfying  from CSI-RS configuration 3

Antenna port number  where  for CSI-RS resource number .  The sequence is given by Table 6.10.5.2-0B.

Table 6.10.5.2-0B: The sequence  for CDM8 with 24 CSI-RS antenna ports.

| 15, 25, 31 |  |
| --- | --- |
| 16, 26, 32 |  |
| 19, 29, 35 |  |
| 20, 30, 36 |  |
| 17, 23, 33 |  |
| 18, 24, 34 |  |
| 21, 27, 37 |  |
| 22, 28, 38 |  |

Multiple CSI reference signal configurations can be used in a given cell. A UE can be configured with multiple sets of CSI reference signals,

-	one or more configurations for CSI reporting for which the UE shall assume non-zero transmission power for the CSI-RS, and

-	zero or more configurations for which the UE shall assume zero transmission power, and

-	zero or more configurations valid across the system downlink bandwidth as part of the discovery signals for which the UE shall assume non-zero transmission power for the CSI-RS.

The CSI-RS configurations for which the UE shall assume non-zero transmission power are provided by higher layers.

The CSI-RS configurations for which the UE shall assume zero transmission power in a subframe are given by a bitmap derived according to clause 7.2.7 in MFA TS 36.213 [4]. For each bit set to one in the 16-bit bitmap, the UE shall assume zero transmission power for the resource elements corresponding to the four CSI reference signal column in Tables 6.10.5.2-1 and 6.10.5.2-2 for normal and extended cyclic prefix, respectively, except for resource elements that overlap with those for which the UE shall assume non-zero transmission power CSI-RS as configured by higher layers. The most significant bit corresponds to the lowest CSI reference signal configuration index and subsequent bits in the bitmap correspond to configurations with indices in increasing order.

CSI reference signals not corresponding to higher layer configured parameters csi-RS-ConfigNZP-ApList or csi-RS-ConfigZP-Ap can only occur in

-	downlink slots where  fulfils the condition in Tables 6.10.5.2-1 and 6.10.5.2-2 for normal and extended cyclic prefix, respectively, and

-	where the subframe number fulfils the conditions in clause 6.10.5.3.

CSI reference signals corresponding to either higher layer configured parameter csi-RS-ConfigNZP-ApList or csi-RS-ConfigZP-Ap can only occur in

-	downlink slots where  fulfils the condition in Tables 6.10.5.2-1 and 6.10.5.2-2 for normal and extended cyclic prefix, respectively.

The UE shall assume that CSI reference signals are not transmitted

-	in the DwPTS for special subframe configuration 0, 5 and 9 for normal cyclic prefix and special subframe configuration 0, 4 and 7 for extended cyclic prefix, in case of frame structure type 2,

- 	in the DwPTS for normal CP for the case of CDMType equal to CDM8 and the number of CSI-RS antenna ports equal to 24,

-	in subframes where PDSCH/EPDCCH transmission starts in the second slot of a subframe for frame structure type 3,

-	in subframes where PDSCH/EPDCCH transmission ends prior to the end of a subframe for frame structure type 3,

-	in an empty subframe where there is no PDSCH or discovery signal transmission for frame structure type 3,

-	in subframes where transmission of a CSI-RS would collide with SystemInformationBlockType1 messages,

-	in the primary cell in subframes configured for transmission of paging messages in the primary cell for any UE with the cell-specific paging configuration.

For special subframe configuration {1, 2, 6, or 7}, a UE does not expect to be configured with one of CSI-RS configurations {1, 2, 3, 4, 6, 7, 8, 9, 12, 13, 14, 15, 16, 17} in DwPTS for normal CP.

The UE shall assume that none of the CSI reference signals corresponding to a CSI reference signal configuration are transmitted in subframes where transmission of any of those CSI reference signals would collide with transmission of synchronization signals or the core part of PBCH.

For MF, CSI-RS may be configured in the same subframe as the DRS and if configured, the UE may assume that the CSI-RS is transmitted and may ignore the presence of MF-PBCH.

Note: The CSI-RS is in this case used for CSI-RSRP measurements.

Resource elements  used for transmission of CSI reference signals on any of the antenna ports in the set , where , , , , , , , , , , , , , , ,  or  shall not be used for transmission of PDSCH on any antenna port in the same slot if higher layer parameter CDMType is not configured, or is configured to CDM2.

Resource elements  used for transmission of CSI reference signals on any of the antenna ports in the set , where

-	,  or  for CSI reference signals on 12 ports, or

-	, ,  or  for CSI reference signals on 16 ports, or

-	, , ,  or for CSI reference signals on 20 ports, or

-	, , , ,  or  for CSI reference signals on 24 ports, or

-	, , , ,,  or  for CSI reference signals on 28 ports, or

-	, , , , , ,  or  for CSI reference signals on 32 ports

shall not be used for transmission of PDSCH on any antenna port in the same slot if higher layer parameter CDMType is configured to CDM4.

Resource elements  used for transmission of CSI reference signals on any of the antenna ports in the set , where

-	,  or  for CSI reference signals on 24 ports, or

-	, ,  or  for CSI reference signals on 32 ports

shall not be used for transmission of PDSCH on any antenna port in the same slot if higher layer parameter CDMType is configured to CDM8.

The mapping for CSI reference signal configuration 0 is illustrated in Figures 6.10.5.2-1 and 6.10.5.2-2.

Table 6.10.5.2-1: Mapping from CSI reference signal configuration to  for normal cyclic prefix

| CSI-RS config. | Number of CSI reference signals configured |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 or 2 |  |  |  | 4 |  |  |  | 8 |  |  |  |
|  | Normal subframe |  | Special subframe |  | Normal subframe |  | Special subframe |  | Normal subframe |  | Special subframe |  |
| 0 | (9,5) | 0 | (9,5) | 0 | (9,5) | 0 | (9,5) | 0 | (9,5) | 0 | (9,5) | 0 |
| 1 | (11,2) | 1 | (11,5) | 0 | (11,2) | 1 | (11,5) | 0 | (11,2) | 1 | (11,5) | 0 |
| 2 | (9,2) | 1 | (9,2) | 1 | (9,2) | 1 | (9,2) | 1 | (9,2) | 1 | (9,2) | 1 |
| 3 | (7,2) | 1 | (7,5) | 0 | (7,2) | 1 | (7,5) | 0 | (7,2) | 1 | (7,5) | 0 |
| 4 | (9,5) | 1 |  |  | (9,5) | 1 |  |  | (9,5) | 1 |  |  |
| 5 | (8,5) | 0 | (8,5) | 0 | (8,5) | 0 | (8,5) | 0 |  |  |  |  |
| 6 | (10,2) | 1 | (10,5) | 0 | (10,2) | 1 | (10,5) | 0 |  |  |  |  |
| 7 | (8,2) | 1 | (8,2) | 1 | (8,2) | 1 | (8,2) | 1 |  |  |  |  |
| 8 | (6,2) | 1 | (6,5) | 0 | (6,2) | 1 | (6,5) | 0 |  |  |  |  |
| 9 | (8,5) | 1 |  |  | (8,5) | 1 |  |  |  |  |  |  |
| 10 | (3,5) | 0 | (3,5) | 0 |  |  |  |  |  |  |  |  |
| 11 | (2,5) | 0 | (2,5) | 0 |  |  |  |  |  |  |  |  |
| 12 | (5,2) | 1 | (5,5) | 0 |  |  |  |  |  |  |  |  |
| 13 | (4,2) | 1 | (4,5) | 0 |  |  |  |  |  |  |  |  |
| 14 | (3,2) | 1 | (3,2) | 1 |  |  |  |  |  |  |  |  |
| 15 | (2,2) | 1 | (2,2) | 1 |  |  |  |  |  |  |  |  |
| 16 | (1,2) | 1 | (1,5) | 0 |  |  |  |  |  |  |  |  |
| 17 | (0,2) | 1 | (0,5) | 0 |  |  |  |  |  |  |  |  |
| 18 | (3,5) | 1 |  |  |  |  |  |  |  |  |  |  |
| 19 | (2,5) | 1 |  |  |  |  |  |  |  |  |  |  |
| 20 | (11,1) | 1 |  |  | (11,1) | 1 |  |  | (11,1) | 1 |  |  |
| 21 | (9,1) | 1 |  |  | (9,1) | 1 |  |  | (9,1) | 1 |  |  |
| 22 | (7,1) | 1 |  |  | (7,1) | 1 |  |  | (7,1) | 1 |  |  |
| 23 | (10,1) | 1 |  |  | (10,1) | 1 |  |  |  |  |  |  |
| 24 | (8,1) | 1 |  |  | (8,1) | 1 |  |  |  |  |  |  |
| 25 | (6,1) | 1 |  |  | (6,1) | 1 |  |  |  |  |  |  |
| 26 | (5,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 27 | (4,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 28 | (3,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 29 | (2,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 30 | (1,1) | 1 |  |  |  |  |  |  |  |  |  |  |
| 31 | (0,1) | 1 |  |  |  |  |  |  |  |  |  |  |

Note:	. Configurations 0 – 19 for normal subframes are available for frame structure types 1, 2 and 3. Configurations 20 – 31 and configurations for special subframes are available for frame structure type 2 only.

Table 6.10.5.2-2: Mapping from CSI reference signal configuration to  for extended cyclic prefix.

| CSI-RS config. | Number of CSI reference signals configured |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 or 2 |  |  |  | 4 |  |  |  | 8 |  |  |  |
|  | Normal subframe |  | Special subframe |  | Normal subframe |  | Special subframe |  | Normal Subframe |  | Special subframe |  |
| 0 | (11,4) | 0 | (11,4) | 0 | (11,4) | 0 | (11,4) | 0 | (11,4) | 0 | (11,4) | 0 |
| 1 | (9,4) | 0 | (9,4) | 0 | (9,4) | 0 | (9,4) | 0 | (9,4) | 0 | (9,4) | 0 |
| 2 | (10,4) | 1 |  |  | (10,4) | 1 |  |  | (10,4) | 1 |  |  |
| 3 | (9,4) | 1 |  |  | (9,4) | 1 |  |  | (9,4) | 1 |  |  |
| 4 | (5,4) | 0 | (5,4) | 0 | (5,4) | 0 | (5,4) | 0 |  |  |  |  |
| 5 | (3,4) | 0 | (3,4) | 0 | (3,4) | 0 | (3,4) | 0 |  |  |  |  |
| 6 | (4,4) | 1 |  |  | (4,4) | 1 |  |  |  |  |  |  |
| 7 | (3,4) | 1 |  |  | (3,4) | 1 |  |  |  |  |  |  |
| 8 | (8,4) | 0 | (8,4) | 0 |  |  |  |  |  |  |  |  |
| 9 | (6,4) | 0 | (6,4) | 0 |  |  |  |  |  |  |  |  |
| 10 | (2,4) | 0 | (2,4) | 0 |  |  |  |  |  |  |  |  |
| 11 | (0,4) | 0 | (0,4) | 0 |  |  |  |  |  |  |  |  |
| 12 | (7,4) | 1 |  |  |  |  |  |  |  |  |  |  |
| 13 | (6,4) | 1 |  |  |  |  |  |  |  |  |  |  |
| 14 | (1,4) | 1 |  |  |  |  |  |  |  |  |  |  |
| 15 | (0,4) | 1 |  |  |  |  |  |  |  |  |  |  |
| 16 | (11,1) | 1 | (11,1) | 1 | (11,1) | 1 | (11,1) | 1 | (11,1) | 1 | (11,1) | 1 |
| 17 | (10,1) | 1 | (10,1) | 1 | (10,1) | 1 | (10,1) | 1 | (10,1) | 1 | (10,1) | 1 |
| 18 | (9,1) | 1 | (9,1) | 1 | (9,1) | 1 | (9,1) | 1 | (9,1) | 1 | (9,1) | 1 |
| 19 | (5,1) | 1 | (5,1) | 1 | (5,1) | 1 | (5,1) | 1 |  |  |  |  |
| 20 | (4,1) | 1 | (4,1) | 1 | (4,1) | 1 | (4,1) | 1 |  |  |  |  |
| 21 | (3,1) | 1 | (3,1) | 1 | (3,1) | 1 | (3,1) | 1 |  |  |  |  |
| 22 | (8,1) | 1 | (8,1) | 1 |  |  |  |  |  |  |  |  |
| 23 | (7,1) | 1 | (7,1) | 1 |  |  |  |  |  |  |  |  |
| 24 | (6,1) | 1 | (6,1) | 1 |  |  |  |  |  |  |  |  |
| 25 | (2,1) | 1 | (2,1) | 1 |  |  |  |  |  |  |  |  |
| 26 | (1,1) | 1 | (1,1) | 1 |  |  |  |  |  |  |  |  |
| 27 | (0,1) | 1 | (0,1) | 1 |  |  |  |  |  |  |  |  |

Note:	. Configurations 0 – 15 for normal subframes are available for both frame structure type 1 and type 2. Configurations 16 – 27 and configurations for special subframes are available for frame structure type 2 only.

Figure 6.10.5.2-1: Mapping of CSI reference signals (CSI configuration 0, normal cyclic prefix)

Figure 6.10.5.2-2: Mapping of CSI reference signals (CSI configuration 0, extended cyclic prefix)

#### 6.10.5.3	CSI reference signal subframe configuration

The subframe configuration period  and the subframe offset  for the occurence of CSI reference signals are listed in Table 6.10.5.3-1. The parameter  can be configured separately for CSI reference signals for which the UE shall assume non-zero and zero transmission power. Subframes containing CSI reference signals that do not correspond to either higher layer configured parameter csi-RS-ConfigNZP-ApList or csi-RS-ConfigZP-Ap shall satisfy .

Table 6.10.5.3-1: CSI reference signal subframe configuration

| CSI-RS-SubframeConfig | CSI-RS periodicity (subframes) | CSI-RS subframe offset (subframes) |
| --- | --- | --- |
| 0 – 4 | 5 |  |
| 5 – 14 | 10 |  |
| 15 – 34 | 20 |  |
| 35 – 74 | 40 |  |
| 75 – 154 | 80 |  |

## 6.11	Synchronization signals

There are 504 unique physical-layer cell identities. The physical-layer cell identities are grouped into 168 unique physical-layer cell-identity groups, each group containing three unique identities. The grouping is such that each physical-layer cell identity is part of one and only one physical-layer cell-identity group. A physical-layer cell identity is thus uniquely defined by a numberin the range of 0 to 167, representing the physical-layer cell-identity group, and a number in the range of 0 to 2, representing the physical-layer identity within the physical-layer cell-identity group.

### 6.11.1	Primary synchronization signal (PSS)

#### 6.11.1.1	Sequence generation

The sequence  used for the primary synchronization signal is generated from a frequency-domain Zadoff-Chu sequence according to

where the Zadoff-Chu root sequence index  is given by Table 6.11.1.1-1.

Table 6.11.1.1-1: Root indices for the primary synchronization signal

|  | Root index |
| --- | --- |
| 0 | 25 |
| 1 | 29 |
| 2 | 34 |

#### 6.11.1.2	Mapping to resource elements

The mapping of the sequence to resource elements depends on the frame structure. The UE shall not assume that the primary synchronization signal is transmitted on the same antenna port as any of the downlink reference signals. The UE shall not assume that any transmission instance of the primary synchronization signal is transmitted on the same antenna port, or ports, used for any other transmission instance of the primary synchronization signal.

The sequence  shall be mapped to the resource elements according to

For frame structure type 1, the primary synchronization signal shall be mapped to the last OFDM symbol in slots 0 and 10.

For frame structure type 2, the primary synchronization signal shall be mapped to the third OFDM symbol in subframes 1 and 6. Resource elements  in the OFDM symbols used for transmission of the primary synchronization signal where

are reserved and not used for transmission of the primary synchronization signal.

For frame structure type 3, the primary synchronization signal shall be mapped according to frame structure type 1 with the following exceptions:

-	the primary synchronization signal shall be transmitted only if the corresponding subframe is non-empty and at least 12 OFDM symbols are transmitted,

-	a primary synchronization signal being part of a discovery signal shall be transmitted in the last OFDM symbol of the first slot of a discovery signal occasion.

### 6.11.1MF1	MF Primary Synchronization Signal (MF-PSS)

#### 6.11.1MF1.1	Sequence generation

The MF-PSS sequence is generated similarly to as defined in Section 6.11.1.1, where the Zadoff-Chu root sequence index is different and described in Table 6.11.1A.1-2.

Table 6.11.1A.1-2: Root indices for the enhanced primary synchronization signal

|  | MF-PSS Root Index (u) |
| --- | --- |
| 0 | 40 |
| 1 | 44 |
| 2 | 59 |

#### 6.11.1MF1.2	Mapping to resource elements

The mapping of the sequence to resource elements follows mapping of primary synchronization signal for frame structure type 3 as defined in 6.11.1.2 with the following modifications and exceptions:

- MF-PSS shall be mapped to the fourth OFDM symbol in

  - a subframe containing DRS

  - subframe 0 whenever PDSCH or PDCCH is transmitted on this subframe.

- MF-PSS is not transmitted on subframe 5 when not part of a DRS.

MF-PSS is transmitted using the same antenna port as the primary synchronization signal.

### 6.11.2	Secondary synchronization signal (SSS)

#### 6.11.2.1	Sequence generation

The sequence used for the second synchronization signal is an interleaved concatenation of two length-31 binary sequences. The concatenated sequence is scrambled with a scrambling sequence given by the primary synchronization signal.

The combination of two length-31 sequences defining the secondary synchronization signal differs between subframes according to

where . The indices  and  are derived from the physical-layer cell-identity group  according to

where the output of the above expression is listed in Table 6.11.2.1-1.

The two sequences  and  are defined as two different cyclic shifts of the m-sequence  according to

where, , is defined by

with initial conditions.

The two scrambling sequences  and  depend on the primary synchronization signal and are defined by two different cyclic shifts of the m-sequence  according to

where  is the physical-layer identity within the physical-layer cell identity group  and , , is defined by

with initial conditions .

The scrambling sequences  and  are defined by a cyclic shift of the m-sequence  according to

where  and  are obtained from Table 6.11.2.1-1 and  , , is defined by

with initial conditions .

Table 6.11.2.1-1: Mapping between physical-layer cell-identity group  and the indices  and

| 0 | 0 | 1 | 34 | 4 | 6 | 68 | 9 | 12 | 102 | 15 | 19 | 136 | 22 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 35 | 5 | 7 | 69 | 10 | 13 | 103 | 16 | 20 | 137 | 23 | 28 |
| 2 | 2 | 3 | 36 | 6 | 8 | 70 | 11 | 14 | 104 | 17 | 21 | 138 | 24 | 29 |
| 3 | 3 | 4 | 37 | 7 | 9 | 71 | 12 | 15 | 105 | 18 | 22 | 139 | 25 | 30 |
| 4 | 4 | 5 | 38 | 8 | 10 | 72 | 13 | 16 | 106 | 19 | 23 | 140 | 0 | 6 |
| 5 | 5 | 6 | 39 | 9 | 11 | 73 | 14 | 17 | 107 | 20 | 24 | 141 | 1 | 7 |
| 6 | 6 | 7 | 40 | 10 | 12 | 74 | 15 | 18 | 108 | 21 | 25 | 142 | 2 | 8 |
| 7 | 7 | 8 | 41 | 11 | 13 | 75 | 16 | 19 | 109 | 22 | 26 | 143 | 3 | 9 |
| 8 | 8 | 9 | 42 | 12 | 14 | 76 | 17 | 20 | 110 | 23 | 27 | 144 | 4 | 10 |
| 9 | 9 | 10 | 43 | 13 | 15 | 77 | 18 | 21 | 111 | 24 | 28 | 145 | 5 | 11 |
| 10 | 10 | 11 | 44 | 14 | 16 | 78 | 19 | 22 | 112 | 25 | 29 | 146 | 6 | 12 |
| 11 | 11 | 12 | 45 | 15 | 17 | 79 | 20 | 23 | 113 | 26 | 30 | 147 | 7 | 13 |
| 12 | 12 | 13 | 46 | 16 | 18 | 80 | 21 | 24 | 114 | 0 | 5 | 148 | 8 | 14 |
| 13 | 13 | 14 | 47 | 17 | 19 | 81 | 22 | 25 | 115 | 1 | 6 | 149 | 9 | 15 |
| 14 | 14 | 15 | 48 | 18 | 20 | 82 | 23 | 26 | 116 | 2 | 7 | 150 | 10 | 16 |
| 15 | 15 | 16 | 49 | 19 | 21 | 83 | 24 | 27 | 117 | 3 | 8 | 151 | 11 | 17 |
| 16 | 16 | 17 | 50 | 20 | 22 | 84 | 25 | 28 | 118 | 4 | 9 | 152 | 12 | 18 |
| 17 | 17 | 18 | 51 | 21 | 23 | 85 | 26 | 29 | 119 | 5 | 10 | 153 | 13 | 19 |
| 18 | 18 | 19 | 52 | 22 | 24 | 86 | 27 | 30 | 120 | 6 | 11 | 154 | 14 | 20 |
| 19 | 19 | 20 | 53 | 23 | 25 | 87 | 0 | 4 | 121 | 7 | 12 | 155 | 15 | 21 |
| 20 | 20 | 21 | 54 | 24 | 26 | 88 | 1 | 5 | 122 | 8 | 13 | 156 | 16 | 22 |
| 21 | 21 | 22 | 55 | 25 | 27 | 89 | 2 | 6 | 123 | 9 | 14 | 157 | 17 | 23 |
| 22 | 22 | 23 | 56 | 26 | 28 | 90 | 3 | 7 | 124 | 10 | 15 | 158 | 18 | 24 |
| 23 | 23 | 24 | 57 | 27 | 29 | 91 | 4 | 8 | 125 | 11 | 16 | 159 | 19 | 25 |
| 24 | 24 | 25 | 58 | 28 | 30 | 92 | 5 | 9 | 126 | 12 | 17 | 160 | 20 | 26 |
| 25 | 25 | 26 | 59 | 0 | 3 | 93 | 6 | 10 | 127 | 13 | 18 | 161 | 21 | 27 |
| 26 | 26 | 27 | 60 | 1 | 4 | 94 | 7 | 11 | 128 | 14 | 19 | 162 | 22 | 28 |
| 27 | 27 | 28 | 61 | 2 | 5 | 95 | 8 | 12 | 129 | 15 | 20 | 163 | 23 | 29 |
| 28 | 28 | 29 | 62 | 3 | 6 | 96 | 9 | 13 | 130 | 16 | 21 | 164 | 24 | 30 |
| 29 | 29 | 30 | 63 | 4 | 7 | 97 | 10 | 14 | 131 | 17 | 22 | 165 | 0 | 7 |
| 30 | 0 | 2 | 64 | 5 | 8 | 98 | 11 | 15 | 132 | 18 | 23 | 166 | 1 | 8 |
| 31 | 1 | 3 | 65 | 6 | 9 | 99 | 12 | 16 | 133 | 19 | 24 | 167 | 2 | 9 |
| 32 | 2 | 4 | 66 | 7 | 10 | 100 | 13 | 17 | 134 | 20 | 25 | - | - | - |
| 33 | 3 | 5 | 67 | 8 | 11 | 101 | 14 | 18 | 135 | 21 | 26 | - | - | - |

#### 6.11.2.2	Mapping to resource elements

The mapping of the sequence to resource elements depends on the frame structure. In a subframe for frame structure type 1 and 3 and in a half-frame for frame structure type 2, the same antenna port as for the primary synchronization signal shall be used for the secondary synchronization signal.

The sequence  shall be mapped to resource elements according to

Resource elements  where

are reserved and not used for transmission of the secondary synchronization signal.

### 6.11.2MF1	MF secondary synchronization signal (MF-SSS)

#### 6.11.2MF1.1	Sequence generation

The MF-SSS sequence generation is identical to the SSS sequence generation described in Section 6.11.2.1.

#### 6.11.2MF1.2	Mapping to resource elements

The mapping of the sequence to resource elements follows secondary synchronization signal mapping for frame structure type 3, as defined in Section 6.11.2.2, with the following modifications and exceptions:

- MF-SSS shall be mapped to the third OFDM symbol in

  - a subframe containing DRS

  - subframe 0 whenever PDSCH or PDCCH is transmitted on this subframe.

- MF-SSS is not transmitted on subframe 5 when not part of a DRS.

MF-SSS is transmitted using the same antenna port as the secondary synchronization signal.

## 6.11A	Discovery signal

A discovery signal occasion for a cell consists of a period with a duration of

-	one to five consecutive subframes for frame structure type 1

-	two to five consecutive subframes for frame structure type 2

-	12 OFDM symbols within one non-empty subframe for frame structure type 3

-	additionally, for MF, the 14 OFDM symbols within one non-empty subframe in frame structure type 3

where the UE in the downlink subframes may assume presence of a discovery signal consisting of

-	cell-specific reference signals on antenna port 0 in all downlink subframes and in DwPTS of all special subframes in the period for frame structure type 1 and 2

-	cell specific reference signals on antenna port 0 when higher layer parameters indicate only one configured antenna port for cell specific reference signals for a serving cell using frame structure type 3

-	cell specific reference signals on antenna port 0 and antenna port 1 when higher layer parameters indicate at least two configured antenna ports for cell specific reference signals for a serving cell using frame structure type 3

- 	cell specific reference signals on antenna port 0 and antenna port 1 when higher layer configured parameter presenceAntennaPort1 is signalled to be 1, for a neighbour cell when using frame structure type 3

-	primary synchronization signal in the first subframe of the period for frame structure types 1 and 3 or the second subframe of the period for frame structure type 2,

-	secondary synchronization signal in the first subframe of the period, and

-	non-zero-power CSI reference signals in zero or more subframes in the period. The configuration of non-zero-power CSI reference signals part of the discovery signal is obtained as described in clause 6.10.5.2

- 	For MF: MF primary and secondary synchronization signals, MF physical broadcast channel, and broadcast PDSCH and associated PDCCH in the first subframe of the period.

For frame structures 1 and 2 the UE may assume a discovery signal occasion once every dmtc-Periodicity.

For frame structure type 3,

- the UE may assume a discovery signal occasion may occur in any subframe within the discovery signals measurement timing configuration in clause 5.5.2.10 of MFA TS 36.331 [9].

- in MF, the UE may assume a discovery signal occasion may occur in any subframe within the serving cell discovery signals measurement timing configuration in Section 5.5.2.10 of MFA TS 36.331 [9].

For frame structure type 3, simultaneous transmission of a discovery signal and PDSCH/PDCCH/EPDCCH may occur in subframes 0 and 5 only.

For frame structure type 3,

-	the UE may assume that a discovery signal occasion occurs in the first subframe containing a primary synchronization signal, secondary synchronization signal and cell-specific reference signals within the discovery measurement timing configuration in clause 5.5.2.10 of MFA TS 36.331 [9].

-	in MF, the UE may assume that a discovery signal occasion occurs in the first subframe containing a primary synchronization signal, secondary synchronization signal and cell-specific reference signals within the serving cell discovery measurement timing configuration in Section 5.5.2.10 of MFA TS 36.331 [9].

For MF, UE shall assume twelve OFDM symbols in the subframe carrying DRS unless subframe length is known based on common PDCCH signalling as defined in Section 13A of MFA TS 36.213 [4].

## 6.12	OFDM baseband signal generation

The time-continuous signal  on antenna port  in OFDM symbol  in a downlink slot is defined by

for    where  and. The variable  equals 2048 for  subcarrier spacing and 4096 for  subcarrier spacing.

The OFDM symbols in a slot shall be transmitted in increasing order of , starting with , where OFDM symbol starts at time   within the slot. In case the first OFDM symbol(s) in a slot use normal cyclic prefix and the remaining OFDM symbols use extended cyclic prefix, the starting position the OFDM symbols with extended cyclic prefix shall be identical to those in a slot where all OFDM symbols use extended cyclic prefix. Thus there will be a part of the time slot between the two cyclic prefix regions where the transmitted signal is not specified.

Table 6.12-1 lists the value of that shall be used. Note that different OFDM symbols within a slot in some cases have different cyclic prefix lengths.

In case NB-IoT is supported, the OFDM baseband signal generation is defined in subclause 10.2.8.

Table 6.12-1: OFDM parameters

| Configuration |  | Cyclic prefix length |
| --- | --- | --- |
| Normal cyclic prefix |  |  |
| Extended cyclic prefix |  |  |

## 6.13	Modulation and upconversion

Modulation and upconversion to the carrier frequency of the complex-valued OFDM baseband signal for each antenna port is shown in Figure 6.13-1. The filtering required prior to transmission is defined by the requirements in MFA TS 36.104 [6].

Figure 6.13-1: Downlink modulation

# 7	Generic functions

## 7.1	Modulation mapper

The modulation mapper takes binary digits, 0 or 1, as input and produces complex-valued modulation symbols, x=I+jQ, as output.

### 7.1.1	BPSK

In case of BPSK modulation, a single bit, , is mapped to a complex-valued modulation symbol x=I+jQ according to Table 7.1.1-1.

Table 7.1.1-1: BPSK modulation mapping

|  | I | Q |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |

### 7.1.2	QPSK

In case of QPSK modulation, pairs of bits, , are mapped to complex-valued modulation symbols x=I+jQ according to Table 7.1.2-1.

Table 7.1.2-1: QPSK modulation mapping

|  | I | Q |
| --- | --- | --- |
| 00 |  |  |
| 01 |  |  |
| 10 |  |  |
| 11 |  |  |

### 7.1.3	16QAM

In case of 16QAM modulation, quadruplets of bits, , are mapped to complex-valued modulation symbols x=I+jQ according to Table 7.1.3-1.

Table 7.1.3-1: 16QAM modulation mapping

|  | I | Q |
| --- | --- | --- |
| 0000 |  |  |
| 0001 |  |  |
| 0010 |  |  |
| 0011 |  |  |
| 0100 |  |  |
| 0101 |  |  |
| 0110 |  |  |
| 0111 |  |  |
| 1000 |  |  |
| 1001 |  |  |
| 1010 |  |  |
| 1011 |  |  |
| 1100 |  |  |
| 1101 |  |  |
| 1110 |  |  |
| 1111 |  |  |

### 7.1.4	64QAM

In case of 64QAM modulation, hextuplets of bits, , are mapped to complex-valued modulation symbols x=I+jQ according to Table 7.1.4-1.

Table 7.1.4-1: 64QAM modulation mapping

|  | I | Q |  | I | Q |
| --- | --- | --- | --- | --- | --- |
| 000000 |  |  | 100000 |  |  |
| 000001 |  |  | 100001 |  |  |
| 000010 |  |  | 100010 |  |  |
| 000011 |  |  | 100011 |  |  |
| 000100 |  |  | 100100 |  |  |
| 000101 |  |  | 100101 |  |  |
| 000110 |  |  | 100110 |  |  |
| 000111 |  |  | 100111 |  |  |
| 001000 |  |  | 101000 |  |  |
| 001001 |  |  | 101001 |  |  |
| 001010 |  |  | 101010 |  |  |
| 001011 |  |  | 101011 |  |  |
| 001100 |  |  | 101100 |  |  |
| 001101 |  |  | 101101 |  |  |
| 001110 |  |  | 101110 |  |  |
| 001111 |  |  | 101111 |  |  |
| 010000 |  |  | 110000 |  |  |
| 010001 |  |  | 110001 |  |  |
| 010010 |  |  | 110010 |  |  |
| 010011 |  |  | 110011 |  |  |
| 010100 |  |  | 110100 |  |  |
| 010101 |  |  | 110101 |  |  |
| 010110 |  |  | 110110 |  |  |
| 010111 |  |  | 110111 |  |  |
| 011000 |  |  | 111000 |  |  |
| 011001 |  |  | 111001 |  |  |
| 011010 |  |  | 111010 |  |  |
| 011011 |  |  | 111011 |  |  |
| 011100 |  |  | 111100 |  |  |
| 011101 |  |  | 111101 |  |  |
| 011110 |  |  | 111110 |  |  |
| 011111 |  |  | 111111 |  |  |

### 7.1.5	256QAM

In case of 256QAM modulation, octuplets of bits, , are mapped to complex-valued modulation symbols  according to Table 7.1.5-1.

Table 7.1.5-1: 256QAM modulation mapping

|  | I | Q |  | I | Q |  | I | Q |  | I | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00000000 | 5 | 5 | 01000000 | 5 | -5 | 10000000 | -5 | 5 | 11000000 | -5 | -5 |
| 00000010 | 7 | 5 | 01000010 | 7 | -5 | 10000010 | -7 | 5 | 11000010 | -7 | -5 |
| 00000011 | 7 | 7 | 01000011 | 7 | -7 | 10000011 | -7 | 7 | 11000011 | -7 | -7 |
| 00000100 | 5 | 3 | 01000100 | 5 | -3 | 10000100 | -5 | 3 | 11000100 | -5 | -3 |
| 00000101 | 5 | 1 | 01000101 | 5 | -1 | 10000101 | -5 | 1 | 11000101 | -5 | -1 |
| 00000110 | 7 | 3 | 01000110 | 7 | -3 | 10000110 | -7 | 3 | 11000110 | -7 | -3 |
| 00000111 | 7 | 1 | 01000111 | 7 | -1 | 10000111 | -7 | 1 | 11000111 | -7 | -1 |
| 00001000 | 3 | 5 | 01001000 | 3 | -5 | 10001000 | -3 | 5 | 11001000 | -3 | -5 |
| 00001001 | 3 | 7 | 01001001 | 3 | -7 | 10001001 | -3 | 7 | 11001001 | -3 | -7 |
| 00001010 | 1 | 5 | 01001010 | 1 | -5 | 10001010 | -1 | 5 | 11001010 | -1 | -5 |
| 00001011 | 1 | 7 | 01001011 | 1 | -7 | 10001011 | -1 | 7 | 11001011 | -1 | -7 |
| 00001100 | 3 | 3 | 01001100 | 3 | -3 | 10001100 | -3 | 3 | 11001100 | -3 | -3 |
| 00001101 | 3 | 1 | 01001101 | 3 | -1 | 10001101 | -3 | 1 | 11001101 | -3 | -1 |
| 00001110 | 1 | 3 | 01001110 | 1 | -3 | 10001110 | -1 | 3 | 11001110 | -1 | -3 |
| 00001111 | 1 | 1 | 01001111 | 1 | -1 | 10001111 | -1 | 1 | 11001111 | -1 | -1 |
| 00010000 | 5 | 11 | 01010000 | 5 | -11 | 10010000 | -5 | 11 | 11010000 | -5 | -11 |
| 00010001 | 5 | 9 | 01010001 | 5 | -9 | 10010001 | -5 | 9 | 11010001 | -5 | -9 |
| 00010010 | 7 | 11 | 01010010 | 7 | -11 | 10010010 | -7 | 11 | 11010010 | -7 | -11 |
| 00010011 | 7 | 9 | 01010011 | 7 | -9 | 10010011 | -7 | 9 | 11010011 | -7 | -9 |
| 00010100 | 5 | 13 | 01010100 | 5 | -13 | 10010100 | -5 | 13 | 11010100 | -5 | -13 |
| 00010101 | 5 | 15 | 01010101 | 5 | -15 | 10010101 | -5 | 15 | 11010101 | -5 | -15 |
| 00010110 | 7 | 13 | 01010110 | 7 | -13 | 10010110 | -7 | 13 | 11010110 | -7 | -13 |
| 00010111 | 7 | 15 | 01010111 | 7 | -15 | 10010111 | -7 | 15 | 11010111 | -7 | -15 |
| 00011000 | 3 | 11 | 01011000 | 3 | -11 | 10011000 | -3 | 11 | 11011000 | -3 | -11 |
| 00011001 | 3 | 9 | 01011001 | 3 | -9 | 10011001 | -3 | 9 | 11011001 | -3 | -9 |
| 00011010 | 1 | 11 | 01011010 | 1 | -11 | 10011010 | -1 | 11 | 11011010 | -1 | -11 |
| 00011011 | 1 | 9 | 01011011 | 1 | -9 | 10011011 | -1 | 9 | 11011011 | -1 | -9 |
| 00011100 | 3 | 13 | 01011100 | 3 | -13 | 10011100 | -3 | 13 | 11011100 | -3 | -13 |
| 00011101 | 3 | 15 | 01011101 | 3 | -15 | 10011101 | -3 | 15 | 11011101 | -3 | -15 |
| 00011110 | 1 | 13 | 01011110 | 1 | -13 | 10011110 | -1 | 13 | 11011110 | -1 | -13 |
| 00011111 | 1 | 15 | 01011111 | 1 | -15 | 10011111 | -1 | 15 | 11011111 | -1 | -15 |
| 00100000 | 11 | 5 | 01100000 | 11 | -5 | 10100000 | -11 | 5 | 11100000 | -11 | -5 |
| 00100001 | 11 | 7 | 01100001 | 11 | -7 | 10100001 | -11 | 7 | 11100001 | -11 | -7 |
| 00100010 | 9 | 5 | 01100010 | 9 | -5 | 10100010 | -9 | 5 | 11100010 | -9 | -5 |
| 00100011 | 9 | 7 | 01100011 | 9 | -7 | 10100011 | -9 | 7 | 11100011 | -9 | -7 |
| 00100100 | 11 | 3 | 01100100 | 11 | -3 | 10100100 | -11 | 3 | 11100100 | -11 | -3 |
| 00100101 | 11 | 1 | 01100101 | 11 | -1 | 10100101 | -11 | 1 | 11100101 | -11 | -1 |
| 00100110 | 9 | 3 | 01100110 | 9 | -3 | 10100110 | -9 | 3 | 11100110 | -9 | -3 |
| 00100111 | 9 | 1 | 01100111 | 9 | -1 | 10100111 | -9 | 1 | 11100111 | -9 | -1 |
| 00101000 | 13 | 5 | 01101000 | 13 | -5 | 10101000 | -13 | 5 | 11101000 | -13 | -5 |
| 00101001 | 13 | 7 | 01101001 | 13 | -7 | 10101001 | -13 | 7 | 11101001 | -13 | -7 |
| 00101010 | 15 | 5 | 01101010 | 15 | -5 | 10101010 | -15 | 5 | 11101010 | -15 | -5 |
| 00101011 | 15 | 7 | 01101011 | 15 | -7 | 10101011 | -15 | 7 | 11101011 | -15 | -7 |
| 00101100 | 13 | 3 | 01101100 | 13 | -3 | 10101100 | -13 | 3 | 11101100 | -13 | -3 |
| 00101101 | 13 | 1 | 01101101 | 13 | -1 | 10101101 | -13 | 1 | 11101101 | -13 | -1 |
| 00101110 | 15 | 3 | 01101110 | 15 | -3 | 10101110 | -15 | 3 | 11101110 | -15 | -3 |
| 00101111 | 15 | 1 | 01101111 | 15 | -1 | 10101111 | -15 | 1 | 11101111 | -15 | -1 |
| 00110000 | 11 | 11 | 01110000 | 11 | -11 | 10110000 | -11 | 11 | 11110000 | -11 | -11 |
| 00110001 | 11 | 9 | 01110001 | 11 | -9 | 10110001 | -11 | 9 | 11110001 | -11 | -9 |
| 00110010 | 9 | 11 | 01110010 | 9 | -11 | 10110010 | -9 | 11 | 11110010 | -9 | -11 |
| 00110011 | 9 | 9 | 01110011 | 9 | -9 | 10110011 | -9 | 9 | 11110011 | -9 | -9 |
| 00110100 | 11 | 13 | 01110100 | 11 | -13 | 10110100 | -11 | 13 | 11110100 | -11 | -13 |
| 00110101 | 11 | 15 | 01110101 | 11 | -15 | 10110101 | -11 | 15 | 11110101 | -11 | -15 |
| 00110110 | 9 | 13 | 01110110 | 9 | -13 | 10110110 | -9 | 13 | 11110110 | -9 | -13 |
| 00110111 | 9 | 15 | 01110111 | 9 | -15 | 10110111 | -9 | 15 | 11110111 | -9 | -15 |
| 00111000 | 13 | 11 | 01111000 | 13 | -11 | 10111000 | -13 | 11 | 11111000 | -13 | -11 |
| 00111001 | 13 | 9 | 01111001 | 13 | -9 | 10111001 | -13 | 9 | 11111001 | -13 | -9 |
| 00111010 | 15 | 11 | 01111010 | 15 | -11 | 10111010 | -15 | 11 | 11111010 | -15 | -11 |
| 00111011 | 15 | 9 | 01111011 | 15 | -9 | 10111011 | -15 | 9 | 11111011 | -15 | -9 |
| 00111100 | 13 | 13 | 01111100 | 13 | -13 | 10111100 | -13 | 13 | 11111100 | -13 | -13 |
| 00111101 | 13 | 15 | 01111101 | 13 | -15 | 10111101 | -13 | 15 | 11111101 | -13 | -15 |
| 00111110 | 15 | 13 | 01111110 | 15 | -13 | 10111110 | -15 | 13 | 11111110 | -15 | -13 |
| 00111111 | 15 | 15 | 01111111 | 15 | -15 | 10111111 | -15 | 15 | 11111111 | -15 | -15 |

## 7.2	Pseudo-random sequence generation

Pseudo-random sequences are defined by a length-31 Gold sequence. The output sequence  of length, where, is defined by

where  and the first m-sequence shall be initialized with. The initialization of the second m-sequence is denoted by  with the value depending on the application of the sequence.

# 8	Timing

## 8.1	Uplink-downlink frame timing

Transmission of the uplink radio frame number  from the UE shall start  seconds before the start of the corresponding downlink radio frame at the UE, where  if the UE is configured with a SCG and  otherwise. For frame structure type 1  and for frame structure type 2  unless stated otherwise in MFA TS 36.213 [4] and for a MF cell

is derived from mf-CommonTimingAdvanceOffset configured by higher layer using Table 8.1-1

| mf-CommonTimingAdvanceOffset | Value |
| --- | --- |
| 0 | 0 (default) |
| 1 | 1096 |
| 2 | 1424 |
| 3 | 1700 |
| 4 | 2192 |
| 5 | 3288 |
| 6 | 3616 |
| 7 | 3892 |
| 8 | 4384 |
| 9 | 5480 |
| 10 | 5808 |
| 11 | 6084 |
| 12 | 6576 |
| 13 | 7672 |
| 14 | spare |
| 15 | spare |

Note that not all slots in a radio frame may be transmitted. One example hereof is TDD, where only a subset of the slots in a radio frame is transmitted.

Figure 8.1-1: Uplink-downlink timing relation

Table 8.1-1

| mf-CommonTimingAdvanceOffset | Value |
| --- | --- |
| 0 | 0 (default) |
| 1 | 1096 |
| 2 | 1424 |
| 3 | 1700 |
| 4 | 2192 |
| 5 | 3288 |
| 6 | 3616 |
| 7 | 3892 |
| 8 | 4384 |
| 9 | 5480 |
| 10 | 5808 |
| 11 | 6084 |
| 12 | 6576 |
| 13 | 7672 |
| 14 | spare |
| 15 | spare |

# 9	Sidelink

## 9.1	Overview

A sidelink is used for ProSe direct communication and ProSe direct discovery between UEs.

### 9.1.1	Physical channels

A sidelink physical channel corresponds to a set of resource elements carrying information originating from higher layers and is the interface defined between MFA TS 36.212 [3] and this present document MFA TS 36.211. The following sidelink physical channels are defined:

-	Physical Sidelink Shared Channel, PSSCH

-	Physical Sidelink Control Channel, PSCCH

-	Physical Sidelink Discovery Channel, PSDCH

-	Physical Sidelink Broadcast Channel, PSBCH

Generation of the baseband signal representing the different physical sidelink channels is illustrated in Figure 5.3-1.

### 9.1.2	Physical signals

A sidelink physical signal is used by the physical layer but does not carry information originating from higher layers. The following sidelink physical signals are defined:

-	Demodulation reference signal

-	Synchronization signal

### 9.1.3	Handling of simultaneous sidelink and uplink/downlink transmissions

For a given frequency, on an uplink subframe included in discTxGapConfig MFA TS 36.331 [9], a UE shall not transmit an uplink transmission that is not a PRACH transmission and that is partly or completely overlapping in time with a PSDCH transmission or a SLSS transmission for PSDCH by the same UE. Else, for a given carrier frequency, a UE shall not transmit a sidelink signal or channel overlapping partly or completely in time with an uplink transmission from the same UE.

For a given carrier frequency, no PSDCH, PSCCH, or PSSCH transmission shall occur from a UE in a sidelink subframe configured for synchronization purposes by the higher-layer parameters

-	syncOffsetIndicator1 or syncOffsetIndicator2 in MFA TS 36.331 [9] if the UE has no serving cell fulfilling the S criterion according to Section 5.2.3.2 in MFA TS 36.304 [10], or

-	syncOffsetIndicator in commSyncConfig or discSyncConfig which includes txParameters in MFA TS 36.331 [9] if the UE has a serving cell fulfilling the S criterion according to Section 5.2.3.2 in MFA TS 36.304 [10]. The UE may assume the same configuration in commSyncConfig and discSyncConfig.

For a given carrier frequency, with the exception of PSSCH transmissions with transmission mode 1 and same sidelink cyclic prefix as PUSCH, no sidelink transmissions shall occur in sidelink subframe  from a UE if uplink SRS is transmitted from the same UE in uplink subframe .

In case of a UE capable of transmission on multiple carriers, sidelink transmission may only occur on a single carrier frequency at a given time.

A UE with limited transmission capabilities, on an uplink subframe included in discTxGapConfig (see MFA TS 36.331 [9]), shall first prioritize a PSDCH transmission or a SLSS transmission for PSDCH over an uplink transmission that is not a PRACH transmission. Else, a UE with limited transmission capabilities shall at a given time first prioritize uplink transmissions, followed by sidelink transmissions.

A UE with limited transmission capabilities shall at a given time prioritize sidelink communication transmissions (PSSS, SSSS, PSBCH, PSSCH, PSCCH) over sidelink discovery transmissions (PSDCH).

A UE with limited reception capabilities, on a downlink subframe included in discRxGapConfig (see MFA TS 36.331 [9]), shall first prioritize reception of PSDCH or reception of SLSS for PSDCH over downlink reception. Else, a UE with limited reception capabilities shall at a given time first prioritize downlink reception over sidelink reception.

A UE with limited reception capabilities shall at a given time first prioritize sidelink communication reception, sidelink discovery reception on carriers configured by the eNodeB, and last sidelink discovery reception on carriers not configured by the eNodeB.

## 9.2	Slot structure and physical resources

Sidelink transmissions are organized into radio frames with a duration of , each consisting of 20 slots of duration . A sidelink subframe consists of two consecutive slots, starting with an even-numbered slot.

### 9.2.1	Resource grid

A transmitted physical channel or signal in a slot is described by a resource grid of  subcarriers and  SC-FDMA symbols. The sidelink bandwidth  if the S criterion according to Section 5.2.3.2 of MFA TS 36.304 [10] is fulfilled for a serving cell having the same uplink carrier frequency as the sidelink, otherwise a preconfigured value is used (see MFA TS 36.331 [9]).

The sidelink cyclic prefix is configured independently for type 1 discovery, type 2B discovery, sidelink transmission mode 1, sidelink transmission mode 2, control signalling, and PSBCH and synchronization signals. Configuration is per resource pool for discovery, sidelink transmission mode 2, and control signalling. The PSBCH and synchronization signals always use the same cyclic prefix.

Only normal cyclic prefix is supported for PSSCH, PSCCH, PSBCH, and synchronization signals for a sidelink configured with transmission mode 3 or 4.

The resource grid is illustrated in Figure 5.2.1-1.

An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. There is one resource grid per antenna port. The antenna ports used for transmission of a physical channel or signal are shown in Table 9.2.1-1.

Table 9.2.1-1: Antenna ports used for different physical channels and signals

| Physical channel or signal | Antenna port number |
| --- | --- |
| PSSCH | 1000 |
| PSCCH | 1000 |
| PSDCH | 1000 |
| PSBCH | 1010 |
| Synchronization signals | 1020 |

### 9.2.2	Resource elements

Each element in the resource grid is called a resource element and is uniquely defined by the index pair  in a slot where  and  are the indices in the frequency and time domains, respectively. Resource element  on antenna port  corresponds to the complex value . When there is no risk for confusion, or no particular antenna port is specified, the index  may be dropped.

Quantities  corresponding to resource elements not used for transmission of a physical channel or a physical signal in a slot shall be set to zero.

### 9.2.3	Resource blocks

A physical resource block is defined as consecutive SC-FDMA symbols in the time domain and consecutive subcarriers in the frequency domain, where  and  are given by Table 9.2.3-1. A physical resource block in the sidelink thus consists of  resource elements, corresponding to one slot in the time domain and 180 kHz in the frequency domain.

Table 9.2.3-1: Resource block parameters

| Configuration |  |  |
| --- | --- | --- |
| Normal cyclic prefix | 12 | 7 |
| Extended cyclic prefix | 12 | 6 |

The relation between the physical resource block number  in the frequency domain and resource elements  in a slot is given by

### 9.2.4	Resource pool

The subframe pools and resource block pools are defined in MFA TS 36.213 [4].

For PSSCH, the number of the current slot in the subframe pool , where  is the number of the current slot within the current sidelink subframe  with  equal to the subscript of , defined in clauses 14.1.4, 14.2.3, 14.1.1.4A, and 14.1.1.4B of MFA TS 36.213 [4] for sidelink transmission modes 1, 2, 3, and 4, respectively

### 9.2.5	Guard period

The last SC-FDMA symbol in a sidelink subframe serves as a guard period and shall not be used for sidelink transmission.

## 9.3	Physical Sidelink Shared Channel

### 9.3.1	Scrambling

The block of bits , where  is the number of bits transmitted on the physical sidelink shared channel in one subframe shall be scrambled according to clause 5.3.1.

The scrambling sequence generator shall be initialised with  at the start of every PSSCH subframe where  is destination identity obtained from the sidelink control channel for transmission modes 1 and 2 and  equals the CRC on the PSCCH for transmission modes 3 and 4.

### 9.3.2	Modulation

Modulation shall be done according to clause 5.3.2. Table 9.3.2-1 specifies the modulation mappings applicable for the physical sidelink shared channel.

Table 9.3.2-1: PSSCH modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PSSCH | QPSK, 16QAM |

### 9.3.3	Layer mapping

Layer mapping shall be done according to clause 5.3.2A assuming a single antenna port, .

### 9.3.4	Transform precoding

Transform precoding shall be done according to clause 5.3.3 with  and  replaced by  and , respectively.

### 9.3.5	Precoding

Precoding shall be done according to clause 5.3.3A assuming a single antenna port, .

### 9.3.6	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in MFA TS 36.213 [4], and mapped in sequence starting with  to physical resource blocks on antenna port  and assigned for transmission of PSSCH. The mapping to resource elements  corresponding to the physical resource blocks assigned for transmission and not used for transmission of reference signals shall be in increasing order of first the index , then the index, starting with the first slot in the subframe. Resource elements in the last SC-FDMA symbol within a subframe shall be counted in the mapping process but not transmitted.

If sidelink frequency hopping is disabled the set of physical resource blocks to be used for transmission is given by  where  is obtained from Section 14.1.1.2.1 in MFA TS 36.213 [4].

If sidelink frequency hopping with type 1 hopping is enabled, the set of physical resource blocks to be used for transmission is given by MFA TS 36.213 [4].

If sidelink frequency hopping with predefined hopping pattern is enabled, the set of physical resource blocks to be used for transmission is given by the sidelink control information together with a predefined pattern in clause 5.3.4 with the following exceptions:

-	only inter-subframe hopping shall be used

-	the number of subbands  is given by higher layers as described in Section 14.1.1.2 in MFA TS 36.213 [4]

-	the quantity  is given by higher layers as described in Section 14.1.1.2 in MFA TS 36.213 [4]

-	the quantity  where  is given by clause 9.2.4

-	the quantity

-	the pseudo-random sequence generator is initialized at the start of each slot fulfilling  with the initialization value   given by hoppingParameter-r12 in MFA TS 36.331 [9]

-	the quantity  shall be replaced by , given by Section 14.1.1.2.1 of MFA TS 36.213 [4]

-	for sidelink transmission mode 1

-

-	for sidelink transmission mode 2

-	 where  is given by Section 14.1.3 of MFA TS 36.213 [4]

-	the quantity  shall be replaced by , given by Section 14.1.1.4 of MFA TS 36.213  [4]

-	the physical resource block to use for transmission  with  given by Section 14.1.3 of MFA TS 36.213 [4]

## 9.4	Physical Sidelink Control Channel

### 9.4.1	Scrambling

The block of bits , where  is the number of bits transmitted on the physical sidelink control channel in one subframe shall be scrambled according to clause 5.3.1.

The scrambling sequence generator shall be initialised with  at the start of every PSCCH subframe.

### 9.4.2	Modulation

Modulation shall be done according to clause 5.3.2. Table 9.4.2-1 specifies the modulation mappings applicable for the physical sidelink control channel.

Table 9.4.2-1: PSCCH modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PSCCH | QPSK |

### 9.4.3	Layer mapping

Layer mapping shall be done according to clause 5.3.2A assuming a single antenna port, .

### 9.4.4	Transform precoding

Transform precoding shall be done according to clause 5.3.3 with  and  replaced by  and , respectively.

### 9.4.5	Precoding

Precoding shall be done according to clause 5.3.3A assuming a single antenna port, .

### 9.4.6	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in MFA TS 36.213 [4], and mapped in sequence starting with  to physical resource blocks on antenna port  and assigned for transmission of PSCCH. The mapping to resource elements  corresponding to the physical resource blocks assigned for transmission and not used for transmission of reference signals shall be in increasing order of first the index , then the index, starting with the first slot in the subframe. Resource elements in the last SC-FDMA symbol within a subframe shall be counted in the mapping process but not transmitted.

## 9.5	Physical Sidelink Discovery Channel

### 9.5.1	Scrambling

The block of bits , where  is the number of bits transmitted on the physical sidelink discovery channel in one subframe shall be scrambled according to clause 5.3.1.

The scrambling sequence generator shall be initialised with  at the start of each PSDCH subframe.

### 9.5.2	Modulation

Modulation shall be done according to clause 5.3.2. Table 9.5.2-1 specifies the modulation mappings applicable for the physical sidelink discovery channel.

Table 9.5.2-1: Sidelink modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PSDCH | QPSK |

### 9.5.3	Layer mapping

Layer mapping shall be done according to clause 5.3.2A assuming a single antenna port, .

### 9.5.4	Transform precoding

Transform precoding shall be done according to clause 5.3.3 with  and  replaced by  and , respectively.

### 9.5.5	Precoding

Precoding shall be done according to clause 5.3.3A assuming a single antenna port, .

### 9.5.6	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in MFA TS 36.213 [4], and mapped in sequence starting with  to physical resource blocks on antenna port  and assigned for transmission of PSDCH. The mapping to resource elements  corresponding to the physical resource blocks assigned for transmission and not used for transmission of reference signals shall be in increasing order of first the index , then the index, starting with the first slot in the subframe. Resource elements in the last SC-FDMA symbol within a subframe shall be counted in the mapping process but not transmitted.

The set of physical resource blocks that shall be used are given by Section 14.3.1 of MFA TS 36.213 [4].

## 9.6	Physical Sidelink Broadcast Channel

### 9.6.1	Scrambling

The block of bits , where  is the number of bits transmitted on the physical sidelink broadcast channel in one subframe, shall be scrambled according to clause 5.3.1. The scrambling sequence generator shall be initialised at the start of every PSBCH subframe with .

### 9.6.2	Modulation

Modulation shall be done according to clause 5.3.2. Table 9.6.2-1 specifies the modulation mappings applicable for the physical sidelink broadcast channel.

Table 9.6.2-1: PSBCH modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| PSBCH | QPSK |

### 9.6.3	Layer mapping

Layer mapping shal be done according to clause 5.3.2A assuming a single antenna port, .

### 9.6.4	Transform precoding

Transform precoding shall be done according to clause 5.3.3 with  and  replaced by  and , respectively.

### 9.6.5	Precoding

Precoding shall be done according to clause 5.3.3A assuming a single antenna port, .

### 9.6.6	Mapping to physical resources

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power  specified in MFA TS 36.213 [4], and mapped in sequence starting with  to physical resource blocks on antenna port . The PSBCH shall use the same set of resource blocks as the synchronization signal. The mapping to resource elements  corresponding to the physical resource blocks used for the PSBCH and not used for transmission of reference signals or synchronization signals shall be in increasing order of first the index , then the index , starting with the first slot in the subframe. The resource-element index  given by

Resource elements in the last SC-FDMA symbol within a subframe should be counted in the mapping process but not transmitted.

## 9.7	Sidelink Synchronization Signals

A physical-layer sidelink synchronization identity is represented by , divided into two sets id_net and id_oon consisting of identities  and , respectively.

### 9.7.1	Primary sidelink synchronization signal

The primary sidelink synchronization signal is transmitted in two adjacent SC-FDMA symbols in the same subframe.

#### 9.7.1.1	Sequence generation

Each of the two sequences  used for the primary sidelink synchronization signal in the two SC-FDMA symbols  is given by clause 6.11.1.1 with root index  if  and  otherwise.

#### 9.7.1.2	Mapping to resource elements

The sequence  shall be multiplied with the amplitude scaling factor  and mapped to resource elements on antenna port 1020 in the first slot in the subframe according to

### 9.7.2	Secondary sidelink synchronization signal

The secondary sidelink synchronization signal is transmitted in two adjacent SC-FDMA symbols in the same subframe.

#### 9.7.2.1	Sequence generation

Each of the two sequences  used for the secondary sidelink synchronization signal is given by clause 6.11.2.1 assuming

-	subframe 0 with  and  for transmission modes 1 and 2, and

-	subframe 5 for transmission modes 3 and 4.

#### 9.7.2.2	Mapping to resource elements

The sequence  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in clause 14.4 in MFA TS 36.213 [4] and mapped to resource elements on antenna port 1020 in the second slot in the subframe according to

## 9.8	Demodulation reference signals

Demodulation reference signals associated with PSSCH, PSCCH, PSDCH, and PSBCH transmission shall be transmitted according to PUSCH in clause 5.5.2.1 with the following exceptions:

-	The parameters in Tables 9.8-1, 9.8-2, and 9.8-3 shall be used.

-	The term PUSCH shall be replaced by PSSCH, PSCCH, PSDCH or PSBCH, depending on the physical channel to which the reference signal is associated.

-	Antenna ports are given by Table 9.2-1.

-	The set of physical resource blocks used in the mapping process shall be identical to the corresponding PSSCH/PSCCH/PSDCH/PSBCH transmission.

-	The index  in the mapping process in clause 5.5.2.1.2 corresponding to the case where higher-layer parameter ul-DMRS-IFDMA is not set shall be identical to that for the corresponding PSSCH/PSCCH/PSDCH/PSBCH transmission.

-	For sidelink transmission modes 3 and 4 on the PSSCH and PSCCH, the mapping shall use  and  for the first slot in the subframe and  and  for the second slot in the subframe.

-	For sidelink transmission modes 3 and 4 on the PSBCH, the mapping shall use  and  for the first slot in the subframe and  for the second slot in the subframe.

-	The pseudo-random sequence generator in clause 5.5.1.3 shall be initialized at the start of each slot fulfilling

-	For sidelink transmission modes 3 and 4 on the PSCCH, the cyclic shift  to be applied for all DM-RS in a subframe shall be chosen according to clause 14.2.1 of MFA TS 36.213 [4].

-	For sidelink transmission modes 1and 2, the quantity  in clause 5.5.2.1.1 takes the values  and for sidelink transmission modes 3and 4, the quantity  in clause 5.5.2.1.1 takes the values .

-	For sidelink transmission modes 3 and 4, the quantity  equals the CRC on the PSCCH.

Table 9.8-1: Reference signal parameters for PSSCH.

| Parameter in clause 5.5.2.1 |  | PSSCH |  |
| --- | --- | --- | --- |
|  |  | Sidelink transmission modes 1 and 2 | Sidelink transmission modes 3 and 4 |
| Group hopping |  | enabled | enabled |
|  |  |  | first DM-RS symbol in a slot second DM-RS symbol in a slot |
| Sequence hopping |  | disabled | disabled |
| Cyclic shift |  |  |  |
| Orthogonal sequence |  |  |  |
| Reference signal length |  |  |  |
| Number of layers |  | 1 | 1 |
| Number of antenna ports |  | 1 | 1 |

Table 9.8-2: Reference signal parameters for PSCCH.

| Parameter in clause 5.5.2.1 |  | PSCCH |  |
| --- | --- | --- | --- |
|  |  | Sidelink transmission modes 1 and 2 | Sidelink transmission modes 3 and 4 |
| Group hopping |  | disabled | disabled |
|  |  | - | - |
|  |  | - | - |
|  |  | 0 | 8 |
| Sequence hopping |  | disabled | disabled |
| Cyclic shift |  | 0 | {0, 3, 6, 9} |
| Orthogonal sequence |  |  |  |
| Reference signal length |  |  |  |
| Number of layers |  | 1 | 1 |
| Number of antenna ports |  | 1 | 1 |

Table 9.8-3: Reference signal parameters for PSDCH and PSBCH.

| Parameter in clause 5.5.2.1 |  | PSDCH | PSBCH |
| --- | --- | --- | --- |
| Group hopping |  | disabled | disabled |
|  |  | 0 |  |
| Sequence hopping |  | disabled | disabled |
| Cyclic shift |  | 0 |  |
| Orthogonal sequence |  |  |  |
| Reference signal length |  |  |  |
| Number of layers |  | 1 | 1 |
| Number of antenna ports |  | 1 | 1 |

## 9.9	SC-FDMA baseband signal generation

The time-continuous signal  for antenna port  in SC-FDMA symbol  in a sidelink slot is defined by clause 5.6 with  replaced by .

The cyclic prefix length for each sidelink channel or signal may differ from that configured for uplink transmissions.

## 9.10	Timing

Transmission of a sidelink radio frame number  from the UE shall start  seconds before the start of the corresponding timing reference frame at the UE. The UE is not required to receive sidelink or downlink transmissions earlier than  after the end of a sidelink transmission.

For PSDCH transmission and sidelink synchronization signal transmission for PSDCH:

if the UE has a serving cell fulfilling the S criterion according to Section 5.2.3.2 of MFA TS 36.304 [10]

-	the timing of reference radio frame  equals that of downlink radio frame  of the cell c as given  in  Section 14.3.1 of MFA TS 36.213 [4] and

-	 is given by clause 8.1,

otherwise

-	the timing of reference radio frame  is implicitly obtained from MFA TS 36.213 [4] and

-	.

For all other sidelink transmissions:

if the UE has a serving cell fulfilling the S criterion according to Section 5.2.3.2 of MFA TS 36.304 [10]

-	the timing of reference radio frame  equals that of downlink radio frame  in the cell with the same uplink carrier frequency as the sidelink and

-	 is given by clause 8.1,

otherwise

-	the timing of reference radio frame  is implicitly obtained from MFA TS 36.213 [4] and

-	.

Figure 9.9-1: Sidelink timing relation.

The quantity  differs between channels and signals according to

# 10	Narrowband IoT

## 10.1	Uplink

### 10.1.1	Overview

#### 10.1.1.1	Physical channels

The following narrowband physical channels are defined:

-	Narrowband Physical Uplink Shared Channel, NPUSCH

-	Narrowband Physical Random Access Channel, NPRACH

#### 10.1.1.2	Physical signals

The following uplink narrowband physical signals are defined:

-	Narrowband demodulation reference signal

### 10.1.2	Slot structure and physical resources

#### 10.1.2.1	Resource grid

A transmitted physical channel or signal in a slot is described by one or several resource grids of  subcarriers and  SC-FDMA symbols. The resource grid is illustrated in Figure 10.1.2.1-1. The slot number within a radio frame is denoted  where  for  and  for .

Figure 10.1.2.1-1: Uplink resource grid for NB-IoT

The uplink bandwidth in terms of subcarriers , and the slot duration  are given in Table 10.1.2.1-1.

Table 10.1.2.1-1: NB-IoT parameters.

| Subcarrier spacing |  |  |
| --- | --- | --- |

A single antenna port  is used for all uplink transmissions.

#### 10.1.2.2	Resource elements

Each element in the resource grid is called a resource element and is uniquely defined by the index pair  in a slot where  and  are the indices in the frequency and time domains, respectively. Resource element  corresponds to the complex value . Quantities  corresponding to resource elements not used for transmission of a physical channel or a physical signal in a slot shall be set to zero.

#### 10.1.2.3	Resource unit

Resource units are used to describe the mapping of the NPUSCH to resource elements. A resource unit is defined as  consecutive SC-FDMA symbols in the time domain and consecutive subcarriers in the frequency domain, where  and  are given by Table 10.1.2.3-1.

Table 10.1.2.3-1: Supported combinations of , , and .

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

### 10.1.3	Narrowband physical uplink shared channel

The narrowband physical uplink shared channel supports two formats:

-	NPUSCH format 1, used to carry the UL-SCH

-	NPUSCH format 2, used to carry uplink control information

#### 10.1.3.1	Scrambling

Scrambling shall be done according to clause 5.3.1. The scrambling sequence generator shall be initialised with  where  is the first slot of the transmission of the codeword. In case of NPUSCH repetitions, the scrambling sequence shall be reinitialised according to the above formula after every  transmissions of the codeword with  and  set to the first slot and the frame, respectively, used for the transmission of the repetition. The quantity  is given by clause 10.1.3.6.

#### 10.1.3.2	Modulation

Modulation shall be done according to clause 5.3.2. Table 10.1.3.2-1 specifies the modulation mappings applicable for the narrowband physical uplink shared channel.

Table 10.1.3.2-1: NPUSCH modulation schemes

| NPUSCH format |  | Modulation scheme |
| --- | --- | --- |
| 1 | 1 | BPSK, QPSK |
|  | >1 | QPSK |
| 2 | 1 | BPSK |

#### 10.1.3.3	Layer mapping

Layer mapping shall be done according to clause 5.3.2A with .

#### 10.1.3.4	Transform precoding

Transform precoding shall be done according to clause 5.3.3 with  and  replaced by .

#### 10.1.3.5	Precoding

Precoding shall be done according to clause 5.3.3A assuming a single antenna port.

#### 10.1.3.6	Mapping to physical resources

NPUSCH can be mapped to one or more than one resource units, , as given by clause 16.5.1.2 of MFA TS 36.213 [4], each of which shall be transmitted  times.

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in MFA TS 36.213 [4], and mapped in sequence starting with  to subcarriers assigned for transmission of NPUSCH. The mapping to resource elements  corresponding to the subcarriers assigned for transmission and not used for transmission of reference signals, shall be in increasing order of first the index , then the index, starting with the first slot in the assigned resource unit.

After mapping to slots, the  slots shall be repeated  additional times, before continuing the mapping of  to the following slot, where

If a mapping to  slots or a repetition of the mapping contains a resource element which overlaps with any configured NPRACH resource according to NPRACH-ConfigSIB-NB, the NPUSCH transmission in overlapped slots is postponed until the next  slots not overlapping with any configured NPRACH resource.

The mapping of  is then repeated until  slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

When higher layer parameter npusch-AllSymbols is set to false, resource elements in SC-FDMA symbols overlapping with a symbol configured with SRS according to srs-SubframeConfig shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH. When higher layer parameter npusch-AllSymbols is set to true, all symbols are transmitted.

### 10.1.4	Demodulation reference signal

#### 10.1.4.1	Reference signal sequence

##### 10.1.4.1.1	Reference signal sequence for

The reference signal sequence for  is defined by

where the binary sequence  is defined by clause 7.2 and shall be initialised with  at the start of the NPUSCH transmission. The quantity  is given by Table 10.1.4.1.1-1 where  for NPUSCH format 2, and for NPUSCH format 1if group hopping is not enabled, and by clause 10.1.4.1.3 if group hopping is enabled for NPUSCH format 1.

Table 10.1.4.1.1-1: Definition of

| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 |
| 2 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 |
| 3 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 |
| 4 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 |
| 5 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| 6 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 |
| 7 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 9 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 |
| 10 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 |
| 11 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 |
| 12 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 |
| 13 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 14 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 |
| 15 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 |

The reference signal sequence for NPUSCH format 1 is given by:

The reference signal sequence for NPUSCH format 2 is given by

where is defined in Table 5.5.2.2.1-2 with the sequence index chosen according to   with  .

##### 10.1.4.1.2	Reference signal sequence for

The reference signal sequences for is defined by a cyclic shift  of a base sequence according to

,

where  is given by Table 10.1.4.1.2-1 for ,  Table 10.1.4.1.2-2 for  and Table 5.5.1.2-1 for .

If group hopping is not enabled, the base sequence index  is given by higher layer parameters threeTone-BaseSequence, sixTone-BaseSequence, and twelveTone-BaseSequence for , , and , respectively. If not signalled by higher layers, the base sequence is given by

If group hopping is enabled, the base sequence index  is given by clause 10.1.4.1.3.

The cyclic shift  for  and  is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 10.1.4.1.2-3. For , .

Table 10.1.4.1.2-1: Definition of  for

| 0 | 1 | -3 | -3 |
| --- | --- | --- | --- |
| 1 | 1 | -3 | -1 |
| 2 | 1 | -3 | 3 |
| 3 | 1 | -1 | -1 |
| 4 | 1 | -1 | 1 |
| 5 | 1 | -1 | 3 |
| 6 | 1 | 1 | -3 |
| 7 | 1 | 1 | -1 |
| 8 | 1 | 1 | 3 |
| 9 | 1 | 3 | -1 |
| 10 | 1 | 3 | 1 |
| 11 | 1 | 3 | 3 |

Table 10.1.4.1.2-2: Definition of  for

| 0 | 1 | 1 | 1 | 1 | 3 | -3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 3 | 1 | -3 | 3 |
| 2 | 1 | -1 | -1 | -1 | 1 | -3 |
| 3 | 1 | -1 | 3 | -3 | -1 | -1 |
| 4 | 1 | 3 | 1 | -1 | -1 | 3 |
| 5 | 1 | -3 | -3 | 1 | 3 | 1 |
| 6 | -1 | -1 | 1 | -3 | -3 | -1 |
| 7 | -1 | -1 | -1 | 3 | -3 | -1 |
| 8 | 3 | -1 | 1 | -3 | -3 | 3 |
| 9 | 3 | -1 | 3 | -3 | -1 | 1 |
| 10 | 3 | -3 | 3 | -1 | 3 | 3 |
| 11 | -3 | 1 | 3 | 1 | -3 | -1 |
| 12 | -3 | 1 | -3 | 3 | -3 | -1 |
| 13 | -3 | 3 | -3 | 1 | 1 | -3 |

Table 10.1.4.1.2-3: Definition of

| threeTone-CyclicShift |  | sixTone-CyclicShift |  |
| --- | --- | --- | --- |
| 0 |  | 0 |  |
| 1 |  | 1 |  |
| 2 |  | 2 |  |
|  |  | 3 |  |

##### 10.1.4.1.3	Group hopping

For the reference signal for NPUSCH format 1, sequence-group hopping can be enabled where the sequence-group number  in slot  is defined by a group hopping pattern  and a sequence-shift pattern  according to

where the number of reference signal sequences available for each resource unit size, is given by Table 10.1.4.1.3-1.

Table 10.1.4.1.3-1: Definition of

| 1 | 16 |
| --- | --- |
| 3 | 12 |
| 6 | 14 |
| 12 | 30 |

Sequence-group hopping can be enabled or disabled by means of the cell-specific parameter groupHoppingEnabled provided by higher layers. Sequence-group hopping for NPUSCH can be disabled for a certain UE through the higher-layer parameter groupHoppingDisabled despite being enabled on a cell basis unless the NPUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

The group-hopping pattern  is given by

where  for  and  is the slot number of the first slot of the resource unit for  . The pseudo-random sequence  is defined by clause 7.2. The pseudo-random sequence generator shall be initialized with  at the beginning of the resource unit for and in every even slot for .

The sequence-shift pattern  is given by

where is given by higher-layer parameter groupAssignmentNPUSCH. If no value is signalled  ,.

#### 10.1.4.2	Mapping to physical resources

The sequence  shall be multiplied with the amplitude scaling factor  and mapped in sequence starting with  to the sub-carriers. 
The set of sub-carriers used in the mapping process shall be identical to the corresponding NPUSCH transmission as defined in clause 10.1.3.6. 
The mapping to resource elements  shall be in increasing order of first, then , and finally the slot number. The values of the symbol index  in a slot are given in Table 10.1.4.2-1.

Table 10.1.4.2-1: Demodulation reference signal location for NPUSCH.

| NPUSCH format | Values for |  |
| --- | --- | --- |
| 1 | 4 | 3 |
| 2 | 0,1,2 | 2,3,4 |

### 10.1.5	SC-FDMA baseband signal generation

For , the time-continuous signal  in SC-FDMA symbol  in a slot is defined by clause 5.6 with the quantity  replaced by .

For , the time-continuous signal  for sub-carrier index  in SC-FDMA symbol  in an uplink slot is defined by

For  where parameters for  and  are given in Table 10.1.5-1,  is the modulation value of symbol  and the phase rotation  is defined by

where  is a symbol counter that is reset at the start of a transmission and incremented for each symbol during the transmission.

Table 10.1.5-1: SC-FDMA parameters for

| Parameter |  |  |
| --- | --- | --- |
|  | 8192 | 2048 |
| Cyclic prefix length | 256 |  |
| Set of values for | -24,-23,…,23 | -6,-5,…,5 |

The SC-FDMA symbols in a slot shall be transmitted in increasing order of , starting with , where SC-FDMA symbol starts at time   within the slot. For , the remaining  in  are not transmitted and used for guard period.

Only normal CP is supported for Narrowband IoT uplink in this release of the specification.

### 10.1.6	Narrowband physical random access channel

#### 10.1.6.1	Time and frequency structure

The physical layer random access preamble is based on single-subcarrier frequency-hopping symbol groups. A symbol group is illustrated in Figure 10.1.6.1-1, consisting of a cyclic prefix of length  and a sequence of 5 identical symbols with total length. The parameter values are listed in Table 10.1.6.1-1.

Figure 10.1.6.1-1: Random access symbol group

Table 10.6.1.1-1: Random access preamble parameters

| Preamble format |  |  |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |

The preamble consisting of 4 symbol groups transmitted without gaps shall be transmitted  times.

The transmission of a random access preamble, if triggered by the MAC layer, is restricted to certain time and frequency resources.

A NPRACH configuration provided by higher layers contains the following:

-	NPRACH resource periodicity  (nprach-Periodicity),

-	frequency location of the first subcarrier allocated to NPRACH  (nprach-SubcarrierOffset),

-	number of subcarriers allocated to NPRACH   (nprach-NumSubcarriers),

-	number of starting sub-carriers allocated to contention based NPRACH random access  (nprach-NumCBRA-StartSubcarriers),

-	number of NPRACH repetitions per attempt  (numRepetitionsPerPreambleAttempt),

-	NPRACH starting time  (nprach-StartTime),

-	Fraction for calculating starting subcarrier index for the range of NPRACH subcarriers reserved for indication of UE support for multi-tone msg3 transmission  (nprach-SubcarrierMSG3-RangeStart).

NPRACH transmission can start only  time units after the start of a radio frame fulfilling . After transmissions of  time units, a gap of  time units shall be inserted.

NPRACH configurations where  are invalid.

The NPRACH starting subcarriers allocated to contention based random access are split in two sets of subcarriers,  and  , where the second set, if present, indicate UE support for multi-tone msg3 transmission.

The frequency location of the NPRACH transmission is constrained within  sub-carriers. Frequency hopping shall be used within the 12 subcarriers, where the frequency location of the ith symbol group is given by  where  and

where  with  being the subcarrier selected by the MAC layer from , and the pseudo random sequence  is given by clause 7.2. The pseudo random sequence generator shall be initialised with .

#### 10.1.6.2	Baseband signal generation

The time-continuous random access signal  for symbol group  is defined by

Where ,  is an amplitude scaling factor in order to conform to the transmit power  specified in clause 16.3.1 in MFA TS 36.213 [4], ,  accounts for the difference in subcarrier spacing between the random access preamble and uplink data transmission,  and the location in the frequency domain controlled by the parameter  is derived from clause 10.1.6.1. The variable is given by Table 10.1.6.2-1.

Table 10.1.6.2-1: Random access baseband parameters

| Preamble format |  |
| --- | --- |
| 0, 1 | 3.75 kHz |

### 10.1.7	Modulation and upconversion

Modulation and upconversion to the carrier frequency of the complex-valued baseband signal or the complex-valued NPRACH baseband signal is shown in Figure 5.8-1. The filtering required prior to transmission is defined by the requirements in MFA TS 36.101 [7].

## 10.2	Downlink

### 10.2.1	Overview

#### 10.2.1.1	Physical channels

A downlink narrowband physical channel corresponds to a set of resource elements carrying information originating from higher layers and is the interface defined between MFA TS 36.212 [3] and this present document MFA TS 36.211.

The following downlink physical channels are defined:

-	Narrowband Physical Downlink Shared Channel, NPDSCH

-	Narrowband Physical Broadcast Channel, NPBCH

-	Narrowband Physical Downlink Control Channel, NPDCCH

#### 10.2.1.2	Physical signals

A downlink narrowband physical signal corresponds to a set of resource elements used by the physical layer but does not carry information originating from higher layers. The following downlink physical signals are defined:

-	Narrowband reference signal, NRS

-	Narrowband synchronization signal

### 10.2.2	Slot structure and physical resource elements

#### 10.2.2.1	Resource grid

The transmitted signal on one antenna port in each slot is described by a resource grid of size one resource block as defined in clause 6.2.3.

Only  is supported.

#### 10.2.2.2	Resource elements

Resource elements are defined according to clause 6.2.2.

#### 10.2.2.3	Guard period for half-duplex FDD operation

Only type-B half-duplex FDD operation is supported.

### 10.2.3	Narrowband physical downlink shared channel

#### 10.2.3.1	Scrambling

Scrambling shall be done according to clause 6.3.1. The scrambling sequence generator shall be initialised with whereis the first slot of the transmission of the codeword.

In case of NPDSCH repetitions and the NPDSCH carrying the BCCH, the scrambling sequence generator shall be reinitialized according to the expression above for each repetition.

In case of NPDSCH repetitions and the NPDSCH is not carrying the BCCH, the scrambling sequence generator shall be reinitialized according to the expression above after every  transmission of the codeword withand set to the first slot and the frame, respectively, used for the transmission of the repetition.

#### 10.2.3.2	Modulation

Modulation shall be done according to clause 6.3.2 using one of the modulation schemes in Table 10.2.3-1

Table 10.2.3-1: Modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| NPDSCH | QPSK |

#### 10.2.3.3	Layer mapping and precoding

Layer mapping and precoding shall be done according to clause 6.6.3 using the same set of antenna ports as the NPBCH.

#### 10.2.3.4	Mapping to resource elements

NPDSCH can be mapped to one or more than one subframes, , as given by clause 16.4.1.5 of MFA TS 36.213 [4], each of which shall be transmitted times.

For each of the antenna ports used for transmission of the physical channel, the block of complex-valued symbols  shall be mapped to resource elements  which meet all of the following criteria in the current subframe:

-	the subframe is not used for transmission of NPBCH, NPSS, or NSSS, and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.4.1.4 of MFA TS 36.213 [4].

The mapping of  in sequence starting with  to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe. For NPDSCH not carrying BCCH, after mapping to a subframe, the subframe shall be repeated for  additional subframes, before continuing the mapping of  to the following subframe. The mapping of  is then repeated until  subframes have been transmitted. For NPDSCH carrying BCCH, the is mapped to  subframes in sequence and then repeated until  subframes have been transmitted.

The NPDSCH transmission can be configured by higher layers with transmission gaps where the NPSDCH transmission is postponed. There are no gaps in the NPDSCH transmission if where  is given by the higher layer parameter dl-GapThreshold and  is given by MFA TS 36.213 [4]. The gap starting frame and subframe is given by  where the gap periodicity,, is given by the higher layer parameter dl-GapPeriodicity. The gap duration in number of subframes is given by ,  where  is given by the higher layer parameter dl-GapDurationCoeff. For NPDSCH carrying the BCCH there are no gaps in the transmission.

The UE shall not expect NPDSCH in subframe  if it is not a NB-IoT downlink subframe, except for transmissions of NPDSCH carrying SystemInformationBlockType1-NB in subframe 4. In case of NPDSCH transmissions, in subframes that are not NB-IoT downlink subframes, the NPDSCH transmission is postponed until the next NB-IoT downlink subframe.

### 10.2.4	Narrowband physical broadcast channel

#### 10.2.4.1	Scrambling

Scrambling shall be done according to clause 6.6.1 with  denoting the number of bits to be transmitted on the NPBCH.  equals 1600 for normal cyclic prefix. The scrambling sequence shall be initialised with  in radio frames fulfilling.

#### 10.2.4.2	Modulation

Modulation shall be done according to clause 6.6.2 using the modulation scheme in Table 10.2.4.2-1

Table 10.2.4.2-1: Modulation schemes for NPBCH

| Physical channel | Modulation schemes |
| --- | --- |
| NPBCH | QPSK |

#### 10.2.4.3	Layer mapping and precoding

Layer mapping and precoding shall be done according to clause 6.6.3 with . The UE shall assume antenna ports  and  are used for the transmission of the narrowband physical broadcast channel.

#### 10.2.4.4	Mapping to resource elements

The block of complex-valued symbols for each antenna port is transmitted in subframe 0 during 64 consecutive radio frames starting in each radio frame fulfilling  and shall be mapped in sequence starting with  to resource elements . The mapping to resource elements  not reserved for transmission of reference signals shall be in increasing order of first the index, then the index . After mapping to a subframe, the subframe shall be repeated in subframe 0 in the 7 following radio frames, before continuing the mapping of to subframe 0 in the following radio frame. The first three OFDM symbols in a subframe shall not be used in the mapping process.

For the purpose of the mapping, the UE shall assume cell-specific reference signals for antenna ports 0-3 and narrowband reference signals for antenna ports 2000 and 2001 being present irrespective of the actual configuration. The frequency shift of the cell-specific reference signals shall be calculated by replacing with  in the calculation of in clause 6.10.1.2.

### 10.2.5	Narrowband physical downlink control channel

#### 10.2.5.1	NPDCCH formats

The narrowband physical downlink control channel carries control information. A narrowband physical control channel is transmitted on an aggregation of one or two consecutive narrowband control channel elements (NCCEs), where a narrowband control channel element corresponds to 6 consecutive subcarriers  in a subframe where NCCE 0 occupies subcarriers 0 through 5 and NCCE 1 occupies subcarriers 6 through 11. The NPDCCH supports multiple formats as listed in Table 10.2.5.1-1. For NPDCCH format 1, both NCCEs belong to the same subframe.

One or two NPDCCHs can be transmitted in a subframe.

Table 10.2.5.1-1: Supported NPDCCH formats

| NPDCCH format | Number of NCCEs |
| --- | --- |
| 0 | 1 |
| 1 | 2 |

#### 10.2.5.2	Scrambling

Scrambling shall be done according to clause 6.8.2. The scrambling sequence shall be initialised at the start of subframe k0 according to MFA TS 36.213 [4] section 16.6 and after every 4th NPDCCH subframe with  where is the first slot of the NPDCCH subframe in which scrambling is (re-)initialized.

#### 10.2.5.3	Modulation

Modulation shall be done according to clause 6.8.3 using the modulation scheme in Table 10.2.5.3-1

Table 10.2.5.3-1: Modulation schemes

| Physical channel | Modulation schemes |
| --- | --- |
| NPDCCH | QPSK |

#### 10.2.5.4	Layer mapping and precoding

Layer mapping and precoding shall be done according to clause 6.6.3 using the same set of antenna ports as the NPBCH.

#### 10.2.5.5	Mapping to resource elements

The block of complex-valued symbols  shall be mapped in sequence starting with  to resource elements  on the associated antenna port which meet all of the following criteria:

-	they are part of the NCCE(s) assigned for the NPDCCH transmission, and

-	they are not used for transmission of NPBCH, NPSS, or NSSS , and

-	they are assumed by the UE not to be used for NRS, and

-	they are not overlapping with resource elements used for PBCH, PSS, SSS, or CRS as defined in clause 6 (if any), and

-	the index  in the first slot in a subframe fulfils  where is given by clause 16.6.1 of MFA TS 36.213 [4].

The mapping to resource elements  on antenna port  meeting the criteria above shall be in increasing order of first the index  and then the index, starting with the first slot and ending with the second slot in a subframe.

The NPDCCH transmission can be configured by higher layers with transmissions gaps where the NPDCCH transmission is postponed. The configuration is the same as described for NPDSCH in clause 10.2.3.4.

The UE shall not expect NPDCCH in subframe  if it is not a NB-IoT downlink subframe. In case of NPDCCH transmissions, in subframes that are not NB-IoT downlink subframes, the NPDCCH transmission is postponed until the next NB-IoT downlink subframe.

### 10.2.6	Narrowband reference signal (NRS)

Before a UE obtains operationModeInfo, the UE may assume narrowband reference signals are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS.

When UE receives higher-layer parameter operationModeInfo indicating guardband or standalone,

Before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS.

After the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes and shall not expect narrowband reference signals in other downlink subframes.

When UE receives higher-layer parameter operationModeInfo indicating inband-SamePCI or inband-DifferentPCI,

Before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #4 and in subframes #9 not containing NSSS.

After the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes and shall not expect narrowband reference signals in other downlink subframes.

#### 10.2.6.1	Sequence generation

The narrowband reference sequence shall be initialised according to clause 6.10.1.1 where   is replaced with .

#### 10.2.6.2	Mapping to resource elements

Narrowband reference signals are transmitted on one or two antenna ports .

If the higher layer indicates UE may assume that  is equal to , UE may assume

-	the number of antenna ports for the cell-specific reference signals as defined in clause 6.10.1 is the same as for the narrowband reference signals,

-	the antenna ports for cell-specific reference signals{0, 1} are equivalent to antenna ports for narrowband reference signals {2000, 2001}, respectively, and

-	 the cell-specificreference signals are available in all subframes where the narrowband reference signals are available.

If the higher layer does not indicate UE may assume that  is equal to , UE may assume

-	the number of antenna port for the cell-specific reference signals as defined in clause 6.10.1 is obtained from the higher layer parameter eutra-NumCRS-Ports.

-	the cell-specific reference signals are available in all subframes where the narrowband reference signals are available.

The reference signal sequence  shall be mapped to complex-valued modulation symbols  used as reference symbols for antenna port  in slot  according to

where

The variables  and  define the position in the frequency domain for the different reference signals where  is given by

The cell-specific frequency shift is given by .

Resource elements  used for transmission of narrowband reference signals on any of the antenna ports in a slot shall not be used for any transmission on any other antenna port in the same slot and set to zero.

Narrowband reference signals shall not be transmitted in  subframes containing NPSS or NSSS.

Figure 10.2.6.2-1 illustrates the resource elements used for reference signal transmission according to the above definition. The notation  is used to denote a resource element used for reference signal transmission on antenna port.

Figure 10.2.6.2-1. Mapping of downlink narrowband reference signals (normal cyclic prefix)

### 10.2.7	Synchronization signals

There are 504 unique physical-layer cell identities indicated by the narrowband secondary synchronization signal.

#### 10.2.7.1	Narrowband primary synchronization signal (NPSS)

##### 10.2.7.1.1	Sequence generation

The sequence  used for the narrowband primary synchronization signal is generated from a frequency-domain Zadoff-Chu sequence according to

where the Zadoff-Chu root sequence index  and for different symbol indices  is given by Table 10.2.7.1.1-1.

Table 10.2.7.1.1-1: Definition of .

| Cyclic prefix length |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | 1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 |

##### 10.2.7.1.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the narrowband primary synchronization signal within a subframe.

UE shall not assume that the narrowband primary synchronization signal is transmitted on the same antenna port as any of the downlink reference signals. The UE shall not assume that the transmissions of the narrowband primary synchronization signal in a given subframe use the same antenna port, or ports, as the narrowband primary synchronization signal in any other subframe.

The sequences  shall be mapped to resource elements  in increasing order of first the index  and then the index  in subframe 5 in every radio frame. For resource elements  overlapping with resource elements where cell-specific reference signals according to clause 6.10 are transmitted, the corresponding sequence element  is not used for the NPSS but counted in the mapping process.

#### 10.2.7.2	Narrowband secondary synchronization signal (NSSS)

##### 10.2.7.2.1	Sequence generation

The sequence  used for the narrowband secondary synchronization signal is generated from a frequency-domain Zadoff-Chu sequence according to

where

The binary sequence  is given by Table 10.2.7.2.1-1. The cyclic shift in frame number  is given by

.

Table 10.2.7.2.1-1: Definition of .

| 0 | [1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1] |
| --- | --- |
| 1 | [1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1] |
| 2 | [1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1] |
| 3 | [1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1 -1 1 1 -1 1 -1 -1 1 -1 1 1 -1 1 -1 -1 1 1 -1 -1 1 -1 1 1 -1] |

##### 10.2.7.2.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the narrowband secondary synchronization signal within a subframe.

UE shall not assume that the narrowband secondary synchronization signal is transmitted on the same antenna port as any of the downlink reference signals. The UE shall not assume that the transmissions of the narrowband secondary synchronization signal in a given subframe use the same antenna port, or ports, as the narrowband secondary synchronization signal in any other subframe.

The sequence  shall be mapped to resource elements  in sequence starting with  in increasing order of first the index  over the 12 assigned subcarriers and then the index over the assigned last  symbols of subframe 9 in radio frames fulfilling , where  is given by Table 10.2.7.2.2-1.

Table 10.2.7.2.2-1: NSSS number of symbols

| Cyclic prefix length |  |
| --- | --- |
| Normal | 11 |

For resource elements  overlapping with resource elements where cell-specific reference signals according to clause 6.10 are transmitted, the corresponding sequence element  is not used for the NSSS but counted in the mapping process.

### 10.2.8	OFDM baseband signal generation

If the higher layer parameter operationModeInfo does not indicate ‘inband-SamePCI’ and samePCI-Indicator does not indicate 'samePCI', then the time-continuous signal  on antenna port  in OFDM symbol  in a downlink slot is defined by

for where , ,  and  is the content of resource element  on antenna port .

If the higher layer parameter operationModeInfo indicates ‘inband-SamePCI’ or samePCI-Indicator indicate 'samePCI', then the time-continuous signal  on antenna port  in OFDM symbol , where is the OFDM symbol index from the start of the last even-numbered subframe, is defined by

for    where  and, if resource element is used for Narrowband IoT, and 0 otherwise, and  is the frequency location of the center of the Narrowband IoT PRB minus the frequency location of the center of the LTE signal.

Only normal CP is supported for Narrowband IoT downlink in this release of the specification.

### 10.2.9	Modulation and upconversion

Modulation and upconversion to the carrier frequency of the complex-valued OFDM baseband signal for each antenna port is shown in Figure 6.13-1. The filtering required prior to transmission is defined by the requirements in MFA TS 36.104 [6].

# Annex A (informative): Change history

| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2006-09-24 | - | - | - |  | Draft version created | - | 0.0.0 |
| 2006-10-09 | - | - | - |  | Updated skeleton | 0.0.0 | 0.0.1 |
| 2006-10-13 | - | - | - |  | Endorsed by RAN1 | 0.0.1 | 0.1.0 |
| 2006-10-23 | - | - | - |  | Inclusion of decision from RAN1#46bis | 0.1.0 | 0.1.1 |
| 2006-11-06 | - | - | - |  | Updated editor’s version | 0.1.1 | 0.1.2 |
| 2006-11-09 | - | - | - |  | Updated editor’s version | 0.1.2 | 0.1.3 |
| 2006-11-10 | - | - | - |  | Endorsed by RAN1#47 | 0.1.3 | 0.2.0 |
| 2006-11-27 | - | - | - |  | Editor’s version, including decisions from RAN1#47 | 0.2.0 | 0.2.1 |
| 2006-12-14 | - | - | - |  | Updated editor’s version | 0.2.1 | 0.2.2 |
| 2007-01-15 | - | - | - |  | Updated editor’s version | 0.2.2 | 0.2.3 |
| 2007-01-19 | - | - | - |  | Endorsed by RAN1#47bis | 0.2.3 | 0.3.0 |
| 2007-02-01 | - | - | - |  | Editor’s version, including decisions from RAN1#47bis | 0.3.0 | 0.3.1 |
| 2007-02-12 | - | - | - |  | Updated editor’s version | 0.3.1 | 0.3.2 |
| 2007-02-16 | - | - | - |  | Endorsed by RAN1#48 | 0.3.2 | 0.4.0 |
| 2007-02-16 | - | - | - |  | Editor’s version, including decisions from RAN1#48 | 0.4.0 | 0.4.1 |
| 2007-02-21 | - | - | - |  | Updated editor’s version | 0.4.1 | 0.4.2 |
| 2007-03-03 | RP_35 | RP-070169 |  |  | For information at RAN#35 | 0.4.2 | 1.0.0 |
| 2007-04-25 | - | - | - |  | Editor’s version, including decisions from RAN1#48bis and RAN1 TDD Ad Hoc | 1.0.0 | 1.0.1 |
| 2007-05-03 | - | - | - | - | Updated editor’s version | 1.0.1 | 1.0.2 |
| 2007-05-08 | - | - | - | - | Updated editor’s version | 1.0.2 | 1.0.3 |
| 2007-05-11 | - | - | - | - | Updated editor’s version | 1.0.3 | 1.0.4 |
| 2007-05-11 | - | - | - | - | Endorsed by RAN1#49 | 1.0.4 | 1.1.0 |
| 2007-05-15 | - | - | - | - | Editor’s version, including decisions from RAN1#49 | 1.1.0 | 1.1.1 |
| 2007-06-05 | - | - | - | - | Updated editor’s version | 1.1.1 | 1.1.2 |
| 2007-06-25 | - | - | - | - | Endorsed by RAN1#49bis | 1.1.2 | 1.2.0 |
| 2007-07-10 | - | - | - | - | Editor’s version, including decisions from RAN1#49bis | 1.2.0 | 1.2.1 |
| 2007-08-10 | - | - | - | - | Updated editor’s version | 1.2.1 | 1.2.2 |
| 2007-08-20 | - | - | - | - | Updated editor’s version | 1.2.2 | 1.2.3 |
| 2007-08-24 | - | - | - | - | Endorsed by RAN1#50 | 1.2.3 | 1.3.0 |
| 2007-08-27 | - | - | - | - | Editor’s version, including decisions from RAN1#50 | 1.3.0 | 1.3.1 |
| 2007-09-05 | - | - | - | - | Updated editor’s version | 1.3.1 | 1.3.2 |
| 2007-09-08 | RP_37 | RP-070729 | - | - | For approval at RAN#37 | 1.3.2 | 2.0.0 |
| 12/09/07 | RP_37 | RP-070729 |  |  | Approved version | 2.0.0 | 8.0.0 |
| 28/11/07 | RP_38 | RP-070949 | 0001 | - | Introduction of optimized FS2 for TDD | 8.0.0 | 8.1.0 |
| 28/11/07 | RP_38 | RP-070949 | 0002 | - | Introduction of scrambling sequences, uplink reference signal sequences, secondary synchronization sequences and control channel processing | 8.0.0 | 8.1.0 |
| 05/03/08 | RP_39 | RP-080219 | 0003 | 1 | Update of uplink reference-signal hopping, downlink reference signals, scrambling sequences, DwPTS/UpPTS lengths for TDD and control channel processing | 8.1.0 | 8.2.0 |
| 28/05/08 | RP_40 | RP-080432 | 0004 | - | Correction of the number of subcarriers in PUSCH transform precoding | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0005 | - | Correction of PHICH mapping | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0006 | - | Correction of PUCCH resource index for PUCCH format 2 | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0007 | 3 | Correction of the predefined hopping pattern for PUSCH | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0008 | - | Non-binary hashing functions | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0009 | 1 | PUCCH format 1 | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0010 | 1 | CR on Uplink DM RS hopping | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0012 | 1 | Correction to limitation of constellation size of ACK transmission in PUSCH | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0015 | 1 | PHICH mapping for one and two antenna ports in extended CP | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0016 | 1 | Correction of PUCCH in absent of mixed format | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0017 | - | Specification of CCE size and PHICH resource indication | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0018 | 3 | Correction of the description of frame structure type 2 | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0019 | - | On Delta^pucch_shift correction | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0021 | - | Corrections to Secondary Synchronization Signal Mapping | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0022 | - | Downlink VRB mapping to PRB for distributed transmission | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0023 | - | Clarification of modulation symbols to REs mapping for DVRB | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0024 | 1 | Consideration on the scrambling of PDSCH | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0025 | - | Corrections to Initialization of DL RS Scrambling | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0026 | 1 | CR on Downlink RS | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0027 | - | CR on Uplink RS | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0028 | 1 | Fixed timing advance offset for LTE TDD and half-duplex FDD | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0029 | 1 | Timing of random access preamble format 4 | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0030 | 1 | Uplink sounding RS bandwidth configuration | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0031 | - | Use of common RS when UE-specific RS are configured | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0032 | 1 | Uplink RS Updates | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0033 | - | Orthogonal cover sequence for shortened PUCCH format 1a and 1b | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0034 | - | Clarification of PDCCH mapping | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0035 | - | TDD PRACH time/frequency mapping | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0036 | - | Cell Specific Uplink Sounding RS Subframe Configuration | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0038 | - | PDCCH length for carriers with mixed MBSFN and Unicast Traffic | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0040 | - | Correction to the scrambling sequence generation for PUCCH, PCFICH, PHICH, MBSFN RS and UE specific RS | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0041 | - | PDCCH coverage in narrow bandwidths | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0042 | - | Closed-Loop and Open-Loop Spatial Multiplexing | 8.2.0 | 8.3.0 |
| 28/05/08 | RP_40 | RP-080432 | 0043 | - | Removal of small-delay CDD | 8.2.0 | 8.3.0 |
| 09/09/08 | RP_41 | RP-080668 | 48 | 1 | Frequency Shifting of UE-specific RS | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 49 | 1 | Correction of PHICH to RE mapping in extended CP subframe | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 50 | - | Corrections to for handling remaining Res | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 51 | - | PRACH configuration for frame structure type 1 | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 52 | 2 | Correction of PUCCH index generation formula | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 53 | - | Orthogonal cover sequence for shortened PUCCH format 1a and 1b | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 54 | - | Correction of mapping of ACK/NAK to binary bit values | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 56 | 2 | Remaining issues on SRS hopping | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 57 | 1 | Correction of n_cs(n_s) and OC/CS remapping for PUCCH formats 1/1a/1b and 2/2a/2b | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 59 | - | Corrections to Rank information scrambling in Uplink Shared Channel | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 60 | - | Definition on the slot number for frame structure type 2 | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 61 | - | Correction of the Npucch sequence upper limit for the formats 1/1a/1b | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 62 | 1 | Clarifications for DMRS parameters | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 63 | - | Correction of n_prs | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 64 | 1 | Introducing missing L1 parameters to 36.211 | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 65 | 3 | Clarification on reception of synchronization signals | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 66 | - | Correction to the downlink/uplink timing | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 67 | - | ACK/NACK Scrambling scheme on PUCCH | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 68 | - | DCI format1C | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 69 | - | Refinement for REG Definition for n = 4 | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 71 | - | Correcting Ncs value for PRACH preamble format 0-3 | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 73 | - | Correction of the half duplex timing advance offset value | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 74 | - | Correction to Precoding for Transmit Diversity | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 75 | - | Clarification on number of OFDM symbols used for PDCCH | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 77 | - | Number of antenna ports for PDSCH | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 78 | - | Correction to Type 2 PUSCH predetermined hopping for Nsb=1 operation | 8.3.0 | 8.4.0 |
| 09/09/08 | RP_41 | RP-080668 | 79 | - | PRACH frequency location | 8.3.0 | 8.4.0 |
| 03/12/08 | RP_42 | RP-081074 | 70 | 1 | Correction for the definition of UE-specific reference signals | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 72 | 2 | Corrections to precoding for large delay CDD | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 80 | - | Correction to the definition of nbar_oc for extended CP | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 81 | 1 | Specification of reserved REs not used for RS | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 82 | 2 | Clarification of the random access preamble transmission timing | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 83 | 1 | Indexing of PRACH resources within the radio frame | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 84 | 6 | Alignment of RAN1/RAN2 specification | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 86 | - | Clarification on scrambling of ACK/NAK bits for PUCCH format 2a/2b | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 87 | - | Correction of introduction of shortened SR | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 88 | - | Corrections to 36.211 | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 89 | - | Clarification on PUSCH DM RS Cyclic Shift Hopping | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 92 | 1 | Correction to the uplink DM RS assignment | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 93 | - | Clarify the RNTI used in scrambling sequence initialization | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 94 | 1 | On linkage Among UL Power Control Parameters | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 95 | - | Clarification on PUSCH pre-determined hopping pattern | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 96 | - | Clarification of SRS sequence-group and base sequence number | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 97 | 1 | SRS subframe configuration | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 98 | - | Remaining SRS details for TDD | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 99 | - | Clarifying UL VRB Allocation | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 100 | - | Clarification on PUCCH resource hopping | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 101 | - | Correction for definition of Qm and a pseudo code syntax error in Scrambling. | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 105 | 1 | Remaining Issues on SRS of TDD | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 106 | - | Correction of reference to RAN4 specification of supported uplink bandwidth | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 107 | - | General corrections to SRS | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 109 | 2 | Correction to PCFICH specification | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 110 | 1 | Correction to Layer Mapping for Transmit Diversity with Four Antenna Ports | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 111 | - | Correction of the mapping of cyclic shift filed in DCI format 0 to the dynamic cyclic shift offset | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 112 | - | DRS collision handling | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 113 | - | Clarification to enable reuse of non-active PUCCH CQI RBs for PUSCH | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 114 | 1 | PUSCH Mirror Hopping operation | 8.4.0 | 8.5.0 |
| 03/12/08 | RP_42 | RP-081074 | 108 | 1 | Extended and normal cyclic prefix in DL and UL for LTE TDD | 8.4.0 | 8.5.0 |
| 04/03/09 | RP_43 | RP-090234 | 115 | 1 | Alignment of PRACH configuration index for FS type 1 and type 2 | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 118 | 1 | Clarification for DRS Collision handling | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 121 | 1 | Removing inverse modulo operation | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 123 | 1 | Clarification on the use of preamble format 4 | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 124 | - | Clarification of RNTI used in scrambling sequence | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 125 | 1 | Clarifying PDCCH RE mapping | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 126 | - | Correction of preamble format 4 timing | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 127 | 2 | Corrections to SRS | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 128 | 2 | Clarification of PDSCH Mapping to Resource Elements | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 129 | 1 | Alignment with correct ASN1 parameter names | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 130 | - | Correction to PUCCH format 1 mapping to physical resources | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 132 | - | Correction to type-2 PUSCH hopping | 8.5.0 | 8.6.0 |
| 04/03/09 | RP_43 | RP-090234 | 134 | - | Alignment of SRS configuration | 8.5.0 | 8.6.0 |
| 27/05/09 | RP_44 | RP-090527 | 135 | - | Correction on UE behavior for PRACH 20ms periodicity | 8.6.0 | 8.7.0 |
| 15/09/09 | RP_45 | RP-090888 | 137 | 1 | Clarification on DMRS sequence for PUSCH | 8.7.0 | 8.8.0 |
| 15/09/09 | RP_45 | RP-090888 | 138 | 1 | Correction to PHICH resource mapping for TDD and to PHICH scrambling | 8.7.0 | 8.8.0 |
| 01/12/09 | RP_46 | RP-091168 | 142 | - | Clarification of the transmit condition for UE specific reference signals | 8.8.0 | 8.9.0 |
| 01/12/09 | RP_46 | RP-091172 | 139 | 2 | Introduction of LTE positioning | 8.9.0 | 9.0.0 |
| 01/12/09 | RP_46 | RP-091177 | 140 | 3 | Editorial corrections to 36.211 | 8.9.0 | 9.0.0 |
| 01/12/09 | RP_46 | RP-091257 | 141 | 1 | Introduction of enhanced dual layer transmission | 8.9.0 | 9.0.0 |
| 16/03/10 | RP_47 | RP-100209 | 144 | 1 | Removal of square brackets on positioning subframe periodicities | 9.0.0 | 9.1.0 |
| 16/03/10 | RP_47 | RP-100209 | 145 | - | Clarification of the CP length of empty OFDM symbols in PRS subframes | 9.0.0 | 9.1.0 |
| 16/03/10 | RP_47 | RP-100210 | 146 | - | Clarification of MBSFN subframe definition | 9.0.0 | 9.1.0 |
| 07/12/10 | RP_50 | RP-101320 | 148 | - | Introduction of Rel-10 LTE-Advanced features in 36.211 | 9.1.0 | 10.0.0 |
| 15/03/11 | RP_51 | RP-110254 | 149 | 1 | Correction on UE behavior for PRACH preamble format 4 | 10.0.0 | 10.1.0 |
| 15/03/11 | RP_51 | RP-110256 | 150 | - | Corrections to Rel-10 LTE-Advanced features in 36.211 | 10.0.0 | 10.1.0 |
| 01/06/11 | RP_52 | RP-110818 | 153 | 2 | PUSCH interaction with periodic SRS | 10.1.0 | 10.2.0 |
| 01/06/11 | RP_52 | RP-110819 | 154 | 1 | Correction on describing PUCCH format 3 | 10.1.0 | 10.2.0 |
| 01/06/11 | RP_52 | RP-110821 | 155 | 3 | Correction on codebooks for CSI-RS based feedback for up to 4 CSI-RS ports. | 10.1.0 | 10.2.0 |
| 01/06/11 | RP_52 | RP-110821 | 156 | - | Correction on overlapping non-zero-power and zero-power CSI-RS configurations | 10.1.0 | 10.2.0 |
| 01/06/11 | RP_52 | RP-110821 | 157 | - | Correction on CSI-RS configuration | 10.1.0 | 10.2.0 |
| 01/06/11 | RP_52 | RP-110821 | 158 | - | PDSCH transmission in MBSFN subframes | 10.1.0 | 10.2.0 |
| 01/06/11 | RP_52 | RP-110823 | 159 | - | Correction on implicit derivation of transmission comb per antenna port for SRS | 10.1.0 | 10.2.0 |
| 01/06/11 | RP_52 | RP-110823 | 160 | - | Uplink DMRS sequence in RACH procedure | 10.1.0 | 10.2.0 |
| 15/09/11 | RP_53 | RP-111229 | 162 | - | Corrections on DMRS for Extended CP | 10.2.0 | 10.3.0 |
| 15/09/11 | RP_53 | RP-111228 | 163 | - | Clarification of applicability of precoding power scaling factors for PDSCH | 10.2.0 | 10.3.0 |
| 15/09/11 | RP_53 | RP-111228 | 164 | - | Correction to modulation and upconversion on PRACH | 10.2.0 | 10.3.0 |
| 15/09/11 | RP_53 | RP-111229 | 165 | - | Clarification on cyclic prefix of PDSCH in MBSFN subframes | 10.2.0 | 10.3.0 |
| 15/09/11 | RP_53 | RP-111229 | 166 | 3 | Corrections on indication in scrambling identity field in DCI format 2B and 2C | 10.2.0 | 10.3.0 |
| 05/12/11 | RP_54 | RP-111668 | 167 | - | A correction to PDSCH precoding for CQI calculation | 10.3.0 | 10.4.0 |
| 05/12/11 | RP_54 | RP-111668 | 168 | - | Correction to figure of CSI-RS pattern in extended-CP subframe | 10.3.0 | 10.4.0 |
| 13/06/12 | RP_56 | RP-120736 | 169 | - | Correction to resource mapping for PDSCH | 10.4.0 | 10.5.0 |
| 13/06/12 | RP_56 | RP-120739 | 171 | - | Correction for DMRS group hopping and sequence hopping | 10.4.0 | 10.5.0 |
| 13/06/12 | RP_56 | RP-120738 | 172 | - | Correction to assumed CSI-RS transmissions in subframes used for paging | 10.4.0 | 10.5.0 |
| 04/09/12 | RP_57 | RP-121274 | 170 | 4 | Introduction of an additional special subframe configuration | 10.5.0 | 11.0.0 |
| 04/09/12 | RP_57 | RP-121272 | 173 | - | Inclusion of Rel-11 features | 10.5.0 | 11.0.0 |
| 04/12/12 | RP_58 | RP-121839 | 175 | - | Correction to assumed CSI-RS transmissions in secondary cells | 11.0.0 | 11.1.0 |
| 04/12/12 | RP_58 | RP-121846 | 176 | - | Correction to assumed CSI-RS transmissions in secondary cells | 11.0.0 | 11.1.0 |
| 26/02/13 | RP_59 | RP-130254 | 178 | - | Clarification of CSI RS mapping to resource elements | 11.1.0 | 11.2.0 |
| 26/02/13 | RP_59 | RP-130254 | 180 | - | Correction to CSI Reference Signals | 11.1.0 | 11.2.0 |
| 26/02/13 | RP_59 | RP-130255 | 181 | - | Additional clarifications/corrections for introducing Rel-11 features | 11.1.0 | 11.2.0 |
| 11/06/13 | RP_60 | RP-130752 | 182 | - | Correction to EPDCCH PRB pair indication | 11.2.0 | 11.3.0 |
| 11/06/13 | RP_60 | RP-130752 | 183 | - | CR on collision between EPDCCH and PSS/SSS/PBCH | 11.2.0 | 11.3.0 |
| 03/09/13 |  |  |  |  | MCC clean-up | 11.3.0 | 11.4.0 |
| 03/09/13 | RP_60 | RP-131250 | 185 | - | Correction to QCL behaviour on CRS | 11.3.0 | 11.4.0 |
| 03/12/13 | RP_62 | RP-131894 | 186 | - | Correction on the derivation of the non-MBSFN region by PCFICH | 11.4.0 | 11.5.0 |
| 03/12/13 | RP_62 | RP-131896 | 184 | 3 | Introduction of Rel 12 feature for Downlink MIMO Enhancement | 11.5.0 | 12.0.0 |
| 03/03/14 | RP_63 | RP-140286 | 187 | - | On PMCH starting symbol in an MBSFN subframe | 12.0.0 | 12.1.0 |
| 10/06/14 | RP_64 | RP-140858 | 189 | - | CR on antenna port definitions | 12.1.0 | 12.2.0 |
| 10/06/14 | RP_64 | RP-140858 | 190 | 1 | Clarification of downlink subframes | 12.1.0 | 12.2.0 |
| 10/06/14 | RP_64 | RP-140862 | 191 | - | Inclusion of eIMTA, TDD-FDD CA, and coverage enhancements | 12.1.0 | 12.2.0 |
| 10/09/14 | RP_65 | RP-141485 | 192 | - | Inclusion of low-cost MTC and 256QAM | 12.2.0 | 12.3.0 |
| 10/09/14 | RP_65 | RP-141477 | 194 | - | CR on port 5 UE-specific reference signal when PDSCH is overlapped with EPDCCH | 12.2.0 | 12.3.0 |
| 08/12/14 | RP_66 | RP-142098 | 195 | 3 | Clarification of PUSCH rate matching with SRS | 12.3.0 | 12.4.0 |
| 08/12/14 | RP_66 | RP-142106 | 197 | 4 | Inclusion of small-cell enhancements | 12.3.0 | 12.4.0 |
| 09/03/15 | RP_67 | RP-150366 | 196 | 7 | Inclusion of ProSe | 12.4.0 | 12.5.0 |
| 09/03/15 | RP_67 | RP-150364 | 198 | - | Correction on 256QAM applicability to PMCH | 12.4.0 | 12.5.0 |
| 09/03/15 | RP_67 | RP-150364 | 199 | - | Correction of discovery signal transmission | 12.4.0 | 12.5.0 |
| 15/06/15 | RP_68 | RP-150935 | 201 | - | Alignment of ProSe parameters | 12.5.0 | 12.6.0 |
| 14/09/15 | RP_69 | RP-151465 | 203 | - | Clarification on SRS BW configuration | 12.6.0 | 12.7.0 |
| 07/12/15 | RP_70 | RP-152036 | 209 | 1 | Modify max TA for dual connectivity | 12.7.0 | 12.8.0 |
| 07/12/15 | RP_70 | RP-152025 | 206 | 2 | Introduction of EB/FD-MIMO | 12.8.0 | 13.0.0 |
| 07/12/15 | RP_70 | RP-152027 | 208 | 1 | Introduction of Rel-13 eCA | 12.8.0 | 13.0.0 |
| 07/12/15 | RP_70 | RP-152125 | 204 | 2 | eD2D CR for 36.211 | 12.8.0 | 13.0.0 |
| 07/12/15 | RP_70 | RP-152258 | 205 | 4 | Introduction of LAA | 12.8.0 | 13.0.0 |

| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2016-03 | RAN#71 | RP-160359 | 210 | - | F | Alignment eD2D CR for 36.211 | 13.1.0 |
| 2016-03 | RAN#71 | RP-160367 | 212 | - | F | Clarification on PDSCH collision with PSS/SSS/PBCH | 13.1.0 |
| 2016-03 | RAN#71 | RP-160357 | 213 | - | F | Correction on support of CA with up to 32 CCs | 13.1.0 |
| 2016-03 | RAN#71 | RP-160357 | 214 | - | F | Correction on PUCCH format 4 and 5 | 13.1.0 |
| 2016-03 | RAN#71 | RP-160360 | 217 | - | F | Correction on DRS subframe in 36.211 | 13.1.0 |
| 2016-03 | RAN#71 | RP-160360 | 218 | - | F | Correction on EPDCCH start symbol in LAA | 13.1.0 |
| 2016-03 | RAN#71 | RP-160360 | 219 | - | F | Correction to MBSFN subframe configuration | 13.1.0 |
| 2016-03 | RAN#71 | RP-160358 | 220 | - | F | CR on CSI-RS configuration for more than eight antenna ports in 36.211 | 13.1.0 |
| 2016-03 | RAN#71 | RP-160358 | 221 | - | F | CR on mismatch between 36.211 and 36.331 | 13.1.0 |
| 2016-03 | RAN#71 | RP-160358 | 222 | - | F | Clarification on additional SC-FDMA symbols in UpPTS for SRS | 13.1.0 |
| 2016-03 | RAN#71 | RP-160358 | 223 | - | F | Correction on Precoding and definition of DMRS ports | 13.1.0 |
| 2016-03 | RAN#71 | RP-160361 | 207 | 9 | B | Introduction of LC/CE MTC | 13.1.0 |
| 2016-06 | RAN#72 | RP-161063 | 216 | 2 | F | CR on CSI-RS transmission in DwPTS | 13.2.0 |
| 2016-06 | RAN#72 | RP-161067 | 224 | 8 | B | Introduction of NB-IoT | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 229 | 2 | F | Collision between PSS/SSS/PBCH and MPDCCH/PDSCH for MTC | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 230 | - | F | DMRS initialization of CSS for MTC | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 231 | - | F | Missing words in PRACH starting subframe paragraph for MTC | 13.2.0 |
| 2016-06 | RAN#72 | RP-161065 | 232 | - | F | Correction to EPDCCH procedures for LAA FS 3 | 13.2.0 |
| 2016-06 | RAN#72 | RP-161063 | 233 | - | F | Clarification on PDSCH mapping to resource elements | 13.2.0 |
| 2016-06 | RAN#72 | RP-161063 | 234 | - | F | CR on CSI-RS description in TS 36.211 | 13.2.0 |
| 2016-06 | RAN#72 | RP-161065 | 235 | - | F | Corrections on the support of ending partial subframe in LAA | 13.2.0 |
| 2016-06 | RAN#72 | RP-161063 | 236 | - | F | Clarification of CSI-RS on extended CP | 13.2.0 |
| 2016-06 | RAN#72 | RP-161063 | 237 | - | F | Correction on description about UpPTS length for preamble format 4 for PRACH | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 238 | - | F | Correction to TS 36.211 for eMTC | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 239 | - | F | Narrow band hopping | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 240 | 1 | F | CR on MPDCCH format for Rmax=1 and 2/4 PRBs | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 241 | 1 | F | Correction on RE mapping in MBSFN subframe for BL/CE UEs in CEModeB | 13.2.0 |
| 2016-06 | RAN#72 | RP-161063 | 242 | - | F | Correction on the description about DMRS | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 243 | - | F | CR for TS36.211 related to 2+4 PRB set | 13.2.0 |
| 2016-06 | RAN#72 | RP-161065 | 244 | - | F | CR on UE assumptions on number of CRS ports in DRS | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 245 | - | F | Some corrections for eMTC | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 247 | - | F | Clarification of MPDCCH over empty CRS tones in PBCH repetition | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 248 | - | F | Scrambling sequence initialization | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 249 | - | F | On MPDCCH AL for 8 EREGs per ECCE in TS 36.211 | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 250 | - | F | Overriding of valid-invalid subframes for R=1 | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 251 | - | F | Scrambling Sequence for paging MPDCCH and PDSCH | 13.2.0 |
| 2016-06 | RAN#72 | RP-161066 | 252 | - | F | Scrambling sequence initialization for PDSCH | 13.2.0 |
| 2016-09 | RAN#73 | RP-161563 | 253 | - | F | Correction on DMRS for NB-IoT in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 254 | 1 | F | Correction on NPRACH in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 255 | - | F | Correction on SC-FDMA signal generation for NB-IoT in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 256 | - | F | Corrections to RRC parameter names for NB-IoT in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 259 | - | F | MPDCCH search-space with Temporary C-RNTI | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 260 | - | F | Correction on NPBCH in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 261 | 1 | F | Correction on UL collisions in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 262 | 1 | F | Correction on NPSS mapping in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 263 | 1 | F | Corrections on the presence of NRS for standalone and guard band operation mode in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161561 | 264 | - | F | Correction on the determination of EPDCCH starting position | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 265 | - | F | Corrections on NPDCCH scrambling in TS 36.211 | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 272 | 1 | F | Frequency hopping for SI and paging messages for BL/CE UE | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 275 | - | F | Scrambling of DL DMRS for BL/CE UE | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 276 | - | F | Enable cross-subframe channel estimation for BL/CE UE | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 278 | - | F | Frequency hopping interval for MPDCCH during random access for BL/CE UE | 13.3.0 |
| 2016-09 | RAN#73 | RP-161565 | 279 | - | F | CR on the correction from SC-FDFMA to SC-FDMA | 13.3.0 |
| 2016-09 | RAN#73 | RP-161561 | 280 | - | F | Correction for PHICH resource reservation on the LAA cell in 36.211 for Rel-13 LAA | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 281 | - | F | Correction on MPDCCH transmission without repetition in special subframes | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 282 | 1 | F | Introduction of a reserved range of NPRACH sub-carriers for contention based access | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 283 | - | F | Clarification of valid subframe in eMTC | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 284 | - | F | Correction of NB-IoT antenna port mapping | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 285 | - | F | Clarification on PRACH system frame number | 13.3.0 |
| 2016-09 | RAN#73 | RP-161562 | 286 | - | F | PUCCH retuning with puncturing for BL/CE UE | 13.3.0 |
| 2016-09 | RAN#73 | RP-161563 | 287 | 1 | F | Phase difference between NRS and CRS | 13.3.0 |
| 2016-09 | RAN#73 | RP-161825 | 288 | 1 | B | Continuous uplink transmission in eMTC | 13.3.0 |
| 2016-09 | RAN#73 | RP-161571 | 266 | 2 | B | Introduction of eLAA | 13.3.0 |
| 2016-09 | RAN#73 | RP-161570 | 267 | 2 | B | Introduction of V2V support | 13.3.0 |
| 2016-12 | RAN#74 | RP-162356 | 309 | - | A | Correction on NZP CSI-RS aggregation for Class A | 14.0.0 |
| 2017-03 | RAN#75 | RP-170607 | 335 | 2 | B | Introduction of eFD-MIMO | 14.1.0 |
| 2017-03 | RAN#75 | RP-170612 | 352 | - | A | Correction on single layer precoding for EPCCH | 14.1.0 |
| 2017-06 | RAN#76 | RP-171194 | 360 | - | F | CR 36.211 for clarification and correction on IFDMA UL-DMRS for eFD-MIMO | 14.2.0 |
| 2017-06 | RAN#76 | RP-171194 | 360 | - | F | CR 36.211 for Clarification on CDM-8 pattern for 24-ports CSI-RS in DwPTS | 14.2.0 |
| 2017-06 | RAN#76 | RP-171194 | 375 | - | F | CR for precoding for spatial multiplexing using antenna ports with UE-specific reference signals in 36.211 | 14.2.0 |
| 2017-06 | RAN#76 | RP-171194 | 376 | - | F | Correction to CSI-RS configuration | 14.2.0 |

# Annex A-MF1 (informative): Change history for MF Specification

| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2016-11 | MFA #7 | - | - | 1 | Introduction of MF | - | 1.0.0 |
| 2017-02 | MFA #9 | mf2017.088.02 | - | 2 | Corrected the equation of the time-continous random access signal s(t) for MF-preamble format 0 Corrected the definition of by MF-sPRACH opportunity per configured interlace considered. | 1.0.0 | 1.0.1 |
| 2017-07 | MFA #10 | mf2017.380.04 | - | 3 | sPUCCH Format 0 number of symbols | 1.0.1 | 1.0.2 |
| 2017-07 | MFA #10 | mf2017.381.00 | - | 3 | Clarification of sequence hopping for MF-sPUCCH/MF-ePUCCH DMRS | 1.0.1 | 1.0.2 |
| 2017-07 | MFA #11 | mf2017.597.02 | - | 3 | Clarification of SRS Interface Allocation | 1.0.1 | 1.0.2 |
| 2017-09 | MFA #12 | ‒ | R1-1704157 | 4 | Introduction of eFD-MIMO | 1.0.2 | 1.0.3 |
| 2017-09 | MFA #12 | ‒ | R1-1703505 | 4 | Correction on single-layer precoding for EPCCH | 1.0.2 | 1.0.3 |
| 2017-09 | MFA #12 | ‒ | R1-1706589 | 4 | Clarification and correction on IFDMA UL-DMRS for eFD-MIMO | 1.0.2 | 1.0.3 |
| 2017-09 | MFA #12 | ‒ | R1-1706751 | 4 | Clarification on CDM-8 pattern for 24-ports CSI-RS in DwPTS | 1.0.2 | 1.0.3 |
| 2017-09 | MFA #12 | ‒ | R1-1709648 | 4 | Precoding for spatial multiplexing using antenna ports with UE-specific reference signals | 1.0.2 | 1.0.3 |
| 2017-09 | MFA #12 | ‒ | R1-1709649 | 4 | Correction to CSI-RS configuration | 1.0.2 | 1.0.3 |