Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016-2017, MulteFire Alliance (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

MFA TS 36.413 V1.0.3 (2017-09)

Technical Specification

MulteFire Alliance

S1 Application Protocol (S1AP)

Release 1.0

The present document has been developed within MulteFire Alliance and may be further elaborated for the purposes of the MulteFire Alliance.
The MulteFire Alliance accepts no liability for any use of this Specification.

Keywords

MF, multefire, radio, layer 1

MulteFire Alliance

Postal address

5177 Brandin Court

Fremont, CA 94538 USA

Internet

http://www.multefire.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016-2017, MulteFire Alliance

This specification is a modification of 3GPP TS 36.413 13.5.0, © 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC) from which permission has been received.

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	13

1	Scope	14

2	References	14

3	Definitions and abbreviations	16

3.1	Definitions	16

3.2	Abbreviations	17

4	General	18

4.1	Procedure Specification Principles	18

4.2	Forwards and Backwards Compatibility	19

4.3	Specification Notations	19

5	S1AP Services	20

6	Services Expected from Signalling Transport	21

7	Functions of S1AP	22

8	S1AP Procedures	24

8.1	List of S1AP Elementary procedures	24

8.2	E-RAB Management procedures	26

8.2.1	E-RAB Setup	26

8.2.1.1	General	26

8.2.1.2	Successful Operation	26

8.2.1.3	Unsuccessful Operation	27

8.2.1.4	Abnormal Conditions	28

8.2.2	E-RAB Modify	28

8.2.2.1	General	28

8.2.2.2	Successful Operation	28

8.2.2.3	Unsuccessful Operation	29

8.2.2.4	Abnormal Conditions	29

8.2.3	E-RAB Release	30

8.2.3.1	General	30

8.2.3.2	Successful Operation	30

8.2.3.3	Abnormal Conditions	31

8.2.4	E-RAB Modification Indication	31

8.2.4.1	General	31

8.2.4.2	Successful Operation	32

8.2.4.3	Unsuccessful Operation	32

8.2.4.4	Abnormal Conditions	33

8.3	Context Management procedures	33

8.3.1	Initial Context Setup	33

8.3.1.1	General	33

8.3.1.2	Successful Operation	33

8.3.1.3	Unsuccessful Operation	36

8.3.1.4	Abnormal Conditions	37

8.3.2	UE Context Release Request (eNB initiated)	37

8.3.2.1	General	37

8.3.2.2	Successful Operation	37

8.3.3	UE Context Release (MME initiated)	38

8.3.3.1	General	38

8.3.3.2	Successful Operation	38

8.3.3.3	Abnormal Conditions	38

8.3.4	UE Context Modification	39

8.3.4.1	General	39

8.3.4.2	Successful Operation	39

8.3.4.3	Unsuccessful Operation	40

8.3.4.4	Abnormal Conditions	41

8.3.5	UE Radio Capability Match	41

8.3.5.1	General	41

8.3.5.2	Successful Operation	41

8.3.5.3	Unsuccessful Operation	41

8.3.5.4	Abnormal Conditions	41

8.3.6	UE Context Modification Indication	41

8.3.6.1	General	41

8.3.6.2	Successful Operation	42

8.3.6.3	Unsuccessful Operation	42

8.3.6.4	Abnormal Conditions	42

8.3.7	UE Context Suspend	42

8.3.7.1	General	42

8.3.7.2	Successful Operation	43

8.3.8	UE Context Resume	43

8.3.8.1	General	43

8.3.8.2	Successful Operation	43

8.3.8.3	Unsuccessful Operation	44

8.3.9	Connection Establishment Indication	44

8.3.9.1	General	44

8.3.9.2	Successful Operation	44

8.3.9.3	Unsuccessful Operation	44

8.3.9.4	Abnormal Conditions	44

8.4	Handover Signalling	45

8.4.1	Handover Preparation	45

8.4.1.1	General	45

8.4.1.2	Successful Operation	45

8.4.1.3	Unsuccessful Operation	48

8.4.1.4	Abnormal Conditions	49

8.4.2	Handover Resource Allocation	49

8.4.2.1	General	49

8.4.2.2	Successful Operation	49

8.4.2.3	Unsuccessful Operation	52

8.4.2.4	Abnormal Conditions	52

8.4.3	Handover Notification	53

8.4.3.1	General	53

8.4.3.2	Successful Operation	53

8.4.3.3	Abnormal Conditions	53

8.4.4	Path Switch Request	53

8.4.4.1	General	53

8.4.4.2	Successful Operation	53

8.4.4.3	Unsuccessful Operation	55

8.4.4.4	Abnormal Conditions	55

8.4.5	Handover Cancellation	55

8.4.5.1	General	55

8.4.5.2	Successful Operation	56

8.4.5.3	Unsuccessful Operation	56

8.4.5.4	Abnormal Conditions	56

8.4.6	eNB Status Transfer	56

8.4.6.1	General	56

8.4.6.2	Successful Operation	56

8.4.6.3	Unsuccessful Operation	57

8.4.6.4	Abnormal Conditions	57

8.4.7	MME Status Transfer	57

8.4.7.1	General	57

8.4.7.2	Successful Operation	57

8.4.7.3	Unsuccessful Operation	58

8.4.7.4	Abnormal Conditions	58

8.5	Paging	58

8.5.1	General	58

8.5.2	Successful Operation	58

8.5.3	Unsuccessful Operation	59

8.5.4	Abnormal Conditions	59

8.6	NAS transport	59

8.6.1	General	59

8.6.2	Successful Operations	60

8.6.2.1	Initial UE Message	60

8.6.2.2	DOWNLINK NAS TRANSPORT	61

8.6.2.3	UPLINK NAS TRANSPORT	62

8.6.2.4	NAS NON DELIVERY INDICATION	62

8.6.2.5	Reroute NAS Request	63

8.6.3	Unsuccessful Operation	63

8.6.4	Abnormal Conditions	63

8.7	Management procedures	63

8.7.1	Reset	63

8.7.1.1	General	63

8.7.1.2	Successful Operation	64

8.7.1.3	Abnormal Conditions	65

8.7.2	Error Indication	66

8.7.2.1	General	66

8.7.2.2	Successful Operation	66

8.7.2.3	Abnormal Conditions	67

8.7.3	S1 Setup	67

8.7.3.1	General	67

8.7.3.2	Successful Operation	67

8.7.3.3	Unsuccessful Operation	68

8.7.3.4	Abnormal Conditions	68

8.7.4	eNB Configuration Update	68

8.7.4.1	General	68

8.7.4.2	Successful Operation	68

8.7.4.3	Unsuccessful Operation	69

8.7.4.4	Abnormal Conditions	69

8.7.5	MME Configuration Update	69

8.7.5.1	General	69

8.7.5.2	Successful Operation	69

8.7.5.3	Unsuccessful Operation	70

8.7.5.4	Abnormal Conditions	70

8.7.6	Overload Start	70

8.7.6.1	General	70

8.7.6.2	Successful Operation	71

8.7.6.3	Unsuccessful Operation	71

8.7.7	Overload Stop	72

8.7.7.1	General	72

8.7.7.2	Successful Operation	72

8.7.7.3	Unsuccessful Operation	72

8.8	S1 CDMA2000 Tunnelling Procedures	72

8.8.1	General	72

8.8.2	Successful Operations	73

8.8.2.1	Downlink S1 CDMA2000 Tunnelling	73

8.8.2.2	Uplink S1 CDMA2000 Tunnelling	73

8.8.3	Unsuccessful Operation	74

8.8.4	Abnormal Conditions	74

8.9	UE Capability Info Indication	74

8.9.1	General	74

8.9.2	Successful Operation	74

8.10	Trace Procedures	75

8.10.1	Trace Start	75

8.10.1.1	General	75

8.10.1.2	Successful Operation	75

8.10.2	Trace Failure Indication	75

8.10.2.1	General	75

8.10.2.2	Successful Operation	76

8.10.3	Deactivate Trace	76

8.10.3.1	General	76

8.10.3.2	Successful Operation	76

8.10.4	Cell Traffic Trace	76

8.10.4.1	General	76

8.10.4.2	Successful Operation	77

8.11	Location Reporting Procedures	77

8.11.1	Location Reporting Control	77

8.11.1.1	General	77

8.11.1.2	Successful Operation	77

8.11.1.3	Abnormal Conditions	77

8.11.2	Location Report Failure Indication	78

8.11.2.1	General	78

8.11.2.2	Successful Operation	78

8.11.3	Location Report	78

8.11.3.1	General	78

8.11.3.2	Successful Operation	78

8.11.3.3	Abnormal Conditions	78

8.12	Warning Message Transmission Procedures	78

8.12.1	Write-Replace Warning	78

8.12.1.1	General	78

8.12.1.2	Successful Operation	79

8.12.1.3	Abnormal Conditions	80

8.12.2	Kill	80

8.12.2.1	General	80

8.12.2.2	Successful Operation	80

8.12.3	PWS Restart Indication	81

8.12.3.1	General	81

8.12.3.2	Successful Operation	81

8.12.4	PWS Failure Indication	81

8.12.4.1	General	81

8.12.4.2	Successful Operation	81

8.13	eNB Direct Information Transfer	81

8.13.1	General	81

8.13.2	Successful Operation	82

8.13.2.1	eNB Direct Information Transfer	82

8.13.3	Abnormal Conditions	82

8.14	MME Direct Information Transfer	82

8.14.1	General	82

8.14.2	Successful Operation	82

8.14.2.1	MME Direct Information Transfer	82

8.14.3	Abnormal Conditions	83

8.15	eNB Configuration Transfer	83

8.15.1	General	83

8.15.2	Successful Operation	83

8.15.2.1	eNB Configuration Transfer	83

8.15.3	Abnormal Conditions	83

8.16	MME Configuration Transfer	83

8.16.1	General	83

8.16.2	Successful Operation	83

8.16.2.1	MME Configuration Transfer	83

8.16.3	Abnormal Conditions	84

8.17	LPPa transport	84

8.17.1	General	84

8.17.2	Successful Operations	85

8.17.2.1	DOWNLINK UE ASSOCIATED LPPA TRANSPORT	85

8.17.2.2	UPLINK UE ASSOCIATED LPPA TRANSPORT	85

8.17.2.3	DOWNLINK NON UE ASSOCIATED LPPA TRANSPORT	85

8.17.2.4	UPLINK NON UE ASSOCIATED LPPA TRANSPORT	86

8.17.3	Unsuccessful Operation	86

8.17.4	Abnormal Conditions	86

9	Elements for S1AP Communication	87

9.1	Message Functional Definition and Content	87

9.1.1	General	87

9.1.2	Message Contents	87

9.1.2.1	Presence	87

9.1.2.2	Criticality	87

9.1.2.3	Range	87

9.1.2.4	Assigned Criticality	87

9.1.3	E-RAB Management Messages	87

9.1.3.1	E-RAB SETUP REQUEST	87

9.1.3.2	E-RAB SETUP RESPONSE	88

9.1.3.3	E-RAB MODIFY REQUEST	89

9.1.3.4	E-RAB MODIFY RESPONSE	90

9.1.3.5	E-RAB RELEASE COMMAND	90

9.1.3.6	E-RAB RELEASE RESPONSE	91

9.1.3.7	E-RAB RELEASE INDICATION	91

9.1.3.8	E-RAB MODIFICATION INDICATION	92

9.1.3.9	E-RAB MODIFICATION CONFIRM	93

9.1.4	Context Management Messages	93

9.1.4.1	INITIAL CONTEXT SETUP REQUEST	93

9.1.4.2	Void	94

9.1.4.3	INITIAL CONTEXT SETUP RESPONSE	94

9.1.4.4	INITIAL CONTEXT SETUP FAILURE	95

9.1.4.5	UE CONTEXT RELEASE REQUEST	95

9.1.4.6	UE CONTEXT RELEASE COMMAND	96

9.1.4.7	UE CONTEXT RELEASE COMPLETE	96

9.1.4.8	UE CONTEXT MODIFICATION REQUEST	96

9.1.4.9	UE CONTEXT MODIFICATION RESPONSE	97

9.1.4.10	UE CONTEXT MODIFICATION FAILURE	97

9.1.4.11	UE RADIO CAPABILITY MATCH REQUEST	97

9.1.4.12	UE RADIO CAPABILITY MATCH RESPONSE	98

9.1.4.13	UE CONTEXT MODIFICATION INDICATION	98

9.1.4.14	UE CONTEXT MODIFICATION CONFIRM	98

9.1.4.15	UE CONTEXT SUSPEND REQUEST	98

9.1.4.16	UE CONTEXT SUSPEND RESPONSE	99

9.1.4.17	UE CONTEXT RESUME REQUEST	99

9.1.4.18	UE CONTEXT RESUME RESPONSE	99

9.1.4.19	UE CONTEXT RESUME FAILURE	100

9.1.4.20	Connection Establishment Indication	100

9.1.5	Handover Signalling Messages	101

9.1.5.1	HANDOVER REQUIRED	101

9.1.5.2	HANDOVER COMMAND	102

9.1.5.3	HANDOVER PREPARATION FAILURE	103

9.1.5.4	HANDOVER REQUEST	104

9.1.5.5	HANDOVER REQUEST ACKNOWLEDGE	105

9.1.5.6	HANDOVER FAILURE	105

9.1.5.7	HANDOVER NOTIFY	106

9.1.5.8	PATH SWITCH REQUEST	107

9.1.5.9	PATH SWITCH REQUEST ACKNOWLEDGE	108

9.1.5.10	PATH SWITCH REQUEST FAILURE	108

9.1.5.11	HANDOVER CANCEL	109

9.1.5.12	HANDOVER CANCEL ACKNOWLEDGE	109

9.1.5.13	eNB STATUS TRANSFER	109

9.1.5.14	MME STATUS TRANSFER	109

9.1.6	PAGING	109

9.1.7	NAS Transport Messages	111

9.1.7.1	INITIAL UE MESSAGE	111

9.1.7.2	DOWNLINK NAS TRANSPORT	112

9.1.7.3	UPLINK NAS TRANSPORT	112

9.1.7.4	NAS NON DELIVERY INDICATION	112

9.1.7.5	REROUTE NAS REQUEST	113

9.1.8	Management messages	113

9.1.8.1	RESET	113

9.1.8.2	RESET ACKNOWLEDGE	113

9.1.8.3	ERROR INDICATION	114

9.1.8.4	S1 SETUP REQUEST	114

9.1.8.5	S1 SETUP RESPONSE	115

9.1.8.6	S1 SETUP FAILURE	116

9.1.8.7	ENB CONFIGURATION UPDATE	116

9.1.8.8	ENB CONFIGURATION UPDATE ACKNOWLEDGE	117

9.1.8.9	ENB CONFIGURATION UPDATE FAILURE	117

9.1.8.10	MME CONFIGURATION UPDATE	117

9.1.8.11	MME CONFIGURATION UPDATE ACKNOWLEDGE	118

9.1.8.12	MME CONFIGURATION UPDATE FAILURE	118

9.1.8.13	OVERLOAD START	118

9.1.8.14	OVERLOAD STOP	119

9.1.9	S1 CDMA2000 Tunnelling Messages	119

9.1.9.1	DOWNLINK S1 CDMA2000 TUNNELLING	119

9.1.9.2	UPLINK S1 CDMA2000 TUNNELLING	120

9.1.10	UE CAPABILITY INFO INDICATION	120

9.1.11	Trace Messages	120

9.1.11.1	TRACE START	120

9.1.11.2	TRACE FAILURE INDICATION	120

9.1.11.3	DEACTIVATE TRACE	121

9.1.12	Location Reporting Messages	121

9.1.12.1	LOCATION REPORTING CONTROL	121

9.1.12.2	LOCATION REPORT FAILURE INDICATION	121

9.1.12.3	LOCATION REPORT	121

9.1.13	Warning Message Transmission Messages	122

9.1.13.1	WRITE-REPLACE WARNING REQUEST	122

9.1.13.2	WRITE-REPLACE WARNING RESPONSE	122

9.1.13.3	KILL REQUEST	122

9.1.13.4	KILL RESPONSE	123

9.1.13.5	PWS RESTART INDICATION	123

9.1.13.6	PWS FAILURE INDICATION	123

9.1.14	eNB DIRECT INFORMATION TRANSFER	124

9.1.15	MME DIRECT INFORMATION TRANSFER	124

9.1.16	eNB CONFIGURATION TRANSFER	124

9.1.17	MME CONFIGURATION TRANSFER	124

9.1.18	CELL TRAFFIC TRACE	125

9.1.19	LPPa Transport Messages	125

9.1.19.1	DOWNLINK UE ASSOCIATED LPPA TRANSPORT	125

9.1.19.2	UPLINK UE ASSOCIATED LPPA TRANSPORT	125

9.1.19.3	DOWNLINK NON UE ASSOCIATED LPPA TRANSPORT	126

9.1.19.4	UPLINK NON UE ASSOCIATED LPPA TRANSPORT	126

9.2	Information Element Definitions	126

9.2.0	General	126

9.2.1	Radio Network Layer Related IEs	126

9.2.1.1	Message Type	126

9.2.1.2	E-RAB ID	127

9.2.1.3	Cause	127

9.2.1.3a	RRC Establishment Cause	131

9.2.1.4	Trace Activation	131

9.2.1.5	Source ID	132

9.2.1.6	Target ID	132

9.2.1.7	Source eNB to Target eNB Transparent Container	133

9.2.1.8	Target eNB to Source eNB Transparent Container	134

9.2.1.9	Source RNC to Target RNC Transparent Container	134

9.2.1.10	Target RNC to Source RNC Transparent Container	134

9.2.1.11	Source BSS to Target BSS Transparent Container	134

9.2.1.12	Target BSS to Source BSS Transparent Container	134

9.2.1.13	Handover Type	134

9.2.1.14	Extended RNC-ID	134

9.2.1.15	E-RAB Level QoS Parameters	135

9.2.1.16	Paging DRX	135

9.2.1.17	Paging Cause	135

9.2.1.18	GBR QoS Information	135

9.2.1.19	Bit Rate	136

9.2.1.20	UE Aggregate Maximum Bit Rate	136

9.2.1.21	Criticality Diagnostics	136

9.2.1.22	Handover Restriction List	137

9.2.1.23	CDMA2000-PDU	138

9.2.1.24	CDMA2000 RAT Type	138

9.2.1.25	CDMA2000 Sector ID	139

9.2.1.26	Security Context	139

9.2.1.27	UE Radio Capability	140

9.2.1.28	CDMA2000 HO Status	140

9.2.1.29	CDMA2000 HO Required Indication	140

9.2.1.30	1xRTT MEID	140

9.2.1.31	eNB Status Transfer Transparent Container	140

9.2.1.32	COUNT Value	142

9.2.1.33	CDMA2000 1xRTT RAND	142

9.2.1.34	Request Type	142

9.2.1.35	CDMA2000 1xRTT SRVCC Info	143

9.2.1.36	E-RAB List	143

9.2.1.37	Global eNB ID	144

9.2.1.38	E-UTRAN CGI	144

9.2.1.39	Subscriber Profile ID for RAT/Frequency priority	144

9.2.1.40	UE Security Capabilities	144

9.2.1.41	Security Key	145

9.2.1.42	UE History Information	145

9.2.1.43	Last Visited Cell Information	145

9.2.1.43a	Last Visited E-UTRAN Cell Information	146

9.2.1.43b	Last Visited GERAN Cell Information	146

9.2.1.44	Message Identifier	146

9.2.1.45	Serial Number	147

9.2.1.46	Warning Area List	147

9.2.1.47	Emergency Area ID	147

9.2.1.48	Repetition Period	147

9.2.1.49	Number of Broadcasts Requested	148

9.2.1.50	Warning Type	148

9.2.1.51	Warning Security Information	148

9.2.1.52	Data Coding Scheme	148

9.2.1.53	Warning Message Contents	148

9.2.1.54	Broadcast Completed Area List	148

9.2.1.55	Inter-system Information Transfer Type	149

9.2.1.56	Source To Target Transparent Container	149

9.2.1.57	Target To Source Transparent Container	150

9.2.1.58	SRVCC Operation Possible	151

9.2.1.59	SRVCC HO Indication	151

9.2.1.60	Allocation and Retention Priority	151

9.2.1.61	Time to wait	152

9.2.1.62	CSG Id	152

9.2.1.63	CSG Id List	152

9.2.1.64	MS Classmark 2	153

9.2.1.65	MS Classmark 3	153

9.2.1.66	Cell Type	153

9.2.1.67	Old BSS to New BSS Information	153

9.2.1.68	Layer 3 Information	153

9.2.1.69	E-UTRAN Round Trip Delay Estimation Info	153

9.2.1.70	Broadcast Cancelled Area List	153

9.2.1.71	Number of Broadcasts	154

9.2.1.72	Concurrent Warning Message Indicator	154

9.2.1.73	CSG Membership Status	155

9.2.1.74	Cell Access Mode	155

9.2.1.75	Extended Repetition Period	155

9.2.1.76	Data Forwarding Not Possible	155

9.2.1.77	PS Service Not Available	155

9.2.1.78	Paging Priority	155

9.2.1.79	Relay Node Indicator	155

9.2.1.80	Correlation ID	156

9.2.1.81	MDT Configuration	156

9.2.1.82	MME Relay Support Indicator	159

9.2.1.83	Management Based MDT Allowed	159

9.2.1.84	GW Context Release Indication	159

9.2.1.85	Voice Support Match Indicator	160

9.2.1.86	M3 Configuration	160

9.2.1.87	M4 Configuration	160

9.2.1.88	M5 Configuration	160

9.2.1.89	MDT PLMN List	160

9.2.1.90	COUNT Value Extended	161

9.2.1.91	Kill-all Warning Messages Indicator	161

9.2.1.92	LHN ID	161

9.2.1.93	User Location Information	161

9.2.1.94	MBSFN-ResultToLog	161

9.2.1.95	EARFCN	162

9.2.1.96	Expected UE Behaviour	162

9.2.1.97	Expected UE Activity Behaviour	162

9.2.1.98	UE Radio Capability for Paging	163

9.2.1.99	ProSe Authorized	163

9.2.1.100	COUNT Value for PDCP SN Length 18	163

9.2.1.101	M6 Configuration	163

9.2.1.102	M7 Configuration	164

9.2.1.103	Assistance Data for Paging	164

9.2.1.104	Assistance Data for Recommended Cells	164

9.2.1.105	Information on Recommended Cells and eNBs for Paging	164

9.2.1.106	Recommended Cells for Paging	165

9.2.1.107	Recommended eNBs for Paging	165

9.2.1.108	Assistance Data for CE capable UEs	166

9.2.1.109	Cell Identifier and Coverage Enhancement Level	166

9.2.1.110	Paging Attempt Information	166

9.2.1.111	Paging eDRX Information	166

9.2.1.112	UE Retention Information	167

9.2.1.113	UE User Plane CIoT Support Indicator	167

9.2.1.114	NB-IoT Default Paging DRX	167

9.2.1.115	NB-IoT Paging eDRX Information	167

9.2.1.116	Bearer Type	167

9.2.1.117	RAT Type	167

9.2.1.118	CE-mode-B Support Indicator	168

9.2.1.119	SRVCC Operation Not Possible	168

9.2.2	Transport Network Layer Related IEs	169

9.2.2.1	Transport Layer Address	169

9.2.2.2	GTP-TEID	169

9.2.2.3	Tunnel Information	169

9.2.3	NAS Related IEs	169

9.2.3.1	LAI	169

9.2.3.2	RAC	169

9.2.3.3	MME UE S1AP ID	170

9.2.3.4	eNB UE S1AP ID	170

9.2.3.5	NAS-PDU	170

9.2.3.6	S-TMSI	170

9.2.3.7	TAC	170

9.2.3.8	PLMN Identity	170

9.2.3.9	GUMMEI	171

9.2.3.10	UE Identity Index value	171

9.2.3.11	IMSI	171

9.2.3.12	MMEC	171

9.2.3.13	UE Paging Identity	172

9.2.3.14	DL Forwarding	172

9.2.3.15	Direct Forwarding Path Availability	172

9.2.3.16	TAI	172

9.2.3.17	Relative MME Capacity	172

9.2.3.18	UE S1AP ID pair	173

9.2.3.19	Overload Response	173

9.2.3.20	Overload Action	173

9.2.3.21	CS Fallback Indicator	173

9.2.3.22	CN Domain	173

9.2.3.23	RIM Transfer	174

9.2.3.24	RIM Information	174

9.2.3.25	RIM Routing Address	174

9.2.3.26	SON Configuration Transfer	174

9.2.3.27	SON Information	175

9.2.3.28	SON Information Reply	175

9.2.3.29	X2 TNL Configuration Info	176

9.2.3.30	NAS Security Parameters from E-UTRAN	176

9.2.3.31	NAS Security Parameters to E-UTRAN	177

9.2.3.32	LPPa-PDU	177

9.2.3.33	Routing ID	177

9.2.3.34	Time Synchronisation Info	177

9.2.3.35	Void	178

9.2.3.36	Traffic Load Reduction Indication	178

9.2.3.37	Additional CS Fallback Indicator	178

9.2.3.38	Masked IMEISV	178

9.2.3.39	SON Information Report	178

9.2.3.40	RLF Report Information	178

9.2.3.41	Muting Pattern Information	179

9.2.3.42	Synchronisation Information	179

9.2.3.43	Listening Subframe Pattern	179

9.2.3.44	MME Group ID	180

9.2.3.45	Additional GUTI	180

9.2.3.46	Extended UE Identity Index Value	180

9.2.3.47	NB-IoT UE Identity Index Value	180

9.2.3.47MF1	  PSP Info	180

9.3	Message and Information Element Abstract Syntax (with ASN.1)	182

9.3.0	General	182

9.3.1	Usage of private message mechanism for non-standard use	182

9.3.2	Elementary Procedure Definitions	183

9.3.3	PDU Definitions	192

9.3.4	Information Element Definitions	236

9.3.5	Common Definitions	266

9.3.6	Constant Definitions	266

9.3.7	Container Definitions	271

9.4	Message Transfer Syntax	274

9.5	Timers	274

10	Handling of Unknown, Unforeseen and Erroneous Protocol Data	275

10.1	General	275

10.2	Transfer Syntax Error	275

10.3	Abstract Syntax Error	275

10.3.1	General	275

10.3.2	Criticality Information	276

10.3.3	Presence Information	276

10.3.4	Not comprehended IE/IE group	277

10.3.4.1	Procedure Code	277

10.3.4.1A	Type of Message	277

10.3.4.2	IEs other than the Procedure Code and Type of Message	277

10.3.5	Missing IE or IE group	278

10.3.6	IEs or IE groups received in wrong order or with too many occurrences or erroneously present	279

10.4	Logical Error	280

10.5	Exceptions	280

10.6	Handling of AP ID	280

Annex A (informative): S1AP Transparent containers content	282

Annex B (normative): IEs for SON Transfer	283

B.1	Tabular definition	283

B.1.1	SON Transfer Application Identity	283

B.1.2	SON Transfer Request Container	283

B.1.3	SON Transfer Response Container	284

B.1.4	SON Transfer Cause	285

B.1.5	Cell Load Reporting Response	287

B.1.6	E-UTRAN Cell Load Reporting Response	287

B.1.7	Multi-Cell Load Reporting Request	288

B.1.8	IRAT Cell ID	288

B.1.9	Multi-Cell Load Reporting Response	288

B.1.10	Cell Load Reporting Cause	289

B.1.11	Event-Triggered Cell Load Reporting Request	290

B.1.12	Event-triggered Cell Load Reporting Response	290

B.1.13	HO Report	290

B.1.14	Cell Activation Request	291

B.1.15	Cell Activation Response	291

B.1.16	Cell State Indication	292

B.1.17	Failure Event Report	292

B.1.18	eHRPD Sector ID	293

B.1.19	eHRPD Sector Load Reporting Response	293

B.1.20	eHRPD Composite Available Capacity	293

B.1.21	eHRPD Sector Capacity Class Value	293

B.1.22	eHRPD Capacity Value	293

B.1.23	Candidate PCI	294

B.2	ASN.1 definition	294

Annex C (informative): Processing of Transparent Containers at the MME	299

Annex D (informative): Change history	300

# Foreword

This Technical Specification has been produced by the MulteFire Alliance (MFA).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e., technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the E-UTRAN radio network layer signalling protocol for the S1 interface. The S1 Application Protocol (S1AP) supports the functions of S1 interface by signalling procedures defined in this document. S1AP is developed in accordance to the general principles stated in 3GPP TS 36.401 [2] and 3GPP TS 36.410 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document. For the interpretation of references, the rules described in Section 4 of MFA TR MF.100 [MF0] apply.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[3]	3GPP TS 36.410: "S1 General Aspects and Principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	Void

[8]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[9]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC)".

[10]	3GPP TS 32.422: "Trace control and configuration management".

[11]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for E-UTRAN access".

[12]	3GPP TS 36.414: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 data transport".

[13]	3GPP TS 23.203: "Policy and charging control architecture"

[14]	MFA TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[15]	MFA TS 33.401: "Security architecture".

[16]	MFA TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRAN); Radio Resource Control (RRC) Protocol Specification".

[17]	3GPP TS 23.272: "Circuit Switched Fallback in Evolved Packet System; Stage 2".

[18]	3GPP TS 48.018: "General Packet Radio Service (GPRS); BSS GPRS Protocol (BSSGP)".

[19]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[20]	MFA TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA), User Equipment (UE) procedures in idle mode".

[21]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[22]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[23]	3GPP TS 48.008: "Mobile Switching Centre-Base Station System (MSC-BSS) interface; Layer 3 specification".

[24]	MFA TS 24.301: "Non-Access Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[25]	3GPP2 A.S0008-C: "Interoperability Specification (IOS) for High Rate Packet Data (HRPD) Radio Access Network Interfaces with Session Control in the Access Network".

[26]	MFA TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[27]	3GPP2 C.S0024-B: "cdma2000 High Rate Packet Data Air Interface Specification".

[28]	3GPP TS 22.220: "Service requirements for Home Node Bs and Home eNode Bs".

[29]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[30] 	3GPP TS 48.016: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN) interface; Network service".

[31]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[32] 	3GPP TS 29.281: "General Packet Radio Service (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[33]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[34]	3GPP TS 36.455: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[35]	3GPP TS 29.060: "GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[36]	3GPP TS 29.274: "Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[37]	3GPP TS 23.139: "3GPP system – fixed broadband access network interworking".

[38]	3GPP TS 23.007: "Technical Specification Group Core Network Terminals; Restoration procedures".

[39] 	3GPP TS 36.104: "Base Station (BS) radio transmission and reception".

[40]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[41]	3GPP TS 36.306: "User Equipment (UE) radio access capabilities".

[MF0]	MFA TR MF.100: “Guidelines for MulteFire Specifications”

[MF1]	MFA TS MF.202: "Architecture for Neutral Host Network Access Mode (stage 2)(Release 1)"

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

In this document the MulteFire (MF) Access Point, as defined in MFA TS MF.202 [MF1], is equivalent with eNB.

In this document the NH (Neutral Host) MME, as defined in MFA TS MF.202 [MF1], is equivalent with MME.

Note: The mapping of the MF-AP and NH-MME terms to eNB and MME could be done in MFA TS 36.300 [14], if these changes should be incorporated by 3GPP.

ACL functionality: A functionality controlling the access to network nodes. In case of Access Control Lists (ACL) functionality is applied in a network node the network node may only accept connections from other peer network nodes once the source addresses of the sending network node is already known in the target node.

CSG Cell: an E-UTRAN cell broadcasting a CSG indicator set to true and a CSG identity. This cell operates in Closed Access Mode as defined in 3GPP TS 22.220 [28].

Dual Connectivity: as defined in MFA TS 36.300 [14].

Elementary Procedure: S1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between eNBs and the EPC. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several S1AP EPs together or together with EPs from other interfaces is specified in stage 2 specifications (e.g., 3GPP TS 23.401 [11] and MFA TS 36.300 [14]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

eNB UE S1AP ID: as defined in 3GPP TS 36.401 [2].

Hybrid Cell: an E-UTRAN cell broadcasting a CSG indicator set to false and a CSG identity. This cell operates in Hybrid Access Mode as defined in 3GPP TS 22.220 [28].

MME UE S1AP ID: as defined in 3GPP TS 36.401 [2].

E-RAB: as defined in 3GPP TS 36.401 [2].

NOTE 1:	The E-RAB is either a default E-RAB or a dedicated E-RAB.

E-RAB ID: the E-RAB ID uniquely identifies an E-RAB for one UE.

NOTE 2:	The E-RAB ID remains unique for the UE even if the UE-associated logical S1-connection is released during periods of user inactivity.

Data Radio Bearer: the Data Radio bearer transports the packets of an E-RAB between a UE and an eNB. There is a one-to-one mapping between the E-RAB and the Data Radio Bearer.

Secondary Cell Group: as defined in MFA TS 36.300 [14].

UE-associated signalling: When S1-AP messages associated to one UE uses the UE-associated logical S1-connection for association of the message to the UE in eNB and EPC.

UE-associated logical S1-connection: The UE-associated logical S1-connection uses the identities MME UE S1AP ID and eNB UE S1AP ID according to definition in 3GPP TS 23.401 [11]. For a received UE associated S1-AP message the MME identifies the associated UE based on the MME UE S1AP ID IE and the eNB identifies the associated UE based on the eNB UE S1AP ID IE. The UE-associated logical S1-connection may exist before the S1 UE context is setup in eNB.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACL	Access Control List

BBF	Broadband Forum

CCO	Cell Change Order

CDMA	Code Division Multiple Access

CID	Cell-ID (positioning method)

CIoT	Cellular Internet of Things

CS	Circuit Switched

CSG	Closed Subscriber Group

CN	Core Network

DCN	Dedicated Core Network

DL	Downlink

eAN	evolved Access Network

ECGI	E-UTRAN Cell Global Identifier

E-CID	Enhanced Cell-ID (positioning method)

eHRPD	evolved High Rate Packet Data

eNB	E-UTRAN NodeB

EP	Elementary Procedure

EPC	Evolved Packet Core

EPS	Evolved Packet System

E-RAB	E-UTRAN Radio Access Bearer

E-SMLC	Evolved Serving Mobile Location Centre

E-UTRAN	Evolved UTRAN

GBR	Guaranteed Bit Rate

GNSS	Global Navigation Satellite System

GUMMEI	Globally Unique MME Identifier

GTP	GPRS Tunnelling Protocol

HFN	Hyper Frame Number

HRPD	High Rate Packet Data

IE	Information Element

IoT	Internet of Things

L-GW	Local GateWay

LHN	Local Home Network

LHN ID	Local Home Network ID

LIPA	Local IP Access

LPPa	LTE Positioning Protocol Annex

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MDT	Minimization of Drive Tests

MME	Mobility Management Entity

NAS	Non Access Stratum

NB-IoT	Narrowband IoT

NNSF	NAS Node Selection Function

OTDOA	Observed Time Difference of Arrival

PS	Packet Switched

ProSe	Proximity Services

PWS	Public Warning System

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PS	Packet Switched

RRC	Radio Resource Control

RIM	RAN Information Management

SCTP	Stream Control Transmission Protocol

SCG	Secondary Cell Group

S-GW	Serving GateWay

SN	Sequence Number

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

S-TMSI	S-Temporary Mobile Subscriber Identity

TAI	Tracking Area Identity

TEID	Tunnel Endpoint Identifier

UE	User Equipment

UE-AMBR	UE-Aggregate Maximum Bitrate

UL	Uplink

UTDOA	Uplink Time Difference of Arrival

# 4	General

## 4.1	Procedure Specification Principles

The principle for specifying the procedure logic is to specify the functional behaviour of the terminating node exactly and completely. Any rule that specifies the behaviour of the originating node shall be possible to be verified with information that is visible within the system.

The following specification principles have been applied for the procedure text in clause 8:

-	The procedure text discriminates between:

1)	Functionality which “shall” be executed

The procedure text indicates that the receiving node “shall” perform a certain function Y under a certain condition. If the receiving node supports procedure X but cannot perform functionality Y requested in the REQUEST message of a Class 1 EP, the receiving node shall respond with the message used to report unsuccessful outcome for this procedure, containing an appropriate cause value.

2)	Functionality which “shall, if supported” be executed

The procedure text indicates that the receiving node “shall, if supported,” perform a certain function Y under a certain condition. If the receiving node supports procedure X, but does not support functionality Y, the receiving node shall proceed with the execution of the EP, possibly informing the requesting node about the not supported functionality.

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included. For requirements on including Criticality Diagnostics IE, see clause 10.

## 4.2	Forwards and Backwards Compatibility

The forwards and backwards compatibility of the protocol is assured by mechanism where all current and future messages, and IEs or groups of related IEs, include ID and criticality fields that are coded in a standard format that will not be changed in the future. These parts can always be decoded regardless of the standard version.

## 4.3	Specification Notations

For the purposes of the present document, the following notations apply:

Procedure	When referring to an elementary procedure in the specification the Procedure Name is written with the first letters in each word in upper case characters followed by the word “procedure”, e.g., E-RAB procedure.

Message	When referring to a message in the specification the MESSAGE NAME is written with all letters in upper case characters followed by the word “message”, e.g., MESSAGE NAME message.

IE	When referring to an information element (IE) in the specification the Information Element Name is written with the first letters in each word in upper case characters and all letters in Italic font followed by the abbreviation “IE”, e.g., Information Element IE.

Value of an IE	When referring to the value of an information element (IE) in the specification the “Value” is written as it is specified in subclause 9.2 enclosed by quotation marks, e.g., “Value”.

# 5	S1AP Services

S1AP provides the signalling service between E-UTRAN and the evolved packet core (EPC) that is required to fulfil the S1AP functions described in clause 7. S1AP services are divided into two groups:

Non UE-associated services:	They are related to the whole S1 interface instance between the eNB and MME utilising a non UE-associated signalling connection.

UE-associated services:	They are related to one UE. S1AP functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UE in question.

# 6	Services Expected from Signalling Transport

The signalling connection shall provide in sequence delivery of S1AP messages. S1AP shall be notified if the signalling connection breaks.

# 7	Functions of S1AP

The S1AP protocol has the following functions:

-	E-RAB management function: This overall functionality is responsible for setting up, modifying and releasing E-RABs, which are triggered by the MME. The release and modification of E-RABs may be triggered by the eNB as well.

-	Initial Context Transfer function: This functionality is used to establish an S1UE context in the eNB, to setup the default IP connectivity, to setup one or more E-RAB(s) if requested by the MME, and to transfer NAS signalling related information to the eNB if needed.

-	UE Capability Info Indication function: This functionality is used to provide the UE Capability Info when received from the UE to the MME.

-	Mobility Functions for UEs in LTE_ACTIVE in order to enable

-	a change of eNBs within SAE/LTE (Inter MME/Serving SAE-GW Handovers) via the S1 interface (with EPC involvement).

-	a change of RAN nodes between different RATs (Inter-3GPP-RAT Handovers) via the S1 interface (with EPC involvement).

-	Paging: This functionality provides the EPC with the capability to page the UE.

-	S1 interface management functions comprise the:

-	Reset functionality to ensure a well defined initialisation on the S1 interface.

-	Error Indication functionality to allow a proper error reporting/handling in cases where no failure messages are defined.

-	Overload function to indicate the load situation in the control plane of the S1 interface.

-	Load balancing function to ensure equally loaded MMEs within an MME pool area

-	S1 Setup functionality for initial S1 interface setup for providing configuration information

-	eNB and MME Configuration Update functions are to update application level configuration data needed for the eNB and MME to interoperate correctly on the S1 interface.

-	NAS Signalling transport function between the UE and the MME is used:

-	to transfer and reroute NAS signalling related information and to establish the S1 UE context in the eNB.

-	to transfer NAS signalling related information when the S1 UE context in the eNB is already established.

-	S1 UE context Release function: This functionality is responsible to manage the release of UE specific context in the eNB and the MME.

-	UE Context Modification function: This functionality allows to modify the established UE Context partly.

-	UE Context Resumption function: This functionality allows keeping the UE Context in the eNB for a UE in RRC_IDLE that has been enabled to use User Plane EPS Optimization (see 3GPP TS 23.401 [11]) and to resume the RRC connection without the need to re-establish the UE Context.

-	Status Transfer: This functionality transfers PDCP SN Status information from source eNB to target eNB in support of in-sequence delivery and duplication avoidance for intra LTE handover.

-	Trace function: This functionality is to control a trace session recording for a UE in ECM_CONNECTED or to control an MDT session transferring MDT measurements collected by the UE.

-	Location Reporting: This functionality allows MME to be aware of the UE’s current location.

-	LPPa Signalling transport:  This functionality transfers LPPa messages between eNB and E-SMLC over the S1 interface.

-	S1 CDMA2000 Tunnelling function: This functionality is to carry CDMA2000 signalling between UE and CDMA2000 RAT over the S1 Interface.

-	Warning message transmission function:
This functionality provides the means to start and overwrite the broadcasting of warning message.

-	RAN Information Management (RIM) function: This functionality allows the request and transfer of RAN information (e.g., GERAN system information) between two RAN nodes via the core network.

-	Configuration Transfer function: This functionality allows the request and transfer of RAN configuration information (e.g., SON information) between two RAN nodes via the core network.

-	UE Radio Capability Match function. The functionality enables the eNB to derive and provide an indication to the MME whether the UE radio capabilities are compatible with the network configuration for voice continuity.

-	PWS Restart Indication function. The functionality enables the eNB to inform the MME that PWS information for some or all cells of the eNB are available for reloading from the CBC if needed.

-	PWS Failure Indication function. The functionality enables the eNB to inform the MME that ongoing PWS operation for one or more cells of the eNB has failed.

-	Connection Establishment Indication function. The functionality enables the MME to complete the establishment of the UE-associated logical S1-connection.

# 8	S1AP Procedures

## 8.1	List of S1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| E-RAB Setup | E-RAB SETUP REQUEST | E-RAB SETUP RESPONSE |  |
| E-RAB Modify | E-RAB MODIFY REQUEST | E-RAB MODIFY RESPONSE |  |
| E-RAB Modification Indication | E-RAB MODIFICATION INDICATION | E-RAB MODIFICATION CONFIRM |  |
| E-RAB Release | E-RAB RELEASE COMMAND | E-RAB RELEASE RESPONSE |  |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| S1 Setup | S1 SETUP REQUEST | S1 SETUP RESPONSE | S1 SETUP FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| MME Configuration Update | MME CONFIGURATION UPDATE | MME CONFIGURAION UPDATE ACKNOWLEDGE | MME CONFIGURATION UPDATE FAILURE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| Kill | KILL REQUEST | KILL RESPONSE |  |
| UE Radio Capability Match | UE RADIO CAPABILITY MATCH REQUEST | UE RADIO CAPABILITY MATCH RESPONSE |  |
| UE Context Modification Indication | UE CONTEXT MODIFICATION INDICATION | UE CONTEXT MODIFICATION CONFIRM |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE |  |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Handover Notification | HANDOVER NOTIFY |
| E-RAB Release Indication | E-RAB RELEASE INDICATION |
| Paging | PAGING |
| Initial UE Message | INITIAL UE MESSAGE |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| NAS non delivery indication | NAS NON DELIVERY INDICATION |
| Error Indication | ERROR INDICATION |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| DownlinkS1 CDMA2000 Tunnelling | DOWNLINK S1 CDMA2000 TUNNELLING |
| Uplink S1 CDMA2000 Tunnelling | UPLINK S1 CDMA2000 TUNNELLING |
| UE Capability Info Indication | UE CAPABILITY INFO INDICATION |
| eNB Status Transfer | eNB STATUS TRANSFER |
| MME Status Transfer | MME STATUS TRANSFER |
| Deactivate Trace | DEACTIVATE TRACE |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| eNB Direct Information Transfer | eNB DIRECT INFORMATION TRANSFER |
| MME Direct Information Transfer | MME DIRECT INFORMATION TRANSFER |
| eNB Configuration Transfer | eNB CONFIGURATION TRANSFER |
| MME Configuration Transfer | MME CONFIGURATION TRANSFER |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Downlink UE Associated LPPa Transport | DOWNLINK UE ASSOCIATED LPPA TRANSPORT |
| Uplink UE Associated LPPa Transport | UPLINK UE ASSOCIATED LPPA TRANSPORT |
| Downlink Non UE Associated LPPa Transport | DOWNLINK NON UE ASSOCIATED LPPA TRANSPORT |
| Uplink Non UE Associated LPPa Transport | UPLINK NON UE ASSOCIATED LPPA TRANSPORT |
| PWS Restart Indication | PWS RESTART INDICATION |
| Reroute NAS Request | REROUTE NAS REQUEST |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |

The following applies concerning interference between Elementary Procedures:

-	The Reset procedure takes precedence over all other EPs.

- The UE Context Release procedure takes precedence over all other EPs that are using the UE-associated signalling.

## 8.2	E-RAB Management procedures

### 8.2.1	E-RAB Setup

#### 8.2.1.1	General

The purpose of the E-RAB Setup procedure is to assign resources on Uu and S1 for one or several E-RABs and to setup corresponding Data Radio Bearers for a given UE. The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: E-RAB Setup procedure. Successful operation.

The MME initiates the procedure by sending an E-RAB SETUP REQUEST message to the eNB.

-	The E-RAB SETUP REQUEST message shall contain the information required by the eNB to build the E-RAB configuration consisting of at least one E-RAB and for each E-RAB to setup include an E-RAB to be Setup Item IE.

Upon reception of the E-RAB SETUP REQUEST message, and if resources are available for the requested configuration, the eNB shall execute the requested E-RAB configuration. For each E-RAB and based on the E-RAB level QoS parameters IE the eNB shall establish a Data Radio Bearer and allocate the required resources on Uu. The eNB shall pass the NAS-PDU IE and the value contained in the E-RAB ID IE received for the E-RAB for each established Data Radio Bearer to the UE. The eNB does not send the NAS PDUs associated to the failed Data radio bearers to the UE. The eNB shall allocate the required resources on S1 for the E-RABs requested to be established.

If the Correlation ID IE is included in the E-RAB SETUP REQUEST message towards the eNB with L-GW function for LIPA operation, then the eNB shall use this information for LIPA operation for the concerned E-RAB.

If the SIPTO Correlation ID IE is included in the E-RAB SETUP REQUEST message towards the eNB with L-GW function for SIPTO@LN operation, then the eNB shall use this information for SIPTO@LN operation for the concerned E-RAB.

If the Bearer Type IE is included in the E-RAB SETUP REQUEST message and is set to “non IP”, then the eNB shall not perform header compression for the concerned E-RAB.

The E-RAB SETUP REQUEST message may contain

-	the UE Aggregate Maximum Bit Rate IE.

If the UE Aggregate Maximum Bit Rate IE is included in the E-RAB SETUP REQUEST the eNB shall

-	replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context;

-	use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the UE Aggregate Maximum Bit Rate IE is not contained in the E-RAB SETUP REQUEST message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

The eNB shall establish or modify the resources according to the values of the Allocation and Retention Priority IE (priority level and pre-emption indicators) and the resource situation as follows:

-	The eNB shall consider the priority level of the requested E-RAB, when deciding on the resource allocation.

-	The priority levels and the pre-emption indicators may (individually or in combination) be used to determine whether the E-RAB setup has to be performed unconditionally and immediately. If the requested E-RAB is marked as “may trigger pre-emption” and the resource situation requires so, the eNB may trigger the pre-emption procedure which may then cause the forced release of a lower priority E-RAB which is marked as “pre-emptable”. Whilst the process and the extent of the pre-emption procedure are operator-dependent, the pre-emption indicators shall be treated as follows:

1.	The values of the last received Pre-emption Vulnerability IE and Priority Level IE shall prevail.

2.	If the Pre-emption Capability IE is set to “may trigger pre-emption”, then this allocation request may trigger the pre-emption procedure.

3.	If the Pre-emption Capability IE is set to “shall not trigger pre-emption”, then this allocation request shall not trigger the pre-emption procedure.

4.	If the Pre-emption Vulnerability IE is set to “pre-emptable”, then this E-RAB shall be included in the pre-emption process.

5.	If the Pre-emption Vulnerability IE is set to “not pre-emptable”, then this E-RAB shall not be included in the pre-emption process.

6.	If the Priority Level IE is set to “no priority” the given values for the Pre-emption Capability IE and Pre-emption Vulnerability IE shall not be considered. Instead the values “shall not trigger pre-emption” and “not pre-emptable” shall prevail.

-	The E-UTRAN pre-emption process shall keep the following rules:

1.	E-UTRAN shall only pre-empt E-RABs with lower priority, in ascending order of priority.

2.	The pre-emption may be done for E-RABs belonging to the same UE or to other UEs.

The eNB shall report to the MME, in the E-RAB SETUP RESPONSE message, the result for all the requested E-RABs.

-	A list of E-RABs which are successfully established shall be included in the E-RAB Setup List IE.

-	A list of E-RABs which failed to be established, if any, shall be included in the E-RAB Failed to Setup List IE.

In case of the establishment of an E-RAB the EPC must be prepared to receive user data before the E-RAB SETUP RESPONSE message has been received.

When the eNB reports unsuccessful establishment of an E-RAB, the cause value should be precise enough to enable the MME to know the reason for an unsuccessful establishment, e.g., “Radio resources not available”, “Failure in the Radio Interface Procedure”.

Interactions with Handover Preparation procedure:

If a handover becomes necessary during E-RAB Setup, the eNB may interrupt the ongoing E-RAB Setup procedure and initiate the Handover Preparation procedure as follows:

1.	The eNB shall send the E-RAB SETUP RESPONSE message in which the eNB shall indicate, if necessary

-	all the E-RABs fail with an appropriate cause value, e.g., ”S1 intra system Handover triggered”, “S1 inter system Handover triggered” or “X2 Handover triggered”.

2.	The eNB shall trigger the handover procedure.

#### 8.2.1.3	Unsuccessful Operation

The unsuccessful operation is specified in the successful operation section.

#### 8.2.1.4	Abnormal Conditions

If the eNB receives a E-RAB SETUP REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in 3GPP TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the eNB shall consider the establishment of the corresponding E-RAB as failed.

If the eNB receives an E-RAB SETUP REQUEST message containing several E-RAB ID IEs (in the E-RAB To Be Setup List IE) set to the same value, the eNB shall report the establishment of the corresponding E-RABs as failed in the E-RAB SETUP RESPONSE with the appropriate cause value, e.g., “Multiple E-RAB ID instances”.

If the eNB receives an E-RAB SETUP REQUEST message containing a E-RAB ID IE (in the E-RAB To Be Setup List IE) set to the value that identifies an active E-RAB (established before the E-RAB SETUP REQUEST message was received), the eNB shall report the establishment of the new E-RAB as failed in the E-RAB SETUP RESPONSE with the appropriate cause value, e.g., “Multiple E-RAB ID instances”.

If the eNB receives an E-RAB SETUP REQUEST message containing both the Correlation ID and the SIPTO Correlation ID IEs for the same E-RAB, the eNB shall consider the establishment of the corresponding E-RAB as failed.

### 8.2.2	E-RAB Modify

#### 8.2.2.1	General

The purpose of the E-RAB Modify procedure is to enable modifications of already established E-RABs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: E-RAB Modify procedure. Successful operation.

The MME initiates the procedure by sending an E-RAB MODIFY REQUEST message to the eNB.

- The E-RAB MODIFY REQUEST message shall contain the information required by the eNB to modify one or several E-RABs of the existing E-RAB configuration.

Information shall be present in the E-RAB MODIFY REQUEST message only when any previously set value for the E-RAB configuration is requested to be modified.

Upon reception of the E-RAB MODIFY REQUEST message, and if resources are available for the requested target configuration, the eNB shall execute the modification of the requested E-RAB configuration. For each E-RAB that shall be modified and for which the Transport Information IE is not included and based on the new E-RAB level QoS parameters IE the eNB shall modify the Data Radio Bearer configuration and change allocation of resources on Uu according to the new resource request. The eNB shall pass the NAS-PDU IE received for the E-RAB to the UE when modifying the Data Radio Bearer configuration. The eNB does not send the NAS PDUs associated to the failed Data radio bearers to the UE. The eNB shall change allocation of resources on S1 according to the new resource request.

If the E-UTRAN failed to modify an E-RAB the E-UTRAN shall keep the E-RAB configuration as it was configured prior the E-RAB MODIFY REQUEST.

The E-RAB MODIFY REQUEST message may contain the

-	the UE Aggregate Maximum Bit Rate IE.

If the UE Aggregate Maximum Bit Rate IE is included in the E-RAB MODIFY REQUEST, the eNB shall

- replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context;

- use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the UE Aggregate Maximum Bit Rate IE is not contained in the E-RAB MODIFY REQUEST message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

The modification of resources according to the values of the Allocation and Retention Priority IE shall follow the principles described for the E-RAB Setup procedure.

If the Transport Information IE is included in the E-RAB MODIFY REQUEST message, the eNB shall use the included information as the new S-GW address and uplink packet destination for the relevant E-RAB as defined in 3GPP TS 23.401 [11], and it shall ignore the E-RAB Level QoS Parameters and NAS-PDU IEs for the same E-RAB.

The eNB shall report to the MME, in the E-RAB MODIFY RESPONSE message, the result for all the requested E-RABs to be modified.

-	A list of E-RABs which are successfully modified shall be included in the E-RAB Modify List IE.

-	A list of E-RABs which failed to be modified, if any, shall be included in the E-RAB Failed to Modify List IE.

When the eNB reports unsuccessful modification of an E-RAB, the cause value should be precise enough to enable the MME to know the reason for an unsuccessful modification, e.g., “Radio resources not available”, “Failure in the Radio Interface Procedure”.

In case of a modification of an E-RAB the EPC must be prepared to receive user data according to the modified E-RAB profile prior to the E-RAB MODIFY RESPONSE message.

Interactions with Handover Preparation procedure:

If a handover becomes necessary during E-RAB modify, the eNB may interrupt the ongoing E-RAB Modify procedure and initiate the Handover Preparation procedure as follows:

1.	The eNB shall send the E-RAB MODIFY RESPONSE message in which the eNB shall indicate, if necessary

-	all the E-RABs fail with an appropriate cause value, e.g., “S1 intra system Handover triggered”, “S1 inter system Handover triggered” or “X2 Handover triggered”.

2.	The eNB shall trigger the handover procedure.

#### 8.2.2.3	Unsuccessful Operation

The unsuccessful operation is specified in the successful operation section.

#### 8.2.2.4	Abnormal Conditions

If the eNB receives a E-RAB MODIFY REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in 3GPP TS 23.203 [13]) for a E-RAB previously configured as a non-GBR bearer (as defined in 3GPP TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the eNB shall consider the modification of the corresponding E-RAB as failed.

If the eNB receives an E-RAB MODIFY REQUEST message containing several E-RAB ID IEs (in the E-RAB to be Modified List IE) set to the same value, the eNB shall report the modification of the corresponding E-RABs as failed in the E-RAB MODIFY RESPONSE with the appropriate cause value, e.g., “Multiple E-RAB ID instances”.

If the eNB receives an E-RAB MODIFY REQUEST message containing some E-RAB ID IEs that eNB does not recognize, the eNB shall report the corresponding invalid E-RABs as failed in the E-RAB MODIFY RESPONSE with the appropriate cause value, e.g., “Unknown E-RAB ID”.

### 8.2.3	E-RAB Release

#### 8.2.3.1	General

The purpose of the E-RAB Release procedure is to enable the release of already established E-RABs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.3.2	Successful Operation

##### 8.2.3.2.1	E-RAB Release – MME initiated

Figure 8.2.3.2.1-1: E-RAB Release procedure. Successful operation.

The MME initiates the procedure by sending an E-RAB RELEASE COMMAND message.

The E-RAB RELEASE COMMAND message shall contain the information required by the eNB to release at least one E-RAB in the E-RAB To Be Released List IE. If a NAS-PDU IE is contained in the message, the eNB shall pass it to the UE.

Upon reception of the E-RAB RELEASE COMMAND message the eNB shall execute the release of the requested E-RABs. For each E-RAB to be released the eNB shall release the corresponding Data Radio Bearer and release the allocated resources on Uu. The eNB shall pass the value contained in the E-RAB ID IE received for the E-RAB to the radio interface protocol for each Data Radio Bearer to be released. The eNB shall release allocated resources on S1 for the E-RABs requested to be released.

The E-RAB RELEASE COMMAND message may contain

-	the UE Aggregate Maximum Bit Rate IE.

If the UE Aggregate Maximum Bit Rate IE is included in the E-RAB RELEASE COMMAND the eNB shall

- replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context; the eNB shall use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the UE Aggregate Maximum Bit Rate IE is not contained in the E-RAB RELEASE COMMAND message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

The eNB shall report to the MME, in the E-RAB RELEASE RESPONSE message, the result for all the E-RABs to be released.

-	A list of E-RABs which are released successfully shall be included in the E-RAB Release List IE.

-	A list of E-RABs which failed to be released, if any, shall be included in the E-RAB Failed to Release List IE.

The eNB shall be prepared to receive an E-RAB RELEASE COMMAND message on an established UE-associated logical S1-connection containing an E-RAB Release List IE at any time and shall always reply to it with an E-RAB RELEASE RESPONSE message.

The eNB shall, if supported, report in the E-RAB RELEASE RESPONSE message location information of the UE in the User Location Information IE.

After sending an E-RAB RELEASE RESPONSE message containing an E-RAB ID within the E-RAB Release List IE, the eNB shall be prepared to receive an E-RAB SETUP REQUEST message requesting establishment of an E-RAB with this E-RAB ID.

If the User Location Information IE is included in the E-RAB RELEASE RESPONSE message, the MME shall handle this information as specified in 3GPP TS 23.401 [11].

##### 8.2.3.2.2	E-RAB Release Indication – eNB initiated

Figure 8.2.3.2.2-1: E-RAB Release INDICATION procedure. Successful operation.

The eNB initiates the procedure by sending an E-RAB RELEASE INDICATION message towards the MME.

The E-RAB RELEASE INDICATION message shall contain at least one E-RAB released at the eNB, in the E-RAB Released List IE.

The eNB shall, if supported, report in the E-RAB RELEASE INDICATION message location information of the UE in the User Location Information IE.

Upon reception of the E-RAB RELEASE INDICATION message the MME shall normally initiate the appropriate release procedure on the core network side for the E-RABs identified in the E-RAB RELEASE INDICATION message.

If the User Location Information IE is included in the E-RAB RELEASE INDICATION message, the MME shall handle this information as specified in 3GPP TS 23.401 [11].

Interaction with UE Context Release Request procedure:

If the eNB wants to remove all remaining E-RABs, e.g., for user inactivity, the UE Context Release Request procedure shall be used instead.

#### 8.2.3.3	Abnormal Conditions

If the eNB receives an E-RAB RELEASE COMMAND message containing multiple E-RAB ID IEs (in the E-RAB To Be Released List IE) set to the same value, the eNB shall initiate the release of one corresponding E-RAB and ignore the duplication of the instances of the selected corresponding E-RABs.

If the MME receives an E-RAB RELEASE INDICATION message containing multiple E-RAB ID IEs (in the E-RAB Released List IE) set to the same value, the MME shall initiate the release of one corresponding E-RAB and ignore the duplication of the instances of the selected corresponding E-RABs.

If the eNB receives an E-RAB RELEASE COMMAND message containing some E-RAB ID IEs that eNB does not recognize, the eNB shall report the corresponding invalid E-RABs as failed in the E-RAB RELEASE RESPONSE message with the appropriate cause, e.g., “Unknown E-RAB ID”.

### 8.2.4	E-RAB Modification Indication

#### 8.2.4.1	General

The purpose of the E-RAB Modification Indication procedure is to enable the eNB to request modifications of already established E-RABs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: E-RAB Modification Indication procedure. Successful operation.

The eNB initiates the procedure by sending an E-RAB MODIFICATION INDICATION message to the MME.

The Transport Layer Address IE and DL GTP TEID IE included in the E-RAB To Be Modified Item IEs IE in the E-RAB MODIFICATION INDICATON message shall be considered by the MME as the new DL address of the E-RABs. The Transport Layer Address IE and DL GTP TEID IE included in the E-RAB Not To Be Modified Item IEs IE in the E-RAB MODIFICATION INDICATION message shall be considered by the MME as the E-RABs with unchanged DL address

The E-RAB MODIFICATION CONFIRM message shall contain the result for all the E-RABs that were requested to be modified according to the E-RAB To Be Modified Item IEs IE of the E-RAB MODIFICATION INDICATION message as follows:

-	A list of E-RABs which are successfully modified shall be included in the E-RAB Modify List IE.

-	A list of E-RABs which failed to be modified, if any, shall be included in the E-RAB Failed to Modify List IE.

-	A list of E-RABs which are to be released, if any, shall be included in the E-RAB To Be Released List IE.

If the E-RAB Failed to Modify List IE is received in the E-RAB MODIFICATION CONFIRM message, the eNB shall either

-	release all corresponding E-UTRA and E-UTRAN resources for the concerned E-RAB or

-	keep the previous transport information before sending the E-RAB MODIFICATION INDICATION message unchanged for the concerned E-RAB.

If the E-RAB To Be Released List IE is received in the E-RAB MODIFICATION CONFIRM message, the eNB shall release all corresponding E-UTRA and E-UTRAN resources for the concerned E-RAB.

If the CSG Membership Info IE is included in the E-RAB MODIFICATION INDICATION message, the MME shall use the information for CSG membership verification as specified in MFA TS 36.300 [14] and provide the result of the membership verification in the CSG Membership Status IE contained in the E-RAB MODIFICATION CONFIRM message.

If PLMN Identity IE is received in the CSG Membership Info IE in the E-RAB MODIFICATION INDICATION message, the MME shall use it for CSG membership verification as specified in MFA TS 36.300 [14].

When the MME reports unsuccessful modification of an E-RAB, the cause value should be precise enough to enable the eNB to know the reason for an unsuccessful modification.

If the Tunnel Information for BBF IE is received in the E-RAB MODIFICATION INDICATION message, the MME shall, if supported, use it in the core network as specified in 3GPP TS 23.139 [37].

#### 8.2.4.3	Unsuccessful Operation

The unsuccessful operation is specified in the successful operation section.

#### 8.2.4.4	Abnormal Conditions

Interaction with UE Context Release Request procedure:

If the E-RAB MODIFICATION INDICATION message does not contain all the E-RABs previously included in the UE Context, the MME shall trigger the UE Context Release procedure.

If the E-RAB MODIFICATION INDICATION message contains several E-RAB ID IEs set to the same value, the MME shall trigger the UE Context Release procedure.

If the CSG Membership Info IE in the E-RAB MODIFICATION INDICATION message does not contain the Cell Access Mode IE set to "hybrid", the MME shall trigger the UE Context Release procedure.

## 8.3	Context Management procedures

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE Context including E-RAB context, the Security Key, Handover Restriction List, UE Radio capability and UE Security Capabilities etc. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial Context Setup procedure. Successful operation.

In case of the establishment of an E-RAB the EPC must be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the MME. If no UE-associated logical S1-connection exists, the UE-associated logical S1-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain within the E-RAB to be Setup List IE the information required by the eNB to build the new E-RAB configuration consisting of at least one additional E-RAB.

The E-RAB to be Setup Item IE may contain:

-	the NAS-PDU IE,

-	the Correlation ID IE in case of LIPA operation,

-	the SIPTO Correlation ID IE in case of SIPTO@LN operation,

-	the Bearer Type IE.

The INITIAL CONTEXT SETUP REQUEST message may contain

-	the Trace Activation IE.

-	the Handover Restriction List IE, which may contain roaming or access restrictions.

-	the UE Radio Capability IE.

-	the Subscriber Profile ID for RAT/Frequency priority IE.

-	the CS Fallback Indicator IE.

-	the SRVCC Operation Possible IE.

-	the CSG Membership Status IE.

-	the Registered LAI IE.

-	the GUMMEI IE, which indicates the MME serving the UE, and shall only be present according to subclauses 4.6.2 and 4.7.6.6 of MFA TS 36.300 [14].

-	the MME UE S1AP ID 2 IE, which indicates the MME UE S1AP ID assigned by the MME, and shall only be present according to subclause 4.6.2 of MFA TS 36.300 [14].

-	the Management Based MDT Allowed IE.

-	the Management Based MDT PLMN List IE.

-	the Additional CS Fallback Indicator IE.

-	the Masked IMEISV IE.

-	the Expected UE Behaviour IE.

-	the ProSe Authorized IE.

-	the UE User Plane CIoT Support Indicator IE.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE, if available in the MME.

If the Correlation ID IE is included in the INITIAL CONTEXT SETUP REQUEST message towards the eNB with L-GW function for LIPA operation, then the eNB shall use this information for LIPA operation for the concerned E-RAB.

If the SIPTO Correlation ID IE is included in the INITIAL CONTEXT SETUP REQUEST message towards the eNB with L-GW function for SIPTO@LN operation, then the eNB shall use this information for SIPTO@LN operation for the concerned E-RAB.

If the Bearer Type IE is included in the INITIAL CONTEXT SETUP REQUEST message and is set to “non IP”, then the eNB shall not perform header compression for the concerned E-RAB.

If the Masked IMEISV IE is contained in the INITIAL CONTEXT SETUP REQUEST the target eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the Expected UE Behaviour IE is included in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information and may use it to determine the RRC connection time.

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the eNB shall

-	attempt to execute the requested E-RAB configuration.

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

-	pass the value contained in the E-RAB ID IE and the NAS-PDU IE received for the E-RAB for each established Data radio bearer to the radio interface protocol. The eNB shall not send the NAS PDUs associated to the failed Data radio bearers to the UE.

-	store the received Handover Restriction List in the UE context.

-	store the received UE Radio Capability in the UE context.

-	store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in MFA TS 36.300 [14].

-	store the received SRVCC Operation Possible in the UE context and use it as defined in 3GPP TS 23.216 [9].

-	store the received UE Security Capabilities in the UE context.

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in MFA TS 33.401 [15].

-	store the received CSG Membership Status, if supported, in the UE context.

-	store the received Management Based MDT Allowed information, if supported, in the UE context.

-	store the received Management Based MDT PLMN List information, if supported, in the UE context.

-	store the received ProSe Authorization information, if supported, in the UE context.

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context.

The allocation of resources according to the values of the Allocation and Retention Priority IE shall follow the principles described for the E-RAB Setup procedure.

The eNB shall use the information in the Handover Restriction List IE if present in the INITIAL CONTEXT SETUP REQUEST message to

-	determine a target for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, except if the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is not present in which case the eNB may use the information in the Handover Restriction List IE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall consider that no roaming and no access restriction apply to the UE. The eNB shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the setup E-RABs has a particular ARP value (3GPP TS 23.401 [11]);

-	the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is not present and, in case the Handover Restriction List IE is applied, no suitable target is found, in which case it shall process according to 3GPP TS 23.272 [17];

-	the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is set to “no restriction”, in which case it shall process according to 3GPP TS 23.272 [17].

If the Trace Activation IE is included in the INITIAL CONTEXT SETUP REQUEST message then eNB shall, if supported, initiate the requested trace function as described in 3GPP TS 32.422 [10]. In particular, the eNB shall, if supported:

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in 3GPP TS 32.422 [10];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in 3GPP TS 32.422 [10];

-	if the Trace Activation IE includes the MDT Activation IE ,within the MDT Configuration IE, set to “Immediate MDT Only”, “Logged MDT only” or “Logged MBSFN MDT”, initiate the requested MDT session as described in 3GPP TS 32.422 [10] and the eNB shall ignore Interfaces To Trace IE, and Trace Depth IE.

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session.

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the eNB may use it to propagate the MDT Configuration as described in 3GPP TS 37.320 [31].

-	if the Trace Activation IE includes the MBSFN-ResultToLog IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in 3GPP TS 37.320 [31].

-	if the Trace Activation IE includes the MBSFN-AreaId IE in the MBSFN-ResultToLog IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in 3GPP TS 37.320 [31].

If the CS Fallback Indicator IE is included in the INITIAL CONTEXT SETUP REQUEST message, it indicates that the UE Context to be set-up is subject to CS Fallback. The eNB shall reply with the INITIAL CONTEXT SETUP RESPONSE message and then act as defined in 3GPP TS 23.272 [17].

If the Registered LAI IE is included in the INITIAL CONTEXT SETUP REQUEST message, it indicates that the eNB may take the Registered LAI IE into account when selecting the target cell or frequency and then act as defined in 3GPP TS 23.272 [17].

If the UE Security Capabilities IE included in the INITIAL CONTEXT SETUP REQUEST message only contains the EIA0 algorithm as defined in MFA TS 33.401 [15] and if this EIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the eNB (see MFA TS 33.401 [15]), the eNB shall take it into use and ignore the keys received in the Security Key IE.

If the GUMMEI IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information in the UE context and use it for subsequent X2 handovers.

If the MME UE S1AP ID 2 IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information in the UE context and use it for subsequent X2 handovers.

If the Management Based MDT Allowed IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall use it, if supported, together with information in the Management Based MDT PLMN List IE, if available in the UE context, to allow subsequent selection of the UE for management based MDT defined in 3GPP TS 32.422 [10].

If the UE User Plane CIoT Support Indicator IE is included in the INITIAL CONTEXT SETUP REQUEST message and is set to "supported", the eNB shall, if supported, consider that User Plane CIoT EPS Optimisation as specified in 3GPP TS 23.401 [11] is supported for the UE.

The eNB shall report to the MME, in the INITIAL CONTEXT SETUP RESPONSE message, the successful establishment of the security procedures with the UE, and, the result for all the requested E-RABs in the following way:

-	A list of E-RABs which are successfully established shall be included in the E-RAB Setup List IE

-	A list of E-RABs which failed to be established shall be included in the E-RAB Failed to Setup List IE.

When the eNB reports the unsuccessful establishment of an E-RAB, the cause value should be precise enough to enable the MME to know the reason for the unsuccessful establishment, e.g., “Radio resources not available”, “Failure in the Radio Interface Procedure”.

After sending the INITIAL CONTEXT SETUP RESPONSE message, the procedure is terminated in the eNB.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: Initial Context Setup procedure. Unsuccessful operation.

If the eNB is not able to establish an S1 UE context, or cannot even establish one non GBR bearer it shall consider the procedure as failed and reply with the INITIAL CONTEXT SETUP FAILURE message.

#### 8.3.1.4	Abnormal Conditions

If the eNB receives an INITIAL CONTEXT SETUP REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in 3GPP TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the eNB shall consider the establishment of the corresponding E-RAB as failed.

If the eNB receives an INITIAL CONTEXT SETUP REQUEST message containing several E-RAB ID IEs (in the E-RAB to Be Setup List IE) set to the same value, the eNB shall consider the establishment of the corresponding E-RABs as failed.

If the supported algorithms for encryption defined in the Encryption Algorithms IE in the UE Security Capabilities IE, plus the mandated support of EEA0 in all UEs (MFA TS 33.401 [15]), do not match any allowed algorithms defined in the configured list of allowed encryption algorithms in the eNB (MFA TS 33.401 [15]), the eNB shall reject the procedure using the INITIAL CONTEXT SETUP FAILURE message.

If the supported algorithms for integrity defined in the Integrity Protection Algorithms IE in the UE Security Capabilities IE, plus the mandated support of the EIA0 algorithm in all UEs (MFA TS 33.401 [15]), do not match any allowed algorithms defined in the configured list of allowed integrity protection algorithms in the eNB (MFA TS 33.401 [15]), the eNB shall reject the procedure using the INITIAL CONTEXT SETUP FAILURE message.

If the CSG Membership Status IE is not included in the INITIAL CONTEXT SETUP REQUEST message and the cell accessed by the UE is a hybrid cell, the eNB shall reject the procedure using the INITIAL CONTEXT SETUP FAILURE message.

If the eNB receives an INITIAL CONTEXT SETUP REQUEST message containing both the Correlation ID and the SIPTO Correlation ID IEs for the same E-RAB, the eNB shall consider the establishment of the corresponding E-RAB as failed.

### 8.3.2	UE Context Release Request (eNB initiated)

#### 8.3.2.1	General

The purpose of the UE Context Release Request procedure is to enable the eNB to request the MME to release the UE-associated logical S1-connection due to E-UTRAN generated reasons, e.g., “TX2RELOCOverall Expiry”. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: UE Context Release Request procedure. Successful operation.

The eNB controlling a UE-associated logical S1-connection initiates the procedure by generating a UE CONTEXT RELEASE REQUEST message towards the affected MME node.

The UE CONTEXT RELEASE REQUEST message shall indicate the appropriate cause value, e.g., “User Inactivity”, “Radio Connection With UE Lost”, “CSG Subscription Expiry”, “CS Fallback triggered”, “Redirection towards 1xRTT”, “Inter-RAT Redirection”, “UE Not Available for PS Service”, for the requested UE-associated logical S1-connection release.

Interactions with UE Context Release procedure:

The UE Context Release procedure should be initiated upon reception of a UE CONTEXT RELEASE REQUEST message.

### 8.3.3	UE Context Release (MME initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the MME to order the release of the UE-associated logical connection due to various reasons, e.g., completion of a transaction between the UE and the EPC, or completion of successful handover, or completion of handover cancellation, or release of the old UE-associated logical S1-connection when two UE-associated logical S1-connections toward the same UE is detected after the UE has initiated the establishment of a new UE-associated logical S1-connection, or the UE is no longer allowed to access the CSG cell (i.e., the UE becomes a non-member of the currently used CSG cell). The procedure uses UE-associated S1 connection.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release procedure. Successful operation.

The MME initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the eNB.

The UE CONTEXT RELEASE COMMAND message shall contain the UE S1AP ID pair IE if available, otherwise the message shall contain the MME UE S1AP ID IE.

The MME provides the cause IE set to “Load Balancing TAU Required” in the UE CONTEXT RELEASE COMMAND message sent to the eNB for all load balancing and offload cases in the MME.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the eNB shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message. In case of eNB supporting L-GW function for LIPA and/or SIPTO@LN operation, the eNB shall also release any related tunnel resources. In case of successful handover, the eNB using L-GW function for SIPTO@LN operation shall also request using intra-node signalling the collocated L-GW to release the SIPTO@LN PDN connection as defined in 3GPPTS 23.401 [11].

The eNB shall, if supported, report in the UE CONTEXT RELEASE COMPLETE message location information of the UE in the User Location Information IE.

If the User Location Information IE is included in the UE CONTEXT RELEASE COMPLETE message, the MME shall handle this information as specified in 3GPP TS 23.401 [11].

If the Information on Recommended Cells and eNBs for Paging IE is included in the UE CONTEXT RELEASE COMPLETE message, the MME shall, if supported, store it and may use it for subsequent paging.

If the Cell Identifier and Coverage Enhancement Level IE is included in the UE CONTEXT RELEASE COMPLETE message, the MME shall, if supported, store it and use it for subsequent paging.

#### 8.3.3.3	Abnormal Conditions

If the UE Context Release procedure is not initiated towards the eNB before the expiry of the timer TS1RELOCOverall, the eNB shall request the MME to release the UE context.

If the UE returns to the eNB before the reception of the UE CONTEXT RELEASE COMMAND message or the expiry of the timer TS1RELOCOverall, the eNB shall stop the TS1RELOCOverall and continue to serve the UE.

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE Context, e.g., with the Security Key or the Subscriber Profile ID for RAT/Frequency priority. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation.

The UE CONTEXT MODIFICATION REQUEST message may contain.

-	the Security Key IE.

-	the Subscriber Profile ID for RAT/Frequency priority IE.

-	the UE Aggregate Maximum Bit Rate IE.

-	the CS Fallback Indicator IE.

-	the UE Security Capabilities IE.

-	the CSG Membership Status IE.

-	the Registered LAI IE.

-	the Additional CS Fallback Indicator IE.

-	the ProSe Authorized IE.

-	the SRVCC Operation Possible IE.

-	the SRVCC Operation Not Possible IE

Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the eNB shall

-	store the received Security Key IE, take it into use and associate it with the initial value of NCC as defined in MFA TS 33.401 [15]

-	store the UE Security Capabilities IE and take them into use together with the received keys according to MFA TS 33.401 [15].

-	store the Subscriber Profile ID for RAT/Frequency priority IE and use it as defined in TS 36.300 [14].

If the UE Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message the eNB shall:

-	replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context;

- use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the CSG Membership Status IE is received in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall take the following action:

-	If the cell that serves the UE is a hybrid cell, the eNB shall store the value contained in the CSG Membership Status IE and replace any previously stored membership status value by this new one. It shall then use it as defined in MFA TS 36.300 [14].

-	If the cell that serves the UE is a CSG cell, and the CSG Membership Status IE is set to “not-member”, the eNB should initiate actions to ensure that the UE is no longer served by the CSG cell as defined in MFA TS 36.300 [14].

-	If the UE is in dual connectivity operation and the cell configured as SCG is a hybrid cell, the eNB shall inform the eNB serving the SCG of the updated CSG membership status.

If the UE Aggregate Maximum Bit Rate IE is not contained in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

If the CS Fallback Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST message, it indicates that the concerned UE Context is subject to CS Fallback. The eNB shall reply with the UE CONTEXT MODIFICATION RESPONSE message and then act as defined in 3GPP TS 23.272 [17]. If the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is not present and, in case the Handover Restriction List information that may exist in the UE context is applied, no suitable target is found, or if the CS Fallback Indicator IE is set to “CS Fallback High Priority” and the Additional CS Fallback Indicator IE is set to “no restriction”, the eNB shall consider that no roaming and no access restriction apply to the UE and process according to 3GPP TS 23.272 [17].

If the Registered LAI IE is included in the UE CONTEXT MODIFICATION REQUEST message, it indicates that the eNB may take the Registered LAI IE into account when selecting the target cell or frequency and then act as defined in 3GPP TS 23.272 [17].

If the ProSe Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the eNB shall, if supported, update its ProSe authorization information for the UE accordingly. If the ProSe Authorized IE includes one or more IEs set to “not authorized”, the eNB shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant ProSe service(s).

If the SRVCC Operation Possible IE is included in UE CONTEXT MODIFICATION REQUEST message, the eNB shall store content of the received SRVCC Operation Possible IE in the UE context and, if supported, use it as defined in 3GPP TS 23.216 [9].

If the SRVCC Operation Not Possible IE is included in UE CONTEXT MODIFICATION REQUEST message, the eNB shall, if supported, remove the SRVCC Operation Possible information from the UE context.

The eNB shall report, in the UE CONTEXT MODIFICATION RESPONSE message to the MME the successful update of the UE context.

After sending the UE CONTEXT MODIFICATION RESPONSE message, the procedure is terminated in the eNB.

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation.

In case the UE context update cannot be performed successfully the eNB shall respond with the UE CONTEXT MODIFICATION FAILURE message to the MME with an appropriate cause value in the Cause IE.

#### 8.3.4.4	Abnormal Conditions

If the eNB receives both the CS Fallback Indicator IE and one of the security IEs (either the Security Key IE or the UE Security Capabilities IE) in the UE Context Modification Request message, the eNB shall ignore both IEs and send back the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

### 8.3.5	UE Radio Capability Match

#### 8.3.5.1	General

The purpose of the UE Radio Capability Match procedure is for the MME to request the eNB to derive and provide an indication to the MME whether the UE radio capabilities are compatible with the network configuration for voice continuity.

The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: UE Radio Capability Match. Successful operation

The MME initiates the procedure by sending a UE RADIO CAPABILITY MATCH REQUEST message. If the UE-associated logical S1-connection is not established, the MME shall allocate a unique MME UE S1AP ID to be used for the UE and include the MME UE S1AP ID IE in the UE RADIO CAPABILITY MATCH REQUEST message; by receiving the MME UE S1AP ID IE in the UE RADIO CAPABILITY MATCH REQUEST message, the eNB establishes the UE-associated logical S1-connection.

Upon receipt of the UE RADIO CAPABILITY MATCH REQUEST message, the eNB shall act as defined in the 3GPP TS 23.401 [11] and respond with a UE RADIO CAPABILITY MATCH RESPONSE message.

If the UE Radio Capability IE is contained in the UE RADIO CAPABILITY MATCH REQUEST message, the eNB shall use it to determine the value of the Voice Support Match Indicator IE to be included in the UE RADIO CAPABILITY MATCH RESPONSE message.

#### 8.3.5.3	Unsuccessful Operation

Not applicable.

#### 8.3.5.4	Abnormal Conditions

Not applicable.

### 8.3.6	UE Context Modification Indication

#### 8.3.6.1	General

The purpose of the UE Context Modification Indication procedure is for the eNB to request the modifications on the established UE Context.

The procedure uses UE-associated signalling.

In the current version of the specification, this procedure is only used for membership verification, as described in MFA TS 36.300 [14].

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: UE Context Modification Indication. Successful operation

If the CSG Membership Info IE is included in the UE CONTEXT MODIFICATION INDICATION message, the MME shall use the information for CSG membership verification as specified in MFA TS 36.300 [14] and provide the result of the membership verification in the CSG Membership Status IE contained in the UE CONTEXT MODIFICATION CONFIRM message.

If no CSG Membership Info IE is received in the UE CONTEXT MODIFICATION INDICATION message and the UE was previously configured with resources from a hybrid cell, the MME shall consider that the UE has moved into an open access cell.

If PLMN Identity IE is received in the CSG Membership Info IE in the UE CONTEXT MODIFICATION INDICATION message, the MME shall use it for CSG membership verification as specified in MFA TS 36.300 [14].

#### 8.3.6.3	Unsuccessful Operation

Not applicable.

#### 8.3.6.4	Abnormal Conditions

If the CSG Membership Info IE in the UE CONTEXT MODIFICATION message does not contain the Cell Access Mode IE set to "hybrid"the MME shall trigger the UE Context Release procedure.

### 8.3.7	UE Context Suspend

#### 8.3.7.1	General

The purpose of the UE Context Suspend procedure is to suspend the UE context, the UE-associated logical S1-connection and the related bearer contexts in the E-UTRAN and the EPC.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: UE Context Suspend procedure. Successful operation.

The eNB initiates the procedure by sending the UE CONTEXT SUSPEND REQUEST message to the MME.

Upon receipt of the UE CONTEXT SUSPEND REQUEST the MME shall act as defined in 3GPP TS 23.401 [11].

Upon receipt of the UE CONTEXT SUSPEND RESPONSE message the eNB shall suspend the UE context, the UE-associated logical S1-connection and the related bearer contexts and send the UE to RRC_IDLE.

If the Information on Recommended Cells and eNBs for Paging IE is included in the UE CONTEXT SUSPEND REQUEST message, the MME shall, if supported, store it and may use it for subsequent paging.

If the Cell Identifier and Coverage Enhancement Level IE is included in the UE CONTEXT SUSPEND REQUEST message, the MME shall, if supported, store it and use it for subsequent paging.

### 8.3.8	UE Context Resume

#### 8.3.8.1	General

The purpose of the UE Context Resume procedure is to indicate to the MME that the UE has resumed the suspended RRC connection and to request the MME to resume the UE context, UE-associated logical S1-connection and the related bearer contexts in the EPC.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: UE Context Resume procedure. Successful operation.

The eNB initiates the procedure by sending the UE CONTEXT RESUME REQUEST message to the MME. If the eNB is not able to admit all suspended E-RABs the eNB shall indicate this in the E-RABs Failed To Resume List IE.

Upon receipt of the UE CONTEXT RESUME REQUEST message the MME shall act as defined in 3GPP TS 23.401 [11] and respond with the UE CONTEXT RESUME RESPONSE. If the MME is not able to admit all suspended E-RABs the MME shall indicate this in the E-RABs Failed To Resume List IE.

The eNB shall release resources for each E-RAB failed to resume and shall assume that the EPC has released respective resources as well.

#### 8.3.8.3	Unsuccessful Operation

Figure 8.3.8.3-1: UE Context Resume procedure. Unsuccessful operation.

If the MME is not able to resume a single E-RAB it releases the UE-associated logical S1-connection by sending the UE CONTEXT RESUME FAILURE message to the eNB. Upon reception of the UE CONTEXT RESUME FAILURE message the eNB shall release the RRC connection as specified in MFA TS 36.331 [16] and release all related signalling and user data transport resources.

### 8.3.9	Connection Establishment Indication

#### 8.3.9.1	General

The purpose of the Connection Establishment Indication procedure is to enable the MME to complete the establishment of the UE-associated logical S1-connection. The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: Connection Establishment Indication procedure. Successful operation.

The MME initiates the procedure by sending a CONNECTION ESTABLISHMENT INDICATION message to the eNB.

The MME shall allocate a unique MME UE S1AP ID to be used for the UE and include that in the CONNECTION ESTABLISHMENT INDICATION message.

If the UE Radio Capability IE is included in the CONNECTION ESTABLISHMENT INDICATION message, the eNB shall store this information in the UE context, use it as defined in MFA TS 36.300 [14].

#### 8.3.9.3	Unsuccessful Operation

Not applicable.

#### 8.3.9.4	Abnormal Conditions

Not applicable.

## 8.4	Handover Signalling

### 8.4.1	Handover Preparation

#### 8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the EPC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source eNB initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving MME. When the source eNB sends the HANDOVER REQUIRED message, it shall start the timer TS1RELOCprep. The source eNB shall indicate the appropriate cause value for the handover in the Cause IE.

The source eNB shall include the Source to Target Transparent Container IE in the HANDOVER REQUIRED message.

In case of intra-system handover, the information in the Source to Target Transparent Container IE shall be encoded according to the definition of the Source eNB to Target eNB Transparent Container IE. In case of handover to UTRAN, the information in the Source to Target Transparent Container IE shall be encoded according to the Source RNC to Target RNC Transparent Container IE definition as specified in 3GPP TS 25.413 [19] and the source eNB shall include the UE History Information IE in the Source RNC to Target RNC Transparent Container IE. If the handover is to GERAN A/Gb mode then the information in the Source to Target Transparent Container IE shall be encoded according to the definition of the Source BSS to Target BSS Transparent Container IE as described in 3GPP TS 48.018 [18].

When the preparation, including the reservation of resources at the target side is ready, the MME responds with the HANDOVER COMMAND message to the source eNB.

If the Target to Source Transparent Container IE has been received by the MME from the handover target then the transparent container shall be included in the HANDOVER COMMAND message.

Upon reception of the HANDOVER COMMAND message the source eNB shall stop the timer TS1RELOCprep and start the timer TS1RELOCOverall.

In case of intra-system handover, the information in the Target to Source Transparent Container IE shall be encoded according to the definition of the Target eNB to Source eNB Transparent Container IE. In case of inter-system handover to UTRAN, the information in the Target to Source Transparent Container IE shall be encoded according to the Target RNC to Source RNC Transparent Container IE definition as specified in 3GPP TS 25.413 [19]. In case of inter-system handover to GERAN A/Gb mode, the information in the Target to Source Transparent Container IE shall be encoded according to the Target BSS to Source BSS Transparent Container IE definition as described in 3GPP TS 48.018 [18].

If there are any E-RABs that could not be admitted in the target, they shall be indicated in the E-RABs to Release List IE.

If the DL forwarding IE is included within the Source eNB to Target eNB Transparent Container IE of the HANDOVER REQUIRED message and it is set to “DL forwarding proposed”, it indicates that the source eNB proposes forwarding of downlink data.

If the MME receives the Direct Forwarding Path Availability IE in the HANDOVER REQUIRED message indicating that a direct data path is available, it shall handle it as specified in 3GPP TS 23.401 [11].

If the CSG Id IE and no Cell Access Mode IE are received in the HANDOVER REQUIRED message, the MME shall perform the access control according to the CSG Subscription Data of that UE and, if the access control is successful or if at least one of the E-RABs has a particular ARP value (see 3GPP TS 23.401 [11]), it shall continue the handover and propagate the CSG Id IE to the target side. If the access control is unsuccessful but at least one of the E-RABs has a particular ARP value (see 3GPP TS 23.401 [11]) the MME shall also provide the CSG Membership Status IE set to “non member” to the target side.

If the CSG Id IE and the Cell Access Mode IE set to “hybrid” are received in the HANDOVER REQUIRED message, the MME shall provide the membership status of the UE and the CSG Id to the target side.

The source eNB shall include the SRVCC HO Indication IE in the HANDOVER REQUIRED message if the SRVCC operation is needed as defined in 3GPP TS 23.216 [9]. The source eNB shall indicate to the MME in the SRVCC HO Indication IE if the handover shall be prepared for PS and CS domain or only for CS domain. The SRVCC HO Indication IE is set according to the target cell capability and UE capability. In case the target system is GERAN without DTM support or the UE is without DTM support, the source eNB shall indicate “CS only” in the SRVCC HO Indication IE and “PS service not available” in PS Service Not Available IE. In case the target system is either GERAN with DTM but without DTM HO support and the UE is supporting DTM or the target system is UTRAN without PS HO support, the source eNB shall indicate “CS only” in the SRVCC HO Indication IE. Otherwise, the source eNB shall indicate “PS and CS” in the SRVCC HO Indication IE.

In case of inter-system handover from E-UTRAN, the source eNB shall indicate in the Target ID IE, in case the target system is UTRAN, the Target RNC-ID of the RNC (including the Routing Area Code only in case the UTRAN PS domain is involved), in case the target system is GERAN the Cell Global Identity (including the Routing Area Code only in case the GERAN PS domain is involved) of the cell in the target system.

In case of inter-system handover from E-UTRAN to UTRAN, the source eNB shall, if supported, include the HO Cause Value IE in the UE History Information IE of the HANDOVER REQUIRED message.

In case the SRVCC operation is performed and the SRVCC HO Indication IE indicates that handover shall be prepared only for CS domain, and if

-	the target system is GERAN, then the source eNB

-	shall encode the information in the Source to Target Transparent Container IE within the HANDOVER REQUIRED message, according to the definition of the Old BSS to New BSS information IE as specified in 3GPP TS 48.008 [23], and

-	shall not include the Source to Target Transparent Container Secondary IE in the HANDOVER REQUIRED message;

-	the target system is UTRAN, then the source eNB

-	shall encode the information in the Source to Target Transparent Container IE within the HANDOVER REQUIRED message according to the definition of the Source RNC to Target RNC Transparent Container IE as specified in 3GPP TS 25.413 [19],

-	shall include the UE History Information IE in the Source RNC to Target RNC Transparent Container IE, and

-	shall not include the Source to Target Transparent Container Secondary IE in the HANDOVER REQUIRED message.

In case the SRVCC operation is performed, the SRVCC HO Indication IE in the HANDOVER REQUIRED message indicates that handover shall be prepared for PS and CS domain, and if

-	the target system is GERAN with DTM HO support, then the source eNB

-	shall encode the information in the Source to Target Transparent Container IE within the HANDOVER REQUIRED message according to the definition of the Source BSS to Target BSS Transparent Container IE as described in 3GPP TS 48.018 [18],and

-	shall include the Source to Target Transparent Container Secondary IE in the HANDOVER REQUIRED message and encode information in it according to the definition of the Old BSS to New BSS information IE as specified in 3GPP TS 48.008 [23];

-	the target system is UTRAN, then the source eNB

-	shall encode the information in the Source to Target Transparent Container IE within the HANDOVER REQUIRED message according to the definition of the Source RNC to Target RNC Transparent Container IE as specified in 3GPP TS 25.413 [19],

-	shall include the UE History Information IE in the Source RNC to Target RNC Transparent Container IE, and

-	shall not include the Source to Target Transparent Container Secondary IE in the HANDOVER REQUIRED message.

In case the SRVCC operation is performed, the SRVCC HO Indication IE in the HANDOVER REQUIRED message indicates that handover shall be prepared only for CS domain, and if

-	the target system is GERAN, then the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Layer 3 Information IE as specified in 3GPP TS 48.008 [23], and

-	shall not include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message;

-	the target system is UTRAN, then the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Target RNC to Source RNC Transparent Container IE as specified in 3GPP TS 25.413 [19], and

-	shall not include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message.

In case the SRVCC operation is performed, the SRVCC HO Indication IE in the HANDOVER REQUIRED message indicates that handover shall be prepared for PS and CS domain,

-	the target system is GERAN with DTM HO support, and if

-	the Handover Preparation procedure has succeeded in the CS and PS domain, then the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Layer 3 Information IE as specified in 3GPP TS 48.008 [23], and

-	shall include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message and encode information in it according to the definition of the Target BSS to Source BSS Transparent Container IE as specified in 3GPP TS 48.018 [18];

-	the Handover Preparation procedure has succeeded in the CS domain only, then the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Layer 3 Information IE as specified in 3GPP TS 48.008 [23], and

-	shall not include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message;

-	the target system is UTRAN, then the Handover Preparation procedure shall be considered successful if the Handover Preparation procedure has succeeded in the CS domain, and the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Target RNC to Source RNC Transparent Container IE as specified in 3GPP TS 25.413 [19], and

-	shall not include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message.

If the HANDOVER COMMAND message contains the DL GTP-TEID IE and the DL Transport Layer Address IE for a given bearer in the E-RABs Subject to Forwarding List IE, then the source eNB shall consider that the forwarding of downlink data for this given bearer is possible.

If the HANDOVER COMMAND message contains the UL GTP-TEID IE and the UL Transport Layer Address IE for a given bearer in the E-RABs Subject to Forwarding List IE, then it means the target eNB has requested the forwarding of uplink data for this given bearer.

Interactions with E-RAB Management procedures:

If, after a HANDOVER REQUIRED message is sent and before the Handover Preparation procedure is terminated, the source eNB receives an MME initiated E-RAB Management procedure on the same UE associated signalling connection, the source eNB shall either:

1.	cancel the Handover Preparation procedure by executing the Handover Cancel procedure with an appropriate cause value. After successful completion of the Handover Cancel procedure, the source eNB shall continue the MME initiated E-RAB Management procedure

or

2.	terminate the MME initiated E-RAB Management procedure by sending the appropriate response message with an appropriate cause value, e.g., “S1 intra system Handover Triggered”, “S1 inter system Handover Triggered” to the MME and then the source eNB shall continue with the handover procedure.

#### 8.4.1.3	Unsuccessful Operation

Figure 8.4.1.3-1: Handover preparation: unsuccessful operation

If the EPC or the target system is not able to accept any of the bearers or a failure occurs during the Handover Preparation, the MME sends the HANDOVER PREPARATION FAILURE message with an appropriate cause value to the source eNB.

If the CSG Id IE and no Cell Access Mode IE are received in the HANDOVER REQUIRED message and the access control is unsuccessful and none of the E-RABs has a particular ARP value (see 3GPP TS 23.401 [11]) the MME shall send the HANDOVER PREPARATION FAILURE message with an appropriate cause value to the source eNB, except when one of the E-RABs has a particular ARP value (see 3GPP TS 23.401 [11]). Upon reception, the source eNB may decide to prevent handover for that UE towards CSG (Closed Access Mode) cells with corresponding CSG Id.

Interaction with Handover Cancel procedure:

If there is no response from the EPC to the HANDOVER REQUIRED message before timer TS1RELOCprep expires in the source eNB, the source eNB should cancel the Handover Preparation procedure by initiating the Handover Cancel procedure with the appropriate value for the Cause IE. The source eNB shall ignore any HANDOVER COMMAND message or HANDOVER PREPARATION FAILURE message received after the initiation of the Handover Cancel procedure.

#### 8.4.1.4	Abnormal Conditions

If the eNB receives at least one E-RAB ID included in the E-RABs Subject to Forwarding List IE without at least one valid associated tunnel address pair (in either UL or DL), then the eNB shall consider it as a logical error and act as described in subclause 10.4. A GTP tunnel address pair is considered valid if both the GTP-TEID IE and the Transport Layer Address IE are present.

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target eNB for the handover of a UE.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The MME initiates the procedure by sending the HANDOVER REQUEST message to the target eNB. The HANDOVER REQUEST message may contain the Handover Restriction List IE, which contains roaming or access restrictions.

If the Handover Restriction List IE is contained in the HANDOVER REQUEST message, the target eNB shall store this information in the UE context. This information shall however not be considered whenever one of the handed over E-RABs has a particular ARP value (3GPP TS 23.401 [11]).

The target eNB shall use the information in Handover Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the HANDOVER REQUEST message, the target eNB shall consider that no roaming and no access restriction apply to the UE.

Upon reception of the HANDOVER REQUEST message the eNB shall store the received UE Security Capabilities IE in the UE context and use it to prepare the configuration of the AS security relation with the UE.

If the SRVCC Operation Possible IE is included in the HANDOVER REQUEST message, the target eNB shall store the content of the received SRVCC Operation Possible IE in the UE context and, if supported, use it as defined in 3GPP TS 23.216 [9].

Upon reception of the HANDOVER REQUEST message the eNB shall store the received Security Context IE in the UE context and the eNB shall use it to derive the security configuration as specified in MFA TS 33.401 [15].

If the Trace Activation IE is included in the HANDOVER REQUEST message, the target eNB shall if supported, initiate the requested trace function as described in 3GPP TS 32.422 [10]. In particular, the eNB shall, if supported:

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in 3GPP TS 32.422 [10];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in 3GPP TS 32.422 [10];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT Only”, “Logged MDT only” or “Logged MBSFN MDT”, initiate the requested MDT session as described in 3GPP TS 32.422 [10] and the target eNB shall ignore Interfaces To Trace IE, and Trace Depth IE.

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session.

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the eNB may use it to propagate the MDT Configuration as described in 3GPP TS 37.320 [31].

-	if the Trace Activation IE includes the MBSFN-ResultToLog IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in 3GPP TS 37.320 [31].

-	if the Trace Activation IE includes the MBSFN-AreaId IE in the MBSFN-ResultToLog IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in 3GPP TS 37.320 [31].

If the CSG Id IE is received in the HANDOVER REQUEST message, the eNB shall compare the received value with the CSG Id broadcast by the target cell.

If the CSG Membership Status IE is received in the HANDOVER REQUEST message and the CSG Membership Status is set to “member”, the eNB may provide the QoS to the UE as for member provided that the CSG Id received in the HANDOVER REQUEST messages corresponds to the CSG Id broadcast by the target cell.

If the CSG Membership Status IE and the CSG Id IE are received in the HANDOVER REQUEST message and the CSG Id does not correspond to the CSG Id broadcast by the target cell, the eNB may provide the QoS to the UE as for a non member and shall send back in the HANDOVER REQUEST ACKNOWLEDGE message the actual CSG Id broadcast by the target cell.

If the target cell is CSG cell or hybrid cell, the target eNB shall include the CSG ID IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the target eNB receives the CSG Id IE and the CSG Membership Status IE is set to “non member” in the HANDOVER REQUEST message and the target cell is a closed cell and at least one of the E-RABs has a particular ARP value (see 3GPP TS 23.401 [11]), the eNB shall send back the HANDOVER REQUEST ACKNOWLEDGE message to the MME accepting those E-RABs and failing the other E-RABs.

If the Subscriber Profile ID for RAT/Frequency priority IE is contained in the Source eNB to Target eNB Transparent Container IE, the target eNB shall store the content of the received Subscriber Profile ID for RAT/Frequency priority IE in the UE context and use it as defined in MFA TS 36.300 [14].

Upon reception of the UE History Information IE, which is included within the Source eNB to Target eNB Transparent Container IE in the HANDOVER REQUEST message, the target eNB shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

Upon reception of the UE History Information from the UE IE, which is included within the Source eNB to Target eNB Transparent Container IE in the HANDOVER REQUEST message, the target eNB shall, if supported, store the collected information, to be used for future handover preparations.

If the Mobility Information IE is included within the Source eNB to Target eNB Transparent Container IE in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information and use it as defined in MFA TS 36.300 [14].

If the Expected UE Behaviour IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information and may use it to determine the RRC connection time.

If the Bearer Type IE is included in the HANDOVER REQUEST message and is set to “non IP”, then the eNB shall not perform header compression for the concerned E-RAB.

After all necessary resources for the admitted E-RABs have been allocated, the target eNB shall generate the HANDOVER REQUEST ACKNOWLEDGE message. The target eNB shall include in the E-RABs Admitted List IE the E-RABs for which resources have been prepared at the target cell. The E-RABs that have not been admitted in the target cell, if any, shall be included in the E-RABs Failed to Setup List IE.

If the HANDOVER REQUEST message contains the Data Forwarding Not Possible IE associated with a given E-RAB within the E-RABs To Be Setup List IE set to “Data forwarding not possible”, then the target eNB may decide not to include the DL Transport Layer Address IE and the DL GTP-TEID IE and for intra LTE handover the UL Transport Layer Address IE and the UL GTP-TEID IE within the E-RABs Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message for that E-RAB.

For each bearer that target eNB has decided to admit and for which DL forwarding IE is set to “DL forwarding proposed”, the target eNB may include the DL GTP-TEID IE and the DL Transport Layer Address IE within the E-RABs Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message indicating that it accepts the proposed forwarding of downlink data for this bearer.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL GTP-TEID IE and the UL Transport Layer Address IE for a given bearer in the E-RABs Admitted List IE, then it means the target eNB has requested the forwarding of uplink data for this given bearer.

If the Request Type IE is included in the HANDOVER REQUEST message, then the target eNB should perform the requested location reporting functionality for the UE as described in subclause 8.11.

If the UE Security Capabilities IE included in the HANDOVER REQUEST message only contains the EIA0 algorithm as defined in MFA TS 33.401 [15] and if this EIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the eNB (see MFA TS 33.401 [15]), the eNB shall take it into use and ignore the keys received in the Security Context IE.

The GUMMEI IE shall only be contained in the HANDOVER REQUEST message according to subclauses 4.6.2 and 4.7.6.6 of MFA TS 36.300 [14]. If the GUMMEI IE is present, the target eNB shall store this information in the UE context and use it for subsequent X2 handovers.

The MME UE S1AP ID 2 IE shall only be contained in the HANDOVER REQUEST message according to subclause 4.6.2 of MFA TS 36.300 [14].If the MME UE S1AP ID 2 IE is present, the target eNB shall store this information in the UE context and use it for subsequent X2 handovers.

If the Management Based MDT Allowed IE only or the Management Based MDT Allowed IE and the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, store the received information in the UE context, and use this information to allow subsequent selections of the UE for management based MDT defined in 3GPP TS 32.422 [10].

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the HANDOVER REQUEST contains a Target Cell ID IE, as part of the Source eNB to Target eNB Transparent Container IE, for a cell which is no longer active, the eNB may respond with an HANDOVER REQUEST ACKNOWLEDGE in case the PCI of the deactivated cell is in use by another active cell.

If the ProSe Authorized IE is contained in the HANDOVER REQUEST message and it contains one or more IEs set to “authorized”, the eNB shall, if supported, consider that the UE is authorized for the relevant ProSe service(s).

If the UE User Plane CIoT Support Indicator IE is included in the HANDOVER REQUEST message and is set to "supported", the eNB shall, if supported, consider that User Plane CIoT EPS Optimisation as specified in 3GPP TS 23.401 [11] is supported for the UE.

If the CE-mode-B Support Indicator IE is included in the HANDOVER REQUEST ACKNOWLEDGE message and set to "supported", the MME shall, if supported, use the extended NAS timer settings for the UE as specified in MFA TS 24.301 [24].

#### 8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: Handover resource allocation: unsuccessful operation

If the target eNB does not admit at least one non-GBR E-RAB, or a failure occurs during the Handover Preparation, it shall send the HANDOVER FAILURE message to the MME with an appropriate cause value.

If the target eNB does not receive the CSG Membership Status IE but does receive the CSG Id IE in the HANDOVER REQUEST message and the CSG Id does not correspond to the CSG Id of the target cell, the target eNB shall send the HANDOVER FAILURE message to the MME with an appropriate cause value.

If the target eNB receives a HANDOVER REQUEST message containing RRC Container IE that does not include required information as specified in MFA TS 36.331 [16], the target eNB shall send the HANDOVER FAILURE message to the MME.

#### 8.4.2.4	Abnormal Conditions

If the target eNB receives a HANDOVER REQUEST message containing a E-RAB Level QoS Parameters IE which contains a QCI IE indicating a GBR bearer (as defined in 3GPP TS 23.203 [13]), and which does not contain the GBR QoS Information IE, the target eNB shall not admit the corresponding E-RAB.

If the target eNB receives a HANDOVER REQUEST message containing several E-RAB ID IEs (in the E-RABs To Be Setup List IE) set to the same value, the target eNB shall not admit the corresponding E-RABs.

If the Subscriber Profile ID for RAT/Frequency priority IE is not contained in the Source eNB to Target eNB Transparent Container IE whereas available in the source eNB, the target eNB shall trigger a local error handling.

NOTE:	It is assumed that the information needed to verify this condition is visible within the system, see subclause 4.1.

If the supported algorithms for encryption defined in the Encryption Algorithms IE in the UE Security Capabilities IE, plus the mandated support of EEA0 in all UEs (see MFA TS 33.401 [15]), do not match any allowed algorithms defined in the configured list of allowed encryption algorithms in the eNB (see MFA TS 33.401 [15]), the target eNB shall reject the procedure using the HANDOVER FAILURE message.

If the supported algorithms for integrity defined in the Integrity Protection Algorithms IE in the UE Security Capabilities IE, plus the mandated support of the EIA0 algorithm in all UEs (see MFA TS 33.401 [15]), do not match any allowed algorithms defined in the configured list of allowed integrity protection algorithms in the eNB (see TS 33.401 [15]), the target eNB shall reject the procedure using the HANDOVER FAILURE message.

If the target eNB receives a HANDOVER REQUEST message which does not contain the Handover Restriction List IE, and the serving PLMN cannot be determined otherwise by the eNB, the target eNB shall reject the procedure using the HANDOVER FAILURE message.

If the target eNB receives a HANDOVER REQUEST message containing the Handover Restriction List IE, and the serving PLMN indicated is not supported by the target cell, the target eNB shall reject the procedure using the HANDOVER FAILURE message.

### 8.4.3	Handover Notification

#### 8.4.3.1	General

The purpose of the Handover Notification procedure is to indicate to the MME that the UE has arrived to the target cell and the S1 handover has been successfully completed.

#### 8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Handover notification

The target eNB shall send the HANDOVER NOTIFY message to the MME when the UE has been identified in the target cell and the S1 handover has been successfully completed.

If the Tunnel Information for BBF IE is received in the HANDOVER NOTIFY message, the MME shall, if supported, use it in the core network as specified in 3GPP TS 23.139 [37].

If the LHN ID IE is included in the HANDOVER NOTIFY message, the MME shall, if supported, use it as specified in 3GPP TS 23.401 [11].

#### 8.4.3.3	Abnormal Conditions

Not applicable.

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to request the switch of a downlink GTP tunnel towards a new GTP tunnel endpoint.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The eNB initiates the procedure by sending the PATH SWITCH REQUEST message to the MME.

If the ‎E-RAB To Be Switched in Downlink List IE in the PATH SWITCH REQUEST message does not include all E-RABs previously included in the UE Context, the MME shall consider the non included E-RABs as implicitly released by the eNB.

After all necessary updates including the UP path switch have been successfully completed in the EPC for at least one of the E-RABs included in the PATH SWITCH REQUEST E-RAB To Be Switched in Downlink List IE, the MME shall send the PATH SWITCH REQUEST ACKNOWLEDGE message to the eNB and the procedure ends. The UE-associated logical S1-connection shall be established at reception of the PATH SWITCH REQUEST ACKNOWLEDGE message.

In case the EPC failed to perform the UP path switch for at least one, but not all, of the E-RABs included in the PATH SWITCH REQUEST E-RAB To Be Switched in Downlink List IE, the MME shall include the E-RABs it failed to perform UP path switch in the PATH SWITCH REQUEST ACKNOWLEDGE E-RAB To Be Released List IE. In this case, the eNB shall release the corresponding data radio bearers, and the eNB shall regard the E-RABs indicated in the E-RAB To Be Released List IE as being fully released.

If the CSG Id IE and no Cell Access Mode IE are received in the PATH SWITCH REQUEST message, the MME shall use it in the core network as specified in 3GPP TS 23.401 [11]. If the CSG Id IE and the Cell Access Mode IE set to “hybrid” are received in the PATH SWITCH REQUEST message, the MME shall decide the membership status of the UE and use it in the core network as specified in 3GPP TS 23.401 [11]. If no CSG Id IE and no Cell Access Mode IE are received in the PATH SWITCH REQUEST message and the UE was previously either in a CSG cell or in a hybrid cell, the MME shall consider that the UE has moved into a cell that is neither a CSG cell nor a hybrid cell and use this as specified in 3GPP TS 23.401 [11].

If the GUMMEI of the MME currently serving the UE is available at the eNB (see MFA TS 36.300 [14]) the eNB shall include the Source MME GUMMEI IE within the PATH SWITCH REQUEST message.

Upon reception of the PATH SWITCH REQUEST ACKNOWLEDGE message the eNB shall store the received Security Context IE in the UE context and the eNB shall use it for the next X2 handover or Intra eNB handovers as specified in MFA TS 33.401 [15].

The PATH SWITCH REQUEST ACKNOWLEDGE message may contain

-	the UE Aggregate Maximum Bit Rate IE.

-	the MME UE S1AP ID 2 IE, which indicates the MME UE S1AP ID assigned by the MME.

If the UE Aggregate Maximum Bit Rate IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message the eNB shall

- replace the previously provided UE Aggregate Maximum Bit Rate by the received UE Aggregate Maximum Bit Rate in the UE context; the eNB shall use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

If the UE Aggregate Maximum Bit Rate IE is not contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall use the previously provided UE Aggregate Maximum Bit Rate which is stored in the UE context.

In case the EPC decides to change the uplink termination point of the tunnels, it may include the E-RAB To Be Switched in Uplink List IE in the PATH SWITCH REQUEST ACKNOWLEDGE message to specify a new uplink transport layer address and uplink GTP-TEID for each respective E-RAB for which it wants to change the uplink tunnel termination point.

When the eNB receives the PATH SWITCH REQUEST ACKNOWLEDGE message and if this message includes the E-RAB To Be Switched in Uplink List IE, the eNB shall start delivering the uplink packets of the concerned E-RABs to the new uplink tunnel endpoints as indicated in the message.

When the eNB receives the PATH SWITCH REQUEST ACKNOWLEDGE message including the CSG Membership Status IE, and if the cell that serves the UE is a hybrid cell, the eNB shall use it as defined in MFA TS 36.300 [14].

If the MME UE S1AP ID 2 IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall store this information in the UE context and use it for subsequent X2 handovers.

If the Tunnel Information for BBF IE is received in the PATH SWITCH REQUEST message, the MME shall, if supported, use it in the core network as specified in 3GPP TS 23.139 [37].

If the LHN ID IE is included in the PATH SWITCH REQUEST message, the MME shall, if supported, use it as specified in 3GPP TS 23.401 [11].

If the ProSe Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the eNB shall, if supported, update its ProSe authorization information for the UE accordingly. If the ProSe Authorized IE includes one or more IEs set to “not authorized”, the eNB shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant ProSe service(s).

If the UE User Plane CIoT Support Indicator IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and is set to "supported", the eNB shall, if supported, consider that User Plane CIoT EPS Optimisation as specified in 3GPP TS 23.401[11] is supported for the UE.

#### 8.4.4.3	Unsuccessful Operation

Figure 8.4.4.3-1: Path switch request: unsuccessful operation

If the EPC fails to switch the downlink GTP tunnel endpoint towards a new GTP tunnel endpoint for all E-RABs included in the E-RAB To Be Switched in Downlink List IE during the execution of the Path Switch Request procedure, the MME shall send the PATH SWITCH REQUEST FAILURE message to the eNB with an appropriate cause value. In this case, the eNB should decide its subsequent actions and the MME should behave as described in 3GPP TS 23.401 [11].

#### 8.4.4.4	Abnormal Conditions

If the MME receives a PATH SWITCH REQUEST message containing several E-RAB ID IEs (in the E-RAB To Be Switched in Downlink List IE) set to the same value, the MME shall send the PATH SWITCH REQUEST FAILURE message to the eNB.

If the MME receives a PATH SWITCH REQUEST message without the CSG Membership Status IE, and the cell accessed by the UE is a hybrid cell with a different CSG from the source cell or the source cell does not have a CSG ID, the MME shall send the PATH SWITCH REQUEST FAILURE message to the eNB.

If the CSG Membership Status IE is not included in the PATH SWITCH REQUEST ACKNOWLEDGE message and the cell accessed by the UE is a hybrid cell with a different CSG from the source cell or the source cell does not have a CSG ID, the eNB shall consider the procedure as unsuccessfully terminated and initiate local error handling.

### 8.4.5	Handover Cancellation

#### 8.4.5.1	General

The purpose of the Handover Cancel procedure is to enable a source eNB to cancel an ongoing handover preparation or an already prepared handover.

The procedure uses UE-associated signalling.

#### 8.4.5.2	Successful Operation

Figure 8.4.5.2-1: Handover Cancel procedure. Successful operation.

The source eNB initiates the procedure by sending a HANDOVER CANCEL message to the EPC.

The HANDOVER CANCEL message shall indicate the reason for cancelling the handover with the appropriate value of the Cause IE.

Upon reception of a HANDOVER CANCEL message, the EPC shall terminate the ongoing Handover Preparation procedure, release any resources associated with the handover preparation and send a HANDOVER CANCEL ACKNOWLEDGE message to the source eNB.

Transmission and reception of a HANDOVER CANCEL ACKNOWLEDGE message terminate the procedure in the EPC and in the source eNB. After this, the source eNB does not have a prepared handover for that UE-associated logical S1-connection.

#### 8.4.5.3	Unsuccessful Operation

Not applicable.

#### 8.4.5.4	Abnormal Conditions

If the source eNB becomes aware of the fact that an expected HANDOVER CANCEL ACKNOWLEDGE message is missing, the source eNB shall consider the Handover Cancellation as successfully terminated.

### 8.4.6	eNB Status Transfer

#### 8.4.6.1	General

The purpose of the eNB Status Transfer procedure is to transfer the uplink PDCP-SN and HFN receiver status and the downlink PDCP-SN and HFN transmitter status from the source to the target eNB via the MME during an intra LTE S1 handover for each respective E-RAB for which PDCP-SN and HFN status preservation applies.

#### 8.4.6.2	Successful Operation

Figure 8.4.6.2-1: eNB Status Transfer procedure

The source eNB initiates the procedure by stopping assigning PDCP-SNs to downlink SDUs and sending the eNB STATUS TRANSFER message to the MME at the point in time when it considers the transmitter/receiver status to be frozen.

-	For each E-RAB for which PDCP-SN and HFN status preservation applies the source eNB shall include the E-RAB ID IE, the UL COUNT value IE and the DL COUNT value IE within the E-RABs Subject to Status Transfer Item IE in the eNB Status Transfer Transparent Container IE of the eNB STATUS TRANSFER message.

-	In case of 15 bit long PDCP-SN, for each E-RAB for which PDCP-SN and HFN status preservation applies, the source eNB shall additionally include the UL COUNT Value Extended IE and the DL COUNT Value Extended IE within the E-RABs Subject to Status Transfer Item IE.

-	In case of 18 bit long PDCP-SN, for each E-RAB for which PDCP-SN and HFN status preservation applies, the source eNB shall additionally include the UL COUNT Value for PDCP SN Length 18 IE and the DL COUNT Value for PDCP SN Length 18 IE within the E-RABs Subject to Status Transfer Item IE.

The source eNB may also include in the eNB STATUS TRANSFER message the missing and the received uplink SDUs in the Receive Status Of UL PDCP SDUs IE, or in the Receive Status Of UL PDCP SDUs Extended IE in case of 15 bit long PDCP-SN, or in the Receive Status Of UL PDCP SDUs for PDCP SN Length 18 IE in case of 18 bit long PDCP-SN, for each bearer for which the source eNB has accepted the request from the target eNB for uplink forwarding.

#### 8.4.6.3	Unsuccessful Operation

Not applicable.

#### 8.4.6.4	Abnormal Conditions

Not applicable.

### 8.4.7	MME Status Transfer

#### 8.4.7.1	General

The purpose of the MME Status Transfer procedure is to transfer the uplink PDCP-SN and HFN receiver status and the downlink PDCP-SN and HFN transmitter status from the source to the target eNB via the MME during an S1 handover for each respective E-RAB for which PDCP-SN and HFN status preservation applies.

#### 8.4.7.2	Successful Operation

Figure 8.4.7.2-1: MME Status Transfer procedure

The MME initiates the procedure by sending the MME STATUS TRANSFER message to the eNB. The target eNB using Full Configuration for this handover as per MFA TS 36.300 [14] shall ignore the information received in this message.

For each bearer within the E-RABs Subject to Status Transfer List IE within the eNB Status Transfer Transparent Container IE for which the UL COUNT value IE is received in the MME STATUS TRANSFER message, the target eNB shall apply the contained information and shall not deliver any uplink packet which has a PDCP-SN lower than the value contained in the PDCP-SN IE of this IE. If the UL COUNT Value Extended IE or UL COUNT Value for PDCP SN Length 18 IE is included in the E-RABs Subject to Status Transfer Item IE, the target eNB shall, if supported, use the value contained in the PDCP-SN Extended IE in the UL COUNT Value Extended IE or PDCP-SN Length 18 IE of the UL COUNT Value for PDCP SN Length 18 IE instead of the value contained in the PDCP-SN IE of the UL COUNT value IE.

For each bearer in E-RABs Subject to Status Transfer List IE within the eNB Status Transfer Transparent Container IE received in the MME STATUS TRANSFER message, the target eNB shall use DL COUNT value IE for the first downlink packet for which there is no PDCP-SN yet assigned. If the DL COUNT Value Extended IE or DL COUNT Value for PDCP SN Length 18 IE is included in the E-RABs Subject to Status Transfer Item IE, the target eNB shall, if supported, use the DL COUNT Value Extended IE or DL COUNT Value for PDCP SN Length 18 IE instead of the DL COUNT value IE.

If the Receive Status Of UL PDCP SDUs IE or the Receive Status Of UL PDCP SDUs Extended IE or the Receive Status Of UL PDCP SDUs for PDCP SN Length 18 IE is included for at least one bearer in the eNB Status Transfer Transparent Container IE of the MME STATUS TRANSFER message, the target eNB may use it in a Status Report message sent to the UE over the radio interface.

#### 8.4.7.3	Unsuccessful Operation

Not applicable.

#### 8.4.7.4	Abnormal Conditions

If the target eNB receives this message for a UE for which no prepared handover exists at the target eNB, the target eNB shall ignore the message.

## 8.5	Paging

### 8.5.1	General

The purpose of the Paging procedure is to enable the MME to page a UE in the specific eNB.

### 8.5.2	Successful Operation

Figure 8.5.2-1: Paging procedure

The MME initiates the paging procedure by sending the PAGING message to the eNB.

At the reception of the PAGING message, the eNB shall perform paging of the UE in cells which belong to tracking areas as indicated in the List of TAIs IE.

The CN Domain IE shall be transferred transparently to the UE.

The Paging DRX IE may be included in the PAGING message, and if present the eNB shall use it according to MFA TS 36.304 [20].

A list of CSG IDs may be included in the PAGING message.

If included, the E-UTRAN may use the list of CSG IDs to avoid paging the UE at CSG cells whose CSG ID does not appear in the list.

For each cell that belongs to any of the TAs indicated in the List of TAIs IE, the eNB shall generate one page on the radio interface.

The Paging Priority IE may be included in the PAGING message, and if present the eNB may use it according to 3GPP TS 23.401 [11] and 3GPP TS 23.272 [17].

If the UE Radio Capability for Paging IE is included in the PAGING message, the eNB may use it to apply specific paging schemes.

If the Assistance Data for Recommended Cells IE is included in the Assistance Data for Paging IE it may be used, together with the Paging Attempt Information IE if also present according to MFA TS 36.300 [14].

If the Assistance Data for CE capable UEs IE is included in the Assistance Data for Paging IE, it may be used for paging the indicated CE capable UE, together with the Paging Attempt Information IE according to MFA TS 36.300 [14].

If the Next Paging Area Scope IE is included in the Paging Attempt Information IE it may be  used for paging the UE according to MFA TS 36.300 [14].

If the Paging eDRX Information IE is included in the PAGING message, the eNB shall, if supported, use it according to MFA TS 36.304 [20]. If the Paging Time Window IE is included in the Paging eDRX Information IE, the eNB shall take this information into account to determine the UE’s paging occasion according to MFA TS 36.304 [20]. The eNB should take into account the reception time of the PAGING message on the S1-MME interface to determine when to page the UE.

If the Extended UE Identity Index Value IE is included in the PAGING message, the eNB shall, if supported, use it to identify the paging resources to be used according to MFA TS 36.304 [20].

If the NB-IoT Paging eDRX Information IE is included in the PAGING message, the eNB shall, if supported, use it according to MFA TS 36.304 [20]. If the NB-IoT Paging Time Window IE is included in the NB-IoT Paging eDRX Information IE, the eNB shall take this information into account to determine the UE’s paging occasion according to MFA TS 36.304 [20]. The eNB should take into account the reception time of the PAGING message on the S1-MME interface to determine when to page the UE.

If the NB-IoT UE Identity Index Value IE is included in the PAGING message, the eNB shall, if supported, use it to identify the paging resources to be used according to MFA TS 36.304 [20].

### 8.5.3	Unsuccessful Operation

Not applicable.

### 8.5.4	Abnormal Conditions

Not applicable.

## 8.6	NAS transport

### 8.6.1	General

The purpose of the NAS Transport procedure is to carry UE – MME signalling over the S1 Interface. The NAS messages are not interpreted by the eNB, and their content is outside the scope of this specification. The procedure may use an existing UE-associated logical S1-connection. If no UE-associated logical S1-connection exists, the establishment of the UE-associated logical S1-connection is initiated (and may be established) as part of the procedure.

The NAS messages are transported in an IE of the INITIAL UE MESSAGE, DOWNLINK NAS TRANSPORT, UPLINK NAS TRANSPORT or REROUTE NAS REQUEST messages.

### 8.6.2	Successful Operations

#### 8.6.2.1	Initial UE Message

Figure 8.6.2.1-1: Initial UE Message procedure

When the eNB has received from the radio interface the first UL NAS message transmitted on an RRC connection to be forwarded to an MME, the eNB shall invoke the NAS Transport procedure and send the INITIAL UE MESSAGE message to the MME including the NAS message as a NAS-PDU IE. The eNB shall allocate a unique eNB UE S1AP ID to be used for the UE and the eNB shall include this identity in the INITIAL UE MESSAGE message. In case of network sharing, the selected PLMN is indicated by the PLMN Identity IE within the TAI IE included in the INITIAL UE MESSAGE message. When the eNB has received from the radio interface the S-TMSI IE, it shall include it in the INITIAL UE MESSAGE message. If the eNB does not support NNSF and the eNB has received from the radio interface the GUMMEI IE, the eNB may include it in the INITIAL UE MESSAGE message. If the eNB does not support NNSF and the eNB has received from the radio interface the GUMMEI Type IE, the eNB may include it in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment originating from a CSG cell, the CSG Id IE shall be included in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment originating from a Hybrid cell, the CSG Id IE and the Cell Access Mode IE shall be included in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment triggered by a Relay Node as defined in MFA TS 36.300 [14], the GW Transport Layer Address IE and the Relay Node Indicator IE shall be included in the INITIAL UE MESSAGE message.

If the eNB has a L-GW function for LIPA operation, it shall include the GW Transport Layer Address IE in the INITIAL UE MESSAGE message.

If the SIPTO L-GW Transport Layer Address IE is received in the INITIAL UE MESSAGE message, the MME shall, if supported, use it for SIPTO@LN operation as sepecified in 3GPP TS 23.401 [11].

If the LHN ID IE is included in the INITIAL UE MESSAGE message, the MME shall, if supported, use it as specified in 3GPP TS 23.401 [11].

If the Tunnel Information for BBF IE is received in the INITIAL UE MESSAGE message, the MME shall, if supported, use it in the core network as specified in 3GPP TS 23.139 [37].

If the MME Group ID IE is included in the INITIAL UE MESSAGE message this indicates that the message is a rerouted message and the MME shall, if supported, use the IE as described in 3GPP TS 23.401 [11].

If the UE Usage Type IE is included in the INITIAL UE MESSAGE message, then the selected MME in the DCN shall if supported, use it as defined in 3GPP TS 23.401 [11].

NOTE:	The first UL NAS message is always received in the RRC CONNECTION SETUP COMPLETE message.

If the CE-mode-B Support Indicator IE is included in the INITIAL UE MESSAGE message and set to "supported", the MME shall, if supported, use the extended NAS timer settings for the UE as specified in MFA TS 24.301 [24].

#### 8.6.2.2	DOWNLINK NAS TRANSPORT

Figure 8.6.2.2-1: DOWNLINK NAS Transport Procedure

If the MME only needs to send a NAS message transparently via the eNB to the UE and a UE-associated logical S1-connection exists for the UE or if the MME has received the eNB UE S1AP ID IE in an INITIAL UE MESSAGE message, the MME shall send a DOWNLINK NAS TRANSPORT message to the eNB including the NAS message as a NAS-PDU IE. If the UE-associated logical S1-connection is not established, the MME shall allocate a unique MME UE S1AP ID to be used for the UE and include that in the DOWNLINK NAS TRANSPORT message; by receiving the MME UE S1AP ID IE in the DOWNLINK NAS TRANSPORT, the eNB establishes the UE-associated logical S1-connection.

The NAS-PDU IE contains an MME – UE message that is transferred without interpretation in the eNB.

The DOWNLINK NAS TRANSPORT message may contain the Handover Restriction List IE, which may contain roaming or access restrictions.

If the Handover Restriction List IE is contained in the DOWNLINK NAS TRANSPORT message, the eNB shall store this information in the UE context.

The eNB shall use the information in Handover Restriction List IE if present in the DOWNLINK NAS TRANSPORT message to:

-	determine a target for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the DOWNLINK NAS TRANSPORT message and there is no previously stored Handover restriction information, the eNB shall consider that no roaming and no access restriction apply to the UE.

If the Subscriber Profile ID for RAT/Frequency priority IE is included in DOWNLINK NAS TRANSPORT message, the eNB shall, if supported, use it as defined in MFA TS 36.300 [14].

If the SRVCC Operation Possible IE is included in DOWNLINK NAS TRANSPORT message, the eNB shall store it in the UE context and, if supported, use it as defined in 3GPP TS 23.216 [9].

If the UE Radio Capability IE is included in the DOWNLINK NAS TRANSPORT message, the eNB shall store this information in the UE context, use it as defined in MFA TS 36.300 [14].

#### 8.6.2.3	UPLINK NAS TRANSPORT

Figure 8.6.2.3-1: UPLINK NAS TRANSPORT Procedure

When the eNB has received from the radio interface a NAS message to be forwarded to the MME to which a UE-associated logical S1-connection for the UE exists, the eNB shall send the UPLINK NAS TRANSPORT message to the MME including the NAS message as a NAS-PDU IE. The eNB shall include the TAI and ECGI of the current cell in every S1-AP UPLINK NAS TRANSPORT message.

The NAS-PDU IE contains a UE – MME message that is transferred without interpretation in the eNB.

If the GW Transport Layer Address IE is received in the UPLINK NAS TRANSPORT message, the MME shall, if supported, use it for LIPA operation as specified in 3GPP TS 23.401 [11].

If the SIPTO L-GW Transport Layer Address IE is received in the UPLINK NAS TRANSPORT message, the MME shall, if supported, use it for SIPTO@LN operation as specified in 3GPP TS 23.401 [11].

If the LHN ID IE is included in the UPLINK NAS TRANSPORT message, the MME shall, if supported, use it as specified in 3GPP TS 23.401 [11].

#### 8.6.2.4	NAS NON DELIVERY INDICATION

Figure 8.6.2.4-1: NAS NON DELIVERY INDICATION Procedure

When the eNB decides not to start the delivery of a NAS message that has been received over a UE-associated logical S1-connection or the eNB is unable to ensure that the message has been received by the UE, it shall report the non-delivery of this NAS message by sending a NAS NON DELIVERY INDICATION message to the MME including the non-delivered NAS message within the NAS-PDU IE and an appropriate cause value within an appropriate Cause IE, e.g., “S1 intra system Handover Triggered”, “S1 inter system Handover Triggered” or “X2 Handover Triggered”.

#### 8.6.2.5	Reroute NAS Request

Figure 8.6.2.5-1: Reroute NAS Request Procedure

The purpose of the Reroute NAS Request procedure is to enable the MME to request for a rerouting of the INITIAL UE MESSAGE message to the MME in the indicated DCN.

The MME initiates the procedure by sending a REROUTE NAS REQUEST message to the eNB. The eNB shall, if supported, reroute the INITIAL UE MESSAGE message to the MME in the DCN indicated by the MME Group ID IE as described in 3GPP TS 23.401 [11].

If the Additional GUTI IE is included in the REROUTE NAS REQUEST message, then the eNB shall if supported, use it when selecting the MME in the DCN as defined in 3GPP TS 23.401 [11].

If the UE Usage Type IE is included in the REROUTE NAS REQUEST message, then the eNB shall if supported, include it towards the selected MME in the DCN as defined in 3GPP TS 23.401 [11].

### 8.6.3	Unsuccessful Operation

Not applicable.

### 8.6.4	Abnormal Conditions

If the S-TMSI is not received by the MME in the INITIAL UE MESSAGE message whereas expected, the MME shall consider the procedure as failed.

## 8.7	Management procedures

### 8.7.1	Reset

#### 8.7.1.1	General

The purpose of the Reset procedure is to initialise or re-initialise the E-UTRAN, or part of E-UTRAN S1AP UE-related contexts, in the event of a failure in the EPC or vice versa. This procedure does not affect the application level configuration data exchanged during, e.g., the S1 Setup procedure.

The procedure uses non-UE associated signalling.

#### 8.7.1.2	Successful Operation

##### 8.7.1.2.1	Reset Procedure Initiated from the MME

Figure 8.7.1.2.1-1: Reset procedure initiated from the MME. Successful operation.

In the event of a failure at the MME, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the eNB.

At reception of the RESET message the eNB shall release all allocated resources on S1 and Uu related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the indicated UE contexts including S1AP ID.

After the eNB has released all assigned S1 resources and the UE S1AP IDs for all indicated UE associations which can be used for new UE-associated logical S1-connections over the S1 interface, the eNB shall respond with the RESET ACKNOWLEDGE message. The eNB does not need to wait for the release of radio resources to be completed before returning the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical S1-connection list IE, then:

-	The eNB shall use the MME UE S1AP ID IE and/or the eNB UE S1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The eNB shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical S1-connection Item IE in the UE-associated logical S1-connection list IE. The UE-associated logical S1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical S1-connections. Empty UE-associated logical S1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the MME UE S1AP ID IE is included in the UE-associated logical S1-connection Item IE for a UE association, the eNB shall include the MME UE S1AP ID IE in the corresponding UE-associated logical S1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the eNB UE S1AP ID IE is included in the UE-associated logical S1-connection Item IE for a UE association, the eNB shall include the eNB UE S1AP ID IE in the corresponding UE-associated logical S1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same S1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

##### 8.7.1.2.2	Reset Procedure Initiated from the E-UTRAN

Figure 8.7.1.2.2-1: Reset procedure initiated from the E-UTRAN. Successful operation.

In the event of a failure at the eNB, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the MME.

At reception of the RESET message the MME shall release all allocated resources on S1 related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the S1AP ID for the indicated UE associations.

After the MME has released all assigned S1 resources and the UE S1AP IDs for all indicated UE associations which can be used for new UE-associated logical S1-connections over the S1 interface, the MME shall respond with the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical S1-connection list IE, then:

-	The MME shall use the MME UE S1AP ID IE and/or the eNB UE S1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The MME shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical S1-connection Item IE in the UE-associated logical S1-connection list IE. The UE-associated logical S1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical S1-connections. Empty UE-associated logical S1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the MME UE S1AP ID IE is included in the UE-associated logical S1-connection Item IE for a UE association, the MME shall include the MME UE S1AP ID IE in the corresponding UE-associated logical S1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the eNB UE S1AP ID IE is included in a UE-associated logical S1-connection Item IE for a UE association, the MME shall include the eNB UE S1AP ID IE in the corresponding UE-associated logical S1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same S1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

#### 8.7.1.3	Abnormal Conditions

##### 8.7.1.3.1	Abnormal Condition at the EPC

If the RESET message includes the UE-associated logical S1-connection list IE, but neither the MME UE S1AP ID IE nor the eNB UE S1AP ID IE is present for a UE-associated logical S1-connection Item IE, then the MME shall ignore the UE-associated logical S1-connection Item IE. The MME may return the empty UE-associated logical S1-connection Item IE in the UE-associated logical S1-connection list IE in the RESET ACKNOWLEDGE message.

##### 8.7.1.3.2	Abnormal Condition at the E-UTRAN

If the RESET message includes the UE-associated logical S1-connection list IE, but neither the MME UE S1AP ID IE nor the eNB UE S1AP ID IE is present for a UE-associated logical S1-connection Item IE, then the eNB shall ignore the UE-associated logical S1-connection Item IE. The eNB may return the empty UE-associated logical S1-connection Item IE in the UE-associated logical S1-connection list IE in the RESET ACKNOWLEDGE message.

##### 8.7.1.3.3	Crossing of Reset Messages

If a Reset procedure is ongoing in the eNB and the eNB receives a RESET message from the peer entity on the same S1 interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received RESET message, the eNB shall respond with the RESET ACKNOWLEDGE message as described in 8.7.1.2.1.

If a Reset procedure is ongoing in the MME and the MME receives a RESET message from the peer entity on the same S1 interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received RESET message, the MME shall respond with the RESET ACKNOWLEDGE message as described in 8.7.1.2.2.

### 8.7.2	Error Indication

#### 8.7.2.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE associated signalling, then the Error Indication procedure uses UE associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: Error Indication procedure, MME originated. Successful operation.

Figure 8.7.2.2-2: Error Indication procedure, eNB originated. Successful operation.

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE. In case the Error Indication procedure is triggered by utilising UE associated signalling the MME UE S1AP ID IE and the eNB UE S1AP ID IE shall be included in the ERROR INDICATION message. If one or both of MME UE S1AP ID IE and the eNB UE S1AP ID IE are not correct, the cause shall be set to appropriate value, e.g., “Unknown or already allocated MME UE S1AP ID”, “Unknown or already allocated eNB UE S1AP ID” or “Unknown or inconsistent pair of UE S1AP ID”.

#### 8.7.2.3	Abnormal Conditions

Not applicable.

### 8.7.3	S1 Setup

#### 8.7.3.1	General

The purpose of the S1 Setup procedure is to exchange application level data needed for the eNB and the MME to correctly interoperate on the S1 interface. This procedure shall be the first S1AP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received and clears MME overload state information at the eNB. If the eNB and MME do not agree on retaining the UE Contexts this procedure also re-initialises the E-UTRAN S1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do. If the eNB initiating the S1 Setup procedure supports a CSG cell, the procedure shall report the CSG ID(s) of the supported CSGs.

#### 8.7.3.2	Successful Operation

Figure 8.7.3.2-1: S1 Setup procedure: Successful Operation.

The eNB initiates the procedure by sending a S1 SETUP REQUEST message including the appropriate data to the MME. The MME responds with a S1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used for the duration of the TNL association. When this procedure is finished, the S1 interface is operational and other S1 messages can be exchanged.

If the eNB initiating the S1 SETUP procedure supports one (or more) CSG cell(s), the S1 SETUP REQUEST message shall contain the CSG ID(s) of the supported CSG(s).

If the S1 SETUP REQUEST message contains the eNB Name IE the MME may use this IE as a human readable name of the eNB.

If the S1 SETUP RESPONSE message contains the MME Name IE the eNB may use this IE as a human readable name of the MME.

If the MME Relay Support Indicator IE is included in the S1 SETUP RESPONSE message, the eNB shall consider this information when selecting an appropriate MME for the Relay Node.

If the UE Retention Information IE set to “ues-retained“ was included in the S1 SETUP REQUEST message, then the MME may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the S1 SETUP RESPONSE message.

If the NB-IoT Default Paging DRX IE is included in the S1 SETUP REQUEST message, the MME will take it into account as specified in MFA TS 36.300 [14].

If the S1-SETUP RESPONSE message contains PSP IDs, the eNB shall announce these identities as PSPs (see MFA TS MF.202 [MF1]).

#### 8.7.3.3	Unsuccessful Operation

Figure 8.7.3.3-1: S1 Setup procedure: Unsuccessful Operation.

If the MME cannot accept the setup, it should respond with a S1 SETUP FAILURE and appropriate cause value.

If the S1 SETUP FAILURE message includes the Time To Wait IE, the eNB shall wait at least for the indicated time before reinitiating the S1 setup towards the same MME.

#### 8.7.3.4	Abnormal Conditions

If the eNB initiates the procedure by sending a S1 SETUP REQUEST message including the PLMN Identity IEs and none of the PLMNs provided by the eNB is identified by the MME, then the MME shall reject the eNB S1 Setup Request procedure with the appropriate cause value, e.g., “Unknown PLMN”.

### 8.7.4	eNB Configuration Update

#### 8.7.4.1	General

The purpose of the eNB Configuration Update procedure is to update application level configuration data needed for the eNB and the MME to interoperate correctly on the S1 interface. This procedure does not affect existing UE-related contexts, if any.

#### 8.7.4.2	Successful Operation

Figure 8.7.4.2-1: ENB Configuration Update procedure: Successful Operation.

The eNB initiates the procedure by sending an ENB CONFIGURATION UPDATE message to the MME including an appropriate set of updated configuration data that it has just taken into operational use. The MME responds with ENB CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If information element(s) is/are not included in the ENB CONFIGURATION UPDATE message, the MME shall interpret that the corresponding configuration data is/are not changed and shall continue to operate the S1 with the existing related configuration data.

If the supported TA(s) is/are to be updated, the whole list of supported TAs, including those that are not to be updated, shall be included in the Supported TAs IE. The MME shall overwrite the whole list of TAs.

If the supported CSG ID(s) is/are to be updated, the whole list of supported CSG IDs, including those that are not to be updated, shall be included in the CSG Id List IE. The MME shall overwrite the whole list of CSG Ids.

If the ENB CONFIGURATION UPDATE message contains the eNB Name IE, the MME may use this IE as a human readable name of the eNB.

If the Default Paging DRX IE is included, the MME shall overwrite any previously stored default paging DRX value for the eNB.

If the NB-IoT Default Paging DRX IE is included in the ENB CONFIGURATION UPDATE message, the MME shall overwrite any previously stored NB-IoT default paging DRX value for the eNB.

The updated configuration data shall be stored in both the eNB and the MME and used for the duration of the TNL association or until any further update is triggered by the eNB.

The eNB may initiate a further eNB Configuration Update procedure only after a previous eNB Configuration Update procedure has been completed.

#### 8.7.4.3	Unsuccessful Operation

Figure 8.7.4.3-1: ENB Configuration Update procedure: Unsuccessful Operation.

If the MME cannot accept the update, it shall respond with an ENB CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the ENB CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the eNB shall wait at least for the indicated time before reinitiating the ENB Configuration Update procedure towards the same MME. Both nodes shall continue to operate the S1 with their respective configuration data.

#### 8.7.4.4	Abnormal Conditions

If the eNB after initiating eNB Configuration Update procedure receives neither an ENB CONFIGURATION UPDATE ACKOWLEDGE nor an ENB CONFIGURATION UPDATE FAILURE message, the eNB may reinitiate a further eNB Configuration Update procedure towards the same MME, provided that the content of the new ENB CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged ENB CONFIGURATION UPDATE message.

### 8.7.5	MME Configuration Update

#### 8.7.5.1	General

The purpose of the MME Configuration Update procedure is to update application level configuration data needed for the eNB and MME to interoperate correctly on the S1 interface. This procedure does not affect existing UE-related contexts, if any.

#### 8.7.5.2	Successful Operation

Figure 8.7.5.2-1: MME Configuration Update procedure: Successful Operation.

The MME initiates the procedure by sending an MME CONFIGURATION UPDATE message including the appropriate updated configuration data to the eNB. The eNB responds with an MME CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If information element(s) is/are not included in the MME CONFIGURATION UPDATE message, the eNB shall interpret that the corresponding configuration data is not changed and shall continue to operate the S1 with the existing related configuration data.

If the served PLMNs is/are to be updated, the eNB shall overwrite the whole list of PLMNs.

If the MME CONFIGURATION UPDATE message contains the MME Name IE, the eNB may use this IE as a human readable name of the MME.

The updated configuration data shall be stored in the respective node and used for the duration of the TNL association or until any further update is performed from the MME.

The MME may initiate a further MME Configuration Update procedure only after a previous MME Configuration Update procedure has been completed.

If the PSP IDs are to be updated, the eNB shall overwrite the whole list of PSP IDs. If all PSP IDs should be removed, the MME shall remove NHAMI as a served PLMN (see MFA TS MF.202 [MF1]).

#### 8.7.5.3	Unsuccessful Operation

Figure 8.7.5.3-1: MME Configuration Update: Unsuccessful Operation.

If the eNB cannot accept the update, it shall respond with an MME CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the MME CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE the MME shall wait at least for the indicated time before reinitiating the MME Configuration Update procedure towards the same eNB. Both nodes shall continue to operate the S1 with the existing configuration data.

#### 8.7.5.4	Abnormal Conditions

If the MME neither receives an MME CONFIGURATION UPDATE ACKOWLEDGE nor an MME CONFIGURATION UPDATE FAILURE message, the MME may reinitiate MME Configuration Update procedure towards the same eNB provided that the content of the new MME CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged MME CONFIGURATION UPDATE message.

### 8.7.6	Overload Start

#### 8.7.6.1	General

The purpose of the Overload Start procedure is to inform an eNB to reduce the signalling load towards the concerned MME.

The procedure uses non-UE associated signalling.

#### 8.7.6.2	Successful Operation

Figure 8.7.6.2-1: Overload Start procedure

The eNB receiving the OVERLOAD START message shall assume the MME from which it receives the message as being in an overloaded state.

If the Overload Action IE in the Overload Response IE within the OVERLOAD START message is set to

-	"reject RRC connection establishments for non-emergency mobile originated data transfer" (i.e., reject traffic corresponding to RRC cause "mo-data", "mo-VoiceCall" and "delayTolerantAccess" in MFA TS 36.331 [16]), or

-	"reject RRC connection establishments for signalling" (i.e., reject traffic corresponding to RRC cause "mo-data", "mo-signalling", "mo-VoiceCall" and "delayTolerantAccess" in MFA TS 36.331 [16]), or

-	"only permit RRC connection establishments for emergency sessions and mobile terminated services" (i.e., only permit traffic corresponding to RRC cause "emergency" and "mt-Access" in MFA TS 36.331 [16]), or

-	"only permit RRC connection establishments for high priority sessions and mobile terminated services" (i.e., only permit traffic corresponding to RRC cause "highPriorityAccess" and "mt-Access" in MFA TS 36.331 [16]), or

-	"reject only RRC connection establishment for delay tolerant access" (i.e., only reject traffic corresponding to RRC cause "delayTolerantAccess" in MFA TS 36.331 [16]),

the eNB shall:

-	if the Traffic Load Reduction Indication IE is included in the OVERLOAD START message and, if supported, reduce the signalling traffic indicated as to be rejected by the indicated percentage,

-	otherwise ensure that only the signalling traffic not indicated as to be rejected is sent to the MME.

NOTE:	When the Overload Action IE is set to "only permit RRC connection establishments for emergency sessions and mobile terminated services", emergency calls with RRC cause "highPriorityAccess" from high priority users are rejected (see MFA TS 24.301 [24]).

If the GUMMEI List IE is present, the eNB shall, if supported, use this information to identify to which traffic the above defined rejections shall be applied.

If an overload action is ongoing and the eNB receives a further OVERLOAD START message, the eNB shall replace the ongoing overload action with the newly requested one. If the GUMMEI List IE is present, the eNB replaces applicable ongoing actions according to MFA TS 36.300 [14], clauses 4.6.2, 4.7.4 and 19.2.2.12.

#### 8.7.6.3	Unsuccessful Operation

Not applicable.

### 8.7.7	Overload Stop

#### 8.7.7.1	General

The purpose of the Overload Stop procedure is to signal to an eNB the MME is connected to that the overload situation at the MME has ended and normal operation shall resume.

The procedure uses non-UE associated signalling.

#### 8.7.7.2	Successful Operation

Figure 8.7.7.2.-1: Overload Stop procedure

The eNB receiving the OVERLOAD STOP message shall assume that the overload situation at the MME from which it receives the message has ended and shall resume normal operation for the applicable traffic towards this MME.

If the GUMMEI List IE is present, the eNB shall, if supported, use this information to identify which traffic to cease rejecting, and proceed according to MFA TS 36.300 [14], clauses 4.6.2, 4.7.4 and 19.2.2.12. If no particular overload action is ongoing for a particular GUMMEI value, the eNB shall ignore this value.

#### 8.7.7.3	Unsuccessful Operation

Not applicable.

## 8.8	S1 CDMA2000 Tunnelling Procedures

### 8.8.1	General

The purpose of S1 CDMA2000 Tunnelling procedures is to carry CDMA2000 signalling between UE and CDMA2000 RAT over the S1 Interface. This includes signalling for pre-registration of UE with CDMA2000 HRPD network, signalling for handover preparation for handover from E-UTRAN to CDMA2000 HRPD/1xRTT and pre-registration and paging of UE with CDMA2000 1xRTT CS system. The CDMA2000 messages are not interpreted by the eNB, and their content is outside the scope of this specification, however, additional information may be sent along with the tunnelled CDMA2000 message to assist the eNB and the MME in the tunnelling procedure. These procedures use an established UE-associated logical S1-connection.

The CDMA2000 messages are transported in an IE of the DOWNLINK S1 CDMA2000 TUNNELLING or UPLINK S1 CDMA2000 TUNNELLING messages.

### 8.8.2	Successful Operations

#### 8.8.2.1	Downlink S1 CDMA2000 Tunnelling

Figure 8.8.2.1-1: Downlink S1 CDMA2000 Tunnelling Procedure

If a CDMA2000 message needs to be sent from the MME to a given UE and a UE-associated logical S1-connection exists for that given UE, the MME should send a DOWNLINK S1 CDMA2000 TUNNELLING message to the eNB including the CDMA2000 message in the CDMA2000-PDU IE. The eNB forwards the received CDMA2000-PDU IE to the UE along with an indication of the RAT Type associated with the CDMA2000-PDU IE based on the CDMA2000 RAT Type IE.

If the MME receives handover status information along with the tunnelled downlink CDMA2000 message, the MME should include the handover status information in the CDMA2000 HO Status IE in the DOWNLINK S1 CDMA2000 TUNNELLING message.

If the DOWNLINK S1 CDMA2000 TUNNELLING message contains the E-RABs Subject to Forwarding List IE, it indicates that DL forwarding is available for the indicated E-RABs towards the tunnel endpoint identified by the DL GTP-TEID IE for those E-RABs.

#### 8.8.2.2	Uplink S1 CDMA2000 Tunnelling

Figure 8.8.2.2-1: Uplink S1 CDMA2000 Tunnelling Procedure

When the eNB has received from the radio interface a CDMA2000 message to be forwarded to the MME in which a UE-associated logical S1-connection for a given UE exists, the eNB shall send the UPLINK S1 CDMA2000 TUNNELLING message to the MME including the CDMA2000 message in the CDMA2000-PDU IE.

If the MME receives the CDMA2000 HO Required Indication IE set to “true” in UPLINK S1 CDMA2000 TUNNELLING message, the MME shall send the necessary handover preparation information to the CDMA2000 target RAT.

If the MME receives any of the CDMA2000 1xRTT SRVCC Info IE, or the CDMA2000 1xRTT RAND IE in the UPLINK S1 CDMA2000 TUNNELLING message, the MME shall forward the received information to the CDMA2000 1xRTT RAT.

If the MME receives the E-UTRAN Round Trip Delay Estimation Info IE in the UPLINK S1 CDMA2000 TUNNELLING message, the MME shall forward the received information to the target HRPD access. The MME shall forward the received CDMA2000 Sector ID IE and CDMA2000-PDU IE to the proper destination node in the CDMA2000 RAT.

Interactions with E-RAB Management procedures:

If, after an UPLINK S1 CDMA2000 TUNNELLING message with CDMA2000 HO Required Indication IE set to “true” is sent before the DOWNLINK S1 CDMA2000 TUNNELLING message with CDMA2000 HO Status IE is received, the source eNB receives an MME initiated E-RAB Management procedure on the same UE associated signalling connection, the source eNB shall terminate the MME initiated E-RAB Management procedure by sending the appropriate response message with an appropriate cause value, e.g., “S1 inter system Handover Triggered”, to the MME.

### 8.8.3	Unsuccessful Operation

Not applicable

### 8.8.4	Abnormal Conditions

If the eNB receives at least one E-RAB ID included in the E-RABs Subject to Forwarding Items IE without any associated DL GTP-TEID and DL Transport Layer Address pair in the DOWNLINK S1 CDMA2000 TUNNELLING message, the eNB shall consider it as a logical error and act as described in subclause 10.4.

The eNB shall ignore the UL GTP-TEID IE and/or UL Transport Layer Address IE in the E-RABs Subject to Forwarding Items IE, when the IEs are included in the DOWNLINK S1 CDMA2000 TUNNELLING message.

## 8.9	UE Capability Info Indication

### 8.9.1	General

The purpose of the UE Capability Info Indication procedure is to enable the eNB to provide to the MME UE capability-related information.

### 8.9.2	Successful Operation

Figure 8.9.2-1: UE Capability Info Indication procedure. Successful operation.

The eNB controlling a UE-associated logical S1-connection initiates the procedure by sending a UE CAPABILITY INFO INDICATION message to the MME including the UE capability information. The UE CAPABILITY INFO INDICATION message may also include paging specific UE capability information within the UE Radio Capability for Paging IE. The UE capability information received by the MME shall replace any previously stored UE capability information in the MME for the UE.

## 8.10	Trace Procedures

### 8.10.1	Trace Start

#### 8.10.1.1	General

The purpose of the Trace Start procedure is to allow the MME to request the eNB to initiate a trace function for a UE. The procedure uses UE-associated signalling. If no UE-associated logical S1-connection exists, the UE-associated logical S1-connection shall be established as part of the procedure.

#### 8.10.1.2	Successful Operation

Figure 8.10.1.2-1: Trace Start procedure.

The MME initiates the procedure by sending a TRACE START message. On receipt of a TRACE START message, the eNB shall initiate the requested trace function as described in 3GPP TS 32.422 [10].

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT and Trace”, the eNB shall if supported, initiate the requested trace session and MDT session as described in 3GPP TS 32.422 [10].

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT Only”, “Logged MDT only” or “Logged MBSFN MDT”, the target eNB shall, if supported, initiate the requested MDT session as described in 3GPP TS 32.422 [10] and the target eNB shall ignore Interfaces To Trace IE, and Trace Depth IE.

If the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, the eNB shall, if supported, store this information and take it into account in the requested MDT session.

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT Only”, “Logged MDT only” or “Logged MBSFN MDT” and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the eNB may use it to propagate the MDT Configuration as described in 3GPP TS 37.320 [31].

If the Trace Activation IE includes the MBSFN-ResultToLog IE, within the MDT Configuration IE, the eNB shall, if supported, take it into account for MDT Configuration as described in 3GPP TS 37.320 [31].

If the Trace Activation IE includes the MBSFN-AreaId IE in the MBSFN-ResultToLog IE, within the MDT Configuration IE, the eNB shall, if supported, take it into account for MDT Configuration as described in 3GPP TS 37.320 [31].

Interactions with other procedures:

If the eNB is not able to initiate the trace session due to ongoing handover of the UE to another eNB, the eNB shall initiate a Trace Failure Indication procedure with the appropriate cause value.

### 8.10.2	Trace Failure Indication

#### 8.10.2.1	General

The purpose of the Trace Failure Indication procedure is to allow the eNB to inform the MME that a Trace Start procedure or a Deactivate Trace procedure has failed due to an interaction with a handover procedure. The procedure uses UE-associated signalling.

#### 8.10.2.2	Successful Operation

Figure 8.10.2.2-1: Trace Failure Indication procedure.

The eNB initiates the procedure by sending a TRACE FAILURE INDICATION message. Upon reception of the TRACE FAILURE INDICATION message, the MME shall take appropriate actions based on the failure reason indicated by the Cause IE.

### 8.10.3	Deactivate Trace

#### 8.10.3.1	General

The purpose of the Deactivate Trace procedure is to allow the MME to request the eNB to stop the trace session, for the indicated trace reference.

#### 8.10.3.2	Successful Operation

Figure 8.10.3.2-1: Deactivate Trace procedure. Successful operation.

The MME invokes the Deactivate Trace procedure by sending a DEACTIVATE TRACE message to the eNB as described in 3GPP TS 32.422 [10].

Upon reception of this message, the eNB shall stop the trace session for the indicated trace reference in the E-UTRAN Trace ID IE.

Interactions with other procedures:

If the eNB is not able to stop the trace session due to ongoing handover of the UE to another eNB, the eNB shall initiate a Trace Failure Indication procedure with the appropriate cause value.

### 8.10.4	Cell Traffic Trace

#### 8.10.4.1	General

The purpose of the Cell Traffic Trace procedure is to send the allocated Trace Recording Session Reference and the Trace Reference to MME. The procedure uses UE-associated signalling.

#### 8.10.4.2	Successful Operation

Figure 8.10.4.2-1: Cell Traffic Trace procedure. Successful operation.

The procedure is initiated with a CELL TRAFFIC TRACE message sent from the eNB to the MME.

If the Privacy Indicator IE is included in the message, the MME shall take the information into account for anonymisation of MDT data (see 3GPP TS 32.422 [10]).

## 8.11	Location Reporting Procedures

### 8.11.1	Location Reporting Control

#### 8.11.1.1	General

The purpose of Location Reporting Control procedure is to allow the MME to request the eNB to report where the UE is currently located. The procedure uses UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Location Reporting Control procedure. Successful operation.

The MME initiates the procedure by sending a LOCATION REPORTING CONTROL message. On receipt of a LOCATION REPORTING CONTROL message the eNB shall perform the requested location reporting control action for the UE.

The Request Type IE indicates to the eNB whether:

-	to report directly;

-	to report upon change of serving cell, or

-	to stop reporting at change of serving cell.

If reporting upon change of serving cell is requested, the eNB shall report whenever the UE changes its serving cell to another cell belonging to the eNB.

The Request Type IE also indicates what type of location information the eNB shall report. The location information is E-UTRAN CGI and TAI.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

### 8.11.2	Location Report Failure Indication

#### 8.11.2.1	General

The Location Report Failure Indication procedure is initiated by an eNB in order to inform the MME that a Location Reporting Control procedure has failed due to an interaction with a handover procedure. The procedure uses UE-associated signalling.

#### 8.11.2.2	Successful Operation

Figure 8.11.2.2-1: Location Report Failure Indication procedure.

Upon reception of the LOCATION REPORT FAILURE INDICATION message the MME shall take appropriate actions based on the failure reason indicated by the Cause IE.

### 8.11.3	Location Report

#### 8.11.3.1	General

The purpose of Location Report procedure is to provide the UE’s current location to the MME. The procedure uses UE-associated signalling.

#### 8.11.3.2	Successful Operation

Figure 8.11.3.2-1: Location Report procedure. Successful operation.

The eNB initiates the procedure by generating a LOCATION REPORT message. The LOCATION REPORT message may be used as a response to a LOCATION REPORTING CONTROL message.

In case reporting at change of serving cell has been requested, the eNB shall send a LOCATION REPORT message whenever the information given to the EPC in any S1AP message is not anymore valid.

#### 8.11.3.3	Abnormal Conditions

Not applicable.

## 8.12	Warning Message Transmission Procedures

### 8.12.1	Write-Replace Warning

#### 8.12.1.1	General

The purpose of Write-Replace Warning procedure is to start or overwrite the broadcasting of warning messages.

The procedure uses non UE-associated signalling.

#### 8.12.1.2	Successful Operation

Figure 8.12.1.2-1: Write-Replace Warning procedure. Successful operation.

The MME initiates the procedure by sending a WRITE-REPLACE WARNING REQUEST message to the eNB.

Upon receipt of the WRITE-REPLACE WARNING REQUEST, eNB shall prioritise its resources to process the warning message.

If, in a certain area, broadcast of a warning message is already ongoing and the eNB receives a WRITE-REPLACE WARNING REQUEST message with Message Identifier IE and/or Serial Number IE which are different from those in the warning message being broadcast, and if the Concurrent Warning Message Indicator IE is not present, the eNB shall replace the warning message being broadcast with the newly received one for that area.

If the eNB receives a WRITE-REPLACE WARNING REQUEST message with a warning message identified by the Message Identifier IE and Serial Number IE and if there are no prior warning messages being broadcast in any of warning areas indicated in the Warning Area List IE, the eNB shall broadcast the received warning message for those area(s).

If, in a certain area, broadcast of one or more warning messages are already ongoing and the eNB receives a WRITE-REPLACE WARNING REQUEST message with a Message Identifier IE and/or Serial Number IE which are different from those in any of the warning messages being broadcast, and if the Concurrent Warning Message Indictor IE is present, the eNB shall schedule the received warning message for broadcast, for that area.

If the Concurrent Warning Message Indicator IE is present and if a value “0” is received in the Number of Broadcast Requested IE, the eNB shall broadcast the received warning message indefinitely until requested otherwise to stop broadcasting, except if the Repetition Period IE is set to “0”.

If, in a certain area, broadcast of one or more warning messages are already ongoing and the eNB receives a WRITE-REPLACE WARNING REQUEST message with Message Identifier IE and Serial Number IE which correspond to one of the warning messages already being broadcast in that area, the eNB shall not start a new broadcast or replace an existing one but it shall still reply by sending a WRITE-REPLACE WARNING RESPONSE message which includes the Broadcast Completed Area List IE set according to the ongoing broadcast.

If Warning Area List IE is not included in the WRITE-REPLACE WARNING REQUEST message, the eNB shall broadcast the indicated message in all of the cells within the eNB.

If Warning Type IE is included in WRITE-REPLACE WARNING REQUEST message, the eNB shall broadcast the Primary Notification irrespective of the setting of the Repetition Period IE and the Number of Broadcasts Requested IE, and process the Primary Notification according to MFA TS 36.331 [16].

If the Warning Security Information IE is included in the WRITE-REPLACE WARNING REQUEST message, the eNB shall send this IE together with the Warning Type IE in the Primary Notification.

If the Data Coding Scheme IE and the Warning Message Contents IE are both included in the WRITE-REPLACE WARNING REQUEST message, the eNB shall schedule a broadcast of the warning message according to the value of the Repetition Period IE and Number of Broadcasts Requested IE and process the warning message according to MFA TS 36.331 [16].

The eNB acknowledges the WRITE-REPLACE WARNING REQUEST message by sending a WRITE-REPLACE WARNING RESPONSE message to the MME.

If the Broadcast Completed Area List IE is not included in the WRITE-REPLACE WARNING RESPONSE message, the MME shall consider that the broadcast is unsuccessful in all the cells within the eNB.

If the Extended Repetition Period IE is included in the WRITE-REPLACE WARNING REQUEST message, the eNB shall ignore the value in the Repetition Period IE.

#### 8.12.1.3	Abnormal Conditions

If the Concurrent Warning Message Indicator IE is not present and if a value “0” is received in the Number of Broadcast Requested IE, the eNB shall not broadcast the received secondary notification.

If Concurrent Warning Message Indicator IE is included and if a value “0” is received in the Repetition Period IE, the eNB shall not broadcast the received warning message except if the Number of Broadcast Requested IE is set to “1”.

If Concurrent Warning Message Indicator IE is not included and if a value “0” is received in the Repetition Period IE, the eNB shall not broadcast the received secondary notification except if the Number of Broadcast Requested IE is set to “1”.

### 8.12.2	Kill

#### 8.12.2.1	General

The purpose of Kill procedure is to cancel an already ongoing broadcast of a warning message.

The procedure uses non UE-associated signalling.

#### 8.12.2.2	Successful Operation

Figure 8.12.2.2-1: Kill procedure. Successful operation.

The MME initiates the procedure by sending a KILL REQUEST message to the eNB.

If the eNB receives a KILL REQUEST message and broadcast of the warning message identified by the Message Identifier and Serial Number IE is ongoing in an area indicated within the Warning Area List IE, the eNB shall stop broadcasting the warning message within that area and discard the warning message for that area.

If the Warning Area List IE is not included in the KILL REQUEST message, the eNB shall stop broadcasting and discard the warning message identified by the Message Identifier IE and the Serial Number IE in all of the cells in the eNB.

The eNB shall acknowledge the KILL REQUEST message by sending the KILL RESPONSE message, with the Message Identifier IE and the Serial Number IE copied from the KILL REQUEST message and shall, if there is an area to report where an ongoing broadcast was stopped successfully, include the Broadcast Cancelled Area List IE.

If an area included in the Warning Area List IE in the KILL REQUEST message does not appear in the Broadcast Cancelled Area List IE, the MME shall consider that the eNB had no ongoing broadcast to stop for the same Message Identifier and Serial Number in that area.

If the Broadcast Cancelled Area List IE is not included in the KILL RESPONSE message, the MME shall consider that the eNB had no ongoing broadcast to stop for the same Message Identifier and Serial Number.

If the Kill-all Warning Messages Indicator IE is present in the KILL REQUEST message, then the eNB shall stop broadcasting and discard all warning messages for the area as indicated in the Warning Area List IE or in all the cells of the eNB if the Warning Area List IE is not included. The eNB shall acknowledge the KILL REQUEST message by sending the KILL RESPONSE message, with the Message Identifier IE and the Serial Number IE copied from the KILL REQUEST message and shall, if there is area to report where an ongoing broadcast was stopped successfully, include the Broadcast Cancelled Area List IE with the Number of Broadcasts IE set to 0.

### 8.12.3	PWS Restart Indication

#### 8.12.3.1	General

The purpose of PWS Restart Indication procedure is to inform the MME that PWS information for some or all cells of the eNB are available for reloading from the CBC if needed. The procedure uses non UE-associated signalling.

#### 8.12.3.2	Successful Operation

Figure 8.12.3.2-1: PWS Restart Indication procedure. Successful operation.

The eNB initiates the procedure by sending a PWS RESTART INDICATION message to the MME. On receipt of a PWS RESTART INDICATION message, the MME shall act as defined in 3GPP TS 23.007 [38].

If the Emergency Area ID is available, the eNB shall also include it in the Emergency Area ID List for Restart IE.

### 8.12.4	PWS Failure Indication

#### 8.12.4.1	General

The purpose of PWS Failure Indication procedure is to inform the MME that ongoing PWS operation for one or more cells of the eNB has failed. The procedure uses non UE-associated signalling.

#### 8.12.4.2	Successful Operation

Figure 8.12.4.2-1: PWS Failure Indication procedure. Successful operation.

The eNB initiates the procedure by sending a PWS FAILURE INDICATION message to the MME. On receipt of a PWS FAILURE INDICATION message, the MME shall act as defined in 3GPP TS 23.041 [29].

## 8.13	eNB Direct Information Transfer

### 8.13.1	General

The purpose of the eNB Direct Information Transfer procedure is to transfer RAN information from the eNB to the MME in unacknowledged mode. The MME does not interpret the transferred RAN information.

This procedure uses non-UE associated signalling.

### 8.13.2	Successful Operation

#### 8.13.2.1	eNB Direct Information Transfer

Figure 8.13.1.2-1: ENB Direct Information Transfer procedure. Successful operation.

The procedure is initiated with an ENB DIRECT INFORMATION TRANSFER message sent from the eNB to the MME.

The RIM Transfer IE within the Inter-system Information Transfer Type IE shall contain the RIM Routing Address IE that identifies the final RAN destination node where the RIM information needs to be transferred to by the core network. In case of transfer to UTRAN the source eNB shall include the RAC IE in the Target RNC-ID IE within the RIM Routing Address IE.

### 8.13.3	Abnormal Conditions

Not applicable.

## 8.14	MME Direct Information Transfer

### 8.14.1	General

The purpose of the MME Direct Information Transfer procedure is to transfer RAN information from the MME to the eNB in unacknowledged mode.

This procedure uses non-UE associated signalling.

### 8.14.2	Successful Operation

#### 8.14.2.1	MME Direct Information Transfer

Figure 8.14.1.2-1: MME Direct Information Transfer procedure. Successful operation.

The procedure is initiated with a MME DIRECT INFORMATION TRANSFER message sent from the MME to the eNB.

The Inter-system Information Transfer Type IE indicates the nature of the transferred information. When the transferred information is of RIM nature, the RIM Information IE within the RIM Transfer IE shall contain a BSSGP RIM PDU. The RIM Routing Address IE shall not be present since the eNB is the final destination node.

### 8.14.3	Abnormal Conditions

Not applicable.

## 8.15	eNB Configuration Transfer

### 8.15.1	General

The purpose of the eNB Configuration Transfer procedure is to transfer RAN configuration information from the eNB to the MME in unacknowledged mode. The MME does not interpret the transferred RAN configuration information.

This procedure uses non-UE associated signalling.

### 8.15.2	Successful Operation

#### 8.15.2.1	eNB Configuration Transfer

Figure 8.15.2.1-1: eNB Configuration Transfer procedure. Successful operation.

The procedure is initiated with an ENB CONFIGURATION TRANSFER message sent from the eNB to the MME.

If the MME receives the SON Configuration Transfer IE, it shall transparently transfer the SON Configuration Transfer IE towards the eNB indicated in the Target eNB-ID IE which is included in the SON Configuration Transfer IE.

### 8.15.3	Abnormal Conditions

Not applicable.

## 8.16	MME Configuration Transfer

### 8.16.1	General

The purpose of the MME Configuration Transfer procedure is to transfer RAN configuration information from the MME to the eNB in unacknowledged mode.

This procedure uses non-UE associated signalling.

### 8.16.2	Successful Operation

#### 8.16.2.1	MME Configuration Transfer

Figure 8.16.2.1-1: MME Configuration Transfer procedure. Successful operation.

The procedure is initiated with an MME CONFIGURATION TRANSFER message sent from the MME to the eNB.

If the eNB receives, in the SON Configuration Transfer IE, the SON Information IE containing the SON Information Request IE, it may transfer back the requested information towards the eNB indicated in the Source eNB-ID IE of the SON Configuration Transfer IE by initiating the eNB Configuration Transfer procedure. If the X2 TNL Configuration Info IE contains the eNB Indirect X2 Transport Layer Addresses IE, the eNB may use it for the X2 TNL establishment, and may transfer back the received eNB Indirect X2 Transport Layer Addresses towards the eNB indicated in the Source eNB-ID IE of the SON Configuration Transfer IE by initiating the eNB Configuration Transfer procedure.

If the eNB receives, in the SON Configuration Transfer IE, the X2 TNL Configuration Info IE containing the eNB X2 Extended Transport Layer Addresses IE, it may use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the eNB receives, in the SON Configuration Transfer IE, the SON Information IE containing the SON Information Reply IE including the X2 TNL Configuration Info IE as an answer to a former request, it may use it to initiate the X2 TNL establishment. If the X2 TNL Configuration Info IE contains the eNB Indirect X2 Transport Layer Addresses IE, the eNB may use it for the X2 TNL establishment.

In case the IP-Sec Transport Layer Address IE is present and the GTP Transport Layer Addresses IE within the eNB X2 Extended Transport Layer Addresses IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel end point given in by the IP-Sec Transport Layer Address IE.

In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the end points given by the list of addresses in eNB GTP Transport Layer Addresses IE within the eNB X2 Extended Transport Layer Addresses IE.

In case the eNB GTP Transport Layer Addresses IE is empty and the IP-Sec Transport Layer Address IE is present, SCTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel end point given in the IP-Sec Transport Layer Address IE, within the eNB X2 Extended Transport Layer Addresses IE.

If the eNB is configured to use one IPsec tunnel for all S1 and X2 traffic (IPsec star topology) then the traffic to the peer eNB shall be routed through this IPsec tunnel and the IP-Sec Transport Layer Address IE shall be ignored.

If the eNB receives the SON Information IE containing the SON Information Reply IE including the Time Synchronisation Info IE as an answer to a former request, it may use it for over-the-air synchronisation by means of network listening and for triggering muting activation request.

If the eNB receives the SON Information IE containing the SON Information Report IE it may use it as specified in MFA TS 36.300 [14].

If the eNB receives the SON Information IE containing the SON Information Request IE set to “Activate Muting”, the eNB should consider activating for over-the-air synchronisation by means of network listening, taking into account information on the selected source of synchronisation cell and the cells as indicated by the Aggressor E-CGI List IE. In case the Aggressor E-CGI List IE is not present, the eNB may consider the request applicable to all cells.

If the eNB receives the SON Information IE containing the SON Information Reply IE including the Muting Pattern Information IE as an answer to a former request, it may use it for over-the-air synchronisation by means of network listening. The Muting Pattern Information IE may apply to all cells that were requested to mute.

If the eNB receives the SON Information IE containing the SON Information Request IE set to “Deactivate Muting”, the eNB may consider deactivating muting for over-the-air synchronisation that was activated by a former muting request from the corresponding eNB.

### 8.16.3	Abnormal Conditions

Not applicable.

## 8.17	LPPa transport

### 8.17.1	General

The purpose of the LPPa Transport procedure is to carry LPPa signalling (defined in 3GPP TS 36.455 [34]) between eNB and E-SMLC over the S1 Interface as defined in 3GPP TS 36.455 [34]. The procedure may use UE-associated signalling or non-UE associated signalling. The UE-associated signalling is used to support E-CID and UTDOA positioning of a specific UE. The non-UE associated signalling is used to obtain assistance data from an eNB to support OTDOA positioning for any UE.

### 8.17.2	Successful Operations

#### 8.17.2.1	DOWNLINK UE ASSOCIATED LPPA TRANSPORT

Figure 8.17.2.1-1: DOWNLINK UE ASSOCIATED LPPA Transport Procedure

The MME initiates the procedure by sending the DOWNLINK UE ASSOCIATED LPPA TRANSPORT message to eNB.

#### 8.17.2.2	UPLINK UE ASSOCIATED LPPA TRANSPORT

Figure 8.17.2.2-1: UPLINK UE ASSOCIATED LPPA TRANSPORT Procedure

The eNB initiates the procedure by sending the UPLINK UE ASSOCIATED LPPA TRANSPORT message to MME.

#### 8.17.2.3	DOWNLINK NON UE ASSOCIATED LPPA TRANSPORT

Figure 8.17.2.3-1: DOWNLINK NON UE ASSOCIATED LPPA Transport Procedure

The MME initiates the procedure by sending the DOWNLINK NON UE ASSOCIATED LPPA TRANSPORT message to eNB.

#### 8.17.2.4	UPLINK NON UE ASSOCIATED LPPA TRANSPORT

Figure 8.17.2.4-1: UPLINK NON UE ASSOCIATED LPPA TRANSPORT Procedure

The eNB initiates the procedure by sending the UPLINK NON UE ASSOCIATED LPPA TRANSPORT message to MME.

### 8.17.3	Unsuccessful Operation

Not applicable

### 8.17.4	Abnormal Conditions

If an MME receives an UPLINK UE ASSOCIATED LPPA TRANSPORT message with an unknown Routing ID for the UE, the MME shall ignore the message.

If an MME receives an UPLINK NON UE ASSOCIATED LPPA TRANSPORT message indicating an unknown or unreachable Routing ID, the MME shall ignore the message.

# 9	Elements for S1AP Communication

## 9.1	Message Functional Definition and Content

### 9.1.1	General

### 9.1.2	Message Contents

#### 9.1.2.1	Presence

All information elements in the message descriptions below are marked mandatory, optional or conditional according to table 4.

Table 4: Meaning of abbreviations used in S1AP messages

| Abbreviation | Meaning |
| --- | --- |
| M | IEs marked as Mandatory (M) shall always be included in the message. |
| O | IEs marked as Optional (O) may or may not be included in the message. |
| C | IEs marked as Conditional (C) shall be included in a message only if the condition is satisfied. Otherwise the IE shall not be included. |

#### 9.1.2.2	Criticality

Each Information Element or Group of Information Elements may have criticality information applied to it.
Following cases are possible:

Table 5: Meaning of content within “Criticality” column

| Abbreviation | Meaning |
| --- | --- |
| – | No criticality information is applied explicitly. |
| YES | Criticality information is applied. This is usable only for non-repeatable IEs |
| GLOBAL | The IE and all its repetitions together have one common criticality information. This is usable only for repeatable IEs. |
| EACH | Each repetition of the IE has its own criticality information. It is not allowed to assign different criticality values to the repetitions. This is usable only for repeatable IEs. |

#### 9.1.2.3	Range

The Range column indicates the allowed number of copies of repetitive IEs/IE groups.

#### 9.1.2.4	Assigned Criticality

This column provides the actual criticality information as defined in subclause 10.3.2, if applicable.

### 9.1.3	E-RAB Management Messages

#### 9.1.3.1	E-RAB SETUP REQUEST

This message is sent by the MME and is used to request the eNB to assign resources on Uu and S1 for one or several E-RABs.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | reject |
| E-RAB to be Setup List |  | 1 |  |  | YES | reject |
| >E-RAB To Be Setup Item IEs |  | 1 .. <maxnoof E-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | EPC TEID. | - |  |
| >>NAS-PDU | M |  | 9.2.3.5 |  | - |  |
| >>Correlation ID | O |  | 9.2.1.80 |  | YES | ignore |
| >>SIPTO Correlation ID | O |  | Correlation ID 9.2.1.80 |  | YES | ignore |
| >>Bearer Type | O |  | 9.2.1.116 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.3.2	E-RAB SETUP RESPONSE

This message is sent by the eNB and is used to report the outcome of the request from the E-RAB SETUP REQUEST message.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| E-RAB Setup List |  | 0..1 |  |  | YES | ignore |
| >E-RAB Setup Item IEs |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  |  |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | eNB TEID. | - |  |
| E-RAB Failed to Setup List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB Setup List IE and in E-RAB Failed to Setup List IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.3.3	E-RAB MODIFY REQUEST

This message is sent by the MME and is used to request the eNB to modify the Data Radio Bearers and the allocated resources on Uu and S1 for one or several E-RABs or to change the S-GW as defined in 3GPP TS 23.401 [11].

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | reject |
| E-RAB to be Modified List |  | 1 |  |  | YES | reject |
| >E-RAB To Be Modified Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>NAS-PDU | M |  | 9.2.3.5 |  | - |  |
| >>Transport Information | O |  |  |  | YES | reject |
| >>>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>>UL GTP TEID | M |  | GTP-TEID 9.2.2.2 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.3.4	E-RAB MODIFY RESPONSE

This message is sent by the eNB and is used to report the outcome of the request from the E-RAB MODIFY REQUEST message.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| E-RAB Modify List |  | 0..1 |  |  | YES | ignore |
| >E-RAB Modify Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| E-RAB Failed to Modify List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB Modify List IE and E-RAB Failed to Modify List IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.3.5	E-RAB RELEASE COMMAND

This message is sent by the MME and is used to request the eNB to release allocated resources on Uu and S1 for one or several E-RABs.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | reject |
| E-RAB To Be Released List | M |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB To Be Released List IE. | YES | ignore |
| NAS-PDU | O |  | 9.2.3.5 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.3.6	E-RAB RELEASE RESPONSE

This message is sent by the eNB and is used to report the outcome of the request from the E-RAB RELEASE COMMAND message.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| E-RAB Release List |  | 0..1 |  |  | YES | ignore |
| >E-RAB Release Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| E-RAB Failed to Release List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB Release List IE and E-RAB Failed to Release List IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| User Location Information | O |  | 9.2.1.93 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.3.7	E-RAB RELEASE INDICATION

This message is sent by the eNB and is used to indicate the MME to release one or several E-RABs for one UE.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-RAB Released List | M |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB Released List IE. | YES | ignore |
| User Location Information | O |  | 9.2.1.93 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.3.8	E-RAB MODIFICATION INDICATION

This message is sent by the eNB and is used to request the MME to apply the indicated modification for one or several E-RABs.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-RAB to be Modified List |  | 1 |  |  | YES | reject |
| >E-RAB to Be Modified Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>DL GTP TEID | M |  | GTP-TEID 9.2.2.2 |  | - |  |
| E-RAB not to be Modified List |  | 0..1 |  |  | YES | reject |
| >E-RAB not to Be Modified Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>DL GTP TEID | M |  | GTP-TEID 9.2.2.2 |  | - |  |
| CSG Membership Info |  | 0..1 |  |  | YES | reject |
| >CSG Membership Status | M |  | 9.2.1.73 |  | - |  |
| >CSG Id | M |  | 9.2.1.62 |  | - |  |
| >Cell Access Mode | O |  | 9.2.1.74 |  | - |  |
| >PLMN Identity | O |  | 9.2.3.8 |  | - |  |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.3 | Indicating HeNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.3.9	E-RAB MODIFICATION CONFIRM

This message is sent by the MME and is used to report the outcome of the request from the E-RAB MODIFICATION INDICATION message.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| E-RAB Modify List |  | 0..1 |  |  | YES | ignore |
| >E-RAB Modify Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| E-RAB Failed to Modify List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in the E-RAB MODIFICATION CONFIRM message. | YES | ignore |
| E-RAB To Be Released List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in the E-RAB MODIFICATION CONFIRM message. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

### 9.1.4	Context Management Messages

#### 9.1.4.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the MME to request the setup of a UE context.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.1.20 |  | YES | reject |
| E-RAB to Be Setup List |  | 1 |  |  | YES | reject |
| >E-RAB to Be Setup Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 |  | - |  |
| >>NAS-PDU | O |  | 9.2.3.5 |  | - |  |
| >>Correlation ID | O |  | 9.2.1.80 |  | YES | ignore |
| >>SIPTO Correlation ID | O |  | Correlation ID 9.2.1.80 |  | YES | ignore |
| >>Bearer Type | O |  | 9.2.1.116 |  | YES | reject |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | reject |
| Security Key | M |  | 9.2.1.41 | The KeNB is provided after the key-generation in the MME, see MFA TS 33.401 [15]. | YES | reject |
| Trace Activation | O |  | 9.2.1.4 |  | YES | ignore |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| CS Fallback Indicator | O |  | 9.2.3.21 |  | YES | reject |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Registered LAI | O |  | 9.2.3.1 |  | YES | ignore |
| GUMMEI | O |  | 9.2.3.9 | This IE indicates the MME serving the UE. | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| Management Based MDT Allowed | O |  | 9.2.1.83 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |
| Additional CS Fallback Indicator | C-ifCSFBhighpriority |  | 9.2.3.37 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.1.96 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCSFBhighpriority | This IE shall be present if the CS Fallback Indicator IE is set to “CS Fallback High Priority”. |

#### 9.1.4.2	Void

#### 9.1.4.3	INITIAL CONTEXT SETUP RESPONSE

This message is sent by the eNB to confirm the setup of a UE context.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| E-RAB Setup List |  | 1 |  |  | YES | ignore |
| >E-RAB Setup Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 |  | - |  |
| E-RAB Failed to Setup List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB Setup List IE and E-RAB Failed to Setup List IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.4.4	INITIAL CONTEXT SETUP FAILURE

This message is sent by the eNB to indicate that the setup of the UE context was unsuccessful.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.4.5	UE CONTEXT RELEASE REQUEST

This message is sent by the eNB to request the release of the UE-associated S1-logical connection over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| GW Context Release Indication | O |  | 9.2.1.84 |  | YES | reject |

#### 9.1.4.6	UE CONTEXT RELEASE COMMAND

This message is sent by the MME to request the release of the UE-associated S1-logical connection over the S1 interface.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| CHOICE UE S1AP IDs | M |  |  |  | YES | reject |
| >UE S1AP ID pair |  |  |  |  |  |  |
| >>UE S1AP ID pair | M |  | 9.2.3.18 |  |  |  |
| >MME UE S1AP ID |  |  |  |  |  |  |
| >>MME UE S1AP ID | M |  | 9.2.3.3 |  |  |  |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |

#### 9.1.4.7	UE CONTEXT RELEASE COMPLETE

This message is sent by the eNB to confirm the release of the UE-associated S1-logical connection over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| User Location Information | O |  | 9.2.1.93 |  | YES | ignore |
| Information on Recommended Cells and eNBs for Paging | O |  | 9.2.1.105 |  | YES | ignore |
| Cell Identifier and Coverage Enhancement Level | O |  | 9.2.1.109 |  | YES | ignore |

#### 9.1.4.8	UE CONTEXT MODIFICATION REQUEST

This message is sent by the MME to provide UE Context information changes to the eNB.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Security Key | O |  | 9.2.1.41 | A fresh KeNB is provided after performing a key-change on the fly procedure in the MME, see MFA TS 33.401 [15]. | YES | reject |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | ignore |
| CS Fallback Indicator | O |  | 9.2.3.21 |  | YES | reject |
| UE Security Capabilities | O |  | 9.2.1.40 |  | YES | reject |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Registered LAI | O |  | 9.2.3.1 |  | YES | ignore |
| Additional CS Fallback Indicator | C-ifCSFBhighpriority |  | 9.2.3.37 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| SRVCC Operation Not Possible | O |  | 9.2.1.119 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCSFBhighpriority | This IE shall be present if the CS Fallback Indicator IE is set to “CS Fallback High Priority”. |

#### 9.1.4.9	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the eNB to confirm the performed UE context updates.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.4.10	UE CONTEXT MODIFICATION FAILURE

This message is sent by the eNB in case the performed UE context update is not successful.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.4.11	UE RADIO CAPABILITY MATCH REQUEST

This message is sent by the MME to request the compatibility between the UE radio capabilities and network configuration.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |

#### 9.1.4.12	UE RADIO CAPABILITY MATCH RESPONSE

This message is sent by the eNB to report the compatibility between the UE radio capabilities and network configuration.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Voice Support Match Indicator | M |  | 9.2.1.85 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.4.13	UE CONTEXT MODIFICATION INDICATION

This message is sent by the eNB to request the MME to modify the UE Context information.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| CSG Membership Info |  | 0..1 |  |  | YES | reject |
| >CSG Membership Status | M |  | 9.2.1.73 |  | - |  |
| >CSG Id | M |  | 9.2.1.62 |  | - |  |
| >Cell Access Mode | O |  | 9.2.1.74 |  | - |  |
| >PLMN Identity | O |  | 9.2.3.8 |  | - |  |

#### 9.1.4.14	UE CONTEXT MODIFICATION CONFIRM

This message is sent by the MME to confirm the modification of the UE Context information.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.4.15	UE CONTEXT SUSPEND REQUEST

This message is sent by the eNB to request the MME to suspend the UE context and the related bearer contexts.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Information on Recommended Cells and eNBs for Paging | O |  | 9.2.1.105 |  | YES | ignore |
| Cell Identifier and Coverage Enhancement Level | O |  | 9.2.1.109 |  | YES | ignore |

#### 9.1.4.16	UE CONTEXT SUSPEND RESPONSE

This message is sent by the MME to indicate to the eNB the UE context and the related bearer contexts have been suspended.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.4.17	UE CONTEXT RESUME REQUEST

This message is sent by the eNB to request the MME to indicate that the suspended RRC connection has been resumed.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-RAB Failed To Resume List |  | 0..1 |  |  | YES | reject |
| >E-RAB Failed To Resume Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Cause | M |  | 9.2.1.3 |  | - |  |
| RRC Resume Cause | O |  | RRC Establishment Cause 9.2.1.3a |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.4.18	UE CONTEXT RESUME RESPONSE

This message is sent by the MME to indicate to the eNB that the UE context and the related bearer contexts have been resumed in the EPC.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| E-RAB Failed To Resume List |  | 0..1 |  |  | YES | reject |
| >E-RAB Failed To Resume Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Cause | M |  | 9.2.1.3 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.1.4.19	UE CONTEXT RESUME FAILURE

This message is sent by the MME to indicate to the eNB that resumption of the UE context and the related bearer contexts has failed in the EPC.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.4.20	Connection Establishment Indication

This message is sent by the MME to complete the establishment of the UE-associated logical S1-connection.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |

### 9.1.5	Handover Signalling Messages

#### 9.1.5.1	HANDOVER REQUIRED

This message is sent by the source eNB to the MME to request the preparation of resources at the target.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Handover Type | M |  | 9.2.1.13 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Target ID | M |  | 9.2.1.6 |  | YES | reject |
| Direct Forwarding Path Availability | O |  | 9.2.3.15 |  | YES | ignore |
| SRVCC HO Indication | O |  | 9.2.1.59 |  | YES | reject |
| Source to Target Transparent Container | M |  | 9.2.1.56 |  | YES | reject |
| Source to Target Transparent Container Secondary | O |  | Source to Target Transparent Container 9.2.1.56 |  | YES | reject |
| MS Classmark 2 | C- ifSRVCCtoGERAN |  | 9.2.1.64 |  | YES | reject |
| MS Classmark 3 | C- ifSRVCCtoGERAN |  | 9.2.1.65 |  | YES | ignore |
| CSG Id | O |  | 9.2.1.62 |  | YES | reject |
| Cell Access Mode | O |  | 9.2.1.74 |  | YES | reject |
| PS Service Not Available | O |  | 9.2.1.77 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSRVCCtoGERAN | This IE shall be present if the Handover Type IE is set to the “Value” LTEtoGERAN and the SRVCC HO Indication IE is present. |

#### 9.1.5.2	HANDOVER COMMAND

This message is sent by the MME to inform the source eNB that resources for the handover have been prepared at the target side.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Handover Type | M |  | 9.2.1.13 |  | YES | reject |
| NAS Security Parameters from E-UTRAN | C-iftoUTRANGERAN |  | 9.2.3.30 | The eNB shall use this IE as specified in MFA TS 33.401 [15]. | YES | reject |
| E-RABs Subject to Forwarding List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Subject to Forwarding Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>DL Transport Layer Address | O |  | 9.2.2.1 |  | - |  |
| >>DL GTP-TEID | O |  | 9.2.2.2 | To deliver forwarded DL PDCP SDUs. | - |  |
| >>UL Transport Layer Address | O |  | 9.2.2.1 |  | - |  |
| >>UL GTP-TEID | O |  | 9.2.2.2 | To deliver forwarded UL PDCP SDUs. | - |  |
| E-RABs to Release List | O |  | E-RAB List 9.2.1.36 |  | YES | ignore |
| Target to Source Transparent Container | M |  | 9.2.1.57 |  | YES | reject |
| Target to Source Transparent Container Secondary | O |  | Target to Source Transparent Container 9.2.1.57 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| iftoUTRANGERAN | This IE shall be present if the Handover Type IE is set to the value “LTEtoUTRAN “ or “LTEtoGERAN”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.1.5.3	HANDOVER PREPARATION FAILURE

This message is sent by the MME to inform the source eNB that the Handover Preparation has failed.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.5.4	HANDOVER REQUEST

This message is sent by the MME to the target eNB to request the preparation of resources.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| Handover Type | M |  | 9.2.1.13 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.1.20 |  | YES | reject |
| E-RABs To Be Setup List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Setup Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | To deliver UL PDUs. | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>Data Forwarding Not Possible | O |  | 9.2.1.76 |  | YES | ignore |
| >>Bearer Type | O |  | 9.2.1.116 |  | YES | reject |
| Source to Target Transparent Container | M |  | 9.2.1.56 |  | YES | reject |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | reject |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| Trace Activation | O |  | 9.2.1.4 |  | YES | ignore |
| Request Type | O |  | 9.2.1.34 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| Security Context | M |  | 9.2.1.26 |  | YES | reject |
| NAS Security Parameters to E-UTRAN | C-iffromUTRANGERAN |  | 9.2.3.31 | The eNB shall use this IE as specified in MFA TS 33.401 [15]. | YES | reject |
| CSG Id | O |  | 9.2.1.62 |  | YES | reject |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| GUMMEI | O |  | 9.2.3.9 | This IE indicates the MME serving the UE. | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| Management Based MDT Allowed | O |  | 9.2.1.83 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.1.96 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| C-iffromUTRANGERAN | This IE shall be present if the Handover Type IE is set to the value “UTRANtoLTE” or “GERANtoLTE”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.1.5.5	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target eNB to inform the MME about the prepared resources at the target.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 | Allocated at the target eNB. | YES | ignore |
| E-RABs Admitted List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | To deliver DL PDUs. | - |  |
| >>DL Transport Layer Address | O |  | 9.2.2.1 |  | - |  |
| >>DL GTP-TEID | O |  | 9.2.2.2 | To deliver forwarded DL PDCP SDUs. | - |  |
| >>UL Transport Layer Address | O |  | 9.2.2.1 |  | - |  |
| >>UL GTP-TEID | O |  | 9.2.2.2 | To deliver forwarded UL PDCP SDUs. | - |  |
| E-RABs Failed to Setup List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and E-RABs Failed to Setup List IE. | YES | ignore |
| Target to Source Transparent Container | M |  | 9.2.1.57 |  | YES | reject |
| CSG Id | O |  | 9.2.1.62 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| Cell Access Mode | O |  | 9.2.1.74 |  | YES | ignore |
| CE-mode-B Support Indicator | O |  | 9.2.1.118 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.1.5.6	HANDOVER FAILURE

This message is sent by the target eNB to inform the MME that the preparation of resources has failed.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.5.7	HANDOVER NOTIFY

This message is sent by the target eNB to inform the MME that the UE has been identified in the target cell and the S1 handover has been completed.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| TAI | M |  | 9.2.3.16 |  | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.3 | Indicating HeNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |

#### 9.1.5.8	PATH SWITCH REQUEST

This message is sent by the eNB to request the MME to switch DL GTP tunnel termination point(s) from one end-point to another.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-RAB To Be Switched in Downlink List |  | 1 |  |  | YES | reject |
| >E-RABs Switched in Downlink Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 | To deliver DL PDUs. | - |  |
| Source MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| TAI | M |  | 9.2.3.16 |  | YES | ignore |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | ignore |
| CSG Id | O |  | 9.2.1.62 |  | YES | ignore |
| Cell Access Mode | O |  | 9.2.1.74 |  | YES | ignore |
| Source MME GUMMEI | O |  | 9.2.3.9 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.3 | Indicating HeNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.1.5.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the MME to inform the eNB that the path switch has been successfully completed in the EPC.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | O |  | 9.2.1.20 |  | YES | ignore |
| E-RAB To Be Switched in Uplink List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Switched in Uplink Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 |  | - |  |
| E-RAB To Be Released List | O |  | E-RAB List 9.2.1.36 | A value for E-RAB ID shall only be present once in E-RAB To Be Switched in Uplink List IE and E-RAB to Be Released List IE. | YES | ignore |
| Security Context | M |  | 9.2.1.26 | One pair of {NCC, NH} is provided. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.1.5.10	PATH SWITCH REQUEST FAILURE

This message is sent by the MME to inform the eNB that a failure has occurred in the EPC during the Path switch request procedure.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.5.11	HANDOVER CANCEL

This message is sent by the source eNB to the MME to request the cancellation of an ongoing handover.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |

#### 9.1.5.12	HANDOVER CANCEL ACKNOWLEDGE

This message is sent by the MME to the source eNB to confirm that the ongoing handover was cancelled.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.5.13	eNB STATUS TRANSFER

This message is sent by the source eNB to transfer the PDCP SN receiver and transmitter status.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| eNB Status Transfer Transparent Container | M |  | 9.2.1.31 |  | YES | reject |

#### 9.1.5.14	MME STATUS TRANSFER

This message is sent by the MME to transfer the PDCP-SN receiver and transmitter status.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| eNB Status Transfer Transparent Container | M |  | 9.2.1.31 |  | YES | reject |

### 9.1.6	PAGING

This message is sent by the MME and is used to page a UE in one or several tracking areas.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.2.3.10 |  | YES | ignore |
| UE Paging Identity | M |  | 9.2.3.13 |  | YES | ignore |
| Paging DRX | O |  | 9.2.1.16 |  | YES | ignore |
| CN Domain | M |  | 9.2.3.22 |  | YES | ignore |
| List of TAIs |  | 1 |  |  | YES | ignore |
| >TAI List Item |  | 1 .. <maxnoofTAIs> |  |  | EACH | ignore |
| >>TAI | M |  | 9.2.3.16 |  | - |  |
| CSG Id List |  | 0..1 |  |  | GLOBAL | ignore |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Paging Priority | O |  | 9.2.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.2.1.103 |  | YES | ignore |
| Paging eDRX Information | O |  | 9.2.1.111 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.46 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.2.1.115 |  | YES | ignore |
| NB-IoT UE Identity Index value | O |  | 9.2.3.47 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIs | Maximum no. of TAIs. Value is 256. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |

### 9.1.7	NAS Transport Messages

#### 9.1.7.1	INITIAL UE MESSAGE

This message is sent by the eNB to transfer the initial layer 3 message to the MME over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | reject |
| TAI | M |  | 9.2.3.16 | Indicating the Tracking Area from which the UE has sent the NAS message. | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 | Indicating the E-UTRAN CGI from which the UE has sent the NAS message. | YES | ignore |
| RRC Establishment Cause | M |  | 9.2.1.3a |  | YES | ignore |
| S-TMSI | O |  | 9.2.3.6 |  | YES | reject |
| CSG Id | O |  | 9.2.1.62 |  | YES | reject |
| GUMMEI | O |  | 9.2.3.9 |  | YES | reject |
| Cell Access Mode | O |  | 9.2.1.74 |  | YES | reject |
| GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating GW Transport Layer Address if the GW is collocated with eNB. | YES | ignore |
| Relay Node Indicator | O |  | 9.2.1.79 | Indicating a relay node. | YES | reject |
| GUMMEI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.3 | Indicating HeNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating SIPTO L-GW Transport Layer Address if the SIPTO L-GW is collocated with eNB. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |
| MME Group ID | O |  | 9.2.3.44 |  | YES | ignore |
| UE Usage Type | O |  | INTEGER (0..255) |  | YES | ignore |
| CE-mode-B Support Indicator | O |  | 9.2.1.118 |  | YES | ignore |

#### 9.1.7.2	DOWNLINK NAS TRANSPORT

This message is sent by the MME and is used for carrying NAS information over the S1 interface.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | reject |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |

#### 9.1.7.3	UPLINK NAS TRANSPORT

This message is sent by the eNB and is used for carrying NAS information over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| TAI | M |  | 9.2.3.16 |  | YES | ignore |
| GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating GW Transport Layer Address if the GW is collocated with eNB. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating SIPTO L-GW Transport Layer Address if the SIPTO L-GW is collocated with eNB. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |

#### 9.1.7.4	NAS NON DELIVERY INDICATION

This message is sent by the eNB and is used for reporting the non delivery of a NAS PDU previously received within a DOWNLINK NAS TRANSPORT message over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |

#### 9.1.7.5	REROUTE NAS REQUEST

This message is sent by the MME in order to request for a rerouting of the INITIAL UE MESSAGE to a DCN.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| MME UE S1AP ID | O |  | 9.2.3.3 |  | YES | ignore |
| S1 Message | M |  | OCTET STRING | Contains the INITIAL UE MESSAGE | YES | reject |
| MME Group ID | M |  | 9.2.3.44 |  | YES | reject |
| Additional GUTI | O |  | 9.2.3.45 |  | YES | ignore |
| UE Usage Type | O |  | INTEGER (0..255) |  | YES | ignore |

### 9.1.8	Management messages

#### 9.1.8.1	RESET

This message is sent by both the MME and the eNB and is used to request that the S1 interface, or parts of the S1 interface, to be reset.

Direction: MME  eNB and eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >S1 interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all,…) |  | - |  |
| >Part of S1 interface |  |  |  |  |  |  |
| >>UE-associated logical S1-connection list |  | 1 |  |  | - |  |
| >>>UE-associated logical S1-connection Item |  | 1 .. <maxnoofIndividualS1ConnectionsToReset> |  |  | EACH | reject |
| >>>>MME UE S1AP ID | O |  | 9.2.3.3 |  | - |  |
| >>>>eNB UE S1AP ID | O |  | 9.2.3.4 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualS1ConnectionsToReset | Maximum no. of UE-associated logical S1-connections allowed to reset in one message. Value is 256. |

#### 9.1.8.2	RESET ACKNOWLEDGE

This message is sent by both the MME and the eNB as a response to a RESET message.

Direction: eNB  MME and MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| UE-associated logical S1-connection list |  | 0..1 |  |  | YES | ignore |
| >UE-associated logical S1-connection Item |  | 1 .. <maxnoofIndividualS1ConnectionsToReset> |  |  | EACH | ignore |
| >>MME UE S1AP ID | O |  | 9.2.3.3 |  | - |  |
| >>eNB UE S1AP ID | O |  | 9.2.3.4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualS1ConnectionsToReset | Maximum no. of UE-associated logical S1-connections allowed to reset in one message. Value is 256. |

#### 9.1.8.3	ERROR INDICATION

This message is sent by both the MME and the eNB and is used to indicate that some error has been detected in the node.

Direction: MME  eNB and eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | O |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | O |  | 9.2.3.4 |  | YES | ignore |
| Cause | O |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.8.4	S1 SETUP REQUEST

This message is sent by the eNB to transfer information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Global eNB ID | M |  | 9.2.1.37 |  | YES | reject |
| eNB Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the eNB. | GLOBAL | reject |
| >TAC | M |  | 9.2.3.7 | Broadcast TAC. | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| >RAT-Type | O |  | 9.2.1.117 | RAT-Type associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | 9.2.1.16 |  | YES | ignore |
| CSG Id List |  | 0..1 |  |  | GLOBAL | reject |
| >CSG Id |  | 1 .. <maxnoofCSGIds> | 9.2.1.62 |  |  |  |
| UE Retention Information | O |  | 9.2.1.112 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.2.1.114 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 6. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |

#### 9.1.8.5	S1 SETUP RESPONSE

This message is sent by the MME to transfer information for a TNL association.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Served GUMMEIs |  | 1..<maxnoofRATs> |  | The LTE related pool configuration is included on the first place in the list. | GLOBAL | reject |
| >Served PLMNs |  | 1..<maxnoofPLMNsPerMME> |  |  | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >Served GroupIDs |  | 1..<maxnoofGroupIDs> |  |  | - |  |
| >>MME Group ID | M |  | OCTET STRING (SIZE(2)) |  | - |  |
| >Served MMECs |  | 1..<maxnoofMMECs> |  |  | - |  |
| >>MME Code | M |  | 9.2.3.12 |  | - |  |
| Relative MME Capacity | M |  | 9.2.3.17 |  | YES | ignore |
| MME Relay Support Indicator | O |  | 9.2.1.82 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| UE Retention Information | O |  | 9.2.1.112 |  | YES | Ignore |
| PSP list |  | 0..<maxnoofPSPInfo> |  |  | GLOBAL | Ignore |
| >PSP Info | M |  | 9.2.3.mf1 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPLMNsPerMME | Maximum no. of PLMNs per MME. Value is 32. |
| maxnoofRATs | Maximum no. of RATs. Value is 8. |
| maxnoofGroupIDs | Maximum no. of GroupIDs per node per RAT. Value is 65535. |
| maxnoofMMECs | Maximum no. of MMECs per node per RAT. Value is 256. |
| maxnoofPSPInfo | Maximum no. of PSP Info per MME. Value is 1024. |

#### 9.1.8.6	S1 SETUP FAILURE

This message is sent by the MME to indicate S1 Setup failure.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Time to wait | O |  | 9.2.1.61 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.8.7	ENB CONFIGURATION UPDATE

This message is sent by the eNB to transfer updated information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| eNB Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Supported TAs |  | 0..<maxnoofTACs> |  | Supported TAs in the eNB. | GLOBAL | reject |
| >TAC | M |  | 9.2.3.7 | Broadcast TAC. | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >RAT-Type | O |  | 9.2.1.117 | RAT Type associated with the TAC of the indicated PLMN(s). | YES | reject |
| CSG Id List |  | 0..1 |  |  | GLOBAL | reject |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Default Paging DRX | O |  | 9.2.1.16 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.2.1.114 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 6. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |

#### 9.1.8.8	ENB CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the MME to acknowledge the eNB transfer updated information for a TNL association.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.8.9	ENB CONFIGURATION UPDATE FAILURE

This message is sent by the MME to indicate S1 eNB Configuration Update failure.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Time to wait | O |  | 9.2.1.61 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.8.10	MME CONFIGURATION UPDATE

This message is sent by the MME to transfer updated information for a TNL association.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Served GUMMEIs |  | 0..<maxnoofRATs> |  | The LTE related pool configuration is included on the first place in the list. | GLOBAL | reject |
| >Served PLMNs |  | 1..<maxnoofPLMNsPerMME> |  |  | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >Served GroupIDs |  | 1..<maxnoofGroupIDs> |  |  | - |  |
| >>MME GroupID | M |  | OCTET STRING (SIZE(2)) |  | - |  |
| >Served MMECs |  | 1..<maxnoofMMECs> |  |  | - |  |
| >>MME Code | M |  | 9.2.3.12 |  | - |  |
| Relative MME Capacity | O |  | 9.2.3.17 |  | YES | Reject |
| PSP list |  | 0..<maxnoofPSPInfo> |  |  | GLOBAL | ignore |
| >PSP Info | M |  | 9.2.3.mf1 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPLMNsPerMME | Maximum no. of PLMNs per MME. Value is 32. |
| maxnoofRATs | Maximum no. of RATs. Value is 8. |
| maxnoofGroupIDs | Maximum no. of GroupIDs per node per RAT. Value is 65535. |
| maxnoofMMECs | Maximum no. of MMECs per node per RAT. Value is 256. |
| maxnoofPSPInfo | Maximum no. of PSP Info per MME. Value is 1024. |

#### 9.1.8.11	MME CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the eNB to acknowledge the MME transfer updated information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.8.12	MME CONFIGURATION UPDATE FAILURE

This message is sent by the eNB to indicate S1 MME Configuration Update failure.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Time to wait | O |  | 9.2.1.61 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.8.13	OVERLOAD START

This message is sent by the MME and is used to indicate to the eNB that the MME is overloaded.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| Overload Response | M |  | 9.2.3.19 |  | YES | reject |
| GUMMEI List |  | 0..1 |  |  | YES | ignore |
| >GUMMEI List Item |  | 1..<maxnoofMMECs> |  |  | EACH | ignore |
| >>GUMMEI | M |  | 9.2.3.9 |  | - |  |
| Traffic Load Reduction Indication | O |  | 9.2.3.36 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMMECs | Maximum no. of MMECs per node per RAT. Value is 256. |

#### 9.1.8.14	OVERLOAD STOP

This message is sent by the MME and is used to indicate that the MME is no longer overloaded.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| GUMMEI List |  | 0..1 |  |  | YES | ignore |
| >GUMMEI List Item |  | 1..<maxnoofMMECs> |  |  | EACH | ignore |
| >>GUMMEI | M |  | 9.2.3.9 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMMECs | Maximum no. of MMECs per node per RAT. Value is 256. |

### 9.1.9	S1 CDMA2000 Tunnelling Messages

#### 9.1.9.1	DOWNLINK S1 CDMA2000 TUNNELLING

This message is sent by the MME and is used for carrying CDMA2000 information over the S1 interface.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-RABs Subject to Forwarding List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Subject to Forwarding Item IEs |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>DL Transport Layer Address | O |  | 9.2.2.1 |  | - |  |
| >>DL GTP-TEID | O |  | 9.2.2.2 | This IE indicates the tunnel endpoint for forwarding of DL data. | - |  |
| >>UL Transport Layer Address | O |  | 9.2.2.1 |  | - |  |
| >>UL GTP-TEID | O |  | 9.2.2.2 |  | - |  |
| CDMA2000 HO Status | O |  | 9.2.1.28 |  | YES | ignore |
| CDMA2000 RAT Type | M |  | 9.2.1.24 |  | YES | reject |
| CDMA2000-PDU | M |  | 9.2.1.23 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.1.9.2	UPLINK S1 CDMA2000 TUNNELLING

This message is sent by the eNB and is used for carrying CDMA2000 information over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| CDMA2000 RAT Type | M |  | 9.2.1.24 |  | YES | reject |
| CDMA2000 Sector ID | M |  | 9.2.1.25 |  | YES | reject |
| CDMA2000 HO Required Indication | O |  | 9.2.1.29 |  | YES | ignore |
| CDMA2000 1xRTT SRVCC Info | O |  | 9.2.1.35 |  | YES | reject |
| CDMA2000 1xRTT RAND | O |  | 9.2.1.33 |  | YES | reject |
| CDMA2000-PDU | M |  | 9.2.1.23 |  | YES | reject |
| E-UTRAN Round Trip Delay Estimation Info | O |  | 9.2.1.69 |  | YES | ignore |

### 9.1.10	UE CAPABILITY INFO INDICATION

This message is sent by the eNB to provide UE Radio Capability information to the MME.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Radio Capability | M |  | 9.2.1.27 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |

### 9.1.11	Trace Messages

#### 9.1.11.1	TRACE START

This message is sent by the MME to initiate trace recording for a UE.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Trace Activation | M |  | 9.2.1.4 |  | YES | ignore |

#### 9.1.11.2	TRACE FAILURE INDICATION

This message is sent by the eNB to indicate that a Trace Start procedure or a Deactivate Trace procedure has failed for a UE.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-UTRAN Trace ID | M |  | OCTET STRING (SIZE(8)) | As per E-UTRAN Trace ID IE in Trace Activation IE (9.2.1.4). | YES | ignore |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |

#### 9.1.11.3	DEACTIVATE TRACE

This message is sent by the MME to deactivate trace.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-UTRAN Trace ID | M |  | OCTET STRING (SIZE(8)) | As per E-UTRAN Trace ID IE in Trace Activation IE (9.2.1.4). | YES | ignore |

### 9.1.12	Location Reporting Messages

#### 9.1.12.1	LOCATION REPORTING CONTROL

This message is sent by the MME and is used to request the eNB to report where the UE is currently located.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Request Type | M |  | 9.2.1.34 |  | YES | ignore |

#### 9.1.12.2	LOCATION REPORT FAILURE INDICATION

This message is sent by the eNB and is used to indicate the failure of location report.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |

#### 9.1.12.3	LOCATION REPORT

This message is sent by the eNB and is used to provide the UE’s location to the MME.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| TAI | M |  | 9.2.3.16 |  | YES | ignore |
| Request Type | M |  | 9.2.1.34 | The Request Type IE is sent as it has been provided. | YES | ignore |

### 9.1.13	Warning Message Transmission Messages

#### 9.1.13.1	WRITE-REPLACE WARNING REQUEST

This message is sent by the MME to request the start or overwrite of the broadcast of a warning message.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Message Identifier | M |  | 9.2.1.44 |  | YES | reject |
| Serial Number | M |  | 9.2.1.45 |  | YES | reject |
| Warning Area List | O |  | 9.2.1.46 |  | YES | ignore |
| Repetition Period | M |  | 9.2.1.48 |  | YES | reject |
| Extended Repetition Period | O |  | 9.2.1.75 |  | YES | reject |
| Number of Broadcasts Requested | M |  | 9.2.1.49 |  | YES | reject |
| Warning Type | O |  | 9.2.1.50 |  | YES | ignore |
| Warning Security Information | O |  | 9.2.1.51 | See 3GPP TS 23.041 [29]. | YES | ignore |
| Data Coding Scheme | O |  | 9.2.1.52 |  | YES | ignore |
| Warning Message Contents | O |  | 9.2.1.53 |  | YES | ignore |
| Concurrent Warning Message Indicator | O |  | 9.2.1.72 |  | YES | reject |

#### 9.1.13.2	WRITE-REPLACE WARNING RESPONSE

This message is sent by the eNB to acknowledge the MME on the start or overwrite request of a warning message.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Message Identifier | M |  | 9.2.1.44 |  | YES | reject |
| Serial Number | M |  | 9.2.1.45 |  | YES | reject |
| Broadcast Completed Area List | O |  | 9.2.1.54 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.13.3	KILL REQUEST

This message is forwarded by the MME to eNB to cancel an already ongoing broadcast of a warning message

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Message Identifier | M |  | 9.2.1.44 |  | YES | reject |
| Serial Number | M |  | 9.2.1.45 |  | YES | reject |
| Warning Area List | O |  | 9.2.1.46 |  | YES | ignore |
| Kill-all Warning Messages Indicator | O |  | 9.2.1.91 |  | YES | reject |

#### 9.1.13.4	KILL RESPONSE

This message is sent by the eNB to indicate the list of warning areas where cancellation of the broadcast of the identified message was successful and unsuccessful.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Message Identifier | M |  | 9.2.1.44 |  | YES | reject |
| Serial Number | M |  | 9.2.1.45 |  | YES | reject |
| Broadcast Cancelled Area List | O |  | 9.2.1.70 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

#### 9.1.13.5	PWS RESTART INDICATION

This message is sent by the eNB to inform the MME that PWS information for some or all cells of the eNB are available for reloading from the CBC if needed.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| E-CGI List for Restart |  | 1..<maxnoofCellsforRestart> |  |  | EACH | reject |
| >E-CGI | M |  | 9.2.1.38 |  | - | - |
| Global eNB ID | M |  | 9.2.1.37 |  | YES | reject |
| TAI List for Restart |  | 1 .. <maxnoofRestartTAIs> |  |  | EACH | reject |
| >TAI | M |  | 9.2.3.16 |  | - | - |
| Emergency Area ID List for Restart |  | 0 .. <maxnoofRestartEmergencyAreaIDs> |  |  | EACH | reject |
| >Emergency Area ID | M |  | 9.2.1.47 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforRestart | Maximum no. of Cell ID subject for reloading warning messages broadcast. Value is 256. |
| maxnoofRestartTAIs | Maximum no. of TAI subject for reloading warning message broadcast. Value is 2048. |
| maxnoofRestartEmergencyAreaID | Maximum no. of Emergency Area ID subject for reloading warning message broadcast. Value is 256. |

#### 9.1.13.6	PWS FAILURE INDICATION

This message is sent by the eNB to inform the MME that ongoing PWS operation for one or more cells of the eNB has failed.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| PWS failed E-CGI List |  | 1..<maxnoofCellsineNB> |  |  | EACH | reject |
| >E-CGI | M |  | 9.2.1.38 |  | - | - |
| Global eNB ID | M |  | 9.2.1.37 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsineNB | Maximum no. of cells that can be served by an eNB. Value is 256. |

### 9.1.14	eNB DIRECT INFORMATION TRANSFER

This message is sent by the eNB in order to transfer specific information.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| Inter-system Information Transfer Type | M |  | 9.2.1.55 |  | YES | reject |

### 9.1.15	MME DIRECT INFORMATION TRANSFER

This message is sent by the MME in order to transfer specific information.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| Inter-system Information Transfer Type | M |  | 9.2.1.55 |  | YES | reject |

### 9.1.16	eNB CONFIGURATION TRANSFER

This message is sent by the eNB in order to transfer RAN configuration information.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.2.3.26 |  | YES | ignore |

### 9.1.17	MME CONFIGURATION TRANSFER

This message is sent by the MME in order to transfer RAN configuration information.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.2.3.26 |  | YES | ignore |

### 9.1.18	CELL TRAFFIC TRACE

This message is sent by eNB to transfer specific information.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| E-UTRAN Trace ID | M |  | OCTET STRING (SIZE(8)) | The E-UTRAN Trace ID IE is composed of the following: Trace Reference defined in 3GPP TS 32.422 [10] (leftmost 6 octets, with PLMN information coded as in 9.2.3.8), and Trace Recording Session Reference defined in 3GPP TS 32.422 [10] (last 2 octets). | YES | ignore |
| E-UTRAN CGI | M |  | 9.2.1.38 |  | YES | ignore |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.2.2.1 | Defined in 3GPP TS 32.422 [10] | YES | ignore |
| Privacy Indicator | O |  | ENUMERATED (Immediate MDT, Logged MDT, ...) |  | YES | ignore |

### 9.1.19	LPPa Transport Messages

#### 9.1.19.1	DOWNLINK UE ASSOCIATED LPPA TRANSPORT

This message is sent by the MME and is used for carrying LPPa message over the S1 interface.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Routing ID | M |  | 9.2.3.33 |  | YES | reject |
| LPPa-PDU | M |  | 9.2.3.32 |  | YES | reject |

#### 9.1.19.2	UPLINK UE ASSOCIATED LPPA TRANSPORT

This message is sent by the eNB and is used for carrying LPPa message over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Routing ID | M |  | 9.2.3.33 |  | YES | reject |
| LPPa-PDU | M |  | 9.2.3.32 |  | YES | reject |

#### 9.1.19.3	DOWNLINK NON UE ASSOCIATED LPPA TRANSPORT

This message is sent by the MME and is used for carrying LPPa message over the S1 interface.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| Routing ID | M |  | 9.2.3.33 |  | YES | reject |
| LPPa-PDU | M |  | 9.2.3.32 |  | YES | reject |

#### 9.1.19.4	UPLINK NON UE ASSOCIATED LPPA TRANSPORT

This message is sent by the eNB and is used for carrying LPPa message over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| Routing ID | M |  | 9.2.3.33 |  | YES | reject |
| LPPa-PDU | M |  | 9.2.3.32 |  | YES | reject |

## 9.2	Information Element Definitions

### 9.2.0	General

Subclause 9.2 presents the S1AP IE definitions in tabular format. The corresponding ASN.1 definition is presented in subclause 9.3. In case there is contradiction between the tabular format in subclause 9.2 and the ASN.1 definition, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional elements, where the tabular format shall take precedence.

The messages have been defined in accordance to the guidelines specified in 3GPP TR 25.921 [40].

When specifying information elements which are to be represented by bitstrings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bitstrings from other specifications, the first bit of the bitstring contains the first bit of the concerned information;

### 9.2.1	Radio Network Layer Related IEs

#### 9.2.1.1	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Code | M |  | INTEGER (0..255) |  |
| >Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, …) |  |

#### 9.2.1.2	E-RAB ID

This element uniquely identifies a radio access bearer for a particular UE, which makes the E-RAB ID unique over one S1 connection. The E-RAB ID shall remain the same for the duration of the E-RAB even if the UE-associated logical S1-connection is released or moved using S1 handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-RAB ID | M |  | INTEGER (0..15, …) |  |

#### 9.2.1.3	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the S1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, TX2RELOCOverall Expiry, Successful Handover, Release due to E-UTRAN Generated Reason, Handover Cancelled, Partial Handover, Handover Failure In Target EPC/eNB Or Target System, Handover Target not allowed, TS1RELOCoverall Expiry, TS1RELOCprep Expiry, Cell not available, Unknown Target ID, No Radio Resources Available in Target Cell, Unknown or already allocated MME UE S1AP ID, Unknown or already allocated eNB UE S1AP ID, Unknown or inconsistent pair of UE S1AP ID, Handover desirable for radio reasons, Time critical handover, Resource optimisation handover, Reduce load in serving cell, User inactivity, Radio Connection With UE Lost, Load Balancing TAU Required, CS Fallback Triggered, UE Not Available For PS Service, Radio resources not available, Failure in the Radio Interface Procedure, Invalid QoS combination, Inter-RAT redirection, Interaction with other procedure, Unknown E-RAB ID, Multiple E-RAB ID instances, Encryption and/or integrity protection algorithms not supported, S1 intra system Handover triggered, S1 inter system Handover triggered, X2 Handover triggered …, Redirection towards 1xRTT, Not supported QCI value, invalid CSG Id) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >NAS |  |  |  |  |
| >>NAS Cause | M |  | ENUMERATED (Normal Release, Authentication failure, Detach, Unspecified, …, CSG Subscription Expiry) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, Unknown PLMN, …) |  |

The meaning of the different cause values is described in the following table. In general, “not supported” cause values indicate that the related capability is missing. On the other hand, “not available” cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause |  | Meaning |
| --- | --- | --- |
| Unspecified |  | Sent for radio network layer cause when none of the specified cause values applies. |
| TX2RELOCOverall Expiry |  | The timer guarding the handover that takes place over X2 has abnormally expired. |
| Successful Handover |  | Successful handover. |
| Release due to E-UTRAN generated reason |  | Release is initiated due to E-UTRAN generated reason. |
| Handover Cancelled |  | The reason for the action is cancellation of Handover. |
| Partial Handover |  | Provides a reason for the handover cancellation. The HANDOVER COMMAND message from MME contained E-RABs to Release List IE and the source eNB estimated service continuity for the UE would be better by not proceeding with handover towards this particular target eNB. |
| Handover Failure In Target EPC/eNB Or Target System |  | The handover failed due to a failure in target EPC/eNB or target system. |
| Handover Target not allowed |  | Handover to the indicated target cell is not allowed for the UE in question. |
| TS1RELOCoverall Expiry |  | The reason for the action is expiry of timer TS1RELOCoverall. |
| TS1RELOCprep Expiry |  | Handover Preparation procedure is cancelled when timer TS1RELOCprep expires. |
| Cell not available |  | The concerned cell is not available. |
| Unknown Target ID |  | Handover rejected because the target ID is not known to the EPC. |
| No radio resources available in target cell |  | Load on target cell is too high. |
| Unknown or already allocated MME UE S1AP ID |  | The action failed because the MME UE S1AP ID is either unknown, or (for a first message received at the eNB) is known and already allocated to an existing context. |
| Unknown or already allocated eNB UE S1AP ID |  | The action failed because the eNB UE S1AP ID is either unknown, or (for a first message received at the MME) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE S1AP ID |  | The action failed because both UE S1AP IDs are unknown, or are known but do not define a single UE context. |
| Handover Desirable for Radio Reasons |  | The reason for requesting handover is radio related. |
| Time Critical Handover |  | Handover is requested for time critical reason i.e., this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| Resource Optimisation Handover |  | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Reduce Load in Serving Cell |  | Load on serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| User Inactivity |  | The action is requested due to user inactivity on all E-RABs, e.g., S1 is requested to be released in order to optimise the radio resources. |
| Radio Connection With UE Lost |  | The action is requested due to losing the radio connection to the UE. |
| Load Balancing TAU Required |  | The action is requested for all load balancing and offload cases in the MME. |
| CS Fallback triggered |  | The action is due to a CS fallback that has been triggered. When it is included in UE CONTEXT RELEASE REQUEST message, it indicates the PS service suspension is not required in the EPC. |
| UE Not Available for PS Service |  | The action is requested due to a CS fallback to GERAN that has been triggered. When it is included in the UE CONTEXT RELEASE REQUEST message, it indicates that the PS service suspension is required in the EPC due to the target GERAN cell or the UE has no DTM capability. |
| Radio resources not available |  | No requested radio resources are available. |
| Invalid QoS combination |  | The action was failed because of invalid QoS combination. |
| Inter-RAT Redirection |  | The release is requested due to inter-RAT redirection. When it is included in UE CONTEXT RELEASE REQUEST message, it indicates the PS service suspension is not required in the EPC. |
| Failure in the Radio Interface Procedure |  | Radio interface procedure has failed. |
| Interaction with other procedure |  | The action is due to an ongoing interaction with another procedure. |
| Unknown E-RAB ID |  | The action failed because the E-RAB ID is unknown in the eNB. |
| Multiple E-RAB ID Instances |  | The action failed because multiple instance of the same E-RAB had been provided to the eNB. |
| Encryption and/or integrity protection algorithms not supported |  | The eNB is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| S1 Intra system Handover triggered | The action is due to a S1 intra system handover that has been triggered. |  |
| S1 Inter system Handover triggered | The action is due to a S1 inter system handover that has been triggered. |  |
| X2 Handover triggered | The action is due to an X2 handover that has been triggered. |  |
| Redirection towards 1xRTT | The release of the UE-associated logical S1 connection is requested due to redirection towards a 1xRTT system e.g., CS fallback to 1xRTT, or SRVCC to 1xRTT, when the PS service suspension is required in the EPC. During this procedure, the radio interface message might but need not include redirection information. |  |
| Not supported QCI Value | The E-RAB setup failed because the requested QCI is not supported. |  |
| Invalid CSG Id | The CSG ID provided to the target eNB was found invalid. |  |

| Transport Layer cause | Meaning |
| --- | --- |
| Transport Resource Unavailable | The required transport resources are not available. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

| NAS cause | Meaning |
| --- | --- |
| Normal Release | The release is normal. |
| Authentication Failure | The action is due to authentication failure. |
| Detach | The action is due to detach. |
| Unspecified | Sent when none of the above cause values applies but still the cause is NAS related. |
| CSG Subscription Expiry | The action is due to the UE becoming a non-member of the currently used CSG. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated “reject”. |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated “ignore and notify”. |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | Control processing overload. |
| Not Enough User Plane Processing Resources Available | No enough resources are available related to user plane processing. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |
| Unspecified Failure | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer, NAS or Protocol. |
| Unknown PLMN | The MME does not identify any PLMN provided by the eNB. |

#### 9.2.1.3a	RRC Establishment Cause

The purpose of the RRC Establishment Cause IE is to indicate to the MME the reason for RRC Connection Establishment or RRC Connection Resume as received from the UE in the EstablishmentCause, EstablishmentCause-NB or ResumeCause defined in MFA TS 36.331 [16].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC Establishment Cause | M |  | ENUMERATED(emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, …,delayTolerantAccess, mo-VoiceCall, mo-ExceptionData) |  |

#### 9.2.1.4	Trace Activation

Defines parameters related to a trace activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRAN Trace ID | M |  | OCTET STRING (SIZE(8)) | The E-UTRAN Trace ID IE is composed of the following: Trace Reference defined in 3GPP TS 32.422 [10] (leftmost 6 octets, with PLMN information coded as in 9.2.3.8), and Trace Recording Session Reference defined in 3GPP TS 32.422 [10] (last 2 octets). |  |  |
| Interfaces To Trace | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents a eNB interface: first bit =S1-MME, second bit =X2, third bit =Uu: other bits reserved for future use. Value ‘1’ indicates ‘should be traced’. Value ‘0’ indicates ‘should not be traced’. |  |  |
| Trace depth | M |  | ENUMERATED( minimum, medium, maximum, MinimumWithoutVendorSpecificExtension, MediumWithoutVendorSpecificExtension, MaximumWithoutVendorSpecificExtension, …) | Defined in 3GPP TS 32.422 [10]. |  |  |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.2.2.1 | Defined in 3GPP TS 32.422 [10]. |  |  |
| MDT Configuration | O |  | 9.2.1.81 |  | YES | ignore |

#### 9.2.1.5	Source ID

Void.

#### 9.2.1.6	Target ID

The Target ID IE identifies the target for the handover. The target ID may be, e.g., the target Global eNB-ID (for intra SAE/LTE), the RNC-ID (for SAE/LTE-UMTS handover) or the Cell Global ID of the handover target (in case of SAE/LTE to GERAN A/Gb mode handover).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Target ID | M |  |  |  | - | - |
| >Target eNB-ID |  |  |  |  | - | - |
| >>Global eNB ID | M |  | 9.2.1.37 |  | - | - |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  | - | - |
| >Target RNC-ID |  |  |  |  | - | - |
| >>LAI | M |  | 9.2.3.1 |  | - | - |
| >>RAC | O |  | 9.2.3.2 |  | - | - |
| >>RNC-ID | M |  | INTEGER (0..4095) | If the Extended RNC-ID IE is included in the Target ID IE, the RNC-ID IE shall be ignored. | - | - |
| >>Extended RNC-ID | O |  | 9.2.1.14 | The Extended RNC-ID IE shall be used if the RNC identity has a value larger than 4095. | - | - |
| >CGI |  |  |  |  | - | - |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - | - |
| >>LAC | M |  | OCTET STRING (SIZE(2)) | 0000 and FFFE not allowed. | - | - |
| >>CI | M |  | OCTET STRING (SIZE(2)) |  | - | - |
| >>RAC | O |  | 9.2.3.2 |  | - | - |

#### 9.2.1.7	Source eNB to Target eNB Transparent Container

The Source eNB to target eNB Transparent Container IE is an information element that is produced by the source eNB and is transmitted to the target eNB. For inter-system handovers to E-UTRAN, the IE is transmitted from the external handover source to the target eNB.

This IE is transparent to the EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC Handover Preparation Information message as defined in subclause 10.2.2 of MFA TS 36.331 [16]. | - |  |
| E-RABs Information List |  | 0..1 |  |  | - |  |
| >E-RABs Information Item |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>DL Forwarding | O |  | 9.2.3.14 |  | - |  |
| Target Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | - |  |
| UE History Information | M |  | 9.2.1.42 |  | - |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the external handover source provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (see MFA TS 36.331 [16]) | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.2.1.8	Target eNB to Source eNB Transparent Container

The Target eNB to Source eNB Transparent Container IE is an information element that is produced by the target eNB and is transmitted to the source eNB. For inter-system handovers to E-UTRAN, the IE is transmitted from the target eNB to the external relocation source.

This IE is transparent to EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC E-UTRA Handover Command message as defined in subclause 10.2.2 of MFA TS 36.331 [16]. | - |  |

#### 9.2.1.9	Source RNC to Target RNC Transparent Container

This IE is used to transparently pass radio related information between the handover source and the handover target through the EPC. This container is used for inter 3GPP RAT handovers from SAE/LTE to UTRAN.

This IE is defined in 3GPP TS 25.413 [19].

#### 9.2.1.10	Target RNC to Source RNC Transparent Container

This container is used to transparently pass radio related information between the handover target and the handover source through the EPC. This container is used inter 3GPP RAT handovers from SAE/LTE to UTRAN.

This IE is defined in 3GPP TS 25.413 [19].

#### 9.2.1.11	Source BSS to Target BSS Transparent Container

This container is used to transparently pass radio related information between the handover source and the handover target through the EPC. This container is used for inter 3GPP RAT handovers from SAE/LTE to GERAN A/Gb mode.

This IE is defined in 3GPP TS 48.018 [18].

#### 9.2.1.12	Target BSS to Source BSS Transparent Container

This container is used to transparently pass radio related information between the handover source and the handover target through the EPC. This container is used for inter 3GPP RAT handovers from SAE/LTE to GERAN A/Gb mode.

This IE is defined in 3GPP TS 48.018 [18].

#### 9.2.1.13	Handover Type

This IE indicates which kind of handover was triggered in the source side.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Type | M |  | ENUMERATED (IntraLTE, LTEtoUTRAN, LTEtoGERAN, UTRANtoLTE, GERANtoLTE) |  |

#### 9.2.1.14	Extended RNC-ID

The Extended RNC-ID is used to identify an RNC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended RNC-ID | M |  | INTEGER (4096..65535) | The Extended RNC-ID IE shall be used if the RNC identity has a value larger than 4095. |

#### 9.2.1.15	E-RAB Level QoS Parameters

This IE defines the QoS to be applied to an E-RAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-RAB Level QoS Parameters |  |  |  |  |
| >QCI | M |  | INTEGER (0..255) | QoS Class Identifier defined in 3GPP TS 23.401 [11]. Coding specified in 3GPP TS 23.203 [13]. |
| >Allocation and Retention Priority | M |  | 9.2.1.60 |  |
| >GBR QoS Information | O |  | 9.2.1.18 | This IE applies to GBR bearers only and shall be ignored otherwise. |

#### 9.2.1.16	Paging DRX

This IE indicates the Paging DRX as defined in MFA TS 36.304 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Paging DRX | M |  | ENUMERATED(32, 64, 128, 256, …) |  | - |  |

#### 9.2.1.17	Paging Cause

Void.

#### 9.2.1.18	GBR QoS Information

This IE indicates the maximum and guaranteed bit rates of a GBR bearer for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-RAB Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.1.19 | Desc.: This IE indicates the maximum downlink E-RAB Bit Rate as specified in 3GPP TS 23.401 [11] for this bearer. |
| E-RAB Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.1.19 | Desc.: This IE indicates the maximum uplink E-RAB Bit Rate as specified in 3GPP TS 23.401 [11] for this bearer. |
| E-RAB Guaranteed Bit Rate Downlink | M |  | Bit Rate 9.2.1.19 | Desc.: This IE indicates the downlink guaranteed E-RAB Bit Rate as specified in 3GPP TS 23.401 [11] (provided that there is data to deliver) for this bearer. |
| E-RAB Guaranteed Bit Rate Uplink | M |  | Bit Rate 9.2.1.19 | Desc.: This IE indicates the uplink guaranteed E-RAB Bit Rate as specified in 3GPP TS 23.401 [11] (provided that there is data to deliver) for this bearer. |

#### 9.2.1.19	Bit Rate

This IE indicates the number of bits delivered by E-UTRAN in UL or to E-UTRAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR bearer, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate |  |  | INTEGER (0..10,000,000,000) | The unit is: bit/s. |

#### 9.2.1.20	UE Aggregate Maximum Bit Rate

The UE Aggregate Maximum Bitrate is applicable for all Non-GBR bearers per UE which is defined for the Downlink and the Uplink direction and provided by the MME to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Aggregate Maximum Bit Rate |  |  |  | Applicable for non-GBR E-RABs. |
| >UE Aggregate Maximum Bit Rate Downlink | M |  | Bit Rate 9.2.1.19 | This IE indicates the UE Aggregate Maximum Bit Rate as specified in 3GPP TS 23.401 [11] in the downlink direction. |
| >UE Aggregate Maximum Bit Rate Uplink | M |  | Bit Rate 9.2.1.19 | This IE indicates the UE Aggregate Maximum Bit Rate as specified in 3GPP TS 23.401 [11] in the uplink direction. Receiving both the UE Aggregate Maximum Bit Rate Downlink IE and the UE Aggregate Maximum Bit Rate Uplink IE equal to value zero shall be considered as a logical error by the eNB. |

#### 9.2.1.21	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the eNB or the MME when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing.

For further details on how to use the Criticality Diagnostics IE, (see clause 10).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED(initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED(reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| Information Element Criticality Diagnostics |  | 0 .. <maxnoof Errors> |  |  |
| >IE Criticality | M |  | ENUMERATED(reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value ‘ignore’ shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE. |
| >Type of Error | M |  | ENUMERATED(not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxnoofErrors is 256. |

#### 9.2.1.22	Handover Restriction List

This IE defines roaming or access restrictions for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, e.g., handover and CCO, or for SCG selection during dual connectivity operation. If the eNB receives the Handover Restriction List IE, it shall overwrite previously received restriction information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Serving PLMN | M |  | 9.2.3.8 |  |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of “equivalent PLMNs” as defined in MFA TS 24.301 [24]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the Serving PLMN and Equivalent PLMNs. |
| >PLMN Identity | M |  | 9.2.3.8 |  |
| Forbidden TAs |  | 0..<maxnoofEPLMNsPlusOne> |  | Intra LTE roaming restrictions. |
| >PLMN Identity | M |  | 9.2.3.8 | The PLMN of forbidden TACs. |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  |
| >>TAC | M |  | 9.2.3.7 | The TAC of the forbidden TAI. |
| Forbidden LAs |  | 0..<maxnoofEPLMNsPlusOne> |  | Inter-3GPP RAT roaming restrictions. |
| >PLMN Identity | M |  | 9.2.3.8 |  |
| >Forbidden LACs |  | 1..<maxnoofForbLACs> |  |  |
| >>LAC | M |  | OCTET STRING (SIZE(2)) |  |
| Forbidden inter RATs | O |  | ENUMERATED(ALL, GERAN, UTRAN, CDMA2000, …, GERAN and UTRAN, CDMA2000 and UTRAN) | Inter-3GPP and 3GPP2 RAT access restrictions. |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMN Ids. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of equivalent PLMN Ids plus one. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbLACs | Maximum no. of forbidden Location Area Codes. Value is 4096. |

#### 9.2.1.23	CDMA2000-PDU

This information element contains a CDMA2000 message between the UE and CDMA2000 RAT that is transferred without interpretation in the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CDMA2000-PDU | M |  | OCTET STRING |  |

#### 9.2.1.24	CDMA2000 RAT Type

In the uplink, this information element, along with the CDMA2000 Sector ID IE is used for routing the tunnelled CDMA2000 message to the proper destination node in the CDMA2000 RAT and is set by the eNB to the CDMA2000 RAT type received from the UE.

NOTE:	In the downlink, this information element is used by the eNB to provide an indication of the RAT Type associated with the tunnelled CDMA2000 message to the UE to help it route the tunnelled downlink CDMA2000 message to the appropriate CDMA upper layer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CDMA2000 RAT Type | M |  | ENUMERATED (HRPD, 1xRTT, …) | This IE is used to identify which CDMA2000 RAT the tunnelled CDMA2000 signalling is associated with. The source of this information in the uplink is the UE and in the downlink it is the CDMA2000 system. |

#### 9.2.1.25	CDMA2000 Sector ID

This information element, along with the RAT Type IE is used for routing the tunnelled CDMA2000 message to the proper destination node in the CDMA2000 RAT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CDMA2000 Sector ID | M |  | OCTET STRING | This IE is set to CDMA2000 Reference Cell ID corresponding to the HRPD/1xRTT sector under the HRPD AN/1xBS towards which the signalling is performed. The CDMA2000 Reference Cell ID is statically configured in the eNB. If the RAT type is HRPD, this IE contains the HRPD Sector ID as specified in 3GPP2 C.S0024-B [27]. If the RAT type is 1x RTT, this IE is encoded as the Reference Cell ID IE in 3GPP2 A.S0008-C [25]. |

#### 9.2.1.26	Security Context

The purpose of the Security Context IE is to provide security related parameters to the eNB which are used to derive security keys for user plane traffic and RRC signalling messages and for security parameter generation for subsequent X2 or intra eNB Handovers, or for the security parameters for the current S1 Handover. For intra LTE S1 Handover one pair of {NCC, NH} is provided for 1-hop security, see MFA TS 33.401 [15].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Counter (NCC) defined in MFA TS 33.401 [15]. For inter-RAT Handover into LTE the Next Hop Chaining Count IE takes the value defined for NCC at initial setup, i.e., Next Hop Chaining Count IE = “0”. |
| Next-Hop NH | M |  | 9.2.1.41 Security Key | The NH together with the NCC is used to derive the security configuration as defined in MFA TS 33.401 [15]. For inter RAT Handover the Next-Hop NH IE is the KeNB to be used in the new configuration. |

#### 9.2.1.27	UE Radio Capability

This IE contains UE Radio Capability information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Radio Capability | M |  | OCTET STRING | Includes either the UERadioAccessCapabilityInformation message as defined in 10.2.2 of MFA TS 36.331 [16], or the UERadioAccessCapabilityInformation-NB message as defined in 10.6.2 of MFA TS 36.331 [16]. |

#### 9.2.1.28	CDMA2000 HO Status

This IE is used to indicate to the eNB which initiated an inter-RAT HO towards CDMA2000 about the outcome of the handover preparation to CDMA2000.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CDMA2000 HO Status | M |  | ENUMERATED (HO Success, HO Failure, …) | This IE indicates the status of the handover resource allocation in the CDMA2000 RAT. |

#### 9.2.1.29	CDMA2000 HO Required Indication

This information element is set by the eNB to provide an indication about whether the UE has initiated the handover preparation with the CDMA2000 RAT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CDMA2000 HO Required Indication | M |  | ENUMERATED (true, …) | This IE indicates to MME that handover preparation to CDMA2000 has been started. It helps MME to decide when to send certain handover preparation information for HRPD (see 3GPP TS 23.402 [8]) and 1xRTT (see 3GPP TS 23.216 [9]) to the CDMA2000 RAT. |

#### 9.2.1.30	1xRTT MEID

Void.

#### 9.2.1.31	eNB Status Transfer Transparent Container

The eNB Status Transfer Transparent Container IE is an information element that is produced by the source eNB and is transmitted to the target eNB. This IE is used for the intra SAE/LTE S1 handover case.

This IE is transparent to the EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RABs Subject to Status Transfer List |  | 1 |  |  | - | - |
| >E-RABs Subject to Status Transfer Item |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - | - |
| >>UL COUNT value | M |  | COUNT Value 9.2.1.32 | PDCP-SN and HFN of first missing UL PDCP SDU in case of 12 bit long PDCP-SN. | - | - |
| >>DL COUNT value | M |  | COUNT Value 9.2.1.32 | PDCP-SN and HFN that the target eNB should assign for the next DL SDU not having an SN yet in case of 12 bit long PDCP-SN. | - | - |
| >>Receive Status Of UL PDCP SDUs | O |  | BIT STRING (SIZE(4096)) | PDCP Sequence Number = (First Missing SDU Number + bit position) modulo 4096. 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. |  |  |
| >>UL COUNT Value Extended | O |  | COUNT Value Extended 9.2.1.90 | PDCP-SN and HFN of first missing UL PDCP SDU in case of 15 bit long PDCP-SN. | YES | ignore |
| >>DL COUNT Value Extended | O |  | COUNT Value Extended 9.2.1.90 | PDCP-SN and HFN that the target eNB should assign for the next DL SDU not having an SN yet in case of 15 bit long PDCP-SN. | YES | ignore |
| >>Receive Status Of UL PDCP SDUs Extended | O |  | BIT STRING (SIZE(1..16384)) | The IE is used in case of 15 bit long PDCP-SN in this release. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | YES | ignore |
| >>UL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.1.100 | PDCP-SN and HFN of first missing UL PDCP SDU in case of 18 bit long PDCP-SN. | YES | ignore |
| >>DL COUNT Value for PDCP SN Length 18 | O |  | COUNT Value for PDCP SN Length 18 9.2.1.100 | PDCP-SN and HFN that the target eNB should assign for the next DL SDU not having an SN yet in case of 18 bit long PDCP-SN. | YES | ignore |
| >>Receive Status Of UL PDCP SDUs for PDCP SN Length 18 | O |  | BIT STRING (SIZE(1..131072)) | The IE is used in case of 18 bit long PDCP-SN. The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

#### 9.2.1.32	COUNT Value

This IE contains a PDCP sequence number and a hyper frame number in case of 12 bit long PDCP-SN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP-SN | M |  | INTEGER (0..4095) |  | - | - |
| HFN | M |  | INTEGER (0..1048575) |  | - | - |

#### 9.2.1.33	CDMA2000 1xRTT RAND

This information element is a random number generated by the eNB and tunnelled to the 1xCS IWS (see 3GPP TS 23.402 [8]) and is transparent to MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CDMA2000 1xRTT RAND | M |  | OCTET STRING | This IE is a Random Challenge that is used for authentication of UE during 1xCS registration, eCSFB to 1xRTT or handover from E-UTRAN to CDMA2000 1xRTT RAT. This IE is coded as the RAND (32bits) of the Authentication Challenge Parameter (RAND) in 3GPP2 A.S0008-C [25]. |

#### 9.2.1.34	Request Type

The purpose of the Request Type IE is to indicate the type of location request to be handled by the eNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Request Type |  |  |  |  |
| >Event Type | M |  | ENUMERATED(Direct, Change of service cell, Stop Change of service cell, …) |  |
| >Report Area | M |  | ENUMERATED (ECGI, …) |  |

#### 9.2.1.35	CDMA2000 1xRTT SRVCC Info

This IE defines SRVCC related information elements that are assembled by the MME to be tunnelled transparently to the 1xCS IWS (see 3GPP TS 23.402 [8]) system.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CDMA2000 1xRTT SRVCC Info |  |  |  |  |
| >CDMA2000 1xRTT MEID | M |  | OCTET STRING | This information element is the Mobile Equipment Identifier or Hardware ID that is tunnelled from the UE and is transparent to the eNB. This IE is used to derive a MEID-based PLCM that is used for channelization in CDMA2000 1xRTT network. |
| >CDMA2000 1xRTT Mobile Subscription Information | M |  | OCTET STRING | This IE provides the list of UE supported 1x RTT Band classes and Band Subclasses. It is provided by the UE to the eNB as part of the UE capability. It is transparent to the eNB. |
| >CDMA2000 1xRTT Pilot List | M |  | OCTET STRING | This IE provides the measured pilot information. Encoded as the Pilot List IE from the A21-1x air interface signalling message in 3GPP2 A.S0008-C [25]. |

#### 9.2.1.36	E-RAB List

This IE contains a list of E-RAB IDs with a cause value. It is used for example to indicate failed bearers or bearers to be released.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RAB List Item |  | 1 .. <maxnoofE-RABs> |  |  | EACH | ignore |
| >E-RAB ID | M |  | 9.2.1.2 |  | - | - |
| >Cause | M |  | 9.2.1.3 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RAB allowed towards one UE, the maximum value is 256. |

#### 9.2.1.37	Global eNB ID

This information element is used to globally identify an eNB (see 3GPP TS 36.401 [2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| CHOICE eNB ID | M |  |  |  |
| >Macro eNB ID |  |  |  |  |
| >>Macro eNB ID | M |  | BIT STRING (SIZE(20)) | Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Home eNB ID |  |  |  |  |
| >>Home eNB ID | M |  | BIT STRING (SIZE(28)) | Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of the cell served by the eNB. |

#### 9.2.1.38	E-UTRAN CGI

This information element is used to globally identify a cell (see 3GPP TS 36.401 [2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| Cell Identity | M |  | BIT STRING (SIZE(28)) | The leftmost bits of the Cell Identity correspond to the eNB ID (defined in subclause 9.2.1.37). |

#### 9.2.1.39	Subscriber Profile ID for RAT/Frequency priority

The Subscriber Profile ID IE for RAT/Frequency Selection Priority is used to define camp priorities in Idle mode and to control inter-RAT/inter-frequency handover in Active mode (see MFA TS 36.300 [14]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subscriber Profile ID for RAT/Frequency Priority | M |  | INTEGER (1..256) |  |

#### 9.2.1.40	UE Security Capabilities

The UE Security Capabilities IE defines the supported algorithms for encryption and integrity protection in the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Security Capabilities |  |  |  |  |
| >Encryption Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an encryption algorithm: “all bits equal to 0” – UE supports no other algorithm than EEA0, “first bit” – 128-EEA1, “second bit” – 128-EEA2, “third bit” – 128-EEA3, other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in MFA TS 33.401 [15]. |
| >Integrity Protection Algorithms | M |  | BIT STRING (SIZE(16, …)) | Each position in the bitmap represents an integrity protection algorithm: “all bits equal to 0” – UE supports no other algorithm than EIA0, “first bit” – 128-EIA1, “second bit” – 128-EIA2, “third bit” – 128-EIA3, other bits reserved for future use. Value ‘1’ indicates support and value ‘0’ indicates no support of the algorithm. Algorithms are defined in MFA TS 33.401 [15]. |

#### 9.2.1.41	Security Key

The Security Key IE is used to apply security in the eNB for different scenarios as defined in MFA TS 33.401 [15].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Security Key | M |  | BIT STRING (SIZE(256)) | Key material for KeNB or Next Hop Key as defined in MFA TS 33.401 [15] |

#### 9.2.1.42	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1 .. <maxnoOfCells> |  | Most recent information is added to the top of this list. | - | - |
| >Last Visited Cell Information | M |  | 9.2.1.43 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoOfCells | Maximum length of the list. Value is 16. |

#### 9.2.1.43	Last Visited Cell Information

The Last Visited Cell Information may contain E-UTRAN or UTRAN cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Last Visited Cell Information | M |  |  |  | - | - |
| >E-UTRAN Cell |  |  |  |  |  |  |
| >>Last Visited E-UTRAN Cell Information | M |  | 9.2.1.43a |  | - | - |
| >UTRAN Cell |  |  |  |  |  |  |
| >>Last Visited UTRAN Cell Information | M |  | OCTET STRING | Defined in 3GPP TS 25.413 [19]. | - | - |
| >GERAN Cell |  |  |  |  |  |  |
| >>Last Visited GERAN Cell Information | M |  | 9.2.1.43b |  | - | - |

#### 9.2.1.43a	Last Visited E-UTRAN Cell Information

The Last Visited E-UTRAN Cell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| Cell Type | M |  | 9.2.1.66 |  | - |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the cell in seconds. If the UE stays in a cell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. | YES | ignore |
| HO Cause Value | O |  | 9.2.1.3 | The cause for the handover from the E-UTRAN cell. | YES | ignore |

#### 9.2.1.43b	Last Visited GERAN Cell Information

The Last Visited Cell Information for GERAN is currently undefined.

NOTE:	If in later Releases this is defined, the choice type may be extended with the actual GERAN specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Last Visited GERAN Cell Information | M |  |  |  | - |  |
| >Undefined | M |  | NULL |  | - |  |

#### 9.2.1.44	Message Identifier

The purpose of the Message Identifier IE is to identify the warning message. Message Identifier IE is set by the EPC and transferred to the UE by the eNB

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Message Identifier | M |  | BIT STRING (SIZE(16)) | This IE is set by the EPC, transferred to the UE by the eNB. The eNB shall treat it as an identifier of the message. |

#### 9.2.1.45	Serial Number

The Serial Number IE identifies a particular message from the source and type indicated by the Message Identifier and is altered every time the message with a given Message Identifier is changed.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Serial Number | M |  | BIT STRING (SIZE(16)) |  |

#### 9.2.1.46	Warning Area List

The Warning Area List IE indicates the areas where the warning message needs to be broadcast or cancelled.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Warning Area | M |  |  |  |
| >Cell ID List |  | 1 .. <maxnoofCellID> |  |  |
| >>E-CGI | M |  | 9.2.1.38 |  |
| >TAI List for Warning |  | 1 .. <maxnoofTAIforWarning> |  |  |
| >>TAI | M |  | 9.2.3.16 |  |
| >Emergency Area ID List |  | 1 .. <maxnoofEmergencyAreaID> |  |  |
| >>Emergency Area ID | M |  | 9.2.1.47 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellID | Maximum no. of Cell ID subject for warning message broadcast. Value is 65535. |
| maxnoofTAIforWarning | Maximum no. of TAI subject for warning message broadcast. Value is 65535. |
| maxnoofEmergencyAreaID | Maximum no. of Emergency Area ID subject for warning message broadcast. Value is 65535. |

#### 9.2.1.47	Emergency Area ID

The Emergency Area ID IE is used to indicate the area which has the emergency impact.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Emergency Area ID | M |  | OCTET STRING (SIZE(3)) | Emergency Area ID may consist of several cells. Emergency Area ID is defined by the operator. |

#### 9.2.1.48	Repetition Period

The Repetition Period IE indicates the periodicity of the warning message to be broadcast.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Repetition Period | M |  | INTEGER (0..4095) | The unit of value 1 to 4095 is [second]. |

#### 9.2.1.49	Number of Broadcasts Requested

The Number of Broadcast Requested IE indicates the number of times a message is to be broadcast.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number of Broadcasts Requested | M |  | INTEGER (0..65535) |  |

#### 9.2.1.50	Warning Type

The Warning Type IE indicates types of the disaster. This IE also indicates that a Primary Notification is included. This IE can be used by the UE to differentiate the type of alert according to the type of disaster.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Warning Type | M |  | OCTET STRING (SIZE(2)) |  |

#### 9.2.1.51	Warning Security Information

The Warning Security Information IE provides the security information needed for securing the Primary Notification.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Warning Security Information | M |  | OCTET STRING(SIZE(50)) |  |

#### 9.2.1.52	Data Coding Scheme

The Data Coding Scheme IE identifies the alphabet or coding employed for the message characters and message handling at the UE (it is passed transparently from the EPC to the UE).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Data Coding Scheme | M |  | BIT STRING (SIZE(8)) |  |

#### 9.2.1.53	Warning Message Contents

The Warning Message Content IE contains user information, e.g., the message with warning contents, and will be broadcast over the radio interface.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Warning Message Contents | M |  | OCTET STRING (SIZE(1..9600)) | The length of this IE varies between 1 to 9600 bytes. |

#### 9.2.1.54	Broadcast Completed Area List

The Broadcast Completed Area List IE indicates the areas where either resources are available to perform the broadcast or where broadcast is performed successfully.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Broadcast Completed Area | M |  |  |  |
| >Broadcast Completed Area |  |  |  |  |
| >>Cell ID Broadcast |  | 1 .. <maxnoofCellID> |  |  |
| >>>E-CGI | M |  | 9.2.1.38 |  |
| >TAI Broadcast |  |  |  |  |
| >>TAI Broadcast |  | 1 .. <maxnoofTAIforWarning> |  |  |
| >>>TAI | M |  | 9.2.3.16 |  |
| >>>Completed Cell in TAI List |  | 1 .. <maxnoofCellinTAI> |  |  |
| >>>>E-CGI | M |  |  |  |
| >Emergency Area ID |  |  |  |  |
| >>Emergency Area ID Broadcast |  | 1 .. <maxnoofEmergencyAreaID> |  |  |
| >>>Emergency Area ID | M |  | 9.2.1.47 |  |
| >>>Completed Cell in Emergency Area ID List |  | 1 .. <maxnoofCellinEAI> |  |  |
| >>>>E-CGI | M |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellID | Maximum no. of Cell ID subject for warning message broadcast. Value is 65535. |
| maxnoofTAIforWarning | Maximum no. of TAI subject for warning message broadcast. Value is 65535. |
| maxnoofEmergencyAreaID | Maximum no. of Emergency Area ID subject for warning message broadcast. Value is 65535. |
| maxnoofCellinTAI | Maximum no. of Cell ID within a TAI. Value is 65535. |
| maxnoofCellinEAI | Maximum no. of Cell ID within an Emergency Area. Value is 65535. |

#### 9.2.1.55	Inter-system Information Transfer Type

The Inter-system Information Type IE indicates the type of information that the eNB requests to transfer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system Information Transfer Type | M |  |  |  |
| >RIM |  |  |  |  |
| >>RIM Transfer | M |  | 9.2.3.23 |  |

#### 9.2.1.56	Source To Target Transparent Container

The Source to Target Transparent Container IE is an information element that is used to transparently pass radio related information from the handover source to the handover target through the EPC; it is produced by the source RAN node and is transmitted to the target RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Source to Target Transparent Container | M |  | OCTET STRING | This IE includes a transparent container from the source RAN node to the target RAN node. The octets of the OCTET STRING are encoded according to the specifications of the target system. Note: in the current version of the specification, this IE may either carry the Source eNB to Target eNB Transparent Container IE or the Source RNC to Target RNC Transparent Container IE as defined in 3GPP TS 25.413 [19] or the Source BSS to Target BSS Transparent Container Contents of the Source BSS to Target BSS Transparent Container IE as defined in 3GPP TS 48.018 [18] or the Old BSS to New BSS information elements field of the Old BSS to New BSS information IE as defined in 3GPP TS 48.008 [23]. |

#### 9.2.1.57	Target To Source Transparent Container

The Target to Source Transparent Container IE is an information element that is used to transparently pass radio related information from the handover target to the handover source through the EPC; it is produced by the target RAN node and is transmitted to the source RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target to Source Transparent Container | M |  | OCTET STRING | This IE includes a transparent container from the target RAN node to the source RAN node. The octets of the OCTET STRING are coded according to the specifications of the target system. Note: in the current version of the specification, this IE may either carry the Target eNB to Source eNB Transparent Container IE or the Target RNC to Source RNC Transparent Container IE as defined in 3GPP TS 25.413 [19] or the Target BSS to Source BSS Transparent Container Contents of the Target BSS to Source BSS Transparent Container IE as defined in 3GPP TS 48.018 [18] or the Layer 3 Information field of the Layer 3 Information IE as defined in 3GPP TS 48.008 [23]. |

#### 9.2.1.58	SRVCC Operation Possible

This element indicates that both UE and MME are SRVCC-capable. E-UTRAN behaviour on receipt of this IE is specified in 3GPP TS 23.216 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRVCC operation possible | M |  | ENUMERATED (Possible, …) |  |

#### 9.2.1.59	SRVCC HO Indication

This information element is set by the source eNB to provide an indication that E-RAB may be subjected to handover via SRVCC means.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRVCC HO Indication | M |  | ENUMERATED (PS and CS, CS only, …) |  |

#### 9.2.1.60	Allocation and Retention Priority

This IE specifies the relative importance compared to other E-RABs for allocation and retention of the E-UTRAN Radio Access Bearer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allocation/Retention Priority |  |  |  |  |
| >Priority Level | M |  | INTEGER (0..15) | Desc.: This IE should be understood as “priority of allocation and retention” (see 3GPP TS 23.401 [11]). Usage: Value 15 means “no priority”. Values between 1 and 14 are ordered in decreasing order of priority, i.e., 1 is the highest and 14 the lowest. Value 0 shall be treated as a logical error if received. |
| >Pre-emption Capability | M |  | ENUMERATED(shall not trigger pre-emption, may trigger pre-emption) | Desc.: This IE indicates the pre-emption capability of the request on other E-RABs Usage: The E-RAB shall not pre-empt other E-RABs or, the E-RAB may pre-empt other E-RABs The Pre-emption Capability indicator applies to the allocation of resources for an E-RAB and as such it provides the trigger to the pre-emption procedures/processes of the eNB. |
| >Pre-emption Vulnerability | M |  | ENUMERATED(not pre-emptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the E-RAB to preemption of other E-RABs. Usage: The E-RAB shall not be pre-empted by other E-RABs or the E-RAB may be pre-empted by other RABs. Pre-emption Vulnerability indicator applies for the entire duration of the E-RAB, unless modified and as such indicates whether the E-RAB is a target of the pre-emption procedures/processes of the eNB. |

#### 9.2.1.61	Time to wait

This IE defines the minimum allowed waiting times.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time to wait | M |  | ENUMERATED(1s, 2s, 5s, 10s, 20s, 60s) |  |

#### 9.2.1.62	CSG Id

This information element indicates the identifier of the Closed Subscriber Group, as defined in 3GPP TS 23.003 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSG Id | M |  | BIT STRING (SIZE (27)) |  |

#### 9.2.1.63	CSG Id List

Void.

#### 9.2.1.64	MS Classmark 2

The coding of this element is described in 3GPP TS 48.008 [23].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MS Classmark 2 | M |  | OCTET STRING | Coded as the value part of the Classmark Information Type 2 IE defined in 3GPP TS 48.008 [23]. |

#### 9.2.1.65	MS Classmark 3

The coding of this element is described in 3GPP TS 48.008 [23].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MS Classmark 3 | M |  | OCTET STRING | Coded as the value part of the Classmark Information Type 3 IE defined in 3GPP TS 48.008 [23]. |

#### 9.2.1.66	Cell Type

The cell type provides the cell coverage area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Size | M |  | ENUMERATED (verysmall, small, medium, large, …) |  | - | - |

#### 9.2.1.67	Old BSS to New BSS Information

This container is used to transparently pass radio related information between the handover source and the handover target through the EPC. This container is used for inter 3GPP RAT handovers from SAE/LTE to GERAN A/Gb mode.

This IE is defined in 3GPP TS 48.008 [23].

#### 9.2.1.68	Layer 3 Information

This container is used to transparently pass radio related information between the handover target and the handover source through the EPC. This container is used for inter 3GPP RAT handovers from SAE/LTE to GERAN A/Gb mode.

This IE is defined in 3GPP TS 48.008 [23].

#### 9.2.1.69	E-UTRAN Round Trip Delay Estimation Info

This IE contains the information to assist target HRPD access with the acquisition of the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-UTRAN Round Trip Delay Estimation Info | M |  | INTEGER (0..2047) | Includes the Round Trip Delay between the eNB and the UE. The unit is 16Ts (see subclause 4.2.3 in MFA TS 36.213 [26]). |

#### 9.2.1.70	Broadcast Cancelled Area List

The Broadcast Cancelled Area List IE indicates the areas where broadcast was stopped successfully.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Broadcast Cancelled Area | M |  |  |  |
| >CID Cancelled |  |  |  |  |
| >>Cell ID Cancelled |  | 1 .. <maxnoofCellID> |  |  |
| >>>E-CGI | M |  | 9.2.1.38 |  |
| >>>Number of Broadcasts | M |  | 9.2.1.71 |  |
| >TAI Cancelled |  |  |  |  |
| >>TAI Cancelled |  | 1 .. <maxnoofTAIforWarning > |  |  |
| >>>TAI | M |  | 9.2.3.16 |  |
| >>>Cancelled Cell in TAI List |  | 1 .. <maxnoofCellinTAI> |  |  |
| >>>>E-CGI | M |  |  |  |
| >>>>Number of Broadcasts | M |  | 9.2.1.71 |  |
| >Emergency Area Cancelled |  |  |  |  |
| >>Emergency Area ID Cancelled |  | 1 .. <maxnoofEmergencyAreaID> |  |  |
| >>>Emergency Area ID | M |  | 9.2.1.47 |  |
| >>>Cancelled Cell in Emergency Area ID List |  | 1 .. <maxnoofCellinEAI> |  |  |
| >>>>E-CGI | M |  |  |  |
| >>>>Number of Broadcasts | M |  | 9.2.1.71 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellID | Maximum no. of Cell ID subject for warning message broadcast. Value is 65535. |
| maxnoofTAIforWarning | Maximum no. of TAI subject for warning message broadcast. Value is 65535. |
| maxnoofEmergencyAreaID | Maximum no. of Emergency Area ID subject for warning message broadcast. Value is 65535. |
| maxnoofCellinTAI | Maximum no. of Cell ID within a TAI. Value is 65535. |
| maxnoofCellinEAI | Maximum no. of Cell ID within an Emergency Area. Value is 65535. |

#### 9.2.1.71	Number of Broadcasts

The Number of Broadcasts IE indicates the number of times that a particular message has been broadcast in a given warning area.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number of Broadcasts | M |  | INTEGER(0..65535) | This IE is set to ‘0’ if valid results are not known or not available. It is set to 65535 if the counter results have overflown. |

#### 9.2.1.72	Concurrent Warning Message Indicator

The Concurrent Warning Message Indicator IE indicates to eNB that the received warning message is a new message to be scheduled for concurrent broadcast with any other ongoing broadcast of warning messages.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Concurrent Warning Message Indicator | M |  | ENUMERATED(true) | This IE is used to identify a PWS type warning system which allows the broadcast of multiple concurrent warning messages over the radio. |

#### 9.2.1.73	CSG Membership Status

This element indicates the membership status of the UE to a particular CSG.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSG Membership Status | M |  | ENUMERATED (member, not-member) |  |

#### 9.2.1.74	Cell Access Mode

This element indicates the access mode of the cell accessed by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Access Mode | M |  | ENUMERATED (hybrid, …) |  |

#### 9.2.1.75	Extended Repetition Period

The Extended Repetition Period IE indicates the periodicity of the warning message to be broadcast.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Extended Repetition Period | M |  | INTEGER (4096..217-1) | The Extended Repetition Period IE is used if the Repetition Period has a value larger than 4095. Unit [second]. |

#### 9.2.1.76	Data Forwarding Not Possible

This information element indicates that the MME decided that the corresponding E-RAB bearer will not be subject to data forwarding.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Forwarding Not Possible | M |  | ENUMERATED (Data forwarding not possible, …) |  |

#### 9.2.1.77	PS Service Not Available

This IE indicates that the UE is not available for the PS service in the target cell in case of SRVCC to GERAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PS Service Not Available | M |  | ENUMERATED (PS service not Available, …) |  |

#### 9.2.1.78	Paging Priority

This element indicates the paging priority for paging a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Priority | M |  | ENUMERATED (PrioLevel1, PrioLevel2, PrioLevel3, PrioLevel4, PrioLevel5, PrioLevel6, PrioLevel7, PrioLevel8, …) | Lower value codepoint indicates higher priority. |

#### 9.2.1.79	Relay Node Indicator

This element indicates a relay node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Relay Node Indicator | M |  | ENUMERATED (true, …) |  |

#### 9.2.1.80	Correlation ID

This information element is the GTP Tunnel Endpoint Identifier or GRE key to be used for the user plane transport between eNB and the L-GW described in 3GPP TS 23.401 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation ID | M |  | OCTET STRING (SIZE(4)) |  |

#### 9.2.1.81	MDT Configuration

The IE defines the MDT configuration parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED(Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…, Logged MBSFN MDT) |  | - | - |
| CHOICE Area Scope of MDT | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for MDT |  | 1 .. <maxnoofCellIDforMDT> |  |  |  | - |
| >>>E-CGI | M |  | 9.2.1.38 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |  | - |
| >>>TAC | M |  | 9.2.3.7 | The TAI is derived using the current serving PLMN. | - | - |
| >PLMN Wide |  |  | NULL |  | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  | 9.2.3.16 |  | - | - |
| CHOICE MDT Mode | M |  |  |  | - | - |
| >Immediate MDT |  |  |  |  |  | - |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in 3GPP TS 37.320 [31]. First Bit = M1, Second Bit= M2, Third Bit = M3, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - | - |
| >>M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, …, A2event-triggered periodic) | This IE shall be ignored if the Measurements to Activate IE has the first bit set to “0”. | - | - |
| >>M1 Threshold Event A2 | C-ifM1A2trigger |  |  | Included in case of event-triggered or event-triggered periodic reporting for measurement M1. | - | - |
| >>>CHOICE Threshold | M |  |  |  | - | - |
| >>>>RSRP |  |  |  |  |  | - |
| >>>>>Threshold RSRP | M |  | INTEGER (0..97) | This IE is defined in MFA TS 36.331 [16]. | - | - |
| >>>>RSRQ |  |  |  |  |  | - |
| >>>>>Threshold RSRQ | M |  | INTEGER (0..34) | This IE is defined in MFA TS 36.331 [16]. | - | - |
| >>M1 Periodic reporting | C-ifperiodicMDT |  |  | Included in case of periodic or event-triggered periodic reporting for measurement M1. | - | - |
| >>>Report interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60) | This IE is defined in MFA TS 36.331 [16]. | - | - |
| >>>Report amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity) | Number of reports. | - | - |
| >>M3 Configuration | C-ifM3 |  | 9.2.1.86 |  | YES | ignore |
| >>M4 Configuration | C-ifM4 |  | 9.2.1.87 |  | YES | ignore |
| >>M5 Configuration | C-ifM5 |  | 9.2.1.88 |  | YES | ignore |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in 3GPP TS 37.320 [31]. First Bit = GNSS Second Bit = E-CID information. Other bits are reserved for future use and are ignored if received. Value “1” indicates “activate” and value “0” indicates “do not activate”. The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to “1”. | YES | ignore |
| >>M6 Configuration | C-ifM6 |  | 9.2.1.101 |  | YES | ignore |
| >>M7 Configuration | C-ifM7 |  | 9.2.1.102 |  | YES | ignore |
| >Logged MDT |  |  |  |  |  | - |
| >>Logging interval | M |  | ENUMERATED (1.28, 2.56, 5.12, 10.24, 20.48, 30.72, 40.96 and 61.44) | This IE is defined in MFA TS 36.331 [16]. Unit: [second]. | - | - |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90 and 120) | This IE is defined in MFA TS 36.331 [16]. Unit: [minute]. | - | - |
| >Logged MBSFN MDT |  |  |  |  | YES | ignore |
| >>Logging interval | M |  | ENUMERATED (1.28, 2.56, 5.12, 10.24, 20.48, 30.72, 40.96 and 61.44) | This IE is defined in MFA TS 36.331 [16]. Unit: [second]. | - | - |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90 and 120) | This IE is defined in MFA TS 36.331 [16]. Unit: [minute]. | - | - |
| >>MBSFN-ResultToLog | O |  | MBSFN-ResultToLog 9.2.1.94 |  | - | - |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifM1A2trigger | This IE shall be present if the Measurements to Activate IE has the first bit set to “1” and the M1 Reporting Trigger IE is set to “A2event-triggered” or to “A2event-triggered periodic”. |
| ifperiodicMDT | This IE shall be present if the M1 Reporting Trigger IE is set to “periodic”, or to “A2event-triggered periodic”. |
| ifM3 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to “1”. |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to “1”. |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to “1”. |

#### 9.2.1.82	MME Relay Support Indicator

This element is set by the MME to advertise its support of Relay functionalities.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MME Relay Support Indicator | M |  | ENUMERATED (true, ...) |  |

#### 9.2.1.83	Management Based MDT Allowed

This information element is used by the eNB to allow selection of the UE for management based MDT as described in 3GPP TS 32.422 [10].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Management Based MDT Allowed | M |  | ENUMERATED (Allowed, ...) |  |

#### 9.2.1.84	GW Context Release Indication

This information element is set by the eNB to provide an indication that the MME may release any resources related to the signalled S1 UE context (see MFA TS 36.300 [14]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GW Context Release Indication | M |  | ENUMERATED (true, …) | This IE indicates to the MME that the eNB has successfully performed an X2 HO for the UE to a target eNB. |

#### 9.2.1.85	Voice Support Match Indicator

This information element is set by the eNB to provide an indication whether the UE radio capabilities are compatible with the network configuration for voice continuity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Voice Support Match Indicator | M |  | ENUMERATED (Supported, Not Supported …) |  |

#### 9.2.1.86	M3 Configuration

This IE defines the parameters for M3 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M3 Collection Period | M |  | ENUMERATED (ms100, ms1000, ms10000, …) |  |

#### 9.2.1.87	M4 Configuration

This IE defines the parameters for M4 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M4 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M4 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.2.1.88	M5 Configuration

This IE defines the parameters for M5 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M5 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M5 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.2.1.89	MDT PLMN List

The purpose of the MDT PLMN List IE is to provide the list of PLMN allowed for MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT PLMN List |  | 1..<maxnoofMDTPLMNs> |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | Maximum no. of PLMNs in the MDT PLMN list. Value is 16. |

#### 9.2.1.90	COUNT Value Extended

This IE contains a PDCP sequence number and a hyper frame number in case of 15 bit long PDCP-SN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP-SN Extended | M |  | INTEGER (0..32767) |  | - | - |
| HFN Modified | M |  | INTEGER (0..131071) |  | - | - |

#### 9.2.1.91	Kill-all Warning Messages Indicator

The Kill-all Warning Messages Indicator IE indicates to the eNB to stop all already ongoing broadcast of warning messages in the eNB or in an area.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Kill-all Warning Message Indicator | M |  | ENUMERATED(true) |  |

#### 9.2.1.92	LHN ID

The LHN ID IE is used to indicate the LHN ID of the eNB, as defined in 3GPP TS 23.003 [21].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Local Home Network ID | M |  | OCTET STRING (SIZE (32..256)) | Identifies the Local Home Network. |

#### 9.2.1.93	User Location Information

This IE provides location information of a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| User Location Information |  |  |  |  |
| >E-UTRAN CGI | M |  | 9.2.1.38 |  |
| >TAI | M |  | 9.2.3.16 |  |

#### 9.2.1.94	MBSFN-ResultToLog

This IE provides information on the MBMS area in which the MBSFN MDT result is logged.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MBSFN-ResultToLog |  | 1..<maxnoofMBSFNAreaMDT > |  |  |
| >MBSFN-AreaId | O |  | INTEGER (0..255) |  |
| >CarrierFreq | M |  | EARFCN 9.2.1.95 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSFNAreaMDT | Maximum number of MBSFN areas configured for logged MBSFN MDT. Value is 8. |

#### 9.2.1.95	EARFCN

The E-UTRA Absolute Radio Frequency Channel Number defines the carrier frequency used in a cell for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| EARFCN | M |  | INTEGER (0 .. maxEARFCN, ...) | The relation between EARFCN and carrier frequency (in MHz) are defined in 3GPP TS 36.104 [39]. |

| Range bound | Explanation |
| --- | --- |
| maxEARFCN | Maximum value of EARFCNs. Value is 262143. |

#### 9.2.1.96	Expected UE Behaviour

This IE defines the behaviour of a UE with predictable activity and/or mobility behaviour, to assist the eNB in determining the optimum RRC connection time.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Expected UE Activity Behaviour | O |  | 9.2.1.97 |  |
| Expected HO Interval | O |  | ENUMERATED (sec15, sec30, sec60, sec90, sec120, sec180, long-time, ...) | Indicates the expected time interval between inter-eNB handovers. If "long-time" is included, the interval between inter-eNB handovers is expected to be longer than 180 seconds. |

#### 9.2.1.97	Expected UE Activity Behaviour

Indicates information about the expected "UE activity behaviour" as defined in 3GPP TS 23.401 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Expected Activity Period | O |  | INTEGER (1..30¦40¦50¦60¦80¦ 100¦120¦150¦180¦ 181, ...) | If this IE is set to "181" the expected activity time is longer than 180 seconds. The remaining values indicate the expected activity time in [seconds]. |
| Expected Idle Period | O |  | INTEGER (1..30¦40¦50¦60¦80¦ 100¦120¦150¦180¦ 181, ...) | If this IE is set to "181" the expected idle time is longer than 180 seconds. The remaining values indicate the expected idle time in [seconds]. |
| Source of UE Activity Behaviour Information | O |  | ENUMERATED (subscription information, statistics, ...) | If "subscription information" is indicated, the information contained in the Expected Activity Period IE and the Expected Idle Period IE, if present, is derived from subscription information. If "statistics" is indicated, the information contained in the Expected Activity Period IE and the Expected Idle Period IE, if present, is derived from statistical information. |

#### 9.2.1.98	UE Radio Capability for Paging

This IE contains paging specific UE Radio Capability information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Radio Capability for Paging | M |  | OCTET STRING | Includes either the UERadioPagingInformation message as defined in 10.2.2 of MFA TS 36.331 [16], or the UERadioPagingInformation-NB message as defined in 10.6.2 of MFA TS 36.331 [16]. |

#### 9.2.1.99	ProSe Authorized

This IE provides information on the authorization status of the UE for ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for ProSe Direct Discovery | - | - |
| ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for ProSe Direct Communication | - | - |
| ProSe UE-to-Network Relaying | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized to act as ProSe UE-to-Network Relay | YES | ignore |

#### 9.2.1.100	COUNT Value for PDCP SN Length 18

This IE contains a PDCP sequence number and a hyper frame number in case of 18 bit long PDCP-SN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP-SN Length 18 | M |  | INTEGER (0..262143) |  | - | - |
| HFN for PDCP-SN Length 18 | M |  | INTEGER (0..16383) |  | - | - |

#### 9.2.1.101	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, …) |  |
| M6 Delay Threshold | C-ifUL |  | ENUMERATED (ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, …) |  |
| M6 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

| Condition | Explanation |
| --- | --- |
| ifUL | This IE shall be present if the M6 Links to log IE is set to “uplink” or to “both-uplink-and-downlink”. |

#### 9.2.1.102	M7 Configuration

This IE defines the parameters for M7 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | INTEGER (1..60, …) | Unit: minutes |
| M7 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.2.1.103	Assistance Data for Paging

This IE provides assistance information for paging optimisation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Data for Recommended Cells | O |  | 9.2.1.104 |  |
| Assistance Data for CE capable UEs | O |  | 9.2.1.108 |  |
| Paging Attempt Information | O |  | 9.2.1.110 |  |

#### 9.2.1.104	Assistance Data for Recommended Cells

This IE provides assistance information for paging in recommended cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Cells for Paging | M |  | 9.2.1.106 |  |

#### 9.2.1.105	Information on Recommended Cells and eNBs for Paging

This IE provides information on recommended cells and eNBs for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Cells for Paging | M |  | 9.2.1.106 |  |
| Recommended eNBs for Paging | M |  | 9.2.1.107 |  |

#### 9.2.1.106	Recommended Cells for Paging

This IE contains the recommended cells for paging.

This IE is transparent to the EPC.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Recommended Cell List |  | 1 |  |  |
| >Recommended Cell Item IEs |  | 1 .. <maxnoofRecommendedCells> |  | Includes visited and non-visited cells, where visited cells are listed in the order the UE visited them with the most recent cell being the first in the list. Non-visited cells are included immediately after the visited cell they are associated with. |
| >>E-UTRAN CGI | M |  | 9.2.1.38 |  |
| >>Time Stayed in Cell | O |  | INTEGER (0..4095) | This is included for visited cells and indicates the time a UE stayed in a cell in seconds. If the UE stays in a cell more than 4095 seconds, this IE is set to 4095. |

| Range bound | Explanation |
| --- | --- |
| maxnoofRecommendedCells | Maximum no. of recommended Cells, the maximum value is 16. |

#### 9.2.1.107	Recommended eNBs for Paging

This IE contains recommended targets for paging.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Recommended eNB List |  |  |  |  |
| >Recommended eNB Item IEs |  | 1 .. <maxnoofRecommendedeNBs> |  | Includes visited and non-visited eNBs, where visited eNBs are listed in the order the UE visited them with the most recent eNB being the first in the list. Non-visited eNBs are included after the visited eNB they are associated with. |
| >>Choice MME Paging Target |  |  |  | The MME paging target is either an eNB identity or a TAI as specified in MFA TS 36.300 [14]. |
| >>>eNB |  |  |  |  |
| >>>>Global eNB ID | M |  | 9.2.1.37 |  |
| >>>TAI |  |  |  |  |
| >>>>TAI | M |  | 9.2.3.16 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRecommendedeNBs | Maximum no. of recommended eNBs, the maximum value is 16. |

#### 9.2.1.108	Assistance Data for CE capable UEs

This IE provides information for paging for CE capable UEs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Identifier and Coverage Enhancement Level | M |  | 9.2.1.109 |  |

#### 9.2.1.109	Cell Identifier and Coverage Enhancement Level

This IE provides information for paging CE capable UEs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  |
| Coverage Enhancement Level | M |  | OCTET STRING | Includes either the UEPagingCoverageInformation message as defined in 10.2.2 of MFA TS 36.331 [16], or the UEPagingCoverageInformation-NB message as defined in 10.6.2 of MFA TS 36.331 [16]. |

#### 9.2.1.110	Paging Attempt Information

This IE includes information related to the paging count over S1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Attempt Count | M |  | INTEGER (1..16,...) | Shall be set as specified in MFA TS 36.300 [14]. |
| Intended Number of Paging Attempts | M |  | INTEGER (1..16,…) | Intended number of paging attempts (see MFA TS 36.300 [14]). |
| Next Paging Area Scope | O |  | ENUMERATED (same, changed, …) | Indicates whether the paging area scope will change or not at next paging attempt. Usage specified in MFA TS 36.300 [14]. |

#### 9.2.1.111	Paging eDRX Information

This IE indicates the Paging eDRX parameters as defined in MFA TS 36.304 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging eDRX Cycle | M |  | ENUMERATED (hfhalf, hf1, hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, …) | TeDRX defined in MFA TS 36.304 [20]. Unit: [number of hyperframes]. |
| Paging Time Window | O |  | ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, …) | Unit: [1.28 second]. |

#### 9.2.1.112	UE Retention Information

This information element allows the eNB and the MME to indicate whether prior UE related contexts and signalling connections are intended to be retained.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Retention Information | M |  | ENUMERATED (ues-retained,…) |  |

#### 9.2.1.113	UE User Plane CIoT Support Indicator

This IE indicates whether User Plane CIoT EPS Optimisation as specified in 3GPP TS 23.401[11] is supported for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE User Plane CIoT Support Indicator | M |  | ENUMERATED (supported, …) |  |

#### 9.2.1.114	NB-IoT Default Paging DRX

This IE indicates the NB-IoT Default Paging DRX as defined in MFA TS 36.304 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NB-IoT Default Paging DRX | M |  | ENUMERATED(128, 256, 512, 1024, …) | Unit: [number of radioframes] |

#### 9.2.1.115	NB-IoT Paging eDRX Information

This IE indicates the NB-IoT Paging eDRX parameters as defined in MFA TS 36.304 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NB-IoT Paging eDRX Cycle | M |  | ENUMERATED (hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) | TeDRX defined in MFA TS 36.304 [20]. Unit: [number of hyperframes]. |
| NB-IoT Paging Time Window | O |  | ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, …) | Unit: [2.56 seconds] |

#### 9.2.1.116	Bearer Type

This IE is used to support Non-IP data as specified in 3GPP TS 23.401 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bearer Type | M |  | ENUMERATED (non IP, …) |  |

#### 9.2.1.117	RAT Type

This element is provided by the eNB to inform about the RAT Type.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAT Type | M |  | ENUMERATED (NB-IOT, ...) |  |

#### 9.2.1.118	CE-mode-B Support Indicator

This IE indicates whether CE-mode-B as specified in 3GPP TS 36.306 [41] is supported for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CE-mode-B Support Indicator | M |  | ENUMERATED (supported, …) |  |

#### 9.2.1.119	SRVCC Operation Not Possible

This element indicates that SRVCC operation is not possible any more.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRVCC Operation Not Possible | M |  | ENUMERATED (notPossible, …) |  |

### 9.2.2	Transport Network Layer Related IEs

#### 9.2.2.1	Transport Layer Address

This information element is an IP address.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Address | M |  | BIT STRING (SIZE(1..160, …)) | The Radio Network Layer is not supposed to interpret the address information. It should pass it to the transport layer for interpretation. For details on the Transport Layer Address, see 3GPP TS 36.414 [12]. |

#### 9.2.2.2	GTP-TEID

This information element is the GTP Tunnel Endpoint Identifier to be used for the user plane transport between eNB and the serving gateway.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GTP-TEID | M |  | OCTET STRING (SIZE(4)) | For details and range, see 3GPP TS 29.281 [32]. |

#### 9.2.2.3	Tunnel Information

The Tunnel Information IE indicates the transport layer address and UDP port number.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Address | M |  | 9.2.2.1 | HeNB’s Transport Layer Address. |
| UDP Port Numbers | O |  | OCTET STRING (SIZE(2)) | UDP Port Numbers if NAT/NAPT is deployed in the BBF access network. |

### 9.2.3	NAS Related IEs

#### 9.2.3.1	LAI

This information element is used to uniquely identify a Location Area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LAI |  |  |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |
| >LAC | M |  | OCTET STRING (SIZE(2)) | 0000 and FFFE not allowed. |

#### 9.2.3.2	RAC

This information element is used to identify a Routing Area within a Location Area. It is used for PS services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAC | M |  | OCTET STRING (SIZE(1)) |  |

#### 9.2.3.3	MME UE S1AP ID

The MME UE S1AP ID uniquely identifies the UE association over the S1 interface within the MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MME UE S1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.2.3.4	eNB UE S1AP ID

The eNB UE S1AP ID uniquely identifies the UE association over the S1 interface within the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| eNB UE S1AP ID | M |  | INTEGER (0 .. 224 -1) |  |

#### 9.2.3.5	NAS-PDU

This information element contains an EPC – UE or UE – EPC message that is transferred without interpretation in the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NAS-PDU | M |  | OCTET STRING |  |

#### 9.2.3.6	S-TMSI

The Temporary Mobile Subscriber Identity is used for security reasons, to hide the identity of a subscriber.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MMEC | M |  | 9.2.3.12 |  |  |  |
| M-TMSI | M |  | OCTET STRING (SIZE (4)) | M-TMSI is unique within MME that allocated it. |  |  |

#### 9.2.3.7	TAC

This information element is used to uniquely identify a Tracking Area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAC | M |  | OCTET STRING (SIZE (2)) |  |

#### 9.2.3.8	PLMN Identity

This information element indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE (3)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n -The PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |

#### 9.2.3.9	GUMMEI

This information element indicates the globally unique MME identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GUMMEI |  |  |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |
| >MME Group ID | M |  | OCTET STRING (SIZE(2)) |  |
| >MME code | M |  | 9.2.3.12 |  |

#### 9.2.3.10	UE Identity Index value

The UE Identity Index value IE is used by the eNB to calculate the Paging Frame (see MFA TS 36.304 [20]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | BIT STRING (SIZE(10)) | Coded as specified in MFA TS 36.304 [20]. |

#### 9.2.3.11	IMSI

This information element contains an International Mobile Subscriber Identity, which is commonly used to identify the UE in the CN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IMSI | M |  | OCTET STRING (SIZE (3..8)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bit 4 to 1 of octet n encoding digit 2n-1 - bit 8 to 5 of octet n encoding digit 2n -Number of decimal digits shall be from 6 to 15 starting with the digits from the PLMN identity. When the IMSI is made of an odd number of digits, the filler digit shall be added at the end to make an even number of digits of length 2N. The filler digit shall then be consequently encoded as bit 8 to 5 of octet N. |

#### 9.2.3.12	MMEC

This information element represents the MME Code to uniquely identify an MME within an MME pool area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MMEC | M |  | OCTET STRING (SIZE (1)) |  |

#### 9.2.3.13	UE Paging Identity

This IE represents the Identity with which the UE is paged.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE Paging Identity | M |  |  |  |
| >S-TMSI |  |  |  |  |
| >>S-TMSI | M |  | 9.2.3.6 |  |
| >IMSI |  |  |  |  |
| >>IMSI | M |  | 9.2.3.11 |  |

#### 9.2.3.14	DL Forwarding

This information element indicates that the E-RAB is proposed for forwarding of downlink packets.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Forwarding |  |  |  |  |
| >DL Forwarding | M |  | ENUMERATED (DL forwarding proposed, …) |  |

#### 9.2.3.15	Direct Forwarding Path Availability

The availability of a direct forwarding path shall be determined by the source eNB. The EPC behaviour on receipt of this IE is specified in 3GPP TS 23.401 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Direct Forwarding Path Availability | M |  | ENUMERATED (Direct Path Available, …) |  |

#### 9.2.3.16	TAI

This information element is used to uniquely identify a Tracking Area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAI |  |  |  |  |
| >PLMN Identity | M |  | 9.2.3.8 |  |
| >TAC | M |  | 9.2.3.7 |  |

#### 9.2.3.17	Relative MME Capacity

This IE indicates the relative processing capacity of an MME with respect to the other MMEs in the pool in order to load-balance MMEs within a pool defined in 3GPP TS 23.401 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Relative MME Capacity | M |  | INTEGER (0..255) |  |

#### 9.2.3.18	UE S1AP ID pair

This IE contains a pair of UE S1AP identities.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | - | - |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | - | - |

#### 9.2.3.19	Overload Response

The Overload Response IE indicates the required behaviour of the eNB in an overload situation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Overload Response | M |  |  |  |
| >Overload Action |  |  |  |  |
| >>Overload Action | M |  | 9.2.3.20 |  |

#### 9.2.3.20	Overload Action

The Overload Action IE indicates which signalling traffic is subject to rejection by the eNB in an MME overload situation as defined in 3GPP TS 23.401 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Overload Action | M |  | ENUMERATED (Reject RRC connection establishments for non-emergency MO DT, Reject RRC connection establishments for Signalling, Permit Emergency Sessions and mobile terminated services only, …, Permit High Priority Sessions and mobile terminated services only, Reject delay tolerant access) |  |

#### 9.2.3.21	CS Fallback Indicator

The IE indicates that a fallback to the CS domain is needed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CS Fallback Indicator | M |  | ENUMERATED(CS Fallback required, … , CS Fallback High Priority) |  |

#### 9.2.3.22	CN Domain

This IE indicates whether Paging is originated from the CS or PS domain.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Domain | M |  | ENUMERATED(PS, CS) |  |

#### 9.2.3.23	RIM Transfer

This IE contains the RIM Information (e.g. NACC information) and additionally in uplink transfers the RIM routing address of the destination of this RIM information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RIM Transfer |  |  |  |  |
| >RIM Information | M |  | 9.2.3.24 |  |
| >RIM Routing Address | O |  | 9.2.3.25 |  |

#### 9.2.3.24	RIM Information

This IE contains the RIM Information (e.g., NACC information) i.e., the BSSGP RIM PDU from the RIM application part contained in the eNB, or the BSSGP RIM PDU to be forwarded to the RIM application part in the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RIM Information |  |  |  |  |
| >RIM Information | M |  | OCTET STRING | Contains the BSSGP RIM PDU as defined in 3GPP TS 48.018 [18]. |

#### 9.2.3.25	RIM Routing Address

This IE identifies the destination node where the RIM Information needs to be routed by the CN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE RIM Routing Address | M |  |  |  |  |  |
| >GERAN-Cell-ID |  |  |  |  | - |  |
| >>LAI | M |  | 9.2.3.1 |  | - |  |
| >>RAC | M |  | 9.2.3.2 |  | - |  |
| >>CI | M |  | OCTET STRING (SIZE(2)) |  | - |  |
| >Target RNC-ID |  |  |  |  | - |  |
| >>LAI | M |  | 9.2.3.1 |  | - |  |
| >>RAC | O |  | 9.2.3.2 |  | - |  |
| >>RNC-ID | M |  | INTEGER (0..4095) | If the Extended RNC-ID IE is included in the Target ID IE, the RNC-ID IE shall be ignored. | - |  |
| >>Extended RNC-ID | O |  | 9.2.1.14 | The Extended RNC-ID IE shall be used if the RNC identity has a value larger than 4095. | - |  |
| >eHRPD Sector ID |  |  |  |  | - |  |
| >>eHRPD Sector ID | M |  | OCTET STRING (SIZE(16)) | Contains the eHRPD Sector ID as defined in 3GPP2 C.S0024-B [27] sub-section 13.9. | - |  |

#### 9.2.3.26	SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, and additionally includes the eNB identifier of the destination of this configuration information and the eNB identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SON Configuration Transfer |  |  |  |  |  |  |
| >Target eNB-ID | M |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.1.37 |  |  |  |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  |  |  |
| >Source eNB-ID | M |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.1.37 |  |  |  |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  |  |  |
| >SON Information | M |  | 9.2.3.27 |  |  |  |
| >X2 TNL Configuration Info | C-ifSONInformationRequest |  | 9.2.3.29 | Source eNB X2 TNL Configuration Info. | YES | ignore |
| >Synchronisation Information | C-if Activate Muting |  | 9.2.3.42 | Information on cell selected as source of synchronisation and aggressor cells. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSONInformationRequest | This IE shall be present if the SON Information IE contains the SON Information Request IE set to “X2TNL Configuration Info” |
| ifActivateMuting | This IE shall be present if the SON Information IE contains the SON Information Request IE set to “Activate Muting” |

#### 9.2.3.27	SON Information

This IE identifies the nature of the configuration information transferred, i.e., a request, a reply or a report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE SON Information | M |  |  |  |  |  |
| >SON Information Request |  |  |  |  |  |  |
| >>SON Information Request | M |  | ENUMERATED(X2 TNL Configuration Info, …, Time synchronisation Info, Activate Muting, Deactivate Muting) |  | - |  |
| >SON Information Reply |  |  |  |  |  |  |
| >>SON Information Reply | M |  | 9.2.3.28 |  | - |  |
| >SON Information Report |  |  |  |  |  |  |
| >>SON Information Report | M |  | 9.2.3.39 |  | YES | ignore |

#### 9.2.3.28	SON Information Reply

This IE contains the configuration information to be replied to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SON Information Reply |  |  |  |  |  |  |
| >X2 TNL Configuration Info | O |  | 9.2.3.29 |  |  |  |
| >Time Synchronisation Info | O |  | 9.2.3.34 |  | YES | ignore |
| >Muting Pattern Information | O |  | 9.2.3.41 |  | YES | ignore |

#### 9.2.3.29	X2 TNL Configuration Info

The X2 TNL Configuration Info IE is used for signalling X2 TNL Configuration information for automatic X2 SCTP association establishment.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| eNB X2 Transport Layer Addresses |  | 1 .. <maxnoofeNBX2TLAs> |  |  |  |  |
| >Transport Layer Address | M |  | 9.2.2.1 | Transport Layer Addresses for X2 SCTP end-point. |  |  |
| eNB X2 Extended Transport Layer Addresses |  | 0 .. <maxnoofeNBX2ExtTLAs> |  |  | YES | ignore |
| >IP-Sec Transport Layer Address | O |  | 9.2.2.1 | Transport Layer Addresses for IP-Sec end-point. | - | - |
| >eNB GTP Transport Layer Addresses |  | 0 .. <maxnoofeNBX2GTPTLAs> |  |  | - | - |
| >>GTP Transport Layer Address | M |  | 9.2.2.1 | GTP Transport Layer Addresses for GTP end-points (used for data forwarding over X2). | - | - |
| eNB Indirect X2 Transport Layer Addresses |  | 0 .. <maxnoofeNBX2TLAs> |  |  | YES | ignore |
| >Transport Layer Address | O |  | 9.2.2.1 | Transport Layer Addresses for Indirect X2 SCTP end-point. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofeNBX2TLAs | Maximum no. of eNB X2 Transport Layer Addresses for an SCTP end-point. Value is 2. |
| maxnoofeNBX2ExtTLAs | Maximum no. of eNB X2 Extended Transport Layer Addresses in the message. Value is 16. |
| maxnoofeNBX2GTPTLAs | Maximum no. of eNB X2 GTP Transport Layer Addresses for an GTP end-point in the message. Value is 16. |

#### 9.2.3.30	NAS Security Parameters from E-UTRAN

The purpose of the NAS Security Parameters from E-UTRAN IE is to provide security related parameters for I-RAT handovers from E-UTRAN via the eNB to the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NAS Security Parameters from E-UTRAN | M |  | OCTET STRING | Coded as the value part of NAS security parameters from E-UTRA IE defined in MFA TS 24.301 [24]. |

#### 9.2.3.31	NAS Security Parameters to E-UTRAN

The purpose of the NAS Security Parameters to E-UTRAN IE is to provide security related parameters for I-RAT handovers to E-UTRAN via the RNC or BSS to the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NAS Security Parameters to E-UTRAN | M |  | OCTET STRING | Coded as the value part of NAS security parameters to E-UTRA IE defined in MFA TS 24.301 [24]. |

#### 9.2.3.32	LPPa-PDU

This information element contains an eNB – E-SMLC or E-SMLC – eNB message that is transferred without interpretation in the MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LPPa-PDU | M |  | OCTET STRING |  |

#### 9.2.3.33	Routing ID

This information element is used to identify an E-SMLC within the EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Routing ID | M |  | INTEGER (0..255) |  |

#### 9.2.3.34	Time Synchronisation Info

The Time Synchronisation Info IE is used for signalling stratum level, synchronisation status and muting availability for over-the-air synchronisation using network listening.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Synchronisation Info |  |  |  |  |  |  |
| >Stratum Level | M |  | INTEGER (0..3, …) |  |  |  |
| >Synchronisation status | M |  | ENUMERATED(Synchronous, Asynchronous, …) |  |  |  |
| >Muting Availability Indication | O |  | ENUMERATED (Available, Unavailable, …) | Indicates availability of muting activation. | YES | ignore |

#### 9.2.3.35	Void

#### 9.2.3.36	Traffic Load Reduction Indication

The Traffic Load Reduction Indication IE indicates the percentage of the type of traffic relative to the instantaneous incoming rate at the eNB, as indicated in the Overload Action IE, to be rejected.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Traffic Load Reduction Indication | M |  | INTEGER (1..99) |  |

#### 9.2.3.37	Additional CS Fallback Indicator

The IE indicates whether the restrictions contained in the Handover Restriction List IE apply or not to the CS Fallback High Priority call.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Additional CS Fallback Indicator | M |  | ENUMERATED(no restriction, restriction, ...) |  |

#### 9.2.3.38	Masked IMEISV

This information element contains the IMEISV value with a mask, to identify a terminal model without identifying an individual Mobile Equipment.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Masked IMEISV | M |  | BIT STRING (SIZE (64)) | Coded as the International Mobile station Equipment Identity and Software Version Number (IMEISV) defined in 3GPP TS 23.003 [21] with the last 4 digits of the SNR masked by setting the corresponding bits to 1. The first to fourth bits correspond to the first digit of the IMEISV, the fifth to eighth bits correspond to the second digit of the IMEISV, and so on. |

#### 9.2.3.39	SON Information Report

This IE contains the configuration information to be transferred to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >RLF Report Information |  |  |  |  |
| >>RLF Report Information | M |  | 9.2.3.40 |  |

#### 9.2.3.40	RLF Report Information

This IE contains the RLF report information to be transferred to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE RLF Report Container | M |  | OCTET STRING | rlf-Report-r9 contained in UEInformationResponse message as defined in MFA TS 36.331 [16]. |
| UE RLF Report Container for extended bands | O |  | OCTET STRING | rlf-Report-v9e0 contained in the UEInformationResponse message (see MFA TS 36.331 [16]) |

#### 9.2.3.41	Muting Pattern Information

This information element contains muting pattern information that can be used for over-the-air synchronisation using network listening.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Muting Pattern Period | M |  | ENUMERATED (0, 1280, 2560, 5120, 10240, …) | Period for repetition of muted subframe in milliseconds. Value ‘0’ indicates that the muting request is not fulfilled. |
| Muting Pattern Offset | O |  | INTEGER (0..10239,…) | Offset in number of subframes of the muting pattern starting from subframe 0 in a radio frame where SFN = 0. If this IE is not present, the receiving eNB may consider the requested muting pattern offset in the former request has been accepted. |

#### 9.2.3.42	Synchronisation Information

This information element contains information concerning the cell selected as source of synchronisation signal by the sending eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Source Stratum Level | O |  | INTEGER (0..3, …) | Stratum Level of cell selected as synchronisation source. The range of this IE is limited to 0..2. |
| Listening Subframe Pattern | O |  | 9.2.3.43 | Subframe pattern where the Reference Signals can be detected for synchronisation. |
| Aggressor Cell List |  | 0..1 |  | List of cells for which the muting pattern need to be activated. |
| >Aggressor E-CGI List |  | 1..<maxnoofCellsineNB> |  |  |
| >>E-CGI | M |  | 9.2.1.38 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.2.3.43	Listening Subframe Pattern

This information element contains information concerning the pattern of subframes where the reference signals can be detected for the purpose of over the air synchronisation via network listening.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Pattern Period | M |  | ENUMERATED (1280, 2560, 5120, 10240, …) | Period in milliseconds for repetition of the subframe where reference signals are available. |
| Pattern Offset | M |  | INTEGER (0..10239,…) | Offset in number of subframes of the reference signals starting from subframe 0 in a radio frame where SFN = 0. |

#### 9.2.3.44	MME Group ID

This information element contains information concerning the MME Group ID that identifies a group of MME’s.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MME Group ID | M |  | OCTET STRING (SIZE(2)) | The MME Group ID is defined in 3GPP TS 23.003 [21] |

#### 9.2.3.45	Additional GUTI

This information element contains DCN related information to for identification of a CN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GUMMEI | M |  | 9.2.3.9 |  |
| M-TMSI | M |  | OCTET STRING (SIZE (4)) |  |

#### 9.2.3.46	Extended UE Identity Index Value

The Extended UE Identity Index Value IE is used by the eNB to calculate the paging resources to be used for the UE, as defined in MFA TS 36.304 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended UE Identity Index Value | M |  | BIT STRING (SIZE(14)) | Corresponds to the UE_ID used to determine the Paging Narrowband as specified in MFA TS 36.304 [20]. |

#### 9.2.3.47	NB-IoT UE Identity Index Value

The NB-IoT UE Identity Index Value IE is used by the eNB to calculate the paging resources to be used for the UE, as defined in MFA TS 36.304 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NB-IoT UE Identity Index Value | M |  | BIT STRING (SIZE(12)) | Coded as specified in MFA TS 36.304 [20]. |

#### 9.2.3.47MF1			PSP Info

The PLMN PSP Info IE is used by the eNB to announce the available PLMN PSPs, as defined in MFA TS MF.202 [MF1].

The OID PSP Info IE is used by the eNB to announce the available OID PSPs, as defined in MFA TS MF.202 [MF1].

The Hash PSP Info IE is used by the eNB to announce the available Hash PSPs, as defined in MFA TS MF.202 [MF1].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE PSP Info | M |  |  |  |
| >PLMN PSP Info |  |  |  |  |
| >>PLMN PSP ID | M |  | BIT STRING (SIZE(24)) | Identity that eNB should announce as a PLMN based PSP as specified in MFA TS MF.202 [MF1]. |
| >> PLMN PSP S2a Support Indicator | O |  | ENUMERATED (true) | Indicates if PLMN PSP supports S2a home anchoring |
| >OID PSP Info |  |  |  |  |
| >>OID PSP ID | M |  | BIT STRING (SIZE(24)) | Identity that eNB should announce as a 24 bit OID based PSP as specified in MFA TS MF.202 [MF1]. |
| >Hash PSP Info |  |  |  |  |
| >>Hash PSP ID | M |  | BIT STRING (SIZE(24)) | Identity that eNB should announce as a hash based PSP (i.e., short-form PSP-IDs) as specified in MFA TS MF.202 [MF1]. |

## 9.3	Message and Information Element Abstract Syntax (with ASN.1)

### 9.3.0	General

S1AP ASN.1 definition conforms to ITU-T Rec. X.691 [4], ITU-T Rec. X.680 [5] and ITU-T Rec. X.681 [6].

The ASN.1 definition specifies the structure and content of S1AP messages. S1AP messages can contain any IEs specified in the object set definitions for that message without the order or number of occurrence being restricted by ASN.1. However, for this version of the standard, a sending entity shall construct a S1AP message according to the PDU definitions module and with the following additional rules:

- IEs shall be ordered (in an IE container) in the order they appear in object set definitions.

- Object set definitions specify how many times IEs may appear. An IE shall appear exactly once if the presence field in an object has value "mandatory". An IE may appear at most once if the presence field in an object has value "optional" or "conditional". If in a tabular format there is multiplicity specified for an IE (i.e., an IE list) then in the corresponding ASN.1 definition the list definition is separated into two parts. The first part defines an IE container list where the list elements reside. The second part defines list elements. The IE container list appears as an IE of its own. For this version of the standard an IE container list may contain only one kind of list elements.

NOTE:	In the above “IE” means an IE in the object set with an explicit ID. If one IE needs to appear more than once in one object set, then the different occurrences will have different IE IDs.

If a S1AP message that is not constructed as defined above is received, this shall be considered as Abstract Syntax Error, and the message shall be handled as defined for Abstract Syntax Error in subclause 10.3.6.

Subclause 9.3 presents the Abstract Syntax of S1AP protocol with ASN.1. In case there is contradiction between the ASN.1 definition in this subclause and the tabular format in subclause 9.1 and 9.2, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional elements, where the tabular format shall take precedence.

### 9.3.1	Usage of private message mechanism for non-standard use

The private message mechanism for non-standard use may be used:

-	for special operator- (and/or vendor) specific features considered not to be part of the basic functionality, i.e., the functionality required for a complete and high-quality specification in order to guarantee multivendor interoperability;

-	by vendors for research purposes, e.g., to implement and evaluate new algorithms/features before such features are proposed for standardisation.

The private message mechanism shall not be used for basic functionality. Such functionality shall be standardised.

### 9.3.2	Elementary Procedure Definitions

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

S1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM S1AP-CommonDataTypes

CellTrafficTrace,

DeactivateTrace,

DownlinkUEAssociatedLPPaTransport,

DownlinkNASTransport,

DownlinkNonUEAssociatedLPPaTransport,

DownlinkS1cdma2000tunnelling,

ENBDirectInformationTransfer,

ENBStatusTransfer,

ENBConfigurationUpdate,

ENBConfigurationUpdateAcknowledge,

ENBConfigurationUpdateFailure,

ErrorIndication,

HandoverCancel,

HandoverCancelAcknowledge,

HandoverCommand,

HandoverFailure,

HandoverNotify,

HandoverPreparationFailure,

HandoverRequest,

HandoverRequestAcknowledge,

HandoverRequired,

InitialContextSetupFailure,

InitialContextSetupRequest,

InitialContextSetupResponse,

InitialUEMessage,

KillRequest,

KillResponse,

LocationReportingControl,

LocationReportingFailureIndication,

LocationReport,

MMEConfigurationUpdate,

MMEConfigurationUpdateAcknowledge,

MMEConfigurationUpdateFailure,

MMEDirectInformationTransfer,

MMEStatusTransfer,

NASNonDeliveryIndication,

OverloadStart,

OverloadStop,

Paging,

PathSwitchRequest,

PathSwitchRequestAcknowledge,

PathSwitchRequestFailure,

PrivateMessage,

Reset,

ResetAcknowledge,

S1SetupFailure,

S1SetupRequest,

S1SetupResponse,

E-RABModifyRequest,

E-RABModifyResponse,

E-RABModificationIndication,

E-RABModificationConfirm,

E-RABReleaseCommand,

E-RABReleaseResponse,

E-RABReleaseIndication,

E-RABSetupRequest,

E-RABSetupResponse,

TraceFailureIndication,

TraceStart,

UECapabilityInfoIndication,

UEContextModificationFailure,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextReleaseRequest,

UERadioCapabilityMatchRequest,

UERadioCapabilityMatchResponse,

UplinkUEAssociatedLPPaTransport,

UplinkNASTransport,

UplinkNonUEAssociatedLPPaTransport,

UplinkS1cdma2000tunnelling,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

ENBConfigurationTransfer,

MMEConfigurationTransfer,

PWSRestartIndication,

UEContextModificationIndication,

UEContextModificationConfirm,

RerouteNASRequest,

PWSFailureIndication,

UEContextSuspendRequest,

UEContextSuspendResponse,

UEContextResumeRequest,

UEContextResumeResponse,

UEContextResumeFailure,

ConnectionEstablishmentIndication

FROM S1AP-PDU-Contents

id-CellTrafficTrace,

id-DeactivateTrace,

id-downlinkUEAssociatedLPPaTransport,

id-downlinkNASTransport,

id-downlinkNonUEAssociatedLPPaTransport,

id-DownlinkS1cdma2000tunnelling,

id-eNBStatusTransfer,

id-ErrorIndication,

id-HandoverCancel,

id-HandoverNotification,

id-HandoverPreparation,

id-HandoverResourceAllocation,

id-InitialContextSetup,

id-initialUEMessage,

id-ENBConfigurationUpdate,

id-Kill,

id-LocationReportingControl,

id-LocationReportingFailureIndication,

id-LocationReport,

id-eNBDirectInformationTransfer,

id-MMEConfigurationUpdate,

id-MMEDirectInformationTransfer,

id-MMEStatusTransfer,

id-NASNonDeliveryIndication,

id-OverloadStart,

id-OverloadStop,

id-Paging,

id-PathSwitchRequest,

id-PrivateMessage,

id-Reset,

id-S1Setup,

id-E-RABModify,

id-E-RABModificationIndication,

id-E-RABRelease,

id-E-RABReleaseIndication,

id-E-RABSetup,

id-TraceFailureIndication,

id-TraceStart,

id-UECapabilityInfoIndication,

id-UEContextModification,

id-UEContextRelease,

id-UEContextReleaseRequest,

id-UERadioCapabilityMatch,

id-uplinkUEAssociatedLPPaTransport,

id-uplinkNASTransport,

id-uplinkNonUEAssociatedLPPaTransport,

id-UplinkS1cdma2000tunnelling,

id-WriteReplaceWarning,

id-eNBConfigurationTransfer,

id-MMEConfigurationTransfer,

id-PWSRestartIndication,

id-UEContextModificationIndication,

id-RerouteNASRequest,

id-PWSFailureIndication,

id-UEContextSuspend,

id-UEContextResume,

id-ConnectionEstablishmentIndication

FROM S1AP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

S1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

S1AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	S1AP-ELEMENTARY-PROCEDURE.&procedureCode		({S1AP-ELEMENTARY-PROCEDURES}),

criticality		S1AP-ELEMENTARY-PROCEDURE.&criticality			({S1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			S1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({S1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	S1AP-ELEMENTARY-PROCEDURE.&procedureCode		({S1AP-ELEMENTARY-PROCEDURES}),

criticality		S1AP-ELEMENTARY-PROCEDURE.&criticality			({S1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			S1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({S1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	S1AP-ELEMENTARY-PROCEDURE.&procedureCode		({S1AP-ELEMENTARY-PROCEDURES}),

criticality		S1AP-ELEMENTARY-PROCEDURE.&criticality			({S1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			S1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({S1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

S1AP-ELEMENTARY-PROCEDURES S1AP-ELEMENTARY-PROCEDURE ::= {

S1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

S1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

S1AP-ELEMENTARY-PROCEDURES-CLASS-1 S1AP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation				|

handoverResourceAllocation		|

pathSwitchRequest 				|

e-RABSetup						|

e-RABModify						|

e-RABRelease					|

initialContextSetup				|

handoverCancel					|

kill							|

reset							|

s1Setup							|

uEContextModification			|

uEContextRelease				|

eNBConfigurationUpdate			|

mMEConfigurationUpdate			|

writeReplaceWarning				,

...,

uERadioCapabilityMatch						|

e-RABModificationIndication					|

uEContextModificationIndication				|

uEContextSuspend							|

uEContextResume

}

S1AP-ELEMENTARY-PROCEDURES-CLASS-2 S1AP-ELEMENTARY-PROCEDURE ::= {

handoverNotification			|

e-RABReleaseIndication			|

paging 							|

downlinkNASTransport			|

initialUEMessage				|

uplinkNASTransport				|

errorIndication					|

nASNonDeliveryIndication		|

uEContextReleaseRequest			|

downlinkS1cdma2000tunnelling	|

uplinkS1cdma2000tunnelling		|

uECapabilityInfoIndication		|

eNBStatusTransfer				|

mMEStatusTransfer				|

deactivateTrace					|

traceStart						|

traceFailureIndication			|

cellTrafficTrace				|

locationReportingControl		|

locationReportingFailureIndication	|

locationReport					|

overloadStart					|

overloadStop					|

eNBDirectInformationTransfer	|

mMEDirectInformationTransfer	|

eNBConfigurationTransfer		|

mMEConfigurationTransfer		|

privateMessage					,

...,

downlinkUEAssociatedLPPaTransport	|

uplinkUEAssociatedLPPaTransport	|

downlinkNonUEAssociatedLPPaTransport	|

uplinkNonUEAssociatedLPPaTransport	|

pWSRestartIndication			|

rerouteNASRequest				|

pWSFailureIndication						|

connectionEstablishmentIndication

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequired

SUCCESSFUL OUTCOME		HandoverCommand

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-HandoverPreparation

CRITICALITY				reject

}

handoverResourceAllocation S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverFailure

PROCEDURE CODE			id-HandoverResourceAllocation

CRITICALITY				reject

}

handoverNotification S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverNotify

PROCEDURE CODE			id-HandoverNotification

CRITICALITY				ignore

}

pathSwitchRequest S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PathSwitchRequest

SUCCESSFUL OUTCOME		PathSwitchRequestAcknowledge

UNSUCCESSFUL OUTCOME	PathSwitchRequestFailure

PROCEDURE CODE			id-PathSwitchRequest

CRITICALITY				reject

}

e-RABSetup S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-RABSetupRequest

SUCCESSFUL OUTCOME		E-RABSetupResponse

PROCEDURE CODE			id-E-RABSetup

CRITICALITY				reject

}

e-RABModify S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-RABModifyRequest

SUCCESSFUL OUTCOME		E-RABModifyResponse

PROCEDURE CODE			id-E-RABModify

CRITICALITY				reject

}

e-RABRelease S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-RABReleaseCommand

SUCCESSFUL OUTCOME		E-RABReleaseResponse

PROCEDURE CODE			id-E-RABRelease

CRITICALITY				reject

}

e-RABReleaseIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-RABReleaseIndication

PROCEDURE CODE			id-E-RABReleaseIndication

CRITICALITY				ignore

}

initialContextSetup S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialContextSetupRequest

SUCCESSFUL OUTCOME		InitialContextSetupResponse

UNSUCCESSFUL OUTCOME 	InitialContextSetupFailure

PROCEDURE CODE			id-InitialContextSetup

CRITICALITY				reject

}

uEContextReleaseRequest S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseRequest

PROCEDURE CODE			id-UEContextReleaseRequest

CRITICALITY				ignore

}

paging S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Paging

PROCEDURE CODE			id-Paging

CRITICALITY				ignore

}

downlinkNASTransport S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkNASTransport

PROCEDURE CODE			id-downlinkNASTransport

CRITICALITY				ignore

}

initialUEMessage S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialUEMessage

PROCEDURE CODE			id-initialUEMessage

CRITICALITY				ignore

}

uplinkNASTransport S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkNASTransport

PROCEDURE CODE			id-uplinkNASTransport

CRITICALITY				ignore

}

nASNonDeliveryIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NASNonDeliveryIndication

PROCEDURE CODE			id-NASNonDeliveryIndication

CRITICALITY				ignore

}

handoverCancel S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

SUCCESSFUL OUTCOME		HandoverCancelAcknowledge

PROCEDURE CODE			id-HandoverCancel

CRITICALITY				reject

}

reset S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-Reset

CRITICALITY				reject

}

errorIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-ErrorIndication

CRITICALITY				ignore

}

s1Setup S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		S1SetupRequest

SUCCESSFUL OUTCOME		S1SetupResponse

UNSUCCESSFUL OUTCOME 	S1SetupFailure

PROCEDURE CODE			id-S1Setup

CRITICALITY				reject

}

eNBConfigurationUpdate S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENBConfigurationUpdate

SUCCESSFUL OUTCOME		ENBConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME 	ENBConfigurationUpdateFailure

PROCEDURE CODE			id-ENBConfigurationUpdate

CRITICALITY				reject

}

mMEConfigurationUpdate S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MMEConfigurationUpdate

SUCCESSFUL OUTCOME		MMEConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME 	MMEConfigurationUpdateFailure

PROCEDURE CODE			id-MMEConfigurationUpdate

CRITICALITY				reject

}

downlinkS1cdma2000tunnelling S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkS1cdma2000tunnelling

PROCEDURE CODE			id-DownlinkS1cdma2000tunnelling

CRITICALITY				ignore

}

uplinkS1cdma2000tunnelling S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkS1cdma2000tunnelling

PROCEDURE CODE			id-UplinkS1cdma2000tunnelling

CRITICALITY				ignore

}

uEContextModification S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequest

SUCCESSFUL OUTCOME		UEContextModificationResponse

UNSUCCESSFUL OUTCOME 	UEContextModificationFailure

PROCEDURE CODE			id-UEContextModification

CRITICALITY				reject

}

uECapabilityInfoIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UECapabilityInfoIndication

PROCEDURE CODE			id-UECapabilityInfoIndication

CRITICALITY				ignore

}

uEContextRelease S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseCommand

SUCCESSFUL OUTCOME		UEContextReleaseComplete

PROCEDURE CODE			id-UEContextRelease

CRITICALITY				reject

}

eNBStatusTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENBStatusTransfer

PROCEDURE CODE			id-eNBStatusTransfer

CRITICALITY				ignore

}

mMEStatusTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MMEStatusTransfer

PROCEDURE CODE			id-MMEStatusTransfer

CRITICALITY				ignore

}

deactivateTrace S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

traceStart S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

traceFailureIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceFailureIndication

PROCEDURE CODE			id-TraceFailureIndication

CRITICALITY				ignore

}

cellTrafficTrace S1AP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-CellTrafficTrace

CRITICALITY				ignore

}

locationReportingControl S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReportingControl

PROCEDURE CODE			id-LocationReportingControl

CRITICALITY				ignore

}

locationReportingFailureIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReportingFailureIndication

PROCEDURE CODE			id-LocationReportingFailureIndication

CRITICALITY				ignore

}

locationReport S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReport

PROCEDURE CODE			id-LocationReport

CRITICALITY				ignore

}

overloadStart S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OverloadStart

PROCEDURE CODE			id-OverloadStart

CRITICALITY				ignore

}

overloadStop S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OverloadStop

PROCEDURE CODE			id-OverloadStop

CRITICALITY				reject

}

writeReplaceWarning S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		WriteReplaceWarningRequest

SUCCESSFUL OUTCOME		WriteReplaceWarningResponse

PROCEDURE CODE			id-WriteReplaceWarning

CRITICALITY				reject

}

eNBDirectInformationTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENBDirectInformationTransfer

PROCEDURE CODE			id-eNBDirectInformationTransfer

CRITICALITY				ignore

}

mMEDirectInformationTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MMEDirectInformationTransfer

PROCEDURE CODE			id-MMEDirectInformationTransfer

CRITICALITY				ignore

}

eNBConfigurationTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ENBConfigurationTransfer

PROCEDURE CODE			id-eNBConfigurationTransfer

CRITICALITY				ignore

}

mMEConfigurationTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MMEConfigurationTransfer

PROCEDURE CODE			id-MMEConfigurationTransfer

CRITICALITY				ignore

}

privateMessage S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-PrivateMessage

CRITICALITY				ignore

}

pWSRestartIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSRestartIndication

PROCEDURE CODE			id-PWSRestartIndication

CRITICALITY				ignore

}

kill S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		KillRequest

SUCCESSFUL OUTCOME		KillResponse

PROCEDURE CODE			id-Kill

CRITICALITY				reject

}

downlinkUEAssociatedLPPaTransport S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkUEAssociatedLPPaTransport

PROCEDURE CODE			id-downlinkUEAssociatedLPPaTransport

CRITICALITY				ignore

}

uplinkUEAssociatedLPPaTransport S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkUEAssociatedLPPaTransport

PROCEDURE CODE			id-uplinkUEAssociatedLPPaTransport

CRITICALITY				ignore

}

downlinkNonUEAssociatedLPPaTransport S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkNonUEAssociatedLPPaTransport

PROCEDURE CODE			id-downlinkNonUEAssociatedLPPaTransport

CRITICALITY				ignore

}

uplinkNonUEAssociatedLPPaTransport S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkNonUEAssociatedLPPaTransport

PROCEDURE CODE			id-uplinkNonUEAssociatedLPPaTransport

CRITICALITY				ignore

}

uERadioCapabilityMatch S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UERadioCapabilityMatchRequest

SUCCESSFUL OUTCOME		UERadioCapabilityMatchResponse

PROCEDURE CODE			id-UERadioCapabilityMatch

CRITICALITY				reject

}

e-RABModificationIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-RABModificationIndication

SUCCESSFUL OUTCOME		E-RABModificationConfirm

PROCEDURE CODE			id-E-RABModificationIndication

CRITICALITY				reject

}

uEContextModificationIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationIndication

SUCCESSFUL OUTCOME		UEContextModificationConfirm

PROCEDURE CODE			id-UEContextModificationIndication

CRITICALITY				reject

}

rerouteNASRequest S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RerouteNASRequest

PROCEDURE CODE			id-RerouteNASRequest

CRITICALITY				reject

}

pWSFailureIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSFailureIndication

PROCEDURE CODE			id-PWSFailureIndication

CRITICALITY				ignore

}

uEContextSuspend S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextSuspendRequest

SUCCESSFUL OUTCOME		UEContextSuspendResponse

PROCEDURE CODE			id-UEContextSuspend

CRITICALITY				reject

}

uEContextResume S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextResumeRequest

SUCCESSFUL OUTCOME		UEContextResumeResponse

UNSUCCESSFUL OUTCOME 	UEContextResumeFailure

PROCEDURE CODE			id-UEContextResume

CRITICALITY				reject

}

connectionEstablishmentIndication S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConnectionEstablishmentIndication

PROCEDURE CODE			id-ConnectionEstablishmentIndication

CRITICALITY				reject

}

END

### 9.3.3	PDU Definitions

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UEAggregateMaximumBitrate,

BearerType,

Cause,

CellAccessMode,

Cdma2000HORequiredIndication,

Cdma2000HOStatus,

Cdma2000OneXSRVCCInfo,

Cdma2000OneXRAND,

Cdma2000PDU,

Cdma2000RATType,

Cdma2000SectorID,

EUTRANRoundTripDelayEstimationInfo,

CNDomain,

ConcurrentWarningMessageIndicator,

CriticalityDiagnostics,

CSFallbackIndicator,

CSG-Id,

CSG-IdList,

CSGMembershipStatus,

Data-Forwarding-Not-Possible,

Direct-Forwarding-Path-Availability,

Global-ENB-ID,

EUTRAN-CGI,

ENBname,

ENB-StatusTransfer-TransparentContainer,

ENB-UE-S1AP-ID,

ExtendedRepetitionPeriod,

GTP-TEID,

GUMMEI,

GUMMEIType,

HandoverRestrictionList,

HandoverType,

Masked-IMEISV,

LAI,

LPPa-PDU,

ManagementBasedMDTAllowed,

MDTPLMNList,

MMEname,

MMERelaySupportIndicator,

MME-UE-S1AP-ID,

MSClassmark2,

MSClassmark3,

NAS-PDU,

NASSecurityParametersfromE-UTRAN,

NASSecurityParameterstoE-UTRAN,

OverloadResponse,

PagingDRX,

PagingPriority,

PLMNidentity,

ProSeAuthorized,

RIMTransfer,

RelativeMMECapacity,

RequestType,

E-RAB-ID,

E-RABLevelQoSParameters,

E-RABList,

RelayNode-Indicator,

Routing-ID,

SecurityKey,

SecurityContext,

ServedGUMMEIs,

SONConfigurationTransfer,

Source-ToTarget-TransparentContainer,

SourceBSS-ToTargetBSS-TransparentContainer,

SourceeNB-ToTargeteNB-TransparentContainer,

SourceRNC-ToTargetRNC-TransparentContainer,

SubscriberProfileIDforRFP,

SRVCCOperationNotPossible,

SRVCCOperationPossible,

SRVCCHOIndication,

SupportedTAs,

TAI,

Target-ToSource-TransparentContainer,

TargetBSS-ToSourceBSS-TransparentContainer,

TargeteNB-ToSourceeNB-TransparentContainer,

TargetID,

TargetRNC-ToSourceRNC-TransparentContainer,

TimeToWait,

TraceActivation,

TrafficLoadReductionIndication,

E-UTRAN-Trace-ID,

TransportLayerAddress,

UEIdentityIndexValue,

UEPagingID,

UERadioCapability,

UERadioCapabilityForPaging,

UE-RetentionInformation,

UE-S1AP-IDs,

UE-associatedLogicalS1-ConnectionItem,

UESecurityCapabilities,

S-TMSI,

MessageIdentifier,

SerialNumber,

WarningAreaList,

RepetitionPeriod,

NumberofBroadcastRequest,

WarningType,

WarningSecurityInfo,

DataCodingScheme,

WarningMessageContents,

BroadcastCompletedAreaList,

RRC-Establishment-Cause,

BroadcastCancelledAreaList,

PS-ServiceNotAvailable,

GUMMEIList,

Correlation-ID,

GWContextReleaseIndication,

PrivacyIndicator,

VoiceSupportMatchIndicator,

TunnelInformation,

KillAllWarningMessages,

TransportInformation,

LHN-ID,

UserLocationInformation,

AdditionalCSFallbackIndicator,

ECGIListForRestart,

TAIListForRestart,

EmergencyAreaIDListForRestart,

ExpectedUEBehaviour,

Paging-eDRXInformation,

Extended-UEIdentityIndexValue,

MME-Group-ID,

Additional-GUTI,

PWSfailedECGIList,

CellIdentifierAndCELevelForCECapableUEs,

AssistanceDataForPaging,

InformationOnRecommendedCellsAndENBsForPaging,

UE-Usage-Type,

PSP-list,

UEUserPlaneCIoTSupportIndicator,

NB-IoT-DefaultPagingDRX,

NB-IoT-Paging-eDRXInformation,

CE-mode-B-SupportIndicator,

NB-IoT-UEIdentityIndexValue

FROM S1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-SingleContainer{},

S1AP-PRIVATE-IES,

S1AP-PROTOCOL-EXTENSION,

S1AP-PROTOCOL-IES,

S1AP-PROTOCOL-IES-PAIR

FROM S1AP-Containers

id-AssistanceDataForPaging,

id-uEaggregateMaximumBitrate,

id-BearerType,

id-Cause,

id-CellAccessMode,

id-CellIdentifierAndCELevelForCECapableUEs,

id-cdma2000HORequiredIndication,

id-cdma2000HOStatus,

id-cdma2000OneXSRVCCInfo,

id-cdma2000OneXRAND,

id-cdma2000PDU,

id-cdma2000RATType,

id-cdma2000SectorID,

id-EUTRANRoundTripDelayEstimationInfo,

id-CNDomain,

id-ConcurrentWarningMessageIndicator,

id-CriticalityDiagnostics,

id-CSFallbackIndicator,

id-CSG-Id,

id-CSG-IdList,

id-CSGMembershipStatus,

id-Data-Forwarding-Not-Possible,

id-DefaultPagingDRX,

id-Direct-Forwarding-Path-Availability,

id-Global-ENB-ID,

id-EUTRAN-CGI,

id-eNBname,

id-eNB-StatusTransfer-TransparentContainer,

id-eNB-UE-S1AP-ID,

id-GERANtoLTEHOInformationRes,

id-GUMMEI-ID,

id-GUMMEIType,

id-HandoverRestrictionList,

id-HandoverType,

id-Masked-IMEISV,

id-InformationOnRecommendedCellsAndENBsForPaging,

id-InitialContextSetup,

id-Inter-SystemInformationTransferTypeEDT,

id-Inter-SystemInformationTransferTypeMDT,

id-LPPa-PDU,

id-NAS-DownlinkCount,

id-ManagementBasedMDTAllowed,

id-ManagementBasedMDTPLMNList,

id-MMEname,

id-MME-UE-S1AP-ID,

id-MSClassmark2,

id-MSClassmark3,

id-NAS-PDU,

id-NASSecurityParametersfromE-UTRAN,

id-NASSecurityParameterstoE-UTRAN,

id-OverloadResponse,

id-pagingDRX,

id-PagingPriority,

id-RelativeMMECapacity,

id-RequestType,

id-Routing-ID,

id-E-RABAdmittedItem,

id-E-RABAdmittedList,

id-E-RABDataForwardingItem,

id-E-RABFailedToModifyList,

id-E-RABFailedToReleaseList,

id-E-RABFailedtoSetupItemHOReqAck,

id-E-RABFailedToSetupListBearerSURes,

id-E-RABFailedToSetupListCtxtSURes,

id-E-RABFailedToSetupListHOReqAck,

id-E-RABFailedToBeReleasedList,

id-E-RABFailedToResumeListResumeReq,

id-E-RABFailedToResumeItemResumeReq,

id-E-RABFailedToResumeListResumeRes,

id-E-RABFailedToResumeItemResumeRes,

id-E-RABModify,

id-E-RABModifyItemBearerModRes,

id-E-RABModifyListBearerModRes,

id-E-RABRelease,

id-E-RABReleaseItemBearerRelComp,

id-E-RABReleaseItemHOCmd,

id-E-RABReleaseListBearerRelComp,

id-E-RABReleaseIndication,

id-E-RABSetup,

id-E-RABSetupItemBearerSURes,

id-E-RABSetupItemCtxtSURes,

id-E-RABSetupListBearerSURes,

id-E-RABSetupListCtxtSURes,

id-E-RABSubjecttoDataForwardingList,

id-E-RABToBeModifiedItemBearerModReq,

id-E-RABToBeModifiedListBearerModReq,

id-E-RABToBeModifiedListBearerModInd,

id-E-RABToBeModifiedItemBearerModInd,

id-E-RABNotToBeModifiedListBearerModInd,

id-E-RABNotToBeModifiedItemBearerModInd,

id-E-RABModifyListBearerModConf,

id-E-RABModifyItemBearerModConf,

id-E-RABFailedToModifyListBearerModConf,

id-E-RABToBeReleasedListBearerModConf,

id-E-RABToBeReleasedList,

id-E-RABReleasedList,

id-E-RABToBeSetupItemBearerSUReq,

id-E-RABToBeSetupItemCtxtSUReq,

id-E-RABToBeSetupItemHOReq,

id-E-RABToBeSetupListBearerSUReq,

id-E-RABToBeSetupListCtxtSUReq,

id-E-RABToBeSetupListHOReq,

id-E-RABToBeSwitchedDLItem,

id-E-RABToBeSwitchedDLList,

id-E-RABToBeSwitchedULList,

id-E-RABToBeSwitchedULItem,

id-E-RABtoReleaseListHOCmd,

id-ProSeAuthorized,

id-SecurityKey,

id-SecurityContext,

id-ServedGUMMEIs,

id-SONConfigurationTransferECT,

id-SONConfigurationTransferMCT,

id-Source-ToTarget-TransparentContainer,

id-Source-ToTarget-TransparentContainer-Secondary,

id-SourceMME-UE-S1AP-ID,

id-SRVCCOperationNotPossible,

id-SRVCCOperationPossible,

id-SRVCCHOIndication,

id-SubscriberProfileIDforRFP,

id-SupportedTAs,

id-S-TMSI,

id-TAI,

id-TAIItem,

id-TAIList,

id-Target-ToSource-TransparentContainer,

id-Target-ToSource-TransparentContainer-Secondary,

id-TargetID,

id-TimeToWait,

id-TraceActivation,

id-TrafficLoadReductionIndication,

id-E-UTRAN-Trace-ID,

id-UEIdentityIndexValue,

id-UEPagingID,

id-UERadioCapability,

id-UERadioCapabilityForPaging,

id-UTRANtoLTEHOInformationRes,

id-UE-associatedLogicalS1-ConnectionListResAck,

id-UE-associatedLogicalS1-ConnectionItem,

id-UE-RetentionInformation,

id-UESecurityCapabilities,

id-UE-S1AP-IDs,

id-ResetType,

id-MessageIdentifier,

id-SerialNumber,

id-WarningAreaList,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-WarningType,

id-WarningSecurityInfo,

id-DataCodingScheme,

id-WarningMessageContents,

id-BroadcastCompletedAreaList,

id-BroadcastCancelledAreaList,

id-RRC-Establishment-Cause,

id-TraceCollectionEntityIPAddress,

maxnoofTAIs,

maxnoofErrors,

maxnoofE-RABs,

maxnoofIndividualS1ConnectionsToReset,

maxnoofEmergencyAreaID,

maxnoofCellID,

maxnoofTAIforWarning,

maxnoofCellinTAI,

maxnoofCellinEAI,

id-ExtendedRepetitionPeriod,

id-PS-ServiceNotAvailable,

id-RegisteredLAI,

id-GUMMEIList,

id-SourceMME-GUMMEI,

id-MME-UE-S1AP-ID-2,

id-GW-TransportLayerAddress,

id-RelayNode-Indicator,

id-Correlation-ID,

id-MMERelaySupportIndicator,

id-GWContextReleaseIndication,

id-PrivacyIndicator,

id-VoiceSupportMatchIndicator,

id-Tunnel-Information-for-BBF,

id-SIPTO-Correlation-ID,

id-SIPTO-L-GW-TransportLayerAddress,

id-KillAllWarningMessages,

id-TransportInformation,

id-LHN-ID,

id-UserLocationInformation,

id-AdditionalCSFallbackIndicator,

id-ECGIListForRestart,

id-TAIListForRestart,

id-EmergencyAreaIDListForRestart,

id-ExpectedUEBehaviour,

id-Paging-eDRXInformation,

id-extended-UEIdentityIndexValue,

id-CSGMembershipInfo,

id-MME-Group-ID,

id-Additional-GUTI,

id-S1-Message,

id-PWSfailedECGIList,

id-PWSFailureIndication,

id-UE-Usage-Type,

id-PSP-list,

id-UEUserPlaneCIoTSupportIndicator,

id-NB-IoT-DefaultPagingDRX,

id-NB-IoT-Paging-eDRXInformation,

id-CE-mode-B-SupportIndicator,

id-NB-IoT-UEIdentityIndexValue,

id-RRC-Resume-Cause

FROM S1AP-Constants;

-- **************************************************************

--

-- Common Container Lists

--

-- **************************************************************

E-RAB-IE-ContainerList			{ S1AP-PROTOCOL-IES      : IEsSetParam }	::= ProtocolIE-ContainerList     { 1, maxnoofE-RABs,   {IEsSetParam} }

E-RAB-IE-ContainerPairList		{ S1AP-PROTOCOL-IES-PAIR : IEsSetParam }	::= ProtocolIE-ContainerPairList { 1, maxnoofE-RABs,   {IEsSetParam} }

ProtocolError-IE-ContainerList	{ S1AP-PROTOCOL-IES      : IEsSetParam }	::= ProtocolIE-ContainerList     { 1, maxnoofE-RABs,   {IEsSetParam} }

-- **************************************************************

--

-- HANDOVER PREPARATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Handover Required

--

-- **************************************************************

HandoverRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { HandoverRequiredIEs} },

...

}

HandoverRequiredIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-TargetID								CRITICALITY reject	TYPE TargetID							PRESENCE mandatory}|

{ ID id-Direct-Forwarding-Path-Availability		CRITICALITY ignore	TYPE Direct-Forwarding-Path-Availability		PRESENCE optional}|

{ ID id-SRVCCHOIndication						CRITICALITY reject	TYPE SRVCCHOIndication					PRESENCE optional}|

{ ID id-Source-ToTarget-TransparentContainer	CRITICALITY reject	TYPE Source-ToTarget-TransparentContainer	PRESENCE mandatory}|

{ ID id-Source-ToTarget-TransparentContainer-Secondary	CRITICALITY reject	TYPE Source-ToTarget-TransparentContainer	PRESENCE optional}|

{ ID id-MSClassmark2							CRITICALITY reject	TYPE MSClassmark2						PRESENCE conditional}|

{ ID id-MSClassmark3							CRITICALITY ignore	TYPE MSClassmark3						PRESENCE conditional}|

{ ID id-CSG-Id									CRITICALITY reject	TYPE CSG-Id								PRESENCE optional}|

{ ID id-CellAccessMode							CRITICALITY reject	TYPE CellAccessMode						PRESENCE optional}|

{ ID id-PS-ServiceNotAvailable					CRITICALITY ignore	TYPE PS-ServiceNotAvailable			PRESENCE optional},

...

}

-- **************************************************************

--

-- Handover Command

--

-- **************************************************************

HandoverCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { HandoverCommandIEs} },

...

}

HandoverCommandIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID							PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY reject	TYPE ENB-UE-S1AP-ID							PRESENCE mandatory}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType							PRESENCE mandatory}|

{ ID id-NASSecurityParametersfromE-UTRAN		CRITICALITY reject	TYPE NASSecurityParametersfromE-UTRAN			PRESENCE conditional

-- This IE shall be present if HandoverType IE is set to value "LTEtoUTRAN" or "LTEtoGERAN" --}|

{ ID id-E-RABSubjecttoDataForwardingList		CRITICALITY ignore	TYPE E-RABSubjecttoDataForwardingList			PRESENCE optional}|

{ ID id-E-RABtoReleaseListHOCmd					CRITICALITY ignore	TYPE E-RABList								PRESENCE optional}|

{ ID id-Target-ToSource-TransparentContainer	CRITICALITY reject	TYPE Target-ToSource-TransparentContainer		PRESENCE mandatory}|

{ ID id-Target-ToSource-TransparentContainer-Secondary	CRITICALITY reject	TYPE Target-ToSource-TransparentContainer	PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional},

...

}

E-RABSubjecttoDataForwardingList ::= E-RAB-IE-ContainerList { {E-RABDataForwardingItemIEs} }

E-RABDataForwardingItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABDataForwardingItem					CRITICALITY ignore	TYPE E-RABDataForwardingItem			PRESENCE mandatory	},

...

}

E-RABDataForwardingItem ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

dL-transportLayerAddress			TransportLayerAddress 													OPTIONAL,

dL-gTP-TEID							GTP-TEID 																OPTIONAL,

uL-TransportLayerAddress			TransportLayerAddress													OPTIONAL,

uL-GTP-TEID							GTP-TEID																OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { E-RABDataForwardingItem-ExtIEs} }			OPTIONAL,

...

}

E-RABDataForwardingItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- Handover Preparation Failure

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { HandoverPreparationFailureIEs} },

...

}

HandoverPreparationFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- HANDOVER RESOURCE ALLOCATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Handover Request

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {HandoverRequestIEs} },

...

}

HandoverRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType						PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate				CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE mandatory}|

{ ID id-E-RABToBeSetupListHOReq					CRITICALITY reject	TYPE E-RABToBeSetupListHOReq			PRESENCE mandatory}|

{ ID id-Source-ToTarget-TransparentContainer	CRITICALITY reject	TYPE Source-ToTarget-TransparentContainer	PRESENCE mandatory}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities			PRESENCE mandatory}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList			PRESENCE optional}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation					PRESENCE optional}|

{ ID id-RequestType								CRITICALITY ignore	TYPE RequestType						PRESENCE optional}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible			PRESENCE optional}|

{ ID id-SecurityContext							CRITICALITY reject	TYPE SecurityContext					PRESENCE mandatory}|

{ ID id-NASSecurityParameterstoE-UTRAN			CRITICALITY reject	TYPE NASSecurityParameterstoE-UTRAN		PRESENCE conditional

-- This IE shall be present if the Handover Type IE is set to the value "UTRANtoLTE" or "GERANtoLTE" --				}|

{ ID id-CSG-Id									CRITICALITY reject	TYPE CSG-Id								PRESENCE optional}|

{ ID id-CSGMembershipStatus						CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-GUMMEI-ID								CRITICALITY ignore	TYPE GUMMEI								PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2						CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-ManagementBasedMDTAllowed				CRITICALITY ignore	TYPE ManagementBasedMDTAllowed			PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList						PRESENCE optional}|

{ ID id-Masked-IMEISV							CRITICALITY ignore	TYPE Masked-IMEISV						PRESENCE optional}|

{ ID id-ExpectedUEBehaviour						CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-ProSeAuthorized							CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator			CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator 		PRESENCE optional},

...

}

E-RABToBeSetupListHOReq 					::= E-RAB-IE-ContainerList { {E-RABToBeSetupItemHOReqIEs} }

E-RABToBeSetupItemHOReqIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSetupItemHOReq				CRITICALITY reject	TYPE E-RABToBeSetupItemHOReq			PRESENCE mandatory	},

...

}

E-RABToBeSetupItemHOReq ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

e-RABlevelQosParameters				E-RABLevelQoSParameters,

iE-Extensions						ProtocolExtensionContainer { {E-RABToBeSetupItemHOReq-ExtIEs} }		OPTIONAL,

...

}

E-RABToBeSetupItemHOReq-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Data-Forwarding-Not-Possible		CRITICALITY ignore	EXTENSION Data-Forwarding-Not-Possible	PRESENCE optional}|

{ ID id-BearerType							CRITICALITY reject	EXTENSION BearerType					PRESENCE optional},

...

}

-- **************************************************************

--

-- Handover Request Acknowledge

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {HandoverRequestAcknowledgeIEs} },

...

}

HandoverRequestAcknowledgeIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY ignore	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-E-RABAdmittedList						CRITICALITY ignore	TYPE E-RABAdmittedList					PRESENCE mandatory}|

{ ID id-E-RABFailedToSetupListHOReqAck			CRITICALITY ignore	TYPE E-RABFailedtoSetupListHOReqAck		PRESENCE optional}|

{ ID id-Target-ToSource-TransparentContainer	CRITICALITY reject	TYPE Target-ToSource-TransparentContainer	PRESENCE mandatory}|

{ ID id-CSG-Id									CRITICALITY ignore	TYPE CSG-Id								PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-CellAccessMode							CRITICALITY ignore	TYPE CellAccessMode						PRESENCE optional}|

{ ID id-CE-mode-B-SupportIndicator				CRITICALITY ignore	TYPE CE-mode-B-SupportIndicator			PRESENCE optional},

...

}

E-RABAdmittedList 					::= E-RAB-IE-ContainerList { {E-RABAdmittedItemIEs} }

E-RABAdmittedItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABAdmittedItem			CRITICALITY ignore	TYPE E-RABAdmittedItem			PRESENCE mandatory	},

...

}

E-RABAdmittedItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

dL-transportLayerAddress		TransportLayerAddress	OPTIONAL,

dL-gTP-TEID						GTP-TEID				OPTIONAL,

uL-TransportLayerAddress		TransportLayerAddress	OPTIONAL,

uL-GTP-TEID						GTP-TEID				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABAdmittedItem-ExtIEs} }	OPTIONAL,

...

}

E-RABAdmittedItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABFailedtoSetupListHOReqAck 					::= E-RAB-IE-ContainerList { {E-RABFailedtoSetupItemHOReqAckIEs} }

E-RABFailedtoSetupItemHOReqAckIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABFailedtoSetupItemHOReqAck			CRITICALITY ignore	TYPE E-RABFailedToSetupItemHOReqAck			PRESENCE mandatory	},

...

}

E-RABFailedToSetupItemHOReqAck ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

cause				Cause,

iE-Extensions					ProtocolExtensionContainer { { E-RABFailedToSetupItemHOReqAckExtIEs} }			OPTIONAL,

...

}

E-RABFailedToSetupItemHOReqAckExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- Handover Failure

--

-- **************************************************************

HandoverFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { HandoverFailureIEs} },

...

}

HandoverFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- HANDOVER NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Handover Notify

--

-- **************************************************************

HandoverNotify ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { HandoverNotifyIEs} },

...

}

HandoverNotifyIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-EUTRAN-CGI						CRITICALITY ignore	TYPE EUTRAN-CGI				PRESENCE mandatory}|

{ ID id-TAI								CRITICALITY ignore	TYPE TAI					PRESENCE mandatory}|

-- Extension for Release 11 to support BBAI --

{ ID id-Tunnel-Information-for-BBF		CRITICALITY ignore	TYPE TunnelInformation		PRESENCE optional}|

{ ID id-LHN-ID							CRITICALITY ignore	TYPE LHN-ID					PRESENCE optional},

...

}

-- **************************************************************

--

-- PATH SWITCH REQUEST ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Path Switch Request

--

-- **************************************************************

PathSwitchRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PathSwitchRequestIEs} },

...

}

PathSwitchRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-E-RABToBeSwitchedDLList			CRITICALITY reject	TYPE E-RABToBeSwitchedDLList	PRESENCE mandatory}|

{ ID id-SourceMME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-EUTRAN-CGI						CRITICALITY ignore	TYPE EUTRAN-CGI					PRESENCE mandatory}|

{ ID id-TAI								CRITICALITY ignore	TYPE TAI						PRESENCE mandatory}|

{ ID id-UESecurityCapabilities			CRITICALITY ignore	TYPE UESecurityCapabilities		PRESENCE mandatory}|

{ ID id-CSG-Id							CRITICALITY ignore	TYPE CSG-Id						PRESENCE optional}|

{ ID id-CellAccessMode					CRITICALITY ignore	TYPE CellAccessMode				PRESENCE optional}|

{ ID id-SourceMME-GUMMEI				CRITICALITY ignore	TYPE GUMMEI						PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus		PRESENCE optional}|

-- Extension for Release 11 to support BBAI --

{ ID id-Tunnel-Information-for-BBF		CRITICALITY ignore	TYPE TunnelInformation			PRESENCE optional}|

{ ID id-LHN-ID							CRITICALITY ignore	TYPE LHN-ID						PRESENCE optional},

...

}

E-RABToBeSwitchedDLList					::= E-RAB-IE-ContainerList { {E-RABToBeSwitchedDLItemIEs} }

E-RABToBeSwitchedDLItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSwitchedDLItem			CRITICALITY reject	TYPE E-RABToBeSwitchedDLItem			PRESENCE mandatory	},

...

}

E-RABToBeSwitchedDLItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

iE-Extensions					ProtocolExtensionContainer { { E-RABToBeSwitchedDLItem-ExtIEs} }			OPTIONAL,

...

}

E-RABToBeSwitchedDLItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- Path Switch Request Acknowledge

--

-- **************************************************************

PathSwitchRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PathSwitchRequestAcknowledgeIEs} },

...

}

PathSwitchRequestAcknowledgeIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY ignore	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY ignore	TYPE UEAggregateMaximumBitrate			PRESENCE optional}|

{ ID id-E-RABToBeSwitchedULList			CRITICALITY ignore	TYPE E-RABToBeSwitchedULList			PRESENCE optional}|

{ ID id-E-RABToBeReleasedList			CRITICALITY ignore	TYPE E-RABList							PRESENCE optional}|

{ ID id-SecurityContext					CRITICALITY reject	TYPE SecurityContext					PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2				CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-ProSeAuthorized					CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator	CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator	PRESENCE optional},

...

}

E-RABToBeSwitchedULList ::= E-RAB-IE-ContainerList { {E-RABToBeSwitchedULItemIEs} }

E-RABToBeSwitchedULItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSwitchedULItem		CRITICALITY ignore	TYPE E-RABToBeSwitchedULItem		PRESENCE mandatory	},

...

}

E-RABToBeSwitchedULItem ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

iE-Extensions						ProtocolExtensionContainer { { E-RABToBeSwitchedULItem-ExtIEs} }	OPTIONAL,

...

}

E-RABToBeSwitchedULItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- Path Switch Request Failure

--

-- **************************************************************

PathSwitchRequestFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PathSwitchRequestFailureIEs} },

...

}

PathSwitchRequestFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- HANDOVER CANCEL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Handover Cancel

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { HandoverCancelIEs} },

...

}

HandoverCancelIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- Handover Cancel Request Acknowledge

--

-- **************************************************************

HandoverCancelAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { HandoverCancelAcknowledgeIEs} },

...

}

HandoverCancelAcknowledgeIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- E-RAB SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- E-RAB Setup Request

--

-- **************************************************************

E-RABSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E-RABSetupRequestIEs} },

...

}

E-RABSetupRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE optional	}|

{ ID id-E-RABToBeSetupListBearerSUReq	CRITICALITY reject	TYPE E-RABToBeSetupListBearerSUReq	PRESENCE mandatory	},

...

}

E-RABToBeSetupListBearerSUReq ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABToBeSetupItemBearerSUReqIEs} }

E-RABToBeSetupItemBearerSUReqIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSetupItemBearerSUReq	 CRITICALITY reject 	TYPE E-RABToBeSetupItemBearerSUReq 	PRESENCE mandatory },

...

}

E-RABToBeSetupItemBearerSUReq ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RABlevelQoSParameters			E-RABLevelQoSParameters,

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

nAS-PDU							NAS-PDU,

iE-Extensions					ProtocolExtensionContainer { {E-RABToBeSetupItemBearerSUReqExtIEs} } OPTIONAL,

...

}

E-RABToBeSetupItemBearerSUReqExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Correlation-ID			CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-SIPTO-Correlation-ID	CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-BearerType				CRITICALITY reject	EXTENSION BearerType			PRESENCE optional},

...

}

-- **************************************************************

--

-- E-RAB Setup Response

--

-- **************************************************************

E-RABSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E-RABSetupResponseIEs} },

...

}

E-RABSetupResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY ignore	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY ignore	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-E-RABSetupListBearerSURes				CRITICALITY ignore	TYPE E-RABSetupListBearerSURes	PRESENCE optional	}|

{ ID id-E-RABFailedToSetupListBearerSURes		CRITICALITY ignore	TYPE E-RABList					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

E-RABSetupListBearerSURes ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABSetupItemBearerSUResIEs} }

E-RABSetupItemBearerSUResIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABSetupItemBearerSURes	 CRITICALITY ignore 	TYPE E-RABSetupItemBearerSURes 	PRESENCE mandatory },

...

}

E-RABSetupItemBearerSURes ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

transportLayerAddress		TransportLayerAddress,

gTP-TEID					GTP-TEID,

iE-Extensions				ProtocolExtensionContainer { {E-RABSetupItemBearerSUResExtIEs} } OPTIONAL,

...

}

E-RABSetupItemBearerSUResExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- E-RAB MODIFY ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- E-RAB Modify Request

--

-- **************************************************************

E-RABModifyRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E-RABModifyRequestIEs} },

...

}

E-RABModifyRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID						CRITICALITY reject	TYPE MME-UE-S1AP-ID							PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID						CRITICALITY reject	TYPE ENB-UE-S1AP-ID							PRESENCE mandatory	}|

{ ID id-uEaggregateMaximumBitrate			CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE optional	}|

{ ID id-E-RABToBeModifiedListBearerModReq	CRITICALITY reject	TYPE E-RABToBeModifiedListBearerModReq		PRESENCE mandatory	},

...

}

E-RABToBeModifiedListBearerModReq ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABToBeModifiedItemBearerModReqIEs} }

E-RABToBeModifiedItemBearerModReqIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeModifiedItemBearerModReq	 CRITICALITY reject 	TYPE E-RABToBeModifiedItemBearerModReq 	PRESENCE mandatory },

...

}

E-RABToBeModifiedItemBearerModReq ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RABLevelQoSParameters			E-RABLevelQoSParameters,

nAS-PDU							NAS-PDU,

iE-Extensions					ProtocolExtensionContainer { {E-RABToBeModifyItemBearerModReqExtIEs} } OPTIONAL,

...

}

E-RABToBeModifyItemBearerModReqExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-TransportInformation	CRITICALITY reject	EXTENSION TransportInformation		PRESENCE optional},

...

}

-- **************************************************************

--

-- E-RAB Modify Response

--

-- **************************************************************

E-RABModifyResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E-RABModifyResponseIEs} },

...

}

E-RABModifyResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY ignore	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-E-RABModifyListBearerModRes		CRITICALITY ignore	TYPE E-RABModifyListBearerModRes		PRESENCE optional	}|

{ ID id-E-RABFailedToModifyList			CRITICALITY ignore	TYPE E-RABList							PRESENCE optional	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

E-RABModifyListBearerModRes ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABModifyItemBearerModResIEs} }

E-RABModifyItemBearerModResIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABModifyItemBearerModRes		CRITICALITY ignore	TYPE E-RABModifyItemBearerModRes		PRESENCE mandatory},

...

}

E-RABModifyItemBearerModRes ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

iE-Extensions				ProtocolExtensionContainer { {E-RABModifyItemBearerModResExtIEs} } OPTIONAL,

...

}

E-RABModifyItemBearerModResExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- E-RAB RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- E-RAB Release Command

--

-- **************************************************************

E-RABReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {E-RABReleaseCommandIEs} },

...

}

E-RABReleaseCommandIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY reject	TYPE UEAggregateMaximumBitrate		PRESENCE optional	}|

{ ID id-E-RABToBeReleasedList			CRITICALITY ignore	TYPE E-RABList						PRESENCE mandatory	}|

{ ID id-NAS-PDU							CRITICALITY ignore	TYPE NAS-PDU						PRESENCE optional	},

...

}

-- **************************************************************

--

-- E-RAB Release Response

--

-- **************************************************************

E-RABReleaseResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { E-RABReleaseResponseIEs } },

...

}

E-RABReleaseResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY ignore	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-E-RABReleaseListBearerRelComp	CRITICALITY ignore	TYPE E-RABReleaseListBearerRelComp	PRESENCE optional	}|

{ ID id-E-RABFailedToReleaseList		CRITICALITY ignore	TYPE E-RABList							PRESENCE optional	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	}|

-- Extension for Release 12 to support User Location Information --

{ ID id-UserLocationInformation			CRITICALITY ignore	TYPE UserLocationInformation			PRESENCE optional	},

...

}

E-RABReleaseListBearerRelComp ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABReleaseItemBearerRelCompIEs} }

E-RABReleaseItemBearerRelCompIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABReleaseItemBearerRelComp	CRITICALITY ignore	TYPE E-RABReleaseItemBearerRelComp	PRESENCE mandatory },

...

}

E-RABReleaseItemBearerRelComp ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

iE-Extensions					ProtocolExtensionContainer { {E-RABReleaseItemBearerRelCompExtIEs} } OPTIONAL,

...

}

E-RABReleaseItemBearerRelCompExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- E-RAB RELEASE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- E-RAB Release Indication

--

-- **************************************************************

E-RABReleaseIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E-RABReleaseIndicationIEs} },

...

}

E-RABReleaseIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-E-RABReleasedList				CRITICALITY ignore	TYPE E-RABList						PRESENCE mandatory	}|

-- Extension for Release 12 to support User Location Information --

{ ID id-UserLocationInformation			CRITICALITY ignore	TYPE UserLocationInformation		PRESENCE optional	},

...

}

-- **************************************************************

--

-- INITIAL CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Initial Context Setup Request

--

-- **************************************************************

InitialContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {InitialContextSetupRequestIEs} },

...

}

InitialContextSetupRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE mandatory}|

{ ID id-E-RABToBeSetupListCtxtSUReq		CRITICALITY reject	TYPE E-RABToBeSetupListCtxtSUReq		PRESENCE mandatory}|

{ ID id-UESecurityCapabilities			CRITICALITY reject	TYPE UESecurityCapabilities				PRESENCE mandatory}|

{ ID id-SecurityKey						CRITICALITY reject	TYPE SecurityKey						PRESENCE mandatory}|

{ ID id-TraceActivation					CRITICALITY ignore	TYPE TraceActivation					PRESENCE optional}|

{ ID id-HandoverRestrictionList			CRITICALITY ignore	TYPE HandoverRestrictionList			PRESENCE optional}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability					PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	TYPE SubscriberProfileIDforRFP			PRESENCE optional}|

{ ID id-CSFallbackIndicator				CRITICALITY reject	TYPE CSFallbackIndicator				PRESENCE optional}|

{ ID id-SRVCCOperationPossible			CRITICALITY ignore	TYPE SRVCCOperationPossible				PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-RegisteredLAI					CRITICALITY ignore	TYPE LAI								PRESENCE optional}|

{ ID id-GUMMEI-ID						CRITICALITY ignore	TYPE GUMMEI								PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2				CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-ManagementBasedMDTAllowed		CRITICALITY ignore	TYPE ManagementBasedMDTAllowed			PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList		CRITICALITY ignore	TYPE MDTPLMNList						PRESENCE optional}|

{ ID id-AdditionalCSFallbackIndicator	CRITICALITY ignore	TYPE AdditionalCSFallbackIndicator	PRESENCE conditional}|

{ ID id-Masked-IMEISV					CRITICALITY ignore	TYPE Masked-IMEISV						PRESENCE optional}|

{ ID id-ExpectedUEBehaviour				CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-ProSeAuthorized					CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator	CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator	PRESENCE optional},

...

}

E-RABToBeSetupListCtxtSUReq ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABToBeSetupItemCtxtSUReqIEs} }

E-RABToBeSetupItemCtxtSUReqIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeSetupItemCtxtSUReq	CRITICALITY reject	TYPE E-RABToBeSetupItemCtxtSUReq		PRESENCE mandatory	},

...

}

E-RABToBeSetupItemCtxtSUReq ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RABlevelQoSParameters			E-RABLevelQoSParameters,

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

nAS-PDU							NAS-PDU		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABToBeSetupItemCtxtSUReqExtIEs} } OPTIONAL,

...

}

E-RABToBeSetupItemCtxtSUReqExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Correlation-ID					CRITICALITY ignore	EXTENSION Correlation-ID			PRESENCE optional}|

{ ID id-SIPTO-Correlation-ID			CRITICALITY ignore	EXTENSION Correlation-ID			PRESENCE optional}|

{ ID id-BearerType						CRITICALITY reject	EXTENSION BearerType				PRESENCE optional},

...

}

-- **************************************************************

--

-- Initial Context Setup Response

--

-- **************************************************************

InitialContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {InitialContextSetupResponseIEs} },

...

}

InitialContextSetupResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID						CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID						CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-E-RABSetupListCtxtSURes				CRITICALITY ignore	TYPE E-RABSetupListCtxtSURes		PRESENCE mandatory	}|

{ ID id-E-RABFailedToSetupListCtxtSURes		CRITICALITY ignore	TYPE E-RABList						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

E-RABSetupListCtxtSURes ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABSetupItemCtxtSUResIEs} }

E-RABSetupItemCtxtSUResIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABSetupItemCtxtSURes		CRITICALITY ignore	TYPE E-RABSetupItemCtxtSURes	PRESENCE mandatory	},

...

}

E-RABSetupItemCtxtSURes ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

transportLayerAddress 			TransportLayerAddress,

gTP-TEID						GTP-TEID,

iE-Extensions					ProtocolExtensionContainer { {E-RABSetupItemCtxtSUResExtIEs} } OPTIONAL,

...

}

E-RABSetupItemCtxtSUResExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- Initial Context Setup Failure

--

-- **************************************************************

InitialContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {InitialContextSetupFailureIEs} },

...

}

InitialContextSetupFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{PagingIEs}},

...

}

PagingIEs S1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue			CRITICALITY ignore	TYPE UEIdentityIndexValue				PRESENCE mandatory}|

{ ID id-UEPagingID						CRITICALITY ignore	TYPE UEPagingID							PRESENCE mandatory}|

{ ID id-pagingDRX						CRITICALITY ignore	TYPE PagingDRX							PRESENCE optional}|

{ ID id-CNDomain						CRITICALITY ignore	TYPE CNDomain							PRESENCE mandatory}|

{ ID id-TAIList							CRITICALITY ignore	TYPE TAIList							PRESENCE mandatory}|

{ ID id-CSG-IdList						CRITICALITY ignore	TYPE CSG-IdList							PRESENCE optional}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority						PRESENCE optional}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging			PRESENCE optional}|

-- Extension for Release 13 to support Paging Optimisation and Coverage Enhancement paging –-

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging			PRESENCE optional}|

{ ID id-Paging-eDRXInformation			CRITICALITY ignore	TYPE Paging-eDRXInformation				PRESENCE optional}|

{ ID id-extended-UEIdentityIndexValue	CRITICALITY ignore	TYPE Extended-UEIdentityIndexValue	PRESENCE optional}|

{ ID id-NB-IoT-Paging-eDRXInformation	CRITICALITY ignore	TYPE NB-IoT-Paging-eDRXInformation	PRESENCE optional}|

{ ID id-NB-IoT-UEIdentityIndexValue		CRITICALITY ignore	TYPE NB-IoT-UEIdentityIndexValue		PRESENCE optional},

...

}

TAIList::= SEQUENCE (SIZE(1.. maxnoofTAIs)) OF ProtocolIE-SingleContainer {{TAIItemIEs}}

TAIItemIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-TAIItem	 CRITICALITY ignore		TYPE TAIItem	PRESENCE mandatory },

...

}

TAIItem ::= SEQUENCE {

tAI 							TAI,

iE-Extensions					ProtocolExtensionContainer { {TAIItemExtIEs} } OPTIONAL,

...

}

TAIItemExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- UE CONTEXT RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Context Release Request

--

-- **************************************************************

UEContextReleaseRequest ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{UEContextReleaseRequest-IEs}},

...

}

UEContextReleaseRequest-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-GWContextReleaseIndication	CRITICALITY reject	TYPE GWContextReleaseIndication		PRESENCE optional	},

...

}

-- **************************************************************

--

-- UE Context Release Command

--

-- **************************************************************

UEContextReleaseCommand ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{UEContextReleaseCommand-IEs}},

...

}

UEContextReleaseCommand-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-UE-S1AP-IDs					CRITICALITY reject	TYPE UE-S1AP-IDs					PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- UE Context Release Complete

--

-- **************************************************************

UEContextReleaseComplete ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{UEContextReleaseComplete-IEs}},

...

}

UEContextReleaseComplete-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID								CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID								CRITICALITY ignore	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

-- Extension for Release 12 to support User Location Information --

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation				PRESENCE optional}|

-- Extension for Release 13 to support Paging Optimisation

{ ID id-InformationOnRecommendedCellsAndENBsForPaging	CRITICALITY ignore	TYPE InformationOnRecommendedCellsAndENBsForPaging		PRESENCE optional}|

-- Extension for Release 13 to support coverage enhancement paging –

{ ID id-CellIdentifierAndCELevelForCECapableUEs		CRITICALITY ignore	TYPE CellIdentifierAndCELevelForCECapableUEs	PRESENCE optional},

...

}

-- **************************************************************

--

-- UE CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Context Modification Request

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-SecurityKey						CRITICALITY reject	TYPE SecurityKey						PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	TYPE SubscriberProfileIDforRFP			PRESENCE optional}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY ignore	TYPE UEAggregateMaximumBitrate			PRESENCE optional}|

{ ID id-CSFallbackIndicator				CRITICALITY reject	TYPE CSFallbackIndicator				PRESENCE optional}|

{ ID id-UESecurityCapabilities			CRITICALITY reject	TYPE UESecurityCapabilities				PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-RegisteredLAI					CRITICALITY ignore	TYPE LAI								PRESENCE optional}|

{ ID id-AdditionalCSFallbackIndicator	CRITICALITY ignore	TYPE AdditionalCSFallbackIndicator	PRESENCE conditional}|

{ ID id-ProSeAuthorized					CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-SRVCCOperationPossible			CRITICALITY ignore	TYPE SRVCCOperationPossible				PRESENCE optional}|

{ ID id-SRVCCOperationNotPossible		CRITICALITY ignore	TYPE SRVCCOperationNotPossible			PRESENCE optional},

...

}

-- **************************************************************

--

-- UE Context Modification Response

--

-- **************************************************************

UEContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationResponseIEs} },

...

}

UEContextModificationResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- UE Context Modification Failure

--

-- **************************************************************

UEContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationFailureIEs} },

...

}

UEContextModificationFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- UE RADIO CAPABILITY MATCH ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Radio Capability Match Request

--

-- **************************************************************

UERadioCapabilityMatchRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UERadioCapabilityMatchRequestIEs} },

...

}

UERadioCapabilityMatchRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional	},

...

}

-- **************************************************************

--

-- UE Radio Capability Match Response

--

-- **************************************************************

UERadioCapabilityMatchResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UERadioCapabilityMatchResponseIEs} },

...

}

UERadioCapabilityMatchResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory	}|

{ ID id-VoiceSupportMatchIndicator	CRITICALITY reject	TYPE VoiceSupportMatchIndicator		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- NAS TRANSPORT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

-- DOWNLINK NAS TRANSPORT

--

-- **************************************************************

DownlinkNASTransport ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{DownlinkNASTransport-IEs}},

...

}

DownlinkNASTransport-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory}|

{ ID id-HandoverRestrictionList		CRITICALITY ignore	TYPE HandoverRestrictionList		PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP	CRITICALITY ignore	TYPE SubscriberProfileIDforRFP		PRESENCE optional}|

{ ID id-SRVCCOperationPossible		CRITICALITY ignore	TYPE SRVCCOperationPossible			PRESENCE optional}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional},

...

}

-- **************************************************************

--

-- INITIAL UE MESSAGE

--

-- **************************************************************

InitialUEMessage ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{InitialUEMessage-IEs}},

...

}

InitialUEMessage-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory}|

{ ID id-TAI							CRITICALITY reject	TYPE TAI							PRESENCE mandatory}|

{ ID id-EUTRAN-CGI					CRITICALITY ignore	TYPE EUTRAN-CGI						PRESENCE mandatory}|

{ ID id-RRC-Establishment-Cause		CRITICALITY ignore	TYPE RRC-Establishment-Cause		PRESENCE mandatory}|

{ ID id-S-TMSI						CRITICALITY reject	TYPE S-TMSI							PRESENCE optional}|

{ ID id-CSG-Id						CRITICALITY reject	TYPE CSG-Id							PRESENCE optional}|

{ ID id-GUMMEI-ID					CRITICALITY reject	TYPE GUMMEI							PRESENCE optional}|

{ ID id-CellAccessMode				CRITICALITY reject	TYPE CellAccessMode					PRESENCE optional}|

{ ID id-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress			PRESENCE optional}|

{ ID id-RelayNode-Indicator			CRITICALITY reject	TYPE RelayNode-Indicator			PRESENCE optional}|

{ ID id-GUMMEIType					CRITICALITY ignore	TYPE GUMMEIType						PRESENCE optional}|

-- Extension for Release 11 to support BBAI --

{ ID id-Tunnel-Information-for-BBF	CRITICALITY ignore	TYPE TunnelInformation				PRESENCE optional}|

{ ID id-SIPTO-L-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress	PRESENCE optional}|

{ ID id-LHN-ID						CRITICALITY ignore	TYPE LHN-ID							PRESENCE optional}|

{ ID id-MME-Group-ID				CRITICALITY ignore	TYPE MME-Group-ID					PRESENCE optional}|

{ ID id-UE-Usage-Type				CRITICALITY ignore	TYPE UE-Usage-Type					PRESENCE optional}|

{ ID id-CE-mode-B-SupportIndicator	CRITICALITY ignore	TYPE CE-mode-B-SupportIndicator		PRESENCE optional},

...

}

-- **************************************************************

--

-- UPLINK NAS TRANSPORT

--

-- **************************************************************

UplinkNASTransport ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{UplinkNASTransport-IEs}},

...

}

UplinkNASTransport-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory}|

{ ID id-EUTRAN-CGI					CRITICALITY ignore	TYPE EUTRAN-CGI						PRESENCE mandatory}|

{ ID id-TAI							CRITICALITY ignore	TYPE TAI							PRESENCE mandatory}|

{ ID id-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress			PRESENCE optional}|

{ ID id-SIPTO-L-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress	PRESENCE optional}|

{ ID id-LHN-ID						CRITICALITY ignore	TYPE LHN-ID							PRESENCE optional},

...

}

-- **************************************************************

--

-- NAS NON DELIVERY INDICATION

--

-- **************************************************************

NASNonDeliveryIndication ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{NASNonDeliveryIndication-IEs}},

...

}

NASNonDeliveryIndication-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-NAS-PDU						CRITICALITY ignore	TYPE NAS-PDU					PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- REROUTE NAS REQUEST

--

-- **************************************************************

RerouteNASRequest ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RerouteNASRequest-IEs}},

...

}

RerouteNASRequest-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-eNB-UE-S1AP-ID			CRITICALITY reject	TYPE ENB-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-MME-UE-S1AP-ID			CRITICALITY ignore	TYPE MME-UE-S1AP-ID			PRESENCE optional}|

{ ID id-S1-Message				CRITICALITY reject	TYPE OCTET STRING			PRESENCE mandatory}|

{ ID id-MME-Group-ID			CRITICALITY reject	TYPE MME-Group-ID			PRESENCE mandatory}|

{ ID id-Additional-GUTI			CRITICALITY ignore	TYPE Additional-GUTI		PRESENCE optional}|

{ ID id-UE-Usage-Type			CRITICALITY ignore	TYPE UE-Usage-Type			PRESENCE optional},

...

}

-- **************************************************************

--

-- RESET ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

ResetIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

ResetType ::= CHOICE {

s1-Interface					ResetAll,

partOfS1-Interface				UE-associatedLogicalS1-ConnectionListRes,

...

}

ResetAll ::= ENUMERATED {

reset-all,

...

}

UE-associatedLogicalS1-ConnectionListRes ::= SEQUENCE (SIZE(1.. maxnoofIndividualS1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalS1-ConnectionItemRes } }

UE-associatedLogicalS1-ConnectionItemRes S1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalS1-ConnectionItem	CRITICALITY reject	TYPE UE-associatedLogicalS1-ConnectionItem	PRESENCE mandatory},

...

}

-- **************************************************************

--

-- Reset Acknowledge

--

-- **************************************************************

ResetAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetAcknowledgeIEs} },

...

}

ResetAcknowledgeIEs S1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalS1-ConnectionListResAck		CRITICALITY ignore	TYPE UE-associatedLogicalS1-ConnectionListResAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UE-associatedLogicalS1-ConnectionListResAck ::= SEQUENCE (SIZE(1.. maxnoofIndividualS1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalS1-ConnectionItemResAck } }

UE-associatedLogicalS1-ConnectionItemResAck 	S1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalS1-ConnectionItem	 CRITICALITY ignore 	TYPE UE-associatedLogicalS1-ConnectionItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

-- ERROR INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Error Indication

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ErrorIndicationIEs}},

...

}

ErrorIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID				PRESENCE optional	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID				PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

-- S1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- S1 Setup Request

--

-- **************************************************************

S1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {S1SetupRequestIEs} },

...

}

S1SetupRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Global-ENB-ID				CRITICALITY reject	TYPE Global-ENB-ID				PRESENCE mandatory}|

{ ID id-eNBname						CRITICALITY ignore	TYPE ENBname					PRESENCE optional}|

{ ID id-SupportedTAs				CRITICALITY reject	TYPE SupportedTAs				PRESENCE mandatory}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX					PRESENCE mandatory}|

{ ID id-CSG-IdList					CRITICALITY reject	TYPE CSG-IdList					PRESENCE optional}|

{ ID id-UE-RetentionInformation		CRITICALITY ignore	TYPE UE-RetentionInformation	PRESENCE optional}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX	PRESENCE optional},

...

}

-- **************************************************************

--

-- S1 Setup Response

--

-- **************************************************************

S1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {S1SetupResponseIEs} },

...

}

S1SetupResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MMEname						CRITICALITY ignore	TYPE MMEname					PRESENCE optional}|

{ ID id-ServedGUMMEIs				CRITICALITY reject	TYPE ServedGUMMEIs				PRESENCE mandatory}|

{ ID id-RelativeMMECapacity			CRITICALITY ignore	TYPE RelativeMMECapacity		PRESENCE mandatory}|

{ ID id-MMERelaySupportIndicator	CRITICALITY ignore	TYPE MMERelaySupportIndicator	PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-UE-RetentionInformation		CRITICALITY ignore	TYPE UE-RetentionInformation	PRESENCE optional}|

{ ID id-PSP-list					CRITICALITY ignore	TYPE PSP-list				PRESENCE optional},

...

}

-- **************************************************************

--

-- S1 Setup Failure

--

-- **************************************************************

S1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {S1SetupFailureIEs} },

...

}

S1SetupFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

-- ENB CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- eNB Configuration Update

--

-- **************************************************************

ENBConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ENBConfigurationUpdateIEs} },

...

}

ENBConfigurationUpdateIEs S1AP-PROTOCOL-IES ::= {

{ ID id-eNBname						CRITICALITY ignore	TYPE ENBname					PRESENCE optional}|

{ ID id-SupportedTAs				CRITICALITY reject	TYPE SupportedTAs				PRESENCE optional}|

{ ID id-CSG-IdList					CRITICALITY reject	TYPE CSG-IdList					PRESENCE optional}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX	PRESENCE optional},

...

}

-- **************************************************************

--

-- eNB Configuration Update Acknowledge

--

-- **************************************************************

ENBConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ENBConfigurationUpdateAcknowledgeIEs} },

...

}

ENBConfigurationUpdateAcknowledgeIEs S1AP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

-- eNB Configuration Update Failure

--

-- **************************************************************

ENBConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ENBConfigurationUpdateFailureIEs} },

...

}

ENBConfigurationUpdateFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

-- MME CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- MME Configuration Update

--

-- **************************************************************

MMEConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {MMEConfigurationUpdateIEs} },

...

}

MMEConfigurationUpdateIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MMEname					CRITICALITY ignore	TYPE MMEname				PRESENCE optional	}|

{ ID id-ServedGUMMEIs			CRITICALITY reject	TYPE ServedGUMMEIs			PRESENCE optional	}|

{ ID id-RelativeMMECapacity		CRITICALITY reject	TYPE RelativeMMECapacity	PRESENCE optional	}|

{ ID id-PSP-list				CRITICALITY ignore	TYPE PSP-list				PRESENCE optional	}

,

...

}

-- **************************************************************

--

-- MME Configuration Update Acknowledge

--

-- **************************************************************

MMEConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {MMEConfigurationUpdateAcknowledgeIEs} },

...

}

MMEConfigurationUpdateAcknowledgeIEs S1AP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- MME Configuration Update Failure

--

-- **************************************************************

MMEConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {MMEConfigurationUpdateFailureIEs} },

...

}

MMEConfigurationUpdateFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

-- DOWNLINK S1 CDMA2000 TUNNELLING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Downlink S1 CDMA2000 Tunnelling

--

-- **************************************************************

DownlinkS1cdma2000tunnelling ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {DownlinkS1cdma2000tunnellingIEs} },

...

}

DownlinkS1cdma2000tunnellingIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID						CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID						CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-E-RABSubjecttoDataForwardingList	CRITICALITY ignore	TYPE E-RABSubjecttoDataForwardingList	PRESENCE optional	}|

{ ID id-cdma2000HOStatus					CRITICALITY ignore	TYPE Cdma2000HOStatus					PRESENCE optional	}|

{ ID id-cdma2000RATType						CRITICALITY reject	TYPE Cdma2000RATType					PRESENCE mandatory	}|

{ ID id-cdma2000PDU							CRITICALITY reject	TYPE Cdma2000PDU						PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- UPLINK S1 CDMA2000 TUNNELLING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Uplink S1 CDMA2000 Tunnelling

--

-- **************************************************************

UplinkS1cdma2000tunnelling ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {UplinkS1cdma2000tunnellingIEs} },

...

}

UplinkS1cdma2000tunnellingIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory	}|

{ ID id-cdma2000RATType							CRITICALITY reject	TYPE Cdma2000RATType					PRESENCE mandatory	}|

{ ID id-cdma2000SectorID						CRITICALITY reject	TYPE Cdma2000SectorID					PRESENCE mandatory	}|

{ ID id-cdma2000HORequiredIndication			CRITICALITY ignore	TYPE Cdma2000HORequiredIndication		PRESENCE optional	}|

{ ID id-cdma2000OneXSRVCCInfo					CRITICALITY reject	TYPE Cdma2000OneXSRVCCInfo			PRESENCE optional	}|

{ ID id-cdma2000OneXRAND						CRITICALITY reject	TYPE Cdma2000OneXRAND					PRESENCE optional	}|

{ ID id-cdma2000PDU								CRITICALITY reject	TYPE Cdma2000PDU						PRESENCE mandatory	}|

{ ID id-EUTRANRoundTripDelayEstimationInfo		CRITICALITY ignore	TYPE EUTRANRoundTripDelayEstimationInfo		PRESENCE optional	},

-- Extension for Release 9 to assist target HRPD access with the acquisition of the UE --

...

}

-- **************************************************************

--

-- UE CAPABILITY INFO INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Capability Info Indication

--

-- **************************************************************

UECapabilityInfoIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UECapabilityInfoIndicationIEs} },

...

}

UECapabilityInfoIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory}|

{ ID id-UERadioCapabilityForPaging	CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional},

...

}

-- **************************************************************

--

-- eNB STATUS TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- eNB Status Transfer

--

-- **************************************************************

ENBStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ENBStatusTransferIEs} },

...

}

ENBStatusTransferIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID								CRITICALITY reject	TYPE MME-UE-S1AP-ID		PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID								CRITICALITY reject	TYPE ENB-UE-S1AP-ID		PRESENCE mandatory}|

{ ID id-eNB-StatusTransfer-TransparentContainer		CRITICALITY reject	TYPE ENB-StatusTransfer-TransparentContainer	PRESENCE mandatory},

...

}

-- **************************************************************

--

-- MME STATUS TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- MME Status Transfer

--

-- **************************************************************

MMEStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {MMEStatusTransferIEs} },

...

}

MMEStatusTransferIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID								CRITICALITY reject	TYPE MME-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID								CRITICALITY reject	TYPE ENB-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-eNB-StatusTransfer-TransparentContainer		CRITICALITY reject	TYPE ENB-StatusTransfer-TransparentContainer		PRESENCE mandatory},

...

}

-- **************************************************************

--

-- TRACE ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

-- Trace Start

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {TraceStartIEs} },

...

}

TraceStartIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation				CRITICALITY ignore	TYPE TraceActivation			PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- Trace Failure Indication

--

-- **************************************************************

TraceFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {TraceFailureIndicationIEs} },

...

}

TraceFailureIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-E-UTRAN-Trace-ID			CRITICALITY ignore	TYPE E-UTRAN-Trace-ID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- DEACTIVATE TRACE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Deactivate Trace

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DeactivateTraceIEs} },

...

}

DeactivateTraceIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID		CRITICALITY reject	TYPE MME-UE-S1AP-ID		PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID		CRITICALITY reject	TYPE ENB-UE-S1AP-ID		PRESENCE mandatory	}|

{ ID id-E-UTRAN-Trace-ID	CRITICALITY ignore	TYPE E-UTRAN-Trace-ID	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- CELL TRAFFIC TRACE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Cell Traffic Trace

--

-- **************************************************************

CellTrafficTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{ { CellTrafficTraceIEs } },

...

}

CellTrafficTraceIEs S1AP-PROTOCOL-IES ::= {

{ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ID id-E-UTRAN-Trace-ID					CRITICALITY ignore	TYPE E-UTRAN-Trace-ID			PRESENCE mandatory	}|

{ID id-EUTRAN-CGI						CRITICALITY ignore	TYPE EUTRAN-CGI					PRESENCE mandatory	}|

{ID id-TraceCollectionEntityIPAddress	CRITICALITY ignore	TYPE TransportLayerAddress		PRESENCE mandatory	}|

{ID id-PrivacyIndicator					CRITICALITY ignore	TYPE PrivacyIndicator			PRESENCE optional	},

...

}

-- **************************************************************

--

-- LOCATION ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

-- Location Reporting Control

--

-- **************************************************************

LocationReportingControl ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { LocationReportingControlIEs} },

...

}

LocationReportingControlIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID			CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-RequestType				CRITICALITY ignore	TYPE RequestType				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- Location Report Failure Indication

--

-- **************************************************************

LocationReportingFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { LocationReportingFailureIndicationIEs} },

...

}

LocationReportingFailureIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID			CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- Location Report

--

-- **************************************************************

LocationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { LocationReportIEs} },

...

}

LocationReportIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID			CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-EUTRAN-CGI				CRITICALITY ignore	TYPE EUTRAN-CGI					PRESENCE mandatory	}|

{ ID id-TAI						CRITICALITY ignore	TYPE TAI						PRESENCE mandatory	}|

{ ID id-RequestType				CRITICALITY ignore	TYPE RequestType				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- OVERLOAD ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

-- Overload Start

--

-- **************************************************************

OverloadStart ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {OverloadStartIEs} },

...

}

OverloadStartIEs S1AP-PROTOCOL-IES ::= {

{ ID id-OverloadResponse					CRITICALITY reject	TYPE OverloadResponse				PRESENCE mandatory	}|

{ ID id-GUMMEIList							CRITICALITY ignore	TYPE GUMMEIList						PRESENCE optional	}|

{ ID id-TrafficLoadReductionIndication		CRITICALITY ignore	TYPE TrafficLoadReductionIndication	PRESENCE optional	},

...

}

-- **************************************************************

--

-- Overload Stop

--

-- **************************************************************

OverloadStop ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {OverloadStopIEs} },

...

}

OverloadStopIEs S1AP-PROTOCOL-IES ::= {

{ ID id-GUMMEIList								CRITICALITY ignore	TYPE GUMMEIList						PRESENCE optional	},

...

}

-- **************************************************************

--

-- WRITE-REPLACE WARNING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Write-Replace Warning Request

--

-- **************************************************************

WriteReplaceWarningRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {WriteReplaceWarningRequestIEs} },

...

}

WriteReplaceWarningRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier					CRITICALITY reject	TYPE MessageIdentifier					PRESENCE mandatory	}|

{ ID id-SerialNumber						CRITICALITY reject	TYPE SerialNumber						PRESENCE mandatory	}|

{ ID id-WarningAreaList						CRITICALITY ignore	TYPE WarningAreaList					PRESENCE optional	}|

{ ID id-RepetitionPeriod					CRITICALITY reject	TYPE RepetitionPeriod					PRESENCE mandatory	}|

{ ID id-ExtendedRepetitionPeriod			CRITICALITY reject	TYPE ExtendedRepetitionPeriod			PRESENCE optional	}|

{ ID id-NumberofBroadcastRequest			CRITICALITY reject	TYPE NumberofBroadcastRequest			PRESENCE mandatory	}|

{ ID id-WarningType							CRITICALITY ignore	TYPE WarningType						PRESENCE optional	}|

{ ID id-WarningSecurityInfo					CRITICALITY ignore	TYPE WarningSecurityInfo				PRESENCE optional	}|

{ ID id-DataCodingScheme					CRITICALITY ignore	TYPE DataCodingScheme					PRESENCE optional	}|

{ ID id-WarningMessageContents				CRITICALITY ignore	TYPE WarningMessageContents				PRESENCE optional	}|

{ ID id-ConcurrentWarningMessageIndicator	CRITICALITY reject	TYPE ConcurrentWarningMessageIndicator	PRESENCE optional	},

...

}

-- **************************************************************

--

-- Write-Replace Warning Response

--

-- **************************************************************

WriteReplaceWarningResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {WriteReplaceWarningResponseIEs} },

...

}

WriteReplaceWarningResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier				CRITICALITY reject	TYPE MessageIdentifier					PRESENCE mandatory	}|

{ ID id-SerialNumber					CRITICALITY reject	TYPE SerialNumber						PRESENCE mandatory	}|

{ ID id-BroadcastCompletedAreaList		CRITICALITY ignore	TYPE BroadcastCompletedAreaList			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- eNB DIRECT INFORMATION TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- eNB Direct Information Transfer

--

-- **************************************************************

ENBDirectInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ENBDirectInformationTransferIEs}},

...

}

ENBDirectInformationTransferIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Inter-SystemInformationTransferTypeEDT	CRITICALITY reject	TYPE Inter-SystemInformationTransferType		PRESENCE mandatory	},

...

}

Inter-SystemInformationTransferType ::= CHOICE {

rIMTransfer		RIMTransfer,

...

}

-- **************************************************************

--

-- MME DIRECT INFORMATION TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- MME Direct Information Transfer

--

-- **************************************************************

MMEDirectInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MMEDirectInformationTransferIEs}},

...

}

MMEDirectInformationTransferIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Inter-SystemInformationTransferTypeMDT	CRITICALITY reject	TYPE Inter-SystemInformationTransferType	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- eNB CONFIGURATION TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- eNB Configuration Transfer

--

-- **************************************************************

ENBConfigurationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ENBConfigurationTransferIEs}},

...

}

ENBConfigurationTransferIEs S1AP-PROTOCOL-IES ::= {

{ ID id-SONConfigurationTransferECT		CRITICALITY ignore	TYPE SONConfigurationTransfer	PRESENCE optional	},

...

}

-- **************************************************************

--

-- MME CONFIGURATION TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- MME Configuration Transfer

--

-- **************************************************************

MMEConfigurationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MMEConfigurationTransferIEs}},

...

}

MMEConfigurationTransferIEs S1AP-PROTOCOL-IES ::= {

{ ID id-SONConfigurationTransferMCT		CRITICALITY ignore	TYPE SONConfigurationTransfer	PRESENCE optional	},

...

}

-- **************************************************************

--

-- PRIVATE MESSAGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Private Message

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs			PrivateIE-Container       {{PrivateMessageIEs}},

...

}

PrivateMessageIEs S1AP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

-- KILL PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Kill Request

--

-- **************************************************************

KillRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {KillRequestIEs} },

...

}

KillRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier			CRITICALITY reject	TYPE MessageIdentifier		PRESENCE mandatory}|

{ ID id-SerialNumber				CRITICALITY reject	TYPE SerialNumber			PRESENCE mandatory}|

{ ID id-WarningAreaList				CRITICALITY ignore	TYPE WarningAreaList		PRESENCE optional}|

{ ID id-KillAllWarningMessages		CRITICALITY reject	TYPE KillAllWarningMessages	PRESENCE optional},

...

}

-- **************************************************************

--

-- Kill Response

--

-- **************************************************************

KillResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {KillResponseIEs} },

...

}

KillResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier			CRITICALITY reject	TYPE MessageIdentifier					PRESENCE mandatory	}|

{ ID id-SerialNumber				CRITICALITY reject	TYPE SerialNumber						PRESENCE mandatory	}|

{ ID id-BroadcastCancelledAreaList	CRITICALITY ignore	TYPE BroadcastCancelledAreaList			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- PWS RESTART INDICATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- PWS Restart Indication

--

-- **************************************************************

PWSRestartIndication::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PWSRestartIndicationIEs}},

...

}

PWSRestartIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-ECGIListForRestart				CRITICALITY reject	TYPE ECGIListForRestart					PRESENCE mandatory}|

{ ID id-Global-ENB-ID					CRITICALITY reject	TYPE Global-ENB-ID						PRESENCE mandatory}|

{ ID id-TAIListForRestart				CRITICALITY reject	TYPE TAIListForRestart					PRESENCE mandatory}|

{ ID id-EmergencyAreaIDListForRestart	CRITICALITY reject	TYPE EmergencyAreaIDListForRestart	PRESENCE optional},

...

}

-- **************************************************************

--

-- PWS Failure Indication

--

-- **************************************************************

PWSFailureIndication::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PWSFailureIndicationIEs}},

...

}

PWSFailureIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-PWSfailedECGIList		CRITICALITY reject	TYPE PWSfailedECGIList	PRESENCE mandatory}|

{ ID id-Global-ENB-ID			CRITICALITY reject	TYPE Global-ENB-ID		PRESENCE mandatory},

...

}

-- **************************************************************

--

-- LPPA TRANSPORT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

-- DOWNLINK UE ASSOCIATED LPPA TRANSPORT

--

-- **************************************************************

DownlinkUEAssociatedLPPaTransport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{DownlinkUEAssociatedLPPaTransport-IEs}},

...

}

DownlinkUEAssociatedLPPaTransport-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID			CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-Routing-ID				CRITICALITY reject	TYPE Routing-ID					PRESENCE mandatory	}|

{ ID id-LPPa-PDU    			CRITICALITY reject	TYPE LPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- UPLINK UE ASSOCIATED LPPA TRANSPORT

--

-- **************************************************************

UplinkUEAssociatedLPPaTransport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{UplinkUEAssociatedLPPaTransport-IEs}},

...

}

UplinkUEAssociatedLPPaTransport-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID			CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-Routing-ID				CRITICALITY reject	TYPE Routing-ID					PRESENCE mandatory	}|

{ ID id-LPPa-PDU				CRITICALITY reject	TYPE LPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- DOWNLINK NON UE ASSOCIATED LPPA TRANSPORT

--

-- **************************************************************

DownlinkNonUEAssociatedLPPaTransport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{DownlinkNonUEAssociatedLPPaTransport-IEs}},

...

}

DownlinkNonUEAssociatedLPPaTransport-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-Routing-ID				CRITICALITY reject	TYPE Routing-ID					PRESENCE mandatory	}|

{ ID id-LPPa-PDU				CRITICALITY reject	TYPE LPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- UPLINK NON UE ASSOCIATED LPPA TRANSPORT

--

-- **************************************************************

UplinkNonUEAssociatedLPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container       {{UplinkNonUEAssociatedLPPaTransport-IEs}},

...

}

UplinkNonUEAssociatedLPPaTransport-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-Routing-ID				CRITICALITY reject	TYPE Routing-ID					PRESENCE mandatory	}|

{ ID id-LPPa-PDU				CRITICALITY reject	TYPE LPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- E-RAB MODIFICATION INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- E-RAB Modification Indication

--

-- **************************************************************

E-RABModificationIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { E-RABModificationIndicationIEs} },

...

}

E-RABModificationIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-E-RABToBeModifiedListBearerModInd		CRITICALITY reject	TYPE E-RABToBeModifiedListBearerModInd		PRESENCE mandatory}|

{ ID id-E-RABNotToBeModifiedListBearerModInd	CRITICALITY reject	TYPE E-RABNotToBeModifiedListBearerModInd	PRESENCE optional}|

{ ID id-CSGMembershipInfo						CRITICALITY reject	TYPE CSGMembershipInfo					PRESENCE optional}|

-- Extension for Release 11 to support BBAI --

{ ID id-Tunnel-Information-for-BBF				CRITICALITY ignore	TYPE TunnelInformation					PRESENCE optional},

...

}

E-RABToBeModifiedListBearerModInd ::= E-RAB-IE-ContainerList { {E-RABToBeModifiedItemBearerModIndIEs} }

E-RABToBeModifiedItemBearerModIndIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABToBeModifiedItemBearerModInd		CRITICALITY reject	TYPE E-RABToBeModifiedItemBearerModInd		PRESENCE mandatory},

...

}

E-RABToBeModifiedItemBearerModInd ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

transportLayerAddress			TransportLayerAddress,

dL-GTP-TEID						GTP-TEID,

iE-Extensions					ProtocolExtensionContainer { { E-RABToBeModifiedItemBearerModInd-ExtIEs} }			OPTIONAL,

...

}

E-RABToBeModifiedItemBearerModInd-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABNotToBeModifiedListBearerModInd ::= E-RAB-IE-ContainerList { {E-RABNotToBeModifiedItemBearerModIndIEs} }

E-RABNotToBeModifiedItemBearerModIndIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABNotToBeModifiedItemBearerModInd		CRITICALITY reject	TYPE E-RABNotToBeModifiedItemBearerModInd		PRESENCE mandatory},

...

}

E-RABNotToBeModifiedItemBearerModInd ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

transportLayerAddress			TransportLayerAddress,

dL-GTP-TEID						GTP-TEID,

iE-Extensions					ProtocolExtensionContainer { { E-RABNotToBeModifiedItemBearerModInd-ExtIEs} }		OPTIONAL,

...

}

E-RABNotToBeModifiedItemBearerModInd-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CSGMembershipInfo ::= SEQUENCE {

cSGMembershipStatus		CSGMembershipStatus,

cSG-Id					CSG-Id,

cellAccessMode			CellAccessMode	OPTIONAL,

pLMNidentity			PLMNidentity	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { CSGMembershipInfo-ExtIEs} }		OPTIONAL,

...

}

CSGMembershipInfo-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- E-RAB Modification Confirm

--

-- **************************************************************

E-RABModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container { {E-RABModificationConfirmIEs} },

...

}

E-RABModificationConfirmIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID							CRITICALITY ignore	TYPE MME-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID							CRITICALITY ignore	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-E-RABModifyListBearerModConf			CRITICALITY ignore	TYPE E-RABModifyListBearerModConf	PRESENCE optional}|

{ ID id-E-RABFailedToModifyListBearerModConf	CRITICALITY ignore	TYPE E-RABList						PRESENCE optional}|

{ ID id-E-RABToBeReleasedListBearerModConf		CRITICALITY ignore	TYPE E-RABList						PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-CSGMembershipStatus						CRITICALITY ignore	TYPE CSGMembershipStatus			PRESENCE optional},

...

}

E-RABModifyListBearerModConf ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABModifyItemBearerModConfIEs} }

E-RABModifyItemBearerModConfIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABModifyItemBearerModConf		CRITICALITY ignore	TYPE E-RABModifyItemBearerModConf		PRESENCE mandatory},

...

}

E-RABModifyItemBearerModConf ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

iE-Extensions				ProtocolExtensionContainer { {E-RABModifyItemBearerModConfExtIEs} } OPTIONAL,

...

}

E-RABModifyItemBearerModConfExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- UE CONTEXT MODIFICATION INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Context Modification Indication

--

-- **************************************************************

UEContextModificationIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationIndicationIEs} },

...

}

UEContextModificationIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID			CRITICALITY reject	TYPE ENB-UE-S1AP-ID			PRESENCE mandatory}|

{ ID id-CSGMembershipInfo		CRITICALITY reject	TYPE CSGMembershipInfo		PRESENCE optional},

...

}

-- **************************************************************

--

-- UE Context Modification Confirm

--

-- **************************************************************

UEContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container { {UEContextModificationConfirmIEs} },

...

}

UEContextModificationConfirmIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-CSGMembershipStatus			CRITICALITY ignore	TYPE CSGMembershipStatus		PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

-- UE CONTEXT SUSPEND ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Context Suspend Request

--

-- **************************************************************

UEContextSuspendRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSuspendRequestIEs} },

...

}

UEContextSuspendRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID									CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID									CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-InformationOnRecommendedCellsAndENBsForPaging	CRITICALITY ignore	TYPE InformationOnRecommendedCellsAndENBsForPaging PRESENCE optional}|

{ ID id-CellIdentifierAndCELevelForCECapableUEs			CRITICALITY ignore	TYPE CellIdentifierAndCELevelForCECapableUEs	PRESENCE optional},

...

}

-- **************************************************************

--

-- UE Context Suspend Response

--

-- **************************************************************

UEContextSuspendResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container { {UEContextSuspendResponseIEs} },

...

}

UEContextSuspendResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

-- UE CONTEXT RESUME ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Context Resume Request

--

-- **************************************************************

UEContextResumeRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextResumeRequestIEs} },

...

}

UEContextResumeRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID										CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID									CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-E-RABFailedToResumeListResumeReq 				CRITICALITY reject	TYPE E-RABFailedToResumeListResumeReq		PRESENCE optional}|

{ ID id-RRC-Resume-Cause								CRITICALITY ignore	TYPE RRC-Establishment-Cause	PRESENCE optional},

...

}

E-RABFailedToResumeListResumeReq ::= E-RAB-IE-ContainerList { {E-RABFailedToResumeItemResumeReqIEs} }

E-RABFailedToResumeItemResumeReqIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABFailedToResumeItemResumeReq	CRITICALITY reject	TYPE E-RABFailedToResumeItemResumeReq	PRESENCE mandatory},

...

}

E-RABFailedToResumeItemResumeReq ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

cause								Cause,

iE-Extensions						ProtocolExtensionContainer { { E-RABFailedToResumeItemResumeReq-ExtIEs} }		OPTIONAL,

...

}

E-RABFailedToResumeItemResumeReq-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- UE Context Resume Response

--

-- **************************************************************

UEContextResumeResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextResumeResponseIEs} },

...

}

UEContextResumeResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID						CRITICALITY ignore	TYPE MME-UE-S1AP-ID							PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID						CRITICALITY ignore	TYPE ENB-UE-S1AP-ID							PRESENCE mandatory}|

{ ID id-E-RABFailedToResumeListResumeRes	CRITICALITY reject	TYPE E-RABFailedToResumeListResumeRes		PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional},

...

}

E-RABFailedToResumeListResumeRes ::= E-RAB-IE-ContainerList { {E-RABFailedToResumeItemResumeResIEs} }

E-RABFailedToResumeItemResumeResIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABFailedToResumeItemResumeRes	CRITICALITY reject	TYPE E-RABFailedToResumeItemResumeRes	PRESENCE mandatory},

...

}

E-RABFailedToResumeItemResumeRes ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

cause								Cause,

iE-Extensions						ProtocolExtensionContainer { { E-RABFailedToResumeItemResumeRes-ExtIEs} }		OPTIONAL,

...

}

E-RABFailedToResumeItemResumeRes-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- UE Context Resume Failure

--

-- **************************************************************

UEContextResumeFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextResumeFailureIEs} },

...

}

UEContextResumeFailureIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID						CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID						CRITICALITY ignore	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional},

...

}

-- **************************************************************

--

-- Connection Establishment Indication

--

-- **************************************************************

ConnectionEstablishmentIndication::= SEQUENCE {

protocolIEs			ProtocolIE-Container { {ConnectionEstablishmentIndicationIEs} },

...

}

ConnectionEstablishmentIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY ignore	TYPE MME-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY ignore	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability			PRESENCE optional},

...

}

END

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RABInformationListItem,

id-E-RABItem,

id-Bearers-SubjectToStatusTransfer-Item,

id-Time-Synchronisation-Info,

id-x2TNLConfigurationInfo,

id-eNBX2ExtendedTransportLayerAddresses,

id-MDTConfiguration,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-HO-Cause,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-SignallingBasedMDTPLMNList,

id-MobilityInformation,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-eNBIndirectX2TransportLayerAddresses,

id-Muting-Availability-Indication,

id-Muting-Pattern-Information,

id-Synchronisation-Information,

id-uE-HistoryInformationFromTheUE,

id-LoggedMBSFNMDT,

id-SON-Information-Report,

id-RecommendedCellItem,

id-RecommendedENBItem,

id-ProSeUEtoNetworkRelaying,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-M6Configuration,

id-M7Configuration,

id-RAT-Type,

maxnoofCSGs,

maxnoofE-RABs,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofPLMNsPerMME,

maxnoofTACs,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofForbLACs,

maxnoofForbTACs,

maxnoofCells,

maxnoofCellID,

maxnoofEmergencyAreaID,

maxnoofTAIforWarning,

maxnoofCellinTAI,

maxnoofCellinEAI,

maxnoofeNBX2TLAs,

maxnoofeNBX2ExtTLAs,

maxnoofeNBX2GTPTLAs,

maxnoofRATs,

maxnoofGroupIDs,

maxnoofMMECs,

maxnoofTAforMDT,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofCellsforRestart,

maxnoofRestartTAIs,

maxnoofRestartEmergencyAreaIDs,

maxnoofMBSFNAreaMDT,

maxEARFCN,

maxnoofCellsineNB,

maxnoofRecommendedCells,

maxnoofRecommendedENBs,

maxnoofPSPInfo

FROM S1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM S1AP-CommonDataTypes

ProtocolExtensionContainer{},

S1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

S1AP-PROTOCOL-IES

FROM S1AP-Containers;

-- A

Additional-GUTI::= SEQUENCE {

gUMMEI					GUMMEI,

m-TMSI					M-TMSI,

iE-Extensions			ProtocolExtensionContainer { {Additional-GUTI-ExtIEs} } OPTIONAL,

...

}

Additional-GUTI-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

AreaScopeOfMDT ::= CHOICE {

cellBased					CellBasedMDT,

tABased						TABasedMDT,

pLMNWide					NULL,

...,

tAIBased					TAIBasedMDT

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForCECapableUEs ::= SEQUENCE {

cellIdentifierAndCELevelForCECapableUEs		CellIdentifierAndCELevelForCECapableUEs,

iE-Extensions								ProtocolExtensionContainer { { InformationForCECapableUEs-ExtIEs} } OPTIONAL,

...

}

InformationForCECapableUEs-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForPaging ::= SEQUENCE {

assistanceDataForRecommendedCells	AssistanceDataForRecommendedCells		OPTIONAL,

assistanceDataForCECapableUEs		AssistanceDataForCECapableUEs			OPTIONAL,

pagingAttemptInformation			PagingAttemptInformation				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { AssistanceDataForPaging-ExtIEs} }	OPTIONAL,

...

}

AssistanceDataForPaging-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForRecommendedCells ::= SEQUENCE {

recommendedCellsForPaging	RecommendedCellsForPaging,

iE-Extensions				ProtocolExtensionContainer { { AssistanceDataForRecommendedCells-ExtIEs} }	OPTIONAL,

...

}

AssistanceDataForRecommendedCells-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- B

Bearers-SubjectToStatusTransferList ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { { Bearers-SubjectToStatusTransfer-ItemIEs } }

Bearers-SubjectToStatusTransfer-ItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-Bearers-SubjectToStatusTransfer-Item	CRITICALITY ignore	TYPE Bearers-SubjectToStatusTransfer-Item 	PRESENCE mandatory	},

...

}

Bearers-SubjectToStatusTransfer-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

uL-COUNTvalue							COUNTvalue,

dL-COUNTvalue							COUNTvalue,

receiveStatusofULPDCPSDUs				ReceiveStatusofULPDCPSDUs			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {Bearers-SubjectToStatusTransfer-ItemExtIEs} } OPTIONAL,

...

}

Bearers-SubjectToStatusTransfer-ItemExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ULCOUNTValueExtended						CRITICALITY ignore	EXTENSION COUNTValueExtended				PRESENCE optional}|

{ ID id-DLCOUNTValueExtended						CRITICALITY ignore	EXTENSION COUNTValueExtended				PRESENCE optional}|

{ ID id-ReceiveStatusOfULPDCPSDUsExtended			CRITICALITY ignore	EXTENSION ReceiveStatusOfULPDCPSDUsExtended		PRESENCE optional}|

{ ID id-ULCOUNTValuePDCP-SNlength18					CRITICALITY ignore	EXTENSION COUNTvaluePDCP-SNlength18			PRESENCE optional}|

{ ID id-DLCOUNTValuePDCP-SNlength18					CRITICALITY ignore	EXTENSION COUNTvaluePDCP-SNlength18			PRESENCE optional}|

{ ID id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18	CRITICALITY ignore	EXTENSION ReceiveStatusOfULPDCPSDUsPDCP-SNlength18	PRESENCE optional},

...

}

BearerType ::= ENUMERATED {

non-IP,

...

}

BitRate	::= INTEGER (0..10000000000)

BPLMNs ::= SEQUENCE (SIZE(1.. maxnoofBPLMNs)) OF PLMNidentity

BroadcastCancelledAreaList ::= CHOICE {

cellID-Cancelled				CellID-Cancelled,

tAI-Cancelled					TAI-Cancelled,

emergencyAreaID-Cancelled		EmergencyAreaID-Cancelled,

...

}

BroadcastCompletedAreaList ::= CHOICE {

cellID-Broadcast				CellID-Broadcast,

tAI-Broadcast					TAI-Broadcast,

emergencyAreaID-Broadcast		EmergencyAreaID-Broadcast,

...

}

-- C

CancelledCellinEAI ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CancelledCellinEAI-Item

CancelledCellinEAI-Item ::= SEQUENCE {

eCGI					EUTRAN-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions			ProtocolExtensionContainer { {CancelledCellinEAI-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellinEAI-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CancelledCellinTAI ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CancelledCellinTAI-Item

CancelledCellinTAI-Item ::= SEQUENCE{

eCGI				EUTRAN-CGI,

numberOfBroadcasts	NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellinTAI-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellinTAI-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

nas					CauseNas,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

unknown-PLMN,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

tx2relocoverall-expiry,

successful-handover,

release-due-to-eutran-generated-reason,

handover-cancelled,

partial-handover,

ho-failure-in-target-EPC-eNB-or-target-system,

ho-target-not-allowed,

tS1relocoverall-expiry,

tS1relocprep-expiry,

cell-not-available,

unknown-targetID,

no-radio-resources-available-in-target-cell,

unknown-mme-ue-s1ap-id,

unknown-enb-ue-s1ap-id,

unknown-pair-ue-s1ap-id,

handover-desirable-for-radio-reason,

time-critical-handover,

resource-optimisation-handover,

reduce-load-in-serving-cell,

user-inactivity,

radio-connection-with-ue-lost,

load-balancing-tau-required,

cs-fallback-triggered,

ue-not-available-for-ps-service,

radio-resources-not-available,

failure-in-radio-interface-procedure,

invalid-qos-combination,

interrat-redirection,

interaction-with-other-procedure,

unknown-E-RAB-ID,

multiple-E-RAB-ID-instances,

encryption-and-or-integrity-protection-algorithms-not-supported,

s1-intra-system-handover-triggered,

s1-inter-system-handover-triggered,

x2-handover-triggered,

...,

redirection-towards-1xRTT,

not-supported-QCI-value,

invalid-CSG-Id

}

CauseTransport ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

CauseNas ::= ENUMERATED {

normal-release,

authentication-failure,

detach,

unspecified,

...,

csg-subscription-expiry

}

CellAccessMode ::= ENUMERATED {

hybrid,

...

}

CellIdentifierAndCELevelForCECapableUEs ::= SEQUENCE {

global-Cell-ID		EUTRAN-CGI,

cELevel				CELevel,

iE-Extensions		ProtocolExtensionContainer { { CellIdentifierAndCELevelForCECapableUEs-ExtIEs} } OPTIONAL,

...

}

CellIdentifierAndCELevelForCECapableUEs-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- Coverage Enhancement level encoded according to MFA TS 36.331 [16] --

CELevel ::= OCTET STRING

CE-mode-B-SupportIndicator ::= ENUMERATED {

supported,

...

}

CellIdentity ::= BIT STRING (SIZE (28))

CellID-Broadcast ::= SEQUENCE (SIZE(1..maxnoofCellID)) OF CellID-Broadcast-Item

CellID-Broadcast-Item ::= SEQUENCE {

eCGI				EUTRAN-CGI,

iE-Extensions		ProtocolExtensionContainer { {CellID-Broadcast-Item-ExtIEs} } OPTIONAL,

...

}

CellID-Broadcast-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CellID-Cancelled::= SEQUENCE (SIZE(1..maxnoofCellID)) OF CellID-Cancelled-Item

CellID-Cancelled-Item ::= SEQUENCE {

eCGI				EUTRAN-CGI,

numberOfBroadcasts	NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CellID-Cancelled-Item-ExtIEs} } OPTIONAL,

...

}

CellID-Cancelled-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CellBasedMDT::= SEQUENCE {

cellIdListforMDT	CellIdListforMDT,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforMDT ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF EUTRAN-CGI

Cdma2000PDU	::= OCTET STRING

Cdma2000RATType ::= ENUMERATED {

hRPD,

onexRTT,

...

}

Cdma2000SectorID ::= OCTET STRING

Cdma2000HOStatus ::= ENUMERATED {

hOSuccess,

hOFailure,

...

}

Cdma2000HORequiredIndication ::= ENUMERATED {

true,

...

}

Cdma2000OneXSRVCCInfo ::= SEQUENCE {

cdma2000OneXMEID			Cdma2000OneXMEID,

cdma2000OneXMSI				Cdma2000OneXMSI,

cdma2000OneXPilot			Cdma2000OneXPilot,

iE-Extensions				ProtocolExtensionContainer { {Cdma2000OneXSRVCCInfo-ExtIEs} } OPTIONAL,

...

}

Cdma2000OneXSRVCCInfo-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

Cdma2000OneXMEID ::= OCTET STRING

Cdma2000OneXMSI ::= OCTET STRING

Cdma2000OneXPilot ::= OCTET STRING

Cdma2000OneXRAND ::= OCTET STRING

Cell-Size ::= ENUMERATED {verysmall, small, medium, large, ...}

CellType ::= SEQUENCE {

cell-Size				Cell-Size,

iE-Extensions			ProtocolExtensionContainer { { CellType-ExtIEs}}	OPTIONAL,

...

}

CellType-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CGI ::= SEQUENCE {

pLMNidentity	PLMNidentity,

lAC				LAC,

cI				CI,

rAC				RAC												OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { {CGI-ExtIEs} }		OPTIONAL,

...

}

CGI-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CI					::= OCTET STRING (SIZE (2))

CNDomain ::= ENUMERATED {

ps,

cs

}

ConcurrentWarningMessageIndicator ::= ENUMERATED {

true

}

Correlation-ID		::= OCTET STRING (SIZE (4))

CSFallbackIndicator ::= ENUMERATED {

cs-fallback-required,

...,

cs-fallback-high-priority

}

AdditionalCSFallbackIndicator ::= ENUMERATED {

no-restriction,

restriction,

...

}

CSG-Id		::= BIT STRING (SIZE (27))

CSG-IdList ::= SEQUENCE (SIZE (1.. maxnoofCSGs)) OF CSG-IdList-Item

CSG-IdList-Item ::= SEQUENCE {

cSG-Id			CSG-Id,

iE-Extensions	ProtocolExtensionContainer { {CSG-IdList-Item-ExtIEs} }	OPTIONAL,

...

}

CSG-IdList-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CSGMembershipStatus ::= ENUMERATED {

member,

not-member

}

COUNTvalue ::= SEQUENCE {

pDCP-SN			PDCP-SN,

hFN				HFN,

iE-Extensions	ProtocolExtensionContainer { {COUNTvalue-ExtIEs} } OPTIONAL,

...

}

COUNTvalue-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

COUNTValueExtended  ::= SEQUENCE {

pDCP-SNExtended		PDCP-SNExtended,

hFNModified			HFNModified,

iE-Extensions		ProtocolExtensionContainer { {COUNTValueExtended-ExtIEs} } OPTIONAL,

...

}

COUNTValueExtended-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

COUNTvaluePDCP-SNlength18 ::= SEQUENCE {

pDCP-SNlength18				PDCP-SNlength18,

hFNforPDCP-SNlength18		HFNforPDCP-SNlength18,

iE-Extensions				ProtocolExtensionContainer { {COUNTvaluePDCP-SNlength18-ExtIEs} } OPTIONAL,

...

}

COUNTvaluePDCP-SNlength18-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode														OPTIONAL,

triggeringMessage				TriggeringMessage													OPTIONAL,

procedureCriticality			Criticality															OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer {{CriticalityDiagnostics-ExtIEs}}		OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1.. maxnoofErrors)) OF CriticalityDiagnostics-IE-Item

CriticalityDiagnostics-IE-Item ::= SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError 			TypeOfError,

iE-Extensions			ProtocolExtensionContainer {{CriticalityDiagnostics-IE-Item-ExtIEs}}	OPTIONAL,

...

}

CriticalityDiagnostics-IE-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- D

DataCodingScheme ::= BIT STRING (SIZE (8))

DL-Forwarding ::= ENUMERATED {

dL-Forwarding-proposed,

...

}

Direct-Forwarding-Path-Availability ::= ENUMERATED {

directPathAvailable,

...

}

Data-Forwarding-Not-Possible ::= ENUMERATED {

data-Forwarding-not-Possible,

...

}

-- E

EARFCN ::= INTEGER(0..maxEARFCN, ...)

ECGIList ::= SEQUENCE (SIZE(1..maxnoofCellID)) OF EUTRAN-CGI

PWSfailedECGIList ::= SEQUENCE (SIZE(1..maxnoofCellsineNB)) OF EUTRAN-CGI

EmergencyAreaIDList ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaID

EmergencyAreaID ::= OCTET STRING (SIZE (3))

EmergencyAreaID-Broadcast ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaID-Broadcast-Item

EmergencyAreaID-Broadcast-Item ::= SEQUENCE {

emergencyAreaID			EmergencyAreaID,

completedCellinEAI		CompletedCellinEAI,

iE-Extensions			ProtocolExtensionContainer { {EmergencyAreaID-Broadcast-Item-ExtIEs} }	OPTIONAL,

...

}

EmergencyAreaID-Broadcast-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaID-Cancelled ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaID-Cancelled-Item

EmergencyAreaID-Cancelled-Item ::= SEQUENCE {

emergencyAreaID			EmergencyAreaID,

cancelledCellinEAI		CancelledCellinEAI,

iE-Extensions			ProtocolExtensionContainer { {EmergencyAreaID-Cancelled-Item-ExtIEs} }	OPTIONAL,

...

}

EmergencyAreaID-Cancelled-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

CompletedCellinEAI ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CompletedCellinEAI-Item

CompletedCellinEAI-Item ::= SEQUENCE {

eCGI					EUTRAN-CGI,

iE-Extensions			ProtocolExtensionContainer { {CompletedCellinEAI-Item-ExtIEs} }	OPTIONAL,

...

}

CompletedCellinEAI-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ECGI-List ::= SEQUENCE (SIZE(1..maxnoofCellsineNB)) OF EUTRAN-CGI

EmergencyAreaIDListForRestart	::= SEQUENCE (SIZE(1..maxnoofRestartEmergencyAreaIDs)) OF EmergencyAreaID

ENB-ID ::= CHOICE {

macroENB-ID			BIT STRING (SIZE(20)),

homeENB-ID			BIT STRING (SIZE(28)),

...

}

GERAN-Cell-ID ::= SEQUENCE {

lAI				LAI,

rAC				RAC,

cI				CI,

iE-Extensions			ProtocolExtensionContainer { { GERAN-Cell-ID-ExtIEs} }	OPTIONAL,

...

}

GERAN-Cell-ID-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

Global-ENB-ID ::= SEQUENCE {

pLMNidentity			PLMNidentity,

eNB-ID					ENB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalENB-ID-ExtIEs} }		OPTIONAL,

...

}

GlobalENB-ID-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

GUMMEIList::= SEQUENCE (SIZE (1.. maxnoofMMECs)) OF GUMMEI

ENB-StatusTransfer-TransparentContainer		::= SEQUENCE {

bearers-SubjectToStatusTransferList		Bearers-SubjectToStatusTransferList,

iE-Extensions			ProtocolExtensionContainer { {ENB-StatusTransfer-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

ENB-StatusTransfer-TransparentContainer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-UE-S1AP-ID				::= INTEGER (0..16777215)

ENBname ::= PrintableString (SIZE (1..150,...))

ENBX2TLAs ::= SEQUENCE (SIZE(1.. maxnoofeNBX2TLAs)) OF TransportLayerAddress

EncryptionAlgorithms ::= BIT STRING (SIZE (16,...))

EPLMNs ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMNidentity

EventType	::= ENUMERATED {

direct,

change-of-serve-cell,

stop-change-of-serve-cell,

...

}

E-RAB-ID		::= INTEGER (0..15, ...)

E-RABInformationList	::= SEQUENCE (SIZE (1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { { E-RABInformationListIEs } }

E-RABInformationListIEs S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABInformationListItem			CRITICALITY ignore	TYPE E-RABInformationListItem			PRESENCE mandatory	},

...

}

E-RABInformationListItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding					DL-Forwarding		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABInformationListItem-ExtIEs} }			OPTIONAL,

...

}

E-RABInformationListItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABList ::= SEQUENCE (SIZE(1.. maxnoofE-RABs)) OF ProtocolIE-SingleContainer { {E-RABItemIEs} }

E-RABItemIEs 	S1AP-PROTOCOL-IES ::= {

{ ID id-E-RABItem	 CRITICALITY ignore 	TYPE E-RABItem 	PRESENCE mandatory },

...

}

E-RABItem ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

cause						Cause,

iE-Extensions				ProtocolExtensionContainer { {E-RABItem-ExtIEs} } OPTIONAL,

...

}

E-RABItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABLevelQoSParameters ::= SEQUENCE {

qCI			QCI,

allocationRetentionPriority		AllocationAndRetentionPriority,

gbrQosInformation					GBR-QosInformation												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {E-RABQoSParameters-ExtIEs} }	OPTIONAL,

...

}

E-RABQoSParameters-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-CGI ::= SEQUENCE {

pLMNidentity			PLMNidentity,

cell-ID					CellIdentity,

iE-Extensions			ProtocolExtensionContainer { {EUTRAN-CGI-ExtIEs} } OPTIONAL,

...

}

EUTRAN-CGI-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRANRoundTripDelayEstimationInfo ::= INTEGER (0..2047)

ExpectedUEBehaviour ::= SEQUENCE {

expectedActivity		ExpectedUEActivityBehaviour OPTIONAL,

expectedHOInterval		ExpectedHOInterval		 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExpectedUEBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod					ExpectedActivityPeriod					OPTIONAL,

expectedIdlePeriod						ExpectedIdlePeriod						OPTIONAL,

sourceofUEActivityBehaviourInformation	SourceOfUEActivityBehaviourInformation	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExpectedUEActivityBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExtendedRNC-ID					::= INTEGER (4096..65535)

ExtendedRepetitionPeriod ::= INTEGER (4096..131071)

Extended-UEIdentityIndexValue ::= BIT STRING (SIZE (14))

-- F

ForbiddenInterRATs ::= ENUMERATED {

all,

geran,

utran,

cdma2000,

...,

geranandutran,

cdma2000andutran

}

ForbiddenTAs ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ForbiddenTAs-Item

ForbiddenTAs-Item ::= SEQUENCE {

pLMN-Identity		PLMNidentity,

forbiddenTACs		ForbiddenTACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenTAs-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenTAs-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenTACs ::= SEQUENCE (SIZE(1..maxnoofForbTACs)) OF TAC

ForbiddenLAs ::= SEQUENCE (SIZE(1..maxnoofEPLMNsPlusOne)) OF ForbiddenLAs-Item

ForbiddenLAs-Item ::= SEQUENCE {

pLMN-Identity		PLMNidentity,

forbiddenLACs		ForbiddenLACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenLAs-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenLAs-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenLACs ::= SEQUENCE (SIZE(1..maxnoofForbLACs)) OF LAC

-- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEID					::= OCTET STRING (SIZE (4))

GUMMEI			::= SEQUENCE {

pLMN-Identity		PLMNidentity,

mME-Group-ID		MME-Group-ID,

mME-Code			MME-Code,

iE-Extensions		ProtocolExtensionContainer { {GUMMEI-ExtIEs} } OPTIONAL,

...

}

GUMMEI-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

GUMMEIType ::= ENUMERATED {

native,

mapped,

...

}

GWContextReleaseIndication ::= ENUMERATED {

true,

...

}

-- H

HandoverRestrictionList ::= SEQUENCE {

servingPLMN					PLMNidentity,

equivalentPLMNs				EPLMNs					OPTIONAL,

forbiddenTAs				ForbiddenTAs			OPTIONAL,

forbiddenLAs				ForbiddenLAs			OPTIONAL,

forbiddenInterRATs			ForbiddenInterRATs		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {HandoverRestrictionList-ExtIEs} }	OPTIONAL,

...

}

HandoverRestrictionList-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

HandoverType ::= ENUMERATED {

intralte,

ltetoutran,

ltetogeran,

utrantolte,

gerantolte,

...

}

Hash-PSP-Info ::= SEQUENCE {

hash-PSP-ID					BIT STRING (SIZE(24)),

iE-Extensions				ProtocolExtensionContainer { { Hash-PSP-Info-ExtIEs} } OPTIONAL,

...

}

Hash-PSP-Info-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

HFN ::= INTEGER (0..1048575)

HFNModified ::= INTEGER (0..131071)

HFNforPDCP-SNlength18 ::= INTEGER (0..16383)

-- I

Masked-IMEISV ::= BIT STRING (SIZE (64))

ImmediateMDT ::= SEQUENCE {

measurementsToActivate		MeasurementsToActivate,

m1reportingTrigger			M1ReportingTrigger,

m1thresholdeventA2			M1ThresholdEventA2				OPTIONAL,

-- Included in case of event-triggered, or event-triggered periodic reporting for measurement M1

m1periodicReporting			M1PeriodicReporting				OPTIONAL,

-- Included in case of periodic or event-triggered periodic reporting

iE-Extensions				ProtocolExtensionContainer { { ImmediateMDT-ExtIEs} } OPTIONAL,

...

}

ImmediateMDT-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-M3Configuration		CRITICALITY ignore	EXTENSION M3Configuration		PRESENCE conditional}|

{ ID id-M4Configuration		CRITICALITY ignore	EXTENSION M4Configuration		PRESENCE conditional}|

{ ID id-M5Configuration		CRITICALITY ignore	EXTENSION M5Configuration		PRESENCE conditional}|

{ ID id-MDT-Location-Info	CRITICALITY ignore	EXTENSION MDT-Location-Info		PRESENCE optional}|

{ ID id-M6Configuration		CRITICALITY ignore	EXTENSION M6Configuration		PRESENCE conditional}|

{ ID id-M7Configuration		CRITICALITY ignore	EXTENSION M7Configuration		PRESENCE conditional},

...

}

IMSI	::=	OCTET STRING (SIZE (3..8))

InformationOnRecommendedCellsAndENBsForPaging ::= SEQUENCE {

recommendedCellsForPaging	RecommendedCellsForPaging,

recommendENBsForPaging		RecommendedENBsForPaging,

iE-Extensions				ProtocolExtensionContainer { { InformationOnRecommendedCellsAndENBsForPaging-ExtIEs} }	OPTIONAL,

...

}

InformationOnRecommendedCellsAndENBsForPaging-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

IntegrityProtectionAlgorithms ::= BIT STRING (SIZE (16,...))

IntendedNumberOfPagingAttempts ::= INTEGER (1..16, ...)

InterfacesToTrace ::= BIT STRING (SIZE (8))

-- J

-- K

KillAllWarningMessages ::= ENUMERATED {true}

-- L

LAC	::= OCTET STRING (SIZE (2))

LAI ::= SEQUENCE {

pLMNidentity				PLMNidentity,

lAC				LAC,

iE-Extensions			ProtocolExtensionContainer { {LAI-ExtIEs} } OPTIONAL,

...

}

LAI-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedCell-Item ::= CHOICE {

e-UTRAN-Cell					LastVisitedEUTRANCellInformation,

uTRAN-Cell						LastVisitedUTRANCellInformation,

gERAN-Cell						LastVisitedGERANCellInformation,

...

}

LastVisitedEUTRANCellInformation ::= SEQUENCE {

global-Cell-ID					EUTRAN-CGI,

cellType						CellType,

time-UE-StayedInCell			Time-UE-StayedInCell,

iE-Extensions					ProtocolExtensionContainer { { LastVisitedEUTRANCellInformation-ExtIEs} } OPTIONAL,

...

}

LastVisitedEUTRANCellInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-11 to support enhanced granularity for time UE stayed in cell --

{ ID id-Time-UE-StayedInCell-EnhancedGranularity	CRITICALITY ignore	EXTENSION Time-UE-StayedInCell-EnhancedGranularity	PRESENCE optional}|

{ ID id-HO-Cause									CRITICALITY ignore	EXTENSION Cause							PRESENCE optional},

...

}

LastVisitedUTRANCellInformation	::= OCTET STRING

LastVisitedGERANCellInformation ::= CHOICE {

undefined						NULL,

...

}

L3-Information				::= OCTET STRING

-- This is a dummy IE used only as a reference to the actual definition in relevant specification.

LPPa-PDU ::=  OCTET STRING

LHN-ID ::=  OCTET STRING(SIZE (32..256))

Links-to-log ::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

ListeningSubframePattern ::= SEQUENCE {

pattern-period				ENUMERATED {ms1280, ms2560, ms5120, ms10240, ...},

pattern-offset				INTEGER (0..10239, ...),

iE-Extensions				ProtocolExtensionContainer { { ListeningSubframePattern-ExtIEs} } OPTIONAL,

...

}

ListeningSubframePattern-ExtIEs	S1AP-PROTOCOL-EXTENSION ::= {

...

}

LoggedMDT ::= SEQUENCE {

loggingInterval				LoggingInterval,

loggingDuration				LoggingDuration,

iE-Extensions				ProtocolExtensionContainer { {LoggedMDT-ExtIEs} } OPTIONAL,

...

}

LoggedMDT-ExtIEs	S1AP-PROTOCOL-EXTENSION ::= {

...

}

LoggingInterval ::= ENUMERATED {ms128, ms256, ms512, ms1024, ms2048, ms3072, ms4096, ms6144}

LoggingDuration ::= ENUMERATED {m10, m20, m40, m60, m90, m120}

LoggedMBSFNMDT ::= SEQUENCE {

loggingInterval				LoggingInterval,

loggingDuration				LoggingDuration,

mBSFN-ResultToLog			MBSFN-ResultToLog		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { LoggedMBSFNMDT-ExtIEs } } OPTIONAL,

...

}

LoggedMBSFNMDT-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- M

M3Configuration ::= SEQUENCE {

m3period			M3period,

iE-Extensions		ProtocolExtensionContainer { { M3Configuration-ExtIEs} } OPTIONAL,

...

}

M3Configuration-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

M3period ::= ENUMERATED {ms100, ms1000, ms10000, ... }

M4Configuration ::= SEQUENCE {

m4period			M4period,

m4-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M4Configuration-ExtIEs} } OPTIONAL,

...

}

M4Configuration-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

M5period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6delay-threshold	M6delay-threshold		OPTIONAL,

-- This IE shall be present if the M6 Links to log IE is set to “uplink” or to “both-uplink-and-downlink” --

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

M6report-Interval ::= ENUMERATED { ms1024, ms2048, ms5120, ms10240, ... }

M6delay-threshold ::= ENUMERATED { ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, ... }

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

M7period ::= INTEGER(1..60, ...)

MDT-Activation 	::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

logged-MDT-only,

...,

logged-MBSFN-MDT

}

MDT-Location-Info ::= BIT STRING (SIZE (8))

MDT-Configuration ::= SEQUENCE {

mdt-Activation		MDT-Activation,

areaScopeOfMDT		AreaScopeOfMDT,

mDTMode				MDTMode,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SignallingBasedMDTPLMNList			CRITICALITY ignore	EXTENSION MDTPLMNList	PRESENCE optional	},

...

}

ManagementBasedMDTAllowed ::= ENUMERATED {allowed, ...}

MBSFN-ResultToLog ::= SEQUENCE (SIZE(1..maxnoofMBSFNAreaMDT)) OF MBSFN-ResultToLogInfo

MBSFN-ResultToLogInfo ::= SEQUENCE {

mBSFN-AreaId		INTEGER (0..255)		OPTIONAL,

carrierFreq			EARFCN,

iE-Extensions		ProtocolExtensionContainer { { MBSFN-ResultToLogInfo-ExtIEs} } OPTIONAL,

...

}

MBSFN-ResultToLogInfo-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMNidentity

PrivacyIndicator ::= ENUMERATED {

immediate-MDT,

logged-MDT,

...

}

MDTMode ::= CHOICE {

immediateMDT				ImmediateMDT,

loggedMDT					LoggedMDT,

...,

mDTMode-Extension			MDTMode-Extension

}

MDTMode-Extension ::= ProtocolIE-SingleContainer {{ MDTMode-ExtensionIE }}

MDTMode-ExtensionIE S1AP-PROTOCOL-IES ::= {

{ ID id-LoggedMBSFNMDT		CRITICALITY ignore	TYPE LoggedMBSFNMDT		PRESENCE mandatory}

}

MeasurementsToActivate ::= BIT STRING (SIZE (8))

MeasurementThresholdA2 ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

...

}

MessageIdentifier	::= BIT STRING (SIZE (16))

MobilityInformation ::= BIT STRING (SIZE(32))

MMEname ::= PrintableString (SIZE (1..150,...))

MMEPagingTarget ::= CHOICE {

global-ENB-ID		Global-ENB-ID,

tAI					TAI,

...

}

MMERelaySupportIndicator ::= ENUMERATED {true, ...}

MME-Group-ID	::= OCTET STRING (SIZE (2))

MME-Code		::= OCTET STRING (SIZE (1))

MME-UE-S1AP-ID	::= INTEGER (0..4294967295)

M-TMSI			::= OCTET STRING (SIZE (4))

MSClassmark2	::= OCTET STRING

MSClassmark3	::= OCTET STRING

MutingAvailabilityIndication ::= ENUMERATED {

available,

unavailable,

...

}

MutingPatternInformation ::= SEQUENCE {

muting-pattern-period				ENUMERATED {ms0, ms1280, ms2560, ms5120, ms10240, ...},

muting-pattern-offset				INTEGER (0..10239, ...)		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {MutingPatternInformation-ExtIEs} } OPTIONAL,

...

}

MutingPatternInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- N

NAS-PDU ::=  OCTET STRING

NASSecurityParametersfromE-UTRAN ::= OCTET STRING

NASSecurityParameterstoE-UTRAN ::= OCTET STRING

NB-IoT-DefaultPagingDRX ::= ENUMERATED {

v128,

v256,

v512,

v1024,

...

}

NB-IoT-Paging-eDRXInformation ::= SEQUENCE {

nB-IoT-paging-eDRX-Cycle		NB-IoT-Paging-eDRX-Cycle,

nB-IoT-pagingTimeWindow			NB-IoT-PagingTimeWindow			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { NB-IoT-Paging-eDRXInformation-ExtIEs} } OPTIONAL,

...

}

NB-IoT-Paging-eDRXInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

NB-IoT-Paging-eDRX-Cycle ::= ENUMERATED{hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, ...}

NB-IoT-PagingTimeWindow ::= ENUMERATED{s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, ...}

NB-IoT-UEIdentityIndexValue ::= BIT STRING (SIZE (12))

NextPagingAreaScope ::= ENUMERATED {

same,

changed,

...

}

NumberofBroadcastRequest ::= INTEGER (0..65535)

NumberOfBroadcasts ::= INTEGER (0..65535)

-- O

OID-PSP-Info ::= SEQUENCE {

oID-PSP-ID					BIT STRING (SIZE(24)),

iE-Extensions				ProtocolExtensionContainer { { OID-PSP-Info-ExtIEs} } OPTIONAL,

...

}

OID-PSP-Info-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

OldBSS-ToNewBSS-Information		::= OCTET STRING

-- This is a dummy IE used only as a reference to the actual definition in relevant specification.

OverloadAction ::= ENUMERATED {

reject-non-emergency-mo-dt,

reject-rrc-cr-signalling,

permit-emergency-sessions-and-mobile-terminated-services-only,

...,

permit-high-priority-sessions-and-mobile-terminated-services-only,

reject-delay-tolerant-access

}

OverloadResponse ::= CHOICE {

overloadAction					OverloadAction,

...

}

-- P

PagingAttemptInformation ::= SEQUENCE {

pagingAttemptCount					PagingAttemptCount,

intendedNumberOfPagingAttempts		IntendedNumberOfPagingAttempts,

nextPagingAreaScope					NextPagingAreaScope		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PagingAttemptInformation-ExtIEs} } OPTIONAL,

...

}

PagingAttemptInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

PagingAttemptCount ::= INTEGER (1..16, ...)

Paging-eDRXInformation ::= SEQUENCE {

paging-eDRX-Cycle			Paging-eDRX-Cycle,

pagingTimeWindow			PagingTimeWindow			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Paging-eDRXInformation-ExtIEs} } OPTIONAL,

...

}

Paging-eDRXInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

Paging-eDRX-Cycle ::= ENUMERATED{hfhalf, hf1, hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, ...}

PagingTimeWindow ::= ENUMERATED{s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, ...}

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

PDCP-SN ::= INTEGER (0..4095)

PDCP-SNExtended ::= INTEGER (0..32767)

PDCP-SNlength18 ::= INTEGER (0..262143)

M1PeriodicReporting ::= SEQUENCE {

reportInterval				ReportIntervalMDT,

reportAmount				ReportAmountMDT,

iE-Extensions				ProtocolExtensionContainer { { M1PeriodicReporting-ExtIEs} } OPTIONAL,

...

}

M1PeriodicReporting-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

PLMNidentity 				::= TBCD-STRING

PLMN-PSP-Info ::= SEQUENCE {

pLMN-PSP-ID						BIT STRING (SIZE(24)),

pLMN-PSP-S2a-Support-Indicator		PLMN-PSP-S2a-Support-Indicator OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PLMN-PSP-Info-ExtIEs} } OPTIONAL,

...

}

PLMN-PSP-Info-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

PLMN-PSP-S2a-Support-Indicator ::= ENUMERATED {

true,

...

}

Port-Number		::= OCTET STRING (SIZE (2))

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel				::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

ProSeAuthorized ::= SEQUENCE {

proSeDirectDiscovery		ProSeDirectDiscovery									OPTIONAL,

proSeDirectCommunication	ProSeDirectCommunication								OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {ProSeAuthorized-ExtIEs} }	OPTIONAL,

...

}

ProSeAuthorized-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ProSeUEtoNetworkRelaying	CRITICALITY ignore	EXTENSION ProSeUEtoNetworkRelaying		PRESENCE optional},

...

}

ProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

ProSeUEtoNetworkRelaying ::= ENUMERATED {

authorized,

not-authorized,

...

}

ProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

PSP-Info ::= CHOICE {

pLMN-PSP-Info	PLMN-PSP-Info,

oID-PSP-Info	OID-PSP-Info,

hash-PSP-Info	Hash-PSP-Info,

...

}

PSP-list ::= SEQUENCE (SIZE(1..maxnoofPSPInfo)) OF PSP-Info

PS-ServiceNotAvailable ::= ENUMERATED {

ps-service-not-available,

...

}

-- Q

QCI						::= INTEGER (0..255)

-- R

ReceiveStatusofULPDCPSDUs ::= BIT STRING (SIZE(4096))

ReceiveStatusOfULPDCPSDUsExtended ::= BIT STRING (SIZE(1..16384))

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18 ::= BIT STRING (SIZE(1..131072))

RecommendedCellsForPaging ::= SEQUENCE {

recommendedCellList			RecommendedCellList,

iE-Extensions				ProtocolExtensionContainer { { RecommendedCellsForPaging-ExtIEs} }	OPTIONAL,

...

}

RecommendedCellsForPaging-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedCellList ::= SEQUENCE (SIZE(1.. maxnoofRecommendedCells)) OF ProtocolIE-SingleContainer { { RecommendedCellItemIEs } }

RecommendedCellItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-RecommendedCellItem	CRITICALITY ignore 	TYPE RecommendedCellItem		PRESENCE mandatory },

...

}

RecommendedCellItem::= SEQUENCE {

eUTRAN-CGI				EUTRAN-CGI,

timeStayedInCell		INTEGER (0..4095)		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RecommendedCellsForPagingItem-ExtIEs} }	OPTIONAL,

...

}

RecommendedCellsForPagingItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedENBsForPaging ::= SEQUENCE {

recommendedENBList		RecommendedENBList,

iE-Extensions			ProtocolExtensionContainer { { RecommendedENBsForPaging-ExtIEs} }	OPTIONAL,

...

}

RecommendedENBsForPaging-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedENBList::= SEQUENCE (SIZE(1.. maxnoofRecommendedENBs)) OF ProtocolIE-SingleContainer { { RecommendedENBItemIEs } }

RecommendedENBItemIEs S1AP-PROTOCOL-IES ::= {

{ ID id-RecommendedENBItem	CRITICALITY ignore	TYPE RecommendedENBItem		PRESENCE mandatory },

...

}

RecommendedENBItem ::= SEQUENCE {

mMEPagingTarget			MMEPagingTarget,

iE-Extensions			ProtocolExtensionContainer { { RecommendedENBItem-ExtIEs} }	OPTIONAL,

...

}

RecommendedENBItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

RelativeMMECapacity				::= INTEGER (0..255)

RelayNode-Indicator ::= ENUMERATED {

true,

...

}

RAC					::= OCTET STRING (SIZE (1))

RAT-Type ::= ENUMERATED {

nbiot,

...

}

ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, rinfinity}

ReportIntervalMDT ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60}

M1ReportingTrigger ::= ENUMERATED{

periodic,

a2eventtriggered,

...,

a2eventtriggered-periodic

}

RequestType	::= SEQUENCE {

eventType 				EventType,

reportArea 				ReportArea,

iE-Extensions			ProtocolExtensionContainer { { RequestType-ExtIEs} }	OPTIONAL,

...

}

RequestType-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

RIMTransfer ::= SEQUENCE {

rIMInformation			RIMInformation,

rIMRoutingAddress		RIMRoutingAddress		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RIMTransfer-ExtIEs} }	OPTIONAL,

...

}

RIMTransfer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

RIMInformation ::= OCTET STRING

RIMRoutingAddress ::= CHOICE {

gERAN-Cell-ID			GERAN-Cell-ID,

...,

targetRNC-ID			TargetRNC-ID,

eHRPD-Sector-ID			OCTET STRING (SIZE(16))

}

ReportArea ::= ENUMERATED {

ecgi,

...

}

RepetitionPeriod ::= INTEGER (0..4095)

RLFReportInformation ::= SEQUENCE {

uE-RLF-Report-Container							UE-RLF-Report-Container,

uE-RLF-Report-Container-for-extended-bands		UE-RLF-Report-Container-for-extended-bands		OPTIONAL,

iE-Extensions									ProtocolExtensionContainer {{ RLFReportInformation-ExtIEs}} OPTIONAL,

...

}

RLFReportInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

RNC-ID ::= INTEGER (0..4095)

RRC-Container ::= OCTET STRING

RRC-Establishment-Cause ::= ENUMERATED {

emergency,

highPriorityAccess,

mt-Access,

mo-Signalling,

mo-Data,

...,

delay-TolerantAccess,

mo-VoiceCall,

mo-ExceptionData

}

ECGIListForRestart ::= SEQUENCE (SIZE(1..maxnoofCellsforRestart)) OF EUTRAN-CGI

Routing-ID ::= INTEGER (0..255)

-- S

SecurityKey	::= BIT STRING (SIZE(256))

SecurityContext ::= SEQUENCE {

nextHopChainingCount		INTEGER (0..7),

nextHopParameter			SecurityKey,

iE-Extensions				ProtocolExtensionContainer { { SecurityContext-ExtIEs} }	OPTIONAL,

...

}

SecurityContext-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

SerialNumber ::= BIT STRING (SIZE (16))

SONInformation ::= CHOICE{

sONInformationRequest		SONInformationRequest,

sONInformationReply			SONInformationReply,

...,

sONInformation-Extension	SONInformation-Extension

}

SONInformation-Extension ::= ProtocolIE-SingleContainer {{ SONInformation-ExtensionIE }}

SONInformation-ExtensionIE S1AP-PROTOCOL-IES ::= {

{ ID id-SON-Information-Report	CRITICALITY ignore	TYPE SONInformationReport	PRESENCE mandatory}

}

SONInformationRequest ::= ENUMERATED {

x2TNL-Configuration-Info,

...,

time-Synchronisation-Info,

activate-Muting,

deactivate-Muting}

SONInformationReply ::= SEQUENCE {

x2TNLConfigurationInfo			X2TNLConfigurationInfo			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer {{SONInformationReply-ExtIEs}} OPTIONAL,

...

}

SONInformationReply-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 9 to transfer Time synchronisation information --

{ID id-Time-Synchronisation-Info		CRITICALITY ignore	EXTENSION TimeSynchronisationInfo		PRESENCE optional},

...,

{ID id-Muting-Pattern-Information		CRITICALITY ignore	EXTENSION MutingPatternInformation	PRESENCE optional}

}

SONInformationReport ::= CHOICE{

rLFReportInformation		RLFReportInformation,

...

}

SONConfigurationTransfer ::= SEQUENCE {

targeteNB-ID					TargeteNB-ID,

sourceeNB-ID					SourceeNB-ID,

sONInformation					SONInformation,

iE-Extensions			ProtocolExtensionContainer { { SONConfigurationTransfer-ExtIEs} }			OPTIONAL,

...

}

SONConfigurationTransfer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 10 to transfer the IP addresses of the eNB initiating the ANR action --

{ID id-x2TNLConfigurationInfo		CRITICALITY ignore	EXTENSION X2TNLConfigurationInfo				PRESENCE conditional

-- This IE shall be present if the SON Information IE contains the SON Information Request IE and the SON Information Request IE is set to “X2TNL Configuration Info” --}|

-- Extension for Release 12 to transfer information concerning the source cell of synchronisation and the aggressor cell --

{ID id-Synchronisation-Information	CRITICALITY ignore	EXTENSION SynchronisationInformation			PRESENCE conditional

-- This IE shall be present if the SON Information IE contains the SON Information Request IE set to “ Activate Muting ” --},

...

}

SynchronisationInformation ::= SEQUENCE {

sourceStratumLevel				StratumLevel				OPTIONAL,

listeningSubframePattern		ListeningSubframePattern	OPTIONAL,

aggressoreCGI-List				ECGI-List					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {SynchronisationInformation-ExtIEs} } OPTIONAL,

...

}

SynchronisationInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

Source-ToTarget-TransparentContainer ::= OCTET STRING

-- This IE includes a transparent container from the source RAN node to the target RAN node.

-- The octets of the OCTET STRING are encoded according to the specifications of the target system.

SourceBSS-ToTargetBSS-TransparentContainer		::= OCTET STRING

-- This is a dummy IE used only as a reference to the actual definition in relevant specification.

SourceeNB-ID ::= SEQUENCE {

global-ENB-ID	Global-ENB-ID,

selected-TAI	TAI,

iE-Extensions	ProtocolExtensionContainer { {SourceeNB-ID-ExtIEs} } OPTIONAL

}

SourceeNB-ID-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

SRVCCOperationNotPossible ::= ENUMERATED {

notPossible,

...

}

SRVCCOperationPossible ::= ENUMERATED {

possible,

...

}

SRVCCHOIndication ::= ENUMERATED {

pSandCS,

cSonly,

...

}

SourceeNB-ToTargeteNB-TransparentContainer		::= SEQUENCE {

rRC-Container				RRC-Container,

e-RABInformationList		E-RABInformationList			OPTIONAL,

targetCell-ID				EUTRAN-CGI,

subscriberProfileIDforRFP	SubscriberProfileIDforRFP		OPTIONAL,

uE-HistoryInformation		UE-HistoryInformation,

iE-Extensions				ProtocolExtensionContainer { {SourceeNB-ToTargeteNB-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

SourceeNB-ToTargeteNB-TransparentContainer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ID id-MobilityInformation				CRITICALITY ignore	EXTENSION MobilityInformation				PRESENCE optional}|

{ID id-uE-HistoryInformationFromTheUE	CRITICALITY ignore	EXTENSION UE-HistoryInformationFromTheUE		PRESENCE optional},

...

}

SourceRNC-ToTargetRNC-TransparentContainer		::= OCTET STRING

-- This is a dummy IE used only as a reference to the actual definition in relevant specification.

ServedGUMMEIs ::= SEQUENCE (SIZE (1.. maxnoofRATs)) OF ServedGUMMEIsItem

ServedGUMMEIsItem ::= SEQUENCE {

servedPLMNs				ServedPLMNs,

servedGroupIDs			ServedGroupIDs,

servedMMECs				ServedMMECs,

iE-Extensions			ProtocolExtensionContainer { {ServedGUMMEIsItem-ExtIEs} }	OPTIONAL,

...

}

ServedGUMMEIsItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ServedGroupIDs ::= SEQUENCE (SIZE(1.. maxnoofGroupIDs)) OF MME-Group-ID

ServedMMECs ::= SEQUENCE (SIZE(1.. maxnoofMMECs)) OF MME-Code

ServedPLMNs ::= SEQUENCE (SIZE(1.. maxnoofPLMNsPerMME)) OF PLMNidentity

SubscriberProfileIDforRFP ::= INTEGER (1..256)

SupportedTAs ::= SEQUENCE (SIZE(1.. maxnoofTACs)) OF SupportedTAs-Item

SupportedTAs-Item ::=	SEQUENCE  {

tAC					TAC,

broadcastPLMNs		BPLMNs,

iE-Extensions		ProtocolExtensionContainer { {SupportedTAs-Item-ExtIEs} } OPTIONAL,

...

}

SupportedTAs-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 13 to transfer RAT-Type per TAC --

{ID id-RAT-Type		CRITICALITY reject	EXTENSION RAT-Type		PRESENCE optional},

...

}

StratumLevel ::= INTEGER (0..3, ...)

SynchronisationStatus ::= ENUMERATED { synchronous, asynchronous, ... }

TimeSynchronisationInfo ::= SEQUENCE {

stratumLevel					StratumLevel,

synchronisationStatus			SynchronisationStatus,

iE-Extensions					ProtocolExtensionContainer { { TimeSynchronisationInfo-ExtIEs} } OPTIONAL,

...

}

TimeSynchronisationInfo-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 12 to transfer Muting Availability Indication --

{ID id-Muting-Availability-Indication		CRITICALITY ignore	EXTENSION MutingAvailabilityIndication	PRESENCE optional},

...

}

S-TMSI ::= SEQUENCE {

mMEC	MME-Code,

m-TMSI	M-TMSI,

iE-Extensions		ProtocolExtensionContainer { {S-TMSI-ExtIEs} } OPTIONAL,

...

}

S-TMSI-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

-- T

TAC ::= OCTET STRING (SIZE (2))

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAI

TAIListforWarning ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAI

TAI ::= SEQUENCE {

pLMNidentity			PLMNidentity,

tAC						TAC,

iE-Extensions			ProtocolExtensionContainer { {TAI-ExtIEs} } OPTIONAL,

...

}

TAI-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TAI-Broadcast ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAI-Broadcast-Item

TAI-Broadcast-Item ::= SEQUENCE {

tAI					TAI,

completedCellinTAI	CompletedCellinTAI,

iE-Extensions		ProtocolExtensionContainer { {TAI-Broadcast-Item-ExtIEs} } OPTIONAL,

...

}

TAI-Broadcast-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TAI-Cancelled ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAI-Cancelled-Item

TAI-Cancelled-Item ::= SEQUENCE {

tAI					TAI,

cancelledCellinTAI	CancelledCellinTAI,

iE-Extensions		ProtocolExtensionContainer { {TAI-Cancelled-Item-ExtIEs} } OPTIONAL,

...

}

TAI-Cancelled-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

CompletedCellinTAI ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CompletedCellinTAI-Item

CompletedCellinTAI-Item ::= SEQUENCE{

eCGI				EUTRAN-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellinTAI-Item-ExtIEs} } OPTIONAL,

...

}

CompletedCellinTAI-Item-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TBCD-STRING ::= OCTET STRING (SIZE (3))

TargetID ::= CHOICE {

targeteNB-ID		TargeteNB-ID,

targetRNC-ID		TargetRNC-ID,

cGI					CGI,

...

}

TargeteNB-ID ::= SEQUENCE {

global-ENB-ID		Global-ENB-ID,

selected-TAI		TAI,

iE-Extensions		ProtocolExtensionContainer { {TargeteNB-ID-ExtIEs} } OPTIONAL,

...

}

TargeteNB-ID-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TargetRNC-ID ::= SEQUENCE {

lAI					LAI,

rAC					RAC 		OPTIONAL,

rNC-ID				RNC-ID,

extendedRNC-ID		ExtendedRNC-ID		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TargetRNC-ID-ExtIEs} } OPTIONAL,

...

}

TargetRNC-ID-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TargeteNB-ToSourceeNB-TransparentContainer		::= SEQUENCE {

rRC-Container		RRC-Container,

iE-Extensions		ProtocolExtensionContainer { {TargeteNB-ToSourceeNB-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargeteNB-ToSourceeNB-TransparentContainer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

Target-ToSource-TransparentContainer ::= OCTET STRING

-- This IE includes a transparent container from the target RAN node to the source RAN node.

-- The octets of the OCTET STRING are coded according to the specifications of the target system.

TargetRNC-ToSourceRNC-TransparentContainer		::= OCTET STRING

-- This is a dummy IE used only as a reference to the actual definition in relevant specification.

TargetBSS-ToSourceBSS-TransparentContainer		::= OCTET STRING

-- This is a dummy IE used only as a reference to the actual definition in relevant specification.

M1ThresholdEventA2 ::= SEQUENCE {

measurementThreshold	MeasurementThresholdA2,

iE-Extensions			ProtocolExtensionContainer { { M1ThresholdEventA2-ExtIEs} } OPTIONAL,

...

}

M1ThresholdEventA2-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

Threshold-RSRP ::= INTEGER(0..97)

Threshold-RSRQ ::= INTEGER(0..34)

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

Time-UE-StayedInCell ::= INTEGER (0..4095)

Time-UE-StayedInCell-EnhancedGranularity ::= INTEGER (0..40950)

TransportInformation ::= SEQUENCE {

transportLayerAddress				TransportLayerAddress,

uL-GTP-TEID							GTP-TEID,

...

}

TransportLayerAddress		::= BIT STRING (SIZE(1..160, ...))

TraceActivation ::= SEQUENCE {

e-UTRAN-Trace-ID					E-UTRAN-Trace-ID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions						ProtocolExtensionContainer { { TraceActivation-ExtIEs} }	OPTIONAL,

...

}

TraceActivation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-10 to support MDT --

{ ID id-MDTConfiguration	CRITICALITY ignore	EXTENSION MDT-Configuration		PRESENCE optional },

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

E-UTRAN-Trace-ID ::=  OCTET STRING (SIZE (8))

TrafficLoadReductionIndication ::= INTEGER (1..99)

TunnelInformation ::= SEQUENCE {

transportLayerAddress	TransportLayerAddress,

uDP-Port-Number			Port-Number			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {Tunnel-Information-ExtIEs} } OPTIONAL,

...

}

Tunnel-Information-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

TAIListForRestart ::= SEQUENCE (SIZE(1..maxnoofRestartTAIs)) OF TAI

-- U

UEAggregateMaximumBitrate ::= SEQUENCE {

uEaggregateMaximumBitRateDL		BitRate,

uEaggregateMaximumBitRateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { {UEAggregate-MaximumBitrates-ExtIEs} } OPTIONAL,

...

}

UEAggregate-MaximumBitrates-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-RetentionInformation ::= ENUMERATED {

ues-retained,

...}

UE-S1AP-IDs ::= CHOICE{

uE-S1AP-ID-pair		UE-S1AP-ID-pair,

mME-UE-S1AP-ID		MME-UE-S1AP-ID,

...

}

UE-S1AP-ID-pair ::= SEQUENCE{

mME-UE-S1AP-ID		MME-UE-S1AP-ID,

eNB-UE-S1AP-ID		ENB-UE-S1AP-ID,

iE-Extensions		ProtocolExtensionContainer { {UE-S1AP-ID-pair-ExtIEs} } OPTIONAL,

...

}

UE-S1AP-ID-pair-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-associatedLogicalS1-ConnectionItem ::= SEQUENCE {

mME-UE-S1AP-ID		MME-UE-S1AP-ID OPTIONAL,

eNB-UE-S1AP-ID		ENB-UE-S1AP-ID OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UE-associatedLogicalS1-ConnectionItemExtIEs} } OPTIONAL,

...

}

UE-associatedLogicalS1-ConnectionItemExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

UEIdentityIndexValue	::=	BIT STRING (SIZE (10))

UE-HistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCells)) OF LastVisitedCell-Item

UE-HistoryInformationFromTheUE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the VisitedCellInfoList field contained in the UEInformationResponse message as defined in MFA TS 36.331 [16]

UEPagingID ::= CHOICE {

s-TMSI		S-TMSI,

iMSI		IMSI,

...

}

UERadioCapability ::= OCTET STRING

UERadioCapabilityForPaging ::= OCTET STRING

UE-RLF-Report-Container ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the rlf-Report-r9 field contained in the UEInformationResponse message as defined in MFA TS 36.331 [16]

UE-RLF-Report-Container-for-extended-bands ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the rlf-Report-v9e0 contained in the UEInformationResponse message as defined in MFA TS 36.331 [16]

UESecurityCapabilities ::= SEQUENCE {

encryptionAlgorithms			EncryptionAlgorithms,

integrityProtectionAlgorithms	IntegrityProtectionAlgorithms,

iE-Extensions					ProtocolExtensionContainer { { UESecurityCapabilities-ExtIEs} }	OPTIONAL,

...

}

UESecurityCapabilities-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-Usage-Type ::= INTEGER (0..255)

UserLocationInformation ::= SEQUENCE {

eutran-cgi 				EUTRAN-CGI,

tai						TAI,

iE-Extensions			ProtocolExtensionContainer { { UserLocationInformation-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

UEUserPlaneCIoTSupportIndicator ::= ENUMERATED {

supported,

...

}

-- V

VoiceSupportMatchIndicator ::= ENUMERATED {

supported,

not-supported,

...

}

-- W

WarningAreaList ::= CHOICE {

cellIDList						ECGIList,

trackingAreaListforWarning		TAIListforWarning,

emergencyAreaIDList				EmergencyAreaIDList,

...

}

WarningType ::= OCTET STRING (SIZE (2))

WarningSecurityInfo ::= OCTET STRING (SIZE (50))

WarningMessageContents ::= OCTET STRING (SIZE(1..9600))

-- X

X2TNLConfigurationInfo ::= SEQUENCE {

eNBX2TransportLayerAddresses	ENBX2TLAs,

iE-Extensions					ProtocolExtensionContainer { { X2TNLConfigurationInfo-ExtIEs} } OPTIONAL,

...

}

X2TNLConfigurationInfo-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 10 to transfer the IPsec and U-plane addresses during ANR action --

{ID id-eNBX2ExtendedTransportLayerAddresses		CRITICALITY ignore	EXTENSION ENBX2ExtTLAs	PRESENCE optional}|

-- Extension for Release 12 to transfer the IP addresses of the X2 GW --

{ID id-eNBIndirectX2TransportLayerAddresses	CRITICALITY ignore	EXTENSION ENBIndirectX2TransportLayerAddresses	PRESENCE optional},

...

}

ENBX2ExtTLAs ::= SEQUENCE (SIZE(1.. maxnoofeNBX2ExtTLAs)) OF ENBX2ExtTLA

ENBX2ExtTLA ::= SEQUENCE {

iPsecTLA					TransportLayerAddress		OPTIONAL,

gTPTLAa						ENBX2GTPTLAs				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ENBX2ExtTLA-ExtIEs} } OPTIONAL,

...

}

ENBX2ExtTLA-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

ENBX2GTPTLAs ::= SEQUENCE (SIZE(1.. maxnoofeNBX2GTPTLAs)) OF TransportLayerAddress

ENBIndirectX2TransportLayerAddresses ::= SEQUENCE (SIZE(1..maxnoofeNBX2TLAs)) OF TransportLayerAddress

-- Y

-- Z

END

### 9.3.5	Common Definitions

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

S1AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0..65535),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..65535)

ProtocolIE-ID		::= INTEGER (0..65535)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessfull-outcome }

END

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

S1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM S1AP-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-HandoverPreparation						ProcedureCode ::= 0

id-HandoverResourceAllocation				ProcedureCode ::= 1

id-HandoverNotification						ProcedureCode ::= 2

id-PathSwitchRequest						ProcedureCode ::= 3

id-HandoverCancel							ProcedureCode ::= 4

id-E-RABSetup								ProcedureCode ::= 5

id-E-RABModify								ProcedureCode ::= 6

id-E-RABRelease								ProcedureCode ::= 7

id-E-RABReleaseIndication					ProcedureCode ::= 8

id-InitialContextSetup						ProcedureCode ::= 9

id-Paging									ProcedureCode ::= 10

id-downlinkNASTransport						ProcedureCode ::= 11

id-initialUEMessage							ProcedureCode ::= 12

id-uplinkNASTransport						ProcedureCode ::= 13

id-Reset									ProcedureCode ::= 14

id-ErrorIndication							ProcedureCode ::= 15

id-NASNonDeliveryIndication					ProcedureCode ::= 16

id-S1Setup									ProcedureCode ::= 17

id-UEContextReleaseRequest					ProcedureCode ::= 18

id-DownlinkS1cdma2000tunnelling				ProcedureCode ::= 19

id-UplinkS1cdma2000tunnelling				ProcedureCode ::= 20

id-UEContextModification					ProcedureCode ::= 21

id-UECapabilityInfoIndication				ProcedureCode ::= 22

id-UEContextRelease							ProcedureCode ::= 23

id-eNBStatusTransfer						ProcedureCode ::= 24

id-MMEStatusTransfer						ProcedureCode ::= 25

id-DeactivateTrace							ProcedureCode ::= 26

id-TraceStart								ProcedureCode ::= 27

id-TraceFailureIndication					ProcedureCode ::= 28

id-ENBConfigurationUpdate					ProcedureCode ::= 29

id-MMEConfigurationUpdate					ProcedureCode ::= 30

id-LocationReportingControl					ProcedureCode ::= 31

id-LocationReportingFailureIndication		ProcedureCode ::= 32

id-LocationReport							ProcedureCode ::= 33

id-OverloadStart							ProcedureCode ::= 34

id-OverloadStop								ProcedureCode ::= 35

id-WriteReplaceWarning						ProcedureCode ::= 36

id-eNBDirectInformationTransfer				ProcedureCode ::= 37

id-MMEDirectInformationTransfer				ProcedureCode ::= 38

id-PrivateMessage							ProcedureCode ::= 39

id-eNBConfigurationTransfer					ProcedureCode ::= 40

id-MMEConfigurationTransfer					ProcedureCode ::= 41

id-CellTrafficTrace							ProcedureCode ::= 42

id-Kill										ProcedureCode ::= 43

id-downlinkUEAssociatedLPPaTransport		ProcedureCode ::= 44

id-uplinkUEAssociatedLPPaTransport			ProcedureCode ::= 45

id-downlinkNonUEAssociatedLPPaTransport		ProcedureCode ::= 46

id-uplinkNonUEAssociatedLPPaTransport		ProcedureCode ::= 47

id-UERadioCapabilityMatch					ProcedureCode ::= 48

id-PWSRestartIndication						ProcedureCode ::= 49

id-E-RABModificationIndication				ProcedureCode ::= 50

id-PWSFailureIndication						ProcedureCode ::= 51

id-RerouteNASRequest						ProcedureCode ::= 52

id-UEContextModificationIndication			ProcedureCode ::= 53

id-ConnectionEstablishmentIndication		ProcedureCode ::= 54

id-UEContextSuspend							ProcedureCode ::= 55

id-UEContextResume							ProcedureCode ::= 56

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxnoofCSGs								INTEGER ::= 256

maxnoofE-RABs							INTEGER ::= 256

maxnoofTAIs								INTEGER ::= 256

maxnoofTACs								INTEGER ::= 256

maxnoofErrors							INTEGER ::= 256

maxnoofBPLMNs							INTEGER ::= 6

maxnoofPLMNsPerMME						INTEGER ::= 32

maxnoofEPLMNs							INTEGER ::= 15

maxnoofEPLMNsPlusOne					INTEGER ::= 16

maxnoofForbLACs							INTEGER ::= 4096

maxnoofForbTACs							INTEGER ::= 4096

maxnoofIndividualS1ConnectionsToReset	INTEGER ::= 256

maxnoofCells							INTEGER ::= 16

maxnoofCellsineNB						INTEGER ::= 256

maxnoofTAIforWarning					INTEGER ::= 65535

maxnoofCellID							INTEGER ::= 65535

maxnoofEmergencyAreaID					INTEGER ::= 65535

maxnoofCellinTAI						INTEGER ::= 65535

maxnoofCellinEAI						INTEGER ::= 65535

maxnoofeNBX2TLAs						INTEGER ::= 2

maxnoofeNBX2ExtTLAs						INTEGER ::= 16

maxnoofeNBX2GTPTLAs						INTEGER ::= 16

maxnoofRATs								INTEGER ::= 8

maxnoofGroupIDs							INTEGER ::= 65535

maxnoofMMECs							INTEGER ::= 256

maxnoofCellIDforMDT						INTEGER ::= 32

maxnoofTAforMDT							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::= 16

maxnoofCellsforRestart					INTEGER ::= 256

maxnoofRestartTAIs						INTEGER ::= 2048

maxnoofRestartEmergencyAreaIDs			INTEGER ::= 256

maxEARFCN								INTEGER ::= 262143

maxnoofMBSFNAreaMDT						INTEGER ::= 8

maxnoofRecommendedCells					INTEGER ::= 16

maxnoofRecommendedENBs					INTEGER ::= 16

maxnoofPSPInfo							INTEGER ::= 1024

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-MME-UE-S1AP-ID									ProtocolIE-ID ::= 0

id-HandoverType										ProtocolIE-ID ::= 1

id-Cause											ProtocolIE-ID ::= 2

id-SourceID											ProtocolIE-ID ::= 3

id-TargetID											ProtocolIE-ID ::= 4

id-eNB-UE-S1AP-ID									ProtocolIE-ID ::= 8

id-E-RABSubjecttoDataForwardingList					ProtocolIE-ID ::= 12

id-E-RABtoReleaseListHOCmd							ProtocolIE-ID ::= 13

id-E-RABDataForwardingItem							ProtocolIE-ID ::= 14

id-E-RABReleaseItemBearerRelComp					ProtocolIE-ID ::= 15

id-E-RABToBeSetupListBearerSUReq					ProtocolIE-ID ::= 16

id-E-RABToBeSetupItemBearerSUReq					ProtocolIE-ID ::= 17

id-E-RABAdmittedList								ProtocolIE-ID ::= 18

id-E-RABFailedToSetupListHOReqAck					ProtocolIE-ID ::= 19

id-E-RABAdmittedItem								ProtocolIE-ID ::= 20

id-E-RABFailedtoSetupItemHOReqAck					ProtocolIE-ID ::= 21

id-E-RABToBeSwitchedDLList							ProtocolIE-ID ::= 22

id-E-RABToBeSwitchedDLItem							ProtocolIE-ID ::= 23

id-E-RABToBeSetupListCtxtSUReq						ProtocolIE-ID ::= 24

id-TraceActivation									ProtocolIE-ID ::= 25

id-NAS-PDU											ProtocolIE-ID ::= 26

id-E-RABToBeSetupItemHOReq							ProtocolIE-ID ::= 27

id-E-RABSetupListBearerSURes						ProtocolIE-ID ::= 28

id-E-RABFailedToSetupListBearerSURes				ProtocolIE-ID ::= 29

id-E-RABToBeModifiedListBearerModReq				ProtocolIE-ID ::= 30

id-E-RABModifyListBearerModRes						ProtocolIE-ID ::= 31

id-E-RABFailedToModifyList							ProtocolIE-ID ::= 32

id-E-RABToBeReleasedList							ProtocolIE-ID ::= 33

id-E-RABFailedToReleaseList							ProtocolIE-ID ::= 34

id-E-RABItem										ProtocolIE-ID ::= 35

id-E-RABToBeModifiedItemBearerModReq				ProtocolIE-ID ::= 36

id-E-RABModifyItemBearerModRes						ProtocolIE-ID ::= 37

id-E-RABReleaseItem									ProtocolIE-ID ::= 38

id-E-RABSetupItemBearerSURes						ProtocolIE-ID ::= 39

id-SecurityContext									ProtocolIE-ID ::= 40

id-HandoverRestrictionList							ProtocolIE-ID ::= 41

id-UEPagingID										ProtocolIE-ID ::= 43

id-pagingDRX										ProtocolIE-ID ::= 44

id-TAIList											ProtocolIE-ID ::= 46

id-TAIItem											ProtocolIE-ID ::= 47

id-E-RABFailedToSetupListCtxtSURes					ProtocolIE-ID ::= 48

id-E-RABReleaseItemHOCmd							ProtocolIE-ID ::= 49

id-E-RABSetupItemCtxtSURes							ProtocolIE-ID ::= 50

id-E-RABSetupListCtxtSURes							ProtocolIE-ID ::= 51

id-E-RABToBeSetupItemCtxtSUReq						ProtocolIE-ID ::= 52

id-E-RABToBeSetupListHOReq							ProtocolIE-ID ::= 53

id-GERANtoLTEHOInformationRes						ProtocolIE-ID ::= 55

id-UTRANtoLTEHOInformationRes						ProtocolIE-ID ::= 57

id-CriticalityDiagnostics 							ProtocolIE-ID ::= 58

id-Global-ENB-ID									ProtocolIE-ID ::= 59

id-eNBname											ProtocolIE-ID ::= 60

id-MMEname											ProtocolIE-ID ::= 61

id-ServedPLMNs										ProtocolIE-ID ::= 63

id-SupportedTAs										ProtocolIE-ID ::= 64

id-TimeToWait										ProtocolIE-ID ::= 65

id-uEaggregateMaximumBitrate						ProtocolIE-ID ::= 66

id-TAI												ProtocolIE-ID ::= 67

id-E-RABReleaseListBearerRelComp					ProtocolIE-ID ::= 69

id-cdma2000PDU										ProtocolIE-ID ::= 70

id-cdma2000RATType									ProtocolIE-ID ::= 71

id-cdma2000SectorID									ProtocolIE-ID ::= 72

id-SecurityKey										ProtocolIE-ID ::= 73

id-UERadioCapability								ProtocolIE-ID ::= 74

id-GUMMEI-ID										ProtocolIE-ID ::= 75

id-E-RABInformationListItem							ProtocolIE-ID ::= 78

id-Direct-Forwarding-Path-Availability				ProtocolIE-ID ::= 79

id-UEIdentityIndexValue								ProtocolIE-ID ::= 80

id-cdma2000HOStatus									ProtocolIE-ID ::= 83

id-cdma2000HORequiredIndication						ProtocolIE-ID ::= 84

id-E-UTRAN-Trace-ID									ProtocolIE-ID ::= 86

id-RelativeMMECapacity								ProtocolIE-ID ::= 87

id-SourceMME-UE-S1AP-ID								ProtocolIE-ID ::= 88

id-Bearers-SubjectToStatusTransfer-Item				ProtocolIE-ID ::= 89

id-eNB-StatusTransfer-TransparentContainer			ProtocolIE-ID ::= 90

id-UE-associatedLogicalS1-ConnectionItem			ProtocolIE-ID ::= 91

id-ResetType										ProtocolIE-ID ::= 92

id-UE-associatedLogicalS1-ConnectionListResAck		ProtocolIE-ID ::= 93

id-E-RABToBeSwitchedULItem							ProtocolIE-ID ::= 94

id-E-RABToBeSwitchedULList							ProtocolIE-ID ::= 95

id-S-TMSI											ProtocolIE-ID ::= 96

id-cdma2000OneXRAND									ProtocolIE-ID ::= 97

id-RequestType										ProtocolIE-ID ::= 98

id-UE-S1AP-IDs										ProtocolIE-ID ::= 99

id-EUTRAN-CGI										ProtocolIE-ID ::= 100

id-OverloadResponse									ProtocolIE-ID ::= 101

id-cdma2000OneXSRVCCInfo							ProtocolIE-ID ::= 102

id-E-RABFailedToBeReleasedList						ProtocolIE-ID ::= 103

id-Source-ToTarget-TransparentContainer				ProtocolIE-ID ::= 104

id-ServedGUMMEIs									ProtocolIE-ID ::= 105

id-SubscriberProfileIDforRFP						ProtocolIE-ID ::= 106

id-UESecurityCapabilities							ProtocolIE-ID ::= 107

id-CSFallbackIndicator								ProtocolIE-ID ::= 108

id-CNDomain											ProtocolIE-ID ::= 109

id-E-RABReleasedList								ProtocolIE-ID ::= 110

id-MessageIdentifier								ProtocolIE-ID ::= 111

id-SerialNumber										ProtocolIE-ID ::= 112

id-WarningAreaList									ProtocolIE-ID ::= 113

id-RepetitionPeriod									ProtocolIE-ID ::= 114

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 115

id-WarningType										ProtocolIE-ID ::= 116

id-WarningSecurityInfo								ProtocolIE-ID ::= 117

id-DataCodingScheme									ProtocolIE-ID ::= 118

id-WarningMessageContents							ProtocolIE-ID ::= 119

id-BroadcastCompletedAreaList						ProtocolIE-ID ::= 120

id-Inter-SystemInformationTransferTypeEDT			ProtocolIE-ID ::= 121

id-Inter-SystemInformationTransferTypeMDT			ProtocolIE-ID ::= 122

id-Target-ToSource-TransparentContainer				ProtocolIE-ID ::= 123

id-SRVCCOperationPossible							ProtocolIE-ID ::= 124

id-SRVCCHOIndication								ProtocolIE-ID ::= 125

id-NAS-DownlinkCount								ProtocolIE-ID ::= 126

id-CSG-Id											ProtocolIE-ID ::= 127

id-CSG-IdList										ProtocolIE-ID ::= 128

id-SONConfigurationTransferECT						ProtocolIE-ID ::= 129

id-SONConfigurationTransferMCT						ProtocolIE-ID ::= 130

id-TraceCollectionEntityIPAddress					ProtocolIE-ID ::= 131

id-MSClassmark2										ProtocolIE-ID ::= 132

id-MSClassmark3										ProtocolIE-ID ::= 133

id-RRC-Establishment-Cause							ProtocolIE-ID ::= 134

id-NASSecurityParametersfromE-UTRAN					ProtocolIE-ID ::= 135

id-NASSecurityParameterstoE-UTRAN					ProtocolIE-ID ::= 136

id-DefaultPagingDRX									ProtocolIE-ID ::= 137

id-Source-ToTarget-TransparentContainer-Secondary	ProtocolIE-ID ::= 138

id-Target-ToSource-TransparentContainer-Secondary	ProtocolIE-ID ::= 139

id-EUTRANRoundTripDelayEstimationInfo				ProtocolIE-ID ::= 140

id-BroadcastCancelledAreaList						ProtocolIE-ID ::= 141

id-ConcurrentWarningMessageIndicator				ProtocolIE-ID ::= 142

id-Data-Forwarding-Not-Possible						ProtocolIE-ID ::= 143

id-ExtendedRepetitionPeriod							ProtocolIE-ID ::= 144

id-CellAccessMode									ProtocolIE-ID ::= 145

id-CSGMembershipStatus 								ProtocolIE-ID ::= 146

id-LPPa-PDU											ProtocolIE-ID ::= 147

id-Routing-ID										ProtocolIE-ID ::= 148

id-Time-Synchronisation-Info						ProtocolIE-ID ::= 149

id-PS-ServiceNotAvailable							ProtocolIE-ID ::= 150

id-PagingPriority									ProtocolIE-ID ::= 151

id-x2TNLConfigurationInfo							ProtocolIE-ID ::= 152

id-eNBX2ExtendedTransportLayerAddresses				ProtocolIE-ID ::= 153

id-GUMMEIList										ProtocolIE-ID ::= 154

id-GW-TransportLayerAddress							ProtocolIE-ID ::= 155

id-Correlation-ID									ProtocolIE-ID ::= 156

id-SourceMME-GUMMEI									ProtocolIE-ID ::= 157

id-MME-UE-S1AP-ID-2									ProtocolIE-ID ::= 158

id-RegisteredLAI									ProtocolIE-ID ::= 159

id-RelayNode-Indicator								ProtocolIE-ID ::= 160

id-TrafficLoadReductionIndication					ProtocolIE-ID ::= 161

id-MDTConfiguration									ProtocolIE-ID ::= 162

id-MMERelaySupportIndicator							ProtocolIE-ID ::= 163

id-GWContextReleaseIndication						ProtocolIE-ID ::= 164

id-ManagementBasedMDTAllowed						ProtocolIE-ID ::= 165

id-PrivacyIndicator									ProtocolIE-ID ::= 166

id-Time-UE-StayedInCell-EnhancedGranularity			ProtocolIE-ID ::= 167

id-HO-Cause											ProtocolIE-ID ::= 168

id-VoiceSupportMatchIndicator						ProtocolIE-ID ::= 169

id-GUMMEIType										ProtocolIE-ID ::= 170

id-M3Configuration									ProtocolIE-ID ::= 171

id-M4Configuration									ProtocolIE-ID ::= 172

id-M5Configuration									ProtocolIE-ID ::= 173

id-MDT-Location-Info								ProtocolIE-ID ::= 174

id-MobilityInformation								ProtocolIE-ID ::= 175

id-Tunnel-Information-for-BBF						ProtocolIE-ID ::= 176

id-ManagementBasedMDTPLMNList						ProtocolIE-ID ::= 177

id-SignallingBasedMDTPLMNList						ProtocolIE-ID ::= 178

id-ULCOUNTValueExtended								ProtocolIE-ID ::= 179

id-DLCOUNTValueExtended								ProtocolIE-ID ::= 180

id-ReceiveStatusOfULPDCPSDUsExtended				ProtocolIE-ID ::= 181

id-ECGIListForRestart								ProtocolIE-ID ::= 182

id-SIPTO-Correlation-ID								ProtocolIE-ID ::= 183

id-SIPTO-L-GW-TransportLayerAddress					ProtocolIE-ID ::= 184

id-TransportInformation								ProtocolIE-ID ::= 185

id-LHN-ID											ProtocolIE-ID ::= 186

id-AdditionalCSFallbackIndicator					ProtocolIE-ID ::= 187

id-TAIListForRestart								ProtocolIE-ID ::= 188

id-UserLocationInformation							ProtocolIE-ID ::= 189

id-EmergencyAreaIDListForRestart					ProtocolIE-ID ::= 190

id-KillAllWarningMessages							ProtocolIE-ID ::= 191

id-Masked-IMEISV									ProtocolIE-ID ::= 192

id-eNBIndirectX2TransportLayerAddresses				ProtocolIE-ID ::= 193

id-uE-HistoryInformationFromTheUE					ProtocolIE-ID ::= 194

id-ProSeAuthorized									ProtocolIE-ID ::= 195

id-ExpectedUEBehaviour								ProtocolIE-ID ::= 196

id-LoggedMBSFNMDT									ProtocolIE-ID ::= 197

id-UERadioCapabilityForPaging						ProtocolIE-ID ::= 198

id-E-RABToBeModifiedListBearerModInd				ProtocolIE-ID ::= 199

id-E-RABToBeModifiedItemBearerModInd				ProtocolIE-ID ::= 200

id-E-RABNotToBeModifiedListBearerModInd				ProtocolIE-ID ::= 201

id-E-RABNotToBeModifiedItemBearerModInd				ProtocolIE-ID ::= 202

id-E-RABModifyListBearerModConf						ProtocolIE-ID ::= 203

id-E-RABModifyItemBearerModConf						ProtocolIE-ID ::= 204

id-E-RABFailedToModifyListBearerModConf				ProtocolIE-ID ::= 205

id-SON-Information-Report							ProtocolIE-ID ::= 206

id-Muting-Availability-Indication					ProtocolIE-ID ::= 207

id-Muting-Pattern-Information						ProtocolIE-ID ::= 208

id-Synchronisation-Information						ProtocolIE-ID ::= 209

id-E-RABToBeReleasedListBearerModConf				ProtocolIE-ID ::= 210

id-AssistanceDataForPaging							ProtocolIE-ID ::= 211

id-CellIdentifierAndCELevelForCECapableUEs			ProtocolIE-ID ::= 212

id-InformationOnRecommendedCellsAndENBsForPaging	ProtocolIE-ID ::= 213

id-RecommendedCellItem								ProtocolIE-ID ::= 214

id-RecommendedENBItem								ProtocolIE-ID ::= 215

id-ProSeUEtoNetworkRelaying							ProtocolIE-ID ::= 216

id-ULCOUNTValuePDCP-SNlength18						ProtocolIE-ID ::= 217

id-DLCOUNTValuePDCP-SNlength18						ProtocolIE-ID ::= 218

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18			ProtocolIE-ID ::= 219

id-M6Configuration									ProtocolIE-ID ::= 220

id-M7Configuration									ProtocolIE-ID ::= 221

id-PWSfailedECGIList								ProtocolIE-ID ::= 222

id-MME-Group-ID										ProtocolIE-ID ::= 223

id-Additional-GUTI									ProtocolIE-ID ::= 224

id-S1-Message										ProtocolIE-ID ::= 225

id-CSGMembershipInfo								ProtocolIE-ID ::= 226

id-Paging-eDRXInformation							ProtocolIE-ID ::= 227

id-UE-RetentionInformation							ProtocolIE-ID ::= 228

id-UE-Usage-Type									ProtocolIE-ID ::= 230

id-extended-UEIdentityIndexValue					ProtocolIE-ID ::= 231

id-RAT-Type											ProtocolIE-ID ::= 232

id-BearerType										ProtocolIE-ID ::= 233

id-NB-IoT-DefaultPagingDRX							ProtocolIE-ID ::= 234

id-E-RABFailedToResumeListResumeReq					ProtocolIE-ID ::= 235

id-E-RABFailedToResumeItemResumeReq					ProtocolIE-ID ::= 236

id-E-RABFailedToResumeListResumeRes					ProtocolIE-ID ::= 237

id-E-RABFailedToResumeItemResumeRes					ProtocolIE-ID ::= 238

id-NB-IoT-Paging-eDRXInformation					ProtocolIE-ID ::= 239

id-UEUserPlaneCIoTSupportIndicator 					ProtocolIE-ID ::= 241

id-CE-mode-B-SupportIndicator 						ProtocolIE-ID ::= 242

id-SRVCCOperationNotPossible						ProtocolIE-ID ::= 243

id-NB-IoT-UEIdentityIndexValue 						ProtocolIE-ID ::= 244

id-RRC-Resume-Cause									ProtocolIE-ID ::= 245

id-PSP-list											ProtocolIE-ID ::= 1000

END

### 9.3.7	Container Definitions

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

S1AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

Presence,

PrivateIE-ID,

ProtocolExtensionID,

ProtocolIE-ID

FROM S1AP-CommonDataTypes

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs

FROM S1AP-Constants;

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

S1AP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 					UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

S1AP-PROTOCOL-IES-PAIR ::= CLASS {

&id					ProtocolIE-ID 				UNIQUE,

&firstCriticality	Criticality,

&FirstValue,

&secondCriticality	Criticality,

&SecondValue,

&presence			Presence

}

WITH SYNTAX {

ID				&id

FIRST CRITICALITY		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

-- Class Definition for Protocol Extensions

--

-- **************************************************************

S1AP-PROTOCOL-EXTENSION ::= CLASS {

&id				ProtocolExtensionID			UNIQUE,

&criticality	Criticality,

&Extension,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

EXTENSION		&Extension

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

S1AP-PRIVATE-IES ::= CLASS {

&id				PrivateIE-ID,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {S1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer {S1AP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {S1AP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				S1AP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		S1AP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			S1AP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {S1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {S1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					S1AP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	S1AP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			S1AP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	S1AP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			S1AP-PROTOCOL-IES-PAIR.&SecondValue		({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, S1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-SingleContainer {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, S1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

-- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {S1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {S1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					S1AP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			S1AP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		S1AP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {S1AP-PRIVATE-IES : IEsSetParam } ::=

SEQUENCE (SIZE (1.. maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {S1AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id					S1AP-PRIVATE-IES.&id				({IEsSetParam}),

criticality			S1AP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value				S1AP-PRIVATE-IES.&Value				({IEsSetParam}{@id})

}

END

## 9.4	Message Transfer Syntax

S1AP shall use the ASN.1 Basic Packed Encoding Rules (BASIC-PER) Aligned Variant as transfer syntax as specified in ITU-T Rec. X.691 [4].

## 9.5	Timers

TS1RELOCprep

-	Specifies the maximum time for the Handover Preparation procedure in the source eNB.

TS1RELOCoverall

-	Specifies the maximum time for the protection of the overall handover procedure in the source eNB.

TX2RELOCOverall

-	it is specified in 3GPP TS 36.423 [22].

# 10	Handling of Unknown, Unforeseen and Erroneous Protocol Data

## 10.1	General

Protocol Error cases can be divided into three classes:

-	Transfer Syntax Error.

-	Abstract Syntax Error.

-	Logical Error.

Protocol errors can occur in the following functions within a receiving node:

Figure 10.1-1: Protocol Errors in S1AP.

The information stated in subclauses 10.2, 10.3 and 10.4, to be included in the message used when reporting an error, is what at minimum shall be included. Other optional information elements within the message may also be included, if available. This is also valid for the case when the reporting is done with a response message. The latter is an exception to what is stated in subclause 4.1.

## 10.2	Transfer Syntax Error

A Transfer Syntax Error occurs when the receiver is not able to decode the received physical message. Transfer syntax errors are always detected in the process of ASN.1 decoding. If a Transfer Syntax Error occurs, the receiver should initiate Error Indication procedure with appropriate cause value for the Transfer Syntax protocol error.

Examples for Transfer Syntax Errors are:

-	Violation of value ranges in ASN.1 definition of messages. E.g., if an IE has a defined value range of 0 to 10 (ASN.1: INTEGER (0..10)), and 12 will be received, then this will be treated as a transfer syntax error.

-	Violation in list element constraints. E.g., if a list is defined as containing 1 to 10 elements, and 12 elements will be received, than this case will be handled as a transfer syntax error.

-	Missing mandatory elements in ASN.1 SEQUENCE definitions (as sent by the originator of the message).

-	Wrong order of elements in ASN.1 SEQUENCE definitions (as sent by the originator of the message).

## 10.3	Abstract Syntax Error

### 10.3.1	General

An Abstract Syntax Error occurs when the receiving functional S1AP entity:

1.	receives IEs or IE groups that cannot be understood (unknown IE ID);

2.	receives IEs for which the logical range is violated (e.g., ASN.1 definition: 0 to 15, the logical range is 0 to 10, while values 11 to 15 are undefined), and 12 will be received; this case will be handled as an abstract syntax error using criticality information sent by the originator of the message);

3.	does not receive IEs or IE groups but according to the specified presence of the concerning object, the IEs or IE groups should have been present in the received message.

4.	receives IEs or IE groups that are defined to be part of that message in wrong order or with too many occurrences of the same IE or IE group;

5.	receives IEs or IE groups but according to the conditional presence of the concerning object and the specified condition, the IEs or IE groups should not have been present in the received message.

Cases 1 and 2 (not comprehended IE/IE group) are handled based on received Criticality information. Case 3 (missing IE/IE group) is handled based on Criticality information and Presence information for the missing IE/IE group specified in the version of the specification used by the receiver. Case 4 (IEs or IE groups in wrong order or with too many occurrences) and Case 5 (erroneously present conditional IEs or IE groups) result in rejecting the procedure.

If an Abstract Syntax Error occurs, the receiver shall read the remaining message and shall then for each detected Abstract Syntax Error that belong to cases 1-3 act according to the Criticality Information and Presence Information for the IE/IE group due to which Abstract Syntax Error occurred in accordance with subclauses 10.3.4 and 10.3.5. The handling of cases 4 and 5 is specified in subclause 10.3.6.

### 10.3.2	Criticality Information

In the S1AP messages there is criticality information set for individual IEs and/or IE groups. This criticality information instructs the receiver how to act when receiving an IE or an IE group that is not comprehended, i.e., the entire item (IE or IE group) which is not (fully or partially) comprehended shall be treated in accordance with its own criticality information as specified in subclause 10.3.4.

In addition, the criticality information is used in case of the missing IE/IE group abstract syntax error (see subclause 10.3.5).

The receiving node shall take different actions depending on the value of the Criticality Information. The three possible values of the Criticality Information for an IE/IE group are:

-	Reject IE.

-	Ignore IE and Notify Sender.

-	Ignore IE.

The following rules restrict when a receiving entity may consider an IE, an IE group, or an EP not comprehended (not implemented), and when action based on criticality information is applicable:

1.	IE or IE group: When one new or modified IE or IE group is implemented for one EP from a standard version, then other new or modified IEs or IE groups specified for that EP in that standard version shall be considered comprehended by a receiving entity (some may still remain unsupported).

2.	EP: The comprehension of different EPs within a standard version or between different standard versions is not mandated. Any EP that is not supported may be considered not comprehended, even if another EP from that standard version is comprehended, and action based on criticality shall be applied.

### 10.3.3	Presence Information

For many IEs/IE groups which are optional according to the ASN.1 transfer syntax, S1AP specifies separately if the presence of these IEs/IE groups is optional or mandatory with respect to RNS application by means of the presence field of the concerning object of class S1AP-PROTOCOL-IES, S1AP-PROTOCOL-IES-PAIR, S1AP-PROTOCOL-EXTENSION or S1AP-PRIVATE-IES.

The presence field of the indicated classes supports three values:

1.	Optional;

2.	Conditional;

3.	Mandatory.

If an IE/IE group is not included in a received message and the presence of the IE/IE group is mandatory or the presence is conditional and the condition is true according to the version of the specification used by the receiver, an abstract syntax error occurs due to a missing IE/IE group.

If an IE/IE group is included in a received message and the presence of the IE/IE group is conditional and the condition is false according to the version of the specification used by the receiver, an abstract syntax error occurs due to this erroneously present conditional IE/IE group.

### 10.3.4	Not comprehended IE/IE group

#### 10.3.4.1	Procedure Code

The receiving node shall treat the different types of received criticality information of the Procedure Code IE according to the following:

Reject IE:

-	If a message is received with a Procedure Code IE marked with “Reject IE” which the receiving node does not comprehend, the receiving node shall reject the procedure using the Error Indication procedure.

Ignore IE and Notify Sender:

-	If a message is received with a Procedure Code IE marked with “Ignore IE and Notify Sender” which the receiving node does not comprehend, the receiving node shall ignore the procedure and initiate the Error Indication procedure.

Ignore IE:

-	If a message is received with a Procedure Code IE marked with “Ignore IE” which the receiving node does not comprehend, the receiving node shall ignore the procedure.

When using the Error Indication procedure to reject a procedure or to report an ignored procedure it shall include the Procedure Code IE, the Triggering Message IE, and the Procedure Criticality IE in the Criticality Diagnostics IE.

#### 10.3.4.1A	Type of Message

When the receiving node cannot decode the Type of Message IE, the Error Indication procedure shall be initiated with an appropriate cause value.

#### 10.3.4.2	IEs other than the Procedure Code and Type of Message

The receiving node shall treat the different types of received criticality information of an IE/IE group other than the Procedure Code IE and Type of Message IE according to the following:

Reject IE:

-	If a message initiating a procedure is received containing one or more IEs/IE group marked with “Reject IE” which the receiving node does not comprehend; none of the functional requests of the message shall be executed. The receiving node shall reject the procedure and report the rejection of one or more IEs/IE group using the message normally used to report unsuccessful outcome of the procedure. In case the information received in the initiating message was insufficient to determine a value for all IEs that are required to be present in the message used to report the unsuccessful outcome of the procedure, the receiving node shall instead terminate the procedure and initiate the Error Indication procedure.

-	If a message initiating a procedure that does not have a message to report unsuccessful outcome is received containing one or more IEs/IE groups marked with “Reject IE” which the receiving node does not comprehend, the receiving node shall terminate the procedure and initiate the Error Indication procedure.

-	If a response message is received containing one or more IEs marked with “Reject IE”, that the receiving node does not comprehend, the receiving node shall consider the procedure as unsuccessfully terminated and initiate local error handling.

Ignore IE and Notify Sender:

-	If a message initiating a procedure is received containing one or more IEs/IE groups marked with “Ignore IE and Notify Sender” which the receiving node does not comprehend, the receiving node shall ignore the content of the not comprehended IEs/IE groups, continue with the procedure as if the not comprehended IEs/IE groups were not received (except for the reporting) using the understood IEs/IE groups, and report in the response message of the procedure that one or more IEs/IE groups have been ignored. In case the information received in the initiating message was insufficient to determine a value for all IEs that are required to be present in the response message, the receiving node shall instead terminate the procedure and initiate the Error Indication procedure.

-	if a message initiating a procedure that does not have a message to report the outcome of the procedure is received containing one or more IEs/IE groups marked with “Ignore IE and Notify Sender” which the receiving node does not comprehend, the receiving node shall ignore the content of the not comprehended IEs/IE groups, continue with the procedure as if the not comprehended IEs/IE groups were not received (except for the reporting) using the understood IEs/IE groups, and initiate the Error Indication procedure to report that one or more IEs/IE groups have been ignored.

-	If a response message is received containing one or more IEs/IE groups marked with “Ignore IE and Notify Sender” which the receiving node does not comprehend, the receiving node shall ignore the content of the not comprehended IEs/IE groups, continue with the procedure as if the not comprehended IEs/IE groups were not received (except for the reporting) using the understood IEs/IE groups and initiate the Error Indication procedure.

Ignore IE:

-	If a message initiating a procedure is received containing one or more IEs/IE groups marked with “Ignore IE” which the receiving node does not comprehend, the receiving node shall ignore the content of the not comprehended IEs/IE groups and continue with the procedure as if the not comprehended IEs/IE groups were not received using the understood IEs/IE groups.

-	If a response message is received containing one or more IEs/IE groups marked with “Ignore IE” which the receiving node does not comprehend, the receiving node shall ignore the content of the not comprehended IEs/IE groups and continue with the procedure as if the not comprehended IEs/IE groups were not received using the understood IEs/IE groups.

When reporting not comprehended IEs/IE groups marked with “Reject IE” or “Ignore IE and Notify Sender” using a response message defined for the procedure, the Information Element Criticality Diagnostics IE shall be included in the Criticality Diagnostics IE for each reported IE/IE group.

When reporting not comprehended IEs/IE groups marked with “Reject IE” or “Ignore IE and Notify Sender” using the Error Indication procedure, the Procedure Code IE, the Triggering Message IE, Procedure Criticality IE, and the Information Element Criticality Diagnostics IE shall be included in the Criticality Diagnostics IE for each reported IE/IE group.

### 10.3.5	Missing IE or IE group

The receiving node shall treat the missing IE/IE group according to the criticality information for the missing IE/IE group in the received message specified in the version of this specification used by the receiver:

Reject IE:

- if a received message initiating a procedure is missing one or more IEs/IE groups with specified criticality “Reject IE”; none of the functional requests of the message shall be executed. The receiving node shall reject the procedure and report the missing IEs/IE groups using the message normally used to report unsuccessful outcome of the procedure. In case the information received in the initiating message was insufficient to determine a value for all IEs that are required to be present in the message used to report the unsuccessful outcome of the procedure, the receiving node shall instead terminate the procedure and initiate the Error Indication procedure.

-	if a received message initiating a procedure that does not have a message to report unsuccessful outcome is missing one or more IEs/IE groups with specified criticality “Reject IE”, the receiving node shall terminate the procedure and initiate the Error Indication procedure.

-	if a received response message is missing one or more IEs/IE groups with specified criticality “Reject IE, the receiving node shall consider the procedure as unsuccessfully terminated and initiate local error handling.

Ignore IE and Notify Sender:

-	if a received message initiating a procedure is missing one or more IEs/IE groups with specified criticality “Ignore IE and Notify Sender”, the receiving node shall ignore that those IEs are missing and continue with the procedure based on the other IEs/IE groups present in the message and report in the response message of the procedure that one or more IEs/IE groups were missing. In case the information received in the initiating message was insufficient to determine a value for all IEs that are required to be present in the response message, the receiving node shall instead terminate the procedure and initiate the Error Indication procedure.

-	if a received message initiating a procedure that does not have a message to report the outcome of the procedure is missing one or more IEs/IE groups with specified criticality “Ignore IE and Notify Sender”, the receiving node shall ignore that those IEs are missing and continue with the procedure based on the other IEs/IE groups present in the message and initiate the Error Indication procedure to report that one or more IEs/IE groups were missing.

-	if a received response message is missing one or more IEs/IE groups with specified criticality “Ignore IE and Notify Sender”, the receiving node shall ignore that those IEs are missing and continue with the procedure based on the other IEs/IE groups present in the message and initiate the Error Indication procedure to report that one or more IEs/IE groups were missing.

Ignore IE:

- if a received message initiating a procedure is missing one or more IEs/IE groups with specified criticality “Ignore IE”, the receiving node shall ignore that those IEs are missing and continue with the procedure based on the other IEs/IE groups present in the message.

- if a received response message is missing one or more IEs/IE groups with specified criticality “Ignore IE”, the receiving node shall ignore that those IEs/IE groups are missing and continue with the procedure based on the other IEs/IE groups present in the message.

When reporting missing IEs/IE groups with specified criticality “Reject IE” or “Ignore IE and Notify Sender” using a response message defined for the procedure, the Information Element Criticality Diagnostics IE shall be included in the Criticality Diagnostics IE for each reported IE/IE group.

When reporting missing IEs/IE groups with specified criticality “Reject IE” or “Ignore IE and Notify Sender” using the Error Indication procedure, the Procedure Code IE, the Triggering Message IE, Procedure Criticality IE, and the Information Element Criticality Diagnostics IE shall be included in the Criticality Diagnostics IE for each reported IE/IE group.

### 10.3.6	IEs or IE groups received in wrong order or with too many occurrences or erroneously present

If a message with IEs or IE groups in wrong order or with too many occurrences is received or if IEs or IE groups with a conditional presence are present when the condition is not met (i.e., erroneously present), the receiving node shall behave according to the following:

-	If a message initiating a procedure is received containing IEs or IE groups in wrong order or with too many occurrences or erroneously present, none of the functional requests of the message shall be executed. The receiving node shall reject the procedure and report the cause value “Abstract Syntax Error (Falsely Constructed Message)” using the message normally used to report unsuccessful outcome of the procedure. In case the information received in the initiating message was insufficient to determine a value for all IEs that are required to be present in the message used to report the unsuccessful outcome of the procedure, the receiving node shall instead terminate the procedure and initiate the Error Indication procedure.

-	If a message initiating a procedure that does not have a message to report unsuccessful outcome is received containing IEs or IE groups in wrong order or with too many occurrences or erroneously present, the receiving node shall terminate the procedure and initiate the Error Indication procedure, and use cause value “Abstract Syntax Error (Falsely Constructed Message)”.

-	If a response message is received containing IEs or IE groups in wrong order or with too many occurrences or erroneously present, the receiving node shall consider the procedure as unsuccessfully terminated and initiate local error handling.

When determining the correct order only the IEs specified in the specification version used by the receiver shall be considered.

## 10.4	Logical Error

Logical error situations occur when a message is comprehended correctly, but the information contained within the message is not valid (i.e., semantic error), or describes a procedure which is not compatible with the state of the receiver. In these conditions, the following behaviour shall be performed (unless otherwise specified) as defined by the class of the elementary procedure, irrespective of the criticality information of the IEs/IE groups containing the erroneous values.

Class 1:

Where the logical error occurs in a request message of a class 1 procedure, and the procedure has a message to report this unsuccessful outcome, this message shall be sent with an appropriate cause value. Typical cause values are:

-	Semantic Error.

-	Message not compatible with receiver state.

Where the logical error is contained in a request message of a class 1 procedure, and the procedure does not have a message to report this unsuccessful outcome, the procedure shall be terminated and the Error Indication procedure shall be initiated with an appropriate cause value. The Procedure Code IE and the Triggering Message IE within the Criticality Diagnostics IE shall then be included in order to identify the message containing the logical error.

Where the logical error exists in a response message of a class 1 procedure, the procedure shall be considered as unsuccessfully terminated and local error handling shall be initiated.

Class 2:

Where the logical error occurs in a message of a class 2 procedure, the procedure shall be terminated and the Error Indication procedure shall be initiated with an appropriate cause value. The Procedure Code IE and the Triggering Message IE within the Criticality Diagnostics IE shall then be included in order to identify the message containing the logical error.

## 10.5	Exceptions

The error handling for all the cases described hereafter shall take precedence over any other error handling described in the other subclauses of clause 10.

- If any type of error (Transfer Syntax Error, Abstract Syntax Error or Logical Error) is detected in the ERROR INDICATION message, it shall not trigger the Error Indication procedure in the receiving Node but local error handling.

- In case a response message or Error Indication message needs to be returned, but the information necessary to determine the receiver of that message is missing, the procedure shall be considered as unsuccessfully terminated and local error handling shall be initiated.

- If an error that terminates a procedure occurs, the returned cause value shall reflect the error that caused the termination of the procedure even if one or more abstract syntax errors with criticality “ignore and notify” have earlier occurred within the same procedure.

- If an AP ID error is detected, the error handling as described in subclause 10.6 shall be applied.

## 10.6	Handling of AP ID

NOTE:	The “first message”, the “first returned message” and the “last message” as used below correspond to messages for a UE-associated logical connection. The “first message” has a new AP ID from the sending node and the “first returned message” is the first response message, which has a new APID from the node sending the “first returned message”. Thereafter the two APIDs are included in all messages over the UE-associated logical connection unless otherwise allowed by the specification. The “last message” is a message sent by a node in order to complete the termination of a given UE-associated logical connection, such that no other messages for the same connection are expected in either direction.

If a node receives a first message that includes a remote AP ID which is erroneous, e.g., an AP ID which has been stored previously for another UE-associated logical connection for the same peer node, the receiving node shall initiate an Error Indication procedure with inclusion of only the previously received AP ID from the peer node and an appropriate cause value. In this case, both nodes shall initiate a local release of any established UE-associated logical connection having the erroneous AP ID as local or remote identifier.

If a node receives a first returned message that includes a remote AP ID which has been stored previously for another UE-associated logical connection for the same peer node, or that includes an AP ID pair which is inconsistent (e.g., the local AP ID is unknown or already allocated to another UE-associated logical connection), the receiving node shall initiate an Error Indication procedure with inclusion of the received AP IDs from the peer node and an appropriate cause value. Both nodes shall initiate a local release of any established UE-associated logical connection (for the same S1 interface) having these AP IDs as local or remote identifier.

If a node receives a message (other than the first or first returned messages) that includes AP ID(s) identifying a logical connection which is unknown to the node (for the same S1 interface):

-	if this message is not the last message for this UE-associated logical connection, the node shall initiate an Error Indication procedure with inclusion of the received AP ID(s) from the peer node and an appropriate cause value. Both nodes shall initiate a local release of any established UE-associated logical connection (for the same S1 interface) having the erroneous AP ID(s) as local or remote identifier.

-	if this message is the last message for this UE-associated logical connection, the receiving node shall initiate a local release of any established UE-associated logical connection (for the same S1 interface) that have either the local or remote AP ID(s) as identifiers.

# Annex A (informative): S1AP Transparent containers content

Transparent containers are used in order to transfer information from one RAN node to another RAN node. Depending on the particular scenario the behaviour of both involved RAN nodes may be either specified according to the same radio system or according to different radio systems. During an inter-system handover the source RAN node has to adopt to the target RAN node and its requirements. Therefore the container content is encoded according to the rules which are specified for the target radio system.

In S1AP, there is a single transparent container defined for transporting information from the source to the target RAN node and a single transparent container for transporting information from the target to the source RAN node during handover preparation: the Source to Target Transparent Container IE and the Target to Source Transparent Container IE, which may carry either E-UTRAN, UTRAN or GERAN specific information.

NOTE:	The definition of generic transparent containers for handover purposes allows to transport them through the core network in a RAT-agnostic way.

In subclause 8.4.1.2, it is described how the transparent container shall be encoded with respect to the scenario in which it is used.

The table below is showing all possible scenarios and definitions according to which the content of the transparent container shall be encoded. Additionally the reference to the specification defining particular IE is given.

Table A.1. Specification of Transparent Containers referenced in S1AP.

| Scenario | Source to Target Transparent Container IE in S1AP: HANDOVER REQUIRED message |  | Target to Source Transparent Container IE in S1AP: HANDOVER COMMAND message |  |
| --- | --- | --- | --- | --- |
|  | Name of the IE | Definition in specification | Name of the IE | Definition in specification |
| Intra E-UTRAN handover | Source eNB to Target eNB Transparent Container | 36.413 | Target eNB to Source eNB Transparent Container | 36.413 |
| Inter-system handover to UTRAN or SRVCC operation to UTRAN | Source RNC to Target RNC Transparent Container | 25.413 | Target RNC to Source RNC Transparent Container | 25.413 |
| Inter-system handover to GERAN (PS domain only) | Source BSS to Target BSS Transparent Container Contents of the Source BSS to Target BSS Transparent Container | 48.018 | Target BSS to Source BSS Transparent Container Contents of the Target BSS to Source BSS Transparent Container | 48.018 |
| SRVCC operation to GERAN without DTM support or SRVCC operation to GERAN with DTM but without DTM HO support | Old BSS to New BSS information elements field of the Old BSS to New BSS information | 48.008 | Layer 3 Information field of the Layer 3 Information | 48.008 |
| SRVCC operation to GERAN with DTM HO support | Source BSS to Target BSS Transparent Container Contents of the Source BSS to Target BSS Transparent Container (in the Source to Target Transparent Container IE); Old BSS to New BSS information elements field of the Old BSS to New BSS information (in the Source to Target Transparent Container Secondary IE) | 48.018 48.008 | Layer 3 Information field of the Layer 3 Information (in the Target to Source Transparent Container IE); Target BSS to Source BSS Transparent Container Contents of the Target BSS to Source BSS Transparent Container (in the Target to Source Transparent Container Secondary IE) | 48.008 48.018 |

# Annex B (normative): IEs for SON Transfer

This annex defines IEs used by the SON Transfer RIM application (TS 3GPP 48.018 [18]).

## B.1	Tabular definition

### B.1.1	SON Transfer Application Identity

This IE indicates the application identity within the SON Transfer application.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SON Transfer Application Identity | M |  | ENUMERATED (Cell Load Reporting, …, Multi-Cell Load Reporting, Event-Triggered Cell Load Reporting, HO Reporting, E-UTRAN Cell Activation, Energy Savings Indication, Failure Event Reporting) | The receiving RAN node, including the eHRPD eAN, shall discard any RAN-INFORMATION-REQUEST/Multiple Report PDU containing this IE with value set to “Cell Load Reporting”, "Multi-Cell Load Reporting", “HO Reporting”, “E-UTRAN Cell Activation”, “Energy Savings Indication” or "Failure Event Reporting". The receiving eHRPD eAN shall discard any RAN-INFORMATION-REQUEST/Single Report PDU containing this IE with value set to “Cell Load Reporting”, “HO Reporting”, “E-UTRAN Cell Activation”, “Energy Savings Indication” or "Failure Event Reporting". |

### B.1.2	SON Transfer Request Container

This container transfers request information for the SON Transfer application.

NOTE:	The length of the SON Transfer Request Container IE shall remain compatible with the maximum message size on the Gb interface, this maximum size being determined depending on the lower layers used on the interface and on their configuration, a typical (default) limitation being 1600 octets for a Frame Relay sub-network as stated in 3GPP TS 48.016 [30].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Transfer Application | M |  |  |  |
| >Cell Load Reporting |  |  | NULL |  |
| >Multi-Cell Load Reporting |  |  |  |  |
| >>Multi-Cell Load Reporting Request | M |  | B.1.7 |  |
| >Event-Triggered Cell Load Reporting |  |  |  |  |
| >>Event-Triggered Cell Load Reporting Request | M |  | B.1.11 |  |
| >HO Reporting |  |  |  |  |
| >>HO Report | M |  | B.1.13 |  |
| >E-UTRAN Cell Activation |  |  |  |  |
| >>Cell Activation Request | M |  | B.1.14 |  |
| >Energy Savings Indication |  |  |  |  |
| >>Cell State Indication | M |  | B.1.16 |  |
| >Failure Event Reporting |  |  |  |  |
| >>Failure Event Report | M |  | B.1.17 |  |

### B.1.3	SON Transfer Response Container

This container transfers response information for the SON Transfer application.

NOTE:	The length of the SON Transfer Response Container IE shall remain compatible with the maximum message size on the Gb interface, this maximum size being determined depending on the lower layers used on the interface and on their configuration, a typical (default) limitation being 1600 octets for a Frame Relay sub-network as stated in 3GPP TS 48.016 [30].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Transfer Application | M |  |  |  |
| >Cell Load Reporting |  |  |  |  |
| >>Cell Load Reporting Response | M |  | B.1.5 |  |
| >Multi-Cell Load Reporting |  |  |  |  |
| >>Multi-Cell Load Reporting Response | M |  | B.1.9 |  |
| >Event-Triggered Cell Load Reporting |  |  |  |  |
| >>Event-triggered Cell Load Reporting Response | M |  | B.1.12 |  |
| >HO Reporting |  |  | NULL |  |
| >E-UTRAN Cell Activation |  |  |  |  |
| >>Cell Activation Response | M |  | B.1.15 |  |
| >Energy Savings Indication |  |  | NULL | The Reporting Cell Identifier field in the RAN-INFORMATION Application Container for SON Transfer (see 3GPP TS 48.018 [18]) shall be the same as received in the RAN-INFORMATION-REQUEST Application Container. The RAT Discriminator field shall be set to 'E-UTRAN'. |
| >Failure Event Reporting |  |  | NULL | The Reporting Cell Identifier field in the RAN-INFORMATION Application Container for SON Transfer (see 3GPP TS 48.018 [18]) shall be the same as received in the RAN-INFORMATION-REQUEST Application Container. The RAT Discriminator field shall be set to 'E-UTRAN'. |

### B.1.4	SON Transfer Cause

This container indicates the cause why the Application Error Container IE for the SON Transfer application defined in 3GPP TS 48.018 [18] is sent.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Transfer Application | M |  |  |  |
| >Cell Load Reporting |  |  |  |  |
| >>Cell Load Reporting Cause | M |  | B.1.10 |  |
| >Multi-Cell Load Reporting |  |  |  |  |
| >>Cell Load Reporting Cause | M |  | B.1.10 |  |
| >Event-Triggered Cell Load Reporting |  |  |  |  |
| >>Cell Load Reporting Cause | M |  | B.1.10 |  |
| >HO Reporting |  |  |  |  |
| >>HO Reporting Cause | M |  | ENUMERATED (Application Container Syntax Error, Inconsistent Reporting Cell Identifier, Unspecified, …) |  |
| >E-UTRAN Cell Activation |  |  |  |  |
| >>Cell Activation Cause | M |  | ENUMERATED (Application Container Syntax Error, Inconsistent Reporting Cell Identifier, Unspecified, …) |  |
| >Energy Savings Indication |  |  |  |  |
| >>Cell State Indication Cause | M |  | ENUMERATED (Application Container Syntax Error, Inconsistent Reporting Cell Identifier, Unspecified, …) |  |
| >Failure Event Reporting |  |  |  |  |
| >>Failure Event Reporting Cause | M |  | ENUMERATED (Application Container Syntax Error, Inconsistent Reporting Cell Identifier, Unspecified, …) |  |

| HO Reporting Cause | Meaning |
| --- | --- |
| Application Container Syntax Error | The Application Container IE is syntactically incorrect. |
| Inconsistent Reporting Cell Identifier | - In case the reporting RAT is GERAN: the Reporting Cell Identifier in the Application Container IE does not match with the Destination Cell Identifier IE value (in the case of a RAN-INFORMATION-REQUEST PDU) or with the Source Cell Identifier IE value (in the case of a RAN-INFORMATION PDU) of the RIM header. - In case the reporting RAT is UTRAN or E-UTRAN: the cell identified by Reporting Cell Identifier in the Application Container IE is unknown in the RNC (UTRAN case) or in the eNodeB (E-UTRAN case) identified by the Destination Cell Identifier IE value in the RAN-INFORMATION-REQUEST PDU. |
| Unspecified | Sent when none of the above cause values applies. |

| Cell Activation Cause | Meaning |
| --- | --- |
| Application Container Syntax Error | The Application Container IE is syntactically incorrect. |
| Inconsistent Reporting Cell Identifier | - In case the reporting RAT is E-UTRAN: The Reporting Cell Identifier in the Application Container IE is unknown in the eNB identified by the Destination Cell Identifier IE value of the RIM header of a RAN-INFORMATION-REQUEST PDU or the reporting cell identifier in the Application Container IE does not match with the Source Cell Identifier IE value of the RIM header of a RAN-INFORMATION PDU. |
| Unspecified | Sent when none of the above cause values applies. |

| Cell State Indication Cause | Meaning |
| --- | --- |
| Application Container Syntax Error | The Application Container IE is syntactically incorrect. |
| Inconsistent Reporting Cell Identifier | - In case the reporting RAT is E-UTRAN: The Reporting Cell Identifier in the Application Container IE does not match with the Source Cell Identifier IE value of the RIM header of a RAN-INFORMATION-REQUEST PDU or the reporting cell identifier in the Application Container IE does not match with the Destination Cell Identifier IE value of the RIM header of a RAN-INFORMATION PDU. |
| Unspecified | Sent when none of the above cause values applies. |

| Failure Event Reporting Cause | Meaning |
| --- | --- |
| Application Container Syntax Error | The Application Container IE is syntactically incorrect. |
| Inconsistent Reporting Cell Identifier | - In case the reporting RAT is E-UTRAN: The Reporting Cell Identifier in the Application Container IE does not match with the Source Cell Identifier IE value of the RIM header of a RAN-INFORMATION-REQUEST PDU or the reporting cell identifier in the Application Container IE does not match with the Destination Cell Identifier IE value of the RIM header of a RAN-INFORMATION PDU. |
| Unspecified | Sent when none of the above cause values applies |

### B.1.5	Cell Load Reporting Response

This IE contains response information for inter-RAT cell load reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting RAT | M |  |  |  |
| >E-UTRAN |  |  |  |  |
| >>E-UTRAN Response | M |  | E-UTRAN Cell Load Reporting Response B.1.6 |  |
| >UTRAN |  |  |  |  |
| >>UTRAN Response | M |  | OCTET STRING | Contains the Cell Load Information Group IE as defined in 3GPP TS 25.413 [19]. The receiver shall ignore the value of the Source Cell Identifier IE within the Cell Load Information Group IE. |
| >GERAN |  |  |  |  |
| >>GERAN Response | M |  | OCTET STRING | Contains the Cell Load Information Group IE as defined in 3GPP TS 48.008 [23]. The receiver shall ignore the value of the Cell Identifier IE within the Cell Load Information Group IE. |
| >eHRPD |  |  |  |  |
| >>eHRPD Response | M |  | eHRPD Sector Load Reporting Response B.1.19 |  |

### B.1.6	E-UTRAN Cell Load Reporting Response

This IE contains response information for inter-RAT cell load reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Composite Available Capacity Group | M |  | OCTET STRING | Contains the Composite Available Capacity Group IE as defined in 3GPP TS 36.423 [22]. |

### B.1.7	Multi-Cell Load Reporting Request

This IE contains request information for inter-RAT multi-cell load reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Cell List |  | 1 .. <maxnoofIRATReportingCells> |  | One of the IRAT Cell IDs contained in this list shall be carried in the Reporting Cell Identifier field in the RAN-INFORMATION-REQUEST Application Container for SON Transfer (TS 48.018). |
| >IRAT Cell ID | M |  | B.1.8 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIRATReportingCells | Maximum no. cells to be included. Value is 128. |

### B.1.8	IRAT Cell ID

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting RAT | M |  |  |  |
| >E-UTRAN |  |  |  |  |
| >>Cell Identifier | M |  | OCTET STRING | Contains the E-UTRAN CGI IE as defined in 9.2.1.38. |
| >UTRAN |  |  |  |  |
| >>Cell Identifier | M |  | OCTET STRING | Contains the Source Cell Identifier IE as defined in 3GPP TS 25.413 [19]. |
| >GERAN |  |  |  |  |
| >>Cell Identifier | M |  | OCTET STRING | Contains the Cell Identifier IE as defined in 3GPP TS 48.018 [18]. |
| >eHRPD |  |  |  |  |
| >>eHRPD Sector ID | M |  | B.1.18 |  |

### B.1.9	Multi-Cell Load Reporting Response

This IE contains response information for inter-RAT multi-cell load reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting Cell List |  | 1 .. <maxnoofIRATReportingCells> |  |  |
| >CHOICE Reporting RAT | M |  |  |  |
| >>E-UTRAN |  |  |  |  |
| >>>E-UTRAN Response | M |  |  |  |
| >>>>Cell Identifier | M |  | OCTET STRING | Contains the E-UTRAN CGI IE as defined in 9.2.1.38. |
| >>>>E-UTRAN Cell Load Reporting Response | M |  | B.1.6 |  |
| >>UTRAN |  |  |  |  |
| >>>UTRAN Response | M |  | OCTET STRING | Contains the Cell Load Information Group IE as defined in 3GPP TS 25.413 [19]. |
| >>GERAN |  |  |  |  |
| >>>GERAN Response | M |  | OCTET STRING | Contains the Cell Load Information Group IE as defined in 3GPP TS 48.008 [23]. |
| >>eHRPD |  |  |  |  |
| >>>eHRPD Sector ID | M |  | B.1.18 |  |
| >>>eHRPD Sector Load Reporting Response | M |  | B.1.19 |  |

### B.1.10	Cell Load Reporting Cause

This IE contains request information for inter-RAT cell load reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Load Reporting Cause | M |  | ENUMERATED (Application Container Syntax Error, Inconsistent Reporting Cell Identifier, Unspecified, …) |  |

The meaning of the different cause values is described in the following table.

| Cell Load Reporting Cause | Meaning |
| --- | --- |
| Application Container Syntax Error | The Application Container IE is syntactically incorrect. |
| Inconsistent Reporting Cell Identifier | - In case the reporting RAT is GERAN or eHRPD: the Reporting Cell Identifier in the Application Container IE does not match with the Destination Cell Identifier IE value (in the case of a RAN-INFORMATION-REQUEST PDU) or with the Source Cell Identifier IE value (in the case of a RAN-INFORMATION PDU) of the RIM header. - In case the reporting RAT is UTRAN or E-UTRAN: the cell identified by Reporting Cell Identifier in the Application Container IE is unknown in the RNC (UTRAN case) or in the eNodeB (E-UTRAN case) identified by the Destination Cell Identifier IE value in the RAN-INFORMATION-REQUEST PDU. |
| Unspecified | Sent when none of the above cause values applies |

### B.1.11	Event-Triggered Cell Load Reporting Request

This IE contains request  information for inter-RAT cell load reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number Of Measurement Reporting Levels | M |  | ENUMERATED (2, 3, 4, 5, 10, ...) | The reporting node divides the cell load scale into the indicated number of reporting levels, evenly distributed on a linear scale below the reporting node's threshold for overload. The reporting node sends a report each time the cell load changes from one reporting level to another, and when the cell load enters and exits overload state. If the reporting RAT is eHRPD, triggering is based on sector load. |

### B.1.12	Event-triggered Cell Load Reporting Response

This IE contains response information for event-triggered inter-RAT cell load reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Load | M |  | Cell Load Reporting Response B.1.5 |  |
| Overload Flag | O |  | ENUMERATED (Overload, ...) | If the reporting RAT is eHRPD, when this IE is present the sector load exceeds the threshold for overload. For other reporting RATs, when this IE is present the cell load exceeds the threshold for overload. |

### B.1.13	HO Report

This IE contains information for too early inter-RAT HO without connection failure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| HO Type | M |  | ENUMERATED (LTE to UTRAN, LTE to GERAN, …) |  |
| HO Report Type | M |  | ENUMERATED (Unnecessary HO to another RAT, …, Early IRAT Handover) | The "Early IRAT Handover" code-point shall be used by the RNC according to 3GPP TS 25.413 [19]. |
| HO Source ID | M |  | IRAT Cell ID B.1.8 | Contains the cell ID of the source cell for the HO. This IE shall contain an E-UTRAN CGI, and shall be set to the same value as the Reporting Cell Identifier IE in 3GPP TS 48.018 [18] |
| HO Target ID | M |  | IRAT Cell ID B.1.8 | Contains the cell ID of the target cell for the HO. This IE shall contain either a UTRAN Cell ID or a GERAN Cell ID. |
| Candidate Cell List |  | 1 .. <maxnoofCandidateCells> |  |  |
| >Candidate Cell ID | M |  | IRAT Cell ID B.1.8 | This IE contains an E-UTRAN CGI. |
| Candidate PCI List |  | 0..1 |  |  |
| >Candidate PCIs |  | 1 .. <maxnoofCandidateCells> |  |  |
| >>Candidate PCI | M |  | B.1.23 | This IE includes the Primary Cell Identifier and the EARFCN of detected cells not included in the Candidate Cell List IE and for which an E-UTRAN CGI could not be derived. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. |

### B.1.14	Cell Activation Request

This IE contains request information for inter-RAT Cell Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cells to Activate List |  | 1 .. <maxnoofCellineNB> |  | One of the cell IDs contained in this list shall be carried in the Reporting Cell Identifier field in the RAN-INFORMATION-REQUEST Application Container for SON Transfer (TS 48.018 [18]). |
| >Cell Identifier | M |  | OCTET STRING | Contains the E-UTRAN CGI IE as defined in 9.2.1.38. |
| Minimum Activation Time | O |  | INTEGER (1..60) | Seconds |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

### B.1.15	Cell Activation Response

This IE contains response information for inter-RAT Cell Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activated Cells List |  | 0 .. <maxnoofCellineNB> |  |  |
| >Cell Identifier | M |  | OCTET STRING | Contains the E-UTRAN CGI IE as defined in 9.2.1.38. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

### B.1.16	Cell State Indication

This IE contains notification information for inter-RAT Cell Activation and Deactivation

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Notification Cell List |  | 1 .. <maxnoofCellineNB> |  | One of the cell IDs contained in this list shall be carried in the Reporting Cell Identifier field in the RAN-INFORMATION-REQUEST Application Container for SON Transfer (TS 48.018 [18]). |
| >Cell Identifier | M |  | OCTET STRING | Contains the E-UTRAN CGI IE as defined in 9.2.1.38. |
| >Notify Flag | M |  | ENUMERATED (Activated, Deactivated, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

### B.1.17	Failure Event Report

This IE contains information for inter-RAT handover with connection failure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Failure Event Report Type | M |  |  |  |
| >Too Early inter-RAT HO report from E-UTRAN |  |  |  | The Reporting Cell Identifier field in the RAN-INFORMATION Application Container for SON Transfer (see 3GPP TS 48.018 [18]) shall be the same as the Last Serving Cell Identity in the UE RLF Report. |
| >>UE RLF Report Container | M |  | OCTET STRING | RLF Report contained in the UEInformationResponse message (see MFA TS 36.331 [16]) |
| >>Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the external handover source provides it in the Source eNB to target eNB Transparent Container in order to enable later analysis of the conditions that led to a wrong HO. |

### B.1.18	eHRPD Sector ID

This IE contains the eHRPD Sector ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| eHRPD Sector ID | M |  | OCTET STRING (SIZE(16)) | Defined in 3GPP2 C.S0024-B [27] sub-section 13.9 |

### B.1.19	eHRPD Sector Load Reporting Response

This IE indicates the overall available resource level in the eHRPD sector in downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| eHRPD Composite Available Capacity Downlink | M |  | eHRPD Composite Available Capacity B.1.20 | For the downlink |
| eHRPD Composite Available Capacity Uplink | M |  | eHRPD Composite Available Capacity B.1.20 | For the uplink |

### B.1.20	eHRPD Composite Available Capacity

This IE indicates the overall available resource level in the eHRPD sector in either Downlink or Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| eHRPD Sector Capacity Class Value | M |  | B.1.21 |  |
| eHRPD Capacity Value | M |  | B.1.22 | ‘0’ indicates no resource is available, Measured on a linear scale. |

### B.1.21	eHRPD Sector Capacity Class Value

This IE indicates the value that classifies the eHRPD sector capacity with regards to cells in other RATs. The IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| eHRPD Sector Capacity Class Value | M |  | INTEGER (1..100,...) | Value 1 indicates the minimum sector capacity, and 100 indicates the maximum sector capacity. There should be a linear relation between sector capacity and eHRPD Sector Capacity Class Value. |

### B.1.22	eHRPD Capacity Value

This IE indicates the amount of resources that are available for load balancing relative to the total eHRPD resources. A sector is expected to accept traffic corresponding to the indicated available capacity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| eHRPD Capacity Value | M |  | INTEGER (0..100) | Value 0 indicates no available capacity, and 100 indicates maximum available capacity . Capacity Value should be measured on a linear scale. |

### B.1.23	Candidate PCI

This IE contains the Primary Cell Identity and the frequency of a detected LTE cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PCI | M |  | INTEGER (0..503) | Physical Cell Identifier of the detected cell |
| EARFCN | M |  | OCTET STRING | Contains the EARFCN IE as defined in 9.2.1.95. |

## B.2	ASN.1 definition

-- **************************************************************

--

-- IE definitions for the SON Transfer application

-- The IEs in this ASN.1 module shall be defined and encoded

-- using the same rules as applicable for the S1AP-IEs module.

--

-- **************************************************************

SonTransfer-IEs

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

--

-- Generic IEs for the SON Transfer application

--

SONtransferApplicationIdentity ::= ENUMERATED {

cell-load-reporting,

...,

multi-cell-load-reporting,

event-triggered-cell-load-reporting,

ho-reporting,

eutran-cell-activation,

energy-savings-indication,

failure-event-reporting

}

SONtransferRequestContainer ::= CHOICE{

cellLoadReporting					NULL,

...,

multiCellLoadReporting				MultiCellLoadReportingRequest,

eventTriggeredCellLoadReporting		EventTriggeredCellLoadReportingRequest,

hOReporting							HOReport,

eutranCellActivation				CellActivationRequest,

energySavingsIndication				CellStateIndication,

failureEventReporting				FailureEventReport

}

SONtransferResponseContainer ::= CHOICE{

cellLoadReporting					CellLoadReportingResponse,

...,

multiCellLoadReporting				MultiCellLoadReportingResponse,

eventTriggeredCellLoadReporting	EventTriggeredCellLoadReportingResponse,

hOReporting							NULL,

eutranCellActivation				CellActivationResponse,

energySavingsIndication				NULL,

failureEventReporting				NULL

}

SONtransferCause ::= CHOICE {

cellLoadReporting					CellLoadReportingCause,

...,

multiCellLoadReporting				CellLoadReportingCause,

eventTriggeredCellLoadReporting		CellLoadReportingCause,

hOReporting							HOReportingCause,

eutranCellActivation				CellActivationCause,

energySavingsIndication				CellStateIndicationCause,

failureEventReporting				FailureEventReportingCause

}

CellLoadReportingCause ::= ENUMERATED {

application-container-syntax-error,

inconsistent-reporting-cell-identifier,

unspecified,

...

}

HOReportingCause ::= ENUMERATED {

application-container-syntax-error,

inconsistent-reporting-cell-identifier,

unspecified,

...

}

CellActivationCause ::= ENUMERATED {

application-container-syntax-error,

inconsistent-reporting-cell-identifier,

unspecified,

...

}

CellStateIndicationCause ::= ENUMERATED {

application-container-syntax-error,

inconsistent-reporting-cell-identifier,

unspecified,

...

}

FailureEventReportingCause ::= ENUMERATED {

application-container-syntax-error,

inconsistent-reporting-cell-identifier,

unspecified,

...

}

--

-- IEs for Cell Load Reporting application

--

CellLoadReportingResponse::= CHOICE{

eUTRAN			EUTRANcellLoadReportingResponse,

uTRAN			OCTET STRING,

gERAN			OCTET STRING,

...,

eHRPD			EHRPDSectorLoadReportingResponse

}

CompositeAvailableCapacityGroup ::= OCTET STRING

EUTRANcellLoadReportingResponse ::= SEQUENCE {

compositeAvailableCapacityGroup		CompositeAvailableCapacityGroup,

...

}

--

-- IEs for Multi-Cell Load Reporting application

--

EUTRANResponse::= SEQUENCE {

cell-ID			OCTET STRING,

eUTRANcellLoadReportingResponse		EUTRANcellLoadReportingResponse,

...

}

EHRPD-Sector-ID ::= OCTET STRING (SIZE (16))

IRAT-Cell-ID ::= CHOICE{

eUTRAN			OCTET STRING,

uTRAN			OCTET STRING,

gERAN			OCTET STRING,

...,

eHRPD			EHRPD-Sector-ID

}

RequestedCellList ::= SEQUENCE (SIZE(1.. maxnoofIRATReportingCells)) OF IRAT-Cell-ID

MultiCellLoadReportingRequest::= SEQUENCE {

requestedCellList						RequestedCellList,

...

}

ReportingCellList-Item ::= SEQUENCE {

cell-ID									IRAT-Cell-ID,

...

}

ReportingCellList ::= SEQUENCE (SIZE(1.. maxnoofIRATReportingCells)) OF ReportingCellList-Item

MultiCellLoadReportingResponse ::= SEQUENCE (SIZE(1.. maxnoofIRATReportingCells)) OF MultiCellLoadReportingResponse-Item

MultiCellLoadReportingResponse-Item ::= CHOICE{

eUTRANResponse							EUTRANResponse,

uTRANResponse							OCTET STRING,

gERANResponse							OCTET STRING,

...,

eHRPD									EHRPDMultiSectorLoadReportingResponseItem

}

--

-- IEs for Event-triggered Cell Load Reporting application

--

NumberOfMeasurementReportingLevels ::= ENUMERATED {

rl2,

rl3,

rl4,

rl5,

rl10,

...

}

EventTriggeredCellLoadReportingRequest ::= SEQUENCE {

numberOfMeasurementReportingLevels		NumberOfMeasurementReportingLevels,

...

}

OverloadFlag ::= ENUMERATED {

overload,

...

}

EventTriggeredCellLoadReportingResponse ::= SEQUENCE {

cellLoadReportingResponse				CellLoadReportingResponse,

overloadFlag							OverloadFlag						OPTIONAL,

...

}

--

-- IEs for HO Reporting application

--

HOReport::= SEQUENCE {

hoType					HoType,

hoReportType			HoReportType,

hosourceID				IRAT-Cell-ID,

hoTargetID				IRAT-Cell-ID,

candidateCellList		CandidateCellList,

...,

candidatePCIList		CandidatePCIList	OPTIONAL

}

HoType ::= ENUMERATED {

ltetoutran,

ltetogeran,

...

}

HoReportType ::= ENUMERATED {

unnecessaryhotoanotherrat,

...,

earlyirathandover

}

CandidateCellList ::= SEQUENCE (SIZE(1..maxnoofcandidateCells)) OF IRAT-Cell-ID

CandidatePCIList ::= SEQUENCE (SIZE(1..maxnoofcandidateCells)) OF CandidatePCI

CandidatePCI ::= SEQUENCE {

pCI			INTEGER (0..503),

eARFCN		OCTET STRING,

...

}

--

-- IEs for E-UTRAN Cell Activation application

--

CellActivationRequest ::= SEQUENCE {

cellsToActivateList		CellsToActivateList,

minimumActivationTime	INTEGER (1..60)		OPTIONAL,

...

}

CellsToActivateList ::= SEQUENCE (SIZE(1.. maxnoofCellineNB)) OF CellsToActivateList-Item

CellsToActivateList-Item ::= SEQUENCE {

cell-ID					OCTET STRING,

...

}

CellActivationResponse ::= SEQUENCE {

activatedCellsList		ActivatedCellsList,

...

}

ActivatedCellsList ::= SEQUENCE (SIZE(0.. maxnoofCellineNB)) OF ActivatedCellsList-Item

ActivatedCellsList-Item ::= SEQUENCE {

cell-ID					OCTET STRING,

...

}

--

-- IEs for Energy Savings Indication application

--

CellStateIndication ::= SEQUENCE {

notificationCellList			NotificationCellList,

...

}

NotificationCellList ::= SEQUENCE (SIZE(1.. maxnoofCellineNB)) OF NotificationCellList-Item

NotificationCellList-Item ::= SEQUENCE {

cell-ID					OCTET STRING,

notifyFlag				NotifyFlag,

...

}

NotifyFlag ::= ENUMERATED {

activated,

deactivated,

...

}

FailureEventReport::= CHOICE {

tooEarlyInterRATHOReportFromEUTRAN			TooEarlyInterRATHOReportReportFromEUTRAN,

...

}

TooEarlyInterRATHOReportReportFromEUTRAN ::= SEQUENCE {

uERLFReportContainer	OCTET STRING, -- as defined in MFA TS 36.331 [16] --

mobilityInformation		MobilityInformation		OPTIONAL,

...

}

MobilityInformation ::= BIT STRING (SIZE(32))

--

-- IEs for reporting of eHRPD load

--

EHRPDCapacityValue ::= INTEGER (0..100)

EHRPDSectorCapacityClassValue ::= INTEGER (1..100, ...)

EHRPDSectorLoadReportingResponse ::= SEQUENCE {

dL-EHRPD-CompositeAvailableCapacity			EHRPDCompositeAvailableCapacity,

uL-EHRPD-CompositeAvailableCapacity			EHRPDCompositeAvailableCapacity,

...

}

EHRPDCompositeAvailableCapacity ::= SEQUENCE {

eHRPDSectorCapacityClassValue				EHRPDSectorCapacityClassValue,

eHRPDCapacityValue							EHRPDCapacityValue,

...

}

EHRPDMultiSectorLoadReportingResponseItem ::= SEQUENCE {

eHRPD-Sector-ID							EHRPD-Sector-ID,

eHRPDSectorLoadReportingResponse		EHRPDSectorLoadReportingResponse,

...

}

-- **************************************************************

--

-- Constants

--

-- **************************************************************

maxnoofIRATReportingCells					INTEGER ::= 128

maxnoofcandidateCells						INTEGER ::= 16

maxnoofCellineNB							INTEGER ::= 256

END

# Annex C (informative): Processing of Transparent Containers at the MME

The encoding of the Source to Target Transparent Container and Target to Source Transparent Container IEs in this specification is different from the one specified in 3GPP TS 25.413 [19].

Irrespective of the mobility scenario (inter-RAT or intra-LTE), the MME always processes these IEs in the following way:

-	The MME shall convey to the eNodeB the information received within

-	the GTPv1-C "UTRAN transparent field" of the "UTRAN Transparent Container" IE across the Gn-interface (see subclause 7.7.38 of 3GPP TS 29.060 [35]), or

-	the GTPv1-C "BSS Container" (value part octets 4-n) of the "BSS Container" IE across the Gn- interface (see subclause 7.7.72 of 3GPP TS 29.060 [35]), or

-	the GTPv2 "F-container field" of the "F-Container" IE across the S3/S10- interface (see subclause 8.48 of 3GPP TS 29.274 [36]).

by including it in the octets of the OCTET STRING of the Source to Target Transparent Container IE, the Target to Source Transparent Container IE or the Target to Source Transparent Container Secondary IE of the corresponding S1AP message.

-	The MME shall convey to the GTP peer the information received within the octets of the OCTET STRING of the Source to Target Transparent Container IE, the Target to Source Transparent Container IE or the Target to Source Transparent Container Secondary IE by including it in

-	the GTPv1-C "UTRAN transparent field" of the "UTRAN Transparent Container" IE across the Gn- interface (see subclause 7.7.38 of TS 3GPP 29.060 [35]), or

-	the GTPv1-C "BSS Container" (value part octets 4-n) of the "BSS Container" IE across the Gn- interface (see subclause 7.7.72 of 3GPP TS 29.060 [35]), or

-	the GTPv2 "F-container field" of the "F-Container" IE across the S3/S10- interface (see subclause 8.48 of 3GPP TS 29.274 [36]).

# Annex D (informative): Change history

| TSG # | TSG Doc. | CR | Rev | Subject/Comment | New |
| --- | --- | --- | --- | --- | --- |
| 38 |  |  |  | Specification approved at TSG-RAN and placed under change control | 8.0.0 |
| 39 | RP-080080 | 0058 |  | RAN3 agreed changes for TS 36.413 | 8.1.0 |
| 40 | RP-080304 | 0059 | 1 | RAN3 agreed changes for TS 36.413 | 8.2.0 |
| 41 | RP-080584 | 0223 |  | changes to TS36.413 agreed in RAN3#61 | 8.3.0 |
| 42 | RP-080846 | 0325 | 1 | changes to TS36.413 agreed in RAN3#62 | 8.4.0 |
| 43 | RP-090083 | 0327 |  | Adding extension container in SEQUENCE type for forward compatibility | 8.5.0 |
| 43 | RP-090091 | 0331 | 1 | Corrections on S1AP: eNB configuration update procedure | 8.5.0 |
| 43 | RP-090086 | 0332 | 1 | Corrections on S1AP: Paging procedure | 8.5.0 |
| 43 | RP-090089 | 0333 | 1 | Handling detection of two S1 connections towards one UE | 8.5.0 |
| 43 | RP-090089 | 0334 | 1 | Interaction between UE Context Release Request and UE Context Release procedure | 8.5.0 |
| 43 | RP-090246 | 0337 | 2 | IP address retrieval for ANRF | 8.5.0 |
| 43 | RP-090083 | 0340 |  | Modification of RRC context indexing | 8.5.0 |
| 43 | RP-090086 | 0342 | 1 | Completion of LTE cause values | 8.5.0 |
| 43 | RP-090090 | 0345 | 1 | Correction of served GUMMEIs | 8.5.0 |
| 43 | RP-090086 | 0346 | 1 | Correction of Initial Context Setup | 8.5.0 |
| 43 | RP-090086 | 0349 | 1 | Clarification of path switch failure | 8.5.0 |
| 43 | RP-090091 | 0350 | 2 | Correction of eNB Status Transfer | 8.5.0 |
| 43 | RP-090083 | 0356 |  | Addition of the description of Timer TX2RELOCOverall | 8.5.0 |
| 43 | RP-090089 | 0357 | 1 | New cause value “Interaction with other procedure” | 8.5.0 |
| 43 | RP-090087 | 0359 | 1 | S1AP Review on Location Reporting procedures | 8.5.0 |
| 43 | RP-090089 | 0366 | 1 | Definition on parameters related to a trace activation | 8.5.0 |
| 43 | RP-090090 | 0368 | 2 | Adding EUTRAN CELL TRAFFIC TRACE message over S1 interfaces | 8.5.0 |
| 43 | RP-090091 | 0369 | 2 | Adding MS Classmark 2 and MS Clssmark 3 IEs over S1 interface | 8.5.0 |
| 43 | RP-090086 | 0370 | 1 | New Invalid E-RAB Id causes | 8.5.0 |
| 43 | RP-090091 | 0371 | 2 | S1AP Review: S1 Handover Cancel procedure | 8.5.0 |
| 43 | RP-090158 | 0372 | 2 | S1AP Review: Write-Replace Warning procedure | 8.5.0 |
| 43 | RP-090246 | 0374 | 1 | Definition of Cell Type | 8.5.0 |
| 43 | RP-090085 | 0375 | 1 | Abnormal condition related to UE Security Capabilities | 8.5.0 |
| 43 | RP-090245 | 0376 |  | Removal of UE Security Capabilities IE from HANDOVER NOTIFY message | 8.5.0 |
| 43 | RP-090086 | 0378 | 1 | Corrections for the procedure concurrency | 8.5.0 |
| 43 | RP-090091 | 0380 | 2 | Clarification of eNB Name and MME Name IE’s | 8.5.0 |
| 43 | RP-090083 | 0392 |  | Clarifications on access control at handover | 8.5.0 |
| 43 | RP-090087 | 0393 | 1 | Paging response | 8.5.0 |
| 43 | RP-090077 | 0394 |  | Correction on usage of UE History Information | 8.5.0 |
| 43 | RP-090086 | 0395 | 1 | Delete the UDP port in the note for GTP-TEID | 8.5.0 |
| 43 | RP-090245 | 0396 |  | S1AP CR on CDMA2000 RAT Type | 8.5.0 |
| 43 | RP-090246 | 0397 | 1 | Editorial Updates TS 36.413 | 8.5.0 |
| 43 | RP-090091 | 0398 | 3 | NAS Security Parameters for to/from E-UTRAN/UTRAN handovers | 8.5.0 |
| 43 | RP-090085 | 0399 | 1 | Updates for Next Hop Chaining Count | 8.5.0 |
| 43 | RP-090245 | 0401 |  | Transparent Container content – informative annex | 8.5.0 |
| 43 | RP-090093 | 0404 | 1 | Transparent container handling in case of SRVCC operation to GERAN | 8.5.0 |
| 43 | RP-090090 | 0405 | 2 | Changes to S1AP to support paging optimization | 8.5.0 |
| 43 | RP-090245 | 0406 | 3 | S1 handover Clean up | 8.5.0 |
| 43 | RP-090087 | 0407 | 1 | Support blocking 3GPP2 handover | 8.5.0 |
| 43 | RP-090091 | 0410 | 2 | Inclusion of eNB default paging DRX in S1 setup and configuration update | 8.5.0 |
| 43 | RP-090087 | 0412 | 1 | Explicit resetting of overload state information on S1 Setup | 8.5.0 |
| 43 | RP-090090 | 0413 | 2 | Clarify Security Context IE description | 8.5.0 |
| 43 | RP-090091 | 0414 | 2 | Criticality corrections in 36.413 | 8.5.0 |
| 43 | RP-090245 | 0415 |  | Add abnormal conditions section to UE Context Release and fix tabular error | 8.5.0 |
| 43 | RP-090245 | 0419 |  | Consistent references to S1AP | 8.5.0 |
| 43 | RP-090090 | 0424 | 2 | Two new cause values in the Cause IE | 8.5.0 |
| 43 | RP-090089 | 0425 |  | Alignment of QCI range | 8.5.0 |
| 43 | RP-090089 | 0426 |  | Remove the Handover Type IE from the HANDOVER REQUEST ACKNOWLEDGE message | 8.5.0 |
| 43 | RP-090090 | 0427 | 1 | Correction of the trace procedural text and trace related IEs | 8.5.0 |
| March 2009 | - | - | - | Minor corrections before freezing of ASN.1 | 8.5.1 |
| 44 | RP-090637 | 0504 | 2 | Editorial Updates | 8.6.0 |
| 44 | RP-090637 | 0512 |  | Correction of RAN#43 CR implementation | 8.6.0 |
| 44 | RP-090637 | 0510 |  | Explicitly allow TRACE START to be the first UE-associated message received at the eNB | 8.6.0 |
| 44 | RP-090637 | 0507 | 1 | Clarification of UE Capability Info Indication | 8.6.0 |
| 44 | RP-090637 | 0500 | 1 | Mandatory UE History Information IE in HANDOVER REQUIRED For Inter-RAT HO from E-UTRAN to UMTS | 8.6.0 |
| 44 | RP-090637 | 0482 | 1 | Clarify eNB may send Release msg rather than RRC Reject msg on receiving OVERLOAD Start msg | 8.6.0 |
| 44 | RP-090637 | 0480 | 1 | Clarify reporting of duplicate E-RABs in E-RAB RESPONSE | 8.6.0 |
| 44 | RP-090637 | 0468 |  | Correction of security parameters | 8.6.0 |
| 44 | RP-090637 | 0463 | 1 | Emergency call Indicator during CS Fallback | 8.6.0 |
| 44 | RP-090638 | 0438 | 2 | Correction on Path Switch Request procedure | 8.6.0 |
| 44 | RP-090644 | 0443 | 2 | Removing ‘outcome’ element from the Triggering Message IE | 8.6.0 |
| 44 | RP-090644 | 0448 | 1 | Missing S1AP functions | 8.6.0 |
| 44 | RP-090644 | 0451 | 1 | Correction of abnormal conditions in UE Context Release | 8.6.0 |
| 44 | RP-090644 | 0452 | 1 | Clarification of E-UTRAN Trace ID in Cell Traffice Trace message | 8.6.0 |
| 44 | RP-090644 | 0453 |  | Removal of duplication description of MME UE S1AP ID and eNB UE S1AP ID | 8.6.0 |
| 44 | RP-090644 | 0455 | 1 | Abnormal condition for Handover Cancellation | 8.6.0 |
| 44 | RP-090640 | 0458 | 3 | NNSF for HeNB GW deployment scenario | 8.6.0 |
| 44 | RP-090640 | 0503 | 1 | Transparent Container Coding | 8.6.0 |
| 44 | RP-090640 | 0471 | 2 | Some Editorial Corrections on ASN.1 | 8.6.0 |
| 44 | RP-090640 | 0492 |  | Failure of the eNB Configuration Update procedure | 8.6.0 |
| 44 | RP-090640 | 0484 |  | Rephrasing of abnormal conditions for S1 setup | 8.6.0 |
| 44 | RP-090640 | 0494 |  | Cause value for inter-RAT Redirection | 8.6.0 |
| 44 | RP-090628 | 0464 | 2 | NAS PDU in E-RAB Release Command | 8.6.0 |
| 44 | RP-090636 | 0491 |  | Alignment of eNB configuration update procedure | 8.6.0 |
| 44 | RP-090636 | 0476 | 2 | Add that a non-GBR must be received and admitted on S1-HO | 8.6.0 |
| 44 | RP-090636 | 0461 | 1 | Clarification of Security Context to be used in HANDOVER REQUEST message | 8.6.0 |
| 44 | RP-090636 | 0459 |  | Correction the text about the Handover Resource Allocation procedure | 8.6.0 |
| 44 | RP-090636 | 0502 |  | Clarification for RAT list in S1 Setup Response and MME configuration Update | 8.6.0 |
| 44 | RP-090636 | 0501 | 1 | Range bound for maximal number of PLMNs per MME and GUMMEIs | 8.6.0 |
| June 2009 |  |  |  | Correction of an ASN.1 implementation error of CR0463r1 in RP-090637 (R3-091456) | 8.6.1 |
| 45 | RP-090767 | 0515 | 1 | Corrections for 36.413 | 8.7.0 |
| 45 | RP-090964 | 0522 |  | SRVCC to GERAN/UTRAN | 8.7.0 |
| 45 | RP-090964 | 0531 |  | Clean up the Terminology of home eNB in S1AP | 8.7.0 |
| 45 | RP-090964 | 0534 |  | Specify how report dup E-RAB ID in Tabular and replace MME with EPC in 8.3.1.2 | 8.7.0 |
| 45 | RP-090964 | 0536 | 1 | Indirect path use by the MME | 8.7.0 |
| 45 | RP-090767 | 0537 | 1 | Handling of not supported QCI values | 8.7.0 |
| 45 | RP-090964 | 0538 | 1 | E-RABs subject to forwarding | 8.7.0 |
| 45 | RP-090767 | 0540 | 1 | Mandatory NAS PDU in E-RAB Release Command | 8.7.0 |
| 45 | RP-090767 | 0542 | 1 | Missing reference and specification for encoding the CDMA2000 Pilot List | 8.7.0 |
| 45 | RP-090767 | 0547 | 1 | CR on Repetition Period IE | 8.7.0 |
| 45 | RP-090767 | 0551 |  | Miscellaneous correction to 36.413v8.6.1 | 8.7.0 |
| 45 | RP-090768 | 0553 |  | ASN1 object identified correction | 8.7.0 |
| 45 | RP-090767 | 0554 |  | Interaction between Initial Context Setup/UE Context Modification and Handover Preparation/Redirection procedures during CS Fallback | 8.7.0 |
| 09/2009 |  |  |  | Rel-9 version is created based on v.8.7.0 | 9.0.0 |
| 45 | RP-090767 | 0521 | 3 | Adding the RTD information in UPLINK CDMA2000 TUNNELING | 9.0.0 |
| 45 | RP-090787 | 0543 | 1 | Handling of Emergency Calls in Limited Service Mode | 9.0.0 |
| 45 | RP-090787 | 0544 | 1 | Emergency Calls Mobility Handling | 9.0.0 |
| 45 | RP-090776 | 0548 | 1 | S1AP Kill procedure for cancellation of PWS warning messages | 9.0.0 |
| 45 | RP-090776 | 0549 | 1 | S1AP Write-Replace Warning procedure for PWS/CMAS | 9.0.0 |
| 46 | RP-091191 | 0513 | 4 | Support for paging optimization with CSG membership changes | 9.1.0 |
| 46 | RP-091191 | 0550 | 3 | Inclusion of Access Mode and Subscription Status for UE prioritisation in LTE hybrid cells | 9.1.0 |
| 46 | RP-091194 | 0557 |  | Handling of Multiple concurrent CMAS Warning Notifications | 9.1.0 |
| 46 | RP-091189 | 0563 | 2 | CR for Transportation support for LPPa | 9.1.0 |
| 46 | RP-091195 | 0567 | 3 | Introducing the “Data Forwarding Not Possible” indication to HANDOVER REQUEST | 9.1.0 |
| 46 | RP-091183 | 0569 |  | ASN.1 correction for BroadcastCompleteAreaList | 9.1.0 |
| 46 | RP-091183 | 0571 | 1 | Correction on abnormal handling of Subscriber Profile ID for RAT/Frequency priority IE | 9.1.0 |
| 46 | RP-091368 | 0580 |  | Align IE’s in Tabular for two messages with their ASN.1 for R9 | 9.1.0 |
| 46 | RP-091183 | 0589 | 2 | Rejection Criteria for Overload | 9.1.0 |
| 46 | RP-091369 | 0592 | 2 | Introduction of inbound LTE mobility | 9.1.0 |
| 46 | RP-091194 | 0605 | 1 | Repetition Period for CMAS | 9.1.0 |
| 46 | RP-091183 | 0607 |  | Correction of E-RAB Modify | 9.1.0 |
| 46 | RP-091183 | 0616 | 1 | Clarification on handover restriction | 9.1.0 |
| 46 | RP-091183 | 0618 | 2 | Correction of Transport Layer Address | 9.1.0 |
| 46 | RP-091183 | 0621 | 1 | Missing reference and unclear handling of the CDMA2000 Sector ID | 9.1.0 |
| 47 | RP-100214 | 0623 | 1 | Correction of RTD range | 9.2.0 |
| 47 | RP-100214 | 0625 | 1 | Correction of path switch failure | 9.2.0 |
| 47 | RP-100213 | 0626 |  | Fix for Mobile terminated calls rejection in eNodeB | 9.2.0 |
| 47 | RP-100229 | 0627 |  | Introduction of PLMN-related abnormal conditions during HO in network sharing scenarios | 9.2.0 |
| 47 | RP-100222 | 0628 |  | Correction of CSG Cell and Hybrid Cell Definition | 9.2.0 |
| 47 | RP-100214 | 0629 | 3 | NCC Initialization in eNB at the Initial Connection Setup | 9.2.0 |
| 47 | RP-100228 | 0631 |  | Inter RAT Mobility Load Balance on S1 | 9.2.0 |
| 47 | RP-100213 | 0634 |  | Crrection in DOWNLIN S1 CDMA2000 TUNNELING Procedure | 9.2.0 |
| 47 | RP-100222 | 0639 | 3 | CSG expiry Handling | 9.2.0 |
| 47 | RP-100229 | 0641 | 1 | CMAS and ETWS action if Number of Broadcasts Requested IE set to 0 | 9.2.0 |
| 47 | RP-100229 | 0645 | 1 | Description of Transparent Container Encoding | 9.2.0 |
| 47 | RP-100230 | 0647 | 2 | Rapporteur’s update for S1AP protocol | 9.2.0 |
| 47 | RP-100213 | 0649 |  | Removing the restriction for Primary Notification | 9.2.0 |
| 47 | RP-100214 | 0651 | 1 | CDMA2000 1xRTT RAND format | 9.2.0 |
| 47 | RP-100213 | 0659 | 1 | Handling of the CDMA2000 RAT and Sector ID | 9.2.0 |
| 47 | RP-100214 | 0661 | 2 | Handling of CSG ID check failure in LTE hybrid cells | 9.2.0 |
| 47 | RP-100225 | 0664 | 1 | Transfer Encoding of LPPa PDUs over S1 | 9.2.0 |
| 47 | RP-100214 | 0666 | 1 | Correction of connection establishment | 9.2.0 |
| 47 | RP-100214 | 0667 | 1 | Correction of S1 Release | 9.2.0 |
| 47 | RP-100228 | 0678 |  | Creation of annex for SON Transfer and Cell Load Reporting RIM application. | 9.2.0 |
| 47 | RP-100230 | 0679 |  | Support of time and frequency synchronization using network listening | 9.2.0 |
| 04/2010 |  |  |  | ToC updated | 9.2.1 |
| 04/2010 |  |  |  | Corrupted headers and ASN.1 fixed | 9.2.2 |
| 48 | RP-100592 | 0682 | 1 | E-UTRAN Trace ID Abnormal Conditions | 9.3.0 |
| 48 | RP-100599 | 0683 | 2 | Clarification on DTM and PS Handover | 9.3.0 |
| 48 | RP-100599 | 0687 |  | Correction on UE Security Capability handling in UE Context Modification procedure | 9.3.0 |
| 48 | RP-100599 | 0693 |  | Clarification on processing Extended Repetition Period IE | 9.3.0 |
| 48 | RP-100599 | 0694 | 1 | List more apt cause in Interactions with E-RAB Management procedures section | 9.3.0 |
| 48 | RP-100596 | 0695 | 1 | Missing ETWS action if Repetition period set to 0 | 9.3.0 |
| 48 | RP-100599 | 0701 | 2 | Correction of shall to shall if supported | 9.3.0 |
| 48 | RP-100599 | 0710 |  | Correction of no DTM support | 9.3.0 |
| 48 | RP-100599 | 0711 | 2 | Correction of forbidden inter-RAT | 9.3.0 |
| 48 | RP-100599 | 0716 | 1 | Rapporteur’s update for S1AP protocol | 9.3.0 |
| 48 | RP-100599 | 0717 | 1 | S1AP Transparent containers compatible maximum message size | 9.3.0 |
| 49 | RP-100908 | 0726 | 1 | Explicit PLMN coding in Trace IEs | 9.4.0 |
| 49 | RP-100908 | 0731 | 3 | Cause value for UE context release during CSFB | 9.4.0 |
| 49 | RP-100906 | 0738 | 1 | CS Fallback Indication and Handover Restriction List | 9.4.0 |
| 49 | RP-100908 | 0741 | 1 | Correction of Repetition Period | 9.4.0 |
| 49 | RP-100908 | 0742 | 1 | Notification of Location Reporting Failure | 9.4.0 |
| 49 | RP-100908 | 0743 | 1 | Correction of UE AMBR | 9.4.0 |
| 49 | RP-100908 | 0745 |  | Simultaneous Rekeying and CSFB | 9.4.0 |
| 49 | RP-100908 | 0751 |  | Delete references to 23.041 in Tabular | 9.4.0 |
| 50 | RP-101271 | 0753 |  | Handling of CDMA2000 HO Required Indication | 9.5.0 |
| 50 | RP-101270 | 0755 |  | Correction of E-RAB Data Forwarding in HANDOVER COMMAND and DOWNLINK S1 CDMA2000 TUNNELING | 9.5.0 |
| 50 | RP-101271 | 0756 |  | Clarification on Handover Restriction List | 9.5.0 |
| 50 | RP-101271 | 0761 | 4 | Multiple PLMNs Selection in eNodeB for CS fallback | 9.5.0 |
| 50 | RP-101271 | 0780 | 2 | Clarification on SRVCC procedure in case of PS handover failure | 9.5.0 |
| 50 | RP-101271 | 0783 | 1 | Correction of GBR and MBR | 9.5.0 |
| 50 | RP-101271 | 0799 |  | Clarification on the overload action only accepting emergency and MT sessions | 9.5.0 |
| 12/2010 |  |  |  | Rel-10 version created based on v 9.5.0 | 10.0.0 |
| 50 | RP-101272 | 0752 | 2 | Prioritised handling of MPS session in S1-AP PAGING message | 10.0.0 |
| 50 | RP-101272 | 0754 | 2 | Alignment of tabular with ASN.1 for S1 Setup message | 10.0.0 |
| 50 | RP-101272 | 0764 | 2 | Enhancement of the IP address exchange mechanism for ANR purposes | 10.0.0 |
| 50 | RP-101304 | 0768 | 1 | Inter-RAT cell load reporting for multiple cells | 10.0.0 |
| 50 | RP-101304 | 0769 | 2 | Event-triggered inter-RAT cell load reporting | 10.0.0 |
| 50 | RP-101272 | 0776 | 3 | Introduction of a new overload action IE to permit high priority access | 10.0.0 |
| 50 | RP-101304 | 0791 | 2 | Inter-RAT MRO for Detection of too early inter-RAT handover with no RLF | 10.0.0 |
| 50 | RP-101281 | 0794 | 2 | Adding List of GUMMEIs to Overload related messages | 10.0.0 |
| 50 | RP-101272 | 0797 | 1 | Incorrect causes in the Error Indication msg | 10.0.0 |
| 50 | RP-101279 | 0798 | 4 | X2 handover support | 10.0.0 |
| 50 | RP-101272 | 0800 | 1 | Clarification on the overload action only accepting emergency and MT sessions | 10.0.0 |
| 01/2011 |  |  |  | Editorial change: highlighting removed | 10.0.1 |
| SP-49 | SP-100629 |  |  | Clarification on the use of References (TS 21.801 CR#0030) | 10.1.0 |
| 51 | RP-110231 | 0801 |  | Correct the criticality for two new IEs to support X2 HO for HeNB | 10.1.0 |
| 51 | RP-110239 | 0802 |  | Clean-up for Rel-10 enhancements of SON Transfer application | 10.1.0 |
| 51 | RP-110226 | 0803 |  | Clarification containers for CS only SRVCC towards UTRAN without PS HO support | 10.1.0 |
| 51 | RP-110225 | 0804 |  | Correction to the editor notes | 10.1.0 |
| 51 | RP-110225 | 0805 |  | Correction on CSG Subcription List | 10.1.0 |
| 51 | RP-110222 | 0808 |  | Correction of CSFB related Cause Values | 10.1.0 |
| 51 | RP-110236 | 0809 | 2 | Relay Node indication to MME | 10.1.0 |
| 51 | RP-110236 | 0810 |  | GUMMEI List in Overload Start and Overload Stop message | 10.1.0 |
| 51 | RP-110222 | 0812 |  | ASN.1 Correction for the Broadcast Cancelled Area List IE | 10.1.0 |
| 51 | RP-110227 | 0813 | 2 | LIPA Impact In RAN3 | 10.1.0 |
| 51 | RP-110227 | 0814 |  | S1 Release for LIPA Bearer | 10.1.0 |
| 51 | RP-110230 | 0815 | 2 | Support for MDT | 10.1.0 |
| 51 | RP-110236 | 0820 | 1 | Advertising support to RNs at the MME | 10.1.0 |
| 51 | RP-110225 | 0823 | 1 | Introduction of SPID into DOWNLINK NAS TRANSPORT message | 10.1.0 |
| 51 | RP-110226 | 0824 | 2 | NNSF Abbreviation and other Editorials | 10.1.0 |
| 51 | RP-110226 | 0827 | 2 | Clarification on TEID value range for S1AP | 10.1.0 |
| 51 | RP-110222 | 0833 | 2 | Correction of Write Replace Warning abnormal condition | 10.1.0 |
| 51 | RP-110226 | 0839 |  | Correction of the name for Time Synchronization Info IE | 10.1.0 |
| 51 | RP-110226 | 0840 | 1 | Typo correction in Message Type IE table | 10.1.0 |
| 51 | RP-110231 | 0848 | 1 | Correction of Source MME GUMMEI IE criticality in PATH SWITCH REQUEST message | 10.1.0 |
| 51 | RP-110226 | 0852 | 1 | Correction of Duplicated Warning Messages | 10.1.0 |
| 51 | RP-110234 | 0854 | 1 | Introduction of MTC Overload Support | 10.1.0 |
| 51 | RP-110231 | 0857 | 3 | Correction of Mobility to Open HeNBs | 10.1.0 |
| 51 | RP-110226 | 0860 |  | S1AP Procedure Text General Clean-up | 10.1.0 |
| 51 | RP-110225 | 0863 |  | Correction to the Semantics Description of TAC | 10.1.0 |
| 51 | RP-110226 | 0864 |  | Introduction of a Stepwise Load Reduction Indication for the Overload procedure in Stage 3 | 10.1.0 |
| 52 | RP-110695 | 0865 | 1 | MDT correction for TAI | 10.2.0 |
| 52 | RP-110688 | 0870 | 1 | Usage of the transparent containers for SRVCC | 10.2.0 |
| 52 | RP-110688 | 0871 | 1 | Removal of DTM capability for UTRAN PS HO | 10.2.0 |
| 52 | RP-110687 | 0874 | 1 | UE context release correction | 10.2.0 |
| 52 | RP-110700 | 0878 |  | Correction to the semantic description of Cell Load Reporting Cause IE | 10.2.0 |
| 52 | RP-110682 | 0885 | 1 | Correction of Target ID | 10.2.0 |
| 52 | RP-110689 | 0886 | 2 | Review of Initial Context Setup | 10.2.0 |
| 52 | RP-110689 | 0887 |  | Correction of SPID | 10.2.0 |
| 52 | RP-110689 | 0889 | 1 | Overload Consistency Handling | 10.2.0 |
| 52 | RP-110689 | 0892 | 2 | Clarification of “Redirection towards 1xRTT” cause code | 10.2.0 |
| 52 | RP-110695 | 0900 | 3 | Support for MDT user consent | 10.2.0 |
| 52 | RP-110684 | 0903 |  | Correction of References | 10.2.0 |
| 52 | RP-110686 | 0904 | 2 | General clean-up before Rel-10 ASN.1 closure | 10.2.0 |
| 52 | RP-110698 | 0905 | 1 | Clarification of MME,HeNB GW and Relay Node functions | 10.2.0 |
| 52 | RP-110687 | 0910 | 3 | Error Handling for LIPA | 10.2.0 |
| 52 | RP-110695 | 0911 | 2 | MDT amendments | 10.2.0 |
| 52 | RP-110695 | 0912 | 1 | Correction of trace function and trace session | 10.2.0 |
| 52 | RP-110714 | 0913 | 2 | Remove the UE context in the source HeNB-GW after HeNB-HeNB X2 HO | 10.2.0 |
| 53 | RP-111197 | 0914 |  | Correction on the Order of Transparent Containers | 10.3.0 |
| 53 | RP-111196 | 0919 | 1 | Correction of an ASN.1 typo regarding ManagementBasedMDTAllowed | 10.3.0 |
| 53 | RP-111197 | 0923 | 1 | Data Forwarding correction | 10.3.0 |
| 53 | RP-111195 | 0924 | 2 | Definition of value of bit in Measurements to Activate | 10.3.0 |
| 53 | RP-111195 | 0927 | 1 | Correction of RIM function decsription | 10.3.0 |
| 53 | RP-111196 | 0928 | - | Missing procedure code for “Kill” | 10.3.0 |
| 53 | RP-111196 | 0930 | 1 | Correction of Emergency Call | 10.3.0 |
| 53 | RP-111198 | 0933 | 2 | Container Issue | 10.3.0 |
| 53 | RP-111196 | 0935 | 1 | Correction of SRVCC | 10.3.0 |
| 53 | RP-111197 | 0940 | - | Clarification on PLMN Identity | 10.3.0 |
| 54 | RP-111648 | 0941 |  | Definition of Maximum no. of candidate cells | 10.4.0 |
| 54 | RP-111651 | 0943 |  | Correction of Emergency Call | 10.4.0 |
| 54 | RP-111651 | 0944 |  | Correction of the annex on the processing of transparent containers at MME | 10.4.0 |
| 54 | RP-111648 | 0945 | 1 | GW Context Release Indication correction | 10.4.0 |
| 54 | RP-111649 | 0954 | 3 | Alignment on privacy requirements for MDT | 10.4.0 |
| 55 | RP-120233 | 0956 | 1 | Corrections for SON Transfer RIM application | 10.5.0 |
| 55 | RP-120234 | 0969 | 2 | Correct of reset | 10.5.0 |
| 55 | RP-120234 | 0978 | 2 | Octet String for E-CGI | 10.5.0 |
| 56 | RP-120744 | 0980 | - | Correction on ETWS and CMAS | 10.6.0 |
| 06/2012 |  |  |  | Rel-11 version created based on v 10.6.0 | 11.0.0 |
| 56 | RP-120751 | 0981 | - | Introduction of the Security Algorithm (ZUC) | 11.0.0 |
| 56 | RP-120752 | 0998 | 2 | Correction on Emergency ARP Value | 11.0.0 |
| 56 | RP-120752 | 1007 | 1 | Improved granularity for the time UE stayed in cell | 11.0.0 |
| 56 | RP-120747 | 1010 | - | SON Transfer application for IRAT Network Energy Savings | 11.0.0 |
| 57 | RP-121140 | 1011 | 2 | UE Radio Capability Match Indicator for Voice Continuity | 11.1.0 |
| 57 | RP-121140 | 1018 |  | Correction of GUMMEI | 11.1.0 |
| 57 | RP-121138 | 1034 | 1 | Corrections for IRAT Network Energy Savings | 11.1.0 |
| 57 | RP-121135 | 1041 | 2 | Addition of HO cause value to the UE history information in S1AP | 11.1.0 |
| 57 | RP-121138 | 1044 | 1 | Energy Saving UE Measurement (“Probing”) | 11.1.0 |
| 58 | RP-121730 | 1048 | 3 | Introduction of new MDT measurements | 11.2.0 |
| 58 | RP-121736 | 1049 |  | Verification of HeNB | 11.2.0 |
| 58 | RP-121732 | 1052 | 2 | Membership verification during Path Switch Request procedure (Option A) | 11.2.0 |
| 58 | RP-121737 | 1056 | 1 | Rapporteur editorial corrections | 11.2.0 |
| 58 | RP-121737 | 1057 |  | Rapporteur correction of constants’ names | 11.2.0 |
| 58 | RP-121730 | 1061 | 2 | Multi-PLMN MDT | 11.2.0 |
| 58 | RP-121736 | 1062 |  | Correction of Capability Match Request | 11.2.0 |
| 58 | RP-121731 | 1068 | 2 | Introduce support for Inter-RAT MRO | 11.2.0 |
| 58 | RP-121739 | 1074 | 2 | New Information for BBF access | 11.2.0 |
| 58 | RP-121736 | 1075 | 1 | Establishment of UE-associated logical S1-connection in eNB | 11.2.0 |
| 02/2013 |  |  |  | History table update | 11.2.1 |
| 59 | RP-130211 | 1095 | - | Correction of GUMMEI Type Criticality | 11.3.0 |
| 59 | RP-130211 | 1097 | 2 | ASN.1 review for S1AP | 11.3.0 |
| 59 | RP-130211 | 1100 | 1 | Clarification of Warning Area List IE | 11.3.0 |
| 59 | RP-130212 | 1106 | 2 | Invalidation of ETWS with security feature | 11.3.0 |
| 59 | RP-130211 | 1108 | 3 | Correction of Classmark Encoding | 11.3.0 |
| 59 | RP-130210 | 1109 | 1 | S1AP modification for PDCP SN extension | 11.3.0 |
| 60 | RP-130641 | 1110 | 1 | Correction for the MDT Location Information IE | 11.4.0 |
| 60 | RP-130643 | 1111 |  | Correction of the presence of the X2 TNL Configuration Info IE inside the SON Configuration Transfer IE tabular definition | 11.4.0 |
| 60 | RP-130643 | 1116 | 1 | Correction of Kill | 11.4.0 |
| 61 | RP-131182 | 1127 | 1 | Correction on LPPa Signalling Transport Function to support UTDOA | 11.5.0 |
| 61 | RP-131183 | 1140 | 2 | Correction of terminology concerning the mobility restriction function | 11.5.0 |
| 62 | RP-131902 | 1147 | 3 | Correction of Handover Restriction List | 11.6.0 |
| 62 | RP-131902 | 1150 | 1 | Correction for Load Balancing Related cause value CR for 36413 | 11.6.0 |
| 62 | RP-131901 | 1153 | 4 | Correction on CSFB high priority indication | 11.6.0 |
| 62 | RP-131902 | 1164 | 1 | Correction of UE Radio Capability Match | 11.6.0 |
| 62 | RP-131909 | 1128 | 4 | Introduction of Collocated L-GW for SIPTO@LN | 12.0.0 |
| 62 | RP-131910 | 1143 | 3 | Kill All Warning Messages | 12.0.0 |
| 62 | RP-131979 | 1166 | 1 | Update of reference to 3GPP2 specification | 12.0.0 |
| 62 | RP-131909 | 1171 | 1 | Introduction of SIPTO@LN Stand-Alone in S1AP | 12.0.0 |
| 63 | RP-140296 | 1151 | 6 | Introduce support for load reporting between LTE and eHRPD | 12.1.0 |
| 63 | RP-140297 | 1163 | 3 | Reporting of User Location Information at E-RAB release | 12.1.0 |
| 63 | RP-140297 | 1180 | 1 | New CSFB high priority indication for eMPS and emergency call | 12.1.0 |
| 63 | RP-140298 | 1181 | 1 | Introduction of Restart Indication for PWS | 12.1.0 |
| 63 | RP-140295 | 1196 | 2 | Correction of contradictions for kill-all functionality | 12.1.0 |
| 64 | RP-140906 | 1179 | 3 | Provide IMEISV to eNB to identify UE characteristics | 12.2.0 |
| 64 | RP-140897 | 1185 | 4 | Enhance TNL Address Discovery procedure for X2 GW | 12.2.0 |
| 64 | RP-140894 | 1191 | 4 | Correction of SRVCC to GERAN | 12.2.0 |
| 64 | RP-140902 | 1200 | 1 | Correction on Kill-all Warning Messages Indicator | 12.2.0 |
| 64 | RP-140903 | 1208 | 1 | Correction of OCTET STRING for eHRPD Sector ID | 12.2.0 |
| 64 | RP-140905 | 1215 | 1 | Correction of MME STATUS TRANSFER | 12.2.0 |
| 64 | RP-140905 | 1220 | - | Correction on Inter-RAT Cell ID in SON Transfer | 12.2.0 |
| 65 | RP-141520 | 1216 | 2 | Introduction of the UE history reported from the UE | 12.3.0 |
| 65 | RP-141522 | 1219 | 5 | Introduction of MBMS MDT | 12.3.0 |
| 65 | RP-141518 | 1231 | 2 | Introduction of an indication of the expected UE behaviour | 12.3.0 |
| 65 | RP-141513 | 1243 | 2 | Correction of Transparent Container encoding for PS Handover to GERAN | 12.3.0 |
| 65 | RP-141514 | 1246 | 2 | Correction of Transparent Containers usage in annex C | 12.3.0 |
| 65 | RP-141521 | 1255 | 1 | Paging enhancements for Low Complexity UE | 12.3.0 |
| 66 | RP-142082 | 1198 | 9 | Addition of RLF reporting over S1 | 12.4.0 |
| 66 | RP-142089 | 1214 | 8 | Introduction of Dual Connectivity | 12.4.0 |
| 66 | RP-142093 | 1238 | 2 | Rapporteur Review | 12.4.0 |
| 66 | RP-142088 | 1257 | 8 | Enabling Radio Interface based Synchronisation via S1 Signalling | 12.4.0 |
| 66 | RP-142095 | 1274 | 2 | HO Report Enhancements to reduce IRAT configuration | 12.4.0 |
| 67 | RP-150353 | 1230 | 6 | ProSe UE Authorization in S1AP | 12.5.0 |
| 67 | RP-150356 | 1276 | 1 | Corrections of SON configuration transfer | 12.5.0 |
| 67 | RP-150356 | 1281 | 2 | Rapporteur Review-ASN.1 consistency check | 12.5.0 |
| 67 | RP-150352 | 1285 | 2 | Correction of reloading PWS Alerts | 12.5.0 |
| 68 | RP-150943 | 1289 | 2 | Add indication in the E-RAB MODIFICATION CONFIRM for E-RAB(s) that shall be released | 12.6.0 |
| 68 | RP-150943 | 1293 |  | Adding Criticality Diagnostics in E-RAB Modification Confirm message | 12.6.0 |
| 68 | RP-150944 | 1305 | 3 | Masked IMEISV IE correction | 12.6.0 |
| 68 | RP-150943 | 1310 | 1 | Correction of Muting procedure | 12.6.0 |
| 68 | RP-150944 | 1311 | 2 | Correction of PWS Broadcast Completed Area List | 12.6.0 |
| 68 | RP-150944 | 1312 | 1 | Updating SRVCC Operation Possible in EUTRAN | 12.6.0 |
| 06/2015 |  |  |  | Rel-13 version created based on v 12.6.0 | 13.0.0 |
| 68 | RP-150945 | 1303 | 2 | eNB behaviour for IRAT handovers in AAS | 13.0.0 |
| 70 | RP-152100 | 1315 | 6 | ProSe UE Relaying Support in S1AP | 13.1.0 |
| 70 | RP-152099 | 1345 | 2 | Extension of PDCP SN | 13.1.0 |
| 70 | RP-152102 | 1348 | 4 | Adding CSG support to DC | 13.1.0 |
| 70 | RP-152096 | 1362 | 4 | Introduction of Dedicated Core Network (DCN) feature | 13.1.0 |
| 70 | RP-152102 | 1368 | 1 | Support of SIPTO and LIPA for DC | 13.1.0 |
| 70 | RP-152103 | 1369 | 3 | Introduction of feMDT | 13.1.0 |
| 70 | RP-152102 | 1372 | - | Tunnel Information of BBAI in Dual Connectivity | 13.1.0 |
| 70 | RP-152108 | 1373 | 2 | Introduction of PWS Failure Indication message | 13.1.0 |
| 71 | RP-160442 | 1377 | 2 | Introduction of Paging Optimisation and Paging for Coverage Enhancement capable UEs | 13.2.0 |
| 71 | RP-160447 | 1387 | 1 | Addition of new RRC establishment cause to S1AP for VoLTE | 13.2.0 |
| 71 | RP-160443 | 1388 | 2 | Introduction of eDRX parameters in the paging message | 13.2.0 |
| 71 | RP-160449 | 1398 |  | Rapporteur Review on 36.413 | 13.2.0 |
| 71 | RP-160449 | 1401 | 2 | UE context retention at SCTP recovery | 13.2.0 |
| 71 | RP-160448 | 1403 | 1 | Providing UE Usage Type in Dedicated Core Network Reroute NAS Request procedure | 13.2.0 |
| 71 | RP-160451 | 1408 | 2 | Correction on CSG support in DC enhancement | 13.2.0 |
| 71 | RP-160442 | 1411 | 2 | Introduction of new UE Identity in the paging message | 13.2.0 |
|  | MFA-5 | mf2016.365 | 2 | Modification for MulteFire in 36.413 | Mf2016.462.01 |
|  | MFA-6 | Mf2016.586 | 1 | Updates for TS 36.413 | Mf2016.462.02 |
|  | MFA-7 | Mf2016.819 | 1 | S2a indicator and ASN.1 changes for TS 36.413 | Mf2016.462.03 |
|  | MFA-7 |  |  | Corrected the MulteFire NHN reference and some minor ASN.1 errors introduced in Mf2016.462.03 | Mf2016.462.04 |
|  | MFA-8 |  | 2 | Editorial Corrections and references update | 1.0.1 |
|  | MFA-9 | Mf2017.067.00 | 3 | Update baseline based on 3GPP TS 36.413 version 13.5.0, plus several minor editorial corrections | 1.0.2 |
|  | MFA-12 | Mf2017.844.01 | 4 | Removal of last PSP id on S1AP | 1.0.3 |