MFA TS MF.201 V1.0.2 (2017-02)

Technical Specification

MulteFire Alliance

Architecture for PLMN Access Mode 
Stage 2 (Release 1.0)

The present document has been developed within MulteFire Alliance and may be further elaborated for the purposes of the MulteFire Alliance.
The MulteFire Alliance accepts no liability for any use of this Specification.

Keywords

MF, multefire, neutral host network, stage 2, NHN

MulteFire Alliance

Postal address

5177 Brandin Court

Fremont, CA 94538 USA

Internet

http://www.multefire.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016-2017, MulteFire Alliance

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

1	Scope	3

2	References	3

3	Definitions, symbols and abbreviations	4

3.1	Definitions	4

3.2	Symbols	4

3.3	Abbreviations	4

4	Overview	4

5	Architecture Model and Concepts	4

5.1	Concepts	4

5.2	Architecture Reference Model	5

5.3	Network Elements	6

5.4	Reference Points	6

5.5	Identities	6

5.6	Network Discovery and Selection	7

5.7	Bearer model and QoS Concepts	7

5.8	Charging	7

5.9	Interworking with 3GPP systems	7

5.10	Information Storage	7

5.11	IMS Emergency Session Support	7

5.12	Warning Message Delivery	7

5.13	Multimedia Priority Services (MPS)	7

5.14	Isolated E-UTRAN Operation for Public Safety (IOPS)	8

5.15	Access Information and Restriction	8

5.16	MF RAN Sharing	8

5.17	Features not supported with MF RAN	8

Annex A:	Change History	9

# 1	Scope

The present document provides a specification on the deployment of MulteFire technology when the MulteFire network is used to access the 3GPP Rel-13 EPC. The related use cases are described in MFA TR MF.101 [2]. MulteFire cells may support two operating modes:

- PLMN access mode

- NHN access mode

Public Land Mobile Network (PLMN) access mode provides connectivity to EPCs of specific PLMNs. Mechanisms for PLMN access mode are based on 3GPP EPS specifications, such as 3GPP TS 23.401 [3]. MulteFire-specific issues for PLMN access mode are described in this document.

Neutral Host Network (NHN) access mode provides connectivity to an IP Network. Mechanisms for NHN access mode are based on 3GPP EPS specifications, such as 3GPP TS 23.401 [3]. Issues that are specific for NHN access mode are described in MFA TS MF.202 [7].

# 2	References

For interpretations of references, the defined rules in Section 4 of MFA TR MF.100 [0] apply.

[0]	MulteFire Alliance (MFA). Guidelines for MulteFire Specifications. MFA TR MF.100.

[1]	Third Generation Partnership Project (3GPP). Vocabulary for 3GPP Specifications. 3GPP TR 21.905.

[2]	MulteFire Alliance (MFA). MulteFire Deployment Scenarios and Requirements (Release 1). MFA TR MF.101.

[3]	Third Generation Partnership Project (3GPP). General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access. 3GPP TS 23.401.

[4]	MulteFire Alliance (MFA), in collaboration with Third Generation Partnership Project (3GPP). Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall Description; Stage 2. MFA TS 36.300.

[5]	Third Generation Partnership Project (3GPP). Architecture enhancements for Non-3GPP accesses. 3GPP TS 23.402.

[6]	Void

[7]	MulteFire Alliance (MFA). Architecture for Neutral Host Network Access Mode: Stage 2 (Release 1). MFA TS MF.202.

[8]	Void

[9]	Third Generation Partnership Project (3GPP). Service aspects and requirements for network sharing. 3GPP TR 22.951.

[10]	Third Generation Partnership Project (3GPP). Network architecture. 3GPP TS 23.002

[11]	Third Generation Partnership Project (3GPP). Technical Realization of Cell Broadcast Service (CBS). 3GPP TS 23.041.

[12]	MulteFire Alliance (MFA), in collaboration with Third Generation Partnership Project (3GPP). Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode. MFA TS 36.304.

[13]	MulteFire Alliance (MFA), in collaboration with Third Generation Partnership Project (3GPP). Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 1). MFA TS 36.331.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions described in 3GPP TR 21.905 [1], 3GPP TR 22.951 [9] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TR 22.951 [9].

## 3.2	Symbols

## 3.3	Abbreviations

| Abbreviation | Definition |
| --- | --- |
| CAI | Cell Access Information |
| CIoT | Cellular-based Internet of Things |
| GWCN | Gateway Core Network |
| IOPS | Isolated E-UTRAN Operation for Public Safety |
| MF | MulteFire |
| MF-AP | MulteFire Access Point |
| MF-UE | MulteFire User Equipment |
| MOCN | Multi-Operator Core Network |
| MPS | Multimedia Priority Service |
| NHAMI | Neutral Host Access Mode Information |
| NHN | Neutral Host Network |
| NHN-ID | Neutral Host Network Identifier |
| PCC | Policy and Charging Control |
| PGW | Packet Data Network Gateway |
| SGW | Serving Gateway |
| TAC | Tracking Area Code |
| ULI | User Location Information |

# 4	Overview

This document describes the network architecture for the deployment when the MulteFire RAN is used to access 3GPP Rel-13 EPC. This architecture addresses the Standalone Deployment Scenario without External Interworking and Tight 3GPP IW Deployment Scenarios of MFA TR MF.101 [2].

# 5	Architecture Model and Concepts

## 5.1	Concepts

The concept for MulteFire PLMN access is that the MF RAN is connected to a 3GPP EPC in a similar manner as E-UTRAN is connected to EPC. MF RAN supports all functions of Rel-13 E-UTRAN in the same way as a Rel-13 E-UTRAN from EPC point of view unless it is explicitly described otherwise in this specification. The internal behaviour of MF RAN is described in MFA TS 36.300 [4].

Figure 5.1-1 shows the relationship between an MF Cell, the PLMN and the NHN. An MF Cell may support PLMN access mode for specific PLMNs. When the MF Cell supports PLMN access mode for a PLMN, it broadcasts the corresponding PLMN ID. The MF Cell is considered to be part of the PLMN whose PLMN-ID it broadcasts. RAN-sharing solutions may be used to share MF cells across multiple PLMNs.

The PLMN architecture for PLMN access mode is specified in 3GPP specifications such as 3GPP TS 23.002 [10] and 3GPP TS 23.401 [3]. MF-specific extensions for PLMN architecture are described in this document.

An MF cell may support NHN access mode for a specific Neutral Host Network. When the MF cell supports NHN access mode for a specific NHN, it broadcasts the Neutral Host Access Mode Indicator (NHAMI) and the NHN-ID of the accessible NHN. The MF cell is considered to be part of the NHN whose NHN-ID it broadcasts. RAN-sharing solutions may be used to share MF cells across one NHN and one or more PLMNs.

NHN architecture for NHN access mode is based on 3GPP specifications such as 3GPP TS 23.002 [10] and 3GPP TS 23.401 [3]. NHN-specific issues are described further in MFA TS MF.202 [7].

A UE can attach a MulteFire cell simultaneously using both PLMN and NHN access mode, only if it is capable of maintaining multiple active RRC connections (multiple radios for example).

For the scenario when one MF cell supports both PLMN access mode and NHN access mode, MFA TS MF.202 [7] describes what PLMN ID to use for the different PLMN-based parameters.

Figure 5.1-1: Relation between MF Cell, PLMNs and NHN

## 5.2	Architecture Reference Model

Figure 5.2-1 presents the reference architecture model for MulteFire PLMN access. The reference points within EPC are defined in 3GPP TS 23.401 [3].

Figure 5.2-1: Architecture For Tight 3GPP Interworking

Note 1:	Not all the of the 3GPP EPC reference points are shown in this figure for the sake of simplicity.

Note 2:	This architecture is valid in roaming and non-roaming scenarios. In the roaming scenario, the MulteFire RAN is connected to the EPC of VPLMN as any other 3GPP RAN.

Note 3:	Multiple EPCs can be connected to a single MF RAN.

## 5.3	Network Elements

All the network elements in the EPC are defined in 3GPP TS 23.401 [3].

The following new network elements are introduced:

| Network Element | Description |
| --- | --- |
| MF-AP | A MulteFire AP provides similar functionality as an eNB in LTE networks. Details are specified in MFA TS 36.300 [4]. |
| MF-UE | A 3GPP UE that supports functions needed to use the MulteFire RAN. |

## 5.4	Reference Points

All reference points between network elements not discussed in this section are defined in 3GPP TS 23.401 [3].

| Reference Point | Description |
| --- | --- |
| S1-MME | Reference point for the control plane protocol between the MulteFire RAN and the MME. The functionality and the protocol stack of this reference point is the same as S1-MME reference point, which is defined in 3GPP TS 23.401 [3] between E-UTRAN and MME. |
| S1-U | Reference point between the MulteFire RAN and Serving Gateway (SGW). The functionality and the protocol stack of this reference point is the same as an S1-U reference point, which is defined in 3GPP TS 23.401 [3] between E-UTRAN and the SGW. |
| Uu-M | Reference point between an MF-AP and an MF-UE. The functionality of this reference point is the similar to Uu reference point. Details are specified in MFA TS 36.300 [4]. |
| X2 | Reference point between two MF-APs and point between a MF-AP and an eNB. The functionality and the protocol stack for this reference point is defined in MFA TS 36.300 [4]. |

## 5.5	Identities

Identities are used in the same way as described in 3GPP TS 23.401 [3].

The MF RAN shares the same RAT type as E-UTRAN.

Different TAC values can be used for differentiation of MF RAN from E-UTRAN (see section 5.18). EPC can differentiate the MF RAN from the E-UTRAN by reading TAC values, which can be used in charging and PCC rules differentiation. The impact is that MME, SGW and PGW can be required to have configuration on what TAC values that are used by the MF RAN and/or the E-UTRAN.

## 5.6	Network Discovery and Selection

Network discovery and selection happens in the same way, as specified by 3GPP. A PLMN-based MF service is considered equivalent to LTE nodes in network discovery and selection, unless HPLMN provides dedicated preferences. Prioritization between MF and 3GPP radios is specified in MFA TS 36.304 [12].

## 5.7	Bearer model and QoS Concepts

The same bearer model and QoS parameters are used, as those used described in 3GPP TS 23.401 [3].

The PCC rules may be different for MF RAN from the rules used for E-UTRAN. PCC rules differentiation between MF RAN and E-UTRAN can be realized according to different TAC values.

Note:	The user experience for GBR bearers may not be the same in a MulteFire network as in a 3GPP network, due to possible interferences in unlicensed spectrum.

## 5.8	Charging

The same architecture is used for charging as the architecture described in 3GPP TS 23.401 [3].

Charging may be different in MF RAN from the charging architecture used by the E-UTRAN. Charging differentiation can be realized according to different TAC values.

## 5.9	Interworking with 3GPP systems

Handover between a MF cell and a 3GPP cell is supported with the same procedures as handover between an LTE cell to another 3GPP cell. The differing radio levels are described MFA TS 36.300 [4].

## 5.10	Information Storage

The same information stored in EPC is used as specified by 3GPP. The MF RAN shares the same RAT type with E-UTRAN, and the stored TAC values could be used to identify the MF RAN from the E-UTRAN. Regional restriction of UE subscription in HSS could be set, considering the distinction of TACs between MF RAN and E-UTRAN. An MME can set the forbidden TAs in handover restriction list accordingly.

## 5.11	IMS Emergency Session Support

An MF cell in PLMN access mode may support IMS emergency sessions, depending on local regulations. When an MF cell supports IMS emergency sessions the relevant 3GPP specifications shall be followed (see Section 4.2.12 of 3GPP TS 23.401 [3]). When an MF cell is used to support an IMS emergency session, possible interferences due to the use of unlicensed bands should be considered.

## 5.12	Warning Message Delivery

If warning message delivery is supported by the PLMN, then warning message delivery over an MF cell must follow the 3GPP specifications (see 3GPP TS 23.041 [11]). As it not guaranteed that interference will or will not occur in unlicensed spectrum used by an MF cell, no guarantee is possible that an MF-AP is able to deliver a warning message in any timeframe.

Note: 	In order to increase the probability of successful warning message delivery, operators could use longer repetition period and more frequent message delivery attempt in MF-APs than the values used in eNBs.

## 5.13	Multimedia Priority Services (MPS)

An MF cell may provide MPS in the same way as an LTE cell. When an MF cell is used to provide MPS the possible interferences due to the use unlicensed spectrum should be taken into account.

## 5.14	Isolated E-UTRAN Operation for Public Safety (IOPS)

An MF-AP may be used to provide IOPS, as it is a special configuration and deployment option, as described in Annex K of 3GPP TS 23.401 [3]. When an MF cell is used to provide IOPS, the possible interferences due to the use unlicensed spectrum should be taken into account.

## 5.15	Access Information and Restriction

Different TAC values can be used to differentiate MF RAN from E-UTRAN so that the TAC value can be used to derive if UE is camping on MF RAN or E-UTRAN.

When a UE moves between an MF cell and an E-UTRAN cell, the MME can be aware of the access change by reading the TAC reported by RAN. Subsequently, the MME forwards the changed TAC to EPC in the ULI. The EPC can identify if the UE is accessing MF RAN or E-UTRAN by reading the TAC reported by the MME.

This can be used for example in these cases:

- The PCC framework can have different policies based on the TAC value.

- Charging systems can do differentiated charging based on the TAC value.

- Subscribers can be provisioned in HSS to be restricted to access MF RAN (using regional subscriptions).

When a UE moves between a MF cell and an E-UTRAN cell in the idle mode using TAU procedure (without the active flag set), then the MME will not send updated ULI to PGW (via SGW) if the PGW has not explicitly subscribed for the information (using Change Reporting Action). The new ULI information will instead be sent in a subsequent procedure when there is a message from MME to SGW (for example at service request).

## 5.16	MF RAN Sharing

MF RAN may support the same RAN sharing methods as E-UTRAN, i.e., MOCN and GWCN. In these RAN-sharing methods a single MF RAN can be shared by a group of PLMNs.

MF RAN may also support RAN sharing between multiple PLMN groups. When an MF RAN supports sharing across such multiple PLMN groups, the MF RAN is configured to broadcast separate Cell Access Info for each PLMN group. MF RAN also exposes separate S1 and X2 interface instances for each such PLMN group as if the interface instances would originate from separate MF APs.

The EPC nodes utilized within each such PLMN group are not made aware of MF RAN S1 instances nor cell access information parameters that are related to other such PLMN groups. From the EPC point of view, there is no difference in S1 procedures compared to those with an MF RAN, which serves only EPC nodes belonging to that specific PLMN group. From the MF RAN point of view the operation towards the EPC nodes within each such PLMN group is the same as if the MF RAN would be connected just towards that PLMN group.

MF UEs detecting broadcast of multiple CAIs by one MF cell, shall perform CAI selection as specified in MFA TS 36.331 [13].

## 5.17	Features not supported with MF RAN

The following features in 3GPP TS 23.401 [3] are not supported by the MF RAN in PLMN access mode:

- Dual connectivity

- RAN-relaying function when MF is the donor

- MBMS

- RAN-assisted WLAN interworking features specified in MFA TS 36.300 [4].

- Proximity-based services

- Extended idle mode DRX (eDRX)

- CIoT Optimization

The MF-AP shall behave as an eNB that does not support these features.

# Annex A:	Change History

| Date | Document Number | Comment |
| --- | --- | --- |
| 2016-03 | mf2016.099.00 | Initial version from MFA #3 |
| 2016-04 | mf2016.099.01 | Review version from MFA #4 incorporating changes from mf2016.193.02 and mf2016.199.02 |
| 2016-04 | mf2016.099.02 | Approved version from MFA #4 incorporating changes from mf2016.193.02 and mf2016.199.02 |
| 2016-09 | mf2016.099.03 | Approved version from MFA #6 incorporating changes from mf2016.581.01 and mf2016.587.02 |
| 2016-11 | mf2016.099.04 | Approved version from MFA #7 incorporating changes from mf2016.811.01 |
| 2016-11 | MFA TS MF.201-v1.0.0 | Initial Rel-1 version (same as previous version) |
| 2017-01 | MFA TS MF.201-v1.0.1 | Approved version from MFA #8 incorporating changes from mf2016.1017.01 |
| 2017-02 | MFA TS MF.201-v1.0.2 | Approved version from MFA #9 incorporating changes from mf2017.060.00 |