MulteFire Alliance

Architecture for Neutral Host Network Access Mode
Stage 2 (Release 1)

MFA TS MF.202 V1.0.4 (2017-09)

Technical Specification

The present document has been developed within MulteFire Alliance and may be further elaborated for the purposes of the MulteFire Alliance.
The MulteFire Alliance accepts no liability for any use of this Specification.

Keywords

multefire, neutral host network, stage 2, NHN, release 1

MulteFire Alliance

Postal address

5177 Brandin Court

Fremont, CA 94538 USA

Internet

http://www.multefire.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016-2017, MulteFire Alliance

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	10

5	Architecture Model and Concepts	11

5.1	Concepts	11

5.2	Architecture Reference Model	11

5.3	Network Elements	12

5.4	Reference Points	13

5.5	Neutral Host System Information Broadcast	13

5.6	Utilization of AAA servers for NHN access mode	14

5.7	Identities	14

5.7.1	NHN Access Mode Specific Identities	14

5.7.2	Custom rules for using existing 3GPP identifiers for NHN access mode	15

5.8	UE Aspects	16

5.8.1	Subscription and credentials	16

5.9	Interworking with 3GPP systems	17

5.10	Untrusted Access and Trusted Access Modes	18

5.10.1	General	18

5.10.2	NHN-specific considerations for using SWa and STa interfaces	18

5.11	Network Discovery and Selection	19

5.11.1	General	19

5.11.2	Initial PSP Discovery	19

5.11.3	PSP and NHN network selection	19

5.11.4	NHN reselection	20

5.11.5	Network selection outcome	20

5.11.6	Out-of-band service discovery	20

5.11.7	NHN Service Discovery	20

5.12	Authentication and Security	22

5.12.1	General	22

5.12.2	EAP/NAS sublayer interaction	24

5.12.3	EAP over NAS authentication procedures	25

5.12.4	KASME Derivation from MSK	31

5.13	Access Service Authorization	31

5.13.1	Predefined Access Service	31

5.13.2 	Dynamically Selectable Access Service	32

5.14	Online Sign Up	33

5.14.1	General	33

5.14.2	Online Signup Procedure	34

5.15	Bearer model and QoS Concepts	36

5.16	Charging	36

5.17	Multiple PDN Support	36

5.18	Location Reporting	36

5.19	Information Storage	36

5.20	MDT	36

5.21	Warning Message Delivery	37

5.22	Features that are not supported or modified	37

6	Functional Description and Procedures	37

6.1	Control and User Plane Protocol Stacks	37

6.1.1	Control Plane Protocol Stack	37

6.1.2	User Plane Protocol Stack	38

6.2	Procedures within MulteFire RAN	38

6.2.1	General	38

6.2.2	Initial Attach	39

6.2.3	Tracking Area Update	42

6.2.4	S1 Setup	43

6.2.5	MME Configuration Update	43

6.2.6	Bearer-related procedures	44

6.2.7	X2-based handover	45

6.2.8	S1-based handover	45

6.2.9	Service Request	45

6.2.10	Detach-related procedures	46

6.2.11	Multiple PDN Support	47

6.3	Procedures within NHN Core	47

6.3.1	General	47

6.3.2	Requirements for NHN Core Network Elements	47

Annex A (informative): Change History	55

Annex B (Informative): Out-of-Band Service Discovery	56

Annex C (informative): Access service authorization examples	57

C.1 General	57

C.2 Examples	57

Annex D: PSP and NHN Selection (Normative)	59

# 1	Scope

MulteFire cells may support two operating modes:

- PLMN access mode

- NHN access mode

PLMN access mode provides connectivity to EPCs of specific PLMNs. Mechanisms for PLMN access mode are based on 3GPP EPS specifications, as described in 3GPP TS 23.401[3]. MulteFire-specific issues for PLMN access mode are described in MFA TS MF.201 [6].

NHN (Neutral Host Network) access mode provides connectivity to an IP Network. Mechanisms for NHN access mode are based on 3GPP EPS specifications, as described in 3GPP TS 23.401 [3]. Issues that are specific for NHN access mode are described in this document.

# 2	References

For interpretations of references, the defined rules in Section 4 of MFA TR MF.100 [0] apply.

[0]	MulteFire Alliance (MFA). Guidelines for MulteFire Specifications. MFA TR MF.100.

[1]	Third Generation Partnership Project (3GPP). Vocabulary for 3GPP Specifications, 3GPP TR 21.905.

[2]	MulteFire Alliance (MFA). MulteFire Deployment Scenarios and Requirements (Release 1), MFA TR MF.101.

[3]	Third Generation Partnership Project (3GPP). General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access. 3GPP TS 23.401.

[4]	Void

[5]	Third Generation Partnership Project (3GPP). Architecture enhancements for non-3GPP accesses. 3GPP TS 23.402.

[6]	MulteFire Alliance (MFA), in collaboration with Third Generation Partnership Project (3GPP). MulteFire Alliance Architecture for PLMN access mode (Stage 2) (Release 1). MFA TS MF.201.

[7]	MulteFire Alliance (MFA), in collaboration with Third Generation Partnership Project (3GPP). 3GPP System Architecture Evolution (SAE); Security architecture. MFA TS 33.401.

[8]	The Internet Engineering Task Force (IETF). Diameter Base Protocol, edited by V. Fajardo et al., October 2012. IETF RFC 6733, also available at https://tools.ietf.org/html/rfc6733.

[9]	The Internet Engineering Task Force (IETF). RADIUS (Remote Authentication Dial In User Service) Support for Extensible Authentication Protocol (EAP), edited by B. Aboba and P. Calhoun, September 2003. IETF RFC 3579, also available at https://tools.ietf.org/html/rfc3579.

[10]	The Internet Engineering Task Force (IETF). The Network Access Identifier, A. DeKok, May 2015. IETF RFC 7542, also available at https://tools.ietf.org/html/rfc7542.

[11]	The Internet Engineering Task Force (IETF). Extensible Authentication Protocol (EAP) Key Management Framework, edited by B. Aboba, D. Simon, and P. Eronen, August 2008. IETF RFC 5247, also available at https://tools.ietf.org/html/rfc5247.

[12]	The Internet Engineering Task Force (IETF). The EAP-TLS Authentication Protocol, edited by D. Simon, B. Aboba, and R. Hurst, March 2008. IETF RFC 5216, also available at https://tools.ietf.org/html/rfc5216.

[13]	The Internet Engineering Task Force (IETF). Extensible Authentication Protocol Tunnelled Transport Layer Security Authenticated Protocol Version 0 (EAP-TTLSv0), edited by P. Funk and S. Blake-Wilson, August 2008. IETF RFC 5281, also available at https://tools.ietf.org/html/rfc5281.

[14]	The Internet Engineering Task Force (IETF). Microsoft PPP CHAP Extensions, Version 2, edited by G. Zorn, January 2000. IETF RFC 2759, also available at https://tools.ietf.org/html/rfc2759.

[15]	The Internet Engineering Task Force (IETF). Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA’), edited by J. Arkko, V. Lehtovira, and P. Eronen, May 2009. IETF RFC 5448, also available at https://tools.ietf.org/html/rfc5448.

[16]	The Internet Engineering Task Force (IETF). Transport Layer Security (TLS) Extensions: Extension Definitions, edited by D. Eastlake, January 2011. IETF RFC 6066, also available at https://tools.ietf.org/html/rfc6066.

[17]	Third Generation Partnership Project (3GPP). 3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses. 3GPP TS 33.402.

[18]	Third Generation Partnership Project (3GPP). Evolved Packet System (EPS); 3GPP EPS AAA interfaces. 3GPP TS 29.273,

[19]	Third Generation Partnership Project (3GPP). Numbering, addressing and identification. 3GPP TS 23.003.

[20]	Third Generation Partnership Project (3GPP). Network Domain Security (NDS); Authentication Framework. 3GPP TS 33.310.

[21]	Wi-Fi Alliance. Hotspot 2.0 (Release 2) Technical Specification. Available at http://www.wi-fi.org/file/hotspot-20-release-2-technical-specification-package-v110-0 (Registration required to view).

[22]	The Internet Engineering Task Force (IETF). 	Diameter Extensible Authentication Protocol (EAP) Application, edited by P. Eronen, T. Hiller, and G. Zorn, August 2005. IETF RFC 4072, also available at https://tools.ietf.org/html/rfc4072.

[23]	Third Generation Partnership Project (3GPP). Policy and charging control architecture. 3GPP TS 23.203.

[24]	MulteFire Alliance (MFA), in collaboration with Third Generation Partnership Project (3GPP). MulteFire Alliance S1 Application Protocol (S1AP) (Release 1). MFA TS 36.413.

[25]	The Internet Engineering Task Force (IETF). Diameter Network Access Server Application, edited by G. Zorn, April 2014. IETF RFC 7155, also available at https://tools.ietf.org/html/rfc7155.

[26]	The Internet Engineering Task Force (IETF). RADIUS Accounting, edited by C. Rigney, June 2000. IETF RFC 2866, also available at https://tools.ietf.org/html/rfc2866.

[27]	The Internet Engineering Task Force (IETF). US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF), edited by D. Eastlake and T. Hansen, May 2011. IETF RFC 6234, also available at https://tools.ietf.org/html/rfc2866.

[28]	Third Generation Partnership Project (3GPP). Technical Realization of Cell Broadcast Service (CBS). 3GPP TS 23.041.

[29]	Third Generation Partnership Project (3GPP). Mobile Radio Interface Layer 3 specification; Core network protocols; Stage 3. 3GPP TS 24.008.

[30]	MulteFire Alliance. MulteFire Subscription Management (Stage 3) (Release 1). MFA TS MF.301.

[31]	IEEE Standards Association. Guidelines for Use of the 24-bit Organizationally Unique Identifiers (OUI).

[32]	IEEE Standards Association. Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications, March 2012. IEEE 802.11-2012, Part 11.

[33]	Third Generation Partnership Project (3GPP). Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode. 3GPP TS 23.122, version 13.3.0.

[34]	Void

[35]	Third Generation Partnership Project (3GPP). Network-Based IP Flow Mobility (NBIFOM); Stage 2. 3GPP TS 23.161.

[36]	Third Generation Partnership Project (3GPP). Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP Access Networks; Stage 3. 3GPP TS 24.302.

[37]	Third Generation Partnership Project (3GPP). 3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS), Tunnelling Protocol for Control plane (GTPv2-C); Stage 3. 3GPP TS 29.274.

[38]	Void

[39]	The Internet Engineering Task Force (IETF). RADIUS Attributes for Tunnel Protocol Support, edited by G. Zorn et al., October 2012. IETF RFC 2868, also available at https://tools.ietf.org/html/rfc2868.

[40]	The Internet Engineering Task Force (IETF). RADIUS Filter Rule Attribute, edited by P. Congdon, M. Sanchez, and B. Aboba, April 2007. IETF RFC 4849, also available at https://tools.ietf.org/html/rfc4849.

[41]	MulteFire Alliance (MFA), in collaboration with Third Generation Partnership Project (3GPP). MulteFire Alliance Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 1). MFA TS 36.331.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions listed in 3GPP TR 21.905 [1], MFA TR MF.101 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and MFA TR MF.101 [2].

| Term | Definition |
| --- | --- |
| Subscription certificate | A certificate used by a Service Provider to authenticate subscriber and grant access to a user via a Neutral Host Network |
| Device certificate | A certificate tied to the device and used by the Service Provider to verify device uniquely |
| EPC Routed PDN connection | A PDN connection routed to the PLMN PDN-GW over the S2a interface in the NHN Trusted Access Mode |
| Non-EPC Routed PDN connection | A PDN connection directly routed to an external IP network by the NHN in the Untrusted or Trusted Access Mode |

## 3.2	Symbols

## 3.3	Abbreviations

| Abbreviation | Definition |
| --- | --- |
| AAA | Authentication, Authorization, and Accounting |
| ACA | Accounting Answer |
| ACR | Accounting Request |
| ANID | Access Network ID |
| ASR/ASA | Abort Session Request/Answer |
| AVP | Attribute Value Pair |
| BSSID | Basic Service Set Identifier |
| CA | Certificate Authority |
| CHAP | Challenge-Handshake Authentication Protocol |
| CIoT | Cellular-based Internet of Things |
| CSFB | Circuit-Switched Fall Back |
| DCN | Dedicated Core Network(s) |
| DEA | Diameter-EAP-Answer |
| DER | Diameter-EAP-Request |
| DNS | Domain Name System |
| EAP | Extensible Authentication Protocol |
| EAP-RSP | EAP Response Transfer |
| EAP-TLS | EAP Transport Layer Security |
| EAP-TTLS | EAP Tunnelled Transport Layer Security |
| ePDG | Evolved Packet Data Gateway |
| EPS-AKA | Evolved Packet System Authentication and Key Agreement |
| FQDN | Fully Qualified Domain Name |
| GUTI | Globally Unique Temporary ID |
| IoT | Internet of Things |
| ISR | Idle Mode Signalling Reduction |
| L-AAA | Local Authentication, Authorization, and Accounting |
| LWA | LTE-WLAN Aggregation |
| LWIP | LTE-WLAN Integration with IPsec Tunnel |
| MCM | Multi-Connection Mode |
| MF | MulteFire |
| MNO | Mobile Network Operator |
| MS-CHAP | Microsoft Challenge-Handshake Authentication Protocol |
| MSBs | Most Significant Bits |
| MSK | Master Session Key |
| NBIFOM | Network-Based IP Flow Mobility |
| NH | Neutral Host |
| NH GW | Neutral Host Gateway |
| NH MME | Neutral Host Mobile Management Entity |
| NHAMI | Neutral Host Access Mode Indicator |
| NHN | Neutral Host Network |
| NHN-Name | Neutral Host Network Name |
| NHN-ID | Neutral Host Network Identifier |
| NSWO | Non-Seamless WLAN Offload |
| OCSP | Online Certificate Status Protocol |
| OID | Organizational Identifier |
| OMA | Open Mobile Alliance |
| OSU | Online Signup System |
| P-IMSI | Pseudo-IMSI |
| PCC | Policy and Charging Control |
| PDN-GW | Packet Data Network Gateway |
| PDP | Packet Data Protocol |
| PLMN-ID | Public Land Mobile Network Identifier |
| PSP | Participating Service Provider |
| PSP-ID | Participating Service Provider Identifier |
| RFSP | RAT/Frequency Selection Priority |
| SDP | Service Delivery Platform |
| SGW | Serving Gateway |
| SOAP-XML | Simple Object Access Protocol-Extensible Markup Language |
| TAU | Tracking Area Update |
| TWAG | Trusted WLAN Access Gateway |
| TWAN | Trusted WLAN Access Network |
| TWAP | Trusted WLAN Access Point |
| UE ID | User Equipment Identifier |
| UTF-8 | Unicode Transformation Format, 8-bit |
| WLCP | Wireless LAN Control Plane Protocol |

# 4	Overview

This document describes the network architecture for the deployments where the MulteFire RAN is used in Neutral Host Network (NHN) access mode. This architecture addresses the Standalone Deployment Scenario without External Interworking and Standalone Deployment Scenario with External Interworking, as described in MFA TR MF.101 [2].

# 5	Architecture Model and Concepts

## 5.1	Concepts

An MF cell may support PLMN access mode for specific PLMNs. When the MF cell supports PLMN access mode for a PLMN, it broadcasts the corresponding PLMN-ID. The MF cell is considered to be part of the PLMN whose PLMN-ID it broadcasts. RAN sharing solutions may be used to share MF cells across multiple PLMNs.

PLMN architecture for PLMN access mode is specified in several 3GPP specifications, such as 3GPP TS 23.002 and 3GPP TS 23.401 [3]. Any MF-specific extensions for PLMN architecture are described in MFA TS MF.201 [6].

An MF cell may support NHN access mode for a specific NHN. When the MF cell supports NHN access mode for a specific NHN, it broadcasts the Neutral Host Access Mode Indicator (NHAMI) and the NHN-ID of the accessible NHN. The MF cell is considered to be part of the NHN whose NHN-ID it broadcasts. RAN-sharing solutions may be used to share MF cells across one NHN and one or more PLMNs.

NHN architecture for NHN access mode is based on 3GPP specifications such as 3GPP TS 23.002 and 3GPP TS 23.401 [3]. Issues that are specific for NHN are described in this document.

One UE can attach a MulteFire cell simultaneously using both PLMN and NHN access mode only if it is capable of maintaining multiple active RRC connections (multiple radios for example).

Figure 5.1-1: Relation between MF Cell, PLMNs and NHN

Like PLMNs, each NHN is a self-contained, standalone deployment. NHN may support Neutral Host-compliant UEs associated with a subscription from a remote Participating Service Provider (PSP). The NHN authenticates and authorizes a device to connect using either a PSP AAA or a 3GPP AAA. Once authorized, NHN provides the device with IP connectivity to an External IP Network or access to PLMN's IP services over S2a interface.

## 5.2	Architecture Reference Model

Figure 5.2-1 shows the reference architecture model.

Figure 5.2-1: MulteFire NHN Architecture

## 5.3	Network Elements

All the network elements in EPC are defined in 3GPP specifications.

| Network Element | Description |
| --- | --- |
| UE | A 3GPP UE that supports functions needed to use MulteFire network. |
| NH MME | The NH MME provides similar functionality as a MME in EPC. |
| NH GW | The NH GW provides similar functionality as a combined SGW/PGW for non-EPC routed PDN connections. For EPC Routed PDN connections, NHN-GW functionality is similar to a SGW in interactions with the MF-APs over the S1 interface and is similar to the TWAG in interactions with the PLMN PDN-GWs over the S2a interface. Note: The WLAN user-plane related functions of TWAN are not performed by the NHN. |
| Local AAA Proxy | An AAA proxy that is part of the NHN. It provides the AAA functionalities required for interworking with PSP AAA and 3GPP AAAs. |
| PSP AAA | The AAA server using non-USIM credentials that is associated with the PSP, and may be either internal or external to the NHN. |
| 3GPP AAA | It is the 3GPP AAA server as defined in 3GPP TS 23.402 [5]. |
| PDN-GW | Gateway in the PLMN core network which terminates the SGi interface towards the PDN as specified in 3GPP TS 23.401 [3]. |

## 5.4	Reference Points

All the reference points between 3GPP network elements are defined in 3GPP specifications.

| Reference Point | Description |
| --- | --- |
| AAA | Reference point between the Local AAA proxy in the Neutral Host Core Network and the PSP AAA. The functionality of this reference point is to provide authentication, authorization, and accounting for MF network based on non-USIM credentials external to the NHN. |
| AAA-GW-N | Reference point between the NH GW and the Local AAA Proxy. The functionality of this reference point is to provide accounting for MF network. |
| AAA-MME-N | Reference point between the NH MME and the Local AAA Proxy. The functionality of this reference point is to provide authentication and authorization for the MF network. |
| S1-MME-N | Reference point for the control plane protocol between MulteFire AP and NH MME. The functionality of this reference point is similar to the S1-MME reference point, as defined in 3GPP TS 23.401 [3] between E-UTRAN and the MME. |
| STa-N | Reference point between the Local AAA proxy in the Neutral Host Core Network and PLMN 3GPP AAA. Functionality of this reference point is similar to the STa reference point, as defined in 3GPP TS 23.402 [5] between Trusted Non 3GPP Access and 3GPP AAA Server. The NHN-specific changes are described in Section 5.10.2 of this specification. |
| SWa-N | Reference point between the Local AAA proxy in the Neutral Host Core Network and 3GPP AAA server. Functionality of this reference point is similar to the SWa reference point, as defined in 3GPP TS 23.402 [5] between Untrusted Non 3GPP Access and 3GPP AAA Server. The NHN-specific changes are described in Section 5.10.2 of this specification. |
| Uu-N | Reference point between MulteFire AP and MulteFire UE. |

## 5.5	Neutral Host System Information Broadcast

MF cells supporting NHN access mode broadcast additional information enabling Neutral Host-enabled UEs to discover the accessible NHN identity, support for OSU, a list of PSPs and the PSPs that support S2a.

MF cells supporting NHN access mode broadcast NHAMI as a PLMN-ID within the Cell Access Info that is applicable for the NHN.

## 5.6	Utilization of AAA servers for NHN access mode

NHN interacts with PSP AAA servers through the Local AAA proxy server for the purposes of authentication, authorization and accounting of Neutral Host service for an accessing UE. NHNs do not use the S6a interface for providing Neutral Host service.

- NHN access mode uses EAP authentication instead of EPS-AKA which is used for PLMN access mode. EAP signalling is transported between UE and the NH MME in the NHN using extended NAS protocol.

- EAP signalling is transported between the Local AAA proxy in the NHN and the PSP AAA in the PSP using RADIUS (see IETF RFC 3579 [9]) or Diameter (see IETF RFC 6733 [8]), depending on mutual agreement.

- EAP signalling is transported between the Local AAA proxy in the NHN and the 3GPP AAA in the PLMN using Diameter (see IETF RFC 6733 [8]).

- Service authorization happens between PSP and NHN using RADIUS/Diameter.

- Accounting information exchange happens between NHN and PSP using the same protocol as for EAP signalling (RADIUS/Diameter)

Following a decision to select a PSP, a UE indicates its PSP selection by providing its identity and home PSP domain in a form of NAI (see IETF RFC 7542 [10]). The local AAA Proxy uses the realm portion of the NAI to route AAA transactions for the UE.

## 5.7	Identities

### 5.7.1	NHN Access Mode Specific Identities

The following new identities are introduced:

- Neutral Host Network ID (NHN-ID): An identifier of the NHN accessible via the MF cell which is advertised in broadcast channels.

  - Two categories of NHN-IDs are available for NHNs:

    - Locally administrated

    - Universally administrated

  - Length of the NHN-ID is 40 bits

  - Locally and universally administrated identifiers are distinguished by most significant bit: NHN-ID is locally administrated if the bit is 1.

  - The remaining 39 bits in a locally administrated NHN-ID shall be randomized to minimize collision probability between different NHN-IDs.

  - The MFA administers universally administrated NHN-IDs and guarantee their uniqueness.

  - UE performs cell reselection or cell selection between two MF-Cells with the same NHN-ID. The UE can assume that they belong to the same NHN and thus support the same set of PSPs

  - UE performs MF network selection between MF cells with different NHN-IDs

- Neutral Host Network Name (NHN-Name): A friendly name of the NHN network. This is advertised in broadcast channels but potentially less frequently than NHN-ID.

  - NHN-Name is optional. It may be used for manual network selection.

  - NHN-Name may be broadcasted.

  - The length of the NHN-Name shall not exceed 64 characters

- Participating Service Provider ID (PSP-ID): ID that identifies a service provider which provides subscriptions for the NHN. A PSP-ID is used for automatic service discovery where UE matches the PSP-ID against the credentials or subscription the user has for NHN.

  - For PLMN use case the PSP-ID is the PLMN-ID of the PSP

    - The PSP-ID size is 24 bits

    - UE derives the EAP-AKA’ user identity from the PLMN-ID, as specified in 3GPP TS 23.003 [19].

    - This PSP-specific PLMN-ID is advertised in System Information.

  - For other use cases the PSP-ID is based on a domain name or Organizational Identifier (OID) as defined in HS2.0.

  - The following applies for an OID-based PSP-ID if the OID is 24 bits long (i.e., OUI-24/MA-L) [31]:

    - The subscriber profile for an OID shall include credentials and a realm that can be used to access the NHN.

    - The OID is advertised in System Information.

  - The following applies for an OID-based PSP-ID if the OID is longer than 24 bits (e.g., MA-M or MA-S) [31]:

    - UE having a subscriber profile including a detected OID, shall include credentials and a realm that can be used to access the NHN.

    - PSP-ID based on an OID longer than 24 bits has a short form and a long form.

    - The short form PSP-ID is calculated from the long form PSP ID, formatted as specified in Section 8.4.1.31 of IEEE 802.11-2012, Part 11 [32] by calculating the SHA-256 hash value (see IETF RFC 6234 [27]) of the OID and then extracting and using the 24 most significant bits as the short-form PSP-ID.

    - Only short form PSP-ID is advertised in System Information.

  - The following applies for domain based PSP-ID:

    - The subscriber profile for a domain based PSP-ID shall include credentials and a realm that can be used to access the NHN.

    - A PSP with a valid domain name shall use the domain name as PSP-ID. A valid domain name is issued according to Internet Corporation for Assigned Names and Numbers – ICANN regulations (http://www.icann.org). In other cases, the PSP-ID shall be free form name complying with a domain name format.

    - The domain based PSP-ID has a short form and a long form

    - The short form PSP-ID is calculated from the long form PSP ID by calculating the SHA-256 hash value (see IETF RFC 6234 [27]) of the long form and then extract and use the 24 most significant bits as the short-form PSP-ID.

    - Only short form PSP-IDs may be advertised in System Information.

  - A full list of PSP-IDs (PLMN/OID/long form PSP ID) and respective PSP friendly names are available via PSP Query.

- Neutral Host Access Mode Indicator (NHAMI): A reserved global value that is used in lieu of network specific PLMN-ID values within all MulteFire networks enabling NHN Access Mode

  - Note that all NHNs utilize the same NHAMI value and thus NHAMI cannot be interpreted as an actual identifier for the specific NHN.

- Neutral Host Subscriber Identifier: Neutral Host Service is based on UE identifying itself directly to the selected PSP using EAP signalling. After the EAP exchange, the PSP provides a Neutral Host Subscriber Identifier in NAI format back to the NHN. Neutral Host Subscriber Identifier is used as subscriber identifier in all interactions between the NHN and the PSP.

### 5.7.2	Custom rules for using existing 3GPP identifiers for NHN access mode

The following custom rules are specified for NHN access mode:

- When accessing a given NHN with given PSP credentials, while no valid EMM context is available at the UE for the {NHN-PSP}-pair, the UE shall use a common predefined value as the IMSI in the Attach Request. This IMSI contains the NHAMI and 9 zeros. Usage of this value signifies that UE requests NHN access mode and that a new EMM context is to be created for the UE.

- When Attach request is received from the UE for which the NH MME cannot acquire valid EMM context, the NH MME shall initiate the EAP authentication process. NH MME may initiate EAP authentication process even if there is a EMM context but NH MME considers previous EAP authentication obsolete (timed out).

- During this process the UE provides its credentials associated with the selected PSP, which allows the NH MME to execute EAP authentication through the Local AAA proxy with the PSP AAA of the PSP or the 3GPP AAA of the PLMN.

- Upon successful completion of the EAP authentication, the Local AAA proxy creates a locally unique NHN internal identity (unique within NHN) for the UE session, and delivers it to NH MME.

- The NHN internal identity may follow the same format defined for IMSI specified in ITU-T E.212 (P-IMSI).

- Care shall be taken to ensure that a generated P-IMSI pattern does not collide with any other P-IMSI pattern generated for another current UE session, or any real IMSI.

- When negotiation of connected mode results in the trusted NHN access mode described in Section 5.10, the Local AAA proxy shall set the NHN internal identity to the real IMSI of the mobile subscription. The real IMSI of the mobile subscription shall be derived by the Local AAA proxy from the Permanent User Identity information element of the Mobile-Node-Identifier AVP received from the PSP AAA (in this case from the 3GPP AAA over the STa reference point) in the last DEA command of the authentication procedure containing the EAP Success indication. The Local AAA proxy shall reconstruct the IMSI from the Permanent User Identity information element in the NAI format using rules defined in 3GPP TS 23.003 [19].

- The NHN internal identity (the P-IMSI when the format defined for IMSI is used) is stored at NH MME and is used as a pointer to the UE within the NHN.

- The NHN internal identity is not provided to the UE.

- In all subsequent attach procedures to the same NHN with the same PSP credentials, as long as UE possesses a valid EMM context for the {NHN; PSP}-pair, UE identifies itself with respective GUTI. Note that UE maintains separate EMM contexts, including separate GUTIs, for each accessed {NHN; PSP}-pair.

- If NHAMI is the only PLMN-ID within the Cell Access Info utilized for NHN, then all 3GPP identifiers applicable for the NHN and containing a PLMN-ID (e.g., Global eNB ID, EGCI, TAI, GUMMEI, GUTI) use the NHAMI as the PLMN-ID value. These identifiers shall be considered unique only within the specific NHN.

- If also other PLMN-IDs than NHAMI are included within the Cell Access Info utilized for NHN, then the parameters Global eNB ID and ECGI will be based on the 1st PLMN id broadcasted within that Cell Access -Info. The 1st PLMN id is the primary PLMN for the group and thus shall not be the NHAMI. PLMN-ID in TAI, GUMMEI and GUTI is the selected PLMN-ID, which is the NHAMI in case the UE is attaching using NHN access mode.

- Calculation of paging occasions for NHN access mode UEs uses S-TMSI instead of IMSI.

## 5.8	UE Aspects

### 5.8.1	Subscription and credentials

The UE shall have credentials to authenticate to a PSP and gain NHN access mode service. The PSP may not authorize access if user does not have valid subscription in the PSP leading to service denial.

UE may have credentials to multiple different PSPs.

USIM may be used for both PLMN access mode and NHN access mode authentication, but authorization may be subject to different subscriptions.

User shall be able to erase any information associated with credentials (e.g. authentication credentials, network discovery and selection criteria, temporary identifiers mapping to an EMM context in NHN) from the device (‘factory reset / USIM removal’) due to repair, resell or other second-hand usage.

Non-USIM UE authentication credentials shall be stored and processed securely on the UE.

Note:	The mechanism for secure storage and processing of non-USIM UE authentication credentials is left to the UE implementation.

Non-USIM Subscription certificates must be provisioned securely in the UE using one of the following mechanisms:

- Provisioned during manufacture: Subscription certificates for EAP-TLS may be provisioned in the UE during manufacture by the device vendor. The definition of such mechanisms is out of scope of this specification.

- Out of band mechanisms: Subscription certificates may be provisioned securely using an out of band mechanism, e.g., application on the device, MDM. The definition of such mechanisms is out of scope of this specification.

- Online Signup: Subscription certificates may be provisioned in band using the Online Signup procedure as described in Section 5.14. A UE is required to have a device certificate in order to initiate attach to NHN for Online Signup.

The device certificate is tied to the device and used by the Service Provider to verify the device uniquely. A device certificate is required if UE needs to use Online Signup for provisioning a subscription certificate or if EAP-TTLS is used for UE authentication. The UE shall have exactly one device certificate if the UE supports Online Signup or EAP-TTLS. The device certificate shall be stored and processed securely on the UE and the format of the certificate is described in MFA TS MF.301 [30].

The device certificate can be provisioned in the UE using the following mechanisms:

- Provisioned during manufacturing: The device certificate may be provisioned in the UE during manufacture by the device vendor.

- Out-of-band mechanisms: The device certificate may be provisioned securely using an out-of-band mechanism, e.g., application on the device, MDM.

The definition of different device certificate provisioning processes is out of scope of this specification.

### 5.8.2	Access Barring

In NHN AM, a UE shall behave as a UE that does not belong to any special access classes (access classes 11 to 15) and shall follow the access-bearing check specified in Section 5.3.3.11 of MFA TS 36.331 [41].

## 5.9	Interworking with 3GPP systems

From the perspective of a 3GPP network, the MulteFire network may appear as a Trusted Non-3GPP Access Network or an Untrusted Non-3GPP IP Access network. In order to provide mobility with IP address preservation the solutions specified for Untrusted Non-3GPP IP Access network can be used, as described in 3GPP TS 23.402 [5]. When the NHN is treated as an Untrusted Network, access is made via 3GPP ePDG to 3GPP network while the MulteFire UE is in the MulteFire network.

When the NHN is treated as Trusted Non-3GPP IP Access and the PSP is the PLMN operator, the following applies:

- For mobility with IP address preservation from NHN to 3GPP, the solution specified for Trusted Non-3GPP IP Access network should be used, as described in 3GPP TS 23.402 [5].

- For mobility with IP address preservation from 3GPP to NHN, the following adaptations from the solution specified for Trusted Non-3GPP IP in 3GPP TS 23.402 [5] are applicable:

  - If the UE has no existing PDN connection in MulteFire

    - UE executes Steps 1 through 5 in Figure 16.10.2.1-1 of 3GPP TS 23.402 [5], according to Figure 6.2.2-2 (Attach for trusted access mode), setting Request Type in the PDN Connection Request to Handover

    - Step 6 in Figure 16.10.2.1-1 of 3GPP TS 23.402 [5] is maintained

To move any additional PDN connections to MulteFire, the following procedure is applicable:

  - If the UE has an existing PDN connection in MulteFire

    - UE executes Step 5 in Figure 16.10.2.1-1 of 3GPP TS 23.402 [5], according to Section 6.2.11.2, with setting the Request Type in the PDN Connection Request to Handover

    - Step 6 in Figure 16.10.2.1-1 of 3GPP TS 23.402 [5] is maintained

The UE performs these steps as many times as needed to move additional PDN connections to MulteFire.

Figure 5.9-1 illustrates the architecture interworking with 3GPP systems.

Figure 5.9-1: Architecture Interworking with 3GPP systems

No direct interface is specified between any type of 3GPP RAN and MulteFire Network.

The use of IP Multimedia Services (IMS) from NHN can happen either via EPC routed PDN connection or via ePDG. Direct IMS access from NHN without using EPC is not specified.

## 5.10	Untrusted Access and Trusted Access Modes

### 5.10.1	General

In the Untrusted access mode, the NHN may provide one or more simultaneous Non-EPC Routed PDN connections to the UE. Provisioning of configuration parameters for Untrusted access mode is not specified. For Untrusted access mode the AAA or SWa interface may be used between the PSP AAA/3GPPP AAA and the NHN Local AAA Proxy.

In Trusted access mode, the NHN provides one or more simultaneous EPC-routed PDN connections to the UE that are routed to the PLMN PDN-GW using the S2a interface. A non-EPC routed PDN connection can also be provided in Trusted access mode based on UE request when the NHN receives authorization for this type of service from the 3GPP AAA server. To enable Trusted access mode, a PLMN shall be selected as PSP, and EAP-AKA’ shall be performed using STa. The subscription information of the UE including the list of permitted APNs is received by the NHN during the EAP-AKA’ procedure over the STa interface as defined in 3GPP TS 23.402 [5].

The MF-AP advertises the S2a indicator for PLMN-based PSPs in the System Information. The presence of this indication implies that the NHN has an S2a and STa interface with the relevant PLMN. This information can be used by the UE in NHN selection procedures.

The PLMN determines whether the Trusted or Untrusted access mode is used, as specified in 3GPP TS 23.402 [5] and 3GPP TS 29.273 [18] when a UE selects a PLMN as PSP. The 3GPP AAA server sends an indication to the Local AAA server over SWa/STa about the selected mode in AN-Trusted AVP, as specified in 3GPP TS 29.273 [18] and the Local AAA server shall forward this information to the NH MME. The 3GPP AAA server shall send a trust relationship indicator to the UE during the EAP-AKA’ based access authentication using a AT_TRUST_IND attribute as specified in 3GPP TS 24.302 [36].

### 5.10.2	NHN-specific considerations for using SWa and STa interfaces

When EAP-AKA’ is performed with a PLMN acting as PSP, the specifications for general non-3GPP access networks defined in 3GPP TS 29.273 [18] are followed, with the exceptions specified in this section. Extensions defined for Trusted WLAN interworking shall not be used unless it is explicitly required in this section.

The ANID AVP shall be created as follows:

- “MULTEFIRE” constant character string shall be used as a prefix

- The NHN-ID represented as a 10-digit hexadecimal number using UTF-8 encoding of the digits.

When Trusted Access mode (S2a) for the selected PLMN is supported:

- DER-flagged AVP are sent by the NHN as it would be sent by a TWAN. The NSWO-Capability-Indication flag indicates if non-EPC routed PDN connections are supported by the NHN: if it is set, then it indicates that the NHN supports non-EPC routed PDN connections in Trusted access mode. The other bits of DER-flags are not used and shall be cleared. If the NHN does not send the DER-Flags then it is an indication that the NHN does not support non-EPC routed PDN connection in Trusted access mode (S2a).

- The Access Type value shall be set VIRTUAL or any value according the agreement between the operator of the NHN and PLMN.

Note:	The selection of Access Type value should be considered carefully by the operators as the value sent by the NHN is populated over other 3GPP EPC interfaces.

When the 3GPP AAA server selects Trusted Access mode (S2a):

- DEA-flagged AVP are sent by the 3GPP AAA as it would be sent to a TWAN. Only the NSWO-authorization flag is used: if it is set, then it indicates that the UE is authorized to create non-EPC routed PDN connections in Trusted access mode. Otherwise the NHN shall not allow the creation of non-EPC routed PDN connections for the UE. The other bits of DER-flags are not used and shall be cleared.

## 5.11	Network Discovery and Selection

### 5.11.1	General

Network selection for NHN access mode consists of the following main components:

- UE configuration for NHN access mode

  - Prioritized list of configured PSPs

    - PSP-ID

    - Subscriber profile including credentials associated with configured PSP

- Discovery of NHNs and PSPs

  - UE autonomously scans applicable frequency bands for MF cells

  - UE detects that the MF cell supports NHN access mode

  - UE identifies the NHN identity and NHN-friendly name

  - UE identifies the list of PSPs supported by the NHN

- PSP and NHN selection (automated selection case)

  - UE detects a match between configured PSP-IDs and available PSP-IDs

  - UE selects the PSP among the discovered PSPs to use for NHN access mode, based on the prioritized list of configured PSPs

  - UE connects to a NHN serving the PSP

- PSP and NHN selection (manual selection case)

  - UE identifies available PSP-IDs and NHNs and their friendly names

  - The selection of the PSP-ID and NHN is based on user input and out of scope of the specification.

Network selection for Online Signup (OSU) consist of following main components:

  - UE detects NHNs offering OSU services and collects OSU information from them

  - UE presents NHNs and OSU PSPs to the user for OSU PSP selection.

  - UE connects to the NHN using OSU attach and anonymous TLS served by OSU AAA. UE accesses PSP OSU URL for service offering.

A NHN may be operated by one of the advertised PSPs or by a third party. MulteFire cells operated by the same NHN operator and broadcasting the same NHN-ID shall provide access to same set of PSPs.

UE may have simultaneous registrations with a PLMN in 3GPP RAN and a PSP in NHN, subject to device capabilities. NHN radio connection is logically independent of 3GPP radio connection.

UE may have multiple simultaneous registrations (EMM Contexts) with different {NHN,PSP} pairs. They are independent from a network point of view, and each of them is accessed via dedicated RRC connection. One RRC connection can be used only for one EMM context as the RRC connection is created under EMM context-specific security context. If UE wishes to associate with a new PSP while having existing active RRC connection, a new RRC connection must be created for this. Old RRC connections must be released first unless the UE is able to maintain multiple active RRC connections (multiple radios for example).

### 5.11.2	Initial PSP Discovery

UE discovers PSPs from system information.

The network shall broadcast one or more PSP IDs in system information. This enables UEs to discover the available PSPs.

### 5.11.3	PSP and NHN network selection

In automatic network selection mode, the user preference has highest priority in PSP selection.

In manual network selection mode, the UE shall present a list of available PSPs. Presentation is out of the scope.

More detailed descriptions of the mechanisms applicable to PSP and NHN selection is included in Annex D.

Network selection for Online Signup is described in Section 5.14.

After selecting PSP and NHN, UE indicates NHN access mode service request by including NHAMI into RRC Connection Setup procedure. The MulteFire AP shall then select a NH MME instead of an EPC MME.

UE creates the EAP Identity from subscription data in the subscriber profile, unless USIM-based credentials are used. In such cases, the identity is derived from PLMN, as specified in 3GPP TS 23.003 [19].

If UE has a valid EMM context for the selected {NHN,PSP} pair, UE shall provide the respective GUTI in the NAS ATTACH request as UE identity. If MME cannot bind the used UE identity to an existing EMM context, or cannot resolve the context from an old MME, or just chooses to ignore the GUTI, the MME shall run EAP authentication with the UE and the PSP AAA server. The PSP is discovered from the NAI realm in EAP User Identity.

The UE shall not provide IMSI in the initial UE identity IE, even if UE seeks to use IMSI-based PSP credentials for NHN. If UE does not have valid GUTI for the {NHN,PSP} pair, UE follows custom rules for the user identity specified in Section 5.7.2.

### 5.11.4	NHN reselection

If the UE has been successfully authenticated in an NHN for a PSP, the UE may record the {NHN,PSP} pair and store EMM context for it. UE may use the respective NHN-ID in future cell selection or reselection procedures without explicit PSP discovery and assume recorded PSP is supported. This has the following benefits:

- Efficient network discovery

- Faster service recovery

### 5.11.5	Network selection outcome

If UE is able to authenticate successfully with the selected PSP and UE is authorized for the service, the network selection was successful. If EAP authentication fails, or user authorization fails, this is indicated to UE using respective NAS Reject message. The EAP authentication and user authorization result may also be provided to the UE according to respective EAP mechanism, like using Notification message in EAP-AKA’.

### 5.11.6	Out-of-band service discovery

The UE may use an out-of-band service discovery procedure, as described in Annex B.

### 5.11.7	NHN Service Discovery

The NH MME is configured with one or multiple PSPs that provide access via this NHN. The NH MME can provide Network or PSP specific information to the UE via a new query/response protocol, Service Discovery Protocol (SDP).

The SDP query for PSP information provides list of all PSP long names, the PSP friendly names with one or more human languages and the PSP support indicator for OSU service. The SDP query for OSU information provides PSP specific OSU information including the URI the UE shall use to access OSU server and the NAI UE shall use in the OSU access authentication. UE may request OSU information for one or more PSPs.

Figure 5.11.7-1 shows the protocol stack for SDP query/response between UE and NH MME. SDP signalling is transported over NAS transport via new NAS messages that can be sent/transmitted without the need to establish a secure exchange of NAS messages, i.e., these NAS messages can be received plain (not integrity protected, nor encrypted) if there is no security context.

Figure 5.11.7-1 Protocol stack for SDP query/response between UE and NH-MME

Figure 5.11.7-2 shows the call flow for SDP query/response for service discovery.

Figure 5.11.7-2 SDP Query/Response for MF NHN deployment

- MF AP broadcasts NHN-specific information like NHAMI, NHN-ID, etc.

- UE detects NHN deployment, and determines that additional information about this network is required before the UE accesses this network.

- UE establishes an RRC connection with MF-AP, with the RRC establishment cause set to “MO-signalling”.

- UE establishes an RRC connection with MF-AP. If the query procedure does not involve human participation, the RRC establishment cause is set to “delayTolerantAccess-v1020” and “MO-signalling” otherwise. UE shall also include Device Properties IE with low priority indicator into the Uplink Query NAS Transport message if delay-tolerant access is made.

- UE sends an SDP query over a new message Uplink Query NAS Transport for service discovery. The UE may request specific network information from the NH MME in this step. No state is maintained from the SDP query in NH MME.

- The NH MME returns the queried information to the mobile device in SDP response via a Downlink Response NAS transport message.

- (Optional) The UE may request additional network information via subsequent SDP queries.

- The NH MME or UE may release the RRC and S1 connection after the last SDP Query/Response, or keep the connections established to use for Attach or for Credential Provisioning.

The Uplink/Downlink Query/Response NAS Transport messages can be sent/received between the UE and NH MME even when there is no secure exchange of NAS messages established.

## 5.12	Authentication and Security

### 5.12.1	General

Authentication and key agreement is performed using Extensible Authentication Protocol (EAP) authentication over the Neutral Host MulteFire network (see IETF RFC 5247 [11]).

Each EAP authentication involves executing an EAP method (e.g., EAP-TLS, EAP-TTLS, EAP-AKA’, etc.). The EAP method and the associated credential selection is a deployment decision. The UE and the EAP authentication server use EAP method negotiation to dynamically select a method during network access authentication.

For device and/or user authentication based on certificates (X.509, 802.1ar), UE shall support EAP-TLS (see IETF RFC 5216 [12]).

For user authentication based on username and password, UE shall support EAP-TTLS (see IETF RFC 5281 [13]) with MS-CHAP v2 (see IETF RFC 2759 [14]).

For user authentication with the 3GPP Operator based on USIM, UE shall support EAP-AKA’ (see IETF RFC 5448 [15]). The EAP-AKA’ is used as it is specified for SWa for Untrusted access mode, and STa for Trusted access mode as defined in 3GPP TS 33.402 [17].

For EAP methods that utilize server certificates, the UE should check the revocation status of AAA server’s X.509 certificate at the time of network access authentication. The UE should use, and the PSP AAA shall support OCSP extensions during TLS handshake (see IETF RFC 6066 [16]).

Figure 5.12.1-1 shows the protocol stack for EAP authentication over a Neutral Host MulteFire network.

Figure 5.12.1-1: MulteFire Neutral Host Protocol Stack for EAP authentication

The EAP authenticator function resides in the Neutral Host MulteFire network, and forwards messages to Service Provider AAA

EAP signaling occurs between the UE and the EAP authenticator in the MulteFire network. The NAS layer is responsible for transporting EAP signaling between the UE and the MulteFire core network.

The MulteFire NH MME function and EAP authenticator function can be collocated. The interface between the MulteFire NH MME NAS and EAP authenticator in the MulteFire network is unspecified.

Note:	The Master Session Key (MSK) is exchanged between the EAP authenticator and the NH-MME, and should be done in a secure way.

The EAP layer is responsible for negotiation of the EAP authentication method to be used, mutual authentication and key agreement. EAP authentication delivers a MSK from which KASME is derived.

Once authentication via EAP is successful, and KASME is derived from the MSK:

- The session key derivation from KASME is the same as for E-UTRA, as defined in MFA TS 33.401 [7].

- The new security context is placed at NAS via the Security Mode Command as described in MFA TS 33.401 [7].

- NAS and AS security handling is the same as for E-UTRA, as defined in MFA TS 33.401 [7].

Figure 5.12.1-2 shows the overview of call flow when EAP authentication method is used in NHN.

Figure 5.12.1-2: Overview of EAP-Based Authentication Procedure

- The UE establishes an RRC connection with MF AP.

- The UE sends an Initial NAS message (e.g. Attach/TAU Request) to the MF MME.

- The MF MME decides to initiate EAP authentication, and notifies the EAP authenticator function to initiate EAP authentication.

- EAP authentication NAS transport is used to exchange EAP signaling between UE and EAP authenticator at the MulteFire NHN. Based on the Network Access Identifier (NAI) in EAP payload sent by the UE, the EAP Authenticator performs EAP procedures with the appropriate PSP AAA server through the Local AAA Proxy. The interface between the EAP Authenticator and AAA is based on Diameter (see IETF RFC 4072 [22]). The interface between the Local AAA proxy and the PSP AAA is based on RADIUS or Diameter. The interface between the Local AAA proxy and the 3GPP AAA is based on Diameter and follows specifications for the SWa interface for Untrusted access mode and STa interface for Trusted access mode, as defined in 3GPP TS 29.273 [18]

- Upon successful authentication, the UE and the NH MME derive the KASME from the EAP keying material MSK as defined in Section 5.12.4 of this specification.

- The NH MME sends a Security Mode Command to the UE indicating the NAS security is activated.

- The NH MME continues the NAS procedure. For example, for attach procedure Steps 17 to 24 as listed in Section 5.3.2.1 of 3GPP TS 23.401 [3] are performed.

### 5.12.2	EAP/NAS sublayer interaction

This section defines at both UE and MME primitives and internal messaging between NAS sublayer and EAP sublayer at a very high level.

The messaging in the UE side is as follows:

- NAS to EAP:

  - Initiate EAP authentication:

    - When NAS has initiated attach procedure and receives a DOWNLINK EAP AUTHENTICATION NAS TRANSPORT message it indicates EAP sublayer to start.

    - It also includes initial EAP signalling received in DOWNLINK EAP AUTHENTICATION NAS TRANSPORT payload.

  - Deliver downlink EAP message from MME.

  - Terminate:

    - If NAS aborts attach procedure.

- EAP to NAS:

  - Deliver uplink EAP signalling

  - EAP success

    - Contains MSK

    - NAS will derive KASME from MSK.

  - EAP permanent failure

    - If EAP sublayer reached permanent failure, it indicates to NAS that EAP authentication has failed.

The messaging in the MME side is as follows:

- NAS to EAP:

  - Initiate EAP authentication:

    - When NAS receives and attaches request with a specific IMSI with a reserved MNC and MCC and rest of IMSI as zeros to indicate EAP-AKA’ type registration.

  - Deliver uplink EAP message received from MME.

  - Terminate:

    - If NAS aborts attach procedure.

- EAP to NAS:

  - Deliver downlink EAP signalling

  - EAP permanent failure

    - If EAP sublayer reached permanent failure it indicates to NAS that EAP authentication has failed.

### 5.12.3	EAP over NAS authentication procedures

#### 5.12.3.1	EAP over NAS authentication procedure overview

NAS transport is used to communicate EAP signalling between UE and EAP authenticator at the MulteFire NHN. Based on EAP Identity sent by the UE, the EAP Authenticator performs EAP procedures with the appropriate AAAs. Two new NAS messages are defined to communicate EAP signaling

- Downlink EAP Authentication NAS transport

- Uplink EAP Authentication NAS transport

NAS signaling occurs between the MulteFire NH MME and UE. The interface between EAP layer and NAS is internal in the UE, and is left unspecified at the NHN.

#### 5.12.3.2	EAP-AKA’ procedure

EAP-AKA’ authentication is executed on requests for attach to the 3GPP PLMN using USIM-based credentials.

Figure 5.12.3.2-1 shows the initial attach call flow with EAP-AKA’ authentication procedure.

Figure 5.12.3.2-1 Call Flow for Initial Attach with EAP-AKA’

- The UE provides an indication to initiate registration procedure.

Note:	The initial attach request message is not integrity-protected nor ciphered. If the UE has a security context stored for this NHN, it will send the initial NAS message (Attach request, TAU request, service request) integrity protected and including the GUTI.

- The NH MME decides to initiate EAP authentication.

- EAP-RSP/Identity message contains the UE identity complying with NAI format specified in Section 19.3 of 3GPP TS 23.003 [19]. (e.g., “6<IMSI>@nai.epc.mnc<MNC>.mcc<MCC>.3gppnetwork.org” for EAP-AKA authentication with the 3GPP network through the NHN access.)

- The message is routed towards the proper 3GPP AAA server based on the realm part of the NAI, as specified in 3GPP TS 23.003 [19]. The routing path shall include the Local AAA proxy server as the proxy, and may include one or several AAA proxies. The Access Type and the identity of the access network in which the authenticator resides, shall be included by the authenticator in the Diameter message forwarded to the Local AAA Proxy. The Local AAA proxy shall include the NHN identity in the Access Network Identity information element mapped to the ANID AVP of the SWa DER message sent to the 3GPP AAA.

Steps 5 through 9 follow the same steps as listed in Section 6.2 of 3GPP TS 33.402 [17].

- All requirements of Step 10, as described in Section 6.2, in 3GPP TS 33.402 [17], are followed with the following deviation: The 3GPP AAA Server shall identify the subscriber as a candidate for authentication with EAP-AKA’ due to its access through the NHN. The Diameter Server at 3GPP AAA should resolve the identity of the Diameter Client at the Local AAA proxy to a specific NHN. The Diameter Server at the 3GPP AAA should then verify that the NHN served by the verified Local AAA proxy is authorized to use the Access Network Identity received in Step 4. (This verification is based on established security association for the Diameter session between the Local AAA proxy and the 3GPP AAA). If there is a mismatch, the 3GPP AAA shall reject the session. The 3GPP AAA server includes the Access Network Identity associated with the NHN, based on the value provided in Step 4, in a message sent to the HSS.

Note:	Step 10 implies that the 3GPP AAA server maintains an association between the authenticated identity of the Diameter Client and the authorized Access Network Identities which is the same as the NHN ID.

- Step 12 follows the same step listed in Section 6.2 of 3GPP TS 33.402 [17].

- EAP-REQ/AKA’-Challenge includes the following attributes: AT_RAND, AT_AUTN, AT_MAC, AT_KDF, AT_KDF_INPUT. AT_KDF indicates the key derivation function index and is set to “1”. AT_KDF_INPUT attribute contains the Access Network Identity associated with the NHN and sent to the HSS in Step 10., which will be used for key derivation at the UE.

- The EAP-RQ/AKA’-Challenge is sent to the UE over the NAS transport.

- Step 15. follows the same step listed in Section 6.2 of 3GPP TS 33.402 [17]. Specifically, the UE extracts the Access Network Identity from the message received in Step 14. UE shall compare the broadcasted NHN-ID with the received access network identity. If they do not match, then the UE aborts the procedure.

- The EAP-RSP/AKA’-Challenge is sent to the NH MME over the NAS transport.

Steps 17 through 23 follow the same steps listed in Section 6.2 of 3GPP TS 33.402 [17].

- The EAP-Success is sent to the UE over the NAS transport.

- Both UE and NH MME derive the KASME from the MSK as defined in Section 5.12.4 of this specification.

NH-MME invokes NHN air interface security as defined for E-UTRA; this is specified in 3GPP TS 33.401 [7]

Figure 5.3.12.2-2 shows the key hierarchy scheme in the UE and NHN, when EAP-AKA’ is used for authentication.

Figure 5.12.3.2-2 Key Hierarchy Using EAP-AKA’

#### 5.12.3.3	EAP-TLS

The EAP-TLS client in UE shall support at least one, and the EAP-TLS Server (PSP AAA server) shall as a minimum support cryptographic suites defined in Annex E of 3GPP TS 33.310 [20].

The EAP-TLS executes as described below in this section. If the EAP-TLS terminates with EAP-Failure, the UE and the authenticator in the NH MME shall perform the disconnection procedure. Furthermore, if the UE received network rejection information via EAP Notification, then the UE shall detach from the NHN.

The UE shall parse the server’s X.509 certificate sent to it by the AAA during EAP-TLS. The UE shall verify that the PSP FQDN is a suffix match of the domain name in at least one of the DNSName SubjectAltName extensions.

Note: 	The PSP FQDN is a part of the UE subscription profile.

If a SubjectAltName of type DNSName is not present, then the domain name from the FQDN shall be a suffix match to the CommonName portion of the SubjectName. If neither of these conditions holds, then verification fails.

If the EAP session is completed successfully (i.e. the UE receives EAP Success), the UE shall act depending on the realm match or the mismatch. In case of realm match, i.e. when the PSP FQDN extracted from the AAA certificate matches expected realm of the PSP, (see Section 6.1.1 of Hotspot 2.0 (Release 2) Technical Specification [21]), when receiving EAP Success, the UE shall continue the connection procedure. Otherwise, in case of realm mismatch, the UE shall reject the connection.

Figure 5.12.3.3-1 shows the initial attach call flow with EAP-TLS authentication procedure. Each step is self-explanatory. So, we add some details only if a special attention is needed.

Figure 5.12.3.3.-1 Call flow for initial attach with EAP-TLS

If the UE has a security context stored for this NHN network, it will send the initial NAS message (Attach request, TAU request, service request) integrity protected and including the GUTI.

In Step 2., the NH MME decides to initiate EAP authentication. In such a case, the NH MME sends an EAP-REQ/Identity to the UE.

In Step 3, NAI in EAP-RSP/Identity is set to the NH MME.

Note:	The user identity may be protected by sending, for example, anonymous@<ServiceProviderRealm> (see Section 2.1.4 of IETF RFC 5216 [12]. The use of an anonymous username preserves anonymity of the user. In this case the actual UE ID will be in a Client certificate sent after Step 14 (TLS Finished), under the TLS protection. In other words, the PSP AAA continues the TLS handshake requesting the Client certificate again, under the protection of established TLS.

In Step 9, the PSP AAA may request that the Client at the UE sends its certificate. The use of either Manufacturer-provided Device Certificate or the PSP-provided User Certificate as a Client Certificate is PSP policy-specific. This policy is provisioned into the UE at the time of setting the subscription with the PSP.

This request is sent in the EAP-RQ/EAP-TLS:Certificate Request message, and is forwarded in Step 10 to the UE in the NAS transport.

In Step 11, if the Client Certificate was requested in Step 9, the UE may return the Client Certificate. The choice of either Device or User Certificate is dictated by the subscription policy provisioned by the selected PSP. UE shall include it in the EAP-RSP/EAP-TLS transported to the NH MME over NAS. This response is forwarded to the PSP AAA via the Local AAA proxy in Step 12.

As a result of successful completion of EAP-TLS procedure, the MSK is computed in Step 17 by the AAA and provided to the authenticator at the NHN in Step 18. The same MSK is computed at the UE.

In Step 21 the EAP Success is routed to the UE over the NAS transport.

In Step 22 both UE and NH MME derive the KASME from the MSK as defined in Section 5.12.4 of this specifications.

Figure5.12.3.3.-2 shows the key derivation scheme in EAP-TLS.

Figure 5.12.3.3-2 Key hierarchy with the use of EAP-TLS

#### 5.12.3.4		EAP-TTLS

When EAP-TTLS is used, and the subscription credentials are based on User ID and password, the MS-CHAPv2 shall be used for subscription authentication as the inner method.

Note:	Although support for the MS-CHAPv2 is mandated, its use of it is not mandated, and other inner methods are allowed.

The EAP-TTLS client in UE and the EAP-TTLS Server (PSP AAA server) shall, as a minimum, support cryptographic suites defined in Annex E of 3GPP TS 33.310 [20].

The UE shall parse the server’s X.509 certificate sent to it by the AAA during EAP-TLS. The UE shall verify that the PSP FQDN is a suffix match of the domain name in at least one of the DNSName SubjectAltName extensions.

Note:		The PSP Fully Qualified Domain Name (FQDN) is a part of the UE subscription profile.

If a SubjectAltName of type DNSName is not present, then the domain name from the FQDN shall be a suffix match to the CommonName portion of the SubjectName. If neither of these conditions holds, then verification fails.

If the EAP session is completed successfully (i.e. the UE receives EAP Success), the UE shall act depending on the realm match or the mismatch. In case of a realm match, i.e., when the PSP FQDN extracted from the AAA certificate matches expected realm of the PSP (see Section 6.1.1 of Hotspot 2.0 (Release 2) Technical Specification [21]), when receiving EAP Success, the UE shall continue the connection procedure. Otherwise, in case of realm mismatch, the UE shall reject the connection.

EAP-TTLS follows the call flow for EAP-TLS, as illustrated in Figure 5.12.3.4-1 of IETF RFC 5281 [13] with the following deviations:

- In Step 5, the PSP AAA shall set the Type to indicate EAP-TTLS and the S (Start) bit set, as defined in IETF RFC 5281 [13].

- In Step 9, as in the EAP-TLS (see Section 5.12.3.3) the PSP AAA shall request the Client Certificate.

- In Step 11, the UE shall include its unique device certificate in the EAP-RSP/EAP-TTLS transported to the NH MME over NAS, and forwarded to the PSP AAA via the Local AAA proxy in Step 12. The device certificate may be provisioned via mechanisms described in Section 5.8.2. The PSP AAA shall declare EAP failure if the UE does not provide a device certificate. Otherwise, the PSP shall proceed through and including Step 17.

Validating the device certificate is up to the PSP policy (but it is recommended). If the PSP decides to validate the device certificate then it is expected that the device certificate has a chain to a trust anchor CA certificate which is trusted by the PSP AAA. If the certificate validation fails, then the PSP AAA shall declare EAP failure.

- Before executing Step 18, the PSP AAA shall invoke the Inner authentication method based on MS-CHAPv2 as defined in IETF RFC 2759 [14]. When EAP-TTLS is used with MS-CHAPv2, the subscriber credential shall be the identifier and password used for MS-CHAPv2. Upon successful completion of authentication based on Inner method, the PSP AAA proceeds with Step 18.

Figure 5.12.3.3-2 shows the key hierarchy when EAP-TTLS is used.

### 5.12.4	KASME Derivation from MSK

KASME is defined as 256 MSBs (Most Significant Bits) of MSK. (MSK is 64 bytes long.)

## 5.13	Access Service Authorization

### 5.13.1	Predefined Access Service

The NHN operator and a PSP can pre-agree on specific access service to be provided for the UEs authorized by the PSP. In this case, information on this specific access service is preconfigured into the NHN Local AAA together with the PSP AAA realm. These predefined access services may be similar to those that can be dynamically selected by the PSPs. The definition of these predefined access services is however outside of the scope of MFA specifications.

Upon receiving AAA Authorization Response from a PSP for which a predefined access service is configured at L-AAA, NHN activates the preconfigured access service for the authorized UE.

### 5.13.2 	Dynamically Selectable Access Service

#### 5.13.2.1 Overview

In case where no predefined access service is associated with the PSP, the access service is dynamically defined based on the information received in the PSP’s AAA Authorization Response. Three types of NHN access services can be dynamically defined:

- Open Access Service

- Filtered Access Service

- Tunnelled Access Service

Note:	Authorization instructions received from the PSP do not override any access restrictions that apply to NHN due to local regulation or otherwise.

#### 5.13.2.2 Open Access Service

Open Access Service refers to a service where the PSP does not set any session specific restrictions on UE communication with the external IP network.

NHN selects Open Access Service for the UE when all the following conditions are met:

- No predefined access service is configured at the Local AAA for the PSP

- No Filtering information is included in the received AAA Authorization Response

- No Tunnelling information is included in the received AAA Authorization Response

When Open Access Service is selected:

- NHN allocates IP address for the UE

- NHN provides DNS and DHCP services for the UE

- NHN does not apply session specific restrictions on UEs IP communication with IP hosts in the external IP network

#### 5.13.2.3 Filtered Access Service

Filtered Access Service refers to a service where UE is allowed to communicate only with specific IP hosts or/and IP subnets in the external IP network.

NHN selects Filtered Access Service for the UE when all the following conditions are met:

- No predefined access service is configured at the Local AAA for the PSP

- Filtering information is included in the received AAA Authorization Response

- No Tunnelling information is included in the received AAA Authorization Response

When Filtered Access Service is selected:

- NHN allocates IP address for the UE

- NHN provides DNS and DHCP services for the UE

- NHN restricts UEs IP communication in the external IP network to PSP authorized IP hosts or/and IP subnets. Restriction is achieved by NHN blocking all non-authorized IP packets

Support for Filtered Access Service is optional. If the NHN is not able to fully comply with the received Filtering information, NHN shall reject the UE session.

#### 5.13.2.4 Tunnelled Access Service

Tunnelled Access Service refers to a service where all UEs IP communication is encapsulated and forwarded between the NHN and a PSP indicated IP host.

NHN selects Tunnelled Access Service for the UE when all the following conditions are met:

- No predefined access service is configured at the Local AAA for the PSP

- No Filtering information is included in the received AAA Authorization Response

- Tunnelling information is included in the received AAA Authorization Response

When Tunnelled Access Service is selected:

- Either NHN or PSP allocates IP address for the UE

- NHN encapsulates and forwards all IP data from the UE towards the PSP-indicated IP host

- NHN decapsulates and forwards all IP data received from the PSP-indicated IP host towards the UE

- NHN does not allow UE to directly communicate with any other IP host in the external IP network

Support for Tunnelled Access Service is optional. If NHN is not able to fully comply with the received Tunnelling information, NHN shall reject the UE session.

## 5.14	Online Sign Up

### 5.14.1	General

Subscription for NHN access mode can be offered to the MF UE using the OSU. If an NHN supports OSU this shall be broadcasted by the network. The UE can learn detailed OSU support information using SDP Query for PSP discovery and SDP Query for OSU information, as described in Section 5.10.7.

The UE shall cross check any friendly name of the selected OSU PSP that was received using the SDP Query for PSP discovery and displayed to the user, against the friendly names found in the OSU AAA and OSU server certificates of the PSP, as described in Section 7.3.2 of Hotspot 2.0 (Release 2) Technical Specification [21].

If the UE does not have credentials required to access the NHN, the UE may initiate a limited service state access to the selected NHN to access an OSU provisioning server. The UE shall use OSU specific Attach Type in the Attach Request to indicate its intention to engage with OSU process. UE shall request a PDN connection for a default Access Point Name (APN).

Upon receiving this request the NH MME initiates EAP authentication for OSU service. The UE shall use the
PSP-specific OSU NAI as the user identity and anonymous TLS as the authentication method. OSU NAI selects the PSP AAA server to complete the authentication and to provide authorization data for the OSU service. The authorization process and data is further described in Section 5.13 of this specification.

If authentication and authorization are successful, NH MME establishes a PDN connection for OSU services based on OSU Configuration Data and received authorization data from the PSP AAA, if any. The default EPS bearer of a PDN Connection associated with the OSU shall be restricted for the OSU session and shall not allow any other type of traffic. The NH MME configuration data may include traffic flow template (TFT) to limit access only to OSU service, or this may be pre-configured into the NH GW for the OSU APN. The TFT shall allow access to used OSU server addresses. The authorization data may also include indicators affecting PDN setup (like tunnel information or filters) that need to be provided to the NH GW.

The PDN connection for OSU services shall not be changed to other type of PDN connection and vice versa. The NHN shall block any traffic that is not related to providing OSU service. The NH MME shall reject any additional PDN Connection requests from the UE. The UE shall not request any bearer resource modification for the OSU PDN connection and the NH MME shall reject any such attempt.

The OSU Client in the UE initiates a TLS connection to the discovered OSU server URL of the selected OSU PSP. The OSU Server authentication is performed using the OSU server certificates and authorized trust anchor root CA certificates. Validation of the OSU server certificate follows the procedure defined in Section 7.3.2.2 in Hotspot 2.0 (Release 2) Technical Specification [21]. Once the TLS session is setup between the OSU Client and the OSU server, HTTP-based messages are exchanged for registration of the user, and provisioning of subscription credentials and other policy related information. The provisioning process follows the procedures defined in Section 8 of Hotspot 2.0 (Release 2) Technical Specification [21], with additional clarifications described in [30].

During this exchange, the OSU server may order UE to open a browser to the provided URL to provide information, such as contact information and payment method, as required by the OSU PSP to obtain an account. The UE could provide this information in an automated manner (e.g., without end user involvement) via an optionally-embedded registration schema, or the user could manually enter the information. The OSU PSP may choose to ignore the registration schema. This allows the PSP and UE to exchange any information the OSU PSP requires, but then automatic provisioning is not possible. Provisioned credentials and related metadata are bound to this account.

To allow the OSU-attached UE to get access to normal services after completion of the provisioning procedure, the UE shall explicitly detach and reattach to normal services.

Figure 5.14.1-1: Provisioning Protocol Stack

The Provisioning Protocol Stack (Figure 5.14.1-1) in the UE consists of the following elements:

- Control Plane stack:

  - Responsible for discovery of NHNs, list of service providers and/or PLMN IDs of MNOs offering service through the NHN.

  - Responsible for discovery of online provisioning protocol and additional information (like Online Signup) for each service provider if available.

  - Responsible for setting up a PDN connection for online provisioning.

- Provisioning function:

  - Potentially part of credential management function.

  - Requests the lower layers discovery of possible online provisioning information.

  - Requests the NAS to establish a PDN connection for provisioning.

  - Handles actual provisioning protocol and procedure towards the Provisioning/Online signup server.

A UE may also contact the OSU provisioning server via any IP connectivity. The result of the procedure (e.g. credentials and policies provisioned in the UE) shall be the same as the procedure described in this section; the UE can use the provisioned credentials in the same way. The specification of this type of provisioning is out of the scope this specification.

### 5.14.2	Online Signup Procedure

This section outlines a high-level solution for Online Sign Up.

The UE provides Device certificate when accessing a MulteFire NHN for online provisioning. The device credentials, once successfully authenticated, do not entitle the device to receive normal services, but rather access is still restricted to provisioning only.

The Device certificate may be provisioned using mechanisms mentioned in Section 5.8.2. The Device certificate has a chain to a trust anchor CA certificate which is trusted by the PSP OSU AAA and OSU servers.

When the UE accesses the network for provisioning with Device certificate:

- The UE is authenticated with trust anchor and security keys are put into place using EAP-TLS.

- If authentication is successful, a PDN connection for provisioning is established with security context.

Figure 5.14.2-1 shows the call flow for Online Signup.

Figure 5.14.2-1: Online Signup procedure – Call Flow

- UE discovers a MulteFire access point (MF-AP) and performs service discovery to receive information of Online Credential Provisioning.

- The Provisioning function in the UE initiates the online provisioning by requesting NAS to provide temporary access for credential provisioning. The UE performs Attach procedure indicating that the UE is seeking online sign up using a special Attach Type.

- NH MME initiates EAP to authenticate the device. EAP-TLS follows steps 2 through 21 described in Figure 5.12.3.3-1 with the following deviations:

  - In Step 3, the user identity used is the OSU NAI received via the SDP query procedure for OSU information or via preconfiguration in the UE.

  - In step 9 the PSP OSU AAA shall request for the device certificate and in Step 11 UE shall provide the device certificate. Validating the device certificate is up-to the PSP policy (but it is recommended). UE shall have exactly one device certificate for OSU.

During EAP-TLS the MSK is derived.

NH MME shall ensure that the UE does not use OSU authorization in normal attach. This can be based on authorization data received during authentication or on local configuration.

- If EAP-TLS is successful MSK is provided to the NH MME NAS and UE NAS. KASME is derived from MSK, and from there all security keys are derived, as shown in Figure 5.12.3.3-2.

- The UE and network continue attach procedure, starting with the SMC to create a new security context. This security context is only valid during the provisioning process, i.e., the UE enters a substate of EMM-REGISTERED that does not allow normal service, only accesses a PDN connection restricted to provisioning with a specific (set of) OSU server(s).

- The interaction with the OSU server is handled by the Provisioning function in the UE. The UE initiates the Subscription selection and Subscription certificate provisioning with the OSU server over HTTPS, using OMA DM or SOAP-XML, as defined in Hotspot 2.0 (Release 2) Technical Specification [21]. The OSU server shall request and the UE shall provide the device certificate. Validating the device certificate is up to the PSP policy (but it is recommended).

- Upon successful provisioning of the device, the OSU server updates the AAA server about this new subscription information and the Subscription certificate is provisioned in the UE.

- The Detach procedure is initiated, to remove the UE context for provisioning only. RRC connection is released during detach procedure.

- The UE establishes a new RRC connection and performs attach procedure using the new set of credentials as described in Section 5.12.3.3.

## 5.15	Bearer model and QoS Concepts

The same bearer model is used as in LTE. In order to avoid to EPC impacts the same Quality of Service (QoS) parameters are used. The policy management of QoS is up to network implementation. It is not specified whether the PCRF or some other policy management framework is used in the network.

The bearer-related procedures for Neutral Host Access Mode are defined in Section 6.2.6.

## 5.16	Charging

No internal or external charging architecture and interfaces for NHN are specified. Charging characteristics are routed through the NHN for EPC-routed PDN connections as specified in 3GPP.

## 5.17	Multiple PDN Support

NHN supports IP network access by providing connectivity to a default PDN. Access to additional PDNs is optional. If access to additional PDNs is supported, the procedures for multiple-PDN support (see Section 6.2.11) are used. Configuration data required to access both the default PDN and any additional PDN is left to NHN UE and core network implementation.

## 5.18	Location Reporting

Location reporting is optionally supported by NHN as in LTE (see 3GPP TS 23.401 [3]) with the exception that a NHN will not have access to the “UE-dedicated Presence Reporting Area”, defined in the subscriber profile (HSS). The NHN needs to use a “Core Network predefined Presence Reporting Area”, predefined in MME/SGSN.

Note:	If GTPv2-C is used internally in NHN, the NH GW will get updated UE location information in most UE procedures.

For the untrusted interworking model, the location information provided to ePDG and PGW in the MNO EPC is the same as for 3GPP Un-trusted Non-3GPP IP access (see 3GPP TS 23.402 [5]).

For the trusted interworking model, the location information provided to PGW in the MNO EPC is the same as for 3GPP Trusted WLAN Access (see 3GPP TS 23.402 [5]). The TWAN identifier is formatted as specified in Section 6.2.1.2.

## 5.19	Information Storage

There is no S6a interface for NHN Access Mode, so the HSS data described in Section 5.8.1 of 3GPP TS 23.401 [3] is not available. Information related to features supported in NHN Access Mode and required by the NHN for operation is maintained either at the L-AAA or at the NH MME. PSP-specific configurations (e.g., Access Point Name (APN) configuration information) may also be used according to a bilateral agreement with the NHN and PSP. For trusted access, information provided by the 3GPP AAA server is supported as described in 3GPP TS 23.402 [5] and 3GPP TS 29.273 [18].

## 5.20	MDT

MDT shall only be used with user consent.

Note:	User consent cannot be provided as part of subscription profile from HSS as in PLMN access mode.

## 5.21	Warning Message Delivery

Warning Messages Delivery may be supported. Warning message delivery over S1 and Uu-N interfaces follows 3GPP specifications (see 3GPP TS 23.041 [28]) and the application of the other part of the architecture (e.g., SBc interface) is not specified. As it cannot be guaranteed that interference will not happen in unlicensed band used by an MF cell, no guarantee is possible that a MF-AP is able to deliver a warning message in any timeframe.

Note:	In order to increase the probability of successful warning message delivery, operators could use longer repetition periods and more frequent message delivery attempts in MF-APs than the values used in eNBs.

## 5.22	Features that are not supported or modified

As UTRAN and GERAN cannot be connected to NHNs the following features are not relevant in NHN access mode:

- CSFB and 1xCSFB

- SRVCC

- Idle Mode Signalling Reduction (ISR)

The following features are not support in NHN access mode:

- Dual Connectivity

- Relaying Function when MF-AP acts as a donor eNB

- Multimedia Broadcast/Multicast Service (MBMS)

- RAN-assisted WLAN interworking features (e.g. lwIP, LWA)

- EPC-level WLAN IW features (e.g. IFOM, MAPCON, NBIFOM)

Note:	EPC can support Non-3GPP IW features such as NBIFOM using NHN as non-3GPP Access Network, but this is transparent to NHN (it is not a feature provided by NHN). NBIFOM using NHN as Trusted non-3GPP Access Network requires support for NBIFOM signaling in the NHN, as specified in 3GPP TS 23.161 [35] .

- Proximity-based services

- CIoT Optimization-related features

- Emergency Services

- Multimedia Priority Services

- CSG-related features

- LIPA feature

- Selected IP Traffic Offload (SIPTO)

- Features supporting Machine Type Communications (see Section 4.3.17 of 3GPP TS 23.401 [3])

- Dedicated Core Networks (DCN)

- Isolated E-UTRAN Operation for Public Safety (IOPS)

The MF-AP shall behave as an eNB that does not support these features.

# 6	Functional Description and Procedures

## 6.1	Control and User Plane Protocol Stacks

### 6.1.1	Control Plane Protocol Stack

Figure 6.1.1-1: UE-MME protocol stack

### 6.1.2	User Plane Protocol Stack

Figure 6.1.2-1: User Plane protocol stack for Non-EPC Routed PDNs

Figure 6.1.2-2: User Plane protocol stack for EPC Routed PDNs

## 6.2	Procedures within MulteFire RAN

### 6.2.1	General

#### 6.2.1.1		General considerations for Untrusted access mode

The MF NHN procedures are performed as defined in 3GPP TS 23.401 [3], with the following adaptations:

- The NH GW is a combined SGW/PGW.

- The NH MME is a MME.

- The MF AP is an eNB.

- GUTI, GUMMEI, TAI, ECGI are specified according to the rules in Section 5.7.2.

- NH MME or NH GW relocation is not specified.

- PCRF and the related interfaces are not specified for NHNs.

- All messages related to GERAN and UTRAN interworking (e.g., ISR) are not supported.

#### 6.2.1.2		General considerations for Trusted access mode

For Trusted access mode, the procedures between the NHN and PLMN network follow those defined in 3GPP TS 23.402 [5].

The following adaptations apply:

- For EPC-Routed PDN connections the NH GW functionality is similar to a SGW in interactions with the MF-APs over the S1 interface and is similar to the TWAG in interactions with the PLMN PDN-GWs over the S2a interface.

- The Local AAA proxy performs the functionality required to terminate STa interface.

The access-specific procedures outlined in 3GPP TS 23.402 [5] are the MF NHN internal procedures that follow the procedures outlined in 3GPP TS 23.401 [3], with the following adaptations:

- The NH MME is an MME.

- The MF AP is an eNB.

- GUTI, GUMMEI, TAI, ECGI are specified according to the rules in Section 5.7.2.

- NH MME or NH GW relocation is not specified.

- PCRF and the related interfaces are not specified for NHNs.

- All messages related GERAN and UTRAN interworking (e.g. ISR) are not supported.

The following considerations are applicable for GTP-based S2a to avoid any MulteFire-specific changes in the interface:

- All parameters shall be used in the way as a TWAN supporting MCM would use them according to 3GPP TS 29.274 [37]; e.g. Trusted WLAN Mode Indication shall be used to indicate that MCM is supported, APCO shall not be used. This rule also applies to the RAT Type value that shall be set to WLAN. Parameters defined for TWAN (e.g. TWAN-FQ-CSID, S2a-U TWAN F-TEID) shall be used to send NH GW-related GTP parameters.

- WLCP PDN Connection Modification Support Indication in Indication Flags shall be set to indicate that NHN supports PDN connection modification procedures.

- TWAN Identifier shall be sent by the NH GW and shall be created in the following way:

  - SSID can contain any value. An NHN may send a constant string such as “MulteFire” or the first 32 characters of the NHN-Name.

  - First byte of BSSID shall be set to 3 (locally administered and multicast MAC address) since this will not make sense as a BSSID for WLAN AP. The last 5 bytes of BSSID shall be set to NHN-ID.

  - For other fields, the same requirements apply as in case of a WLAN. The NHN operator may send its name in the TWAN operator name field.

To establish a non-EPC routed PDN connection, UEs in Trusted access mode shall send an APN value (see Section 10.5.6.1 of 3GPP TS 24.008 [29]) including the APN operator identifier created from the NHAMI (see Section 9.1.2 of 3GPP TS 23.003 [19]). The NH MME shall always interpret a request with an APN value, including the APN operator identifier created from the NHAMI as a request to establish a non-EPC routed PDN connection. The UE may request a non-EPC routed PDN connection either as a part of initial attach (see Section 6.2.2), or during the establishment of new PDN connectivity (see Section 6.2.11).

Note:	The use of an APN value, including the APN operator identifier created from the NHAMI, is also allowed in Untrusted access mode to establish a Non-EPC routed PDN connection. 3GPP PLMNs creates APN operator identifiers from their PLMN IDs. Thus, they never use an APN including the APN operator identifier created from the NHAMI.

### 6.2.2	Initial Attach

#### 6.2.2.1	General

The NHN Initial Attach procedure is an adaptation of the 3GPP E-UTRAN Initial Attach procedure for providing access to the MulteFire NHN by an MF UE. The adaptations for the NHN operation are:

- There is no S6a interface. All subscription information comes from the Local AAA, either directly or in its role as proxy to a Remote AAA or 3GPP AAA.

- The NHN Access Mode utilizes EAP authentication framework for user authentication and keying material derivation. The authentication and security functions are defined in Section 5.12 on authentication and security.

- The NH GW is the accounting client and accounting records are sent to the Local AAA proxy. The Local AAA proxy delivers the external accounting information to the external AAA servers in a form supported by the external AAA interface.

The attach procedures for the Untrusted access and Trusted access modes are described in Sections 6.2.2.2 and 6.2.2.3, respectively. Common aspects of the attach procedure are described in this Section.

The Initial Attach procedure for NHN Access Mode is a modified version of the E-UTRAN Initial Attach procedure (see Section 5.3.2.1 of 3GPP TS 23.401 [3]). Two types of initial attach are supported:

- Attach with NHAMI IMSI, the attach procedure executed by the UE when it has no valid EMM context for the selected PSP in the NHN.

- Attach with GUTI, used for attach procedures executed by the UE when it has a valid EMM context for the selected PSP in the NHN.

The following table identifies optional messages and their applicability for NHN-based Attach procedures:

| Optional Message | Trigger Condition | NHN Applicability |
| --- | --- | --- |
| Identity Req/Resp | UE is unknown in both old and new MME. | Not applicable. UE identity is provided only during EAP procedure. If the MME cannot acquire a valid MM context for the UE, MME shall run new EAP authentication with the UE. |
| Authentication | Attach with NHAMI IMSI or integrity check of Attach message during Attach with GUTI fails or MME considers existing security keys for the MM context to be obsolete. | EAP is triggered; NAS EAP Authentication performed. |
| Update/Cancel Location | Updates UE location in HSS and retrieves subscription information. | Not applicable. No HSS in NHN, so these procedures are not supported. |
| Notify Req/Resp | Informs HSS of PDN GW for mobility with non-3GPP networks. | Not applicable. No HSS in NHN so this procedure is not supported. |

The use of existing 3GPP identifiers in the Attach procedure for the NHN access mode follows the rules listed in Section 5.7.2.

#### 6.2.2.2	Initial Attach for NHN Untrusted access mode

Figure 6.2.2-1 shows the Attach Procedure for Untrusted Access Mode.

Figure 6.2.2-1: Attach Procedure for Untrusted access mode

Note 1:	The NHN architecture described in Section 5.2 does not specify interface behavior among the NHN network elements. The behavior for Steps 3-4 and 7-16 are not specified in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3].

Note 2:	User data is initiated as described in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3].

- Step 1 is the same as Steps 1-2 shown in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3].

- If the trigger conditions for initiating authentication and security occur, as described in Step 5a in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3], then the NH MME initiates authentication and security functions as defined in Section 5.12.

- The Ciphered Options Request/Response transaction may be executed as described in TS 23.401 Figure 5.3.2.1-1 Step 6, when the APN needs to be retrieved from the UE.

- The NHN establishes the session using the parameters provided in Step 1. How the session is created is implementation-specific.

- Step 5 is the same as Steps 17-22 in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3].

- Optional additional session modification within NHN. The details of this step are implementation-specific.

The parameters included in the messages described in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3] may not be included in the NHN Attach Procedure since the associated feature(s) may not be supported. See Section 5.22 for the features not supported in the Neutral Host Access Mode.

Note:	An MF UE may also create a session with a PDN GW in an external PLMN. The method used by the UE is described in Section 7.2.4 of 3GPP TS 23.402 [5]. This will have requirements on the UE (e.g., support for IMSI known by the LTE network).

#### 6.2.2.3	Initial Attach for NHN Trusted access mode

Figure 6.2.2-2 shows the Attach Procedure for Trusted access mode.

Figure 6.2.2-2: Attach procedure for trusted access mode

- Step 1 is the same as Steps 1-2 described in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3]. If S2a support is indicated for the selected PSP, and the UE intends to establish a non-EPC routed PDN connection as a part of the attach procedure, then the UE shall indicate the intention in the attach request, as specified in Section 6.2.1.2.

- If the trigger conditions for initiating authentication and security occur, as described in Step 5a in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3], then the NH MME initiates authentication and security functions as defined in Section 5.12. Negotiation of connection mode is described in Section 5.10.1.

- Step 3 is the same as Step 6 shown in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3].

- If the UE requests the creation of an EPC-routed PDN connection in Step 1, then Step 4 is the same as Steps 10-14 shown in Figure 16.2.1-1 of 3GPP TS 23.402 [5]. This Step uses information received by the NH MME in Steps 1-3. The mechanism of transfer of information from NH MME to NH GW is out of scope of the specification.
If the UE requests the creation of a non-EPC routed PDN connection in Step 1, then Step 4 described in Figure 6.2.2-1 is performed.

- Step 5 is the same as Steps 17-24 described in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3]. This uses information received by the NH GW from the PLMN PDN GW in the previous step. The transfer of information from the NH GW to NH MME is out of scope of the specification.

- Optional additional session modification within NHN. The details of this step are implementation specific.

The parameters included in the messages described in Figure 5.3.2.1-1 of 3GPP TS 23.401 [3] and Figure 16.2.1-1 of 3GPP TS 23.402 [5] may not be included in the NHN Attach Procedure since the associated feature(s) may not be supported. Consult Section 5.22 for the features not supported in the Neutral Host Access Mode.

### 6.2.3	Tracking Area Update

The NHN Tracking Area Update (TAU) procedure is an adaptation of the 3GPP E-UTRAN TAU procedure for the MulteFire NHN. The adaptations for the NHN operation are:

- There is no S6a interface. No subscribed RFSP index is provided.

- The NHN Access Mode utilizes EAP authentication framework for user authentication and keying material derivation. The authentication and security functions are defined in Section 5.12 on authentication and security.

- The P-IMSI is used as a substitution for IMSI internally in the core network in the Untrusted access mode.

The following table lists procedures which are adapted or not used for NHN-based TAU.

| TAU Procedure | Trigger Condition | NHN Applicability |
| --- | --- | --- |
| Authentication | Integrity check of TAU message during fails | EAP is triggered; NAS EAP Authentication performed |
| Update/Cancel Location | Updates UE location in HSS if MME relocation occurs | Not applicable. No HSS in NHN so these procedures are not supported |

Figure 6.2.3-1 shows the Tracking Area Update Procedure for the Neutral Host Access Mode.

Figure 6.2.3-1: Tracking Area Update Procedure

Note:	The NHN architecture described in Section 5.2 does not support MME relocation. As such, Steps 4-5 and
7-19 described in Figure 5.3.3.2-1 3GPP TS 23.401 [3], do not apply.

- Step 1 is the same as Steps 1-3 described in Figure 5.3.3.2-1 of 3GPP TS 23.401 [3].

- If the trigger conditions described Step 6 shown in Figure 5.3.3.2-1 of 3GPP TS 23.401 [3] for initiating authentication and security occur, then the NH MME initiates authentication and security functions as defined in Section 5.12.

- Optional session modification within NHN. The details of this step are implementation specific.

- Step 4 is the same as Steps 20-21 described in Figure 5.3.3.2-1 of 3GPP TS 23.401 [3].

The parameters included in the messages described in Figure 5.3.3.2-1 o of 3GPP TS 23.401 [3] may not be included in the NHN Tracking Area Update Procedure, since the associated feature(s) may not be supported. See Section 5.22 for the features not supported in the Neutral Host Access Mode.

### 6.2.4	S1 Setup

S1 Setup is done according to MFA TS 36.413 [24], with the following deviations from the 3GPP procedures:

- MF-AP sends NHAMI in the parameter “Broadcasted PLMNs” in S1 setup request. If the MF-AP also uses PLMN access mode, the MF-AP shall also include the native PLMN identities.

- The NH MME sends NHAMI in the parameter “Served PLMNs”. If the NH MME is used as a MME for PLMN access mode, the NH MME will also include the served native PLMN identities.

Note:	If the MF-AP uses both PLMN access mode and NHN access mode and MME and NH MME are separated, the MME will send native PLMN identities as “served PLMNs”.

- NH-MME must include PSP information in the S1 setup response. The required PSP information is the list of PSP-identities as they are announced in System Information (i.e., short-form PSP-ID for PSP identities based on an OID longer than 24 bits and domain names). The NH MME must include the same set of PSP identities to all MF-APs in the NHN. If the MF-AP supports to send PSP identities with different frequencies, the MF-AP should use the order of the list to determine what PSP identities to send most frequently. The list is in declining order i.e., the PSP identity that should be sent most frequently is the first entry. The MF-AP may override the order provided by NH MME (e.g., based on local configuration in MF-AP).

### 6.2.5	MME Configuration Update

S1 Configuration Update is done according to MFA TS 36.413 [24], with the following deviations from the 3GPP procedures:

- The NH MME can provide updated PSP information in the MME configuration update message.

When NH-MME is configured to remove a PSP, the NH-MME shall perform a MME-initiated detach procedure for all UEs that are using subscriptions belonging to the PSP. NH-MME shall also send an MME configuration update to all the MF-APs with the updated PSP information.

### 6.2.6	Bearer-related procedures

#### 6.2.6.1	Bearer-related procedures for Non-EPC Routed PDN connections

The bearer-related procedures are performed according to of 3GPP TS 23.401 [3], with these additional adaptations.

In particular:

1.	Dedicated-bearer activation follows the procedure defined in Section 5.4.1 of 3GPP TS 23.401 [3], where:

-	Dynamic PCC/PCRF-related Steps 1 and 12 are not specified for the NHN.

-	Steps 2 and 11 are internal to the NH GW.

-	Steps 3 and 10 are not specified, as they are internal to the NHN.

2.	NH GW-initiated bearer modification, with a bearer QoS update procedure, follows the procedure defined in Section 5.4.2.1 of 3GPP TS 23.401 [3], where:

-	Dynamic PCC/PCRF-related Steps 1 and 12 are not specified for the NHN.

-	Steps 2 and 11 are skipped, as they are internal to the NH GW.

-	Steps 3 and 10 are not specified, as they are internal to the NHN.

3.	NH GW-initiated bearer modification, without a bearer QoS update, follows the procedure defined in Section 5.4.3 of 3GPP TS 23.401 [3], where:

-	Dynamic PCC/PCRF-related Steps 1 and 10 are not specified for the NHN.

-	Steps 2 and 9 are internal to the NH GW.

-	Steps 3 and 8 are not specified, as they are internal to the NHN.

4.	NH GW-initiated bearer deactivation follows the procedure defined in Section 5.4.4.1, where:

-	Dynamic PCC/PCRF-related Steps 1 and 10 are not specified for NHN.

-	Steps 2 and 9 are internal to the NH GW.

-	Steps 3a and 8a are not specified, as they are internal to the NHN.

5.	NH MME -initiated dedicated bearer deactivation follows the procedure defined in Section 5.4.4.2 of 3GPP TS 23.401 [3], where:

-	Dynamic PCC/PCRF-related Step 4 is not specified for the NHN.

-	Steps 3, 5 and 9 are internal to the NH GW.

-	Steps 2, 6 and 8 are not specified, as they are internal to the NHN.

6.	UE-requested bearer resource modification follows the procedure defined in Section 5.4.5 of 3GPP TS 23.401 [3], where:

-	Dynamic PCC/PCRF-related Steps 4 and 6 are not specified for the NHN.

-	Step 3 is internal to the NH GW.

-	Step 2 is not specified, as it is internal to the NHN.

Note:	The HSS-initiated subscribed QoS modification procedure, as defined in 3GPP TS 23.401 [3], is not applicable to the NHN as there is no S6a interface and the PSP/3GPP AAA server does not provide subscribed QoS information.

#### 6.2.6.2	Bearer-related procedures for EPC-Routed PDN connections

For the procedures in this Section the following apply:

-	In references to 3GPP TS 23.402 [5] NHN functions as the TWAN. Specifically, the NH-GW functions as a TWAG in interactions with the PLMN PDN GW, and the Local AAA proxy functions as a TWAP in interactions with the PLMN 3GPPP AAA protocols.

-	In references to of 3GPP TS 23.401 [3], the NH GW functions as the SGW, and the NH-MME functions as the MME.

-	Session creation, modification and deactivation messages between the NH-MME and the NH-GW are implementation-specific.

1.	Dedicated bearer activation follows the procedure defined in Figure 16.5-1 of 3GPP TS 23.402 [5], where:

-	Step 3 follows Steps 	3-10 as described in Figure 5.4.1-1 of 3GPP TS 23.401 [3].

2.	The PDN GW-initiated bearer modification procedure follows the procedure defined in Figure 16.6-1 of 3GPP TS 23.402 [5], where:

-	Step 3 follows Steps 	3-10 as described in Figure 5.4.2.1-1 of 3GPP TS 23.401 [3].

3.	The HSS-initiated bearer modification procedure follows the procedure defined in Figure 16.6.2-1 of 3GPP TS 23.402 [5], where:

-	Step 5 follows Steps 3-10 as described in Figure 5.4.2.1-1 of 3GPP TS 23.401 [3].

4. 	The PDN GW-initiated bearer deactivation follows the procedure defined in Figure 16.4.1-1 of 3GPP TS 23.402 [5], with the following deviations:

-	Steps 3-5 are replaced by Steps 3-8 as described in Figure 5.4.4.1-1 of 3GPP TS 23.401 [3].

### 6.2.7	X2-based handover

X2-based handover is an intra-MF handover between MF-APs of the same NHN and is performed according to Section 5.5.1.1.2 of 3GPP TS 23.401 [3], but with the following adaptation:

- The Handover Type value is “intraLTE”.

- Optional session modification messages between the NH-MME and the NH-GW are implementation-specific.

There are no changes to the procedure based on the steps specified in 3GPP TS 23.401 [3].

If Tracking Area Update (TAU) is performed as part of the handover, it is done according to Section 6.2.3. UE context information transferred between MF-APs is the same as for LTE.

### 6.2.8	S1-based handover

The S1-based handover is an intra-MF handover and is performed according to Section 5.5.1.2 of 3GPP TS 23.401 [3], with the following additional adaptations:

- The S1 handover is supported within a NHN and not specified between NHNs.

- The Handover Type value is “intraLTE”.

- Optional session modification messages between the NH-MME and the NH-GW are implementation-specific.

There are no changes to the procedure steps specified in 3GPP TS 23.401 [3].

If TAU is done as part of the handover, it is done according to Section 6.2.3.

The Source-to-Target Transparent Container, the Target-to-Source Transparent Container, and the eNB Status Transfer Transparent Container are used as specified in MFA TS 36.413 [24]. These containers are transparent to the NH core.

### 6.2.9	Service Request

The MF NHN supports both a UE-triggered Service Request and a Network-triggered Service Request. The Service Request is performed according to Section 5.3.4 of 3GPP TS 23.401 [3], with the following additional adaptations:

- The NHN Access Mode utilizes an EAP authentication framework for user authentication and keying material derivation. The authentication and security functions are defined in Section 5.12.

- The “Stop Paging” signal from the NH GW to the NH MME is not used, as there is no ISR support.

- Paging only uses S-TMSI.

The following table identifies messages which are modified or adapted for Service Request.

| Optional Message | Trigger Condition | NHN Applicability |
| --- | --- | --- |
| Authentication | The MME considers existing security keys for the MME context to be obsolete or not existent | EAP is triggered; NAS EAP authentication performed |
| Stop Paging | Used if ISR is supported and paging response is received during a Network-triggered Service Request | Not used since ISR is not supported |

There are no changes to the procedure steps specified in 3GPP TS 23.401 [3], except that session-related messages between the NH-MME and the NH-GW are implementation-specific.

### 6.2.10	Detach-related procedures

#### 6.2.10.1	Detach-related procedures for Untrusted access mode

This section covers the following procedures:

- UE-initiated detach procedure specified in Section 5.3.8.2.1 of 3GPP TS 23.401 [3]

- MME-initiated detach procedure specified in Section 5.3.8.3 of 3GPP TS 23.401 [3]

- HSS-initiated detach procedure specified in Section 5.3.8.4 of 3GPP TS 23.401 [3]

The detach procedures are performed as described in Section 5.3.8 of 3GPP TS 23.401 [3], with the following additional adaptations:

- There is no S6a interface, so the HSS-initiated detach procedure is not supported.

- The Local AAA proxy may initiate a Detach for any reason (e.g., the L-AAA proxy cannot support prescribed local filtering) by instructing the NH MME to terminate the user’s session. If Diameter is used on the 
AAA-MME-N interface, the Diameter Abort Session Request/Answer (ASR/ASA) commands can be used for this purpose. To indicate the reason for Detach, the Local AAA proxy may include the Termination-Cause AVP in the Abort Session Request (ASR) command to the NH MME.

- The NH MME initiates the detach procedure when the NH MME is instructed by the Local AAA to terminate the user’s session. If the Local AAA proxy indicates the cause for session termination, the NH MME may use the appropriate EMM Error Code to indicate the detach reason to the UE.

- The session deactivation messages between the NH-MME and NH-GW are implementation-specific.

There are no changes to the procedure steps specified in 3GPP TS 23.401 [3] for the UE-initiated Detach or the
MME-initiated Detach procedures, except that the session deactivation messages between the NH-MME and NH-GW are implementation-specific.

#### 6.2.10.2	Detach-related procedures for Trusted access mode

1.	The UE-initiated detach procedure follows the steps as described in Figure 16.7.1.1-1 of 3GPP TS 23.402 [5], with the following modifications:

- Step 1 is replaced by Step 1 as described in Figure 5.3.8.2-1 of 3GPP TS 23.401 [3].

- Step 6 is replaced by Steps 11-12 as described in Figure 5.3.8.2-1 3GPP TS 23.401 [3].

- The MME-initiated detach procedure follows Steps 2-5 as described in Figure 16.7.1.1-1.

- The NHN may perform S1 release procedure after execution of Step 5, as defined in 3GPP TS 23.401 [3].

- The HSS/AAA-initiated detach procedures follows the steps as described in Figure 16.7.1.2-1 of 3GPP TS 23.402 [5].

Note:	The NHN may perform the S1 release procedure after execution of Step 3, as defined in 3GPP TS 23.401 [3] after execution of Step 3.

The session deactivation messages between the NH-MME and NH-GW are implementation-specific.

### 6.2.11	Multiple PDN Support

#### 6.2.11.1	Multiple PDN Support for Non-EPC Routed PDNs

The MF NHN supports both the UE-requested PDN connectivity, and the UE or NH MME-requested PDN disconnection procedures. These procedures are done according to Section 5.10.2 and 5.10.3 of 3GPP TS 23.401 [3], with the following additional adaptations:

- The Notify message is not supported in the UE-requested PDN connectivity procedure.

- Accounting information provided to the Local AAA follows the rules defined in Section 6.3.2.3.2.2.

- The procedures described in this section shall not be used to terminate the last PDN connection of a UE, as "Attach without PDN Connectivity" is not supported. The UE shall use the UE-initiated Detach procedure and the NH MME shall use the NH MME-initiated detach procedure to release the last PDN connection.

In Trusted access mode the UE shall indicate if it requests a Non-EPC routed PDN connection, as specified in Section 6.2.1.2.

In Untrusted access mode the UE may indicate that it requests a Non-EPC routed PDN connection, as Specified in Section 6.2.1.2.

#### 6.2.11.2	Multiple PDN Support for EPC Routed PDNs

1.	UE requested PDN connectivity follows the steps described in Figure 16.8.1-1 of 3GPP TS 23.402 [5], with the following deviations:

-	Step 1 is replaced by Step 1 described in Figure 5.10.2-1 in 3GPP TS 23.401 [3].

-	Step 7 is replaced by Steps 7-12 described in Figure 5.10.2-1 in 3GPP TS 23.401 [3]. The session deactivation messages between the NH-MME and NH-GW are implementation-specific.

2.	The UE/NHN-initiated PDN disconnection follows the steps described in Figure 16.9.1-1 of 3GPP TS 23.402 [5], with the following deviations:

-	Step 1 is replaced by Step 1 described in Figure 5.10.3-1 from 3GPP TS 23.401 [3].

-	Steps 6-9 are replaced by Steps 7-10 described in Figure 5.10.3-1 from 3GPP TS 23.401 [3]. The session deactivation messages between the NH-MME and NH-GW are implementation-specific.

## 6.3	Procedures within NHN Core

### 6.3.1	General

Protocols and procedures within NHN are not standardized. The following subsections describe requirements for the NHN Core network elements.

### 6.3.2	Requirements for NHN Core Network Elements

#### 6.3.2.1	NH-MME

In addition to performing MME functions defined in 3GPP TS 23.401 [3], the NH MME shall act as the EAP authenticator and a Diameter Client towards the Local AAA Proxy.

#### 6.3.2.2	NH-GW

For non-EPC-routed PDN connections the NH GW acts a combined SGW/PDN-GW. For EPC-routed PDN connections the NH GW shall act as a SGW for interfacing with the MF APs (as defined in 3GPP TS 23.401 [3]), and as a TWAG for interfacing with the PLMN PDN-GW (as defined in 3GPP TS 23.402 [5]).

#### 6.3.2.3	Local AAA Proxy

##### 6.3.2.3.1	General

If the PSP AAA supports Diameter, the Local AAA proxy shall act as a Diameter Client towards the PSP AAA.

If PSP AAA supports RADIUS, the Local AAA proxy shall perform the Diameter-to-RADIUS protocol translation and act as a RADIUS Client towards the PSP AAA.

- The Local AAA proxy shall act as a Diameter client towards the 3GPP AAA and support specifications for the SWa reference point in Untrusted access mode, and STa reference point in Trusted access mode as defined in 3GPP TS 23.402 [5], with the NHN-specific considerations described in Section 5.10.2:

  - The Local AAA proxy shall act as a Diameter server towards the NH MME.

  - The Local AAA proxy shall act as a Diameter server towards the NH GW.

##### 6.3.2.3.2	Support for Authentication and Authorization

The Local AAA proxy shall act as a Diameter proxy to transport the EAP payload between PSP AAA or 3GPP AAA and the NH MME.

The Local AAA proxy shall process the NAI presented as a User Identity Information Element of the User-Name AVP of the Diameter EAP request received from the Diameter client at the NH MME. The Local AAA proxy shall use the realm part of the NAI to initiate a request towards a specific PSP AAA or 3GPP AAA.

Upon recognizing the EAP Success payload sent by the EAP Server at the PSP AAA or the 3GPP AAA, the Local AAA proxy shall create the Pseudo IMSI (P-IMSI) for the current UE session and deliver it to the NH MME in the MulteFire-specific AVP outside the EAP payload. The Local AAA proxy shall associate the P-IMSI with the Chargeable-User-ID received from the PSP AAA or 3GPP AAA, and store this association for the duration of the UE session.

##### 6.3.2.3.3	Support for Accounting

###### 6.3.2.3.3.1	General

The NHN may support accounting. In NHN access mode, accounting information is provided by the NHN to the PSP. If NHN supports accounting, it shall at least support the session-based accounting.

The accounting information is generated in the NH GW and provided to the Local AAA Proxy. The accounting information can be provided by the Local AAA proxy to the accounting AAA server operated by the PSP either via basic Diameter accounting or via RADIUS accounting interfaces.

When a PSP is the 3GPP MNO, this section defines providing the accounting information by the NHN to the accounting AAA server associated with the 3GPP MNO, which may or may not be integrated with 3GPP AAA. Because neither SWa nor STa interfaces defined between the Local AAA proxy and the 3GPP AAA support accounting, the accounting interface between the Local AAA proxy and the accounting AAA associated with the 3GPP is expected to follow requirements for the interface between the Local AAA proxy and the accounting AAA server operated by the PSP, as defined in this section. The use of 3GPP RF-based accounting interface is not defined in this specification.

###### 6.3.2.3.3.2	Diameter-based Accounting

For Diameter-based accounting an accounting interface between the Local AAA proxy acting as accounting client, and the accounting AAA server, shall follow the requirements defined in IETF RFC 7155 [25].

Accounting information, including indications of accounting start, accounting stop, and interim reporting of accounting data as requested by the accounting AAA server or initiated by internal NH GW triggers, are needed to support the session-based accounting. Required information is communicated by the Local AAA proxy to the accounting AAA server using Diameter Accounting Request (ACR) command and is acknowledged by the accounting AAA server using the Diameter Accounting Answer (ACA) command.

To correlate accounting application with authentication and authorization session the Local AAA proxy shall use the correlation rules defined in Section 9.6 of IETF RFC 6733 [8]. Specifically, the Local AAA proxy shall use the same Diameter Session ID (Session-ID) attribute for accounting application as was used in authentication and authorization application for the same session.

The Local AAA proxy shall map the accounting information identity used within the NHN (e.g., P-IMSI) to the Diameter User Name (User-Name) attribute, provided by the PSP/3GPP AAA in the Diameter EAP authentication response (DEA), for the same session used in the accounting application with the accounting AAA server. The usage guidelines of the User-Name AVP between the Local AAA proxy and the accounting AAA server are defined in Section 2.8.1 of IETF RFC 4072 [22].

Figure 6.3.2.3.3.2-1 shows flow of messages carrying accounting information.

Fig.6.3.2.3.3.2-1: NHN Diameter-based Accounting

- The UE attaches to the NHN, and a session is created. The procedure follows Figure 6.2.2-1 in Section 6.2.2 of this document.

- Once the PDP is established and bearer is activated, the NH GW sends an indication to the Local AAA proxy of the Start of Accounting session. This indication carries the NHN internal identity (e.g., P-IMSI) associated with the session within the NHN.

Note:	Steps 2-5 may proceed in parallel with Steps 3-4 described in Figure 6.2.2-1 in Section 6.2.2 of this document. The Local AAA proxy constructs the Diameter ACR, as defined in IETF RFC 7155 [25], and sends it to the accounting AAA server.

  - In the final DEA of authentication phase of Step 1 that carries the EAP Success, the PSP/3GPP AAA may provide the User-Name AVP that overrides the initially-used User-Name in the DER. The latest received User-Name shall be used by the Local AAA proxy for accounting application associated with the current session.

  - To accommodate Diameter-to-RADIUS translation when RADIUS-based accounting is used and the interim accounting reporting is expected, the accounting AAA server may also provide the Account-Interim-Interval AVP.

  - The Local AAA proxy maps the NHN internal identity associated with the session to the User-Name attribute received from the PSP/3GPP AAA in Step 1.

  - The Local AAA proxy sets the Accounting-Record-Type to START RECORD,

  - The Local AAA proxy includes the timestamp of the start of accounting session in the Event-Timestamp AVP.

  - To accommodate Diameter-to-RADIUS translation, the Local AAA proxy shall also include the Acct-Session-ID attribute. If the Acct-Session-ID was included in the Diameter DER/DEA exchange with the PSP/3GPP AAA in the EAP authentication phase of Step 1, the Diameter ACR generated by the Local AAA proxy shall use the latest such Acct-Session-ID AVP for the associated accounting session.

  - To allow the linking of multiple accounting sessions for the UE, if the Acct-Multi-Session-ID AVP was received from the PSP/3GPP AAA in a Diameter DEA during authorization phase in Step 1, the Local AAA proxy shall include the same Acct-Multi-Session-ID AVP in the constructed ACR.

  - When a new accounting sub-session is added to an existing accounting session for the UE that establishes multiple PDN connections as described in Section 6.2.11, the Local AAA proxy shall construct a new Diameter ACR with added Accounting-Sub-Session AVP for each added sub-session.

  - The combination of a Session-ID and Accounting-Sub-Session-ID for each sub-session shall be unique, and managed, as defined in Section 9.8.6 of IETF RFC 6733 [8].

- The accounting AAA server returns the ACA message with Accounting-Record-Type set to START_RECORD to the Local AAA Proxy, and possibly Acct-Interim-Interval (AII) AVP set to non-zero value indicating the desired intermediate accounting interval.

- The Local AAA proxy acknowledges the start of accounting session to the NH GW. With this acknowledgement, the Local AAA proxy sends the accounting parameters to the NH GW, such as the periodicity of reporting accounting data, and other information needed to report as required by the accounting AAA server.

- As defined by the accounting parameters communicated to the NH GW in Step 5, or based on interim reporting triggers set by the accounting application (i.e., every time re-authentication or re-authorization occurs, etc.), the NH GW may need to periodically report an intermediate accounting data. When either the AII elapses or conditions for sending accounting information are recognized at the NH GW, the NH GW may send an interim accounting report that contains accounting data.

- The Local AAA proxy sends an ACR with Accounting-Record-Type AVP set to INTERIM_RECORD to the accounting AAA server.

- The interim accounting report is acknowledged by the accounting AAA server in the returned Diameter ACA.

- The interim accounting report is acknowledged to the NH GW by the Local AAA Proxy.

- The session is terminated by the UE in this example, as described in Section 6.2.10 of this document. It could also be terminated by the NH MME for any reason, such as by the Local AAA proxy command, or by the NH GW action. The active bearer is deleted at the NH GW.

- Once the PDN connection is deactivated and bearer removed, the NH GW sends an Accounting Request to the Local AAA proxy with indication of the Stop of Accounting session. This indication carries the NHN internal identity assigned for the session.

- The Local AAA proxy maps the NHN internal identity associated with the session to the User-Name and Session-ID and optionally, Acct-Session-ID, and constructs the Diameter ACR. The Local AAA proxy setts the Accounting-Record-Type to STOP RECORD, as defined in IETF RFC 7155 [25]. The ACR is sent to the accounting AAA server.

  - If a current accounting session contains sub-sessions (see Section 6.2.11), and all sub-sessions for a given Session-ID are terminated, the Local AAA proxy shall not include the Accounting-Sub-Session AVP in the ACR; otherwise, the Local AAA proxy shall include the Accounting-Sub-Session AVP set to a number of a sub-session being terminated.

- The accounting AAA server returns the ACA message with Accounting-Record-Type set to STOP_RECORD to the Local AAA Proxy.

- The Local AAA proxy acknowledges the stop of accounting session to the NH GW. The Local AAA proxy deletes session specific parameters, such as NHN internal identity associated with the session, associated User-Name, Session ID, etc. Once this message is received by the NH GW, all parameters associated with current accounting session may be deleted at the NH GW.

The following list of Diameter AVPs for the ACR command, as specified in IETF RFC 7155 [25], shall be supported by the Local AAA proxy in order to accommodate accounting requirements listed in this section:         <AC-Request> ::= < Diameter Header: 271, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Accounting-Record-Type }

{ Accounting-Record-Number }

{ Acct-Application-Id }

{ User-Name }

[ Accounting-Sub-Session-Id ]

[ Acct-Session-Id ]

[ Acct-Multi-Session-Id ]

[ Origin-AAA-Protocol ]

[ Origin-State-Id ]

[ Destination-Host ]

[ Event-Timestamp ]

[ Acct-Delay-Time ]

[ Termination-Cause ]

[ Accounting-Input-Octets ]

[ Accounting-Input-Packets ]

[ Accounting-Output-Octets ]

[ Accounting-Output-Packets ]

[ Acct-Authentic ]

[ Accounting-Auth-Method ]

[ Acct-Link-Count ]

[ Acct-Session-Time ]

[ Acct-Tunnel-Connection ]

[ Acct-Tunnel-Packets-Lost ]

[ Accounting-Realtime-Required ]

[ Acct-Interim-Interval ]

The following list of Diameter AVPs for the ACA command as specified in IETF RFC 7155 [25] shall be supported by the Local AAA proxy in order to accommodate accounting requirements listed in this section:

<AC-Answer> ::= < Diameter Header: 271, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

{ Accounting-Record-Type }

{ Accounting-Record-Number }

{ Acct-Application-Id }

[ User-Name ]

[ Accounting-Sub-Session-Id ]

[ Acct-Session-Id ]

[ Acct-Multi-Session-Id ]

[ Event-Timestamp ]

[ Error-Message ]

[ Error-Reporting-Host ]

* [ Failed-AVP ]

[ Origin-AAA-Protocol ]

[ Origin-State-Id ]

[ Termination-Cause ]

[ Accounting-Realtime-Required ]

[ Acct-Interim-Interval ]

###### 6.3.2.3.3.3	RADIUS-based Accounting

For RADIUS-based accounting, an accounting interface between the Local AAA proxy acting as the accounting client and the accounting AAA server shall follow the requirements defined in IETF RFC 2866 [26].

Accounting information, including indications of accounting start, accounting stop, interim reporting of accounting data as requested by the accounting AAA server, are needed to support the session-based accounting. Required information is communicated by the Local AAA proxy to the accounting AAA server using the RADIUS Accounting-Request packet and is acknowledged using the RADIUS Accounting-Response packet.

The Local AAA proxy matches the Identifier field in the received Accounting-Response packet with the Identifier field in the pending Accounting-Request packet that waits to be acknowledged.

To correlate the accounting session with the authentication and authorization session, the Local AAA proxy shall use the same RADIUS User-Name attribute as was sent by authentication and authorization application for the same session. The Local AAA proxy shall map the accounting information identity used within the NHN (e.g., P-IMSI) to the RADIUS user name (User-Name) attribute, provided by the PSP/3GPP AAA during authentication and authorization phase of the session.

To match accounting records associated with the same accounting session, the Local AAA proxy shall use the same Acct-Session-ID attribute in all Accounting-Request packets. The Acct-Session-ID may be exchanged during authentication and authorization session. If it does, the Local AAA proxy shall use the same Acct-Session-ID in all Accounting-Request packets for that session.

Figure 6.3.2.3.3.3-1 shows flow of messages carrying accounting information.

Fig. 6.3.2.3.3.3-1:	NHN RADIUS-based Accounting

- UE attaches to the NHN. The session is activated on the NH GW. The procedure follows Section 6.2.2.

- Once the PDP is established and bearer is activated, the NH GW sends an indication to the Local AAA proxy of the Start of Accounting session. This indication carries the NHN internal identity associated with the session.

Note:	Steps 2-5 may proceed in parallel with steps 3-4 as described in Fig. 6.2.2-1 in Section 6.2.2 of this document.

- The Local AAA proxy constructs the RADIUS Accounting-Request packet, as defined in IETF RFC 2866 [26], and sends it to the accounting AAA server.

  - In the final DEA of authentication phase of Step 1 that carries the EAP Success, the PSP/3GPP AAA may provide the User-Name AVP that overrides the initially used User-Name in the DER. The latest received User-Name shall be used by the Local AAA proxy for accounting application associated with the current session.

  - When RADIUS-based accounting is used, and the interim accounting reporting is expected, the PSP/3GPP AAA shall also provide in the DEA command during the authentication phase the Acct-Interim-Interval AVP.

  - The Local AAA proxy maps the NHN internal identity associated with the session to the User-Name attribute received from the PSP/3GPP AAA in Step 1.

  - The Local AAA proxy sets the Accounting-Status-Type attribute to START.

  - The Local AAA proxy includes the Acct-Session-ID attribute. If the Acct-Session-ID was included in the Diameter DER/DEA exchange with the PSP/3GPP AAA in EAP authentication phase (Step 1), the Accounting-Request packet generated by the Local AAA proxy shall use the latest such Acct-Session-ID AVP for the associated accounting session.

  - To allow linking multiple accounting sessions for the UE, if the Acct-Multi-Session-ID attribute was received from the PSP/3GPP AAA in a Diameter DEA during authorization phase in Step 1, the Local AAA proxy shall include the same Acct-Multi-Session-ID attribute in the constructed Accounting-Request packet.

  - To allow linking multiple simultaneous accounting sub-sessions for the UE that established multiple PDN connections, possibly on multiple NH-GWs, the Local AAA proxy shall include the Acct-Link-Count attribute in the constructed Accounting-Request packet for each of the multi-link accounting sub-session. The combination of an Acct-Session-ID and Acct-Link-Count for each of the linked sub-session shall be unique, and managed as defined in Section 5.12 of IETF RFC 2866 [26].

- The accounting AAA server returns Accounting-Response packet, acknowledging the Accounting-Request in Step 3.

- The Local AAA proxy acknowledges the start of accounting session to the NH GW. With this acknowledgement the Local AAA proxy sends to the NH GW the accounting parameters such as Interim interval of reporting accounting data, and other information needed to report as required by the accounting AAA server.

- As defined by the accounting parameters communicated to the NH GW in Step 5, or based on interim reporting triggers set by the accounting application, i.e. every time re-Authentication or re-authorization occurs, etc., or local configuration the NH GW may need to periodically report an interim accounting data. When either AII elapses or conditions for sending accounting information are recognized at the NH GW, the NH GW may send an interim accounting report that contains accounting data.

- The Local AAA proxy sends an Accounting-Request with Accounting-Status-Type attribute set to Interim-Update to the accounting AAA server.

- Interim accounting report is acknowledged in the returned Accounting-Response packet.

- The interim accounting report is acknowledged to the NH GW by the Local AAA Proxy.

- The session is terminated by the UE in this example, as in Section 6.2.10 of this specification. It could also be terminated by the NH MME for any reason, such as by the Local AAA proxy command, or by the NH GW action. The active bearer is deleted at the NH GW.

- Once the PDP context is deactivated and bearer removed, the NH GW sends an Accounting Request to the Local AAA proxy with indication of the Stop of Accounting session. This indication carries the NHN internal identity assigned for the session.

- The Local AAA proxy maps the NHN internal identity associated with the session to the User-Name and Session-ID and optionally, Acct-Session-ID, and constructs the RADIUS Accounting-Request packet. The Local AAA proxy setts the Accounting-Status-Type to STOP, as defined in IETF RFC 2866 [26]. The Accounting-Request is sent to the accounting AAA server.

  - If a current accounting session contains linked sub-sessions, and all sub-sessions for a given Acct-Session-ID are terminated, the Local AAA proxy shall not include the Acct-Link-Count attribute in the Accounting-Request; otherwise, the Local AAA proxy shall include the Acct-Link-Count attribute set to a number of a linked sub-session being terminated.

- The accounting AAA server returns Accounting-Response packet to the Local AAA Proxy.

- The Local AAA proxy acknowledges the stop of accounting session to the NH GW. The Local AAA proxy deletes session specific parameters, such as NHN internal identity associated with the session, associated User-Name, Session ID, etc. Once this message is received by the NH GW, all parameters associated with current accounting session may be deleted at the NH GW.

# Annex A (informative): Change History

| Date | Document Number | Comment |
| --- | --- | --- |
| 2016-06 | mf2016.100.02 | Approved version from MFA #5 incorporating changes: mf2016.331.01, mf2016.363.01, mf2016.364.01, mf2016.374.02, mf2016.375.02, mf2016.376.02, mf2016.440.01, mf2016.379.01, mf2016.381.03, mf2016.322.01, mf2016.323.00, mf2016.352.01 and mf2016.366.02 |
| 2016-09 | mf2016.100.03 | Approved version from MFA #6 incorporating changes: mf2016.594.03, mf2016.590.04, mf2016.574.03, mf2016.584.01, mf2016.585.03, mf2016.582.01, mf2016.583.01, mf2016.588.02, mf2016.618.02, mf2016.619.02, mf2016.680.02, mf2016.681.02, mf2016.683.01 |
| 2016-11 | MFA TS MF.202 v1.0.0 | Approved version from MFA #7 incorporating changes: mf2016.799.00, mf2016.800.01, mf2016.810.03, mf2016.815.04, mf2016.816.02, mf2016.817.01, mf2016.818.01, mf2016.821.01, mf2016.827.01, mf2016.876.02 |
| 2017-01 | MFA TS MF.202 v1.0.1 | Approved version from MFA #8 incorporating changes: mf2016.978.00, mf2016.976.01 |
| 2017-02 | MFA TS MF.202 v1.0.2 | Approved version from MFA #9 incorporating changes described in mf2017.061.02 |
| 2017-06 | MFA TS MF.202 v1.0.3 | Draft version from MFA #10.1 (Conference Call) incorporating changes from mf2017.584.01 and mf2017.650.00. |
| 2017-09 | MFA TS MF.202 v1.0.4 | Approved version after MFA #12 incorporating changes from mf2017.836.00 and mf2017.870.01. |

# Annex B (Informative): Out-of-Band Service Discovery

This section describes an out-of-band service discovery mechanism. The service capability information for the NHN is hosted in a web resource reachable via the Internet. The UE queries this web-resource out-of-band i.e., using an existing internet connection over a different network.

Figure Annex B-1 depicts a call flow for the out of band service discovery procedure.

Figure Annex B-1: Out-of-Band Service Discovery for NHNs

- The MF AP broadcasts NHN-specific information like NHAMI, NHN-ID, etc. In addition, the MF AP broadcasts an indication indicating if Out-of-Band Service Discovery is supported by this MF NHN.

- The UE detects NHN deployment and determines that additional information about this network is required before the UE accesses this network.

- Based on information received in the broadcast messages the UE constructs the web resource location of the Out-of-Band Service Discovery server. Alternately, the web resource location is configured in the UE. The web resource location can be identified by an IP address, FQDN or URI.

- UE uses an existing IP connection to the internet to query the web resource and acquires service information for this NHN. The transport and content of these messages is for future study.

Based on the information received, the UE may decide to attach to the NHN or perform credential provisioning.

# Annex C (informative): Access service authorization examples

## C.1 General

When dynamic access service authorization is used, the AAA Authorization Response from the PSP defines the Access Service, which the NHN provides for the authorized UE. Authorized access service is indicated to the NHN using parameters as specified for Diameter in IETF RFC 7155 [25] and IETF RFC 6733 [8] as well as for RADIUS in IETF RFC 2868 [39] and IETF RFC 4849 [40].

The access service to be provided can be largely controlled by the following parameters:

- Tunnelling related Parameters

  - Tunnel Type	(Radius Attribute Type 64, Diameter AVP Code 64)

  - Tunnel Medium Type	(Radius Attribute Type 65, Diameter AVP Code 65)

  - Tunnel-Server-Endpoint	(Radius Attribute Type 67, Diameter AVP Code 67)

  - Framed IP address	(Radius Attribute Type 8, Diameter AVP Code 8)

- Filtering related Parameters

  - IP Filter Rule	(As specified in RFC 3588)

    - (action   dir   proto   from src   to dst)

## C.2 Examples

#### C.2.1 Open Access

In cases where the PSP desires to allow full open access for the UE, the PSP should not include any Tunnelling-related parameters nor any Filtering-related parameters to the Authorization Response message.

#### C.2.2 Bidirectional-Filtered Access with a specific IP subnet

In cases where the PSP desires to authorize the UE bidirectional communication only with IP hosts in IP subnet 1.2.3.0/24, the following parameters could be included in Authorization Response:

- Tunnelling-related parameters

  - N/A

- Filtering-related parameters

  - IP Filter Rule 1:	(permit   in   ip   proto   from 0.0.0.0/0    to 1.2.3.0/24)

  - IP Filter Rule 2:	(permit  out  ip  proto  from 1.2.3.0/24   to 0.0.0.0/0)

#### C.2.3 Tunnelled Access with GRE over IPv4

In cases where the PSP desires that all IP communication is to occur using GRE over IPv4 via IP host 1.2.3.4 the following parameters could be included in Authorization Response:

- Tunnelling related Parameters

  - Tunnel Type:		 	GRE

  - Tunnel Medium Type	 	IPv4

  - Tunnel-Server-Endpoint 	1.2.3.4

- Filtering related Parameters

  - N/A

Depending on how the PSP prefers the UE IP address allocation to happen, the PSP may also include the Framed IP address (Radius Attribute Type 8/Diameter AVP Code 8) to its Authorization Response as follows:

- The NHN allocates an NHN-specific IP address for the UE: Don’t include Framed IP address.

- The NHN allocates an IP address for the UE by performing a DHCP query towards the PSP-informed Tunnel-Server-Endpoint: Set Framed IP address to 0xFFFFFFFE.

- The NHN does not allocate and IP address for the UE, but allows the PSP to allocate it via a UE-triggered DHCP as specified in 3GPP for External PDN Address Allocation: Set Framed IP address to 0xFFFFFFFF.

- The NHN allocates a PSP-specific pre-allocated IP address 1.2.3.4 for the UE: Set Framed IP address to {4 octet presentation of 1.2.3.4}.

# Annex D: PSP and NHN Selection (Normative)

This section specifies further details on PSP and NHN selection.

PSP and NHN selection shall be done based on the terms defined in 3GPP TS 23.122 [33]. When an MS is capable of connecting to PSPs via NHNs and is performing network selection of NHN and PSP based on the terms defined in 3GPP TS 23.122 the following parameters apply:

- 3GPP TS 23.122 term “no SIM in the MS” is interpreted as no SIM or any other credentials (e.g. certificates) available for NHN access in the MS.

- 3GPP TS 23.122 term “PLMN Selection” is interpreted as PSP Selection.

- 3GPP TS 23.122 term “Registration on a PLMN” is interpreted as Registration on a PSP/NHN pair

- 3GPP TS 23.122 term “Registered PLMN” is interpreted as Registered PSP/NHN pair

- 3GPP TS 23.122 term “Selected PLMN” is interpreted as Selected PSP/NHN pair

- 3GPP TS 23.122 Automatic Network Selection Mode:

  - In addition to the listed “PLMN/access technology combination” categories, also a further automatic selection category exists for the MS:

    - Each PSP/NHN access technology combination configured for automatic network selection in the UE (in priority order based on user preference and UE implementation).

  - The selection priority of PSP/NHN access technology combinations from this further category compared to the PLMN/access technology combinations from the categories listed in 3GPP TS 23.122 [33] is not specified and is implementation-dependent.

Note:	Depending on UE capabilities and implementation, the following may be possible: A dual radio UE may have completely separate radios for PLMN/access technology combinations and for PSP/NHN access technology combinations. In this case, for automatic network selection purposes, one radio may have entries only in selection categories within TS23.122 (i.e., PLMNs), while the other radio may have entries only in the further selection category introduced above (i.e., PSPs). On the other hand, a single radio UE may have entries across all the categories (i.e., PLMNs and PSPs) available for making the automatic network selection for the single radio the UE has.

- 3GPP TS 23.122 Manual Network Selection Mode:

  - In addition to the listed categories for presenting the available PLMNs to the user, also a further presentation category exists for the MS:

    - PSP/NHN access technology combinations

  - The presentation priority of PSP/NHN access technology combinations from this further category compared to the PLMN/access technology combinations from the categories listed in 3GPP TS 23.122 [33] is not specified and is implementation-dependent.

Note:	Depending on UE capabilities and implementation the following may be possible: A dual radio UE may have completely separate radios for PLMN/access technology combinations and for PSP/NHN access technology combinations. In this case, for manual network selection purposes, one radio may have entries only in presentation categories within those listed within 3GPP TS 23.122 [33] (i.e., PLMNs), while the other radio may have entries only in the further presentation category introduced above (i.e., PSPs). On the other hand, a single radio UE may have entries from all the categories (i.e., PLMNs and PSPs) available for presenting the user with the manual network selection options for the single radio the UE has. Possible sub-categorization of NHN access technology is not specified and is implementation dependent. A MS may consider the following as separate subcategories, each with its own selection/presentation priority for a given PSP:

    - A specific NHN, as identified by its NHN-ID
(e.g., to consider a PSP only via a specific NHN)

    - NHNs supporting S2a for the PSP
(e.g., to consider a PSP only via NHNs supporting S2a for the PSP)

    - NHNs supporting the PSP

- When MS is registered to a PSP/NHN pair, the MS behavior to discover availability of PLMNs or/and higher priority PSPs or PSP/NHN combinations is not specified and is implementation-dependent.