MFA TS MF.301 V1.0.2 (2017-02)

Technical Specification

MulteFire Alliance

MulteFire Subscription Management Stage 3

(Release 1)

The present document has been developed within MulteFire Alliance and may be further elaborated for the purposes of the MulteFire Alliance.
The MulteFire Alliance accepts no liability for any use of this Specification.

Keywords

multefire, neutral host network, NHN, subscription, certificate

MulteFire Alliance

Postal address

5177 Brandin Court

Fremont, CA 94538 USA

Internet

http://www.multefire.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016-2017, MulteFire Alliance

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

1	Scope	3

2	References	3

3	Definitions, symbols and abbreviations	4

3.1	Definitions	4

3.2	Abbreviations	4

4	SDP containers	5

4.0	Overview	5

4.1	Query/Response Message container IE for SDP	5

4.2	SDP container type	5

4.3	SDP version	5

4.4	SDP container for PSP discovery query	5

4.5	SDP container for OSU information query	5

4.6	SDP container for PSP discovery response	6

4.7	SDP container for OSU information response	7

5.	Subscription provisioning	7

5.1	Overview	7

5.2	Mobile device management tree	8

6	Management objects	8

6.1	SubscriptionProfile MO	9

6.1.1	Graphical representation	9

6.1.2	Node descriptions	10

6.2	DevDetail MO vendor specific extensions	11

6.2.1	Graphical representation	11

6.2.2	Node descriptions	12

7	MulteFire Certificate Policy Specifications	13

7.0	Overview	13

7.1	Certificates for the MulteFire OSU	14

7.2	MulteFire Trust Root Certificates	14

7.3	MulteFire OSU Server Certificates	15

7.4	MulteFire Device Certificates	16

Annex A (informative): Change history	17

# 1	Scope

The present document defines the MulteFire subscription establishment and management, including definitions of Subscription Profile Management Object (MO) that contains subscription specific parameters supporting PSP subscriptions and service discovery in NHN access mode, SDP containers for SDP query process, the provisioning process to deploy the MO, and requirements for the format and issuing process of public key certificates used in MulteFire.

The scope of this document does not include establishment and management of USIM-based subscriptions required for access to 3GPP resources.

# 2	References

For interpretations of references, the defined rules in Section 4 of MFA TR MF.100 [0] apply.

[0]	MulteFire Alliance (MFA). Guidelines for MulteFire Specifications. MFA TR MF.100.

[1]	Third Generation Partnership Project (3GPP). Vocabulary for 3GPP Specifications, 3GPP TR 21.905.

[2]	MulteFire Alliance (MFA). MulteFire Deployment Scenarios and Requirements (Release 1), MFA TR MF.101.

[3]	Wi-Fi Alliance. Hotspot 2.0 (Release 2) Technical Specification. Available at http://www.wi-fi.org/file/hotspot-20-release-2-technical-specification-package-v110-0 (Registration required to view).

[4]	Wi-Fi Alliance. Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications, Version 1.1.

[5]	The Internet Engineering Task Force (IETF). Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile, edited by D. Cooper et al., May 2008. IETF RFC 5280, also available at https://www.ietf.org/rfc/rfc5280.txt.

[6]	MulteFire Alliance (MFA). Architecture for Neutral Host Network Access Mode Stage 2 (Release 1). MFA TS MF 202.

[7]	The Internet Engineering Task Force (IETF). X.509 Internet Public Key Infrastructure Online Certificate Status Protocol – OCSP, edited by M. Myers et al., June 1999. IETF RFC 2560, also available at https://www.ietf.org/rfc/rfc2560.txt.

[8]	Open Mobile Alliance (OMA). Enabler Release Definition for OMA Device Management, Approved Version 1.2, February 2007. OMA-ERELD-DM-V1_2, also available at http://www.openmobilealliance.org/release/DM/V1_2-20070209-A/OMA-ERELD-DM-V1_2-20070209-A.pdf.

[8A]	Open Mobile Alliance (OMA). OMA Device Management Protocol Candidate Version 1.2, August 2005. OMA-TS-DM_Protocol-V1_2, also available at http://member.openmobilealliance.org/ftp/Public_documents/DM/Permanent_documents/ (see index).

[8B]	Open Mobile Alliance (OMA). OMA Device Management Tree and Description Approved 1.2, February 2007. OMA-TS-DM_TND-V1_2, also available at http://www.openmobilealliance.org/release/DM/V1_2-20070209-A/OMA-TS-DM_TNDS-V1_2-20070209-A.pdf.

[8C]	Open Mobile Alliance (OMA). OMA Device Management Tree and Description Serialization, April 2006. OMA-TS-DM_TNDS-V1_2, also available at http://devwww.openmobilealliance.org/release/DM/V1_2-20060424-C/OMA-TS-DM-TNDS-V1_2-20060406-C.pdf.

[9]	The World Wide Web Consortium (W3C). SOAP Version 1.2 Part 1: Messaging Framework (Second Edition), April 2007. Available at http://www.w3.org/TR/soap12-part1/.

[10]	IEEE Standards Association. Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications, March 2012. IEEE 802.11-2012, Part 11:

[11]	Open Mobile Alliance (OMA). OMA Device Management Standardized Objects, Approved Version 1.2, February 2007. OMA-TS-DM_StdObj-V1_2_2-20070209, also available at http://member.openmobilealliance.org/ftp/Public_documents/DM/Permanent_documents/ (see index).

[12]	The Internet Engineering Task Force (IETF). UTF-8, a transformation format of ISO 10646, edited by F. Yergeau, November 2003. IETF RFC 3629, also available at https://www.ietf.org/rfc/rfc3629.txt.

[13]	The Internet Engineering Task Force (IETF). Uniform Resource Identifier (URI): Generic Syntax, edited by T. Berners-Lee, R. Fielding, and L. Masinter, January 2005. IETF RFC 3986, also available at https://www.ietf.org/rfc/rfc3986.txt.

[14]	The Internet Engineering Task Force (IETF). The Network Access Identifier, edited by A.Dekok, May 2015. IETF RFC 7542, also available at https://www.ietf.org/rfc/rfc7542.txt.

[15]	MulteFire Alliance (MFA), in collaboration with Third Generation Partnership Project (3GPP). Non-Access-Stratum (NAS) Protocol for Evolved Packet System (EPS), MFA TS 24.301.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions listed in 3GPP TR 21.905 [1], MFA TS MF.202 [6], as well as in Hotspot 2.0 (Release 2) Technical Specification [3] and Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications [4] apply. A term defined in this document takes precedence over the the same term as defined (if any), in other referenced documents listed in this section.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations provided in MFA TS MF.202 [6], as well as in Hotspot 2.0 (Release 2) Technical Specification [3] and Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications [4] apply.

| Abbreviation | Definition |
| --- | --- |
| AAA | Authentication, Authorization, and Accounting |
| ACL | Access Control List |
| ANQP | Access Network Query Protocol |
| CA | Certificate Authority |
| MF | MulteFire |
| MFA | MulteFire Alliance |
| MO | Management Object |
| NHN | Neutral Host Network |
| OID | Organizational Identifier |
| OMA | Open Mobile Alliance |
| OSU | Online Signup System |
| PKI | Public Key Infrastructure |
| PSP | Participating Service Provider |
| PSP-ID | Participating Service Provider Identifier |
| SDP | Service Delivery Platform |
| SOAP | Simple Object Access Protocol |

# 4	SDP containers

## 4.0	Overview

The generic query procedure for MFA TS MF.202 [6]. The NAS messages where query type specific containers are exchanged within the Query/Response Message container IE are defined in MulteFire-specific adaptations for MFA TS 24.301 [15].

This Query/Response Message container IE for SDP Query procedures are defined in this specification. The different SDP Query and Response types are listed in Table 4.2-1. Respective procedures are referred as SDP Query for PSP discovery and SDP Query for OSU information, both of which are described further in Section 5.11.7 of MFA TS MF.202 [6].

## 4.1	Query/Response Message container IE for SDP

The generic Query/Response Message container structure for SDP is shown in Figure 4.1-1.

| SDP container type | octet 1 |
| --- | --- |
| Length of SDP container | octet 2 octet 3 |
| Version | octet 4 |
| SDP container | octet 5 octet 5+n |

Figure 4.1-1: Query/Response Message container IE for SDP

## 4.2	SDP container type

The purpose of the SDP container type element is to specify the type of the SDP container included into the Query/Response Message container NAS IE.

The PSP Discovery query and OSU information query containers are used in Uplink Query NAS Transport messages for SDP and PSP discovery response and OSU information response containers in Downlink Response NAS Transport messages for SDP.

Table 4.2-1: SDP container type

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | PSP discovery query |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | PSP discovery response |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | OSU information query |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | OSU information response |
| All other values are reserved |  |  |  |  |  |  |  |  |  |

## 4.3	SDP version

The SDP version value shall be 1 in this specification release. It will be monotonically increased if a modification is introduced which changes SDP container content structures.

## 4.4	SDP container for PSP discovery query

This release does not define PSP discovery query specific SDP container. The SDP container length shall be set to 0.

## 4.5	SDP container for OSU information query

The SDP container for OSU information query message contains the PSP-ID of the OSU service provider for which the UE wishes to receive OSU information. The PSP-ID format is either a short form PSP-ID or OID.

Figure 4.5-1 shows how the SDP container coding for OSU information query.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of the container (6) |  |  |  |  |  |  |  |  | octet 1 octet 2 |
| Reserved |  |  |  |  |  |  | PSP-ID type |  | octet 3 |
| Short form PSP-ID or OID-24 |  |  |  |  |  |  |  |  | octet 4 octet 6 |

Figure 4.5-1: SDP container for OSU information query

The PSP-ID type bit is set when contained PSP-ID is presented as a short form PSP-ID, and otherwise as a 24 bit OID (OID-24). OIDs longer than 24 bits are presented as short form PSP-ID.

Both short form PSP-ID and OID are coded in 3 octets as described in Section 6.2.4 of MFA TS MF.202 [6].

## 4.6	SDP container for PSP discovery response

The SDP container for PSP discovery response provides a list of PSP specific information. Each element includes PSP long form name, PSP friendly names in multiple languages and indication whether PSP supports OSU and thereby additional OSU information is available of the PSP via SDP Query for OSU information procedure.

SDP container for PSP discovery response is coded as shown in Figure 4.6-1.

| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of PSP information 1 |  |  |  |  |  |  |  |  | octet 1 octet 2 |
| Spare |  |  |  |  |  | PSP-ID type2 |  | OSU support | octet 3 |
| Length of long form PSP name/OID 1 |  |  |  |  |  |  |  |  | octet 4 |
| PSP long form name/OID 1 (k octets) |  |  |  |  |  |  |  |  | octet 5 octet 5+k |
| Length of PSP friendly names |  |  |  |  |  |  |  |  | octet 6+k octet 7+k |
| Length of PSP friendly name 1 |  |  |  |  |  |  |  |  | octet 8 + k |
| Language code 1 |  |  |  |  |  |  |  |  | octet 9+k octet 11+k |
| PSP friendly name 1 (n octets) |  |  |  |  |  |  |  |  | octet 12+k octet 12+k+n |
| … |  |  |  |  |  |  |  |  |  |
| PSP friendly name z |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  | (start of remaining PSP information elements) |
| Length of PSP information/OID v |  |  |  |  |  |  |  |  |  |
| PSP information/OID v |  |  |  |  |  |  |  |  |  |

Figure 4.6-1: SDP container for PSP information response

When PSP-ID type2 bit is set, the following PSP identity is an OID and otherwise a PSP long form name.

The format of the OID is specified in Section 8.4.1.31 of IEEE 802.11-2012, Part 11 [10].

## 4.7	SDP container for OSU information response

The SDP container for OSU information response provides PSP-specific OSU information on requested OSU provider.

Figure 4.7-1 shows the SDP response message container coding for OSU information.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of OSU information |  |  |  |  |  |  |  |  | octet 1 octet 2 |  |
| Reserved |  |  |  |  |  |  | PSP-ID type |  | octet 3 |
| Short form PSP-ID or OID |  |  |  |  |  |  |  |  | octet 4 octet 6 |
| Length of OSU Server URI |  |  |  |  |  |  |  |  | octet 7 octet 8 |  |
| OSU Server URI (k octets) |  |  |  |  |  |  |  |  | octet 9 octet 9+k |  |
| Length of OSU NAI |  |  |  |  |  |  |  |  | octet 10+k |  |
| OSU NAI (n octets) |  |  |  |  |  |  |  |  | octet 11+k octet 11+k+n |  |

Figure 4.7-1: SDP container for OSU information response

The PSP-ID type and short form PSP-ID and OID are in accordance with Section 4.5 of this specification.

The OSU Server URI is the URI of the OSU server that is used for OSU with the PSP. The content of the OSU Server URI is formatted in accordance with parameters defined in IETF RFC 3986 [13].

OSU NAI is the user identity the UE shall use in anonymous TLS authentication for OSU attach. It is formatted in accordance with the parameters defined in IETF RFC 7542 [14].

# 5.	Subscription provisioning

Subscription provisioning comprises:

- Credential-provisioning and related metadata-provisioning for network access

- PSP information provisioning for network discovery and selection

This section is adapted from Chapter 8 of Hotspot 2.0 (Release 2) Technical Specification [3].

## 5.1	Overview

Sections 7.3, 7.5, 7.6, and Chapter 8 of Hotspot 2.0 (Release 2) Technical Specification [3] apply, with the following adaptations:

- The SP is considered synonymous to the PSP.

- There is no policy provisioning. Note: this implies that Section 7.3.6 of Hotspot 2.0 (Release 2) Technical Specification [3] is not applicable.

- There is no subscription remediation. Note: this implies that Section 7.3.5 of Hotspot 2.0 (Release 2) Technical Specification [3] is not applicable.

- There is no Anonymous EAP-TLS. Note: this implies that Sections 7.3.4 and 7.7 of Hotspot 2.0 (Release 2) Technical Specification [3] are not applicable.

- There is no PolicyUpdate node and related procedures. Note: this implies that Sections 8.1.5, 8.3.4, and 8.4.4 of Hotspot 2.0 (Release 2) Technical Specification [3] are not applicable.

- In Figure 31 of Hotspot 2.0 (Release 2) Technical Specification [3], the OSU server is the only subscription server defined.

- ANQP defined in HS2.0 [3] is replaced with SDP Query procedures defined in clause 4 of this specification.

- The SubscriptionProfile MO defined in Section 6.1 of this specification is used in place of PerProviderSubsription MO defined in Hotspot 2.0 (Release 2) Technical Specification [3].

- The NHN term replaces the Wi-Fi (AP/AN) terminology used in Hotspot 2.0 (Release 2) Technical Specification [3].

- There is no equivalent to 3GPP AAA initiated PerProviderSubsription MO provisioning procedure defined in Hotspot 2.0 (Release 2) Technical Specification [3]. Procedure is not applied.

- The WNM-Notification Request/Response defined in Hotspot 2.0 (Release 2) Technical Specification [3] is not applied.

- Instead of using the disassociate procedure defined in Hotspot 2.0 (Release 2) Technical Specification [3], the RRC Connection Release procedure is applied as defined in Section 5.11.7 of MFA TS MF.202 [6].

- Instead of using associate procedure defined in Hotspot 2.0 (Release 2) Technical Specification [3], the RRC Connection Request procedure is applied as defined in Section 5.11.7 of MFA TS MF.202 [6].

- Instead of using 802.11 authentication, the EAP authentication over NAS is applied as defined in Section 5.12.2 of MFA TS MF.202 [6].

As support for Device certificate is mandatory for UEs that support the OSU procedure, the OSU server shall request a client certificate already in the initial TLS setup, as specified Section 5.14.2 of MFA TS MF.202 [6]. This is contrary to Step 1 in Figures 35 and 47 of Hotspot 2.0 (Release 2) Technical Specification [3]. In Figures 35 and 47 in Hotspot 2.0 (Release 2) Technical Specification [3], Steps 2 through 5 do not apply.

- The terminology “HotSpot 2.0 trusted CA database” in Section 7.3.1 of Hotspot 2.0 (Release 2) Technical Specification [3] is replaced with the terminology “MFA trusted CA database”.

- The terminology “Operator Friendly Name” is replaced with the terminology “MFA PSP Friendly Name”

- The reserved value “id-wfa-hotspot-friendlyName” is replaced with the reserved value “id-mfa-psp-friendlyName”. This value is the type-id of the X.509 otherName containing the MFA PSP Friendly Name.

- The reserved OID value “1.3.6.1.4.1.40808.1.1.1” (identifying the reserved value id-WFA-hotspot-friendlyName) is replaced by the reserved OID value “1.3.6.1.4.1.[MFA Code].1.1.1” (identifying the reserved value “id-mfa-psp-friendlyName”).

- Instead of using HS2.0 access authentication as described in Section 7.6.2 in Hotspot 2.0 (Release 2) Technical Specification [3], MFA NHN network access authentication is applied as specified in Sections 5.12.3.3 (EAP-TLS) or 5.12.3.4 (EAP-TTLS) of MFA TS MF.202 [6]

- The terminology “HS2.0 Extended Key Usage” is replaced with the terminology “MFA NHN Extended Key Usage”

- The terminology “HS2.0 EKU” is replaced with the terminology “MFA NHN EKU”

- The reserved value “id-kp-HS2.0Auth” is replaced with the reserved value “id-kp-mfa-NHNAuth”

- The reserved OID value “{1.3.6.1.4.1.40808.1.1.2}” (identifying the reserved value “id-kp-HS2.0Auth”) is replaced with “1.3.6.1.4.1.[MFA Code].1.1.2” (identifying the reserved value “id-kp-mfa-NHNAuth”).

Note:	The MFA-specific Organization Code [MFA Code] needs to be requested from and allocated by IANA.

## 5.2	Mobile device management tree

Section 8.1 of Hotspot 2.0 (Release 2) Technical Specification [3] applies with the following adaptations, in addition to the adaptations defined in Section 5.1 in this specification:

- Instead of using root node “./Wi-Fi/SP FQDN*”, the MF uses root node “./MulteFire/PSP FQDN*”

- Section 8.3.3 (Subscription Management) using OMA DM applies for subscription update only. Remediation is not supported.

- Section 8.3.4 (Policy Provisioning) using OMA DM does not apply

- Section 8.4.3 (Subscription Management) using SOAP applies for subscription update only. Remediation is not supported.

- Section 8.4.4 (Policy Provisioning) using SOAP does not apply

- Section 8.5 (Provisioning of a mobile device that has a SIM card) does not apply

# 6	Management objects

The Management object (MO) intends to follow the PerProviderSubscription MO and principles defined in Hotspot 2.0 (Release 2) Technical Specification [3], but is adapted to MulteFire usage.

The MOs specified in Sections 6.1 and 6.2 are defined in accordance with OMA-TS-DM_TND-V1_2 [8B]. All MOs transferred between the mobile device and the online signup subscription server shall be encoded according to OMA-TS-DM_TNDS-V1_2 [8C].

## 6.1	SubscriptionProfile MO

This section defines the MulteFire SubscriptionProfile MO for PSP-specific subscriptions. The MO is used to manage MulteFire subscription profiles stored into the UE. The profile provides information for network discovery and credentials for network access.

This MO is defined according to the principles specified in OMA-TS-DM_TND-V1_2 [8B] and is therefore an OMA DM-compliant MO, although it may be used with both the OMA-TS-DM_Protocol-V1_2 [8A] and XML transfer protocols described in SOAP Version 1.2 Part 1: Messaging Framework [9].

The mobile device shall use the provisioned SubscriptionProfile MO to select and authenticate a network in accordance with the identifiers, credentials and related metadata contained in the MO.

The MO provisioning can happen via MulteFire online signup session (in-band), via external means (out-of-band) or manually by user. Manual option applies to username/password type of credentials. Only the entity that has created the MO is able to modify the MO, unless the associated ACL says otherwise.

Online signup subscription servers shall generate valid XML names for all the dynamic nodes in the SubscriptionProfile MO provisioned to the mobile device. These XML names shall be unique within the MO.

Note:	Figure 6.1.1-1 and Figure 6.2.1-1 indicate dynamic nodes with the notation “<X+>”.

The implementation shall use case insensitive comparison when processing SubscriptionProfile MO node names. The case of the names shall be maintained, e.g., when uploading the MO to the server from which it was received.

The use of term “up to” in this section indicates the expectation that a leaf node should not be any longer than a specified limit and that all lengths up to that limit should/shall (depending on the context) be supported, e.g., “Mobile device implementations shall support Username strings up to 63 octets long”.

### 6.1.1	Graphical representation

Figure 6.1.1-1 shows the SubscriptionProfile MO. This is adapted from the PerProviderSubscription MO, as described in Hotspot 2.0 (Release 2) Technical Specification [3].

Figure 6.1.1-1: MulteFire subscription profile

### 6.1.2	Node descriptions

Node descriptions as defined in Section 9.1.2 of Hotspot 2.0 (Release 2) Technical Specification [3], applies with following exceptions:

- Instead of PerProviderSubscription node, the SubscriptionProfile node is used.

- The PerProviderSubscription/<X+>/SubscriptionParameters node is not applied.

- The PerProviderSubscription/<X+>/Policy node is not applied.

- The PerProviderSubscription/<X+>/HomeSP/NetworkID is not applied.

- The PerProviderSubscription/<X+>/Credential/SIM/EAPType has only one parameter value: EAP-AKA’

The following nodes detect PSP IDs that allow the UE to authenticate using the credentials in PerProviderSubscription/<X+>/Credential node. The nodes represent full-form PSP IDs:

- PerProviderSubscription/<X+>/HomeSP /FQDN

- PerProviderSubscription/<X+>/HomeSP/OtherHomePartners/<X+>/FQDN

- PerProviderSubscription/<X+>/Credential/Realm

## 6.2	DevDetail MO vendor specific extensions

This section defines the MulteFire vendor-specific extensions to the DevDetail standard OMA DM MO that contains data needed for the credential provisioning. The DevDetail MO is defined in OMA-TS-DM_StdObj-V1_2_2 [11].

The MO Identifier for this vendor-specific extension shall be: “urn:mfa:mo-ext:multefire-devdetail-ext:1.0”.

### 6.2.1	Graphical representation

Figure 6.2.1-1 shows the structure of the MFA vendor-specific extensions to the OMA DM DevDetail standard MO as defined in OMA-TS-DM_StdObj-V1_2_2 [11]. The “DevDetail”, “Ext?”, “Bearer?” and “LrgObj” interior nodes are part of the defined OMA DM MO and shall not be duplicated; they are only shown in the figure to provide context. The ellipsis in Figure 6.2.1-1 indicates that other nodes, which are not relevant to these vendor-specific extensions, are not shown.

The mobile device shall generate a valid XML name for each dynamic node in the Wi-Fi extensions to the DevDetail MO sent to a subscription or policy server. Each XML name shall be unique within the MO.

Note:	Dynamic nodes are indicated in Figure 6.2.1-1 by the notation “<X+>”.

The Wi-Fi MO extension is defined in this section. This extension shall be uniquely identified by using following path in the DevDetail subtree: “./DevDetail/Ext/org.wfa/MulteFire”.

Figure 6.2.1-1: Graphical representation of the Vendor-specific extension
to the DevDetail Standard MO

### 6.2.2	Node descriptions

In order to preserve the capability for the mobile device to add the new nodes illustrated in Figure 6.2.1-1, the DevDetail node and its child x* node in a MulteFire-compliant mobile device shall have its ACL set to "Add=*&Get=*" (see Section 7.7.1.2 in OMA-TS-DM_StdObj-V1_2_2 [8B]); furthermore, the mobile device shall not permit any server to change the value of the ACL on either of these nodes.

Only the MulteFire node is defined in this section and all nodes in this MO are encoded using UTF-8 asper IETF RFC 3629 [12], unless stated otherwise. In the following descriptions, the node Status indicates whether or not the mobile device (OMA DM Client) needs to support the node. If the Status is “Required”, then the mobile device shall support that node, provided the parent node of this node is supported. If the Status is “Optional”, the mobile device is not required to support the node.

MulteFire

| Status | Occurrence | Format | Access Types |
| --- | --- | --- | --- |
| Required | One | Node | Get |

This interior node acts as a container for mobile device information related to PSP credential provisioning. In order to preserve the capability for the mobile device to provide this information in this MO to any PSP with which the user has a subscription, the scope (see Section 6.2.3 in OMA-TS-DM_StdObj-V1_2_2 [8B]) of the MulteFire node shall be permanent and it is default ACL shall be set to "Add=*&Get=*" (see Section 7.7.1.2 in OMA-TS-DM_StdObj-V1_2_2 [8B]); the mobile device shall not permit any server to change this ACL. The MO Identifier for this node shall be: “urn:mfa:mo-ext:multefire-devdetail-ext:1.0

MulteFire/EAPMethodList

This interior node is identical to the Wi-Fi/EAPMethodList node as described in Section 9.2.2 of Hotspot 2.0 (Release 2) Technical Specification [3]. The definition in the Hotspot 2.0 specification applies.

MulteFire/PSPCertificate

This interior node is identical to the Wi-Fi/SPCertificate node as described in Section 9.2.2 of Hotspot 2.0 (Release 2) Technical Specification [3]. The definition in the Hotspot 2.0 specification applies.

MulteFire/ManufacturingCertificate

This interior node is identical to the Wi-Fi/ManufacturingCertificate as described in Section 9.2.2 of Hotspot 2.0 (Release 2) Technical Specification [3]. The definition in the Hotspot 2.0 specification applies.

MulteFire/DevID

| Status | Occurrence | Format | Access Types |
| --- | --- | --- | --- |
| Required | One | Chr | Get |

This leaf node contains a vendor-specific unique device identifier from the device certificate, if one is present in the device certificate. For X.509-based device certificates, the Common Name (CN) component of the DName may be used. Otherwise it contains vendor specific unique device identifier permanently stored into the device.

MulteFire/ClientTriggerRedirectURI

This leaf node is identical to the Wi-Fi/ClientTriggerRedirectURI leaf node as described in Section 9.2.2 of Hotspot 2.0 (Release 2) Technical Specification [3]. The definition in the Hotspot 2.0 specification applies.

Wi-Fi/Ops

This interior node is identical to the Wi-Fi/OPS as described in Section 9.2.2 of Hotspot 2.0 (Release 2) Technical Specification [3]. The definition in the Hotspot 2.0 specification applies.

Wi-Fi/Ext

| Status | Occurrence | Format | Access Types |
| --- | --- | --- | --- |
| Optional | ZeroOrOne | Node | Get |

This interior node acts as a container for further extensions.

# 7	MulteFire Certificate Policy Specifications

## 7.0	Overview

This Section describes requirements for certificates used in procedure defined for the MulteFire Neutral Host Network (NHN). These requirements are based on Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications [4] with the adaptations described in this document.

The scope of this Section includes:

- A profile of X.509 Certificates used for Online Sign-Up, as defined in IETF RFC 5280 [5], that documents constraints on the allowable certificate fields. The certificate profile includes specifications of:

  - Trust Root certificates for validation of OSU server certificates

  - OSU server certificates applicable for OSU AAA and OSU Servers.

- A profile of X.509 Certificates used for Device Certificates, as defined in IETF RFC 5280 [5], that documents constraints on the allowable certificate fields.

- Requirements on the issuing Certificate Authority for verification of identity information and correspondence between identified entities and the fields in the certificate.

- Requirements for revocation support of OSU Server Certificates using the OCSP protocol, as defined in IETF RFC 2560 [7].

MulteFire also uses certificates to authenticate the PSP AAA servers. The requirements for PSP AAA certificates are not in scope of this specification.

The MulteFire NHN architecture does not include either an OSU AAA or the OSU server. These functional elements are only supported in the PSP domain. The MulteFire NHN shall only allow https traffic to OSU servers in the domain of selected PSP.

## 7.1	Certificates for the MulteFire OSU

The MulteFire specification uses four types of public certificates for the authentication of OSU servers and devices during the OSU process.

- MulteFire Trust Root CA certificates provisioned in the UE for validation of received server certificates.

- Trust Root CA certificates accessible by the OSU AAA and OSU servers for validation of received Device Certificates.

Note:	Selection of a Trust Root CA for validation of the Device certificate is based on pre-agreement between the PSP OSU infrastructure and the device manufacturer, and is outside the scope of this specification.

- MulteFire OSU server certificates provisioned in the OSU AAA and the OSU Server in the PSP domain.

- MulteFire Device Certificates provisioned in the UE.

All MulteFire certificates are X.509v3 public key certificates, as defined in IETF RFC 5280 [5].

Each OSU server has a certificate signed by a Certificate Authority (CA) whose root certificate is trusted by the mobile device.

The requirements for the MulteFire trust hierarchy are specified below:

- There will be at least two different MulteFire Trust Root CAs authorized by MulteFire Alliance.

- The MulteFire OSU server certificates shall be issued by a Trust Root CA authorized by MulteFire Alliance.

- OSU server certificates shall only be used for authenticating the OSU server and OSU AAA server to the UE as defined by the MulteFire specifications.

- CA vendors in the MulteFire OSU Server PKI shall support OCSP as defined in IETF RFC 2560 [7] for the distribution of revocation lists.

- The device certificates shall be issued by a CA chosen by the device manufacturer.

- Device certificates shall only be used for authenticating the device as defined by the MulteFire specifications.

## 7.2	MulteFire Trust Root Certificates

Table 7.2-1 lists the MulteFire-specific adaptations of constraints on fields in a Hotspot 2.0 OSU Trust Root certificate, as defined in Table 2 of Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications [4].

Table 7.2-1: MulteFire Trust Root Certificate Profile

| Selected Certificate Fields | Profile Specific Constraints HS 2.0 | Profile Specific Constraints MulteFire | M/O1 | ASN.1 Type & OIDs |
| --- | --- | --- | --- | --- |
| Validity | The validity period should be 30 years from the issuing date (about 2043). UTCTime values shall be used. | The validity period should be 30 years from the issuing date (about 2048). UTCTime values shall be used. | M | SEQUENCE {notBefore Time, notAfter Time} |
| Subject | The "subject" field shall only contain the following attributes: C=US O=WFA Hotspot 2.0 CN=Hotspot 2.0 Trust Root CA - <ID#>. Note: ID# identifies the CA hosting the root certificate. For example, for the NetworkFX Trust Root CA, the CN is set to “Hotspot 2.0 Trust Root CA - 01”. ID# = 01 for NetworkFX ID# = 02 for Verizon ID# = 03 for DigiCert | The "subject" field shall only contain the following attributes: C=Issuing Country O=MFA CN= MFA Trust Root CA - <ID#>. Note: ID# identifies the CA hosting the root certificate. For example, for the NetworkFX Trust Root CA, the CN is set to “MFA Trust Root CA - 01”. ID# = 01 for NetworkFX ID# = 02 for Symantec ID# = 03 for DigiCert | M | Name . |

Notes

1:		M = Mandatory

## 7.3	MulteFire OSU Server Certificates

This section specifies MulteFire-specific adaptations of constraints on fields in a Hotspot 2.0 OSU Server certificate defined in Section 3.4 of Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications [4].

An OSU server certificate:

- Shall contain the OSU server FQDN as a DNSName type in the subjectAltName field.

- Shall contain a Friendly Name field.

- The UTF-8 encoded Friendly Name shall be an otherName sequence to the subjectAltName and shall be encoded with an ASN.1 type of UTF8String. If multiple Friendly Name values are required (same PSP, multiple human languages), then multiple otherName fields are present in the certificate. The type-id of the otherName shall be an id-mfa-psp-friendlyName. The id-mfa-psp-friendlyName contains exactly 1 language code and Friendly Name for a PSP. In the case where a Friendly Name is to be included in more than one human language, there shall be as many id-mfa-psp-friendlyName objects as there are human languages included. The payload of the id-mfa-psp-friendlyName is the concatenation of the Language Code and Friendly Name. The format of the Language Code and the Friendly Name is defined in MFA TS MF.202 [6]. For example, for two human languages the following Friendly Names would be included as two separate otherName objects in the SubjectAltName: “engMulteFire Alliance” and “fraMultefire Alliance”.

Table 7.3.-1 lists the MulteFire-specific adaptations of constraints on fields in a Hotspot 2.0 OSU Server certificate defined in Table 4 of Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications [4].

Table 7.3.-1: MulteFire OSU Server Certificate

| Selected Certificate Fields | Profile Specific Constraints HS 2.0 (For reference) | Profile Specific Constraints MulteFire | M/O | ASN.1 Type & OIDs |
| --- | --- | --- | --- | --- |
| subject | The "subject" name shall set the "CN" to the FQDN of the server. The "OU" portion of the name shall be set to "Hotspot 2.0 Online Sign Up Server". If there are multiple FQDNs in the subjectAltName field the certificate requester should choose which one is put in the subject common name. | The "subject" name shall set the "CN" to the FQDN of the server. The "OU" portion of the name shall be set to "Multefire Online Sign Up Server". If there are multiple FQDNs in the subjectAltName field the certificate requester should choose which one is put in the subject common name. | M | Name |
| logotype | The "logotype" with critical set to FALSE and contains the "HashAlgAndValue" for a logo that is associated with the service provider. Note – RFC 3709 specified inclusion of SHA-1 hash values are not required. | Not used in MulteFire | C See Note | mediaType IA5String logotypeHash HashAlgAndValue logotypeURI IA5String LogotypeImageInfo |

Note:	The use of Logotype is mandatory in Hotspot 2.0 OSU server certificate and optional in MulteFire OSU server certificate. When present in MulteFire OSU server certificate, its validation by the mobile device may be skipped.

## 7.4	MulteFire Device Certificates

This section specifies the format of MulteFire device certificate. It uses a Hotspot 2.0 OSU Server certificate as a baseline, as defined in Section 3.4 of Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications [4].

Table 7.4-1 describes the MulteFire-specific adaptations for device certificates compared to the Hotspot 2.0 OSU Server certificate defined in Table 4 of Hotspot 2.0 (Release 2) Online Sign Up Certificate Policy Specifications [4].

Table 7.4-1: MulteFire Device Certificate

| Selected Certificate Fields | Profile Specific Constraints HS 2.0 OSU Server Certificates (For reference) | Profile Specific Constraints MulteFire Device Certificates | M/O | ASN.1 Type & OIDs |
| --- | --- | --- | --- | --- |
| Validity | The validity period should be 2 years from the issuing date. UTCTime values shall be used. | The validity period should be 10 years from the issuing date since the device certificate shall be valid for the lifetime of the device. UTCTime values shall be used. | M | SEQUENCE {notBefore Time, notAfter Time } |
| subject | The "subject" name shall set the "CN" to the FQDN of the server. The "OU" portion of the name shall be set to "Hotspot 2.0 Online Sign Up Server". If there are multiple FQDNs in the subjectAltName field the certificate requester should choose which one is put in the subject common name. | The "subject" name shall set the "CN" to a device FQDN with the format: <IMEI number of device>.IMEI.<vendor domain name> The "OU" portion of the name shall be set to "MulteFire Device Certificate". | M | Name |
| subjectPublicKeyInfo | The "algorithm" subfield of "subjectPublicKeyInfo" shall be RSA (OID 1.2.840.113549.1.1.1) | Any algorithm may be used which supports "extKeyUsage" of "Client Authentication". | M | SEQUENCE {algorithm AlgorithmIdentifier, subjectPublicKey BIT STRING } |
| extKeyUsage | "extKeyUsage" is critical and shall set "KeyPurposeId" to "Server Authentication". | "extKeyUsage" is critical and shall set "KeyPurposeId" to "Client Authentication". | M |  |
| subjectAltName | The "subjectAltName" is critical and shall contain one or more Friendly Names that identify the service provider. The Friendly Names shall be in UTF-8 format and encoded with an ASN.1 type of UTF8String. The "subjectAltName" field shall include one or more "dNSName" values that provide the FQDN of the server. | The subjectAltName with type dNSName shall contain the same FQDN as the subject field. Exactly one instance of subjectAltName shall be present. | M |  |
| logotype | The "logotype" with critical set to FALSE and contains the "HashAlgAndValue" for a logo that is associated with the service provider. Note – RFC 3709 specified inclusion of SHA-1 hash values are not required. | Not used for MulteFire device certificates. | C See Note | mediaType IA5String logotypeHash HashAlgAndValue logotypeURI IA5String LogotypeImageInfo |

Note:	The use of Logotype is mandatory in a Hotspot 2.0 OSU server certificate and optional in a MulteFire device certificate. When present in MulteFire device certificate, its validation by the OSU Server or AAA Server may be skipped.

# Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2016-09 | 6 | Mf2016.447 |  | 02 | MulteFire Subscription Profile MO, SDP and OSU (Stage-3) |  | 0.0.0 |
| 2016-10 | 7 | Mf2016.447 |  | 03 | Includes the following CRs: mf2016.769.01; mf2016.801.00 | 0.0.0 | 0.0.1 |
| 2016-10 | 7 | Mf2016.447 |  | 03 | Renumbered list of References, aligned references throughout the document with the list. | 0.0.0 | 0.0.1 |
| 2016-10 | 7 | Mf2016.447 |  | 03 | Added Annex A: Change History; Added Table of Contents. | 0.0.0 | 0.0.1 |
| 2016-10 | 7 | Mf2016.447 |  | 04 | Editorial corrections | 0.0.1 | 0.0.2 |
| 2016-10 | 7 | Mf2016.447 |  | 05 | Title page added | 0.0.2 | 0.0.3 |
| 2017-01 | 8 | Mf2016.1019 |  | 06 | Editorial corrections and updates to references throughout specification. | 1.0.0 | 1.0.1 |
| 2017-02 | 9 | Mf2017.062 |  | 07 | Updates approved at TSG meeting 9; initially approved and compiled in Mf2017.062.00 | 1.0.1 | 1.0.2 |