MFA TS 36.101 V1.0.0 (2017-12)

Technical Specification

MulteFire Alliance;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

User Equipment (UE) radio transmission and reception

(Release 1.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

MF, multefire, radio, release 1.0

MulteFire Alliance

Postal address

5177 Brandin Court

Fremont, CA 94538 USA

Internet

http://www.multefire.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, MulteFire Alliance

This specification is a modification of 3GPP TS 36.101, V14.3.0 © 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC) from which permission has been received.

All rights reserved.

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	26

1	Scope	27

2	References	28

3	Definitions, symbols and abbreviations	29

3.1	Definitions	29

3.2	Symbols	30

3.3	Abbreviations	33

4	General	35

4.1	Relationship between minimum requirements and test requirements	35

4.2	Applicability of minimum requirements	35

4.3	Void	35

4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category 1bis, UE category NB1 and NB2, V2X Communication)	35

4.4	RF requirements in later releases	37

5	Operating bands and channel arrangement	38

5.1	General	38

5.2	Void	38

5.3	Void	38

5.4	Void	38

5.5	Operating bands	38

5.5A	Operating bands for CA	40

5.5B	Operating bands for UL-MIMO	49

5.5C	Operating bands for Dual Connectivity	49

5.5D	Operating bands for ProSe	51

5.5E	Operating bands for UE category 0, UE category M1 and UE category 1bis	51

5.5F	Operating bands for category NB1 and NB2	51

5.5G	Operating bands for V2X Communication	52

5.6	Channel bandwidth	52

5.6.1	Channel bandwidths per operating band	53

5.6A	Channel bandwidth for CA	55

5.6A.1	Channel bandwidths per operating band for CA	57

5.6B	Channel bandwidth for UL-MIMO	106

5.6B.1	Void	106

5.6C	Channel bandwidth for Dual Connectivity	106

5.6C.1	Void	106

5.6D	Channel bandwidth for ProSe	106

5.6D.1	Channel bandwidths per operating band for ProSe	106

5.6F	Channel bandwidth for category NB1 and NB2	107

5.6G	Channel bandwidth for V2X Communication	108

5.6G.1	Channel bandwidths per operating band for V2X Communication	108

5.7	Channel arrangement	109

5.7.1	Channel spacing	109

5.7.1A	Channel spacing for CA	110

5.7.1F	Channel spacing for category NB1 and NB2	110

5.7.2	Channel raster	110

5.7.2A	Channel raster for CA	110

5.7.2F	Channel raster for category NB1 and NB2	110

5.7.3	Carrier frequency and EARFCN	110

5.7.3F	Carrier frequency and EARFCN for category NB1 and NB2	113

5.7.4	TX–RX frequency separation	114

5.7.4A	TX–RX frequency separation for CA	114

5.7.4E	TX–RX frequency separation for category M1	115

5.7.4F	TX–RX frequency separation for category NB1 and NB2	115

6	Transmitter characteristics	116

6.1	General	116

6.2	Transmit power	116

6.2.1	Void	116

6.2.2	UE maximum output power	116

6.2.2A	UE maximum output power for CA	118

6.2.2B	UE maximum output power for UL-MIMO	120

6.2.2C	 Void	122

6.2.2D	UE maximum output power for ProSe	122

6.2.2E	UE maximum output power for Category M1 UE	122

6.2.2F	UE maximum output power for category NB1 and NB2	123

6.2.2G	UE maximum output power for V2X Commincation	124

6.2.3	UE maximum output power for modulation / channel bandwidth	125

6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA	126

6.2.3B	UE maximum output power for modulation / channel bandwidth for UL-MIMO	128

6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe	128

6.2.3E	UE maximum output power for modulation / channel bandwidth for category M1	129

6.2.3F	UE maximum output power for modulation / channel bandwidth for category NB1 and NB2	129

6.2.3G	UE maximum output power for modulation / channel bandwidth for V2X Communication	130

6.2.3G.1	MPR for Power class 3 V2X UE	130

6.2.3G.2	MPR for Power class 2 V2X UE	131

6.2.4	UE maximum output power with additional requirements	132

6.2.4A	UE maximum output power with additional requirements for CA	145

6.2.4A.1	A-MPR for CA_NS_01 for CA_1C	146

6.2.4A.2	A-MPR for CA_NS_02 for CA_1C	147

6.2.4A.3	A-MPR for CA_NS_03 for CA_1C	148

6.2.4A.4	A-MPR for CA_NS_04	148

6.2.4A.5	A-MPR for CA_NS_05 for CA_38C	149

6.2.4A.6	A-MPR for CA_NS_06	150

6.2.4A.7	A-MPR for CA_NS_07	151

6.2.4A.8	A-MPR for CA_NS_08	152

6.2.4B	UE maximum output power with additional requirements for UL-MIMO	153

6.2.4D	UE maximum output power with additional requirements for ProSe	154

6.2.4E	UE maximum output power with additional requirements for category M1 UE	154

6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE	155

6.2.4G	UE maximum output power with additional requirements for V2X Communication	155

6.2.5	Configured transmitted power	157

6.2.5A	Configured transmitted power for CA	190

6.2.5B	Configured transmitted power for UL-MIMO	192

6.2.5C	Configured transmitted power for Dual Connectivity	193

6.2.5D	Configured transmitted power for ProSe	194

6.2.5F	Configured transmitted Power for category NB1 and NB2	196

6.2.5G	Configured transmitted power for V2X Communication	197

6.3	Output power dynamics	200

6.3.1	(Void)	200

6.3.2	Minimum output power	200

6.3.2.1	Minimum requirement	200

6.3.2A	UE Minimum output power for CA	200

6.3.2A.1	Minimum requirement for CA	200

6.3.2B	UE Minimum output power for UL-MIMO	201

6.3.2B.1	Minimum requirement	201

6.3.2C	Void	201

6.3.2D	UE Minimum output power for ProSe	201

6.3.2F	UE Minimum output power for category NB1 and NB2	201

6.3.2G	UE Minimum output power for V2X Communication	202

6.3.3	Transmit OFF power	202

6.3.3.1	Minimum requirement	202

6.3.3A	UE Transmit OFF power for CA	202

6.3.3A.1	Minimum requirement for CA	202

6.3.3B	UE Transmit OFF power for UL-MIMO	203

6.3.3B.1	Minimum requirement	203

6.3.3D	Transmit OFF power for ProSe	203

6.3.3F	Transmit OFF power for category NB1 and NB2	204

6.3.3G	Transmit OFF power for V2X Communication	204

6.3.4	ON/OFF time mask	204

6.3.4.1	General ON/OFF time mask	204

6.3.4.2	PRACH and SRS time mask	205

6.3.4.2.1	PRACH time mask	205

6.3.4.2.2	SRS time mask	205

6.3.4.3	Slot / Sub frame boundary time mask	206

6.3.4.4	PUCCH / PUSCH / SRS time mask	207

6.3.4A	ON/OFF time mask for CA	209

6.3.4B	ON/OFF time mask for UL-MIMO	210

6.3.4D	ON/OFF time mask for ProSe	210

6.3.4D.1	General time mask for ProSe	210

6.3.4D.2	PSSS/SSSS time mask	210

6.3.4D.3	PSSS / SSSS / PSBCH time mask	211

6.3.4D.4	PSSCH / SRS time mask	212

6.3.4F	ON/OFF time mask for category NB1 and NB2	212

6.3.4F.1	General ON/OFF time mask	212

6.3.4F.2	NPRACH time mask	213

6.3.4G	ON/OFF time mask for V2X Communication	213

6.3.4G.1	PSSS / SSSS / PSBCH time mask	213

6.3.5	Power Control	214

6.3.5.1	Absolute power tolerance	214

6.3.5.1.1	Minimum requirements	214

6.3.5.2	Relative Power tolerance	215

6.3.5.2.1	Minimum requirements	215

6.3.5.3	Aggregate power control tolerance	216

6.3.5.3.1	Minimum requirement	216

6.3.5A	Power control for CA	216

6.3.5A.1	Absolute power tolerance	216

6.3.5A.1.1	Minimum requirements	216

6.3.5A.2	Relative power tolerance	217

6.3.5A.2.1	Minimum requirements	217

6.3.5A.3	Aggregate power control tolerance	217

6.3.5A.3.1	Minimum requirements	217

6.3.5B	Power control for UL-MIMO	218

6.3.5D	Power Control for ProSe	218

6.3.5D.1	Absolute power tolerance	218

6.3.5E	Power control for category M1	218

6.3.5E.1	Absolute power tolerance	218

6.3.5E.2	Relative Power tolerance	218

6.3.5E.3	Aggregate power control tolerance	219

6.3.5E.3.1	Minimum requirement	219

6.3.5F	Power Control for category NB1 and NB2	219

6.3.5F.1	Absolute power tolerance	219

6.3.5F.2	Relative power tolerance	220

6.3.5F.3	Aggregate power control tolerance for category NB1 and NB2	220

6.3.5F.3.1	Minimum requirement	220

6.3.5G	Power Control for V2X Communication	220

6.3.5G.1	Absolute power tolerance	221

6.4	Void	221

6.5	Transmit signal quality	221

6.5.1	Frequency error	221

6.5.1A 	Frequency error for CA	221

6.5.1B	Frequency error for UL-MIMO	221

6.5.1D	Frequency error for ProSe	221

6.5.1E	Frequency error for UE category M1	221

6.5.1F	Frequency error for UE category NB1 and NB2	222

6.5.1G	Frequency error for V2X Communication	222

6.5.2	Transmit modulation quality	222

6.5.2.1	Error Vector Magnitude	222

6.5.2.1.1	Minimum requirement	223

6.5.2.2	Carrier leakage	223

6.5.2.2.1	Minimum requirements	223

6.5.2.3	In-band emissions	224

6.5.2.3.1	Minimum requirements	224

6.5.2.4	EVM equalizer spectrum flatness	225

6.5.2.4.1	Minimum requirements	225

6.5.2A	Transmit modulation quality for CA	226

6.5.2A.1	Error Vector Magnitude	226

6.5.2A.2	Carrier leakage for CA	227

6.5.2A.2.1	Minimum requirements	227

6.5.2A.3	In-band emissions	227

6.5.2A.3.1	Minimum requirement for CA	227

6.5.2B	Transmit modulation quality for UL-MIMO	229

6.5.2B.1	Error Vector Magnitude	230

6.5.2B.2	Carrier leakage	230

6.5.2B.3	In-band emissions	230

6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO	230

6.5.2D	Transmit modulation quality for ProSe	230

6.5.2D.1	Error Vector Magnitude	230

6.5.2D.2	Carrier leakage	230

6.5.2D.3	In-band emissions	231

6.5.2D.4	EVM equalizer spectrum flatness for ProSe	231

6.5.2E	Transmit modulation quality for category M1	231

6.5.2E.1	Error Vector Magnitude	231

6.5.2E.2	Carrier leakage	231

6.5.2E.2.1	Minimum requirements	231

6.5.2E.3	In-band emissions	231

6.5.2E.3.1	Minimum requirements	231

6.5.2F	Transmit modulation quality for Category NB1 and NB2	232

6.5.2F.1	Error Vector Magnitude	232

6.5.2F.2	Carrier leakage	232

6.5.2F.3	In-band emissions	232

6.5.2G	Transmit modulation quality for V2X Communication	233

6.5.2G.1	Error Vector Magnitude	233

6.5.2G.2	Carrier leakage	233

6.5.2G.3	In-band emissions	234

6.5.2G.4	EVM equalizer spectrum flatness	234

6.6	Output RF spectrum emissions	234

6.6.1	Occupied bandwidth	234

6.6.1.1	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	235

6.6.1A	Occupied bandwidth for CA	235

6.6.1B	Occupied bandwidth for UL-MIMO	235

6.6.1F	Occupied bandwidth for category NB1 and NB2	235

6.6.1G	Occupied bandwidth for V2X Communication	236

6.6.2	Out of band emission	236

6.6.2.1	Spectrum emission mask	236

6.6.2.1.1	Minimum requirement	236

6.6.2.1A	Spectrum emission mask for CA	236

6.6.2.2	Additional spectrum emission mask	237

6.6.2.2.1	Minimum requirement (network signalled value "NS_03", “NS_11”, "NS_20", and “NS_21”)	238

6.6.2.2.2	Minimum requirement (network signalled value "NS_04")	238

6.6.2.2.3	Minimum requirement (network signalled value "NS_06" or “NS_07”)	239

6.6.2.2.4	Minimum requirement (network signalled value "NS_XX")	239

6.6.2.2.5	Minimum requirement (network signalled value “NS_27”)	240

6.6.2.2.6	Minimum requirement (network signalled value "NS_28”)	240

6.6.2.2A	Additional Spectrum Emission Mask for CA	240

6.6.2.2A.1	Minimum requirement (network signalled value "CA_NS_04")	240

6.6.2.3	Adjacent Channel Leakage Ratio	241

6.6.2.3.1	Minimum requirement E-UTRA	241

6.6.2.3.1a	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	242

6.6.2.3.1A	Void	243

6.6.2.3.1Aa	Void	243

6.6.2.3.2	Minimum requirements UTRA	243

6.6.2.3.2A	Minimum requirement UTRA for CA	243

6.6.2.3.3A	Minimum requirements for CA E-UTRA	245

6.6.2.4	Void	246

6.6.2.4.1	Void	246

6.6.2A	Void	246

6.6.2B	Out of band emission for UL-MIMO	246

6.6.2C	Void	246

6.6.2D	Out of band emission for ProSe	246

6.6.2F	Out of band emission for category NB1 and NB2	247

6.6.2F.1	Spectrum emission mask	247

6.6.2F.2	Void	247

6.6.2F.3	Adjacent Channel Leakage Ratio for category NB1 and NB2	247

6.6.2G	Out of band emission for V2X Communication	248

6.6.3	Spurious emissions	248

6.6.3.1	Minimum requirements	249

6.6.3.1A	Minimum requirements for CA	249

6.6.3.2	Spurious emission band UE co-existence	250

6.6.3.2A	Spurious emission band UE co-existence for CA	259

6.6.3.3	Additional spurious emissions	271

6.6.3.3.1	Minimum requirement (network signalled value "NS_05")	271

6.6.3.3.2	Minimum requirement (network signalled value “NS_07”)	271

6.6.3.3.3	Minimum requirement (network signalled value “NS_08”)	272

6.6.3.3.4	Minimum requirement (network signalled value “NS_09”)	272

6.6.3.3.5	Minimum requirement (network signalled value "NS_12")	272

6.6.3.3.6	Minimum requirement (network signalled value “NS_13”)	273

6.6.3.3.7	Minimum requirement (network signalled value “NS_14”)	273

6.6.3.3.8	Minimum requirement (network signalled value “NS_15”)	273

6.6.3.3.9	Minimum requirement (network signalled value “NS_16”)	273

6.6.3.3.10	Minimum requirement (network signalled value “NS_17”)	274

6.6.3.3.11	Minimum requirement (network signalled value “NS_18”)	274

6.6.3.3.12	Minimum requirement (network signalled value “NS_19”)	274

6.6.3.3.13	Minimum requirement (network signalled value “NS_11”)	274

6.6.3.3.14	Minimum requirement (network signalled value “NS_20”)	275

6.6.3.3.15	Minimum requirement (network signalled value “NS_21”)	275

6.6.3.3.16	Minimum requirement (network signalled value "NS_22")	275

6.6.3.3.17	Minimum requirement (network signalled value “NS_23”)	276

6.6.3.3.18	Void	276

6.6.3.3.19	Minimum requirement (network signalled value "NS_04")	276

6.6.3.3.20	Minimum requirement (network signalled value “NS_24”)	277

6.6.3.3.21	Minimum requirement (network signalled value “NS_25”)	277

6.6.3.3.22	Minimum requirement (network signalled value “NS_26”)	277

6.6.3.3.23	Minimum requirement (network signalled value “NS_27”)	278

6.6.3.3.24	Minimum requirement (network signalled value “NS_28”)	278

6.6.3.3.25	Minimum requirement (network signalled value “NS_29”)	278

6.6.3.3.26	Minimum requirement (network signalled value “NS_30”)	279

6.6.3.3.27	Minimum requirement (network signalled value “NS_31”)	280

6.6.3.3A	Additional spurious emissions for CA	281

6.6.3.3A.1	Minimum requirement for CA_1C (network signalled value "CA_NS_01")	281

6.6.3.3A.2 	Minimum requirement for CA_1C (network signalled value "CA_NS_02")	281

6.6.3.3A.3	Minimum requirement for CA_1C (network signalled value "CA_NS_03")	281

6.6.3.3A.4	Minimum requirement for CA_38C (network signalled value "CA_NS_05")	282

6.6.3.3A.5	Minimum requirement for CA_7C (network signalled value "CA_NS_06")	282

6.6.3.3A.6	Minimum requirement for CA_39C and CA_39C-41A (network signalled value "CA_NS_07")	282

6.6.3.3A.7	Minimum requirement for CA_42C (network signalled value "CA_NS_08")	283

6.6.3A	Void	283

6.6.3B	Spurious emission for UL-MIMO	283

6.6.3C	Void	283

6.6.3D	Spurious emission for ProSe	283

6.6.3F	Spurious emission for category NB1 and NB2	284

6.6.3G	Spurious emission for V2X Communication	284

6.6A	Void	284

6.6B	Void	284

6.7	Transmit intermodulation	284

6.7.1	Minimum requirement	284

6.7.1A	Minimum requirement for CA	285

6.7.1B	Minimum requirement for UL-MIMO	285

6.7.1F	Minimum requirement for category NB1 and NB2	286

6.7.1G	Minimum requirement for V2X Communication	286

6.8	Void	286

6.8.1	Void	286

6.8A	Void	286

6.8B	Time alignment error for UL-MIMO	286

6.8B.1	Minimum Requirements	286

7	Receiver characteristics	287

7.1	General	287

7.2	Diversity characteristics	287

7.3	Reference sensitivity power level	288

7.3.1	Minimum requirements (QPSK)	288

7.3.1A	Minimum requirements (QPSK) for CA	318

7.3.1B	Minimum requirements (QPSK) for UL-MIMO	381

7.3.1D	Minimum requirements (QPSK) for ProSe	381

7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, and 1bis	383

7.3.1F	Minimum requirements for UE category NB1 and NB2	388

7.3.1F.1	Reference sensitivity for UE category NB1 and NB2	388

7.3.1F.2	Sensitivity with repetitions for UE category NB1 and NB2	388

7.3.1G	Minimum requirements (QPSK) for V2X	389

7.3.2	Void	390

7.4	Maximum input level	390

7.4.1	Minimum requirements	390

7.4.1A	Minimum requirements for CA	391

7.4.1B	Minimum requirements for UL-MIMO	392

7.4.1D	Minimum requirements for ProSe	392

7.4.1F	Minimum requirements for category NB1 and NB2	392

7.4.1G	Minimum requirements for V2X	393

7.4A	Void	393

7.4A.1	Void	393

7.5	Adjacent Channel Selectivity (ACS)	393

7.5.1	Minimum requirements	393

7.5.1A	Minimum requirements for CA	394

7.5.1B	Minimum requirements for UL-MIMO	398

7.5.1D	Minimum requirements for ProSe	398

7.5.1F	Minimum requirements for category NB1 and NB2	399

7.5.1G	Minimum requirements for V2X	400

7.6	Blocking characteristics	402

7.6.1	In-band blocking	402

7.6.1.1	Minimum requirements	402

7.6.1.1A	Minimum requirements for CA	403

7.6.1.1D	Minimum requirements for ProSe	407

7.6.1.1F	Minimum requirements for category NB1 and NB2	408

7.6.1.1G	Minimum requirements for V2X	409

7.6.2	Out-of-band blocking	411

7.6.2.1	Minimum requirements	411

7.6.2.1A	Minimum requirements for CA	412

7.6.2.1D	Minimum requirements for ProSe	415

7.6.2.1F	Minimum requirements for category NB1 and NB2	416

7.6.2.1G	Minimum requirements for V2X	416

7.6.3	Narrow band blocking	418

7.6.3.1	Minimum requirements	418

7.6.3.1A	Minimum requirements for CA	418

7.6.3.1D	Minimum requirements for ProSe	419

7.6A	Void	420

7.6B	Blocking characteristics for UL-MIMO	420

7.7	Spurious response	420

7.7.1	Minimum requirements	420

7.7.1A	Minimum requirements for CA	421

7.7.1B	Minimum requirements for UL-MIMO	422

7.7.1D	Minimum requirements for ProSe	422

7.7.1F	Minimum requirements for UE category NB1 and NB2	423

7.7.1G	Minimum requirements for V2X	423

7.8	Intermodulation characteristics	424

7.8.1	Wide band intermodulation	424

7.8.1.1	Minimum requirements	424

7.8.1A	Minimum requirements for CA	425

7.8.1B	Minimum requirements for UL-MIMO	427

7.8.1D	Minimum requirements for ProSe	427

7.8.1F	Minimum requirements for category NB1 and NB2	428

7.8.1G	Minimum requirements	428

7.8.2	Void	429

7.9	Spurious emissions	429

7.9.1	Minimum requirements	430

7.9.1A	Minimum requirements	430

7.10	Receiver image	430

7.10.1	Void	430

7.10.1A	Minimum requirements for CA	430

7.10.1G	Minimum requirements for V2X Communication	431

8	Performance requirement	432

8.1	General	432

8.1.1	Receiver antenna capability	432

8.1.1.1	Simultaneous unicast and MBMS operations	433

8.1.1.2	Dual-antenna receiver capability in idle mode	433

8.1.2	Applicability of requirements	433

8.1.2.1	Applicability of requirements for different channel bandwidths	433

8.1.2.2	Definition of CA capability	433

8.1.2.2A	Definition of dual connectivity capability	436

8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets	437

8.1.2.3A	Applicability and test rules for different dual connectivity configuration and bandwidth combination set	439

8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	440

8.1.2.3C	Applicability and test rules for SDR tests for different CA/DC configurations and bandwidth combination sets	442

8.1.2.4	Test coverage for different number of component carriers	442

8.1.2.5	Applicability of performance requirements for Type B receiver	443

8.1.2.6	Applicability of performance requirements for 4Rx capable UEs	443

8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx	443

8.1.2.6.2	Applicability rule and antenna connection for CA and DC tests with 2Rx	445

8.1.2.6.3	Applicability rule and antenna connection for single carrier tests with 4Rx	445

8.1.2.6.4	Applicability rule for 256QAM tests	445

8.1.2.6.5	Applicability rule and antenna connection for CA and DC tests with 4Rx	446

8.1.2.7	Applicability of Enhanced Downlink Control Channel Performance Requirements	449

8.1.2.8	Applicability of performance requirements for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations	450

8.1.2.9	Applicability of SDR requirements for CA and LAA	450

8.1.3	UE category and UE DL category	450

8.2	Demodulation of PDSCH (Cell-Specific Reference Symbols)	451

8.2.1	FDD (Fixed Reference Channel)	451

8.2.1.1	Single-antenna port performance	451

8.2.1.1.1	Minimum Requirement	451

8.2.1.1.2	Void	456

8.2.1.1.3	Void	456

8.2.1.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	456

8.2.1.2	Transmit diversity performance	457

8.2.1.2.1	Minimum Requirement 2 Tx Antenna Port	457

8.2.1.2.2	Minimum Requirement 4 Tx Antenna Port	457

8.2.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	458

8.2.1.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	460

8.2.1.2.4	Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	462

8.2.1.2.5	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	464

8.2.1.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	465

8.2.1.3	Open-loop spatial multiplexing performance	466

8.2.1.3.1	Minimum Requirement 2 Tx Antenna Port	466

8.2.1.3.1A	Soft buffer management test	469

8.2.1.3.1B	Enhanced Performance Requirement Type C –2Tx Antenna Ports	471

8.2.1.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	471

8.2.1.3.2	Minimum Requirement 4 Tx Antenna Port	472

8.2.1.3.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	473

8.2.1.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	477

8.2.1.4	Closed-loop spatial multiplexing performance	479

8.2.1.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	479

8.2.1.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	480

8.2.1.4.1B	Enhanced Performance Requirement Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	480

8.2.1.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	482

8.2.1.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	485

8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	487

8.2.1.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	488

8.2.1.4.2A	Enhanced Performance Requirement Type C – Multi-layer Spatial Multiplexing 2Tx Antenna Ports	488

8.2.1.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	489

8.2.1.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	492

8.2.1.5	MU-MIMO	494

8.2.1.6	[Control channel performance: D-BCH and PCH]	494

8.2.1.7	Carrier aggregation with power imbalance	494

8.2.1.7.1	Minimum Requirement	494

8.2.1.8	Intra-band non-contiguous carrier aggregation with timing offset	495

8.2.1.8.1	Minimum Requirement	495

8.2.1.9	HST-SFN performance	496

8.2.1.9.1	Minimum Requirement	496

8.2.2	TDD (Fixed Reference Channel)	497

8.2.2.1	Single-antenna port performance	497

8.2.2.1.1	Minimum Requirement	497

8.2.2.1.2	Void	501

8.2.2.1.3	Void	501

8.2.2.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	501

8.2.2.2	Transmit diversity performance	502

8.2.2.2.1	Minimum Requirement 2 Tx Antenna Port	502

8.2.2.2.2	Minimum Requirement 4 Tx Antenna Port	502

8.2.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	503

8.2.2.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	505

8.2.2.2.4	Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	507

8.2.2.2.5	Minimum Requirement 2 Tx Antenna Port (when EIMTA-MainConfigServCell-r12 is configured)	509

8.2.2.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	509

8.2.2.2.7	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	511

8.2.2.3	Open-loop spatial multiplexing performance	512

8.2.2.3.1	Minimum Requirement 2 Tx Antenna Port	512

8.2.2.3.1A	Soft buffer management test	514

8.2.2.3.1B	Enhanced Performance Requirement Type C - 2Tx Antenna Ports	515

8.2.2.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	515

8.2.2.3.2	Minimum Requirement 4 Tx Antenna Port	516

8.2.2.3.3	Minimum Requirement 2Tx antenna port (demodulation subframe overlaps with aggressor cell ABS)	517

8.2.2.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	521

8.2.2.4	 Closed-loop spatial multiplexing performance	523

8.2.2.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	523

8.2.2.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	524

8.2.2.4.1B	Enhanced Performance Requirement Type A – Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	524

8.2.2.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	526

8.2.2.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	528

8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	530

8.2.2.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	531

8.2.2.4.2A	Enhanced Performance Requirement Type C Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	532

8.2.2.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	533

8.2.2.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	536

8.2.2.4.4	Void	538

8.2.2.5	MU-MIMO	538

8.2.2.6	[Control channel performance: D-BCH and PCH]	538

8.2.2.7	Carrier aggregation with power imbalance	538

8.2.2.7.1	Minimum Requirement	538

8.2.2.8	Intra-band contiguous carrier aggregation with minimum channel spacing	539

8.2.2.8.1	Minimum Requirement	539

8.2.2.9	HST-SFN performance	540

8.2.2.9.1	Minimum Requirement	540

8.2.3	TDD FDD CA (Fixed Reference Channel)	541

8.2.3.1	Single-antenna port performance	542

8.2.3.1.1	Minimum Requirement for FDD PCell	542

8.2.3.1.2	Minimum Requirement for TDD PCell	545

8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port	548

8.2.3.2.1	Minimum Requirement for FDD PCell	548

8.2.3.2.1A	Soft buffer management test for FDD PCell	551

8.2.3.2.2	Minimum Requirement for TDD PCell	552

8.2.3.2.2A	Soft buffer management test for TDD PCell	555

8.2.3.3	 Closed-loop spatial multiplexing performance 4Tx Antenna Port	556

8.2.3.3.1	Minimum Requirement for FDD PCell	556

8.2.3.3.2	Minimum Requirement for TDD PCell	559

8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity	562

8.2.4	LAA	564

8.2.4.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port	564

8.2.4.1.1	FDD PCell (FDD single carrier)	564

8.2.4.1.2	TDD PCell (TDD single carrier)	566

8.3	Demodulation of PDSCH (User-Specific Reference Symbols)	569

8.3.1	FDD	569

8.3.1.1	Single-layer Spatial Multiplexing	570

8.3.1.1A	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	571

8.3.1.1B	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	574

8.3.1.1C	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference model	577

8.3.1.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	579

8.3.1.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference model	580

8.3.1.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	581

8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)	583

8.3.1.1H	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	585

8.3.1.2	Dual-Layer Spatial Multiplexing	586

8.3.1.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	588

8.3.1.3	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	589

8.3.1.3.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	589

8.3.1.3.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	591

8.3.1.3.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	593

8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	594

8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	596

8.3.2	TDD	598

8.3.2.1	Single-layer Spatial Multiplexing	599

8.3.2.1A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	601

8.3.2.1B	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	602

8.3.2.1C	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	604

8.3.2.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference	606

8.3.2.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	607

8.3.2.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference	608

8.3.2.1G	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	609

8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)	611

8.3.2.1I	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	612

8.3.2.2	Dual-Layer Spatial Multiplexing	613

8.3.2.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	614

8.3.2.3	Dual-Layer Spatial Multiplexing (with multiple CSI-RS configurations)	615

8.3.2.4	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	616

8.3.2.4.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	616

8.3.2.4.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	618

8.3.2.4.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	620

8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	621

8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	623

8.3.3	LAA	626

8.3.3.1	Dual-Layer Spatial Multiplexing with DM-RS	626

8.3.3.1.1	FDD PCell (FDD single carrier)	626

8.3.3.1.2	TDD Pcell (TDD single carrier)	629

8.4	Demodulation of PDCCH/PCFICH	632

8.4.1	FDD	632

8.4.1.1	Single-antenna port performance	633

8.4.1.2	Transmit diversity performance	633

8.4.1.2.1	Minimum Requirement 2 Tx Antenna Port	633

8.4.1.2.2	Minimum Requirement 4 Tx Antenna Port	633

8.4.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	634

8.4.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	639

8.4.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port under Asynchronous Network	645

8.4.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	646

8.4.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	647

8.4.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	648

8.4.2	TDD	649

8.4.2.1	Single-antenna port performance	650

8.4.2.2	Transmit diversity performance	650

8.4.2.2.1	Minimum Requirement 2 Tx Antenna Port	650

8.4.2.2.2	Minimum Requirement 4 Tx Antenna Port	651

8.4.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	651

8.4.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	655

8.4.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	659

8.4.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	660

8.4.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	661

8.4.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	662

8.4.3	LAA	663

8.4.3.1	Transmit diversity performance	663

8.4.3.1.1	FDD Pcell (FDD single carrier)	663

8.4.3.1.2	TDD Pcell (TDD single carrier)	664

8.5	Demodulation of PHICH	665

8.5.1	FDD	666

8.5.1.1	Single-antenna port performance	666

8.5.1.2	Transmit diversity performance	666

8.5.1.2.1	Minimum Requirement 2 Tx Antenna Port	666

8.5.1.2.2	Minimum Requirement 4 Tx Antenna Port	667

8.5.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	667

8.5.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	669

8.5.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports under Asynchronous Network	672

8.5.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	673

8.5.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	674

8.5.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	675

8.5.2	TDD	676

8.5.2.1	Single-antenna port performance	677

8.5.2.2	Transmit diversity performance	677

8.5.2.2.1	Minimum Requirement 2 Tx Antenna Port	677

8.5.2.2.2	Minimum Requirement 4 Tx Antenna Port	678

8.5.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	678

8.5.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	680

8.5.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	682

8.5.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	683

8.5.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	684

8.5.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	685

8.6	Demodulation of PBCH	686

8.6.1	FDD	686

8.6.1.1	Single-antenna port performance	686

8.6.1.2	Transmit diversity performance	687

8.6.1.2.1	Minimum Requirement 2 Tx Antenna Port	687

8.6.1.2.2	Minimum Requirement 4 Tx Antenna Port	687

8.6.1.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	687

8.6.2	TDD	689

8.6.2.1	Single-antenna port performance	689

8.6.2.2	Transmit diversity performance	689

8.6.2.2.1	Minimum Requirement 2 Tx Antenna Port	689

8.6.2.2.2	Minimum Requirement 4 Tx Antenna Port	689

8.6.2.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	690

8.6.2MF1	MF-PBCH	691

8.7	Sustained downlink data rate provided by lower layers	691

8.7.1	FDD (single carrier and CA)	691

8.7.2	TDD (single carrier and CA)	701

8.7.3	FDD (EPDCCH scheduling)	705

8.7.4	TDD (EPDCCH scheduling)	707

8.7.5	TDD FDD CA	709

8.7.5.1	Minimum Requirement FDD PCell	710

8.7.5.2	Minimum Requirement TDD PCell	714

8.7.6	FDD (DC)	719

8.7.7	TDD (DC)	726

8.7.8	TDD FDD (DC)	729

8.7.9	FDD (4 layer MIMO)	732

8.7.10	TDD (4 layer MIMO)	734

8.7.11	TDD FDD CA (4 layer MIMO)	736

8.7.11.1	Void	738

8.7.12	LAA	738

8.7.12.1	FDD CA in licensed bands	738

8.7.12.2	TDD CA in licensed bands	740

8.7.12.3	TDD-FDD CA in licensed bands	742

8.7.13	FDD (DC 4 layer MIMO)	745

8.7.14	TDD (DC 4 layer MIMO)	746

8.7.15	TDD FDD CA (DC 4 layer MIMO)	748

8.8	Demodulation of EPDCCH	750

8.8.1	Distributed Transmission	750

8.8.1.1	FDD	750

8.8.1.1.1	Void	751

8.8.1.2	TDD	751

8.8.1.2.1	Void	752

8.8.2	Localized Transmission with TM9	752

8.8.2.1	FDD	752

8.8.2.1.1	Void	753

8.8.2.1.2	Void	754

8.8.2.2	TDD	754

8.8.2.2.1	Void	755

8.8.2.2.2	Void	755

8.8.3	Localized transmission with TM10 Type B quasi co-location type	755

8.8.3.1	FDD	755

8.8.3.2	TDD	758

8.8.4	Enhanced Downlink Control Channel Performance Requirements Type A - Localized Transmission with CRS Interference Model	761

8.8.4.1	FDD	761

8.8.4.2	TDD	762

8.8.5	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM9 Interference Model	764

8.8.5.1	TDD	764

8.8.6	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM3 Interference Model	765

8.8.6.1	FDD	765

8.9	Demodulation (single receiver antenna)	766

8.9.1	PDSCH	766

8.9.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	766

8.9.1.1.1	Transmit diversity performance (Cell-Specific Reference Symbols)	766

8.9.1.1.2	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	767

8.9.1.1.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	768

8.9.1.2	TDD (Fixed Reference Channel)	769

8.9.1.2.1	Transmit diversity performance (Cell-Specific Reference Symbols)	770

8.9.1.2.2	 Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	771

8.9.1.2.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	772

8.9.2	PHICH	774

8.9.2.1	FDD and half-duplex FDD	774

8.9.2.1.1	Transmit diversity performance	774

8.9.2.2	TDD	774

8.9.2.2.1	Transmit diversity performance	774

8.9.3	PBCH	774

8.9.3.1	FDD and half-duplex FDD	774

8.9.3.1.1	Transmit diversity performance	774

8.9.3.2	TDD	775

8.9.3.2.1	Transmit diversity performance	775

8.10	Demodulation (4 receiver antenna ports)	775

8.10.1	PDSCH	775

8.10.1.1	FDD (Fixed Reference Channel)	775

8.10.1.1.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	775

8.10.1.1.1A	Transmit diversity performance wit Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	776

8.10.1.1.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	778

8.10.1.1.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	778

8.10.1.1.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (Cell-Specific Reference Symbols)	779

8.10.1.1.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	780

8.10.1.1.5A	Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	783

8.10.1.1.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	784

8.10.1.1.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	785

8.10.1.1.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	787

8.10.1.1.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas (Cell-Specific Reference Symbols)	787

8.10.1.1.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	788

8.10.1.2	TDD (Fixed Reference Channel)	789

8.10.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	790

8.10.1.2.1A	Transmit diversity performance with Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	791

8.10.1.2.2	Open-loop spatial multiplexing performance  with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	792

8.10.1.2.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	792

8.10.1.2.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	793

8.10.1.2.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	794

8.10.1.2.5A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	797

8.10.1.2.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	798

8.10.1.2.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	799

8.10.1.2.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	801

8.10.1.2.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas	801

8.10.1.2.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	802

8.10.2	PDCCH/PCFICH	803

8.10.2.1	FDD	803

8.10.2.1.1	Single-antenna port performance	804

8.10.2.1.2	Transmit diversity performance with 2 Tx Antenna Ports	804

8.10.2.1.3	Transmit diversity performance with 4 Tx Antenna Ports	804

8.10.2.2	TDD	805

8.10.2.2.1	Single-antenna port performance	805

8.10.2.2.2	Transmit diversity performance with 2 Tx Antenna Ports	805

8.10.2.2.3	Transmit diversity performance with 4 Tx Antenna Ports	806

8.10.3	PHICH	806

8.10.3.1	FDD	806

8.10.3.1.1	Single Tx Antenna Port performance	806

8.10.3.1.2	Transmit diversity performance with 2 Tx Antenna Ports	807

8.10.3.1.3	Transmit diversity performance with 4 Tx Antenna Ports	807

8.10.3.2	TDD	807

8.10.3.2.1	Single Tx Antenna Port performance	808

8.10.3.2.2	Transmit diversity performance with 2 Tx Antenna Ports	808

8.10.3.2.3	Transmit diversity performance with 4 Tx Antenna Ports	808

8.10.4	ePDCCH	809

8.10.4.1	Distributed Transmission with 4Rx	809

8.10.4.1.1	FDD	809

8.10.4.1.2	TDD	810

8.10.4.2	Localized Transmission with TM9 and 4Rx	811

8.10.4.2.1	FDD	811

8.10.4.2.2	TDD	812

8.11	Demodulation (UE supporting coverage enhancement)	813

8.11.1	PDSCH	813

8.11.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	813

8.11.1.1.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	813

8.11.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	814

8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)	816

8.11.1.2	TDD (Fixed Reference Channel)	817

8.11.1.2.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	818

8.11.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	819

8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)	821

8.11.2	MPDCCH	822

8.11.2.1	FDD and half-duplex FDD	822

8.11.2.1.1	CE Mode A	823

8.11.2.1.2	CE Mode B	823

8.11.2.2	TDD	824

8.11.2.2.1	CE Mode A	824

8.11.2.2.2	CE Mode B	825

8.11.3	PBCH	825

8.11.3.1	FDD and half-duplex FDD	825

8.11.3.1.1	Transmit diversity performance	825

8.11.3.2	TDD	826

8.11.3.2.1	Transmit diversity performance	826

8.12	Demodulation of Narrowband IoT	827

8.12.1	NPDSCH	827

8.12.1.1	Half-duplex FDD	827

8.12.1.1.1	Minimum Requirements for In-band	827

8.12.1.1.2	Minimum Requirements for Standalone/Guard-band	828

8.12.2	NPDCCH	829

8.12.2.1	Half-duplex FDD	829

8.12.2.1.1	Single-antenna performance	830

8.12.2.1.2	Transmit diversity performance	830

8.12.3	Demodulation of NPBCH	830

8.12.3.1	HD-FDD	831

8.12.3.1.1	Single-antenna port performance with single NPBCH TTI	831

8.12.3.1.2	Transmit diversity performance	831

8.13	Demodulation of PDSCH CA and DC(4 receiver antenna ports)	832

8.13.1	FDD (CA and DC)	832

8.13.1.1	Closed-loop spatial multiplexing performance	832

8.13.1.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	832

8.13.1.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	835

8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	837

8.13.1.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	837

8.13.2	TDD (CA and DC)	840

8.13.2.1	Closed-loop spatial multiplexing performance	840

8.13.2.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	840

8.13.2.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	842

8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	844

8.13.2.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	844

8.13.3	TDD-FDD CA	846

8.13.3.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port	847

8.13.3.1.1	Minimum Requirement for FDD PCell	847

8.13.3.1.2	Minimum Requirement for TDD PCell	850

8.13.3.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	853

8.13.3.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for FDD PCell	853

8.13.3.2.2	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for TDD PCell	856

9	Reporting of Channel State Information	861

9.1	General	861

9.1.1	Applicability of requirements	861

9.1.1.1	Applicability of requirements for different channel bandwidths	861

9.1.1.2	Applicability and test rules for different CA configurations and bandwidth combination sets	861

9.1.1.2A	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	862

9.1.1.3	Test coverage for different number of componenet carriers	863

9.1.1.4	Applicability of performance requirements for 4Rx capable UEs	863

9.1.1.4.1	Applicability rule and antenna connection for single carrier tests with 2Rx	863

9.1.1.4.2	Applicability rule and antenna connection for CA tests with 2Rx	865

9.1.1.4.3	Applicability rule and antenna connection for single carrier tests with 4Rx	865

9.2	CQI reporting definition under AWGN conditions	866

9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols)	866

9.2.1.1	FDD	866

9.2.1.2	TDD	867

9.2.1.3	FDD (CSI measurements in case two CSI subframe sets are configured)	868

9.2.1.4	TDD (CSI measurements in case two CSI subframe sets are configured)	870

9.2.1.5	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	872

9.2.1.6	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	874

9.2.1.7	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	876

9.2.1.8	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	877

9.2.2	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	877

9.2.2.1	FDD	877

9.2.2.2	TDD	878

9.2.3	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	879

9.2.3.1	FDD	879

9.2.3.1A	FDD (With channelMeasRestriction configured)	880

9.2.3.2	TDD	881

9.2.3.2A	TDD (With channelMeasRestriction configured)	882

9.2.4	Minimum requirement PUCCH 1-1 (With Single CSI Process)	883

9.2.4.1	FDD	884

9.2.4.1A	FDD (With interferenceMeasRestriction configured)	886

9.2.4.2	TDD	888

9.2.4.2A	TDD (With interferenceMeasRestriction configured)	890

9.2.5	Minimum requirement PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured)	892

9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	894

9.2.6.1	Frame structure type 3 with FDD Pcell	894

9.2.6.2	Frame structure type 3 with TDD Pcell	896

9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	898

9.2.7.1	Frame structure type 3 wth FDD Pcell	898

9.2.7.2	Frame structure type 3 wth TDD Pcell	900

9.3	CQI reporting under fading conditions	903

9.3.1	Frequency-selective scheduling mode	903

9.3.1.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	904

9.3.1.1.1	FDD	904

9.3.1.1.2	TDD	905

9.3.1.1.3	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	906

9.3.1.1.4	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	909

9.3.1.1.5	TDD (when csi-SubframeSet –r12 is configured)	911

9.3.1.2	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	913

9.3.1.2.1	FDD	913

9.3.1.2.2	TDD	914

9.3.1.2.3	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	916

9.3.1.2.4	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	917

9.3.1.2.5	Void	918

9.3.1.2.6	TDD (when csi-SubframeSet –r12 is configured with one CSI process)	918

9.3.2	Frequency non-selective scheduling mode	921

9.3.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	921

9.3.2.1.1	FDD	921

9.3.2.1.2	TDD	923

9.3.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	925

9.3.2.2.1	FDD	925

9.3.2.2.2	TDD	926

9.3.3	Frequency-selective interference	928

9.3.3.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbol)	928

9.3.3.1.1	FDD	928

9.3.3.1.2	TDD	929

9.3.3.2	Void	930

9.3.3.2.1	Void	930

9.3.3.2.2	Void	930

9.3.4	UE-selected subband CQI	930

9.3.4.1	Minimum requirement PUSCH 2-0 (Cell-Specific Reference Symbols)	930

9.3.4.1.1	FDD	930

9.3.4.1.2	TDD	931

9.3.4.2	Minimum requirement PUCCH 2-0 (Cell-Specific Reference Symbols)	932

9.3.4.2.1	FDD	932

9.3.4.2.2	TDD	934

9.3.5	Additional requirements for enhanced receiver Type A	936

9.3.5.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	936

9.3.5.1.1	FDD	936

9.3.5.1.2	TDD	937

9.3.5.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	939

9.3.5.2.1	FDD	939

9.3.5.2.2	TDD	941

9.3.6	Minimum requirement (With multiple CSI processes)	943

9.3.6.1	FDD	943

9.3.6.2	TDD	947

9.3.7	Minimum requirement PUSCH 3-2	950

9.3.7.1	FDD	950

9.3.7.2	TDD	951

9.3.8	Additional requirements for enhanced receiver Type B	953

9.3.8.1	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	953

9.3.8.1.1	FDD	953

9.3.8.1.2	TDD	954

9.3.8.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	956

9.3.8.2.1	FDD	956

9.3.8.2.2	TDD	958

9.3.8.3	Minimum requirement with CSI process	960

9.3.8.3.1	FDD	960

9.3.8.3.2	TDD	962

9.4	Reporting of Precoding Matrix Indicator (PMI)	964

9.4.1	Single PMI	965

9.4.1.1	Minimum requirement PUSCH 3-1 (Cell-Specific Reference Symbols)	965

9.4.1.1.1	FDD	965

9.4.1.1.2	TDD	966

9.4.1.2	Minimum requirement PUCCH 2-1 (Cell-Specific Reference Symbols)	967

9.4.1.2.1	FDD	967

9.4.1.2.2	TDD	969

9.4.1.3	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	970

9.4.1.3.1	FDD	970

9.4.1.3.2	TDD	971

9.4.1.3.3	FDD (with Class A 12Tx codebook)	973

9.4.1.3.4	TDD (with Class A 12Tx codebook)	975

9.4.1.4	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	977

9.4.1.4.1	FDD (with 4Tx enhanced codebook)	977

9.4.1.4.2	TDD (with 4Tx enhanced codebook)	979

9.4.1.4.3	FDD (with Class B alternative codebook for one CSI-RS resource configured)	981

9.4.1.4.4	TDD (with Class B alternative codebook for one CSI-RS resource configured)	983

9.4.1a	Void	985

9.4.1a.1	Void	985

9.4.1a.1.1	Void	985

9.4.1a.1.2	Void	985

9.4.2	Multiple PMI	985

9.4.2.1	Minimum requirement PUSCH 1-2 (Cell-Specific Reference Symbols)	985

9.4.2.1.1	FDD	985

9.4.2.1.2	TDD	986

9.4.2.2	Minimum requirement PUSCH 2-2 (Cell-Specific Reference Symbols)	987

9.4.2.2.1	FDD	987

9.4.2.2.2	TDD	988

9.4.2.3	Minimum requirement PUSCH 1-2 (CSI Reference Symbol)	989

9.4.2.3.1	FDD	989

9.4.2.3.2	TDD	991

9.4.2.3.3	FDD (with 4Tx enhanced codebook)	993

9.4.2.3.4	TDD (with 4Tx enhanced codebook)	995

9.4.2.3.5	FDD (with Class A 16Tx codebook)	997

9.4.2.3.6	TDD (with Class A 16Tx codebook)	999

9.4.3	Void	1001

9.4.3.1	Void	1001

9.4.3.1.1	Void	1001

9.4.3.1.2	Void	1001

9.5	 Reporting of Rank Indicator (RI)	1001

9.5.1	Minimum requirement (Cell-Specific Reference Symbols)	1001

9.5.1.1	FDD	1001

9.5.1.2	TDD	1002

9.5.2	Minimum requirement (CSI Reference Symbols)	1003

9.5.2.1	FDD	1003

9.5.2.2	TDD	1005

9.5.3	Minimum requirement (CSI measurements in case two CSI subframe sets are configured)	1007

9.5.3.1	FDD	1007

9.5.3.2	TDD	1008

9.5.4	Minimum requirement (CSI measurements in case two CSI subframe sets are configured and CRS assistance information are configured)	1010

9.5.4.1	FDD	1010

9.5.4.2	TDD	1012

9.5.5	Minimum requirement (with CSI process)	1014

9.5.5.1	FDD	1015

9.5.5.2	TDD	1016

9.6	Additional requirements for carrier aggregation	1018

9.6.1	Periodic reporting on multiple cells (Cell-Specific Reference Symbols)	1018

9.6.1.1	FDD	1018

9.6.1.2	TDD	1022

9.6.1.3	TDD-FDD CA with FDD PCell	1023

9.6.1.4	TDD-FDD CA with TDD PCell	1026

9.7	 CSI reporting (Single receiver antenna)	1030

9.7.1	CQI reporting definition under AWGN conditions	1030

9.7.1.1	FDD and half-duplex FDD	1030

9.7.1.2	TDD	1030

9.7.1.3	FDD (Category 1bis UE)	1031

9.7.1.4	TDD (Category 1bis UE)	1032

9.7.2	CQI reporting under fading conditions	1033

9.7.2.1	FDD and half-duplex FDD	1033

9.7.2.2	TDD	1034

9.7.2.3	FDD (Category 1bis UE)	1035

9.7.2.4	TDD (Category 1bis UE)	1036

9.8	CSI reporting (UE supporting coverage enhancement)	1037

9.8.1	CQI reporting definition under AWGN conditions	1037

9.8.1.1	FDD and half-duplex FDD	1037

9.8.1.2	TDD	1038

9.8.2	UE-selected subband CQI	1039

9.8.2.1	FDD and half-duplex FDD	1039

9.8.2.2	TDD	1041

9.9	CSI reporting for 4Rx UE	1043

9.9.1	CQI reporting definition under AWGN conditions	1043

9.9.1.1	Minimum requirement PUCCH 1-0 with Rank 1 (Cell-Specific Reference Symbols)	1043

9.9.1.1.1	FDD	1043

9.9.1.1.2	TDD	1044

9.9.1.2	Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)	1044

9.9.1.2.1	FDD	1044

9.9.1.2.2	TDD	1045

9.9.1.3	Minimum requirement PUCCH 1-1 with Rank 4 (Cell-Specific Reference Symbols)	1046

9.9.1.3.1	FDD	1047

9.9.1.3.2	TDD	1047

9.9.1.4	Minimum requirement PUCCH 1-1 with Rank 3 (CSI Reference Symbols)	1048

9.9.1.4.1	FDD	1048

9.9.1.4.2	TDD	1049

9.9.2	CQI reporting definition under fading conditions	1050

9.9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol) for enhanced receiver Type A	1050

9.9.2.1.1	FDD	1051

9.9.2.1.2	TDD	1052

9.9.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol) for enhanced receiver Type A	1054

9.9.2.2.1	FDD	1054

9.9.2.2.2	TDD	1055

9.9.3	Reporting of Precoding Matrix Indicator (PMI) for 4Rx UE	1057

9.9.3.1	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1057

9.9.3.1.1	TDD	1057

9.9.4	 Reporting of Rank Indicator (RI)	1059

9.9.4.1	Minimum requirement (Cell-Specific Reference Symbols)	1059

9.9.4.1.1	FDD	1059

9.9.4.1.2	TDD	1061

9.9.4.2	Minimum requirement (CSI Reference Symbols)	1062

9.9.4.2.1	FDD	1062

9.9.4.2.2	TDD	1064

9.10	Reporting of CSI-RS Resource Indicator (CRI)	1066

9.10.1	Minimum requirement (PUSCH 3-1)	1066

9.10.1.1	FDD	1066

9.10.1.2	TDD	1068

10	Performance requirement (MBMS)	1071

10.1	FDD (Fixed Reference Channel)	1071

10.1.1	Minimum requirement	1071

10.2	TDD (Fixed Reference Channel)	1072

10.2.1	Minimum requirement	1072

11	Performance requirement (ProSe Direct Discovery)	1074

11.1	General	1074

11.1.1	Applicability of requirements	1074

11.1.2	Reference DRX configuration	1074

11.2	Demodulation of PSDCH (single link performance)	1074

11.2.1	FDD (in-coverage)	1074

11.2.2	TDD (in-coverage)	1075

11.2.3	FDD (out-of-coverage)	1076

11.3	Power imbalance performance with two links	1077

11.3.1	FDD	1077

11.3.2	TDD	1078

11.4	Multiple timing reference test	1079

11.4.1	FDD	1080

11.5	Maximum Sidelink processes test	1081

11.5.1	FDD	1081

11.5.2	TDD	1082

12	Performance requirement (ProSe Direct Communication)	1085

12.1	General	1085

12.1.1	Applicability of requirements	1085

12.1.1.1	Applicability of requirements for different channel bandwidths	1085

12.1.1.2	Test coverage for different number of component carriers	1085

12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets	1085

12.1.2	Reference DRX configuration	1085

12.2	Demodulation of PSSCH	1086

12.2.1	FDD	1086

12.3	Demodulation of PSCCH	1087

12.3.1	FDD	1087

12.4	Demodulation of PSBCH	1088

12.4.1	FDD	1088

12.5	Power imbalance performance with two links	1089

12.5.1	FDD	1089

12.6	Multiple timing reference test	1091

12.6.1	FDD	1091

12.7	Maximum Sidelink processes test	1092

12.7.1	FDD	1093

12.8	Sustained downlink data rate with active Sidelink	1094

13	Channel access procedures	1096

13.1	Uplink channel access procedure	1096

13.1.1	Channel access parameters	1096

13.1.2	Minimum requirement	1096

Annex A (normative): Measurement channels	1097

A.1	General	1097

A.2	UL reference measurement channels	1097

A.2.1	General	1097

A.2.1.1	Applicability and common parameters	1097

A.2.1.2	Determination of payload size	1097

A.2.1.3	Overview of UL reference measurement channels	1098

A.2.2	Reference measurement channels for FDD	1114

A.2.2.1	Full RB allocation	1114

A.2.2.1.1	QPSK	1114

A.2.2.1.2	16-QAM	1116

A.2.2.1.3	64-QAM	1117

A.2.2.1.4	256 QAM	1117

A.2.2.2	Partial RB allocation	1117

A.2.2.2.1	QPSK	1118

A.2.2.2.2	16-QAM	1120

A.2.2.2.3	64-QAM	1121

A.2.2.2.4	256 QAM	1122

A.2.2.3	Void	1123

A.2.3	Reference measurement channels for TDD	1123

A.2.3.1	Full RB allocation	1124

A.2.3.1.1	QPSK	1124

A.2.3.1.2	16-QAM	1126

A.2.3.1.3	64-QAM	1128

A.2.3.1.4	256 QAM	1128

A.2.3.2	Partial RB allocation	1128

A.2.3.2.1	QPSK	1129

A.2.3.2.2	16-QAM	1132

A.2.3.2.3	64-QAM	1136

A.2.3.2.4	256 QAM	1138

A.2.3.3	Void	1138

A.2.4	Reference measurement channels for UE category NB1	1139

A.3	DL reference measurement channels	1139

A.3.1	General	1139

A.3.1.1	Overview of DL reference measurement channels	1140

A.3.2	Reference measurement channel for receiver characteristics	1155

A.3.3	Reference measurement channels for PDSCH performance requirements (FDD)	1180

A.3.3.1	Single-antenna transmission (Common Reference Symbols)	1180

A.3.3.2	Multi-antenna transmission (Common Reference Symbols)	1185

A.3.3.2.1	Two antenna ports	1185

A.3.3.2.2	Four antenna ports	1189

A.3.3.3	Reference Measurement Channel for UE-Specific Reference Symbols	1192

A.3.3.3.0	Two antenna ports (no CSI-RS)	1192

A.3.3.3.1	Two antenna port (CSI-RS)	1193

A.3.3.3.2	Four antenna ports (CSI-RS)	1195

A.3.3.3.2A	Eight antenna ports (CSI-RS)	1199

A.3.3.3.3	Twelve antenna port (CSI-RS)	1200

A.3.3.3.4	Sixteen antenna port (CSI-RS)	1201

A.3.4	Reference measurement channels for PDSCH performance requirements (TDD)	1203

A.3.4.1	Single-antenna transmission (Common Reference Symbols)	1203

A.3.4.2	Multi-antenna transmission (Common Reference Signals)	1211

A.3.4.2.1	Two antenna ports	1211

A.3.4.2.2	Four antenna ports	1219

A.3.4.3	Reference Measurement Channels for UE-Specific Reference Symbols	1223

A.3.4.3.1	Single antenna port (Cell Specific)	1223

A.3.4.3.2	Two antenna ports (Cell Specific)	1224

A.3.4.3.3	Two antenna ports (CSI-RS)	1226

A.3.4.3.4	Four antenna ports (CSI-RS)	1228

A.3.4.3.5	Eight antenna ports (CSI-RS)	1233

A.3.4.3.6	Twelve antenna ports (CSI-RS)	1236

A.3.4.3.7	Sixteen antenna ports (CSI-RS)	1237

A.3.5	Reference measurement channels for PDCCH/PCFICH performance requirements	1239

A.3.5.1	FDD	1239

A.3.5.2	TDD	1239

A.3.5.3	LAA	1240

A.3.6	Reference measurement channels for PHICH performance requirements	1240

A.3.7	Reference measurement channels for PBCH performance requirements	1240

A.3.8	Reference measurement channels for MBMS performance requirements	1241

A.3.8.1	FDD	1241

A.3.8.2	TDD	1243

A.3.9	Reference measurement channels for sustained downlink data rate provided by lower layers	1245

A.3.9.1	FDD	1245

A.3.9.2	TDD	1248

A.3.9.3	FDD (EPDCCH scheduling)	1255

A.3.9.4	TDD (EPDCCH scheduling)	1256

A.3.9.5	LAA	1258

A.3.10	Reference Measurement Channels for EPDCCH performance requirements	1260

A.3.10.1	FDD	1260

A.3.10.2	TDD	1260

A.3.11	Reference Measurement Channels for MPDCCH performance requirements	1260

A.3.11.1	FDD and half-duplex FDD	1260

A.3.11.2	TDD	1261

A.3.12	Reference measurement channels for NPDSCH performance requirements	1262

A.3.12.1	In-band	1262

A.3.12.1.2	Two-antenna transmission	1262

A.3.12.2	Standalone/Guard-band	1263

A.3.12.2.1	Single-antenna transmission	1263

A.3.13	Reference measurement channels for NPDCCH performance requirements	1263

A.3.13.1	Half-duplex FDD	1263

A.3.14	Reference measurement channels for NPBCH performance requirements for Cat NB1 UEs	1264

A.3.15	Reference Measurement Channels for LAA SCell with frame structure Type-3	1264

A.3.15.1	Multi-antenna transmission (Common Reference Symbols)	1264

A.3.15.1.1	Four antenna ports	1264

A.3.15.2	Reference Measurement Channel for UE-Specific Reference Symbols	1265

A.3.15.2.1	Two antenna ports (CSI-RS)	1265

A.4	CSI reference measurement channels	1265

A.5	OFDMA Channel Noise Generator (OCNG)	1275

A.5.1	OCNG Patterns for FDD	1275

A.5.1.1	OCNG FDD pattern 1: One sided dynamic OCNG FDD pattern	1275

A.5.1.2	OCNG FDD pattern 2: Two sided dynamic OCNG FDD pattern	1276

A.5.1.3	OCNG FDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1276

A.5.1.4	OCNG FDD pattern 4: One sided dynamic OCNG FDD pattern for MBMS transmission	1277

A.5.1.5	OCNG FDD pattern 5: One sided dynamic 16QAM modulated OCNG FDD pattern	1277

A.5.1.6	OCNG FDD pattern 6: dynamic OCNG FDD pattern when user data is in 2 non-contiguous blocks	1278

A.5.1.7	OCNG FDD pattern 7: dynamic OCNG FDD pattern when user data is in multiple non-contiguous blocks	1278

A.5.1.8	OCNG FDD pattern 8: Dynamic OCNG FDD pattern for TM10 transmission	1279

A.5.2	OCNG Patterns for TDD	1280

A.5.2.1	OCNG TDD pattern 1: One sided dynamic OCNG TDD pattern	1280

A.5.2.2	OCNG TDD pattern 2: Two sided dynamic OCNG TDD pattern	1281

A.5.2.3	OCNG TDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1282

A.5.2.4	OCNG TDD pattern 4: One sided dynamic OCNG TDD pattern for MBMS transmission	1282

A.5.2.5	OCNG TDD pattern 5: One sided dynamic 16QAM modulated OCNG TDD pattern	1283

A.5.2.6	OCNG TDD pattern 6: dynamic OCNG TDD pattern when user data is in 2 non-contiguous blocks	1284

A.5.2.7	OCNG TDD pattern 7: dynamic OCNG TDD pattern when user data is in multiple non-contiguous blocks	1285

A.5.2.8	OCNG TDD pattern 8: Dynamic OCNG TDD pattern for TM10 transmission	1285

A.5.3	OCNG Patterns for Narrowband IoT	1286

A.5.3.1	Narrowband IoT OCNG pattern 1	1286

A.5.4	OCNG Patterns for frame structure type 3	1287

A.5.4.1	OCNG FS3 pattern 1: One sided dynamic OCNG frame structure type 3 pattern	1287

A.6	Sidelink reference measurement channels	1288

A.6.1	General	1288

A.6.2	Reference measurement channel for receiver characteristics	1288

A.6.3	Reference measurement channels for PSDCH performance requirements	1290

A.6.4	Reference measurement channels for PSCCH performance requirements	1291

A.6.5	Reference measurement channels for PSSCH performance requirements	1291

A.6.6	Reference measurement channels for PSBCH performance requirements	1292

A.7	Sidelink reference resource pool configurations	1293

A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests	1293

A.7.1.1	FDD	1293

A.7.1.2	TDD	1296

A.7.2	Reference resource pool configurations for ProSe Direct Communication demodulation tests	1298

A.7.2.1	FDD	1298

A.8	V2X reference measurement channels	1303

A.8.1	General	1303

A.8.2	Reference measurement channel for receiver characteristics	1304

A.8.3	Reference measurement channel for transmitter characteristics	1305

Annex B (normative): Propagation conditions	1307

B.1	Static propagation condition	1307

B.1.1	UE Receiver with 2Rx	1307

B.1.2	UE Receiver with 4Rx	1307

B.2	Multi-path fading propagation conditions	1308

B.2.1	Delay profiles	1308

B.2.2	Combinations of channel model parameters	1309

B.2.3	MIMO Channel Correlation Matrices	1309

B.2.3.1	Definition of MIMO Correlation Matrices	1309

B.2.3.2	MIMO Correlation Matrices at High, Medium and Low Level	1311

B.2.3A	MIMO Channel Correlation Matrices using cross polarized antennas	1315

B.2.3A.1	Definition of MIMO Correlation Matrices using cross polarized antennas	1315

B.2.3A.2	Spatial Correlation Matrices using cross polarized antennas at eNB and UE sides	1316

B.2.3A.2.1	Spatial Correlation Matrices at eNB side	1316

B.2.3A.2.2	Spatial Correlation Matrices at UE side	1316

B.2.3A.3	MIMO Correlation Matrices using cross polarized antennas	1316

B.2.3A.4	Beam steering approach	1319

B.2.3B	MIMO Channel Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1319

B.2.3B.1	Definition of MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1320

B.2.3B.2	Spatial Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1321

B.2.3B.2.1	Spatial Correlation Matrices at eNB side	1321

B.2.3B.2.2	Spatial Correlation Matrices at UE side	1321

B.2.3B.3	MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1321

B.2.3B.4	Beam steering approach	1324

B.2.4	Propagation conditions for CQI tests	1325

B.2.4.1	Propagation conditions for CQI tests with multiple CSI processes	1325

B.2.5	Void	1325

B.2.6	MBSFN Propagation Channel Profile	1325

B.3	High speed train scenario	1326

B.3A	HST-SFN scenario	1327

B.4	Beamforming Model	1330

B.4.1	Single-layer random beamforming (Antenna port 5, 7, or 8)	1330

B.4.1A	Single-layer random beamforming (Antenna port 7, 8, 11 or 13 with enhanced DMRS table configured)	1331

B.4.2	Dual-layer random beamforming (antenna ports 7 and 8)	1331

B.4.3	Generic beamforming model (antenna ports 7-14)	1332

B.4.4	Random beamforming for EPDCCH distributed transmission (Antenna port 107 and 109)	1332

B.4.5	Random beamforming for EPDCCH localized transmission (Antenna port 107, 108, 109 or 110)	1333

B.4.6	Beamforming model for CRI test	1333

B.5	Interference models for enhanced performance requirements Type-A	1334

B.5.1	Dominant interferer proportion	1334

B.5.2	Transmission mode 3 interference model	1335

B.5.3	Transmission mode 4 interference model	1335

B.5.4	Transmission mode 9 interference model	1335

B.6	Interference models for enhanced performance requirements Type-B	1336

B.6.1	Transmission mode 2 interference model	1336

B.6.2	Transmission mode 3 interference model	1336

B.6.3	Transmission mode 4 interference model	1337

B.6.4	Transmission mode 9 interference model	1337

B.6.5	CRS interference model	1337

B.6.6	Random interference model	1338

B.7	Interference models for enhanced downlink control channel performance requirements Type A and B	1338

B.7.1	PDCCH, PCFICH and PHICH interference model	1339

B.8	Burst transmission models for Frame structure type 3	1339

B.8.1	Burst transmission model for one LAA SCell	1339

Annex C (normative): Downlink Physical Channels	1341

C.1	General	1341

C.2	Set-up	1341

C.3	Connection	1341

C.3.1	Measurement of Receiver Characteristics	1341

C.3.2	Measurement of Performance requirements	1342

C.3.3	Aggressor cell power allocation for Measurement of Performance Requirements when ABS is Configured	1344

C.3.4	Power Allocation for Measurement of Performance Requirements when Quasi Co-location Type B: same Cell ID	1344

C.3.5	Simplified CA testing method	1345

C.3.6	Measurement of Receiver Characteristics for Narrowband IoT	1345

Annex D (normative): Characteristics of the interfering signal	1347

D.1	General	1347

D.2	Interference signals	1347

Annex E (normative): Environmental conditions	1348

E.1	 General	1348

E.2	 Environmental	1348

E.2.1	Temperature	1348

E.2.2	Voltage	1348

E.2.3	Vibration	1349

Annex F (normative): Transmit modulation	1350

F.1	Measurement Point	1350

F.2	Basic Error Vector Magnitude measurement	1350

F.3	Basic in-band emissions measurement	1351

F.4	Modified signal under test	1351

F.5	Window length	1353

F.5.1	Timing offset	1353

F.5.2	Window length	1353

F.5.3	Window length for normal CP	1353

F.5.4	Window length for Extended CP	1354

F.5.5	Window length for PRACH	1354

F.5.F	Window length for category NB1	1355

F.6	Averaged EVM	1355

F.6.F	Averaged EVM for category NB1	1356

F.7	Spectrum Flatness	1356

Annex G (informative): Reference sensitivity level in lower SNR	1357

G.1	General	1357

G.2	 Typical receiver sensitivity performance (QPSK)	1357

G.3	Reference measurement channel for REFSENSE in lower SNR	1360

Annex H (normative): Modified MPR behavior	1363

H.1	Indication of modified MPR behavior	1363

Annex I (informative): Change history	1364

Annex I-MF (informative): Change history for MF Specification	1397