MFA TS 36.133 V1.0.0 (2017-12)

Technical Specification

MulteFire Alliance

Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management

(Release 1.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

MF, multefire, LTE

MulteFire Alliance

Postal address

5177 Brandin Court

Fremont, CA 94538 USA

Internet

www.multefire.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, MulteFire Alliance

This specification is a modification of 3GPP TS 36.133 Version 14.3.0, © 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC) from which permission has been received.

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	65

1	Scope	66

2	References	67

3	Definitions, symbols and abbreviations	69

3.1	Definitions	69

3.2	Symbols	70

3.3	Abbreviations	72

3.4	Test tolerances	74

3.5	Additional notation	74

3.5.1	Groups of bands	74

3.6	General	76

3.6.1	Applicability of requirements in this specification version	76

4	E-UTRAN RRC_IDLE state mobility	80

4.1	Cell Selection	80

4.2	Cell Re-selection	80

4.2.1	Introduction	80

4.2.2	Requirements	80

4.2.2.1	Measurement and evaluation of serving cell	81

4.2.2.2	Void	82

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	82

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	84

4.2.2.5	Measurements of inter-RAT cells	87

4.2.2.5.1	Measurements of UTRAN FDD cells	88

4.2.2.5.2	Measurements of UTRAN TDD cells	89

4.2.2.5.3	Measurements of GSM cells	91

4.2.2.5.4	Measurements of HRPD cells	92

4.2.2.5.5	Measurements of cdma2000 1X	94

4.2.2.6	Evaluation of cell re-selection criteria	95

4.2.2.7	Maximum interruption in paging reception	95

4.2.2.8	void	96

4.2.2.9	UE measurement capability	96

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	97

4.2.2.10	Reselection to CSG cells	97

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	97

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	98

4.2.2.11	Measurement and evaluation requirements for UE in enhanced coverage	99

4.2.2.11.1	Measurement and evaluation of serving cell	99

4.2.2.11.2	Measurement and evaluation of intra-frequency E-UTRAN cells	100

4.2.2.12	Maximum interruption in paging reception for Category M1 UEs in normal coverage	102

4.2.2.13	Maximum interruption in paging reception for Category M1 UEs in extended coverage	103

4.3	Minimization of Drive Tests (MDT)	103

4.3.1	Introduction	103

4.3.2	Measurements	104

4.3.2.1	Requirements	104

4.3.3	Relative Time Stamp Accuracy	104

4.3.3.1	Requirements	104

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	104

4.3.4.1	 Requirements	104

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	104

4.3.5.1	Requirements for timeSinceFailure	105

4.4	MBSFN Measurements	105

4.4.1	Introduction	105

4.4.2	MBSFN RSRP measurements	105

4.4.3	MBSFN RSRQ measurements	105

4.4.4	MCH BLER measurements	106

4.5	Proximity-based Services	106

4.5.1	Introduction	106

4.5.2	Requirements	106

4.5.2.1	Interruptions with ProSe Direct Discovery	106

4.5.2.2	Interruptions with ProSe Direct Communication	106

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	106

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	107

4.6	Cell Selection and Re-selection Requirements for UE category NB1	107

4.6.1	Cell Selection	108

4.6.2	Cell Re-selection	108

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	108

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	109

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	110

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	111

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	113

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	114

4.6.2.7	Maximum interruption in paging reception in normal coverage	116

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	116

4.6.2.8	UE measurement capability	116

4.7	Cell Selection and Re-selection Requirements for UE category M2	116

4.7.1	Cell Selection	116

4.7.2	Cell Re-selection	116

4.7.2.1	Cell Re-selection requirements for UE category M2 in normal coverage	117

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M2 in normal coverage	117

4.7.2.1.2	Measurements of intra-frequency cells for UE category M2 in normal coverage	117

4.7.2.1.3	Measurements of inter-frequency cells for UE category M2 in normal coverage	117

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M2 in normal coverage	117

4.7.2.2	Cell Re-selection requirements for UE category M2 in enhanced coverage	117

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M2 in enhanced coverage	117

4.7.2.2.2	Measurements of intra-frequency cells for UE category M2 in enhanced coverage	117

4.7.2.2.3	Measurements of inter-frequency cells for UE category M2 in enhanced coverage	117

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M2 in enhanced coverage	119

5	E-UTRAN RRC_CONNECTED state mobility	121

5.1	E-UTRAN Handover	121

5.1.1	Introduction	121

5.1.2	Requirements	121

5.1.2.1	E-UTRAN FDD – FDD	121

5.1.2.1.1	Handover delay	121

5.1.2.1.2	Interruption time	122

5.2.2.2	E-UTRAN FDD – TDD	123

5.2.2.2.1	(Void)	123

5.2.2.2.2	(Void)	123

5.2.2.3	E-UTRAN TDD – FDD	123

5.2.2.3.1	(Void)	123

5.2.2.3.2	(Void)	123

5.2.2.4	E-UTRAN TDD – TDD	123

5.2.2.4.1	Handover delay	124

5.2.2.4.2	Interruption time	124

5.2.2.5	E-UTRAN HD–FDD	126

5.2.2.5.1	Handover delay	126

5.2.2.5.2	Interruption time	126

5.2.2MF1	MF – MF	128

5.2.2MF1.1	Handover delay	128

5.2.2MF1.2	Interruption time	128

5.2.2MF2	FDD – MF	129

5.2.2MF3	TDD – MF	129

5.3	Handover to other RATs	129

5.3.1	E-UTRAN - UTRAN FDD Handover	129

5.3.1.1	Introduction	129

5.3.1.1.1	Handover delay	129

5.3.1.1.2	Interruption time	129

5.3.2	E-UTRAN - UTRAN TDD Handover	130

5.3.2.1	Introduction	130

5.3.2.2	Requirements	130

5.3.2.2.1	Handover delay	130

5.3.2.2.2	Interruption time	130

5.3.3	E-UTRAN - GSM Handover	131

5.3.3.1	Introduction	131

5.3.3.2	Requirements	131

5.3.3.2.1	Handover delay	131

5.3.3.2.2	Interruption time	131

5.4	Handover to Non-3GPP RATs	132

5.4.1	E-UTRAN – HRPD Handover	132

5.4.1.1	Introduction	132

5.4.1.1.1	Handover delay	132

5.4.1.1.2	Interruption time	132

5.4.2	E-UTRAN – cdma2000 1X Handover	133

5.4.2.1	Introduction	133

5.4.2.1.1	Handover delay	133

5.4.2.1.2	Interruption time	133

5.5	E-UTRAN Handover for Cat-M1 UEs in CEModeA	133

5.5.1	Introduction	133

5.5.2	Requirements	134

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	134

5.5.2.1.1	Handover delay	134

5.5.2.1.2	Interruption time	134

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	134

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	135

5.5.2.3.1	Handover delay	135

5.5.2.3.2	Interruption time	135

5.6	E-UTRAN Handover for Cat-M1 UEs in CEModeB	135

5.6.1	Introduction	135

5.6.2	Requirements	136

5.6.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	136

5.6.2.1.1	Handover delay	136

5.6.2.1.2	Interruption time	136

5.6.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	136

5.6.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	137

5.6.2.3.1	Handover delay	137

5.6.2.3.2	Interruption time	137

6	RRC Connection Mobility Control	138

6.1	RRC Re-establishment	138

6.1.1	Introduction	138

6.1.2	Requirements	138

6.1.2.1	UE Re-establishment delay requirement for E-UTRAN FDD and TDD	138

6.1.2.1MF1	UE Re-establishment delay requirement (MF)	138

6.2	Random Access	139

6.2.1	Introduction	139

6.2.2	Requirements	139

6.2.2.1	Contention based random access	140

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	140

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	140

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	140

6.2.2.1.4	Void	140

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	140

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	140

6.2.2.2	Non-Contention based random access	140

6.2.2.2.1	Correct behaviour when receiving Random Access Response	140

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	140

6.2.3	Requirements for Cat-M1 UEs	141

6.3	RRC Connection Release with Redirection	141

6.3.1	Introduction	141

6.3.2	Requirements	141

6.3.2.1	RRC connection release with redirection to UTRAN FDD	141

6.3.2.2	RRC connection release with redirection to GERAN	142

6.3.2.3	RRC connection release with redirection to UTRAN TDD	142

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	143

6.4.1	Introduction	143

6.4.2	Requirements	143

6.5	RRC Re-establishment for NB-IoT UEs	143

6.5.1	Introduction	143

6.5.2	Requirements	143

6.5.2.1	UE Re-establishment delay requirement in normal coverage	144

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	144

6.6	Random Access for UE category NB1	144

6.6.1	Introduction	145

6.6.2	Requirements	145

6.6.2.1	Correct behaviour when receiving Random Access Response reception	145

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	145

6.6.2.3	Correct behaviour when receiving a NACK on msg3	145

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	145

6.6.2.5	Correct behaviour when contention Resolution timer expires	145

6.6.3	Requirements for NPRACH configuration	146

6.7	RRC Re-establishment for Cat-M1 UEs	146

6.7.1	Introduction	146

6.7.2	Requirements	146

6.7.2.1	UE Re-establishment delay requirement for CEModeA	146

6.7.2.2	UE Re-establishment delay requirement for CEModeB	147

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	147

6.8.1	Introduction	147

6.8.2	Requirements	148

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	148

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	148

6.9.1	Introduction	148

6.9.2	Requirements	148

7	Timing and signalling characteristics	150

7.1	UE transmit timing	150

7.1.1	Introduction	150

7.1.2	Requirements	150

7.2	UE timer accuracy	151

7.2.1	Introduction	151

7.2.2	Requirements	152

7.3	Timing Advance	152

7.3.1	Introduction	152

7.3.2	Requirements	152

7.3.2.1	Timing Advance adjustment delay	152

7.3.2.2	Timing Advance adjustment accuracy	152

7.4	Cell phase synchronization accuracy (TDD)	152

7.4.1	Definition	152

7.4.2	Minimum requirements	152

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	153

7.5.1	Introduction	153

7.5.2	eNodeB Synchronization Requirements	153

7.5.2.1	Synchronized E-UTRAN	153

7.5.2.2	Non-Synchronized E-UTRAN	154

7.6	Radio Link Monitoring	154

7.6.1	Introduction	154

7.6.2	Requirements	156

7.6.2.1	Minimum requirement when no DRX is used	156

7.6.2.2	Minimum requirement when DRX is used	157

7.6.2.3	Minimum requirement at transitions	159

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	159

7.6.2.5	Minimum requirement under IDC Interference	159

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	159

7.7.1	Introduction	159

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	159

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	160

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells	161

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	162

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	162

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	163

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	164

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	164

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	164

7.7.10MF1	SCell Activation Delay Requirement for Deactivated MF PUCCH SCell under Frame Structure 3	165

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	166

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	166

7.7.12MF1	SCell Activation Delay Requirement for Deactivated MF PUCCH SCell with Multiple SCells under Frame Structure 3	167

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	167

7.8	Interruptions with Carrier Aggregation	168

7.8.1	Introduction	168

7.8.2	Requirements	168

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	168

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	168

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	168

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	169

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	169

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	169

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	169

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	169

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	170

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	171

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	171

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	171

7.8.2.13	Interruptions at SRS carrier based switching	172

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	173

7.9.1	Introduction	173

7.9.2	Minimum Requirements for Interband Carrier Aggregation	173

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	173

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	174

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	174

7.10.1	Introduction	174

7.10.2	Requirements	174

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	174

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	174

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	175

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	175

7.11	Radio Link Monitoring for UE Category 0	175

7.11.1	Introduction	175

7.11.2	Requirements for FD-FDD and TDD	177

7.11.2.1	Minimum requirement when no DRX is used	177

7.11.2.2	Minimum requirement when DRX is used	177

7.11.2.3	Minimum requirement at transitions	178

7.11.3	Requirements for HD-FDD	178

7.11.3.1	Minimum requirement when no DRX is used	178

7.11.3.2	Minimum requirement when DRX is used	178

7.11.3.3	Minimum requirement at transitions	179

7.12	Interruptions with Dual Connectivity	179

7.12.1	Introduction	179

7.12.2	Requirements	180

7.12.2.1	Interruptions at PSCell addition/release	180

7.12.2.2	Interruptions at transitions between active and non-active during DRX	180

7.12.2.3	Interruptions at transitions from non-DRX to DRX	180

7.12.2.4	Interruptions at SCell addition/release	180

7.12.2.5	Interruptions at SCell activation/deactivation	181

7.12.2.6	Interruptions during measurements on SCC	181

7.12.2.7	Interruptions at SRS carrier based switching	182

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	182

7.13.1	Definition	183

7.13.2	Minimum requirements	183

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	183

7.14.1	Introduction	183

7.14.2	PSCell Addition Delay Requirement	183

7.14.3	PSCell Release Delay Requirement	184

7.15	Maximum Receive Timing Difference in Dual Connectivity	184

7.15.1	Introduction	184

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	184

7.16	Proximity-based Services	184

7.16.1	Introduction	185

7.16.2	Requirements	185

7.16.2.1	ProSe UE transmission timing	185

7.16.2.1.1	Serving cell or PCell as timing reference	185

7.16.2.1.2	SCell or non-serving cell as timing reference	185

7.16.3	Interruptions with ProSe	186

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	186

7.16.3.2	Interruptions at ProSe Direct Communication configuration	186

7.16.3.3	Interruptions during ProSe Direct Discovery	186

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	187

7.16.3.5	Interruptions during ProSe Direct Communication	187

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	187

7.16.4.1	Measurement and evaluation of selected cell	188

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	188

7.16.5	Selection / Reselection of ProSe relay UE	189

7.17	Maximum Transmission Timing Difference in Dual Connectivity	189

7.17.1	Introduction	189

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	189

7.18.1	Introduction	190

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	190

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	190

7.19	Radio Link Monitoring for UE Category M1	190

7.19.1	Introduction	190

7.19.2	Requirements for FD-FDD and TDD CE mode A	190

7.19.2.1	Minimum requirement when no DRX is used	191

7.19.2.2	Minimum requirement when DRX is used	191

7.19.2.3	Minimum requirement at transitions	192

7.19.3	Requirements for HD-FDD with CE mode A	192

7.19.3.1	Minimum requirement when no DRX is used	192

7.19.3.2	Minimum requirement when DRX is used	193

7.19.3.3	Minimum requirement at transitions	194

7.19.4	Requirements for FD-FDD and TDD with CE mode B	194

7.19.4.1	Minimum requirement when no DRX is used	194

7.19.4.2	Minimum requirement when DRX is used	195

7.19.4.3	Minimum requirement at transitions	196

7.19.5	Requirements for HD-FDD with CE mode B	196

7.19.5.1	Minimum requirement when no DRX is used	196

7.19.5.2	Minimum requirement when DRX is used	196

7.19.5.3	Minimum requirement at transitions	197

7.20	UE transmit timing for NB-IoT	197

7.20.1	Introduction	197

7.20.2	Requirements	198

7.21	 UE timer accuracy for NB-IoT	198

7.21.1	Introduction	198

7.21.2	Requirements	198

7.22	Timing Advance for NB-IoT	199

7.22.1	Introduction	199

7.22.2	Requirements	199

7.22.2.1	Timing Advance adjustment delay	199

7.22.2.2	Timing Advance adjustment accuracy	199

7.23	Radio Link Monitoring for Category NB1 UE	199

7.23.1	Introduction	199

7.23.2	Requirements for HD-FDD Category NB1 UE	199

7.23.2.1	Minimum requirement when no DRX is used	200

7.23.2.2	Minimum requirement when DRX is used	200

7.23.2.3	Minimum requirement at transitions	201

7.24	UE transmit timing for Category M1	201

7.24.1	Introduction	201

7.24.2	Requirements	201

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	202

7.25.1	Definition	203

7.25.2	Minimum requirements	203

8	UE Measurements Procedures in RRC_CONNECTED State	204

8.1	General Measurement Requirements	204

8.1.1	Introduction	204

8.1.2	Requirements	204

8.1.2.1	UE measurement capability	204

8.1.2.1.1	Monitoring of multiple layers using gaps	206

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	208

8.1.2.2	E-UTRAN intra frequency measurements	209

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	209

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	214

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	219

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	220

8.1.2.3	E-UTRAN inter frequency measurements	221

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	221

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	225

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	230

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	230

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	230

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	231

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	232

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	233

8.1.2.4	Inter RAT measurements	234

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	235

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	239

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	240

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	244

8.1.2.4.5	E-UTRAN FDD – GSM measurements	244

8.1.2.4.6	E-UTRAN TDD – GSM measurements	251

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	252

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	254

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	254

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	254

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	254

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	255

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	255

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	255

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	257

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	257

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	257

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	257

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	258

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	258

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	261

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	261

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	261

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	263

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	265

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	266

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	267

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	269

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	271

8.1.2.7	E-UTRAN E-CID Measurements	273

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	273

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	275

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	277

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	277

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	278

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	278

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	281

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	285

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	289

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	293

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	293

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	293

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	293

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	294

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	294

8.2.1	Introduction	294

8.2.2	Requirements	295

8.3	Measurements for E-UTRA carrier aggregation	297

8.3.1	Introduction	297

8.3.2	Measurements of the primary component carrier	297

8.3.3	Measurements of a secondary component carrier	298

8.3.3.1	Measurements of a secondary component carrier with active SCell	298

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	298

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	298

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	299

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	301

8.4.1	Introduction	301

8.4.2	Measurements on the primary component carrier	302

8.4.3	Measurements on a secondary component carrier	302

8.4.4	Measurements on both primary component carrier and a secondary component carrier	303

8.4.5	Measurements on different secondary component carriers	304

8.5	Measurements for UE category 0	306

8.5.1	Introduction	306

8.5.2 	Requirements	306

8.5.2.1 	E-UTRAN intra frequency measurements	306

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	306

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	310

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	312

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	316

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	317

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	317

8.6	Discovery signal measurements	318

8.6.1	Introduction	318

8.6.2	Requirements for CRS based discovery signal measurements	319

8.6.2.1	E-UTRAN intra frequency measurements	319

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	319

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	322

8.6.2.2	E-UTRAN inter frequency measurements	325

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	325

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	328

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	331

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	331

8.6.3	Requirements for CSI-RS based discovery signal measurements	332

8.6.3.1	E-UTRAN intra frequency measurements	332

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	332

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	335

8.6.3.2	E-UTRAN inter frequency measurements	338

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	338

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	341

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	344

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	344

8.7	Discovery signal measurements for E-UTRA carrier aggregation	344

8.7.1	Introduction	345

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	345

8.7.2.1	Measurements of the primary component carrier	345

8.7.2.2	Measurements of a secondary component carrier	345

8.7.2.3	Measurements of a secondary component carrier with active SCell	345

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	345

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	345

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	347

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	349

8.7.3.1	Measurements of the primary component carrier	349

8.7.3.2	Measurements of a secondary component carrier	349

8.7.3.3	Measurements of a secondary component carrier with active SCell	349

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	349

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	349

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	351

8.8	Measurements for E-UTRA dual connectivity	352

8.8.1	Introduction	353

8.8.2	Intra-frequency measurements requirements on PCell	353

8.8.3	Intra-frequency measurements requirements on PSCell	353

8.8.4	Inter-frequency and inter-RAT measurement requirements	353

8.8.5	Intra-frequency measurements with autonomous gaps	353

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	353

8.8.5.2	ECGI reporting delay	354

8.8.6	Inter-frequency measurements with autonomous gaps	355

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	355

8.8.6.2	ECGI reporting delay	356

8.8.7	SSTD Measurements	356

8.8.7.1	Introduction	356

8.8.7.2	SSTD Measurement requirements	356

8.8.7.3	SSTD Measurement Reporting Delay	357

8.8.8	Intra-frequency measurements requirements on SCell	357

8.9	MBSFN Measurements	357

8.9.1	Introduction	357

8.9.2	MBSFN RSRP Measurements	357

8.9.3	MBSFN RSRQ Measurements	357

8.9.4	MCH BLER Measurements	358

8.10	Proximity-based Services	358

8.10.1	Introduction	358

8.10.2	Requirements	358

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	358

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	359

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	359

8.11.1	Introduction	359

8.11.2	CRS based discovery signal measurements	360

8.11.2.1	E-UTRAN intra-frequency measurements	360

8.11.2.1.1	Requirements	360

8.11.2.1.1.1	Requirements when no DRX is used	360

8.11.2.2	E-UTRAN inter-frequency measurements	366

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	366

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	372

8.11.3	CSI-RS based discovery signal measurements	372

8.11.3.1	E-UTRAN intra-frequency measurements	372

8.11.3.1.1	Requirements	372

8.11.3.2	E-UTRAN inter-frequency measurements	376

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	377

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	381

8.11.4	RSSI measurements	381

8.11.4.1	E-UTRAN intra-frequency measurements	381

8.11.4.2	E-UTRAN inter-frequency measurements	382

8.11.5	Channel occupancy measurements	382

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	382

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	382

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	383

8.12.1	Introduction	383

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	383

8.12.2.1	Introduction	383

8.12.2.2	Measurements of a secondary component carrier	383

8.12.2.3	Measurements of a secondary component carrier with active SCell	383

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	383

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	383

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	386

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	389

8.12.3.1	Introduction	389

8.12.3.2	Measurements of a secondary component carrier	389

8.12.3.3	Measurements of a secondary component carrier with active SCell	389

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	389

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	390

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	392

8.13	Measurements for UE Category M1	394

8.13.1	Introduction	394

8.13.2	Requirements for UE category M1 with CE mode A	394

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	394

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	394

8.13.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	398

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	400

8.13.2.2	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	404

8.13.2.2.1	E-UTRAN FDD Intra-frequency E-CID RSRP Measurements	404

8.13.2.2.2	E-UTRAN HD-FDD Intra-frequency E-CID RSRP Measurements	404

8.13.2.2.3	E-UTRAN TDD Intra-frequency E-CID RSRP Measurements	405

8.13.3	Requirements for UE category M1 with CE mode B	405

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	405

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	406

8.13.3.1.2 	E-UTRAN intra frequency measurements for HD-FDD	410

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	413

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	417

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	418

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	418

8.13.3.2	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	419

8.13.3.2.1	E-UTRAN FDD Intra-frequency E-CID RSRP Measurements	419

8.13.3.2.2	E-UTRAN HD-FDD Intra-frequency E-CID RSRP Measurements	420

8.13.3.2.3	E-UTRAN TDD Intra-frequency E-CID RSRP Measurements	420

8.14	Measurements for UE category NB1	421

8.14.1	Introduction	421

8.14.2	NB-IoT intra frequency measurements under normal coverage	421

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	421

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	421

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	422

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	422

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	422

8.15	Measurements for UE Category M2	422

8.15.1	Introduction	422

8.15.2	Requirements for UE category M2 with CE mode A	422

8.15.2.1	Maximum allowed layers for multiple monitoring for UE category M2 with CE mode A	422

8.15.2.2	E-UTRAN intra frequency measurements by UE category M2 with CE mode A	423

8.15.2.2.1	E-UTRAN FDD intra frequency measurements	423

8.15.2.2.2	E-UTRAN intra frequency measurements for HD-FDD	427

8.15.2.2.3	E-UTRAN TDD intra frequency measurements	430

8.15.2.3	E-UTRAN inter frequency measurements by UE category M2 with CE mode A	434

8.15.2.3.1	E-UTRAN FDD - FDD inter frequency measurements	434

8.15.2.3.2	E-UTRAN inter-frequency measurements for HD-FDD	438

8.15.2.3.3	E-UTRAN TDD inter frequency measurements	441

8.15.2.4	E-UTRAN E-CID Measurements Requirements for UE category M2 with CE mode A	445

8.15.2.4.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeA	445

8.15.2.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeA	445

8.15.2.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeA	446

8.15.2.4.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeA	446

8.15.2.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeA	447

8.15.2.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeA	447

8.15.3.1	Maximum allowed layers for multiple monitoring for UE category M2 with CE mode B	448

8.15.3.2	E-UTRAN intra frequency measurements by UE category M2 with CE mode B	448

8.15.3.2.1	E-UTRAN FDD intra frequency measurements	448

8.15.3.2.2	E-UTRAN intra frequency measurements for HD-FDD	452

8.15.3.2.3	E-UTRAN TDD intra frequency measurements	455

8.15.3.3	E-UTRAN inter frequency measurements by UE category M2 with CE Mode B	460

8.15.3.3.1	E-UTRAN FDD - FDD inter frequency measurements	460

8.15.3.3.2 	E-UTRAN inter-frequency measurements for HD-FDD	464

8.15.3.3.3	E-UTRAN TDD inter frequency measurements	467

8.15.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M2 UE in CEModeB	472

8.15.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeB	472

8.15.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeB	473

8.15.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M2 UE in CEModeB	473

8.15.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeB	474

8.15.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M2 UE in CEModeB	474

9	Measurements performance requirements for UE	475

9.1	E-UTRAN measurements	475

9.1.1	Introduction	475

9.1.2	Intra-frequency RSRP Accuracy Requirements	475

9.1.2.1	Absolute RSRP Accuracy	475

9.1.2.2	Relative Accuracy of RSRP	476

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	477

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	477

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	478

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	479

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	480

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	481

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	482

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	482

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	482

9.1.3	Inter-frequency RSRP Accuracy Requirements	483

9.1.3.1	Absolute RSRP Accuracy	483

9.1.3.2	Relative Accuracy of RSRP	484

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	485

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	486

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	486

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	486

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	487

9.1.4	RSRP Measurement Report Mapping	488

9.1.5	Intra-frequency RSRQ Accuracy Requirements	488

9.1.5.1	Absolute RSRQ Accuracy	488

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	489

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	490

9.1.5.4	Absolute WB-RSRQ Accuracy	491

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	492

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	493

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	493

9.1.6	Inter-frequency RSRQ Accuracy Requirements	494

9.1.6.1	Absolute RSRQ Accuracy	494

9.1.6.2	Relative Accuracy of RSRQ	495

9.1.6.3	Absolute WB-RSRQ Accuracy	495

9.1.6.4	Relative WB-RSRQ Accuracy	496

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	497

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	498

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	499

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	499

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	500

9.1.7	RSRQ Measurement Report Mapping	501

9.1.8	Power Headroom	501

9.1.8.1	Period	502

9.1.8.2	Reporting Delay	502

9.1.8.3	Void	502

9.1.8.4	Report Mapping	502

9.1.9	UE Rx – Tx time difference	502

9.1.9.1	Measurement Requirement	502

9.1.9.2	Measurement Report mapping	503

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	504

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	505

9.1.10	Reference Signal Time Difference (RSTD)	506

9.1.10.1	Intra-Frequency Accuracy Requirement	506

9.1.10.2	Inter-Frequency Accuracy Requirement	507

9.1.10.3	RSTD Measurement Report Mapping	509

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	509

9.1.11	Carrier aggregation measurement accuracy	510

9.1.11.1	Primary component carrier accuracy requirement	510

9.1.11.2	Secondary component carrier accuracy requirement	510

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	511

9.1.11.4	Secondary component carrier relative accuracy requirement	511

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	511

9.1.13	Measurement accuracy for UE category 0	511

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	511

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	512

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	513

9.1.14	Accuracy requirements for Discovery Signal Measurements	514

9.1.14.1	Introduction	514

9.1.14.2	RSRP measurements in discovery signal occasions	514

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	515

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	515

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	516

9.1.14.3.3	CSI-RSRP measurement report mapping	518

9.1.14.4	RSRQ measurements in discovery signal occasions	518

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	518

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	519

9.1.15.1.1	Primary component carrier accuracy requirement	519

9.1.15.1.2	Secondary component carrier accuracy requirement	519

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	519

9.1.15.1.4	Secondary component carrier relative accuracy requirement	519

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	519

9.1.15.2.1	Primary component carrier accuracy requirement	519

9.1.15.2.2	Secondary component carrier accuracy requirement	519

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	520

9.1.15.2.4	Secondary component carrier relative accuracy requirement	520

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	520

9.1.17	RS-SINR Measurements	521

9.1.17.1	Measurement Report Mapping	521

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	521

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	521

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	522

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	522

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	522

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	523

9.1.18.1	Introduction	523

9.1.18.2	RSRP measurements	523

9.1.18.2.1	RSRP measurement report mapping	523

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	523

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	524

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	524

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	525

9.1.18.3	RSRQ measurements	525

9.1.18.3.1	RSRQ measurement report mapping	525

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	526

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	526

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	527

9.1.18.4	CSI-RSRP measurements	527

9.1.18.4.1	CSI-RSRP measurement report mapping	527

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	528

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	528

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	529

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	529

9.1.18.5	RSSI measurements	530

9.1.18.5.1	RSSI measurement report mapping	530

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	530

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	531

9.1.18.6	Channel occupancy measurements	531

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	531

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	531

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	532

9.1.19.1	Introduction	532

9.1.19.2	Accuracy requirements for measurements on SCC	532

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	532

9.1.20	SFN and Subframe Time Difference (SSTD)	533

9.1.20.1	SSTD Accuracy Requirement	533

9.1.20.2	SSTD Measurement Report Mapping	533

9.1.21	Measurement accuracy for UE category M1	534

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	534

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	535

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	536

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	537

9.1.21.5	RSRP Measurement Report Mapping	538

9.1.22	Measurement accuracy for UE Category NB1	539

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	539

9.1.22.2	Void	539

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode	539

9.1.22.4	Void	540

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	540

9.1.22.6	Void	541

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	541

9.1.22.8	Void	541

9.1.23	Power Headroom for UE Category NB1	541

9.1.23.1	Period	541

9.1.23.2	Reporting Delay	541

9.1.23.3	Report Mapping for UE Category NB1	542

9.1.23.3.1	Report Mapping for UE Category NB1 in normal coverage	542

9.1.23.3.2	Report Mapping for UE Category NB1 in enhanced coverage	542

9.2	UTRAN FDD Measurements	542

9.2.1	UTRAN FDD CPICH RSCP	542

9.2.2	Void	543

9.2.3	UTRAN FDD CPICH Ec/No	543

9.3	UTRAN TDD Measurements	543

9.3.1	UTRAN TDD P-CCPCH RSCP	544

9.3.2	Void	544

9.3.3	Void	544

9.4	GSM Measurements	544

9.4.1	GSM carrier RSSI	544

9.5	CDMA2000 1x RTT Measurements	545

9.5.1	CDMA2000 1x RTT Pilot Strength	545

9.6	PCMAX,c	545

9.6.1	Report Mapping	545

9.6.2	Estimation Period	545

9.6.3	Reporting Delay	545

9.7	IEEE802.11 Measurements	546

9.7.1	WLAN RSSI	546

9.7.2	WLAN RSSI Measurement Report Mapping	546

9.8	MBSFN Measurements	546

9.8.1	Introduction	546

9.8.2	MBSFN RSRP	546

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	546

9.8.2.2	MBSFN RSRP measurement report mapping	547

9.8.3	MBSFN RSRQ	547

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	547

9.8.3.2	MBSFN RSRQ measurement report mapping	548

9.8.4	MCH BLER	548

9.8.4.1	Measurement report mapping for MCH BLER	548

9.8.4.2	Measurement report mapping for MCH Block Number	549

9.9	ProSe Measurements	550

9.9.1	Introduction	550

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	550

9.9.2.1	Absolute S-RSRP Accuracy	550

9.9.2.2	Relative Accuracy of S-RSRP	551

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	552

9.9.3.1	Absolute SD-RSRP Accuracy	552

9.9.3.2	Relative Accuracy of SD-RSRP	553

9.10	V2X Measurements	554

9.10.1	Introduction	554

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	554

9.10.2.1	Absolute S-RSRP Accuracy	554

9.10.2.2	Relative Accuracy of S-RSRP	554

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	555

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	555

9.10.4	S-RSSI Measurement Accuracy Requirements	556

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	556

9.10.4.2	Intra-frequency relative S-RSSI measurement accuracy requirements	556

10	Measurements Performance Requirements for E-UTRAN	558

10.1	Received Interference Power	558

10.1.1	Absolute accuracy requirement	558

10.1.2	Relative accuracy requirement	558

10.1.3	Received Interference Power measurement report mapping	558

10.2	Angle of Arrival (AOA)	558

10.2.1	Range/mapping	558

10.3	Timing Advance (TADV)	559

10.3.1	Report mapping	559

11	ProSe Requirements in Any Cell Selection state	560

11.1	Introduction	560

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	560

11.2.1	Introduction	560

11.2.2	ProSe UE transmission timing	560

11.3	Initiation/Cease of SLSS Transmissions	560

11.3.1	Introduction	560

11.3.2	Requirements	561

11.4	Measurements for ProSe in Any Cell Selection State	561

11.4.1	Introduction	561

11.4.2	Requirements	561

11.4.2.1	E-UTRA FDD	561

11.4.2.2	E-UTRA TDD	562

11.5	Selection / Reselection of ProSe Synchronization Reference	562

11.5.1	Introduction	562

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	562

11.5.2.1	Introduction	562

11.5.2.2	Requirements	562

11.6	Void	563

11.7	Selection / Reselection of ProSe relay UE	563

11.7.1	Introduction	563

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	563

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	564

12.1	Introduction	564

12.2	Transmit Timing	564

12.2.1	GNSS as timing reference	564

12.3	Interruption	564

12.4	Reliability of GNSS signal	564

13	V2X Requirements	566

13.1	Introduction	566

13.2	UE Transmit Timing	566

13.2.1	Introduction	566

13.2.2	GNSS as timing reference	566

13.2.3	eNB as timing reference	566

13.2.4	SyncRef UE as timing reference	567

13.3	Initiation/Cease of SLSS Transmissions	567

13.3.1	Introduction	567

13.3.1.1	Initiation/Cease of SLSS transmissions with eNB as timing reference	567

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as timing reference	568

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as timing reference	568

13.4	Selection / Reselection of V2X Synchronization Reference	568

13.5	Autonomous Resource Selection/Reselection measurements	569

13.5.1	Introduction	569

13.5.2	PSSCH-RSRP measurements	569

13.5.3	S-RSSI measurements	569

13.6	Congestion Control measurements	569

13.7	Interruption	570

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	570

13.7.2	V2X Sidelink Communication Dropping due to synchronization source change	570

Annex A (normative): Test Cases	571

A.1	Purpose of annex	571

A.2	Requirement classification for statistical testing	571

A.2.1	Types of requirements in TS 36.133	571

A.2.1.1	Time and delay requirements on UE higher layer actions	571

A.2.1.2	Measurements of power levels, relative powers and time	571

A.2.1.3	Implementation requirements	572

A.2.1.4	Physical layer timing requirements	572

A.3	RRM test configurations	573

A.3.1	Reference Measurement Channels	573

A.3.1.1	PDSCH	573

A.3.1.1.1	FDD	573

A.3.1.1.2	TDD	574

A.3.1.1.3	FDD for UE category 0	576

A.3.1.1.4	HD-FDD for UE category 0	577

A.3.1.1.5	TDD for UE category 0	578

A.3.1.1.6	Frame Structure 3	579

A.3.1.2	PCFICH/PDCCH/PHICH	580

A.3.1.2.1	FDD	580

A.3.1.2.2	TDD	580

A.3.1.2.3	HD-FDD for UE category 0	581

A.3.1.2.4	FS 3	581

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	582

A.3.1.3.1	FDD in CEModeA	582

A.3.1.3.2	HD-FDD in CEModeA	582

A.3.1.3.3	TDD in CEModeA	583

A.3.1.3.4	FDD in CEModeB	583

A.3.1.3.5	HD-FDD in CEModeB	584

A.3.1.3.6	TDD in CEModeB	584

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	585

A.3.1.4.1	FDD in CEModeA	585

A.3.1.4.2	HD-FDD in CEModeA	586

A.3.1.4.3	TDD in CEModeA	587

A.3.1.4.4	FDD in CEModeB	588

A.3.1.4.5	 HD-FDD in CEModeB	589

A.3.1.4.6	TDD in CEModeB	590

A.3.1.5	NPDSCH Reference Channel for UE category NB1	591

A.3.1.5.1	HD-FDD in-band operation	591

A.3.1.5.2	Void	592

A.3.1.5.3	HD-FDD standalone operation	592

A.3.1.5.4	Void	593

A.3.1.5.5	HD-FDD guard band operation	593

A.3.1.5.6	Void	593

A.3.1.6	NPDCCH Reference Channel for UE category NB1	593

A.3.1.6.1	HD-FDD in-band operation	593

A.3.1.6.2	Void	594

A.3.1.6.3	HD-FDD standalone operation	594

A.3.1.6.4	Void	594

A.3.1.6.5	HD-FDD guard band operation	594

A.3.1.6.6	Void	594

A.3.2	OFDMA Channel Noise Generator (OCNG)	594

A.3.2.1	OCNG Patterns for FDD	594

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	595

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	596

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	597

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	598

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	598

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	599

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	599

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	599

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	600

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	601

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	601

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	602

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	602

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	603

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	603

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	604

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	604

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	605

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	605

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	606

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	606

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	607

A.3.2.2	OCNG Patterns for TDD	607

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	608

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	609

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	609

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	610

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	610

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	611

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	612

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	613

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	613

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	614

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	615

A.3.2.3	OCNG Patterns for Narrowband IoT	615

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	616

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	617

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	618

A.3.3	Reference DRX Configurations	618

A.3.4	ABS Transmission Configurations	619

A.3.4.1	Non-MBSFN ABS Transmission Configurations	619

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	619

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	619

A.3.4.2	MBSFN ABS Transmission Configurations	620

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	620

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	621

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	621

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	621

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	622

A.3.6.1	Introduction	622

A.3.7	Test Cases with Different Channel Bandwidths	622

A.3.7.1	Introduction	622

A.3.7.2	Principle of testing	622

A.3.8	Antenna Configuration	622

A.3.8.1	Antenna connection for 4 Rx capable UEs	622

A.3.8.1.1	Introduction	622

A.3.8.1.2	Principle of testing	622

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	624

A.3.9.1	Introduction	624

A.3.9.2	Principle of testing	624

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	625

A.3.10.1	Introduction	625

A.3.10.2	Principle of testing	625

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	625

A.3.11.1	Introduction	625

A.3.11.2	Principle of Testing	625

A.3.12	Proximity-based Services	625

A.3.12.1	Introduction	625

A.3.12.2	Reference DRX configurations for ProSe tests	626

A.3.12.3	Test Cases with Different Channel Bandwidths	626

A.3.12.3.1	Introduction	626

A.3.12.3.2	Principle of testing	626

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	627

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	633

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	635

A.3.12.6.1	FDD	635

A.3.12.7	Reference measurement channels for ProSe Direct Communication	635

A.3.12.7.1	FDD	635

A.3.12.8	ProSe Receive Traffic Generator	636

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	636

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	636

A.3.13	Time Offset between Cells	637

A.3.13.1	Introduction	637

A.3.13.2	Definition	637

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	637

A.3.14.1	Introduction	637

A.3.14.2	Principle of testing	637

A.3.15	Dual connectivity test cases with different combination of duplex mode	637

A.3.15.1	Introduction	637

A.3.15.2	Principle of testing	637

A.3.16	Reference PRACH Configurations	638

A.3.17	Listen before talk model	638

A.3.17.1	Introduction	638

A.3.17.2	Definition	639

A.3.18	Reference NPRACH Configurations	639

A.3.19	Dual connectivity test cases with different bandwidth combinations	640

A.3.19.1	Introduction	640

A.3.19.2	Principle of testing	640

A.3.20	Category M1 UE Test Cases	640

A.3.20.1	Introduction	640

A.3.20.2	Principle of Cat-M1 UE Testing	641

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	641

A.3.21.1	Introduction	641

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	642

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	643

A.4	E-UTRAN RRC_IDLE state	643

A.4.2	Cell Re-Selection	643

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	643

A.4.2.1.1	Test Purpose and Environment	643

A.4.2.1.2	Test Requirements	645

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	646

A.4.2.2.1	Test Purpose and Environment	646

A.4.2.2.2	Test Requirements	647

A.4.2.2MF1	MF Intra frequency case	648

A.4.2.2MF1.1	Test Purpose and Environment	648

A.4.2.2MF1.2	Test Requirements	650

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	650

A.4.2.3.1	Test Purpose and Environment	650

A.4.2.3.2	Test Requirements	652

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	653

A.4.2.4.1	Test Purpose and Environment	653

A.4.2.4.2	Test Requirements	654

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	655

A.4.2.5.1	Test Purpose and Environment	655

A.4.2.5.2	Test Requirements	656

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	657

A.4.2.6.1	Test Purpose and Environment	657

A.4.2.6.2	Test Requirements	658

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	659

A.4.2.7.1	Test Purpose and Environment	659

A.4.2.7.2	Test Requirements	660

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	661

A.4.2.8.1	Test Purpose and Environment	661

A.4.2.8.2	Test Requirements	662

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	663

A.4.2.9.1	Test Purpose and Environment	663

A.4.2.9.2	Test Requirements	663

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	663

A.4.2.10.1	Test Purpose and Environment	663

A.4.2.10.2	Test Requirements	666

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	666

A.4.2.11.1	Test Purpose and Environment	666

A.4.2.11.2	Test Requirements	669

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	669

A.4.2.12.1	Test Purpose and Environment	669

A.4.2.12.2	Test Requirements	671

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	672

A.4.2.13.1	Test Purpose and Environment	672

A.4.2.13.2	Test Requirements	673

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	674

A.4.2.14.1	Test Purpose and Environment	674

A.4.2.14.2	Test Requirements	675

A.4.2.15	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	676

A.4.2.15.1	Test Purpose and Environment	676

A.4.2.15.2	Test Requirements	677

A.4.2.16	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	678

A.4.2.16.1	Test Purpose and Environment	678

A.4.2.16.2	Test Requirements	679

A.4.2.17	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	680

A.4.2.17.1	Test Purpose and Environment	680

A.4.2.17.2	Test Requirements	681

A.4.2.18	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	682

A.4.2.18.1	Test Purpose and Environment	682

A.4.2.18.2	Test Requirements	684

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	685

A.4.2.19.1	Test Purpose and Environment	685

A.4.2.19.2	Test Requirements	687

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	688

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	688

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	688

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	690

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	693

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	696

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	697

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	701

A.4.3.2.1	Test Purpose and Environment	701

A.4.3.2.2	Test Requirements	703

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	703

A.4.3.2A.1	Test Purpose and Environment	703

A.4.3.2A.2	Test Requirements	707

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	707

A.4.3.3.1	Test Purpose and Environment	707

A.4.3.3.2	Test Requirements	710

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	710

A.4.3.3A.1	Test Purpose and Environment	710

A.4.3.3A.2	Test Requirements	714

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	715

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	715

A.4.3.4.1.2	Test Requirements	717

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	717

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	720

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	722

A.4.4	E-UTRAN to GSM Cell Re-Selection	726

A.4.4.1	E-UTRAN FDD – GSM:	726

A.4.4.1.1	Test Purpose and Environment	726

A.4.4.1.2	Test Requirements	728

A.4.4.2	E-UTRAN TDD – GSM:	728

A.4.4.2.1	Test Purpose and Environment	728

A.4.4.2.2	Test Requirements	730

A.4.5	E-UTRAN to HRPD Cell Re-Selection	731

A.4.5.1	E-UTRAN FDD – HRPD	731

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	731

A.4.5.2	E-UTRAN TDD – HRPD	733

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	733

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	736

A.4.6.1	E-UTRAN FDD – cdma2000 1X	736

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	736

A.4.6.1.1.1	Test Purpose and Environment	736

A.4.6.1.1.2	Test Requirements	739

A.4.6.2	E-UTRAN TDD – cdma2000 1X	739

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	739

A.5	E-UTRAN RRC CONNECTED Mode Mobility	742

A.5.1	E-UTRAN Handover	742

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	742

A.5.1.1.1	Test Purpose and Environment	742

A.5.1.1.2	Test Requirements	744

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	744

A.5.1.2.1	Test Purpose and Environment	744

A.5.1.2.2	Test Requirements	746

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	746

A.5.1.3.1	Test Purpose and Environment	746

A.5.1.3.2	Test Requirements	748

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	748

A.5.1.4.1	Test Purpose and Environment	748

A.5.1.4.2	Test Requirements	750

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	750

A.5.1.5.1	Test Purpose and Environment	750

A.5.1.5.2	Test Requirements	752

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	752

A.5.1.6.1	Test Purpose and Environment	752

A.5.1.6.2	Test Requirements	754

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	754

A.5.1.7.1	Test Purpose and Environment	754

A.5.1.7.2	Test Requirements	757

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	757

A.5.1.8.1	Test Purpose and Environment	757

A.5.1.8.2	Test Requirements	760

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	760

A.5.1.9.1	Test Purpose and Environment	760

A.5.1.9.2	Test Requirements	761

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	761

A.5.1.10.1	Test Purpose and Environment	761

A.5.1.10.2	Test Requirements	763

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	763

A.5.1.11.1	Test Purpose and Environment	763

A.5.1.11.2	Test Requirements	765

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	765

A.5.1.12.1	Test Purpose and Environment	765

A.5.1.12.2	Test Requirements	767

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	767

A.5.1.13.1	Test Purpose and Environment	767

A.5.1.13.2	Test Requirements	769

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	770

A.5.1.14.1	Test Purpose and Environment	770

A.5.1.14.2	Test Requirements	771

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	772

A.5.1.15.1	Test Purpose and Environment	772

A.5.1.15.2	Test Requirements	773

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	774

A.5.1.16.1	Test Purpose and Environment	774

A.5.1.16.2	Test Requirements	775

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	776

A.5.1.17.1	Test Purpose and Environment	776

A.5.1.17.2	Test Requirements	777

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	778

A.5.1.18.1	Test Purpose and Environment	778

A.5.1.18.2	Test Requirements	779

A.5.1MF1	E-UTRAN MF - MF Intra frequency handover	780

A.5.1MF1.1	Test Purpose and Environment	780

A.5.1MF1.2	Test Requirements	782

A.5.1MF2	E-UTRAN MF – MF Inter frequency handover	783

A.5.1MF2.1	Test Purpose and Environment	783

A.5.1MF2.2	Test Requirements	785

A.5.2	E-UTRAN Handover to other RATs	786

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	786

A.5.2.1.1	Test Purpose and Environment	786

A.5.2.1.2	Test Requirements	787

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	788

A.5.2.2.1	Test Purpose and Environment	788

A.5.2.2.2	Test Requirements	791

A.5.2.3	E-UTRAN FDD- GSM Handover	791

A.5.2.3.1	Test Purpose and Environment	791

A.5.2.3.2	Test Requirements	792

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	793

A.5.2.4.1	Test Purpose and Environment	793

A.5.2.4.2	Test Requirements	795

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	795

A.5.2.5.1	Test Purpose and Environment	795

A.5.2.5.2	Test Requirements	798

A.5.2.6	E-UTRAN TDD - GSM Handover	798

A.5.2.6.1	Test Purpose and Environment	798

A.5.2.6.2	Test Requirements	800

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	801

A.5.2.7.1	Test Purpose and Environment	801

A.5.2.7.2	Test Requirements	803

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	803

A.5.2.8.1	Test Purpose and Environment	803

A.5.2.8.2	Test Requirements	804

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	805

A.5.2.9.1	Test Purpose and Environment	805

A.5.2.9.2	Test Requirements	806

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	807

A.5.2.10.1	Test Purpose and Environment	807

A.5.2.10.2	Test Requirements	809

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	809

A.5.2.10A.1	Test Purpose and Environment	809

A.5.2.10A.2	Test Requirements	812

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	812

A.5.2.10B.1	Test Purpose and Environment	812

A.5.2.10B.2	Test Requirements	815

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	815

A.5.2.11.1	Test Purpose and Environment	815

A.5.2.11.2	Test Requirements	816

A.5.3	E-UTRAN Handover to Non-3GPP RATs	816

A.5.3.1	E-UTRAN FDD – HRPD Handover	816

A.5.3.1.1	Test Purpose and Environment	816

A.5.3.1.2	Test Requirements	819

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	819

A.5.3.2.1	Test Purpose and Environment	819

A.5.3.2.2	Test Requirements	822

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	822

A.5.3.3.1	Test Purpose and Environment	822

A.5.3.3.2	Test Requirements	824

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	825

A.5.3.4.1	Test Purpose and Environment	825

A.5.3.4.2	Test Requirements	826

A.5.3.5	E-UTRAN TDD – HRPD Handover	827

A.5.3.5.1	Test Purpose and Environment	827

A.5.3.5.2	Test Requirements	829

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	830

A.5.3.6.1	Test Purpose and Environment	830

A.5.3.6.2	Test Requirements	833

A.6	RRC Connection Control	833

A.6.1	RRC Re-establishment	833

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	833

A.6.1.1.1	Test Purpose and Environment	833

A.6.1.1.2	Test Requirements	834

A.6.1MF1	MF Intra-frequency RRC Re-establishment	835

A.6.1MF1.1	Test Purpose and Environment	835

A.6.1MF1.2	Test Requirements	837

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	838

A.6.1.2.1	Test Purpose and Environment	838

A.6.1.2.2	Test Requirements	839

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	840

A.6.1.3.1	Test Purpose and Environment	840

A.6.1.3.2	Test Requirements	841

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	842

A.6.1.4.1	Test Purpose and Environment	842

A.6.1.4.2	Test Requirements	843

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	844

A.6.1.5.1	Test Purpose and Environment	844

A.6.1.5.2	Test Requirements	844

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	844

A.6.1.6.1	Test Purpose and Environment	844

A.6.1.6.2	Test Requirements	846

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	847

A.6.1.7.1	Test Purpose and Environment	847

A.6.1.7.2	Test Requirements	848

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	849

A.6.1.8.1	Test Purpose and Environment	849

A.6.1.8.2	Test Requirements	850

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	851

A.6.1.9.1	Test Purpose and Environment	851

A.6.1.9.2	Test Requirements	852

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	853

A.6.1.10.1	Test Purpose and Environment	853

A.6.1.10.2	Test Requirements	854

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	855

A.6.1.11.1	Test Purpose and Environment	855

A.6.1.11.2	Test Requirements	856

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	857

A.6.1.12.1	Test Purpose and Environment	857

A.6.1.12.2	Test Requirements	858

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	859

A.6.1.13.1	Test Purpose and Environment	859

A.6.1.13.2	Test Requirements	860

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	861

A.6.1.14.1	Test Purpose and Environment	861

A.6.1.14.2	Test Requirements	862

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	863

A.6.1.15.1	Test Purpose and Environment	863

A.6.1.15.2	Test Requirements	865

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	866

A.6.1.16.1	Test Purpose and Environment	866

A.6.1.16.2	Test Requirements	868

A.6.2	Random Access	869

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	869

A.6.2.1.1	Test Purpose and Environment	869

A.6.2.1.2	Test Requirements	870

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	871

A.6.2.2.1	Test Purpose and Environment	871

A.6.2.2.2	Test Requirements	872

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	873

A.6.2.3.1	Test Purpose and Environment	873

A.6.2.3.2	Test Requirements	875

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	876

A.6.2.4.1	Test Purpose and Environment	876

A.6.2.4.2	Test Requirements	878

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	878

A.6.2.5.1	Test Purpose and Environment	878

A.6.2.5.2	Test Requirements	879

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	879

A.6.2.6.1	Test Purpose and Environment	879

A.6.2.6.2	Test Requirements	879

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	880

A.6.2.7.1	Test Purpose and Environment	880

A.6.2.7.2	Test Requirements	882

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	883

A.6.2.8.1	Test Purpose and Environment	883

A.6.2.8.2	Test Requirements	885

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	886

A.6.2.9.1	Test Purpose and Environment	886

A.6.2.9.2	Test Requirements	888

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	890

A.6.2.10.1	Test Purpose and Environment	890

A.6.2.10.2	Test Requirements	892

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	894

A.6.2.11.1	Test Purpose and Environment	894

A.6.2.11.2	Test Requirements	896

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	898

A.6.2.12.1	Test Purpose and Environment	898

A.6.2.12.2	Test Requirements	900

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	902

A.6.2.13.1	Test Purpose and Environment	902

A.6.2.13.2	Test Requirements	904

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	906

A.6.2.14.1	Test Purpose and Environment	906

A.6.2.14.2	Test Requirements	908

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	910

A.6.2.15.1	Test Purpose and Environment	910

A.6.2.15.2	Test Requirements	912

A.6.2MF1	MF – Contention Based Random Access Test	914

A.6.2MF1.1	Test Purpose and Environment	914

A.6.2MF1.2	Test Requirements	916

A.6.2MF1.2.1	Random Access Response Reception	916

A.6.2MF1.2.2	No Random Access Response reception	916

A.6.2MF1.2.3	Receiving a NACK on msg3	916

A.6.2MF1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	917

A.6.2MF1.2.5	Reception of a Correct Message over Temporary C-RNTI	917

A.6.2MF1.2.6	Contention Resolution Timer expiry	917

A.6.2MF2	MF – Non-Contention Based Random Access Test	917

A.6.2MF2.1	Test Purpose and Environment	917

A.6.2MF2.2	Test Requirements	919

A.6.2MF2.2.1	Random Access Response Reception	919

A.6.2MF2.2.2	No Random Access Response Reception	919

A.6.3	RRC Connection Release with Redirection	919

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	919

A.6.3.1.1	Test Purpose and Environment	919

A.6.3.1.2	Test Requirements	921

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	921

A.6.3.2.1	Test Purpose and Environment	921

A.6.3.2.2	Test Requirements	924

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	924

A.6.3.3.1	Test Purpose and Environment	924

A.6.3.3.2	Test Requirements	925

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	926

A.6.3.4.1	Test Purpose and Environment	926

A.6.3.4.2	Test Requirements	927

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	928

A.6.3.5.1	Test Purpose and Environment	928

A.6.3.5.2	Test Requirements	930

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	930

A.6.3.6.1	Test Purpose and Environment	930

A.6.3.6.2	Test Requirements	932

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	932

A.6.3.7.1	Test Purpose and Environment	932

A.6.3.7.2	Test Requirements	935

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	935

A.6.3.8.1	Test Purpose and Environment	935

A.6.3.8.2	Test Requirements	938

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	938

A.6.3.9.1	Test Purpose and Environment	938

A.6.3.9.2	Test Requirements	940

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	940

A.6.3.10.1	Test Purpose and Environment	940

A.6.3.10.2	Test Requirements	942

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	942

A.6.3.11.1	Test Purpose and Environment	942

A.6.3.11.2	Test Requirements	944

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	944

A.6.3.12.1	Test Purpose and Environment	944

A.6.3.12.2	Test Requirements	947

A.7	Timing and Signalling Characteristics	948

A.7.1	UE Transmit Timing	948

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	948

A.7.1.1.1	Test Purpose and Environment	948

A.7.1.1.2	Test Requirements	949

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	950

A.7.1.2.1	Test Purpose and Environment	950

A.7.1.2.2	Test Requirements	952

A.7.1.2MF1	E-UTRAN MF - UE Transmit Timing Accuracy Tests	953

A.7.1.2MF1.1	Test Purpose and Environment	953

A.7.1.2MF1.2	Test Requirements	955

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	955

A.7.1.3.1	Test Purpose and Environment	955

A.7.1.3.2	Test Requirements	957

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	958

A.7.1.4.1	Test Purpose and Environment	958

A.7.1.4.2	Test Requirements	960

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	961

A.7.1.4A.1	Test Purpose and Environment	961

A.7.1.4A.2	Test Requirements	961

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	961

A.7.1.5.1	Test Purpose and Environment	961

A.7.1.5.2	Test Requirements	962

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	962

A.7.1.6.1	Test Purpose and Environment	962

A.7.1.6.2	Test Requirements	964

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	965

A.7.1.7.1	Test Purpose and Environment	965

A.7.1.7.2	Test Requirements	966

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	967

A.7.1.7A.1	Test Purpose and Environment	967

A.7.1.7A.2	Test Requirements	967

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	967

A.7.1.7B.1	Test Purpose and Environment	967

A.7.1.7B.2	Test Requirements	968

A.7.1.8	Void	968

A.7.1.8.1	Void	968

A.7.1.8.2	Void	968

A.7.1.9	Void	968

A.7.1.9.1	Void	968

A.7.1.9.2	Void	968

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	968

A.7.1.10.1	Test Purpose and Environment	968

A.7.1.10.2	Test Requirements	970

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	970

A.7.1.11.1	Test Purpose and Environment	970

A.7.1.11.2	Test Requirements	972

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	972

A.7.1.12.1	Test Purpose and Environment	972

A.7.1.12.2	Test Requirements	974

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	974

A.7.1.13.1	Test Purpose and Environment	974

A.7.1.13.2	Test Requirements	976

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	977

A.7.1.14.1	Test Purpose and Environment	977

A.7.1.14.2	Test Requirements	978

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	978

A.7.1.15.1	Test Purpose and Environment	978

A.7.1.15.2	Test Requirements	979

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	980

A.7.1.16.1	Test Purpose and Environment	980

A.7.1.16.2	Test Requirements	981

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	982

A.7.1.17.1	Test Purpose and Environment	982

A.7.1.17.2	Test Requirements	984

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	984

A.7.1.18.1	Test Purpose and Environment	984

A.7.1.18.2	Test Requirements	986

A.7.2	UE Timing Advance	986

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	986

A.7.2.1.1	Test Purpose and Environment	986

A.7.2.1.2	Test Requirements	988

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	988

A.7.2.2.1	Test Purpose and Environment	988

A.7.2.2.2	Test Requirements	990

A.7.2.2MF1	MF – UE Timing Advance Adjustment Accuracy Test	990

A.7.2.2MF1.1	Test Purpose and Environment	990

A.7.2.2MF1.2	Test Requirements	992

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	992

A.7.2.3.1	Test Purpose and Environment	992

A.7.2.3.2	Test Requirements	992

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	992

A.7.2.4.1	Test Purpose and Environment	992

A.7.2.4.2	Test Requirements	995

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	995

A.7.2.5.1	Test Purpose and Environment	995

A.7.2.5.2	Test Requirements	997

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	997

A.7.2.5A.1	Test Purpose and Environment	997

A.7.2.5A.2	Test Requirements	998

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	998

A.7.2.5B.1	Test Purpose and Environment	998

A.7.2.5B.2	Test Requirements	998

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	999

A.7.2.6.1	Test Purpose and Environment	999

A.7.2.6.2	Test Requirements	1001

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1001

A.7.2.7.1	Test Purpose and Environment	1001

A.7.2.7.2	Test Requirements	1003

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1003

A.7.2.8.1	Test Purpose and Environment	1003

A.7.2.8.2	Test Requirements	1005

A.7.2.9	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage	1005

A.7.2.9.1	Test Purpose and Environment	1005

A.7.2.9.2	Test Requirements	1006

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1007

A.7.2.10.1	Test Purpose and Environment	1007

A.7.2.10.2	Test Requirements	1008

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1008

A.7.2.11.1	Test Purpose and Environment	1008

A.7.2.11.2	Test Requirements	1010

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1010

A.7.2.12.1	Test Purpose and Environment	1010

A.7.2.12.2	Test Requirements	1012

A.7.3	Radio Link Monitoring	1013

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1013

A.7.3.1.1	Test Purpose and Environment	1013

A.7.3.1.2	Test Requirements	1017

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1017

A.7.3.2.1	Test Purpose and Environment	1017

A.7.3.2.2	Test Requirements	1020

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1020

A.7.3.3.1	Test Purpose and Environment	1020

A.7.3.3.2	Test Requirements	1024

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1024

A.7.3.4.1	Test Purpose and Environment	1024

A.7.3.4.2	Test Requirements	1027

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1027

A.7.3.5.1	Test Purpose and Environment	1027

A.7.3.5.2	Test Requirements	1030

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1031

A.7.3.6.1	Test Purpose and Environment	1031

A.7.3.6.2	Test Requirements	1034

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1034

A.7.3.7.1	Test Purpose and Environment	1034

A.7.3.7.2	Test Requirements	1037

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1037

A.7.3.8.1	Test Purpose and Environment	1037

A.7.3.8.2	Test Requirements	1040

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1040

A.7.3.9.1	Test Purpose and Environment	1040

A.7.3.9.2	Test Requirements	1043

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1043

A.7.3.10.1	Test Purpose and Environment	1043

A.7.3.10.2	Test Requirements	1046

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1046

A.7.3.11.1	Test Purpose and Environment	1046

A.7.3.11.2	Test Requirements	1050

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1050

A.7.3.12.1	Test Purpose and Environment	1050

A.7.3.12.2	Test Requirements	1054

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1054

A.7.3.13.1	Test Purpose and Environment	1054

A.7.3.13.2	Test Requirements	1057

A.7.3.14	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1057

A.7.3.14.1	Test Purpose and Environment	1057

A.7.3.14.2	Test Requirements	1060

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1060

A.7.3.15.1	Test Purpose and Environment	1060

A.7.3.15.2	Test Requirements	1063

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1063

A.7.3.16.1	Test Purpose and Environment	1063

A.7.3.16.2	Test Requirements	1066

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1066

A.7.3.17.1	Test Purpose and Environment	1066

A.7.3.17.2	Test Requirements	1069

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1069

A.7.3.18.1	Test Purpose and Environment	1069

A.7.3.18.2	Test Requirements	1072

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1072

A.7.3.19.1	Test Purpose and Environment	1072

A.7.3.19.2	Test Requirements	1076

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1076

A.7.3.20.1	Test Purpose and Environment	1076

A.7.3.20.2	Test Requirements	1080

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1080

A.7.3.21.1	Test Purpose and Environment	1080

A.7.3.21.2	Test Requirements	1084

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1084

A.7.3.22.1	Test Purpose and Environment	1084

A.7.3.22.2	Test Requirements	1088

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1088

A.7.3.23.1	Test Purpose and Environment	1088

A.7.3.23.2	Test Requirements	1089

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1089

A.7.3.24.1	Test Purpose and Environment	1089

A.7.3.24.2	Test Requirements	1090

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1090

A.7.3.25.1	Test Purpose and Environment	1090

A.7.3.25.2	Test Requirements	1091

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1091

A.7.3.26.1	Test Purpose and Environment	1091

A.7.3.26.2	Test Requirements	1094

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1094

A.7.3.27.1	Test Purpose and Environment	1094

A.7.3.27.2	Test Requirements	1097

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1097

A.7.3.28.1	Test Purpose and Environment	1097

A.7.3.28.2	Test Requirements	1100

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1100

A.7.3.29.1	Test Purpose and Environment	1100

A.7.3.29.2	Test Requirements	1103

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1103

A.7.3.30.1	Test Purpose and Environment	1103

A.7.3.30.2	Test Requirements	1106

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1106

A.7.3.31.1	Test Purpose and Environment	1106

A.7.3.31.2	Test Requirements	1109

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1109

A.7.3.32.1	Test Purpose and Environment	1109

A.7.3.32.2	Test Requirements	1112

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1112

A.7.3.33.1	Test Purpose and Environment	1112

A.7.3.33.2	Test Requirements	1115

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1115

A.7.3.34.1	Test Purpose and Environment	1115

A.7.3.34.2	Test Requirements	1118

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1118

A.7.3.35.1	Test Purpose and Environment	1118

A.7.3.35.2	Test Requirements	1121

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1121

A.7.3.36.1	Test Purpose and Environment	1121

A.7.3.36.2	Test Requirements	1124

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1124

A.7.3.37.1	Test Purpose and Environment	1124

A.7.3.37.2	Test Requirements	1127

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1127

A.7.3.38.1	Test Purpose and Environment	1127

A.7.3.38.2	Test Requirements	1130

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1130

A.7.3.39.1	Test Purpose and Environment	1130

A.7.3.39.2	Test Requirements	1133

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1134

A.7.3.40.1	Test Purpose and Environment	1134

A.7.3.40.2	Test Requirements	1136

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1137

A.7.3.41.1	Test Purpose and Environment	1137

A.7.3.41.2	Test Requirements	1140

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1140

A.7.3.42.1	Test Purpose and Environment	1140

A.7.3.42.2	Test Requirements	1143

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1143

A.7.3.43.1	Test Purpose and Environment	1143

A.7.3.43.2	Test Requirements	1146

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1146

A.7.3.44.1	Test Purpose and Environment	1146

A.7.3.44.2	Test Requirements	1149

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1150

A.7.3.45.1	Test Purpose and Environment	1150

A.7.3.45.2	Test Requirements	1152

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1153

A.7.3.46.1	Test Purpose and Environment	1153

A.7.3.46.2	Test Requirements	1156

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1156

A.7.3.47.1	Test Purpose and Environment	1156

A.7.3.47.2	Test Requirements	1159

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1159

A.7.3.48.1	Test Purpose and Environment	1159

A.7.3.48.2	Test Requirements	1162

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1162

A.7.3.49.1	Test Purpose and Environment	1162

A.7.3.49.2	Test Requirements	1165

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1165

A.7.3.50.1	Test Purpose and Environment	1165

A.7.3.50.2	Test Requirements	1168

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1168

A.7.3.51.1	Test Purpose and Environment	1168

A.7.3.51.2	Test Requirements	1171

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1171

A.7.3.52.1	Test Purpose and Environment	1171

A.7.3.52.2	Test Requirements	1174

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1174

A.7.3.53.1	Test Purpose and Environment	1174

A.7.3.53.2	Test Requirements	1177

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1177

A.7.3.54.1	Test Purpose and Environment	1177

A.7.3.54.2	Test Requirements	1180

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1180

A.7.3.55.1	Test Purpose and Environment	1180

A.7.3.55.2	Test Requirements	1183

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1183

A.7.3.56.1	Test Purpose and Environment	1183

A.7.3.56.2	Test Requirements	1186

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1186

A.7.3.57.1	Test Purpose and Environment	1186

A.7.3.57.2	Test Requirements	1189

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1189

A.7.3.58.1	Test Purpose and Environment	1189

A.7.3.58.2	Test Requirements	1192

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1192

A.7.3.59.1	Test Purpose and Environment	1192

A.7.3.59.2	Test Requirements	1195

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1195

A.7.3.60.1	Test Purpose and Environment	1195

A.7.3.60.2	Test Requirements	1199

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1199

A.7.3.61.1	Test Purpose and Environment	1199

A.7.3.61.2	Test Requirements	1203

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1203

A.7.3.62.1	Test Purpose and Environment	1203

A.7.3.62.2	Test Requirements	1207

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1207

A.7.3.63.1	Test Purpose and Environment	1207

A.7.3.63.2	Test Requirements	1211

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1211

A.7.3.64.1	Test Purpose and Environment	1211

A.7.3.64.2	Test Requirements	1214

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1215

A.7.3.65.1	Test Purpose and Environment	1215

A.7.3.65.2	Test Requirements	1218

A.7.3MF1	Radio Link Monitoring in MF	1219

A.7.3MF1.1	MF Radio Link Monitoring Test for Out-of-sync	1219

A.7.3MF1.1.1	Test Purpose and Environment	1219

A.7.3MF1.1.2	Test Requirements	1221

A.7.3MF1.2	MF Radio Link Monitoring Test for In-sync	1222

A.7.3MF1.2.1	Test Purpose and Environment	1222

A.7.3MF1.2.2	Test Requirements	1224

A.7.3MF1.3	MF Radio Link Monitoring Test for DRS Loss Tolerance	1224

A.7.3MF1.3.1	Test Purpose and Environment	1224

A.7.3MF1.3.2	Test Requirements	1226

A.7.3MF1.4	MF Radio Link Monitoring Test for Extended DRS Failure	1227

A.7.3MF1.4.1	Test Purpose and Environment	1227

A.7.3MF1.4.2	Test Requirements	1229

A.7.3MF1.5	MF Radio Link Monitoring Test for Floating DRS Tolerance	1229

A.7.3MF1.5.1	Test Purpose and Environment	1229

A.7.3MF1.5.2	Test Requirements	1231

A.7.3MF1.6	MF Radio Link Monitoring Test for Out-of-sync in DRX	1232

A.7.3MF1.6.1	Test Purpose and Environment	1232

A.7.3MF1.6.2	Test Requirements	1234

A.7.3MF1.7	MF Radio Link Monitoring Test for In-sync in DRX	1234

A.7.3MF1.7.1	Test Purpose and Environment	1234

A.7.3MF1.7.2	Test Requirements	1237

A.7.4	Interruption for Dual Connectivity	1237

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1237

A.7.4.1.1	Test Purpose and Environment	1237

A.7.4.1.2	Test Requirements	1239

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1239

A.7.4.2.1	Test Purpose and Environment	1239

A.7.4.2.2	Test Requirements	1241

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1241

A.7.4.3.1	Test Purpose and Environment	1241

A.7.4.3.2	Test Requirements	1243

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1243

A.7.4.4.1	Test Purpose and Environment	1243

A.7.4.4.2	Test Requirements	1245

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1245

A.7.4.5.1	Test Purpose and Environment	1245

A.7.4.5.2	Test Requirements	1247

A.7.5	Proximity-based Services	1247

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1247

A.7.5.1.1	Test Purpose and Environment	1247

A.7.5.1.2	Test Requirements	1248

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1249

A.7.5.2.1	Test Purpose and Environment	1249

A.7.5.1.2	Test Requirements	1250

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1250

A.7.5.3.1	Test Purpose and Environment	1250

A.7.5.3.2	Test Requirements	1252

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1253

A.7.5.4.1	Test Purpose and Environment	1253

A.7.5.4.2	Test Requirements	1254

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1254

A.7.5.5.1	Test Purpose and Environment	1254

A.7.5.5.2	Test Requirements	1256

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1257

A.7.5.6.1	Test Purpose and Environment	1257

A.7.5.6.2	Test Requirements	1258

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1259

A.7.5.7.1	Test Purpose and Environment	1259

A.7.5.7.2	Test Requirements	1261

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1261

A.7.5.8.1	Test Purpose and Environment	1261

A.7.5.8.2	Test Requirements	1263

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1264

A.7.5.9.1	Test Purpose and Environment	1264

A.7.5.9.2	Test Requirements	1266

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1267

A.7.5.10.1	Test Purpose and Environment	1267

A.7.5.10.2	Test Requirements	1269

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1270

A.7.5.11.1	Test Purpose and Environment	1270

A.7.5.11.2	Test Requirements	1272

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1272

A.7.5.12.1	Test Purpose and Environment	1272

A.7.5.12.2	Test Requirements	1276

A.8	UE Measurements Procedures	1277

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1277

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1277

A.8.1.1.1	Test Purpose and Environment	1277

A.8.1.1.2	Test Requirements	1278

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1279

A.8.1.2.1	Test Purpose and Environment	1279

A.8.1.2.2	Test Requirements	1280

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1281

A.8.1.3.1	Test Purpose and Environment	1281

A.8.1.3.2	Test Requirements	1283

A.8.1.4	Void	1283

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1283

A.8.1.5.1	Test Purpose and Environment	1283

A.8.1.5.2	Test Requirements	1285

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1285

A.8.1.6.1	Test Purpose and Environment	1285

A.8.1.6.2	Test Requirements	1287

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1287

A.8.1.7.1	Test Purpose and Environment	1287

A.8.1.7.2	Test Requirements	1289

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1290

A.8.1.8.1	Test Purpose and Environment	1290

A.8.1.8.2	Test Requirements	1292

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1293

A.8.1.9.1	Test Purpose and Environment	1293

A.8.1.9.2	Test Requirements	1293

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1293

A.8.1.10.1	Test Purpose and Environment	1293

A.8.1.10.2	Test Requirements	1294

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1294

A.8.1.11.1	Test Purpose and Environment	1294

A.8.1.11.2	Test Requirements	1296

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1297

A.8.1.12.1	Test Purpose and Environment	1297

A.8.1.12.2	Test Requirements	1298

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1299

A.8.1.13.1	Test Purpose and Environment	1299

A.8.1.13.2	Test Requirements	1301

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1301

A.8.1.14.1	Test Purpose and Environment	1301

A.8.1.14.2	Test Requirements	1303

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1304

A.8.1.15.1	Test Purpose and Environment	1304

A.8.1.15.2	Test Requirements	1305

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1306

A.8.1.16.1	Test Purpose and Environment	1306

A.8.1.16.2	Test Requirements	1308

A.8.1.17	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1308

A.8.1.17.1	Test Purpose and Environment	1308

A.8.1.17.2	Test Requirements	1310

A.8.1.18	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1311

A.8.1.18.1	Test Purpose and Environment	1311

A.8.1.18.2	Test Requirements	1313

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1313

A.8.1.19.1	Test Purpose and Environment	1313

A.8.1.19.2	Test Requirements	1315

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1316

A.8.1.20.1	Test Purpose and Environment	1316

A.8.1.20.2	Test Requirements	1318

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1318

A.8.1.21.1	Test Purpose and Environment	1318

A.8.1.21.2	Test Requirements	1320

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1321

A.8.1.22.1	Test Purpose and Environment	1321

A.8.1.22.2	Test Requirements	1323

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1323

A.8.1.23.1	Test Purpose and Environment	1323

A.8.1.23.2	Test Requirements	1324

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1325

A.8.1.24.1	Test Purpose and Environment	1325

A.8.1.24.2	Test Requirements	1326

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1327

A.8.1.25.1	Test Purpose and Environment	1327

A.8.1.25.2	Test Requirements	1329

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1329

A.8.1.26.1	Test Purpose and Environment	1329

A.8.1.26.2	Test Requirements	1331

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1332

A.8.1.27.1	Test Purpose and Environment	1332

A.8.1.27.2	Test Requirements	1333

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1334

A.8.1.28.1	Test Purpose and Environment	1334

A.8.1.28.2	Test Requirements	1336

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1336

A.8.1.29.1	Test Purpose and Environment	1336

A.8.1.29.2	Test Requirements	1338

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1339

A.8.1.30.1	Test Purpose and Environment	1339

A.8.1.30.2	Test Requirements	1341

A.8.1.31	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1341

A.8.1.31.1	Test Purpose and Environment	1341

A.8.1.31.2	Test Requirements	1343

A.8.1.32	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1344

A.8.1.32.1	Test Purpose and Environment	1344

A.8.1.32.2	Test Requirements	1345

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1346

A.8.1.33.1	Test Purpose and Environment	1346

A.8.1.33.2	Test Requirements	1347

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1348

A.8.1.34.1	Test Purpose and Environment	1348

A.8.1.34.2	Test Requirements	1349

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1350

A.8.1.35.1	Test Purpose and Environment	1350

A.8.1.35.2	Test Requirements	1351

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1352

A.8.1.36.1	Test Purpose and Environment	1352

A.8.1.36.2	Test Requirements	1353

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1354

A.8.1.37.1	Test Purpose and Environment	1354

A.8.1.37.2	Test Requirements	1356

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1356

A.8.1.38.1	Test Purpose and Environment	1356

A.8.1.38.2	Test Requirements	1357

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1358

A.8.1.39.1	Test Purpose and Environment	1358

A.8.1.39.2	Test Requirements	1360

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1360

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1360

A.8.2.1.1	Test Purpose and Environment	1360

A.8.2.1.2	Test Requirements	1362

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1363

A.8.2.2.1	Test Purpose and Environment	1363

A.8.2.2.2	Test Requirements	1365

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1365

A.8.2.3.1	Test Purpose and Environment	1365

A.8.2.3.2	Test Requirements	1367

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1368

A.8.2.4.1	Test Purpose and Environment	1368

A.8.2.4.2	Test Requirements	1370

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1370

A.8.2.5.1	Test Purpose and Environment	1370

A.8.2.5.2	Test Requirements	1372

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1373

A.8.2.6.1	Test Purpose and Environment	1373

A.8.2.6.2	Test Requirements	1375

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1376

A.8.2.7.1	Test Purpose and Environment	1376

A.8.2.7.2	Test Requirements	1377

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1378

A.8.2.8.1	Test Purpose and Environment	1378

A.8.2.8.2	Test Requirements	1380

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1380

A.8.2.9.1	Test Purpose and Environment	1380

A.8.2.9.2	Test Requirements	1382

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1383

A.8.2.10.1	Test Purpose and Environment	1383

A.8.2.10.2	Test Requirements	1385

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1385

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1385

A.8.3.1.1	Test Purpose and Environment	1385

A.8.3.1.2	Test Requirements	1387

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1387

A.8.3.2.1	Test Purpose and Environment	1387

A.8.3.2.2	Test Requirements	1390

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1390

A.8.3.3.1	Test Purpose and Environment	1390

A.8.3.3.2	Test Requirements	1393

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1393

A.8.3.4.1	Test Purpose and Environment	1393

A.8.3.4.2	Test Requirements	1395

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1395

A.8.3.5.2	Test Requirements	1397

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1398

A.8.3.6.1	Test Purpose and Environment	1398

A.8.3.6.2	Test Requirements	1399

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1400

A.8.3.7.1	Test Purpose and Environment	1400

A.8.3.7.2	Test Requirements	1402

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1403

A.8.3.8.1	Test Purpose and Environment	1403

A.8.3.8.2	Test Requirements	1406

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1406

A.8.3.9.1	Test Purpose and Environment	1406

A.8.3.9.2	Test Requirements	1409

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1410

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1410

A.8.4.1.1	Test Purpose and Environment	1410

A.8.4.1.2	Test Requirements	1411

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1411

A.8.4.2.1	Test Purpose and Environment	1411

A.8.4.2.2	Test Requirements	1414

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1414

A.8.4.3.1	Test Purpose and Environment	1414

A.8.4.3.2	Test Requirements	1416

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1417

A.8.4.4.1	Test Purpose and Environment	1417

A.8.4.4.2	Test Requirements	1419

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1420

A.8.4.5.1	Test Purpose and Environment	1420

A.8.4.5.2	Test Requirements	1422

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1422

A.8.4.6.1	Test Purpose and Environment	1422

A.8.4.6.2	Test Requirements	1423

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1423

A.8.4.7.1	Test Purpose and Environment	1423

A.8.4.7.2	Test Requirements	1427

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1427

A.8.4.8.1	Test Purpose and Environment	1427

A.8.4.8.2	Test Requirements	1430

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1430

A.8.4.9.1	Test Purpose and Environment	1430

A.8.4.9.2	Test Requirements	1434

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1435

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1435

A.8.5.1.1	Test Purpose and Environment	1435

A.8.5.1.2	Test Requirements	1437

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1437

A.8.5.2.1	Test Purpose and Environment	1437

A.8.5.2.2	Test Requirements	1439

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1439

A.8.5.3.1	Test Purpose and Environment	1439

A.8.5.3.2	Test Requirements	1442

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1443

A.8.5.4.1	Test Purpose and Environment	1443

A.8.5.4.2	Test Requirements	1445

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1445

A.8.5.5.1	Test Purpose and Environment	1445

A.8.5.5.2	Test Requirements	1447

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1447

A.8.5.6.1	Test Purpose and Environment	1447

A.8.5.6.2	Test Requirements	1448

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1449

A.8.5.7.1	Test Purpose and Environment	1449

A.8.5.7.2	Test Requirements	1449

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1449

A.8.5.8.1	Test Purpose and Environment	1449

A.8.5.8.2	Test Requirements	1452

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1452

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1452

A.8.6.1.1	Test Purpose and Environment	1452

A.8.6.1.2	Test Requirements	1455

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1455

A.8.6.2.1	Test Purpose and Environment	1455

A.8.6.2.2	Test Requirements	1458

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1458

A.8.6.3.1	Test Purpose and Environment	1458

A.8.6.3.2	Test Requirements	1461

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1461

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1461

A.8.7.1.1	Test Purpose and Environment	1461

A.8.7.1.2	Test Requirements	1464

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1464

A.8.7.2.1	Test Purpose and Environment	1464

A.8.7.2.2	Test Requirements	1466

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1467

A.8.7.3.1	Test Purpose and Environment	1467

A.8.7.3.2	Test Parameters	1467

A.8.7.3.3	Test Requirements	1469

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1469

A.8.7.4.1	Test Purpose and Environment	1469

A.8.7.4.2	Test Requirements	1472

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1472

A.8.7.5.1	Test Purpose and Environment	1472

A.8.7.5.2	Test Requirements	1475

A.8.7A	TBD	1475

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1475

A.8.7A.1.1	Test Purpose and Environment	1475

A.8.7A.1.2	Test Requirements	1478

A.8.8	E-UTRAN FDD – GSM Measurements	1478

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1478

A.8.8.1.1	Test Purpose and Environment	1478

A.8.8.1.2	Test Requirements	1480

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1480

A.8.8.2.1	Test Purpose and Environment	1480

A.8.8.2.2	Test Requirements	1482

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1483

A.8.8.3.1	Test Purpose and Environment	1483

A.8.8.3.2	Test Requirements	1484

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1485

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1485

A.8.9.1.1	Test Purpose and Environment	1485

A.8.9.1.2	Test Requirements	1487

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1487

A.8.9.2.1	Test Purpose and Environment	1487

A.8.9.2.2	Test Requirements	1489

A.8.10	E-UTRAN TDD – GSM Measurements	1489

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1489

A.8.10.1.1	Test Purpose and Environment	1489

A.8.10.1.2	Test Requirements	1491

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1491

A.8.10.2.1	Test Purpose and Environment	1491

A.8.10.2.2	Test Requirements	1494

A.8.11	Monitoring of Multiple Layers	1494

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1494

A.8.11.1.1	Test Purpose and Environment	1494

A.8.11.1.2	Test Requirements	1496

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1497

A.8.11.2.1	Test Purpose and Environment	1497

A.8.11.2.2	Test Requirements	1498

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1499

A.8.11.3.1	Test Purpose and Environment	1499

A.8.11.3.2	Test Requirements	1501

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1501

A.8.11.4.1	Test Purpose and Environment	1501

A.8.11.4.2	Test Requirements	1504

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1504

A.8.11.5.1	Test Purpose and Environment	1504

A.8.11.5.2	Test Requirements	1507

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1507

A.8.11.6.1	Test Purpose and Environment	1507

A.8.11.6.2	Test Requirements	1509

A.8.12	RSTD Intra-frequency Measurements	1510

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1510

A.8.12.1.1	Test Purpose and Environment	1510

A.8.12.1.2	Test Requirements	1514

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1514

A.8.12.2.1	Test Purpose and Environment	1514

A.8.12.2.2	Test Requirements	1518

A.8.13	RSTD Inter-frequency Measurements	1518

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1518

A.8.13.1.1	Test Purpose and Environment	1518

A.8.13.1.2	Test Requirements	1522

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1522

A.8.13.2.1	Test Purpose and Environment	1522

A.8.13.2.2	Test Requirements	1526

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	1526

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1526

A.8.14.1.1	Test Purpose and Environment	1526

A.8.14.1.2	Test Requirements	1528

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1529

A.8.14.2.1	Test Purpose and Environment	1529

A.8.14.2.2	Test Requirements	1531

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1531

A.8.14.3.1	Test Purpose and Environment	1531

A.8.14.3.2	Test Requirements	1533

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	1534

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1534

A.8.15.1.1	Test Purpose and Environment	1534

A.8.15.1.2	Test Requirements	1535

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1536

A.8.15.2.1	Test Purpose and Environment	1536

A.8.15.2.2	Test Requirements	1538

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1538

A.8.15.3.1	Test Purpose and Environment	1538

A.8.15.3.2	Test Requirements	1540

A.8.16	E-UTRAN Carrier Aggregation Measurements	1541

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	1541

A.8.16.1.1	Test Purpose and Environment	1541

A.8.16.1.2	Test Requirements	1543

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	1544

A.8.16.2.1	Test Purpose and Environment	1544

A.8.16.2.2	Test Requirements	1546

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1547

A.8.16.3.1	Test Purpose and Environment	1547

A.8.16.3.2	Test Requirements	1548

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	1549

A.8.16.4.1	Test Purpose and Environment	1549

A.8.16.4.2	Test Requirements	1551

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1552

A.8.16.5.1	Test Purpose and Environment	1552

A.8.16.5.2	Test Requirements	1552

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1553

A.8.16.6.1	Test Purpose and Environment	1553

A.8.16.6.2	Test Requirements	1553

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1553

A.8.16.7.1	Test Purpose and Environment	1553

A.8.16.7.2	Test Requirements	1554

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1554

A.8.16.8.1	Test Purpose and Environment	1554

A.8.16.8.2	Test Requirements	1555

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1555

A.8.16.9.1	Test Purpose and Environment	1555

A.8.16.9.2	Test Requirements	1556

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1556

A.8.16.10.1	Test Purpose and Environment	1556

A.8.16.10.2	Test Requirements	1557

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1557

A.8.16.11.1	Test Purpose and Environment	1557

A.8.16.11.2	Test Requirements	1558

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1558

A.8.16.12.1	Test Purpose and Environment	1558

A.8.16.12.2	Test Requirements	1559

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	1559

A.8.16.13.1	Test Purpose and Environment	1559

A.8.16.13.2	Test Requirements	1560

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	1560

A.8.16.14.1	Test Purpose and Environment	1560

A.8.16.14.2	Test Requirements	1560

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	1561

A.8.16.15.1	Test Purpose and Environment	1561

A.8.16.7.2	Test Requirements	1561

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	1561

A.8.16.16.1	Test Purpose and Environment	1561

A.8.16.16.2	Test Requirements	1562

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	1562

A.8.16.17.1	Test Purpose and Environment	1562

A.8.16.17.2	Test Requirements	1564

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	1565

A.8.16.17A.1	Test Purpose and Environment	1565

A.8.16.17A.2	Test Requirements	1565

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1565

A.8.16.17B.1	Test Purpose and Environment	1565

A.8.16.17B.2	Test Requirements	1566

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1566

A.8.16.17C.1	Test Purpose and Environment	1566

A.8.16.17C.2	Test Requirements	1566

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	1566

A.8.16.18.1	Test Purpose and Environment	1566

A.8.16.18.2	Test Requirements	1569

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	1570

A.8.16.18A.1	Test Purpose and Environment	1570

A.8.16.18A.2	Test Requirements	1570

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1570

A.8.16.18B.1	Test Purpose and Environment	1570

A.8.16.18B.2	Test Requirements	1571

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1571

A.8.16.18C.1	Test Purpose and Environment	1571

A.8.16.18C.2	Test Requirements	1571

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	1571

A.8.16.18D.1	Test Purpose and Environment	1571

A.8.16.18D.2	Test Requirements	1572

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	1572

A.8.16.19.1	Test Purpose and Environment	1572

A.8.16.19.2	Test Requirements	1574

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1574

A.8.16.19A.1	Test Purpose and Environment	1574

A.8.16.19A.2	Test Requirements	1575

A.8.16.19B	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1575

A.8.16.19B.1	Test Purpose and Environment	1575

A.8.16.19B.2	Test Requirements	1575

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1575

A.8.16.19C.1	Test Purpose and Environment	1575

A.8.16.19C.2	Test Requirements	1576

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	1576

A.8.16.20.1	Test Purpose and Environment	1576

A.8.16.20.2	Test Requirements	1578

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1579

A.8.16.20A.1	Test Purpose and Environment	1579

A.8.16.20A.2	Test Requirements	1579

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1579

A.8.16.20B.1	Test Purpose and Environment	1579

A.8.16.20B.2	Test Requirements	1580

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1580

A.8.16.20C.1	Test Purpose and Environment	1580

A.8.16.20C.2	Test Requirements	1580

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	1580

A.8.16.20D.1	Test Purpose and Environment	1580

A.8.16.20D.2	Test Requirements	1581

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	1581

A.8.16.21.1	Test Purpose and Environment	1581

A.8.16.21.2	Test Requirements	1583

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	1584

A.8.16.22.1	Test Purpose and Environment	1584

A.8.16.22.2	Test Requirements	1584

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	1584

A.8.16.23.1	Test Purpose and Environment	1584

A.8.16.23.2	Test Requirements	1586

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	1587

A.8.16.24.1	Test Purpose and Environment	1587

A.8.16.24.2	Test Requirements	1589

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	1590

A.8.16.25.1	Test Purpose and Environment	1590

A.8.16.25.2	Test Requirements	1592

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	1593

A.8.16.26.1	Test Purpose and Environment	1593

A.8.16.26.2	Test Requirements	1595

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1595

A.8.16.27.1	Test Purpose and Environment	1595

A.8.16.27.2	Test Requirements	1599

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1599

A.8.16.28.1	Test Purpose and Environment	1599

A.8.16.28.2	Test Requirements	1603

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1603

A.8.16.29.1	Test Purpose and Environment	1603

A.8.16.29.2	Test Requirements	1607

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1607

A.8.16.30.1	Test Purpose and Environment	1607

A.8.16.30.2	Test Requirements	1611

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	1611

A.8.16.31.1	Test Purpose and Environment	1611

A.8.16.31.2	Test Requirements	1615

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	1615

A.8.16.32.1	Test Purpose and Environment	1615

A.8.16.32.2	Test Requirements	1619

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1619

A.8.16.33.1	Test Purpose and Environment	1619

A.8.16.33.2	Test Requirements	1623

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1623

A.8.16.34.1	Test Purpose and Environment	1623

A.8.16.34.2	Test Requirements	1627

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	1627

A.8.16.35.1	Test Purpose and Environment	1627

A.8.16.35.2	Test Requirements	1630

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	1630

A.8.16.36.1	Test Purpose and Environment	1630

A.8.16.36.2	Test Requirements	1632

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	1633

A.8.16.37.1	Test Purpose and Environment	1633

A.8.16.37.2	Test Requirements	1635

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	1636

A.8.16.38.1	Test Purpose and Environment	1636

A.8.16.38.2	Test Requirements	1638

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	1639

A.8.16.39.1	Test Purpose and Environment	1639

A.8.16.39.2	Test Requirements	1642

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	1642

A.8.16.40.1	Test Purpose and Environment	1642

A.8.16.40.2	Test Requirements	1645

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	1645

A.8.16.41.1	Test Purpose and Environment	1645

A.8.16.41.2	Test Requirements	1648

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	1648

A.8.16.42.1	Test Purpose and Environment	1648

A.8.16.42.2	Test Requirements	1651

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	1652

A.8.16.43.1	Test Purpose and Environment	1652

A.8.16.43.2	Test Requirements	1655

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	1655

A.8.16.44.1	Test Purpose and Environment	1655

A.8.16.44.2	Test Requirements	1657

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	1657

A.8.16.45.1	Test Purpose and Environment	1657

A.8.16.45.2	Test Requirements	1660

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	1660

A.8.16.46.1	Test Purpose and Environment	1660

A.8.16.46.2	Test Requirements	1663

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	1663

A.8.16.47.1	Test Purpose and Environment	1663

A.8.16.47.2	Test Requirements	1666

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	1666

A.8.16.48.1	Test Purpose and Environment	1666

A.8.16.48.2	Test Requirements	1668

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	1669

A.8.16.49.1	Test Purpose and Environment	1669

A.8.16.49.2	Test Requirements	1671

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	1672

A.8.16.50.1	Test Purpose and Environment	1672

A.8.16.50.2	Test Requirements	1674

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	1675

A.8.16.51.1	Test Purpose and Environment	1675

A.8.16.51.2	Test Requirements	1679

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	1679

A.8.16.52.1	Test Purpose and Environment	1679

A.8.16.52.2	Test Requirements	1683

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	1683

A.8.16.53.1	Test Purpose and Environment	1683

A.8.16.53.2	Test Requirements	1688

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	1688

A.8.16.54.1	Test Purpose and Environment	1688

A.8.16.54.2	Test Requirements	1693

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1693

A.8.16.55.1	Test Purpose and Environment	1693

A.8.16.55.2	Test Requirements	1698

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1698

A.8.16.56.1	Test Purpose and Environment	1698

A.8.16.56.2	Test Requirements	1703

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	1703

A.8.16.57.1	Test Purpose and Environment	1703

A.8.16.57.2	Test Requirements	1706

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	1706

A.8.16.58.1	Test Purpose and Environment	1706

A.8.16.58.2	Test Requirements	1709

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	1709

A.8.16.59.1	Test Purpose and Environment	1709

A.8.16.59.2	Test Requirements	1713

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	1713

A.8.16.60.1	Test Purpose and Environment	1713

A.8.16.60.2	Test Requirements	1717

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	1717

A.8.16.61.1	Test Purpose and Environment	1717

A.8.16.61.2	Test Requirements	1720

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	1720

A.8.16.62.1	Test Purpose and Environment	1720

A.8.16.62.2	Test Requirements	1723

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	1723

A.8.16.63.1	Test Purpose and Environment	1723

A.8.16.63.2	Test Requirements	1727

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	1727

A.8.16.64.1	Test Purpose and Environment	1727

A.8.16.64.2	Test Requirements	1731

A.8.16.65	5 DL PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	1731

A.8.16.65.1	Test Purpose and Environment	1731

A.8.16.65.2	Test Requirements	1735

A.8.16.67	5 DL PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	1735

A.8.16.67.1	Test Purpose and Environment	1735

A.8.16.67.2	Test Requirements	1739

A.8.16.68	5 DL PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	1740

A.8.16.68.1	Test Purpose and Environment	1740

A.8.16.68.2	Test Requirements	1743

A.8.16.69	E-UTRAN FDD 5DL CA activation and deactivation of unknown SCell in non-DRX	1744

A.8.16.69.1	Test Purpose and Environment	1744

A.8.16.69.2	Test Requirements	1747

A.8.16.70	E-UTRAN TDD 5DL CA activation and deactivation of unknown SCell in non-DRX	1747

A.8.16.70.1	Test Purpose and Environment	1747

A.8.16.70.2	Test Requirements	1750

A.8.16.66	5 DL PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	1750

A.8.16.66.1	Test Purpose and Environment	1750

A.8.16.66.2	Test Requirements	1754

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	1754

A.8.16.71.1	Test Purpose and Environment	1754

A.8.16.71.2	Test Requirements	1758

A.8.16.72	E-UTRAN 5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	1758

A.8.16.72.1	Test Purpose and Environment	1758

A.8.16.72.2	Test Requirements	1762

A.8.16.73	E-UTRAN FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1762

A.8.16.73.1	Test Purpose and Environment	1762

A.8.16.73.2	Test Requirements	1767

A.8.16.74	E-UTRAN TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1767

A.8.16.74.1	Test Purpose and Environment	1767

A.8.16.74.2	Test Requirements	1772

A.8.16.75	5 DL PCell in FDD CA activation and deactivation of known SCell in non-DRX	1772

A.8.16.75.1	Test Purpose and Environment	1772

A.8.16.75.2	Test Requirements	1776

A.8.16.76	5 DL PCell in TDD CA activation and deactivation of known SCell in non-DRX	1776

A.8.16.76.1	Test Purpose and Environment	1776

A.8.16.76.2	Test Requirements	1781

A.8.16.77	E-UTRAN FDD 5DL CA activation and deactivation of know SCell in non-DRX	1781

A.8.16.77.1	Test Purpose and Environment	1781

A.8.16.77.2	Test Requirements	1784

A.8.16.78	E-UTRAN TDD 5DL CA activation and deactivation of know SCell in non-DRX	1784

A.8.16.78.1	Test Purpose and Environment	1784

A.8.16.78.2	Test Requirements	1788

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	1788

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	1788

A.8.17.1.1	Test Purpose and Environment	1788

A.8.17.1.2	Test Requirements	1793

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	1793

A.8.17.2.1	Test Purpose and Environment	1793

A.8.17.2.2	Test Requirements	1798

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	1798

A.8.17.3.1	Test Purpose and Environment	1798

A.8.17.3.2	Test Requirements	1799

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	1799

A.8.17.4.1	Test Purpose and Environment	1799

A.8.17.4.2	Test Requirements	1800

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	1800

A.8.17.5.1	Test Purpose and Environment	1800

A.8.17.5.2	Test Requirements	1801

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	1801

A.8.17.6.1	Test Purpose and Environment	1801

A.8.17.6.2	Test Requirements	1802

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	1803

A.8.17.7.1	Test Purpose and Environment	1803

A.8.17.7.2	Test Requirements	1804

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	1804

A.8.17.8.1	Test Purpose and Environment	1804

A.8.17.8.2	Test Requirements	1805

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	1805

A.8.17.9.1	Test Purpose and Environment	1805

A.8.17.9.2	Test Requirements	1806

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	1806

A.8.17.10.1	Test Purpose and Environment	1806

A.8.17.10.2	Test Requirements	1811

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	1812

A.8.17.11.1	Test Purpose and Environment	1812

A.8.17.11.2	Test Requirements	1817

A.8.18	E-UTRAN TDD – HRPD Measurements	1818

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	1818

A.8.18.1.1	Test Purpose and Environment	1818

A.8.18.1.2	Test Requirements	1821

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	1821

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	1821

A.8.19.1.1	Test Purpose and Environment	1821

A.8.19.1.2	Test Requirements	1823

A.8.20	Inter-frequency/RAT Measurements in CA mode	1824

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1824

A.8.20.1.1	Test Purpose and Environment	1824

A.8.20.1.2	Test Requirements	1825

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1826

A.8.20.2.1	Test Purpose and Environment	1826

A.8.20.2.2	Test Requirements	1827

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	1828

A.8.20.2A.1	Test Purpose and Environment	1828

A.8.20.2A.2	Test Requirements	1828

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	1828

A.8.20.2B.1	Test Purpose and Environment	1828

A.8.20.2B.2	Test Requirements	1830

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1830

A.8.20.3.1	Test Purpose and Environment	1830

A.8.20.3.2	Test Requirements	1833

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1833

A.8.20.4.1	Test Purpose and Environment	1833

A.8.20.4.2	Test Requirements	1836

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	1836

A.8.20.4A.1	Test Purpose and Environment	1836

A.8.20.4A.2	Test Requirements	1836

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	1837

A.8.20.4B.1	Test Purpose and Environment	1837

A.8.20.4B.2	Test Requirements	1839

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	1839

A.8.21.1	Test Purpose and Environment	1839

A.8.21.2 Test Requirements	1843

A.8.22	E-UTRAN Discovery Signal Measurements	1844

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	1844

A.8.22.1.1	Test Purpose and Environment	1844

A.8.22.1.2	Test Requirements	1846

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	1846

A.8.22.2.1	Test Purpose and Environment	1846

A.8.22.2.2	Test Requirements	1849

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	1849

A.8.22.3.1	Test Purpose and Environment	1849

A.8.22.3.2	Test Requirements	1852

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	1852

A.8.22.4.1	Test Purpose and Environment	1852

A.8.22.4.2	Test Requirements	1855

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	1855

A.8.22.5.1	Test Purpose and Environment	1855

A.8.22.5.2	Test Requirements	1858

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	1858

A.8.22.6.1	Test Purpose and Environment	1858

A.8.22.6.2	Test Requirements	1861

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	1861

A.8.22.7.1	Test Purpose and Environment	1861

A.8.22.7.2	Test Requirements	1864

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	1864

A.8.22.8.1	Test Purpose and Environment	1864

A.8.22.8.2	Test Requirements	1867

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	1867

A.8.22.9.1	Test Purpose and Environment	1867

A.8.22.9.2	Test Requirements	1870

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	1870

A.8.22.10.1	Test Purpose and Environment	1870

A.8.22.10.2	Test Requirements	1873

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	1873

A.8.22.11.1	Test Purpose and Environment	1873

A.8.22.11.2	Test Requirements	1876

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	1876

A.8.22.12.1	Test Purpose and Environment	1876

A.8.22.12.2	Test Requirements	1879

A.8.23	E-UTRAN Dual Connectivity Measurements	1879

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	1879

A.8.23.1.1	Test Purpose and Environment	1879

A.8.23.1.2	Test Requirements	1882

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	1882

A.8.23.2.1	Test Purpose and Environment	1882

A.8.23.2.2	Test Requirements	1885

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	1885

A.8.23.3.1	Test Purpose and Environment	1885

A.8.23.3.2	Test Requirements	1888

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	1888

A.8.23.4.1	Test Purpose and Environment	1888

A.8.23.4.2	Test Requirements	1891

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	1891

A.8.23.5.1	Test Purpose and Environment	1891

A.8.23.5.2	Test Requirements	1894

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	1894

A.8.23.6.1	Test Purpose and Environment	1894

A.8.23.6.2	Test Requirements	1897

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	1897

A.8.23.7.1	Test Purpose and Environment	1897

A.8.23.7.2	Test Requirements	1900

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	1900

A.8.23.8.1	Test Purpose and Environment	1900

A.8.23.8.2	Test Requirements	1903

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	1903

A.8.23.9.1	Test Purpose and Environment	1903

A.8.23.9.2	Test Requirements	1905

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	1906

A.8.23.10.1	Test Purpose and Environment	1906

A.8.23.10.2	Test Requirements	1909

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	1909

A.8.23.11.1	Test Purpose and Environment	1909

A.8.23.11.2	Test Requirements	1912

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	1912

A.8.23.12.1	Test Purpose and Environment	1912

A.8.23.12.2	Test Requirements	1915

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	1915

A.8.23.13.1	Test Purpose and Environment	1915

A.8.23.13.2	Test Requirements	1918

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	1918

A.8.23.14.1	Test Purpose and Environment	1918

A.8.23.14.2	Test Requirements	1921

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	1921

A.8.23.15.1	Test Purpose and Environment	1921

A.8.23.15.2	Test Requirements	1924

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	1924

A.8.23.16.1	Test Purpose and Environment	1924

A.8.23.16.2	Test Requirements	1926

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	1927

A.8.23.17.1	Test Purpose and Environment	1927

A.8.23.17.2	Test Requirements	1929

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	1929

A.8.23.18.1	Test Purpose and Environment	1929

A.8.23.18.2	Test Requirements	1931

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	1932

A.8.23.19.1	Test Purpose and Environment	1932

A.8.23.19.2	Test Requirements	1933

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	1934

A.8.23.20.1	Test Purpose and Environment	1934

A.8.23.20.2	Test Requirements	1936

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	1937

A.8.23.21.1	Test Purpose and Environment	1937

A.8.23.21.2	Test Requirements	1938

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	1939

A.8.23.22.1	Test Purpose and Environment	1939

A.8.23.22.2	Test Requirements	1941

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	1941

A.8.23.23.1	Test Purpose and Environment	1941

A.8.23.23.2	Test Requirements	1943

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	1944

A.8.23.24.1	Test Purpose and Environment	1944

A.8.23.24.2	Test Requirements	1946

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	1947

A.8.23.25.1	Test Purpose and Environment	1947

A.8.23.25.2	Test Requirements	1949

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	1950

A.8.23.26.1	Test Purpose and Environment	1950

A.8.23.26.2	Test Requirements	1952

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	1953

A.8.23.27.1	Test Purpose and Environment	1953

A.8.23.27.2	Test Requirements	1956

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	1956

A.8.23.28.1	Test Purpose and Environment	1956

A.8.23.28.2	Test Requirements	1958

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	1959

A.8.23.29.1	Test Purpose and Environment	1959

A.8.23.29.2	Test Requirements	1961

A.8.24	Proximity-based Services	1961

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	1961

A.8.24.1.1	Test Purpose and Environment	1962

A.8.24.1.2	Test Requirements	1963

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	1963

A.8.24.2.1	Test Purpose and Environment	1964

A.8.24.2.2	Test Requirements	1965

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	1966

A.8.24.3.1	Test Purpose and Environment	1966

A.8.24.3.2	Test Requirements	1967

A.8.25	E-UTRAN-WLAN Measurements	1968

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	1968

A.8.25.1.1	Test Purpose and Environment	1968

A.8.25.1.2	Test Requirements	1970

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	1970

A.8.25.2.1	Test Purpose and Environment	1970

A.8.25.2.2	Test Requirements	1972

A.8.26	Frame Structure 3 (FS3)	1972

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	1972

A.8.26.1.1	Test Purpose and Environment	1972

A.8.26.1.2	Test Requirements	1974

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	1975

A.8.26.2.1	Test Purpose and Environment	1975

A.8.26.2.2	Test Requirements	1977

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	1978

A.8.26.3.1	Test Purpose and Environment	1978

A.8.26.3.2	Test Requirements	1981

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	1981

A.8.26.4.1	Test Purpose and Environment	1981

A.8.26.4.2	Test Requirements	1983

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	1984

A.8.26.5.1	Test Purpose and Environment	1984

A.8.26.5.2	Test Requirements	1986

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	1986

A.8.26.6.1	Test Purpose and Environment	1986

A.8.26.6.2	Test Requirements	1988

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	1989

A.8.26.7.1	Test Purpose and Environment	1989

A.8.26.7.2	Test Requirements	1991

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	1991

A.8.26.8.1	Test Purpose and Environment	1991

A.8.26.8.2	Test Requirements	1994

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1994

A.8.26.9.1	Test Purpose and Environment	1994

A.8.26.9.2	Test Requirements	1996

A.8.26.10	E-UTRAN TDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1997

A.8.26.10.1	Test Purpose and Environment	1997

A.8.26.10.2	Test Requirements	1999

A.8.26MF1	MF SCell activation and deactivation of known SCell with MF PCell in non-DRX	1999

A.8.26MF1.1	Test Purpose and Environment	1999

A.8.26MF1.2	Test Requirements	2002

A.8.26MF2	MF intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with MF	2002

A.8.26MF2.1	Test Purpose and Environment	2002

A.8.26MF2.2	Test Requirements	2004

A.8.26MF3	MF intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with MF	2005

A.8.26MF3.1	Test Purpose and Environment	2005

A.8.26MF3.2	Test Requirements	2007

A.8.26MF4	MF inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2007

A.8.26MF4.1	Test Purpose and Environment	2007

A.8.26MF4.2	Test Requirements	2010

A.9	Measurement Performance Requirements	2011

A.9.1	RSRP	2011

A.9.1.1	FDD Intra frequency case	2011

A.9.1.1.1	Test Purpose and Environment	2011

A.9.1.1.2	Test parameters	2011

A.9.1.1.3	Test Requirements	2013

A.9.1.2	TDD Intra frequency case	2013

A.9.1.2.1	Test Purpose and Environment	2013

A.9.1.2.2	Test parameters	2013

A.9.1.2.3	Test Requirements	2014

A.9.1.3	FDD—FDD Inter frequency case	2015

A.9.1.3.1	Test Purpose and Environment	2015

A.9.1.3.2	Test parameters	2015

A.9.1.3.3	Test Requirements	2017

A.9.1.4	TDD—TDD Inter frequency case	2017

A.9.1.4.1	Test Purpose and Environment	2017

A.9.1.4.2	Test parameters	2017

A.9.1.4.3	Test Requirements	2019

A.9.1.5	FDD—TDD Inter frequency case	2020

A.9.1.5.1	Test Purpose and Environment	2020

A.9.1.5.2	Test parameters	2020

A.9.1.5.3	Test Requirements	2021

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2022

A.9.1.6.1	Test Purpose and Environment	2022

A.9.1.6.2	Test parameters	2022

A.9.1.6.3	Test Requirements	2024

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2024

A.9.1.7.1	Test Purpose and Environment	2024

A.9.1.7.2	Test parameters	2024

A.9.1.7.3	Test Requirements	2026

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2026

A.9.1.8.1	Test Purpose and Environment	2026

A.9.1.8.2	Test parameters	2026

A.9.1.8.3	Test Requirements	2029

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2029

A.9.1.9.1	Test Purpose and Environment	2029

A.9.1.9.2	Test parameters	2029

A.9.1.9.3	Test Requirements	2032

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2032

A.9.1.10.1	Test Purpose and Environment	2032

A.9.1.10.2	Test parameters	2032

A.9.1.10.3	Test Requirements	2034

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2034

A.9.1.11.1	Test Purpose and Environment	2034

A.9.1.11.2	Test parameters	2034

A.9.1.11.3	Test Requirements	2037

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2037

A.9.1.12.1	Test Purpose and Environment	2037

A.9.1.12.2	Test parameters	2037

A.9.1.12.3	Test Requirements	2038

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2038

A.9.1.13.1	Test Purpose and Environment	2038

A.9.1.13.2	Test parameters	2038

A.9.1.13.3	Test Requirements	2039

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2039

A.9.1.14.1	Test Purpose and Environment	2039

A.9.1.14.2	Test parameters	2039

A.9.1.14.3	Test Requirements	2042

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2042

A.9.1.15.1	Test Purpose and Environment	2042

A.9.1.15.2	Test parameters	2042

A.9.1.15.3	Test Requirements	2044

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2045

A.9.1.16.1	Test Purpose and Environment	2045

A.9.1.16.2	Test parameters	2045

A.9.1.16.3	Test Requirements	2046

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2047

A.9.1.17.1	Test Purpose and Environment	2047

A.9.1.17.2	Test parameters	2047

A.9.1.17.3	Test Requirements	2048

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2049

A.9.1.18.1	Test Purpose and Environment	2049

A.9.1.18.2	Test parameters	2049

A.9.1.18.3	Test Requirements	2050

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2050

A.9.1.19.1	Test Purpose and Environment	2051

A.9.1.19.2	Test parameters	2051

A.9.1.19.3	Test Requirements	2051

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2051

A.9.1.20.1	Test Purpose and Environment	2051

A.9.1.20.2	Test parameters	2051

A.9.1.20.3	Test Requirements	2052

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2053

A.9.1.21.1	Test Purpose and Environment	2053

A.9.1.21.2	Test parameters	2053

A.9.1.21.3	Test Requirements	2053

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2053

A.9.1.22.1	Test Purpose and Environment	2053

A.9.1.22.2	Test parameters	2054

A.9.1.22.3	Test Requirements	2057

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2057

A.9.1.23.1	Test Purpose and Environment	2057

A.9.1.23.2	Test parameters	2057

A.9.1.23.3	Test Requirements	2060

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2060

A.9.1.24.1	Test Purpose and Environment	2060

A.9.1.24.2	Test parameters	2060

A.9.1.24.3	Test Requirements	2061

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2061

A.9.1.25.1	Test Purpose and Environment	2061

A.9.1.25.2	Test parameters	2061

A.9.1.25.3	Test Requirements	2063

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2063

A.9.1.26.1	Test Purpose and Environment	2063

A.9.1.26.2	Test parameters	2063

A.9.1.26.3	Test Requirements	2065

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2065

A.9.1.27.1	Test Purpose and Environment	2065

A.9.1.27.2	Test parameters	2065

A.9.1.27.3	Test Requirements	2067

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2067

A.9.1.28.1	Test Purpose and Environment	2067

A.9.1.28.2	Test parameters	2067

A.9.1.28.3	Test Requirements	2069

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2069

A.9.1.29.1	Test Purpose and Environment	2069

A.9.1.29.2	Test parameters	2069

A.9.1.29.3	Test Requirements	2071

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2071

A.9.1.30.1	Test Purpose and Environment	2071

A.9.1.30.2	Test parameters	2072

A.9.1.30.3	Test Requirements	2074

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2074

A.9.1.31.1	Test Purpose and Environment	2074

A.9.1.31.2	Test parameters	2074

A.9.1.31.3	Test Requirements	2076

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2076

A.9.1.32.1	Test Purpose and Environment	2076

A.9.1.32.2	Test parameters	2077

A.9.1.32.3	Test Requirements	2079

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2079

A.9.1.33.1	Test Purpose and Environment	2079

A.9.1.33.2	Test parameters	2079

A.9.1.33.3	Test Requirements	2081

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2081

A.9.1.34.1	Test Purpose and Environment	2081

A.9.1.34.2	Test parameters	2081

A.9.1.34.3	Test Requirements	2083

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2083

A.9.1.35.1	Test Purpose and Environment	2083

A.9.1.35.2	Test parameters	2083

A.9.1.35.3	Test Requirements	2085

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2086

A.9.1.36.1	Test Purpose and Environment	2086

A.9.1.36.2	Test parameters	2086

A.9.1.36.3	Test Requirements	2088

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2088

A.9.1.37.1	Test Purpose and Environment	2088

A.9.1.37.2	Test parameters	2088

A.9.1.37.3	Test Requirements	2090

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2091

A.9.1.38.1	Test Purpose and Environment	2091

A.9.1.38.2	Test parameters	2091

A.9.1.38.3	Test Requirements	2093

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2094

A.9.1.39.1	Test Purpose and Environment	2094

A.9.1.39.2	Test parameters	2094

A.9.1.39.3	Test Requirements	2098

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2098

A.9.1.40.1	Test Purpose and Environment	2098

A.9.1.40.2	Test parameters	2099

A.9.1.40.3	Test Requirements	2103

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2103

A.9.1.41.1	Test Purpose and Environment	2103

A.9.1.41.2	Test parameters	2103

A.9.1.41.3	Test Requirements	2105

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2105

A.9.1.42.1	Test Purpose and Environment	2105

A.9.1.42.2	Test parameters	2105

A.9.1.42.3	Test Requirements	2107

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2107

A.9.1.43.1	Test Purpose and Environment	2107

A.9.1.43.2	Test parameters	2107

A.9.1.43.3	Test Requirements	2108

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2109

A.9.1.44.1	Test Purpose and Environment	2109

A.9.1.44.2	Test parameters	2109

A.9.1.44.3	Test Requirements	2113

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2113

A.9.1.45.1	Test Purpose and Environment	2113

A.9.1.45.2	Test parameters	2113

A.9.1.45.3	Test Requirements	2117

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2117

A.9.1.46.1	Test Purpose and Environment	2117

A.9.1.46.2	Test parameters	2117

A.9.1.46.3	Test Requirements	2123

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2124

A.9.1.47.1	Test Purpose and Environment	2124

A.9.1.47.2	Test parameters	2124

A.9.1.47.3	Test Requirements	2130

A.9.1.48	5 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2130

A.9.1.48.1	Test Purpose and Environment	2130

A.9.1.48.2	Test parameters	2131

A.9.1.48.3	Test Requirements	2138

A.9.1.49	5 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2139

A.9.1.49.1	Test Purpose and Environment	2139

A.9.1.49.2	Test parameters	2139

A.9.1.49.3	Test Requirements	2146

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2147

A.9.1.50.1	Test Purpose and Environment	2147

A.9.1.50.2	Test parameters	2147

A.9.1.50.3	Test Requirements	2151

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2152

A.9.1.51.1	Test Purpose and Environment	2152

A.9.1.51.2	Test parameters	2152

A.9.1.51.3	Test Requirements	2156

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2157

A.9.1.52.1	Test Purpose and Environment	2157

A.9.1.52.2	Test parameters	2157

A.9.1.52.3	Test Requirements	2159

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2159

A.9.1.52A.1	Test Purpose and Environment	2159

A.9.1.52A.2	Test parameters	2159

A.9.1.52A.3	Test Requirements	2160

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2160

A.9.1.53.1	Test Purpose and Environment	2160

A.9.1.53.2	Test parameters	2161

A.9.1.53.3	Test Requirements	2163

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2163

A.9.1.53A.1	Test Purpose and Environment	2163

A.9.1.53A.2	Test parameters	2163

A.9.1.53A.3	Test Requirements	2164

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2164

A.9.1.54.1	Test Purpose and Environment	2164

A.9.1.54.2	Test parameters	2165

A.9.1.54.3	Test Requirements	2166

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2166

A.9.1.55.1	Test Purpose and Environment	2166

A.9.1.55.2	Test parameters	2166

A.9.1.55.3	Test Requirements	2169

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2169

A.9.1.56.1	Test Purpose and Environment	2169

A.9.1.56.2	Test parameters	2169

A.9.1.56.3	Test Requirements	2171

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2172

A.9.1.57.1	Test Purpose and Environment	2172

A.9.1.57.2	Test parameters	2172

A.9.1.57.3	Test Requirements	2174

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2174

A.9.1.57A.1	Test Purpose and Environment	2174

A.9.1.57A.2	Test parameters	2174

A.9.1.57A.3	Test Requirements	2175

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2175

A.9.1.58.1	Test Purpose and Environment	2175

A.9.1.58.2	Test parameters	2176

A.9.1.58.3	Test Requirements	2178

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2178

A.9.1.58A.1	Test Purpose and Environment	2178

A.9.1.58A.2	Test parameters	2178

A.9.1.58A.3	Test Requirements	2179

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2179

A.9.1.59.1	Test Purpose and Environment	2179

A.9.1.59.2	Test parameters	2180

A.9.1.59.3	Test Requirements	2181

A.9.1.60	Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2181

A.9.1.60.1	Test Purpose and Environment	2181

A.9.1.60.2	Test parameters	2181

A.9.1.60.3	Test Requirements	2184

A.9.1.61	Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2184

A.9.1.61.1	Test Purpose and Environment	2184

A.9.1.61.2	Test parameters	2184

A.9.1.61.3	Test Requirements	2186

A.9.1MF1	MF intra frequency absolute and relative RSRP accuracies	2186

A.9.1MF1.1	Test Purpose and Environment	2186

A.9.1MF1.2	Test parameters	2187

A.9.1MF1.3	Test Requirements	2188

A.9.2	RSRQ	2188

A.9.2.1	FDD Intra frequency case	2188

A.9.2.1.1	Test Purpose and Environment	2188

A.9.2.1.2	Test parameters	2188

A.9.2.1.3	Test Requirements	2190

A.9.2.2	TDD Intra frequency case	2190

A.9.2.2.1	Test Purpose and Environment	2190

A.9.2.2.2	Test parameters	2190

A.9.2.2.3	Test Requirements	2192

A.9.2.3	FDD—FDD Inter frequency case	2192

A.9.2.3.1	Test Purpose and Environment	2192

A.9.2.3.2	Test parameters	2192

A.9.2.3.3	Test Requirements	2194

A.9.2.4	TDD—TDD Inter frequency case	2194

A.9.2.4.1	Test Purpose and Environment	2194

A.9.2.4.2	Test parameters	2194

A.9.2.4.3	Test Requirements	2196

A.9.2.4A	FDD—TDD Inter frequency case	2197

A.9.2.4A.1	Test Purpose and Environment	2197

A.9.2.4A.2	Test parameters	2197

A.9.2.4A.3	Test Requirements	2199

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2199

A.9.2.5.1	Test Purpose and Environment	2199

A.9.2.5.2	Test parameters	2200

A.9.2.5.3	Test Requirements	2202

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2202

A.9.2.6.1	Test Purpose and Environment	2202

A.9.2.6.2	Test parameters	2202

A.9.2.6.3	Test Requirements	2204

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2204

A.9.2.7.1	Test Purpose and Environment	2204

A.9.2.7.2	Test parameters	2204

A.9.2.7.3	Test Requirements	2207

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2207

A.9.2.8.1	Test Purpose and Environment	2207

A.9.2.8.2	Test parameters	2207

A.9.2.8.3	Test Requirements	2210

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2210

A.9.2.9.1	Test Purpose and Environment	2210

A.9.2.9.2	Test parameters	2210

A.9.2.9.3	Test Requirements	2213

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2213

A.9.2.10.1	Test Purpose and Environment	2213

A.9.2.10.2	Test parameters	2213

A.9.2.10.3	Test Requirements	2216

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2216

A.9.2.11.1	Test Purpose and Environment	2216

A.9.2.11.2	Test parameters	2216

A.9.2.11.3	Test Requirements	2217

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2217

A.9.2.12.1	Test Purpose and Environment	2217

A.9.2.12.2	Test parameters	2217

A.9.2.12.3	Test Requirements	2218

A.9.2.13	Void	2219

A.9.2.13.1	Void	2219

A.9.2.13.2	Void	2219

A.9.2.13.3	Void	2219

A.9.2.14	Void	2219

A.9.2.14.1	Void	2219

A.9.2.14.2	Void	2219

A.9.2.14.3	Void	2219

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2219

A.9.2.15.1	Test Purpose and Environment	2219

A.9.2.15.2	Test parameters	2219

A.9.2.15.3	Test Requirements	2222

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2222

A.9.2.16.1	Test Purpose and Environment	2222

A.9.2.16.2	Test parameters	2222

A.9.2.16.3	Test Requirements	2226

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2226

A.9.2.17.1	Test Purpose and Environment	2226

A.9.2.17.2	Test parameters	2226

A.9.2.17.3	Test Requirements	2228

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2228

A.9.2.18.1	Test Purpose and Environment	2228

A.9.2.18.2	Test parameters	2228

A.9.2.18.3	Test Requirements	2230

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2230

A.9.2.19.1	Test Purpose and Environment	2230

A.9.2.19.2	Test parameters	2230

A.9.2.19.3	Test Requirements	2232

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2232

A.9.2.20.1	Test Purpose and Environment	2232

A.9.2.20.2	Test parameters	2232

A.9.2.20.3	Test Requirements	2234

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2234

A.9.2.21.1	Test Purpose and Environment	2234

A.9.2.21.2	Test parameters	2234

A.9.2.21.3	Test Requirements	2236

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2236

A.9.2.22.1	Test Purpose and Environment	2236

A.9.2.22.2	Test parameters	2237

A.9.2.22.3	Test Requirements	2237

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2237

A.9.2.23.1	Test Purpose and Environment	2237

A.9.2.23.2	Test parameters	2237

A.9.2.23.3	Test Requirements	2239

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2239

A.9.2.24.1	Test Purpose and Environment	2239

A.9.2.24.2	Test parameters	2239

A.9.2.24.3	Test Requirements	2240

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2240

A.9.2.25.1	Test Purpose and Environment	2240

A.9.2.25.2	Test parameters	2240

A.9.2.25.3	Test Requirements	2243

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2243

A.9.2.26.1	Test Purpose and Environment	2243

A.9.2.26.2	Test parameters	2243

A.9.2.26.3	Test Requirements	2246

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2246

A.9.2.27.1	Test Purpose and Environment	2246

A.9.2.27.2	Test parameters	2246

A.9.2.27.3	Test Requirements	2247

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2247

A.9.2.28.1	Test Purpose and Environment	2247

A.9.2.28.2	Test parameters	2247

A.9.2.28.3	Test Requirements	2249

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2249

A.9.2.29.1	Test Purpose and Environment	2249

A.9.2.29.2	Test parameters	2249

A.9.2.29.3	Test Requirements	2251

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2251

A.9.2.30.1	Test Purpose and Environment	2251

A.9.2.30.2	Test parameters	2251

A.9.2.30.3	Test Requirements	2253

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2253

A.9.2.31.1	Test Purpose and Environment	2253

A.9.2.31.2	Test parameters	2253

A.9.2.31.3	Test Requirements	2255

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2255

A.9.2.32.1	Test Purpose and Environment	2255

A.9.2.32.2	Test parameters	2255

A.9.2.32.3	Test Requirements	2257

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2257

A.9.2.33.1	Test Purpose and Environment	2257

A.9.2.33.2	Test parameters	2257

A.9.2.33.3	Test Requirements	2259

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2259

A.9.2.34.1	Test Purpose and Environment	2259

A.9.2.34.2	Test parameters	2259

A.9.2.34.3	Test Requirements	2261

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2261

A.9.2.35.1	Test Purpose and Environment	2261

A.9.2.35.2	Test parameters	2261

A.9.2.35.3	Test Requirements	2263

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2263

A.9.2.36.1	Test Purpose and Environment	2263

A.9.2.36.2	Test parameters	2263

A.9.2.36.3	Test Requirements	2264

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2265

A.9.2.37.1	Test Purpose and Environment	2265

A.9.2.37.2	Test parameters	2265

A.9.2.37.3	Test Requirements	2266

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2267

A.9.2.38.1	Test Purpose and Environment	2267

A.9.2.38.2	Test parameters	2267

A.9.2.38.3	Test Requirements	2270

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2270

A.9.2.39.1	Test Purpose and Environment	2270

A.9.2.39.2	Test parameters	2270

A.9.2.39.3	Test Requirements	2273

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2273

A.9.2.40.1	Test Purpose and Environment	2273

A.9.2.40.2	Test parameters	2273

A.9.2.40.3	Test Requirements	2275

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2275

A.9.2.41.1	Test Purpose and Environment	2275

A.9.2.41.2	Test parameters	2276

A.9.2.41.3	Test Requirements	2278

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2278

A.9.2.42.1	Test Purpose and Environment	2278

A.9.2.42.2	Test parameters	2278

A.9.2.42.3	Test Requirements	2280

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2280

A.9.2.43.1	Test Purpose and Environment	2280

A.9.2.43.2	Test parameters	2280

A.9.2.43.3	Test Requirements	2282

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2282

A.9.2.44.1	Test Purpose and Environment	2282

A.9.2.44.2	Test parameters	2282

A.9.2.44.3	Test Requirements	2284

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2284

A.9.2.45.1	Test Purpose and Environment	2284

A.9.2.45.2	Test parameters	2284

A.9.2.45.3	Test Requirements	2287

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2288

A.9.2.46.1	Test Purpose and Environment	2288

A.9.2.46.2	Test parameters	2288

A.9.2.46.3	Test Requirements	2291

A.9.2.47	5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2292

A.9.2.47.1	Test Purpose and Environment	2292

A.9.2.47.2	Test parameters	2292

A.9.2.47.3	Test Requirements	2295

A.9.2.48	5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2295

A.9.2.48.1	Test Purpose and Environment	2295

A.9.2.48.2	Test parameters	2295

A.9.2.48.3	Test Requirements	2298

A.9.2.49	Absolute and relative RSRQ accuracies in 5 DL FDD CA	2298

A.9.2.49.1	Test Purpose and Environment	2298

A.9.2.49.2	Test parameters	2298

A.9.2.49.3	Test Requirements	2302

A.9.2.50	Absolute and relative RSRQ accuracies in 5 DL TDD CA	2303

A.9.2.50.1	Test Purpose and Environment	2303

A.9.2.50.2	Test parameters	2303

A.9.2.50.3	Test Requirements	2307

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2307

A.9.2.51.1	Test Purpose and Environment	2307

A.9.2.51.2	Test parameters	2308

A.9.2.51.3	Test Requirements	2311

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2311

A.9.2.52.1	Test Purpose and Environment	2311

A.9.2.52.2	Test parameters	2311

A.9.2.52.3	Test Requirements	2313

A.9.2MF1	MF intra frequency absolute and relative RSRQ accuracies	2314

A.9.2MF1.1	Test Purpose and Environment	2314

A.9.2MF1.2	Test parameters	2314

A.9.2MF1.3	Test Requirements	2315

A.9.3	UTRAN FDD CPICH RSCP	2316

A.9.3.1	E-UTRAN FDD	2316

A.9.3.1.1	Test Purpose and Environment	2316

A.9.3.1.2	Parameters	2316

A.9.3.1.3	Test Requirements	2318

A.9.3.2	E-UTRAN TDD	2318

A.9.3.2.1	Test Purpose and Environment	2318

A.9.3.2.2	Parameters	2318

A.9.3.2.3	Test Requirements	2320

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2320

A.9.3.3.1	Test Purpose and Environment	2320

A.9.3.3.2	Parameters	2320

A.9.3.3.3	Test Requirements	2321

A.9.4	UTRAN FDD CPICH Ec/No	2322

A.9.4.1	E-UTRAN FDD	2322

A.9.4.1.1	Test Purpose and Environment	2322

A.9.4.1.2	Parameters	2322

A.9.4.1.3	Test Requirements	2324

A.9.4.2	E-UTRAN TDD	2325

A.9.4.2.1	Test Purpose and Environment	2325

A.9.4.2.2	Parameters	2325

A.9.4.2.3	Test Requirements	2327

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2328

A.9.4.3.1	Test Purpose and Environment	2328

A.9.4.3.2	Parameters	2328

A.9.4.3.3	Test Requirements	2329

A.9.5	UTRAN TDD measurement	2329

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2329

A.9.5.1.1	Test Purpose and Environment	2329

A.9.5.1.2	Test parameters	2329

A.9.5.1.3	Test Requirements	2331

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2331

A.9.5.2.1	Test Purpose and Environment	2331

A.9.5.2.2	Test parameters	2331

A.9.5.2.3	Test Requirements	2333

A.9.6	GSM Carrier RSSI	2334

A.9.6.1	E-UTRAN FDD	2334

A.9.6.1.1	Test Purpose and Environment	2334

A.9.6.1.2	Test Requirements	2335

A.9.6.2	E-UTRAN TDD	2335

A.9.6.2.1	Test Purpose and Environment	2335

A.9.6.2.2	Test Requirements	2337

A.9.7	UE Rx – Tx Time Difference	2337

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2337

A.9.7.1.1	Test Purpose and Environment	2337

A.9.7.1.2	Test parameters	2337

A.9.7.1.3	Test Requirements	2339

A.9.7.2	E-UTRA TDD	2339

A.9.7.2.1	Test Purpose and Environment	2339

A.9.7.2.2	Test parameters	2339

A.9.7.2.3	Test Requirements	2341

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2341

A.9.7.3.1	Test Purpose and Environment	2341

A.9.7.3.2	Test parameters	2341

A.9.7.3.3	Test Requirements	2344

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2344

A.9.7.4.1	Test Purpose and Environment	2344

A.9.7.4.2	Test Parameters	2344

A.9.7.4.3	Test Requirements	2347

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2347

A.9.7.5.1	Test Purpose and Environment	2347

A.9.7.5.2	Test parameters	2347

A.9.7.5.3	Test Requirements	2350

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2350

A.9.7.6.1	Test Purpose and Environment	2350

A.9.7.6.2	Test Parameters	2350

A.9.7.6.3	Test Requirements	2353

A.9.8	RSTD	2353

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2353

A.9.8.1.1	Test Purpose and Environment	2353

A.9.8.1.2	Test Requirements	2355

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2355

A.9.8.2.1	Test Purpose and Environment	2355

A.9.8.2.2	Test Requirements	2357

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2358

A.9.8.3.1	Test Purpose and Environment	2358

A.9.8.3.2	Test Requirements	2359

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2359

A.9.8.4.1	Test Purpose and Environment	2359

A.9.8.4.2	Test Requirements	2361

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2361

A.9.8.5.1	Test Purpose and Environment	2361

A.9.8.5.2	Test Requirements	2364

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2364

A.9.8.6.1	Test Purpose and Environment	2364

A.9.8.6.2	Test Requirements	2367

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2367

A.9.8.7.1	Test Purpose and Environment	2367

A.9.8.7.2	Test Requirements	2368

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2368

A.9.8.8.1	Test Purpose and Environment	2368

A.9.8.8.2	Test Requirements	2369

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2369

A.9.8.9.1	Test Purpose and Environment	2369

A.9.8.9.2	Test Requirements	2370

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2370

A.9.8.10.1	Test Purpose and Environment	2370

A.9.8.10.2	Test Requirements	2371

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2371

A.9.8.11.1	Test Purpose and Environment	2371

A.9.8.11.2	Test Requirements	2372

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2372

A.9.8.12.1	Test Purpose and Environment	2372

A.9.8.12.2	Test Requirements	2373

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2373

A.9.8.13.1	Test Purpose and Environment	2373

A.9.8.13.2	Test Requirements	2374

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2374

A.9.8.14.1	Test Purpose and Environment	2374

A.9.8.14.2	Test Requirements	2379

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2379

A.9.8.15.1	Test Purpose and Environment	2379

A.9.8.15.2	Test Requirements	2383

A.9.9	RSRP and RSRQ on the serving cell	2383

A.9.9.1	FDD Intra frequency serving cell case	2383

A.9.9.1.1	Test Purpose and Environment	2383

A.9.9.1.2	Test parameters	2383

A.9.9.1.3	Test Requirements	2385

A.9.9.2	TDD Intra frequency serving cell case	2385

A.9.9.2.1	Test Purpose and Environment	2385

A.9.9.2.2	Test parameters	2385

A.9.9.2.3	Test Requirements	2387

A.9.10	SSTD	2387

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2387

A.9.10.1.1	Test Purpose and Environment	2387

A.9.10.1.2	Test parameters	2387

A.9.10.1.3	Test Requirements	2389

A.9.10.3	FDD—FDD Inter frequency case	2389

A.9.10.3.1	Test Purpose and Environment	2389

A.9.10.3.2	Test parameters	2389

A.9.10.3.3	Test Requirements	2393

A.9.10.4	TDD—TDD Inter frequency case	2393

A.9.10.4.1	Test Purpose and Environment	2393

A.9.10.4.2	Test parameters	2393

A.9.10.4.3	Test Requirements	2398

A.9.11 RSSI	2398

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2398

A.9.11.1.1	Test Purpose and Environment	2398

A.9.11.1.2	Test parameters	2398

A.9.11.1.3	Test Requirements	2400

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2400

A.9.11.2.1	Test Purpose and Environment	2400

A.9.11.2.2	Test parameters	2400

A.9.11.2.3	Test Requirements	2402

A.9.12	Channel occupancy	2402

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2402

A.9.12.1.1	Test Purpose and Environment	2402

A.9.12.1.2	Test parameters	2402

A.9.12.1.3	Test Requirements	2404

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2404

A.9.12.2.1	Test Purpose and Environment	2404

A.9.12.2.2	Test parameters	2404

A.9.12.2.3	Test Requirements	2406

A.9.13	RS-SINR	2406

A.9.13.1	FDD Intra-Frequency Case	2406

A.9.13.1.1	Test Purpose and Environment	2406

A.9.13.1.2	Test parameters	2406

A.9.13.1.3	Test Requirements	2408

A.9.13.2	TDD Intra-Frequency Case	2408

A.9.13.2.1	Test Purpose and Environment	2408

A.9.13.2.2	Test parameters	2408

A.9.13.2.3	Test Requirements	2410

A.9.13.5	FDD—TDD Inter frequency case	2410

A.9.13.5.1	Test Purpose and Environment	2410

A.9.13.5.2	Test parameters	2410

A.9.13.5.3	Test Requirements	2414

A.9.13.6	TDD—FDD Inter frequency case	2414

A.9.13.6.1	Test Purpose and Environment	2414

A.9.13.6.2	Test parameters	2414

A.9.13.6.3	Test Requirements	2417

A.10	Proximity-based Services in Any Cell Selection State	2417

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2417

A.10.1.1	Test Purpose and Environment	2417

A.10.1.2	Test Requirements	2418

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2419

A.10.2.1	Test Purpose and Environment	2419

A.10.2.2	Test Requirements	2420

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2420

A.10.3.1	Test Purpose and Environment	2420

A.10.3.2	Test Requirements	2422

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2423

A.10.4.1	Test Purpose and Environment	2423

A.10.4.2	Test Requirements	2425

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2425

A.11.1	V2V UE Transmission Timing Accuracy Test	2425

A.11.1.1	Test Purpose and Environment	2425

A.11.1.2	Test requirements	2426

A.11.2	Interruptions due to V2V sidelink communication	2426

A.11.2.1	Test Purpose and Environment	2426

Annex B (normative): Conditions for RRM requirements applicability for operating bands	2428

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	2428

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	2428

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	2428

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	2428

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	2430

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	2430

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	2431

B.2.1	Conditions for E-UTRAN intra-frequency measurements	2431

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	2431

B.2.3	Conditions for E-UTRAN inter-frequency measurements	2431

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	2432

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	2432

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	2433

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	2433

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	2433

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2434

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	2434

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	2434

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	2435

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	2435

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2435

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2436

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	2436

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	2436

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2436

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2437

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	2437

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	2438

B.3	Conditions for measurements performance requirements for UE	2439

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	2439

B.3.2	Void	2439

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	2439

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	2440

B.3.5	Conditions for UE Rx – Tx time difference	2440

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	2440

B.3.7	Conditions for inter-frequency RSTD measurements	2440

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	2440

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	2440

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	2441

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2441

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2441

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	2441

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2441

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	2441

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	2441

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2442

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	2442

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	2442

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2443

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	2443

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	2443

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2443

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	2443

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	2443

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	2443

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	2443

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	2443

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2444

B.3.21.1	Conditions for RSRP measurements	2444

B.3.21.2	Conditions for RSRQ measurements	2444

B.3.21.3	Conditions for CSI-RSRP measurements	2444

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2444

B.3.22.1	Conditions for RSRP measurements	2444

B.3.22.2	Void	2445

B.3.22.3	Conditions for CSI-RSRP measurements	2445

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2445

B.3.23.1	Conditions for RSRP measurements	2445

B.3.23.2	Conditions for RSRQ measurements	2445

B.3.23.3	Conditions for CSI-RSRP measurements	2445

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2446

B.3.24.1	Conditions for RSRP measurements	2446

B.3.24.2	Conditions for RSRQ measurements	2446

B.3.24.3	Conditions for CSI-RSRP measurements	2446

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2446

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2446

B.4	RRM Requirements Exceptions	2447

B.4.1	General	2447

B.4.2	Receiver sensitivity relaxation for UE supporting CA	2447

B.4.3	Receiver sensitivity relaxation for UE configured with CA	2447

B.4.3.1	Inter-band carrier aggregation	2447

B.4.3.2	Intra-band non-contiguous carrier aggregation	2447

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	2447

B.5	Conditions for Measurement Performance Requirements for ProSe UE	2448

B.5.1	Conditions for S-RSRP Accuracy Requirements	2448

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	2448

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2448

B.5.4	Conditions for SD-RSRP Accuracy Requirements	2449

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	2449

B.6	Conditions for V2X	2450

B.6.1	Conditions for GNSS Reliability Requirements	2450

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	2450

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	2450

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2450

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	2451

Annex C (informative): Change history:	2452

Annex C-MF (informative): Change history:	2493