# Foreword

This Technical Specification has been produced by the MulteFire Alliance (MFA).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as documented in MFA TR MF.100 [MF-3].

# 1	Scope

The present document specifies the Radio Resource Control protocol for the radio interface between UE and E-UTRAN as well as for the radio interface between RN and E-UTRAN.

The scope of the present document also includes:

-	the radio related information transported in a transparent container between source eNB and target eNB upon inter eNB handover;

-	the radio related information transported in a transparent container between a source or target eNB and another system upon inter RAT handover.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document. For the present RRC specification, non-specific reference refers to the latest 3GPP Release 14 version of the document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	MFA TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer ".

[4]	MFA TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); UE Procedures in Idle Mode".

[5]	MFA TS 36.306 "Evolved Universal Terrestrial Radio Access (E-UTRA); UE Radio Access Capabilities".

[6]	MFA TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 36.322:"Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) Specification".

[9]	MFA TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[10]	3GPP TS 22.011: "Service accessibility".

[11]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[12]	3GPP2 C.S0002-F v1.0: "Physical Layer Standard for cdma2000 Spread Spectrum Systems".

[13]	ITU-T Recommendation X.680 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[14]	ITU-T Recommendation X.681 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[15]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[16]	MFA TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[17]	3GPP TS 25.101: "Universal Terrestrial Radio Access (UTRA); User Equipment (UE) radio transmission and reception (FDD)".

[18]	3GPP TS 25.102: "Universal Terrestrial Radio Access (UTRA); User Equipment (UE) radio transmission and reception (TDD)".

[19]	3GPP TS 25.331:"Universal Terrestrial Radio Access (UTRA); Radio Resource Control (RRC); Protocol specification".

[20]	3GPP TS 45.005: "Radio transmission and reception".

[21]	MFA TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[22]	MFA TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[23]	MFA TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[24]	3GPP2 C.S0057-E v1.0: "Band Class Specification for cdma2000 Spread Spectrum Systems".

[25]	3GPP2 C.S0005-F v1.0: "Upper Layer (Layer 3) Signaling Standard for cdma2000 Spread Spectrum Systems".

[26]	3GPP2 C.S0024-C v2.0: "cdma2000 High Rate Packet Data Air Interface Specification".

[27]	3GPP TS 23.003: "Numbering, addressing and identification".

[28]	3GPP TS 45.008: "Radio subsystem link control".

[29]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[30]	3GPP TS 25.123: "Requirements for Support of Radio Resource Management (TDD)".

[31]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access (E-UTRA); Architecture description".

[32]	MFA TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[33]	3GPP2 A.S0008-C v4.0: "Interoperability Specification (IOS) for High Rate Packet Data (HRPD) Radio Access Network Interfaces with Session Control in the Access Network"

[34]	3GPP2 C.S0004-F v1.0: "Signaling Link Access Control (LAC) Standard for cdma2000 Spread Spectrum Systems"

[35]	MFA TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[36]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".

[37]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[38]	3GPP TS 23.038: "Alphabets and Language".

[39]	MFA TS 36.413: "Evolved Universal Terrestrial Radio Access (E-UTRAN); S1 Application Protocol (S1 AP)".

[40]	3GPP TS 25.304: "Universal Terrestrial Radio Access (UTRAN); User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode".

[41]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[42]	MFA TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[43]	3GPP TS 45.005: "GSM/EDGE Radio transmission and reception".

[44]	3GPP2 C.S0087-A v2.0: "E-UTRAN - cdma2000 HRPD Connectivity and Interworking Air Interface Specification"

[45]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control (RRC) protocol".

[46]	3GPP TS 25.223: "Spreading and modulation (TDD)".

[47]	MFA TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[48]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer - Measurements".

[49]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[50]	3GPP TS 45.010:	"Radio subsystem synchronization".

[51]	3GPP TS 23.272: "Circuit Switched Fallback in Evolved Packet System; Stage 2".

[52]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[53]	3GPP2 C.S0097-0 v3.0: "E-UTRAN - cdma2000 1x Connectivity and Interworking Air Interface Specification".

[54]	3GPP TS 36.355: "LTE Positioning Protocol (LPP)".

[55]	3GPP TS 36.216: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer for relaying operation".

[56]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[57]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[58]	3GPP TS 32.422: "Telecommunication management; Subsriber and equipment trace; Trace control and confiuration management".

[59]	3GPP TS 22.368: "Service Requirements for Machine Type Communications; Stage 1".

[60]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[61]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[62]	3GPP TS 22.146: "Multimedia Broadcast/Multicast Service (MBMS); Stage 1".

[63]	3GPP TR 36.816: "Evolved Universal Terrestrial Radio Access (E-UTRA); Study on signalling and procedure for interference avoidance for in-device coexistence".

[64]	IS-GPS-200F: "Navstar GPS Space Segment/Navigation User Segment Interfaces".

[65]	3GPP TS 25.307: "Requirement on User Equipments (UEs) supporting a release-independent frequency band".

[66]	3GPP TS 24.312: "Access Network Discovery and Selection Function (ANDSF) Management Object (MO)".

[67]	IEEE 802.11-2012, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.

[68]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[69]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3".

[70]	3GPP TS 24.333: "Proximity-services (ProSe) Management Objects (MO)".

[71]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2- Measurements".

[72]	3GPP TS 24.105: "Application specific Congestion control for Data Communication (ACDC) Management Object (MO)".

[73]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2".

[74]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks".

[75]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses; Stage-2".

[76]	Wi-Fi Alliance® Technical Committee, Hotspot 2.0 Technical Task Group Hotspot 2.0 (Release 2) Technical Specification Version 3.11.

[77]	3GPP TS 22.101: "Service aspects; Service principles".

[78]	3GPP TS 36.307: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) supporting a release-independent frequency band".

[MF-1]	MFA TS MF.201: MulteFire Network Architecture for PLMN access mode (Stage 2).

[MF-2]	MFA TS MF.202: MulteFire Network Architecture for Neutral Host Network access mode (Stage 2).

[MF-3]	MFA TR MF.100: “Guidelines for MulteFire Specifications (Release 1)”

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Anchor carrier: In NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB are transmitted.

Bandwidth Reduced: Refers to operation in downlink and uplink with a limited channel bandwidth of 6 PRBs.

Cellular IoT EPS Optimisation: Provides improved support of small data transfer, as defined in MFA TS 24.301 [35].

Commercial Mobile Alert System: Public Warning System that delivers Warning Notifications provided by Warning Notification Providers to CMAS capable UEs.

Common access barring parameters: The common access barring parameters refer to the access class barring parameters that are broadcast in SystemInformationBlockType2 outside the list of PLMN specific parameters (i.e. in ac-BarringPerPLMN-List).

Control plane CIoT EPS optimisation: Enables support of efficient transport of user data (IP, non-IP or SMS) over control plane via the MME without triggering data radio bearer establishment, as defined in MFA TS 24.301 [35].

CSG member cell: A cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN and for which the CSG whitelist of the UE includes an entry comprising cell's CSG ID and the respective PLMN identity.

Dual Connectivity: A UE in RRC_CONNECTED is configured with Dual Connectivity when configured with a Master and a Secondary Cell Group.

EU-Alert: Public Warning System that delivers Warning Notifications provided by Warning Notification Providers using the same AS mechanisms as defined for CMAS.

Field: The individual contents of an information element are referred as fields.

Floor: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Information element: A structural element containing a single or multiple fields is referred as information element.

Korean Public Alert System (KPAS): Public Warning System that delivers Warning Notifications provided by Warning Notification Providers using the same AS mechanisms as defined for CMAS.

Master Cell Group: For a UE not configured with DC, the MCG comprises all serving cells. For a UE configured with DC, the MCG concerns a subset of the serving cells comprising of the PCell and zero or more secondary cells.

MBMS service: MBMS bearer service as defined in 3GPP TS 23.246 [56] (i.e. provided via an MRB or an SC-MRB).

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 180 kHz.

NB-IoT UE: A UE that uses NB-IoT.

Non-anchor carrier: In NB-IoT, a carrier where the UE does not assume that NPSS/NSSS/NPBCH/SIB-NB are transmitted.

Primary Cell: The cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure, or the cell indicated as the primary cell in the handover procedure.

Primary Secondary Cell: The SCG cell in which the UE is instructed to perform random access when performing the SCG change procedure.

Primary Timing Advance Group: Timing Advance Group containing the PCell or the PSCell.

PUCCH SCell: An SCell configured with PUCCH.

Secondary Cell: A cell, operating on a secondary frequency, which may be configured once an RRC connection is established and which may be used to provide additional radio resources.

Secondary Cell Group: For a UE configured with DC, the subset of serving cells not part of the MCG, i.e. comprising of the PSCell and zero or more other secondary cells.

Secondary Timing Advance Group: Timing Advance Group neither containing the PCell nor the PSCell. A secondary timing advance group contains at least one cell with configured uplink.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/ DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of one or more cells comprising of the primary cell and all secondary cells.

Sidelink: UE to UE interface for sidelink communication and sidelink discovery. The sidelink corresponds to the PC5 interface as defined in 3GPP TS 23.303 [68].

Sidelink communication: AS functionality enabling ProSe Direct Communication as defined in 3GPP TS 23.303 [68], between two or more nearby UEs, using E-UTRA technology but not traversing any network node.

Sidelink discovery: AS functionality enabling ProSe Direct Discovery as defined in 3GPP TS 23.303 [68], using E-UTRA technology but not traversing any network node.

User plane CIoT EPS optimisation: Enables support for change from EMM-IDLE mode to EMM-CONNECTED mode without the need for using the Service Request procedure, as defined in MFA TS 24.301 [35].

Timing Advance Group: A group of serving cells that is configured by RRC and that, for the cells with an UL configured, use the same timing reference cell and the same Timing Advance value. A Timing Advance Group only includes cells of the same cell group i.e. it either includes MCG cells or SCG cells.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], MFA TS 36.300 [9] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] or MFA TS 36.300 [9].

1xRTT	CDMA2000 1x Radio Transmission Technology

AB	Access Barring

ACDC	Application specific Congestion control for Data Communication

ACK	Acknowledgement

AM	Acknowledged Mode

ANDSF	Access Network Discovery and Selection Function

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

BCCH	Broadcast Control Channel

BCD	Binary Coded Decimal

BCH	Broadcast Channel

BL	Bandwidth reduced Low complexity

BLER	Block Error Rate

BR	Bandwidth Reduced

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

CA	Carrier Aggregation

CCCH	Common Control Channel

CCO	Cell Change Order

CE	Coverage Enhancement

CG	Cell Group

CIoT	Cellular IoT

CMAS	Commercial Mobile Alert Service

CP	Control Plane

C-RNTI	Cell RNTI

CRS	Cell-specific Reference Signal

CSFB	CS fallback

CSG	Closed Subscriber Group

CSI	Channel State Information

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DFN	Direct Frame Number

DL	Downlink

DL-SCH	Downlink Shared Channel

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

EAB	Extended Access Barring

eDRX	Extended DRX

EHPLMN	Equivalent Home Public Land Mobile Network

eIMTA	Enhanced Interference Management and Traffic Adaptation

ENB	Evolved Node B

EPC	Evolved Packet Core

EPDCCH	Enhanced Physical Downlink Control Channel

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

GERAN	GSM/EDGE Radio Access Network

GNSS	Global Navigation Satellite System

G-RNTI	Group RNTI

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

HFN	Hyper Frame Number

HPLMN	Home Public Land Mobile Network

HRPD	CDMA2000 High Rate Packet Data

H-SFN	Hyper SFN

IDC	In-Device Coexistence

IE	Information element

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

IoT	Internet of Things

ISM	Industrial, Scientific and Medical

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

LAA	Licensed-Assisted Access

LWA	LTE-WLAN Aggregation

LWAAP	LTE-WLAN Aggregation Adaptation Protocol

LWIP	LTE-WLAN Radio Level Integration with IPsec Tunnel

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCG	Master Cell Group

MCPTT	Mission Critical Push To Talk

MDT	Minimization of Drive Tests

MF	MulteFire

MIB	Master Information Block

MO	Mobile Originating

MPDCCH	MTC Physical Downlink Control Channel

MRB	MBMS Point to Multipoint Radio Bearer

MRO	Mobility Robustness Optimisation

MSI	MCH Scheduling Information

MT	Mobile Terminating

N/A	Not Applicable

NACC	Network Assisted Cell Change

NAICS		Network Assisted Interference Cancellation/Suppression

NAS	Non Access Stratum

NB-IoT	NarrowBand Internet of Things

NHN	Neutral Host Network

NPBCH	Narrowband Physical Broadcast channel

NPDCCH	Narrowband Physical Downlink Control channel

NPDSCH	Narrowband Physical Downlink Shared channel

NPRACH	Narrowband Physical Random Access channel

NPSS	Narrowband Primary Synchronization Signal

NPUSCH	Narrowband Physical Uplink Shared channel

NRS	Narrowband Reference Signal

NSSS 	Narrowband Secondary Synchronization Signal

PCCH	Paging Control Channel

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PMK	Pairwise Master Key

ProSe	Proximity based Services

PS	Public Safety (in context of sidelink), Packet Switched (otherwise)

PSCell	Primary Secondary Cell

PSK	Pre-Shared Key

PTAG	Primary Timing Advance Group

PUCCH	Physical Uplink Control Channel

QCI	QoS Class Identifier

QoS	Quality of Service

RACH	Random Access CHannel

RAT	Radio Access Technology

RB	Radio Bearer

RCLWI	RAN Controlled LTE-WLAN Integration

RLC	Radio Link Control

RMTC	RSSI Measurement Timing Configuration

RN	Relay Node

RNTI	Radio Network Temporary Identifier

ROHC	RObust Header Compression

RPLMN	Registered Public Land Mobile Network

RRC	Radio Resource Control

RSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

SAE	System Architecture Evolution

SAP	Service Access Point

SC	Sidelink Control

SCell	Secondary Cell

SCG	Secondary Cell Group

SC-MRB	Single Cell MRB

SC-RNTI	Single Cell RNTI

SD-RSRP	Sidelink Discovery Reference Signal Received Power

SFN	System Frame Number

SI	System Information

SIB	System Information Block

SI-RNTI	System Information RNTI

SL	Sidelink

SLSS	Sidelink Synchronisation Signal

SMC	Security Mode Control

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRB	Signalling Radio Bearer

S-RSRP	Sidelink Reference Signal Received Power

SSAC	Service Specific Access Control

SSTD	SFN and Subframe Timing Difference

STAG	Secondary Timing Advance Group

S-TMSI	SAE Temporary Mobile Station Identifier

TA	Tracking Area

TAG	Timing Advance Group

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TM	Transparent Mode

TPC-RNTI	Transmit Power Control RNTI

T-RPT	Time Resource Pattern of Transmission

TTT	Time To Trigger

UE	User Equipment

UICC	Universal Integrated Circuit Card

UL	Uplink

UL-SCH	Uplink Shared Channel

UM	Unacknowledged Mode

UP	User Plane

UTC	Coordinated Universal Time

UTRAN	Universal Terrestrial Radio Access Network

VoLTE	Voice over Long Term Evolution

WLAN	Wireless Local Area Network

WT	WLAN Termination

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

# 4	General

## 4.1	Introduction

In this specification, (parts of) procedures and messages specified for the UE equally apply to the RN for functionality necessary for the RN. There are also (parts of) procedures and messages which are only applicable to the RN in its communication with the E-UTRAN, in which case the specification denotes the RN instead of the UE. Such RN-specific aspects are not applicable to the UE.

NB-IoT is a non backward compatible variant of E-UTRAN supporting a reduced set of functionality. In this specification, (parts of) procedures and messages specified for the UE equally apply to the UE in NB-IoT. There are also some features and related procedures and messages that are not supported by UEs in NB-IoT.

In particular, the following features are not supported in NB-IoT and corresponding procedures and messages do not apply to the UE in NB-IoT:

-	Connected mode mobility (Handover and measurement reporting);

-	Inter-RAT cell reselection or inter-RAT mobility in connected mode;

-	CSG;

-	Relay Node (RN);

-	Carrier Aggregation (CA);

-	Dual connectivity (DC);

-	GBR (QoS);

-	ACB, EAB, SSAC and ACDC;

-	MBMS;

-	Self-configuration and self-optimisation;

-	Measurement logging and reporting for network performance optimisation;

-	Public warning systems e.g. CMAS, ETWS and PWS;

-	Real time services (including emergency call);

-	CS services and CS fallback;

-	In-device coexistence;

-	RAN assisted WLAN interworking;

-	Network-assisted interference cancellation/suppression;

-	Sidelink (including direct communication and direct discovery).

NOTE: In regard to mobility, NB-IoT is a separate RAT from E-UTRAN.

In this specification, there are also (parts of) procedures and messages which are only applicable to UEs in NB-IoT, in which case this is stated explicitly.

This specification is organised as follows:

-	sub-clause 4.2 describes the RRC protocol model;

-	sub-clause 4.3 specifies the services provided to upper layers as well as the services expected from lower layers;

-	sub-clause 4.4 lists the RRC functions;

-	clause 5 specifies RRC procedures, including UE state transitions;

-	clause 6 specifies the RRC message in a mixed format (i.e. tabular & ASN.1 together);

-	clause 7 specifies the variables (including protocol timers and constants) and counters to be used by the UE;

-	clause 8 specifies the encoding of the RRC messages;

-	clause 9 specifies the specified and default radio configurations;

-	clause 10 specifies the RRC messages transferred across network nodes;

-	clause 11 specifies the UE capability related constraints and performance requirements.

## 4.2	Architecture

### 4.2.1	UE states and state transitions including inter RAT

A UE is in RRC_CONNECTED when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows:

-	RRC_IDLE:

-	A UE specific DRX may be configured by upper layers (not applicable for NB-IoT);

-	UE controlled mobility;

-	The UE:

-	Monitors a Paging channel to detect incoming calls, system information change, for ETWS capable UEs, ETWS notification, and for CMAS capable UEs, CMAS notification;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Acquires system information.

-	Performs logging of available measurements together with location and time for logged measurement configured UEs.

-	RRC_CONNECTED:

-	Transfer of unicast data to/from UE.

-	At lower layers, the UE may be configured with a UE specific DRX.

-	For UEs supporting CA, use of one or more SCells, aggregated with the PCell, for increased bandwidth;

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth;

-	Network controlled mobility, i.e. handover and cell change order with optional network assistance (NACC) to GERAN (not applicable for NB-IoT);

-	The UE:

-	Monitors a Paging channel and/ or System Information Block Type 1 contents to detect system information change, for ETWS capable UEs, ETWS notification, and for CMAS capable UEs, CMAS notification (not applicable for NB-IoT);

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	Provides channel quality and feedback information (not applicable for NB-IoT);

-	Performs neighbouring cell measurements and measurement reporting (not applicable for NB-IoT);

-	Acquires system information (not applicable for NB-IoT).

The following figure not only provides an overview of the RRC states in E-UTRA, but also illustrates the mobility support between E-UTRAN, UTRAN and GERAN.

Figure 4.2.1-1: E-UTRA states and inter RAT mobility procedures, 3GPP

The following figure illustrates the mobility support between E-UTRAN, CDMA2000 1xRTT and CDMA2000 HRPD. The details of the CDMA2000 state models are out of the scope of this specification.

Figure 4.2.1-2: Mobility procedures between E-UTRA and CDMA2000

The inter-RAT handover procedure(s) supports the case of signalling, conversational services, non-conversational services and combinations of these.

In addition to the state transitions shown in Figure 4.2.1-1 and Figure 4.2.1-2, there is support for connection release with redirection information from E-UTRA RRC_CONNECTED to GERAN, UTRAN and CDMA2000 (HRPD Idle/ 1xRTT Dormant mode).

For NB-IoT, mobility between E-UTRA and UTRAN, GERAN and between E-UTRA and CDMA2000 1xRTT and CDMA2000 HRPD is not supported and hence only the E-UTRA states depicted in Figure 4.2.1-1 are applicable.

### 4.2.2	Signalling radio bearers

"Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RB) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined:

-	SRB0 is for RRC messages using the CCCH logical channel;

-	SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel;

-	For NB-IoT, SRB1bis is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the activation of security, all using DCCH logical channel;

-	SRB2 is for RRC messages which include logged measurement information as well as for NAS messages, all using DCCH logical channel. SRB2 has a lower-priority than SRB1 and is always configured by E-UTRAN after security activation. SRB2 is not applicable for NB-IoT.

In downlink piggybacking of NAS messages is used only for one dependant (i.e. with joint success/ failure) procedure: bearer establishment/ modification/ release. In uplink NAS message piggybacking is used only for transferring the initial NAS message during connection setup.

NOTE:		The NAS messages transferred via SRB2 are also contained in RRC messages, which however do not include any RRC protocol control information.

Once security is activated, all RRC messages on SRB1 and SRB2, including those containing NAS or non-3GPP messages, are integrity protected and ciphered by PDCP. NAS independently applies integrity protection and ciphering to the NAS messages.

For a UE configured with DC, all RRC messages, regardless of the SRB used and both in downlink and uplink, are transferred via the MCG.

## 4.3	Services

### 4.3.1	Services provided to upper layers

The RRC protocol offers the following services to upper layers:

-	Broadcast of common control information;

-	Notification of UEs in RRC_IDLE, e.g. about a terminating call, for ETWS, for CMAS;

-	Transfer of dedicated control information, i.e. information for one specific UE.

### 4.3.2	Services expected from lower layers

In brief, the following are the main services that RRC expects from lower layers:

-	PDCP: integrity protection and ciphering;

-	RLC: reliable and in-sequence transfer of information, without introducing duplicates and with support for segmentation and concatenation.

Further details about the services provided by Packet Data Convergence Protocol layer (e.g. integrity and ciphering) are provided in 3GPP TS 36.323 [8]. The services provided by Radio Link Control layer (e.g. the RLC modes) are specified in 3GPP TS 36.322 [7]. Further details about the services provided by Medium Access Control layer (e.g. the logical channels) are provided in MFA TS 36.321 [6]. The services provided by physical layer (e.g. the transport channels) are specified in MFA TS 36.302 [3].

## 4.4	Functions

The RRC protocol includes the following main functions:

-	Broadcast of system information:

-	Including NAS common information;

-	Information applicable for UEs in RRC_IDLE, e.g. cell (re-)selection parameters, neighbouring cell information and information (also) applicable for UEs in RRC_CONNECTED, e.g. common channel configuration information.

-	Including ETWS notification, CMAS notification (not applicable for NB-IoT);

-	RRC connection control:

-	Paging;

-	Establishment/ modification/ suspension / resumption / release of RRC connection, including e.g. assignment/ modification of UE identity (C-RNTI), establishment/ modification/ release of SRB1, SRB1bis and SRB2, access class barring;

-	Initial security activation, i.e. initial configuration of AS integrity protection (SRBs) and AS ciphering (SRBs, DRBs);

-	For RNs, configuration of AS integrity protection for DRBs;

-	RRC connection mobility including e.g. intra-frequency and inter-frequency handover, associated security handling, i.e. key/ algorithm change, specification of RRC context information transferred between network nodes;

NOTE 1:	In NB-IoT, only key change (but no re-keying) at RRC Connection Resumption and RRC context information transfer are applicable.

-	Establishment/ modification/ release of RBs carrying user data (DRBs);

-	Radio configuration control including e.g. assignment/ modification of ARQ configuration, HARQ configuration, DRX configuration;

-	For RNs, RN-specific radio configuration control for the radio interface between RN and E-UTRAN;

-	In case of CA, cell management including e.g. change of PCell, addition/modification/ release of SCell(s) and addition/modification/release of STAG(s);

-	In case of DC, cell management including e.g. change of PSCell, addition/modification/ release of SCG cell(s) and addition/modification/release of SCG TAG(s).

-	QoS control including assignment/ modification of semi-persistent scheduling (SPS) configuration information for DL and UL, assignment/ modification of parameters for UL rate control in the UE, i.e. allocation of a priority and a prioritised bit rate (PBR) for each RB (not applicable for NB-IoT);

-	Recovery from radio link failure;

-	In case of LWA, RCLWI and LWIP, WLAN mobility set management including e.g. addition/modification/ release of WLAN(s) from the WLAN mobility set;

-	Inter-RAT mobility including e.g. security activation, transfer of RRC context information (not applicable for NB-IoT);

-	Measurement configuration and reporting (not applicable for NB-IoT):

-	Establishment/ modification/ release of measurements (e.g. intra-frequency, inter-frequency and inter- RAT measurements);

-	Setup and release of measurement gaps;

-	Measurement reporting;

-	Other functions including e.g. transfer of dedicated NAS information and non-3GPP dedicated information, transfer of UE radio access capability information, support for E-UTRAN sharing (multiple PLMN identities);

-	Generic protocol error handling;

-	Support of self-configuration and self-optimisation (not applicable for NB-IoT);

-	Support of measurement logging and reporting for network performance optimisation (see 3GPP TS 37.320 [60]) (not applicable for NB-IoT);

NOTE 2:	Random access is specified entirely in the MAC including initial transmission power estimation.

# 5	Procedures

## 5.1	General

### 5.1.1	Introduction

The procedural requirements are structured according to the main functional areas: system information (5.2), connection control (5.3), inter-RAT mobility (5.4) and measurements (5.5). In addition sub-clause 5.6 covers other aspects e.g. NAS dedicated information transfer, UE capability transfer, sub-clause 5.7 specifies the generic error handling, sub-clause 5.8 covers MBMS, clause 5.8 cover SC-PTM (i.e. MBMS service reception via SC-MRB) sub-clause 5.9 covers RN-specific procedures and sub-clause 5.10 covers sidelink.

For NB-IoT, only a subset of the above procedural requirements applies: system information (5.2), connection control (5.3), some part of other aspects (5.6), and general error handling (5.7). Subclauses inter-RAT mobility (5.4), measurements (5.5), MBMS (5.8), RN procedures (5.9) and Sidelink (5.10) are not applicable in NB-IoT.

### 5.1.2	General requirements

The UE shall:

1> 	process the received messages in order of reception by RRC, i.e. the processing of a message shall be completed before starting the processing of a subsequent message;

NOTE 1:	E-UTRAN may initiate a subsequent procedure prior to receiving the UE's response of a previously initiated procedure.

1>	if the received message is a MulteFire (MF) message transmitted over PCCH, CCCH or DCCH:

2>	apply the contents of message-3GPP and message-MF, at least one of which must be present, when forming the message, following the combining and presence criteria of sections 5.7.5MF and 6.7MF;

2>	if an MF version of an IE is present in message-MF;

3>	it extends and, where behaviour is conflicting, takes precedence over the 3GPP version in message-3GPP;

3>	unless otherwise stated, procedures described for messages and IEs for E-UTRAN are also applicable to MF messages and MF IEs with the same generic type identifier;

NOTE MF1: 	MF messages transmitted over PCCH, CCCH and DCCH consist of either a baseline message inherited from E-UTRA (message-3GPP), an associated MF-specific message (message-MF), or both. message-MF can be a self-contained message replacing the baseline message or alternatively, it can be an extension to the baseline message. The MF message versions are transmitted on the same signalling radio bearer (SRB), SAP, logical channel, and in the same direction as specified for the associated 3GPP message.

1> 	within a sub-clause execute the steps according to the order specified in the procedural description;

1> 	consider the term 'radio bearer' (RB) to cover SRBs and DRBs but not MRBs unless explicitly stated otherwise;

1> 	set the rrc-TransactionIdentifier in the response message, if included, to the same value as included in the message received from E-UTRAN that triggered the response message;

NOTE MF2: 	If an rrc-TransactionIdentifier is included in the MF message, it is included in message-3GPP if present, otherwise it is included in message-MF.

1> 	upon receiving a choice value set to setup:

2> 	apply the corresponding received configuration and start using the associated resources, unless explicitly specified otherwise;

1> 	upon receiving a choice value set to release:

2> 	clear the corresponding configuration and stop using the associated resources;

1> 	upon handover to E-UTRA; or

1> 	upon receiving an RRCConnectionReconfiguration message including the fullConfig:

2> 	apply the Conditions in the ASN.1 for inclusion of the fields for the DRB/PDCP/RLC setup during the reconfiguration of the DRBs included in the drb-ToAddModList;

NOTE 2:	At each point in time, the UE keeps a single value for each field except for during handover when the UE temporarily stores the previous configuration so it can revert back upon handover failure. In other words: when the UE reconfigures a field, the existing value is released except for during handover.

NOTE 3:	Although not explicitly stated, the UE initially considers all functionality to be deactivated/ released until it is explicitly stated that the functionality is setup/ activated. Correspondingly, the UE initially considers lists to be empty e.g. the list of radio bearers, the list of measurements.

1> 	upon receiving an extension field comprising the entries in addition to the ones carried by the original field (regardless of whether E-UTRAN may signal more entries in total); apply the following generic behaviour if explicitly stated to be applicable:

2> 	create a combined list by concatenating the additional entries included in the extension field to the original field while maintaining the order among both the original and the additional entries;

2> 	for the combined list, created according to the previous, apply the same behaviour as defined for the original field;

NOTE 4:	A field comprising a list of entries normally includes ‘list’ in the field name. The typical way to extend (the size of) such a list is to introduce a field comprising the additional entries, which should include ‘listExt’ in the name of the field/ IE. E.g. field1List-RAT, field1ListExt-RAT.

## 5.2	System information

### 5.2.1	Introduction

#### 5.2.1.1	General

System information is divided into the MasterInformationBlock (MIB) and a number of SystemInformationBlocks (SIBs). The MIB includes a limited number of most essential and most frequently transmitted parameters that are needed to acquire other information from the cell, and is transmitted on BCH. SIBs other than SystemInformationBlockType1 are carried in SystemInformation (SI) messages and mapping of SIBs to SI messages is flexibly configurable by schedulingInfoList included in SystemInformationBlockType1, with restrictions that: each SIB is contained only in a single SI message, and at most once in that message; only SIBs having the same scheduling requirement (periodicity) can be mapped to the same SI message; SystemInformationBlockType2 is always mapped to the SI message that corresponds to the first entry in the list of SI messages in schedulingInfoList. For MF, SystemInformationBlockType2 is not mapped to any SI message, but is transmitted within SystemInformationBlockTypeMF1. There may be multiple SI messages transmitted with the same periodicity. SystemInformationBlockType1 and all SI messages are transmitted on DL-SCH.

The Bandwidth reduced Low Complexity (BL) UEs and UEs in Coverage Enhancement (CE) apply Bandwidth Reduced (BR) version of the SIB or SI messages. A UE considers itself in enhanced coverage as specified in MFA TS 36.304 [4]. In this and subsequent clauses, anything applicable for a particular SIB or SI message equally applies to the corresponding BR version unless explicitly stated otherwise.

For NB-IoT, a reduced set of system information block with similar functionality but different content is defined; the UE applies the NB-IoT (NB) version of the MIB and the SIBs. These are denoted MasterInformationBlock-NB and SystemInformationBlockTypeX-NB in this specification. All other system information blocks (without NB suffix) are not applicable to NB-IoT; this is not further stated in the corresponding text.

For MF, MF specific system information blocks, SystemInformationBlockTypeMFx (SIB-MF), are defined. SIB-MFs other than SystemInformationBlockTypeMF1 are carried in SystemInformation-MF (SI-MF) messages, and a SI-MF message may include a 3GPP SystemInformation message containing 3GPP SIBs. The mapping of SIB-MFs and SIBs to SI-MF is configurable by schedulingInfoList included in SystemInformationBlockTypeMF1. SystemInformationBlockTypeMF1 and all SI-MF messages are transmitted on DL-SCH.

NOTE 1: 	The physical layer imposes a limit to the maximum size a SIB can take. When DCI format 1C is used the maximum allowed by the physical layer is 1736 bits (217 bytes) while for format 1A the limit is 2216 bits (277 bytes), see MFA TS 36.212 [22] and MFA TS 36.213 [23]. For BL UEs and UEs in CE, the maximum SIB and SI message size is 936 bits, see MFA TS 36.213 [23]. For NB-IoT, the maximum SIB and SI message size is 680 bits, see MFA TS 36.213 [23].

In addition to broadcasting, E-UTRAN may provide SystemInformationBlockType1, including the same parameter values, via dedicated signalling i.e., within an RRCConnectionReconfiguration message. MF may alternatively provide SystemInformationBlockType-MF1.

The UE applies the system information acquisition and change monitoring procedures for the PCell, except when being a BL UE or a UE in CE or a NB-IoT UE in RRC_CONNECTED mode while T311 is not running. For an SCell, E-UTRAN provides, via dedicated signalling, all system information relevant for operation in RRC_CONNECTED when adding the SCell. However, a UE that is configured with DC shall aquire the MasterInformationBlock of the PSCell but use it only to determine the SFN timing of the SCG, which may be different from the MCG. Upon change of the relevant system information of a configured SCell, E-UTRAN releases and subsequently adds the concerned SCell, which may be done with a single RRCConnectionReconfiguration message. If the UE is receiving or interested to receive an MBMS service in a cell, the UE shall apply the system information acquisition and change monitoring procedure to acquire parameters relevant for MBMS operation and apply the parameters acquired from system information only for MBMS operation for this cell.

NOTE 2:	E-UTRAN may configure via dedicated signalling different parameter values than the ones broadcast in the concerned SCell.

For MF, SystemInformationBlockTypeMF1 contains all mandatory parts of SystemInformationBlockType1 and SystemInformationBlockType2, plus additional MF specific information elements. Optional parts of SystemInformationBlockType1and SystemInformationBlockType2 may also be included. SystemInformationBlockType2 is not mapped to any SI messages in schedulingInfoList in SystemInformationBlockType1.

NOTE MF1: 	Except for SIB-MF1, the content in SIB-MFx is not necessarily related to the content in SIBx.

NOTE MF2: 	For MF, procedures and actions related to reception of SystemInformationBlockType1 and SystemInformationBlockType2, also apply when they are received via SystemInformationBlockTypeMF1.

An RN configured with an RN subframe configuration does not need to apply the system information acquisition and change monitoring procedures. Upon change of any system information relevant to an RN, E-UTRAN provides the system information blocks containing the relevant system information to an RN configured with an RN subframe configuration via dedicated signalling using the RNReconfiguration message. For RNs configured with an RN subframe configuration, the system information contained in this dedicated signalling replaces any corresponding stored system information and takes precedence over any corresponding system information acquired through the system information acquisition procedure. The dedicated system information remains valid until overridden.

NOTE 3:	E-UTRAN may configure an RN, via dedicated signalling, with different parameter values than the ones broadcast in the concerned cell.

5.2.1.2	Scheduling

The MIB uses a fixed schedule with a periodicity of 40 ms and repetitions made within 40 ms. The first transmission of the MIB is scheduled in subframe #0 of radio frames for which the SFN mod 4 = 0, and repetitions are scheduled in subframe #0 of all other radio frames. For TDD/FDD system with a bandwidth larger than 1.4 MHz that supports BL UEs or UEs in CE, MIB transmission may be repeated in subframe#9 of the previous radio frame for FDD and subframe #5 of the same radio frame for TDD.

For MF, the MIB-MF provides the same content for a period of 40 ms, and repetitions of the same MIB-MF content may be made within these 40 ms. Changes of MIB-MF take place only in radio frames for which SFN mod 4 = 0. The scheduling period for MIB-MF starts in radio frames for which SFN mod 4 = 0. For the first occurrence within this scheduling period, the MIB-MF transmission may occur in any subframe within the radio frame in case it is scheduled in the subframe containing the discovery signal, which uses a configurable scheduling periodicity (dmtc-Periodicity, see Section 5.5.2.10MF1). Otherwise, the first transmission of MIB-MF may be scheduled in subframe #0 of radio frames for which SFN mod 4 = 0. The repetitions of MIB-MF may be scheduled only in subframe #0 of radio frames for which SFN mod 4 ≠ 0. Since the MIB-MF transmission timing may vary when transmitted in radio frames for which SFN mod 4 = 0, the sf-Offset field within the MIB-MF payload is used to indicate the subframe offset compared to subframe #0 or subframe #5 for frames where SFN mod 4 = 0 as depicted in Figure 5.2.1.2-MF1.

Scheduling period boundary; SFN mod 4 = 0

Figure 5.2.1.2-MF1. sf-Offset value for MIB-MF transmissions at the scheduling period boundary.

The SystemInformationBlockType1 uses a fixed schedule with a periodicity of 80 ms and repetitions made within 80 ms. The first transmission of SystemInformationBlockType1 is scheduled in subframe #5 of radio frames for which the SFN mod 8 = 0, and repetitions are scheduled in subframe #5 of all other radio frames for which SFN mod 2 = 0.

For BL UEs or UEs in CE, MIB is applied which may be provided with additional repetitions, while for SIB1 and further SI messages, separate messages are used which are scheduled independantly and with content that may differ. The separate instance of SIB1 is named as SystemInformationBlockType1-BR. The SystemInformationBlockType1-BR uses a schedule with a periodicity of 80ms. TBS for SystemInformationBlockType1-BR and the repetitions made within 80ms are indicated via schedulingInfoSIB1-BR in MIB.

The SystemInformationBlockTypeMF1 is scheduled with the discovery signal which may be transmitted in any subframe of the serving cell DMTC occasion. SystemInformationBlockTypeMF1 may also be transmitted in any subframe #0 outside the serving cell DMTC occasion.

The SI messages are transmitted within periodically occurring time domain windows (referred to as SI-windows) using dynamic scheduling. Each SI message is associated with a SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI is transmitted. The length of the SI-window is common for all SI messages, and is configurable. Within the SI-window, the corresponding SI message can be transmitted a number of times in any subframe other than MBSFN subframes, uplink subframes in TDD, and subframe #5 of radio frames for which SFN mod 2 = 0. For MF operation, scheduling outside the DMTC occasion is restricted to any subframe #0. The UE acquires the detailed time-domain scheduling (and other information, e.g. frequency-domain scheduling, used transport format) from decoding SI-RNTI on PDCCH (see MFA TS 36.321 [6]). For a BL UE or a UE in CE, the detailed time/frequency domain scheduling information for the SI messages is provided in SystemInformationBlockType1-BR.

For UEs other than BL UE or UEs in CE SI-RNTI is used to address SystemInformationBlockType1 as well as all SI messages.

SystemInformationBlockType1 configures the SI-window length and the transmission periodicity for the SI messages.

#### 5.2.1.2a	Scheduling for NB-IoT

The MasterInformationBlock-NB (MIB-NB) uses a fixed schedule with a periodicity of 640 ms and repetitions made within 640 ms. The first transmission of the MIB-NB is scheduled in subframe #0 of radio frames for which the SFN mod 64 = 0 and repetitions are scheduled in subframe #0 of all other radio frames. The transmissions are arranged in 8 independently decodable blocks of 80 ms duration.

The SystemInformationBlockType1-NB (SIB1-NB) uses a fixed schedule with a periodicity of 2560 ms. SIB1-NB transmission occurs in subframe #4 of every other frame in 16 continuous frames. The starting frame for the first transmission of the SIB1-NB is derived from the cell PCID and the number of repetitions within the 2560 ms period and repetitions are made, equally spaced, within the 2560 ms period (see MFA TS 36.213 [23]). TBS for SystemInformationBlockType1-NB and the repetitions made within the 2560 ms are indicated by schedulingInfoSIB1 field in the MIB-NB.

The SI messages are transmitted within periodically occurring time domain windows (referred to as SI-windows) using scheduling information provided in SystemInformationBlockType1-NB. Each SI message is associated with a SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI is transmitted. The length of the SI-window is common for all SI messages, and is configurable.

Within the SI-window, the corresponding SI message can be transmitted a number of times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS.The UE acquires the detailed time/frequency domain scheduling information and other information, e.g. used transport format for the SI messages from schedulingInfoList field in SystemInformationBlockType1-NB. The UE is not required to accumulate several SI messages in parallel but may need to accumulate a SI message across multiple SI windows, depending on coverage condition.

SystemInformationBlockType1-NB configures the SI-window length and the transmission periodicity for all SI messages.

#### 5.2.1.3	System information validity and notification of changes

Change of system information (other than for ETWS, CMAS and EAB parameters and other than for AB parameters for NB-IoT) only occurs at specific radio frames, i.e. the concept of a modification period is used. System information may be transmitted a number of times with the same content within a modification period, as defined by its scheduling. The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SystemInformationBlockType1-BR, modification period boundaries for BL UEs and UEs in CE are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. For NB-IoT, H-SFN is always provided and the modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0.

To enable system information update notification for RRC_IDLE UEs configured to use a DRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 256 =0. For NB-IoT, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0.

When the network changes (some of the) system information, it first notifies the UEs about this change, i.e. this may be done throughout a modification period. In the next modification period, the network transmits the updated system information. These general principles are illustrated in figure 5.2.1.3-1, in which different colours indicate different system information. Upon receiving a change notification, the UE not configured to use a DRX cycle that is longer than the modification period acquires the new system information immediately from the start of the next modification period. Upon receiving a change notification applicable to eDRX, a UE in RRC_IDLE configured to use a DRX cycle that is longer than the modification period acquires the updated system information immediately from the start of the next eDRX acquisition period. The UE applies the previously acquired system information until the UE acquires the new system information. The possible boundaries of modification for SystemInformationBlockType1-BR are defined by SFN values for which SFN mod 512 = 0 except for notification of ETWS/CMAS for which the eNB may change SystemInformationBlockType1-BR content at any time. For NB-IoT, the possible boundaries of modification for SystemInformationBlockType1-NB are defined by SFN values for which (H-SFN * 1024 + SFN) mod 4096 = 0.

Figure 5.2.1.3-1: Change of system Information

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. If the UE is in RRC_CONNECTED or is not configured to use a DRX cycle longer than the modification period in RRC_IDLE, and receives a Paging message including the systemInfoModification, it knows that the system information will change at the next modification period boundary. If a UE in RRC_IDLE is configured to use a DRX cycle longer than the modification period, and the notification is received in a Paging message including the systemInfoModification-eDRX, it acquires the updated system information at the next eDRX acquisition period boundary.Although the UE may be informed about changes in system information, no further details are provided e.g. regarding which system information will change, except if systemInfoValueTagSI is received by BL UEs or UEs in CE.

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell. In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification and EAB modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and MFA TS 36.212 [22].

NOTE:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

SystemInformationBlockType1 (or MasterInformationBlock-NB in NB-IoT) includes a value tag systemInfoValueTag, that indicates if a change has occurred in the SI messages. UEs may use systemInfoValueTag, e.g. upon return from out of coverage, to verify if the previously stored SI messages are still valid. Additionally, for other than BL UEs or UEs in CE or NB-IoT UEs, the UE considers stored system information to be invalid after 3 hours from the moment it was successfully confirmed as valid, unless specified otherwise. BL UE or UE in CE considers stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid, unless the UE is configured by parameter si-ValidityTime to consider stored system information to be invalid 3 hours after validity confirmation. NB-IoT UE considers stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid. If a BL UE, UE in CE or NB-IoT UE in RRC_CONNECTED state considers the stored system information invalid, the UE shall continue using the stored system information while in RRC_CONNECTED state in the serving cell.

For BL UEs or UEs in CE or NB-IoT UEs, the change of specific SI message can additionally be indicated by a SI message specific value tag systemInfoValueTagSI. If systemInfoValueTag included in the SystemInformationBlockType1-BR (or MasterInformationBlock-NB in NB-IoT) is different from the one of the stored system information and if systemInfoValueTagSI is included in the SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) for a specific SI message and is different from the stored one, the UE shall consider this specific SI message to be invalid. If only systemInfoValueTag is included and is different from the stored one, the BL UE or UE in CE should consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, SystemInformationBlockType12 and SystemInformationBlockType14 to be invalid; the NB-IoT UE should consider any stored system information except SystemInformationBlockType14-NB to be invalid.

E-UTRAN may not update systemInfoValueTag upon change of some system information e.g. ETWS information, CMAS information, regularly changing parameters like time information (SystemInformationBlockType8, SystemInformationBlockType16, hyperSFN-MSB in SystemInformationBlockType1-NB), EAB and AB parameters. Similarly, E-UTRAN may not include the systemInfoModification within the Paging message upon change of some system information.

The UE that is not configured to use a DRX cycle longer than the modification period verifies that stored system information remains valid by either checking systemInfoValueTag in SystemInformationBlockType1 (or MasterInformationBlock-NB in NB-IoT) after the modification period boundary, or attempting to find the systemInfoModification indication at least modificationPeriodCoeff times during the modification period in case no paging is received, in every modification period. If no paging message is received by the UE during a modification period, the UE may assume that no change of system information will occur at the next modification period boundary. If UE in RRC_CONNECTED, during a modification period, receives one paging message, it may deduce from the presence/ absence of systemInfoModification whether a change of system information other than ETWS information, CMAS information and EAB parameters will occur in the next modification period or not.

When the RRC_IDLE UE is configured with a DRX cycle that is longer than the modification period, and at least one modification period boundary has passed since the UE last verified validity of stored system information, the UE verifies that stored system information remains valid by checking the systemInfoValueTag before establishing or resuming an RRC connection.

ETWS and/or CMAS capable UEs in RRC_CONNECTED shall attempt to read paging at least once every defaultPagingCycle to check whether ETWS and/or CMAS notification is present or not.

#### 5.2.1.4	Indication of ETWS notification

ETWS primary notification and/ or ETWS secondary notification can occur at any point in time. The Paging message is used to inform ETWS capable UEs in RRC_IDLE and UEs in RRC_CONNECTED about presence of an ETWS primary notification and/ or ETWS secondary notification. If the UE receives a Paging message including the etws-Indication, it shall start receiving the ETWS primary notification and/ or ETWS secondary notification according to schedulingInfoList contained in SystemInformationBlockType1. If the UE receives Paging message including the etws-Indication while it is acquiring ETWS notification(s), the UE shall continue acquiring ETWS notification(s) based on the previously acquired schedulingInfoList until it re-acquires schedulingInfoList in SystemInformationBlockType1.

NOTE:	The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the etws-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType10 and SystemInformationBlockType11. The UE may or may not receive a Paging message including the etws-Indication and/or systemInfoModification when ETWS is no longer scheduled.

ETWS primary notification is contained in SystemInformationBlockType10 and ETWS secondary notification is contained in SystemInformationBlockType11. Segmentation can be applied for the delivery of a secondary notification. The segmentation is fixed for transmission of a given secondary notification within a cell (i.e. the same segment size for a given segment with the same messageIdentifier, serialNumber and warningMessageSegmentNumber). An ETWS secondary notification corresponds to a single CB data IE as defined according to 3GPP TS 23.041 [37].

#### 5.2.1.5	Indication of CMAS notification

CMAS notification can occur at any point in time. The Paging message is used to inform CMAS capable UEs in RRC_IDLE and UEs in RRC_CONNECTED about presence of one or more CMAS notifications. If the UE receives a Paging message including the cmas-Indication, it shall start receiving the CMAS notifications according to schedulingInfoList contained in SystemInformationBlockType1. If the UE receives Paging message including the cmas-Indication while it is acquiring CMAS notification(s), the UE shall continue acquiring CMAS notification(s) based on the previously acquired schedulingInfoList until it re-acquires schedulingInfoList in SystemInformationBlockType1.

NOTE:	The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the cmas-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType12. The UE may or may not receive a Paging message including the cmas-Indication and/or systemInfoModification when SystemInformationBlockType12 is no longer scheduled.

CMAS notification is contained in SystemInformationBlockType12. Segmentation can be applied for the delivery of a CMAS notification. The segmentation is fixed for transmission of a given CMAS notification within a cell (i.e. the same segment size for a given segment with the same messageIdentifier, serialNumber and warningMessageSegmentNumber). E-UTRAN does not interleave transmissions of CMAS notifications, i.e. all segments of a given CMAS notification transmission are transmitted prior to those of another CMAS notification. A CMAS notification corresponds to a single CB data IE as defined according to 3GPP TS 23.041 [37].

#### 5.2.1.6	Notification of EAB parameters change

Change of EAB parameters can occur at any point in time. The EAB parameters are contained in SystemInformationBlockType14. The Paging message is used to inform EAB capable UEs in RRC_IDLE about a change of EAB parameters or that SystemInformationBlockType14 is no longer scheduled. If the UE receives a Paging message including the eab-ParamModification, it shall acquire SystemInformationBlockType14 according to schedulingInfoList contained in SystemInformationBlockType1. If the UE receives a Paging message including the eab-ParamModification while it is acquiring SystemInformationBlockType14, the UE shall continue acquiring SystemInformationBlockType14 based on the previously acquired schedulingInfoList until it re-acquires schedulingInfoList in SystemInformationBlockType1.

NOTE:	The EAB capable UE is not expected to periodically check schedulingInfoList contained in SystemInformationBlockType1.

#### 5.2.1.7	Access Barring parameters change in NB-IoT

Change of Access Barring (AB) parameters can occur at any point in time. The AB parameters are contained in SystemInformationBlockType14-NB. Update of the AB parameters does not impact the systemInfoValueTag in the MasterInformationBlock-NB or the systemInfoValueTagSI in SystemInformationBlockType1-NB.

A NB-IoT UE checks ab-Enabled indication in theMasterInformationBlock-NB to know whether access barring is enabled. If access barring is enabled the UE shall not initiate the RRC connection establishment / resume for all access causes except mobile terminating calls until the UE has a valid version of SystemInformationBlockType14-NB.

### 5.2.2	System information acquisition

#### 5.2.2.1	General

Figure 5.2.2.1-1: System information acquisition, normal

Figure 5.2.2.1-MF1:  MF specific System Information acquisition

The UE applies the system information acquisition procedure to acquire the AS- and NAS- system information that is broadcasted by the E-UTRAN. The procedure applies to UEs in RRC_IDLE and UEs in RRC_CONNECTED.

For BL UE and UE in CE, specific conditions apply, as specified below.

#### 5.2.2.2	Initiation

The UE shall apply the system information acquisition procedure upon selecting (e.g. upon power on) and upon re-selecting a cell, after handover completion, after entering E-UTRA from another RAT, upon return from out of coverage, upon receiving a notification that the system information has changed, upon receiving an indication about the presence of an ETWS notification, upon receiving an indication about the presence of a CMAS notification, upon receiving a notification that the EAB parameters have changed, upon receiving a request from CDMA2000 upper layers and upon exceeding the maximum validity duration. Unless explicitly stated otherwise in the procedural specification, the system information acquisition procedure overwrites any stored system information, i.e. delta configuration is not applicable for system information and the UE discontinues using a field if it is absent in system information unless explicitly specified otherwise.

In RRC_CONNECTED, BL UEs and UEs in CE are required to acquire system information when T311 is running or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell.

NOTE:	Upon handover, E-UTRAN provides system information required by the UE in RRC_CONNECTED except MIB with RRC signalling, i.e. systemInformationBlockType1Dedicated and mobilityControlInfo.

#### 5.2.2.3	System information required by the UE

The UE shall:

1> 	ensure having a valid version, as defined below, of (at least) the following system information, also referred to as the 'required' system information:

2> 	if in RRC_IDLE:

3> 	if the UE is a NB-IoT UE:

4> 	the MasterInformationBlock-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB;

3> 	else:

4> 	the MasterInformationBlock and SystemInformationBlockType1 (or SystemInformationBlockType1-BR depending on whether the UE is a BL UE or the UE in CE) as well as SystemInformationBlockType2 through SystemInformationBlockType8 (depending on support of the concerned RATs), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking);

2> 	if in RRC_CONNECTED; and

2> 	the UE is not a BL UE; and

2> 	the UE is not in CE; and

2> 	the UE is not a NB-IoT UE:

3> 	the MasterInformationBlock, SystemInformationBlockType1 and SystemInformationBlockType2 as well as SystemInformationBlockType8 (depending on support of CDMA2000), SystemInformationBlockType17 (depending on support of RAN-assisted WLAN interworking);

2> 	if in RRC_CONNECTED and T311 is running; and

2> 	the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE;

3> 	the MasterInformationBlock (or MasterInformationBlock-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT);

1> 	delete any stored system information after 3 hours or 24 hours from the moment it was confirmed to be valid as defined in 5.2.1.3, unless specified otherwise;

1> 	consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, systemInformationBlockType12 and systemInformationBlockType14 (systemInformationBlockType14-NB in NB-IoT) to be invalid if systemInfoValueTag included in the SystemInformationBlockType1 (MasterInformationBlock-NB in NB-IoT) is different from the one of the stored system information and in case of NB-IoT UEs, BL UEs and UEs in CE, systemInfoValueTagSI is not broadcasted. Otherwise consider system information validity as defined in 5.2.1.3;

#### 5.2.2.4	System information acquisition by the UE

The UE shall:

1> 	apply the specified BCCH configuration defined in 9.1.1.1 or BR-BCCH configuration defined in 9.1.1.8;

1> 	if the procedure is triggered by a system information change notification:

2> 	if the UE uses an idle DRX cycle longer than the modification period:

3> 	start acquiring the required system information, as defined in 5.2.2.3, from the next eDRX acquisition period boundary;

2> 	else

3> 	start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received;

NOTE 1:	The UE continues using the previously received system information until the new system information has been acquired.

1> 	if the UE is in RRC_IDLE and enters a cell for which the UE does not have stored a valid version of the system information required in RRC_IDLE, as defined in 5.2.2.3:

2> 	acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_IDLE, as defined in 5.2.2.3;

1> 	following successful handover completion to a PCell for which the UE does not have stored a valid version of the system information required in RRC_CONNECTED, as defined in 5.2.2.3:

2> 	acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_CONNECTED, as defined in 5.2.2.3;

2> 	upon acquiring the concerned system information:

3> 	discard the corresponding radio resource configuration information included in the radioResourceConfigCommon previously received in a dedicated message, if any;

1> 	following a request from CDMA2000 upper layers:

2> 	acquire SystemInformationBlockType8, as defined in 5.2.3;

1> 	neither initiate the RRC connection establishment/resume procedure nor initiate transmission of the RRCConnectionReestablishmentRequest message until the UE has a valid version of the MasterInformationBlock (MasterInformationBlock-NB in NB-IoT) and SystemInformationBlockType1 (SystemInformationBlockType1-NB in NB-IoT) messages as well as SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT);

1> 	not initiate the RRC connection establishment/resume procedure subject to EAB until the UE has a valid version of SystemInformationBlockType14, if broadcast;

1> 	if the UE is ETWS capable:

2> 	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3> 		discard any previously buffered warningMessageSegment;

3> 	clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

2> 	when the UE acquires SystemInformationBlockType1 following ETWS indication, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3> 	if schedulingInfoList indicates that SystemInformationBlockType10 is present:

4> 	if the UE is in CE:

5> 	start acquiring SystemInformationBlockType10;

4> 	else

5> 	start acquiring SystemInformationBlockType10 immediately;

3> 	if schedulingInfoList indicates that SystemInformationBlockType11 is present:

4> 	start acquiring SystemInformationBlockType11 immediately;

NOTE 2:	UEs shall start acquiring SystemInformationBlockType10 and SystemInformationBlockType11 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

1> 	if the UE is CMAS capable:

2> 	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3> 	discard any previously buffered warningMessageSegment;

3> 	clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment;

2> 	when the UE acquires SystemInformationBlockType1 following CMAS indication, upon entering a cell during RRC_IDLE, following successful handover and upon connection re-establishment:

3> 	if schedulingInfoList indicates that SystemInformationBlockType12 is present:

4> 	acquire SystemInformationBlockType12;

NOTE 3:	UEs shall start acquiring SystemInformationBlockType12 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

1> 	if the UE is interested to receive MBMS services:

2> 	if the UE is capable of MBMS reception as specified in 5.8:

3> 	if schedulingInfoList indicates that SystemInformationBlockType13 is present and the UE does not have stored a valid version of this system information block:

4> 	acquire SystemInformationBlockType13;

2> 	if the UE is capable of SC-PTM reception as specified in 5.8a:

3> 	if schedulingInfoList indicates that SystemInformationBlockType20 is present and the UE does not have stored a valid version of this system information block:

4> 	acquire SystemInformationBlockType20;

2> 	if the UE is capable of MBMS Service Continuity:

3> 	if schedulingInfoList indicates that SystemInformationBlockType15 is present and the UE does not have stored a valid version of this system information block:

4> 	acquire SystemInformationBlockType15;

1> 	if the UE is EAB capable:

2> 	when the UE does not have stored a valid version of SystemInformationBlockType14 upon entering RRC_IDLE, or when the UE acquires SystemInformationBlockType1 following EAB parameters change notification, or upon entering a cell during RRC_IDLE, or before establishing an RRC connection if using eDRX with DRX cycle longer than the modification period:

3> 	if schedulingInfoList indicates that SystemInformationBlockType14 is present:

4> 	start acquiring SystemInformationBlockType14 immediately;

3> 	else:

4> 	discard SystemInformationBlockType14, if previously received;

NOTE 4:	EAB capable UEs start acquiring SystemInformationBlockType14 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

NOTE 5:	EAB capable UEs maintain an up to date SystemInformationBlockType14 in RRC_IDLE.

1> 	if the UE is capable of sidelink communication and is configured by upper layers to receive or transmit sidelink communication:

2> 	if the cell used for sidelink communication meets the S-criteria as defined in MFA TS 36.304 [4]; and

2> 	if schedulingInfoList indicates that SystemInformationBlockType18 is present and the UE does not have stored a valid version of this system information block:

3> 	acquire SystemInformationBlockType18;

1> 	if the UE is capable of sidelink discovery and is configured by upper layers to receive or transmit sidelink discovery announcements on the primary frequency:

2> 	if schedulingInfoList of the serving cell/ PCell indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3> 	acquire SystemInformationBlockType19;

1> 	if the UE is capable of sidelink discovery and, for each of the one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19 and for which the UE is configured by upper layers to receive sidelink discovery announcements on:

2> 	if SystemInformationBlockType19 of the serving cell/ PCell does not provide the corresponding reception resources; and

2> 	if schedulingInfoList of the cell on the concerned frequency indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3> 	acquire SystemInformationBlockType19;

1> 	if the UE is capable of sidelink discovery and, for each of the one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19 and for which the UE is configured by upper layers to transmit sidelink discovery announcements on:

2> 	if SysemInformationBlockType19 of the serving cell/ PCell includes discTxResourcesInterFreq which is set to acquireSI-FromCarrier; and

2> 	if schedulingInfoList of the cell on the concerned frequency indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3> 	acquire SystemInformationBlockType19;

1> 	if the UE is a NB-IoT UE and if ab-Enabled included in MasterInformationBlock-NB is set to TRUE:

2> 	not initiate the RRC connection establishment/resume procedure for all access causes except mobile terminating calls until the UE has a valid version of SystemInformationBlockType14-NB;

The UE may apply the received SIBs immediately, i.e. the UE does not need to delay using a SIB until all SI messages have been received. The UE may delay applying the received SIBs until completing lower layer procedures associated with a received or a UE originated RRC message, e.g. an ongoing random access procedure.

NOTE 6:	While attempting to acquire a particular SIB, if the UE detects from schedulingInfoList that it is no longer present, the UE should stop trying to acquire the particular SIB.

#### 5.2.2.5	Essential system information missing

The UE shall:

1> 	if in RRC_IDLE or in RRC_CONNECTED while T311 is running:

2> 	if the UE is unable to acquire the MasterInformationBlock (MasterInformationBlock-NB in NB-IoT); or

2> 	if the UE is neither a BL UE nor in CE nor in NB-IoT and the UE is unable to acquire the SystemInformationBlockType1; or

2> 	if the BL UE or UE in CE is unable to acquire SystemInformationBlockType1-BR or SystemInformationBlockType1-BR is not scheduled; or

2> 	if the NB-IoT UE is unable to acquire the SystemInformationBlockType1-NB:

3> 	consider the cell as barred in accordance with MFA TS 36.304 [4]; and

3> 	perform barring as if intraFreqReselection is set to allowed, and as if the csg-Indication is set to FALSE;

2> 	else if the UE is unable to acquire the SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT):

3> 	treat the cell as barred in accordance with MFA TS 36.304 [4];

5.2.2.6	Actions upon reception of the MasterInformationBlock message

Upon receiving the MasterInformationBlock message the UE shall:

1> 	apply the radio resource configuration included in the phich-Config;

1> 	if the UE is in RRC_IDLE or if the UE is in RRC_CONNECTED while T311 is running:

2> 	if the UE has no valid system information stored according to 5.2.2.3 for the concerned cell:

3> 	apply the received value of dl-Bandwidth to the ul-Bandwidth until SystemInformationBlockType2 is received;

Upon receiving the MasterInformationBlock-MF message the UE shall:

1>	if the UE is in RRC_IDLE or if the UE is in RRC_CONNECTED while T311 is running:

2>	if the UE has no valid system information stored according to 5.2.2.3 for the concerned cell:

3>	apply the received value of dl-Bandwidth to the ul-Bandwidth until SystemInformationBlockType2 is received;

1>	if the UE has not established subframe timing for the cell:

2> if the message was transmitted in radio frame SFN mod 4 = 0

3> use the sf-Offset parameter included.

2> else if not transmitted in radio frame SFN mod 4 = 0:

3> ignore the sf-Offset parameter.

Upon receiving the MasterInformationBlock-NB message the UE shall:

1> 	apply the radio resource configuration included in accordance with the operationModeInfo.

5.2.2.7	Actions upon reception of the SystemInformationBlockType1 message

Upon receiving the SystemInformationBlockType1 either via broadcast or via dedicated signalling, the UE shall:

1> 	if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and

1> 	if the UE is a category 0 UE according to MFA TS 36.306 [5]; and

1> 	if category0Allowed is not included in SystemInformationBlockType1:

2> 	consider the cell as barred in accordance with MFA TS 36.304 [4];

1> 	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2> 	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2> 	forward the cellIdentity to upper layers;

2> 	forward the trackingAreaCode to upper layers;

1> 	else:

2> 	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE and it is not a downlink only band; or

2> 	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE and they are not downlink only bands:

3> 	forward the cellIdentity to upper layers;

3> 	forward the trackingAreaCode to upper layers;

3> 	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo or the multiBandInfoList-v10j0 is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo or multiBandInfoList-v10j0:

4> 	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo or multiBandInfolist-v10j0;

4> 	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

5> 	apply the additionalPmax;

4> 	else:

5> 	apply the p-Max;

3> 	else:

4> 	apply the additionalSpectrumEmission in SystemInformationBlockType2 and the p-Max;

2> 	else:

3> 	consider the cell as barred in accordance with MFA TS 36.304 [4]; and

3> 	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

Upon receiving the SystemInformationBlockType1-NB, the UE shall:

1> 	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE and it is not a downlink only band; or

1> 	if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE and they are not downlink only bands:

2> 	forward the cellIdentity to upper layers;

2> 	forward the trackingAreaCode to upper layers;

2> 	forward the attachWithoutPDN-Connectivity to upper layers, if received for the selected PLMN;

2> 	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo:

3> 	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo;

3> 	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

4> 	apply the additionalPmax;

3> 	else:

4> 	apply the p-Max;

2> 	else:

3> 	apply the additionalSpectrumEmission in SystemInformationBlockType2-NB and the p-Max;

1> 	else:

2> 	consider the cell as barred in accordance with MFA TS 36.304 [4]; and

2> 	perform barring as if intraFreqReselection is set to notAllowed.

5.2.2.8	Actions upon reception of SystemInformation messages

No UE requirements related to the contents of the SystemInformation messages apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

5.2.2.9	Actions upon reception of SystemInformationBlockType2

Upon receiving SystemInformationBlockType2, the UE shall:

1> 	apply the configuration included in the radioResourceConfigCommon;

1> 	if upper layers indicate that a (UE specific) paging cycle is configured:

2> 	apply the shortest of the (UE specific) paging cycle and the defaultPagingCycle included in the radioResourceConfigCommon;

1> 	if the mbsfn-SubframeConfigList is included:

2> 	consider that DL assignments may occur in the MBSFN subframes indicated in the mbsfn-SubframeConfigList under the conditions specified in MFA TS 36.213 [23, Section 7.1];

1> 	apply the specified PCCH configuration defined in 9.1.1.3;

1> 	not apply the timeAlignmentTimerCommon;

1> 	if in RRC_CONNECTED and UE is configured with RLF timers and constants values received within rlf-TimersAndConstants:

2> 	not update its values of the timers and constants in ue-TimersAndConstants except for the value of timer T300;

1> 	if in RRC_CONNECTED while T311 is not running; and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators or multipleNS-Pmax:

2> 	disregard the additionalSpectrumEmission and ul-CarrierFreq, if received, while in RRC_CONNECTED;

1> 	forward attachWithoutPDN-Connectivity to upper layers, if received for the selected PLMN;

1> 	forward cp-CIoT-EPS-Optimisation to upper layers, if received for the selected PLMN;

1> 	forward up-CIoT-EPS-Optimisation to upper layers, if received for the selected PLMN;

Upon receiving SystemInformationBlockType2-NB, the UE shall:

1> 	apply the configuration included in the radioResourceConfigCommon;

1> 	apply the defaultPagingCycle included in the radioResourceConfigCommon;

1> 	apply the specified PCCH configuration defined in 9.1.1.3.

5.2.2.10	Actions upon reception of SystemInformationBlockType3

Upon receiving SystemInformationBlockType3, the UE shall:

1> 	if in RRC_IDLE, the redistributionServingInfo is included and the UE is redistribution capable:

2> 	perform E-UTRAN inter-frequency redistribution procedure as specified in MFA TS 36.304 [4, Section 5.2.4.10];

1> 		if in RRC_IDLE, or in RRC_CONNECTED while T311 is running:

2> 	if, for the frequency band selected by the UE (from the procedure in Section 5.2.2.7) to represent the serving cell’s carrier frequency, the freqBandInfo or the multiBandInfoList-v10j0 is present in SystemInformationBlockType3 and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo or multiBandInfoList-v10j0:

3> apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo or multiBandInfoList-v10j0;

3>		if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

4>		apply the additionalPmax;

3> else:

4> apply the p-Max;

2> else:

3> apply the p-Max;

No UE requirements related to the contents of SystemInformationBlockType3-NB apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

5.2.2.11	Actions upon reception of SystemInformationBlockType4

No UE requirements related to the contents of this SystemInformationBlock (SystemInformationBlockType4 or SystemInformationBlockType4-NB) apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

5.2.2.12	Actions upon reception of SystemInformationBlockType5

Upon receiving SystemInformationBlockType5, the UE shall:

1> 	if in RRC_IDLE, the redistributionInterFreqInfo is included and the UE is redistribution capable:

2> 	perform E-UTRAN inter-frequency redistribution procedure as specified in MFA TS 36.304 [4, Section 5.2.4.10];

- if in RRC_IDLE, or in RRC_CONNECTED while T311 is running:

2>	if the frequency band selected by the UE to represent a non-serving E UTRA carrier frequency is not a downlink only band:

3>	if, for the selected frequency band, the freqBandInfo or the multiBandInfoList-v10j0 is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within freqBandInfo or multiBandInfoList-v10j0:

4>		apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo or multiBandInfoList-v10j0;

4>		if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

5>	apply the additionalPmax;

4> else:

5> apply the p-Max;

3> else:

4> apply the p-Max;

No UE requirements related to the contents of SystemInformationBlockType5-NB apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.13	Actions upon reception of SystemInformationBlockType6

No UE requirements related to the contents of this SystemInformationBlock apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.14	Actions upon reception of SystemInformationBlockType7

No UE requirements related to the contents of this SystemInformationBlock apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.15	Actions upon reception of SystemInformationBlockType8

Upon receiving SystemInformationBlockType8, the UE shall:

1> 	if sib8-PerPLMN-List is included and the UE is capable of network sharing for CDMA2000:

2> 	apply the CDMA2000 parameters below corresponding to the RPLMN;

1> 	if the systemTimeInfo is included:

2> 	forward the systemTimeInfo to CDMA2000 upper layers;

1> 	if the UE is in RRC_IDLE and if searchWindowSize is included:

2> 	forward the searchWindowSize to CDMA2000 upper layers;

1> 	if parametersHRPD is included:

2> 	forward the preRegistrationInfoHRPD to CDMA2000 upper layers only if the UE has not received the preRegistrationInfoHRPD within an RRCConnectionReconfiguration message after entering this cell;

2> 	if the cellReselectionParametersHRPD is included:

3> 	forward the neighCellList to the CDMA2000 upper layers;

1> 	if the parameters1XRTT is included:

2> 	if the csfb-RegistrationParam1XRTT is included:

3> 	forward the csfb-RegistrationParam1XRTT to the CDMA2000 upper layers which will use this information to determine if a CS registration/re-registration towards CDMA2000 1xRTT in the EUTRA cell is required;

2> 	else:

3> 	indicate to CDMA2000 upper layers that CSFB Registration to CDMA2000 1xRTT is not allowed;

2> 	if the longCodeState1XRTT is included:

3> 	forward the longCodeState1XRTT to CDMA2000 upper layers;

2> 	if the cellReselectionParameters1XRTT is included:

3> 	forward the neighCellList to the CDMA2000 upper layers;

2> 	if the csfb-SupportForDualRxUEs is included:

3> 	forward csfb-SupportForDualRxUEs to the CDMA2000 upper layers;

2> 	else:

3> 	forward csfb-SupportForDualRxUEs, with its value set to FALSE, to the CDMA2000 upper layers;

2> 	if ac-BarringConfig1XRTT is included:

3> 	forward ac-BarringConfig1XRTT to the CDMA2000 upper layers;

2> 	if the csfb-DualRxTxSupport is included:

3> 	forward csfb-DualRxTxSupport to the CDMA2000 upper layers;

2> 	else:

3> 	forward csfb-DualRxTxSupport, with its value set to FALSE, to the CDMA2000 upper layers;

5.2.2.16	Actions upon reception of SystemInformationBlockType9

Upon receiving SystemInformationBlockType9, the UE shall:

1> 	if hnb-Name is included, forward the hnb-Name to upper layers;

5.2.2.17	Actions upon reception of SystemInformationBlockType10

Upon receiving SystemInformationBlockType10, the UE shall:

1> 	forward the received warningType, messageIdentifier and serialNumber to upper layers;

5.2.2.18	Actions upon reception of SystemInformationBlockType11

Upon receiving SystemInformationBlockType11, the UE shall:

1> 	if there is no current value for messageIdentifier and serialNumber for SystemInformationBlockType11; or

1> 	if either the received value of messageIdentifier or of serialNumber or of both are different from the current values of messageIdentifier and serialNumber for SystemInformationBlockType11:

2> 	use the received values of messageIdentifier and serialNumber for SystemInformationBlockType11 as the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

2> 	discard any previously buffered warningMessageSegment;

2> 	if all segments of a warning message have been received:

3> 	assemble the warning message from the received warningMessageSegment;

3> 	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

3> 	stop reception of SystemInformationBlockType11;

3> 	discard the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

2> 	else:

3> 	store the received warningMessageSegment;

3> 	continue reception of SystemInformationBlockType11;

1> 		else if all segments of a warning message have been received:

2> 	assemble the warning message from the received warningMessageSegment;

2> 	forward the received complete warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

2> 	stop reception of SystemInformationBlockType11;

2> 	discard the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

1> 	else:

2> 	store the received warningMessageSegment;

2> 	continue reception of SystemInformationBlockType11;

The UE should discard any stored warningMessageSegment and the current value of messageIdentifier and serialNumber for SystemInformationBlockType11 if the complete warning message has not been assembled within a period of 3 hours.

#### 5.2.2.19	Actions upon reception of SystemInformationBlockType12

Upon receiving SystemInformationBlockType12, the UE shall:

1> 	if the SystemInformationBlockType12 contains a complete warning message:

2> 	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

2> 	continue reception of SystemInformationBlockType12;

1> 	else:

2> 	if the received values of messageIdentifier and serialNumber are the same (each value is the same) as a pair for which a warning message is currently being assembled:

3> 	store the received warningMessageSegment;

3> 	if all segments of a warning message have been received:

4> 	assemble the warning message from the received warningMessageSegment;

4> 	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

4> 	stop assembling a warning message for this messageIdentifier and serialNumber and delete all stored information held for it;

3> 	continue reception of SystemInformationBlockType12;

2> 	else if the received values of messageIdentifier and/or serialNumber are not the same as any of the pairs for which a warning message is currently being assembled:

3> 	start assembling a warning message for this messageIdentifier and serialNumber pair;

3> 	store the received warningMessageSegment;

3> 	continue reception of SystemInformationBlockType12;

The UE should discard warningMessageSegment and the associated values of messageIdentifier and serialNumber for SystemInformationBlockType12 if the complete warning message has not been assembled within a period of 3 hours.

NOTE: 	The number of warning messages that a UE can re-assemble simultaneously is a function of UE implementation.

#### 5.2.2.20	Actions upon reception of SystemInformationBlockType13

No UE requirements related to the contents of this SystemInformationBlock apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.21	Actions upon reception of SystemInformationBlockType14

No UE requirements related to the contents of this SystemInformationBlock (SystemInformationBlockType14 or SystemInformationBlockType14-NB) apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.22	Actions upon reception of SystemInformationBlockType15

No UE requirements related to the contents of this SystemInformationBlock apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.23	Actions upon reception of SystemInformationBlockType16

No UE requirements related to the contents of this SystemInformationBlock (SystemInformationBlockType16 or SystemInformationBlockType16-NB) apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

#### 5.2.2.24	Actions upon reception of SystemInformationBlockType17

Upon receiving SystemInformationBlockType17, the UE shall:

1> 	if wlan-OffloadConfigCommon corresponding to the RPLMN is included:

2> 	if the UE is not configured with rclwi-Configuration with command set to steerToWLAN:

3> 	apply the wlan-Id-List corresponding to the RPLMN;

2> 	if not configured with the wlan-OffloadConfigDedicated:

3> 	apply the wlan-OffloadConfigCommon corresponding to the RPLMN;

#### 5.2.2.25	Actions upon reception of SystemInformationBlockType18

Upon receiving SystemInformationBlockType18, the UE shall:

1> 	if SystemInformationBlockType18 message includes the commConfig:

2> 	if configured to receive sidelink communication:

3> 	from the next SC period, as defined by sc-Period, use the resource pool indicated by commRxPool for sidelink communication monitoring, as specified in 5.10.3;

2> 	if configured to transmit sidelink communication:

3> 	from the next SC period, as defined by sc-Period, use the resource pool indicated by commTxPoolNormalCommon, commTxPoolNormalCommonExt or by commTxPoolExceptional for sidelink communication transmission, as specified in 5.10.4;

#### 5.2.2.26	Actions upon reception of SystemInformationBlockType19

Upon receiving SystemInformationBlockType19, the UE shall:

1> 	if SystemInformationBlockType19 message includes the discConfig or discConfigPS:

2> 	from the next discovery period, as defined by discPeriod, use the resources indicated by discRxPool, discRxResourcesInterFreq or discRxPoolPS for sidelink discovery monitoring, as specified in 5.10.5;

2> 	if SystemInformationBlockType19 message includes the discTxPoolCommon or discTxPoolPS-Common; and the UE is in RRC_IDLE:

3> 	from the next discovery period, as defined by discPeriod, use the resources indicated by discTxPoolCommon or discTxPoolPS-Common for sidelink discovery announcement, as specified in 5.10.6;

2> 	if the SystemInformationBlockType19 message includes the discTxPowerInfo:

3> 	use the power information included in discTxPowerInfo for sidelink discovery transmission on the serving frequency, as specified in MFA TS 36.213 [23];

1> 	if SystemInformationBlockType19 message includes the discConfigRelay:

2> 	if the SystemInformationBlockType19 message includes the txPowerInfo:

3> 	use the power information included in txPowerInfo for sidelink discovery transmission on the corresponding non-serving frequency, as specified in MFA TS 36.213 [23];

#### 5.2.2.27	Actions upon reception of SystemInformationBlockType20

No UE requirements related to the contents of this SystemInformationBlock apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

### 5.2.2MF1	System information acquisition for MF

5.2.2MF1.1	Actions upon reception of the SystemInformationBlockTypeMF1 message

Upon receiving the SystemInformationBlockTypeMF1, the UE shall:

1> perform all applicable actions as specified in 5.2.2.7 and 5.2.2.9;

1> if cellIdentity and trackingAreaCode need to be forwarded to higher layers according to procedures specified in 5.2.2.7:

2> forward the PLMN specific cellIdentity and trackingAreaCode for each alternative PLMN signalled in altCellAccessInfoList to higher layers if present;

2> forward NHN-Identity to higher layers if present;

2> forward PSP-Identity for the supported PSPs to higher layers if present;

NOTE MF:	The PLMNs signalled in cellAccessRelatedInfo in SIB1 and altCellAccessInfoList in SIB-MF1 are forwarded to higher layers for PLMN selection.

5.2.2MF1.2	Actions upon reception of SystemInformationBlockTypeMF2

Upon receiving the SystemInformationBlockTypeMF2, the UE shall:

1> forward PSP-Identity for the supported PSPs to higher layers;

5.2.2MF1.3	Actions upon reception of SystemInformationBlockTypeMF3

Upon receiving the SystemInformationBlockTypeMF3, the UE shall:

1> forward NHN Name to higher layers if present;

1> forward field content of online sign-up support to higher layers if present.

### 5.2.3	Acquisition of an SI message

When acquiring an SI message, the UE shall:

1> 	determine the start of the SI-window for the concerned SI message as follows:

2> 	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in SystemInformationBlockType1 or SystemInformationBlockTypeMF1;

2> 	determine the integer value x = (n – 1)*w, where w is the si-WindowLength;

2> 	the SI-window starts at the subframe #a, where a = x mod 10, in the radio frame for which SFN mod T = FLOOR(x/10), where T is the si-Periodicity of the concerned SI message;

NOTE:	E-UTRAN should configure an SI-window of 1 ms only if all SIs are scheduled before subframe #5 in radio frames for which SFN mod 2 = 0.

1> 	receive DL-SCH using the SI-RNTI from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received, excluding the following subframes:

2> 	subframe #5 in radio frames for which SFN mod 2 = 0;

2> 	any MBSFN subframes;

2> 	any uplink subframes in TDD;

1> 		if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message;

### 5.2.3a	Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE

When acquiring an SI message, the BL UE or UE in CE or NB-IoT UE shall:

1> 	determine the start of the SI-window for the concerned SI message as follows:

2> 	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT);

2> 	determine the integer value x = (n – 1)*w, where w is the si-WindowLength-BR (or si-WindowLength in NB-IoT);

2> 	if the UE is a NB-IoT UE:

3> 	the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset, where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset);

2> 	else:

3> 	the SI-window starts at the subframe #0 in the radio frame for which SFN mod T = FLOOR(x/10), where T is the si-Periodicity of the concerned SI message;

1> 	if the UE is a NB-IoT UE:

2> 	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB and SystemInformationBlockType1-NB. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

1> 	else:

2> 	receive and accumulate SI message transmissions on DL-SCH on narrowband provided by si-Narrowband, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength-BR, only in radio frames as provided in si-RepetitionPattern and subframes as provided in fdd-DownlinkOrTddSubframeBitmapBR in bandwidthReducedAccessRelatedInfo, or until successful decoding of the accumulated SI message transmissions;

1> 	if the SI message was not possible to decode from the accumulated SI message transmissions by the end of the SI-window, continue reception and accumulation of SI message transmissions on DL-SCH in the next SI-window occasion for the concerned SI message;

## 5.3	Connection control

### 5.3.1	Introduction

#### 5.3.1.1	RRC connection control

RRC connection establishment involves the establishment of SRB1. E-UTRAN completes RRC connection establishment prior to completing the establishment of the S1 connection, i.e. prior to receiving the UE context information from the EPC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the E-UTRAN may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful security activation. However, the UE only accepts a handover message when security has been activated.

NOTE:	In case the serving frequency broadcasts multiple overlapping bands, E-UTRAN can only configure measurements after having obtained the UE capabilities, as the measurement configuration needs to be set according to the band selected by the UE.

Upon receiving the UE context from the EPC, E-UTRAN activates security (both ciphering and integrity protection) using the initial security activation procedure. The RRC messages to activate security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate security is not ciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs) are both integrity protected and ciphered.

After having initiated the initial security activation procedure, E-UTRAN initiates the establishment of SRB2 and DRBs, i.e. E-UTRAN may do this prior to receiving the confirmation of the initial security activation from the UE. In any case, E-UTRAN will apply both ciphering and integrity protection for the RRC connection reconfiguration messages used to establish SRB2 and DRBs. E-UTRAN should release the RRC connection if the initial security activation and/ or the radio bearer establishment fails (i.e. security activation and DRB establishment are triggered by a joint S1-procedure, which does not support partial success).

For SRB2 and DRBs, security is always activated from the start, i.e. the E-UTRAN does not establish these bearers prior to activating security.

For some radio configuration fields, a critical extension has been defined. A switch from the original version of the field to the critically extended version is allowed using any connection reconfiguration. The UE reverts to the original version of some critically extended fields upon handover and re-establishment as specified elsewhere in this specification. Otherwise, switching a field from the critically extended version to the original version is only possible using the handover or re-establishment procedure with the full configuration option. This also applies for fields that are critically extended within a release (i.e. original and extended version defined in same release).

After having initiated the initial security activation procedure, E-UTRAN may configure a UE that supports CA, with one or more SCells in addition to the PCell that was initially configured during connection establishment. The PCell is used to provide the security inputs and upper layer system information (i.e. the NAS mobility information e.g. TAI). SCells are used to provide additional downlink and optionally uplink radio resources. When not configured with DC all SCells the UE is configured with, if any, are part of the MCG. When configured with DC however, some of the SCells are part of a SCG. In this case, user data carried by a DRB may either be transferred via MCG (i.e. MCG-DRB), via SCG (SCG-DRB) or via both MCG and SCG in DL while E-UTRAN configures the CG used in UL (split DRB). An RRC connection reconfiguration message may be used to change the DRB type from MCG-DRB to SCG-DRB or to split DRB, as well as from SCG-DRB or split DRB to MCG-DRB.

SCG change is a synchronous SCG reconfiguration procedure (i.e. involving RA to the PSCell) including reset/ re-establishment of layer 2 and, if SCG DRBs are configured, refresh of security. The procedure is used in a number of different scenarios e.g. SCG establishment, PSCell change, Key refresh, change of DRB type. The UE performs the SCG change related actions upon receiving an RRCConnectionReconfiguration message including mobilityControlInfoSCG, see 5.3.10.10.

The release of the RRC connection normally is initiated by E-UTRAN. The procedure may be used to re-direct the UE to an E-UTRA frequency or an inter-RAT carrier frequency. Only in exceptional cases, as specified within this specification, MFA TS 36.300 [9], MFA TS 36.304 [4] or MFA TS 24.301 [35], may the UE abort the RRC connection, i.e. move to RRC_IDLE without notifying E-UTRAN.

The suspension of the RRC connection is initiated by E-UTRAN. When the RRC connection is suspended, the UE stores the UE AS context and the resumeIdentity, and transitions to RRC_IDLE state. The RRC message to suspend the RRC connection is integrity protected and ciphered. Suspension can only be performed when at least 1 DRB is successfully established.

The resumption of a suspended RRC connection is initiated by upper layers when the UE has a stored UE AS context, RRC connection resume is permitted by E-UTRAN and the UE needs to transit from RRC_IDLE state to RRC_CONNECTED state. When the RRC connection is resumed, RRC configures the UE according to the RRC connection resume procedure based on the stored UE AS context and any RRC configuration received from E-UTRAN. The RRC connection resume procedure re-activates security and re-establishes SRB(s) and DRB(s). The request to resume the RRC connection includes the resumeIdentity. The request is not ciphered, but protected with a message authentication code.

In response to a request to resume the RRC connection, E-UTRAN may resume the suspended RRC connection, reject the request to resume and instruct the UE to either keep or discard the stored context, or setup a new RRC connection.

#### 5.3.1.2	Security

AS security comprises of the integrity protection of RRC signalling (SRBs) as well as the ciphering of RRC signalling (SRBs) and user data (DRBs).

RRC handles the configuration of the security parameters which are part of the AS configuration: the integrity protection algorithm, the ciphering algorithm and two parameters, namely the keyChangeIndicator and the nextHopChainingCount, which are used by the UE to determine the AS security keys upon handover, connection re-establishment and/ or connection resume.

The integrity protection algorithm is common for signalling radio bearers SRB1 and SRB2. The ciphering algorithm is common for all radio bearers (i.e. SRB1, SRB2 and DRBs). Neither integrity protection nor ciphering applies for SRB0.

RRC integrity and ciphering are always activated together, i.e. in one message/ procedure. RRC integrity and ciphering are never de-activated. However, it is possible to switch to a 'NULL' ciphering algorithm (eea0).

The 'NULL' integrity protection algorithm (eia0) is used only for the UE in limited service mode MFA TS 33.401 [32]. In case the 'NULL' integrity protection algorithm is used, 'NULL' ciphering algorithm is also used.

NOTE 1:	Lower layers discard RRC messages for which the integrity check has failed and indicate the integrity verification check failure to RRC.

The AS applies three different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc) and one for the ciphering of user data (KUPenc). All three AS keys are derived from the KeNB key. The KeNB is based on the KASME key, which is handled by upper layers.

Upon connection establishment new AS keys are derived. No AS-parameters are exchanged to serve as inputs for the derivation of the new AS keys at connection establishment.

The integrity and ciphering of the RRC message used to perform handover is based on the security configuration used prior to the handover and is performed by the source eNB.

The integrity and ciphering algorithms can only be changed upon handover. The four AS keys (KeNB, KRRCint, KRRCenc and KUPenc) change upon every handover, connection re-establishment and connection resume. The keyChangeIndicator is used upon handover and indicates whether the UE should use the keys associated with the KASME key taken into use with the latest successful NAS SMC procedure. The nextHopChainingCount parameter is used upon handover, connection re-establishment and connection resume by the UE when deriving the new KeNB that is used to generate KRRCint, KRRCenc and KUPenc (see MFA TS 33.401 [32]). An intra cell handover procedure may be used to change the keys in RRC_CONNECTED.

For each radio bearer an independent counter (COUNT, as specified in 3GPP TS 36.323 [8]) is maintained for each direction. For each DRB, the COUNT is used as input for ciphering. For each SRB, the COUNT is used as input for both ciphering and integrity protection. It is not allowed to use the same COUNT value more than once for a given security key. At connection resume the COUNT is reset. In order to limit the signalling overhead, individual messages/ packets include a short sequence number (PDCP SN, as specified in 3GPP TS 36.323 [8]). In addition, an overflow counter mechanism is used: the hyper frame number (TX_HFN and RX_HFN, as specified in 3GPP TS 36.323 [8]). The HFN needs to be synchronized between the UE and the eNB. The eNB is responsible for avoiding reuse of the COUNT with the same RB identity and with the same KeNB, e.g. due to the transfer of large volumes of data, release and establishment of new RBs. In order to avoid such re-use, the eNB may e.g. use different RB identities for successive RB establishments, trigger an intra cell handover or an RRC_CONNECTED to RRC_IDLE to RRC_CONNECTED transition.

For each SRB, the value provided by RRC to lower layers to derive the 5-bit BEARER parameter used as input for ciphering and for integrity protection is the value of the corresponding srb-Identity with the MSBs padded with zeroes.

In case of DC, a separate KeNB is used for SCG-DRBs (S-KeNB). This key is derived from the key used for the MCG (KeNB) and an SCG counter that is used to ensure freshness. To refresh the S-KeNB e.g. when the COUNT will wrap around, E-UTRAN employs an SCG change, i.e. an RRCConnectionReconfiguration message including mobilityControlInfoSCG. When performing handover, while at least one SCG-DRB remains configured, both KeNB and S-KeNB are refreshed. In such case E-UTRAN performs handover with SCG change i.e. an RRCConnectionReconfiguration message including both mobilityControlInfo and mobilityControlInfoSCG. The ciphering algorithm is common for all radio bearers within a CG but may be different between MCG and SCG. The ciphering algorithm for SCG DRBs can only be changed upon SCG change.

#### 5.3.1.2a	RN security

For RNs, AS security follows the procedures in 5.3.1.2. Furthermore, E-UTRAN may configure per DRB whether or not integrity protection is used. The use of integrity protection may be configured only upon DRB establishment and reconfigured only upon handover or upon the first reconfiguration following RRC connection re-establishment.

To provide integrity protection on DRBs between the RN and the E-UTRAN, the KUPint key is derived from the KeNB key as described in MFA TS 33.401 [32]. The same integrity protection algorithm used for SRBs also applies to the DRBs. The KUPint changes at every handover and RRC connection re-establishment and is based on an updated KeNB which is derived by taking into account the nextHopChainingCount. The COUNT value maintained for DRB ciphering is also used for integrity protection, if the integrity protection is configured for the DRB.

#### 5.3.1.3	Connected mode mobility

In RRC_CONNECTED, the network controls UE mobility, i.e. the network decides when the UE shall connect to which E-UTRA cell(s), or inter-RAT cell. For network controlled mobility in RRC_CONNECTED, the PCell can be changed using an RRCConnectionReconfiguration message including the mobilityControlInfo (handover), whereas the SCell(s) can be changed using the RRCConnectionReconfiguration message either with or without the mobilityControlInfo.

An SCG can be established, reconfigured or released by using an RRCConnectionReconfiguration message with or without the mobilityControlInfo. In case Random Access to the PSCell is required upon SCG reconfiguration, E-UTRAN employs the SCG change procedure (i.e. an RRCConnectionReconfiguration message including the mobilityControlInfoSCG). The PSCell can only be changed using the SCG change procedure and by release and addition of the PSCell.

The network triggers the handover procedure e.g. based on radio conditions, load. To facilitate this, the network may configure the UE to perform measurement reporting (possibly including the configuration of measurement gaps). The network may also initiate handover blindly, i.e. without having received measurement reports from the UE.

Before sending the handover message to the UE, the source eNB prepares one or more target cells. The source eNB selects the target PCell. The source eNB may also provide the target eNB with a list of best cells on each frequency for which measurement information is available, in order of decreasing RSRP. The source eNB may also include available measurement information for the cells provided in the list. The target eNB decides which SCells are configured for use after handover, which may include cells other than the ones indicated by the source eNB. If an SCG is configured, handover involves either SCG release or SCG change. In case the UE was configured with DC, the target eNB indicates in the handover message whether the UE shall release the entire SCG configuration. Upon connection re-establishment, the UE releases the entire SCG configuration except for the DRB configuration, while E-UTRAN in the first reconfiguration message following the re-establishment either releases the DRB(s) or reconfigures the DRB(s) to MCG DRB(s).

The target eNB generates the message used to perform the handover, i.e. the message including the AS-configuration to be used in the target cell(s). The source eNB transparently (i.e. does not alter values/ content) forwards the handover message/ information received from the target to the UE. When appropriate, the source eNB may initiate data forwarding for (a subset of) the DRBs.

After receiving the handover message, the UE attempts to access the target PCell at the first available RACH occasion according to Random Access resource selection defined in MFA TS 36.321 [6], i.e. the handover is asynchronous. Consequently, when allocating a dedicated preamble for the random access in the target PCell, E-UTRA shall ensure it is available from the first RACH occasion the UE may use. Upon successful completion of the handover, the UE sends a message used to confirm the handover.

If the target eNB does not support the release of RRC protocol which the source eNB used to configure the UE, the target eNB may be unable to comprehend the UE configuration provided by the source eNB. In this case, the target eNB should use the full configuration option to reconfigure the UE for Handover and Re-establishment. Full configuration option includes an initialization of the radio configuration, which makes the procedure independent of the configuration used in the source cell(s) with the exception that the security algorithms are continued for the RRC re-establishment.

After the successful completion of handover, PDCP SDUs may be re-transmitted in the target cell(s). This only applies for DRBs using RLC-AM mode and for handovers not involving full configuration option. The further details are specified in 3GPP TS 36.323 [8]. After the successful completion of handover not involving full configuration option, the SN and the HFN are reset except for the DRBs using RLC-AM mode (for which both SN and HFN continue). For reconfigurations involving the full configuration option, the PDCP entities are newly established (SN and HFN do not continue) for all DRBs irrespective of the RLC mode. The further details are specified in 3GPP TS 36.323 [8].

One UE behaviour to be performed upon handover is specified, i.e. this is regardless of the handover procedures used within the network (e.g. whether the handover includes X2 or S1 signalling procedures).

The source eNB should, for some time, maintain a context to enable the UE to return in case of handover failure. After having detected handover failure, the UE attempts to resume the RRC connection either in the source PCell or in another cell using the RRC re-establishment procedure. This connection resumption succeeds only if the accessed cell is prepared, i.e. concerns a cell of the source eNB or of another eNB towards which handover preparation has been performed. The cell in which the re-establishment procedure succeeds becomes the PCell while SCells and STAGs, if configured, are released.

Normal measurement and mobility procedures are used to support handover to cells broadcasting a CSG identity. In addition, E-UTRAN may configure the UE to report that it is entering or leaving the proximity of cell(s) included in its CSG whitelist. Furthermore, E-UTRAN may request the UE to provide additional information broadcast by the handover candidate cell e.g. global cell identity, CSG identity, CSG membership status.

NOTE:	E-UTRAN may use the ‘proximity report’ to configure measurements as well as to decide whether or not to request additional information broadcast by the handover candidate cell. The additional information is used to verify whether or not the UE is authorised to access the target PCell and may also be needed to identify handover candidate cell (PCI confusion i.e. when the physical layer identity that is included in the measurement report does not uniquely identify the cell).

#### 5.3.1.4	Connection control in NB-IoT

In NB-IoT, during the RRC connection establishment procedure, SRB1bis is established implicitly with SRB1. SRB1bis uses the logical channel identity defined in 9.1.2a, with the same configuration as SRB1 but no PDCP entity. SRB1bis is used until security is activated. The RRC messages to activate security (command and successful response) are sent over SRB1 being integrity protected and ciphering is started after completion of the procedure. Once security is activated, new RRC messages shall be transmitted using SRB1. A NB-IoT UE that only supports the Control Plane CIoT EPS optimisation (see MFA TS 24.301 [35]) only establishes SRB1bis.

A NB-IoT UE only supports 0, 1 or 2 DRBs, depending on its capability. A NB-IoT UE that only supports the Control Plane CIoT EPS optimisation (see MFA TS 24.301 [35]) does not need to support any DRBs and associated procedures.

Table 5.3.1.4-1 lists the procedures that are applicable for NB-IoT. All other procedures are not applicable; this is not further stated in the corresponding procedures.

Table 5.3.1.4-1: Connection control procedures applicable to a NB-IoT UE

| Sub-clause | Procedures |
| --- | --- |
| 5.3.2 | Paging |
| 5.3.3 | RRC connection establishment |
|  | RRC connection resume (see NOTE) |
| 5.3.4 | Initial security activation (see NOTE) |
| 5.3.5 | RRC connection reconfiguration (see NOTE) |
| 5.3.7 | RRC connection re-establishment (see NOTE) |
| 5.3.8 | RRC connection release |
| 5.3.9 | RRC connection release requested by upper layers |
| 5.3.10 | Radio resource configuration |
| 5.3.11 | Radio link failure related actions |
| 5.3.12 | UE actions upon leaving RRC_CONNECTED |

NOTE:	Not applicable for a UE that only supports the Control Plane CIoT EPS optimisation (see MFA TS 24.301 [35]).

### 5.3.2	Paging

#### 5.3.2.1	General

Figure 5.3.2.1-1: Paging

The purpose of this procedure is:

-	to transmit paging information to a UE in RRC_IDLE and/ or;

-	to inform UEs in RRC_IDLE and UEs other than NB-IoT UEs in RRC_CONNECTED about a system information change and/ or;

-	to inform UEs other than NB-IoT UEs about an ETWS primary notification and/ or ETWS secondary notification and/ or;

-	to inform UEs other than NB-IoT UEs about a CMAS notification and/ or;

-	to inform UEs other than NB-IoT UEs in RRC_IDLE about an EAB parameters modification and/ or;

-	to inform UEs other than NB-IoT UEs in RRC_IDLE to perform E-UTRAN inter-frequency redistribution procedure.

The paging information is provided to upper layers, which in response may initiate RRC connection establishment, e.g. to receive an incoming call.

#### 5.3.2.2	Initiation

E-UTRAN initiates the paging procedure by transmitting the Paging message at the UE's paging occasion as specified in MFA TS 36.304 [4]. E-UTRAN may address multiple UEs within a Paging message by including one PagingRecord for each UE. E-UTRAN may also indicate a change of system information, and/ or provide an ETWS notification or a CMAS notification in the Paging message.

#### 5.3.2.3	Reception of the Paging message by the UE

Upon receiving the Paging message, the UE shall:

1> if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message:

2> 	if the ue-Identity included in the PagingRecord matches one of the UE identities allocated by upper layers:

3> 	forward the ue-Identity and, except for NB-IoT, the cn-Domain to the upper layers;

1> if the systemInfoModification is included; or

1> if the UE is configured with a DRX cycle longer than the modification period and the systemInfoModification-eDRX is included:

2> 	re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2.

1> if the etws-Indication is included and the UE is ETWS capable:

2> 	re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary;

2> 	if the schedulingInfoList indicates that SystemInformationBlockType10 is present:

3> 	acquire SystemInformationBlockType10;

NOTE:	If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10.

2> 	if the schedulingInfoList indicates that SystemInformationBlockType11 is present:

3> 	acquire SystemInformationBlockType11;

1> 	if the cmas-Indication is included and the UE is CMAS capable:

2> 	re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5;

2> 	if the schedulingInfoList indicates that SystemInformationBlockType12 is present:

3> 	acquire SystemInformationBlockType12;

1> 	if in RRC_IDLE, the eab-ParamModification is included and the UE is EAB capable:

2> 	consider previously stored SystemInformationBlockType14 as invalid;

2> 	re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.6;

2> 	re-acquire SystemInformationBlockType14 using the system information acquisition procedure as specified in 5.2.2.4;

1> if in RRC_IDLE, the redistributionIndication is included and the UE is redistribution capable:

2> 	Perform E-UTRAN inter-frequency redistribution procedure as specified in MFA TS 36.304 [4, Section 5.2.4.10];

### 5.3.3	RRC connection establishment

#### 5.3.3.1	General

Figure 5.3.3.1-1: RRC connection establishment, successful

Figure 5.3.3.1-2: RRC connection establishment, network reject

Figure 5.3.3.1-3: RRC connection resume, successful

Figure 5.3.3.1-4: RRC connection resume fallback to RRC connection establishment, successful

Figure 5.3.3.1-5: RRC connection resume, network reject or release

The purpose of this procedure is to establish or resume an RRC connection. RRC connection establishment involves SRB1 (and SRB1bis for NB-IoT) establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to E-UTRAN.

E-UTRAN applies the procedure as follows:

-	When establishing an RRC connection:

-	to establish SRB1 and, for NB-IoT, SRB1bis;

-	When resuming an RRC connection:

-	to restore the AS configuration from a stored context including resuming SRB(s) and DRB(s).

#### 5.3.3.1a	Conditions for establishing RRC Connection for sidelink communication/ discovery

For sidelink communication an RRC connection is initiated only in the following case:

1> 	if configured by upper layers to transmit non-relay related sidelink communication and related data is available for transmission:

2> 	if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 does not include commTxPoolNormalCommon;

1> 	if configured by upper layers to transmit relay related sidelink communication:

2> 	if the UE is acting as sidelink relay UE; and if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; or

2> 	if the UE has a selected sidelink relay UE; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met and if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 does not include commTxPoolNormalCommon or commTxAllowRelayCommon;

For sidelink discovery an RRC connection is initiated only in the following case:

1> 		if configured by upper layers to transmit non-PS related sidelink discovery announcements:

2> 	if the frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements concerns the camped frequency; and SystemInformationBlockType19 of the cell on which the UE camps does not include discTxPoolCommon-r12; or

2> 	if the frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcesInterFreq included within discResourcesNonPS and set to requestDedicated;

1> 		if configured by upper layers to transmit non-relay PS related sidelink discovery announcements:

2> 	if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements concerns the camped frequency; and SystemInformationBlockType19 of the cell on which the UE camps includes discConfigPS but does not include discTxPoolPS-Common; or

2> 	if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcesInterFreq within discResourcesPS included and set to requestDedicated;

1> 	if configured by upper layers to transmit relay PS related sidelink discovery announcements:

2> 	if the UE is acting as sidelink relay UE; and if the sidelink relay UE threshold conditions as specified in 5.10.10.4 are met; or

2> 	if the UE is selecting a sidelink relay UE / has a selected sidelink relay UE; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met:

3> 	if the frequency on which the UE is configured to transmit relay PS related sidelink discovery announcements concerns the camped frequency; and SystemInformationBlockType19 of the cell on which the UE camps includes discConfigRelay and discConfigPS but does not include discTxPoolPS-Common;

NOTE:	Upper layers initiate an RRC connection. The interaction with NAS is left to UE implementation.

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment or resume of an RRC connection while the UE is in RRC_IDLE.

Except for NB-IoT, upon initiation of the procedure, the UE shall:

1> if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see 3GPP TS 23.122 [11], and MFA TS 24.301 [35]):

2> 	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2> 	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1> 	else

2> 	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

1> 	if SystemInformationBlockType2 contains ACDC-BarringPerPLMN-List and the ACDC-BarringPerPLMN-List contains an ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see 3GPP TS 23.122 [11], and MFA TS 24.301 [35]):

2> 	select the ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2> 	in the remainder of this procedure, use the selected ACDC-BarringPerPLMN entry for ACDC barring check (i.e. presence or absence of access barring parameters in this entry) irrespective of the ACDC-BarringForCommon parameters included in SystemInformationBlockType2;

1> 	else

2> 	in the remainder of this procedure use the ACDC-BarringForCommon (i.e. presence or absence of these parameters) included in SystemInformationBlockType2 for ACDC barring check;

1> if upper layers indicate that the RRC connection is subject to EAB (see MFA TS 24.301 [35]):

2> 	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that EAB is applicable, upon which the procedure ends;

1> 	if upper layers indicate that the RRC connection is subject to ACDC (see MFA TS 24.301 [35]), SystemInformationBlockType2 contains BarringPerACDC-CategoryList, and acdc-HPLMNonly indicates that ACDC is applicable for the UE:

2> 	if the BarringPerACDC-CategoryList contains a BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers:

3> 	select the BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers;

2> 	else:

3> 	select the last BarringPerACDC-Category entry in the BarringPerACDC-CategoryList;

2> 	stop timer T308, if running;

2> 	perform access barring check as specified in 5.3.3.13, using T308 as "Tbarring" and acdc-BarringConfig in the BarringPerACDC-Category as "ACDC barring parameter";

2> 	if access to the cell is barred:

3> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring is applicable due to ACDC, upon which the procedure ends;

1> else if the UE is establishing the RRC connection for mobile terminating calls:

2> 	if timer T302 is running:

3> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile terminating calls is applicable, upon which the procedure ends;

1> else if the UE is establishing the RRC connection for emergency calls:

2> 	if SystemInformationBlockType2 includes the ac-BarringInfo:

3> 	if the ac-BarringForEmergency is set to TRUE:

4> 	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to 3GPP TS 22.011 [10] and 3GPP TS 23.122 [11]:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

5> 	if the ac-BarringInfo includes ac-BarringForMO-Data, and for all of these valid Access Classes for the UE, the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to one:

6> 	consider access to the cell as barred;

4> 	else:

5> 	consider access to the cell as barred;

2> 	if access to the cell is barred:

3> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication, upon which the procedure ends;

1> else if the UE is establishing the RRC connection for mobile originating calls:

2> 	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2> 	if access to the cell is barred:

3> 	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

4> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls is applicable, upon which the procedure ends;

3> 	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

4> 	if timer T306 is not running, start T306 with the timer value of T303;

4> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1> else if the UE is establishing the RRC connection for mobile originating signalling:

2> 	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2> 	if access to the cell is barred:

3> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

1> else if the UE is establishing the RRC connection for mobile originating CS fallback:

2> 	if SystemInformationBlockType2 includes ac-BarringForCSFB:

3> 	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForCSFB as "AC barring parameter";

3> 	if access to the cell is barred:

4> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating CS fallback is applicable, due to ac-BarringForCSFB, upon which the procedure ends;

2> 	else:

3> 	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

3> 	if access to the cell is barred:

4> 	if timer T303 is not running, start T303 with the timer value of T306;

4> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating CS fallback and mobile originating calls is applicable, due to ac-BarringForMO-Data, upon which the procedure ends;

1> else if the UE is establishing the RRC connection for mobile originating MMTEL voice, mobile originating MMTEL video, mobile originating SMSoIP or mobile originating SMS:

2> 	if the UE is establishing the RRC connection for mobile originating MMTEL voice and SystemInformationBlockType2 includes ac-BarringSkipForMMTELVoice; or

2> 	if the UE is establishing the RRC connection for mobile originating MMTEL video and SystemInformationBlockType2 includes ac-BarringSkipForMMTELVideo; or

2> 	if the UE is establishing the RRC connection for mobile originating SMSoIP or SMS and SystemInformationBlockType2 includes ac-BarringSkipForSMS:

3> 	consider access to the cell as not barred;

2> 	else:

3> 	if establishmentCause received from higher layers is set to mo-Signalling (including the case that mo-Signalling is replaced by highPriorityAccess according to MFA TS 24.301 [35] or by mo-VoiceCall according to the subclause 5.3.3.3):

4> 	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

4> 	if access to the cell is barred:

5> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

3> 	if establishmentCause received from higher layers is set to mo-Data (including the case that mo-Data is replaced by highPriorityAccess according to MFA TS 24.301 [35] or by mo-VoiceCall according to the subclause 5.3.3.3):

4> 	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

4> 	if access to the cell is barred:

5> 	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

6> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls is applicable, upon which the procedure ends;

5> 	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

6> 	if timer T306 is not running, start T306 with the timer value of T303;

6> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1> if the UE is resuming an RRC connection:

2> 	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

2> 	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

2> 	release reportProximityConfig and clear any associated proximity status reporting timer;

2> 	release obtainLocationConfig, if configured;

2> 	release idc-Config, if configured;

2> 	release measSubframePatternPCell, if configured;

2> 	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

2> 	release naics-Info for the PCell, if configured;

2> 	release the LWA configuration, if configured, as described in 5.6.14.3;

2> 	release the LWIP configuration, if configured, as described in 5.6.17.3;

1> 	apply the default physical channel configuration as specified in 9.2.4;

1> 	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1> 	apply the default MAC main configuration as specified in 9.2.2;

1> 	apply the CCCH configuration as specified in 9.1.1.2;

1> 	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1> 	start timer T300;

1> 	if the UE is resuming an RRC connection:

2> 	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

1> 	else

2> 	if stored, discard the UE AS context and resumeIdentity;

2> 	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

For NB-IoT, upon initiation of the procedure, the UE shall:

1> 	if the UE is establishing or resuming the RRC connection for mobile originating exception data; or

1> 	if the UE is establishing or resuming the RRC connection for mobile originating data; or

1> 	if the UE is establishing or resuming the RRC connection for delay tolerant access; or

1> 	if the UE is establishing or resuming the RRC connection for mobile originating signalling;

2> 	perform access barring check as specified in 5.3.3.14;

2> 	if access to the cell is barred:

3> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring is applicable, upon which the procedure ends;

1> 	apply the default physical channel configuration as specified in 9.2.4;

1> 	apply the default MAC main configuration as specified in 9.2.2;

1> 	apply the CCCH configuration as specified in 9.1.1.2;

1> 	start timer T300;

1> 		if the UE is establishing an RRC connection:

2> 	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

1> else if the UE is resuming an RRC connection:

2> 	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

NOTE 3:	Upon initiating the connection establishment or resumption procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

5.3.3.3	Actions related to transmission of RRCConnectionRequest message

The UE shall set the contents of RRCConnectionRequest message as follows:

1> 	set the ue-Identity as follows:

2> 	if upper layers provide an S-TMSI:

3> 	set the ue-Identity to the value received from upper layers;

2> 	else:

3> 	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

1> if the UE supports mo-VoiceCall establishment cause and UE is establishing the RRC connection for mobile originating MMTEL voice and SystemInformationBlockType2 includes voiceServiceCauseIndication:

2> 	set the establishmentCause to mo-VoiceCall;

1> 	else:

2> 	set the establishmentCause in accordance with the information received from upper layers;

1> if the UE is a NB-IoT UE:

2> 	if the UE supports multi-tone transmission, include multiToneSupport;

2> 	if the UE supports multi-carrier operation, include multiCarrierSupport;

The UE shall submit the RRCConnectionRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.5.

5.3.3.3a	Actions related to transmission of RRCConnectionResumeRequest message

The UE shall set the contents of RRCConnectionResumeRequest message as follows:

1> 	if the UE is a NB-IoT UE; or

1> 	if field useFullResumeID is signalled in SystemInformationBlockType2:

2> 	set the resumeID to the stored resumeIdentity;

1> 	else

2> 	set the truncatedResumeID to include bits in bit position 9 to 20 and 29 to 40 from the left in the stored resumeIdentity.

- if the UE supports mo-VoiceCall establishment cause and UE is resuming the RRC connection for mobile originating MMTEL voice and SystemInformationBlockType2 includes voiceServiceCauseIndication:

2> 	set the resumeCause to mo-VoiceCall;

1> 	else

2> 	set the resumeCause in accordance with the information received from upper layers;

1> set the shortResumeMAC-I to the 16 least significant bits of the MAC-I calculated:

2> 	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortResumeMAC-Input (or VarShortResumeMAC-Input-NB in NB-IoT);

2> 	with the KRRCint key and the previously configured integrity protection algorithm; and

2> 	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	restore the RRC configuration and security context from the stored UE AS context:

1>	restore the PDCP state and re-establish PDCP entities for SRB1;

1>	resume SRB1;

NOTE:	Until successful connection resumption, SRB1 is used only for transferring the RRCConnectionResume message.

The UE shall submit the RRCConnectionResumeRequest message to lower layers for transmission.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.5.

#### 5.3.3.4	Reception of the RRCConnectionSetup by the UE

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see MFA TS 36.321 [6];

The UE shall:

1> if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

2> 	discard the stored UE AS context and resumeIdentity;

2> 	indicate to upper layers that the RRC connection resume has been fallbacked;

1> 	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1> 	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1> 	stop timer T300;

1> 	stop timer T302, if running;

1> 	stop timer T303, if running;

1> 	stop timer T305, if running;

1> 	stop timer T306, if running;

1> 	stop timer T308, if running;

1> 	perform the actions as specified in 5.3.3.7;

1> 	stop timer T320, if running;

1> 	stop timer T350, if running;

1> 	perform the actions as specified in 5.6.12.4;

1> 	release rclwi-Configuration, if configured, as specified in 5.6.16.2;

1> 	stop timer T360, if running;

1> 	enter RRC_CONNECTED;

1> 	stop the cell re-selection procedure;

1> 	consider the current cell to be the PCell;

1> 	set the content of RRCConnectionSetupComplete message as follows:

2> 	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

3> 	if upper layers provide an S-TMSI:

4> 	set the s-TMSI to the value received from upper layers;

2> 	set the selectedPLMN-Identity to the PLMN selected by upper layers (see 3GPP TS 23.122 [11], and MFA TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1 (or SystemInformationBlockType1-NB in NB-IoT);

2> 	if upper layers provide the 'Registered MME', include and set the registeredMME as follows:

3> 	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers:

4> 	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3> 	set the mmegi and the mmec to the value received from upper layers;

2> 	except for NB-IoT, if upper layers provided the 'Registered MME':

3> 	include and set the gummei-Type to the value provided by the upper layers;

2> 	if the UE supports CIoT EPS optimisation(s):

3> 	include attachWithoutPDN-Connectivity if received from upper layers;

3> 	include up-CIoT-EPS-Optimisation if received from upper layers;

3> 	except for NB-IoT, include cp-CIoT-EPS-Optimisation if received from upper layers;

2> 	if connecting as an RN:

3> 	include the rn-SubframeConfigReq;

2> 	set the dedicatedInfoNAS to include the information received from upper layers;

2> 	except for NB-IoT:

3> 	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

4> 	include rlf-InfoAvailable;

3> 	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4> 	include logMeasAvailableMBSFN;

3> 	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4> 	include logMeasAvailable;

3> 	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

4> 	include connEstFailInfoAvailable;

3> 	include the mobilityState and set it to the mobility state (as specified in MFA TS 36.304 [4]) of the UE just prior to entering RRC_CONNECTED state;

3> 	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

4> 	include the mobilityHistoryAvail;

2> 	if UE needs UL gaps during continuous uplink transmission:

3> 	include ue-CE-NeedULGaps;

2> 	submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends;

#### 5.3.3.4a	Reception of the RRCConnectionResume by the UE

The UE shall:

1> 	stop timer T300;

1> 	restore the PDCP state and re-establish PDCP entities for SRB2 and all DRBs;

1> 	if drb-ContinueROHC is included:

2> 	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

2> 	continue the header compression protocol context for the DRBs configured with the header compression protocol;

1> 	else:

2> 	indicate to lower layers that stored UE AS context is used;

2> 	reset the header compression protocol context for the DRBs configured with the header compression protocol;

1> 	 discard the stored UE AS context and resumeIdentity;

1> 	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1> 	resume SRB2 and all DRBs;

1> 	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1> 	if the RRCConnectionResume message includes the measConfig:

2> 	perform the measurement configuration procedure as specified in 5.5.2;

1> 	stop timer T302, if running;

1> 	stop timer T303, if running;

1> 	stop timer T305, if running;

1> 	stop timer T306, if running;

1> 	stop timer T308, if running;

1> 	perform the actions as specified in 5.3.3.7;

1> 	stop timer T320, if running;

1> 	stop timer T350, if running;

1> 	perform the actions as specified in 5.6.12.4;

1> 	stop timer T360, if running;

1> update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionResume message, as specified in MFA TS 33.401 [32];

1> store the nextHopChainingCount value;

1> derive the KRRCint key associated with the previously configured integrity algorithm, as specified in MFA TS 33.401 [32];

1> 	request lower layers to verify the integrity protection of the RRCConnectionResume message, using the previously configured algorithm and the KRRCint key;

1> 	if the integrity protection check of the RRCConnectionResume message fails:

2> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

1> 	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in MFA TS 33.401 [32];

1> 	configure lower layers to resume integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

1> 	configure lower layers to resume ciphering and to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1> 	enter RRC_CONNECTED;

1> 	indicate to upper layers that the suspended RRC connection has been resumed;

1> 	stop the cell re-selection procedure;

1> 	consider the current cell to be the PCell;

1> 	set the content of RRCConnectionResumeComplete message as follows:

2> 	set the selectedPLMN-Identity to the PLMN selected by upper layers (see 3GPP TS 23.122 [11], and MFA TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2> 	set the dedicatedInfoNAS to include the information received from upper layers;

2> 	except for NB-IoT:

3> 	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

4> 	include rlf-InfoAvailable;

3> 	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4> 	include logMeasAvailableMBSFN;

3> 	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4> 	include logMeasAvailable;

3> 	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

4> 	include connEstFailInfoAvailable;

3> 	include the mobilityState and set it to the mobility state (as specified in MFA TS 36.304 [4]) of the UE just prior to entering RRC_CONNECTED state;

3> 	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

4> 	include mobilityHistoryAvail;

1> 	submit the RRCConnectionResumeComplete message to lower layers for transmission;

1> 	the procedure ends.

#### 5.3.3.5	Cell re-selection while T300, T302, T303, T305, T306, or T308 is running

The UE shall:

1> if cell reselection occurs while T300, T302, T303, T305, T306, or T308 is running:

2> 	if timer T302, T303, T305,T306, and/or T308 is running:

3> 	stop timer T302, T303, T305, T306, and T308, whichever ones were running;

3> 	perform the actions as specified in 5.3.3.7;

2> 	if timer T300 is running:

3> 	stop timer T300;

3> 	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

3> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication;

#### 5.3.3.6	T300 expiry

The UE shall:

1> if timer T300 expires:

2> 	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2> 	if the UE is a NB-IoT UE and connEstFailOffset is included in SystemInformationBlockType2-NB:

3> 	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to MFA TS 36.304 [4];

NOTE 0:	For NB-IoT, the number of times that the UE detects T300 expiry on the same cell before applying connEstFailOffset and the amount of time that the UE applies connEstFailOffset before removing the offset from evaluation of the cell is up to UE implementation.

2> 	else if the UE supports RRC Connection Establishment failure temporary Qoffset and T300 has expired a consecutive connEstFailCount times on the same cell for which txFailParams is included in SystemInformationBlockType2:

3> 	for a period as indicated by connEstFailOffsetValidity:

4> 	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to MFA TS 36.304 [4] and 3GPP TS 25.304 [40];

NOTE 1:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2> 	except for NB-IoT, store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3> 	clear the information included in VarConnEstFailReport, if any;

3> 	set the plmn-Identity to the PLMN selected by upper layers (see 3GPP TS 23.122 [11], and MFA TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

3> 	set the failedCellId to the global cell identity of the cell where connection establishment failure is detected;

3> 	set the measResultFailedCell to include the RSRP and RSRQ, if available, of the cell where connection establishment failure is detected and based on measurements collected up to the moment the UE detected the failure;

3> 	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

4> 	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in MFA TS 36.133 [16].

3> 	if detailed location information is available, set the content of the locationInfo as follows:

4> 	include the locationCoordinates;

4> 	include the horizontalVelocity, if available;

3> 	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the failed random access procedure;

3> 	set contentionDetected to indicate whether contention resolution was not successful as specified in MFA TS 36.321 [6] for at least one of the transmitted preambles for the failed random access procedure;

3> 	set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see MFA TS 36.321 [6];

2> 	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication, upon which the procedure ends;

The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the failure is detected, upon power off or upon detach.

#### 5.3.3.7	T302, T303, T305, T306, or T308 expiry or stop

The UE shall:

1> 	if timer T302 expires or is stopped:

2> 	inform upper layers about barring alleviation for mobile terminating access;

2> 	if timer T303 is not running:

3> 	inform upper layers about barring alleviation for mobile originating calls;

2> 	if timer T305 is not running:

3> 	inform upper layers about barring alleviation for mobile originating signalling;

2> 	if timer T306 is not running:

3> 	inform upper layers about barring alleviation for mobile originating CS fallback;

2> 	if timer T308 is not running:

3> 	inform upper layers about barring alleviation for ACDC;

1> 	if timer T303 expires or is stopped:

2> 	if timer T302 is not running:

3> 	inform upper layers about barring alleviation for mobile originating calls;

1> 	if timer T305 expires or is stopped:

2> 	if timer T302 is not running:

3> 	inform upper layers about barring alleviation for mobile originating signalling;

1> 	if timer T306 expires or is stopped:

2> 	if timer T302 is not running:

3> 	inform upper layers about barring alleviation for mobile originating CS fallback;

1> if timer T308 expires or is stopped:

2> 	if timer T302 is not running:

3> 	inform upper layers about barring alleviation for ACDC;

#### 5.3.3.8	Reception of the RRCConnectionReject by the UE

The UE shall:

1> 	stop timer T300;

1> 	reset MAC and release the MAC configuration;

1> 	except for NB-IoT, start timer T302, with the timer value set to the waitTime;

1> 	if the UE is a NB-IoT UE; or

1> 	if the extendedWaitTime is present and the UE supports delay tolerant access:

2> 	forward the extendedWaitTime to upper layers;

1> if deprioritisationReq is included and the UE supports RRC Connection Reject with deprioritisation:

2> 	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2> 	store the deprioritisationReq until T325 expiry;

NOTE:	The UE stores the deprioritisation request irrespective of any cell reselection absolute priority assignments (by dedicated or common signalling) and regardless of RRC connections in E-UTRAN or other RATs unless specified otherwise.

1> 	if the RRCConnectionReject is received in response to an RRCConnectionResumeRequest:

2> 	if the rrc-SuspendIndication is not present:

3> 	discard the stored UE AS context and resumeIdentity;

3> 	inform upper layers about the failure to resume the RRC connection without suspend indication and that access barring for mobile originating calls, mobile originating signalling, mobile terminating access and except for NB-IoT for mobile originating CS fallback is applicable, upon which the procedure ends;

2> 	else:

3>	suspend SRB1;

3> 	inform upper layers about the failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls, mobile originating signalling, mobile terminating access and except for NB-IoT for mobile originating CS fallback is applicable, upon which the procedure ends;

1> 	else

2> 	inform upper layers about the failure to establish the RRC connection and that access barring for mobile originating calls, mobile originating signalling, mobile terminating access and except for NB-IoT, for mobile originating CS fallback is applicable, upon which the procedure ends;

#### 5.3.3.9	Abortion of RRC connection establishment

If upper layers abort the RRC connection establishment procedure while the UE has not yet entered RRC_CONNECTED, the UE shall:

1> stop timer T300, if running;

1> reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

#### 5.3.3.10	Handling of SSAC related parameters

Upon request from the upper layers, the UE shall:

1> if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see 3GPP TS 23.122 [11], and MFA TS 24.301 [35]):

2> 	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2> 	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1> 	else:

2> 	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

1> set the local variables BarringFactorForMMTEL-Voice and BarringTimeForMMTEL-Voice as follows:

2> 	if ssac-BarringForMMTEL-Voice is present:

3> 	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to 3GPP TS 22.011 [10] and 3GPP TS 23.122 [11], and

NOTE:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

3> 	if, for at least one of these Access Classes, the corresponding bit in the ac-BarringForSpecialAC contained in ssac-BarringForMMTEL-Voice is set to zero:

4> 	set BarringFactorForMMTEL-Voice to one and BarringTimeForMMTEL-Voice to zero;

3> 	else:

4> 	set BarringFactorForMMTEL-Voice and BarringTimeForMMTEL-Voice to the value of ac-BarringFactor and ac-BarringTime included in ssac-BarringForMMTEL-Voice, respectively;

2> 	else set BarringFactorForMMTEL-Voice to one and BarringTimeForMMTEL-Voice to zero;

1> set the local variables BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video as follows:

2> 	if ssac-BarringForMMTEL-Video is present:

3> 	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to 3GPP TS 22.011 [10] and 3GPP TS 23.122 [11], and

3> 	if, for at least one of these Access Classes, the corresponding bit in the ac-BarringForSpecialAC contained in ssac-BarringForMMTEL-Video is set to zero:

4> 	set BarringFactorForMMTEL-Video to one and BarringTimeForMMTEL-Video to zero;

3> 	else:

4> 	set BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video to the value of ac-BarringFactor and ac-BarringTime included in ssac-BarringForMMTEL-Video, respectively;

2> 	else set BarringFactorForMMTEL-Video to one and BarringTimeForMMTEL-Video to zero;

1> forward the variables BarringFactorForMMTEL-Voice, BarringTimeForMMTEL-Voice, BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video to the upper layers;

#### 5.3.3.11	Access barring check

1> if timer T302 or "Tbarring" is running:

2> 	consider access to the cell as barred;

1> else if SystemInformationBlockType2 includes "AC barring parameter":

2> 	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to 3GPP TS 22.011 [10] and 3GPP TS 23.122 [11], and

NOTE:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

2> 	for at least one of these valid Access Classes the corresponding bit in the ac-BarringForSpecialAC contained in "AC barring parameter" is set to zero:

3> 	consider access to the cell as not barred;

2> 	else:

3> 	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3> 	if 'rand' is lower than the value indicated by ac-BarringFactor included in "AC barring parameter":

4> 	consider access to the cell as not barred;

3> 	else:

4> 	consider access to the cell as barred;

1> 	else:

2> 	consider access to the cell as not barred;

1> 	if access to the cell is barred and both timers T302 and "Tbarring" are not running:

2> 	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

2> 	start timer "Tbarring" with the timer value calculated as follows, using the ac-BarringTime included in "AC barring parameter":

"Tbarring" = (0.7+ 0.6 * rand) * ac-BarringTime;

#### 5.3.3.12	EAB check

The UE shall:

1> 	if SystemInformationBlockType14 is present and includes the eab-Param:

2> 	if the eab-Common is included in the eab-Param:

3> 	if the UE belongs to the category of UEs as indicated in the eab-Category contained in eab-Common; and

3> 	if for the Access Class of the UE, as stored on the USIM and with a value in the range 0..9, the corresponding bit in the eab-BarringBitmap contained in eab-Common is set to one:

4> 	consider access to the cell as barred;

3> 	else:

4> 	consider access to the cell as not barred due to EAB;

2> 	else (the eab-PerPLMN-List is included in the eab-Param):

3> 	select the entry in the eab-PerPLMN-List corresponding to the PLMN selected by upper layers (see 3GPP TS 23.122 [11], and MFA TS 24.301 [35]);

3> 	if the eab-Config for that PLMN is included:

4> 	if the UE belongs to the category of UEs as indicated in the eab-Category contained in eab-Config; and

4> 	if for the Access Class of the UE, as stored on the USIM and with a value in the range 0..9, the corresponding bit in the eab-BarringBitmap contained in eab-Config is set to one:

5> 	consider access to the cell as barred;

4> 	else:

5> 	consider access to the cell as not barred due to EAB;

3> 	else:

4> 	consider access to the cell as not barred due to EAB;

1> 	else

2> 	consider access to the cell as not barred due to EAB;

#### 5.3.3.13	Access barring check for ACDC

The UE shall:

1> 	if timer T302 is running:

2> 	consider access to the cell as barred;

1> else if SystemInformationBlockType2 includes "ACDC barring parameter":

2> 	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

2> 	if 'rand' is lower than the value indicated by ac-BarringFactor included in "ACDC barring parameter":

3> 	consider access to the cell as not barred;

2> 	else:

3> 	consider access to the cell as barred;

1> 	else:

2> 	consider access to the cell as not barred;

1> 	if access to the cell is barred and timer T302 is not running:

2> 	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

2> 	start timer "Tbarring" with the timer value calculated as follows, using the ac-BarringTime included in "ACDC barring parameter":

"Tbarring" = (0.7+ 0.6 * rand) * ac-BarringTime.

#### 5.3.3.14	Access Barring check for NB-IoT

The UE shall:

1> if ab-Enabled included in MasterInformationBlock-NB is set to TRUE and SystemInformationBlockType14-NB is broadcast:

2> 	if the ab-Common is included in ab-Param:

3> 	if the UE belongs to the category of UEs as indicated in the ab-Category contained in ab-Common; and

3> 	if for the Access Class of the UE, as stored on the USIM and with a value in the range 0..9, the corresponding bit in the ab-BarringBitmap contained in ab-Common is set to one:

4> 	if the establishmentCause received from higher layers is set to mo-ExceptionData and ab-BarringForExceptionData is set to FALSE in the ab-Common:

5> 	consider access to the cell as not barred;

4> 	else:

5> 	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to 3GPP TS 22.011 [10] and 3GPP TS 23.122 [11] and for at least one of these valid Access Classes for the UE, the corresponding bit in the ab-BarringForSpecialAC contained in ab-Common is set to zero:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

6> 	consider access to the cell as not barred;

5> 	else:

6> 	consider access to the cell as barred;

3> 	else;

4> 	consider access to the cell as not barred;

2> 	else (the ab-PerPLMN-List is included in the ab-Param):

3> 	select the ab-PerPLMN entry in ab-PerPLMN-List corresponding to the PLMN selected by upper layers (see 3GPP TS 23.122 [11], and MFA TS 24.301 [35]);

3> 	if the ab-Config for that PLMN is included:

4> 	if the UE belongs to the category of UEs as indicated in the ab-Category contained in ab-Config; and

4> 	if for the Access Class of the UE, as stored on the USIM and with a value in the range 0..9, the corresponding bit in the ab-BarringBitmap contained in ab-Config is set to one:

5> 	if the establishmentCause received from higher layers is set to mo-ExceptionData and ab-BarringForExceptionData is set to FALSE in the ab-Config:

6> 	consider access to the cell as not barred;

5> 	else:

6> 	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to 3GPP TS 22.011 [10] and 3GPP TS 23.122 [11] and for at least one of these valid Access Classes for the UE, the corresponding bit in the ab-BarringForSpecialAC contained in ab-Config is set to zero:

NOTE 2:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

7> 	consider access to the cell as not barred;

6> 	else:

7> 	consider access to the cell as barred;

4> 	else:

5> 	consider access to the cell as not barred;

3> 	else:

4> 	consider access to the cell as not barred;

1> 	else:

2> 	consider access to the cell as not barred;

### 5.3.4	Initial security activation

#### 5.3.4.1	General

Figure 5.3.4.1-1: Security mode command, successful

Figure 5.3.4.1-2: Security mode command, failure

The purpose of this procedure is to activate AS security upon RRC connection establishment.

#### 5.3.4.2	Initiation

E-UTRAN initiates the security mode command procedure to a UE in RRC_CONNECTED. Moreover, E-UTRAN applies the procedure as follows:

-	when only SRB1 is established, i.e. prior to establishment of SRB2 and/ or DRBs.

#### 5.3.4.3	Reception of the SecurityModeCommand by the UE

The UE shall:

1> derive the KeNB key, as specified in MFA TS 33.401 [32];

1> derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in MFA TS 33.401 [32];

1> 	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1> 	if the SecurityModeCommand message passes the integrity protection check:

2> 	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in MFA TS 33.401 [32];

2> 	if connected as an RN:

3> 	derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in MFA TS 33.401 [32];

2> 	configure lower layers to apply integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent messages received and sent by the UE, including the SecurityModeComplete message;

2> 	configure lower layers to apply ciphering using the indicated algorithm, the KRRCenc key and the KUPenc key after completing the procedure, i.e. ciphering shall be applied to all subsequent messages received and sent by the UE, except for the SecurityModeComplete message which is sent unciphered;

2> 	if connected as an RN:

3> 	configure lower layers to apply integrity protection using the indicated algorithm and the KUPint key, for DRBs that are subsequently configured to apply integrity protection, if any;

2> 	consider AS security to be activated;

2> 	upon RRC connection establishment, if UE does not need UL gaps during continuous uplink transmission:

3> 	configure lower layers to stop using UL gaps during continuous uplink transmission in FDD for SecurityModeComplete message and subsequent uplink transmission in RRC_CONNECTED except for UL transmissions as specified in MFA TS 36.211 [21];

2> 	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

1> 	else:

2> 	continue using the configuration used prior to the reception of the SecurityModeCommand message, i.e. neither apply integrity protection nor ciphering.

2> 	submit the SecurityModeFailure message to lower layers for transmission, upon which the procedure ends;

### 5.3.5	RRC connection reconfiguration

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC connection reconfiguration, successful

Figure 5.3.5.1-2: RRC connection reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/ modify/ release RBs, to perform handover, to setup/ modify/ release measurements, to add/ modify/ release SCells. As part of the procedure, NAS dedicated information may be transferred from E-UTRAN to the UE.

#### 5.3.5.2	Initiation

E-UTRAN may initiate the RRC connection reconfiguration procedure to a UE in RRC_CONNECTED. E-UTRAN applies the procedure as follows:

-	the mobilityControlInfo is included only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended;

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is included only when AS security has been activated;

-	the addition of SCells is performed only when AS security has been activated;

#### 5.3.5.3	Reception of an RRCConnectionReconfiguration not including the mobilityControlInfo by the UE

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1> if this is the first RRCConnectionReconfiguration message after successful completion of the RRC connection re-establishment procedure:

2> 	re-establish PDCP for SRB2 and for all DRBs that are established, if any;

2> 	re-establish RLC for SRB2 and for all DRBs that are established, if any;

2> 	if the RRCConnectionReconfiguration message includes the fullConfig:

3> 	perform the radio configuration procedure as specified in 5.3.5.8;

2> 	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3> 	perform the radio resource configuration procedure as specified in 5.3.10;

2> 	resume SRB2 and all DRBs that are suspended, if any;

NOTE 1:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in 3GPP TS 36.323 [8].

NOTE 2:	The UE may discard SRB2 messages and data that it receives prior to completing the reconfiguration used to resume these bearers.

1> else:

2> 	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3> 	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 3:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

1> 	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2> 	perform SCell release as specified in 5.3.10.3a;

1> 	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2> 	perform SCell addition or modification as specified in 5.3.10.3b;

1> if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1> 	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2> 	perform SCG reconfiguration as specified in 5.3.10.10;

1> 	if the received RRCConnectionReconfiguration includes the systemInformationBlockType1Dedicated:

2> 	perfom the actions upon reception of the SystemInformationBlockType1 message as specified in 5.2.2.7;

1> 	if the RRCConnectionReconfiguration message includes the dedicatedInfoNASList:

2> 	forward each element of the dedicatedInfoNASList to upper layers in the same order as listed;

1> 	if the RRCConnectionReconfiguration message includes the measConfig:

2> 	perform the measurement configuration procedure as specified in 5.5.2;

1> 	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1> 	if the RRCConnectionReconfiguration message includes the otherConfig:

2> 	perform the other configuration procedure as specified in 5.3.10.9;

1> 	if the RRCConnectionReconfiguration message includes the sl-DiscConfig or sl-CommConfig:

2> 	perform the sidelink dedicated configuration procedure as specified in 5.3.10.15;

1> 	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2> 	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1> 	if the RRCConnectionReconfiguration message includes rclwi-Configuration:

2> 	perform the WLAN traffic steering command procedure as specified in 5.6.16.2;

1> 	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2> 	perform the LWA configuration procedure as specified in 5.6.14.2;

1> 	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2> 	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

1> 	upon RRC connection establishment, if UE does not need UL gaps during continuous uplink transmission:

2> 	configure lower layers to stop using UL gaps during continuous uplink transmission in FDD for RRCConnectionReconfigurationComplete message and subsequent uplink transmission in RRC_CONNECTED except for UL transmissions as specified in MFA TS 36.211 [21];

1> 	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

#### 5.3.5.4	Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover)

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1> 	stop timer T310, if running;

1> 	stop timer T312, if running;

1> 	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1> 	stop timer T370, if running;

1> 	if the carrierFreq is included:

2> 	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1> 	else:

2> 	consider the target PCell to be one on the frequency of the source PCell with a physical cell identity indicated by the targetPhysCellId;

1> 	start synchronising to the DL of the target PCell;

NOTE 1:	The UE should perform the handover as soon as possible following the reception of the RRC message triggering the handover, which could be before confirming successful reception (HARQ and ARQ) of this message.

1> 	if BL UE or UE in CE:

2> 	acquire the MasterInformationBlock in the target PCell;

1> 	reset MCG MAC and SCG MAC, if configured;

1> 	re-establish PDCP for all RBs that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in 3GPP TS 36.323 [8].

1> 	re-establish MCG RLC and SCG RLC, if configured, for all RBs that are established;

1> 	configure lower layers to consider the SCell(s) other than the PSCell, if configured, to be in deactivated state;

1> 	apply the value of the newUE-Identity as the C-RNTI;

1> 		if the RRCConnectionReconfiguration message includes the fullConfig:

2> 	perform the radio configuration procedure as specified in 5.3.5.8;

1> 	configure lower layers in accordance with the received radioResourceConfigCommon;

1> 	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1> 	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2> 	perform SCell release as specified in 5.3.10.3a;

1> 	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1> 	if the current UE configuration includes one or more split DRBs and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2> 	perform SCG reconfiguration as specified in 5.3.10.10;

1> 	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

2> 	perform the radio resource configuration procedure as specified in 5.3.10;

1> 	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

2> 	update the KeNB key based on the KASME key taken into use with the latest successful NAS SMC procedure, as specified in MFA TS 33.401 [32];

1> 	else:

2> 	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in MFA TS 33.401 [32];

1> 	store the nextHopChainingCount value;

1> 	if the securityAlgorithmConfig is included in the securityConfigHO:

2> 	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in MFA TS 33.401 [32];

2> 	if connected as an RN:

3> 	derive the KUPint key associated with the integrityProtAlgorithm, as specified in MFA TS 33.401 [32];

2> 	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in MFA TS 33.401 [32];

1> 	else:

2> 	derive the KRRCint key associated with the current integrity algorithm, as specified in MFA TS 33.401 [32];

2> 	if connected as an RN:

3> 	derive the KUPint key associated with the current integrity algorithm, as specified in MFA TS 33.401 [32];

2> 	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in MFA TS 33.401 [32];

1> 	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1> 	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1> 	if connected as an RN:

2> 	configure lower layers to apply the integrity protection algorithm and the KUPint key, for current or subsequently established DRBs that are configured to apply integrity protection, if any;

1> 	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2> 	perform SCell addition or modification as specified in 5.3.10.3b;

1> 	if the received RRCConnectionReconfiguration includes the systemInformationBlockType1Dedicated:

2> 	perfom the actions upon reception of the SystemInformationBlockType1 message as specified in 5.2.2.7;

1> 	perform the measurement related actions as specified in 5.5.6.1;

1> 		if the RRCConnectionReconfiguration message includes the measConfig:

2> 	perform the measurement configuration procedure as specified in 5.5.2;

1> 	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1> 	release reportProximityConfig and clear any associated proximity status reporting timer;

1> 	if the RRCConnectionReconfiguration message includes the otherConfig:

2> 	perform the other configuration procedure as specified in 5.3.10.9;

1> 	if the RRCConnectionReconfiguration message includes the sl-DiscConfig or sl-CommConfig:

2> 	perform the sidelink dedicated configuration procedure as specified in 5.3.10.15;

1> 	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2> 	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1> 	release the LWA configuration, if configured, as described in 5.6.14.3;

1> 	release the LWIP configuration, if configured, as described in 5.6.17.3;

1> 	if the RRCConnectionReconfiguration message includes rclwi-Configuration:

2> 	perform the WLAN traffic steering command procedure as specified in 5.6.16.2;

1> 	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2> 	perform the LWA configuration procedure as specified in 5.6.14.2;

1> 	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2> 	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

1> 	set the content of RRCConnectionReconfigurationComplete message as follows:

2> 	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3> 	include rlf-InfoAvailable;

2> 	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and if T330 is not running:

3> 	include logMeasAvailableMBSFN;

2> 	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3> 	include the logMeasAvailable;

2> 	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3> 	include connEstFailInfoAvailable;

1> 	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1> 	if MAC successfully completes the random access procedure:

2> 	stop timer T304;

2> 	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2> 	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2> 	if the UE is configured to provide IDC indications:

3> 	if the UE has transmitted an InDeviceCoexIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4> 	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2> 	if the UE is configured to provide power preference indications:

3> 	if the UE has transmitted a UEAssistanceInformation message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4> 	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

2> 	if SystemInformationBlockType15 is broadcast by the PCell:

3> 	if the UE has transmitted a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4> 	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4> 	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4> 	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

4> 	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2> 	if SystemInformationBlockType18 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in target PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2> 	if SystemInformationBlockType19 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in target PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

3> 	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

2> 	the procedure ends;

NOTE 4:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell, except for BL UEs or UEs in CE.

#### 5.3.5.5	Reconfiguration failure

The UE shall:

1> 	if the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message:

2> 	continue using the configuration used prior to the reception of RRCConnectionReconfiguration message;

2> 	if security has not been activated:

3> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause other;

2> 	else:

3> 	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the connection reconfiguration procedure ends;

NOTE 1:	The UE may apply above failure handling also in case the RRCConnectionReconfiguration message causes a protocol error for which the generic error handling as defined in 5.7 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.

#### 5.3.5.6	T304 expiry (handover failure)

The UE shall:

1> 	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2> 	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2> 	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3> 	clear the information included in VarRLF-Report, if any;

3> 	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3> 	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the source PCell based on measurements collected up to the moment the UE detected handover failure and in accordance with the following;

4> 	if the UE includes rsrqResult, include the lastServCellRSRQ-Type;

3> 	set the measResultNeighCells to include the best measured cells, other than the source PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4> 	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4> 	if the UE includes rsrqResult, include the rsrq-Type;

4> 	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4> 	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4> 	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4> 	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

3> 	if detailed location information is available, set the content of the locationInfo as follows:

4> 	include the locationCoordinates;

4> 	include the horizontalVelocity, if available;

3> 	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3> 	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3> 	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3> 	set the connectionFailureType to 'hof';

3> 	set the c-RNTI to the C-RNTI used in the source PCell;

2> 	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

#### 5.3.5.7	Void

#### 5.3.5.7a	T307 expiry (SCG change failure)

The UE shall:

1> 	if T307 expires:

NOTE 1:	Following T307 expiry any dedicated preamble, if provided within the rach-ConfigDedicatedSCG, is not available for use by the UE anymore.

2> 	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG change failure;

#### 5.3.5.8	Radio Configuration involving full configuration option

The UE shall:

1>	release/ clear all current dedicated radio configurations except the MCG C-RNTI, the MCG security configuration and the PDCP, RLC, logical channel configurations for the RBs and the logged measurement configuration;

NOTE 1:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig and OtherConfig.

1> 	if the RRCConnectionReconfiguration message includes the mobilityControlInfo:

2> 	release/ clear all current common radio configurations;

2> 	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1> 	else:

2> 	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SystemInformationBlockType2;

1> 	apply the default physical channel configuration as specified in 9.2.4;

1>	if the RRCConnectionReconfiguration message includes the mobilityControlInfo:

2> 	if the mobilityControlInfo contains the carrierFreq, and the carrierFreq belongs to an MF band, as documented in MFA TS 36.101 [42];

3>	further apply the default physical channel configuration as specified in 9.2.4MF;

3>	if the MF extension is absent;

4>	further set the prach-LBT, prach-FreqIndex and prach-NumInterlaces as specified in 9.2MF;

1> 	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1> 	apply the default MAC main configuration as specified in 9.2.2;

1> 	if the UE is a NB-IoT UE; or

1> 	for each srb-Identity value included in the srb-ToAddModList (SRB reconfiguration):

2> 	apply the specified configuration defined in 9.1.2 for the corresponding SRB;

2> 	apply the corresponding default RLC configuration for the SRB specified in 9.2.1.1 for SRB1 or in 9.2.1.2 for SRB2;

2> 	apply the corresponding default logical channel configuration for the SRB as specified in 9.2.1.1 for SRB1 or in 9.2.1.2 for SRB2;

NOTE 2: 	This is to get the SRBs (SRB1 and SRB2 for handover and SRB2 for reconfiguration after reestablishment) to a known state from which the reconfiguration message can do further configuration.

1> 	for each eps-BearerIdentity value included in the drb-ToAddModList that is part of the current UE configuration:

2> 	release the PDCP entity;

2> 	release the RLC entity or entities;

2> 	release the DTCH logical channel;

2> 	release the drb-identity;

NOTE 3:	This will retain the eps-bearerIdentity but remove the DRBs including drb-identity of these bearers from the current UE configuration and trigger the setup of the DRBs within the AS in Section 5.3.10.3 using the new configuration. The eps-bearerIdentity acts as the anchor for associating the released and re-setup DRB. In the AS the DRB re-setup is equivalent with a new DRB setup (including new PDCP and logical channel configurations).

1> 	for each eps-BearerIdentity value that is part of the current UE configuration but not part of the drb-ToAddModList:

2> 	perform DRB release as specified in 5.3.10.2;

### 5.3.6	Counter check

#### 5.3.6.1	General

Figure 5.3.6.1-1: Counter check procedure

The counter check procedure is used by E-UTRAN to request the UE to verify the amount of data sent/ received on each DRB. More specifically, the UE is requested to check if, for each DRB, the most significant bits of the COUNT match with the values indicated by E-UTRAN.

NOTE:	The procedure enables E-UTRAN to detect packet insertion by an intruder (a 'man in the middle').

#### 5.3.6.2	Initiation

E-UTRAN initiates the procedure by sending a CounterCheck message.

NOTE:	E-UTRAN may initiate the procedure when any of the COUNT values reaches a specific value.

#### 5.3.6.3	Reception of the CounterCheck message by the UE

Upon receiving the CounterCheck message, the UE shall:

1>	for each DRB that is established:

2>	if no COUNT exists for a given direction (uplink or downlink) because it is a uni-directional bearer configured only for the other direction:

3>	assume the COUNT value to be 0 for the unused direction;

2>	if the drb-Identity is not included in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of the corresponding COUNT;

2>	else if, for at least one direction, the most significant bits of the COUNT are different from the value indicated in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of the corresponding COUNT;

1>	for each DRB that is included in the drb-CountMSB-InfoList in the CounterCheck message that is not established:

2>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink with the most significant bits set identical to the corresponding values in the drb-CountMSB-InfoList and the least significant bits set to zero;

1>	submit the CounterCheckResponse message to lower layers for transmission upon which the procedure ends;

### 5.3.7	RRC connection re-establishment

#### 5.3.7.1	General

Figure 5.3.7.1-1: RRC connection re-establishment, successful

Figure 5.3.7.1-2: RRC connection re-establishment, failure

The purpose of this procedure is to re-establish the RRC connection, which involves the resumption of SRB1 operation, the re-activation of security and the configuration of only the PCell.

A UE in RRC_CONNECTED, for which security has been activated, may initiate the procedure in order to continue the RRC connection. The connection re-establishment succeeds only if the concerned cell is prepared i.e. has a valid UE context. In case E-UTRAN accepts the re-establishment, SRB1 operation resumes while the operation of other radio bearers remains suspended. If AS security has not been activated, the UE does not initiate the procedure but instead moves to RRC_IDLE directly.

E-UTRAN applies the procedure as follows:

-	to reconfigure SRB1 and to resume data transfer only for this RB;

-	to re-activate AS security without changing algorithms.

#### 5.3.7.2	Initiation

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

1> 	upon detecting radio link failure, in accordance with 5.3.11; or

1> 	upon handover failure, in accordance with 5.3.5.6; or

1> 	upon mobility from E-UTRA failure, in accordance with 5.4.3.5; or

1> 	upon integrity check failure indication from lower layers; or

1> 	upon an RRC connection reconfiguration failure, in accordance with 5.3.5.5;

Upon initiation of the procedure, the UE shall:

1> 	stop timer T310, if running;

1> 	stop timer T312, if running;

1> 	stop timer T313, if running;

1> 	stop timer T307, if running;

1> 	start timer T311;

1> 	stop timer T370, if running;

1> 	suspend all RBs except SRB0;

1> 	reset MAC;

1> 	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

1> 	apply the default physical channel configuration as specified in 9.2.4;

1> 	except for NB-IoT, for the MCG, apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1> 	for the MCG, apply the default MAC main configuration as specified in 9.2.2;

1> 	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

1> 	release reportProximityConfig, if configured and clear any associated proximity status reporting timer;

1> 	release obtainLocationConfig, if configured;

1> 	release idc-Config, if configured;

1> 	release measSubframePatternPCell, if configured;

1> 	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

1> 	release naics-Info for the PCell, if configured;

1> 	if connected as an RN and configured with an RN subframe configuration:

2> 	release the RN subframe configuration;

1> 	release the LWA configuration, if configured, as described in 5.6.14.3;

1> 	release the LWIP configuration, if configured, as described in 5.6.17.3;

1> 	perform cell selection in accordance with the cell selection process as specified in MFA TS 36.304 [4];

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable E-UTRA cell, the UE shall:

1> 	stop timer T311;

1> 	start timer T301;

1> 	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1> 	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

NOTE:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1> 	if the selected cell is a UTRA cell, and if the UE supports Radio Link Failure Report for Inter-RAT MRO, include selectedUTRA-CellId in the VarRLF-Report and set it to the physical cell identity and carrier frequency of the selected UTRA cell;

1> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

#### 5.3.7.4	Actions related to transmission of RRCConnectionReestablishmentRequest message

Except for NB-IoT, if the procedure was initiated due to radio link failure or handover failure, the UE shall:

1> 	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1> 	set the ue-Identity as follows:

2> 	set the c-RNTI to the C-RNTI used in the source PCell (handover and mobility from E-UTRA failure) or used in the PCell in which the trigger for the re-establishment occurred (other cases);

2> 	set the physCellId to the physical cell identity of the source PCell (handover and mobility from E-UTRA failure) or of the PCell in which the trigger for the re-establishment occurred (other cases);

2> 	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3> 	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input (or VarShortMAC-Input-NB in NB-IoT);

3> 	with the KRRCint key and integrity protection algorithm that was used in the source PCell (handover and mobility from E-UTRA failure) or of the PCell in which the trigger for the re-establishment occurred (other cases); and

3> 	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1> 	set the reestablishmentCause as follows:

2> 	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.5 (the UE is unable to comply with the reconfiguration):

3> 	set the reestablishmentCause to the value reconfigurationFailure;

2> 	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3> 	set the reestablishmentCause to the value handoverFailure;

2> 	else:

3> 	set the reestablishmentCause to the value otherFailure;

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

#### 5.3.7.5	Reception of the RRCConnectionReestablishment by the UE

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see MFA TS 36.321 [6];

The UE shall:

1> 	stop timer T301;

1> 	consider the current cell to be the PCell;

1> 	re-establish PDCP for SRB1;

1> 	re-establish RLC for SRB1;

1> 	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1> 	resume SRB1;

NOTE 2:	E-UTRAN should not transmit any message on SRB1 prior to receiving the RRCConnectionReestablishmentComplete message.

1> 	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionReestablishment message, as specified in MFA TS 33.401 [32];

1> 	store the nextHopChainingCount value;

1> 	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in MFA TS 33.401 [32];

1> 	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in MFA TS 33.401 [32];

1> 		if connected as an RN:

2> 	derive the KUPint key associated with the previously configured integrity algorithm, as specified in MFA TS 33.401 [32];

1> 		configure lower layers to activate integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1> 	if connected as an RN:

2> 	configure lower layers to apply integrity protection using the previously configured algorithm and the KUPint key, for subsequently resumed or subsequently established DRBs that are configured to apply integrity protection, if any;

1> 	configure lower layers to apply ciphering using the previously configured algorithm, the KRRCenc key and the KUPenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1> 	if the UE is not a NB-IoT UE:

2> 	set the content of RRCConnectionReestablishmentComplete message as follows:

3> 	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

4> 	include the rlf-InfoAvailable;

3> 	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and if T330 is not running:

4> 	include logMeasAvailableMBSFN;

3> 	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4> 	include the logMeasAvailable;

3> 	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

4> 	include the connEstFailInfoAvailable;

2> 	perform the measurement related actions as specified in 5.5.6.1;

2> 	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1> 	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission;

1> 	if SystemInformationBlockType15 is broadcast by the PCell:

2> 	if the UE has transmitted an MBMSInterestIndication message during the last 1 second preceding detection of radio link failure:

3> 	ensure having a valid version of SystemInformationBlockType15 for the PCell;

3> 	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

3> 	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

3> 	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

1> 	if SystemInformationBlockType18 is broadcast by the PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding detection of radio link failure; or

1> 	if SystemInformationBlockType19 is broadcast by the PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding detection of radio link failure:

2> 	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

1> 	the procedure ends;

#### 5.3.7.6	T311 expiry

Upon T311 expiry, the UE shall:

1> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

#### 5.3.7.7	T301 expiry or selected cell no longer suitable

The UE shall:

1> 	if timer T301 expires; or

1> 	if the selected cell becomes no longer suitable according to the cell selection criteria as specified in MFA TS 36.304 [4]:

2> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

#### 5.3.7.8	Reception of RRCConnectionReestablishmentReject by the UE

Upon receiving the RRCConnectionReestablishmentReject message, the UE shall:

1> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

### 5.3.8	RRC connection release

#### 5.3.8.1	General

Figure 5.3.8.1-1: RRC connection release, successful

The purpose of this procedure is:

-	 to release the RRC connection, which includes the release of the established radio bearers as well as all radio resources;

or:

-	 to suspend the RRC connection, which includes the suspension of the established radio bearers.

#### 5.3.8.2	Initiation

E-UTRAN initiates the RRC connection release procedure to a UE in RRC_CONNECTED.

#### 5.3.8.3	Reception of the RRCConnectionRelease by the UE

The UE shall:

1> 	delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1> 	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

2> 	store the cell reselection priority information provided by the idleModeMobilityControlInfo;

2> 	if the t320 is included:

3> 	start timer T320, with the timer value set according to the value of t320;

1> 	else:

2> 	apply the cell reselection priority information broadcast in the system information;

1> 	if the releaseCause received in the RRCConnectionRelease message indicates loadBalancingTAURequired:

2> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'load balancing TAU required';

1> 	else if the releaseCause received in the RRCConnectionRelease message indicates cs-FallbackHighPriority:

2> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'CS Fallback High Priority';

1> 	else:

2> 	if the extendedWaitTime is present; and

2> 	if the UE supports delay tolerant access or the UE is a NB-IoT UE:

3> 	forward the extendedWaitTime to upper layers;

2> 	if the releaseCause received in the RRCConnectionRelease message indicates rrc-Suspend:

3> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause ‘RRC suspension’;

2> 	else:

3> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

#### 5.3.8.4	T320 expiry

The UE shall:

1> 	if T320 expires:

2> 	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

2> 	apply the cell reselection priority information broadcast in the system information;

### 5.3.9	RRC connection release requested by upper layers

#### 5.3.9.1	General

The purpose of this procedure is to release the RRC connection. Access to the current PCell may be barred as a result of this procedure.

NOTE:	Upper layers invoke the procedure, e.g. upon determining that the network has failed an authentication check, see MFA TS 24.301 [35].

#### 5.3.9.2	Initiation

The UE initiates the procedure when upper layers request the release of the RRC connection. The UE shall not initiate the procedure for power saving purposes.

The UE shall:

1> 	if the upper layers indicate barring of the PCell:

2> 	treat the PCell used prior to entering RRC_IDLE as barred according to MFA TS 36.304 [4];

1> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

### 5.3.10	Radio resource configuration

#### 5.3.10.0	General

The UE shall:

1> 	if the received radioResourceConfigDedicated includes the srb-ToAddModList:

2> 	perform the SRB addition or reconfiguration as specified in 5.3.10.1;

1> 	if the received radioResourceConfigDedicated includes the drb-ToReleaseList:

2> 	perform DRB release as specified in 5.3.10.2;

1> 	if the received radioResourceConfigDedicated includes the drb-ToAddModList:

2> 	perform DRB addition or reconfiguration as specified in 5.3.10.3;

1> 	if the received radioResourceConfigDedicated includes the mac-MainConfig:

2> 	perform MAC main reconfiguration as specified in 5.3.10.4;

1> 	if the received radioResourceConfigDedicated includes sps-Config:

2> 	perform SPS reconfiguration according to 5.3.10.5;

1> 	if the received radioResourceConfigDedicated includes the physicalConfigDedicated:

2> 	reconfigure the physical channel configuration as specified in 5.3.10.6.

1> 	if the received radioResourceConfigDedicated includes the rlf-TimersAndConstants:

2> 	reconfigure the values of timers and constants as specified in 5.3.10.7;

1> 	if the received radioResourceConfigDedicated includes the measSubframePatternPCell:

2> 	reconfigure the time domain measurement resource restriction for the serving cell as specified in 5.3.10.8;

1> 	if the received radioResourceConfigDedicated includes the naics-Info:

2> 	perform NAICS neighbour cell information reconfiguration for the PCell as specified in 5.3.10.13;

1> 	if the received RadioResourceConfigDedicatedPSCell includes the naics-Info:

2> 	perform NAICS neighbour cell information reconfiguration for the PSCell as specified in 5.3.10.13;

1> 	if the received RadioResourceConfigDedicatedSCell-r10 includes the naics-Info:

2> 	perform NAICS neighbour cell information reconfiguration for the SCell as specified in 5.3.10.13;

#### 5.3.10.1	SRB addition/modification

The UE shall:

1> 	if the UE is a NB-IoT UE and SRB1 is not established; or

1> 	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment):

2> if the UE is not a NB-IoT UE that only supports the Control Plane CIoT EPS optimisation:

3> 	apply the specified configuration defined in 9.1.2 for the corresponding SRB;

3> 	establish a PDCP entity and configure it with the current (MCG) security configuration, if applicable;

3> 	establish an (MCG) RLC entity in accordance with the received rlc-Config;

3> 	establish a (MCG) DCCH logical channel in accordance with the received logicalChannelConfig and with the logical channel identity set in accordance with 9.1.2;

2> 	if the UE is a NB-IoT UE:

3> 	apply the specified configuration defined in 9.1.2 for SRB1bis;

3> 	establish an (MCG) RLC entity in accordance with the received rlc-Config;

3> 	establish a (MCG) DCCH logical channel in accordance with the received logicalChannelConfig and with the logical channel identity set in accordance with 9.1.2.1a;

1> 	if the UE is a NB-IoT UE and SRB1 is established; or

1> 	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration (SRB reconfiguration):

2> 	reconfigure the RLC entity in accordance with the received rlc-Config;

2> 	reconfigure the DCCH logical channel in accordance with the received logicalChannelConfig;

#### 5.3.10.2	DRB release

The UE shall:

1> 	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration (DRB release); or

1> 	for each drb-identity value that is to be released as the result of full configuration option according to 5.3.5.8:

2> 	release the PDCP entity;

2> 	release the RLC entity or entities;

2> 	release the DTCH logical channel;

1> 	if the procedure was triggered due to handover:

2> 	indicate the release of the DRB(s) and the eps-BearerIdentity of the released DRB(s) to upper layers after successful handover;

1> 	else:

2> 	indicate the release of the DRB(s) and the eps-BearerIdentity of the released DRB(s) to upper layers immediately.

NOTE:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

#### 5.3.10.3	DRB addition/modification

The UE shall:

1> 	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2> 	if the concerned entry of drb-ToAddModList includes the drb-TypeLWA set to TRUE (i.e. add LWA DRB):

3> 	perform the LWA specific DRB addition or reconfiguration as specified in 5.3.10.3a2;

2> 	if the concerned entry of drb-ToAddModList includes the drb-TypeLWIP (i.e. add LWIP DRB):

3> 	perform LWIP specific DRB addition or reconfiguration as specified in 5.3.10.3a3;

2> 	else if drb-ToAddModListSCG is not received or does not include the drb-Identity value (i.e. add MCG DRB):

3> 	establish a PDCP entity and configure it with the current MCG security configuration and in accordance with the received pdcp-Config;

3> 	establish an MCG RLC entity or entities in accordance with the received rlc-Config;

3> 	establish an MCG DTCH logical channel in accordance with the received logicalChannelIdentity and the received logicalChannelConfig;

2> 	if the RRCConnectionReconfiguration message includes the fullConfig IE:

3> 	associate the established DRB with corresponding included eps-BearerIdentity;

2> 	else:

3> 	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1> 	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration (DRB reconfiguration):

2> 	if the DRB indicated by drb-Identity is an LWA DRB (i.e. LWA to LTE only or reconfigure LWA DRB):

3> 	perform the LWA specific DRB reconfiguration as specified in 5.3.10.3a2;

2> 	else if the concerned entry of drb-ToAddModList includes the drb-TypeLWA set to TRUE (i.e. LTE only to LWA DRB):

3> 	perform the LWA specific DRB reconfiguration as specified in 5.3.10.3a2;

2> 	if the concerned entry of drb-ToAddModList includes the drb-TypeLWIP (i.e. add or reconfigure LWIP DRB):

3> 	perform LWIP specific DRB addition or reconfiguration as specified in 5.3.10.3a3;

2> 	if drb-ToAddModListSCG is not received or does not include the drb-Identity value:

3> 	if the DRB indicated by drb-Identity is an MCG DRB (reconfigure MCG):

4> 	if the pdcp-Config is included:

5> 	reconfigure the PDCP entity in accordance with the received pdcp-Config;

4> 	if the rlc-Config is included:

5> 	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

4> 	if the logicalChannelConfig is included:

5> 	reconfigure the DTCH logical channel in accordance with the received logicalChannelConfig;

NOTE:	Removal and addition of the same drb-Identity in a single radioResourceConfigDedicated is not supported. In case drb-Identity is removed and added due to handover or re-establishment with the full configuration option, the eNB can use the same value of drb-Identity.

#### 5.3.10.3a1	DC specific DRB addition or reconfiguration

For the drb-Identity value for which this procedure is initiated, the UE shall:

1> 	if drb-ToAddModListSCG is received and includes the drb-Identity value; and drb-Identity value is not part of the current UE configuration (i.e. DC specific DRB establishment):

2> 	if drb-ToAddModList is received and includes the drb-Identity value (i.e. add split DRB):

3> 	establish a PDCP entity and configure it with the current MCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModList;

3> 	establish an MCG RLC entity and an MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig included in drb-ToAddModList;

3> 	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

2> 	else (i.e. add SCG DRB):

3> 	establish a PDCP entity and configure it with the current SCG security configuration and in accordance with the pdcp-Config included in drb-ToAddModListSCG;

3> 	establish an SCG RLC entity or entities and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

2> 	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1> 	else (i.e. DC specific DRB modification; drb-ToAddModList and/ or drb-ToAddModListSCG received):

2> 	if the DRB indicated by drb-Identity is a split DRB:

3> 	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. split to MCG):

4> 	release the SCG RLC entity and the SCG DTCH logical channel;

4> 	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4> 	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

3> 	else (i.e. reconfigure split):

4> 	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4> 	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4> 	reconfigure the SCG RLC entity and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

2> 	if the DRB indicated by drb-Identity is an SCG DRB:

3> 	if drb-ToAddModList is received and includes the drb-Identity value, while for this entry drb-TypeChange is included and set to toMCG (i.e. SCG to MCG):

4> 	reconfigure the PDCP entity with the current MCG security configuration and in accordance with the pdcp-Config, if included in drb-ToAddModList;

4> 	reconfigure the SCG RLC entity or entities and the SCG DTCH logical channel to be an MCG RLC entity or entities and an MCG DTCH logical channel;

4> 	reconfigure the MCG RLC entity or entities and/ or the MCG DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig, if included in drb-ToAddModList;

3> 	else (i.e. drb-ToAddModListSCG is received and includes the drb-Identity value i.e. reconfigure SCG):

4> 	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4> 	reconfigure the SCG RLC entity or entities and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

2> 	if the DRB indicated by drb-Identity is an MCG DRB:

3> 	if drb-ToAddModListSCG is received and includes the drb-Identity value, while for this entry drb-Type is included and set to split (i.e. MCG to split):

4> 	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

4> 	reconfigure the MCG RLC entity and/ or the MCG DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

4> 	establish an SCG RLC entity and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG, included in drb-ToAddModListSCG;

3> 	else (i.e. drb-Type is included and set to scg i.e. MCG to SCG):

4> 	reconfigure the PDCP entity with the current SCG security configuration and in accordance with the pdcp-Config, if included in drb-ToAddModListSCG;

4> 	reconfigure the MCG RLC entity or entities and the MCG DTCH logical channel to be an SCG RLC entity or entities and an SCG DTCH logical channel;

4> 	reconfigure the SCG RLC entity or entities and/ or the SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG, if included in drb-ToAddModListSCG;

#### 5.3.10.3a2	LWA specific DRB addition or reconfiguration

For the drb-Identity value for which this procedure is initiated, the UE shall:

1>	if the drb-Identity value is not part of the current UE configuration (i.e. add LWA DRB):

2>	establish a PDCP entity and configure it with the current security configuration and in accordance with the pdcp-Config included in drb-ToAddModList;

2>	establish an RLC entity and an DTCH logical channel in accordance with the rlc-Config, logicalChannelIdentity and logicalChannelConfig included in drb-ToAddModList;

2>	enable data handling for this DRB at the LWAAP entity;

2>	indicate the establishment of the DRB and the eps-BearerIdentity of the established DRB to upper layers;

1>	else if the DRB indicated by drb-Identity is not an LWA DRB (i.e. LTE only to LWA DRB):

2>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

2>	reconfigure the RLC entity and/ or the DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

2>	enable data handling for this DRB at the LWAAP entity;

1>	else if the concerned entry of drb-ToAddModList includes the drb-TypeLWA set to FALSE (i.e. LWA to LTE only DRB):

2>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

2>	reconfigure the RLC entity and/ or the DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

2>	perform PDCP data recovery as specified in 3GPP TS 36.323 [8];

2>	disable data handling for this DRB at the LWAAP entity;

1>	else (i.e. reconfigure LWA DRB):

2>	reconfigure the PDCP entity in accordance with the pdcp-Config, if included in drb-ToAddModList;

2>	reconfigure the RLC entity and/ or the DTCH logical channel in accordance with the rlc-Config and logicalChannelConfig, if included in drb-ToAddModList;

#### 5.3.10.3a3	LWIP specific DRB addition or reconfiguration

For the drb-Identity value for which this procedure is initiated, the UE shall:

1> 	if the drb-TypeLWIP is set to lwip:

2> 	indicate to higher layers to use LWIP resources in both UL and DL for the DRB associated with the drb-Identity;

1> 	if the drb-TypeLWIP is set to lwip-DL-only:

2> 	indicate to higher layers to use LWIP resources in the DL only for the DRB associated with the drb-Identity;

1> 	if the drb-TypeLWIP is set to lwip-UL-only:

2> 	indicate to higher layers to use LWIP resources in the UL only for the DRB associated with the drb-Identity;

1> 	if the drb-TypeLWIP is set to eutran:

2> 	indicate to higher layers to stop using LWIP resources for the DRB associated with the drb-Identity;

#### 5.3.10.3a	SCell release

The UE shall:

1> 	if the release is triggered by reception of the sCellToReleaseList or the sCellToReleaseListSCG:

2> 	for each sCellIndex value included either in the sCellToReleaseList or in the sCellToReleaseListSCG:

3> 	if the current UE configuration includes an SCell with value sCellIndex:

4> 	release the SCell;

1> 	if the release is triggered by RRC connection re-establishment:

2> 	release all SCells that are part of the current UE configuration;

### 5.3.10.3b	SCell addition/modification

The UE shall:

1> 	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2> 	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

2> 	configure lower layers to consider the SCell to be in deactivated state;

2> 	for each measId included in the measIdList within VarMeasConfig:

3> 	if SCells are not applicable for the associated measurement; and

3> 	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4> 	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1> 	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is part of the current UE configuration (SCell modification):

2> 	modify the SCell configuration in accordance with the radioResourceConfigDedicatedSCell, included either in the sCellToAddModList or in the sCellToAddModListSCG;

1>	for each sCellIndex value included in the sCellToAddModList received in message-MF:

2>	modify the SCell configuration in accordance with the measDS-ConfigSCell, radioResourceConfigCommonSCell  and radioResourceConfigDedicatedSCell, included in the sCellToAddModList;

#### 5.3.10.3c	PSCell addition or modification

The UE shall:

1> 		if the PSCell is not part of the current UE configuration (i.e. PSCell addition):

2> 	add the PSCell, corresponding to the cellIdentification, in accordance with the received radioResourceConfigCommonPSCell and radioResourceConfigDedicatedPSCell;

2> 	configure lower layers to consider the PSCell to be in activated state;

1> 	if the PSCell is part of the current UE configuration (i.e. PSCell modification):

2> 	modify the PSCell configuration in accordance with the received radioResourceConfigDedicatedPSCell;

#### 5.3.10.4	MAC main reconfiguration

Except for NB-IoT, the UE shall:

1> 	if the procedure is triggered to perform SCG MAC main reconfiguration:

2> 	if SCG MAC is not part of the current UE configuration (i.e. SCG establishment):

3> 	create an SCG MAC entity;

2> 	reconfigure the SCG MAC main configuration as specified in the following i.e. assuming it concerns the SCG MAC whenever MAC main configuration is referenced and that it is based on the received mac-MainConfigSCG instead of mac-MainConfig:

1> 	reconfigure the MAC main configuration in accordance with the received mac-MainConfig other than stag-ToReleaseList and stag-ToAddModList;

1> 	if the received mac-MainConfig includes the stag-ToReleaseList:

2> 	for each STAG-Id value included in the stag-ToReleaseList that is part of the current UE configuration:

3> 	release the STAG indicated by STAG-Id;

1> 	if the received mac-MainConfig includes the stag-ToAddModList:

2> 	for each stag-Id value included in stag-ToAddModList that is not part of the current UE configuration (STAG addition):

3> 	add the STAG, corresponding to the stag-Id, in accordance with the received timeAlignmentTimerSTAG;

2> 	for each stag-Id value included in stag-ToAddModList that is part of the current UE configuration (STAG modification):

3> 	reconfigure the STAG, corresponding to the stag-Id, in accordance with the received timeAlignmentTimerSTAG;

For NB-IoT, the UE shall:

1> 	reconfigure the MAC main configuration in accordance with the received mac-MainConfig;

#### 5.3.10.5	Semi-persistent scheduling reconfiguration

The UE shall:

1> 	reconfigure the semi-persistent scheduling in accordance with the received sps-Config;

#### 5.3.10.6	Physical channel reconfiguration

Except for NB-IoT, the UE shall:

1> 	if the antennaInfo-r10 is included in the received physicalConfigDedicated and the previous version of this field that was received by the UE was antennaInfo (without suffix i.e. the version defined in REL-8):

2> 	apply the default antenna configuration as specified in 9.2.4;

1> 	if the cqi-ReportConfig-r10 is included in the received physicalConfigDedicated and the previous version of this field that was received by the UE was cqi-ReportConfig (without suffix i.e. the version defined in REL-8):

2> 	apply the default CQI reporting configuration as specified in 9.2.4;

NOTE:	Application of the default configuration involves release of all extensions introduced in REL-9 and later.

1> 	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated;

1> 	if the antennaInfo is included and set to explicitValue:

2> 	if the configured transmissionMode is tm1, tm2, tm5, tm6 or tm7; or

2> 	if the configured transmissionMode is tm8 and pmi-RI-Report is not present; or

2> 	if the configured transmissionMode is tm9 and pmi-RI-Report is not present; or

2> 	if the configured transmissionMode is tm9 and pmi-RI-Report is present and antennaPortsCount within csi-RS is set to an1:

3> 	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

1> 	else if the antennaInfo is included and set to defaultValue:

2> 	release ri-ConfigIndex in cqi-ReportPeriodic, if previously configured;

For NB-IoT, the UE shall:

1> 	if the carrierConfigDedicated is not included in the received physicalConfigDedicated:

2> 	if the UE is configured with a carrier configuration previously received in carrierConfigDedicated:

3> 	use the carrier configuration received in carrierConfigDedicated;

2> 	else:

3> 	use the carrier configuration received for the anchor carrier in the system information;

1> 	else:

2> 	use the carrier configuration received in carrierConfigDedicated;

2> 	start to use the new carrier immediately after the last transport block carrying the RRC message has been acknowledged by the MAC layer, and any subsequent RRC response message sent for the current RRC procedure is therefore sent on the new carrier;

1> 	reconfigure the physical channel configuration in accordance with the received physicalConfigDedicated.

#### 5.3.10.7	Radio Link Failure Timers and Constants reconfiguration

The UE shall:

1> 	if the received rlf-TimersAndConstants is set to release:

2> 	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT);

1> 	else:

2> 	reconfigure the value of timers and constants in accordance with received rlf-TimersAndConstants;

1> 	if the received rlf-TimersAndConstantsSCG is set to release:

2> 	stop timer T313, if running, and

2> 	release the value of timer t313 as well as constants n313 and n314;

1> 	else:

2> 	reconfigure the value of timers and constants in accordance with received rlf-TimersAndConstantsSCG;

#### 5.3.10.8	Time domain measurement resource restriction for serving cell

The UE shall:

1> 	if the received measSubframePatternPCell is set to release:

2> 	release the time domain measurement resource restriction for the PCell, if previously configured

1> 	else:

2> 	apply the time domain measurement resource restriction for the PCell in accordance with the received measSubframePatternPCell;

#### 5.3.10.9	Other configuration

The UE shall:

1> 	if the received otherConfig includes the reportProximityConfig:

2> 	if proximityIndicationEUTRA is set to enabled:

3> 	consider itself to be configured to provide proximity indications for E-UTRA frequencies in accordance with 5.3.14;

2> 	else:

3> 	consider itself not to be configured to provide proximity indications for E-UTRA frequencies;

2> 	if proximityIndicationUTRA is set to enabled:

3> 	consider itself to be configured to provide proximity indications for UTRA frequencies in accordance with 5.3.14;

2> 	else:

3> 	consider itself not to be configured to provide proximity indications for UTRA frequencies;

1> 	if the received otherConfig includes the obtainLocation:

2> 	attempt to have detailed location information available for any subsequent measurement report;

NOTE:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1> 	if the received otherConfig includes the idc-Config:

2> 	if idc-Indication is included (i.e. set to setup):

3> 	consider itself to be configured to provide IDC indications in accordance with 5.6.9;

3> 	if idc-Indication-UL-CA is included (i.e. set to setup):

4> 	consider itself to be configured to indicate UL CA related information in IDC indications in accordance with 5.6.9;

2> 	else:

3> 	consider itself not to be configured to provide IDC indications;

2> 	if autonomousDenialParameters is included:

3> 	consider itself to be allowed to deny any transmission in a particular UL subframe if during the number of subframes indicated by autonomousDenialValidity, preceeding and including this particular subframe, it autonomously denied fewer UL subframes than indicated by autonomousDenialSubframes;

2> 	else:

3> 	consider itself not to be allowed to deny any UL transmission;

1> if the received otherConfig includes the powerPrefIndicationConfig:

2> 	if powerPrefIndicationConfig is set to setup:

3> 	consider itself to be configured to provide power preference indications in accordance with 5.6.10;

2> 	else:

3> 	consider itself not to be configured to provide power preference indications;

#### 5.3.10.10	SCG reconfiguration

The UE shall:

1> if the received scg-Configuration is set to release or includes the mobilityControlInfoSCG (i.e. SCG release/ change):

2> 	if mobilityControlInfo is not received (i.e. SCG release/ change without HO):

3> 	reset SCG MAC, if configured;

3> 	for each drb-Identity value that is part of the current UE configuration:

4> 	if the DRB indicated by drb-Identity is an SCG DRB:

5> 	re-establish the PDCP entity and the SCG RLC entity or entities;

4> 	if the DRB indicated by drb-Identity is a split DRB:

5> 	perform PDCP data recovery and re-establish the SCG RLC entity;

4> 	if the DRB indicated by drb-Identity is an MCG DRB; and

4> 	drb-ToAddModListSCG is received and includes the drb-Identity value, while for this entry drb-Type is included and set to scg (i.e. MCG to SCG):

5> 	re-establish the PDCP entity and the MCG RLC entity or entities;

3> 	configure lower layers to consider the SCG SCell(s), except for the PSCell, to be in deactivated state;

1> 	if the received scg-Configuration is set to release:

2> 	release the entire SCG configuration, except for the DRB configuration (i.e. as configured by drb-ToAddModListSCG);

2> 	if the current UE configuration includes one or more split or SCG DRBs and the received RRCConnectionReconfiguration message includes radioResourceConfigDedicated including drb-ToAddModList:

3> 	reconfigure the SCG or split DRB by drb-ToAddModList as specified in 5.3.10.12;

2> 	stop timer T313, if running;

2> 	stop timer T307, if running;

1> 	else:

2> 	if the received scg-ConfigPartMCG includes the scg-Counter:

3> 	update the S-KeNB key based on the KeNB key and using the received scg-Counter value, as specified in MFA TS 33.401 [32];

3> 	derive the KUPenc key associated with the cipheringAlgorithmSCG included in mobilityControlInfoSCG within the received scg-ConfigPartSCG, as specified in MFA TS 33.401 [32];

3> 	configure lower layers to apply the ciphering algorithm and the KUPenc key;

2> 	if the received scg-ConfigPartSCG includes the radioResourceConfigDedicatedSCG:

3> 	reconfigure the dedicated radio resource configuration for the SCG as specified in 5.3.10.11;

2> 	if the current UE configuration includes one or more split or SCG DRBs and the received RRCConnectionReconfiguration message includes radioResourceConfigDedicated including drb-ToAddModList:

3> 	reconfigure the SCG or split DRB by drb-ToAddModList as specified in 5.3.10.12;

2> 	if the received scg-ConfigPartSCG includes the sCellToReleaseListSCG:

3> 	perform SCell release for the SCG as specified in 5.3.10.3a;

2> 	if the received scg-ConfigPartSCG includes the pSCellToAddMod:

3> 	perform PSCell addition or modification as specified in 5.3.10.3c;

NOTE 0:	This procedure is also used to release the PSCell e.g. PSCell change, SI change for the PSCell.

2> 	if the received scg-ConfigPartSCG includes the sCellToAddModListSCG:

3> 	perform SCell addition or modification as specified in 5.3.10.3b;

2> 	configure lower layers in accordance with mobilityControlInfoSCG, if received;

2> 	if the received scg-ConfigPartSCG includes the mobilityControlInfoSCG (i.e. SCG change):

3> 	resume all SCG DRBs and resume SCG transmission for split DRBs, if suspended;

3> 	stop timer T313, if running;

3> 	start timer T307 with the timer value set to t307, as included in the mobilityControlInfoSCG;

3> 	start synchronising to the DL of the target PSCell;

3> 	initiate the random access procedure on the PSCell, as specified in MFA TS 36.321 [6]:

NOTE 1:	The UE is not required to determine the SFN of the target PSCell by acquiring system information from that cell before performing RACH access in the target PSCell.

3> 	the procedure ends, except that the following actions are performed when MAC successfully completes the random access procedure on the PSCell:

4> 	stop timer T307;

4> 	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PSCell, if any;

4> 	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PSCell (e.g. periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PSCell;

NOTE 2:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

#### 5.3.10.11	SCG dedicated resource configuration

The UE shall:

1> 	if the received radioResourceConfigDedicatedSCG includes the drb-ToAddModListSCG:

2> 	for each drb-Identity value included in the drb-ToAddModListSCG perform the DC specific DRB addition or reconfiguration as specified in 5.3.10.3a1

1> 	if the received radioResourceConfigDedicatedSCG includes the mac-MainConfigSCG:

2> 	perform the SCG MAC main reconfiguration as specified in 5.3.10.4;

1> 	if the received radioResourceConfigDedicatedSCG includes the rlf-TimersAndConstantsSCG:

2> 	reconfigure the values of timers and constants as specified in 5.3.10.7;

5.3.10.12	Reconfiguration SCG or split DRB by drb-ToAddModList

The UE shall:

1> 	for each split or SCG DRBs that is part of the current configuration:

2> 	if the corresponding drb-Identity value is included in the received drb-ToAddModList; and:

2> 	if the corresponding drb-Identity value is not included in the received drb-ToAddModListSCG (i.e. reconfigure split, split to MCG or SCG to MCG):

3> 	perform the DC specific DRB addition or reconfiguration as specified in 5.3.10.3a1;

#### 5.3.10.13	Neighbour cell information reconfiguration

The UE shall:

1> 	if the received naics-Info is set to release:

2> 	instruct lower layer to release all the NAICS neighbour cell information for the concerned cell, if previously configured;

1> 	if the received naics-Info includes the neighCellsToReleaseList-r12:

2> 	for each physCellId-r12 value included in the neighCellsToReleaseList-r12 that is part of the current NAICS neighbour cell information of the concerned cell:

3> 	instruct lower layer to release the NAICS neighbour cell information for the concerned cell;

1> 	if the received naics-Info includes the NeighCellsToAddModList-r12:

2> 	for each physCellId-r12 value included in the neighCellsToAddModList-r12 that is not part of the current NAICS neighbour cell information of the concerned cell:

3> 	instruct lower layer to add the NAICS neighbour cell information for the concerned cell;

2> 	for each physCellId-r12 value included in the neighCellsToAddModList-r12 that is part of the current NAICS neighbour cell information of the concerned cell:

3> 	instruct lower layer to modify the NAICS neighbour cell information in accordance with the received NeighCellsInfo for the concerned cell;

#### 5.3.10.14	Void

#### 5.3.10.15	Sidelink dedicated configuration

The UE shall:

1> 	if the RRCConnectionReconfiguration message includes the sl-CommConfig:

2> 	if commTxResources is included and set to setup:

3> 	from the next SC period use the resources indicated by commTxResources for sidelink communication transmission, as specified in 5.10.4;

2> 	else if commTxResources is included and set to release:

3> 	from the next SC period, release the resources allocated for sidelink communication transmission previously configured by commTxResources;

1> 	if the RRCConnectionReconfiguration message includes the sl-DiscConfig:

2> 	if discTxResources is included and set to setup:

3> 	from the next discovery period, as defined by discPeriod, use the resources indicated by discTxResources for sidelink discovery announcement, as specified in 5.10.6;

2> 	else if discTxResources is included and set to release:

3> 	from the next discovery period, as defined by discPeriod, release the resources allocated for sidelink discovery announcement previously configured by discTxResources;

2> 	if discTxResourcesPS is included and set to setup:

3> 	from the next discovery period, as defined by discPeriod, use the resources indicated by discTxResourcesPS for sidelink discovery announcement, as specified in 5.10.6;

2> 	else if discTxResourcesPS is included and set to release:

3> 	from the next discovery period, as defined by discPeriod, release the resources allocated for sidelink discovery announcement previously configured by discTxResourcesPS;

2> 	if discTxInterFreqInfo is included and set to setup:

3> 	from the next discovery period, as defined by discPeriod, use the resources indicated by discTxInterFreqInfo for sidelink discovery announcement, as specified in 5.10.6;

2> 	else if discTxInterFreqInfo is included and set to release:

3> 	from the next discovery period, as defined by discPeriod, release the resources allocated for sidelink discovery announcement previously configured by discTxInterFreqInfo;

2> 	if discRxGapConfig is included and set to setup:

3> 	from the next gap period, as defined by gapPeriod, use the gaps indicated by discRxGapConfig for sidelink discovery monitoring, as specified in 5.10.5;

2> 	else if discRxGapConfig is included and set to release:

3> 	from the next gap period, as defined by gapPeriod, release the gaps configured for sidelink discovery monitoring previously configured by discRxGapConfig;

2> 	if discTxGapConfig is included and set to setup:

3> 	from the next gap period, as defined by gapPeriod, use the gaps indicated by discTxGapConfig for sidelink discovery announcement, as specified in 5.10.6;

2> 	else if discTxGapConfig is included and set to release:

3> 	from the next gap period, as defined by gapPeriod, release the gaps configured for sidelink discovery announcement previously configured by discTxGapConfig;

2> 	if discSysInfoToReportConfig is included and set to setup:

3> 	start timer T370 with the timer value set to 60s;

2> 	else if discSysInfoToReportConfig is included and set to release:

3> 	stop timer T370 and release discSysInfoToReportConfig;

#### 5.3.10.16	T370 expiry

The UE shall:

1> 	if T370 expires:

2> 	release discSysInfoToReportConfig;

### 5.3.11	Radio link failure related actions

#### 5.3.11.1	Detection of physical layer problems in RRC_CONNECTED

The UE shall:

1> 	upon receiving N310 consecutive "out-of-sync" indications for the PCell from lower layers while neither T300, T301, T304 nor T311 is running:

2> 	start timer T310;

1> 	upon receiving N313 consecutive "out-of-sync" indications for the PSCell from lower layers while T307 is not running:

2> 	start T313;

NOTE:	Physical layer monitoring and related autonomous actions do not apply to SCells except for the PSCell.

#### 5.3.11.2	Recovery of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the PCell from lower layers while T310 is running, the UE shall:

1> 	stop timer T310;

1> 	stop timer T312, if running;

NOTE 1:		In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:		Periods in time where neither "in-sync" nor "out-of-sync" is reported by layer 1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

Upon receiving N314 consecutive "in-sync" indications for the PSCell from lower layers while T313 is running, the UE shall:

1> 	stop timer T313;

#### 5.3.11.3	Detection of radio link failure

The UE shall:

1> 	upon T310 expiry; or

1> 	upon T312 expiry; or

1> 	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1> 	upon indication from MCG RLC that the maximum number of retransmissions has been reached for an SRB or for an MCG or split DRB:

2> 	consider radio link failure to be detected for the MCG i.e. RLF;

2> 	except for NB-IoT, store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3> 	clear the information included in VarRLF-Report, if any;

3> 	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3> 	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3> 	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4> 	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4> 	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4> 	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4> 	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4> 	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

3> 	if detailed location information is available, set the content of the locationInfo as follows:

4> 	include the locationCoordinates;

4> 	include the horizontalVelocity, if available;

3> 	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3> 	set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

3> 	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4> 	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5> 	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5> 	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4> 	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

5> 	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5> 	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3> 	if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

4> 	include the drb-EstablishedWithQCI-1;

3> 	set the connectionFailureType to rlf;

3> 	set the c-RNTI to the C-RNTI used in the PCell;

3> 	set the rlf-Cause to the trigger for detecting radio link failure;

2> 	if AS security has not been activated:

3> 	if the UE is a NB-IoT UE:

4> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

3> 	else:

4> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2> 	else:

3> 	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE shall:

1> 	upon T313 expiry; or

1> 	upon random access problem indication from SCG MAC; or

1> 	upon indication from SCG RLC that the maximum number of retransmissions has been reached for an SCG or split DRB:

2> 	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2> 	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected, upon power off or upon detach.

### 5.3.12	UE actions upon leaving RRC_CONNECTED

Upon leaving RRC_CONNECTED, the UE shall:

1> 	reset MAC;

1> 	stop all timers that are running except T320, T325 and T330;

1> 	if leaving RRC_CONNECTED was triggered by suspension of the RRC:

2> 	re-establish RLC entities for all SRBs and DRBs;

2> 	store the UE AS Context including the current RRC configuration, the current security context, the PDCP state including ROHC state, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell;

2> 	store the following information provided by E-UTRAN:

3> 	the resumeIdentity;

2> 	suspend all SRB(s) and DRB(s), except SRB0;

2> 	indicate the suspension of the RRC connection to upper layers;

2>	 configure lower layers to suspend integrity protection and ciphering;

NOTE:	Ciphering is not applied for the subsequent RRCConnectionResume message used to resume the connection. An integrity check is performed by lower layers, but merely upon request from RRC.

1> 	else:

2> 	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established RBs;

2> 	indicate the release of the RRC connection to upper layers together with the release cause;

1> 	if leaving RRC_CONNECTED was triggered neither by reception of the MobilityFromEUTRACommand message nor by selecting an inter-RAT cell while T311 was running:

2> 	if timer T350 is configured:

3> 	start timer T350;

3> 	apply rclwi-Configuration if configured, otherwise apply the wlan-Id-List corresponding to the RPLMN included in SystemInformationBlockType17;

2> 	else:

3> 	release the wlan-OffloadConfigDedicated, if received;

3> 	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

4> 	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

4> 	apply steerToWLAN if configured, otherwise apply the wlan-Id-List corresponding to the RPLMN included in SystemInformationBlockType17;

2> 	enter RRC_IDLE and perform procedures as specified in MFA TS 36.304 [4, Section 5.2.7];

1> 	else:

2> 	release the wlan-OffloadConfigDedicated, if received;

NOTE:	BL UEs or UEs in CE verifies validity of SI when released to RRC_IDLE.

1> 	indicate the release of LWA configuration, if configured, to upper layers;

1> 	release the LWIP configuration, if configured, as described in 5.6.17.3;

### 5.3.13	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for an indicated serving cell the UE shall:

1> 	apply the default physical channel configuration for cqi-ReportConfig for the indicated serving cell as specified in 9.2.4 and release cqi-ReportConfigSCell, for each SCell that sends HARQ feedback on the indicated serving cell, if any;

1> 	apply the default physical channel configuration for schedulingRequestConfig as specified in 9.2.4, for the concerned CG;

Upon receiving an SRS release request from lower layers, for an indicated serving cell the UE shall:

1> 	apply the default physical channel configuration for soundingRS-UL-ConfigDedicated, as specified in 9.2.4;

NOTE:	Upon PUCCH/ SRS release request, the UE does not modify the soundingRS-UL-ConfigDedicatedAperiodic i.e. it does not apply the default for this field (release).

### 5.3.14	Proximity indication

#### 5.3.14.1	General

Figure 5.3.14.1-1: Proximity indication

The purpose of this procedure is to indicate that the UE is entering or leaving the proximity of one or more CSG member cells. The detection of proximity is based on an autonomous search function as defined in MFA TS 36.304 [4].

#### 5.3.14.2	Initiation

A UE in RRC_CONNECTED shall:

1> 	if the UE enters the proximity of one or more CSG member cell(s) on an E-UTRA frequency while proximity indication is enabled for such E-UTRA cells; or

1> 	if the UE enters the proximity of one or more CSG member cell(s) on an UTRA frequency while proximity indication is enabled for such UTRA cells; or

1> 	if the UE leaves the proximity of all CSG member cell(s) on an E-UTRA frequency while proximity indication is enabled for such E-UTRA cells; or

1> 	if the UE leaves the proximity of all CSG member cell(s) on an UTRA frequency while proximity indication is enabled for such UTRA cells:

2> 	if the UE has previously not transmitted a ProximityIndication for the RAT and frequency during the current RRC connection, or if more than 5 s has elapsed since the UE has last transmitted a ProximityIndication (either entering or leaving) for the RAT and frequency:

3> 	initiate transmission of the ProximityIndication message in accordance with 5.3.14.3;

NOTE:	In the conditions above, "if the UE enters the proximity of one or more CSG member cell(s)" includes the case of already being in the proximity of such cell(s) at the time proximity indication for the corresponding RAT is enabled.

#### 5.3.14.3	Actions related to transmission of ProximityIndication message

The UE shall set the contents of ProximityIndication message as follows:

1> 	if the UE applies the procedure to report entering the proximity of CSG member cell(s):

2> 	set type to entering;

1> 	else if the UE applies the procedure to report leaving the proximity of CSG member cell(s):

2> 	set type to leaving;

1> 	if the proximity indication was triggered for one or more CSG member cell(s) on an E-UTRA frequency:

2> 	set the carrierFreq to eutra with the value set to the E-ARFCN value of the E-UTRA cell(s) for which proximity indication was triggered;

1> 	else if the proximity indication was triggered for one or more CSG member cell(s) on a UTRA frequency:

2> 	set the carrierFreq to utra with the value set to the ARFCN value of the UTRA cell(s) for which proximity indication was triggered;

The UE shall submit the ProximityIndication message to lower layers for transmission.

### 5.3.15	Void

## 5.4	Inter-RAT mobility

### 5.4.1	Introduction

The general principles of connected mode mobility are described in 5.3.1.3. The general principles of the security handling upon connected mode mobility are described in 5.3.1.2.

For the (network controlled) inter RAT mobility from E-UTRA for a UE in RRC_CONNECTED, a single procedure is defined that supports both handover, cell change order with optional network assistance (NACC) and enhanced CS fallback to CDMA2000 1xRTT. In case of mobility to CDMA2000, the eNB decides when to move to the other RAT while the target RAT determines to which cell the UE shall move.

### 5.4.2	Handover to E-UTRA

#### 5.4.2.1	General

Figure 5.4.2.1-1: Handover to E-UTRA, successful

The purpose of this procedure is to, under the control of the network, transfer a connection between the UE and another Radio Access Network (e.g. GERAN or UTRAN) to E-UTRAN.

The handover to E-UTRA procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from UTRAN to E-UTRAN applies only after integrity has been activated in UTRAN.

#### 5.4.2.2	Initiation

The RAN using another RAT initiates the handover to E-UTRA procedure, in accordance with the specifications applicable for the other RAT, by sending the RRCConnectionReconfiguration message via the radio access technology from which the inter-RAT handover is performed.

E-UTRAN applies the procedure as follows:

-	to activate ciphering, possibly using NULL algorithm, if not yet activated in the other RAT;

-	to establish SRB1, SRB2 and one or more DRBs, i.e. at least the DRB associated with the default EPS bearer is established;

#### 5.4.2.3	Reception of the RRCConnectionReconfiguration by the UE

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

1> 	apply the default physical channel configuration as specified in 9.2.4;

1> 	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1> 	apply the default MAC main configuration as specified in 9.2.2;

1> 	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1> 	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1> 	start synchronising to the DL of the target PCell;

1> 	set the C-RNTI to the value of the newUE-Identity;

1> 	for the target PCell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1> 	for the target PCell, apply the uplink bandwidth indicated by (the absence or presence of) the ul-Bandwidth;

1> 	configure lower layers in accordance with the received radioResourceConfigCommon;

1> 	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1> 	perform the radio resource configuration procedure as specified in 5.3.10;

1> 	forward the nas-SecurityParamToEUTRA to the upper layers;

1> 	derive the KeNB key, as specified in MFA TS 33.401 [32];

1> 	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in MFA TS 33.401 [32];

1> 	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in MFA TS 33.401 [32];

1> 	configure lower layers to apply the indicated integrity protection algorithm and the KRRCint key immediately, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1> 	configure lower layers to apply the indicated ciphering algorithm, the KRRCenc key and the KUPenc key immediately, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1> 	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2> 	perform SCell addition as specified in 5.3.10.3b;

1> 	if the RRCConnectionReconfiguration message includes the measConfig:

2> 	perform the measurement configuration procedure as specified in 5.5.2;

1> 	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1> 	if the RRCConnectionReconfiguration message includes the otherConfig:

2> 	perform the other configuration procedure as specified in 5.3.10.9;

1> 	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2> 	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1> 	if the RRCConnectionReconfiguration message includes rclwi-Configuration:

2> 	perform the WLAN traffic steering command procedure as specified in 5.6.16.2;

1> 	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2> 	perform the LWA configuration procedure as specified in 5.6.14.2;

1> 	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2> 	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

1> 	set the content of RRCConnectionReconfigurationComplete message as follows:

2> 	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3> 	include rlf-InfoAvailable;

2> 	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and if T330 is not running:

3> 	include logMeasAvailableMBSFN;

2> 	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3> 	include the logMeasAvailable;

2> 	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3> 	include connEstFailInfoAvailable;

1> 	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1> 	if the RRCConnectionReconfiguration message does not include rlf-TimersAndConstants set to setup:

2> 	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1> 	if MAC successfully completes the random access procedure:

2> 	stop timer T304;

2> 	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2> 	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 1:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2> 	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

NOTE 2:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

#### 5.4.2.4	Reconfiguration failure

The UE shall:

1> 	if the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message:

2> 	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

NOTE 1:	The UE may apply above failure handling also in case the RRCConnectionReconfiguration message causes a protocol error for which the generic error handling as defined in 5.7 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.

#### 5.4.2.5	T304 expiry (handover to E-UTRA failure)

The UE shall:

1> 	upon T304 expiry (handover to E-UTRA failure):

2> 	reset MAC;

2> 	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

### 5.4.3	Mobility from E-UTRA

#### 5.4.3.1	General

Figure 5.4.3.1-1: Mobility from E-UTRA, successful

Figure 5.4.3.1-2: Mobility from E-UTRA, failure

The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using another Radio Access Technology (RAT), e.g. GERAN, UTRA or CDMA2000 systems. The mobility from E-UTRA procedure covers the following type of mobility:

-	handover, i.e. the MobilityFromEUTRACommand message includes radio resources that have been allocated for the UE in the target cell;

-	cell change order, i.e. the MobilityFromEUTRACommand message may include information facilitating access of and/ or connection establishment in the target cell, e.g. system information. Cell change order is applicable only to GERAN; and

-	enhanced CS fallback to CDMA2000 1xRTT, i.e. the MobilityFromEUTRACommand message includes radio resources that have been allocated for the UE in the target cell. The enhanced CS fallback to CDMA2000 1xRTT may be combined with concurrent handover or redirection to CDMA2000 HRPD.

NOTE:	For the case of dual receiver/transmitter enhanced CS fallback to CDMA2000 1xRTT, the DLInformationTransfer message is used instead of the MobilityFromEUTRACommand message (see MFA TS 36.300 [9]).

#### 5.4.3.2	Initiation

E-UTRAN initiates the mobility from E-UTRA procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport message or in response to reception of CS fallback indication for the UE from MME, by sending a MobilityFromEUTRACommand message. E-UTRAN applies the procedure as follows:

-	the procedure is initiated only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended;

#### 5.4.3.3	Reception of the MobilityFromEUTRACommand by the UE

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1> 	stop timer T310, if running;

1> 	stop timer T312, if running;

1> 	if the MobilityFromEUTRACommand message includes the purpose set to handover:

2> 	if the targetRAT-Type is set to utra or geran:

3> 	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3> 	forward the nas-SecurityParamFromEUTRA to the upper layers;

3> 	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

3>	if the targetRAT-Type is set to geran:

4>	use the contents of systemInformation, if provided for PS Handover, as the system information to begin access on the target GERAN cell;

NOTE 1:	If there are DRBs for which no radio bearers are established in the target RAT as indicated in the targetRAT-MessageContainer in the message, the E-UTRA RRC part of the UE does not indicate the release of the concerned DRBs to the upper layers. Upper layers may derive which bearers are not established from information received from the AS of the target RAT.

NOTE 2:	In case of SR-VCC, the DRB to be replaced is specified in 3GPP TS 23.216 [61].

2> 	else if the targetRAT-Type is set to cdma2000-1XRTT or cdma2000-HRPD:

3> 	forward the targetRAT-Type and the targetRAT-MessageContainer to the CDMA2000 upper layers for the UE to access the cell(s) indicated in the inter-RAT message in accordance with the specifications of the CDMA2000 target-RAT;

1> 	else if the MobilityFromEUTRACommand message includes the purpose set to cellChangeOrder:

2> 	start timer T304 with the timer value set to t304, as included in the MobilityFromEUTRACommand message;

2> 	if the targetRAT-Type is set to geran:

3> 	if networkControlOrder is included in the MobilityFromEUTRACommand message:

4> 	apply the value as specified in 3GPP TS 44.060 [36];

3> 	else:

4> 	acquire networkControlOrder and apply the value as specified in 3GPP TS 44.060 [36];

3> 	use the contents of systemInformation, if provided, as the system information to begin access on the target GERAN cell;

2> 	establish the connection to the target cell indicated in the CellChangeOrder;

NOTE 3:	The criteria for success or failure of the cell change order to GERAN are specified in 3GPP TS 44.060 [36].

1> 	if the MobilityFromEUTRACommand message includes the purpose set to e-CSFB:

2> 	if messageContCDMA2000-1XRTT is present:

3> 	forward the messageContCDMA2000-1XRTT to the CDMA2000 upper layers for the UE to access the cell(s) indicated in the inter-RAT message in accordance with the specification of the target RAT;

2> 	if mobilityCDMA2000-HRPD is present and is set to handover:

3> 	forward the messageContCDMA2000-HRPD to the CDMA2000 upper layers for the UE to access the cell(s) indicated in the inter-RAT message in accordance with the specification of the target RAT;

2> 	if mobilityCDMA2000-HRPD is present and is set to redirection:

3> 	forward the redirectCarrierCDMA2000-HRPD to the CDMA2000 upper layers;

NOTE 4:	When the CDMA2000 upper layers in the UE receive both the messageContCDMA2000-1XRTT and messageContCDMA2000-HRPD the UE performs concurrent access to both CDMA2000 1xRTT and CDMA2000 HRPD RAT.

NOTE 5:	The UE should perform the handover, the cell change order or enhanced 1xRTT CS fallback as soon as possible following the reception of the RRC message MobilityFromEUTRACommand, which could be before confirming successful reception (HARQ and ARQ) of this message.

#### 5.4.3.4	Successful completion of the mobility from E-UTRA

Upon successfully completing the handover, the cell change order or enhanced 1xRTT CS fallback, the UE shall:

1> 	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

NOTE:	If the UE performs enhanced 1xRTT CS fallback along with concurrent mobility to CDMA2000 HRPD and the connection to either CDMA2000 1xRTT or CDMA2000 HRPD succeeds, then the mobility from E-UTRA is considered successful.

#### 5.4.3.5	Mobility from E-UTRA failure

The UE shall:

1> 	if T304 expires (mobility from E-UTRA failure); or

1> 	if the UE does not succeed in establishing the connection to the target radio access technology; or

1> 	if the UE is unable to comply with (part of) the configuration included in the MobilityFromEUTRACommand message; or

1> 	if there is a protocol error in the inter RAT information included in the MobilityFromEUTRACommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT:

2> 	stop T304, if running;

2> 	if the cs-FallbackIndicator in the MobilityFromEUTRACommand message was set to TRUE or e-CSFB was present:

3> 	 indicate to upper layers that the CS fallback procedure has failed;

2> 	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, mac-MainConfig and sps-Config;

2> 	initiate the connection re-establishment procedure as specified in 5.3.7;

NOTE:	For enhanced CS fallback to CDMA2000 1xRTT, the above UE behavior applies only when the UE is attempting the enhanced 1xRTT CS fallback and connection to the target radio access technology fails or if the UE is attempting enhanced 1xRTT CS fallback along with concurrent mobility to CDMA2000 HRPD and connection to both the target radio access technologies fails.

### 5.4.4	Handover from E-UTRA preparation request (CDMA2000)

#### 5.4.4.1	General

Figure 5.4.4.1-1: Handover from E-UTRA preparation request

The purpose of this procedure is to trigger the UE to prepare for handover or enhanced 1xRTT CS fallback to CDMA2000 by requesting a connection with this network. The UE may use this procedure to concurrently prepare for handover to CDMA2000 HRPD along with preparation for enhanced CS fallback to CDMA2000 1xRTT. This procedure applies to CDMA2000 capable UEs only.

This procedure is also used to trigger the UE which supports dual Rx/Tx enhanced 1xCSFB to redirect its second radio to CDMA2000 1xRTT.

The handover from E-UTRA preparation request procedure applies when signalling radio bearers are established.

#### 5.4.4.2	Initiation

E-UTRAN initiates the handover from E-UTRA preparation request procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport message or CS fallback indication for the UE, by sending a HandoverFromEUTRAPreparationRequest message. E-UTRA initiates the procedure only when AS security has been activated.

#### 5.4.4.3	Reception of the HandoverFromEUTRAPreparationRequest by the UE

Upon reception of the HandoverFromEUTRAPreparationRequest message, the UE shall:

1> 	if dualRxTxRedirectIndicator is present in the received message:

2> 	forward dualRxTxRedirectIndicator to the CDMA2000 upper layers;

2> 	forward redirectCarrierCDMA2000-1XRTT to the CDMA2000 upper layers, if included;

1> 	else:

2> 	indicate the request to prepare handover or enhanced 1xRTT CS fallback and forward the cdma2000-Type to the CDMA2000 upper layers;

2> 	if cdma2000-Type is set to type1XRTT:

3> 	forward the rand and the mobilityParameters to the CDMA2000 upper layers;

2> 	if concurrPrepCDMA2000-HRPD is present in the received message:

3> 	forward concurrPrepCDMA2000-HRPD to the CDMA2000 upper layers;

2> 	else:

3> 	forward concurrPrepCDMA2000-HRPD, with its value set to FALSE, to the CDMA2000 upper layers;

### 5.4.5	UL handover preparation transfer (CDMA2000)

#### 5.4.5.1	General

Figure 5.4.5.1-1: UL handover preparation transfer

The purpose of this procedure is to tunnel the handover related CDMA2000 dedicated information or enhanced 1xRTT CS fallback related CDMA2000 dedicated information from UE to E-UTRAN when requested by the higher layers. The procedure is triggered by the higher layers on receipt of HandoverFromEUTRAPreparationRequest message. If preparing for enhanced CS fallback to CDMA2000 1xRTT and handover to CDMA2000 HRPD, the UE sends two consecutive ULHandoverPreparationTransfer messages to E-UTRAN, one per addressed CDMA2000 RAT Type. This procedure applies to CDMA2000 capable UEs only.

#### 5.4.5.2	Initiation

A UE in RRC_CONNECTED initiates the UL handover preparation transfer procedure whenever there is a need to transfer handover or enhanced 1xRTT CS fallback related non-3GPP dedicated information. The UE initiates the UL handover preparation transfer procedure by sending the ULHandoverPreparationTransfer message.

5.4.5.3	Actions related to transmission of the ULHandoverPreparationTransfer message

The UE shall set the contents of the ULHandoverPreparationTransfer message as follows:

1> 	include the cdma2000-Type and the dedicatedInfo;

1> 	if the cdma2000-Type is set to type1XRTT:

2> 	include the meid and set it to the value received from the CDMA2000 upper layers;

1> 		submit the ULHandoverPreparationTransfer message to lower layers for transmission, upon which the procedure ends;

#### 5.4.5.4	Failure to deliver the ULHandoverPreparationTransfer message

The UE shall:

1> 	if the UE is unable to guarantee successful delivery of ULHandoverPreparationTransfer messages:

2> 	inform upper layers about the possible failure to deliver the information contained in the concerned ULHandoverPreparationTransfer message;

### 5.4.6	Inter-RAT cell change order to E-UTRAN

#### 5.4.6.1	General

The purpose of the inter-RAT cell change order to E-UTRAN procedure is to transfer, under the control of the source radio access technology, a connection between the UE and another radio access technology (e.g. GSM/ GPRS) to E-UTRAN.

#### 5.4.6.2	Initiation

The procedure is initiated when a radio access technology other than E-UTRAN, e.g. GSM/GPRS, using procedures specific for that RAT, orders the UE to change to an E-UTRAN cell. In response, upper layers request the establishment of an RRC connection as specified in subclause 5.3.3.

NOTE:	Within the message used to order the UE to change to an E-UTRAN cell, the source RAT should specify the identity of the target E-UTRAN cell as specified in the specifications for that RAT.

The UE shall:

1> 	upon receiving an RRCConnectionSetup message:

2> 	consider the inter-RAT cell change order procedure to have completed successfully;

#### 5.4.6.3	UE fails to complete an inter-RAT cell change order

If the inter-RAT cell change order fails the UE shall return to the other radio access technology and proceed as specified in the appropriate specifications for that RAT.

The UE shall:

1> 	upon failure to establish the RRC connection as specified in subclause 5.3.3:

2> 	consider the inter-RAT cell change order procedure to have failed;

NOTE:	The cell change was network ordered. Therefore, failure to change to the target PCell should not cause the UE to move to UE-controlled cell selection.

## 5.5	Measurements

### 5.5.1	Introduction

The UE reports measurement information in accordance with the measurement configuration as provided by E-UTRAN. E-UTRAN provides the measurement configuration applicable for a UE in RRC_CONNECTED by means of dedicated signalling, i.e. using the RRCConnectionReconfiguration or RRCConnectionResume message.

For MF, E-UTRAN provides measurement configurations also via system information, which may be applicable for a UE in RRC_CONNECTED and a UE in RRC_IDLE as described in Section 5.5.2.10MF1. UEs in RRC_IDLE mode do not report any measurement information to E-UTRAN.

The UE can be requested to perform the following types of measurements:

-	Intra-frequency measurements: measurements at the downlink carrier frequency(ies) of the serving cell(s).

-	Inter-frequency measurements: measurements at frequencies that differ from any of the downlink carrier frequency(ies) of the serving cell(s).

-	Inter-RAT measurements of UTRA frequencies.

-	Inter-RAT measurements of GERAN frequencies.

-	Inter-RAT measurements of CDMA2000 HRPD or CDMA2000 1xRTT or WLAN frequencies.

The measurement configuration includes the following parameters:

1.	Measurement objects: The objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object is a single E-UTRA carrier frequency. Associated with this carrier frequency, E-UTRAN can configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not considered in event evaluation or measurement reporting.

-	For inter-RAT UTRA measurements a measurement object is a set of cells on a single UTRA carrier frequency.

-	For inter-RAT GERAN measurements a measurement object is a set of GERAN carrier frequencies.

-	For inter-RAT CDMA2000 measurements a measurement object is a set of cells on a single (HRPD or 1xRTT) carrier frequency.

-	For inter-RAT WLAN measurements a measurement object is a set of WLAN identifiers and optionally a set of WLAN frequencies.

NOTE 1:	Some measurements using the above mentioned measurement objects, only concern a single cell, e.g. measurements used to report neighbouring cell system information, PCell UE Rx-Tx time difference, or a pair of cells, e.g. SSTD measurements between the PCell and the PSCell.

2.	Reporting configurations: A list of reporting configurations where each reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	Reporting format: The quantities that the UE includes in the measurement report and associated information (e.g. number of cells to report).

3.	Measurement identities: A list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is used as a reference number in the measurement report.

4.	Quantity configurations: One quantity configuration is configured per RAT type. The quantity configuration defines the measurement quantities and associated filtering used for all event evaluation and related reporting of that measurement type. One filter can be configured per measurement quantity.

5.	Measurement gaps: Periods that the UE may use to perform measurements, i.e. no (UL, DL) transmissions are scheduled.

E-UTRAN only configures a single measurement object for a given frequency (except for WLAN), i.e. it is not possible to configure two or more measurement objects for the same frequency with different associated parameters, e.g. different offsets and/ or blacklists. E-UTRAN may configure multiple instances of the same event e.g. by configuring two reporting configurations with different thresholds.

The UE maintains a single measurement object list, a single reporting configuration list, and a single measurement identities list. The measurement object list includes measurement objects, that are specified per RAT type, possibly including intra-frequency object(s) (i.e. the object(s) corresponding to the serving frequency(ies)), inter-frequency object(s) and inter-RAT objects. Similarly, the reporting configuration list includes E-UTRA and inter-RAT reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The serving cell(s) - these are the PCell and one or more SCells, if configured for a UE supporting CA.

2.	Listed cells - these are cells listed within the measurement object(s) or, for inter-RAT WLAN, the WLANs matching the WLAN identifiers configured in the measurement object or the WLAN the UE is connected to.

3.	Detected cells - these are cells that are not listed within the measurement object(s) but are detected by the UE on the carrier frequency(ies) indicated by the measurement object(s).

For E-UTRA, the UE measures and reports on the serving cell(s), listed cells, detected cells and, for RSSI and channel occupancy measurements, the UE measures and reports on any reception on the indicated frequency. For inter-RAT UTRA, the UE measures and reports on listed cells and optionally on cells that are within a range for which reporting is allowed by E-UTRAN. For inter-RAT GERAN, the UE measures and reports on detected cells. For inter-RAT CDMA2000, the UE measures and reports on listed cells. For inter-RAT WLAN, the UE measures and reports on listed cells.

NOTE 2:	For inter-RAT UTRA and CDMA2000, the UE measures and reports also on detected cells for the purpose of SON.

NOTE 3:	This specification is based on the assumption that typically CSG cells of home deployment type are not indicated within the neighbour list. Furthermore, the assumption is that for non-home deployments, the physical cell identity is unique within the area of a large macro cell (i.e. as for UTRAN).

Whenever the procedural specification, other than contained in sub-clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

### 5.5.2	Measurement configuration

#### 5.5.2.1	General

E-UTRAN applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig, it includes a measObject for each serving frequency;

-	to configure at most one measurement identity using a reporting configuration with the purpose set to reportCGI;

-	for serving frequencies, set the EARFCN within the corresponding measObject according to the band as used for reception/ transmission;

-	to configure at most one measurement identity using a reporting configuration with ul-DelayConfig;

The UE shall:

1> 	if the received measConfig includes the measObjectToRemoveList:

2> 	perform the measurement object removal procedure as specified in 5.5.2.4;

1> 	if the received measConfig includes the measObjectToAddModList:

2> 	perform the measurement object addition/modification procedure as specified in 5.5.2.5;

1> 	if the received measConfig includes the reportConfigToRemoveList:

2> 	perform the reporting configuration removal procedure as specified in 5.5.2.6;

1> 	if the received measConfig includes the reportConfigToAddModList:

2> 	perform the reporting configuration addition/modification procedure as specified in 5.5.2.7;

1> 	if the received measConfig includes the quantityConfig:

2> 	perform the quantity configuration procedure as specified in 5.5.2.8;

1> 	if the received measConfig includes the measIdToRemoveList:

2> 	perform the measurement identity removal procedure as specified in 5.5.2.2;

1> 	if the received measConfig includes the measIdToAddModList:

2> 	perform the measurement identity addition/modification procedure as specified in 5.5.2.3;

1> 	if the received measConfig includes the measGapConfig:

2> 	perform the measurement gap configuration procedure as specified in 5.5.2.9;

1> 	if the received measConfig includes the s-Measure:

2> 	set the parameter s-Measure within VarMeasConfig to the lowest value of the RSRP ranges indicated by the received value of s-Measure;

1> 	if the received measConfig includes the preRegistrationInfoHRPD:

2> 	forward the preRegistrationInfoHRPD to CDMA2000 upper layers;

1> 	if the received measConfig includes the speedStatePars:

2> 	set the parameter speedStatePars within VarMeasConfig to the received value of speedStatePars;

1> 	if the received measConfig includes the allowInterruptions:

2> 	set the parameter allowInterruptions within VarMeasConfig to the received value of allowInterruptions;

#### 5.5.2.2	Measurement identity removal

The UE shall:

1> 	for each measId included in the received measIdToRemoveList that is part of the current UE configuration in VarMeasConfig:

2> 	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2> 	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2> 	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the measIdToRemoveList includes any measId value that is not part of the current UE configuration.

#### 5.5.2.2a	Measurement identity autonomous removal

The UE shall:

1> 	for each measId included in the measIdList within VarMeasConfig:

2> 	if the associated reportConfig concerns an event involving a serving cell while the concerned serving cell is not configured; or

2> 	 if the associated reportConfig concerns an event involving a WLAN mobility set while the concerned WLAN mobility set is not configured:

3> 	remove the measId from the measIdList within the VarMeasConfig;

3> 	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3> 	stop the periodical reporting timer if running, and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE 1:	The above UE autonomous removal of measId's applies only for measurement events A1, A2, A6, and also applies for events A3 and A5 if configured for PSCell and W2 and W3, if configured.

NOTE 2:	When performed during re-establishment, the UE is only configured with a primary frequency (i.e. the SCell(s) and WLAN mobility set are released, if configured).

#### 5.5.2.3	Measurement identity addition/modification

E-UTRAN applies the procedure as follows:

-	configure a measId only if the corresponding measurement object, the corresponding reporting configuration and the corresponding quantity configuration, are configured;

The UE shall:

1> 	for each measId included in the received measIdToAddModList:

2> 	if an entry with the matching measId exists in the measIdList within the VarMeasConfig:

3> 	replace the entry with the value received for this measId;

2> 	else:

3> 	add a new entry for this measId within the VarMeasConfig;

2> 	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2> 	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

2> 	if the triggerType is set to periodical and the purpose is set to reportCGI in the reportConfig associated with this measId:

3> 	if the measObject associated with this measId concerns E-UTRA:

4> 	if the si-RequestForHO is included in the reportConfig associated with this measId:

5> 	if the UE is a category 0 UE according to MFA TS 36.306 [5]:

6> 	start timer T321 with the timer value set to 190 ms for this measId;

5> 	else:

6> 	start timer T321 with the timer value set to 150 ms for this measId;

4> 	else:

5> 	start timer T321 with the timer value set to 1 second for this measId;

3> 	else if the measObject associated with this measId concerns UTRA:

4> 	if the si-RequestForHO is included in the reportConfig associated with this measId:

5> 	for UTRA FDD, start timer T321 with the timer value set to 2 seconds for this measId;

5> 	for UTRA TDD, start timer T321 with the timer value set to [1 second] for this measId;

4> 	else:

5> 	start timer T321 with the timer value set to 8 seconds for this measId;

3> 	else:

4> 	start timer T321 with the timer value set to 8 seconds for this measId;

#### 5.5.2.4	Measurement object removal

The UE shall:

1> 	for each measObjectId included in the received measObjectToRemoveList that is part of the current UE configuration in VarMeasConfig:

2> 	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2> 	remove all measId associated with this measObjectId from the measIdList within the VarMeasConfig, if any;

2> 	if a measId is removed from the measIdList:

3> 	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3> 	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the measObjectToRemoveList includes any measObjectId value that is not part of the current UE configuration.

#### 5.5.2.5	Measurement object addition/modification

The UE shall:

1> 	for each measObjectId included in the received measObjectToAddModList:

2> 	if an entry with the matching measObjectId exists  or - for MF - is added in the same message in the measObjectList within the VarMeasConfig, for this entry:

3> 	reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, blackCellsToAddModList, whiteCellsToAddModList, altTTT-CellsToAddModList, cellsToRemoveList, blackCellsToRemoveList, whiteCellsToRemoveList, altTTT-CellsToRemoveList, measSubframePatternConfigNeigh, measDS-Config, wlan-ToAddModList and wlan-ToRemoveList;

3> 	if the received measObject includes the cellsToRemoveList:

4> 	for each cellIndex included in the cellsToRemoveList:

5> 	remove the entry with the matching cellIndex from the cellsToAddModList;

3> 	if the received measObject includes the cellsToAddModList:

4> 	for each cellIndex value included in the cellsToAddModList:

5> 	if an entry with the matching cellIndex exists in the cellsToAddModList:

6> 	replace the entry with the value received for this cellIndex;

5> 	else:

6> 	add a new entry for the received cellIndex to the cellsToAddModList;

3> 	if the received measObject includes the blackCellsToRemoveList:

4> 	for each cellIndex included in the blackCellsToRemoveList:

5> 	remove the entry with the matching cellIndex from the blackCellsToAddModList;

NOTE 1:	For each cellIndex included in the blackCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the black list of cells only if all cell indexes containing it are removed.

3> 	if the received measObject includes the blackCellsToAddModList:

4> 	for each cellIndex included in the blackCellsToAddModList:

5> 	if an entry with the matching cellIndex is included in the blackCellsToAddModList:

6> 	replace the entry with the value received for this cellIndex;

5> 	else:

6> 	add a new entry for the received cellIndex to the blackCellsToAddModList;

3> 	if the received measObject includes the whiteCellsToRemoveList:

4> 	for each cellIndex included in the whiteCellsToRemoveList:

5> 	remove the entry with the matching cellIndex from the whiteCellsToAddModList;

NOTE 2:	For each cellIndex included in the whiteCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the white list of cells only if all cell indexes containing it are removed.

3> 	if the received measObject includes the whiteCellsToAddModList:

4> 	for each cellIndex included in the whiteCellsToAddModList:

5> 	if an entry with the matching cellIndex is included in the whiteCellsToAddModList:

6> 	replace the entry with the value received for this cellIndex;

5> 	else:

6> 	add a new entry for the received cellIndex to the whiteCellsToAddModList;

3> 	if the received measObject includes the altTTT-CellsToRemoveList:

4> 	for each cellIndex included in the altTTT-CellsToRemoveList:

5> 	remove the entry with the matching cellIndex from the altTTT-CellsToAddModList;

NOTE 3:	For each cellIndex included in the altTTT-CellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the list of cells only if all cell indexes containing it are removed.

3> 	if the received measObject includes the altTTT-CellsToAddModList:

4> 	for each cellIndex value included in the altTTT-CellsToAddModList:

5> 	if an entry with the matching cellIndex exists in the altTTT-CellsToAddModList:

6> 	replace the entry with the value received for this cellIndex;

5> 	else:

6> 	add a new entry for the received cellIndex to the altTTT-CellsToAddModList;

3> 	if the received measObject includes measSubframePatternConfigNeigh:

4> 	set measSubframePatternConfigNeigh within the VarMeasConfig to the value of the received field

3>	if the received measObject includes neighCellDMTC:

4>	if neighCellDMTC includes measDS-ConfigSync:

5>	handle measDS-ConfigSync the same way as measDS-Config, for the measDS-ConfigSync field in VarMeasConfig

4>	if asyncNeighCells in neighCellDMTC is set to true:

5>	set the asyncNeighCells in VarMeasConfig to true, and

5>	if neighCellDMTC includes measDS-ConfigAsync

6>	and handle measDS-ConfigAsync the same way as measDS-Config, for the measDS-ConfigAsync field in VarMeasConfig

5>	otherwise

6>	if measDS-ConfigAsync is not present (or is marked as released) in VarMeasConfig:

7>	configure measDS-ConfigAsync as if it was present and set to setup, with all other setup fields (dmtc-Periodicity, dmtc-Offset, dmtc-WindowSize, measCSI-RS-ToRemoveList, measCSI-RS-ToAddModList) absent;

4>	if neighCellDMTC includes asyncNeighCells is set to false:

5>		set the asyncNeighCells in VarMeasConfig to false, and

5>		handle measDS-ConfigAsync as if it was present and set to release;

NOTE MF1:	The asyncNeighCells field in VarMeasConfig indicates whether there is DMTC configuration for unsynchronized neighbor cells and set to setup.

3> 	if the received measObject includes measDS-Config:

4> 	if measDS-Config is set to setup:

5> 	if the received measDS-Config includes the measCSI-RS-ToRemoveList:

6> 	for each measCSI-RS-Id included in the measCSI-RS-ToRemoveList:

7> 	remove the entry with the matching measCSI-RS-Id from the measCSI-RS-ToAddModList;

5> 	if the received measDS-Config includes the measCSI-RS-ToAddModList, for each measCSI-RS-Id value included in the measCSI-RS-ToAddModList:

6> 	if an entry with the matching measCSI-RS-Id exists in the measCSI-RS-ToAddModList:

7> 	replace the entry with the value received for this measCSI-RS-Id;

6> 	else:

7> 	add a new entry for the received measCSI-RS-Id to the measCSI-RS-ToAddModList;

5> 	set other fields of the measDS-Config within the VarMeasConfig to the value of the received fields;

5> 	perform the discovery signals measurement timing configuration procedure as specified in 5.5.2.10;

4> 	else:

5> 	release the discovery signals measurement configuration;

3> 	for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any:

4> 	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4> 	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

3> 	if the received measObject includes the wlan-ToRemoveList:

4> 	for each WLAN-Identifiers included in the wlan-ToRemoveList:

5> 	remove the entry with the matching WLAN-Identifiers from the wlan-ToAddModList;

NOTE 3a:	Matching of WLAN-Identifiers requires that all WLAN identifier fields should be same.

3> 	if the received measObject includes the wlan-ToAddModList:

4> 	for each WLAN-Identifiers included in the wlan-ToAddModList:

5> 	add a new entry for the received WLAN-Identifiers to the wlan-ToAddModList;

2> 	else:

3> 	add a new entry for the received measObject to the measObjectList within VarMeasConfig;

NOTE 4:	UE does not need to retain cellForWhichToReportCGI in the measObject after reporting cgi-Info.

#### 5.5.2.6	Reporting configuration removal

The UE shall:

1> 	for each reportConfigId included in the received reportConfigToRemoveList that is part of the current UE configuration in VarMeasConfig:

2> 	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2> 	remove all measId associated with the reportConfigId from the measIdList within the VarMeasConfig, if any;

2> 	if a measId is removed from the measIdList:

3> 	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3> 	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the reportConfigToRemoveList includes any reportConfigId value that is not part of the current UE configuration.

#### 5.5.2.7	Reporting configuration addition/modification

The UE shall:

1> 	for each reportConfigId included in the received reportConfigToAddModList:

2> 	if an entry with the matching reportConfigId exists in the reportConfigList within the VarMeasConfig, for this entry:

3> 	reconfigure the entry with the value received for this reportConfig;

3> 	for each measId associated with this reportConfigId included in the measIdList within the VarMeasConfig, if any:

4> 	remove the measurement reporting entry for this measId from in VarMeasReportList, if included;

4> 	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

2> 	else:

3> 	add a new entry for the received reportConfig to the reportConfigList within the VarMeasConfig;

#### 5.5.2.8	Quantity configuration

The UE shall:

1> 	for each RAT for which the received quantityConfig includes parameter(s):

2> 	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1> 	for each measId included in the measIdList within VarMeasConfig:

2> 		remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2> 	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1> 	if measGapConfig is set to setup:

2> 	if a measurement gap configuration is already setup, release the measurement gap configuration;

2>		for 3GPP:

3> setup the measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition (SFN and subframe of MCG cells):

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in MFA TS 36.133 [16];

2> for MF:

3> setup the measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, gapShiftFactor, and gapLength, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition (SFN and subframe of MCG cells):

(SFN*10 + subframe) mod (MGRP + gapShiftFactor*(gapLength-1)) = gapOffset

NOTE:	The UE applies a single gap, which timing is relative to the MCG cells, even when configured with DC.

1> 	else:

2> 	release the measurement gap configuration;

#### 5.5.2.10	Discovery signals measurement timing configuration

The UE shall setup the discovery signals measurement timing configuration (DMTC) in accordance with the received dmtc-PeriodOffset, i.e., the first subframe of each DMTC occasion occurs at an SFN and subframe of the PCell meeting the following condition:

SFN mod T = FLOOR(dmtc-Offset/10);

subframe = dmtc-Offset mod 10;

with T = dmtc-Periodicity/10;

On the concerned frequency, the UE shall not consider discovery signals transmission in subframes outside the DMTC occasion.

#### 5.5.2.10MF1	Discovery signals measurement timing configuration for MF

A DMTC occasion is defined by the following DMTC paramters: dmtc-Offset, dmtc-Periodicity, and dmtc-WindowSize. The UE may be provided with a full set or a subset of the DMTC parameters.

DMTC configurations are not only used by UEs in RRC_CONNECTED, but also UEs in RRC_IDLE use the DMTC configurations to perform idle mode measurements for cell reselection as specified in MFA TS 36.304 [4].

If a DMTC configuration, i.e. MeasDS-Config, is provided in SIB-MF1, it applies to both UEs in RRC_IDLE and to UEs in RRC_CONNECTED. DMTC configurations provided as part of any other system information are only valid for UEs in RRC_IDLE, while DMTC configurations via RRC signalling are valid for UEs in RRC_CONNECTED.

The UE shall:

1> if dmtc-Periodicity and dmtc-Offset are present in MeasDS-Config,

2> setup the DMTC such that the condition in Section 5.5.2.10 is met

1> if dmtc-WindowSize is present in MeasDS-Config,

2> set the duration of the DMTC occasion equal to the dmtc-WindowSize

NOTE 1: For primary/secondary serving cells, E-UTRAN provides full DMTC configurations.

1>	if only a subset of the DMTC parameters are provided to the UE, i.e. if the DMTC occasion is not fully defined,

2> the UE shall apply received DMTC parameters to determine the DMTC occasion.

1> if neighCellDMTC is provided and

2> the asyncNeighCells is set to true; and measDS-ConfigAsync is absent,

3> the UE shall setup the DMTC assuming any possible discovery signals timing for unsynchronized neighbour cells.

2> if the UE is provided with separate DMTC configurations for synchronized neighbour cells and unsynchronized neighbour cells,

3> the UE shall apply both DMTC configurations.

NOTE 2: 	measDS-ConfigAsync may be used to provide DMTC configuration for neighbour cells with a time drift due to lack of time synchronization between the cells, including to signal a different dmtc-Offset.

1> if the DMTC for a certain frequency is provided both for a measurement object and for a secondary serving cell,

- the UE shall apply both DMTC configurations

NOTE 3: 	On the concerned MF frequency, the UE is not expected to consider MF discovery signals transmission, if neighCellDMTC is not provided

#### 5.5.2.11	RSSI measurement timing configuration

The UE shall setup the RSSI measurement timing configuraton (RMTC) in accordance with the received rmtc-Period, rmtc-SubframeOffset if configured otherwise determined by the UE randomly, i.e. the first symbol of each RMTC occasion occurs at first symbol of an SFN and subframe of the PCell meeting the following condition:

SFN mod T = FLOOR(rmtc-SubframeOffset/10);

subframe = rmtc-SubframeOffset mod 10;

with T = rmtc-Period/10;

On the concerned frequency, the UE shall not consider RSSI measurements outside the configured RMTC occasion which lasts for measDuration for RSSI and channel occupancy measurements.

### 5.5.3	Performing measurements

#### 5.5.3.1	General

For all measurements, except for UE Rx–Tx time difference measurements, RSSI, UL PDCP Packet Delay per QCI measurement, channel occupancy measurements, and except for WLAN measurements of Band, Carrier Info, Available Admission Capacity, Backhaul Bandwidth, Channel Utilization, and Station Count, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria or for measurement reporting.

The UE shall:

1> 	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell as follows:

2> 	for the PCell, apply the time domain measurement resource restriction in accordance with measSubframePatternPCell, if configured;

2> 	if the UE supports CRS based discovery signals measurement:

3> 	for each SCell in deactivated state, apply the discovery signals measurement timing configuration in accordance with measDS-Config, if configured within the measObject corresponding to the frequency of the SCell;

- if the UE has a measConfig with rs-sinr-Config configured, perform RS-SINR (as indicated in the associated reportConfig) measurements as follows:

2> 	perform the corresponding measurements on the frequency indicated in the associated measObject using available idle periods or using autonomous gaps as necessary;

1> 	for each measId included in the measIdList within VarMeasConfig:

2> 	if the purpose for the associated reportConfig is set to reportCGI:

3> 	if si-RequestForHO is configured for the associated reportConfig:

4> 	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using autonomous gaps as necessary;

3> 	else:

4> 	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using available idle periods or using autonomous gaps as necessary;

NOTE 1:	If autonomous gaps are used to perform measurements, the UE is allowed to temporarily abort communication with all serving cell(s), i.e. create autonomous gaps to perform the corresponding measurements within the limits specified in MFA TS 36.133 [16]. Otherwise, the UE only supports the measurements with the purpose set to reportCGI only if E-UTRAN has provided sufficient idle periods.

3> 	try to acquire the global cell identity of the cell indicated by the cellForWhichToReportCGI in the associated measObject by acquiring the relevant system information from the concerned cell;

3> 	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is an E-UTRAN cell:

4> 	try to acquire the CSG identity, if the CSG identity is broadcast in the concerned cell;

4> 	try to acquire the trackingAreaCode in the concerned cell;

4> 	try to acquire the list of additional PLMN Identities, as included in the plmn-IdentityList, if multiple PLMN identities are broadcast in the concerned cell;

4> 	if the includeMultiBandInfo is configured:

5> 	try to acquire the freqBandIndicator in the SystemInformationBlockType1of the concerned cell;

5> 	try to acquire the list of additional frequency band indicators, as included in the multiBandInfoList, if multiple frequency band indicators are included in the SystemInformationBlockType1of the concerned cell;

5> 	try to acquire the freqBandIndicatorPriority, if the freqBandIndicatorPriority is included in the SystemInformationBlockType1of the concerned cell;

NOTE 2:	The 'primary' PLMN is part of the global cell identity.

NOTE MF:	For a PLMN signalled in altCellAccessInfoList, the global cell identity is constructed from that PLMN and the cellIdentity associated with that PLMN. The trackingAreaCode or CSG Identity for that PLMN corresponds to the PLMN specific parameters signalled in altCellAccessInfoList.

3> 	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is a UTRAN cell:

4> 	try to acquire the LAC, the RAC and the list of additional PLMN Identities, if multiple PLMN identities are broadcast in the concerned cell;

4> 	try to acquire the CSG identity, if the CSG identity is broadcast in the concerned cell;

3> 	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is a GERAN cell:

4> 	try to acquire the RAC in the concerned cell;

3> 	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is a CDMA2000 cell and the cdma2000-Type included in the measObject is typeHRPD:

4> 	try to acquire the Sector ID in the concerned cell;

3> 	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is a CDMA2000 cell and the cdma2000-Type included in the measObject is type1XRTT:

4> 	try to acquire the BASE ID, SID and NID in the concerned cell;

2> 	if the ul-DelayConfig is configured for the associated reportConfig:

3> 	ignore the measObject;

3> 	configure the PDCP layer to perform UL PDCP Packet Delay per QCI measurement;

2> 	else:

3> 	if a measurement gap configuration is setup; or

3> 	if the UE does not require measurement gaps to perform the concerned measurements:

4> 	if s-Measure is not configured; or

4> 	if s-Measure is configured and the PCell RSRP, after layer 3 filtering, is lower than this value; or

4> 	if measDS-Config is configured in the associated measObject:

5> 	if the UE supports CSI-RS based discovery signals measurement; and

5> 	if the eventId in the associated reportConfig is set to eventC1 or eventC2, or if reportStrongestCSI-RSs is included in the associated reportConfig:

6> 	perform the corresponding measurements of CSI-RS resources on the frequency indicated in the concerned measObject, applying the discovery signals measurement timing configuration in accordance with measDS-Config in the concerned measObject;

6> 	if reportCRS-Meas is included in the associated reportConfig, perform the corresponding measurements of neighbouring cells on the frequencies indicated in the concerned measObject as follows:

7> 	for neighbouring cells on the primary frequency, apply the time domain measurement resource restriction in accordance with measSubframePatternConfigNeigh, if configured in the concerned measObject;

7> 	apply the discovery signals measurement timing configuration in accordance with measDS-Config in the concerned measObject;

5> 	else:

6> 	perform the corresponding measurements of neighbouring cells on the frequencies and RATs indicated in the concerned measObject as follows:

7> 	for neighbouring cells on the primary frequency, apply the time domain measurement resource restriction in accordance with measSubframePatternConfigNeigh, if configured in the concerned measObject;

7> 	if the UE supports CRS based discovery signals measurement, apply the discovery signals measurement timing configuration in accordance with measDS-Config, if configured in the concerned measObject;

4> 	if the ue-RxTxTimeDiffPeriodical is configured in the associated reportConfig:

5> 	perform the UE Rx–Tx time difference measurements on the PCell;

4> 	if the reportSSTD-Meas is set to true in the associated reportConfig:

5> 	perform SSTD measurements between the PCell and the PSCell;

4> 	if the measRSSI-ReportConfig is configured in the associated reportConfig:

5> 	perform the RSSI and channel occupancy measurements on the frequency indicated in the associated measObject;

2> 	perform the evaluation of reporting criteria as specified in 5.5.4;

NOTE 3:	The s-Measure defines when the UE is required to perform measurements. The UE is however allowed to perform measurements also when the PCell RSRP exceeds s-Measure, e.g., to measure cells broadcasting a CSG identity following use of the autonomous search function as defined in MFA TS 36.304 [4].

NOTE 4:	The UE may not perform the WLAN measurements it is configured with e.g. due to connection to another WLAN based on user preferences as specified in 3GPP TS 23.402 [75] or due to turning off WLAN.

#### 5.5.3.2	Layer 3 filtering

The UE shall:

1> 	for each measurement quantity that the UE performs measurements according to 5.5.3.1:

NOTE 1:	This does not include quantities configured solely for UE Rx-Tx time difference, SSTD measurements and RSSI, channel occupancy measurements, WLAN measurements of Band, Carrier Info, Available Admission Capacity, Backhaul Bandwidth, Channel Utilization, and Station Count, and UL PDCP Packet Delay per QCI measurement i.e. for those types of measurements the UE ignores the triggerQuantity and reportQuantity.

2> 	filter the measured result, before using for evaluation of reporting criteria or for measurement reporting, by the following formula:

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria or for measurement reporting;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and

a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig;

2> 	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to 200 ms;

NOTE 2:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 3:	The filtering is performed in the same domain as used for evaluation of reporting criteria or for measurement reporting, i.e., logarithmic filtering for logarithmic measurements.

NOTE 4:	The filter input rate is implementation dependent, to fulfil the performance requirements set in MFA TS 36.133 [16]. For further details about the physical layer measurements, see MFA TS 36.133 [16].

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If security has been activated successfully, the UE shall:

1> 	for each measId included in the measIdList within VarMeasConfig:

2> 	if the corresponding reportConfig includes a purpose set to reportStrongestCellsForSON:

3> 	consider any neighbouring cell detected on the associated frequency to be applicable;

2> 	else if the corresponding reportConfig includes a purpose set to reportCGI:

3> 	consider any neighbouring cell detected on the associated frequency/ set of frequencies (GERAN) which has a physical cell identity matching the value of the cellForWhichToReportCGI included in the corresponding measObject within the VarMeasConfig to be applicable;

2> 	else:

3> 	if the corresponding measObject concerns E-UTRA:

4> 	if the ue-RxTxTimeDiffPeriodical is configured in the corresponding reportConfig:

5> 	consider only the PCell to be applicable;

4> 	else if the reportSSTD-Meas is set to true in the corresponding reportConfig:

5> 	consider the PSCell to be applicable;

4> 	else if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5> 	consider only the serving cell to be applicable;

4> 	else if eventC1 or eventC2 is configured in the corresponding reportConfig; or if reportStrongestCSI-RSs is included in the corresponding reportConfig:

5> 	consider a CSI-RS resource on the associated frequency to be applicable when the concerned CSI-RS resource is included in the measCSI-RS-ToAddModList defined within the VarMeasConfig for this measId;

4> 	else if measRSSI-ReportConfig is configured in the corresponding reportConfig:

5> 	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4> 	else:

5> 	if useWhiteCellList is set to TRUE:

6> 	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is included in the whiteCellsToAddModList defined within the VarMeasConfig for this measId;

5> 	else:

6> 	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

5> 	for events involving a serving cell on one frequency and neighbours on another frequency, consider the serving cell on the other frequency as a neighbouring cell;

4> 	if the corresponding reportConfig includes alternativeTimeToTrigger and if the UE supports alternativeTimeToTrigger:

5> 	use the value of alternativeTimeToTrigger as the time to trigger instead of the value of timeToTrigger in the corresponding reportConfig for cells included in the altTTT-CellsToAddModList of the corresponding measObject;

3> 	else if the corresponding measObject concerns UTRA or CDMA2000:

4> 	consider a neighbouring cell on the associated frequency to be applicable when the concerned cell is included in the cellsToAddModList defined within the VarMeasConfig for this measId (i.e. the cell is included in the white-list);

NOTE 0:	The UE may also consider a neighbouring cell on the associated UTRA frequency to be applicable when the concerned cell is included in the csg-allowedReportingCells within the VarMeasConfig for this measId, if configured in the corresponding measObjectUTRA (i.e. the cell is included in the range of physical cell identities for which reporting is allowed).

3> 	else if the corresponding measObject concerns GERAN:

4> 	consider a neighbouring cell on the associated set of frequencies to be applicable when the concerned cell matches the ncc-Permitted defined within the VarMeasConfig for this measId;

3> 	else if the corresponding measObject concerns WLAN:

4> 	consider a WLAN on the associated set of frequencies, as indicated by carrierFreq or on all WLAN frequencies when carrierFreq is not present, to be applicable if the WLAN matches all WLAN identifiers of at least one entry within wlan-Id-List for this measId;

2> 	if the triggerType is set to event and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first cell triggers the event):

3> 	include a measurement reporting entry within the VarMeasReportList for this measId;

3> 	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3> 	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3> 	if the UE supports T312 and if useT312 is included for this event and if T310 is running:

4> 	if T312 is not running:

5> 	start timer T312 with the value configured in the corresponding measObject;

3> 	initiate the measurement reporting procedure, as specified in 5.5.5;

2> 	if the triggerType is set to event and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3> 	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3> 	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3> 	if the UE supports T312 and if useT312 is included for this event and if T310 is running:

4> 	if T312 is not running:

5> 	start timer T312 with the value configured in the corresponding measObject;

3> 	initiate the measurement reporting procedure, as specified in 5.5.5;

2> 		if the triggerType is set to event and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3> 	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3> 	if the UE supports T312 and if useT312 is included for this event and if T310 is running:

4> 	if T312 is not running:

5> 	start timer T312 with the value configured in the corresponding measObject;

3> 	if reportOnLeave is set to TRUE for the corresponding reporting configuration or if a6-ReportOnLeave is set to TRUE for the corresponding reporting configuration:

4> 	initiate the measurement reporting procedure, as specified in 5.5.5;

3> 	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4> 	remove the measurement reporting entry within the VarMeasReportList for this measId;

4> 	stop the periodical reporting timer for this measId, if running;

2> 	if the triggerType is set to event and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable CSI-RS resources for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (i.e. a first CSI-RS resource triggers the event):

3> 	include a measurement reporting entry within the VarMeasReportList for this measId;

3> 	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3> 	include the concerned CSI-RS resource(s) in the csi-RS-TriggeredList defined within the VarMeasReportList for this measId;

3> 	initiate the measurement reporting procedure, as specified in 5.5.5;

2> 	if the triggerType is set to event and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable CSI-RS resources not included in the csi-RS-TriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (i.e. a subsequent CSI-RS resource triggers the event):

3> 	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3> 	include the concerned CSI-RS resource(s) in the csi-RS-TriggeredList defined within the VarMeasReportList for this measId;

3> 	initiate the measurement reporting procedure, as specified in 5.5.5;

2> 	if the triggerType is set to event and if the leaving condition applicable for this event is fulfilled for one or more of the CSI-RS resources included in the csi-RS-TriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3> 	remove the concerned CSI-RS resource(s) in the csi-RS-TriggeredList defined within the VarMeasReportList for this measId;

3> 	if c1-ReportOnLeave is set to TRUE for the corresponding reporting configuration or if c2-ReportOnLeave is set to TRUE for the corresponding reporting configuration:

4> 	initiate the measurement reporting procedure, as specified in 5.5.5;

3> 	if the csi-RS-TriggeredList defined within the VarMeasReportList for this measId is empty:

4> 	remove the measurement reporting entry within the VarMeasReportList for this measId;

4> 	stop the periodical reporting timer for this measId, if running;

2> 	if measRSSI-ReportConfig is included and if a (first) measurement result is available:

3> 	include a measurement reporting entry within the VarMeasReportList for this measId;

3> 	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3> 	initiate the measurement reporting procedure as specified in 5.5.5 immediately when RSSI sample values are reported by the physical layer after the first L1 measurement duration;

2> 	else if the purpose is included and set to reportStrongestCells or to reportStrongestCellsForSON and if a (first) measurement result is available:

3> 	include a measurement reporting entry within the VarMeasReportList for this measId;

3> 	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3> 	if the purpose is set to reportStrongestCells and reportStrongestCSI-RSs is not included:

4> 	if the triggerType is set to periodical and the corresponding reportConfig includes the ul-DelayConfig:

5> 	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided by lower layers;

4> 	else if the reportAmount exceeds 1:

5> 	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the PCell;

4> 	else (i.e. the reportAmount is equal to 1):

5> 	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the PCell and for the strongest cell among the applicable cells, or becomes available for the pair of PCell and the PSCell in case of SSTD measurements;

3> 	else:

4> 	initiate the measurement reporting procedure, as specified in 5.5.5, when it has determined the strongest cells on the associated frequency;

2> 	upon expiry of the periodical reporting timer for this measId:

3> 	initiate the measurement reporting procedure, as specified in 5.5.5;

2> 	if the purpose is included and set to reportCGI and if the UE acquired the information needed to set all fields of cgi-Info for the requested cell:

3> 	include a measurement reporting entry within the VarMeasReportList for this measId;

3> 	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3> 	stop timer T321;

3> 	initiate the measurement reporting procedure, as specified in 5.5.5;

2> 	upon expiry of the T321 for this measId:

3> 	include a measurement reporting entry within the VarMeasReportList for this measId;

3> 	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3> 	initiate the measurement reporting procedure, as specified in 5.5.5;

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to event or to periodical while the corresponding measurement is not performed due to the PCell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

NOTE 3:	If the UE is configured with DRX, the UE may delay the measurement reporting for event triggered and periodical triggered measurements until the Active Time, which is defined in MFA TS 36.321 [6].

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when condition A1-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition A1-2, as specified below, is fulfilled;

1> 	for this measurement, consider the primary or secondary cell that is configured on the frequency indicated in the associated measObjectEUTRA to be the serving cell;

Inequality A1-1 (Entering condition)

Inequality A1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigEUTRA for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1> 	for this measurement, consider the primary or secondary cell that is configured on the frequency indicated in the associated measObjectEUTRA to be the serving cell;

Inequality A2-1 (Entering condition)

Inequality A2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigEUTRA for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than PCell/PSCell)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1> 	if usePSCell of the corresponding reportConfig is set to true:

2> 	use the PSCell for Mp, Ofp and Ocp;

1> 	else:

2> 	use the PCell for Mp, Ofp and Ocp;

NOTE	The cell(s) that triggers the event is on the frequency indicated in the associated measObject which may be different from the frequency used by the PCell/PSCell.

Inequality A3-1 (Entering condition)

Inequality A3-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the PCell/PSCell, not taking into account any offsets.

Ofp is the frequency specific offset of the frequency of the PCell/PSCell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the PCell/PSCell).

Ocp is the cell specific offset of the PCell/PSCell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the PCell/PSCell), and is set to zero if not configured for the PCell/PSCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigEUTRA for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled;

Inequality A4-1 (Entering condition)

Inequality A4-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigEUTRA for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.6	Event A5 (PCell/PSCell becomes worse than threshold1 and neighbour becomes better than threshold2)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

1> 	if usePSCell of the corresponding reportConfig is set to true:

2> 	use the PSCell for Mp;

1> 	else:

2> 	use the PCell for Mp;

NOTE:	The cell(s) that triggers the event is on the frequency indicated in the associated measObject which may be different from the frequency used by the PCell/PSCell.

Inequality A5-1 (Entering condition 1)

Inequality A5-2 (Entering condition 2)

Inequality A5-3 (Leaving condition 1)

Inequality A5-4 (Leaving condition 2)

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell/PSCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigEUTRA for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigEUTRA for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.6a	Event A6 (Neighbour becomes offset better than SCell)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when condition A6-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition A6-2, as specified below, is fulfilled;

1> 	for this measurement, consider the (secondary) cell that is configured on the frequency indicated in the associated measObjectEUTRA to be the serving cell;

NOTE:	The neighbour(s) is on the same frequency as the SCell i.e. both are on the frequency indicated in the associated measObject.

Inequality A6-1 (Entering condition)

Inequality A6-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Ms is the measurement result of the serving cell, not taking into account any offsets.

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the serving frequency), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Off is the offset parameter for this event (i.e. a6-Offset as defined within reportConfigEUTRA for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ocn, Ocs, Hys, Off are expressed in dB.

#### 5.5.4.7	Event B1 (Inter RAT neighbour becomes better than threshold)

The UE shall:

1> 	for UTRA and CDMA2000, only trigger the event for cells included in the corresponding measurement object;

1> 	consider the entering condition for this event to be satisfied when condition B1-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition B1-2, as specified below, is fulfilled;

Inequality B1-1 (Entering condition)

Inequality B1-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets. For CDMA 2000 measurement result, pilotStrength is divided by -2.

Ofn is the frequency specific offset of the frequency of the inter-RAT neighbour cell (i.e. offsetFreq as defined within the measObject corresponding to the frequency of the neighbour inter-RAT cell).

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. b1-Threshold as defined within reportConfigInterRAT for this event). For CDMA2000, b1-Threshold is divided by -2.

Mn is expressed in dBm or in dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.8	Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2)

The UE shall:

1> 	for UTRA and CDMA2000, only trigger the event for cells included in the corresponding measurement object;

1> 	consider the entering condition for this event to be satisfied when both condition B2-1 and condition B2-2, as specified below, are fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition B2-3 or condition B2-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality B2-1 (Entering condition 1)

Inequality B2-2 (Entering condition 2)

Inequality B2-3 (Leaving condition 1)

Inequality B2-4 (Leaving condition 2)

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets. For CDMA2000 measurement result, pilotStrength is divided by -2.

Ofn is the frequency specific offset of the frequency of the inter-RAT neighbour cell (i.e. offsetFreq as defined within the measObject corresponding to the frequency of the inter-RAT neighbour cell).

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. b2-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. b2-Threshold2 as defined within reportConfigInterRAT for this event). For CDMA2000, b2-Threshold2 is divided by -2.

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ.

Mn is expressed in dBm or dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.9	Event C1 (CSI-RS resource becomes better than threshold)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when condition C1-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition C1-2, as specified below, is fulfilled;

Inequality C1-1 (Entering condition)

Inequality C1-2 (Leaving condition)

The variables in the formula are defined as follows:

Mcr is the measurement result of the CSI-RS resource, not taking into account any offsets.

Ocr is the CSI-RS specific offset (i.e. csi-RS-IndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the CSI-RS resource), and set to zero if not configured for the CSI-RS resource.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Thresh is the threshold parameter for this event (i.e. c1-Threshold as defined within reportConfigEUTRA for this event).

Mcr, Thresh are expressed in dBm.

Ocr, Hys are expressed in dB.

#### 5.5.4.10	Event C2 (CSI-RS resource becomes offset better than reference CSI-RS resource)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when condition C2-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition C2-2, as specified below, is fulfilled;

NOTE:	The CSI-RS resource(s) that triggers the event is on the same frequency as the reference CSI-RS resource, i.e. both are on the frequency indicated in the associated measObject.

Inequality C2-1 (Entering condition)

Inequality C2-2 (Leaving condition)

The variables in the formula are defined as follows:

Mcr is the measurement result of the CSI-RS resource, not taking into account any offsets.

Ocr is the CSI-RS specific offset of the CSI-RS resource (i.e. csi-RS-IndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the CSI-RS resource), and set to zero if not configured for the CSI-RS resource.

Mref is the measurement result of the reference CSI-RS resource (i.e. c2-RefCSI-RS as defined within reportConfigEUTRA for this event), not taking into account any offsets.

Oref is the CSI-RS specific offset of the reference CSI-RS resource (i.e. csi-RS-IndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the reference CSI-RS resource), and is set to zero if not configured for the reference CSI-RS resource.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Off is the offset parameter for this event (i.e. c2-Offset as defined within reportConfigEUTRA for this event).

Mcr, Mref are expressed in dBm.

Ocr, Oref, Hys, Off are expressed in dB.

#### 5.5.4.11	Event W1 (WLAN becomes better than a threshold)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when wlan-MobilitySet within VarWLAN-MobilityConfig does not contain any entries and condition W1-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition W1-2, as specified below, is fulfilled;

Inequality W1-1 (Entering condition)

Inequality W1-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of WLAN(s) configured in the measurement object, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w1-Threshold as defined within reportConfigInterRAT for this event).

Mn is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.12	Event W2 (All WLAN inside WLAN mobility set becomes worse than threshold1 and a WLAN outside WLAN mobility set becomes better than threshold2)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when both conditions W2-1 and W2-2 as specified below are fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition W2-3 or condition W2-4, i.e. at least one of the two, as specified below is fulfilled;

Inequality W2-1 (Entering condition 1)

Inequality W2-2 (Entering condition 2)

Inequality W2-3 (Leaving condition 1)

Inequality W2-4 (Leaving condition 2)

The variables in the formula are defined as follows:

Ms is the measurement result of WLAN(s) which matches all WLAN identifiers of at least one entry within wlan-MobilitySet in VarWLAN-MobilityConfig, not taking into account any offsets.

Mn is the measurement result of WLAN(s) configured in the measurement object which does not match all WLAN identifiers of any entry within wlan-MobilitySet in VarWLAN-MobilityConfig, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh1 is the threshold parameter for this event (i.e. w2-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. w2-Threshold2 as defined within reportConfigInterRAT for this event).

Mn, Ms are expressed in dBm.

Hys is expressed in dB.

Thresh1 is expressed in the same unit as Ms.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.13	Event W3 (All WLAN inside WLAN mobility set becomes worse than a threshold)

The UE shall:

1> 	consider the entering condition for this event to be satisfied when condition W3-1, as specified below, is fulfilled;

1> 	consider the leaving condition for this event to be satisfied when condition W3-2, as specified below, is fulfilled;

Inequality W3-1 (Entering condition)

Inequality W3-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of WLAN(s) which matches all WLAN identifiers of at least one entry within wlan-MobilitySet in VarWLAN-MobilityConfig, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. w3-Threshold as defined within reportConfigInterRAT for this event).

Ms is expressed in dBm.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

### 5.5.5	Measurement reporting

Figure 5.5.5-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to E-UTRAN. The UE shall initiate this procedure only after successful security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1> 	set the measId to the measurement identity that triggered the measurement reporting;

1> 	set the measResultPCell to include the quantities of the PCell;

1> 	set the measResultServFreqList to include for each SCell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in MFA TS 36.133 [16];

1> 	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2> 	for each serving frequency for which measObjectId is referenced in the measIdList, other than the frequency corresponding with the measId that triggered the measurement reporting:

3> 	set the measResultServFreqList to include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1> 	if there is at least one applicable neighbouring cell to report:

2> 	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3> 	if the triggerType is set to event:

4> 	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3> 	else:

4> 	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE 1:	The reliability of the report (i.e. the certainty it contains the strongest cells on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in MFA TS 36.133 [16].

3> 	for each cell that is included in the measResultNeighCells, include the physCellId;

3> 	if the triggerType is set to event; or the purpose is set to reportStrongestCells or to reportStrongestCellsForSON:

4> 	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5> 	if the measObject associated with this measId concerns E-UTRA:

6> 	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfig in order of decreasing triggerQuantity, i.e. the best cell is included first;

5> 	if the measObject associated with this measId concerns UTRA FDD and if ReportConfigInterRAT includes the reportQuantityUTRA-FDD:

6> 	set the measResult to include the quantities indicated by the reportQuantityUTRA-FDD in order of decreasing measQuantityUTRA-FDD within the quantityConfig, i.e. the best cell is included first;

5> 	if the measObject associated with this measId concerns UTRA FDD and if ReportConfigInterRAT does not include the reportQuantityUTRA-FDD; or

5> 	if the measObject associated with this measId concerns UTRA TDD, GERAN or CDMA2000:

6> 	set the measResult to the quantity as configured for the concerned RAT within the quantityConfig in order of either decreasing quantity for UTRA and GERAN or increasing quantity for CDMA2000 pilotStrength, i.e. the best cell is included first;

3> 	else if the purpose is set to reportCGI:

4> 	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5> 		if the includeMultiBandInfo is configured:

6> 	include the freqBandIndicator;

6> 		if the cell broadcasts the multiBandInfoList, include the multiBandInfoList;

6> 	if the cell broadcasts the freqBandIndicatorPriority, include the freqBandIndicatorPriority;

5> 	if the cell broadcasts a CSG identity:

6> 	include the csg-Identity;

6> 	include the csg-MemberStatus and set it to member if the cell is a CSG member cell;

5> 	if the si-RequestForHO is configured within the reportConfig associated with this measId:

6> 	include the cgi-Info containing all the fields other than the plmn-IdentityList that have been successfully acquired;

6>	include, within the cgi-Info, the field plmn-IdentityList in accordance with the following:

7> 	if the cell is a CSG member cell, determine the subset of the PLMN identities, starting from the second entry of PLMN identities in the broadcast information, that meet the following conditions:

a)	equal to the RPLMN or an EPLMN; and

b)	the CSG whitelist of the UE includes an entry comprising of the concerned PLMN identity and the CSG identity broadcast by the cell;

7> 	if the subset of PLMN identities determined according to the previous includes at least one PLMN identity, include the plmn-IdentityList and set it to include this subset of the PLMN identities;

7> 	if the cell is a CSG member cell, include the primaryPLMN-Suitable if the primary PLMN meets conditions a) and b) specified above;

5> 	else:

6> 	include the cgi-Info containing all the fields that have been successfully acquired and in accordance with the following:

7> 	include in the plmn-IdentityList the list of identities starting from the second entry of PLMN Identities in the broadcast information;

NOTE MF:	For a PLMN signalled in altCellAccessInfoList, the global cell identity is constructed from that PLMN and the cellIdentity associated with that PLMN. The trackingAreaCode or CSG Identity for that PLMN corresponds to the PLMN specific parameters signalled in altCellAccessInfoList.

1> 	for the cells included according to the previous (i.e. covering the PCell, the SCells, the best non-serving cells on serving frequencies as well as neighbouring EUTRA cells) include results according to the extended RSRQ if corresponding results are available according to the associated performance requirements defined in MFA TS 36.133 [16];

1> 	if there is at least one applicable CSI-RS resource to report:

2> 	set the measResultCSI-RS-List to include the best CSI-RS resources up to maxReportCells in accordance with the following:

3> 	if the triggerType is set to event:

4> 	include the CSI-RS resources included in the csi-RS-TriggeredList as defined within the VarMeasReportList for this measId;

3> 	else:

4> 	include the applicable CSI-RS resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE 2:	The reliability of the report (i.e. the certainty it contains the strongest CSI-RS resources on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in MFA TS 36.133 [16].

3> 	for each CSI-RS resource that is included in the measResultCSI-RS-List:

4> 	include the measCSI-RS-Id;

4> 	include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follow:

5> 	set the csi-RSRP-Result to include the quantity indicated in the reportQuantity within the concerned reportConfig in order of decreasing triggerQuantityCSI-RS, i.e. the best CSI-RS resource is included first;

4> 	if reportCRS-Meas is included within the associated reportConfig, and the cell indicated by physCellId of this CSI-RS resource is not a serving cell:

5> 	set the measResultNeighCells to include the cell indicated by physCellId of this CSI-RS resource, and include the physCellId;

5> 	set the rsrpResult to include the RSRP of the concerned cell, if available according to performance requirements in MFA TS 36.133 [16];

5> 	set the rsrqResult to include the RSRQ of the concerned cell, if available according to performance requirements in MFA TS 36.133 [16];

1> 	if the ue-RxTxTimeDiffPeriodical is configured within the corresponding reportConfig for this measId;

2> 	set the ue-RxTxTimeDiffResult to the measurement result provided by lower layers;

2> 	set the currentSFN;

1> 	if the measRSSI-ReportConfig is configured within the corresponding reportConfig for this measId:

2> 	set the rssi-Result to the average of sample value(s) provided by lower layers in the reportInterval;

2> 	set the channelOccupancy to the rounded percentage of sample values which are beyond to the channelOccupancyThreshold within all the sample values in the reportInterval;

1> 	if uplink PDCP delay results are available:

2> 	set the ul-PDCP-DelayResultList to include the uplink PDCP delay results available;

1> 	if the includeLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of the locationInfo as follows:

2> 	include the locationCoordinates;

2> 	if available, include the gnss-TOD-msec;

1> 	if the reportSSTD-Meas is set to true within the corresponding reportConfig for this measId:

2> 	set the measResultSSTD to the measurement results provided by lower layers;

1> 	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1> 	stop the periodical reporting timer, if running;

1> 	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2> 	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1> 	else:

2> 	if the triggerType is set to periodical:

3> 	remove the entry within the VarMeasReportList for this measId;

3> 	remove this measId from the measIdList within VarMeasConfig;

1> 	if the measured results are for CDMA2000 HRPD:

2> 	set the preRegistrationStatusHRPD to the UE's CDMA2000 upper layer's HRPD preRegistrationStatus;

1> 	if the measured results are for CDMA2000 1xRTT:

2> 	set the preRegistrationStatusHRPD to FALSE;

1> 	if the measured results are for WLAN:

2> 	set the measResultListWLAN to include the quantities within the quantityConfigWLAN for the following WLAN(s) up to maxReportCells:

3> 	include WLAN the UE is connected to, if any;

3> 	include WLAN in order of decreasing WLAN RSSI, i.e. the best WLAN is included first, for WLANs which do not match all WLAN identifiers of any entry within wlan-MobilitySet in VarWLAN-MobilityConfig;

2> 	for each included WLAN:

3> 	set wlan-Identifiers to include all WLAN identifiers that can be acquired for the WLAN measured;

3> 	set connectedWLAN to TRUE if the UE is connected to the WLAN measured;

3> 	if reportQuantityWLAN exists within the ReportConfigInterRAT within the VarMeasConfig for this measId:

4> 	if bandRequestWLAN is set to TRUE:

5> 	set bandWLAN to include WLAN band of the WLAN measured;

4> 	if carrierInfoRequestWLAN is set to TRUE:

5> 	set carrierInfoWLAN to include WLAN carrier information of the WLAN measured if it can be acquired;

4> 	if availableAdmissionCapacityRequestWLAN is set to TRUE:

5> 	set the measResult to include avaiableAdmissionCapacityWLAN if it can be acquired;

4> 	if backhaulDL-BandwidthRequestWLAN is set to TRUE:

5> 	set the measResult to include backhaulDL-BandwidthWLAN if it can be acquired;

4> 	if backhaulUL-BandwidthRequestWLAN is set to TRUE:

5> 	set the measResult to include backhaulUL-BandwidthWLAN if it can be acquired;

4> 	if channelUtilizationRequestWLAN is set to TRUE:

5> 	set the measResult to include channelUtilizationWLAN if it can be acquired;

4> 		if stationCountRequestWLAN is set to TRUE:

5> 	set the measResult to include stationCountWLAN if it can be acquired;

1> 	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

### 5.5.6	Measurement related actions

#### 5.5.6.1	Actions upon handover and re-establishment

E-UTRAN applies the handover procedure as follows:

-	when performing the handover procedure, as specified in 5.3.5.4, ensure that a measObjectId corresponding to each handover target serving frequency is configured as a result of the procedures described in this sub-clause and in 5.3.5.4;

-	when changing the band while the physical frequency remains unchanged, E-UTRAN releases the measObject corresponding to the source frequency and adds a measObject corresponding to the target frequency (i.e. it does not reconfigure the measObject);

E-UTRAN applies the re-establishment procedure as follows:

-	when performing the connection re-establishment procedure, as specified in 5.3.7, ensure that a measObjectId corresponding each target serving frequency is configured as a result of the procedure described in this sub-clause and the subsequent connection reconfiguration procedure immediately following the re-establishment procedure;

-	in the first reconfiguration following the re-establishment when changing the band while the physical frequency remains unchanged, E-UTRAN releases the measObject corresponding to the source frequency and adds a measObject corresponding to the target frequency (i.e. it does not reconfigure the measObject);

The UE shall:

1> 	for each measId included in the measIdList within VarMeasConfig:

2> 	if the triggerType is set to periodical:

3> 	remove this measId from the measIdList within VarMeasConfig:

1> 	if the procedure was triggered due to a handover or successful re-establishment and the procedure involves a change of primary frequency, update the measId values in the measIdList within VarMeasConfig as follows:

2> 	if a measObjectId value corresponding to the target primary frequency exists in the measObjectList within VarMeasConfig:

3> 	for each measId value in the measIdList:

4> 	if the measId value is linked to the measObjectId value corresponding to the source primary frequency:

5> 	link this measId value to the measObjectId value corresponding to the target primary frequency;

4> 	else if the measId value is linked to the measObjectId value corresponding to the target primary frequency:

5> 	link this measId value to the measObjectId value corresponding to the source primary frequency;

2> 	else:

3> 	remove all measId values that are linked to the measObjectId value corresponding to the source primary frequency;

1> 	remove all measurement reporting entries within VarMeasReportList;

1> 	stop the periodical reporting timer or timer T321, whichever one is running, as well as associated information (e.g. timeToTrigger) for all measId;

1> 	release the measurement gaps, if activated;

NOTE:	If the UE requires measurement gaps to perform inter-frequency or inter-RAT measurements, the UE resumes the inter-frequency and inter-RAT measurements after the E-UTRAN has setup the measurement gaps.

#### 5.5.6.2	Speed dependant scaling of measurement related parameters

The UE shall adjust the value of the following parameter configured by the E-UTRAN depending on the UE speed: timeToTrigger. The UE shall apply 3 different levels, which are selected as follows:

The UE shall:

1> 	perform mobility state detection using the mobility state detection as specified in MFA TS 36.304 [4] with the following modifications:

2> 	counting handovers instead of cell reselections;

2> 	applying the parameter applicable for RRC_CONNECTED as included in speedStatePars within VarMeasConfig;

1> 	if high mobility state is detected:

2> 	use the timeToTrigger value multiplied by sf-High within VarMeasConfig;

1> 	else if medium mobility state is detected:

2> 	use the timeToTrigger value multiplied by sf-Medium within VarMeasConfig;

1> 	else:

2> 	no scaling is applied;

### 5.5.7	Inter-frequency RSTD measurement indication

#### 5.5.7.1	General

Figure 5.5.7.1-1: Inter-frequency RSTD measurement indication

The purpose of this procedure is to indicate to the network that the UE is going to start/stop OTDOA inter-frequency RSTD measurements which require measurement gaps as specified in MFA TS 36.133 [16, Section 8.1.2.6].

NOTE: 	It is a network decision to configure the measurement gap.

#### 5.5.7.2	Initiation

The UE shall:

1> 	if and only if upper layers indicate to start performing inter-frequency RSTD measurements and the UE requires measurement gaps for these measurements while measurement gaps are either not configured or not sufficient:

2> 	initiate the procedure to indicate start;

NOTE 1:	The UE verifies the measurement gap situation only upon receiving the indication from upper layers. If at this point in time sufficient gaps are available, the UE does not initiate the procedure. Unless it receives a new indication from upper layers, the UE is only allowed to further repeat the procedure in the same PCell once per frequency if the provided measurement gaps are insufficient.

1> 	if and only if upper layers indicate to stop performing inter-frequency RSTD measurements:

2> 	initiate the procedure to indicate stop;

NOTE 2:	The UE may initiate the procedure to indicate stop even if it did not previously initiate the procedure to indicate start.

#### 5.5.7.3	Actions related to transmission of InterFreqRSTDMeasurementIndication message

The UE shall set the contents of InterFreqRSTDMeasurementIndication message as follows:

1> 	set the rstd-InterFreqIndication as follows:

2> 	if the procedure is initiated to indicate start of inter-frequency RSTD measurements:

3> 	set the rstd-InterFreqInfoList according to the information received from upper layers;

2> 	else if the procedure is initiated to indicate stop of inter-frequency RSTD measurements:

3> 	set the rstd-InterFreqIndication to the value stop;

1> 	submit the InterFreqRSTDMeasurementIndication message to lower layers for transmission, upon which the procedure ends;

## 5.6	Other

### 5.6.0	General

For NB-IoT, only a subset of the procedures described in this sub-clause apply.

Table 5.6.0-1 specifies the procedures that are applicable to NB-IoT. All other procedures are not applicable to NB-IoT; this is not further stated in the corresponding procedures.

Table 5.6.0-1: "Other″ Procedures applicable to a NB-IoT UE

| Sub-clause | Procedures |
| --- | --- |
| 5.6.1 | DL information transfer |
| 5.6.2 | UL information transfer |
| 5.6.3 | UE Capability transfer |

### 5.6.1	DL information transfer

#### 5.6.1.1	General

Figure 5.6.1.1-1: DL information transfer

The purpose of this procedure is to transfer NAS or (tunnelled) non-3GPP dedicated information from E-UTRAN to a UE in RRC_CONNECTED.

#### 5.6.1.2	Initiation

E-UTRAN initiates the DL information transfer procedure whenever there is a need to transfer NAS or non-3GPP dedicated information. E-UTRAN initiates the DL information transfer procedure by sending the DLInformationTransfer message.

#### 5.6.1.3	Reception of the DLInformationTransfer by the UE

Upon receiving DLInformationTransfer message, the UE shall:

1> 	if the UE is a NB-IoT UE; or

1> 	if the dedicatedInfoType is set to dedicatedInfoNAS:

2> 	forward the dedicatedInfoNAS to the NAS upper layers.

1> 	if the dedicatedInfoType is set to dedicatedInfoCDMA2000-1XRTT or to dedicatedInfoCDMA2000-HRPD:

2> 	forward the dedicatedInfoCDMA2000 to the CDMA2000 upper layers;

### 5.6.2	UL information transfer

#### 5.6.2.1	General

Figure 5.6.2.1-1: UL information transfer

The purpose of this procedure is to transfer NAS or (tunnelled) non-3GPP dedicated information from the UE to E-UTRAN.

#### 5.6.2.2	Initiation

A UE in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer NAS or non-3GPP dedicated information, except at RRC connection establishment or resume in which case the NAS information is piggybacked to the RRCConnectionSetupComplete or RRCConnectionResumeComplete message correspondingly. The UE initiates the UL information transfer procedure by sending the ULInformationTransfer message. When CDMA2000 information has to be transferred, the UE shall initiate the procedure only if SRB2 is established.

#### 5.6.2.3	Actions related to transmission of ULInformationTransfer message

The UE shall set the contents of the ULInformationTransfer message as follows:

1>	if there is a need to transfer NAS information:

2> 	if the UE is a NB-IoT UE:

3> 	set the dedicatedInfoNAS to include the information received from upper layers;

2> 	else, set the dedicatedInfoType to include the dedicatedInfoNAS;

1> 	if there is a need to transfer CDMA2000 1XRTT information:

2> 	set the dedicatedInfoType to include the dedicatedInfoCDMA2000-1XRTT;

1> 	if there is a need to transfer CDMA2000 HRPD information:

2> 	set the dedicatedInfoType to include the dedicatedInfoCDMA2000-HRPD;

1> 	upon RRC connection establishment, if UE supports the Control Plane CIoT EPS optimisation and UE does not need UL gaps during continuous uplink transmission:

2> 	configure lower layers to stop using UL gaps during continuous uplink transmission in FDD for ULInformationTransfer message and subsequent uplink transmission in RRC_CONNECTED except for UL transmissions as specified in MFA TS 36.211 [21];

1> 	submit the ULInformationTransfer message to lower layers for transmission, upon which the procedure ends;

#### 5.6.2.4	Failure to deliver ULInformationTransfer message

The UE shall:

1> 	if the UE is a NB-IoT UE, AS security is not started and radio link failure occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers; or

1> 	if mobility (i.e. handover, RRC connection re-establishment) occurs before the successful delivery of ULInformationTransfer messages has been confirmed by lower layers:

2> 	inform upper layers about the possible failure to deliver the information contained in the concerned ULInformationTransfer messages;

### 5.6.3	UE capability transfer

#### 5.6.3.1	General

Figure 5.6.3.1-1: UE capability transfer

The purpose of this procedure is to transfer UE radio access capability information from the UE to E-UTRAN.

If the UE has changed its E-UTRAN radio access capabilities, the UE shall request higher layers to initiate the necessary NAS procedures (see 3GPP TS 23.401 [41]) that would result in the update of UE radio access capabilities using a new RRC connection.

NOTE:	Change of the UE's GERAN UE radio capabilities in RRC_IDLE is supported by use of Tracking Area Update.

#### 5.6.3.2	Initiation

E-UTRAN initiates the procedure to a UE in RRC_CONNECTED when it needs (additional) UE radio access capability information.

#### 5.6.3.3	Reception of the UECapabilityEnquiry by the UE

The UE shall:

1> 		for NB-IoT, set the contents of UECapabilityInformation message as follows:

2> 	include the UE Radio Access Capability Parameters within the ue-Capability-Container;

2> 	include ue-RadioPagingInfo;

2> 	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

1> 	else, set the contents of UECapabilityInformation message as follows:

2> 	if the ue-CapabilityRequest is set to true in the UECapabilityEnquiry-MF message:

3>	 include the UE’s MF Radio Access Capability Parameters in the UE-Capability-MF;

2> 	if the ue-CapabilityRequest includes eutra:

3> 	include the UE-EUTRA-Capability within a ue-CapabilityRAT-Container and with the rat-Type set to eutra;

3> 	if the UE supports FDD and TDD:

4> 	set all fields of UECapabilityInformation, except field fdd-Add-UE-EUTRA-Capabilities and tdd-Add-UE-EUTRA-Capabilities (including their sub-fields), to include the values applicable for both FDD and TDD (i.e. functionality supported by both modes);

4> 	if (some of) the UE capability fields have a different value for FDD and TDD:

5> 	if for FDD, the UE supports additional functionality compared to what is indicated by the previous fields of UECapabilityInformation:

6> 	include field fdd-Add-UE-EUTRA-Capabilities and set it to include fields reflecting the additional functionality applicable for FDD;

5> 	if for TDD, the UE supports additional functionality compared to what is indicated by the previous fields of UECapabilityInformation:

6> 	include field tdd-Add-UE-EUTRA-Capabilities and set it to include fields reflecting the additional functionality applicable for TDD;

NOTE 1:		The UE includes fields of XDD-Add-UE-EUTRA-Capabilities in accordance with the following:

-	The field is included only if one or more of its sub-fields (or bits in the feature group indicators string) has a value that is different compared to the value signalled elsewhere within UE-EUTRA-Capability;

(this value signalled elsewhere is also referred to as the Common value, that is supported for both XDD modes)

-	For the fields that are included in XDD-Add-UE-EUTRA-Capabilities, the UE sets:

-	the sub-fields (or bits in the feature group indicators string) that are not allowed to be different to the same value as the Common value;

-	the sub-fields (or bits in the feature group indicators string) that are allowed to be different to a value indicating at least the same functionality as indicated by the Common value;

3> 	else (UE supports single xDD mode):

4> 	set all fields of UECapabilityInformation, except field fdd-Add-UE-EUTRA-Capabilities and tdd-Add-UE-EUTRA-Capabilities (including their sub-fields), to include the values applicable for the xDD mode supported by the UE;

3> 	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message, comprising of band combinations supported by the UE according to the following priority order (i.e. listed in order of decreasing priority):

4> 		include all non-CA bands, regardless of whether UE supports carrier aggregation, only:

-	if the UE includes ue-Category-v1020 (i.e. indicating category 6 to 8); or

-	if for at least one of the non-CA bands, the UE supports more MIMO layers with TM9 and TM10 than implied by the UE category; or

-	if the UE supports TM10 with one or more CSI processes;

4> 	if the UECapabilityEnquiry message includes requestedFrequencyBands and UE supports requestedFrequencyBands:

5> 	include all 2DL+1UL CA band combinations, only consisting of bands included in requestedFrequencyBands;

5> 	include all other CA band combinations, only consisting of bands included in requestedFrequencyBands, and prioritized in the order of requestedFrequencyBands, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

4> 	else (no requested frequency bands):

5> 	include all 2DL+1UL CA band combinations;

5> 	include all other CA band combinations;

4> 	if UE supports maximumCCsRetrieval and if the UECapabilityEnquiry message includes the requestedMaxCCsDL and the requestedMaxCCsUL (i.e. both UL and DL maximums are given):

5> 	remove from the list of candidates the band combinations for which the number of CCs in DL exceeds the value indicated in the requestedMaxCCsDL or for which the number of CCs in UL exceeds the value indicated in the requestedMaxCCsUL;

5> 	indicate in requestedCCsUL the same value as received in requestedMaxCCsUL;

5> 	indicate in requestedCCsDL the same value as received in requestedMaxCCsDL;

4> 	else if UE supports maximumCCsRetrieval and if the UECapabilityEnquiry message includes the requestedMaxCCsDL (i.e. only DL maximum limit is given):

5> 	remove from the list of candidates the band combinations for which the number of CCs in DL exceeds the value indicated in the requestedMaxCCsDL;

5> 	indicate value in requestedCCsDL the same value as received in requestedMaxCCsDL;

4> 	else if UE supports maximumCCsRetrieval and if the UECapabilityEnquiry message includes the requestedMaxCCsUL (i.e. only UL maximum limit is given):

5> 	remove from the list of candidates the band combinations for which the number of CCs in UL exceeds the value indicated in the requestedMaxCCsUL;

5> 	indicate in requestedCCsUL the same value as received in requestedMaxCCsUL;

4> 	if the UE supports reducedIntNonContComb and the UECapabilityEnquiry message includes requestReducedIntNonContComb:

5> set reducedIntNonContCombRequested to true;

5> 	remove from the list of candidates the intra-band non-contiguous CA band combinations which support is implied by another intra-band non-contiguous CA band combination included in the list of candidates as specified in MFA TS 36.306 [5, 4.3.5.21]:

4> 	if the UE supports requestReducedFormat and UE supports skipFallbackCombinations and UECapabilityEnquiry message includes requestSkipFallbackComb:

5> 	set skipFallbackCombRequested to true;

5> 	for each band combination included in the list of candidates (including 2DL+1UL CA band combinations), starting with the ones with the lowest number of DL and UL carriers, that concerns a fallback band combination of another band combination included in the list of candidates as specified in MFA TS 36.306 [5]:

6> 	remove the band combination from the list of candidates;

6> 	include differentFallbackSupported in the band combination included in the list of candidates whose fallback concerns the removed band combination, if its capabilities differ from the removed band combination;

3> 	if the UECapabilityEnquiry message includes requestReducedFormat and UE supports requestReducedFormat:

4> 	include in supportedBandCombinationReduced as many as possible of the band combinations included in the list of candidates, including the non-CA combinations, determined according to the rules and priority order defined above;

3> 	else

4> 	if the UECapabilityEnquiry message includes requestedFrequencyBands and UE supports requestedFrequencyBands:

5> 	include in supportedBandCombination as many as possible of the band combinations included in the list of candidates, including the non-CA combinations and up to 5DL+5UL CA band combinations, determined according to the rules and priority order defined above;

5> 	include in supportedBandCombinationAdd as many as possible of the remaining band combinations included in the list of candidates, (i.e. the candidates not included in supportedBandCombination), up to 5DL+5UL CA band combinations, determined according to the rules and priority order defined above;

4> 	else

5> 	include in supportedBandCombination as many as possible of the band combinations included in the list of candidates, including the non-CA combinations and up to 5DL+5UL CA band combinations, determined according to the rules defined above;

5> 	if it is not possible to include in supportedBandCombination all the band combinations to be included according to the above, selection of the subset of band combinations to be included is left up to UE implementation;

3> 	indicate in requestedBands the same bands and in the same order as included in requestedFrequencyBands, if received;

3> 	if the UE is a category 0 or M1 UE, or supports any UE capability information in ue-RadioPagingInfo, according to MFA TS 36.306 [5]:

4> 	include ue-RadioPagingInfo and set the fields according to MFA TS 36.306 [5];

2> 	if the ue-CapabilityRequest includes geran-cs and if the UE supports GERAN CS domain:

3> 	include the UE radio access capabilities for GERAN CS within a ue-CapabilityRAT-Container and with the rat-Type set to geran-cs;

2> 	if the ue-CapabilityRequest includes geran-ps and if the UE supports GERAN PS domain:

3> 	include the UE radio access capabilities for GERAN PS within a ue-CapabilityRAT-Container and with the rat-Type set to geran-ps;

2> 	if the ue-CapabilityRequest includes utra and if the UE supports UTRA:

3> 	include the UE radio access capabilities for UTRA within a ue-CapabilityRAT-Container and with the rat-Type set to utra;

2> 	if the ue-CapabilityRequest includes cdma2000-1XRTT and if the UE supports CDMA2000 1xRTT:

3> 	include the UE radio access capabilities for CDMA2000 within a ue-CapabilityRAT-Container and with the rat-Type set to cdma2000-1XRTT;

1> 	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

### 5.6.4	CSFB to 1x Parameter transfer

#### 5.6.4.1	General

Figure 5.6.4.1-1: CSFB to 1x Parameter transfer

The purpose of this procedure is to transfer the CDMA2000 1xRTT parameters required to register the UE in the CDMA2000 1xRTT network for CSFB support.

#### 5.6.4.2	Initiation

A UE in RRC_CONNECTED initiates the CSFB to 1x parameter transfer procedure upon request from the CDMA2000 upper layers. The UE initiates the CSFB to 1x parameter transfer procedure by sending the CSFBParametersRequestCDMA2000 message.

#### 5.6.4.3	Actions related to transmission of CSFBParametersRequestCDMA2000 message

The UE shall:

1> 	submit the CSFBParametersRequestCDMA2000 message to lower layers for transmission using the current configuration;

#### 5.6.4.4	Reception of the CSFBParametersResponseCDMA2000 message

Upon reception of the CSFBParametersResponseCDMA2000 message, the UE shall:

1> 	forward the rand and the mobilityParameters to the CDMA2000 1xRTT upper layers;

### 5.6.5	UE Information

#### 5.6.5.1	General

Figure 5.6.5.1-1: UE information procedure

The UE information procedure is used by E-UTRAN to request the UE to report information.

#### 5.6.5.2	Initiation

E-UTRAN initiates the procedure by sending the UEInformationRequest message. E-UTRAN should initiate this procedure only after successful security activation.

#### 5.6.5.3	Reception of the UEInformationRequest message

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1> 	if rach-ReportReq is set to true, set the contents of the rach-Report in the UEInformationResponse message as follows:

2> 	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the last successfully completed random access procedure;

2> 	if contention resolution was not successful as specified in MFA TS 36.321 [6] for at least one of the transmitted preambles for the last successfully completed random access procedure:

3> 	set the contentionDetected to true;

2> 	else:

3> 	set the contentionDetected to false;

1> 	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2> 	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2> 	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2> 	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1> 	if connEstFailReportReq is set to true and the UE has connection establishment failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2> 	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure in E-UTRA;

2> 	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2> 	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1> 	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2> 	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3> 	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3> 	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3> 		include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3> 	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3> 	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3> 	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4> 	include the logMeasAvailable;

1> 	if mobilityHistoryReportReq is set to true:

2> 	include the mobilityHistoryReport and set it to include entries from VarMobilityHistoryReport;

2> 	include in the mobilityHistoryReport an entry for the current cell, possibly after removing the oldest entry if required, and set its fields as follows:

3> 	set visitedCellId to the global cell identity of the current cell:

3> 	set field timeSpent to the time spent in the current cell;

1> 	if the logMeasReport is included in the UEInformationResponse:

2> 	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2> 	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1> 	else:

2> 	submit the UEInformationResponse message to lower layers for transmission via SRB1;

### 5.6.6	Logged Measurement Configuration

#### 5.6.6.1	General

Figure 5.6.6.1-1: Logged measurement configuration

The purpose of this procedure is to configure the UE to perform logging of measurement results while in RRC_IDLE and to perform logging of measurement results for MBSFN in both RRC_IDLE and RRC_CONNECTED. The procedure applies to logged measurements capable UEs that are in RRC_CONNECTED.

NOTE	E-UTRAN may retrieve stored logged measurement information by means of the UE information procedure.

#### 5.6.6.2	Initiation

E-UTRAN initiates the logged measurement configuration procedure to UE in RRC_CONNECTED by sending the LoggedMeasurementConfiguration message.

#### 5.6.6.3	Reception of the LoggedMeasurementConfiguration by the UE

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1> 	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1> 	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1> 	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2> 	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1> 	else:

2> 	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1> 		store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1> 	store the received targetMBSFN-AreaList, if included, in VarLogMeasConfig;

1> 	start timer T330 with the timer value set to the loggingDuration;

#### 5.6.6.4	T330 expiry

Upon expiry of T330 the UE shall:

1> 	release VarLogMeasConfig;

The UE is allowed to discard stored logged measurements, i.e. to release VarLogMeasReport, 48 hours after T330 expiry.

### 5.6.7	Release of Logged Measurement Configuration

#### 5.6.7.1	General

The purpose of this procedure is to release the logged measurement configuration as well as the logged measurement information.

#### 5.6.7.2	Initiation

The UE shall initiate the procedure upon receiving a logged measurement configuration in another RAT. The UE shall also initiate the procedure upon power off or detach.

The UE shall:

1> 		stop timer T330, if running;

1> 	if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport;

### 5.6.8	Measurements logging

#### 5.6.8.1	General

This procedure specifies the logging of available measurements by a UE in RRC_IDLE that has a logged measurement configuration and the logging of available measurements by a UE in both RRC_IDLE and RRC_CONNECTED if targetMBSFN-AreaList is included in VarLogMeasConfig.

#### 5.6.8.2	Initiation

While T330 is running, the UE shall:

1> 	if measurement logging is suspended:

2> 	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1> 	if not suspended, perform the logging in accordance with the following:

2> 	if targetMBSFN-AreaList is included in VarLogMeasConfig:

3> 	if the UE is camping normally on an E-UTRA cell or is connected to E-UTRA; and

3> 	if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; and

3> 	if the PCell (in RRC_CONNECTED) or cell where the UE is camping (in RRC_IDLE) is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4> 	for MBSFN areas, indicated in targetMBSFN-AreaList, from which the UE is receiving MBMS service:

5> 	perform MBSFN measurements in accordance with the performance requirements as specified in MFA TS 36.133 [16];

NOTE 1:	When configured to perform MBSFN measurement logging by targetMBSFN-AreaList, the UE is not required to receive additional MBSFN subframes, i.e. logging is based on the subframes corresponding to the MBMS services the UE is receiving.

5> 	perform logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig, but only for those intervals for which MBSFN measurement results are available as specified in MFA TS 36.133 [16];

2> 	else if the UE is camping normally on an E-UTRA cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3> 	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2> 	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following

3> 	if the UE detected IDC problems during the last logging interval;

4> 	if measResultServCell in VarLogMeasReport is not empty;

5> 	include InDeviceCoexDetected;

5> 	suspend measurement logging from the next logging interval;

4> 	else;

5> 	suspend measurement logging;

NOTE 1A:	The UE may detect the start of IDC problems as early as Phase 1 as described in 23.4 of MFA TS 36.300 [9].

3> set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3> 	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4> 	include the locationCoordinates;

3> 	if targetMBSFN-AreaList is included in VarLogMeasConfig:

4> 	for each MBSFN area, for which the mandatory measurements result fields became available during the last logging interval:

5> 	set the rsrpResultMBSFN, rsrqResultMBSFN to include measurement results that became available during the last logging interval;

5> 	include the fields signallingBLER-Result or dataBLER-MCH-ResultList if the concerned BLER results are availble,

5> 	set the mbsfn-AreaId and carrierFrequency to indicate the MBSFN area in which the UE is receiving MBSFN transmission;

4> 	if in RRC_CONNECTED:

5> 	set the servCellIdentity to indicate global cell identity of the PCell;

5> 	set the measResultServCell to include the layer 3 filtered measured results of the PCell;

5> 	if available, set the measResultNeighCells to include the layer 3 filtered measured results of SCell(s) and neighbouring cell(s) measurements that became available during the last logging interval, in order of decreasing RSRP, for at most the following number of cells: 6 intra-frequency and 3 inter-frequency cells per frequency and according to the following:

6> 	for each cell included, include the optional fields that are available;

5> 	if available, optionally set the measResultNeighCells to include the layer 3 filtered measured results of neighbouring cell(s) measurements that became available during the last logging interval, in order of decreasing RSCP(UTRA)/RSSI(GERAN)/PilotStrength(cdma2000), for at most the following number of cells: 3 inter-RAT cells per frequency (UTRA, cdma2000)/set of frequencies (GERAN), and according to the following:

6> 	for each cell included, include the optional fields that are available;

4> 	if in RRC_IDLE:

5> 	set the servCellIdentity to indicate global cell identity of the serving cell;

5> 	set the measResultServCell to include the quantities of the serving cell;

5> 	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency and according to the following:

6> 	for each neighbour cell included, include the optional fields that are available;

5> 	if available, optionally set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval, for at most the following number of cells: 3 inter-RAT cells per frequency (UTRA, cdma2000)/set of frequencies (GERAN), and according to the following:

6> 	for each cell included, include the optional fields that are available;

4> 	for the cells included according to the previous (i.e. covering previous and current serving cells as well as neighbouring EUTRA cells) include results according to the extended RSRQ if corresponding results are available according to the associated performance requirements defined in MFA TS 36.133 [16];

4> 	for the cells included according to the previous (i.e. covering previous and current serving cells as well as neighbouring EUTRA cells) include RSRQ type if the result was based on measurements using a wider band or using all OFDM symbols;

NOTE 2:	The UE includes the latest results in accordance with the performance requirements as specified in MFA TS 36.133 [16]. E.g. RSRP and RSRQ results are available only if the UE has a sufficient number of results/ receives a sufficient number of subframes during the logging interval.

3> 	else:

4> 	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4> 	set the measResultServCell to include the quantities of the cell the UE is camping on;

4> 	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT and according to the following:

5> 	for each neighbour cell included, include the optional fields that are available;

4> 	for the cells included according to the previous (i.e. covering previous and current serving cells as well as neighbouring EUTRA cells) include results according to the extended RSRQ if corresponding results are available according to the associated performance requirements defined in MFA TS 36.133 [16];

4> 	for the cells included according to the previous (i.e. covering previous and current serving cells as well as neighbouring EUTRA cells) include RSRQ type if the result was based on measurements using a wider band or using all OFDM symbols;

NOTE 3:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or as used for evaluation of reporting criteria or for measurement reporting according to 5.5.3 in RRC_CONNECTED, which are performed in accordance with the performance requirements as specified in MFA TS 36.133 [16].

2> 	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

### 5.6.9	In-device coexistence indication

#### 5.6.9.1	General

Figure 5.6.9.1-1: In-device coexistence indication

The purpose of this procedure is to inform E-UTRAN about (a change of) the In-Device Coexistence (IDC) problems experienced by the UE in RRC_CONNECTED, as described in MFA TS 36.300 [9], and to provide the E-UTRAN with information in order to resolve them.

#### 5.6.9.2	Initiation

A UE capable of providing IDC indications may initiate the procedure when it is configured to provide IDC indications and upon change of IDC problem information.

Upon initiating the procedure, the UE shall:

1> 	if configured to provide IDC indications:

2> 	if the UE did not transmit an InDeviceCoexIndication message since it was configured to provide IDC indications:

3> 	if on one or more frequencies for which a measObjectEUTRA is configured, the UE is experiencing IDC problems that it cannot solve by itself; or

3> 	if configured to provide IDC indications for UL CA; and if on one or more supported UL CA combination comprising of carrier frequencies for which a measurement object is configured, the UE is experiencing IDC problems that it cannot solve by itself:

4> 	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2> 	else:

3> 	if the set of frequencies, for which a measObjectEUTRA is configured and on which the UE is experiencing IDC problems that it cannot solve by itself, is different from the set indicated in the last transmitted InDeviceCoexIndication message; or

3> 	if for one or more of the frequencies in the previously reported set of frequencies, the interferenceDirection is different from the value indicated in the last transmitted InDeviceCoexIndication message; or

3> 	if the TDM assistance information is different from the assistance information included in the last transmitted InDeviceCoexIndication message; or

3> 	if configured to provide IDC indications for UL CA; and if the victimSystemType is different from the value indicated in the last transmitted InDeviceCoexIndication message; or

3> 	if configured to provide IDC indications for UL CA; and if the set of supported UL CA combinations on which the UE is experiencing IDC problems that it cannot solve by itself and that the UE includes in affectedCarrierFreqCombList according to 5.6.9.3, is different from the set indicated in the last transmitted InDeviceCoexIndication message:

4> 	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

NOTE 1:	The term “IDC problems” refers to interference issues applicable across several subframes/slots where not necessarily all the subframes/slots are affected.

NOTE 2:	For the frequencies on which a serving cell or serving cells is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds.
For frequencies on which a SCell or SCells is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell or SCells would result in interference issues that the UE would not be able to solve by itself.
For a non-serving frequency, reporting IDC problems indicates an anticipation that if the non-serving frequency or frequencies became a serving frequency or serving frequencies then this would result in interference issues that the UE would not be able to solve by itself.

#### 5.6.9.3	Actions related to transmission of InDeviceCoexIndication message

The UE shall set the contents of the InDeviceCoexIndication message as follows:

1> 	if there is at least one E-UTRA carrier frequency, for which a measurement object is configured, that is affected by IDC problems:

2> 	include the field affectedCarrierFreqList with an entry for each affected E-UTRA carrier frequency for which a measurement object is configured;

2> 	for each E-UTRA carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly;

2> 	include Time Domain Multiplexing (TDM) based assistance information, unless idc-HardwareSharingIndication is configured and the UE has no Time Doman Multiplexing based assistance information that could be used to resolve the IDC problems:

3> 	if the UE has DRX related assistance information that could be used to resolve the IDC problems:

4> 	include drx-CycleLength, drx-Offset and drx-ActiveTime;

3> 	else (the UE has desired subframe reservation patterns related assistance information that could be used to resolve the IDC problems):

4> 	include idc-SubframePatternList;

3> 	use the MCG as timing reference if TDM based assistance information regarding the SCG is included;

1> 	if the UE is configured to provide UL CA information and there is a supported UL CA combination comprising of carrier frequencies for which a measurement object is configured, that is affected by IDC problems:

2> 	include victimSystemType in ul-CA-AssistanceInfo;

2> 	if the UE sets victimSystemType to wlan or Bluetooth:

3> 	include affectedCarrierFreqCombList in ul-CA-AssistanceInfo with an entry for each supported UL CA combination comprising of carrier frequencies for which a measurement object is configured, that is affected by IDC problems;

2> 	else:

3> 	optionally include affectedCarrierFreqCombList in ul-CA-AssistanceInfo with an entry for each supported UL CA combination comprising of carrier frequencies for which a measurement object is configured, that is affected by IDC problems;

1>	if idc-HardwareSharingIndication is configured, and there is at least one E-UTRA carrier frequency, for which a measurement object is configured, the UE is experiencing hardware sharing problems that it cannot solve by itself:

2>	include the hardwareSharingProblem and set it accordingly;

NOTE 1:	When sending an InDeviceCoexIndication message to inform E-UTRAN the IDC problems, the UE includes all assistance information (rather than providing e.g. the changed part(s) of the assistance information).

NOTE 2:	Upon not anymore experiencing a particular IDC problem that the UE previously reported, the UE provides an IDC indication with the modified contents of the InDeviceCoexIndication message (e.g. by an empty message).

The UE shall submit the InDeviceCoexIndication message to lower layers for transmission.

### 5.6.10	UE Assistance Information

#### 5.6.10.1	General

Figure 5.6.10.1-1: UE Assistance Information

The purpose of this procedure is to inform E-UTRAN of the UE’s power saving preference. Upon configuring the UE to provide power preference indications E-UTRAN may consider that the UE does not prefer a configuration primarily optimised for power saving until the UE explictly indicates otherwise.

#### 5.6.10.2	Initiation

A UE capable of providing power preference indications in RRC_CONNECTED may initiate the procedure in several cases including upon being configured to provide power preference indications and upon change of power preference.

Upon initiating the procedure, the UE shall:

1> 	if configured to provide power preference indications:

2> 	if the UE did not transmit a UEAssistanceInformation message since it was configured to provide power preference indications; or

2> 	if the current power preference is different from the one indicated in the last transmission of the UEAssistanceInformation message and timer T340 is not running:

3> 	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

#### 5.6.10.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message:

1> 	if the UE prefers a configuration primarily optimised for power saving:

2> 	set powerPrefIndication to lowPowerConsumption;

1> 	else:

2> 	start or restart timer T340 with the timer value set to the powerPrefIndicationTimer;

2> 	set powerPrefIndication to normal;

The UE shall submit the UEAssistanceInformation message to lower layers for transmission.

### 5.6.11	Mobility history information

#### 5.6.11.1	General

This procedure specifies how the mobility history information is stored by the UE, covering RRC_CONNECTED and RRC_IDLE.

#### 5.6.11.2	Initiation

If the UE supports storage of mobility history information, the UE shall:

1>	Upon change of cell, consisting of PCell in RRC_CONNECTED or serving cell in RRC_IDLE, to another E-UTRA or inter-RAT cell or when entering out of service:

2>	include an entry in variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	 if the global cell identity of the previous PCell/ serving cell is available:

4>	 include the global cell identity of that cell in the field visitedCellId of the entry;

3>	else:

4>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

3>	set the field timeSpent of the entry as the time spent in the previous PCell/ serving cell;

1>	upon entering E-UTRA (in RRC_CONNECTED or RRC_IDLE) while previously out of service and/ or using another RAT:

2>	include an entry in variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	set the field timeSpent of the entry as the time spent outside E-UTRA;

### 5.6.12	RAN-assisted WLAN interworking

#### 5.6.12.1	General

The purpose of this procedure is to facilitate access network selection and traffic steering between E-UTRAN and WLAN.

If required by upper layers (see 3GPP TS 24.312 [66], the UE shall provide an up-to-date set of the applicable parameters provided by wlan-OffloadConfigCommon or wlan-OffloadConfigDedicated to upper layers, and inform upper layers when no parameters are configured. The parameter set from either wlan-OffloadConfigCommon or wlan-OffloadConfigDedicated is selected as specified in subclauses 5.2.2.24, 5.3.12, 5.6.12.2 and 5.6.12.4.

#### 5.6.12.2	Dedicated WLAN offload configuration

The UE shall:

1>	if the received wlan-OffloadInfo is set to release:

2>	release wlan-OffloadConfigDedicated and t350;

2>	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

3>	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

1>	else:

2>	apply the received wlan-OffloadConfigDedicated:

#### 5.6.12.3	WLAN offload RAN evaluation

The UE shall:

1> 	if the UE is configured with either wlan-OffloadConfigCommon or wlan-OffloadConfigDedicated; and

1> 	if the UE is in RRC_IDLE or none of rclwi-Configuration, lwa-Configuration and lwip-Configuration is configured:

2> 	provide measurement results required for the evaluation of the network selection and traffic steering rules as defined in 3GPP TS 24.312 [66] to upper layers;

2> 	evaluate the network selection and traffic steering rules as defined in MFA TS 36.304 [4] using WLAN identifiers as indicated in other subclauses (either provided in steerToWLAN included in rclwi-Configuration or in wlan-Id-List included in SystemInformationBlockType17);

#### 5.6.12.4	T350 expiry or stop

The UE shall:

1> 	if T350 expires or is stopped:

2> 	release the wlan-OffloadConfigDedicated and t350;

2> 	release rclwi-Configuration if configured;

2> 	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

3> 	apply the wlan-OffloadConfigCommon and the wlan-Id-List corresponding to the RPLMN included in SystemInformationBlockType17;

#### 5.6.12.5	Cell selection/re-selection while T350 is running

The UE shall:

1> 	if, while T350 is running, the UE selects/ reselects a cell which is not the PCell when the wlan-OffloadDedicated was configured:

2> 	stop timer T350;

2> 	perform the actions as specified in 5.6.12.4;

### 5.6.13	SCG failure information

#### 5.6.13.1	General

Figure 5.6.13.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure.

#### 5.6.13.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1> 	upon detecting radio link failure for the SCG, in accordance with 5.3.11; or

1> 	upon SCG change failure, in accordance with 5.3.5.7a; or

1> 	upon stopping uplink transmission towards the PSCell due to exceeding the maximum uplink transmission timing difference when powerControlMode is configured to 1, in accordance with subclause 7.17.2 of 3GPP TS 36.133 [29].

Upon initiating the procedure, the UE shall:

1> 	suspend all SCG DRBs and suspend SCG transmission for split DRBs;

1> 	reset SCG-MAC;

1> 	stop T307;

1> 	initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;

#### 5.6.13.3	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1> 	if the UE initiates transmission of the SCGFailureInformation message to provide SCG radio link failure information:

2> 	include failureType and set it to the trigger for detecting SCG radio link failure;

1> 	else if the UE initiates transmission of the SCGFailureInformation message to provide SCG change failure information:

2> 	include failureType and set it to scg-ChangeFailure;

1> 	else if the UE initiates transmission of the SCGFailureInformation message due to exceeding maximum uplink transmission timing difference:

2> 	include failureType and set it to maxUL-TimingDiff;

1> 	set the measResultServFreqList to include for each SCG cell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in MFA TS 36.133 [16];

1> 	for each SCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1> 	set the measResultNeighCells to include the best measured cells on non-serving E-UTRA frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2> 	if the UE was configured to perform measurements for one or more non-serving EUTRA frequencies and measurement results are available, include the measResultListEUTRA;

2> 	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

### 5.6.14	LTE-WLAN Aggregation

#### 5.6.14.1	Introduction

E-UTRAN can configure the UE to connect to a WLAN and configure bearers for LWA (referred to as LWA DRBs). The UE uses the WLAN parameters received from E-UTRAN in performing WLAN measurements. The UE also performs WLAN connection management as described in 5.6.15 while LWA is configured.

#### 5.6.14.2	Reception of LWA configuration

Upon reception of LWA configuration, the UE shall:

1> 	if the received lwa-Configuration is set to release:

2> 	release the LWA configuration as described in 5.6.14.3;

1> 	else:

2> 	if the received lwa-Config includes lwa-WT-Counter:

3> 	determine the S-KWT key based on the KeNB key and received lwa-WT-Counter value, as specified in MFA TS 33.401 [32];

3> 	forward the S-KWT key to upper layers to be used as a PMK or PSK for WLAN authentication;

2> 	if the received lwa-Config includes lwa-MobilityConfig:

3> 	if the received lwa-MobilityConfig includes wlan-ToReleaseList:

4> 	for each WLAN-Identifiers included in wlan-ToReleaseList:

5> 	remove the WLAN-Identifiers if already part of the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3> 	if the received lwa-MobilityConfig includes wlan-ToAddList:

4> 	for each WLAN-Identifiers included in wlan-ToAddList:

5> 	add the WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3> 	if the received lwa-MobilityConfig includes associationTimer:

4> 	start or restart timer T351 with the timer value set to the associationTimer;

3> 	if the received lwa-MobilityConfig includes successReportRequested:

4> 	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3> 	start WLAN Status Monitoring as described in 5.6.15.4;

#### 5.6.14.3	Release of LWA configuration

To release the LWA configuration, the UE shall:

- for each LWA DRB that is part of the current UE configuration:

2> 	disable data handling for this DRB at the LWAAP entity;

2> 	perform PDCP data recovery as specified in 3GPP TS 36.323 [8];

1> 	delete any existing values in VarWLAN-MobilityConfig and VarWLAN-Status;

1> 	stop timer T351, if running;

1> 	stop WLAN status monitoring and WLAN connection attempts for LWA;

### 5.6.15	WLAN connection management

#### 5.6.15.1	Introduction

WLAN connection management procedures in this section are triggered as specified in other sections where the UE is using a WLAN connection for LWA, RCLWI or LWIP.

The UE stores the current WLAN mobility set, which is a set of one or more WLAN identifier(s) (e.g. BSSID, SSID, HESSID) in wlan-MobilitySet in VarWLAN-MobilityConfig. This WLAN mobility set can be configured and updated by the eNB. A WLAN is considered to be inside the WLAN mobility set if its identifiers match all WLAN identifiers of at least one entry in wlan-MobilitySet and outside the WLAN mobility set otherwise. When the UE receives a new or updated WLAN mobility set, it initiates connection to a WLAN inside the WLAN mobility set, if not already connected to such a WLAN, and starts WLAN status monitoring as described in 5.6.15.4. The UE can perform WLAN mobility within the WLAN mobility set (connect or reconnect to a WLAN inside the WLAN mobility set) without any signalling to E-UTRAN.

The UE reports the WLAN connection status information to E-UTRAN as described in 5.6.15.2. The information in this report is based on the monitoring of WLAN connection as described in 5.6.15.4.

#### 5.6.15.2	WLAN connection status reporting

##### 5.6.15.2.1	General

Figure 5.6.15.2.1-1: WLAN connection status reporting

The purpose of this procedure is to inform E-UTRAN about the status of WLAN connection for LWA, RCLWI, or LWIP.

##### 5.6.15.2.2	Initiation

The UE in RRC_CONNECTED initiates the WLAN status reporting procedure when it connects successfully to a WLAN inside WLAN mobility set while T351 is running after a WLAN mobility set change or after a lwa-WT-Counter update (if success report is requested by the eNB) or its connection or connection attempts to all WLAN(s) inside WLAN mobility set fails in accordance with WLAN Status Monitoring described in 5.6.15.4 or when T351 expires.

Upon initiating the procedure, the UE shall:

1> 	initiate transmission of the WLANConnectionStatusReport message in accordance with 5.6.15.2.3;

##### 5.6.15.2.3	Actions related to transmission of WLANConnectionStatusReport message

The UE shall set the contents of the WLANConnectionStatusReport message as follows:

1>	set wlan-status to status in VarWLAN-Status;

1>	submit the WLANConnectionStatusReport message to lower layers for transmission, upon which the procedure ends;

#### 5.6.15.3	T351 Expiry (WLAN connection attempt timeout)

Upon T351 expiry, the UE shall:

1>	set the status in VarWLAN-Status to failureTimeout;

1>	perform WLAN connection status reporting procedure in 5.6.15.2;

1>	stop WLAN status monitoring and WLAN connection attempts;

#### 5.6.15.4	WLAN status monitoring

To perform WLAN status monitoring, the UE shall:

1>	if UE is not configured with rclwi-Configuration and WLAN connection to a WLAN inside the WLAN mobility set is successfully established or maintained after a WLAN mobility set configuration update or after a lwa-WT-Counter update:

2> 	set the status in VarWLAN-Status to successfulAssociation;

2> 	stop timer T351, if running;

2> 	if successReportRequested in VarWLAN-MobilityConfig is set to TRUE:

3> 	perform WLAN Connection Status Reporting procedure in 5.6.15.2;

1>	if WLAN connection or connection attempts to all WLAN(s) inside WLAN mobility set fails:

2> 	if the failure is due to WLAN radio link issues:

3> 	set the status in VarWLAN-Status to failureWlanRadioLink;

2> 	else if the failure is due to UE internal problems related to WLAN:

3> 	set the status in VarWLAN-Status to failureWlanUnavailable;

NOTE 1:	The UE internal problems related to WLAN includes connection to another WLAN based on user preferences or turning off WLAN connection or connection rejection from WLAN or other WLAN problems.

2> 	stop timer T351, if running;

2> 	perform WLAN Connection Status Reporting procedure in 5.6.15.2;

2> 	if the UE is configured with rclwi-Configuration:

3> 	release rclwi-Configuration and inform upper layers of a move-traffic-from-WLAN indication (see 3GPP TS 24.302 [74]);

2> 	stop WLAN Status Monitoring and WLAN connection attempts;

### 5.6.16	RAN controlled LTE-WLAN interworking

#### 5.6.16.1	General

The purpose of this procedure is to perform RAN-controlled LTE-WLAN interworking (RCLWI) i.e. control access network selection and traffic steering between E-UTRAN and WLAN.

#### 5.6.16.2	WLAN traffic steering command

The UE shall:

1>	if the received rclwi-Configuration is set to setup:

2> 	if the command is set to steerToWLAN:

3> 	inform the upper layers of a move-traffic-to-WLAN indication along with the WLAN identifier lists in steerToWLAN (see 3GPP TS 24.302 [74]);

3> 	store steerToWLAN in wlan-MobilitySet in VarWLAN-MobilityConfig;

3> 	perform the WLAN status monitoring procedure as specified in 5.6.15.4 using steerToWLAN as the WLAN mobility set;

2> 	else:

3> 	inform the upper layers of a move-traffic-from-WLAN indication (see 3GPP TS 24.302 [74]);

3> 	clear wlan-MobilitySet in VarWLAN-MobilityConfig;

3> 	stop performing the WLAN status monitoring procedure as specified in 5.6.15.4;

3> 	delete any existing values in VarWLAN-Status;

1>	else (the rclwi-Configuration is released):

2> 	clear wlan-MobilitySet in VarWLAN-MobilityConfig;

2> 	stop performing the WLAN status monitoring procedure as specified in 5.6.15.4;

2> 	delete any existing values in VarWLAN-Status;

2> inform the upper layers of release of the rclwi-Configuration.

### 5.6.17	LTE-WLAN aggregation with IPsec tunnel

#### 5.6.17.1	General

The WLAN resources that are used over the LWIP tunnel as described in MFA TS 36.300 [9] established as part of LWIP procedures are referred to as 'LWIP resources'. The purpose of this section is to specify procedures to indicate to higher layers to initiate the establishment/ release of the LWIP tunnel over WLAN and to indicate which DRB(s) shall use the LWIP resources.

#### 5.6.17.2	LWIP reconfiguration

The UE shall:

1>	if the received lwip-Configuration is set to release:

2> 	release the LWIP configuration, if configured, as described in 5.6.17.3;

1>	else:

2> 	if lwip-MobilityConfig is included:

3> 	if the received lwip-MobilityConfig includes wlan-ToReleaseList:

4> 	for each WLAN-Identifiers included in wlan-ToReleaseList:

5> 	remove the WLAN-Identifiers if already part of the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3> 	if the received lwip-MobilityConfig includes wlan-ToAddList:

4> 	for each WLAN-Identifiers included in wlan-ToAddList:

5> 	add the WLAN-Identifiers to the current wlan-MobilitySet in VarWLAN-MobilityConfig;

3> 	if the received lwip-MobilityConfig includes associationTimer:

4> 	start timer T351 with the timer value set according to the value of associationTimer;

3> 	if the received lwip-MobilityConfig includes successReportRequested:

4> 	set successReportRequested in VarWLAN-MobilityConfig to the value of successReportRequested;

3> 	start WLAN Status Monitoring as described in 5.6.15.4;

2> 	if tunnelConfigLWIP is included:

3> 	indicate to higher layers to configure the LWIP tunnel according to the received tunnelConfigLWIP (see MFA TS 33.401 [32]);

2> 	if lwip-Counter is included:

3> 	determine the LWIP-PSK based on the KeNB key and received lwip-Counter value, as specified in MFA TS 33.401 [32];

3> 	forward the LWIP-PSK to upper layers for LWIP tunnel establishment;

#### 5.6.17.3	LWIP release

The UE shall:

1>	delete any existing values in VarWLAN-MobilityConfig and VarWLAN-Status;

1>	stop timer T351, if running;

1>	release the lwip-Configuration;

1>	indicate to higher layers to stop all DRBs from using the LWIP resources;

1> 	indicate to higher layers to release the LWIP tunnel (see MFA TS 33.401 [32]);

1> 		stop WLAN status monitoring and WLAN connection attempts for LWIP;

## 5.7	Generic error handling

### 5.7.1	General

The generic error handling defined in the subsequent sub-clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE.

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/ reserved field.

The UE shall consider an MF specific extension to messages as not comprehended when its type does not correspond to the type of the paired 3gpp baseline message, as described in section 6.2.2MF.

### 5.7.2	ASN.1 violation or encoding error

The UE shall:

1> 	when receiving an RRC message on the BCCH, BR-BCCH, PCCH, CCCH, MCCH, SC-MCCH or SBCCH for which the abstract syntax is invalid (see ITU-T Rec. X.680 [13]):

2> 	ignore the message;

NOTE:	This section applies in case one or more fields is set to a value, other than a spare, reserved or extended value, not defined in this version of the transfer syntax. E.g. in the case the UE receives value 12 for a field defined as INTEGER (1..11). In cases like this, it may not be possible to reliably detect which field is in the error hence the error handling is at the message level.

### 5.7.3	Field set to a not comprehended value

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2> 	if a default value is defined for this field:

3> 	treat the message while using the default value defined for this field;

2> 	else if the concerned field is optional:

3> 	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2> 	else:

3> 	treat the message as if the field were absent and in accordance with sub-clause 5.7.4;

### 5.7.4	Mandatory field missing

The UE shall:

1>	if the message includes a field that is mandatory to include in the message (e.g. because conditions for mandatory presence are fulfilled) and that field is absent or treated as absent:

2> 	if the RRC message was received on DCCH or CCCH:

3> 	ignore the message;

2> 	else:

3> 	if the field concerns a (sub-field of) an entry of a list (i.e. a SEQUENCE OF):

4> 	treat the list as if the entry including the missing or not comprehended field was not present;

3> 	else if the field concerns a sub-field of another field, referred to as the 'parent' field i.e. the field that is one nesting level up compared to the erroneous field:

4> 	consider the 'parent' field to be set to a not comprehended value;

4> 	apply the generic error handling to the subsequent 'parent' field(s), until reaching the top nesting level i.e. the message level;

3> 	else (field at message level):

4> 	ignore the message;

NOTE 1:	The error handling defined in these sub-clauses implies that the UE ignores a message with the message type or version set to a not comprehended value.

NOTE 2:	The nested error handling for messages received on logical channels other than DCCH and CCCH applies for errors in extensions also, even for errors that can be regarded as invalid E-UTRAN operation e.g. E-UTRAN not observing conditional presence.

The following ASN.1 further clarifies the levels applicable in case of nested error handling for errors in extension fields.

-- /example/ ASN1START

-- Example with extension addition group

ItemInfoList ::=					SEQUENCE (SIZE (1..max)) OF ItemInfo

ItemInfo ::=						SEQUENCE {

itemIdentity						INTEGER (1..max),

field1								Field1,

field2								Field2					OPTIONAL,			-- Need ON

...

[[	field3-r9						Field3-r9				OPTIONAL,			-- Cond Cond1

field4-r9						Field4-r9				OPTIONAL			-- Need ON

]]

}

-- Example with traditional non-critical extension (empty sequence)

BroadcastInfoBlock1 ::=				SEQUENCE {

itemIdentity						INTEGER (1..max),

field1								Field1,

field2								Field2					OPTIONAL,			-- Need ON

nonCriticalExtension				BroadcastInfoBlock1-v940-IEs	OPTIONAL

}

BroadcastInfoBlock1-v940-IEs::=	SEQUENCE {

field3-r9							Field3-r9				OPTIONAL,			-- Cond Cond1

field4-r9							Field4-r9				OPTIONAL,			-- Need ON

nonCriticalExtension				SEQUENCE {}				OPTIONAL			-- Need OP

}

-- ASN1STOP

The UE shall, apply the following principles regarding the levels applicable in case of nested error handling:

-	an extension additon group is not regarded as a level on its own. E.g. in the ASN.1 extract in the previous, a error regarding the conditionality of field3 would result in the entire itemInfo entry to be ignored (rather than just the extension addition group containing field3 and field4)

-	a traditional nonCriticalExtension is not regarded as a level on its own. E.g. in the ASN.1 extract in the previous, a error regarding the conditionality of field3 would result in the entire BroadcastInfoBlock1 to be ignored (rather than just the non critical extension containing field3 and field4).

### 5.7.5	Not comprehended field

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that the UE does not comprehend:

2> 	treat the rest of the message as if the field was absent;

NOTE:	This section does not apply to the case of an extension to the value range of a field. Such cases are addressed instead by the requirements in section 5.7.3.

### 5.7.5MF1	Not comprehended MF messages or message extensions

The UE shall, when receiving an RRC message on PCCH, CCCH, or DCCH logical channel:

1>		if the message-mf contents do not correspond to the message-3gpp baseline as specified in 6.7MF.2:

2>	treat the message as if message-mf portion was absent, unless otherwise specified in the message procedure in section 5.3;

## 5.8	MBMS

### 5.8.1	Introduction

#### 5.8.1.1	General

In general the control information relevant only for UEs supporting MBMS is separated as much as possible from unicast control information. Most of the MBMS control information is provided on a logical channel specific for MBMS common control information: the MCCH. E-UTRA employs one MCCH logical channel per MBSFN area. In case the network configures multiple MBSFN areas, the UE acquires the MBMS control information from the MCCHs that are configured to identify if services it is interested to receive are ongoing. The action applicable when the UE is unable to simultaneously receive MBMS and unicast services is up to UE implementation. In this release of the specification, an MBMS capable UE is only required to support reception of a single MBMS service at a time, and reception of more than one MBMS service (also possibly on more than one MBSFN area) in parallel is left for UE implementation. The MCCH carries the MBSFNAreaConfiguration message, which indicates the MBMS sessions that are ongoing as well as the (corresponding) radio resource configuration. The MCCH may also carry the MBMSCountingRequest message, when E-UTRAN wishes to count the number of UEs in RRC_CONNECTED that are receiving or interested to receive one or more specific MBMS services.

A limited amount of MBMS control information is provided on the BCCH. This primarily concerns the information needed to acquire the MCCH(s). This information is carried by means of a single MBMS specific SystemInformationBlock: SystemInformationBlockType13. An MBSFN area is identified solely by the mbsfn-AreaId in SystemInformationBlockType13. At mobility, the UE considers that the MBSFN area is continuous when the source cell and the target cell broadcast the same value in the mbsfn-AreaId.

#### 5.8.1.2	Scheduling

The MCCH information is transmitted periodically, using a configurable repetition period. Scheduling information is not provided for MCCH i.e. both the time domain scheduling as well as the lower layer configuration are semi-statically configured, as defined within SystemInformationBlockType13.

For MBMS user data, which is carried by the MTCH logical channel, E-UTRAN periodically provides MCH scheduling information (MSI) at lower layers (MAC). This MCH information only concerns the time domain scheduling i.e. the frequency domain scheduling and the lower layer configuration are semi-statically configured. The periodicity of the MSI is configurable and defined by the MCH scheduling period.

#### 5.8.1.3	MCCH information validity and notification of changes

Change of MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period). The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by means of SystemInformationBlockType13.

When the network changes (some of) the MCCH information, it notifies the UEs about the change during a first modification period. In the next modification period, the network transmits the updated MCCH information. These general principles are illustrated in figure 5.8.1.3-1, in which different colours indicate different MCCH information. Upon receiving a change notification, a UE interested to receive MBMS services acquires the new MCCH information immediately from the start of the next modification period. The UE applies the previously acquired MCCH information until the UE acquires the new MCCH information.

Figure 5.8.1.3-1: Change of MCCH Information

Indication of an MBMS specific RNTI, the M-RNTI (see MFA TS 36.321 [6]), on PDCCH is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about an MCCH information change. When receiving an MCCH information change notification, the UE knows that the MCCH information will change at the next modification period boundary. The notification on PDCCH indicates which of the MCCHs will change, which is done by means of an 8-bit bitmap. Within this bitmap, the bit at the position indicated by the field notificationIndicator is used to indicate changes for that MBSFN area: if the bit is set to "1", the corresponding MCCH will change. No further details are provided e.g. regarding which MCCH information will change. The MCCH information change notification is used to inform the UE about a change of MCCH information upon session start or about the start of MBMS counting.

The MCCH information change notifications on PDCCH are transmitted periodically and are carried on MBSFN subframes only. These MCCH information change notification occasions are common for all MCCHs that are configured, and configurable by parameters included in SystemInformationBlockType13: a repetition coefficient, a radio frame offset and a subframe index. These common notification occasions are based on the MCCH with the shortest modification period.

NOTE 1:	E-UTRAN may modify the MBMS configuration information provided on MCCH at the same time as updating the MBMS configuration information carried on BCCH i.e. at a coinciding BCCH and MCCH modification period. Upon detecting that a new MCCH is configured on BCCH, a UE interested to receive one or more MBMS services should acquire the MCCH, unless it knows that the services it is interested in are not provided by the corresponding MBSFN area.

A UE that is receiving an MBMS service via MRB shall acquire the MCCH information from the start of each modification period. A UE interested to receive MBMS from a carrier on which dl-Bandwidth included in MasterInformationBlock is set to n6 shall acquire the MCCH information at least once every MCCH modification period. A UE that is not receiving an MBMS service via MRB, as well as UEs that are receiving an MBMS service via MRB but potentially interested to receive other services not started yet in another MBSFN area from a carrier on which dl-Bandwidth included in MasterInformationBlock is other than n6, shall verify that the stored MCCH information remains valid by attempting to find the MCCH information change notification at least notificationRepetitionCoeff times during the modification period of the applicable MCCH(s), if no MCCH information change notification is received.

NOTE 2:	In case the UE is aware which MCCH(s) E-UTRAN uses for the service(s) it is interested to receive, the UE may only need to monitor change notifications for a subset of the MCCHs that are configured, referred to as the 'applicable MCCH(s)' in the above.

### 5.8.2	MCCH information acquisition

#### 5.8.2.1	General

Figure 5.8.2.1-1: MCCH information acquisition

The UE applies the MCCH information acquisition procedure to acquire the MBMS control information that is broadcasted by the E-UTRAN. The procedure applies to MBMS capable UEs that are in RRC_IDLE or in RRC_CONNECTED.

#### 5.8.2.2	Initiation

A UE interested to receive MBMS services shall apply the MCCH information acquisition procedure upon entering the corresponding MBSFN area (e.g. upon power on, following UE mobility) and upon receiving a notification that the MCCH information has changed. A UE that is receiving an MBMS service shall apply the MCCH information acquisition procedure to acquire the MCCH, that corresponds with the service that is being received, at the start of each modification period.

Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information unless explicitly specified otherwise.

#### 5.8.2.3	MCCH information acquisition by the UE

An MBMS capable UE shall:

1>	if the procedure is triggered by an MCCH information change notification:

2> 	start acquiring the MBSFNAreaConfiguration message and the MBMSCountingRequest message if present, from the beginning of the modification period following the one in which the change notification was received;

NOTE 1:	The UE continues using the previously received MCCH information until the new MCCH information has been acquired.

1>	if the UE enters an MBSFN area:

2> 	acquire the MBSFNAreaConfiguration message and the MBMSCountingRequest message if present, at the next repetition period;

1>	if the UE is receiving an MBMS service:

2> 	start acquiring the MBSFNAreaConfiguration message and the MBMSCountingRequest message if present, that both concern the MBSFN area of the service that is being received, from the beginning of each modification period;

5.8.2.4	Actions upon reception of the MBSFNAreaConfiguration message

No UE requirements related to the contents of this MBSFNAreaConfiguration apply other than those specified elsewhere e.g. within procedures using the concerned system information, the corresponding field descriptions.

5.8.2.5	Actions upon reception of the MBMSCountingRequest message

Upon receiving MBMSCountingRequest message, the UE shall perform the MBMS Counting procedure as specified in 5.8.4.

### 5.8.3	MBMS PTM radio bearer configuration

#### 5.8.3.1	General

The MBMS PTM radio bearer configuration procedure is used by the UE to configure RLC, MAC and the physical layer upon starting and/or stopping to receive an MRB. The procedure applies to UEs interested to receive one or more MBMS services.

NOTE:	In case the UE is unable to receive an MBMS service due to capability limitations, upper layers may take appropriate action e.g. terminate a lower priority unicast service.

#### 5.8.3.2	Initiation

The UE applies the MRB establishment procedure to start receiving a session of a service it has an interest in. The procedure may be initiated e.g. upon start of the MBMS session, upon (re-)entry of the corresponding MBSFN service area, upon becoming interested in the MBMS service, upon removal of UE capability limitations inhibiting reception of the concerned service.

The UE applies the MRB release procedure to stop receiving a session. The procedure may be initiated e.g. upon stop of the MBMS session, upon leaving the corresponding MBSFN service area, upon losing interest in the MBMS service, when capability limitations start inhibiting reception of the concerned service.

#### 5.8.3.3	MRB establishment

Upon MRB establishment, the UE shall:

1>	establish an RLC entity in accordance with the configuration specified in 9.1.1.4;

1>	configure an MTCH logical channel in accordance with the received locgicalChannelIdentity, applicable for the MRB, as included in the MBSFNAreaConfiguration message;

1>	configure the physical layer in accordance with the pmch-Config, applicable for the MRB, as included in the MBSFNAreaConfiguration message;

1>	inform upper layers about the establishment of the MRB by indicating the corresponding tmgi and sessionId;

#### 5.8.3.4	MRB release

Upon MRB release, the UE shall:

1>	release the RLC entity as well as the related MAC and physical layer configuration;

1>	inform upper layers about the release of the MRB by indicating the corresponding tmgi and sessionId;

### 5.8.4	MBMS Counting Procedure

#### 5.8.4.1	General

Figure 5.8.4.1-1: MBMS Counting procedure

The MBMS Counting procedure is used by the E-UTRAN to count the number of RRC_CONNECTED mode UEs which are receiving via an MRB or interested to receive via an MRB the specified MBMS services.

The UE determines interest in an MBMS service, that is identified by the TMGI, by interaction with upper layers.

#### 5.8.4.2	Initiation

E-UTRAN initiates the procedure by sending an MBMSCountingRequest message.

#### 5.8.4.3	Reception of the MBMSCountingRequest message by the UE

Upon receiving the MBMSCountingRequest message, the UE in RRC_CONNECTED mode shall:

1>	if the SystemInformationBlockType1, that provided the scheduling information for the systemInformationBlockType13 that included the configuration of the MCCH via which the MBMSCountingRequest message was received, contained the identity of the Registered PLMN; and

1>		if the UE is receiving via an MRB or interested to receive via an MRB at least one of the services in the received countingRequestList:

2> 	if more than one entry is included in the mbsfn-AreaInfoList received in the SystemInformationBlockType13 that included the configuration of the MCCH via which the MBMSCountingRequest message was received:

3> 	include the mbsfn-AreaIndex in the MBMSCountingResponse message and set it to the index of the entry in the mbsfn-AreaInfoList within the received SystemInformationBlockType13 that corresponds with the MBSFN area used to transfer the received MBMSCountingRequest message;

2> 	for each MBMS service included in the received countingRequestList:

3> 	if the UE is receiving via an MRB or interested to receive via an MRB this MBMS service:

4> 	include an entry in the countingResponseList within the MBMSCountingResponse message with countingResponseService set it to the index of the entry in the countingRequestList within the received MBMSCountingRequest that corresponds with the MBMS service the UE is receiving or interested to receive;

2> 	submit the MBMSCountingResponse message to lower layers for transmission upon which the procedure ends;

NOTE 1:	UEs that are receiving an MBMS User Service (see 3GPP TS 23.246 [56]) by means of a Unicast Bearer Service (see 3GPP TS 26.346 [57]) (i.e. via a DRB), but are interested to receive the concerned MBMS User Service (see 3GPP TS 23.246 [56]) via an MBMS Bearer Service (i.e. via an MRB), respond to the counting request.

NOTE 2:	If ciphering is used at upper layers, the UE does not respond to the counting request if it can not decipher the MBMS service for which counting is performed (see 3GPP TS 22.146 [62, Section 5.3]).

NOTE 3:	The UE treats the MBMSCountingRequest messages received in each modification period independently. In the unlikely case E-UTRAN would repeat an MBMSCountingRequest (i.e. including the same services) in a subsequent modification period, the UE responds again. The UE provides at most one MBMSCountingResponse message to multiple transmission attempts of an MBMSCountingRequest messages in a given modification period.

### 5.8.5	MBMS interest indication

#### 5.8.5.1	General

Figure 5.8.5.1-1: MBMS interest indication

The purpose of this procedure is to inform E-UTRAN that the UE is receiving or is interested to receive MBMS service(s) via an MRB or SC-MRB, and if so, to inform E-UTRAN about the priority of MBMS versus unicast reception.

#### 5.8.5.2	Initiation

An MBMS or SC-PTM capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment, upon entering or leaving the service area, upon session start or stop, upon change of interest, upon change of priority between MBMS reception and unicast reception or upon change to a PCell broadcasting SystemInformationBlockType15.

Upon initiating the procedure, the UE shall:

1>	if SystemInformationBlockType15 is broadcast by the PCell:

2> 	ensure having a valid version of SystemInformationBlockType15 for the PCell;

2> 	if the UE did not transmit an MBMSInterestIndication message since last entering RRC_CONNECTED state; or

2> 	if since the last time the UE transmitted an MBMSInterestIndication message, the UE connected to a PCell not broadcasting SystemInformationBlockType15:

3> 	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

4> 	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2> 	else:

3> 	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, has changed since the last transmission of the MBMSInterestIndication message; or

3> 	if the prioritisation of reception of all indicated MBMS frequencies compared to reception of any of the established unicast bearers has changed since the last transmission of the MBMSInterestIndication message:

4> 	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

NOTE:	The UE may send an MBMSInterestIndication even when it is able to receive the MBMS services it is interested in i.e. to avoid that the network allocates a configuration inhibiting MBMS reception.

3> 	else if SystemInformationBlockType20 is broadcast by the PCell:

4> 	if since the last time the UE transmitted an MBMSInterestIndication message, the UE connected to a PCell not broadcasting SystemInformationBlockType20; or

4> 	if the set of MBMS services of interest determined in accordance with 5.8.5.3a is different from mbms-Services included in the last transmission of the MBMSInterestIndication message;

5> 	initiate the transmission of the MBMSInterestIndication message in accordance with 5.8.5.4.

#### 5.8.5.3	Determine MBMS frequencies of interest

The UE shall:

1>	consider a frequency to be part of the MBMS frequencies of interest if the following conditions are met:

2> 	at least one MBMS session the UE is receiving or interested to receive via an MRB or SC-MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see MFA TS 36.300 [9] or 3GPP TS 26.346 [57].

2> 	for at least one of these MBMS sessions SystemInformationBlockType15 acquired from the PCell includes for the concerned frequency one or more MBMS SAIs as indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBMS frequencies of interest even though E-UTRAN may (temporarily) not employ an MRB or SC-MRB for the concerned session. I.e. the UE does not verify if the session is indicated on (SC-)MCCH

NOTE 3:	The UE considers the frequencies of interest independently of any synchronization state, e.g. see MFA TS 36.300 [9, Annex J.1]

2> 	the UE is capable of simultaneously receiving MRBs and/or is capable of simultaneously receiving SC-MRBs on the set of MBMS frequencies of interest, regardless of whether a serving cell is configured on each of these frequencies or not; and

2> 	the supportedBandCombination the UE included in UE-EUTRA-Capability contains at least one band combination including the set of MBMS frequencies of interest;

NOTE 4:	Indicating a frequency implies that the UE supports SystemInformationBlockType13 or SystemInformationBlockType20 acquisition for the concerned frequency i.e. the indication should be independent of whether a serving cell is configured on that frequency.

NOTE 5:	When evaluating which frequencies it can receive simultaneously, the UE does not take into account the serving frequencies that are currently configured i.e. it only considers MBMS frequencies it is interested to receive.

NOTE 6:	The set of MBMS frequencies of interest includes at most one frequency for a given physical frequency. The UE only considers a physical frequency to be part of the MBMS frequencies of interest if it supports at least one of the bands indicated for this physical frequency in SystemInformationBlockType1 (for serving frequency) or SystemInformationBlockType15 (for neighbouring frequencies). In this case, E-UTRAN may assume the UE supports MBMS reception on any of the bands supported by the UE (i.e. according to supportedBandCombination).

#### 5.8.5.3a	Determine MBMS services of interest

The UE shall:

1>	consider a MBMS service to be part of the MBMS services of interest if the following conditions are met:

2> 	the UE is SC-PTM capable; and

2> 	the UE is receiving or interested to receive this service via an SC-MRB; and

2> 	one session of this service is ongoing or about to start; and

2> 	one or more MBMS SAIs in the USD for this service is included in SystemInformationBlockType15 acquired from the PCell for a frequency belonging to the set of MBMS frequencies of interest, determined according to 5.8.5.3.

#### 5.8.5.4	Actions related to transmission of MBMSInterestIndication message

The UE shall set the contents of the MBMSInterestIndication message as follows:

1>	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

2> 	include mbms-FreqList and set it to include the MBMS frequencies of interest sorted by decreasing order of interest, using the EARFCN corresponding with freqBandIndicator included in SystemInformationBlockType1 (for serving frequency), if applicable, and the EARFCN(s) as included in SystemInformationBlockType15 (for neighbouring frequencies);

NOTE 1:	The EARFCN included in mbms-FreqList is merely used to indicate a physical frequency the UE is interested to receive i.e. the UE may not support the band corresponding to the included EARFCN (but it does support at least one of the bands indicated in system information for the concerned physical frequency).

2> 	include mbms-Priority if the UE prioritises reception of all indicated MBMS frequencies above reception of any of the unicast bearers;

2> 	if SystemInformationBlockType20 is broadcast by the PCell:

3> 	include mbms-Services and set it to indicate the set of MBMS services of interest determined in accordance with 5.8.5.3a;

NOTE 2:	If the UE prioritises MBMS reception and unicast data cannot be supported because of congestion on the MBMS carrier(s), E-UTRAN may initiate release of unicast bearers. It is up to E-UTRAN implementation whether all bearers or only GBR bearers are released. E-UTRAN does not initiate re-establishment of the released unicast bearers upon alleviation of the congestion.

The UE shall submit the MBMSInterestIndication message to lower layers for transmission.

## 5.8a	SC-PTM

### 5.8a.1	Introduction

#### 5.8a.1.1	General

SC-PTM control information is provided on a specific logical channel: the SC-MCCH. The SC-MCCH carries the SCPTMConfiguration message which indicates the MBMS sessions that are ongoing as well as the (corresponding) information on when each session may be scheduled, i.e.scheduling period, scheduling window and start offset. The SCPTMConfiguration message also provides information about the neighbour cells transmitting the MBMS sessions which are ongoing on the current cell.

A limited amount of SC-PTM control information is provided on the BCCH. This primarily concerns the information needed to acquire the SC-MCCH.

#### 5.8a.1.2	SC-MCCH scheduling

The SC-MCCH information (i.e. information transmitted in messages sent over SC-MCCH) is transmitted periodically, using a configurable repetition period. SC-MCCH transmissions (and the associated radio resources and MCS) are indicated on PDCCH.

#### 5.8a.1.3	SC-MCCH information validity and notification of changes

Change of SC-MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same SC-MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period). The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by means of SystemInformationBlockType20.

When the network changes (some of) the SC-MCCH information, it notifies the UEs about the change in the first subframe which can be used for SC-MCCH transmission in a repetition period. LSB bit in 8-bit bitmap when set to '1' indicates the change in SC-MCCH. Upon receiving a change notification, a UE interested to receive MBMS services transmitted using SC-PTM acquires the new SC-MCCH information starting from the same subframe. The UE applies the previously acquired SC-MCCH information until the UE acquires the new SC-MCCH information.

#### 5.8a.1.4	Procedures

The SC-PTM capable UE receiving or interested to receive MBMS service(s) via SC-MRB applies SC-PTM procedures described in 5.8a and the MBMS interest indication procedure as specified in 5.8.5.

### 5.8a.2	SC-MCCH information acquisition

#### 5.8a.2.1	General

Figure 5.8a.2.1-1: SC-MCCH information acquisition

The UE applies the SC-MCCH information acquisition procedure to acquire the SC-PTM control information that is broadcast by the E-UTRAN. The procedure applies to SC-PTM capable UEs that are in RRC_IDLE or in RRC_CONNECTED.

#### 5.8a.2.2	Initiation

A UE interested to receive MBMS services via SC-MRB shall apply the SC-MCCH information acquisition procedure upon entering the cell broadcasting SystemInformationBlockType20 (e.g. upon power on, following UE mobility) and upon receiving a notification that the SC-MCCH information has changed. A UE that is receiving an MBMS service via SC-MRB shall apply the SC-MCCH information acquisition procedure to acquire the SC-MCCH information that corresponds with the service that is being received, at the start of each modification period.

Unless explicitly stated otherwise in the procedural specification, the SC-MCCH information acquisition procedure overwrites any stored SC-MCCH information, i.e. delta configuration is not applicable for SC-MCCH information and the UE discontinues using a field if it is absent in SC-MCCH information unless explicitly specified otherwise.

#### 5.8a.2.3	SC-MCCH information acquisition by the UE

A SC-PTM capable UE shall:

1>	if the procedure is triggered by an SC-MCCH information change notification:

2> 	start acquiring the SCPTMConfiguration message from the subframe where the change notification was received;

NOTE 1:	The UE continues using the previously received SC-MCCH information until the new SC-MCCH information has been acquired.

1>	if the UE enters a cell broadcasting SystemInformationBlockType20:

2> 	acquire the SCPTMConfiguration message at the next repetition period;

1>	if the UE is receiving an MBMS service via an SC-MRB:

2> 	start acquiring the SCPTMConfiguration message from the beginning of each modification period.

#### 5.8a.2.4	Actions upon reception of the SCPTMConfiguration message

No UE requirements related to the contents of this SCPTMConfiguration apply other than those specified elsewhere e.g. within procedures using the concerned system information, the corresponding field descriptions.

### 5.8a.3	SC-PTM radio bearer configuration

#### 5.8a.3.1	General

The SC-PTM radio bearer configuration procedure is used by the UE to configure RLC, MAC and the physical layer upon starting and/or stopping to receive an SC-MRB transmitted on SC-MTCH. The procedure applies to SC-PTM capable UEs that are in RRC_CONNECTED or in RRC_IDLE and are interested to receive one or more MBMS services via SC-MRB.

NOTE:	In case the UE is unable to receive an MBMS service via an SC-MRB due to capability limitations, upper layers may take appropriate action e.g. terminate a lower priority unicast service.

#### 5.8a.3.2	Initiation

The UE applies the SC-MRB establishment procedure to start receiving a session of a MBMS service it has an interest in. The procedure may be initiated e.g. upon start of the MBMS session, upon entering a cell providing via SC-MRB a MBMS service in which the UE has interest, upon becoming interested in the MBMS service, upon removal of UE capability limitations inhibiting reception of the concerned service.

The UE applies the SC-MRB release procedure to stop receiving a session. The procedure may be initiated e.g. upon stop of the MBMS session, upon leaving the cell where a SC-MRB is established, upon losing interest in the MBMS service, when capability limitations start inhibiting reception of the concerned service.

#### 5.8a.3.3	SC-MRB establishment

Upon SC-MRB establishment, the UE shall:

1>	establish an RLC entity in accordance with the configuration specified in 9.1.1.7;

1>	configure a SC-MTCH logical channel applicable for the SC-MRB and instruct MAC to receive DL-SCH on the cell where the SCPTMConfiguration message was received for the MBMS service for which the SC-MRB is established and using g-RNTI and sc-mtch-SchedulingInfo (if included) in this message for this MBMS service:

1>	configure the physical layer in accordance with the sc-mtch-InfoList, applicable for the SC-MRB, as included in the SCPTMConfiguration message;

1>	inform upper layers about the establishment of the SC-MRB by indicating the corresponding tmgi and sessionId;

#### 5.8a.3.4	SC-MRB release

Upon SC-MRB release, the UE shall:

1>	release the RLC entity as well as the related MAC and physical layer configuration;

1>	inform upper layers about the release of the SC-MRB by indicating the corresponding tmgi and sessionId;

## 5.9	RN procedures

### 5.9.1	RN reconfiguration

#### 5.9.1.1	General

Figure 5.9.1.1-1: RN reconfiguration

The purpose of this procedure is to configure/reconfigure the RN subframe configuration and/or to update the system information relevant for the RN in RRC_CONNECTED.

#### 5.9.1.2	Initiation

E-UTRAN may initiate the RN reconfiguration procedure to an RN in RRC_CONNECTED when AS security has been activated.

#### 5.9.1.3	Reception of the RNReconfiguration by the RN

The RN shall:

1>	if the rn-SystemInfo is included:

2> 	if the systemInformationBlockType1 is included:

3> 	act upon the received SystemInformationBlockType1 as specified in 5.2.2.7;

2> 	if the SystemInformationBlockType2 is included:

3> 	act upon the received SystemInformationBlockType2 as specified in 5.2.2.9;

1>	if the rn-SubframeConfig is included:

2> 	reconfigure lower layers in accordance with the received subframeConfigPatternFDD or subframeConfigPatternTDD;

2> 		if the rpdcch-Config is included:

3> 	reconfigure lower layers in accordance with the received rpdcch-Config;

1>	submit the RNReconfigurationComplete message to lower layers for transmission, upon which the procedure ends;

## 5.10	Sidelink

### 5.10.1	Introduction

The sidelink communication and associated synchronisation resource configuration applies for the frequency at which it was received/ acquired. Moreover, for a UE configured with one or more SCells, the sidelink communication and associated synchronisation resource configuration provided by dedicated signalling applies for the PCell/ the primary frequency. The sidelink discovery and associated synchronisation resource configuration applies for the frequency at which it was received/ acquired or the indicated frequency in the configuration. For a UE configured with one or more SCells, the sidelink discovery and associated synchronisation resource configuration provided by dedicated signalling applies for the the PCell/ the primary frequency / any other indicated frequency.

NOTE 1:	Upper layers configure the UE to receive or transmit sidelink communication on a specific frequency, to monitor or transmit non-PS related sidelink discovery announcements on one or more frequencies or to monitor or transmit PS related sidelink discovery announcements on a specific frequency, but only if the UE is authorised to perform these particular ProSe related sidelink activities.

NOTE 2:	It is up to UE implementation which actions to take (e.g. termination of unicast services, detach) when it is unable to perform the desired sidelink activities, e.g. due to UE capability limitations.

Sidelink communication consists of one-to-many and one-to-one sidelink communication. One-to-many sidelink communication consists of relay related and non-relay related one-to-many sidelink communication. One-to-one sidelink communication consists of relay related and non-relay related one-to-one sidelink communication. In relay related one-to-one sidelink communication the communicating parties consist of one sidelink relay UE and one sidelink remote UE.

Sidelink discovery consists of public safety related (PS related) and non-PS related sidelink discovery. PS related sidelink discovery consists of relay related and non-relay related PS related sidelink discovery. Upper layers indicate to RRC whether a particular sidelink announcement is PS related or non-PS related.

The specification covers the use of UE to network sidelink relays by specifying the additional requirements that apply for a sidelink relay UE and a sidelink remote UE. I.e. for such UEs the regular sidelink UE requirements equally apply unless explicitly stated otherwise.

### 5.10.1a	Conditions for sidelink communication operation

When it is specified that the UE shall perform sidelink communication operation only if the conditions defined in this section are met, the UE shall perform sidelink communication operation only if:

1>	if the UE’s serving cell is suitable (RRC_IDLE or RRC_CONNECTED); and if either the selected cell on the frequency used for sidelink communication operation belongs to the registered or equivalent PLMN as specified in 3GPP TS 24.334 [69] or the UE is out of coverage on the frequency used for sidelink communication operation as defined in MFA TS 36.304 [4, Section 11.4]; or

1>	if the UE is camped on a serving cell (RRC_IDLE) on which it fulfils the conditions to support sidelink communication in limited service state as specified in 3GPP TS 23.303 [68, Section 4.5.6]; and if either the serving cell is on the frequency used for sidelink communication operation or the UE is out of coverage on the frequency used for sidelink communication operation as defined in MFA TS 36.304 [4, Section 11.4]; or

1>	if the UE has no serving cell (RRC_IDLE);

### 5.10.1b	Conditions for PS related sidelink discovery operation

When it is specified that the UE shall perform PS related sidelink discovery operation only if the conditions defined in this section are met, the UE shall perform PS related sidelink discovery operation only if:

1> 	if the UE’s serving cell is suitable (RRC_IDLE or RRC_CONNECTED); and if either the selected cell on the frequency used for PS related sidelink discovery operation belongs to the registered or other PLMN as specified in 3GPP TS 24.334 [69] or the UE is out of coverage on the frequency used for PS related sidelink discovery operation as defined in MFA TS 36.304 [4, Section 11.4]; or

1> 	if the UE is camped on a serving cell (RRC_IDLE) on which it fulfils the conditions to support sidelink discovery in limited service state as specified in 3GPP TS 23.303 [68, Section 4.5.6]; and if either the serving cell is on the frequency used for PS related sidelink discovery operation or the UE is out of coverage on the frequency used for PS related sidelink discovery operation as defined in MFA TS 36.304 [4, Section 11.4]; or

1> 	if the UE has no serving cell (RRC_IDLE);

### 5.10.1c	Conditions for non-PS related sidelink discovery operation

When it is specified that the UE shall perform non-PS related sidelink discovery operation only if the conditions defined in this section are met, the UE shall perform non-PS related sidelink discovery operation only if:

1> 	if the UE’s serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) is suitable; and if the selected cell on the frequency used for non-PS related sidelink discovery operation belongs to the registered or other PLMN as specified in 3GPP TS 24.334 [69].

### 5.10.2	Sidelink UE information

#### 5.10.2.1	General

Figure 5.10.2-1: Sidelink UE information

The purpose of this procedure is to inform E-UTRAN that the UE is interested or no longer interested to receive sidelink communication or discovery, as well as to request assignment or release of transmission resources for sidelink communication or discovery announcements or sidelink discovery gaps and to report parameters related to sidelink discovery from system information of inter-frequency/PLMN cells.

#### 5.10.2.2	Initiation

A UE capable of sidelink communication or discovery that is in RRC_CONNECTED may initiate the procedure to indicate it is (interested in) receiving sidelink communication or discovery in several cases including upon successful connection establishment, upon change of interest, upon change to a PCell broadcasting SystemInformationBlockType18 or SystemInformationBlockType19. A UE capable of sidelink communication or discovery may initiate the procedure to request assignment of dedicated resources for the concerned sidelink communication transmission or discovery announcements or to request sidelink discovery gaps for sidelink discovery transmission or sidelink discovery reception and a UE capable of inter-frequency/PLMN sidelink discovery parameter reporting may initiate the procedure to report parameters related to sidelink discovery from system information of inter-frequency/PLMN cells.

NOTE 1:	A UE in RRC_IDLE that is configured to transmit sidelink communication/ discovery announcements, while SystemInformationBlockType18/ SystemInformationBlockType19 does not include the resources for transmission (in normal conditions), initiates connection establishment in accordance with 5.3.3.1a.

Upon initiating the procedure, the UE shall:

1>	if SystemInformationBlockType18 is broadcast by the PCell:

2> 	ensure having a valid version of SystemInformationBlockType18 for the PCell;

2> 	if configured by upper layers to receive sidelink communication:

3> 	if the UE did not transmit a SidelinkUEInformation message since last entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or

NOTE 2:	After handover/ re-establishment from a source PCell not broadcasting SystemInformationBlockType18 the UE repeats the same interest information that it provided previously as such a source PCell may not forward the interest information.

3> 	if the last transmission of the SidelinkUEInformation message did not include commRxInterestedFreq; or if the frequency configured by upper layers to receive sidelink communication on has changed since the last transmission of the SidelinkUEInformation message:

4> 	initiate transmission of the SidelinkUEInformation message to indicate the sidelink communication reception frequency of interest in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included commRxInterestedFreq:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it is no longer interested in sidelink communication reception in accordance with 5.10.2.3;

2> 	if configured by upper layers to transmit non-relay related one-to-many sidelink communication:

3> 	if the UE did not transmit a SidelinkUEInformation message since last entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or

3> 	if the last transmission of the SidelinkUEInformation message did not include commTxResourceReq; or if the information carried by the commTxResourceReq has changed since the last transmission of the SidelinkUEInformation message:

4> 	initiate transmission of the SidelinkUEInformation message to indicate the non-relay related one-to-many sidelink communication transmission resources required by the UE in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included commTxResourceReq:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it no longer requires non-relay related one-to-many sidelink communication transmission resources in accordance with 5.10.2.3;

2> 	if configured by upper layer to transmit relay related one-to-many sidelink communication:

3> 	if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18, connected to a PCell not broadcasting SystemInformationBlockType19 or broadcasting SystemInformationBlockType19 not including discConfigRelay; or

3> 	if the last transmission of SidelinkUEInformation message did not include commTxResourceReqRelay; or if the information carried by the commTxResourceReqRelay has changed since the last transmission of the SidelinkUEInformation message:

4> 	if the UE is acting as sidelink relay UE:

5> 	initiate transmission of the SidelinkUEInformation message to indicate the relay related one-to-many sidelink communication transmission resources required by the UE in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included commTxResourceReqRelay:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it no longer requires relay related one-to-many sidelink communication transmission resources in accordance with 5.10.2.3;

2> 	if configured by upper layers to transmit non-relay related one-to-one sidelink communication:

3> 	if the UE did not transmit a SidelinkUEInformation message since last entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18 or connected to a PCell broadcasting SystemInformationBlockType18 not including commTxResourceUC-ReqAllowed; or

3> 	if the last transmission of the SidelinkUEInformation message did not include commTxResourceReqUC; or if the information carried by the commTxResourceReqUC has changed since the last transmission of the SidelinkUEInformation message:

4> 	if commTxResourceUC-ReqAllowed is included in SystemInformationBlockType18:

5> 	initiate transmission of the SidelinkUEInformation message to indicate the non-relay related one-to-one sidelink communication transmission resources required by the UE in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included commTxResourceReqUC:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it no longer requires non-relay related one-to-one sidelink communication transmission resources in accordance with 5.10.2.3;

2> 	if configured by upper layers to transmit relay related one-to-one sidelink communication:

3> 	if the UE did not transmit a SidelinkUEInformation message since last entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18, connected to a PCell not broadcasting SystemInformationBlockType19 or broadcasting SystemInformationBlockType19 not including discConfigRelay; or

3> 	if the last transmission of the SidelinkUEInformation message did not include commTxResourceReqRelayUC; or if the information carried by the commTxResourceReqRelayUC has changed since the last transmission of the SidelinkUEInformation message:

4> 	if the UE is acting as sidelink relay UE; or:

4> 	if the UE has a selected sidelink relay UE; and if SystemInformationBlockType19 is broadcast by the PCell and includes discConfigRelay; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met;

5> 	initiate transmission of the SidelinkUEInformation message to indicate the relay related one-to-one sidelink communication transmission resources required by the UE in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included commTxResourceReqRelayUC:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it no longer requires relay related one-to-one sidelink communication transmission resources in accordance with 5.10.2.3;

1>	if SystemInformationBlockType19 is broadcast by the PCell:

2> 	ensure having a valid version of SystemInformationBlockType19 for the PCell;

2> 	if configured by upper layers to receive sidelink discovery announcements on a serving frequency or on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19 of the PCell:

3> 	if the UE did not transmit a SidelinkUEInformation message since last entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType19; or

3> 	if the last transmission of the SidelinkUEInformation message did not include discRxInterest:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it is interested in sidelink discovery reception in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included discRxInterest:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it is no longer interested in sidelink discovery reception in accordance with 5.10.2.3;

2> 	if the UE is configured by upper layers to transmit non-PS related sidelink discovery announcements on the primary frequency or on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19 of the PCell, with discTxResourcesInterFreq included within discResourcesNonPS and not set to noTxOnCarrier:

3> 	if the UE did not transmit a SidelinkUEInformation message since last entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType19 or connected to a PCell broadcasting SystemInformationBlockType19 not including discTxResourcesInterFreq within discResourcesNonPS or discTxResourcesInterFreq did not include all frequencies for which the UE will request resources; or

3> 	if the last transmission of the SidelinkUEInformation message did not include discTxResourceReq; or if the non-PS related sidelink discovery announcement resources required by the UE have changed (i.e. resulting in a change of discTxResourceReq) since the last transmission of the SidelinkUEInformation message:

4> 	initiate transmission of the SidelinkUEInformation message to indicate the non-PS related sidelink discovery announcement resources required by the UE in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included discTxResourceReq:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it no longer requires non-PS related sidelink discovery announcement resources in accordance with 5.10.2.3;

2> 	if configured by upper layers to transmit PS related sidelink discovery announcements on the primary frequency or, in case of non-relay PS related sidelink discovery announcements, on a frequency included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResourcesInterFreq included within discResourcesPS and not set to noTxOnCarrier:

3> 	if the UE did not transmit a SidelinkUEInformation message since last entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType19, connected to a PCell broadcasting SystemInformationBlockType19 not including discConfigPS, or in case of non-relay PS related transmission: (connected to a PCell broadcasting SystemInformationBlockType19 not including discTxResourcesInterFreq within discResourcesPS or for which discTxResourcesInterFreq did not include all frequencies for which the UE will request resources), or in case of relay related PS sidelink discovery announcements: (connected to a PCell broadcasting SystemInformationBlockType19 not including discConfigRelay) sidelink; or

3> 	if the last transmission of the SidelinkUEInformation message did not include discTxResourceReqPS; or if the PS related sidelink discovery announcement resources required by the UE have changed (i.e. resulting in a change of discTxResourceReqPS) since the last transmission of the SidelinkUEInformation message:

4> 	if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or

4> 	if the UE is acting as sidelink relay UE; and if SystemInformationBlockType19 includes discConfigRelay; and if the sidelink relay UE threshold conditions as specified in 5.10.10.4 are met; or

4> 	if the UE is selecting a sidelink relay UE / has a selected sidelink relay UE; and if SystemInformationBlockType19 includes discConfigRelay; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met:

5> 	initiate transmission of the SidelinkUEInformation message to indicate the PS related sidelink discovery announcement resources required by the UE in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included discTxResourceReqPS:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it no longer requires PS related sidelink discovery announcement resources in accordance with 5.10.2.3;

2> 	if configured by upper layers to monitor or transmit sidelink discovery announcements; and if the UE requires sidelink discovery gaps, to perform such actions:

3> 	if the UE did not transmit a SidelinkUEInformation message since last entering RRC_CONNECTED state; or

3> 	if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType19 or connected to a PCell broadcasting SystemInformationBlockType19 not including gapRequestsAllowedCommon while at the same time the UE was not configured with gapRequestsAllowedDedicated; or

3> 	if the last transmission of the SidelinkUEInformation message did not include the gaps required to monitor or transmit the sidelink discovery announcements (i.e. UE requiring gaps to monitor discovery announcements while discRxGapReq was not included or UE requiring gaps to transmit discovery announcements while discTxGapReq was not included); or if the sidelink discovery gaps required by the UE have changed (i.e. resulting in a change of discRxGapReq or discTxGapReq) since the last transmission of the SidelinkUEInformation message:

4> 	if the UE is configured with gapRequestsAllowedDedicated set to true; or

4> 	if the UE is not configured with gapRequestsAllowedDedicated and gapRequestsAllowedCommon is included in SystemInformationBlockType19:

5> 	initiate transmission of the SidelinkUEInformation message to indicate the sidelink discovery gaps required by the UE in accordance with 5.10.2.3;

2> 	else:

3> 	if the last transmission of the SidelinkUEInformation message included discTxGapReq or discRxGapReq:

4> 	initiate transmission of the SidelinkUEInformation message to indicate it no longer requires sidelink discovery gaps in accordance with 5.10.2.3;

2> 	if the UE acquired the relevant parameters from the system information of one or more cells on a carrier included in the discSysInfoToReportConfig and T370 is running:

3> 	if the UE has configured lower layers to transmit or monitor the sidelink discovery announcements on those cells:

4> 	initiate transmission of the SidelinkUEInformation message to report the acquired system information parameters and stop T370;

#### 5.10.2.3	Actions related to transmission of SidelinkUEInformation message

The UE shall set the contents of the SidelinkUEInformation message as follows:

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive sidelink communication or discovery or to request (configuration/ release) of sidelink communication or discovery transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure):

2> 	if SystemInformationBlockType18 is broadcast by the PCell:

3> 	if configured by upper layers to receive sidelink communication:

4> 	include commRxInterestedFreq and set it to the sidelink communication frequency;

3> 	if configured by upper layers to transmit non-relay related one-to-many sidelink communication:

4> 	include commTxResourceReq and set its fields as follows:

5> 	set carrierFreq to indicate the sidelink communication frequency i.e. the same value as indicated in commRxInterestedFreq if included;

5> 	set destinationInfoList to include the non-relay related one-to-many sidelink communication transmission destination(s) for which it requests E-UTRAN to assign dedicated resources;

3> 	if configured by upper layers to transmit non-relay related one-to-one sidelink communication; and

3> 	if commTxResourceUC-ReqAllowed is included in SystemInformationBlockType18:

4> 	include commTxResourceReqUC and set its fields as follows:

5> 	set carrierFreq to indicate the one-to-one sidelink communication frequency i.e. the same value as indicated in commRxInterestedFreq if included;

5> 	set destinationInfoList to include the non-relay related one-to-one sidelink communication transmission destination(s) for which it requests E-UTRAN to assign dedicated resources;

3> 	if configured by upper layers to transmit relay related one-to-one sidelink communication; and

3> 	if SystemInformationBlockType19 is broadcast by the PCell including discConfigRelay; and

3> 	if the UE is acting as sidelink relay UE; or if the UE has a selected sidelink relay UE; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met:

4> 	include commTxResourceReqRelayUC and set its fields as follows:

5> 	set destinationInfoList to include the one-to-one sidelink communication transmission destination(s) for which it requests E-UTRAN to assign dedicated resources;

4> 	include ue-Type and set it to relayUE if the UE is acting as sidelink relay UE and to remoteUE otherwise;

3> 	if configured by upper layers to transmit relay related one-to-many sidelink communication; and

3> 	if SystemInformationBlockType19 is broadcast by the PCell including discConfigRelay; and

3> 	if the UE is acting as sidelink relay UE:

4> 	include commTxResourceReqRelay and set its fields as follows:

5> 	set destinationInfoList to include the one-to-many sidelink communication transmission destination(s) for which it requests E-UTRAN to assign dedicated resources;

4> 	include ue-Type and set it to relayUE;

2> 	if SystemInformationBlockType19 is broadcast by the PCell:

3> 	if configured by upper layers to receive sidelink discovery announcements on a serving frequency or one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19:

4> 	include discRxInterest;

3> 	if the UE is configured by upper layers to transmit non-PS related sidelink discovery announcements:

4> 	for each frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements that concerns the primary frequency or that is included in discInterFreqList with discTxResourcesInterFreq included within discResourcesNonPS and not set to noTxOnCarrier:

5> 	for the first frequency, include discTxResourceReq and set it to indicate the number of discovery messages for sidelink discovery announcement(s) for which it requests E-UTRAN to assign dedicated resources as well as the concerned frequency, if different from the primary;

5> 	for any additional frequency, include discTxResourceReqAddFreq and set it to indicate the number of discovery messages for sidelink discovery announcement(s) for which it requests E-UTRAN to assign dedicated resources as well as the concerned frequency;

3> 	if configured by upper layers to transmit PS related sidelink discovery announcements; and

3> 	if the frequency on which the UE is configured to transmit PS related sidelink discovery announcements either concerns the primary frequency or, in case of non-relay PS related sidelink discovery announcements, is included in discInterFreqList with discTxResources InterFreq included within discResourcesPS and not set to noTxOnCarrier:

4> 	if configured by upper layers to transmit non-relay PS related sidelink discovery announcements and SystemInformationBlockType19 includes discConfigPS; or

4> 	if the UE is acting as sidelink relay UE; and if SystemInformationBlockType19 includes discConfigRelay; and if the sidelink relay UE threshold conditions as specified in 5.10.10.4 are met; or

4> 	if the UE is selecting a sidelink relay UE / has a selected sidelink relay UE; and if SystemInformationBlockType19 includes discConfigRelay; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met:

5> 	include discTxResourceReqPS and set it to indicate the number of discovery messages for PS related sidelink discovery announcement(s) for which it requests E-UTRAN to assign dedicated resources as well as the concerned frequency, if different from the primary;

1>	else if the UE initiates the procedure to request sidelink discovery transmission and/ or reception gaps:

2> 	if the UE is configured with gapRequestsAllowedDedicated set to true; or

2> 	if the UE is not configured with gapRequestsAllowedDedicated and gapRequestsAllowedCommon is included in SystemInformationBlockType19:

3> 	if the UE requires sidelink discovery gaps to monitor the sidelink discovery announcements the UE is configured to monitor by upper layers:

4> 	include discRxGapReq and set it to indicate, for each frequency that either concerns the primary frequency or is included in discInterFreqList on which the UE is configured to monitor sidelink discovery announcements and for which it requires sidelink discovery gaps to do so, the gap pattern(s) as well as the concerned frequency, if different from the primary;

3> 	if the UE requires sidelink discovery gaps to transmit the sidelink discovery announcements the UE is configured to transmit by upper layers:

4> 	include discTxGapReq and set it to indicate, for each frequency that either concerns the primary or is included in discInterFreqList on which the UE is configured to transmit sidelink discovery announcements and for which it requires sidelink discovery gaps to do so, the gap pattern(s) as well as the concerned frequency, if different from the primary;

1>	else if the UE initiates the procedure to report the system information parameters related to sidelink discovery of carriers other than the primary:

2> 	include discSysInfoReportFreqList and set it to report the system information parameter acquired from the cells on those carriers;

The UE shall submit the SidelinkUEInformation message to lower layers for transmission.

### 5.10.3	Sidelink communication monitoring

A UE capable of sidelink communication that is configured by upper layers to receive sidelink communication shall:

1>	 if the conditions for sidelink communication operation as defined in 5.10.1a are met:

2> 	if in coverage on the frequency used for sidelink communication, as defined in MFA TS 36.304 [4, Section 11.4]:

3> 	if the cell chosen for sidelink communication reception broadcasts SystemInformationBlockType18 including commRxPool:

4> 	configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources indicated by commRxPool;

NOTE 1:	If commRxPool includes one or more entries including rxParametersNCell, the UE may only monitor such entries if the associated PSS/SSS or SLSSIDs is detected. When monitoring such pool(s), the UE applies the timing of the concerned PSS/SSS or SLSS.

2> 	else (i.e. out of coverage on the sidelink carrier):

3> 	configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources that were preconfigured (i.e. preconfigComm in SL-Preconfiguration defined in 9.3);

NOTE 2:	The UE may monitor in accordance with the timing of the selected SyncRef UE, or if the UE does not have a selected SyncRef UE, based on the UE’s own timing.

### 5.10.4	Sidelink communication transmission

A UE capable of sidelink communication that is configured by upper layers to transmit non-relay related sidelink communication and has related data to be transmitted or a UE capable of relay related sidelink communication that is configured by upper layers to transmit relay related sidelink communications and satisfies the conditions for relay related sidelink communication specified in this section shall:

1>	if the conditions for sidelink communication operation as defined in 5.10.1a are met:

2> 	if in coverage on the frequency used for sidelink communication, as defined in MFA TS 36.304 [4, Section 11.4]:

3> 	if the UE is in RRC_CONNECTED and uses the PCell for sidelink communication:

4> 	if the UE is configured, by the current PCell/ the PCell in which physical layer problems or radio link failure was detected, with commTxResources set to scheduled:

5> 	if T310 or T311 is running; and if the PCell at which the UE detected physical layer problems or radio link failure broadcasts SystemInformationBlockType18 including commTxPoolExceptional; or

5> 	if T301 is running and the cell on which the UE initiated connection re-establishment broadcasts SystemInformationBlockType18 including commTxPoolExceptional:

6> 	configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources indicated by the first entry in commTxPoolExceptional;

5> 	else:

6> 	configure lower layers to request E-UTRAN to assign transmission resources for sidelink communication;

4> 	else if the UE is configured with commTxPoolNormalDedicated or commTxPoolNormalDedicatedExt:

5> 	if priorityList is included for the entries of commTxPoolNormalDedicated or commTxPoolNormalDedicatedExt:

6> 	configure lower layers to transmit the sidelink control information and the corresponding data using the one or more pools of resources indicated by commTxPoolNormalDedicated or commTxPoolNormalDedicatedExt i.e. indicate all entries of this field to lower layers;

5> 	else:

6> 	configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources indicated by the first entry in commTxPoolNormalDedicated;

3> 	else (i.e. sidelink communication in RRC_IDLE or on cell other than PCell in RRC_CONNECTED):

4> 	if the cell chosen for sidelink communication transmission broadcasts SystemInformationBlockType18:

5> 	if SystemInformationBlockType18 includes commTxPoolNormalCommon:

6> 	if priorityList is included for the entries of commTxPoolNormalCommon or commTxPoolNormalCommonExt:

7> 	configure lower layers to transmit the sidelink control information and the corresponding data using the one or more pools of resources indicated by commTxPoolNormalCommon and/or commTxPoolNormalCommonExt i.e. indicate all entries of these fields to lower layers;

6> 	else:

7> 	configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources indicated by the first entry in commTxPoolNormalCommon;

5> 	else if SystemInformationBlockType18 includes commTxPoolExceptional:

6> 	from the moment the UE initiates connection establishment until receiving an RRCConnectionReconfiguration including sl-CommConfig or until receiving an RRCConnectionRelease or an RRCConnectionReject;

7> 	configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources indicated by the first entry in commTxPoolExceptional;

2> 	else (i.e. out of coverage on sidelink carrier):

3> 	if priorityList is included for the entries of preconfigComm in SL-Preconfiguration defined in 9.3:

4> 	configure lower layers to transmit the sidelink control information and the corresponding data using the one or more pools of resources indicated preconfigComm i.e. indicate all entries of this field to lower layers and in accordance with the timing of the selected SyncRef UE, or if the UE does not have a selected SyncRef UE, based on the UEs own timing;

3> 	else:

4> 	configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources that were preconfigured i.e. indicated by the first entry in preconfigComm in SL-Preconfiguration defined in 9.3 and in accordance with the timing of the selected SyncRef UE, or if the UE does not have a selected SyncRef UE, based on the UEs own timing;

The conditions for relay related sidelink communication are as follows:

1>	if the transmission concerns sidelink relay communication; and the UE is capable of sidelink relay or sidelink remote operation:

2> 	if the UE is in RRC_IDLE; and if the UE has a selected sidelink relay UE: configure lower layers to transmit the sidelink control information and the corresponding data using the resources, as specified previously in this section, only if the following condition is met:

3> 	if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met; and if the UE configured lower layers with a pool of resources included in SystemInformationBlockType18 (i.e. commTxPoolNormalCommon, commTxPoolNormalCommonExt or commTxPoolExceptional); and commTxAllowRelayCommon is included in SystemInformationBlockType18;

2> 	if the UE is in RRC_CONNECTED: configure lower layers to transmit the sidelink control information and the corresponding data using the resources, as specified previously in this section, only if the following condition is met:

3> 	if the UE configured lower layers with resources provided by dedicated signalling (i.e. commTxResources); and the UE is configured with commTxAllowRelayDedicated set to true;

### 5.10.5	Sidelink discovery monitoring

A UE capable of non-PS related sidelink discovery that is configured by upper layers to monitor non-PS related sidelink discovery announcements shall:

1>	for each frequency the UE is configured to monitor non-PS related sidelink discovery announcements on, prioritising the frequencies included in discInterFreqList, if included in SystemInformationBlockType19:

2> 	if the PCell or the cell the UE is camping on indicates the pool of resources to monitor sidelink discovery announcements on by discRxResourcesInterFreq in discResourcesNonPS within discInterFreqList in SystemInformationBlockType19:

3> 	configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxResourcesInterFreqin discResourcesNonPS within SystemInformationBlockType19;

2> 	else if the cell used for sidelink discovery monitoring broadcasts SystemInformationBlockType19:

3> 	configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPool in SystemInformationBlockType19;

2> 	if the UE is configured with discRxGapConfig and requires sidelink discovery gaps to monitor sidelink discovery announcements on the concerned frequency;

3> 	configure lower layers to monitor the concerned frequency using the sidelink discovery gaps indicated by discRxGapConfig;

2> 	else:

3> 	configure lower layers to monitor the concerned frequency without affecting normal operation;

A UE capable of PS related sidelink discovery that is configured by upper layers to monitor PS related sidelink discovery announcements shall:

1> 		if out of coverage on the frequency, as defined in MFA TS 36.304 [4, Section 11.4]:

2>		configure lower layers to monitor sidelink discovery announcements using the pool of resources that were preconfigured (i.e. indicated by discRxPoolList within preconfigDisc in SL-Preconfiguration defined in 9.3);

1> 	else if configured by upper layers to monitor non-relay PS related discovery announcements; and if the PCell or the cell the UE is camping on indicates a pool of resources to monitor sidelink discovery announcements on by discRxResourcesInterFreq in discResourcesPS within discInterFreqList in SystemInformationBlockType19:

2>	configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxResourcesInterFreq in discResourcesPS in SystemInformationBlockType19;

1>	else if configured by upper layers to monitor PS related sidelink discovery announcements; and if the cell used for sidelink discovery monitoring broadcasts SystemInformationBlockType19:

2>	configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPoolPS in SystemInformationBlockType19;

1>	if the UE is configured with discRxGapConfig and requires sidelink discovery gaps to monitor sidelink discovery announcements on the concerned frequency;

2>	configure lower layers to monitor the concerned frequency using the sidelink discovery gaps indicated by discRxGapConfig;

1>	else:

2>	configure lower layers to monitor the concerned frequency without affecting normal operation;

NOTE 1:	The requirement not to affect normal UE operation also applies for the acquisition of sidelink discovery related system and synchronisation information from inter-frequency cells.

NOTE 2:	The UE is not required to monitor all pools simultaneously.

NOTE 3:	It is up to UE implementation to decide whether a cell is sufficiently good to be used to monitor sidelink discovery announcements.

NOTE 4:	If discRxPool, discRxPoolPS or discRxResourcesInterFreq includes one or more entries including rxParameters, the UE may only monitor such entries if the associated SLSSIDs are detected. When monitoring such pool(s) the UE applies the timing of the corresponding SLSS.

### 5.10.6	Sidelink discovery announcement

A UE capable of non-PS related sidelink discovery that is configured by upper layers to transmit non-PS related sidelink discovery announcements shall, for each frequency the UE is configured to transmit such announcements on:

NOTE:	In case the configured resources are insufficient it is up to UE implementation to decide which sidelink discovery announcements to transmit.

1>	if the frequency used to transmit sidelink discovery announcements concerns the serving frequency (RRC_IDLE) or primary frequency (RRC_CONNECTED):

2> 	if the UE’s serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) is suitable as defined in MFA TS 36.304 [4]:

3> 	if the UE is in RRC_CONNECTED (i.e. PCell is used for sidelink discovery announcement):

4> 	if the UE is configured with discTxResources set to scheduled:

5> 	configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResources;

4> 	else if the UE is configured with discTxPoolDedicated (i.e. discTxResources set to ue-Selected):

5> 	select an entry of the list of resource pool entries in discTxPoolDedicated and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a;

3> 	else if T300 is not running (i.e. UE in RRC_IDLE, announcing via serving cell):

4> 	if SystemInformationBlockType19 of the serving cell includes discTxPoolCommon:

5> 	select an entry of the list of resource pool entries in discTxPoolCommon and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a;

1>	else if, for the frequency used to transmit sidelink discovery announcements on, the UE is configured with dedicated resources (i.e. with discTxResources-r12, if discTxCarrierFreq is included in discTxInterFreqInfo, or with discTxResources within discTxInfoInterFreqListAdd in discTxInterFreqInfo); and the conditions for non-PS related sidelink discovery operation as defined in 5.10.1c are met:

2> 	if the UE is configured with discTxResources set to scheduled:

3> 	configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResources;

2> 	else if the UE is configured with discTxResources set to ue-Selected:

3> 	select an entry of the list of resource pool entries in ue-Selected and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a;

1>	else if the frequency used to transmit sidelink discovery announcements on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, and discTxResourcesInterFreq within discResourcesNonPS in the corresponding entry of discInterFreqList is set to discTxPoolCommon (i.e. serving cell/ PCell broadcasts pool of resources) and the conditions for non-PS related sidelink discovery operation as defined in 5.10.1c are met; or

1>	else if discTxPoolCommon is included in SystemInformationBlockType19 acquired from cell selected on the sidelink discovery announcement frequency; and the conditions for non-PS related sidelink discovery operation as defined in 5.10.1c are met:

2> 	select an entry of the list of resource pool entries in discTxPoolCommon and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a;

1>	if the UE is configured with discTxGapConfig and requires sidelink discovery gaps to transmit sidelink discovery announcements on the concerned frequency;

2> 	configure lower layers to transmit on the concerned frequency using the sidelink discovery gaps indicated by discTxGapConfig,

1>	else:

2> 	configure lower layers to transmit on the concerned frequency without affecting normal operation;

A UE capable of PS related sidelink discovery that is configured by upper layers to transmit PS related sidelink discovery announcements shall:

1>	if out of coverage on the frequency used to transmit PS related sidelink discovery announcements as defined in MFA TS 36.304 [4, Section 11.4] and the conditions for PS -related sidelink discovery operation as defined in 5.10.1b are met:

2> 	if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or

2> 	if the UE is selecting a sidelink relay UE/ has a selected sidelink relay UE:

3> 	configure lower layers to transmit sidelink discovery announcements using the pool of resources that were preconfigured and in accordance with the following;

4> 	randomly select, using a uniform distribution, an entry of preconfigDisc in SL-Preconfiguration defined in 9.3;

4> 	using the timing of the selected SyncRef UE, or if the UE does not have a selected SyncRef UE, based on the UEs own timing;

1>	else if the frequency used to transmit sidelink discovery announcements concerns the serving frequency (RRC_IDLE) or primary frequency (RRC_CONNECTED) and the conditions for PS related sidelink discovery operation as defined in 5.10.1b are met:

2> 	if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or

2> 	if the UE is acting as sidelink relay UE; and if the UE is in RRC_IDLE; and if the sidelink relay UE threshold conditions as specified in 5.10.10.4 are met; or

2> 	if the UE is acting as sidelink relay UE; and if the UE is in RRC_CONNECTED; or

2> 	if the UE is selecting a sidelink relay UE / has a selected sidelink relay UE; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met:

3> 	if the UE is configured with discTxPoolPS-Dedicated; or

3> 	if the UE is in RRC_IDLE; and if discTxPoolPS-Common is included in SystemInformationBlockType19:

4> 	select an entry of the list of resource pool entries and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a;

3> 	else if the UE is configured with discTxResourcesPS set to scheduled:

4> 	configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResourcesPS;

1>	else if, for the frequency used to transmit sidelink discovery announcements on, the UE is configured with dedicated resources (i.e. with discTxResourcesPS in discTxInterFreqInfo within sl-DiscConfig); and the conditions for PS related sidelink discovery operation as defined in 5.10.1b are met:

2> 	if configured by upper layers to transmit non-relay PS related sidelink discovery announcements:

3> 	if the UE is configured with discTxResourcesPS set to scheduled:

4> 	configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResourcesPS;

3> 	else if the UE is configured with discTxResourcesPS set to ue-Selected:

4> 	select an entry of the list of resource pool entries in ue-Selected and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a;

1>	else if the frequency used to transmit sidelink discovery announcements on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, while discTxResourcesInterFreq within discResourcesPS in the corresponding entry of discInterFreqList is set to discTxPoolCommon (i.e. serving cell/ PCell broadcasts pool of resources) and the conditions for PS related sidelink discovery operation as defined in 5.10.1b are met:

2> 	if configured by upper layers to transmit non-relay PS related sidelink discovery announcements:

3> 	select an entry of the list of resource pool entries in discTxPoolCommon and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a;

1>	else if discTxPoolPS-Common is included in SystemInformationBlockType19 acquired from cell selected on the sidelink discovery announcement frequency; and the conditions for PS related sidelink discovery operation as defined in 5.10.1b are met:

2> 	if configured by upper layers to transmit non-relay PS related sidelink discovery announcements:

3> 	select an entry of the list of resource pool entries in discTxPoolPS-Common and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a;

1>	if the UE is configured with discTxGapConfig and requires gaps to transmit sidelink discovery announcements on the concerned frequency;

2> 	configure lower layers to transmit on the concerned frequency using the gaps indicated by discTxGapConfig,

1>	else:

2> 	configure lower layers to transmit on the concerned frequency without affecting normal operation;

### 5.10.6a	Sidelink discovery announcement pool selection

A UE that is configured with a list of resource pool entries for sidelink discovery announcement transmission (i.e. by SL-DiscTxPoolList) shall:

1>	if poolSelection is set to rsrpBased:

2> 	select a pool from the list of pools the UE is configured with for which the RSRP measurement of the reference cell selected as defined in 5.10.6b, after applying the layer 3 filter defined by quantityConfig as specified in 5.5.3.2, is in-between threshLow and threshHigh;

1>	else:

2> 	randomly select, using a uniform distribution, a pool from the list of pools the UE is configured with;

1>	configure lower layers to transmit the sidelink discovery announcement using the selected pool of resources;

NOTE 1:	When performing resource pool selection based on RSRP, the UE uses the latest results of the available measurements used for cell reselection evaluation in RRC_IDLE/ for measurement report triggering evaluation in RRC_CONNECTED, which are performed in accordance with the performance requirements specified in MFA TS 36.133 [16].

### 5.10.6b	Sidelink discovery announcement reference carrier selection

A UE capable of sidelink discovery that is configured by upper layers to transmit sidelink discovery announcements shall:

1>	for each frequency the UE is transmitting sidelink discovery announcements on, select a cell to be used as reference for synchronisation and DL measurements in accordance with the following:

2> 	if the frequency concerns the primary frequency:

3> 	use the PCell as reference;

2> 	else if the frequency concerns a secondary frequency:

3> 	use the concerned SCell as reference;

2> 	else if the UE is configured with discTxRefCarrierDedicated for the frequency:

3> 	use the cell indicated by this field as reference;

2> 	else if the UE is configured with refCarrierCommon for the frequency:

3> 	use the serving cell (RRC_IDLE)/ PCell (RRC_CONNECTED) as reference;

2> 	else:

3> 	use the DL frequency paired with the one used to transmit sidelink discovery announcements on as reference;

### 5.10.7	Sidelink synchronisation information transmission

#### 5.10.7.1	General

Figure 5.10.7.1-1: Synchronisation information transmission for sidelink communication, in (partial) coverage

Figure 5.10.7.1-2: Synchronisation information transmission for sidelink communication/ discovery, out of coverage

Figure 5.10.7.1-3: Synchronisation information transmission for sidelink discovery, in (partial) coverage

The purpose of this procedure is to provide synchronisation information to a UE. For sidelink discovery, the synchronisation information concerns a Sidelink Synchronisation Signal (SLSS) and, in case of PS related discovery, also timing information and some additional configuration parameters (i.e. the MasterInformationBlock-SL message), while for sidelink communication it concerns an SLSS and the MasterInformationBlock-SL message. A UE transmits synchronisation information either when E-UTRAN configures it to do so by dedicated signalling (i.e. network based), or when not configured by dedicated signalling (i.e. UE based) and E-UTRAN broadcasts (in coverage) or pre-configures a threshold (out of coverage).

The synchronisation information transmitted by the UE may be derived from information/ signals received from E-UTRAN (in coverage) or received from a UE acting as synchronisation reference for the transmitting UE. In the remainder, the UE acting as synchronisation reference is referred to as SyncRef UE.

#### 5.10.7.2	Initiation

A UE capable of SLSS transmission shall, when transmitting sidelink discovery announcements in accordance with 5.10.6 and when the following conditions are met:

1>	if in coverage on the frequency used for sidelink discovery, as defined in MFA TS 36.304 [4, Section 11.4]:

2> 	if in RRC_CONNECTED; and if networkControlledSyncTx is configured and set to on; or

2> 	if networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType19; and the RSRP measurement of the reference cell, selected as defined in 5.10.6b, is below the value of syncTxThreshIC:

3> 	if the sidelink discovery announcements are not PS related; or if syncTxPeriodic is not included:

4> 	transmit SLSS on the frequency used for sidelink discovery in accordance with 5.10.7.3 and MFA TS 36.211 [21];

3> 	else:

4> 	transmit SLSS on the frequency used for sidelink discovery in accordance with 5.10.7.3 and MFA TS 36.211 [21];

4> 	transmit the MasterInformationBlock-SL message on the frequency used for sidelink discovery, in the same subframe as SLSS, and in accordance with 5.10.7.4;

1>	else (i.e. out of coverage, PS):

2> 	if syncTxThreshOoC is included in the preconfigured sidelink parameters (i.e. SL-Preconfiguration defined in 9.3); and the UE has not selected SyncRef UE or the S-RSRP measurement result of the selected SyncRef UE is below the value of syncTxThreshOoC:

3> 	transmit SLSS on the frequency used for sidelink discovery in accordance with 5.10.7.3 and MFA TS 36.211 [21];

3> 	transmit the MasterInformationBlock-SL message on the frequency used for sidelink discovery, in the same subframe as SLSS, and in accordance with 5.10.7.4;

A UE capable of sidelink communication that is configured by upper layers to transmit sidelink communication shall, irrespective of whether or not it has data to transmit:

1>	if the conditions for sidelink communication operation as defined in 5.10.1a are met:

2> 	if in RRC_CONNECTED; and if networkControlledSyncTx is configured and set to on:

3> 	transmit SLSS in accordance with 5.10.7.3 and MFA TS 36.211 [21];

3> 	transmit the MasterInformationBlock-SL message, in the same subframe as SLSS, and in accordance with 5.10.7.4;

A UE shall, when transmitting sidelink communication in accordance with 5.10.4 and when the following conditions are met:

1>	if in coverage on the frequency used for sidelink communication, as defined in MFA TS 36.304 [4, Section 11.4]:

2> 	if the UE is in RRC_CONNECTED; and networkControlledSyncTx is not configured; and syncTxThreshIC is included in SystemInformationBlockType18; and the RSRP measurement of the cell chosen for sidelink communication transmission is below the value of syncTxThreshIC; or

2> 	if the UE is in RRC_IDLE; and syncTxThreshIC is included in SystemInformationBlockType18; and the RSRP measurement of the cell chosen for sidelink communication transmission is below the value of syncTxThreshIC:

3> 	transmit SLSS in accordance with 5.10.7.3 and MFA TS 36.211 [21];

3> 	transmit the MasterInformationBlock-SL message, in the same subframe as SLSS, and in accordance with 5.10.7.4;

1>	else (i.e. out of coverage):

2> 	if syncTxThreshOoC is included in the preconfigured sidelink parameters (i.e. SL-Preconfiguration defined in 9.3); and the UE has no selected SyncRef UE or the S-RSRP measurement result of the selected SyncRef UE is below the value of syncTxThreshOoC:

3> 	transmit SLSS in accordance with 5.10.7.3 and MFA TS 36.211 [21];

3> 	transmit the MasterInformationBlock-SL message, in the same subframe as SLSS, and in accordance with 5.10.7.4;

#### 5.10.7.3	Transmission of SLSS

The UE shall select the SLSSID and the subframe in which to transmit SLSS as follows:

1>	if triggered by sidelink discovery announcement and in coverage on the frequency used for sidelink discovery, as defined in MFA TS 36.304 [4, Section 11.4]:

2> 	select the SLSSID included in the entry of discSyncConfig included in the received SystemInformationBlockType19, that includes txParameters;

2> 	use syncOffsetIndicator corresponding to the selected SLSSID;

2> 	for each pool used for the transmission of discovery announcements (each corresponding to the selected SLSSID):

3> 	if a subframe indicated by syncOffsetIndicator corresponds to the first subframe of the discovery transmission pool;

4> 	if discTxGapConfig is configured and includes the concerned subframe; or the subframe is not used for regular uplink transmission:

5> 	select the concerned subframe;

3> 	else

4> 	if discTxGapConfig is configured and includes the concerned subframe; or the subframe is not used for regular uplink transmission:

5> 	select the subframe indicated by syncOffsetIndicator that precedes and which, in time domain, is nearest to the first subframe of the discovery transmission pool;

3> 	if the sidelink discovery announcements concern PS; and if syncTxPeriodic is included:

4> 	additionally select each subframe that periodically occurs 40 subframes after the selected subframe;

1>	if triggered by sidelink communication and in coverage on the frequency used for sidelink communication, as defined in MFA TS 36.304 [4, Section 11.4]:

2> 	select the SLSSID included in the entry of commSyncConfig that is included in the received SystemInformationBlockType18 and includes txParameters;

2> 	use syncOffsetIndicator corresponding to the selected SLSSID;

2> 	if in RRC_CONNECTED; and if networkControlledSyncTx is configured and set to on:

3> 	select the subframe(s) indicated by syncOffsetIndicator;

2> 	else (when transmitting communication):

3> 	select the subframe(s) indicated by syncOffsetIndicator within the SC period in which the UE intends to transmit sidelink control information or data;

1>	else (i.e. out of coverage on sidelink carrier):

2> 	select the synchronisation reference UE (i.e. SyncRef UE) as defined in 5.10.8;

2> 	if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlock-SL message received from this UE is set to TRUE; or

2> 	if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlock-SL message received from this UE is set to FALSE while the SLSS from this UE is part of the set defined for out of coverage, see MFA TS 36.211 [21]:

3> 	select the same SLSSID as the SLSSID of the selected SyncRef UE;

3> 	select the subframe in which to transmit the SLSS according to the syncOffsetIndicator1 or syncOffsetIndicator2 included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration defined in 9.3), such that the subframe timing is different from the SLSS of the selected SyncRef UE;

2> 	else if the UE has a selected SyncRef UE:

3> 	select the SLSSID from the set defined for out of coverage having an index that is 168 more than the index of the SLSSID of the selected SyncRef UE, see MFA TS 36.211 [21];

3> 	select the subframe in which to transmit the SLSS according to syncOffsetIndicator1 or syncOffsetIndicator2 included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration defined in 9.3), such that the subframe timing is different from the SLSS of the selected SyncRef UE;

2> 	else (i.e. no SyncRef UE selected):

3> 	randomly select, using a uniform distribution, an SLSSID from the set of sequences defined for out of coverage, see MFA TS 36.211 [21];

3> 	select the subframe in which to transmit the SLSS according to the syncOffsetIndicator1 or syncOffsetIndicator2 (arbitrary selection between these) included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration defined in 9.3);

#### 5.10.7.4	Transmission of MasterInformationBlock-SL message

The UE shall set the contents of the MasterInformationBlock-SL message as follows:

1>	if in coverage on the frequency used for the sidelink operation (communication or discovery) that triggered this procedure as defined in MFA TS 36.304 [4, Section 11.4]:

2> 	set inCoverage to TRUE;

2> 	set sl-Bandwidth to the value of ul-Bandwidth as included in the received SystemInformationBlockType2 of the cell chosen for the concerned sidelink operation;

2> 	if tdd-Config is included in the received SystemInformationBlockType1:

3> 	set subframeAssignmentSL to the value representing the same meaning as of subframeAssignment that is included in tdd-Config in the received SystemInformationBlockType1;

2> 	else:

3> 	set subframeAssignmentSL to none;

2> 	if triggered by sidelink communication; and if syncInfoReserved is included in an entry of commSyncConfig from the received SystemInformationBlockType18;

3> 	set reserved to the value of syncInfoReserved in the received SystemInformationBlockType18;

2> 	if triggered by sidelink discovery; and if syncInfoReserved is included in an entry of discSyncConfig from the received SystemInformationBlockType19;

3> 	set reserved to the value of syncInfoReserved in the received SystemInformationBlockType19;

2> 	else:

3> 	set all bits in reserved to 0;

1>	else if the UE has a selected SyncRef UE (as defined in 5.10.8):

2> 	set inCoverage to FALSE;

2> 	set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the received MasterInformationBlock-SL;

1>	else (i.e. no SyncRef UE selected):

2> 	set inCoverage to FALSE;

2> 	set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. preconfigGeneral in SL-Preconfiguration defined in 9.3);

1>	set directFrameNumber and directSubframeNumber according to the subframe used to transmit the SLSS, as specified in 5.10.7.3;

1>	submit the MasterInformationBlock-SL message to lower layers for transmission upon which the procedure ends;

#### 5.10.7.5	Void

### 5.10.8	Sidelink synchronisation reference

#### 5.10.8.1	General

The purpose of this procedure is to select a synchronisation reference and used a.o. when transmitting sidelink communication, sidelink discovery or synchronisation information.

#### 5.10.8.2	Selection and reselection of synchronisation reference UE (SyncRef UE)

The UE shall:

1>	for the frequency used for sidelink communication or discovery, if out of coverage on that frequency as defined in MFA TS 36.304 [4, Section 11.4]:

2> 	perform a full search (i.e. covering all subframes and all possible SLSSIDs) to detect candidate SLSS, in accordance with MFA TS 36.133 [16]

2> 	when evaluating the one or more detected SLSSIDs, apply layer 3 filtering as specified in 5.5.3.2 using the preconfigured filterCoefficient as defined in 9.3, before using the S-RSRP measurement results;

2> 	if the UE has selected a SyncRef UE:

3> 	if the S-RSRP of the strongest candidate SyncRef UE exceeds the minimum requirement MFA TS 36.133 [16] by syncRefMinHyst and the strongest candidate SyncRef UE belongs to the same priority group as the current SyncRef UE and the S-RSRP of the strongest candidate SyncRef UE exceeds the S-RSRP of the current SyncRef UE by syncRefDiffHyst; or

3> 	if the S-RSRP of the candidate SyncRef UE exceeds the minimum requirement MFA TS 36.133 [16] by syncRefMinHyst and the candidate SyncRef UE belongs to a higher priority group than the current SyncRef UE; or

3> 	if the S-RSRP of the current SyncRef UE is less than the minimum requirement MFA TS 36.133 [16]:

4> 	consider no SyncRef UE to be selected;

2> 	if the UE has not selected a SyncRef UE,

3> 	if the UE detects one or more SLSSIDs for which the S-RSRP exceeds the minimum requirement defined in MFA TS 36.133 [16] by syncRefMinHyst and for which the UE received the corresponding MasterInformationBlock-SL message (candidate SyncRef UEs), select a SyncRef UE according to the following priority order:

4> 	UEs of which inCoverage, included in the MasterInformationBlock-SL message received from this UE, is set to TRUE, starting with the UE with the highest S-RSRP result (priority group 1);

4> 	UE which SLSSID is part of the set defined for in coverage, starting with the UE with the highest S-RSRP result (priority group 2);

4> 	Other UEs, starting with the UE with the highest S-RSRP result (priority group 3);

### 5.10.9	Sidelink common control information

#### 5.10.9.1	General

The sidelink common control information is carried by a single message, the MasterInformationBlock-SL (MIB-SL) message. The MIB-SL includes timing information as well as some configuration parameters and is transmitted via SL-BCH.

The MIB-SL uses a fixed schedule with a periodicity of 40 ms without repetitions. In particular, the MIB-SL is scheduled in subframes indicated by syncOffsetIndicator i.e. for which (10*DFN + subframe number) mod 40 = syncOffsetIndicator.

The sidelink common control information may change at any transmission i.e. neither a modification period nor a change notification mechanism is used.

A UE configured to receive or transmit sidelink communication or PS related sidelink discovery shall:

1>	if the UE has a selected SyncRef UE, as specified in 5.10.8.2:

2> 	ensure having a valid version of the MasterInformationBlock-SL message of that SyncRefUE:

#### 5.10.9.2	Actions related to reception of MasterInformationBlock-SL message

Upon receiving MasterInformationBlock-SL, the UE shall:

1>	apply the values of sl-Bandwidth, subframeAssignmentSL, directFrameNumber and directSubframeNumber included in the received MasterInformationBlock-SL message;

### 5.10.10	Sidelink relay UE operation

#### 5.10.10.1	General

This procedure is used by a UE supporting sidelink relay UE operation and involves evaluation of the AS-layer conditions that need to be met in order for upper layers to configure a sidelink relay UE to receive/ transmit relay related PS sidelink discovery/ relay related sidelink communication. The AS-layer conditions merely comprise of being configured with radio resources that can be used for transmission.

A UE that fulfils the criteria specified in 5.10.10.2 and 5.10.10.3 and that is configured by higher layers accordingly is acting as a sidelink relay UE.

#### 5.10.10.2	AS-conditions for relay related sidelink communication transmission by sidelink relay UE

A UE capable of sidelink relay UE operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink communication transmission if the following conditions are met:

1>	if in RRC_CONNECTED; and if the UE is configured with commTxResources; and the UE is configured with commTxAllowRelayDedicated set to true;

#### 5.10.10.3	AS-conditions for relay PS related sidelink discovery transmission by sidelink relay UE

A UE capable of sidelink relay UE operation shall inform upper layers that it is configured with radio resources that can be used for relay PS related sidelink discovery transmission if the following conditions are met:

1>	if in RRC_IDLE; and if the UE’s serving cell is suitable as defined in MFA TS 36.304 [4]; and if SystemInformationBlockType19 includes discConfigPS including discTxPoolPS-Common and discConfigRelay; and if the sidelink relay UE threshold conditions as specified in 5.10.10.4 are met;

1>	else if in RRC_CONNECTED; and if discTxResourcesPS is configured;

#### 5.10.10.4	Sidelink relay UE threshold conditions

A UE capable of sidelink relay UE operation shall:

1>	if the threshold conditions specified in this section were not met:

2> 	if neither threshHigh nor threshLow is included in relayUE-Config within SystemInformationBlockType19:

3> 	consider the threshold conditions to be met (entry);

2> 	else if threshHigh is not included in relayUE-Config within SystemInformationBlockType19; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHigh by hystMax (also included within relayUE-Config); and

2> 	if threshLow is not included in relayUE-Config within SystemInformationBlockType19; or the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshLow by hystMin (also included within relayUE-Config):

3> 	consider the threshold conditions to be met (entry);

1>	else:

2> 	if threshHigh is included in relayUE-Config within SystemInformationBlockType19; and the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHigh (also included within relayUE-Config); or

2> 	if threshLow is included in relayUE-Config within SystemInformationBlockType19; and the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshLow (also included within relayUE-Config);

3> 	consider the threshold conditions not to be met (leave);

### 5.10.11	Sidelink remote UE operation

#### 5.10.11.1	General

This procedure is used by a UE supporting sidelink remote UE operation and involves evaluation of the AS-layer conditions that need to be met in order for upper layers to configure a sidelink remote UE to receive/ transmit relay related sidelink PS discovery/ relay related sidelink communication. The AS-layer conditions merely comprise of being configured with radio resources that can be used for transmission, as well as whether or not having a selected sidelink relay UE.

#### 5.10.11.2	AS-conditions for relay related sidelink communication transmission by sidelink remote UE

A UE capable of sidelink remote UE operation shall inform upper layers whether it is configured with radio resources that can be used for relay related sidelink communication transmission if the following conditions are met:

1>	if the UE is out of coverage; and is preconfigured with SL-Preconfiguration including discTxPoolList and preconfigRelay;

1>	else if in RRC_IDLE; and if the UE’s serving cell is suitable as defined in MFA TS 36.304 [4]; and if SystemInformationBlockType18 includes commTxPoolNormalCommon and commTxAllowRelayCommon; and if SystemInformationBlockType19 includes discConfigRelay; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met;

1>	else if in RRC_CONNECTED; and if the UE is configured with commTxResources; and the UE is configured with commTxAllowRelayDedicated set to true;

#### 5.10.11.3	AS-conditions for relay PS related sidelink discovery transmission by sidelink remote UE

A UE capable of sidelink remote UE operation shall inform upper layers whether it is configured with radio resources that can be used for relay PS related sidelink discovery transmission if the following conditions are met:

1>	if the UE is out of coverage; and is preconfigured with SL-Preconfiguration including discTxPoolList and preconfigRelay;

1>	else if in RRC_IDLE; and if the UE’s serving cell is suitable as defined in MFA TS 36.304 [4]; and if SystemInformationBlockType19 includes discConfigPS including discTxPoolPS-Common and discConfigRelay; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met;

1>	else if in RRC_CONNECTED; and if discTxResourcesPS is configured;

#### 5.10.11.4	Selection and reselection of sidelink relay UE

A UE capable of sidelink remote UE operation that is configured by upper layers to search for a sidelink relay UE shall:

1>	if out of coverage on the frequency used for sidelink communication, as defined in MFA TS 36.304 [4, Section 11.4]; or

1>	if the serving frequency is used for sidelink communication and the RSRP measurement of the cell on which the UE camps (RRC_IDLE)/ the PCell (RRC_CONNECTED) is below threshHigh within remoteUE-Config:

2> 	search for candidate sidelink relay UEs, in accordance with MFA TS 36.133 [16]

2> 	when evaluating the one or more detected sidelink relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same ProSe Relay UE ID and using the fiterCoefficient in SystemInformationBlockType19 (in coverage) or the preconfigured filterCoefficient as defined in 9.3(out of coverage), before using the SD-RSRP measurement results;

NOTE 1:	The details of the interaction with upper layers are up to UE implementation.

2> 	if the UE does not have a selected sidelink relay UE:

3> 	select a candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoIC (in coverage) or reselectionInfoOoC (out of coverage) by minHyst;

2> 	else if SD-RSRP of the currently selected sidelink relay UE is below q-RxLevMin included in either reselectionInfoIC (in coverage) or reselectionInfoOoC (out of coverage); orif upper layers indicate not to use the currently selected sidelink relay: (i.e. sidelink relay UE reselection):

3> 	select a candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoIC (in coverage) or reselectionInfoOoC (out of coverage) by minHyst;

2> 	else if the UE did not detect any candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoIC (in coverage) or reselectionInfoOoC (out of coverage) by minHyst:

3> 	consider no sidelink relay UE to be selected;

NOTE 2:	The UE may perform sidelink relay UE reselection in a manner resulting in selection of the sidelink relay UE, amongst all candidate sidelink relay UEs meeting higher layer criteria, that has the best radio link quality. Further details, including interaction with upper layers, are up to UE implementation.

#### 5.10.11.5	Sidelink remote UE threshold conditions

A UE capable of sidelink remote UE operation shall:

1>	if the threshold conditions specified in this section were not met:

2> 	if threshHigh is not included in remoteUE-Config within SystemInformationBlockType19; or

2> 	if threshHigh is included in remoteUE-Config within SystemInformationBlockType19; and the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHigh by hystMax (also included within remoteUE-Config):

3> 	consider the threshold conditions to be met (entry);

1>	else:

2> 	if threshHigh is included in remoteUE-Config within SystemInformationBlockType19; and the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHigh (also included within remoteUE-Config):

3> 	consider the threshold conditions not to be met (leave);